Ф.7.03-03

Yeskendirova M.M.

METHODICAL INSTRUCTIONS

to laboratory works on the discipline

“Technology of household chemical goods production”

Shymkent, 2015 y.

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

SOUTH-KAZAKHSTAN STATE UNIVERSITY named after M. AUEZOV

Department “Chemical technology of inorganic substances”

Yeskendirova M.M.

METHODICAL INSTRUCTION

to laboratory works on the discipline

“TECHNOLOGY OF HOUSEHOLD CHEMICAL GOODS PRODUCTION”

for students of speciality

5B072000 – “Chemical technology of inorganic substances”

Shymkent – 2015 y.

UDK 661.185 (075.8)

Compilers: Yeskendirova M.M. Methodical instruction to laboratory works on the discipline “Technology of household chemical goods production”. – Shymkent: M.Auezov SKSU, 2015. – 48 p.

Methodical instruction has been developed according to requirements of a curriculum and a program of the discipline “Technology of household chemical goods production” and includes all necessary information for carrying out of the course of laboratory works.

Methodical instruction contains theoretical information about physicochemical properties of surface-active substances, ways of detergents production and techniques of synthesis and analysis.

Methodical instruction is intended for students of speciality 5B072000 – “Chemical technology of inorganic substances”.

A reviewer: Seitmagzimova G.M. – a candidate of technical sciences, an associate professor of “Technological processes and apparatuses” department of M.Auezov SKSU.

Methodical instruction was discussed and was recommended for print by a meeting of the “CTIS” department (protocol # 6 14.01.2015 y.), the committee on innovation training technologies and methodical provision of higher school of chemical engineering and biotechnology (protocol # 6 15.01.2015 y.).

Methodical instruction was recommended for print by Methodical Council of M. Auezov South-Kazakhstan State University, protocol # ___   _______2015 y.

© M. Auezov South-Kazakhstan State University, 2015 y.

Redactor: Yeskendirova M.

CONTENTS

|  | p. |
| --- | --- |
| Preface | 6 |
| Laboratory work # 1 Determination of stability of washing agents to ions of alkali-earth and heavy metals and their stability in acid mediums | 7 |
| Laboratory work # 2 Determination of dispersive power and surface tension | 11 |
| Laboratory work # 3 Determination of detergents’ critical micelle concentration, wetting ability and pH | 17 |
| Laboratory work # 4 Technology of synthetic detergent production | 23 |
| Laboratory work # 5 Quality control of toilet soap | 32 |
| Engineering safety for work in a chemical laboratory | 43 |
| Literature | 45 |

Preface

Methodical instructions for carrying out of laboratory works on the discipline «Technology of household chemical goods production» contain theoretical information concerning to determination methods of physicochemical properties of surface-active substances such as surface tension, wetting ability, dispersive ability, chemical stability, etc., methods of synthetic detergent and soap production, and also techniques of synthesis and analysis of various kinds of detergents.

Not less important problem is acquisition of skills of laboratory experiment which is the first stage in development and perfection of methods of chemical technology, and also ability to estimate technological indicators of chemical technological processes.

During laboratory researches students should perform laboratory works independently with use of various chemical and physicochemical methods. It will allow receiving skills of independent approach to decision of specific targets, concerning engineers-chemists-technologists, and also to carrying out of research work. Described conditions of laboratory works carrying out are as much as possible approximated to industrial modes.

Methodical instructions cover contents of the course “Technology of household chemical goods production” and contain 5 laboratory works. Each laboratory work includes a purpose and problems, brief theoretical data, a description of equipment, technical and tool means, an installation description, a technique of a work and analyses performance, a procedure of calculation of a process’ technological indicators and control questions. Laboratory works are carried out by groups on 2-3 persons. Before performance of a work students should receive an admission to a laboratory work. After a work performance each student makes out and protects a report on a laboratory work.

Methodical instructions are made on the basis of requirements shown to graduates of higher educational institutions on a speciality «Chemical technology of inorganic substances».

Laboratory work № 1

DETERMINATION OF STABILITY OF WASHING AGENTS TO IONS OF ALKALI-EARTH AND HEAVY METALS AND THEIR STABILITY IN ACID MEDIUMS

1.1 A purpose of the work: studying of methods of determination of washing agents’ stability in acid mediums and to metal ions.

1.2 Theoretical data

Washing action mechanism.

Washing action (removal of pollutants) in a general view it is possible to present as an adsorption-desorption series-parallel process in which a surface S, a pollutant P and a washing substance solution W participate:

к1

SP + 2W  SW + WP

Rates of removal and sedimentation of a pollutant can be presented by the formulae:

1 = К1(SP)(WP)2; -1 = К-1(SW)(PW),

in which the rate constants of direct and inverse processes are equal:

К1 = z exp(-G1/RT); K-1 = z-1 exp(-G-1/RT).

In equilibrium conditions equilibrium constant can be calculated by the equation:

Кeq = К1/K-1 = ехр[(G-1 - G1)/RT] or -G = -G1 + G-1 = RTlnKeq

Therefore, for removal of pollutants it is necessary to reduce the system’s energy by the G value.

Let a polluted surface is characterized by a free energy G1, and a solution of a washing surface active substance – by a free energy G2. When the polluted surface gets to the surface active substance (SAS) water solution, adsorption processes in all three phases S, P and W and between them start to progress which come to an end with an equilibrium establishment, and the system gets a free energy G3. In the process a surface covered with a SAS layer with a free energy G4 is formed. It is impossible to determine absolute values of the free energies, we will write down only energy change:

G1 + G2 =  G1;		G2 + G3 =  G2;		G3 + G4 =  G3.

At immersion of the polluted surface in the washing solution two interfaces P-S and S-air collapse and are replaced with two other surfaces P-W and S-W. Therefore the pollutant’s removal process can be considered as loss of contact between the interfaces. Adsorption on the interface is characterized by Gads, and the mutual mixing of phases, including formation of a mesomorphic phase, is characterized by Gabs. When the pollutant is removed using any physicochemical method, an energy barrier to an equilibrium establishment is a change of free adhesion energy G3. Taking into account the above

G2 = Gads + Gabs; G3 = Gа

Change of free energy Gа, connected with effect of adhesive forces Wa

Gа = Wa S = PW + SW – PS,

where S - a contact surface.

It is obvious, other things being equal a limiting stage of pollutants’ removal is formation of adsorption layers on an interfacial area, destroying connections of the pollutants with the surface. In that case, the total change of free energy G is characterized by adsorption’s work and spreading of a washing substance.

It is possible to distinguish four basic stages in the mechanism of washing action to which dissolution water-soluble pollutants precedes:

- wetting by a washing agent solution with formation of adsorption layers on a surface of a fabric and pollutants and transfer of these layers in the activated condition; as a result of this phenomenon, spreading the SAS layer on microcracks and crushing of particles occur;

- removal of the pollutants from the surface by dispersion (as a result emulsification, slurrying, foaming) and solubilization;

- containment of the pollutants in volume of the solution owing to emulsification, solubilization and, partially, foaming and slurrying;

- shift of the equilibrium “micelle monomer” towards the formation of adsorption layers at sufficient concentration of the SAS.

The choice of washing agents is based on the empirical determination of washing (bleaching) action taking into account a reverse process – sedimentation of pollutants on a surface. Estimation of washing action (WA) is spending by comparison of whiteness of a pollution-free fabric (Т0), polluted one (Т1) and washed one (Т2) under the formula (%):

WA = (lnT2 - lnT1)/(lnT0 - lnT1)100

Often a pollution-free fabric’s whiteness is compared to whiteness of titanium dioxide accepted for the standard. Practically WA values seldom exceed 80 %.

Washing action also depends on рН of a medium. The maximum of washing action of any SAS is observed at рН = 3-4 (except alkylcarboxylates, for which in this case WA = 0). At рН = 6-7 washing action has the minimum value, at рН = 8-10 it raises, and then sharply decreases.

Washing and adsorption properties depend on the nature and position of a hydrophilic group, the nature of a hydrophobic part of a molecule and intermediate groups. So, substances with carboxyl group are characterized by high ability to keep pollutants and foam stability. Their disadvantages are instability in hard water, propensity to decomposition by stronger acids, absence of activity in neutral environments, alkalinity owing to hydrolysis of a salt, a high value of critical micelle concentration (CMC), ability to foaming only at high temperatures, bad washing off at low temperatures.

The surface active substances, containing sulphate groups, have the following advantages: stability in hard water, chemical stability, high moistening and foaming ability (at stability of foam); their solutions have neutral pH, activity in a wide рН interval and low affinity to cellulose and albuminous fibres. However, the washing action of these substances is lower than the washing action of carboxylates, they keep pollutants worse, collapse in acid environments, can cause yellowing of fabrics.

The surface active substances containing sulfonate groups are characterized by high stability in hard water, high emulsifying ability, fast and plentiful foaming at small concentration, chemical stability, good wettability and solubility, have neutral reaction and dissolve fats. However, their washing action and ability to retention of pollutants are low, they irritate skin, interact with albuminous products, cause of yellowing of fibres.

Washing surface active substances differ not only with hydrophilic groups and length of a hydrocarbon radical, but also degree of its branching, presence of double bonds and aromatic rings in a chain. The longer a hydrocarbon chain, the better washing ability, stability of foam, activity at high temperature and ability to retention of pollutants. However, such properties as foaming, solubility, stability in hard water, wetting, and emulsifying ability worsen with the radical length increase. The more branched a hydrophobic part of a SAS’s molecule, the lower its washing action. Presence of a double bond improves solubility and washing action, presence of an aromatic group increases hydrophobic property; thus washing action of monoalkylarenesulfonates is higher than washing action of alkylsulphates. Introduction of alkyl substituents in an aromatic ring reduces washing action.

At manufacture of synthetic detergents and at use of household chemical goods an important characteristic, which determines their washing ability, is stability in acid environments and to ions of various metals. For production of washing solutions hard water is applied very often. Hardness salts, containing in the water, can have an influence on synthetic detergents’ stability.

1.2.1 Order of the work performance

Stability of synthetic washing substances is determined:

- in a solution of calcium and magnesium salts because hard water contains these compounds;

- in solutions containing salts of the metals, which are used for making various equipment (copper, aluminium, iron, lead, nickel and zinc), because at certain conditions these salts can transfer in synthetic detergent solutions.

For the analysis it is necessary to prepare 1 % solution of an investigated substance and 1 % solution (calculating on a waterless substance) of one of the salts (table 1).

Table 1.

| Salt | Sample for preparation of 100 g of 1 % solution, g | Salt | Sample for preparation of 100 g of 1 % solution, g |
| --- | --- | --- | --- |
| CaCl2∙2H2O | 1.327 | Mg4SO4∙2H2O | 2.048 |
| CuSO4∙5H2O | 1.564 | Al2(SO4)3∙18H2O | 1.948 |
| BaCl2∙H2O | 1.173 | Pb(NO3)2 | 1.00 |
| ZnSO4∙7H2O | 1.781 | Ni(NO3)2∙6H2O | 1.591 |

1.2.2 Analysis carrying out

To pour 10 ml of 1 % solution of an investigated washing substance in a conic 50 ml flask. To this solution to add on drops a solution of one of the listed salts before turbidity or precipitation. To mark volume of the added solution. Then the flask is heated up to boiling. If the solution is clarified, it is necessary to add no more than 10 ml of the salt solution; if the precipitate is not formed anymore and the solution does not foam, the addition of the salt solution is stopped.

Stability of washing substances to metal ions is calculated by multiplication of a volume of the added salt solution on 10.

The obtained data are tabulated.

Table 2. Experiment results

| № | Sample | Stability to ions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Na2+ | Mg2+ | Cu2+ | Al3+ | Ba2+ | Fe3+ | Pb2+ | Zn2+ | Ni2+ |

Synthetic surface-active substances are often applied at fulfillment of technological processes in acid mediums. Therefore at a choice of surface-active substances it is necessary to know their stability in acid mediums. For determination of this parameter a 1 % SAS solution is boiled in a flask with a backflow condenser within 15 minutes at various content of an acid in the solution, for example: 0,1 %, 1 % etc.

After the boiling it is necessary to note an external appearance of the solution especially such factors as turbidity, separation of an oily layer, foaming termination.

The obtained data are tabulated (table 3).

Table 3. Experimental results

| № | Sample | Acidity, % |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0,1 | 1 | 5 | 10 |

According to the experimental results we make a conclusion about stability of a researched SAS to ions of various metals and its stability in acid mediums. It should be compare similar data for various washing substances.

1.3 Control questions:

1. Determination technique of washing agents’ stability in acid mediums.

2. Influence of acid concentration on stability of synthetic detergents.

3. Stability of washing agents to metal ions.

4. Comparative characteristic of synthetic detergents with various structures on their stability.

5. SAS classification.

6. Basic properties of SAS.

7. Washing action mechanism.

8. Influence of various factors on washing ability of synthetic detergents.

Laboratory work № 2

DETERMINATION OF DISPERSIVE POWER AND SURFACE TENSION

2.1 A purpose of the work – studying measuring methods of surface tension and suspending ability of detergent solutions

2.2 Theoretical data.

2.2.1 Suspending ability

Microheterogeneous thermodynamically unstable disperse systems, which liquid continuous phase contains uniformly distributed fine solid particles, are named suspensions. Their thermodynamic instability (liability to aggregation) is caused by surplus of total surface energy of particles. Relative stability of a dispersed solid substance in a liquid medium depends on a ratio of attractive and repulsive forces. The repulsive forces arise thanks to the presence of a double electric layer around particles. This layer is formed as result of dissociation of surface functional groups or owing to adsorption of electrolytes and polar molecules on an interphase surface. The double layer consists of a charged surface and particles of opposite charged diffuse layer of counter-ions.

Particles of like charge, naturally, are repulsed. Attractive and repulsive forces are shown at approaching of particles. Interaction energy is determined using a ratio of forces between the particles. Along with repulsion forces of Van der Waals molecular attraction start to operate. If they exceed repulsion forces, a suspension is unstable and collapses with the same rate with which particles collide. If energy barrier created by repulsion forces is comparable with thermal energy кТ, dispersion of solid particles collapses slowly; if energy barrier is considerably higher, destruction rate of suspensions sharply decreases, can even tends to zero. At adhesion of particles there is coagulation of suspensions with separation of a solid phase in sediment. The energy barrier, providing stability of dispersion, arises at value of the double layer potential equal ≈ 20 mV.

The following factors cause stability of disperse systems:

- decrease of surface tension at occurrence of a double layer on a surface of particles;

- interaction of particles with a continuous medium and adsorptions of surface-active substances;

- decrease of aggregation rate connected with change of viscosity and density of a continuous medium, and also with expenditure of energy for destruction of formed surface films.

Suspending ability of detergents can be measured according to their ability to disperse calcium soaps. For this purpose it is necessary to determine a minimum quantity of a detersive surface-active substance which at addition to a calcium soap solution of the set concentration in water of certain rigidity does not cause formation of soap flakes. For traditional SAS suspending ability is expressed as a quantity of a dispersing agent (g) which is necessary for dispersion of 100 g of calcium oleate. This value decreases in the line: oxyethylized tridecanol, n = 15 (1,1) > oleyl [sodium N-methyl (ethyl) sulfonate] amide (1,3) > sodium dodecyl sulphate (25) > sodium dodecyl benzole sulfonate (42) (concentration of calcium chloride solution is 0,882 g/dm3 and temperature is 200С).

The best stabilizers of suspensions are diphilic substances with short aliphatic chains and a branched structure. Formation of suspensions in washing process promotes confinement of pollutants in solution volume. Besides, stability of suspensions plays an essential role at production of detergents with high content of a solid phase.

2.2.2 Surface tension and surface activity

The condition of molecules of a surface liquid layer differs from their condition in volume. Molecules in volume are surrounded from all sides by the same molecules, but molecules in a surface layer interact with molecules of two different phases. Therefore intermolecular forces in a surface layer are not counterbalanced (differ from zero) and are directed towards a phase with which molecules interaction is stronger. The resultant of these forces directed towards this phase, will tend to reduce an interface (phase boundary) of phases, creating surface tension σ.

The result of these interactions is surface energy Gsur. In izobaric-isothermal conditions surface energy is determined as a product of the area of a surface S on surface tension: Gsur = σ ∙ S. As any system tends to decrease of Gsur, in real solutions constantly there is competitive activity for a place in a surface layer.

Surface-active substances at dissolution in water not only aggregate in solution volume, but, as it has already been told, spontaneously concentrate with heat liberation in a surface layer that leads to partial or full replacement of water molecules on a border of a solution with air by adsorbed diphilic molecules. Replacement of molecules on a surface of a solvent by less polar molecules, increase of intermolecular distances in a surface layer owing to fast and reversible exchange by molecules between the surface and volume – such are primary causes of surface tension decrease in the presence of SAS; this property has been named as surface activity.

Surface tension isotherm (fig.2.1) may serve as a characteristic of surface activity. The isotherm contains three sites: at the beginning at growth of concentration there is a gradual decrease σ, then surface tension abruptly decreases, and in the end value σ practically does not change. The reason of the first site is the adsorption of SAS molecules in a surface layer before achievement of limiting adsorption, at which the molecules’ concentration in the surface layer can exceed concentration in volume on several degrees, and also parallel decrease of water surface tension value from 72,7 to 30 mN/m. Energy gain from transition of SAS molecules from solution volume on a surface is named work of adsorption:

W = RT ln S/(∙C),

where S - surface concentration of SAS,  - thickness of a surface layer, which is usually assumed 10-10 m, as radius of action of intermolecular forces ≈ 6 ∙10-10 m.

,mN/m				S, kmole/m3

С, mole/m3					            С, mole/m3

a)							b)

Fig.2.1. Surface tension isotherms in coordinates σ - C (a) and S - C (b).

The second site of the graph σ = f(C) is obviously connected with fast exchange by SAS molecules between an extremely saturated surface layer and formed associates (including micelles) as residence time of molecules in a spherical micelle is too small.

Unlike a system liquid-gas, systems liquid-liquid are characterized by interfacial tension. For such systems surface activity is caused by orientation of aliphatic radicals towards an organic phase, and polar groups – in a water phase. It is obvious, that orientation of the same detergent is various on a border water-gas or hydrocarbon–gas. This distinction is expressed by the Antonov’s empirical rule, that may be written in the simplified kind as follows: 12 = 1 – 2, where 12 - interfacial tension, 1, 2 - surface tension of mutually saturated liquids forming interfacial surface. It is obvious, that a surface layer in such systems includes both hydrophobic, and hydrophilic groups. Work of adsorption from a water phase in an organic liquid can be calculated under the Abramson’s formula:

Wa = 1,76 nCH – (50 – 0) 10-6 SM,

where 1,76 – a difference of free energies of C–H bonds in water and an aliphatic hydrocarbon, kJ/mol; nCH – a number of C–H bonds of a surface-active substance which have passed in an organic phase; 50 – maximum surface tension of aliphatic hydrocarbons, mN/m; 0 – surface tension of an organic liquid, mN/m (1 mN/m = 1 mJ/m2).

It is considered, that a water phase contains 4-12 of С-Н bonds of a SAS molecule that is less, than at micelle formation. Surface activity is characterized not only the relationship  = f(C), but also a function Gr = -d/dC at С0. Taking into account the Gibbs and Langmuir equations this dependence may be expressed the following equation:

Gr =  RT ехр(W/RT),

but for systems gas-liquid Gr is estimated according to the formula:

Gr = (Р – М) / С,

where М – minimum σ value; C – concentration corresponding to limiting adsorption, CCM (critical concentration of micelle formation) is often assumed in the capacity of C.

Thus, surface activity is determined by work of adsorption and consequently depends on nature of SAS, medium, concentration, temperature, electric potential of a surface  (d/d = q, where q - charge density). Criterion of surface activity is change of σ.

2.3 The laboratory work procedure

2.3.1 Determination of dispersive power

Dispersive power of detergents’ solutions in relation to calcium soaps is ability of detergents to prevent formation of calcium soap clots or to keep them in a fine-dispersed state.

Dispersive power in respect of sodium oleate is expressed in percentage. The less synthetic detergent amount required for dispersion of calcium soap is, the higher dispersive power is. For determination of dispersive power it is necessary to prepare an initial solution of sodium oleate (100 g of 100 % of sodium oleate is dissolved in 1 litre of distilled water) and to use hard water (calcium rigidity is 14,26 mg-eq/l:12,12 mg-eq/l and magnesium rigidity is 2,14 mg-eq/l).

Tests are performed in test tubes in diameter of 30 mm and volume of 50 ml at temperature 250С + 20С.

Prepare several solutions of the investigated substance in distilled water: Т1 - 1 %; Т2 - 2,5 %; Т3 - 10 % solutions.

The first dilution: 25 ml of a soap solution (it corresponds to 2,5 g of sodium oleate) is mixed in a test tube with 5 ml of the solution Т1; the solution is diluted by distilled water to 50 ml and is carefully mixed.

The second dilution: 5 ml of the solution prepared at the first dilution is placed in a test tube, is diluted by hard water to 50 ml and is mixed. If sediment does not precipitate, the third dilution is performed. For this purpose 5 ml of the solution after the second dilution is placed in a test tube, is diluted by 50 ml of hard water and is mixed.

If calcium soap is not separated, place 25 ml of the soap solution in a test tube, add smaller quantity of the solution Т1 (< 5 ml) and fulfill tests with smaller quantities before the first separation of calcium soap.

If precipitate is separated already at the second dilution, it is necessary to repeat tests with greater quantities of the solution Т1 ( 5 ml) before the first separation of calcium soap.

If the experiment shows that the 1 % solution (Т1) does not prevent the precipitate sedimentation it is necessary to apply more concentrated solution (i.e. Т2 or Т3).

Dispersive power in relation to 100 % sodium oleate is expressed (in %) under the formula:

Mass of the investigated substance  100

Д = __________________________________________

Mass of sodium oleate (100 %)

Then compare the obtained results and make conclusions about dispersive power of various synthetic detergents.

Table 4.

| № | Sample | Mass of the investigated substance, g | Mass of sodium oleate, g | D, % |
| --- | --- | --- | --- | --- |

2.3.2 Determination of surface tension

The following methods are usually applied for measurement of surface tension: stalagmometric method, bubbles’ maximum pressure method, method of a ring detachment, etc.

Determination of surface tension by a stalagmometer. A size of a drop, formed at the slow outflow of a liquid from a round aperture with known radius at the moment of its detachment, is determined by the liquid’s surface tension since the drop weight is equal to force of the surface tension, tending to keep a drop. This force (P) is equal to 2r.

If the drop weight is known, it is possible to calculate the surface tension value.

Usually do not weigh a separate drop, but calculate a number of drops (N), corresponding to a certain volume (V) of the outflow liquid:

P = V  d  g / N

where d – a liquid density, kg/m3; g - gravity acceleration, m/s2.

Replacing P, we obtain the following equation:

V  d  g / N = 2  r,

For equal volumes of two liquids with density d1 and d2, effluent from the same aperture, there is a ratio:

1 /2 = N2  d1 / N1  d2

If the liquids’ densities differ too little, the ratio assumes more simple kind:

1 / 2 = N2 / N1

If surface tension of one liquid and a number of drops corresponding to certain volume are known, it is possible to determine surface tension of other liquid, having measured a number of drops of this liquid in the same volume.

It is necessary to perform three-four experiments. The result is the arithmetic mean value of these measurements.

Surface tension is calculated under the formula:

=  0  N 0 / N 1

where 0 – water surface tension, N/cm;

N0 – number of water drops;

N1 – number of drops of the investigated solution.

The obtained data are tabulated. Surface tension values of various detergent solutions are compared themselves and with water surface tension.

Table 5. Surface tension values of various detergent solutions.

| № | Sample | V0, ml | 0 10–5, N/cm | N0 | V1, ml | N 1 | 0 N 0 = --------- 10 –5 , N 1 N/cm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |

2.4 Control questions:

- Reasons of decrease of solutions’ surface tension in the SAS presence.

- Influence of SAS concentration in a solution on surface tension.

- Dependence of disperse system stability on various factors.

- Essence of stalagmometric method of surface tension determination.

- Determination technique of detergents’ dispersive ability.

Laboratory work № 3

DETERMINATION of DETERGENTS’ CRITICAL MICELLE CONCENTRATION, WETTING ABILITY and pH

3.1 A purpose of the work: acquaintance with determination methods of detergents’ critical micelle concentration, wetting ability and рН value.

3.2 Theoretical information

3.2.1 Micelle formation

Micelle formation – is a reversible process of molecules’ association in a solution volume with formation of thermodynamically stable coarse particles. A reason of micelle formation is an asymmetric structure of molecules, which at dissolution in water show opposite tendencies. At formation of micelles in water hydrophobic parts of molecules make a hydrocarbon center (fig.2.1), surrounded with a layer of polar groups. In non-polar mediums micelles are formed in which polar groups are placed inside the micelles, and hydrocarbon chains are turned outside.

The association of molecules occurs with heat liberation that testifies to power benefit of this process, i.e. for it the condition G  0 should be satisfied.

A micelle is a microheterogeneous particle possessing excess surface energy. This energy has to be compensated by heat motion, i.e. growth of an entropic component in the equation G = Н – ТS.

Micelles can have spherical, ellipsoid, lamellar and cylindrical forms. Quantity of SAS molecules forming a micelle is named aggregation number. Aggregation of diphilic molecules in micelles can be considered or as gradual association of SAS molecules and their aggregates, or as phase transition. Micelles can have different sizes. A micelle size monotonously decreases with decrease of a monomer’s concentration and size. A total sum of molecular weights of the molecules (ions) containing in a micelle is named a micelle mass. Each diphilic substance can be characterized by a minimal spherical micelle with the radius approximately equal to a length of a stretched polymethylene chain.

A micelle can be imagine as a coarse multiply charged ion, and at dissociation of ionic surface active substances (hydrophilic groups are located on the external sphere of a micelle) only 60-70 % of counter-ions are kept at a surface, the others form a diffusion layer.

At dissolution in water with concentration increase the balance is formed, which shows distribution of washing SAS between micelles and their molecular solution; the balance monomers – micelles is formed quickly. Rate of exchange by a SAS molecule between a water solution and a micelle makes 106 mol-1s-1, rate of return process is 105 … 108 s-1. Thereby lifetime of a molecule in a micelle or in its own hydrocarbon phase makes 10-7-10-6 s.

Concentration, at which micelles are formed in a solution, being in thermodynamic balance with monomer molecules, is called as critical micelle concentration (critical concentration of micelle formation, CCM) and serves as a certain criterion of this process. At CCM achievement it can be observed jump of physicochemical properties of SAS solutions. It serves as a basis for CCM determination; CCM value can be found also from dependence C = f(T), i.e. owing to finding of a triple point coordinate.

At calculation of mole fractions of monomeric (М1) and aggregated (М2) molecules it is supposed that concentration of monomer molecules Сk is equal to CCM:

М1 = Сk/С;		М2 = (С – Сk)/С,

where C – total SAS concentration.

Taking into account the balance monomer  micelle and the fact, that micelle formation is observed at the total SAS concentration ССk, we obtain the formula:

(С – Сk)/К = 1/(mk) + (С – Сk)/m,

which is named micelle formation isotherm and allow to define the dependence K = f(С – Сk) (m – association limit number).

A chemical structure of SAS molecules – their polar group and hydrocarbon radicals – has the basic influence on the CCM value in water solutions:

|  | CCM, mol/dm3 |
| --- | --- |
| 2-(Oktyloxypentaoxyethylene) ethanol С8Н17(ОСН2СН2)6ОН | 9,9 |
| 2-(Oxyloxydioxyethylene) ethanol С8Н17(ОСН2СН2)3ОН | 7,5 |
| 2-(Oktyloxy) ethanol С8Н17ОСН2СН2ОН | 4,9 |
| Sodium salt of oktyloxysulfonic acid С8Н17ОSO3Na | 0,13 |
| Oktyldimethyl-N-oxide С8Н17(СН3)2 N+ –О – | 0,26 |

In a simple case we will consider only one value as a variable – length of a hydrocarbon radical RY, in which Y – a hydrophilic group of water-soluble diphilic substances. In the case of micelle formation in SAS water solutions at invariability of conditions of the process, its quantitative characteristic definitely depends on R.

CCM depends not only on a hydrocarbon radical’s length, but also on degree of its branching. CCM increases if a branched chain contains fragments with unsaturated bonds and hydroxyl groups. Non-ionic detergents are characterized by considerably smaller CCM values. Increase of a hydrophilic group’s polarity promotes CCM increase. Addition of electrolytes makes the same impact, as well as increase of a hydrocarbon radical’s length. Depending on concentration of counter-ions Сi change Сk can be presented so:

ln Сk = - Kэ ln Сi + K0,

where Kэ – a constant equal approximately 0,4-0,6; K0 – a constant.

The increase of SAS concentration leads to greater fixation of counter-ions, therefore hydration of micelles increases. At addition of hydrocarbons and spirits the CCM value decreases proportionally to a number of carbon atoms in their chain. For ionic surface active substances CCM values are defined by metal ions concentration, for cation active SAS – by anion concentration. Properties of non-ionic SAS practically don’t depend on presence of ions in a solution (to certain concentration). For mixed solutions of anion-active and non-ionic surface active substances the CCM value gradually increases at addition of an anion-active component to 90 %, then it is observed the abrupt junction to high CCM values, characteristic for anion-active SAS. Mixtures of anion-active and cation-active SAS at certain ratios do not show at all the properties inherent in detergent solutions.

Solubilization.

Solubilization – is an equilibrium process of dissolution in micelles of insoluble in water (or in other solvent) substances. Solubilization systems are thermodynamically stable isotropic transparent or opalescent liquids. There are systems, in which a continuous phase is a hydrocarbon or water. A SAS in micelle form, causing the solubilization phenomenon, is named a solubilizer or a wedding agent, but a solubilization substance – a solubilizate. Depending on a solubilizate structure, different binding centres of a micelle can take part in the solubilization process.

3.2.2 Wetting

An intermolecular interaction on an interface in systems a liquid - a solid body or two immiscible liquids is referred to wetting phenomenon. Wetting effect is defined by a relation of intramolecular forces of a liquid (cohesion) and intermolecular forces between an interacting liquid and a solid body or other liquid (adhesion). Degree of molecules’ interaction on an interface of two phases is estimated according to adhesion work Wa, related to unit of the area of the formed (broken off) interphase surface. For systems a liquid – a liquid Wa is calculated using the Dupre’s equation:

Wa = 1liq + 2liq – 12liq,

where 1liq, 2liq – surface tension of liquids on a border with air or other gas; 12liq – interfacial tension.

For systems a liquid – a solid body an analogous equation is used:

Wa = l + s – l-s,

where l, s – surface tension of liquids on a border liquid-gas and liquid-solid body; l-s – interfacial surface tension; in some cases adhesion work can be equal to cohesion work Wc (interaction between molecules of the same phase).

Interaction between phases is also shown at liquid spreading on the surface. At achievement of surface forces’ equilibrium on a phase boundary liquid (l) – solid body (s) – gas (g) this interaction can be estimated according to a limiting wetting angle  using the Jung’s equation:

sg = ls + lg∙ cos

For example, at spreading oil (o) on water, the interfacial surface water-air is replaced by the interfacial surface oil-water and the surface oil-air equal in area. Spreading is characterized by spreading factor Кspr = w – (o + ow), or otherwise Кspr = Wа – Wc.

At dissolution of washing SAS in water there is formation of adsorption layers, change of equilibrium of surface forces and wetting angles. So, at removal, for example, oil from a solid surface, interfacial tension does not change, but there is decrease of free energy at the expense of reduction of interfacial tension on the boundary oil-solution, area of the surface covered with the oil, and wetting angle of the solid surface by the water solution. In so doing wetting angle of the solid surface by the oil increases, and the oil drop separates from the surface. Taking into account the above-stated, SAS wetting ability is estimated using values Wа, Wc, Кspr, cos . Influence of SAS concentration on wetting is also estimated by means of wetting isotherms cos  = f(C), which in case of hydrophobic surfaces are similar to the dependence  = f(C). Wetting isotherms of hydrophilic surfaces have a maximum, which is caused by formation of a double SAS layer and according to concentration coordinate is close to limiting adsorption.

As at wetting SAS molecules participate not only in adsorption (thermodynamic factor), but also in diffusion (kinetic factor), it is necessary to take into consideration the wetting time. It is supposed, that for SAS having a high CCM value, wetting depends on surface tension value, and for detergents with a low CCM value, the wetting rate is defined by disintegration of micelles on molecules and their diffusion. Wetting depends also on concentration of SAS solutions; it is optimally shown in the area of critical micelle concentration.

As wetting process is defined by formation of adsorption layers, the best wetting agents are those diphilic substances, which cover a big surface at smaller concentration, i.e. compounds with a branched chain. It is established, that SAS having 10-13 of carbon atoms in an aliphatic chain have the best wetting ability.

In water solutions increase of polarity of hydrophilic groups improves wetting. In distilled water and in the same conditions salts of carboxylic acids have the maximum wetting ability among ionogenic SAS. Wetting increases at addition of electrolytes in solutions of ionogenic detergents, that is caused, as believe, by influence of counter-ions improving spreading. Increase of ethylene oxide groups’ quantity in a corresponding non-ionic SAS lowers wetting ability. Thereby, wetting is a basic stage of washing process.

3.3 Order of the laboratory work performance

3.3.1 Determination of detergent solutions’ рН value

Value of рН of detergent solutions it is possible to determine using a pH instrument with a glass electrode. рН value of buffer solutions must be as close as possible to рН value of investigated solutions. The buffer solutions must be prepared very carefully using twice distilled water, which was distilled over КМО4, and must be stored in well closed vessels.

At measurement of рН of SAS solutions it is impossible to use indicators since surface active substances influence on colouring change of usual indicators. Apparently, it is caused by the same absorption phenomena, which cause changes of colouring of these solutions at critical micelle concentration.

Measuring рН of SAS solutions by means of glass electrodes also can lead to errors. First of all it concerns to solutions of cation-active substances of high concentration.

The received data are tabulated.

Table 6.

| № | Sample | рН | Note |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |

3.3.2 Determination of anion-active detergents’ critical micelle concentration

A technique of CCM determination is based on the fact that the rhodamine dye 6 loses fluorescence if more concentrated solution of an anion-active substance is diluted to the concentration lower CCM at preservation of concentration of the dye.

Reagents:

- Solution N1 – 0,1 g of the rhodamine dye  6 is dissolved in 1 litre of distilled water.

- Solution N2 – 0,5 g of the rhodamine dye 6  is dissolved in 1 litre of distilled water.

- Solution of an anion-active substance in distilled water; the solution concentration should approximately exceed the expected CCM value more than in two times.

Place 5 ml of the anion-active substance solution in a volumetric flask with volume of 50 ml and add 5 ml of the dye solution N1. Then gradually add the dye solution N2 from a burette. Titration is performed in darkness at ultra-violet illumination before disappearance of yellow-green fluorescence of the test sample examined in a narrow layer - in a neck of the volumetric flask. CCM value is calculated using the following formula:

е   100                                                  е  100

CCM = --------------------- , mol/l   or   CCM = ---------------- ,  %

(a + b + x)  М                                      (a + b + x)

where

е – mass of an anion-active substance in the flask for the titration, g;

a – volume of the anion-active substance solution, ml – 5 ml;

b – volume of the added dye solution N1, ml;

х – volume of the dye solution N2 spent for the titration, ml;

M – molecular weight of the anion-active substance.

The received data are tabulated and we do conclusions about CCM value for various detergents.

Table 7.

| № | Sample | е, g | a, ml | b, ml | х, ml | CCM, % |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

3.3.3 Determination of wetting ability

There are different determination methods of detergent solutions’ wetting ability. One of them is described below.

Drevs – Clarcson method. A wetting ability criterion in the given method is submersion time of a balloon of a cotton yarn on the bottom of a cylinder at the expense of displacement of air by a SAS solution.

A balloon of non-boiled grey cotton yarn (weight is 5 g) is suspended on a copper hook (weight is 15 g, width of an aperture in the hook is 6,3 mm). This hook bears cargo in 20 g (fig. 1).

Place 500 ml of the test solution in a chemical glass having volume of 600 ml, heat it to 250С and pour into a graduated cylinder. Immerse the balloon of yarn with the cargo in the cylinder so that the hook was in contact with the surface of the test liquid, let off it and turn on a stopwatch. Mark the time at which the balloon of yarn will drop and the hook will reach the cargo level.

It is necessary to fulfill not less than five measurements using each time a new balloon of yarn. The result is the average of these measurements. The less submersion time of the balloon of yarn is, the higher wetting ability of the test solution is.

3.3 Control questions

- Classification of surface-active substance.

- Basic properties of SAS: surface tension and surface activity; emulsifying properties; foaming ability; wettability; suspending action.

- Critical micelle concentration.

- Influence of SAS concentration on wetting ability.

- Essence of the micelle formation process.

- Influence of various factors on CCM value.

- Solubilization phenomenon.

Laboratory work № 4

Technology of synthetic detergent production

4.1 A purpose of the work: acquaintance with technology of synthetic detergent production, making of a synthetic detergent composition and a finished product analysis.

4.2 Theoretical information

Preparation of a composition at manufacture of powder-like synthetic detergents is an important operation of technological process influencing on a finished product’s quality and work efficiency of a spray-type drier. A composition should have a stable qualitative and quantitative composition and a high mobility, i.e. it should possess optimum viscosity allowing to spray it’s by mechanical atomizers.

A composition’s preparation process consists in batching of liquid and free-flowing components, their mixing in reactors-mixers, filtering, deaeration and homogenization.

Loose raw materials used in compositions of powder-like synthetic detergents possess various solubility and ability to form crystalline hydrates that influences on viscosity and fluidity of the compositions.

Synthetic detergents’ compositions represent water suspensions. In the course of a composition’s preparation solubility of inorganic salts increases with rise of temperature. However it is limited to formation of hydrates.

A ratio of insoluble and soluble powdery materials in a composition makes a big impact on properties of the composition and a produced synthetic detergent. This ratio influences on the composition’s viscosity, density and fluidity. The higher this ratio is, the worse these characteristics are.

The following factors have the greatest influence on qualitative indicators of synthetic detergent compositions: temperature, water content, a minimum quantity of sodium tripolyphosphate and a content of the 1st form in it, minimum preparation time, a share of hydrotropic additives and efficiency of homogenization.

Viscosity of a composition decreases with rise of temperature and increase of water content. It is a consequence of increase of the salt solubility in the composition. At reduction of water content and decrease of a composition’s preparation time (at the same rise of temperature) hydration process decelerates and content of sodium tripolyphosphate hexahydrate, causing decrease of a composition’s viscosity, also decreases. Viscosity of a composition considerable depends on content of the 1st form of sodium tripolyphosphate; if this indicator is 35 %, there is the hydration of sodium tripolyphosphate and the formation of a viscous fine-crystalline composition. The most viscous composition is formed at the maximum quantity of the 2nd form that leads to formation of coarse crystals of sodium tripolyphosphate hexahydrate.

The sodium tripolyphosphate hydration is a necessary stage at the preparation of a synthetic detergent composition, because the presence of incompletely hydrated salt is the reason of low flowability and high miscibility of a finished product.

Properties of a composition depend on carrying out aeration and homogenization of the composition in the course of its preparation. At intensive mixing of liquid and solid phases the composition is aerated, that is undesirable, since at drying of the aerated composition a finished product’s bulk density decreases and dust-like fractions’ content increases. Besides, the air content increase in a composition complicates its transportation and causes pressure fluctuations at its dispersion through atomizers.

Increase of a composition’s density at synthetic detergent manufacture can be reached by deaeration using vacuum or ultrasonic systems. A composition’s homogenization promotes intensification and stabilization of the drying process and manufacture of more homogeneous and strong granules. Application of mechanical homogenizers allows to lower viscosity of a composition almost in 2 times and to reduce sizes of agglomerates and crystal inclusions from 4 mm to 0,2-0,3 mm. Mechanical homogenizers also carry out a role of low-lift pumps since their working bodies are combined in one case.

There are periodic and continuous ways of a composition preparation. The periodic way provides for two apparatuses at least: the first apparatus is used for preparation of a composition, and the second one – for feeding the composition in a drier. The most widespread periodic way includes the preparation of a synthetic detergent composition alternately in one of two mixers supplied with a heating jacket and a mixer. A produced composition is fed through a bottom valve by gravity in a gravity trap for separation of impurities and then it is pumped in a service mixer. Then the composition is given in a homogenizer, a high-pressure pump and on drying.

The continuous method of synthetic detergent preparation has a number of advantages. It allows reducing a composition’s preparation time, increasing productivity of a spray-type drier and manufacturing more concentrated composition at the expense of decrease of sodium tripolyphosphate hydration degree. Besides, it is possible to apply small-sized mixers equipped with effective mixing devices, that favorably affects a composition’s quality indicators.

The composition preparation time is 15-20 minutes. It depends on a chemical composition and sodium tripolyphosphate hydration rate. Temperature in a reactor-mixer is 80-900С.

At composition preparation in a certain technological mode it is possible to regulate a bulk density of a tower powder having the same chemical composition. For example, for the purpose of fast dissolution at use synthetic detergents for tableware washing are produced with a bulk density of 230-280 g/l. Decrease of a bulk density can be reached by increasing a water content in a composition, but it is not favorable with economic point of view, as at these conditions productivity of a drying tower decreases and power inputs increase. Therefore loading of components and preparation of compositions is fulfilled using artificial addition of air in a composition. The synthetic detergent production technology is the following:

- Loading of liquid raw materials (except nonionic SAS) into a reactor in the succession: liquid glass, water, sodium alkylbenzenesulphonate.

2. Switch on a mixer and through a bubbler, mounted in the reactor under the mixer, feed a direct steam for the heating of a composition; heat up the mixture on 100С above that temperature at which the composition preparation is carried out; add cold loose components that reduces the composition temperature to the set one; viscosity of the mixture of liquid components is low, therefore at the bubbling by a direct steam and at a working mixer the mixture’s mixing and aeration occurs.

3. When the mixture’s temperature exceeds the set temperature on 10С, include the heating; at the working mixer, at first, add carboxylmethylcellulose (CMC) and Trilon B from one a common weighting device, then add manually an optical bleach, sodium sulphate, sodium tripolyphosphate and, the last, nonionic SAS.

A synthetic detergent produced by this way has a low bulk density and does not demand additional aeration before its dispersion in atomizers.

For preparation of synthetic detergents with a high bulk density, which are applied for washing in laundry washers, use other order of components’ loading:

- Load all liquid components into a reactor.

- Switch on a mixer and feed steam and loose components simultaneously in the same order as it is described above; nonionic SAS are loaded after the sodium tripolyphosphate’s loading termination.

An optimum components’ loading order is the following: water, an alkali solution, liquid glass, fat acids, sodium alkylbenzenesulphonate, magnesium sulphate, soda ash, 2/3 of sodium tripolyphosphate weight, l/2 of sodium sulphate weight, powdery carboxylmethylcellulose, an optical bleaching agent, a residue of sodium tripolyphosphate and sodium sulphate. Water content in a composition makes 30-32 % (mass).

  - The work procedure

4.3.1 Calculation of reagents’ consumption

Initial data:

Table 8. Synthetic detergent composition (% mass)

| 1.Sodium alkylbenzolesulphonate | 20 |
| --- | --- |
| 2. Nonionic surface-active substances | 2,0 |
| 3. Carboxylmethylcellulose | 9,0 |
| 4.Sodium tripolyphosphate | 40 |
| 5.Sodium sulphate | 25,7 |
| 6.Liquid glass | 1,0 |
| 7.Optical bleaching agent | 0,2 |
| 8.Perfume | 0,2 |
| 9.Water | 10 |

Table 9. Sulfanole composition (% mass)

| 1.Sodium alkylbenzolesulphonate | 45 |
| --- | --- |
| 2.Non-sulfonated compounds | 1,0 |
| 3.Sodium sulphate, sulphite and chloride | 10 |
| 4.Water | 44 |

Table 10. Carboxylmethylcellulose composition (% mass)

| 1.Basic substance in a dry product | 42 |
| --- | --- |
| 2.Water | 13,5 |

Table 11. Other kinds of raw material

| Liquid glass density | 1410 kg/m3 |
| --- | --- |
| Module | 3,36 |
| Sodium sulphate humidity | 0,3 (mass) |

Raw material losses at the production are taken 2 % (mass).

Alkylbenzolesulfonate consumption for preparation of 100 grams of a synthetic detergent:

100  0,2  1,02/0,45 = 45,4 g

where 0,2 – mass fraction of alkylbenzolesulfonate in a finished product;

1,02 – coefficient taking into account production losses;

0,45 – mass fraction of sodium alkylbenzolesulfonate in the initial raw material.

Table 12. Initial raw material contains:

| Sodium alkylbenzolesulfonate | 45,4 * 0,45 = 20,43 g |
| --- | --- |
| Non-sulfonated compounds | 45,4 * 0,01 = 0,45 g |
| Sodium sulphate | 45,4 * 0,1 = 4,54 g |
| Water | 45,4 * 0,44 = 19,98 g |

Nonionic SAS consumption:

100   0,02  1,02 = 2,04 g

where 0,02 – mass fraction of nonionic SAS in a finished product.

Carboxylmethylcellulose consumption:

100  0,09  1,02/0,363 = 25,3 g

where 0,09 – mass fraction of a basic substance in a finished product;

0,363 – mass fraction of a basic substance in the initial raw material

42 (100 – 13,5)/100 = 36,3 %

Technical carboxylmethylcellulose contains:

- water – 25,3  0,135 = 3,4 g

where 0,135 – mass fraction of water in the technical CMC.

- dry substances – 25,3  (100 – 13,5) = 21,9 g

where (100 – 13,5) – mass fraction of dry substances in the technical CMC including:

basic substance – CMC – 25,3  0,363 = 9,2 g

dry substances entering with the CMC – 21,9 – 9,2  = 12,7 g.

Since carboxylmethylcellulose is used as the 7 % water solution, so the water amount, which is necessary for the specified solution preparation makes:

21,9 ∙ (100 - 7)/7 –3,4 = 28,8 g.

For a synthetic detergent preparation it is necessary to take:

- sodium tripolyphosphate: 100  0,4  1,02 = 40,8 g,

where 0,4 – mass fraction of sodium tripolyphosphate in a finished product;

- sodium sulphate: 100  0,257  1,02 = 26,2 g,

where 0,257 – mass fraction of sodium sulphate in a finished product.

Real consumption of sodium sulphate makes:

26,2/0,997 – 4,54 – 12,7 – 0,45 = 8,59 g

where 4,54 – sodium sulphate entering with the sulfonole;

12,7 – dry substances entering with the CMC;

0,45 – non-sulfonated compounds entering with the sulfonole;

0,997 – mass fraction of sodium sulphate in the initial raw material.

Water containing in sodium sulphate:

8,59  0,003 = 0,03 g

- liquid glass: 100  0,01  1,02 – 0,394 = 0,63 g,

where 0,01 – mass fraction of sodium glass in a finished product;

0,394 – mass fraction of dry substances (Na2O + SiO2 + impurities in the liquid glass if the technical product’s density is 1410 kg/m3  and module is 3.36).

Water containing in the liquid sodium glass:

O,63  0,606 = 0,38 g,

where 0,606 – mass fraction of water in the technical glass.

- an optical bleaching agent: 100  0,002 = 0,2 g

perfume: 100  0,002 = 0,2 g,

where 0,002 – mass fraction of bleaching agent and perfume accordingly in a finished product.

Balance of a powdery synthetic detergent preparation is represented in the table 2. At the given ratio of dry substances and water the mixture’s humidity is 19,25 % (mass).

For production of powder-like synthetic detergents by melting in a drying tower humidity of the mixture should make 40 % (mass). Therefore, the additional amount of water is:

920,1  100 / (100 – 40) – 1427 = 106,4  – 1кг

Table 13. Raw material consumption for production of 100 g of synthetic detergent

| Component | Consumption, g |  |  |
| --- | --- | --- | --- |
|  | Total | Dry substances | Water |
| Alkylbenzolesulfonate solution | 45,40 | 25,42 | 19,98 |
| Nonionic surface active substances | 2,04 | 2,04 | - |
| Carboxylmethylcellulose | 54,10 | 21,9 | 32,2 |
| Sodium tripolyphosphate | 40,80 | 40,80 | - |
| Sodium sulphate | 8,59 | 8,56 | 0,03 |
| Liquid sodium glass | 0,63 | 0,25 | 0,38 |
| Optical bleaching agent | 0,2 | 0,2 | - |
| Perfume | 0,2 | 0,2 | - |
| Total | 151,96 | 99,37 | 52,59 |
| Including 0,45 g – losses of non-sulfonated compounds at the drying in a drying tower using a heat-transfer agent |  |  |  |

Table 14. Material balance of powdery synthetic detergent production

| Income |  | Yield |  |
| --- | --- | --- | --- |
| Component | Weight, g | Component | Weight, g |
| Alkylbenzolesulfonate solution | 45,40 | Powdery synthetic detergent (humidity – 10 %) | 1000 |
| Nonionic surface active substances | 2,04 | Initial raw material (loss) | 20 |
| Carboxylmethylcellulose | 312,9 | Evaporated moisture and non-sulfonated compounds | 515,5 |
| Sodium tripolyphosphate | 408,0 |  |  |
| Sodium sulphate | 202,5 |  |  |
| Liquid sodium glass | 25,9 |  |  |
| Optical bleaching agent | 2,0 |  |  |
| Perfume | 2,0 |  |  |
| Water | 106,4 |  |  |
| Total | 1533,5 | Total | 1533,5 |

Hereby, the total mass of initial components (raw material + water), which is necessary for preparation of 1 t of synthetic detergent makes 1533,5 kg.

4.3.2 Calculation of drying process

In order that we may calculate a drying process of a composition in a drying tower, it is necessary to know the following data:

Table 15. Data for drying process calculation

| Drying tower capacity relative to evaporated water | 3000 kg/h |
| --- | --- |
| Water content in a composition | 40 % mass |
| Final humidity of a finished product | 10 % mass |
| Average density of a composition containing 66–65 % mass of dry substance | 1200 kg/m3 |

A composition’s charge in a dryer is calculated according to the equation:

W  (100- W2)           3000 (100 – 10)

G1 = ------------------------- = --------------------- = 10800 kg/h

W1 – W2                    40 – 10

Synthetic detergent output from a drying tower is determined under the equation:

G1(100 – w1)     10800 (100 – 35)

G1 = ------------------- = ------------------- = 7800 kg/h

100 – w2               100 - 10

It is implied that for a composition’s preparation a standard reactor-mixer is used (volume is 5 m3, fill coefficient is 0,6).

Working volume of a reactor:

5  0,6 = 3,0 m3

Synthetic detergent weight produced for one operation:

3,0  1,200 = 3,600 kg

Number of operations necessary for maximum productivity maintenance:

10800/3600 = 3 operation/h

Duration of one operation for preparation of 3600 kg of a composition in one reactor:

60/3 = 20 min

Productivity of a reactor-mixer in one operation:

1000  3600/1533,5 = 2348 kg = 2,348 t

Table 16. Weights of raw material components loaded in the reactor for single operation (kg)

| Alkylbenzolesulfonate | 453,3 2,348 = 1064,4 |
| --- | --- |
| Nonionic surface active substances | 20,4 2,348 = 47,9 |
| Carboxylmethylcellulose | 25,3 2,348 = 59,4 |
| Sodium tripolyphosphate | 408,0 2,348 = 958,0 |
| Sodium sulphate | 202,5 2,348 = 457,5 |
| Liquid sodium glass | 25,9 2,348 = 60,8 |
| Optical bleaching agent | 2,0 2,348 = 4,7 |
| Perfume | 2,0 2,348 = 4,7 |
| Water for preparation of 7 % of carboxylmethylcellulose solution | 287,6 3,348 = 673,9 |
| Water in a mixer | 106,5 2,348 = 250,0 |

Viscosity of the produced composition corresponds to spraying mode of this composition in mechanical atomizers under pressure 40-80 atm.

4.3.3 Synthetic detergent production

A laboratory installation for a universal synthetic detergent production consists of a reactor with capacity of 400-500 ml with a mixer set in motion by an electromotor. Loading of initial materials is made through an aperture in a cover of the reactor. A bubbler is mounted in the reactor under the mixer. The bubbler is used for live steam supply for a composition’s heating. The reactor is placed in a water thermostat, which is supplied with electric heating adjusted on certain temperature. The set temperature is supported by means of a relay connected with a heater and a contact thermometer. For prevention of water evaporation in the course of a composition preparation, the reactor is supplied by a backflow condenser.

The calculated quantity of liquid raw materials (except nonionic surface-active substances) is loaded in the reactor in the following sequence: liquid glass, water, sodium alkylbenzolesulfonate. The reactor is placed in the water thermostat heated to temperature of 80-900С, turn on the mixer and feed live steam through the bubbler for heating of a composition. The mixture is heated on 100С above that temperature, which is necessary for preparation of a composition. After achievement of the necessary temperature the heating is switched off and at the working mixer the calculated quantity of loose components is added in the sequence: CMC, an optical bleaching agent, sodium sulphate, sodium tripolyphosphate and nonionic SAS. The mixture is mixed during 15-20 minutes. The produced composition is poured in a preliminary weighed porcelain cup and is dried in a drying cabinet at temperature 100-105 0С. Then the cup with the produced synthetic detergent is cooled and is weighed; then the synthetic detergent mass is calculated. The product is analyzed according to the technique specified in previous laboratory works. Then it is necessary:

- to determine bulk density of the product;

- to compare quality of the produced product with quality of a standard product;

- to calculate the product yield.

The obtained results are entered in the tables 17 and 18.

The product yield is calculated under the formula:

 = mSD/mtheor.  100 %

where mSD – weight of synthetic detergent produced as a result of the experiment,

mtheor. – weight of SD set for the calculation of a composition.

Table 17. Initial and calculation data

| Условия опыта |  | Reagent consumption, g |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Т, 0С | Т, min | Alkylbenzolesulfonate | Nonionic SAS | CMC | Sodium tripolyphosphate | Sodium sulphate | Liquid glass | Bleaching agent | Perfume | Water |

Table 18. Experimental and analysis results

| SD, g | Stability to ions of alkali and heavy metals | Stability to acid mediums | D, % | , N/cm | рН | CCM, % | Wetting ability | Apparent density, g/cm3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3 Control questions:

1. Stages of a composition preparation.

2. Factors influencing a composition’s properties.

3. Ways of compositions’ preparation.

4. Advantages of the continuous method of a composition’s preparation.

5. An optimum technological mode of a composition’s preparation.

6. An optimum order of loading components.

Laboratory work № 5

Quality control of toilet soap

5.1 An aim: studying methods of toilet soap analysis and determination of toilet soap quality.

5.2 Theory information

Plants and living organisms contain complex substances, so-called lipids, in a structure of some tissues. A basic part of lipids is fats, i.e. complex ethers of glycerin and fat acids – acilglycerins. Alkyl carbon (fat) acids separated from fats are surface-active substances; their aliphatic part contains 12-20 carbon atoms. In cake soap sodium alkyl carboxylates are used.

Soap is a mixture of 60-80 % of sodium alkyl carboxylates with water and auxiliary components. Soap is the first industrial product using for washing and laundering. Soaps are subdivided on toilet and household (laundry) ones. Toilet soap contains 80 % of salts of fat acids – alkyl carboxylates, and household soap – 60-70 %.

Basic and auxiliary raw material for cake soap production

Raw materials for manufacture of toilet and laundry soap are various fats, oils and fat-like substances. There are technical and food fats, fat mass, coconut, palm-stone, palm and tall oils, microbe fats, synthetic alkyl carbon and oil acids, rosin, fillers, dyes, aromatic substances, antioxidants, plasticizing agents and other additives.

Basic raw material

Technical animal fats are produced from any kinds of fat-containing raw material unsuitable for food purposes by extraction, melting or pressing. Technical fats are applied in the manufacture in natural kind or as fat acids separated from them. If it is necessary, fats are preliminary bleached, and produced fat acids are purified.

Fat mass is produced at hydrogenation of vegetable oils and fats of some ground and sea mammals.

Animal food fats (lard, etc.) are applied only for toilet soap manufacture.

Coconut oil is extracted from dried pulp (copra) of fruits of coconut palm trees. This oil has high content of middle- and low-molecular acids (90 %). Acids of coconut oil allow producing homogeneous, plastic and solid soaps, which are stable in mineral environments and form plentiful foam.

Palm-stone oil is produced from stones of olive palm trees. A composition of this oil is close to a composition of coconut oil.

Palm oil is extracted from pulp of fruits of olive palm trees. The oil has a pleasant smell; however, palm oil is painted owing to carotene and demands bleaching.

Rosin is produced from soft resin of coniferous breeds of trees. Basic components of rosin are isomeric pitch acids containing one a carboxyl group. The general formula is С20Н30О2. Sodium and potassium salts of pitch acids are entered in the structure of laundry soap for improvement of plasticity and solubility.

For increasing solubility and plasticity of laundry soap sodium salts of oil acids can be used except for rosin.

Tall oil is industrial waste of the cellulose industry. It consists of a mixture of 40-60 % of fat acids and 30-50 % of pitch acids.

Microbial fat is produced at manufacture of fodder yeast from n-paraffins by a microbiological method. Fat is subjected to alkaline clearing, and then fat acids (С14-С18) are separated from a mixture.

Synthetic fat acids are prepared at oxidation of solid and liquid alkanes by atmospheric oxygen. The fraction C10-C16 is used for toilet soap preparation, the fraction С17-С20 – for production of household soap.

Auxiliary raw material

1) 	 Zinc oxide and titanium oxide (2-5 kg on 1t of soap) – these substances give to soap a homogeneous white colour and eliminate transparency;

2) 	 Dyes (3-350 kg on 1t of soap): rodamine, fluorescine, turquoise, methanil;

3) 	 Perfume (8-15 kg on 1t of soap);

4) 	 Antioxidants (for toilet soaps) – prevent soap’s oxidation;

5) 	 Softeners (for toilet soaps) – give plasticity to soaps;

6) 	 Sodium hydroxide and sodium carbonate.

Standard toilet soap compositions (mass %): coconut oil - 3-17, technical fat - 13-23, food fat - 13-70, lard mass - 0-52, synthetic fat acids of C10-C16 fraction - 0-16.

Composition of high-quality soap includes aliphatic acids containing one non- saturated bond and minimum quantity of low-molecular acids. The toilet soap with such acid structure has high washing and foaming ability, high hardness; it is moderately swell in water and is economy.

Household soap compositions include (mass %): lard mass of vegetable oils - 22-60, animal fats - 5-17, vegetative fats and oils in their waste products - 0-25, synthetic fat acids (fractions C10-C16 and С17-С20) - 12-48, rosin - 0-4, oil acids – 0-2, tall oil - 0-2.

Chemistry and production technology of soap

Two basic reactions lay in the basis of soap production from fats: 1) saponification of acylglycerines and 2) neutralization of alkylcarbon acids.

Formation of alkaline salts from fats (saponification) proceeds in two stages.

The first stage: neutralization of acids by water solution of sodium carbonate. Sodium carbonate – is a salt of a weak acid and a strong alkali. This salt in water medium is exposed to hydrolysis with formation of sodium hydrocarbonate and sodium hydroxide. Sodium hydroxide cooperates with alkyl carbon acids with formation of sodium salt, i.e. soap:

RCOOH + NaOH = RCOONa + H2O

Soap

At rise of temperature above 600С there is sodium hydrocarbonate decomposition and formation of sodium carbonate, carbonic gas and water:

2NaHCO3 = Na2CO3 + CO2 + H2O

The formed sodium carbonate again enters in reaction with acids, and carbonic gas is deleted at 900С.

The second stage consists in neutralization of residue of fat acids by sodium hydroxide and saponification of acyl glycerines, i.e. fats:

3RCOOH + 3NaOH = 3RCOONa + H2O

Hydrolysis of complex ethers is a limiting stage of the process. Due to the described reaction the hydrolysis of complex ethers of carbon acids has received the term "saponification".

Production of soap can be carried out in periodic and continuous ways.

Periodic way. Organic raw material is preliminary improved by repeated salting-out and bleaching by hydrogen hydroperoxide. The clarified raw material is loaded into a soap pan, where saponification and neutralization of acids are carried out. A 28-30 % solution of sodium carbonate (soda ash) is used for the neutralization. Acids are fed with speed 2 t/h approximately. The mixture is mixed by steam and air within 1 hour. Saponification is considered as finished if sodium carbonate content in the reactionary mass does not exceed 0,5 % (mass), thus alkyl carbonate content makes 56 %. For complete saponification of neutral fats, 42 % sodium hydroxide solution is added in the pan at boiling and agitation in such quantity, that its surplus did not exceed 0,3 % (mass). Then the reactionary mixture is settled within 8 hours. The produced soap contains more than 60 % of an active substance. The ready mass is filtered and fed on a vacuum-drying installation.

Continuous way. A fatty composition from a tank by a pump through a heat exchanger at temperature 105-1150С is fed in a saponification apparatus. Sodium hydrocarbonate solution is given from a tank in the same apparatus. A mixture in the first cylindrical part of the apparatus is mixed and blown by steam. Escaping carbon dioxide is released from water and fed in a gas-holder and then - on purifying and compression.

The reactionary mixture from the first cylinder of the apparatus flows in the second cylinder, where it is also mixed. Escaping gas is given in a gas-holder, and the mixture is mixed with sodium hydroxide and through a vertical branch pipe flows in the third cylinder. Saponification of neutral fats and neutralization of residue of fat acids occurs in the third section. The mixture flows on a downpipe in the fourth section, where the reaction comes to the end. A ready product is collected in a receiver tank and fed on preparation of cake soap.

The most widespread production technologies in soap manufacture are methods of continuous processing with use of a vacuum-drying chamber and screw-presses. An active mass of toilet soap from a soap collector by a pump  through a filter  is fed in a reservoir; then by a dosing pump under pressure 0,5 MPa the mass moves in a heater, where it is heated up to 120-1600С. Then the soap is fed inside a shaft of a vacuum-drying chamber and is sprayed through an atomizer in space, where vacuum makes 2-6 kPa. Particles of dried and cooled soap settle as a thin layer on the internal surface of the chamber, from which they are removed by knifes as chips with thickness of 1 mm. The soap chips are given in a two-chamber screw-press, located in the bottom part of the chamber. In the first chamber of the press the soap chip is compacted, plasticized, pressed through a lattice and cut on fine granules. The granules are additionally dried in an intermediate vacuum chamber, and then they are fed in the second chamber of the screw-press, which works similarly as the first one. From the second section of the press the soap as scales is given in a noria and then by means of a screw it moves in a bunker.

Water vapours and soap dust are fed in cyclones. Settled soap particles are pressed and removed as vermicelli using a screw-press.

The soap chip from the bunker is fed in a mixing screw-press. A mixture of necessary additives is fed in the same screw-press also by a dosing pump. The soap mass, feeding from the screw-press, is cut and then it (as vermicelli or tablets) is given on a conveyor, which moves the mixture in the following screw-press. In this press the soap mass is dried in vacuum, cooled, pressed as bars at temperature 40-600С. The bars are given on a conveyor of an automatic cutting machine. The formed pieces on the conveyor are fed in a tunnel blown by cold air. Then the hardened soap is fed on a stamp-press and wrapping automatic devices.

5.3 An order of the laboratory work carrying out

Determination of toilet soap quality is carried out on the following parameters:

а) mass fraction of fat acids;

b) mass fraction of free caustic alkali;

c) mass fraction of free carbonic sodium;

d) the sums of nonsaponifying organic substances and unsaponifiable fats;

e) water-insoluble impurities.

For determination of toilet soap quality the sampling and carrying out of a test are carried out according to GOST 790-89.

A sample for the analysis is prepared as follows: a half of a soap piece is grinded on a grater; the formed shaving is put in a jar with a dense stopper and leaved in an exiccator without water-absorbing agents on 24 hours for uniform distribution of moisture.

5.3.1 Determination of mass fraction of fat acids

Preparation of a sample: 5 ± 0,5g of soap is weighted (writing down the result up to 3-rd decimal sign) and is dissolved in 60 cm3 of the distilled water heated up to boiling.

The solution is cooled up to 35-40°С and put in a separating funnel № 1 in which 5 drops of methyl orange are added, then 20 % solution of hydrochloric or sulfuric acid is added before the occurrence of steady pink shade of water layer.

The solution in the separating funnel № 1 is mixed by circular rotation and cooled. Fat acids are separated from the solution, and then 50 cm3 of ethyl ether is added.

The flask is rinsed twice by distilled water (on 25 cm3), once – 20 % solution of hydrochloric or sulfuric acid (5 cm3) and then – by ethyl ether (25 cm3). After each rinsing of the flask the washing water, acid and ether are placed in the separating funnel №1.

The mixture in the separating funnel is slightly mixed by circular rotation and after settling of acid water layer it is poured out in a separating funnel № 2 in which it is again processed by 30 cm3 of ethyl ether. The water layer settling in the separating funnel № 2 is poured out, and the ether extract is placed in the separating funnel № 1.

An emulsified water layer formed in the separating funnel № 2 is extracted in the third time in a separating funnel № 3 by 25 cm3 of ethyl ether. The water layer settled in the separating funnel № 3 is poured out, and the ether extract from it is placed in the separating funnel № 1.

The separating funnel №2 is rinsed by ethyl ether which is poured out in the separating funnel № 1. The ether extracts of fat acids are washed out three times in the separating funnel № 1 by 10 % solution of sodium chloride (on 30 cm3 of solution) before neutral reaction of the washing water (methyl orange indicator). Then the ether extracts are filtered in a flask preliminary weighed up to constant weight, having placed on the filter about 5 g of waterless sodium sulphate. The separating funnel № 1 is rinsed by ethyl ether. The filter with a deposit is also washed out by ethyl ether. At weak heating on a water bath almost all ether is evaporated from the flask and the deposit is dissolved in a flask containing 30-40 cm3 of preliminary neutralized ethanol. The spirit solution of fat acids is titrated by 0,5 M sodium hydroxide solution at the presence of 2-3 drops of phenolphthalein and boiled on a boiling water bath for distillation of the alcohol.

The flask is dried in a drying cabinet during 2 hours at temperature (120±3)°С then it is cooled in an exiccator 40 minutes and weighed. The result is wrote down up to 4-th decimal sign. The weighing is repeated each hour of the drying. Weight is considered as constant one if a difference between subsequent weighings will not exceed 0,002g. At increase of the weight the data of the previous weighing are took.

Mass fraction of fat acids (X) in percentage is calculated under the formula:

(m1 – V  K  0,011)

Х =  				        100,

m

where m1 – weight of the deposit in the flask after the drying, g;

V - volume of spirit solution of sodium hydroxide with NaOH concentration = 0,5 mole/dm3, spent for the titration, cm3;

K – correction taking into account a ratio of real concentration of sodium hydroxide solution in mole/dm3 to nominal concentration NaOH = 0,5 mole/dm3;

m - weight of the analyzed soap sample, g;

0,011 - the difference between atomic masses of sodium and hydrogen, equivalent 1 cm3 of sodium hydroxide spirit solution with concentration of NaOH = 0,5 mole/dm3.

A result of the measurement is equal the average arithmetic of results of two parallel determinations.

Calculations are carried out up to the first decimal sign.

The result is approximated to a whole number.

Allowable divergences between parallel determinations should not exceed for toilet soap – 0,5 %, for laundry soap - 0,8 % at confidence probability 0,95.

Qualitative number of laundry soap (Nq) is calculated under the formula:

Nq = Х  m/100,

where X - mass fraction of fat acids, %;

m – the average weight of a soap piece, g.

Mass fraction of fat acids in toilet soap in recalculation on nominal weight of a soap piece (Х0) in percentage is calculated under the formula:

Х0 = m1  Х / m2,

where m1 - actual weight of a soap piece, g;

X - actual mass fraction of fat acids, %;

m2 – nominal weight of a piece, g.

5.3.2 Determination of mass fraction of free caustic alkali

Weigh 5 ± 0,5g of soap, the result is written down up to 3-rd decimal sign, then 100 cm3 of the alcohol preliminary neutralized in the presence of phenolphthalein is flowed. The mixture is placed in a flask.

The flask is connected to a backflow condenser, is heated up on a water bath to the soap dissolution, then 25 cm3 of the barium chloride neutralized solution is added in the hot solution.

Not filtering the deposit, the hot solution is titrated at intensive agitation using 0,1 M solution of hydrochloric acid at the presence of a control sample for comparison of the solution’s color.

The control sample is prepared by the same way as the basic sample; a difference consists in the fact that after the cooling of the soap solution by barium chloride phenolphthalein is not added in the flask with the control sample. Titration of the basic sample is conducted up to a color of the control sample.

Mass fraction of free caustic alkali (Х1) in percentage is calculated under the formula:

V  K  0,004  100

Х1 =  				     ,

m

where V - volume of hydrochloric acid solution with concentration of HCl = 0,1 mole/dm3, spent for titration, cm3;

K - correction taking into account a ratio of real concentration of hydrochloric acid in mole/dm3 to nominal concentration of HCl = 0,1 mole/dm3;

m - weight of the analyzed soap sample, g;

0,004 - weight of caustic alkali equivalent to 1 cm3 of hydrochloric acid solution with concentration of HCl = 0,1 mole/dm3.

The result of the test is the average arithmetic of the results of two parallel determinations.

Calculations are fulfilled up to the second decimal sign.

Divergences between results of parallel measurements in percentage at confidence probability 0,95 should not exceed:

for toilet soap: 0,01 (norm of free caustic alkali mass fraction 0,03-0,05 %);

for laundry soap: 0,03 (norm of free caustic alkali mass fraction less 0,15 %);

0,05 (norm of free caustic alkali mass fraction less 0,20 %).

5.3.3 Determination of free carbonic sodium mass fraction

For preparation of a sample 5 ± 0,5 g of soap is weighed in a flask, a result is written down up to the 3-rd decimal sign, then 75 cm3 of alcohol preliminary neutralized in the presence of phenolphthalein is added. The flask is connected to a backflow condenser and is heated up to the soap dissolution.

The solution cooled up to room temperature is titrated by 0,1М solution of hydrochloric acid at the presence of 2-3 drops of phenolphthalein.

At analysis of color soap the titration is carried out at the presence of a control sample. The control sample is prepared by the same way as the basic sample, but without addition of phenolphthalein. The basic sample is titrated before achievement of the control sample’s color.

Mass fraction of free carbonic sodium (Х2) in percentage is calculated under the formula:

V  K  0,4

Х2 = 		       X1      2,65,

m

where V - volume of hydrochloric acid solution with concentration НСl = 0,1 mole/dm3, spent for titration, cm3;

K - correction taking into account a ratio of real concentration of hydrochloric acid in mole/dm3 to nominal concentration of HCl = 0,1 mole/dm3;

m - weight of the analyzed soap sample, g;

Х1 - mass fraction of free caustic alkali in percentage;

0,4 - weight of caustic alkali equivalent to 1 cm3 of hydrochloric acid solution with concentration of НСl = 0,1 mole/dm3, multiplied on 100;

2,65 - recalculation factor of caustic alkali on carbonic sodium.

The average arithmetic of results of two parallel definitions is accepted for the test result.

Calculations are fulfilled up to the second decimal sign.

Allowable divergences between parallel measurements should not exceed 0,05 % at confidence probability 0,95.

Mass fraction of free carbonic sodium in the soap in recalculation on a nominal weight of a soap piece (Х2’) in percentage is calculated under the formula:

Х2 = Х2   m / m1,

where Х2 - mass fraction of free carbonic sodium, %;

m – the average weight of a soap piece, g;

m1 – a nominal weight of a soap piece, g.

Calculation of soda products mass fraction

Mass fraction of soda products (X2’) in percentage in recalculation on Na2O is calculated under the formula:

Х = 0,775 Х1 + 0,590 Х2,

where 0,775 - factor of recalculation of sodium hydroxide on Na2O;

Х1 - mass fraction of free caustic alkali, %;

0,590 - factor of recalculation of carbonic sodium on Na2O;

Х2 - mass fraction of free carbonic sodium, %.

5.3.4 Determination of mass fraction of the sum of nonsaponifying organic substances and unsaponifiable fat

For preparation of a sample 10–15 g of soap is dissolved in a conic flask in 75-100 cm3 of alcohol with volume fraction 60 % at heating on a water bath. Then the solution is placed in a separating funnel № 1, the flask is rinsed by ethanol (60 %) and the washing alcohol is placed in the same funnel.

The solution in the separating funnel is strongly stirred up together with 50 cm3 of petroleum ether and settled. A bottom layer is flowed in a separating funnel № 2 and is again processed by 50 cm3 of the ether. After settling the bottom layer is deleted, and the ether extract is flowed in the separating funnel № 1. Sometimes at processing of soap solutions by ether emulsions are formed. These emulsions should be destroy by addition of 1-3 cm3 of alcohol (60 %) at easy stirring to full removal of soap residues (washing liquid diluted with water and heated should not be painted at the presence of phenolphthalein). The ether solution is filtered in a weighed flask through a paper filter on which approximately 5 g of waterless sodium sulphate is placed. The filter with the sodium sulphate is washed out by petroleum ether. The petroleum ether is evaporated on a water bath, and a residue in the flask is dried in a drying cabinet at 73-75°С up to a constant weight.

The drying of the deposit is finished if a difference between two subsequent weighings will be no more than 0,002.

Mass fraction of the sum of nonsaponifying organic substances and unsaponifiable fat (Х3) in percentage from fat acids weight is calculated under the formula:

Х3 =  m1  100  100 / m   Х,

where m1 – weight of the deposit in the flask after the drying, g;

m – weight of the soap, g;

X - mass fraction of fat acids, %.

The average arithmetic of results of two parallel is accepted for the test result.

Calculations are carried out up to the second decimal sign.

Allowable divergence between parallel measurements must not exceed 0,02 %.

5.3.5 Determination of mass fraction of water-insoluble impurities

For carrying out of this analysis 5-7 g of soap is weighed, the result is written down up to 3-rd decimal sign, then the soap is dissolved in a conic flask in 80-100 cm3 of ethyl alcohol at heating on a water bath.

The solution is filtered through a filter which has been preliminary dried up to a constant weight at (103 ±3)°С. A deposit on the filter is washed out by a hot alcohol before full removal of soap (a test with hydrochloric acid on the absence of turbidity). Then the deposit is washed out by boiling water before full washing up of carbonic sodium (the washing water must not be colored at addition of phenolphthalein).

The filter with the washed deposit is dried in a drying cabinet during 1 hour at temperature (103 ±3)°С, then it is cooled in an exiccator during 40 minutes and is weighed.

A result is written down to the fourth sign.

The subsequent weighings are made through everyone 30 minutes of drying.

Weight is counted as a constant one if a difference between subsequent weighings will not exceed 0,002. At increase of the weight the data of the previous weighing are used.

Mass fraction of water-insoluble impurities (Х4) in percentage is calculated under the formula:

Х4 = m1   100 / m,

where m - weight of the soap sample, g;

m1 - weight of the deposit on the filter, g;

The average arithmetic of results of two parallel measurements is accepted for the test result. Calculations are fulfilled to the second decimal sign. Allowable divergences between parallel determinations must not exceed 0,1 % at confidence probability 0,95.

Mass fraction of water-insoluble impurities in recalculation on a nominal weight of a soap piece (Х4’) in percentage is calculated under the formula:

Х4 = Х4   m / m1,

where Х4 - mass fraction of water-insoluble impurity, %;

m – the average weight of a soap piece, g;

m1 – a nominal weight of a soap piece, g.

Conclusions concerning to quality of investigated toilet soap and it conformity to standard requirements are made according to the results of performed analysis.

5.4 Control questions:

1. Basic components containing in soap composition and their purpose.

2. Chemical reactions proceeding at soap preparation.

3. Basic technological stages of toilet soap manufacture.

4. Standard requirements to content of a basic component and impurities.

5. Determination technique of fat acids content in soap.

6. Methods of determination of free caustic alkali and free carbonic sodium content in soap.

7. Determination technique of nonsaponifying organic substances and water-insoluble impurities in soap.

ENGINEERING SAFETY FOR WORK

IN A CHEMICAL LABORATORY

1. IT IS CATEGORICALLY PROHIBITED suction of acids, alkalis and other caustic liquids in pipettes by a mouth. Sampling of acids and alkalis is carried out only by a spray bulb.

2. All dry reagents and in particular caustic alkalis should be taken only by porcelain spoons or palette knifes. IT IS CATEGORICALLY PROHIBITED to take caustic alkalis by hands.

3. Bottles with bromine, hydrogen peroxide, fluoric acid and other caustic liquids should be opened so that the bottle’s neck was not directed to the side of a working person.

4. It is necessary to work with acids, alkalis and other poisonous and poisoning substances only in a draft hood at the switched on ventilation.

5. It is categorically prohibited to throw out liquids and solid substances immiscible with water as well as strong poisons in a washbowl.

6. Use of reagents without labels or with doubtful labels is categorically prohibited.

7. It is prohibited to work with fire hazard substances near switched electrical devices.

8. It is necessary to transfer vessels with hot liquids by two hands only: one hand holds for a neck, another hand supports under the vessel bottom.

9. Broken glassware must be collected using a scoop and a broom, not by hands.

10. In the event that chemical burns caused by acids, alkalis and other caustic substances you need to fulfill the following:

а) the struck place to wash at once by plenty amount of water, then for acids burns to wash by 10 % soda solution; for alkalis burns to wash by 3-4 % solution of acetic acid or 3 % solution of boric acid;

b) at reddening – to wash out a burnt place by 2 % potassium permanganate solution;

c) after rendering the first help it is necessary in serious cases immediately to address to a doctor;

d) in a case of cuts by glass with simultaneous influence of acid or alkali first of all it is necessary to wash out a wound at once by plenty amount of water and to be sure that there are no glass splinters in the wound, and then to grease edges of a wound with iodine solution and to tie up a cut;

e) in a case of sulphuric acid getting on a skin it is necessary to wash off it by plenty amount of water, and then to neutralize it by weak soda solution and to put bandage moistened by KMnO4 and then to address to a doctor.

11. If an acid is spilled, it is necessary to dilute an acid by water, then to neutralize it by soda and to remove it by a scoop. If sulphuric acid is spilled, it is necessary to dilute it by water, and then to cover it by sand or to wash off by a plenty amount of water and then to neutralize by soda.

12. In a case of alkali spilling, it is necessary to wash off it by water, and then to neutralize by weak acid (acetic, boric).

13. Chromium mixture after washing glassware is poured in the same vessel, where it was before using. It isn’t possible to pour out chromium mixture into a wash-bowl.

14. At heating substances in chemical glassware it is necessary to take into account the following: for heating up to temperature lower 100oС a water bath is applied, up to temperature higher 100oС an oil one is used, and up to temperature higher 200oС a sand bath is used.

15. IT IS STRICTLY PROHIBITED to heat glassware using open fire.

16. IT IS STRICTLY PROHIBITED to take hot glassware during heating by unprotected hands.

17. After finishing work in a chemical laboratory you need to wash hands and to rinse a mouth by water.

LITERATURE

- Ескендирова М.М. Technology of household chemical goods production. Study guide. – Шымкент: ЮКГУ им. М. Ауэзова, 2008. – 98 с.

- Кобылянский Д.А. Новое в оценке качества синтетических моющих средств. - М.: Химическая промышленность, 2011. – 203 с.

- Штюпель Г. Синтетические моющие и очищающие средства. - М.: Химическая промышленность, 2006. – 320 с.

- Бузов Б.А., Алыменкова Н.Д. Производство синтетических моющих средств. - М.: Академия, 2010. – 277 с.

- Абрамзан А.А. Поверхностно-активные вещества. Свойства и применение. 2 изд. - СПб.: Эксосервис, 2001. – 330 с.

Марина Михайловна Ескендирова

МЕТОДИЧЕСКИЕ УКАЗАНИЯ

по выполнению лабораторных работ по дисциплине

«Технология производства продуктов бытовой химии»

Подписано в печать _________

Формат бумаги XxY 1/16

Бумага типографская. Печать офсетная. Объем 3 п.л.

Тираж 20 экз. Заказ №….

© Издание Южно-Казахстанского государственного университета им.М.Ауезова

Издательский центр ЮКГУ им.М.Ауезова, г.Шымкент, пр.Тауке хана,5