Ф.7.03-03

Aslanov M.T., Ismailov S.U.

Methodical instructions to the laboratory work

on discipline

Wireless communication technologies

Shymkent, 2016

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

M.AUEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

Department: «Automation, Telecommunication and Control»

Aslanov M.T., Ismailov S.U.

Methodical instructions to the laboratory work

on discipline

Wireless communication technologies

for students of the specialty 5В071900 – Radio engineering, electronics and telecommunications

Shymkent – 2016

УДК 658.012.011

Authors: Aslanov M.T., Ismailov S.U. Methodical instructions to the laboratory work on discipline: Wireless communication technologies - Shymkent:  M.Auezov SKSU,  2016 - 64p.

Methodical instructions are made according to requirements of the curriculum and the program of discipline Wireless communication technologies and includes all necessary data on execution of a laboratory work.

Methodical instructions are intended for students of specialty 5B071900 – Radio engineering, electronics and telecommunications

Methodical instructions contain the purpose, the contents, rules of design and the job for execution of a laboratory work.

The basis of the preparation of guidelines adopted by the passage of an international internship on "Orleu" at the University of Newcastle (Newcastle University) under the program «Learning and Teaching in Higher Education (LTHE)» in Newcastle, United Kingdom, during the period from November 11, 2015 19 November 2015.

Reviewer:

Saribaev A.S - cand.tech.sci., the associate professor of M.Auezov SKSU

Are considered and recommended for printing by chair meeting of Automation, telecommunication and management (the report № __ from __ ___ 201__) and Methodical provision of higher school “Information technology and energetic” (the report № __ from __ 201___).

It is recommended to the edition Methodical council SKSU of M.Auezov. The report № __ from __ _______ 201___.

M.Auezov South Kazakhstan State university, 2016

Responsible for release Aslanov M.T.

Contents

| Functions in MATLAB. MATLAB – Simulink Communications Blockset starting guide | 7 |
| --- | --- |
| Lab work № 1 Introduction to MATLAB, Simulink, and the Communication toolbox | 30 |
| Lab work № 2 Analog modulation using MATLAB | 41 |
| Lab work № 3 White Gaussian Noise | 45 |
| Lab work № 4 Digitally modulated signals using MATLAB: PAM (ASK), PSK, and QAM | 49 |
| Lab work № 5 Digital modulation using Simulink | 53 |
| Lab work № 6 Generation and analysis of FSK waveform and its transmission using Simulink | 62 |
| Lab work № 7 Fourier analysis using MATLAB | 70 |
| The list of the recommended literature | 76 |

FUNCTIONS IN MATLAB. MATLAB – SIMULINK COMMUNICATIONS BLOCKSET STARTING GUIDE

Creating a function in MATLAB.

All of the M-files that we have seen so far have been script files. Script files are just a collection of MATLAB statements that are stored in a file. When a script file is executed, the result is the same as it would be if all of the commands had been typed directly into the command window. Script files share the command window’s workspace, so any variable that were defined before the script file starts are visible to the script file, and any variable created by the script file remains in the workspace after the script file finishes executing. The Script file has no input arguments and returns no result, but they can communicate with other script files through the data left behind in the workspace.

A MATLAB function is a special type of M-file that runs in its own independent workspace. It receives input data through an input argument list, and returns results to the caller through an output argument list. The general form of a MATLAB function is:

function [outarg1, outarg2, …] = fname (inarg1, inarg2, …)

% H1 comment line

% Other comment lines

…

(Executable code)

…

(return)

Example:

A function to transform a quantity into dB will be:

function dBW = qtodB(P)

% This function receives the number to be transformed into dB and returns it %  in dBW

dBW = 10 * log10(P)

In order for this file to work it must be saved with the name qtodB.m in the working directory.

To execute we will type

>> dB1 = qtodB(100)

dB1 = 20

>>

Exercise:

a) Construct similar functions to calculate:

q to dBm transformations

AdB and AdBm functions

Antenna gain formula (for the 3 types)

Free Space Loss formula (in terms of antenna gains)

Analogue & Digital Modulation Techniques.

AM Modulation.

Double Side-Band

Amplitude modulation is a modulation scheme in which the amplitude of a carrier sinusoidal waveform c(t) is changed as a function of a modulating signal m(t). In DSB-AM, the amplitude of the modulated signal is proportional to the message signal, therefore the time-domain representation of the modulated signal is given by:

Exercise:

For the following message signal:

Using a carrier signal with amplitude 1 and a carrier frequency of 250 Hz, with a value for to of 0.15 sec find and plot:

a) m(t) in the time axis.

b) u(t) in the time axis.

c) The Fourier transform of m(t), and its power spectrum.

d) The Fourier transform of u(t), and its power spectrum.

e) To help compare an illustrate the differences with Conventional AM, change the values of the message in the intervals as follows, and repeat steps a) through d):

Conventional AM (using modulation index)

Conventional AM is quite similar to DSB-AM; the only difference is that in conventional AM, the message m(t) is substituted with the expression  where is the normalized message signal (i.e. ) and n is the modulation index which is a positive constant between 0 and 1.

The net effect of scaling the message signal and adding a constant to it is that the term  is always positive. This makes the demodulation of these signals much easier by employing envelope detectors.

Exercise

Using the signal m(t) provided in Exercise 1 (part e), and assuming a modulation index of 0.85, to=0.15, Ac=1 and fc=250 Hz, find and plot:

a) The message signal in the time axis

b) The modulation expression u(t) in the time axis.

c) The Fourier transform of the message signal and its power spectrum.

d) The Fourier transform of the modulation expression and its power spectrum.

e) Change the modulation index to 0.15, 0.25, 0.5 and observe the differences in all the signals.

Angle Modulation (FM & PM).

Angle modulation schemes, which include frequency modulation (FM) and phase modulation (PM), belong to the class of non-linear modulation schemes. This family of modulation schemes is characterized by their high-bandwidth requirements and good performance in the presence of noise.

Frequency Modulation (FM)

The time domain representation of a frequency modulated signal, when the carrier is and the message signal is given by m(t), is given by:

kf  represents the deviation constant of FM. The frequency domain representation of angle modulated signal is, in genereal very complex due to the non linearity of these modulation schemes. Therefore we will only treat the case where the message signal m(t) is a sinusoidal signal. We will assume that the message signal is:

Then the modulated signal u(t) is of the form:

Where:

Here Wm(t) is the bandwidth of the message.

Exercise:

Using MATLAB and the sinusoidal message m(t) with amplitude 3 and frequency 10 Hz., find and plot.

a) The message in time

b) The modulated signal (u(t)-FM) with a kf =50, and a carrier frequency of 200 Hz, in time and in frequency. (Fourier transform and power spectrum).

c) Now using the message:

with: to=0.15 s, kf =50, fc=200 Hz, find the Fourier transform of the modulated signal u(t) and its power spectrum.

Note: The integral of m in matlab.

Using the vector t constructed using the sample interval selected:

;

% The message is m constructed using a for statement like in lab 1

% int_m will be the calculated integral:

int_m(1)=0;

for i=1:length(t)-1;

int_m(i+1)=int_m(i)+m(i)*ts

end

d) Repeat a),b) and c) using the message given in DSB-AM part (e).

Amplitude Shift Keying (ASK) and Frequency Shift Keying (FSK)

In ASK and FSK, the message signal consist of a bit-stream of digital values (1,0), each one of this discrete values will be multiplied by the carrier creating a seudo-amplitude modulation just like in DSB-AM or by different carriers in FSK creating a modulated function similar to FM.

In order to simulate ASK and FSK in matlab, we need to re-construct the message signal using the sample theorem applied to the carrier in order to obtain a message that has the same number of elements as the selected carrier(s):

Example:

The message m = [1 0 1  1  0] has a data rate of 1 bps., if the carrier signal has a frequency of 5 Hz. Represent the message with the same number of samples as the carrier:

We will sample at a ts of To/32, that is 0.2/32  ts = 6.25 ms. The time vector has the same duration as the message, that is 5 s, but has elements according to the sample interval.

m1=[1 0 1 1 0]

t=[0:ts:length(m1)];	% time vector

for k=length(m1):-1:1;

i = length(m1)-k

z = i*length(t)/length(m1)

for j=(z+1):(i+1)*length(t)/length(m1);

m(fix(j))=m1(length(m1)+1-k);

end;

end;

Implement the previous script in matlab.

b) Check the class notes to the functions used in ASK and BFSK, implement 2 separate matlab functions that will generate the ASK plot and the FSK plot if the message is given as a parameter for the function along with the carrier frequencie(s).

Building a Simple Model

The Basic Steps

This section describes the basic steps in building a model. It explains how to:

• Set simulation parameters with commstartup

• Open a new model window

• Open block libraries

• Move blocks into a model window

• Connect the blocks

• Set block parameters

• Set simulation parameters

• Run the model

Building a model usually involves several iterations, as you decide which blocks to include and what parameter settings to make. In the example in this section, you will refine the model by adding noise. The section explains how to:

• Add noise to the model

• Save the model

The section also explains

• Frames and frame-based processing

• Discrete signals and sample times

• Continuous signals

Using commstartup to Set Simulation Parameters

Before starting to build the model, enter Commstartup at the MATLAB prompt.

This

• Sets the Simulink Boolean logic signals parameter to Off

• Sets default simulation parameters that are optimal for communications models

The Communications Blockset does not support signals with Boolean data types. If you want to use Simulink blocks that output boolean data types, such as the Logical Operator block, in a model with blocks from the Communications Blockset, enter commstartup before building the model.

The commstartup settings apply to any models you create during the current MATLAB session. You must enter commstartup at the beginning of each MATLAB session to establish these settings. If you build a model without entering commstartup and subsequently decide to use Simulink blocks that output signals with boolean data types, turn off the model’s Boolean logic signals parameter by entering

set_param('my_model', 'BooleanDataType', 'off') where my_model.mdl is the name of the model.

Opening a New Model Window

The first step in building a model is to open a new model window. To do so, select New from the File menu, and then select Model. This opens an empty model window, as shown in the following figure.

2-13

Opening Block Libraries

The next step is to select the blocks for the model. These blocks are contained in libraries. To view the libraries for the products you have installed, type simulink at the MATLAB prompt (or, on Microsoft Windows, click the Simulink button on the MATLAB toolbar). If you are using Microsoft

Windows, this displays the Simulink Library Browser, as shown below.

Simulink Library Browser

The left pane displays the installed products, each of which has its own library of blocks. To open a library, click the + sign next to the name of the blockset in the left pane. This displays the contents of the library in the right-hand pane. You can find the blocks you need to build models of communication systems in the libraries of the Communications Blockset, the Signal Processing Blockset, and Simulink.

Moving Blocks into the Model Window

The next step in building the model is to move blocks from the Simulink Library Browser into the model window. To do so,

1 Click the + sign next to Signal Processing Blockset in the left pane of the Library Browser. This displays a list of the Signal Processing Blockset libraries.

2 Click DSP Sources in the left pane. This displays a list of the DSP Sources library blocks in the right pane. If you do not see the Sine Wave block, scroll down the list until it is visible.

3 Click the Sine Wave block and drag it into the model window.

4 Click DSP Sinks in the left pane of the Library Browser.

5 Scroll down in the right pane of the Library Browser until you see the

Vector Scope block, and drag the block into the model window to the right

of the Sine Wave block.

Once a block is in the model window, you can move it to another position by

dragging the block with the mouse.

Dragging a Block into a Model Window

Connecting Blocks

The small arrowhead pointing outward from the right side of the Sine Wave block is an output port for the data the block generates. The arrowhead pointing inward on the Vector Scope block is an input port. To connect the two blocks, click the output port of the SineWave block and drag the mouse toward the input port of the Vector Scope block, as shown in the following figure.

When the pointer is on the input port of the Vector Scope block, release the mouse button. You should see a solid arrow appear, as in the following figure.

If you do not see a solid arrowhead, you have not made a connection. In this case, click the arrowhead again, drag it all the way to the Vector Scope’s input port, and release the mouse button.

Setting Block Parameters

To set parameters for the Sine Wave block, double-click the block to open its dialog, as shown in the following figure. Change the following parameters by clicking in the field next to the parameter, deleting the default setting, and entering the new setting in its place:

1 Set Amplitude to 5.

2 Set Frequency to 30.

3 Set Samples per frame to 100.

4 Click OK.

Note You must set Samples per frame to a value larger than 1 to see an image of the sine wave in the Scope block.

Setting Simulation Parameters

Besides individual block parameters, the model also has overall simulation parameters. To view the current settings,

1 Select the Simulation menu at the top of the model window.

2 Select Configuration parameters to open the Configuration

Parameters dialog box, as shown in the following figure.

Configuration Parameters Dialog Box

If you typed commstartup before creating the model, the Stop time should be set to inf. The Stop time determines the time at which the simulation ends. Setting Stop time to inf causes the simulation to run indefinitely, until you stop it by selecting Stop from the Simulation menu.

The Stop time is not the actual time it takes to run a simulation. The actual run-time for a simulation depends on factors such as the model’s complexity and your computer’s clock speed.

The settings in the Configuration Parameters dialog box affect only the parameters of the current model.

Note To conserve memory in a model, click Data Import/Export on the left, and clear the boxes next to Time and Output under Save to workspace on the right.

Running the Model

Run the model by selecting Start from the Simulation menu. When you do so, a scope window appears, displaying a sine wave as shown in the following figure.

Sine Wave Displayed in a Scope

Note If you do not see the sine wave in the scope, make sure that the Samples per frame parameter for the Sine Wave block is set to 100. When you are finished observing the simulation, select Stop from the Simulation menu.

Adding Noise to the Model

You can add noise to the model using the AWGN Channel block, from the Channels library of the Communications Blockset. The block adds white Gaussian noise to the sine wave. Move the block from the Simulink Library Browser into the model window, as described in “Moving Blocks into the

Model Window”. You can add the block to the model as follows:

1 Extend the line between the Sine Wave block and the Vector Scope block by dragging the Vector Scope block to the right, to make room for the AWGN Channel block.

2 Click the AWGN block and drag it onto the line. This automatically connects the Sine Wave block and the Vector Scope block to the AWGN Channel block.

Sine Wave Plus Noise

Double-click the AWGN Channel block to open its dialog, as shown in the following figure. Then, click the down arrow in the Mode field and select Signal to noise ratio (SNR).

Dialog for the AWGN Channel Block

Now when you run the model, the scope clearly shows the added noise.

Sine Wave with Noise Added

When you are finished observing the simulation, stop the model by selecting Stop from the Simulation menu.

Saving a Model

To save your model for future use, select Save from the File menu. The first time you save the model, this displays the Save As dialog box. In the Save in field, select the directory where you want to save the model (Use your U Drive).

In the File name field, enter a name for the model, such as sine.mdl, and click Save.

To load the model in a future MATLAB session, first change your working directory to the one where you saved the file. You can do this by selecting the directory in the Current Directory field on the MATLAB toolbar. Then enter sine in the MATLAB Command Window.

IMPORTANT NOTES:

Frames and Frame-Based Processing

A frame is a sequence of samples combined into a single vector. By setting Samples per frame to 100 in the Sine Wave block, you set the frame size to 100, so that each frame contains 100 samples. This enables the Vector Scope block to display enough data for a good picture of the sine wave.

Another important reason to set the frame size is that many Communications Blockset blocks require their inputs to be vectors of specific sizes. If you connect a source block, such as the Sine Wave block, to one of these blocks, you can set the input size correctly by setting Samples per frame to the

required value. The model described in “Reducing the Error Rate Using a

Hamming Code” shows how to do this.

In frame-based processing all the samples in a frame are processed simultaneously. In sample-based processing, on the other hand, samples are processed one at a time. The advantage of frame-based processing is that it can greatly increase the speed of a simulation. If you see double lines between

blocks, the model uses frame-based processing.

Discrete Signals and Sample Times

The Sine Wave block in the Signal Processing Blockset generates a discrete signal. This means that it updates the signal at integer multiples of a fixed time interval, called the sample time. You can set the length of this time interval in the Sample time parameter in the block’s dialog. In the example described in “Building a Simple Model”, the Sample time has the default value of 1/1000. All the sources in the Communications Blockset and the Signal Processing Blockset generate discrete signals exclusively. These sources are primarily designed for modeling digital communication

systems.

Continuous Signals

The Simulink libraries also contain blocks that generate continuous signals. This means that they update the signal at variable time intervals, whose length is determined by the numerical solver the simulation uses. For example, the Sine Wave block in the Simulink Sources library can generate a

continuous sine wave.

Note Many blocks in the Communications Blockset accept only discrete signals. To find out whether a block accepts continuous signals, consult the reference page for the block.

Exercises For Simulink

Building a Channel Noise Model

This section shows how to build a simple model of a communication system. The model, shown in the following figure, contains the most basic elements of a communication system: a source for the signal, a channel with noise, and means of detecting errors caused by noise.

Overview of the Model

The channel noise model generates a random binary signal, and then switches the symbols 0 and 1 in the signal, according to a specified error probability, to simulate a channel with noise. The model then calculates the error rate and displays the result. The model contains the following components.

Source

The source for the signal in this model is the Bernoulli Binary Generator block, which generates a random binary sequence.

The Channel

The Binary Symmetric Channel block simulates a channel with noise. The block introduces random errors to the signal by changing a 0 to a 1 or the reverse, with a probability specified by the Error probability parameter in the block’s dialog.

Error Rate Calculation

The Error Rate Calculation block calculates the error rate of the channel. The block has two input ports, labeled Tx, for the transmitted signal, and Rx, for the received signal. The block compares the two signals and checks for errors.

The output of the block is a vector with three entries:

• Bit error rate, which you expect to be approximately .01, since this is the probability of error in the channel

• Number of errors

• Total number of bits that are transmitted

Display

The Display block displays the output of the Error Rate Calculation block. This block, labeled "Error Rate Display," displays the number of errors introduced by the channel noise. When you run the simulation, three small boxes appear in the bl ock, as shown in the following figure, displaying the

vector output from the Error Rate Calculation block.

Error Rate Display

The block displays the output as follows:

• The first entry is the symbol error rate (SER).

• The second entry is the total number of errors.

• The third entry is the total number of comparisons made. The notation 1e+004 is shorthand for 104.

Selecting Blocks for the Channel Noise Model

To build the model, first move its blocks into a new model window, as follows:

1 Type commstartup at the MATLAB prompt to set simulation parameters for the model.

2 Type simulink at the MATLAB prompt to open the Simulink Library Browser.

3 From the File menu, select New, and then Model. This opens a new model window.

4 Drag the following blocks from the Simulink Library Browser into the model window:

• Bernoulli Binary Generator block, from the Data Sources sublibrary of the Comm Sources library

• Binary Symmetric Channel block, from the Channels library

• Error Rate Calculation block, from the Comm Sinks library

• Display block, from the Simulink Sinks library

Setting Parameters in the Channel Noise Model

To set block parameters in the channel noise model, do the following:

1 Double-click the Binary Symmetric Channel block and make the following changes to the default parameters in the block’s dialog:

• Set Error probability to 0.01.

• Clear the Output error vector box. This removes the block’s lower output port, which is not needed for this model.

2 Double-click the Error Rate Calculation block and make the following

changes to the default parameters in the block’s dialog:

• Set Output data to Port to create an output port for the block.

• Select Stop simulation.

Selecting Stop simulation causes the simulation to stop after the target number of errors occurs or the maximum number of symbols is reached.

Initial Seeds

The Bernoulli Binary Generator block and the Binary Symmetric Channel block both use a random number generator to generate random sequences of bits. In both blocks, the Initial seed parameter initializes the random sequence. The initial seeds in the two blocks should have different values to ensure that the source signal and the channel noise are statistically independent. In general, initial seeds should have a different values in all blocks that have an Initial seed parameter.

Connecting the Blocks

Next, connect the blocks as shown in the following figure. Make sure to connect the arrow from the Binary Symmetric Channel block to the input port labeled Rx on the Error Rate Calculation block.

Drawing a Branch Line

The upper line leading from the Bernoulli Binary Generator block to the Error Rate Calculation block, shown in the following figure, is called a branch line. Branch lines carry the same signal to more than one block.

To draw the branch line, follow these steps:

1 Right-click the line between the Bernoulli Random Generator block and the Binary Symmetric Channel block.

2 Move the mouse pointer to the input port labeled Tx on the Error Rate Calculation block, while pressing the right mouse button.

3 Release the mouse button. The end of the branch line should connect to the input port of the Error Rate Calculation block.

4 Click the horizontal section of the branch line and drag it upward until the line is above the Binary Symmetric Channel block. The model should now appear as in the following figure.

Running the Channel Noise Model

To run the model, select Start from the Simulation menu. After a few seconds, the model will stop automatically.

To see all three boxes in the Display block, you must enlarge the block slightly, as follows:

1 Select the Display block and move the mouse pointer to one of the lower

corners of the block, so that a diagonal arrow appears on the corner, as

shown.

2 Drag the corner of the block down with the mouse until three windows

appear, as shown.

The Display block displays the following information:

• The bit error rate

• The number of errors

• The total number of bits that are transmitted

Note that the exact values that appear will vary, depending on the Initial seed parameters in the Bernoulli Binary Generator block and the Binary Symmetric Channel block.

Since the Target number of errors in the dialog for the Error Rate Calculation block is set to 100, the simulation stops when 100 errors have been detected. To save the model, select Save from the File menu, type a name for the model, such as channelnoise, in the File name field, and click Save.

Reducing the Error Rate Using a Hamming Code

We will now describe how to reduce the error rate in the model shown in the figure Channel Noise Model by adding an error-correcting code.

The following figure shows an example that uses a Hamming code.

Building the Hamming Code Model

You can build the Hamming code model by adding blocks to the model shown in the figure Channel Noise Model

1 Type channeldoc at the MATLAB prompt to open the channel noise model. Then save the model as my_hamming in the directory where you keep your work files.

2 Drag the following two Communications Blockset blocks from the Simulink Library Browser into the model window:

• Hamming Encoder block, from the Block sublibrary of the Error Detection and Correction library

• Hamming Decoder block, from the Block sublibrary of the Error Detection and Correction library

3 Click the right border of the model and drag it to the right to widen the model window.

4 Move the Binary Symmetric Channel block, the Error Rate Calculation block, and the Display block to the right by clicking and dragging. This creates more space between the Binary Symmetric Channel block and the blocks next to it. The model should now look like the following figure.

5 Click the Hamming Encoder block and drag it on top of the line between the Bernoulli Binary Generator block and the Binary Symmetric Channel block, to the right of the branch point, as shown in the following figure. Then release the mouse button. The Hamming Encoder block should automatically connect to the line from the Bernoulli Binary Generator block to the Binary Symmetric Channel block.

6 Click the Hamming Decoder block and drag it on top of the line between the Binary Symmetric Channel block and the Error Rate Calculation block.

Using the Hamming Encoder and Decoder Blocks

The Hamming Encoder block encodes the data before it is sent through the channel. The default code is the [7,4] Hamming code, which encodes message words of length 4 into codewords of length 7. As a result, the block converts frames of size 4 into frames of size 7. The code can correct one error in each

transmitted codeword.

For an [n,k] code, the input to the Hamming Encoder block must consist of vectors of size k. In this example, k=4.

The Hamming Decoder block decodes the data after it is sent through the channel. If at most one error is created in a codeword by the channel, the block decodes the word correctly. However, if more than one error occurs, the Hamming Decoder block might decode incorrectly.

Setting Parameters in the Hamming Code Model

Double-click the Bernoulli Binary Generator block and make the following changes to the parameter settings in the block’s dialog, as shown in the following figure:

1 Select the box next to Frame-based outputs in the dialog for the Bernoulli Binary Generator block.

2 Set Samples per frame to 4. This converts the output of the block into frames of size 4, in order to meet the input requirement of the Hamming Encoder Block.

Note Many Communications Blockset blocks, such as the Hamming Encoder block, require their input to be a vector of a specific size. If you connect a source block, such as the Bernoulli Binary Generator block, to one of these blocks, you should select the box next to Frame-based outputs in the dialog for the source, and set Samples per frame to the required value.

Labeling the Display Block

You can change the label that appears below a block to make it more informative. For example, to change the label below the Display block to "Error Rate Display," first select the label with the mouse. This causes a box to appear around the text. Then enter the changes to the text in the box.

Running the Hamming Code Model

To run the model, select Start from the Simulation menu. The model terminates after 100 errors occur. The error rate, displayed in the top window of the Display block, is approximately .001. Note that you get slightly different results if you change the Initial seed parameters in the model or run a simulation for a different length of time.

You expect an error rate of approximately .001 for the following reason. The probability of two or more errors occurring in a codeword of length 7 is

If the codewords with two or more errors were decoded randomly, you would expect about half the bits in the decoded message words to be incorrect. This indicates that .001 is a reasonable value for the bit error rate.

To obtain a lower error rate for the same probability of error, you can try using a Hamming code with larger parameters. To do this, change the parameters Codeword length and Message length in the Hamming Encoder and Decoder block dialogs. You also have to make the appropriate changes to

the parameters of the Bernoulli Binary Generator block and the Binary Symmetric Channel block.

Displaying Frame Sizes

You can display the sizes of data frames in different parts of the model by selecting Signal dimensions from the Port/signal displays submenu of the Format menu at the top of the model window. This is shown in the following figure. Note that the line leading out of the Bernoulli Binary Generator block

is labeled [4x1], indicating that its output consists of column vectors of size 4. Since the Hamming Encoder block uses a [7,4] code, it converts frames of size 4 into frames of size 7, so its output is labeled [7x1].

Modeling a Channel with Modulation

The Binary Symmetric Channel block, which simulates a channel with noise, is useful for building models of channel coding. For other types of applications, you might want to construct a more realistic model of a channel.

For example, you can add modulation and demodulation, and replace the Binary Symmetric Channel block with an AWGN Channel block, which adds white Gaussian noise to the channel. This following figure shows an example that uses binary phase shift keying (BPSK).

Building the BPSK Model

You can build the BPSK model from the one shown in the figure Channel Noise Model. To build the model, follow these steps:

1 Enter channeldoc at the MATLAB prompt to open the channel noise model, and then save the model as my_bpsk in the directory where you keep your work files.

2 Delete the Binary Symmetric Channel block from the model by right-clicking the block and selecting Clear.

3 Move the following blocks from the Simulink Library Browser into the model window, and insert them into the model as shown in the following figure:

• BPSK Modulator Baseband block, from PM in the Digital Baseband Modulation sublibrary of the Modulation library

• AWGN Channel block, from the Channels library

• BPSK Demodulator Baseband block, from PM in the Digital Baseband Modulation sublibrary of the Modulation library

The model should now appear as in the figure below.

Binary Phase Shift Keying

The BPSK Modulator and Demodulator Baseband blocks implement binary phase shift keying (BPSK) modulation. BPSK is a method for modulating a binary signal onto a complex waveform by shifting the phase of the complex signal. In digital baseband BPSK, the symbols 0 and 1 are modulated to the

complex numbers exp(jt) and -exp(jt), respectively, where t is a fixed angle. In this example, t = 0, so these numbers are just 1 and -1.

You can set the value of t in the Phase offset parameter in the dialogs for the BPSK Modulator Baseband block and the BPSK Demodulator Baseband block. The default value is 0.

Setting Parameters in the BPSK Model

To set block parameters in the BPSK model, do the following:

1 Double-click the AWGN Channel block and set Es/No to 4.2.

2 Double-click the Error Rate Calculation block and make the following changes to the default parameters in the block’s dialog:

• Set Output data to Port.

• Check the box next to Stop simulation.

Running the BPSK Model

When you run the model, the Display block shows an error rate of approximately 0.01, the same as in the channel noise model. The BPSK model uses the BPSK Modulator Baseband, the AWGN Channel, and the BPSK Demodulator Baseband blocks to simulate a channel with noise. This provides a more realistic model of a channel than using just the Binary Symmetric Channel block. You can also model other types of channel noise using blocks from the Communications Blockset Channels library.

Reducing the Error Rate with a Cyclic Code

You can improve the error rate in the model shown in the figure BPSK Modulation Model, for certain noise levels, by adding channel coding. An example that uses a binary cyclic code is shown below.

Building the Cyclic Code Model

You can build the cyclic code model by adding blocks to the BPSK model shown in the figure BPSK Modulation Model. To build themodel, follow these steps:

1 Open the BPSK model by entering bpskdoc at the MATLAB prompt. Save the model as my_cyclic in the directory where you keep your work files.

2 Double-click the AWGN Channel block and set Es/No to 7. This value is chosen for this example to illustrate the benefit of using a code. If Es/N0 were much lower than the value chosen here, then the Es/N0 reduction caused by the additional coded bits would be greater than the coding gain provided by the code.

3 Run the simulation and note its error rate (for later comparison in “Running the Cyclic Code Model” on page 2-45). The error rate appears in the top entry of the Display block.

4 To introduce cyclic coding, drag these Communications Blockset blocks from the Simulink Library Browser into the model window:

• Binary Cyclic Encoder block, from the Block sublibrary of the Error Detection and Correction library

• Binary Cyclic Decoder block, from the Block sublibrary of the Error Detection and Correction library

5 Widen the model window and connect the blocks as below.

6 Double-click the Bernoulli Binary Generator block and change these parameters:

• Select Frame-based outputs.

• Set Samples per frame to 21 to match the input requirement of the Binary Cyclic Encoder block.

7 Double-click the Binary Cyclic Encoder block and change these parameters:

• Set Codeword length N to 31.

• Set Message length K to 21.

Make the same changes in the Binary Cyclic Decoder block.

8 Double-click the AWGN Channel block and change these parameters:

• Set Es/No to 7+10*log10(21/31). The second term in this sum accounts for the difference in symbol period compared to the original BPSK model.

• Set Symbol period to 21/31. For more information on setting Symbol period, see “Verifying the Symbol Period” below.

With these parameter values, the AWGN channel block produces the same amount of noise per symbol as in the BPSK model (without coding) in which Es/No is 7. The equivalence of the noise enables you to determine how much the cyclic code improves the bit error rate.

9 Double-click the Error Rate Calculation block and change this parameter:

• Set Maximum number of symbols to 1e7. This extends the simulation, producing a more reliable estimate for the error rate.

Binary Cyclic Encoder and Decoder

The Binary Cyclic Encoder block implements a binary cyclic code. In this example, the block has the following parameter settings:

• Codeword length N = 31

• Message length K = 21

The code rate is given by

This example uses a rate 21/31 code. The codeword length N must have the form 2M - 1, where M is an integer greater than or equal to 3. The input to the block must be a vector whose length is Message length.

The Binary Cyclic Decoder block decodes the demodulated signal. This block must have the same parameter settings as the Binary Cyclic Encoder block.

Running the Cyclic Code Model

When you run the simulation, the bit error rate is less than one-tenth of the error rate in the model that does not have channel coding.

Verifying the Symbol Period

When you compare the cyclic code model to the BPSK model, which does not have channel coding, the ratio of energy per information symbol to noise spectral density, Eb/N0, should be the same in both models. You can use the Symbol period and Es/No parameters in the AWGN Channel block to adjust

the amount of channel noise so that Eb/N0 is the same as in the model without coding. Because the cyclic code has rate 21/31, you set Symbol period to 21/31. For a BPSK simulation with a rate K / N code, set Symbol period to K/N because there are K information symbols for each N channel symbols.

Note that Es/No, the ratio of energy per channel symbol to noise spectral density, is not the same as Eb/N0. To convert between the two, use the formula

where K / N is the ratio of information symbols to channel symbols. Changing the Symbol period to K/N has the same effect as subtracting 10log(K / N) from the Es/No parameter.

Task: Build the FSK Model Shown in the Figure Below:

Task: Building a Frequency-Shift Keying Model

Frequency-shift keying (FSK) is a standard modulation technique in which a digital signal is modulated onto a sinusoidal carrier whose frequency shifts between different values. The Bell Telephone System first used this technique in their Model 103 modem. The model shown in the following figure is an

example of the baseband representation of FSK.

Simulation Parameters:

Bernoulli Generator:

Probability of a zero = 0.5

Sample time = 1/1200

M-FSK Modulator and Demodulator

M-ary Number = 2

Frequency separation = 1000

Samples per Symbol = 5

AWGN Channel

Mode = Signal to Noise Ratio (Eb/No) = 10

Symbol Period = 1/1200

Error-Rate Calculation

Set Receive Delay = 1

Output Data = Port

Relational Operator

Relational Operator = “~=”

Set the Simulation Stop Time parameter to 15 and Run the Model.

Lab work № 1

Introduction to matlab, simulink, and the communication Toolbox

1) Objective

The objective of this lab is to review how to access Matlab, Simulink, and the Communications Toolbox, and to become familiar with the basic operations of these applications.

2) Equipment

Matlab, Simulink, and the Communication Toolbox software.

3)  Matlab

3.1) Introduction

Matlab is a computing environment specially designed for matrix computations. It is widely used for the study of a variety of applications, including circuits, signal processing, control systems, communications, image processing, symbolic mathematics, statistics, neural networks, wavelets, and system identification. Its large library of built-in functions and toolboxes, as well as its graphical capabilities, make it a valuable tool for electrical engineering education and research.

Matlab has an interactive mode in which user commands are interpreted immediately as they are typed. Alternatively, a program (called a script) can be written in advance using a text editor, saved as a file, and then executed in Matlab.

3.2) Matrix Manipulation

The basic objects manipulated by Matlab are two-dimensional matrices (though recent versions can process multidimensional matrices). Recall that a vector is a special case of a matrix that has only one row or one column. In this course, we will define a vector as a column vector, which corresponds to a single column of a matrix, e.g., an N ×1 matrix with N rows and one column. A row vector is obtained from a column vector by using the transpose operator.

You will find that Matlab is extremely powerful when performing matrix manipulations because many scalar operations operate in parallel for all elements of a matrix. This almost eliminates the need for iterative loops employed in most conventional programming languages. For example, in order to generate s ( n ) sin(2 n /1000) for n = 1, . . . , 1000:

In C,

for (n = 1; n <= 1000; n + +)

{

s(n) = sin(2*pi* n/1000)

}

In Matlab, we could use a for loop as follows: for n = 1 : 1000,

s(n) = sin(2*pi*n/1000); end

However, it is much simpler to write: s = sin(2∗pi ∗ (1 : 1000) /1000);

Since Matlab programs are interpreted (not compiled), for loops and while loops are inefficient. They should be avoided whenever possible.

3.3) Graphical Plotting

Matlab supports graphical plotting on the computer screen and to a printer. The command for plotting on the screen is plot, which can be used in several formats as follows.

- plot(y) % Plots vector y versus its index.

- plot(x, y) % Plots vector y versus vector x.

- plot(x, y ’<line type>’) % Plots vector y versus vector x with the specified <line type>.

Possible line types include line, point, and color specifications. The command plot can also take on other forms depending on its argument list. Please refer to the on-line help for a detailed discussion of this command.

A number of commands are used to help generate the desired plot. These include: axis, hold, title, xlabel, ylabel, text, gtext , etc. Some of these will be needed for this lab – please refer to the on- line help for details on these commands. Use the print command to obtain a high-quality hard copy of the current screen plot.

3.4) Obtaining Help

There are two main ways of obtaining help. The first way is by invoking the help command on the Matlab command window to provide you with a list of topics for which help is available. By using help <topic> where topic is the name of a toolbox or function, you will receive either a list of functions available in the toolbox, or an explanation of the function itself, respectively.

The second method of obtaining on -line help is by reading the manuals that are installed on the system in PDF format. You can use a web browser with the Adobe Acrobat Reader plug-in to view these manuals.

4) Simulink

Simulink is a program for simulating signals and dynamic systems. As an extension of Matlab, Simulink adds many features specific to the simulation of dynamic systems while retaining all of Matlab’s general purpose functionality.

Simulink has two phases of use: model definition and model analysis. A typical session starts by either defining a new model or by recalling a previously defined model, and then proceeds to analyze that model. In order to facilitate the model definition, Simulink has a large class of windows called block diagram windows. In these windows, models are created and edited principally by mouse-driven commands. An important part of mastering Simulink is to become familiar with manipulations of various model components in these windows.

After you define a model, you can analyze it either by choosing options from the Simulink menus or by entering commands in the Matlab command window. The progress of an ongoing simulation can be viewed while it is running, and the final results can be made available in the Matlab workspace when the simulation is complete.

5) Pre-work Tasks

5.1) Matlab

Please read the following topics using the on-line help documents (or a book on Matlab).

Command mode operation. On-line help.

Script editing and execution.

Command language: constants, expressions, assignments, m-files, function calls, function definitions, and comments.

Matlab commands: diary, echo, type, !, pause, quit, who, and whos. Predefined variables: ans, i, j, and pi.

Built-in operators for vector manipulation: :(range selector), ’(transpose), +, −, *, /, ˆ,.*, ./, and .ˆ .

Language constructs: for, while, end, if, and break.

Commands to generate graphical plots: plot, bar, stairs, title, xlabel, ylabel, text, gtext, hold, axis, grid, clg, and print.

Built-in functions: abs, angle, clear, conj, cos, exp, imag, real, sin, rem, round, ceiling, floor, fix, fliplr, flipud.

5.2) Simulink

Now we describe how to construct a simple model using Simulink software, and how to simulate that model. The basic techniques you use to construct and simulate this simple model are the same as those for more complex models. The model described in this section integrates a sine wave and displays the result along with the original wave. When completed, the block diagram of the model should look similar to this:

Follow these steps to construct the model:

Click the Simulink icon   in the MATLAB toolbar to enter simulink or click the MATLAB Start button, then select Simulink > Library Browser.

The Library Browser appears. It displays a tree-structured view of the Simulink block libraries installed on your system. You build models by copying blocks from the Library Browser into a model window.

Select File > New > Model in the Simulink Library Browser to construct a new model. An empty model window appears as shown below.

To construct a model, you first copy blocks from the Simulink Library Browser to the model window. To create the simple model in this section, you need four blocks:

  - Sine Wave — To generate an input signal for the model

  - Integrator — To process the input signal

  - Scope — To visualize the signals in the model

  - Mux — To multiplex the input signal and processed signal into a single scope To add blocks to your model:

    - Select the Sources library in the Simulink Library Browser. The Simulink Library Browser displays the Sources library.

- Select the Sine Wave block in the Simulink Library Browser, then drag it to the model window. A copy of the Sine Wave block appears in the model window.

  - Select the Sinks library in the Simulink Library Browser.

  - Select the Scope block from the Sinks library, then drag it to the model window. A Scope block appears in the model window.

  - Select the Continuous library in the Simulink Library Browser.

  - Select the Integrator block from the Continuous library, then drag it to the model window. An Integrator block appears in the model window.

  - Select the Signal Routing library in the Simulink Library Browser.

  - Select the Mux block from the Sinks library, then drag it to the model window. A Mux block appears in the model window.

Before you connect the blocks in your model, you should arrange them logically to make the signal connections as straightforward as possible.

After adding blocks to the model window, you must connect them to represent the signal connections within the model. Notice that each block has angle brackets on one or both sides. These angle brackets represent input and output ports:

    - The > symbol pointing into a block is an input port.

    - The > symbol pointing out of a block is an output port.

To draw a line between two blocks:

  - Position the mouse pointer over the output port on the right side of the Sine Wave block. Note that the pointer changes to a cross hairs (+) shape while over the port.

  - Drag a line from the output port to the top input port of the Mux block. Note that the line is dashed while you hold the mouse button down, and that the pointer changes to a double-lined cross hairs as it approaches the input port of the Mux block. Release the mouse button. The software connects the blocks with an arrow that indicates the direction of signal flow.

The model should look similar to the following figure after making other connections:

The model is almost complete. To finish the model, you must connect the Sine Wave block to the Integrator block. This final connection is somewhat different from the other three. Because the output port of the Sine Wave block already has a connection, you must connect this existing line to the input port of the Integrator block. The new line, called a branch line, carries the same signal that passes from the Sine Wave block to the Mux block. To weld a connection to an existing line:

  - Position the mouse pointer on the line between the Sine Wave and the Mux block.

    - Press and hold the Ctrl key, then drag a line to the Integrator block’s input port. The software draws a line between the starting point and the input port of the Integrator block as shown below.

After you complete the model, you should save it for future use. To save the model:

  - Select File > Save in the model window.

  - Specify the location in which you want to save the model.

  - Enter simple_model in the File name field.

  - Click Save.

The software saves the model with the file name simple_model.mdl.

You can now simulate the system and visualize the results as follows.

Set simulation options such as the start and stop time, and the type of solver that Simulink software uses to solve the model at each time step. You specify these options using the Configuration Parameters dialog box.

To specify simulation options for the sample model:

  - Select Simulation > Configuration Parameters in the model window. The software displays the Configuration Parameters dialog box.

- Enter 20 in the Stop time field.

- Click OK.

Now you are ready to simulate your sample model and observe the simulation results. To run the simulation:

  - Select Simulation > Start in the model window. The software runs the model, stopping when it reaches the stop time specified in the Configuration Parameters dialog box.

  - On computers running the Microsoft® Windows® operating system, you can also click the Start simulation button   and Stop simulation button   in the model window toolbar to start and stop a simulation.

  - Double-click the Scope block in the model window. The Scope window displays the simulation results.

6) Problems

6.1) Using Matlab

- Express the following numbers in polar form and rectangular form using Matlab. a. 1 j 3 2 j 1 j e j /8

| b. |  | 3 | j 2 2e | j /6 | 0.4 |  |
| --- | --- | --- | --- | --- | --- | --- |

- Find the roots of the following expressions using Matlab.

  - 2.72 x 2   1.4 x  6.15  0

  - 4 x 3    25 x  54  0

- Define a function dist_p (x0 , y0 ) which takes a point (x0 , y0 ) and returns a distance from this point to the line y 2 x 4 . Using this function, find distances from the following points.

    - (4,1)

    - (0,0)

| 4. | Calculate and plot c(n ) | n |  | en |  | for n 1, 2,...10. |
| --- | --- | --- | --- | --- | --- | --- |
| 5. | Calculate and plot x (n ) e 0.007n cos(2 n /100 3) for n 1, 2,...,1000. On the |  |  |  |  |  |

same graph, plot e 0.007 n and e 0.007n with different colors and line types. Label the curves to identify each one.

6.  Generate 1  20 vector of uniformly distributed random numbers over ( 3,4) .

6.2) Using Simulink

- Study following Simulink blocks using scope: signal generator, digital clock, step, uniform random number, pulse generator, and sine wave function. The use of uniform random number is outlined below:

o  After these arrangements, start the simulation and observe the result.

o  The figure below indicates a sample simulation result.

- Do similar steps for other sources, that is, signal generator and scope, digital clock and scope, step and scope, pulse generator and scope, and sine wave function and scope. For the values of the ‘Block Parameters’ of the sources, choose your own.

  - Show all of these simulation results and the ‘Block Parameters’ values of the sources at your lab report.

- Follow the steps below to design a multiplier with a square-wave input.

    - Start a Simulink session.

    - Copy the pulse generator block by dragging it to your model. Using the left mouse button, double-click on the pulse generator block. A new window appears that displays all the properties of your selected block. Specify the model fields as follows.

Period: 1

Pulse width: 50% Amplitude: 0.7

    - Next, multiply the output of the signal generator by a gain. You can use a gain block from the math library by placing it in the design window. Double-click on the gain block to open the property window and set the gain to 0.9.

    - Connect the blocks.

    - To analyze the output, drag the Scope block from Sinks of the Simulink block library. Double click the Scope block to open the Scope window. Specify the Scope axes as follows.

Ymax: 1.5 Xmax: −1.5 Time range: 10

    - Select the Simulation menu and then parameters to open the Simulation control window. Set the simulation parameters as follows.

Start time: 0.0

Stop time: 10.0

Solver options Type: Fixed step discrete (no continuous states)

Fixed step size: 0.01

- Select the Simulation menu and then run the simulation by clicking Start.

- Save your model file as a Matlab mdl-file.

6.3) Using Communication Toolbox

Open the Communication Toolbox located in Blocksets&Toolboxes!Comm Tbx Library. This shows all of the elements available to a communication system. Open several of the blocks in order to see what functions are available; for example, look at Modulation and Analog Modem. Within Analog Modem, there are several modulation techniques. Try experimenting with some of the blocks to familiarize yourself with what is available. If you need further help, there is an on -line tutorial in PDF form for the Communications Toolbox which provides descriptions of the available functions and blocks.

7) Lab Report

Answer all questions and provide supporting documents.

REFERENCES:

MATLAB Help, Simulink 7,User’s Guide and Getting Started Guide

Lab work № 2

ANALOG MODULATION USING MATLAB

- OBJECTIVE

To generate AM, DSB-SC AM, PM, and FM waveforms using MATLAB

To download MATLAB generated waveforms into the Agilent E4438C vector signal generator

To observe and analyze the modulated waveforms in both time and frequency domains

- PRELAB

Consider an FM system with the carrier c (t )  cos(2  f c t) and the message signal

Let u(t) denote the modulated signal. Assume that t0	0.18 sec, fc     300 Hz, and the

deviation constant k f      40.

- Obtain the expression for u(t).

- Plot the message and the modulated signals using MATLAB.

- Determine the spectra of the message and the modulated signals using MATLAB.

- LAB WORK

  - Copy the experiment file.

  - Examine and then run the file; observe that by selecting the message signal mt and the modulated signal yt, you are able to see the different modulation types: AM, DSB-SC AM, PM, and FM.

  - Generate these waveforms one by one, observe the MATLAB figure plots, and confirm that they are the theoretically expected waveforms, by writing in your lab report the mathematical expression for each one.

  - This MATLAB code also generates the waveform data to be downloaded into the Agilent E4438C vector signal generator.

    - In the Windows Internet Explorer, set the IP address to 10.10.96.185, this will bring you the Agilent E4438C vector signal generator menu. Press Signal Generator FTP Access and choose the directory of BBG1/WAVEFORM to download the MATLAB generated waveform data (via copy and paste operations) into the Agilent E4438C vector signal generator.

    - Connect I and Q outputs of the vector signal generator (located at the rear) to the CH1 and CH2 of the Agilent DSO6034A oscilloscope.

    - On the vector signal generator, press Mode Setup under the MENUS. Then press Dual ARB soft key on the right menu. After that, press Select Waveform using the soft key on the right of the screen. Using the arrow keys near the numeric keypad select the MATLAB generated waveform and press Select Waveform soft key (which is already displayed as *NONE*).

- Control whether ARB selection is set to On. If ARB selection is set to Off, press the corresponding soft key to set ARB selection to On.

- After performing these steps, either remotely from the vector signal generator menu on the PC screen or locally from the front panel of the vector signal generator, the replica of the waveforms observed as MATLAB figures in the form of message signal mt and the modulated signal yt, should appear on the oscilloscope screen on CH1 and CH2. Note that, once downloaded into the vector signal generator, they are converted into periodic signals. Observe and draw all the waveforms one by one in your lab report, placing mt above yt, so that you can make comparisons. Record the amplitude and time axis information for each plot. Compare these with the settings in the MATLAB code.

Note 1: During the successive downloads, remember to delete the existing file of the same name in the directory, otherwise FTP connection to the signal generator will be blocked.

Note 2: During the successive downloads, remember to set ARB selection to Off, otherwise FTP connection to the signal generator will be blocked.

- Investigate how the frequency and the period of the downloaded waveforms, are related to the settings of fs, fc, fm in the MATLAB code and record your findings in the lab report. Also investigate the amplitude relations between the MATLAB settings and those read from the oscilloscope.

- Include in the given MATLAB code, a modulating message signal, mt, in the form of a decaying exponential, and then let this waveform modulate the presently used carrier in AM, DSB-SC AM, PM, and FM. Obtain the modulated waveform both as a MATLAB plot and also on Agilent DSO6034A oscilloscope. Comment if the waveforms are as theoretically expected.

- Connect the yt output (note that mt corresponds to I channel and yt corresponds to Q channel of the vector signal generator) to the Agilent E4407B spectrum analyzer. Observe and plot in your lab report the frequency spectrum of all generated waveforms.

- Include in your lab report, the general comments and a description of what you have learned from this experiment.

MATLAB CODE

% Amplitude and angle modulation of sinusoidal, square, ramp, and quadratic message signals clear all; close all

fs = 2e3; tstart = 0; tend = 4; t = tstart:tend/fs:tend-1/fs; Am = 1.0; Ac = 1.0;

fc = 10; fm = 1; kp = 25; kf= 25; ka = 0.5; a = 0.1;

T = t(end); t1 = numel(t)/2; % Adjust time scale of 4 seconds

ct = Ac*cos(2*pi*fc*t); % Carrier

m0t = a*cos(2*pi*fm*t); % Sinusoidal message signal

m1t = [Am*ones(1,t1) -Am*ones(1,t1)]; % Square waveform

m2t = [Am*t(1:t1) -2*Am*(t(t1+1:end)-T)/(T-t(t1))]; % Ramp waveform

m3t = [Am*t(1:t1).^2/2 2+Am*(-t(t1+1:end).^2+2*T*t(t1+1:end)+t(t1)^2-2*T*t(t1))/(T-t(t1))]; %Quadratic

yt_AMm0t = Ac*(1 + ka*m0t).*cos(2*pi*fc*t); % AM of sinusoidal waveform yt_AMm1t = Ac*(1 + ka*m1t).*cos(2*pi*fc*t); % AM of square wave yt_AMm2t = Ac*(1 + ka*m2t).*cos(2*pi*fc*t); % AM of ramp waveform yt_AMm3t = Ac*(1 + ka*m3t).*cos(2*pi*fc*t); % AM of quadratic waveform

yt_DSBm0t = Ac*ka*m0t.*cos(2*pi*fc*t); % DSB-SC AM of sinusoidal waveform yt_DSBm1t = Ac*ka*m1t.*cos(2*pi*fc*t); % DSB-SC AM of square wave yt_DSBm2t = Ac*ka*m2t.*cos(2*pi*fc*t); % DSB-SC AM of ramp waveform yt_DSBm3t = Ac*ka*m3t.*cos(2*pi*fc*t); % DSB-SC AM of ramp waveform

yt_PMm0t = Ac*cos(2*pi*fc*t + kp*m0t); % PM of sinusoidal waveform yt_PMm1t = Ac*cos(2*pi*fc*t + kp*m1t); % PM of square wave yt_PMm2t = Ac*cos(2*pi*fc*t + kp*m2t); % PM of ramp waveform yt_PMm3t = Ac*cos(2*pi*fc*t + kp*m3t); % PM of quadratic waveform

yt_FMm0t = Ac*cos(2*pi*fc*t + kf*a*sin(2*pi*fm*t)); % FM of sinusoidal waveform yt_FMm1t = Ac*cos(2*pi*fc*t + kf*m2t); % FM of square waveform

yt_FMm2t = Ac*cos(2*pi*fc*t + kf*m3t); % FM of ramp waveform yt_FMm3t = Ac*cos(2*pi*(fc + kf*m3t).*t); % FM of quadratic waveform

mt = m1t; yt = yt_AMm1t; % Select a message signal and modulated signal to download and plot mt = mt / max(abs(mt)); yt = yt / max(abs(yt)); % Normalization

% Construction of the waveform to be loaded into E4438C ESG vector signal generator waveform(1:2:2*length(yt)) = mt;

waveform(2:2:2*length(yt)) = yt;

waveform = waveform / max(abs(waveform));

waveform = round(32767*waveform); waveform = uint16(mod(65536 + waveform,65536)); if strcmp( computer, 'PCWIN' )

waveform = bitor(bitshift(waveform,-8),bitshift(waveform,8)); end

filename = 'C:\MATLAB7\work\EEM496\Exp2_AM1';

[FID, message] = fopen(filename,'w'); % Open a file to write data if FID == -1 error('Cannot Open File'); end fwrite(FID,waveform,'unsigned short'); fclose(FID);

% Plot the waveforms in MATLAB for verification subplot(2,1,1); plot(t,mt,'LineWidth',4,'Color','Red')

ylabel('Amplitude','FontSize',12,'FontWeight','bold'); title('Message signal','FontSize',12,'FontWeight','bold') axis ([-(max(t)-min(t))*0.0 max(t)*1.0 min(mt)*1.1 max(mt)*1.1]);set(gca,'FontSize',16); subplot(2,1,2);plot(t,yt)

ylabel('Amplitude','FontSize',12,'FontWeight','bold')

title('… Modulation of the message signal','FontSize',12,'FontWeight','bold') set(gcf,'Color',[1 1 1]);set(gca,'FontSize',16);

Lab work №3

WHITE GAUSSIAN NOISE

- OBJECTIVE

To study the delta function in both time and frequency domains

To obtain the autocorrelation and power spectral density of MATLAB generated White Gaussian Noise (WGN)

To obtain the pdf of WGN

To download MATLAB generated WGN into the Agilent E4438C vector signal generator

- LAB WORK

- Copy the experiment file.

- Examine and then run the file using MATLAB; note that three figures are generated.

- In the first figure, observe that the FFT (Fast Fourier Transform) of delta function is flat.

- In the second figure, autocorrelation and power spectral density of MATLAB generated White Gaussian Noise (WGN) are given.

- In the third figure, the pdf of WGN is seen.

- Study these figures thoroughly, while recording them in your lab report.

- After the observations, identify in the m file the sections (by line numbers) that produce the above mentioned three figures.

- At the end of this m file, the time variations of noise signal (the upper figure in Figure

  - is generated with the name “noise_waveform”.

- Download the “noise_waveform” into Agilent E4438C vector signal generator:

    - In the Windows Internet Explorer, set the IP address to 10.10.96.185.

    - Press the Signal Generator FTP Access and choose the directory of BBG1/WAVEFORM.

    - Ensure that CH1 of the Agilent DSO6034A oscilloscope is connected to I (or

Q) output located at the rear of the signal generator.

- On the vector signal generator, press Mode Setup under the MENUS. Then press Dual ARB soft key on the right menu. After that, press Select Waveform using the soft key on the right of the screen. Using the arrow keys near the numeric keypad select the MATLAB generated “noise_waveform”

and press Select Waveform soft key. From the right menu, set ARB selection to On. From Mode Setup, ARB Setup, make sure that ARB Sample Clock is at 100 MHz and IQ Mode Filter is at 40 MHz. After performing these steps, the replica of the noise time waveform observed as MATLAB Figure 3 should appear on the oscilloscope screen on CH1.

Note 1: During the successive downloads, remember to delete the existing file of the same name in this directory, otherwise FTP connection to the signal generator will be blocked.

Note 2: During the successive downloads, remember to set ARB selection to Off, otherwise FTP connection to the signal generator will be blocked.

- Connect one of the remaining I or Q ( I or Q ) from the rear panel of Agilent E4438C vector signal generator to the Agilent E4407B Spectrum Analyzer. By using FREQUENCY Channel button on the spectrum analyzer, set center frequency to 0 MHz, start frequency to –60 MHz, stop frequency to 60 MHz. By using Amplitude Y scale, adjust attenuation to 0 dB, Ref. level to –10 dBm. You can try this step for different attenuation values like 20 dB.

- Initially set var (variance of noise power) in MATLAB m file to –20 (dBW), then by equating n (the number of noise samples used for Figure 3) to 100, 1000, 10000, 100000, 1000000, measure the rms values of noise waveform on the oscilloscope and record these readings in your lab report.

- Use the spectrum analyzer for the same values of n and by pressing Measure, Channel Power, take readings of Occupied BW, Channel Power and Power Spectral Density.

- Repeat the above readings for var = –15, –10, –5, 0. Tabulate these results and record them in your lab report.

- Include in your lab report, the general comments and a description of what you have learned from this experiment.

MATLAB CODE

- This program attempts to verify that the frequency spectrum of White Gaussian Noise is white (i.e. flat)

- First, study the delta function. This is one and a (large) number of zeros

close all; clear all; clc; clf

n = 1000; xt_delta = [ones(1,1) zeros(1,n)]; figure(1)

subplot(2,1,1); plot(1:length(xt_delta),xt_delta,'LineWidth',4,'Color','green') xlabel('t (Time)','FontSize',12,'FontWeight','bold'); ylabel('Amplitude','FontSize',12,'FontWeight','bold');

title('Delta Function','FontSize',12,'FontWeight','bold')

axis ([-15 n 0 1.1]); set(gcf,'Color',[1 1 1]); set(gca,'FontSize',16); Xf_delta = fft(xt_delta,512); Power_Xf = Xf_delta.* conj(Xf_delta) / 512; Power_Xf = Power_Xf / max(Power_Xf); f = 1000*(0:256)/512; subplot(2,1,2); plot(f,Power_Xf(1:257),'LineWidth',4,'Color','Red');

axis ([-(max(f)-min(f))*0.05 max(f)*1.1 0 1.1]); xlabel('f (Frequency)','FontSize',12,'FontWeight','bold'); ylabel('Amplitude','FontSize',12,'FontWeight','bold');

title('Fourier Transform of Delta Function','FontSize',12,'FontWeight','bold') set(gcf,'Color',[1 1 1]); set(gca,'FontSize',16);

- Whiteness of White Gaussian Noise

- As n (number of noise samples) and no_run are increased, uniformity increases n = 1000; m = 1000; no_run = 100;

- m should be the same as n, otherwise correlation deviates towards the edges

rn_ tot=zeros(1,2*m-1); for j=1:no_run,

noise = wgn(1,n,0.01);

rn = xcorr(noise,m-1,'unbiased'); rn_tot = rn_tot + rn;

end;

rn_tot = rn_tot/no_run; figure(2) subplot(2,1,1); plot(-m+1:m-1,rn_tot)

xlabel('\tau (Time)','FontSize',12,'FontWeight','bold'); ylabel('Normalized Amplitude','FontSize',12,'FontWeight','bold'); title('Autocorrelation of WGN','FontSize',12,'FontWeight','bold') set(gcf,'Color',[1 1 1]); set(gca,'FontSize',16);

noise_spect = fftshift(abs(fft(rn_tot))); subplot(2,1,2); plot(-m+1:m-1,noise_spect);

xlabel('f (Frequency)','FontSize',12,'FontWeight','bold'); ylabel('Normalized Power Density','FontSize',12,'FontWeight','bold'); title('Power Spectrum of WGN','FontSize',12,'FontWeight','bold'); set(gcf,'Color',[1 1 1]); set(gca,'FontSize',16);

- The pdf of White Gaussian Noise

- n is the number of noise samples and var (variance) is the noise power in dBW n = 1000000; var = -10; noise = wgn(1,n,var); figure(3)

subplot(2,1,1); plot(1:n,noise);

xlabel('t (Time)','FontSize',12,'FontWeight','bold'); ylabel('Amplitude','FontSize',12,'FontWeight','bold');

title('Time Variation of White Gaussian Noise','FontSize',12,'FontWeight','bold'); set(gcf,'Color',[1 1 1]); set(gca,'FontSize',16);

[nn,xout] = hist(noise,50); subplot(2,1,2); bar(xout,nn/n);

xlabel('Amplitude','FontSize',12,'FontWeight','bold'); ylabel('Noise pdf','FontSize',12,'FontWeight','bold'); set(gcf,'Color',[1 1 1]); set(gca,'FontSize',16);

- Constructing the noise waveform to be loaded into ESG 4438C vector signal generator

- Note that both I and Q are identical in this case

waveform(1:2:2*length(noise)) = noise; waveform(2:2:2*length(noise)) = noise; waveform = waveform / max(abs(waveform)); waveform = round(32767*waveform);

waveform = uint16(mod(65536 + waveform,65536)); if strcmp( computer, 'PCWIN' )

waveform = bitor(bitshift(waveform,-8),bitshift(waveform,8)); end

filename = 'C:\MATLAB7\work\noise_waveform';

[FID, message] = fopen(filename,'w'); % Open a file to write data if FID == -1 error('Cannot Open File'); end fwrite(FID,waveform,'unsigned short'); % write to the file fclose(FID);

Lab work № 4

DIGITALLY MODULATED SIGNALS USING MATLAB:

PAM (ASK), PSK, AND QAM

- OBJECTIVE

To study digitally modulated signals PAM (ASK), PSK, and QAM using MATLAB and construct their constellation diagrams, baseband and passband forms.

- PRELAB

Consider the following three sets of signals (Set I, Set II, and Set III).

Find the geometric representation of the signals in each set. What is the average transmitted energy for each signal set?

- LAB WORK

    - Copy the experiment file.

    - Examine and then run the file, observe the OPs.

    - Initially label the OP of randint as x1 (t), x2 (t),..., xM (t) and the corresponding OP of dmodce as s1 (t), s2 (t),..., sM (t) . Set M = 8 and 16 consecutively, find and draw the corresponding waveforms and write for vectors of PAM (ASK), PSK and QAM. Indicate these on the waveforms and constellation diagrams. Label your drawings clearly. During your runs do not change the original n, Fs, and fc settings.

    - Include your general comments.

MATLAB CODE

- This program shows the illustration of PAM (ASK), PSK, and QAM signals.

  - Both bandpass and passband signals are shown. Keep the n, Fs , and fc settings. clear; clc; close all

n = 4; % Number of symbol intervals M = 16; % Use M-ary modulation

Fd = 1; % Sampling rate of the message signal Fs = 100; % Sampling rate of the modulated signal fc = 18; % Carrier frequency

x = randint(n,1,M); % Random multilevel message signal (from zero to M-1) xt = reshape(repmat(x,1,Fs)',1,n*Fs); t = 1:1:n*Fs; % Arrange time axis

  - PAM (ASK) Signals

  - A) Baseband ASK

sask = real(dmodce(x,Fd,Fs,'ask',M)); modmap('ask',M); set(gcf,'Color',[1 1 1]) pause

subplot(2,1,1); plot(t,xt); title('Input signal x(t) as M-ary levels') axis ([min(t) max(t) -0.1 max(x)*1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); subplot(2,1,2); plot(t,sask); title('ASK baseband signal - s_m(t)'); xlabel('Time Axis') axis ([min(t) max(t) -1.1 1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); pause; clf reset

- B) Bandpass PAM (ASK) uask = dmod(x,fc,Fd,Fs,'ask',M);

subplot(2,1,1); plot(t,sask); set(gcf,'Color',[1 1 1]) title('ASK baseband signal - s_m(t)')

axis ([min(t) max(t) -1.1 1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); subplot(2,1,2); plot(t,uask); title('ASK bandpass signal - u_m(t)'); xlabel('Time Axis') axis ([min(t) max(t) -1.1 1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); pause

- PSK Signals

- A) Baseband PSK

spsk = dmodce(x,Fd,Fs,'psk',M); figure; modmap('psk',M);set(gcf,'Color',[1 1 1]) pause

subplot(2,1,1); plot(t,xt); title('Input signal x(t) as M-ary levels') axis ([min(t) max(t) -0.1 max(x)*1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold');

s1psk = reshape(spsk,Fs/2,2*n);smtpsk = []; for i = 1:2:2*n-1

s3 = [real(s1psk(:,i));imag(s1psk(:,i+1))]; smtpsk = [smtpsk;s3];

end

subplot(2,1,2); plot(t,smtpsk); title('PSK baseband signal - s_m(t)'); xlabel('Time Axis') axis ([min(t) max(t) -1.1 1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); pause;clf reset

% B) Bandpass PSK

upsk = dmod(x,fc,Fd,Fs,'psk',M);

subplot(2,1,1); plot(t,smtpsk); set(gcf,'Color',[1 1 1]) title('PSK baseband signal - s_m(t)')

axis ([min(t) max(t) -1.1 1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); subplot(2,1,2); plot(t,upsk); title('PSK bandpass signal - u_m(t)'); xlabel('Time Axis') axis ([min(t) max(t) -1.1 1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); pause

- QAM Signals

- A) Baseband QAM

sqam = dmodce(x,Fd,Fs,'qask',M); figure; modmap('qask',M); set(gcf,'Color',[1 1 1]) pause

subplot(2,1,1); plot(t,xt); title('Input signal x(t) as M-ary levels') axis ([min(t) max(t) -0.1 max(x)*1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); s1qam = reshape(sqam,Fs/2,2*n); smtqam = [];

for i = 1:2:2*n-1

s3 = [real(s1qam(:,i));imag(s1qam(:,i+1))]; smtqam = [smtqam;s3];

end

subplot(2,1,2); plot(t,smtqam); title('QAM baseband signal - s_m(t)'); xlabel('Time Axis') axis ([min(t) max(t) -M*0.9/2 M*0.9/2])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); pause; clf reset

% B) Bandpass QAM

uqam = dmod(x,fc,Fd,Fs,'qask',M);

subplot(2,1,1); plot(t,smtqam); set(gcf,'Color',[1 1 1]) title('QAM baseband signal - s_m(t)')

axis ([min(t) max(t) -M*0.9/2 M*0.9/2]) set(gca,'XTick',[50 100 150 200 250 300 350 400])

set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold'); subplot(2,1,2); plot(t,uqam); title('QAM bandpass signal - u_m(t)'); xlabel('Time Axis') axis ([min(t) max(t) -1.1 1.1])

set(gca,'XTick',[50 100 150 200 250 300 350 400]) set(gca,'XTickLabel',{'T/2';'T';'3T/2';'2T';'5T/2';'3T';'10T/2';'4T'},'FontWeight','bold');

Lab work № 5

DIGITAL MODULATION USING SIMULINK

- OBJECTIVE

  - To understand the basic principles of digitally modulated signals: PAM, PSK, and QAM

  - To gain experience in using Simulink.

- PRELAB

Consider a digital communication system that transmits information via QAM over a voice-band telephone channel at a rate of 2400 symbols/second. Assume that the channel is AWGN.

- Determine the Ɛb / No required to achieve an error probability of 10-5 at 4800 bps.

- Repeat part (a) for a bit rate of 9600 bps and a bit rate of 19,200 bps.

- What conclusions do you reach from these results?

3) LAB WORK

PART 1: M-ary PAM

- Start a Simulink session.

  - Select File -> New -> Model in the Simulink Library Browser to construct a new model.

  - Go to Communications Blockset -> Comm Sources -> Random Data Sources sub-folder. Drag and drop Random Integer Generator module into the model window. Double-click on this module and make the following settings:

    - M−ary number to 2

    - Initial seed to 37

    - Sample time to 0.1

    - Output Data Type to double

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation >

AM sub-folder. Drag and drop M-PAM Modulator Baseband module into the model window. Double-click on this module and make the following settings:

  - M−ary number to 2

  - Input type to Integer

  - Constellation ordering to Binary

  - Normalization method to Peak Power

  - Peak power (watts) to 1

  - Samples per symbol to 1

- Go to Communications Blockset -> Channels sub-folder. Drag and drop AWGN Channel module into the model window. Double-click on this module and make the following settings:

  - Initial seed to 37

  - Mode to Signal-to-noise ratio (Es/No)

  - Es/No (dB) to 10

  - Input signal power (watts) to 1

  - Symbol period(s) to 0.1

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation >

AM sub-folder. Drag and drop M-PAM Demodulator Baseband module into the model window. Double-click on this module and make the following settings:

  - M−ary number to 2

  - Output type to Integer

  - Constellation ordering to Binary

  - Normalization method to Peak Power

  - Peak power (watts): to 1

  - Samples per symbol to 1

- Go to Communications Blockset -> Comm Sinks sub-folder. Drag and drop Error Rate Calculation module into the model window.

- Go to Simulink -> Sinks sub-folder. Drag and drop the Display module into the model window. Drag the bottom edge of this inserted (Display module) icon to make the display big enough for three entries.

- Go to Communications Blockset -> Comm Sinks sub-folder. Drag and drop two

Discrete-Time Scatter Plot Scope modules into the model window.

- Go to Simulink -> Math Operations sub-folder. Drag and drop two Complex to Real−Imag modules into the model window.

- Go to Simulink -> Sinks sub-folder. Drag and drop two XY Graph modules into the model window.

- Go to Simulink -> Sinks sub-folder. Drag and drop two Scope modules into the model window.

- Go to Simulink Extras -> Additional Sinks sub-folder. Drag and drop two Power Spectral Density modules into the model window.

- Connect all the inserted modules as shown in Figure 1.

Figure 1: Simulink model for 2-PAM Transmission

- Set the simulation parameters (Simulation -> Configuration Parameters) as follows:

  - Start time to 0.0

  - Stop time to 100.0

  - Type to Variable-step

  - Solver to discrete (no continuous states)

  - Max. step size to auto

- Run (Simulation -> Start) the simulation and observe and save all plots and values in BER display.

- Model and simulate again changing M-ary number of Random Integer Generator, M-PAM Modulator Baseband and M-PAM Demodulator Baseband modules to 4, 8, 16, 32 and 64.

PART 2: M-ary PSK

- BPSK

  - Start a Simulink session. Select File -> New -> Model in the Simulink Library Browser to construct a new model.

  - Go to Communications Blockset -> Comm Sources -> Random Data Sources sub-folder. Drag and drop Random Integer Generator module into the model window. Double-click on this module and make the following settings:

    - M−ary number to 2

    - Initial seed to 37

    - Sample time to 0.1

    - Output Data Type to double

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation >

PM sub-folder. Drag and drop BPSK Modulator Baseband module into the model window.

  - Go to Communications Blockset -> Modulation -> Digital Baseband Modulation >

PM sub-folder. Drag and drop BPSK Demodulator Baseband module into the model window.

  - Now, you will need almost the same modules as in Part 1. Repeat the steps in Part 1 for this case.

  - After running the simulation, observe and save all plots and values in BER display.

- QPSK

  - Start a Simulink session. Select File -> New -> Model in the Simulink Library Browser to construct a new model.

  - Go to Communications Blockset -> Comm Sources -> Random Data Sources sub-folder. Drag and drop Random Integer Generator module into the model window. Double-click on this module and make the following settings:

    - M−ary number to 4

    - Initial seed to 37

    - Sample time to 0.1

    - Output Data Type to double

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation >

PM sub-folder. Drag and drop QPSK Modulator Baseband module into the model window.

  - Go to Communications Blockset -> Modulation -> Digital Baseband Modulation >

PM sub-folder. Drag and drop QPSK Demodulator Baseband module into the model window.

  - Now, you will need almost the same modules as in Part 1. Repeat the steps in Part 1 for this case.

  - After running the simulation, observe and save all plots and values in BER display.

- M-PSK

  - Start a Simulink session. Select File -> New -> Model in the Simulink Library Browser to construct a new model.

  - Go to Communications Blockset -> Comm Sources -> Random Data Sources sub-folder. Drag and drop Random Integer Generator module into the model window. Double-click on this module and make the following settings:

    - M−ary number to 2

    - Initial seed to 37

    - Sample time to 0.1

    - Output Data Type to double

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation ->

PM sub-folder. Drag and drop M-PSK Modulator Baseband module into the model window. Double-click on this module and make the following settings:

  - M−ary number to 2

  - Input type to Integer

  - Constellation ordering to Binary

  - Phase offset (rad) to pi/8

  - Output Data Type to double

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation >

PM sub-folder. Drag and drop M-PSK Demodulator Baseband module into the model window. Double-click on this module and make the following settings:

  - M−ary number to 2

  - Output type to Integer

  - Constellation ordering to Binary

  - Phase offset (rad) to pi/8

  - Output Data Type to double

- Now, you will need almost the same modules as in Part 1. Repeat the steps in Part 1 for this case.

- After running the simulation, observe and save all plots and values in BER display.

- Model and simulate again changing M-ary number of Random Integer Generator, M-PSK Modulator Baseband and M-PSK Demodulator Baseband modules to 4, 8, 16, 32 and 64.

PART 3: M-ary QAM

- Start a Simulink session. Select File -> New -> Model in the Simulink Library Browser to construct a new model.

- Go to Communications Blockset -> Comm Sources -> Random Data Sources sub-folder. Drag and drop Random Integer Generator module into the model window. Double-click on this module and make the following settings:

  - M−ary number to 2

  - Initial seed to 37

  - Sample time to 0.1

  - Output Data Type to double

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation > AM sub-folder. Drag and drop Rectangular QAM Modulator Baseband module into the model window. Double-click on this module and make the following settings:

- M−ary number to 2

- Input type to Integer

- Constellation ordering to Binary

- Normalization method to Peak Power

- Peak power (watts): to 1

- Phase offset (rad) to 0

- Output Data Type to double

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation > AM sub-folder. Drag and drop Rectangular QAM Demodulator Baseband module into the model window. Double-click on this module and make the following settings:

  - M−ary number to 2

  - Output type to Integer

  - Constellation ordering to Binary

  - Normalization method to Peak Power

  - Peak power (watts): to 1

  - Phase offset (rad) to 0

- Now, you will need almost the same modules as in Part 1. Repeat the steps in Part 1 for this case.

- After running the simulation, observe and save all plots and values in BER display.

- Model and simulate again changing M-ary number of Random Integer Generator, Rectangular QAM Modulator Baseband and Rectangular QAM Demodulator Baseband modules to 4, 8, 16, 32 and 64.

Lab work № 6

GENERATION AND ANALYSIS OF FSK WAVEFORM AND ITS

TRANSMISSION USING SIMULINK

- OBJECTIVE

  - To generate FSK waveform using MATLAB and understand the basic principles of digital modulation techniques: FSK through modeling and simulations.

  - To download MATLAB generated waveform data of FSK into Agilent E4438C vector signal generator.

  - To observe the waveform of FSK at Agilent DSO6034A oscilloscope and Agilent E4407B Spectrum Analyzer.

  - To gain experience in using Simulink.

- LAB WORK

- Generation of FSK waveforms using MATLAB and Agilent Measurement Devices

  - Copy the experiment file into the directory of your name.

  - Run the file then follow the process that is stated at the MATLAB code. You should enter the bit stream such as [1 0 0 1 0 0 1 1 0 1] and then run the code for different f0 and f1 frequency values for about 3 times using the same bit stream.

  - Follow the same steps that are stated above and make this experiment two more times by changing the bit stream.

NOTE: Do not forget to change the names of the MATLAB generated waveform data

which exists at the line 70 of the MATLAB code.

- After you generate these waveforms, observe the MATLAB figure plots, and confirm that they are the theoretically expected waveforms, by writing in your experiment report the mathematical expression for each one.

- This programme also generates the waveform data to be downloaded into Agilent E4438C vector signal generator. Presently the directory (in your PC) for this is set as ‘C:\Program Files\MATLAB\R2008a\work\fsk’ (Line 70 of MATLAB code).

  - In internet explorer, set the IP address to 10.10.96.185, this will bring you the Agilent E4438C vector signal generator menu. Press Signal Generator FTP Access and choose the directory of BBG1/WAVEFORM to download the MATLAB generated waveform data (via copy and paste operation from the mouse clicks) into Agilent E4438C vector signal generator. Ensure that CH1 and CH2 of the Agilent DSO6034A oscilloscope are connected to I and Q outputs located at the rear of the signal generator. On the vector signal generator, press Mode Setup under the MENUS. Then press Dual ARB soft key on the right menu. After that, press Select Waveform using the soft key on the right of the screen. Using the arrow keys near the numeric keypad select the MATLAB generated waveform and press Select Waveform soft key. From the right menu, control whether ARB selection is set to On. If ARB selection is set to Off position press the corresponding soft key to set ARB selection to On, After performing these steps, either remotely from the vector signal generator menu on the PC screen or locally from the front panel of the vector signal generator, the replica of the waveforms observed as MATLAB figures in the form of bit stream bit and the FSK modulated signal fsk, should appear on the oscilloscope screen on CH1 and

CH2.

Note: During the successive downloads, remember to delete the existing file of the same name in this directory, otherwise FTP connection to the signal generator will be blocked.

Note: During the successive downloads, remember to set ARB selection to Off, otherwise FTP connection to the signal generator will be blocked and the noise_waveform will not be updated.

  - Connect the fsk output (note that bit signal corresponds to I and fsk output corresponds to Q of the vector signal generator) to Agilent E4407B Spectrum Analyser and obtain the spectrum of the generated waveforms then show them at your experiment report.

- Include in your experiment report, the general comments and a description of what you have learned from this experiment.

MATLAB CODE

%%%%% FSK modulation %%%%%

%%

- Follow the process below;

- 1. Write 'fsk' to the command screen of MATLAB

- 2. Enter the bit stream such as [1 1 1 0 0 0 1 0 1 0]

- 3. Enter the first frequency value as an integer value; such as 1,2 etc.

- 4. Enter the second frequency value as an integer value; such as 1,2 etc.

%%

function fsk %initializations clear all;

clc;

g = input('Please enter the bit stream:');

- f0 and f1 must be integers

f0 = input('Please enter the first frequency value:');

f1 = input('Please enter the second frequency value:');

val0=ceil(f0)-f0; val1=ceil(f1)-f1;

if val0 ~=0 || val1 ~=0; error('Frequency must be an integer');

end

if f0<1 || f1<1;

error('Frequency must be bigger than 1'); end

t=0:2*pi/99:2*pi;

cp=[];sp=[];

modd=[];modd1=[];bit=[];

for n=1:length(g); if g(n)==0;

die=ones(1,100);

c=sin(f0*t);

se=zeros(1,100);

else g(n)==1; die=ones(1,100); c=sin(f1*t); se=ones(1,100);

end

cp=[cp die]; modd=[modd c]; bit=[bit se];

end

fsk=cp.*(modd); %the fsk modulation multiplexing

bit = bit / max(abs(bit)); fsk = fsk / max(abs(fsk)); % making the amplitude smaller

- Plotting the graphs subplot(2,1,1);plot(bit,'LineWidth',1.5);grid on; title('Binary Signal');

axis([0 100*length(g) -2.5 2.5]); subplot(2,1,2);plot(fsk,'LineWidth',1.5);grid on; title('FSK modulation');

axis([0 100*length(g) -2.5 2.5]);

- Constructing the waveform to be loaded into ESG 4438C signal generator waveform(1:2:2*length(fsk)) = bit;

waveform(2:2:2*length(fsk)) = fsk;

waveform = waveform / max(abs(waveform)); %maximizing the waveform waveform = round(32767*waveform);

waveform = uint16(mod(65536 + waveform,65536)); if strcmp( computer, 'PCWIN' )

waveform = bitor(bitshift(waveform,-8),bitshift(waveform,8)); end

[FID, message] = fopen('C:\Program Files\MATLAB\R2009a\work\fsk','w'); % Open a file to write

%data

if FID == -1 error('Cannot Open File'); end

fwrite(FID,waveform,'unsigned short'); % write to the file

fclose(FID);

2.2. Analysing M-ary FSK transmission using Simulink

- Start a Simulink session.

- Select File -> New -> Model in the Simulink Library Browser to construct a new model.

- Go to Communications Blockset -> Comm Sources -> Random Data Sources sub-folder. Drag and drop Random Integer Generator module into the model window. Double-click on this module and make the following settings:

  - M−ary number to 2

  - Initial seed to 37

  - Sample time to 0.1

  - Output Data Type to double

4. Go to Communications Blockset -> Modulation -> Digital Baseband Modulation ->

FM sub-folder. Drag and drop M-FSK Modulator Baseband module into the model window. Double-click on this module and make the following settings:

- M−ary number to 2

- Input type to Integer

- Symbol set ordering to Binary

- Frequency separation (Hz) to 6

- Phase continuity to Continuous

- Samples per symbol to 100

- Output Data Type to double

- Go to Communications Blockset -> Channels sub-folder. Drag and drop AWGN Channel module into the model window. Double-click on this module and make the following settings:

  - Initial seed to 37

  - Mode to Signal-to-noise ratio (Eb/No)

  - Eb/No (dB) to 10

  - Number of bits per symbol to 4

  - Input signal power (watts) to 1

  - Symbol period(s) to 0.1

- Go to Communications Blockset -> Modulation -> Digital Baseband Modulation ->

FM sub-folder. Drag and drop M-FSK Demodulator Baseband module into the workspace window. Double-click on this module and make the following settings:

  - M−ary number to 2

  - Output type to Integer

  - Symbol set ordering to Binary

  - Frequency separation (Hz) to 6

  - Samples per symbol to 100

  - Output Data Type to double

- Go to Communications Blockset -> Comm Sinks sub-folder. Drag and drop Error Rate Calculation module into the model window.

- Go to Simulink -> Sinks sub-folder. Drag and drop the Display module into the model window. Drag the bottom edge of this inserted (Display module) icon to make the display big enough for three entries.

- Go to Communications Blockset -> Comm Sinks sub-folder. Drag and drop two Discrete-Time Scatter Plot Scope modules into the model window. Double-click on this

module and make the following settings:

Plotting Properties

- Samples per symbol to 100

- Offset (samples) to 1

- Points displayed to 400

- New points per display to 100

Axes Properties

  - X-axis minimum to -1.25

  - X-axis maximum to 1.25

  - Y-axis minimum to -1.25

  - Y-axis maximum to 1.25

- Go to Simulink -> Math Operations sub-folder. Drag and drop two Complex to Real−Imag modules into the model window.

- Go to Simulink -> Sinks sub-folder. Drag and drop two XY Graph modules into the model window.

- Go to Simulink -> Sinks sub-folder. Drag and drop two Scope modules into the model window.

- Go to Simulink Extras -> Additional Sinks sub-folder. Drag and drop two Power Spectral Density modules into the model window.

- Connect all the inserted modules as shown in Figure 1.

Figure 1: Simulink model for 2-FSK Transmission

- Set the simulation parameters (Simulation -> Configuration Parameters) as follows:

  - Start time to 0.0

  - Stop time to 10.0

  - Type to Variable-step

  - Solver to discrete (no continuous states)

  - Max. step size to auto

- Run (Simulation -> Start) the simulation and observe and save all plots and values in BER display.

- Model and simulate again changing M-ary number of Random Integer Generator, M-FSK Modulator Baseband and M-FSK Demodulator Baseband modules to 4, 8, 16, 32 and 64.

Lab work № 7

FOURIER ANALYSIS USING MATLAB.

Objective: This practical objective is to learn how MATLAB works and how it can be applied to the analysis of different signals in frequency using m-files to perform Fourier transforms and series.

- Introduction to Matlab.

MATLAB® is a special purpose computer program optimized to perform engineering and scientific calculations. It started life as a program designed to perform matrix mathematics, but over the year it has grown into a flexible computing system capable of solving essentially any technical problem.

The MATLAB program implements the MATLAB language and provides a very extensive library of predefined functions to make technical programming tasks easier and more efficient.

  - The command window

When MATLAB starts it opens a window called the command window, an user can enter iterative commands at the command prompt (>>) in the command window and they will be executed on the spot.

If the statement to be typed is too long, it can be typed in several lines if ellipsis (…) are used at the end of each line

Examples:

a) To calculate the are of a circle or radius 2.5 we type the following:

>> area = pi * 2.5^2

area  =

19.6350

b) Long statements: the following commands do the same

Note that when an statement is finished in “;” the result for the calculation will be suppressed. Also note that when the previous command is introduced into the prompt, a new variable called “ans” will be created. This variable can be manipulated just like any other in the program.

  - The Edit Window and M-files

Instead of typing commands directly in the command window, a series of commands may be placed into a file, and the entire file may be executed by typing its name in the command window. Such files are called script files. Script files (and functions) are also known as M-files, because they have a file extension of “.m”

An Edit Window is used to create new M-files or to modify existing ones. An M-file is created with the “File/New/M-file” menu selection and opened with the “File/Open” menu selection from the command window. In each case, the M-file is opened in an Edit Window, which is essentially a programming text editor, with the MATLAB language features highlighted in different colours. Comments in an m-file (which are preceded by the “%” sign) appear in green, variables and numbers appear in black, characters and strings appear in red, and language keywords appear in blue.

  - Important Commands.

The contents of the command window can be cleared at anytime using the clc command. The variables in the workspace are cleared using the clear command. Finally the contents of the graph windows (when MATLAB is used to plot graphs) can be cleared with the clf command.

Help can be obtained typing help in the command window.  The keyword help followed by a function name will provide help in the syntax and purpose of the function.

Exercise 1

Write an m-file that plots the graph of the function for the range with an interval between points of 0.1

Solution:

Writing this program in an m-file, saving it and then typing the file name in the prompt, will plot the results of the aforementioned function. It is important to notice that MATLAB is a matrix oriented program, therefore all the mathematical operations will be done as matrix operations. For example, when constructing a function the values of the argument of the function along with the function evaluation in the argument need to be represented by independent vectors. The vector x are the arguments of the function and the expression [0:0.1:10] means that the vector will be made of all the numbers between 0 and 10 with an interval of 0.1. i.e. 0, 0.1, 0.2 etc. Type the previous commands directly on the prompt without the semicolons.

Exercise 2

Use Matlab Help (or the command lookfor) to learn how to calculate base 10 logarithms.

Exercise 3

Solve, using Matlab, the tutorial questions 1. Compare your results with those obtained in the tutorial session.

- The Fourier Transform using MATLAB

To calculate the Fourier transform of a given signal, MATLAB uses the command fft. When Help FFT is typed in the matlab prompt, the following text appears

>> help fft

FFT Discrete Fourier transform.

FFT(X) is the discrete Fourier transform (DFT) of vector X.  For

matrices, the FFT operation is applied to each column. For N-D

arrays, the FFT operation operates on the first non-singleton

dimension.

FFT(X,N) is the N-point FFT, padded with zeros if X has less

than N points and truncated if it has more.

FFT(X,[],DIM) or FFT(X,N,DIM) applies the FFT operation across the

dimension DIM.

For length N input vector x, the DFT is a length N vector X,

with elements

N

X(k) =       sum  x(n)*exp(-j*2*pi*(k-1)*(n-1)/N), 1 <= k <= N.

n=1

The inverse DFT (computed by IFFT) is given by

N

x(n) = (1/N) sum  X(k)*exp( j*2*pi*(k-1)*(n-1)/N), 1 <= n <= N.

k=1

When calculating a FFT, we need to know certain parameter related to the intended signal:

Sampling time (ts) and sampling frequency (fs). The relationship between them is:

And the selection of 1, defines the other. The selection of ts/fs has to follow the sampling theorem. The idea leading to the sampling theorem is very simple and quite intuitive: the sampling interval for slow varying signals (low frequency) can be very high on the other hand, if the signal changes very fast in time (higher frequency), the sampling interval needs to be reduced. How big or small the sampling interval is, will depend on how high or low the relative frequency of the signal is. If the signal is limited in bandwidth (BW = W), then the upper limit for the sampling interval will be:

In practice, what we really need is the approximate period of the signal and then the sampling frequency needs to be at least twice that period.

Example 1: Lets find the FFT of the sinusoidal signal x(t) with a frequency of 0.5 Hz. and plot its power spectrum along with the signal in the interval t=[-5,5]:

First task is to find out the sampling frequency. For that we know that the period T of the signal is 2 sec., the minimum sampling frequency will be 4. The MATLAB script should start then with the definitions of sampling frequency and time:

fs=4;

ts=1/fs;

The next step is to construct the time vector. The interval is from -5 to 5, and the sampling interval will be the sampling time ts:

t=(-5:ts:5);

Now we need to define our signal x(t), for that we need to create a vector x, this vector values will be the function evaluated in the vector time. In order to perform the operation a for cycle can be used, and we will need an index for the vector x. The script for that is as follows:

x=zeros(size(t));     % this creates a vector x full of zeros with the size of the]

% time vector.

i=0;		        % index for the vector x

for z=-5:ts:5		            % z represents the time argument in the sine function

i=i+1;		            % increment the vector index

x(i)=sin(2*pi*0.5*z);	% evaluates the function

end

When the Fourier transform is to be performed, a parameter n needs to be calculated, this parameter will be where y is the next power of 2 that is greater than the size of the function x vector. n will be the number of points of the fft, and it needs to be a power of 2 in order to save calculation time.

n=2^nextpow2(length(x));

To understand this statement, check in the MATLAB workspace window, the size of the vectors n, t, x and X.

Now the Fourier transform is calculated and the coefficients that are in the vector need to be scaled to by 1/fs in order to obtain the real Fourier transform:

X=fft(x,n);		% Calculates FFT

X1=X/fs;		% Scales the vector X by 1/fs obtaining the Fourier Transform

Then in order to plot this, we need to construct the frequency vector, this vector goes from –fs/2 to fs/2 with interval fs/n. However, because it is a vector, constructing it is not a simple task because of the index (if done in this way, we will obtain a frequency vector with one extra element)

So the frequency vector construction needs to go from 0 to fs at an interval fs*(1-fs/n). This will give a vector of the right size that needs only then to be transported to –fs/2. The Instruction is as follows:

f=[0:fs/n:(fs-fs/n)]-fs/2;

Now we have the Fourier transform vector in X1 and the frequency vector in f, the time vector is t and the time function is x. To plot we will use the multi-plot command

Subplot(x,y,z). Here x represents the number of rows in the window, y is the columns (we want 3 plots vertically so x=3 and y=1). z is the  current plot to be done.

First we draw the time function sampled.

subplot(3,1,1);

plot(t,x);

Remember that the Fourier transform is a complex function, so we can plot its module of its phase, for now we will concentrate only on the module (abs), to plot a fft transform we need to re-define the function to be plotted with the function fftshift(x). This function simply moves the Fourier transform so that the 0 frequency components are located in the centre of the spectrum.

subplot(3,1,2);

plot(f,fftshift(abs(X1)));	% note that the x axis if the vector f no t.

Finally we will plot what is known as the Power Spectrum of the signal, from the power spectrum is from where the bandwidth of the signal can be obtained graphically. It is defined as:

We must remember that X(f) is a complex function, so to obtain the square of the module we will need to multiply it by its conjugate. In MATLAB the multiplication of such vectors needs to be done element to element using the “.*” operator.

subplot(3,1,3);

plot(f,fftshift(abs(X1.*conj(X1))));

Exercises:

1) In the previous example, compare the results obtained when the sampling frequency is reduced to 2 and increased to 10

2) Type the MATLAB script to calculate the cosine function with a frequency of 3 Hz in the same interval.

3) Obtain the FFT, and Power spectrum of the signal x(t) with T0= 1m sec.

4) Obtain the FFT and Power spectrum of a square pulse (amplitude 1, T=2) between t=[5-,5].

5) Obtain the FFT and power spectrum of the triangular pulse of the tutorial 1: (use the same interval [-5,5].

t

1

-1

1

- t

The list of the recommended literature

- Величко В.В. Передача данных в сетях мобильной связи третьего поколения.-М.,2005

- Радиосистемы передачи информации./под ред.В.А.Васин и др.-М.,2005

- Широкополосные беспроводные сети передачи информации./под ред. В.М.Вишневский и др. – М.,2005

- MATLAB Simulink. И.Ф. Цисарь. М.: Солон-Пресс, 2008.-256с. .

- Моделирование электротехнических устройств в MATLAB, SimPowerSystems и Simulink. И. Черных. М.: ИД Питер, 2007, 288 с. ISBN 978-5-388-00020-0.

- Simulink 4. Секреты мастерства. Дж. Дэбни, Т. Харман (пер. М. Симонова)М.: Бином. Лаборатория знаний, 2003,-404 стр. ISBN: 5-94774-048-6, 0-13-1-017085-2.

Aslanov Muradzhan Tokhtasinovich.

Ismailov Serik Umirbaevich.

Methodical instructions to the laboratory work

on discipline

Wireless communication technologies

for students of specialty 5B071900 – Radio engineering,

electronics and telecommunications

Send for the press

Paper format XxY  1/16

Paper typographical. Offset printing. Volume 5 п.л.

Circulation….instance Order № …*

©  Issuing of the M.Auezov South Kazakhstan state university

Publishing center of M.Auezov SKSU, Shymkent,

Tauke Khan street, 5