Ф.7.03-18

M. M. YESKENDIROVA

A.A. KADIRBAYEVA

CHEMISTRY AND TECHNOLOGY OF REACTIVE ACIDS AND SALTS

Shymkent, 2017

Ministry of Education and Science of the Republic of Kazakhstan

M. Auezov South Kazakhstan State University

YESKENDIROVA M. M.

KADIRBAYEVA A.A.

CHEMISTRY AND TECHNOLOGY OF REACTIVE ACIDS AND SALTS

TEXTBOOK

for students of speciality 6M072000 – “Chemical Technology of Inorganic Substances”

Shymkent, 2017

UDK 658.567.1 : 574 (075.8)

BBK 28.08.

Yeskendirova M.M., Kadirbayeva A.A. Chemistry and technology of reactive acids and salts/Textbook. – Shymkent, M. Auezov SKSU, 2017. – 105 p.

The textbook is intended for higher education students of speciality 6M072000 – “Chemical technology of inorganic substances” studying in English. The textbook can be used for preparation to lectures and laboratory classes, as well as for independent work, course and degree student works.

Reviewers:

Zhakipbekova N.O. – a doctor of technical sciences, a professor of the Department “Chemistry” of M. Auezov SKSU;

Korganbayev B.K. – a doctor of technical sciences, a professor of IHTU;

Ashirov A.M. – a doctor of technical sciences, a professor of SKPI

The textbook has been recommended for print by Methodical Council of M. Auezov South Kazakhstan State University, protocol # ___ of ________ 2017 y.

© M. Auezov South-Kazakhstan State University

Contents

| Introduction | 6 |
| --- | --- |
| I chapter. Modern condition and problems of high-purity substance production | 9 |
| 1.1Classification of especially pure substances | 13 |
| II chapter. Influence of external pollutants on processes of deep purification of substances | 17 |
| 2.1 High-pure substance production methods | 20 |
| 2.2Chemical methods of deep purification of substances | 23 |
| 2.3Distillation methods | 31 |
| 2.4Crystallization methods | 39 |
| 3 chapter | 44 |
| 3.1Manufacture of orthophosphoric acid of “p” and “p.f.a” | 44 |
| 3.2Manufacture of chemically pure orthophosphoric acid | 55 |
| 3.3Manufacture of meta- and pyrophosphoric acids | 59 |
| 3.4Manufacturing phosphorous acid | 62 |
| 3.5Manufacture of high-pure phosphoric acid | 66 |
| 4 chapter 4.1 Manufacture of monosubstituted sodium phosphate (sodium dihydrophosphate) of “p” and “p.f.a.” grades | 70 |
| 4.2Manufacture of sodium salts of orthophosphoric acid | 79 |
| 4.3Manufacture of reactive-purity sodium pyrophosphate | 91 |
| 4.4potassium orthophosphates properties and application ares | 101 |
| References | 103 |

Introduction

The industry of chemical reagents and especially pure substances is an independent branch of chemical industry which has a substantial effect on industrial efficiency and development of technical progress. One of the problems facing to the chemical industry is a necessity to increase output of low-tonnage production including fuller satisfaction of needs for especially pure chemical materials and reagents. The great attention is given to manufacturing high-purity materials and substances – ferroelectric materials, especially pure substances, monocrystals, luminophors, etc., which define the progress of electronic, electrotechnical, radio engineering and other branches. Importance of the problem of fuller satisfaction of constantly growing needs of various industries with high-purity substances and chemical reagents with desired properties is emphasized by that fact, that research works in the chemical reagent branch are carried out in accordance with the programs connected with development of production processes and highly sensitive analysis techniques of especially pure substances; with development of wide assortment of low-tonnage chemical reactants with a set chemical composition and a physical structure; with creation of new and improvement of operating manufactures of industrial reactants.

Appearance of the chemical reagent problem has been connected with the necessity to have analytical compounds of necessary quality that caused rates of development, assortment and volumes of manufacturing the reagents. Growth of output of fine organic synthesis products has been also connected with a necessity of manufacture of pharmaceutical preparations and biological products on their basis. In the seventies the manufacture of certain groups of chemical reactants has been intensively developed, among them the reagents for organic and inorganic analysis, medical diagnostics, and instrumental analysis methods and also for use in microelectronics, optical glassmaking, etc.

A specific feature of the present period is manufacture of chemical reagents with desired quality for narrow application fields. Recently chemical reagents and especially pure substances are produced in large scales due to following circumstances: wide use of chemical reactants in modern physical and physicochemical research techniques for production quality control; application of chemical reactants as reference substances at research works, including development of modelling processes on laboratory and pilot installations; use of raw materials of reactive qualification as technological raw materials in connection with increase of requirements to quality of production and environment protection.

According to a trade catalog of “Soyuzreaktiv” the assortment of chemical reagents and especially pure substances in the CIS numbers about 12000 names including:

| Inorganic reagents Organic reagents Biochemical reagents Especially pure substances |  | - 2250 - 8500 - 800 - 300 |
| --- | --- | --- |
|  | including: indicators chemical reagents “pure for analysis” organic reagents for inorganic analysis reagents for chromatography complexons liquid crystals special purpose high-purity substances | - 145 - 900 - 200 - 200 - 90 - 200 - 200 |

Comparison of the assortment of especially pure substances produced in the CIS with the respective assortment of leading foreign companies shows that it is on a production level of some the companies. However, the ranges of the output products considerably differ. Basically the reagent quality corresponds to quality of the foreign products used for analytical purposes.

The further direction of research works in the field of chemical reactants and especially pure substances are development of a broadened line of products including especially pure substances, single crystals, ferrite powders, ferrielectric and piezoelectric materials, luminophors, special-purpose biological preparations; creation of new high-performance technological processes and installations for manufacturing these products; development of automated high sensitive inspection methods of the production quality; definition of prospects of the production development and improvement of its quality.

Now especially pure substances are a base for manufacturing a number of the most important products and materials of modern technics. These compounds owing to a strictly set and a stable structure are applied for development of compact and effective computers, which basis is the devices based on the thinnest films, obtained by an integrated microelectronics method, fiber-optic communication lines, power semi-conductor devices, quantum amplifiers, microwave and light generators, superconducting materials, etc.

Dependence of nuclear and electrophysical characteristics of substances on nature of impurities has caused purification ways from certain impurities to their rather low content (10-6-10-7 %). Today the problem of obtaining absolutely pure substances without an impurity background and their analysis is a hard-to-attain task as theoretical aspects of its decision and practical possibilities have been still insufficiently developed. The content of foreign atoms in the purest substances is actually reduced only on 2-6 orders. At present time a characteristic of an especially pure substance is defined by content of limited impurities. Continuous growth of requirements to improvement of purity of substances defines also growth of theoretical study in the purification field. At the choice of purification ways the preference is given to the methods limiting introduction of additional pollutants, in particular to development of combined cleaning processes with minimal addition of reagents for separation of components of a system. Obtaining the substances containing 10-17-10-18 % of impurity is theoretically possible by means of such multistage processes as molecular distillation, thermal diffusion, zone melting, sorption etc. However it is impracticably because of presence in solutions of poorly soluble substances as submicronic particles of various dispersion degrees. Presence of such the particles in the especially pure solvents, used in microelectronics at manufacturing superhigh-speed integrated circuits, can cause physical defects and failure of the circuits, and also to affect their electric characteristics. Therefore the leading companies include into their trade catalogs a new class of especially pure substances with content of suspended mechanical particles on 1,5-2 order lower, than in other compounds also applied in microelectronics. Improvement or development of essentially new purification methods and increase in sensitivity of analysis techniques were the consequences of new requirements to purity. It has allowed obtaining substances with content of limited impurities at level of 10-6-10-7 %.

With the view of increase in technical level of production the special attention at the development of technological processes is given to creation of flexible automated manufactures on the basis of wide automation of technological processes, in particular with application of generic equipment modules and robotics complexes.

Creation of block-module installations for technological processes is based on the application of unified units meant for carrying out certain technological operations. It allows making non-standard installations from standard blocks-modules, which design should completely meet to requirements and features of a specific production technology. Thus, it is possible to remove a contradiction between a tendency to apply original apparatuses, which take into consideration specificity of processes, peculiar to chemists-technologists and desire of mechanical engineers to standardization and unification. The most responsible operation of development of such apparatuses is selection of unified modules caused by technological features of a process with consideration not only design peculiarities of the modules, but also flow directions. A set of the modules forming both an apparatus structure (a mixer, a reactor, etc.) and a contact element structure (a packing, overflows, etc.) of various standard sizes has been suggested.  In this case it is necessary to investigate a possibility of combination of several technological operations in one apparatus for the purpose of optimization of a process and simplification of its technological circuit. Such module installations and lines give a chance to use them for manufacturing groups of the substances close on physicochemical properties and technologies without considerable reorganization. A block set should operate at independent combination of regime (continuous or periodic) and control (manual or automatic) blocks.

I Chapter. Modern condition and problems of high-purity substance production

High-purity substances are the substances in which impurity content is essentially lower than in the substances with which we deal in daily practice. A quantitative characteristic of a concept “a high-purity substance” is mobile in time. Now a substance is considered as a high-purity one if total impurity content in it does not exceed 10-4 atom % and concentration of individual impurities is in limits of 10-6-10-7 atom % and lower. Impurity content in usual chemical reagents is sufficiently higher. As a rule, concentration of individual impurities and their total content in reactants is higher 10-2-10-4 atom % and 10-1-10-2 atom %, respectively. The volume of high-purity substance output is less in many times than the production volume of chemical reagents. It is necessary to speak more strictly about a high-purity state of a substance, as a chemical composition along with temperature and pressure is one of key parameters of a state of a matter. A range of theoretical and practical problems connected with obtaining, studying of properties, storage, practical application of the substances with minimum impurity content that is what makes a problem of obtaining high-clean substances.

Dependence of properties of substances on their composition was known for a long time and first of all consisted in the interconnection of quality of consumed products and materials with presence in them of some impurities (vinegar in wine, tungsten in a tin ore). Therefore production processes were often supplemented by simplest purification operations (distillation, filtration) for removal of “harmful” impurity. Development of natural sciences allowed finding quantitative laws of influence of impurity on some, first of all, physicochemical properties of substances. These are the Henry's law (1803), the Raoult's law (1888), Vant Hoff equations, which connect an osmotic pressure value and fall of a crystallization point of an ideal solution containing impurities (1885-1890). It has been found out, that influence of impurity on physical and chemical properties of substances is in proportion to a mole impurity ratio and is retraced to a concentration level of 10-2-10-3 mole %. By 40 years of the ХХ century in connection with extensive researches of physicochemical properties of matters a number of substances with concentration of certain impurities of 10-2-10-3 mole % at their total content on a level of the tenth shares of a percent have been obtained. Proper purification methods have been also developed.

A qualitative new period in obtaining pure substances is connected with the intensive development of nuclear physics and solid state physics in 1940-1960 and creation of new technics on the basis of achievements of these sciences. It was found that the interaction of nuclear radiations with individual substances and behaviour of charge carriers in solid bodies are more sensitive to impurity content, than the intermolecular interaction, which define physicochemical properties of substances. The researches carried out at that time and also modern data have shown that dependence of nuclear, electro physical, optical properties of a matter on impurity content in it is retraced to a concentration level of 10-6-10-8 atom %, and in some cases even lower. As it has turned out that some impurities have an influence on properties of substances to a considerably higher extent, than others. These impurities have been named as limited ones as their content defined suitability of substances for manufacturing of those or other equipment and devices. For semi-conductor electronics and nuclear technology it is necessary to have a number of semiconductors, metals, reactants with content of limited impurity about 10-4-10-6 atom % and lower. Such substances, which have been named “especially pure”, have been obtained in the fifties. Total impurity content in them could reach 10-2 atom % at the expense of sufficiently high concentration of unlimited impurities. Development of methods of manufacturing especially pure substances and creation of their industrial production was a major accomplishment of chemical and metallurgical science and industry. The resolution of this problem has in many respects defined success in development of the advanced branches of technics and industry applied especially pure substances.

In the late seventies the requirements to purity of matters have been supplemented with a new aspect: substances should have high purity degree concerning to all impurities, and not only limited ones. This requirement was issued both from researchers, and from industrial experts. Scientists strive for reveal true properties of substances, unmasked by presence of impurity, including those about which developers of a pure substance have not presented any information. Industrial engineers wish to have the materials providing high yield of high-quality apparatuses stable against time degradation. Both the first and the second can be connected with presence of uncontrolled impurities. Thus, at the given stage the matter is obtaining high-purity substances with minimal total impurity content.

Obtaining high-purity substances is an independent scientific and technical problem with own studied object, research methods, theoretical and methodological laws. Purposes of scientific researches in this field can be shortly formulated as a statement, that increase of a purity degree leads to detection of new properties of substances or brighter appearance of known properties, i.e. to increase of our knowledge about substances as a form of existence of matter. Last years the validity of such approach has been confirmed by obtaining optical fibers with insignificant optical losses for long-haul optical communications lines on a base of high-purity silicon dioxide. A problem of applied investigations and industrial production of high-purity substances is development of production technologies of concrete materials and supply of user industries with them. The practical importance of high-purity substances is a common feeling. High-purity substances are an important and integral component of modern scientific and technical progress.

Modern requirements to content of impurities in high-clean substances are defined by a concrete area of their application. Now the number of widely used high-clean substances and especially pure materials on their basis exceeds 2000. These are semi-conductors, materials for microelectronics, electronic engineering and nuclear technics, optical materials, chemical and biochemical reagents and others. There is demand for high-purity materials concerning practically all classes of chemical compounds and elementary substances: elements, oxides, acids, bases, gallides, hydrides, chalcogenides, phosphides, arsenides, antimonides, metalloorganic and elementoorganic compounds, hydrocarbons and their derivatives.

In each of these categories of materials it is possible to single out groups with narrower application. For example, optical materials are subdivided into materials for fiber optics, laser materials, and products for optical mediums. Each the group contains the materials, which have a leading position, for example, silicon, germanium, gallium arsenide in the semi-conductor technics and microelectronics, quartz glass in fiber optics. The general number of such materials makes some tens.

In most cases the existing requirements to impurity content in high-clean materials are defined experimentally. It is possible to note some levels of a substantiation of the requirements to high-purity materials. The first of them is a qualitative substantiation, which consists in identification of an impurity or a group of impurities, which need be removed from a material. The following level is indication of the impurity concentration starting from which the phenomenon taken as a principle of operation of a device is found out. Sometimes instead of impurity content it is convenient to specify a numerical value of any property of a purified substance starting from which the substance is suitable for manufacturing a device with minimum requirements to its parametres and acceptable output of marketable production. It is possible to see an example of such approach in the works devoted to purification of the organic substances used as dielectrics with high specific electric resistance of liquid crystals. Higher levels of the requirements substantiation define influence of certain limited impurities on properties of a high-purity material, and also technical parametres and output of suitable devices made from it. Numerous researches should be performed for achievement of such the level; it is necessary to investigate both a high-clean substance itself, and a process of manufacturing of a device out of it. It can be done not for all groups of high-purity materials and areas of their practical application. In any measure it is “no man’s land” in studying properties of high-purity substances situated between their developers and consumers. Therefore existing requirements represent the qualitative and semiquantitative requirements often formulated with a certain “reserve” as regards purification degree of matters.

The elementary and complex semiconductors, applied in semi-conductor and electronic material science, should have the sum impurity content of 10-3-10-5 atom %. The top accessible concentration limit of the electro active impurities, which are limited for these materials, is usually 10-8 atom %.

Technology of semi-conductor devices and microelectronic products is a multistage technology; it provides for use of numerous chemical reactants, solvents, gases. A surface of the product made of a high-purity material after a contact with the above mentioned compounds can contain different impurities formed at the expense of adsorption and cementation, which are capable to penetrate into the material volume at following stages. For this reason concentration of limited impurities in auxiliary materials should be the same as in a basic semi-conductor material or lower. For example, for especially pure acids and alkalis contents of 20-30 foreign elements are limited, and the concentration of 60-80% from them is about 10-7 %.

The nuclear technology consumes a significant amount of high-clean materials. A set and concentration levels of impurities in them depends on a concrete purpose. For example, water content in the uranium dioxide used for making cores of fuel elements need not exceed 110-3 %.

Among various optical materials the most rigid requirements are shown to the materials for fiber optics. Content of coloring matters (compounds of iron, nickel, chrome and some other elements) in the silicon dioxide applied for manufacturing fiber light conductors with low optical losses should be lower than 1∙10-7-1∙10-8 atom %, content of OH-groups – lower than 1∙10-6 mol %. Considerable quantities of these impurities lead to appreciable increase of optical losses.

Requirements to purity of other materials used in fiber optics (chalcogenide and fluoride glasses, metal fluorides) are the same high especially to quantity of oxygen and hydrogen-containing compounds. Requirements to the high-purity materials applied in industry are developed both towards the assortment diversification and towards decrease of admissible content of micro impurities in traditional materials. There is demand for a number of new high-purity substances: metal oxides and fluorides, some organometallic compounds, organic substances (solvents, liquid crystals), multicomponent oxide and non-oxide glasses. Elementary and complex semiconductors (silicon, germanium, gallium arsenide and phosphide, indium arsenide and antimonide, compounds of a type AІІBVI and their solid solutions) are needed in a purer state than earlier. At the same time a range of controllable impurities broadens and their permissible concentration decreases.

Nuclear radiation detectors are produced of germanium, which contains 10-10-10-11 atom % of impurities. Purer silicon is necessary for power electronics, manufacture of high efficiency solar batteries. Purer complex semiconductors are needed for light sources and receivers including infra-red radiation.

Last years the requirements to high-purity matters pay special attention to homogeneity of materials; in many respects that is connected with a level of micro impurity content. The researches carried out in the seventies have led to the discovery of a new class of impurities – micron and submicronic suspended particles. These particles consist of the compounds insoluble in the given substance. At deep cleaning their behaviour can essentially differ from behaviour of the impurities, which are in a basic substance in dissolved condition /17, 18/. Content of the latter in liquid high-purity substances (if they are not specially purified from suspended particles) can reach 106/1ml of particles and more /19/. The further researches have shown, that the impurities possessing limited solubility in a basic substance can also be as suspended particles; and quantity of an impurity of the same substance, which is as a true solution and as disperse particles, depends on purification conditions (temperature, total impurity concentration) /20,21/.

Suspended particles are extremely undesirable impurities in semi-conductor and optical materials. Content of suspended particles in auxiliary reagents – gases, solvents, etching solutions – should be extremely low so that after their contact to a surface of a high-clean material concentration of micro heterogeneous inclusions on its surface did not increase. In materials for fiber optics foreign impurities can cause nonselective light absorption. Presence of suspended impurities negatively affects not only a value of optical losses of light conductors, but also on their mechanical durability.

It is necessary to provide the demanded impurity content in a final product instead of in an initial high-clean material, for example, in a working volume of a nuclear radiation detector, and not only in the initial germanium; in a fiber light conductor instead of in initial silicon oxide SiO2 or silicon chloride SiCl4 and oxygen. There are the situations when obtaining an initial material with low impurity content does not guarantee manufacturing a final product with specified technical characteristics. In such cases it is necessary to analyze all a technological process, and not only increase of requirements to purity of an initial material.

1.1Classification of especially pure substances

In the nature absolutely* pure substances do not exist. Appearance of an absolutely pure substance would cause at once initiation of a spontaneous and irreversible intensive process of dissolution of the environment components and impurities in this substance. Such the process is characterized by increase in entropy of a system substance-impurity and reduction of its Gibbs thermodynamic potential. Therefore all substances** can be practically considered as solutions (solid, liquid or gaseous) of impurities in a basic component though from the thermodynamic point of view all components of a solution are equivalent and their division into a solvent and dissolved substances has a conditional character. In this case it is possible to use categories of chemical thermodynamics for classification of all substances (as solutions) in accordance with their purity degree.

Still a common international classification of chemical preparations in accordance with their purity degree is not developed. The purest substances applied for chemical analysis and also for scientific investigations have been named as reagents.

__________________________________________________________

* An absolutely pure substance is such a chemically homogeneous system, which consists or of the atoms with the same atomic number and mass number, or “of identical molecules or of various molecules (for example, associated) provided that they make a general phase and are in uninhibited equilibrium with each other”.

** If only a homogeneous medium is considered as a substance, and unstable disperse systems (suspensions) are regarded as a mechanical mixture of substances.

Reagents have the following grades: “pure” (p), “pure for analysis” (p.f.a.), “chemically pure” (c.p.), which should be referred to real solutions and “especially pure” (es.p.) – to extremely diluted solutions. Sometimes the latter can be divided into some sorts.

The “pure”-grade reagents can be successfully applied in different researches both educational and industrial character.

The reagents “pure for analysis”, as the name shows, are intended for the analytical works which are carried out with pinpoint accuracy. Impurity content in these preparations is negligibly small and usually does not introduce appreciable errors in analysis results. These reactants can be well used in research works.

At last, the reagents “chemically pure” are intended for responsible scientific researches; they are used also in analytical laboratories as the substances for determination of titres of working solutions.

These three kinds cover all the general purpose reactants. A list of inadmissible impurities and a limiting value of their content are stipulated by technical specifications. A maximum value of impurity contents in two various reactants of the same category, for example “pure”, can be various and is defined by difficulty of purification of a reactant from this or that impurity.

Preparations of higher purification degree (“especially pure”) are intended only for the special purposes, when even million shares of a percent of an impurity are absolutely inadmissible. Basic consumers of such the reagents are the industry of semi-conductor materials, radio electronics, and quantum electronics. To use expensive especially pure substances for performance of ordinary analytical and scientific researches is absolutely inadmissible and is senselessly.

After the origination of high-purity substance manufacture in 1959 in the former USSR the State scientific and technical committee had been confirmed a new classification system of substances based on their purity degree. In accordance with this classification all pure substances have been divided into three classes: A, B, C. A total impurity content in a substance is expressed as a negative decimal logarithm of their atomic percent and written after the letter which designates the purity grade. The class A is divided into subclasses А1 (content of a base material is 99,9 %) and А2 (99,99 %), i.e. these are the substances with the sum impurity content from 10-1 to 10-2 %. The substances with the total impurity content from 10-3 to 10-6 % make a class B and are designated as В3, В4, В5 and В6. And, at last, the ultrapure substances with impurity content from 10-7 to 10-10 % make a class C, which has subclasses С7-С10. The specified designation system was intended first of all for characteristic of purity degree of semi-conductor substances.

For distinction of the subclasses of high-purity substances a marking system is used. Containers with certain reagents have definite color labels.

| A subclass | A label color | A base material content, % | An impurity content, % |
| --- | --- | --- | --- |
| А1 | Brown | 99,9 | 10-1 |
| А2 | Grey | 99,99 | 10-2 |
| В3 | Dark blue | 99,999 | 10-3 |
| В4 | Blue | 99,9999 | 10-4 |
| В5 | Dark green | 99,99999 | 10-5 |
| В6 | Light green | 99,999999 | 10-6 |
| С7 | Red | 99,9999999 | 10-7 |
| С8 | Pink | 99,99999999 | 10-8 |
| С9 | Orange | 99,999999999 | 10-9 |
| С10 | Light yellow | 99,9999999999 | 10-10 |

There are also other methods of high-clean substance classification. So, scientists of the research institute of chemical reagents and especially pure substances has suggested to characterize purity of a preparation according to the total content of a certain number of micro impurities. For example, for especially pure SiO2 ten impurities are normalized (Аl, B, Fe, Ca, Mg, Na, P, Ti, Sn, Pb) and their total content need not exceed 110-5 %. Such the reagent has a label “es.p. 10-5”. For packing of high-purity reagents it is impossible to apply glassware, which is a source of pollution. Therefore such the preparations are packed in polyethylene or teflon (fluoroplastic-4) jars.

Similar classification is available and in a number of foreign countries. In particular, in the German literature it is accepted to classify high-purity substances in accordance with micro impurity content by the following way: ppm (part per million) and ppb (part per billion), that corresponds to 110-4 and 110-7 atomic %. Such the designation of substance purity causes some confusion in exact estimation of above mentioned values: while the majority of the physicists dealing with solid substances understand ppm as only atomic ppm, chemists, traditionally – only weight ppm. Some English companies designate high-purity substances by a symbol N and their quality, by-turn, is marked by a numeric index. The designation of purity degree by a sign 5N8 corresponds to a base substance content of 99,9998 %; 2N5 – 99,5 %; 3N3 - 99,93 % etc. If an especially pure substance has been produced by a zone melting method the designation of purity degree contains a letter Z, for example, a symbol 6N(Z) means purity of a product of 99,9999 %.

Estimation of purity degree of substances in accordance with the total impurity content is more rational. In particular, in Germany all pure substances are designated by an index R-n, where R (Reinheitsgrad) means purity degree, n=-lgc+2, and c – the total impurity concentration in weight percent. Thus, for example, a purity index R-4 means, that a reagent contains 99,99 weight % of a base substance. An essential disadvantage of such classification of substances’ purity is absence of any information about the number of normalized impurities. Japanese industrial standards for especially pure substances provide for designations “Electro Grade” and “Extra Pure” without any detailed description of their quality.

A number of the countries widely apply such gradation of especially pure substances as “optical pure”, “luminescent purity”, etc. Similar purity gradations have appeared during the pre-war period when scientifically proved requirements to purity of substances have been absent.

Among the numerous spontaneously formed classifications of especially pure substances applied in the CIS by the least correct concept is a “spectral pure matter”. Practice of manufacturing various standard-grade substances shows that the characteristic of purity of a substance cannot be connected with one any analysis method, for example with a spectral technique. Purity of a substance is determined by means of a whole complex of various analysis methods (chemical, spectral, luminescent, polarographic, mass spectral, radioactivation and other ways). In so doing advantages of one or another analysis method (speed, accuracy, sensitivity, and simplicity) are applied for determination of a concrete micro impurity or a group of some micro impurities. It is necessary to mean a circumstance that analysis techniques are continuously improved in a direction of increase of their sensitivity: a spectral pure substance after a while will be considered as a “spectral polluted” one.

On the basis of the above-stated reasons scientists the research institute of chemical reagents and especially pure substances has developed a new classification system of high-pure materials based on the simplest combination of alphabetic and numeric indexes, which can give a general representation about quality of a material. The new system allows giving a certain grade to a special pure substance (a numeric index) depending on the number and content of normalized micro impurities.

For example, the first figure (15) of an index es.p.15-6 is determined by the general number of all normalized micro impurities. The second figure (6) characterizes purity of a substance, i.e. it shows, that the total micro impurity content can be from 110-6 to 410-6 %.

For the especially pure substances in which only organic impurities are controlled the grade is designated by letters “oi” (organic impurity) with a figure (through a dash), which corresponds to a negative decimal logarithm of the total organic impurity percentage, and an index “esp”. So, the grade of especially pure material, in which the total impurity content of organic substances makes 210-3 %, is designated “oi-3 esp”. If a substance is normalized according to content of impurities of inorganic and organic substances it is necessary to take into consideration the content of both inorganic and organic impurities. For example, the grade of especially pure substance in which the total organic impurity content is 410-3 %, and the total content of 8 limited impurities of inorganic substances makes 310-5 %, is designated “oi-3 esp 8-5”.

II CHAPTER. INFLUENCE OF EXTERNAL POLLUTANTS ON PROCESSES OF DEEP PURIFICATION OF SUBSTANCES

The great value was always given to influence of external pollutants on processes of deep purification of substances. Recently in connection with obtaining high-pure materials for semi-conductor technics this factor is a determining one at determination of limiting possibilities of one or another technology. Realization of deep cleaning processes in most cases directly depends on a correct choice of a material for manufacturing chemical equipment and degree of full isolation of a processed product from the air environment at all stages of a technological process.

The problem of obtaining especially pure substances with full right can be compared to a problem of high vacuum production. The more completeness of purification of a substance from impurity, the clearer various leakages, micropores and microcracks in equipment have an influence on penetration of impurities from the atmosphere. In a case of an absolutely chemical-resistant material of equipment a source of impurity is also various surface pollutants, static spaces and pockets in reactors and communications, additional reagents and accessories introduced in a process. At last, gradual corrosion of walls of apparatuses plays a role of the surface possessing high steam tension. Thus, when manufacturing high-pure substances a basic obstacle is a specific “wall problem”, which limits achievement of theoretical purification degrees of substances. Therefore a last stage of fine purification must include the processes with the minimum participation of foreign surfaces (a classical example is a crucibleless variant of zone recrystallization).

The most dangerous for high-pure inorganic substance technology are the atmospheric pollutants, which often are not taken into consideration at the development of cleaning processes. It is considered, that simple “hermetic sealing” of equipment (it should be meant the absence of any reliable control of hermetic sealing) serves as a good guarantee of full isolation of working spaces from the environment. Unfortunately, in so doing some extremely important circumstances are not always considered. The matter is that there is practically no a completely sealed technological process including transportation of intermediate products and raw materials, stages of filtering, drying, calcination, packing or batching etc. Besides it is practically impossible to realize absolute hermetic sealing because of presence of various flange connections, weld seams, gland devices, inspection windows, connecting pipes for input of reagents and control equipment sensors, metering devices, air or hydraulic shutters etc. Moving of liquids is often performed by means of vacuum or compressed air. In all these cases introduction of impurity from the environment is almost inevitable. At assemblage and preparation of equipment in usual conditions air with its pollutants gets in working spaces and communications, stays too long in static spaces of the equipment and strongly increases a “null” cycle of the process during which a sub-standard product is produced.

At last, even at reliable hermetic sealing of junctions of various parts of equipment diffusion of impurities from the environment through an apparatus wall is inevitable. In technology of especially pure inorganic substances polymeric materials are used in overwhelming majority of cases. It has turned out, that the products made out of polymeric materials (sheets, pipes) have minute pores and thin tubules. Besides the air pollutants can diffuse through the hollow spaces formed as a result of continuous oscillatory motion of separate elements of macromolecules. Polymers with a linear structure, not having polar groups (polyethylene, teflon, polyvinylchloride and others) are more flexible and for this reason they also are more permeable for gases, than high-molecular compounds with a spatial structure. It is necessary to notice, that gas diffusion rate sharply increases with rise of temperature, and especially while a polymer passes from a glassy state into an elastic one. Plastic materials are also subjected to microbiological corrosion. Vital functions of the microorganisms settled on polymeric materials can lead to thin punching of the equipment walls and destruction of the polymer. In some cases mould can grow out through the walls made of plastics or rubbers. Till now sensitivity of various polymeric materials to certain microorganism species has not been established.

Thus, solving many technological problems of manufacturing high-pure substances without knowledge of a nature of atmospheric contaminants and taking into account of their influence on purification degree of a concrete substance, without correct estimation of corrosion rate of the equipment material is impossible.

Now not a purification process guarantees against the ingress of micro impurities in reaction zones from the environment (a gaseous phase, an equipment material) and auxiliary reagents (ion exchangers, solvents, precipitants, separating agents). Any technological process of substances’ purification represents a combination of processes of cleaning and pollution. If these two processes take place with equal rate the micro impurity content in a substance does not change. Efficiency of a purification technological process can be estimated in accordance with degree of its approach to the similar process which is realized in ideal conditions at full absence of external contaminants.

A foundation of any purification process is an “elementary” (or “simple”) effect of separation of macro- and micro components of a system. It means reduction of a micro impurity concentration during a single-phase purification process.

A separating element is the least independent cell of a technological process giving some purification degree of a substance. An electrochemical cell, a plate of a rectification column, a mixer-settler of an extraction technological system, etc can serve as the separating elements. A group of in-parallel separating elements which operate with use of identical raw material and produce a product with the same micro impurity content is called as a stage of a technological circuit. Concepts of a separating element and a stage sometimes coincide (for example, a plate in a rectification column can be considered both as a separating element and as a stage). The separating element or the elementary-type stage is characterized by one incoming (feeding) stream and two outgoing ones: the first – with lowered micro impurity content, and the second – enriched by this micro impurity. If purity degree of a substance after one purification stage is insufficient, stages are connected in series. In this case the “elementary” separation effect is repeated many times. A group of the series-connected stages forms a cascade.

Rectification and extraction columns, a set of mixers-settlers for extraction, a system of apparatuses for repeated crystallization are cascades. Obtaining high-pure substances proceeds most effectively in counterflow column-type cascades in which a multistage purification process is performed in one apparatus without any intermediate heat-exchange and mechanical machinery – potential sources of various pollutants.

As follows from the above-stated, the choice of a purification method of a substance is defined not only by values of a separation factor, cleaning completeness and a thermodynamic efficiency coefficient, which characterizes energy consumption for removal of a micro impurity from the given material. A pollution factor, defined by an equipment type (a material kind, constructional features), and “chemical sterility” of the air environment play not a less important role.

A thermodynamic efficiency value (=А0/Аf where А0 – thermodynamic necessary minimum work for removal of a given micro impurity; Аf – actual work inputs) practically always is very low. For example, at separation of light elements’ isotopes by a thermal diffusion method  = 110-8. Very low values of thermodynamic efficiency coefficients speak about substantial irreversibility of applied purification ways including rectification when an elementary separation process in principle is reversible.

In a case of potentially reversible elementary purification processes of a substance (low-temperature rectification, chemical transport reactions, reversible absorption, and multistage vacuum distillation) a thermodynamic efficiency value is defined by power consumption on the ends of a cascade and outside it. On the contrary, at irreversible elementary processes (thermal diffusion, electrolysis, gas diffusion, counterflow gas centrifugation, mass diffusion, irreversible sorption, etc.) a thermodynamic efficiency is connected with separation work. In this case it is necessary to achieve increase of a thermodynamic efficiency coefficient of an elementary purification process and more correct distribution of material streams on stages of cascades. For partially reversible processes (absorption, extraction, azeotropic and extraction rectification, redox reactions, ion exchange) the minimum necessary separation work is proportional to energy consumption in the cascade itself and a thermodynamic efficiency value is defined by the factors typical both for reversible, and irreversible processes of cleaning a substance.

As a production scale of high-pure inorganic substances in many cases is insignificant a role of power consumption decreases; simplicity and chemical resistance of the equipment applied, serviceability, a possibility of creation of “ a chemical-sterile” air environment  become more important factors.

2.1 HIGH-PURE SUBSTANCE PRODUCTION METHODS

At present methods of manufacturing high-pure inorganic substances have gained the large value. They have a number of the features based on principle, which did not occur at realization of usual separation ways of blending components.

One of such the features consists in that deep purification processes of substances cannot be considered in isolation from the environment – a material of equipment, the atmosphere composition, quality of solvents, reagents etc. The environment and a work substance make a complex multiphase and multicomponent system in which all phases participate in a micro impurities (micro components) redistribution process.

Besides, during last stages of a technological fine purification process it has to be dealt with extremely diluted gaseous, liquid or solid solutions of various micro impurities distributed between phases.

High-pure substances as extremely diluted solutions of impurities are characterized by that intermolecular (interionic) interaction continue only between a basic component and micro impurities. Micro impurity ions and molecules are chaotically distributed in a macro component and completely solvate with a maximum coordination number. Therefore further dilution does not change interaction energy of the micro impurity molecule with the molecules of a base component surrounding it.

Substances’ purification techniques are or partially (absorption, extraction, azeotrope and extraction rectification) or completely (diffusion of impurities through porous partitions or a mercury vapor flow, adsorption, thermal diffusion, electromagnetic methods) thermodynamically irreversible. Reversible absorption, crystallization from solutions and distillation in some extent can be rated as thermodynamically reversible purification processes. Full or partial irreversibility of purification methods even of diluted solutions leads to sufficiently high actual energy consumption.

If not to take into consideration influence of apparatus (reactor) walls all the deep purification methods can be roughly divided into two classes: the methods, which are carried out with use of homogeneous systems, placed in electric, gravitational, magnetic or other force field (diffusion, electrophoresis, gas and liquid centrifugal separation, thermal diffusion) and the methods with application of heterogeneous systems (various types of rectification, extraction, simple and zone crystallization etc.).

In a homogeneous system a base material – micro impurities removal of the micro impurities is performed as a result of a concentration gradient between its two areas at influence on the system not only temperature, but also an electric, adsorption, gravitational or magnetic field.

Many methods of cleaning a substance with use of homogeneous systems (diffusion, thermal diffusion, liquid and gas centrifugal separation, electromagnetic methods and others) are applied in the high-pure substance production technology rather seldom because of significant power consumption, extremely complex process equipment and difficulties of a choice of proper constructional materials. In particular, application of electromagnetic purification methods is possible only at considerable ion currents and preservation of sufficient separation ability in the conditions of presence of an enormous space charge caused by other currents. Application of such the methods is expediently only when other ways of cleaning a substance are ineffective. For example, for industrial purification of 235UF6 from foreign fluorides of other isotopes application of a gas diffusion way and gas centrifuges is the most economic.

In the deep purification technology of inorganic substances the methods connected with use of diphasic heterogeneous systems are the most widespread. Occurrence of a phase boundary facilitates not only a process of differentiation of an impurity between various parts of a total system volume, but also separation of a part of the phase space enriched by the impurity. Reiteration of these differentiation and separation stages is what the essence of all modern multistage column purification processes. In heterogeneous processes the impurity distribution between phases occurs as a result of transition of a part of the micro impurity through a phase boundary by means of or simple physical redistribution, or a heterogeneous chemical reaction on the interface.

Complex physical and chemical phenomena take place on an interface during purification of a substance; in particular, two opposite processes can simultaneously proceed: deceleration of mass transfer owing to a slow heterogeneous reaction and acceleration of mass transfer due to the turbulence on the interface. Many phenomena observed on a phase boundary have not been studied in the sufficient measure, which would allow establishing a true mechanism of the mass transfer process of impurities from one phase to another one.

Among the heterogeneous deep purification methods rectification techniques, extraction, some sorption and crystallization ways (for example, zone crystallization) are the most effective. The rectification method, including such its versions as extractive and azeotrope rectification, is one of the leading methods of deep cleaning of liquid substances, even such the systems in which macro- and micro components form azeotropes. Electrochemical and some physicochemical methods of deep purification of substances also possess large potential possibilities.

In the modern chemical technology the gradual selection and isolating the certain direction, connected with manufacturing high-pure substances, is observed, which can be full right named a high chemical technology. Such the isolating in significant measure is a result of the discovery of heavy nuclear fission, nuclear chain reactions and studying the processes of intra-atomic energy release, caused the unprecedented technical progress in the field of isotope separation, obtaining of pure compounds of uranium, thorium, plutonium, zirconium and a number of other elements. Simultaneously with the isolating of high-pure substance technology there is a slow process of development of a science about pure substances. An immediate task of this science was formulated in the thirties of the last century as “putting of the ideas about chemical individuality of substances in harmonous order and critical statement of common methods of preparation of pure bodies”.

Classification of deep purification methods of substances. Unfortunately, the nature has not prepared for people neither simple, nor complex substances in a high-pure state. For example, it has “carefully” mixed all atoms. Any natural substance contains, apparently, all the elements of the periodic system with the only difference, that content of some elements is high, and other ones are contained in insignificant quantity. For example, a native sulphur consists of sulphur atoms making a great bulk of the substance and additionally of atoms of selenium, arsenic, iron, carbon and other elements. Any simple or complex substance is a mixture of many substances; and a chemist problem in this case consists in separation of a base material from this mixture and isolation of other substances from it (or at least the substances which can be found out). When obtaining this or that substance in a flask the impurities accompanying reacting substances partially pass in the reaction products. Besides, in the process always there is formation of by-products. Production of simple and complex substances in a high-pure state consists in their fine purification, in their liberation from impurity. In other words, the deep purification of a substance consists in separation of a mixture of substances (these are all natural and artificial materials), in isolation of a necessary substance from the mixture. The difference between common and deep separation consists only in that the separation depth at manufacturing high-pure substances should be considerably higher and a material of the equipment walls need not pollute the purified substance in some appreciable extent.

The methods applied to deep purification of substances are various. The method choice is defined by properties of a purified substance and the nature of removed impurities. For the present a deep purification theory has not reached such the level that it could be possible to predict, what method would be the best one for cleaning this or that element or substance. In each case it is necessary to carry out special researches giving a chance to choose the optimum method or to find the method allowing to reach needed purity degree.

Deep purification of a substance has demanded improvement of previously known methods of separation of mixtures, for example a recrystallization method, and also development of new methods, for example a transport reaction method.

A characteristic feature of the deep purification methods accepted now is that all of them are based on use of a difference in chemical or physicochemical properties of separated substances (i.e. in the properties caused mainly by a structure of electron shells of atoms or molecules of separated substances). The methods based on a difference in physical properties of separated substances, for example centrifugal or electromagnetic separation, are widely used for isotope separation; for the time being they do not apply for deep purification of substances.

In accordance with the nature of the effect laying in their basis, it is possible to unite all the deep purification methods applied now in two groups: chemical methods and physicochemical methods.

2.2CHEMICAL METHODS OF DEEP PURIFICATION OF SUBSTANCES

Chemical methods of separation of substance mixtures are based on a difference of equilibrium constants or rate constants of the reactions with participation of a base material and impurities. These methods concern the most ancient methods of substance purification. For example, manufacturing any metal is none other than a separation process of atoms of this metal from atoms of other elements accompanying it. Chemical methods of separation of mixtures are a foundation of classical techniques of chemical analysis. Practically all these methods include a stage of separation of impurities from a base material by means of their precipitation as insoluble compounds with the subsequent separation of the sediment or a stage of dissolution of impurities by a reagent in which the base material is not dissolved.

In connection with the problem of obtaining high-pure substances the chemical methods started to be applied also for deep purification of substances. For example, processing of silicon with mineral acids allows turning a considerable part of the metals containing in the silicon into the soluble salts which then can be washed. Thus the metal content in the silicon considerably decreases.

A selective complexing method is widely used for deep cleaning substances from impurity. So, when chemical interaction of water solutions of some alkaline and alkaline-earth metal chlorides and sulphates with sodium diethyl dithiocarbamate it is possible to turn the impurities of iron, cobalt, copper and some other transition metals containing in these salts in poorly soluble compounds like chelates according to the reaction:

S                                           S           S

2(С2Н5)2N–C          + Cu2+  = (C2H5)2N-C         Cu          C-N(C2H5)2 +2N2+

SNa                                       S           S

These compounds precipitate and can be filtered. The given method allows lowering iron content in the salts to 10-5 %, copper content – to 10-6 %.

Other quite effective chemical deep purification methods of substances are methods of selective oxidation or reduction in respect of a purified substance or impurities. Reagents in these processes are halogens (a selective halogenation method), hydrogen (a hydride method) etc. A rather promising technique is a thermal dissociation method, which is used, for example, for separation of some pure metals from their volatile hydrides.

Estimation of limiting possibilities of chemical methods of deep purification of substances

Let a reversible reaction proceeds

(I-1а)

and other reaction parallel to it

(I-1b)

where A – a base substance; А –  an impurity; B – a chemical reagent; С, С – reaction products;  – stoichiometric coefficients; К1, К2 – equilibrium constants.

Apparently from the expressions (I-1a) and (I-1b) a purified substance A will turn into a substance C, and a formed compound С will include an impurity А. Therefore further under the impurity maintenance in products of reactions (I-1a) and (I-1b). Поэтому в дальнейшем под содержанием примеси в продуктах реакций (I-1а) и (I-1б).

The fraction in which a numerator is a relative concentration of an impurity in reaction products, and a denominator is a relative concentration of the impurity in the initial substance equilibrium with the reaction products is called as a separation factor 

= ,                  (I-2)

where х* – a mole (atom) impurity fraction in reaction products; х – a mole (atom) impurity fraction in the initial substance equilibrium to the reaction products. The separation factor characterizes separation efficiency by means of a certain chemical reaction. The more  differs from unity the higher separation efficiency. If =1 the separation does not occur. When an impurity concentration in reaction products is lower than in an initial substance (the impurity concentrates in the remaining substance)  is determined as an inverse of the separation factor because is always expressed as a number more than unity. As during the deep purification of substances х«1 and х*«1 the ratio (I-2) becomes simpler:

х*/х                             (I-3)

Having made an expression (3) for the reactions (I-1a) and (I-1b) we will receive

= ,                           (I-4)

where C – concentration, mol/l. According to an equilibrium constant definition with regard to the reactions (I-1a) and (I-1b) we receive the expression

(I-5)

As follows from the ratios (I-4) and (I-5):

=                   (I-6)

As is obvious from the equation (I-6), the separation factor is directly proportional to a ratio of equilibrium constants of the reactions with participation of an impurity and with participation of a base matter.

For estimation of an order of the separation factor value we will consider a case when stoichiometric coefficients in the reaction equations (I-1a) and (I-1b) are equal to unity. The equation (I-6) will look like thus

=                  (I-7)

An equilibrium constant is connected with thermal effect of a chemical reaction by the known Vant Hoff law:

(I-8)

where ∆Н – thermal effect (enthalpy) of a reaction; Т – temperature, 0К; R – a gas constant.

Using this ratio we will receive

(I-9)

where – a difference in thermal effects of a reaction with participation of an impurity and a reaction with participation of a base matter.

Considering dependence on temperature as negligible and using the equation (I-9) we can receive

(I-10)

As a distinction between the equilibrium constants is basically defined by zero-point energies of reacting agents that

Taking into account the latter and after substitution of the expression (I-7) in the equation (I-10) we will come to the equation

ln=-                               (I-11)

The ratio (I-11) allows estimating a maximum separation factor value, which can be reached in the considered case by means of the reactions (1а and 1b). The difference in thermal effects of chemical reactions seldom exceeds 100 kcal/mol. If to accept ∆H=-100 kcal/mol and to substitute this value in the ratio (I-11)

 1022 at Т = 10000К;

 1073 at Т = 2930К

As follows from the calculated limiting values  that impurity content in the substance subjected to purification can be lowered to trace concentration by means of chemical reactions.

The equilibrium constant can be calculated more strictly using an isotherm equation of a chemical reaction:

lnK = -                            (I-12)

where  – a change of an izobaric-isothermal potential of a system owing to behavior in it of a chemical reaction at temperature Т.

The change of izobaric-isothermal potential is connected with changes of enthalpy and entropy of a reaction system by the following ratio:

=  - Т                          (I-13)

Using the ratios (12) and (13) for the reactions (1а and 1b) we will obtain

(I-14)

where the indexes (I-1) and (I-2) concern the reactions characterized by the equilibrium constants К1 and К2, respectively.

As a rough approximation the difference in changes of a system thermal capacity with reaction temperature growth and also the changes of enthalpy and entropy at the expense of possible phase transitions of reagents and reaction products at the temperature above the standard temperature (2980К) can be neglected. Taking into account this assumption and the expression (I-7) the (I-14) will be transformed:

(I-15)

where  and   – respective variations of a system enthalpy and entropy related to standard conditions; these values for many simple and complex substances are given in thermochemical and physicochemical reference books.

As a concrete example we will make the estimation of a possibility of purifying tellurium from a selenium impurity by a hydride method.

(I-16а)                       (I-16b)

Using standard values of thermodynamic functions for the reactions (I-16a) and (I-16b):

= 23,83 kcal/mol;       = 54,69 kcal/molK;

Исправление

As is evident from the ratio (I-15):

Т = 2980К;     1011;       Т = 4000К;     108;       Т = 10000К;     103.

As follows from the calculations, the hydride purification of tellurium from selenium is sufficiently effective. Efficiency of the hydride cleaning tin from carbon is still higher:

(I-17а)                (I-17б)

Т = 2980К;     1039;       Т = 4000К;     1029;       Т = 10000К;     1010. Исправление

[Calculation of  according to the formula (I-15) is possible and in a case, when 2 = 2 1 as it follows from the equation (I-6).]

Of course, it is necessary to select a suitable temperature regime in each concrete case at which a reaction rate would be high enough.

The effect of purification of substances by chemical methods can be also expressed through a difference of constants of reaction rates between a base material and an impurity with a reagent. Dependence of the separation factor on the rate constants of the reactions (I-1a) and (I-1b) for the case, when stoichiometric coefficients are equal to unity, looks like

(I-18)

where k – a rate constant of a direct reaction; k0 – a pre-exponential factor (Arrhenius frequency factor) in the reaction rate equation; Е – reaction activation energy at a set temperature Т; the indexes 1 and 2 concern the reactions (I-1a) and (I-1b), respectively.

As follows from a theory of absolute rates of chemical reactions, the ratio k20/k10 can vary from 1 to 108. The activation energy changes approximately in the same limits as the reaction thermal effect (enthalpy). Hence, the separation attainable as a result of chemical purification and based on the difference of rate constants of reactions between a base substance and the impurity containing in it with a suitable reagent can be the same effective as well as the separation based on the difference of equilibrium constants of these reactions.

The chemical purification methods are especially effective concerning the elements and the substances essentially differing on chemical properties. Therefore chemists synthesize various substances not being afraid, that walls of reaction apparatuses will collapse in the process. The same supports in them optimism in search of suitable materials for manufacturing the equipment, which walls would not pollute the high-pure substances produced in it.

A method of chemical transport reactions is successfully applied for obtaining high-pure substances. Chemical transport reactions are heterogeneous reversible reactions with participation of a gas phase, which allow realizing mass transport from one part of a system in another one if there is a temperature or pressure difference between these parts. Systems with a temperature difference are usually used for realization of the transport reactions. We will consider transfer of nickel as nickel tetracarbonyl as an example. Polluted nickel is placed in one end of a glass pipe. The pipe is vacuumized and filled with carbon monoxide. Formation of nickel tetracarbonyl takes place in the cold end of the pipe (Т1=45-500С) according to the chemical reaction:

Ni + 4CO        Ni(CO)4                     (II-1)

The gaseous nickel tetracarbonyl diffuses in the hot end of the pipe where it is decomposed under the influence of high temperature (Т2=180-2000С):

Ni(СО)4       Ni + 4CO                     (II-2)

The nickel formed precipitates on the pipe walls, and the carbon monoxide released diffuses to the cold end of the pipe. The reactions (II-1) and (II-2) are parts of the same reversible reaction:

Ni + 4CO  Ni(CO)4                      (II-3)

Equilibrium of the reaction (II-3) at temperature of Т2 is considerably displaced to the left in comparison with that is observed at temperature of Т1. Owing to it all the nickel from the cold end of the pipe can be transported to the hot end. Therefore such the reactions are called as transport reactions. In accordance with the experimental conditions a transferable substance should be solid or liquid and have low vapor pressure, and a chemical reagent (in this case carbon oxide) and a product of its interaction with the transferable substance – nickel tetracarbonyl – should be gaseous. In a general view the transport reaction equation can be written in the following kind:

1A(solid, liquid) + 2В(gas)  3С(gas)           (II-4)

Usually a chemical formula of the transported substance is written in the left part of the equation, and Т2 – is the temperature in the hot zone.

Equilibrium of the reaction (II-4) can be displaced not only temperature influence, but also pressure change. However, it is very difficult to create the high pressure difference necessary for appreciable displacement of the reaction (II-4) equilibrium. To realize this displacement by means of creation of a temperature gradient between the cold and hot zones is much easier. Therefore a two-temperature variant of the transport reaction technique is more widely-spread. The transfer direction is defined by the reaction thermal effect. If a reaction of a base substance with a set reagent is exothermal, then in accordance with the Le Chatelier principle the transport of this substance will occur from the cold zone in the hot one. If the reaction is endothermic, then the initial substance should be placed in a high temperature zone; in this case the substance transfer will occur from the hot zone in the cold one.

A possibility of transfer of a substance by means of chemical reactions has been noted in the second half of the ХIX century. So, for example, it has been established, that under certain conditions mass transfer is possible in following reactions:

Fe2O3 (s) + 6НСl (g)  2FeCl3 (g) + 3Н2О (g)

Si (s) + SiCl4 (g)  2SiCl2 (g)

2Al (s) + АlCl3 (g)  3AlCl (g)

However, practical application of such the phenomenon as mass transfer (transport) has begun only since the thirties of last century in connection with a problem of obtaining pure metals, especially refractory ones. For this purpose the reactions with participation of metal iodides (gaseous iodine was used as a reagent) have been suggested. For example:

Zr (s) + 2I2 (g)  ZrI4 (g)

Such the method of refining metals from impurities has received the name an iodide method. In particular, it has found industrial application for the deep purification of the zirconium, used for manufacture of nuclear reactor components, from a hafnium impurity. Now the chemical transport phenomenon is used for the deep cleaning some substances both simple, and complex.

It is necessary to notice, that the chemical purification methods are usually applied in combination with physicochemical methods (filtering, washing sediments, evaporation, distillation etc.). A rough estimate of the separation effect, which can be reached directly at the deep chemical cleaning substances is of considerable interest.

A comparative assessment of chemical and physicochemical purification methods. As it has been shown above, chemical methods of purification of substances have comprehensive facilities. Consider an example. If to pass a nitrogen-oxygen mixture through the pipe filled with a freshly reduced copper and heated up to some certain temperature, then as a result of the chemical reaction

2Сu + O2           CuO

the oxygen will bind the copper and stay too long in a column. Thus it is possible to reach the high purification degree of nitrogen from oxygen. If to pass the same nitrogen-oxygen mixture through the pipe filled with any adsorbent, for example activated coal, then the gas mixture will be also separated owing to various sorption property of nitrogen and oxygen. But in this case the separation occur less efficiently, than at the application of the pipe with the copper. In the first case the separation is based on a difference in the chemical forces operating between the atoms of copper and oxygen, copper and nitrogen. In the second case the factor is a difference in the forces of intermolecular interaction between coal and oxygen, coal and nitrogen. The intermolecular interaction forces are much weaker than the chemical forces. The difference in the interaction energies of nitrogen and oxygen molecules with the adsorbent is much less, than in energies of the chemical interaction of nitrogen and oxygen with the copper. Therefore the separation effect of the nitrogen-oxygen mixture in the adsorption process, which concerns to the physicochemical techniques, will be substantially lower, than in the reaction of their chemical interaction with the copper. However in spite of the fact that the elementary separation effect occurring in the physicochemical processes is usually insignificant, these methods are widely used for deep purification of substances. Following reasons explain it:

- It is not always possible to select a chemical reaction, which can be applied for effective purification of a certain substance as the chemical methods are improper for cleaning substances from the impurities with properties close to a base material. It is caused by that the circuits of multistage processes of deep purification of substances by means of chemical reactions have not been developed till now. Existing multistage chemical methods of mixture separation (extraction, a chemical exchange technique for isotope separation, etc.) are always accompanied by pollution of a purified substance by chemical reagents, i.e. at their application the deep purification of substances does not occur.

- Physicochemical methods of mixture separation do not demand chemical reagents for the realisation. Therefore, finally these ways are less power-intensive than the chemical methods, especially if to take into consideration that their realisation is almost always connected with a problem of separation of a purified material from reaction products.

- Physicochemical processes, unlike chemical reactions, are rather simply realized in a counter-current regime. It allows to increase manyfold the separation effect and eo ipso to reach a necessary purification degree.

2.3DISTILLATION METHODS

Distillation methods concern the methods of deep purification of substances most developed both theoretically /28,29/ and experimentally /5,29,30/. They are used both a one-stage (simple distillation), and a multistage counterflow (rectification, molecular distillation) variants. These techniques are applied to deep cleaning the substances, whose boiling temperature is in limits from helium boiling point to 1000°С. All high-pure solvents such as ethanol, dichloroethane, isopropanol, acetone, etc are produced by this method.

Distillation methods include simple distillation, rectification and molecular distillation. These methods are based on a difference in compositions of a liquid and the vapor evaporating from its surface. This difference can be characterized by a value of relative volatility of the component under consideration, named in this case as a separation factor.

Designated a mole foreign component fraction in a liquid as х, and a mole fraction of the same component in the vapor equilibrium with the liquid as y, we will obtain in respect to a system “liquid-vapor” the following expression:

(III-1)

In this case essentially it is not important what phase will be considered as the first one; expressions of the type (III-1) are usually made so that a separation factor value  would be more than unity.

A separation factor value  is the major characteristic of a single-phase, elementary separation process. As is evident from the ratio (III-1), if =1 a mixture separation does not occur, i.e. a composition of both equilibrium phases will be identical. The more  differs from unity, the higher separation effect is. Therefore in the separation process theory scientists often use the following coefficient

=  - 1               (III-2)

named an enrichment factor. For the chemical separation techniques »1 practically always and we can accept with satisfactory accuracy that . Therefore the concept “enrichment factor” is used only in the theory of physicochemical separation processes in which the  value sometimes differs from unity a little.

The separation coefficient generally depends on temperature and also on a separated mixture nature and composition. At deep purification of substances when a total impurity content in a substance is insignificant in comparison with a base material content, the mutual influence of impurities can be neglected. So, a multicomponent mixture, what each a substance is, can be considered as a binary mixture consisting of a main component and an impurity, which is characterized the above-mentioned ratio:

х«1  and   у«1                       (III-3)

It characterizes strong diluted solutions (solid, liquid, gaseous). At fulfillment of the condition (III-3) it is also believed, that the separation factor for a mixture “a main component – an impurity” will not depend on its composition and will be a constant at the constant process temperature.

Application of the condition (III-3) and the accepted assumption about constancy of a separation factor value  for the mixture under consideration substantially simplifies many calculations connected with estimation of deep purification effect of substances. Taking into account the condition (III-3) the expression (III-1) can be written as

= х/у                         (III-4)

The ratio (III-4) characterizes the separation process when the impurity concentrates in a liquid phase. In a case when the liquid phase is impoverished by the impurity, the separation factor will be expressed as the following ratio:

 = у/х               (III-5)

Use of the equations (III-4) or (III-5) in calculations is defined by a separation process circuit.

It is necessary to know the separation factor value in each a concrete purification process to estimate efficiency of the technique used, to choose a rational circuit and regime of the process, to calculate equipment size, etc.

Methods of separation factor determination

There are some methods for determination of separation factor.

- A method of evaporation of little amounts of a solution

The simplest way of separation factor determination consists in distillation in equilibrium conditions of a little vapor volume from a great solution amount. In the process it is accepted, that at the evaporation of an insignificant part of the solution the liquid phase composition practically does not change. By analogy it is supposed, that the initial vapor composition during such the evaporation also does not change. Separation factor is determined according to the equation (III-4) or (III-5) after analysis of compositions of the solution and the condensed vapor

- Determination of separation factor by means of a single-phase evaporation apparatus (one theoretical stage apparatus).

This method is more exact, than the first one. If to evaporate some quantity of a solution and to equilibrate the vapor formed in thermodynamic equilibrium with the remained solution that, having carried out the analysis of equilibrium compositions of vapor and liquid phases and used the equations (III-4) and (III-5) it is possible to determine the value. To reach equilibrium of the vapor and liquid phases the evaporation is performed in special apparatuses named one theoretical stage (theoretical plate) effectiveness apparatuses. A great number of designs of such the apparatuses is described in technical literature.

- Rayleigh distillation method.

The installation diagram is represented in fig.8. At partial evaporation of a liquid mixture a macrocomponent – a microcomponent (impurity) the remained liquid will be enriched by a high-boiling component. We will designate the general number of the liquid mixture moles in some moment of the evaporation process as N, and the impurity mole fraction as х. If dN solution moles passes in vapor, the impurity content in the vapor formed will be equal ydN, where y – is the impurity mole fraction in the vapor. As a result of the evaporation the solution amount will decrease and also its composition will change. Therefore it is possible to write down, that

d(Nx) = ydN               (III-6)

Let an impurity is a high-boiling component. Then, substituting the ratio (III-4) in the expression (III-6) we will obtain

хdN + Ndx =                 (III-7)

After separation of variables

(III-8)

Having integrated (III-8) and determining an integration constant from a condition, that х=х0 at N=N0, we will obtain

and                 (Ш-9)

As for diluted solutions it is possible to believe, that N/N0 = V/V0, where V0 – a liquid volume before evaporation; V – a liquid volume in a distillation flask in a specified moment of the evaporation. The equation (III-9) connects a change of the solution concentration with a change of its quantity owing to the evaporation. This equation is called as Rayleigh formula as for the first time a similar ratio has been obtained by Rayleigh in 1896 when separation of an ideal gas mixture by means of diffusion through a porous partition. Having determined the impurity concentration in the solution in the beginning and the end of the evaporation by means of the equation (III-9) it is possible to determine separation factor . More accurate  value can be found graphically in accordance with a straight line inclination in co-ordinates lgx/x0  lgV0/V.

The Rayleigh formula is true under following conditions:

- the liquid composition should be identical in all its volume at any evaporation moment;

- the liquid and the vapor during the evaporation should be in thermodynamic equilibrium.

The first condition is realized by means of sufficiently intensive hashing of the solution (for example, by means of a mixer). The second one proceeds successfully by application of very slow evaporation. Such the evaporation process as a whole has been named as Rayleigh distillations. The maximum evaporation rate value, admissible at determination of α by means of the Rayleigh distillation, can be estimated as follows. In case of high vacuum the evaporation rate of a substance can be characterized by the Langmuir equation:

(III-10)

where L0evap – evaporation rate, mol/cm2∙sec; Р – saturation vapor pressure over the evaporation surface, dyne/cm2, at temperature of Т0К; M – molecular weight of a substance; R – 8,3107 erg/mol∙degree – a gas constant. For the considered Rayleigh distillation case, when the liquid evaporates in the atmosphere of own vapor, which is under the pressure Р lower than the equilibrium pressure P ⃰, respective to the liquid temperature, it is possible to write down

where Levar – actual evaporation rate measured in a distillate quantity per a time unit;  – a correction factor named as an accommodation coefficient; it takes into account a part of the vapor, condensed as a distillate, from the total quantity of the vapor formed in a time unit.

Thus, as follows from the equation (III-11) the maximum evaporation rate depends on a value of the permissible discrepancy between equilibrium pressure Р* and vapor pressure Р under the given evaporation conditions and also on the process temperature and molecular weight of the evaporated substance. At great  values the permissible discrepancy between Р* and Р can be much more, than at values close to unity. In view of the fact that the equation (III-11) includes a rather uncertain quantity  (accommodation coefficient) the allowable evaporation rate is found not its calculation in accordance with the equation (III-11), but experimentally, by means of determination of a dependence of a separation factor value from evaporation rate (condensation rate) (fig.9). Operating rate should be in the area where the further reduction of evaporation rate does not cause changing the separation factor value.

The Rayleigh distillation method allows determining even small α values with satisfactory accuracy. Really, if a ratio of liquid volumes in the beginning and in the end of evaporation V0/V in the equation (III-9) is great enough, a h/h0 value will appreciably difference from unity even at small  values and can be determined with high precision.

In that case when an impurity is a low-boiling component, α determination proceeding from experimental results is carried out by means of a modified Rayleigh formula

(III-12)

This formula follows from the ratios (III-6) and (III-5) or from the formula (III-9) if to use in the latter a reciprocal separation coefficient.

The considered methods of separation factor determination are usually applied when it is necessary to know a specific α value for this or that system, for example, before carrying out rectification purification. On the other hand the separation coefficient in some cases can be determined indirectly on the basis of experimental rectification results. Accuracy of similar determinations is usually insignificant, as it is caused by reliability of the analytical relations, which characterizes the interconnection of α value with separation effect, which can be reached during rectification in those or other conditions (a stationary or non-stationary state, etc.).

Simple distillation

Simple distillation as a purification technique is usually equipped similarly to Rayleigh distillation; the Rayleigh distillation (equilibrium distillation) represents in essence an ideal variant of simple distillation (fig.8).

If the enrichment coefficient is great, then the first condensate (distillate) portions formed at the solution evaporation will not practically contain a high-boiling component. Therefore this component will concentrate in a residue, i.e. in that part of the solution which remains in a distillation flask (a distilling tank). Such simple distillation allows separating a mixture on the fractions consisting generally of individual substances. According to it at the separation of usual mixtures it is possible to obtain several fractions with different compositions; for this reason this variant of simple distillation is sometimes named as “fractional distillation”. However, it is necessary to notice, that this name is more often used as a synonym of another distillation technique – rectification.

An advantage of the simple distillation is simplicity of its realization (fig.8). Consider the water distillation as an example. Water contains the salts, which boiling temperature is usually low. At distillation these salts will concentrate in the liquid remaining in a distillation flask. Content of the salts in a distillate will be negligible. Owing to equipment simplicity the distillation is applied to cleaning the high-boiling substances: metals, salts etc., when manufacturing and operation of more complex equipment for realization of other purification methods present considerable difficulties. Simple distillation is sometimes used also for purification of thermally unstable or air unstable substances. In these cases the distillation process is usually performed at lowered pressure (vacuum distillation) for the purpose of decrease in boiling temperature of the purified substance or for removal of air from the system.

In agreement with analogy of simple distillation with Rayleigh distillation, the impurity content in the residue (in the distillating flask) can be estimated according to the formulas (III-9) or (III-12). For determination of the impurity concentration хd in the distillate fraction we accept, that at any moment of the distillation process the vapor formed is continuously taken away from the system as a distillate. Taking into consideration this assumption and using the formulas (III-9) and (III-12) we will obtain

(III-13)

if the impurity is a high-boiling component; accordingly if the impurity is a low-boiling component:

(III-14)

Of course, at application of the formulas (III-9) and (III-12)-(III-14) for determination of the impurity content in the residue and the distillate it is necessary to know the α value.

In respect to deep purification of substances simple distillation can be also used as a preliminary stage before carrying out multistage separation processes. For separation of mixtures of the substances with close boiling points simple distillation is ineffective.

Rectification

A rectification process is fulfilled in special apparatuses named rectification columns. The column diagram is represented in fig.10. The vapor, formed at boiling a liquid in a boiler of a rectification column A, rises in the top column part, where a condenser C is installed. The condensate flows down in the boiler A. Thus there is countercurrent of the liquid and the vapor in the rectifying part B of the column; between the liquid and the vapor a mass exchange process takes place, i.e. there is the interphase redistribution of the components. The phase conversion occurs on the column ends: the vapor turns into a liquid in the condenser, and the liquid turns into a vapor in the distilling tank. All it leads to multiplication of the elementary separation stage observed at usual evaporation of a liquid. The closer contact between a liquid and a vapor in a rectifying part is, the higher rate of an interphase mass exchange process and separation effect in a column are. Thus, in accordance with the result a rectification column is equivalent to an apparatus consisting of several stages, in each of which there is usual distillation separation; and the stages are connected in such a manner that the distillate from the first stage is a distillating liquid for the second stage etc. So, the rectification is a multistage separation process. As a result of the rectification process a low-boiling component concentrates in a top part of a column, condenses and is taken away from there as a product. In this case the liquid in a condenser formed of the incoming vapor is divided into two parts. A smaller part of the liquid is taken away as a product, and another, bigger, part is returned in the column and used for countercurrent irrigation of the vapor as reflux. A ratio of the irrigation rate to the product bleeding rate is called as a reflux-to-product ratio. Often the product bleeding rate is characterized also by a bleeding degree value, which is a part of the flow taken away as a product. It is easy to show, that these two values are interconnected:

(III-15)

where р –  a bleeding degree; Ф – a reflux-to-product ratio.

If the product from the column is not taken away, р=0, and Ф=. In this case it is said, that the column works in a non-bleeding regime or at full reflux (full irrigation).

If the column operates according to a bleeding mode, i.e. at partial irrigation (р>0, Ф<), the mixture loaded into the column boiler (for example, when separation of ordinary mixtures) can be separated into several fractions with various boiling points by means of continuous removal of the distillate part. In this case the term “rectification” is identified with the term “fractional distillation”. Certainly, every fraction can be a target product; it depends on the needs.

In industrial conditions rectification is usually realized as a continuous process (fig.11). A separated mixture is fed to an average part of a column. Low-boiling components of the mixture are concentrated in the top column part, and high-boiling ones – in the bottom column part. The mixture input point divides the column into two sections. The section, in which the needed component concentrates, is named as a strengthening section. Other section is called as an exhausting section. From the point of view of a rectification theory each of these sections can be considered as a separate column. In addition continuously operating columns are sometimes called as open cycle columns (fig.12).

Thus, at work of a continuously operating column an initial mixture is basically separated on two fractions: a low-boiling fraction, which is partially taken away from the column condenser as a distillate, and a high-boiling one, which is partially removed from the column distillating tank as a residue. With regard to a deep purification case one of these fractions that is what a necessary product.

In laboratory practice periodic columns, i.e. closed cycle columns are more often applied. The rectification theory did develop basically in respect to open cycle columns. This theory is true also both for the closed cycle columns, which have an “infinitely” large boiler, and sufficiently well describes work of the columns with a finite size boiler.

Technical literature contains descriptions of a considerable quantity of rectification column designs. Basically, they differ from each other with operation regimes (continuous or periodic), a contact device and sizes. According to a contact condition between a liquid and a vapor all rectification columns can be divided into three types:

- plate-type columns – a liquid-vapor contact occurs unevenly on the special “plates” horizontally installed in various column sections;

- packing columns – a liquid-vapor contact is realized continuously on all a column height – on a “packing”;

- film columns – in this type columns a liquid flows down from top to down on internal walls in the form of a film with constant thickness. The liquid-vapor contact takes place continuously on all the column height on the film surface.

2.4CRYSTALLIZATION METHODS

At present time crystallization methods of mixture separation, as well as distillation ones, are widespread. So, fractional recrystallization of substances from a solution is applied to purification of some oil refining mixtures and products. In connection with the problem of obtaining high-pure substances two new methods of crystallization refining have been suggested: zone recrystallization (zone melting) and counterflow crystallization from melt. Zone melting is a final purification stage of a number of the high-clean substances produced now. Counterflow melt crystallization, apparently, has also a big prospect because numerous researches have shown that it has advantages in comparison with zone melting concerning process time and power consumption. A basis of all crystallization techniques is a difference in compositions of a liquid (a melt or a solution) and the solid phase (crystals) formed of this liquid during crystallization.

Though any mixture subjected to separation is multicomponent with reference to deep purification of substances the problem of separation effect estimation also can be reduced to consideration of a binary system “a base material – a considered impurity”; first of all it concerns melt crystallization processes. In case of crystallization of substances from a solution the third component – a solvent – usually plays a role of a medium, but sometimes it is a solvation agent. The solvates formed in a system can be considered as individual substances.

A separation factor (distribution factor). An important characteristic of crystallization techniques of mixture separation is a ratio of concentrations of components in equilibrium solid and liquid phases. By analogy with the distillation methods, this ratio is named as a separation factor and can be expressed as

(4-1)

where х – a mole impurity fraction in a solid phase; y – a mole impurity fraction in the liquid phase equilibrium to the solid phase. In order to avoid misunderstanding it is necessary to notice, that at consideration of crystallization processes the value determined by the ratio (4-1) is more often named as a distribution coefficient, and sometimes – a cocrystallization factor.

At the analysis of crystallization separation processes in a case when an impurity component concentrates in a liquid phase, it has to be applied (unlike the distillation methods) the α value less unity as it is visible from the equation (4-1). At deep purification of substances, as it was already marked, it is necessary to deal with diluted solutions. Therefore, as in this case х«1 and у«1, below for determination of the separation coefficient we will use a ratio

= у/х                 (4-2)

The separation factor in the crystallization techniques as well as in the distillation ones basically can be calculated by means of methods of chemical thermodynamics on the assumption of that the impurity chemical potential should have the same value in solid and liquid phases. But as a satisfactory liquid state theory has been not developed till now, similar calculations for the present are not very reliable, and for determination of the separation coefficient it is necessary to know experimental data concerning the equilibrium a solid phase - a liquid phase. The separation factor  can be easily determined in the presence of a fusibility diagram of the given binary system. However for deep cleaning it is necessary to know a numerical value  in the low concentration field of one of the components, i.e. it is necessary to know “an angle” of the fusibility diagrams. Unfortunately, appropriate data are usually absent in technical literature and consequently in most cases it is necessary to resort to the help of experiments for estimation of the separation factor. One of such the methods of α determination with reference to a case of small concentrations of one of the components is a normal oriented crystallization method.

Normal oriented crystallization. Let a vessel 1 (fig.30) is cooled thus (for example, by slow input of the vessel in a cryostat), that the liquid mixture a base substance – an impurity being in it starts to crystallize in the bottom part of the vessel, and the crystallization front gradually moves upwards. It is supposed that the liquid phase is intensively agitated and, so, the liquid remaining in the course of such normal oriented crystallization is homogeneous at any instant of time.

If the separation factor is more than unity then the crystals precipitated will have higher impurity concentration than the remaining liquid. In the process of crystallization front advance the impurity de-enrichment of the liquid will become more appreciable, and the liquid top layer can be substantially released from the impurity. If the separation factor is less than unity then the solid phase formed behind the crystallization front will contain the less impurity amount, than the liquid which is before the crystallization front.

Zone recrystallization (zone melting). The oriented crystallization as a separation method allows obtaining satisfactory enough results only with reference to a mixture of the substances considerably differing on their physical and chemical properties. For mixtures of the substances with close properties the separation factor  usually differs from unity a little, and the specified separation method becomes inefficient. For separation of similar mixtures it is necessary to apply such the method which would allow increasing the effect attainable at the single crystallization, for example, similar that as rectification multiplies the separation effect reached at usual equilibrium evaporation of a liquid. A zone recrystallization technique or as it is usually named a zone melting method is one of them. The idea of this method consists in moving a narrow molten zone along a solid sample, as it is shown in fig.33.

As diffusion processes in a solid phase always proceed much slower, than in a liquid it is possible to consider, that the solid phase, which is in a heating area, turns into a melt without changing its composition. If at the same time crystallization rate of the substance behind the moving liquid zone is small (crystallization rate is equal to melting rate or speed of the zone movement) then the ratio between the impurity concentrations in the melt and in the crystals precipitated of it can be considered close to the equilibrium ratio during each a point of time.

Counterflow crystallization from melt. A method of counterflow crystallization from melt does not have the disadvantages of zone recrystallization first of all concerning of the separation process productivity and duration. This separation method, as well as the zone recrystallization technique, is a multistage process, but it is realized as a counterflow process in a column-type apparatus – a crystallization column. A number of crystallization column designs are described in technical literature; they differ mainly by phase countercurrent conditions. A diagram of one of such the designs is represented in fig.37.

Temperature in the bottom column end is higher, and in the top end is lower than a mixture melting point. The crystals formed in the top part of the column are fed by means of a spiral conveyor (or otherwise) in the bottom column part, where they melt. Hashing of the crystals causes displacement of the melt in the opposite direction that creates the countercurrent motion of solid and liquid phases. As a result of such the process (with reference to a binary system) one of the mixture components will concentrate in the column bottom part from which it can be discharged as a product; other component will concentrate at the top of a column.

In principle, the counterflow crystallization method is similar to other diphasic way of mixture separation – to rectification. The separation in a crystallization column, as well as in a rectification one, is based on a difference of equilibrium phase compositions. At realization of the counterflow crystallization a mixture can be also fed in the middle of the column, which one end is equipped with a crystallization device and another end – with a melting device. In this case the crystallization column will consist in essence of two sections, which are named as exhausting and strengthening sections, respectively.

In spite of the seeming simplicity of a process of counterflow melt crystallization, carried out in a crystallization column, the separation process has sufficiently complex nature. First of all, the separation cumulative effect besides the crystallization effect, which takes place at formation of a solid phase in a crystallizer of a column, will also include the effect of washing the crystals formed from the liquid trapped by the countercurrent moving melt. Secondly, a size of the crystals of the solid phase moving on the column can change, for example, for the account of recrystallization. Therefore, the effect even if partial recrystallization in the column also takes place, similar to that as the partial vapor condensation and liquid evaporation occur directly in a rectifying section of a rectification column. And, thirdly, comparison of results of cleaning the substances with molecular and ionic crystal lattices indicates the diffusion character of interphase mass exchange in ionic crystal lattices, that allows achieve higher purification effect; it is caused higher diffusion rate of an impurity in such the substances.

Crystallization from solution. Crystallization from solution as a separation and purification method of substances has wide application both in laboratory researches, and in industry. This method is successfully used for separation of salt mixtures as in many cases we can apply water as a cheap solvent, and solubility of salts in water usually substantially varies with temperature change. From this point of view various alcohols and esters, acetone, benzene, carbon bisulphide etc. are good solvents for separation of some mixtures of organic substances. Combined solvents are sometimes applied in the oil-refining industry with a view to selective dissolving separate components of a mixture.

As opposed to the melt crystallization, the crystallisation from solution is usually performed in a range of rather low temperatures. In this respect the solution crystallization has an important advantage in comparison with the melt crystallization as with its help it is possible to purify a great number of thermally unstable or refractory materials. But on the other hand, the presence of a solvent sometimes complicates the purification process as solvation (hydration) or solvolysis (hydrolysis) processes take place in a solution. Besides, at the deep purification of substances by the considered way an initial solvent should be on its own account pure enough in order to avoid ingress of an undesirable impurity in the crystals of the purified substance.

Crystallization from solutions is a complex and in essence multistage process. It is necessary to notice, that the general condition of crystal precipitation from a solution is its supersaturation, i.e. excess concentration of a substance in a solution (over its solubility).

As solubility of the majority of solid substances decreases with temperature fall, then the supersaturation causing crystal precipitation can be reached, for example, by means of cooling of hot initial solutions. This variant of the solution crystallization technique has received the name isohydric crystallization; at its realization the solvent quantity in a solution is practically constant. A version of this variant is freeze crystallization, which takes place at cooling of solutions up to temperatures lower than 00С.

Transition of an initial solution in a supersaturated state can be also realized at the expense of partial removal of a solvent by means of the solution evaporation. Such variant is named isothermal crystallization as in this case after achievement of an oversaturated state the solution evaporation occurs at its constant boiling temperature.

The choice of optimum solution crystallization variant depends on the behavior of substance solubility at various temperatures. When solubility of a substance changes a little with temperature change (for example, even decreases with temperature increase), it is rationally to apply isothermal crystallization. Sometimes in practice it is necessary to combine both variants.

There are also other ways of a solid phase separation from a solution, for example, by addition in the solution of any substance which reduces solubility of the separated material; this way is named salting out. At carrying out so-called adductive crystallization the reagent, which forms a less soluble compound or an inclusion complex with the separated substance, is added in the initial solution. These ways can be used as a preliminary stage of concentrating a necessary substance before its subsequent deep purification.

As follows from the stated, the solution crystallization process is characterized by a typical phase transformation. In this sense a difference in principle between crystallization from solutions and crystallization from melts is absent, though every process has its essential features.

At crystallization of a substance from solution it is to some extent contaminated with the impurities which are in the initial liquid phase. A process of impurity capture by a solid phase is generally called coprecipitation. Thus, any transition of an impurity in a solid phase irrespective of the reasons causing this transition is considered as coprecipitation of the impurity with a separated substance. There are true homogeneous coprecipitation and surface heterogeneous one, caused by adsorption. The homogeneous coprecipitation takes place, when a substance and an impurity are isomorphic or isodimorphic, i.e. they can crystallize in a joint crystal lattice. This coprecipitation type is usually called cocrystallization. Conditions of a solid phase separation play an essential role in coprecipitation processes, especially when heterogeneous coprecipitation. The equilibrium between solid and liquid phases can be reached under certain conditions; in this case it is said about equilibrium impurity distribution between the specified phases.

Fractional crystallization. An appreciable separation effect can be reached sometimes even as a result of single crystallization coprecipitation. However in most cases the separation effect can be essentially killed because of inclusions of a mother solution in cracks and pores of the crystals formed, wetting the crystal surface with the mother solution containing an undesirable impurity, etc. Contamination level of the crystals by the impurity basically can be reduced by means of preliminary purification of the initial solution, washing the product or its subsequent recrystallization. Washing the crystalline product by a fresh solvent (sometimes a saturated solution of the crystallized material is applied for this purpose) not always yields good results. So, for example, the mother solution, included in a solid phase crystal, cannot be fully removed from it by washing; for its removal the precipitated solid phase should be subjected to recrystallization. Repeated repetition of the crystallization process allows reaching a necessary purity degree of the needed substance. At repeated recrystallization there is cleaning a base material from the impurities getting to the crystals both at the expense of homogeneous, and at the expense of heterogeneous coprecipitation. There are some variants of repeated recrystallization (for example, with recycling mother solutions and without their recycling). One of such the variants is a fractional crystallization method. The heart of this method is schematically represented in fig.39.

In accordance with the diagram an initial solution by means of crystallization is separated on two fractions – crystals and a mother solution (the first crystallization stage). According to a distribution law both fractions will have the compositions, which differ from the initial solution composition. Each of these fractions is similarly separated on two new fractions. Therefore the result of the second crystallization stage is four fractions. The fractionating process can be repeated before the achievement of specified purification degree of a necessary. If an impurity concentrates in mother solutions then the crystals formed will be an end-product.

3 CHAPTER

3.1MANUFACTURE OF ORTHOPHOSPHORIC ACID OF “P” AND “P.F.A”

Orthophosphoric acid:

empiric formula – Н3РО4                      ОН

structural formula:     H – О = Р            ОН

molecular weight – 98,00 g/mol            ОН

Properties: colourless rhombic-system crystals with density of 1,834 g/cm3. At 42,350С it melts in a colourless odorless syrup-like liquid with density of 1,88 g/cm3. Liquid acid is rather inclined to overcooling. Well soluble in water and ethyl alcohol. It is crystallizes from water solutions with density of 1,75-1,85 g/cm3; the crystallization is realized easier at addition of seed. At heating above 1500С phosphoric acid starts to lose bound water (not having reached 100 % of Н3РО4), turns into Н4Р2О7 and then into НРО3. Н3РО4 is an average acid, its dissociation constant at 250С are: К1 = 7,610-3, К2 = 6,2 10-8, К3 = 4,410-13. Boiling point of 87 % orthophosphoric acid is 159,40С.

Characteristic of initial raw material, auxiliary materials and finished goods

Raw materials for manufacturing orthophosphoric acid are thermal phosphoric acid, sodium sulphide and activated wood crushed carbon.

The characteristic of initial raw materials, auxiliary materials and semiproducts is represented in table 1.

Table 1 – Characteristic of initial raw materials, auxiliary materials and semiproducts for orthophosphoric acid production

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |
| --- | --- | --- | --- | --- |
| 1 | Commercial-grade orthophosphoric acid | According to the initial data of НИЧ КНГФ | 1. Mass fraction of orthophosphoric acid Н3РО4, no more, % 2. Density, not less, g/cm3 3. Mass fraction, no more, % - nitrates (NO3) - sulphates (SO4) - chlorides (Cl) - iron (Fe) - heavy metals (Pb) - arsenic (Аs) | 86-88 1,71-1,72 0,0005 0,002 0,0002 0,001 0,005 0,006 |
| 2 | Sodium sulphide reactive “p” or “p.f.a.” grades | State Standard 2053-77 | Mass fraction of Na2S9H2O, not less, %: “p” “p.f.a.” | 93 97 |
| 3 | Distilled water | State Standard 6709-72 | 1. Residue after evaporation, mg/l, no more 2.Residue after calcination, mg/l, no more 3. Content, mg/l: - ammonia and ammonium salts (NH4), no more, - nitrates (NO3), no more - sulphates (SO3), no more - chlorides (Cl), no more - aluminum (Аl), no more - iron (Fe), no more - calcium (Са), no more - copper (Cu), no more - lead (Pb), no more - zinc (Zn), no more - the substances reducing КМnO4 (О), mg/l - pH | 5,0 1,0 0,02 0,2 0,5 0,02 0,05 0,05 0,8 0,02 0,05 0,2 0,08 5,4-6,6 |
| 4 | Activated wood crushed carbon | State Standard 6217-74 | 1.Iodine adsorption activity, %, not less 2. Total water porosity, cm3/g, not less | 50 1,5 |
| 5 | Filter cloth № 2, a code number 20 | State Standard 6217-74 | According to the State Standard 6217-74 |  |
| 6 | Filter fabric “Lavsan”, a code number 86030 | Ukraine TS 3238-84 |  |  |

Characteristic of “pure” and “pure for analysis” orthophosphoric acid according to the requirements of State Standard 6552-80 is represented in table 2.

Table 2 – Requirements of State Standard 6552-80 to orthophosphoric acid quality

| № | Parameter name | Standard |  |
| --- | --- | --- | --- |
|  |  | “p” | “p.f.a.” |
| 1 | Appearance and color | Should stand the test according to the item 4.2 of State Standard 6552-80 |  |
| 2 | Mass fraction of orthophosphoric acid Н3РО4, not less, % | 85 | 85 |
| 3 | Density , not less, g/cm3 | 1,69 | 1,69 |
| 4 | Mass fraction of the residue after calcination, no more, % | 0,2 | 0,1 |
| 5 | Mass fraction, no more, % - volatile acids | 0,0015 | 0,001 |
|  | - nitrates (NO3) | 0,0005 | 0,0005 |
|  | - sulphates (SO4) | 0,003 | 0,002 |
|  | - chlorides (Cl) | 0,003 | 0,0002 |
|  | - ammonium salts | 0,002 | 0,002 |
|  | - iron (Fe) | 0,002 | 0,0015 |
|  | - arsenic (Аs) | 0,0002 | 0,0001 |
|  | - heavy metals (Pb) | 0,001 | 0,0005 |
|  | - the substances reducing КМnО4 (H3PO4) | 0,05 | 0,005 |

Manufacture of orthophosphoric acid of “p” and “p.f.a.” grades is realized on three technological lines equipped with precipitation columns, desorbers, absorbers; output of each the line is 8700 t/year.

A production way

Reactive orthophosphoric acid of “pure” and “pure for analysis” grades is produced by the method widely applied in industry and based on purification of dry-process phosphoric acid from arsenic and lead by means of their sedimentation by hydrogen sulphide as sulphides. The sulphides precipitated are separated from the acid by filtering, and the hydrogen sulphide excess is removed from the acid by air.

The sedimentation proceeds on following chemical reactions:

2Н3РО4 + Na2S  2NaH2PO4 + H2S        (1)

2H3AsO3 + 3H2S  As2S3 + 6H2O          (2)

Pb(H2PO4)2 + H2S  PbS + 2H3PO4        (3)

The process technological features. The acid filtration is performed with use of special filters “LVAZh-20k”. Desorbers are provided for purging of hydrogen sulphide from the acid. Purification of air from hydrogen sulphide is realized in absorbers.

A flow chart

The production process consists of following stages:

1. Reception and heating a dry-process phosphoric acid

A dry-process 87% phosphoric acid is fed by a pipeline in receiver tanks of a reactive phosphoric acid (RPA) section, where it is heated up to 600С. The temperature is regulated automatically by feeding hot water in a coil. Measurement of the top and bottom levels in the receiver tanks is provided. The acid from the tanks is fed in a pressure tank, then continuously by gravity given in a precipitation column through an induction flow meter. The acid surplus from the pressure tank by means of an overflow line is returned in the receiver tanks. In case of the overflow absence from the pressure tank an alarm system is provided.

2. Preparation of a 3-5% sodium sulphide solution

The water from a distilled water department is fed in a pressure tank, and then it flows by gravity in a pressure tank with volume of 170 l for preparation of a sulfureous sodium solution. Dry sulfureous sodium in quantity of 5-8 kg is fed in the same tank. The obtained solution is mixed up manually before full dissolution of the salt crystals. Then the 3-5 % sodium sulfureous solution by means of an overflow pipe through a float feeder is given in the precipitation column. A filter is installed between the pressure tank and the float feeder to prevent ingress of mechanical impurities.

3. Sedimentation of arsenic and heavy metal impurities by hydrogen sulphide

Phosphoric acid contains arsenic as arsenous acid (H3AsO3), and lead – as monophosphate [Pb(H2PO4)2]. Both the impurities are precipitated from the acid as sulphides. For this purpose the phosphoric acid is mixed with the 3-5% sodium sulfureous solution. The impurity sedimentation process is realized in the precipitation column. The acid is fed in the top column part and evenly sprayed by atomizers on a packing. The acid flows down from the packing in the bottom part of the column. The Na2S solution from the pressure tank is also batched in the bottom part. The hydrogen sulphide formed is subjected to purification.

The feed of phosphoric acid and sulfureous sodium, temperature and vacuum in the column is controlled. As a result of the reaction (1) proceeding in a surface layer of the column tank the hydrogen sulphide is formed, which goes up through the packing and interacts with the acid moving countercurrent. The arsenic and lead impurities, contained in the acid, react with the hydrogen sulphide and form the sulphides, which are precipitated from the solution as yellow-green flakes.

4. Clarification of orthophosphoric acid

The acid with the sulphide (As2S3, PbS) flakes flows spontaneously from the precipitation column on an overflow line in a clarification tank. The acid clarification is fulfilled by activated carbon, which is manually fed in quantity of 209 g per 1 tonne of the acid. For breakup of the arsenic compound layer formed on the acid surface the compressed air bubbling is used. Waste gases are removed from the apparatus by means of a fan. Vacuum in the column is regulated by a manometre (140-160 mm of water).

5. Filtration of orthophosphoric acid for removal of arsenic and lead sulphides

The clarified acid from the tank on the top overflow line spontaneously flows on a filter, which represents the grids covered by two layers of filter fabric “Lavsan” (a code number 86033).

At the moment of activation of a reserve filter for application of a filtering layer a line of double filtration through a working filter is provided.

6. Desorption of hydrogen sulphide from the acid

The filtrate from the filter flows by gravity in a bubble desorber. Removal of H2S from the acid is realized by the compressed air submitted in the desorber from a main pipeline. Before the feeding in the desorber the compressed air is purified from mechanical impurity on a kid-filter and heated up in a heat exchanger to temperature of 70-800С. Automatic blocking of the compressed air feed is provided for the case of overrunning the pressure in the kid-filter more 4 kgf/cm2.

After the desorber the acid spontaneously flows in a commodity phosphoric acid tank, and then pumped by a pump in the storage tanks of chemically pure phosphoric acid production and packing sections. The acid from the commodity acid tank is subjected to chemical analysis for determination of its quality.

7. Air purification from acid splashes

The air after the hydrogen sulphide removal from the acid is given from the desorber through a mist eliminator to a bubble absorber. The absorber is irrigated by a commercial-grade phosphoric acid from a circulating tank, which is fed by a pump through a pressure tank.

The purified air from the absorber is given by a fan in a liquid trap, and then discharged into the atmosphere.

8. Packing and warehousing of the ready product

Depending on its quality the acid is fed in storage tanks of ready product, from which it flows by gravity on packaging in 20-litre bottles or the polyethylene canisters established on pallets, and then is transported to a finished goods warehouse by an electric loader.

Application field

Reactive phosphoric acid of “p” and “p.f.a.” grades is initial raw material for manufacturing chemically pure (“c.p.”), meta- and pyrophosphoric, food orthophosphoric acids and phosphoric acid for electronic industry.

It is applied for carrying out laboratory and special works, to medical purposes.

Production wastes

At the reactive phosphoric acid manufacture of “p” and “p.f.a.” grades three kinds of industrial wastes are formed: gaseous, liquid and solid. Waste gases are formed in the desorber, precipitation column, and also in the tanks. For the environment protection the waste gases are subjected to purification. The air outgoing from the desorber and precipitation column and containing hydrogen sulphide and acid traces, and also the exhaust air from all the tanks is run through liquid traps. Other characteristics of emissions in the atmosphere are represented in table 3.

Waste water in the given production is formed as a result of washout of a deposit from the filter. It contains insoluble harmful substances (PbS, As2S3). The waste water treatment is carried out on a filter-press. The deposit formed (PbS, As2S3) is given to a waste burial place, and the filtrate is discharged in a tank, from which it is pumped to the dry-process phosphoric acid department for dilution of dry-process phosphoric acid. The characteristic of liquid and solid wastes is represented in table 4.

Labor safety

The considered manufacture concerns to fire-safe and explosion-resistant productions except for the precipitation column placement location.

This manufacture is connected with application and formation of 2nd danger type harmful substances (orthophosphoric acid, sulfureous sodium).

The orthophosphoric acid manufacture is characterized by the following points dangerous for operating stuff:

- toxic action of phosphoric acid, sulfureous sodium, hydrogen sulphide, the slime formed at the acid filtration;

- apparatuses operating under pressure (a filter-press).

For protection of the personnel from the poisonous effect it is necessary strictly to observe the operating regime and safety regulations.

Protective individual equipment and first medical aid at affection by harmful substances:

Phosphoric acid. At acid burn it is necessary to wash out carefully the affected area by water and to moisten with a 3-5% potassium permanganate solution.

At acid ingress in eyes it is necessary to wash out them by strong sheet of water and to consult a doctor.

Individual protection: protection of respiratory apparatus and skin. Operating stuff should be provided by filtering gas masks, respirators, hermetic safety goggles, rubber gloves according to requirements of State Standards 12.4.028-76, 12.4.121-83.

In case of acid pouring it is necessary to neutralize it by lime baring from splashing and to wash away by water.

Sulfureous sodium. Careful skin protection. It is recommended to wash hands periodically with a weak acetic acid solution and to apply fatty cream.

Protection of eyes. At work with Na2S it is necessary to apply respirators, goggles, rubber gloves.

Hydrogen sulphide. Emergency treatment at affection by hydrogen sulphide is fresh air. It is recommended to take off tight clothes, to use rest, heat, oxygen inhalations, at respiratory impairment – long artificial oxygen ventilation. Chlorine inhalation can be used

In easy cases, at irritation of upper airways it is recommended to use heat, humid alkaline inhalations, and heated milk with soda, codeine or dionin.

At eye affection: a dark premise, cool lotions with 3% Н3ВО3, application of vaseline or an indifferent ointment, and also novocaine with adrenaline.

Individual protection: operating stuff should observe measures of personal hygiene.

Table 3.  Atmospheric emissions

| An emission type, an apparatus, diameter and height of emission | Emission source quantity | Total volume of waste gases | Periodicity | Emission characteristic |  | Permissible quantity of standardized harmful substances discharged into the anmosphere | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Temperature | Composition |  |  |
| Air containing phosphoric acid drops, hydrogen sulphide Gas pipe Х246 Height 10 m Diameter 350 mm | 1 | No more than 1677 km3/h | Permanently | 30-400С | Н2S=0,0095-0,096 g/m3 Н2О = 2,8gг/m3 | H2S = 0,16-0,17 kg/h Н2О = 4,53-4,7 kg/h |  |

Table 4. Solid and liquid wastes

| Name of a waste product, a department, an apparatus | Bank, transport, paskage | Waste product quantity | Formation periodicity | Solid and liquid waste characteristic |  | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Chemical composition and humidity | Physical parameters |  |
| Solid slime formed at filtration of “p” and “pfa” phosphoric acid | Is fed in a waste burial place | No more than 30,52 kg/day | Periodically | Sulphur compounds – 9,78 %, including: As – 2,23 % Р3PO4 – 65,2-65,5 % H2O – 18,2-18,5 % Carbon – 5,6 – 5,7 % |  | Paste-like yellow-green mass with black inclusions |
| Liquid wastes of “p” and “pfa” phosphoric acid manufacture | Is fed along a conduit to a dry-process phosphoric acid department, then to manufacturing sodium tripolyphosphate | No more than 1680 kg/day | Periodically | H3PO4 – 67,8% As – 0,0045% H2O – 32,5-32,8 % | =1,49 kg/m3 at 250С |  |

Table 5. Waste water

| Name of a drain, a department, an apparatus | Discharge point | Waste water volume | Discharge periodicity | Waste water characteristic |  | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Chemical composition | Permissible quantity of discharged harmful substances |  |
| Acid drains after washing floor, pumps and other washing water | Is fed to a dry-process phosphoric acid department for acid dilution | No more 20 m3/day | Periodically | Acid drains, Content trace H3PO4, no more 67200 mg/l, = 1,12 g/cm3 | 23400 kg/day, including Н3РО4 – 1357 kg/day Н2О – 22043 kg/day |  |

Table 6. Capital equipment specification

| A position number according the circuit | Equipment name | Quantity | A material, a safeguarding method | Technical characteristic |
| --- | --- | --- | --- | --- |
| Е 200 | A distilled water tank | 1 | Steel 10Х17Н13М2Т | Horizontal flat bottom apparatus. Volume 1,5 m3. Length 1500 m. Height 1000 mm. Width 1000 mm. |
| Е 202 | A receiver tank | 2 | Steel 08Х21Н6М2Т | ВПП1-4-10-0. Vertical all-welded flat-bottom coil apparatus. Volume 103. Diameter 2200 mm. Height 2900 mm. The coil area F=2,9 m2. |
| Е 203 | A pressure tank | 3 | Steel 10Х17Н13М2Т | МГ-6/0,63. Vertical jacket-free batchbox. Volume 0,63 m3. Diameter 800 mm. Height 1255 mm. Length 1500 mm. |
| Е 210 | A sodium sulphide dissolver | 3 | Steel 12Х18Н10Т | МГ-6/0,25. Capacitive apparatus. Volume 0,25 m3. Diameter 700 mm. Height 1155 mm. Length 1125 mm. Volume 0,63 m3. Vacuum 160 mm H2O. |
| Е 211 | A precipitation column | 6 | Steel 08Х21Н6М2Т | The apparatus is equipped with a grate for Rashig ring packing and a spraying atomizer. Boiler diameter 750 mm. Total height 3570 mm. |
| Е 212 | An intermediate tank | 3 | Steel 08Х21Н6М2Т Thermal insulation | 8033-6. 30-СА-41. Flat bottom apparatus with detachable flat cap. Volume 6,3 m3. Diameter 1800 mm. Height 2825 mm. The apparatus has an agitator and a coil (F=5,5 m2). Gear motor drive МПО1-15-7,5-5,5/129 АО2-51-6. |
| Н 213 | A centrifugal pump | 6 | A flow part is made from steel 10Х17Н13М2Т | АХ 20/53-Е-СD. Efficiency 20 m3/h. Pressure 53 m liquid column. Electric motor 4А-160 № 15 kW, n = 2900 turn/min. Dimension, mm: 1210х360х510. |
| Ф 214 | A two-filter installation | 3 | Steel 10Х17Н13М2Т | ЛВАЖ-20к. Efficiency of one a filter in terms of filtrate – 7 m3/h. Two vertical filters. Volume of each the filter – 2,1 m3. Filtration area – 20 m3. Filtering plate quantity – 12 pieces. |
| Е 216 | An acid tank | 3 | Steel 08Х21Н6М2Т | ВПП1-4-10-0. Vertical all-welded flat-bottom coil apparatus. Volume – 10 m3. Diameter – 2400 mm. Height – 2300 mm. The coil area F=3 m2. |
| Е 217 | A centrifugal pump | 6 | A flow part is made from steel 10Х17Н13М2Т | Х20/53-Е-СD. Efficiency – 20 m3/h. Pressure – 53 m liquid column. Electric motor 4А132М2. N – 18,5 kW, n – 2900 turn/min. Dimension, mm: 1130х340х438. |
| Е 201 | A distilled water tank | 1 | Steel 12Х18Н10Т | Vertical apparatus with elliptic cap and bottom. V – 1,5 m3. Height – 1000 mm. |
| Е 218 | A suspension storage tank | 2 |  | СЭРИ-6,3-12/р10. Steel enameled reactor. Volume – 6,3 m3. Diameter – 1800 mm. Height – 2925 mm. The apparatus is equipped with an agitator and a discharge device, n = 45 turn/min. |
| Н219 | A centrifugal pump | 4 | A flow part is made from steel 12Х18Н9Т | АХ 20/53-К-CD. Efficiency – 20 m3/h. Pressure – 53 m liquid column. Electric motor 4А16052. N-18,5 kW, n=2900 turn/min. Dimension, mm: 1210х360х510. |
| Ф 220 | A filter press | 3 | Сборка, Steel 08Х22Н6Т | Ф-П-М-16-630/45К. Filtering area – 16 m2. Frame quantity – 20 pieces. Frame space volume – 0,357 m3. Electric motor 4А100 4УЗ. N = 3 kW, n = 1430 turn/min. Dimension, mm: 3420х1337х2795. |
| Е 221 | A clarified water tank | 1 |  | СЭН-6,3-31-12 /10/. Steel enameled tank. Volume – 6,3 m3. Diameter – 1800 mm. Height – 2950 mm. |
| Н 222 | A centrifugal pump | 2 | A flow part is made from steel ????? | Х20/53-К-СО. Efficiency – 20 m3/h. Pressure 53 m liquid column. Electric motor 4А 132М2. N = 16 kW, n = 2900 turn/min. Dimension, mm: 1130х340х438. |
| Ф 224 | A filter “Laik” | 1 | Assembly, steel 3 | Filtering area F = 12,3 m2. 400 ТНГ-16-М1-0/25-2-1 А group. Single-pass, horizontal. Diameter – 426 mm. Length – 2760 mm. |
| Т 225 | A shell-and-tube heat exchanger with stationary tube plates | 1 | Assembly Thermal insulation | Pipe diameter – 25 mm. Pipe length – 2000 m, F = 23 m2. Dimension, mm: 2930х576. |
| С 226 | A rectangular-section foam desorber | 3 | Steel 10Х17Н1342Т | An apparatus is provided for perforated pipes with d = 57 mm. Apparatus section – 1000х500 mm. Total height – 1500 mm. |
| Е 228 | A phosphoric acid clarifier | 3 | Steel 08Х21Н6М2Т Thermal insulation | ВПП1-4-10-0. Vertical all-welded flat-bottom coil apparatus. Volume – 10 m3. Diameter – 2400 mm. Height – 2300 mm. Coil area F = 3 m2. |
| С 232 | A liquid trap | 3 | Steel 08Х21Н6М2Т | Air efficiency – 3500 m3/h. Pressure – 50 mm H2O. Vacuum – 50 mm H2O. Diameter – 630 mm. Total height – 2050 mm. |
| В 233 | A high-pressure centrifugal fan | 6 | Steel 12Х18Н10Т | ВВД-8К-02. Efficiency – 7000 m3/h. Pressure – 600 mm H2O. Fulfillment 2ПРО0 3 pieces; ЛО0 3 pieces; n = 1900 turn/min. Electric motor АО-2-71. N = 285 kW, n = 1500 turn/min. Dimension, mm: 721х1232. |
| Н235/1,2 | A weight pump НD1 600 | 2 | Lined with polyethylene | Efficiency – 1,6 m3/h. Pressure – 10 mm H2O. Electric motor N = 5 kW, n = 2900 turn/min. |
| Е 238 | A circulating tank | 3 | Steel 10Х17Н13М2Т | ВЭЭ 1-3-3,2-0,6. Vertical all-welded elliptic-bottom apparatus. Volume – 2,2 m3. Height – 2615 mm. Diameter – 1400 mm. |
| Н 239 | A centrifugal pump | 6 | A flow part is made from steel 10Х17Н13М2Т | ХРО153-Е-СD. Efficiency – 20 m3/h. Pressure – 53 m liquid column. Electric motor 4А132М2. N = 185 kW, n = 2900 turn/min. Dimension, mm: 1130х340х438. |

Capital equipment

A precipitation column. It is intended for sedimentation of arsenic and lead impurities from technical orthophosphoric acid by a 3-5% sulfureous sodium solution as sulphides. It consists of top cylindrical and bottom boiler parts. The acid is fed in the top part of the column through a sprayer and flows down along a packing (Rashig rings) in the bottom part where a Na2S solution is batched. There is their interaction and the hydrogen sulphide liberated as a result of the reaction rises upwards countercurrent to the acid and reacts with it. There is sedimentation of the impurities as yellow-green flakes.

A filter. It is intended for separation of the precipitated impurities from the acid; consists of two vertical filters, which are equipped with grids covered by two layers of a filtering fabric “Lavsan”. Each the filter has 12 vertical grids. The polluted acid passes consecutively all the filtering grids; the impurities are deposited on the fabric.

A bubble desorber. It is intended for removal of hydrogen sulphide from the acid. The blowing is fulfilled by compressed air. The apparatus is supplied by punched tubes and metal partitions. Compressed air is fed through the tubes; the acid is given by reverse flow. The hydrogen sulphide removal occurs during moving the acid through the partitions.

Productivity, principal dimensions and other parameters are represented in table 6.

Auxiliary equipment

A mist eliminator. It is intended for prevention of splashes entrainment. The caught acid drops flow down in an intermediate tank.

A plate-and-frame filter press. It is intended for wringing of slime after the filter. The filter press represents a set of frames covered by a filtering cloth. The frames are pressed by means of an electric motor and the slime is wrung.

A filter. It is intended for purification of the compressed air fed in the desorber from mechanical impurities.

Productivity, principal dimensions and other parameters are represented in table 6.

3.2MANUFACTURE OF CHEMICALLY PURE ORTHOPHOSPHORIC ACID

Orthophosphoric acid:

empirical formula Н3РО4                                ОН

structural formula:      H – О = Р            ОН

molecular weight 98,00 g/mol                ОН

Properties: colourless rhombic-system crystals with density of 1,834 g/cm3. At 42,350С it melts in a colourless odorless syrup-like liquid with density of 1,88 g/cm3. Liquid acid is rather inclined to overcooling. Well soluble in water and ethyl alcohol. It crystallizes from water solutions with density of 1,75-1,85 g/cm3; the crystallization is realized easier at addition of seed. At heating above 1500С phosphoric acid starts to lose bound water (not having reached 100% of Н3РО4), turns into Н4Р2О7 and then into НРО3. Н3РО4 is an average acid, its dissociation constant at 250С are: К1 = 7,610-3, К2 = 6,2 10-8, К3 = 4,410-13. Boiling point of 87 % orthophosphoric acid is 159,40С.

Characteristic of initial raw material, auxiliary materials and finished goods

Orthophosphoric “c.p.” acid is produced of “p.” and “p.f.a.” orthophosphoric acid.

The characteristic of initial raw materials, auxiliary materials and semiproducts is represented in table 1.

Table 1 – Characteristic of initial raw materials, auxiliary materials and semiproducts for orthophosphoric acid production

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | “p” | “p.f.a.” |
| 1 | “p” and “p.f.a.” orthophosphoric acid | State Standard 6552-80 | 1. Mass fraction of orthophosphoric acid Н3РО4, no less, % 2. Density, not less, g/cm3 3. Mass fraction, no more, % - nitrates (NO3) - sulphates (SO4) - chlorides (Cl) - iron (Fe) - heavy metals (Pb) - arsenic (Аs) - volatile acids - ammonium salts - residue after calcination - substances reducing KMnO4 | 85 1,69 0,0005 0,003 0,003 0,002 0,001 0,0002 0,0015 0,002 0,2 0,05 | 85 1,69 0,0005 0,002 0,0002 0,0015 0,0005 0,0001 0,001 0,002 0,1 0,005 |
| 2 | Distilled water | State Standard 6709-72 | 1. Residue after evaporation, mg/l, no more 2.Residue after calcination, mg/l, no more 3. Content, mg/l: - ammonia and ammonium salts (NH4), no more, - nitrates (NO3), no more - sulphates (SO3), no more - chlorides (Cl), no more - aluminum (Аl), no more - iron (Fe), no more - calcium (Са), no more - copper (Cu), no more - lead (Pb), no more - zinc (Zn), no more - the substances reducing КМnO4 (О), mg/l - pH | 5,0 1,0 0,02 0,2 0,5 0,02 0,05 0,05 0,8 0,02 0,05 0,2 0,08 5,4-6,6 |  |

Characteristic of “chemically pure” orthophosphoric acid according to the requirements of State Standard 6552-80 is represented in table 2.

Table 2 – Requirements of State Standard 6552-80 to chemically pure orthophosphoric acid quality

| № | Parameter name | Standard |
| --- | --- | --- |
|  |  | “c.p.” |
| 1 | Appearance and color | Should stand the test according to the item 4.2 of State Standard 6552-80 |
| 2 | Mass fraction of orthophosphoric acid Н3РО4, not less, % | 87 |
| 3 | Mass fraction of the residue after calcination, no more, % | 0,05 |
| 4 | Mass fraction, no more, % - volatile acids | 0,001 |
|  | - nitrates (NO3) | 0,0005 |
|  | - sulphates (SO4) | 0,0005 |
|  | - chlorides (Cl) | 0,0002 |
|  | - ammonium salts | 0,0005 |
|  | - iron (Fe) | 0,0005 |
|  | - arsenic (Аs) | 0,0001 |
|  | - heavy metals (Pb) | 0,0005 |
|  | - the substances reducing КМnО4 (H3PO4) | 0,003 |

Manufacture of chemically pure orthophosphoric acid is realized in two crystallizer-classifiers with output of each the crystallizer in term of initial acid is 1,57 t/h.

A production technology of chemically pure phosphoric acid is based on recrystallization of a “p” and “p.f.a.” acid mixture with the subsequent wringing and dissolution of crystals in distilled water.

The production process consists of following stages:

1) mixing of “p” and “p.f.a.” acids;

2) crystallization of the phosphoric acid mixture;

3) condensation and centrifugation of crystals;

4) dissolution of the crystals with obtaining “c.p.” orthophosphoric acid.

Technological features of the process. Crystallization is realized in a continuously working apparatus in ascending stream. Cooling the suspension formed is performed in a portable refrigerator. The given apparatus is an experimental-industrial one and all operating indicators should be specified at its service.

A manufacturing diagram. The phosphoric “p” acid from a tank is pumped in a pressure tank from which it is batched in a mixer supplied with an agitator. The “p.f.a.” phosphoric acid from a receiving tank is also pumped through a pressure tank in the mixer. The acid ratio “p”:”p.f.a.” makes 1:2. The obtained acid mixture is pumped to crystallization.

The orthophosphoric acid crystallization is realized in a crystallizer-classifier with suspended crystal layer. In addition, a batch crystallizer equipped with a gate agitator, a jacket and a coil is provided as a reserve apparatus.

Cooling is carried out by chilled water with temperature of 50С. A crystallizer represents a cylindrical apparatus with a central pipe for giving of a solution from an external flow circuit.

A crystalline product is formed at the continuous isothermal crystallization of the acid mixture on the suspended crystal layer. The mother solution from the expanded top part of the crystallizer is fed in the flow circuit and passes through the shell-and-tube heat exchanger, heated with steam, for dissolution of the fines entrained from the crystallizer. The solution temperature in the heat exchanger increases on 20С (to 180С).

Before pumps the circulating mother solution is mixed with the initial concentrated solution, which quantity is in 30 times less than quantity of the circulating mother solution.

Temperature and concentration of the solution after the mixing increases slightly and the solution is close to saturation.

Flowing along the tubes of three tandem heat exchangers, cooled by chilled water, the solution is cooled up to 160С and become supersaturated.

The supersaturated solution along the central pipe is fed into the bottom part of the crystallizer and rises upwards as the ascending stream supporting the growing crystals in a suspended state; at the same time oversaturation of the incoming solution is removed on the suspended crystal layer.

The mother solution from the top part of the crystallizer case is again given in the flow circuit and the process is repeated.

The ready pulp is periodically unloaded from the bottom part of the crystallizer and fed by means of an airlift through a separation vessel in thickeners.

The solution overflow from the crystallizers is fed in thickeners, and drainage – in thickeners or in an off-grade acid tank.

The thickened pulp from the thickeners is given in centrifuges for separation. The clarified acid from the thickeners, and also the mother solution after the centrifuges in a case of crystal overshoot, are given in a mother solution collecting tank from which it is pumped on repeated extraction of crystals to the thickener. To avoid dissolution of the acid crystals and to improve the ratio of solid:liquid the thickeners are supplied by jackets for cooling with chilled water.

A drain from the thickener and the mother crystal-free solution after the centrifuges are given in a mother solution receiver from which pumped to a warehouse of the dry-process phosphoric acid shop,  and then – for manufacturing sodium tripolyphosphate.

The phosphoric acid crystals from the centrifuges are fed on melting and dissolution in tanks-dissolvers; distilled water from pressure tanks is also given in the dissolvers.

The acid obtained has concentration of 85-87% of Н3РО4 and represents the commodity chemically pure acid, which is pumped over in a commodity acid tank and then on packing.

3.3MANUFACTURE OF META- and PYROPHOSPHORIC ACIDS

Metaphosphoric acid

Formula – НРО3

Molecular weight – 79,98

Properties: colourless transparent plates or sticks running in air; become matte with the lapse of time; water-soluble; fixing water it gradually turns into Н3РО4 (the transformation occurs faster in a case of heating with НNO3). It is sublimated at red heat temperature. The preparation consists of a mix of metaphosphoric acid and its sodium salt (a stabilizer).

Pyrophosphoric acid

Formula – Н4Р2О7

Molecular weight – 177,97

Properties: colourless vitreous mass. Melting point is 610С. Easily soluble in water (87,6% at 230С). A pyrophosphoric acid solution is sufficiently stable on the cold; at boiling with HNO3 it very quickly turns into Н3РО4. It is a four-basic acid; its dissociation constants at 250С: К1 = 310-2; К2 = 4,410-3; К3 = 2,510-7; К4 = 5,610-10. The acid forms salts of two types: М4Р2О7 and М2Н2Р2О7 (where M – a monovalent metal). Salts of Н4Р2О7 are poisonous. Н4Р2О7 is inclined to polymerization up to formation of (Н4Р2О7)5.

Characteristic of initial raw material, auxiliary materials and finished goods

Initial raw material for manufacturing meta- and pyrophosphoric acids is reactive orthophosphoric acid (“p.f.a.”) and sodium hydroxide (“p”).

The characteristic of initial raw materials, auxiliary materials and semiproducts is represented in table 1.

Table 1 – Characteristic of initial raw materials, auxiliary materials and semiproducts for meta- and pyrophosphoric acids production

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |
| --- | --- | --- | --- | --- |
| 1 | “p.f.a.” orthophosphoric acid | State Standard 6552-80 | 1. Mass fraction of orthophosphoric acid Н3РО4, no less, % 2. Density, not less, g/cm3 3. Mass fraction, no more, % - nitrates (NO3) - sulphates (SO4) - chlorides (Cl) - iron (Fe) - heavy metals (Pb) - arsenic (Аs) - volatile acids - ammonium salts - residue after calcination - substances reducing KMnO4 | 85 1,69 0,0005 0,002 0,0002 0,0015 0,0005 0,0001 0,001 0,002 0,1 0,005 |
| 2 | Sodium hydroxide “p” | State Standard 4328-77 | Mass fraction, %: - NaOH, not less - Na2CO3, no more - N, no more - SiO2, no more - SO4, no more - PO4, no more - Cl, no more - Al, no more - Fe, no more | 97 1,5 0,001 0,02 0,02 0,01 0,025 0,01 0,002 |

Characteristic of pyrophosphoric acid according to the requirements of State Standard 5653-75 is represented in table 2.

Table 2 – Requirements of State Standard 5653-75 to pyrophosphoric acid quality

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |
| --- | --- | --- | --- | --- |
|  |  |  |  | “p.f.a.” |
| 1 | Pyrophosphoric acid | State Standard 5653-75 | 1. Mass fraction of Н4Р2О7, no less, % Mass fraction, no more, % - chlorides (Cl) - iron (Fe) | 60 0,002 0,003 |

Chemical indices of metaphosphoric acid should correspond to requirements of State Standard 841-76 (table 2').

Table 2' – Requirements of State Standard 841-76 to metaphosphoric acid quality

| № | Parameter name | Standard |
| --- | --- | --- |
|  |  | “p” |
| 1 | Mass fraction of metaphosphoric acid НРО3, not less, % | 60 |
| 2 | Sodium salt of metaphosphoric acid (NaPO3), no more, % | 40 |
| 3 | Mass fraction, no more, % - nitrates (NO3) | 0,0005 |
|  | - sulphates (SO4) | 0,001 |
|  | - chlorides (Cl) | 0,002 |
|  | - iron (Fe) | 0,005 |
|  | - arsenic (Аs) | 0,0002 |
|  | - heavy metals (Pb) | 0,0005 |
|  | - the substances reducing КМnО4 (H3PO4) | Should stand the test according to the item 3.10 of State Standard 841-76 |

A pyrophosphoric acid production line includes four graphite electric furnaces with productivity of 0,5 kg/h.

A metaphosphoric acid production line includes four graphite electric furnaces with productivity of 3 kg/h and four quartz electric furnaces with productivity of 2 kg/h.

A method of production

A production technology of pyrophosphoric acid is based on dehydration of reactive 85% orthophosphoric acid in graphite furnaces at temperature of 250-2600С.

A technology of metaphosphoric acid manufacture is based on dehydration of an orthophosphoric acid and sodium orthophosphate mixture at temperature of 420-5000С in graphite and quartz electric furnaces.

Technological features of the process. In the manufacture of pyro- and metaphosphoric acids improved quartz and graphite furnaces are applied.

A manufacturing circuit

1. Pyrophosphoric acid. Initial raw material for manufacturing pyrophosphoric acid is reactive phosphoric acid. The reactive phosphoric acid from a batchbox by gravity is fed in an initial raw material tank, and then in a graphite furnace for the dehydration. At temperature of 255-2600С the orthophosphoric acid is dehydrated and turns into pyrophosphoric one. The pyrophosphoric acid is collected in a tank and then filtered on a nutsch filter.

The filtered acid by means of vacuum is given in a receiver, from which it is packed in small containers.

2. Metaphosphoric acid. The reactive phosphoric acid, which is the initial raw material for manufacture of metaphosphoric acid, from a batchbox by gravity is fed in a neutralizer. Solid sodium hydroxide is manually added in the neutralizer with small batches at continuous hashing.

A solution of the orthophosphoric acid and sodium orthophosphate mixture with temperature of 68-720С along a heated conduit through a feeding pipe is spontaneously fed in a graphite electric furnace, where the orthophosphoric acid is dehydrated and turns into pyrophosphoric acid and acid sodium pyrophosphate.

The acid-salt mix obtained is continuously given in quartz electric furnaces, where their further dehydration occurs with formation of metaphosphoric acid and sodium metaphosphate.

The mixture of metaphosphoric acid and sodium metaphosphate as a vitriform fusion flows down on the trays installed in a water cooler.

The vitriform metaphosphoric acid and sodium metaphosphate mixture cracks at sharp cooling. The pieces formed are immediately packed into glass jars.

3.4MANUFACTURING PHOSPHOROUS ACID

A chemical formula – Н3РО3

Molecular weight – 81,99

Properties: colourless crystals running in air. Density is 1,651 g/cm3. Melting point is 73,60С; above 2000С the acid is decomposed:

4Н3РО3 = 3Н3РО4 + РН3 

Easily soluble in water (75,6 % at 00С).

Phosphorous acid Н3РО3 is a dibasic acid; its dissociation constants at 250С are: К1=1,610-2, К2=210-7. It is a strong reducer displacing free metals from Аg+ and Hg2+ salts.

Characteristic of initial raw materials, auxiliary materials and finished goods

Phosphorous acid is produced of phosphorus chloride.

The characteristic of initial raw materials, auxiliary materials and semiproducts is represented in table 1.

Table 1 – Characteristic of initial raw materials, auxiliary materials and semiproducts for phosphorous acid production

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |
| --- | --- | --- | --- | --- |
| 1 | Trichlorophosphorus | Technical specifications 6-02-574-70 | 1. Density at 200C, g/cm3 2. Mass fraction of trichlorophosphorus, not less, % 3. Mass fraction of P (V), no more, % 4. Mass fraction of free phosphorus, no more, % 5. Distillation temperature limits at pressure of 760 mm Hg, at which not less than 98% (vol) should be distilled, 0C | 1,574-1,585 98 0,45 absence 2,5 |
| 2 | Distilled water | State Standard 6709-72 | 1. Residue after evaporation, mg/l, no more 2.Residue after calcination, mg/l, no more 3. Content, mg/l: - ammonia and ammonium salts (NH4), no more, - nitrates (NO3), no more - sulphates (SO3), no more - chlorides (Cl), no more - aluminum (Аl), no more - iron (Fe), no more - calcium (Са), no more - copper (Cu), no more - lead (Pb), no more - zinc (Zn), no more - the substances reducing КМnO4 (О), mg/l - pH | 5,0 1,0 0,02 0,2 0,5 0,02 0,05 0,05 0,8 0,02 0,05 0,2 0,08 5,4-6,6 |

Characteristic of “pure” phosphorous acid according to requirements of Technical Specifications 6-09-4023-75 is represented in table 2.

Table 2

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |
| --- | --- | --- | --- | --- |
| 1 | Phosphorous acid of a “p” grade | Technical specifications 6-09-4023-75 | 1. H3PO3 content, %, not less 2. Melting temperature, 0C 3. Water-insoluble substances, %, no more 4. Chlorides, %, no more 5. Iron, %, no more 6. Orthophosphoric acid, %, no more | 98 72-74 0,005 0,003 0,005 2,0 |
| 2 | Technical phosphorous acid |  | H3PO3 content, %, not less | 98 |
| 3 | Hydrochloric acid | Technical specifications 6-01-1194-79 | HCl content, % | 28-29 |

Experimental-industrial installations of manufacturing phosphorous and hypophosphorous acids, purification of phosphoric acid from arsenic and heavy metals

The trial installation capacity is determined by need of the national economy for phosphorous, hypophosphorous and high-pure phosphoric acids.

The installations include reactors with mixing devices for carrying out decomposition of barium hypophosphite by sulfuric acid and hydrolysis of trichlorophosphorus (volume from 160 l to 300 l).

Separation of a phosphorous acid suspension is realized on a centrifuge FMR 633 K-2 (2 pieces), drying of the phosphorous acid crystals is fulfilled in a vacuum-dryer (2 pieces).

A method of production

A phosphorous acid production technology includes hydrolysis of preliminary purified technical trichlorophosphorus with the subsequent evaporation of the solution obtained, crystallization, centrifugation and drying a ready product.

Technological features of the production process. The phosphorous acid, which quality corresponds to the quality of the phosphorous acid used for manufacturing polyethylene terephthalate in the CIS is not produced. There are some ways of phosphorous acid production, but the most widespread technique is trichlorophosphorus hydrolysis.

An advantage of the given technology consists in obtaining a high-quality product with minimum content of Н3РО4 in phosphorous acid.

A manufacturing circuit of phosphorous acid production

Phosphorous acid is produced by hydrolysis of trichlorophosphorus.

Technical trichlorophosphorus is fed in a receiving tank for PСl3 storage, from which it is pumped in a pressure tank. The receiving tank and the pressure tank are preliminary blown by nitrogen.

Trichlorophosphorus from the pressure tank is given in a heat exchanger and heated up to 740С. After that PСl3 is fed in a rectification column filled with steel packing.

A part of the trichlorophosphorus flows down as a liquid in a boiler, in which it is again evaporated.

The vapors of purified PСl3 are condensed in a heat exchanger. The liquid PСl3 is collected in a tank, and then fed in the hydrolysis reactors preliminary filled with distilled water. The reactors before the loading are blown by nitrogen.

Hydrolysis is carried out at temperature of 260С within 3-4 hours; after termination of the hydrolysis water and hydrochloric acid vapors are distilled in vacuum and caught in a packed absorber and a sanitary tower for the purpose of obtaining a commercial hydrochloric acid.

After finishing the distillation the phosphorous acid with the water and НСl residue is fed in another vacuum distillation apparatus for final distillation at temperature of 80-900С to the acid concentration of 97-98%. The melt obtained is crystallized in a crystallizer. The crystals formed are separated on a centrifuge, dried in a vacuum dryer and packed. The distillation residue, formed as the result of PСl3 rectification, is collected in a tank and then processed into phosphorous acid. In accordance with the technology described above the mother solution, formed after the centrifugation, is applied in the process.

The gas from the reactors with temperature of 70-800С containing water vapor splashes of phosphorous acid and hydrogen chloride is fed in a packed absorber and a sanitary tower (a hollow absorber irrigated with water). The weak hydrochloric acid (4-5 %) formed in the tower is given in a tank, from which pumped over in the absorber for irrigation and absorption of hydrogen chloride.

The commercial-grade hydrochloric acid (28-29% of НСl) from the tank is pumped over in a packing section.

The gas purified is sucked away by vacuum pumps and thrown out in the atmosphere.

3.5MANUFACTURE OF HIGH-PURE PHOSPHORIC ACID

(An experimental-industrial installation for purification of phosphoric acid from arsenic and heavy metals)

Characteristic of initial raw material, auxiliary materials and finished goods

Initial raw material for manufacturing a high-pure phosphoric acid is a dry-process phosphoric acid and di-2-ethylhexyl-dithiophosphoric acid.

The characteristic of initial raw materials, auxiliary materials and semiproducts is represented in table 1.

Table 1 – Characteristic of initial raw materials, auxiliary materials and semiproducts for high-pure phosphoric acids production

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |
| --- | --- | --- | --- | --- |
| 1 | Dry-process phosphoric acid | According to initial data of НИЧ КНГФ | Content, % 1. Н3РО4, no less 2. Chlorides, no more 3. Sulphates, no more 4. Heavy metals (Pb), no more 5. Arsenic (Аs), no more 6. Insoluble Ca, Mg, NH4OH in a precipitate, no more 7. Manganese (Mn), no more 8. Potassium (K), no more 9. Sodium (Na), no more P.S. Н3РО4 should not processed with KMnO4 for oxidation of unoxidized phosphorus compound | 86 0,0003 0,003 0,01 0,01 0,005 0,00005 0,005 0,025 |
| 2 | Di-2-ethylhexyl-dithiophosphoric acid | Technical specifications 6-02-3-161-78 | 1. Density, g/cm3, not less 2. Content of di-2-ethylhexyl-dithiophosphoric acid, %, not less | 0,96 80 |
| 3 | Decane | Technical specifications 6-09-3614-74 | 1. Chromaticity, mg of iodine per 100 ml, no more 2. Content of decane (C10H22), %, not less 3. Boiling temperature (at 760 mm Hg), 0C 4. Content of unsaturated hydrocarbons | 1,0 98 173,6-174,6 Test according to the item 4.4 of Technical specifications 6-09-3614-74 |
| 4 | 2-ethylhexanol | Technical specifications 6-05-961-73 | 1. Colour according to a platinum-cobalt scale, not darker 2. Density, g/cm3, in limits 3. Content of aldehydes, % 4. Content of unsaturated compounds, %, no more 5. Acid number of a product, mg KOH/2, no more 6. Boiling temperature limits at 760 mm Hg, 0C a) overpoint, not less b) end boiling point, no more It should be distilled in the specified limits, ml, not less 7. Hydroxyl number of a product, mg KOH/2, not less | № 10 0,830-0,834 Absence 0,02 0,02 181 185 97 426 4 |
| 5 | Technical water ammonia, “B” grade (for industry, 1 sort) | State Standard 9-77 | 1. Mass fraction of ammonia, %, not less 2. Mass concentration of carbon dioxide (CO2), g/l, no more 3. Mass concentration of fixed residue, g/l, no more 4. Mass concentration of copper (Cu), g/l, no more | 25 unstandardized 0,2 unstandardized |
| 6 | Hydrochloric acid | State Standard 3118-77 | 1. Mass fraction of hydrochloric acid (HCl), % 2. Residue after calcination (as sulphates), %, no more 3. Sulphites (SO3), %, no more 4. Sulphates (SO4), %, no more 5. Free chlorine (Cl), %, no more 6. Ammonium salts (NH4), %, no more 7. Iron (Fe), %, no more 8. Arsenic (Аs), %, no more 9. Heavy metals (Pb), %, no more | 35-38 0,001 0,001 0,0005 0,00005 0,0003 0,0001 0,00001 0,0001 |
| 7 | Sulphureous sodium reactive, “p” or “p.f.a.” | State Standard 2053-77 | Mass fraction of Na2S∙9H2O, %, not less: “p” “p.f.a.” | 93 97 |
| 8 | Activated wood crushed carbon | State Standard 6217-74 | 1. Adsorption activity according to iodine, %, not less 2. Total porosity according to water, cm3/g, not less | 50 1,5 |

Characteristic of high-pure phosphoric acid in accordance with requirements of a plant standard is represented in table 2.

Table 2

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |
| --- | --- | --- | --- | --- |
| 1 | High-pure phosphoric acid | Plant standard (according to initial data of НИЧ КНГФ) | Content, % 1. Н3РО4, no less 2. Chlorides, no more 3. Sulphates, no more 4. Heavy metals (Pb), no more 5. Arsenic (Аs), no more 6. Insoluble Ca, Mg, NH4OH in a precipitate, no more 7. Manganese (Mn), no more 8. Potassium (K), no more 9. Sodium (Na), no more | 85 0,0003 0,003 0,001 0,00005 0,005 0,00005 0,005 0,025 |

The phosphoric acid purification  from arsenic and heavy metals with the subsequent extraction of organic compounds is carried out in a reactor-extractor with volume of 100 l (1 piece), and Аs and Pb precipitation as sulphides – in a reactor-precipitator with volume of 100 l (1 piece).

A method of production

Purification of phosphoric acid from arsenic and heavy metals by extraction with an organic solvent – di-2-ethylhexyl-dithiophosphoric acid in decane and 2-ethylhexanol.

The organic leach is purified from impurities and applied many times.

Technological features of the process. The suggested method of phosphoric acid purification from arsenic and heavy metals has no analogues in the CIS and abroad. Applying the ways existing till now it is impossible to reach high purification degree of the phosphoric acid used at preparation of a catalyst C-41 for synthesis of nitrile acrylic acid.

The given method allows purifying the acid to arsenic content of 110-5 %, heavy metal content of 210-4 %.

A manufacturing circuit of phosphoric acid purification from arsenic and heavy metals

For preparation of an extractant the initial di-2-ethylhexyl-dithiophosphoric acid (D2EHDTPA) from a batchbox is given to a reactor-extractor and diluted with decane fed from other measuring tank. Then the extractant is washed five times with distilled water and once an ammonium hydroxide solution.

Each time after stratifying of water and organic phases the bottom water phase is poured out and pass through the absorption column filled with activated carbon. Then the carbon is filtered from the water phase on a nutsch filter; the water phase is fed in an acid drain tank of a packing section with a warehouse of raw materials and a ready product. Then the extractant is again washed several times with water, which is also purified and given to a tank.

The washed extractant is diluted with 2-ethylhexanol from a batchbox. After the dilution the leach prepared is washed with phosphoric acid from a measuring tank; then the acid is purified by the above way and fed to a storage tank.

A dry-process phosphoric acid from a measuring tank is fed to the extractor where the prepared extractant is. The solutions are agitated and settled; the bottom phase – the phosphoric acid purified from As and Pb – is poured in the extractor, in which pure decane from a batchbox is preliminary loaded. Then the phosphoric acid is given in the column with activated carbon, filtered on a nutsch filter and collected in a tank for pure phosphoric acid.

Re-extraction of the extractant is realized in a reactor-extractor filled with hydrochloric acid from a batchbox. The solutions are agitated and settled; the lead-containing hydrochloric acid (the bottom phase) is poured in a precipitation reactor. Then the extractant is washed with water and processed by a water ammonia solution in sequence similar to the hydrochloric acid processing.

Then the extractant is washed with phosphoric acid from a measuring tank, which is later purified in the absorption column, filtered in the nutsch-filter and poured in the tank.

A 5% sulphureous sodium solution is loaded in a reactor-precipitator for sedimentation of arsenic and heavy metal sulphides. The sulphides are filtered in a nutsch filter, the slime formed is buried. The filtrate is discharged in a waste water tank, from which all the washing water is pumped over to a waste water neutralization station.

4 Chapter

4.1 MANUFACTURE of MONOSUBSTITUTED SODIUM PHOSPHATE (SODIUM DIHYDROPHOSPHATE) of “P” and “P.F.A.” GRADES

Monosubstituted sodium phosphate dihydrate (monosodiumphosphate)

Chemical formula – NaH2PO42H2O

Molecular weight – 156,01

Properties: colourless rhombic-system crystals; density is 1,9096 g/cm3. At 57,70С it is melted in the crystallization water, at 1000С it is dehydrated. The further heating up to 2000С leads to formation of pyrophosphate Na2H2P2O7. At temperature of 2440С it turns into sodium metaphosphate. A water solution has acid pH. It is easily soluble in water (46% at 200С) and insoluble in ethyl alcohol.

Application field: in chemical laboratories, at manufacturing pharmaceutical preparations, in photography, food industry, for fabric and wood impregnation with the purpose of giving fire resistance.

Characteristic of initial raw material, auxiliary materials and finished goods

Initial raw material is commercial soda ash (1 sort) and “p” phosphoric acid.

The characteristic of initial raw materials, auxiliary materials and semiproducts is represented in table 1.

Table 1 – Characteristic of initial raw materials, auxiliary materials and semiproducts for monosodiumphosphate production

| № | Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification | Regulated indices with permissible variations |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | “p” | “p.f.a.” |
| 1 | “p” and “p.f.a.” orthophosphoric acid | State Standard 6552-80 | 1. Mass fraction of orthophosphoric acid Н3РО4, no less, % 2. Density, not less, g/cm3 3. Mass fraction, no more, % - nitrates (NO3) - sulphates (SO4) - chlorides (Cl) - iron (Fe) - heavy metals (Pb) - arsenic (Аs) - volatile acids - ammonium salts - residue after calcination - substances reducing KMnO4 | 85 1,69 0,0005 0,003 0,003 0,002 0,001 0,0002 0,0015 0,002 0,2 0,05 | 85 1,69 0,0005 0,002 0,0002 0,0015 0,0005 0,0001 0,001 0,002 0,1 0,005 |
| 2 | Commercial soda ash 1 sort | State Standard 5100-73 | 1. Content of sodium carbonate (Na2CO3), %, not less 2. Mass loss at calcination at 279-3000C, %, no more 3. Chlorides in terms of NaCl, %, no more 4. Iron in terms of Fe2O3, %, no more 5. Water-insoluble substances, %, no more 6. Sulphates in terms of Na2SO4, %, no more | 99,2 0,8 0,5 0,003 0,04 0,05 |  |
| 3 | Distilled water | State Standard 6709-72 | 1. Residue after evaporation, mg/l, no more 2.Residue after calcination, mg/l, no more 3. Content, mg/l: - ammonia and ammonium salts (NH4), no more, - nitrates (NO3), no more - sulphates (SO3), no more - chlorides (Cl), no more - aluminum (Аl), no more - iron (Fe), no more - calcium (Са), no more - copper (Cu), no more - lead (Pb), no more - zinc (Zn), no more - the substances reducing КМnO4 (О), mg/l - pH | 5,0 1,0 0,02 0,2 0,5 0,02 0,05 0,05 0,8 0,02 0,05 0,2 0,08 5,4-6,6 |  |

Characteristic of monosubstituted sodium phosphate dihydrate of “p” and “p.f.a.” grades in accordance with requirements of State Standard 245-76 is represented in table 2.

Table 2 – Requirements of State Standard 245-76 to monosubstituted sodium phosphate dihydrate quality

| № | Parameter name | Standard |  |
| --- | --- | --- | --- |
|  |  | “p.f.a.” | “p” |
| 1 | Mass fraction of monosubstituted sodium phosphate dihydrate NaH2PO42H2O, not less, % | 99 | 98 |
| 2 | Mass fraction of water-insoluble substances, no more, % | 0,005 | 0,005 |
| 3 | Mass fraction of nitrogen from nitrates and nitrides, no more, % | 0,001 | unstandardized |
| 4 | Mass fraction of sulphates (SO4), no more, % | 0,01 | 0,01 |
| 5 | Mass fraction of chlorides (Cl), no more, % | 0,001 | 0,01 |
| 6 | Mass fraction of iron (Fe), no more, % | 0,002 | 0,002 |
| 7 | Mass fraction of arsenic (Аs), no more, %) | 0,00005 | 0,0001 |
| 8 | Mass fraction of heavy metals (Pb), no more, % | 0,00005 | 0,0001 |
| 9 | pH of a 5% solution | 4,2-4,5 | 4,2-4,5 |

Monosubstituted sodium phosphate dihydrate is produced by means of neutralization of reactive orthophosphoric acid by a soda solution with the subsequent evaporation, filtration, crystallization and centrifugation.

Technological features of the process. The suggested manufacture of reactive sodium phosphates unlike the operating one provides for a general unit of obtaining a disodium phosphate solution and a unit of a soda solution preparation. It is expected to produce following salts: disubstituted sodium phosphate 12-hydrate, disubstituted waterless sodium phosphate, trisubstituted sodium phosphate, sodium pyrophosphate and dicalcium phosphate.

Technological equipment is also improved. So, filtration of a disodium phosphate solution is realized on filters FPAKM-5, a sodium pyrophosphate solution is filtered on filters LvAv-20k, and for filtration of a sodium monophosphate solution filters EdGr 1,5-2500k with mechanical removal of a precipitate are provided.

Drum vacuum hoe-type and spray-type dryers are applied for drying the products.

The mother solutions formed at the manufacturing sodium mono-, di-, tri- and pyrophosphates are processed into sodium orthophosphate solutions with the subsequent obtaining sodium tripolyphosphate. The project of calcium salts manufacture includes a general unit of production and purification of a calcium nitrate solution for manufacture of di- and tricalcium phosphates.

Drum-type vacuum filters provided repulping a calcium salt pulp for better washing are used for filtration of di- and tricalcium phosphate suspensions. Drying is realized in a radiation-convective conveyor-type dryer.

Description of a manufacturing circuit of monosubstituted sodium phosphate dihydrate production (“p” and “p.f.a.”)

a) The process stages

The monosubstituted sodium phosphate dihydrate production process based on the neutralization of orthophosphoric acid by a soda ash solution with replacement of one hydrogen ion with a sodium ion proceeds according to the reaction:

2Н3РО4 + Na2CO3  2NaH2PO4 + CO2 + H2O

The following process stages are evaporation, filtration, crystallization and centrifugation of the pulp formed.

The process is continuous and includes following stages:

- reception of initial raw materials;

- neutralization of orthophosphoric acid by a soda solution;

- evaporation of a monosodium phosphate solution;

- filtering the evaporated monosodium phosphate solution;

- crystallization;

- centrifugation of a monosodium phosphate pulp;

- dissolution of the monosodium phosphate crystals;

- recrystallization of the solution;

- packing a ready product – monosodium phosphate.

b) A detailed description of the operating manufacturing circuit (fig.1)

Reception of initial raw materials. A soda ash solution (pos.1) with density =1,28-1,3 g/cm3 is fed in a soda solution batchbox (pos.2) with volume of 0,75 м3. Reactive orthophosphoric acid with =1,62-1,69 g/cm3 from a reactive phosphoric acid section is given in a batchbox (pos.3) with volume of 0,75 m3. The soda solution temperature in the measuring tank (pos.2) is maintained about 40-500С by giving hot water in a jacket; it is registered and automatically regulated by a device (pos.K-102). Absence of overflow from the batchboxes (pos.2) and (pos.3) is controlled by the devices (К-700) and (К-701), respectively, located on a control panel 1.

Neutralization of orthophosphoric acid by a soda ash solution. The soda solution spontaneously from the measuring tank (pos.2) and the orthophosphoric acid from the measuring tank (pos.3) are simultaneously fed in a reactor-neutralizer (pos.4) with volume of 2 m3.

Then the solution on a top overflow pipe is fed in a cascade reactor-neutralizer (pos.4). The orthophosphoric acid solution volume is registered and automatically regulated by the device (К-501) situated on the control panel 1. The neutralization process is realized at temperature of 50-800С. The neutralization reaction is performed at рН=3,5-4,4. The reaction end is determined by means of titration of a sample by a 0,1N hydrochloric acid solution.

An adjustment limit is 0-1 ml of 0,1N HCl. The pH in the reactor-neutralizer (pos.4) is controlled on a local control panel and maintained in the set limit by feeding the orthophosphoric acid and soda solution. The рН in the reactor-neutralizer (pos.4) is continuously registered and automatically regulated by giving the orthophosphoric acid. The reactor-neutralizer is equipped with a turbine mixer with rotation frequency of 195 rpm. The devices (К-702/1,2) on the control panel 1 warned against absence of the monosodium phosphate solution overflow from the reactors-neutralizers.

Evaporation of the monosodium phosphate solution. The monosodium phosphate solution from the reactor-neutralizer (pos.4) along a heated overflow pipe is fed in an evaporator (pos.5) with volume of 2 m3, supplied with a turbine agitator with rotation frequency of 195 rpm.

The evaporation process is carried out at temperature of 100-1100С to the solution density of 1,53-1,56 g/cm3. Temperature in the evaporator is registered on the control panel 1 by a device (К-104). The solution level in the evaporator is registered on the local panel. The emergency overflow from the evaporator (pos.5) is given to a mother solution tank (pos.6) with volume of 5 m3.

Filtration of the evaporated monosodium phosphate solution. The evaporated monosodium phosphate solution from the evaporator (pos.5) on a heated conduit by gravity flows on a filter Eg - Гр - I, Б-2500 кI (pos.6), on which it is purified from impurity. The filter represents a cylindrical reservoir with a compound bottom and a cover; a suspension receiver volume is 2,5 m3; it is supplied by an agitator with rotation frequency of 15 rpm.

The filter works under the pressure no more than 0,4 kPa (4 kPa/cm3) created by feeding compressed undrained air after loading the evaporated monosodium phosphate in the filter.

The filtering is realized at temperature of 70-800С, which is maintained by giving hot water in the filter jacket. The monosodium phosphate solution is filtered through two filtering fabric layers and three layers of the filtering paper laid on a filtering partition and then flows in the bottom part of the filter.

Temperature in the filter (pos.6) is registered on the control panel 1 by a device (К-105). The filtered monosodium phosphate solution from the filter spontaneously flows along a heated conduit in a crystallizer (pos.7).

After termination of the filtration process the filter is washed with a condensate, which is fed by a pump (pos.6) from a condensate storage tank (pos.6).

The condensate after washing the filter is spontaneously fed to the mother solution tank (Р-522).

Crystallization of the monosodium phosphate solution. Crystallization of the filtered monosodium phosphate solution is carried out in periodic crystallizers (pos.7) with volume of 1,6 m3. The crystallizer is equipped with a jacket and a gate agitator, which rotates with frequency of 63 rpm. Cooling the monosodium phosphate solution in the crystallizers is realized in the course of 10-12 hours up to temperature of 18-220С; in the beginning – by turnaround water up to 400С within 4 hours in accordance with a regime of 100С per hour, then by turnaround water within 8 hours, 2-30С per hour. Temperature in the crystallizers (pos.7) is registered by the device (K-100) located on the control panel 1.

Centrifugation of the monosodium phosphate pulp. The monosodium phosphate pulp from the crystallizers (pos.7) spontaneously flows through a charging valve in an intermediate tank, then in an automated horizontal centrifuge FTM-FOZ, К-01 (pos.9) supplied by an unloader. The intermediate tank (pos.8) is equipped with a jacket and a screw-type agitator with rotation frequency of 31 rpm.

The centrifuge operates periodically in following cycles:

- loading the monosodium phosphate pulp;

- drying the monosodium phosphate crystals;

- unloading the crystals;

- regeneration of the centrifuge sieve.

The mother solution and washing water are collected in the mother solution tank (pos.8), from which pumped over by a pump (М-523) in a mother solution storage tank of a disodium phosphate production department, and then to an experimental-industrial installation of mother solution processing in a shop of manufacturing sodium tripolyphosphate № 19. Temperature in the mother solution tank is maintained 50-600С at the expense of the steam giving in the tank jacket; it is registered by a device (К-106) on the control panel 1. The liquid level in the tank is controlled on the local control panel.

The monosodium phosphate crystals are unloaded from the centrifuge (С-517/1) in strong bags with polyethylene bushes, established on a weighing machine (pos.Х-528).

The technology described above allows producing the monosodium phosphate of “p” grade; for obtaining the product of “p.f.a.” grade the given manufacturing circuit should be supplemented with a recrystallization stage.

Dissolution of the monosodium phosphate crystals, recrystallization of the solution obtained. For obtaining the monosodium phosphate of “p.f.a.” grade the crystals, formed in the crystallizers (pos.8 (Р-515)), from the centrifuge (pos.9 (517/2)) are fed in a dissolver (pos.8 (Р-524)) with volume of 1 m3. Distilled water through a measuring tank is also given in the dissolver (pos.8 (Р-524)). Temperature in the dissolver is in limits of 20-900С for full dissolution of the crystals. The monosodium phosphate crystals are added in the solution at the working agitator before the achievement of the solution density of 1,56-1,565 g/cm3.

The monosodium phosphate crystal dissolution takes place at 70-900С. The temperature is maintained by the steam feeding in the dissolver jacket; it is registered by a device (К-107) on the control desk 1. The liquid level in the dissolver is controlled on the local panel. The prepared monosodium phosphate solution with density of 1,56-1,565 g/cm3 is pumped over to the recrystallization in crystallizers (Р-516). If it is necessary feeding the solution can be realized through a filter (512).

Cooling the solution in the crystallizers is carried out with the cooling rate of 10 0С/h before the temperature of 400С by turnaround water, then with the rate of 2-3 0С/h before the temperature of 18-220С by turnaround or chilled water. The temperature in the crystallizer is registered by a device (К-100) located on the control panel 1. The monosodium phosphate pulp from the crystallizer (Р-516) through a discharging valve is poured in an intermediate tank (Р-516/а) and then fed to a centrifuge (517).

Packing the monosodium phosphate. The crystals from the centrifuge are loaded into strong bags with polyethylene bushes established on a weighing machine. The bags are fastened by means of a clamping device. The polyethylene bushes are welded up. The strong bags are sewn up and transported to a warehouse of finished goods.

Packing the ready product in small container is also provided. The monosodium phosphate crystals from the centrifuge (pos.517) are loaded into a trailing cable, which is transported to a unit of small packing of dry reagents, where the monosodium phosphate is packed up in glass jars with capacity of 2 liters. The jars are provided with labels, packed into boxes and transported to a warehouse of finished goods.

The packing and labeling the ready product are fulfilled according to State Standard 245-76.

Industrial wastes

Table 3. Solid and liquid wastes

| Name of a waste kind, a department, an apparatus | Discharge point | Waste quantity, kg/day | Formation periodicity | Solid and liquid waste characteristic |  | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Chemical composition, ??? | Physical parameters, specific weight, kg/m3 |  |
| A mother solution after monosodium phosphate centrifugation. A centrifuge (pos.С-517) |  | 4831,2 1096,2 | Periodical liquid waste | NaH2PO4 – 44,0 H2O – 47,0 H3PO4 – 8,17 NaH2PO4 – 49,57 H2O – 50,43 | 1,45 |  |

Table 4. Emissions into the atmosphere

| Name of an emission, a department, an apparatus, emission diameter and height | Emission source quantity | Total volume of waste gases, ths m3/h | Periodicity | Emission characteristic |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Temperature, 0C | Composition, g/m3 | Permissible quantity of standardizes components |  |
| Exhausts from the technological equipment after a condenser (pos.Т-531). Pipe diameter d = 400 mm. | 1 | 2,0-2,5 | Permanent | 20-30 | СО2 – 13,3-13,5 Н2О – 66,2-82,8 |  |  |

CAPITAL EQUIPMENT

Specification of the capital equipment used at monosodium phosphate manufacture

| № pos. | Equipment name | Quantity | A material, a safeguarding method | Technical characteristic |
| --- | --- | --- | --- | --- |
| Е-508 | A soda solution batchbox | 1 | Steel 12х18 IOI, thermal insulation | Index В 772,3-0,025-ОБК, volume – 0,75 m3. Diameter – 700 mm. Height – 1695 mm. |
| Е-509 | A phosphoric acid batchbox | 1 | Steel 12х18 МIOI | Index В 7721-0,25-ОБК-0,3, volume – 0,75 m3. Diameter – 700 mm. Height – 1260 mm. |
| Е-510 | A reactor-neutralizer | 2 | Steel 08-21 М6 М2Т, thermal insulation | Index 01:03-2 06-СА41. An apparatus with an elemental bulk smooth-top cover. Volume – 2 m3. Diameter – 1400 mm. Height – 3280 mm. The apparatus is provided for a pipe agitator. Rotation frequency – 195 turn/min. Gear motor drive МПД1-10-07-34-3-195-АD-2-32-4. Electric motor АО2-32/4 N = 3 kW. |
| Е-507 | A distilled water batchbox | 1 | Assembly, including aluminum | Index ВПД-10-А. Vertical sectional flat-bottom apparatus. Volume – 1 m3. Height – 1695 mm. |
| Е-511 | An evaporator | 2 | Steel 08х21 М6 М2Т | Index 01-03-2 СБ-СА 41. An apparatus with an elemental bottom, a detachable elemental cover and a smooth welded jacket. Volume – 2 m3. Diameter – 1400 mm. Height – 3125 mm. The apparatus is provided for a pipe agitator. Rotation frequency – 195 turn/min. Gear motor drive МIOI-10-734-3/195-AO2-32-4. N=3 kW. |
| Ф-512 | A low-pressure filter | 1 | Steel 12х12 МIOI | Еg Ip-I, Б-2500 кI type. Dimension, mm: 3010х2025х4950. Collector volume – 3 m3. |
| Е-515 | A crystallizer | 2 | Steel 08х21М6Т, thermal insulation | Еg Ip-I, 5-2500 кI type. Dimension, mm: 3010х4950 mm. Filter collection volume – 0,55 m3. Suspension mixer’s collector volume – 2,5 m3. Filtration surface area – no more 1,5 cm3. |
| С-517 | A centrifuge | 2 | Steel 12х18МIOI | ФГИ-903 К-01 type. An automated filtering horizontal centrifuge with knife loading a precipitate. Working volume – 0,43 m3. Maximum loading – 150 kg. |

4.2MANUFACTURE OF SODIUM SALTS OF ORTHOPHOSPHORIC ACID

1. A general characteristic of the manufacture

1.1 An installation for manufacturing sodium salts of orthophosphoric acid can be applied for production of disubstituted sodium phosphate 12-hydrate and sodium phosphate 12-hydrate.

Disubstituted sodium phosphate is produced by means of neutralization of orthophosphoric acid with a soda ash solution with subsequent filtration, crystallization and washing the crystals formed with distilled water for removal of chlorine ions

Sodium phosphate 12-hydrate is produced by additional neutralization of a disubstituted sodium phosphate solution by sodium hydroxide with subsequent filtration and crystallization.

1.2 The manufacture of disubstituted sodium phosphate is placed in operation in 1937, sodium phosphate 12-hydrate – in 1967.

1.3 Designed capacity of the installation calculated in terms of disubstituted sodium phosphate (a main product) makes 2750 t/y.

1.4 Quantity of technological lines – 1.

1.5 Production method – periodic.

1.6 The installation on manufacturing sodium salts has been designed by Experimental plant of НИУИФ in 1937; a developer of the technological process is НИУИФ.

In 1970, for the purpose of manual skills mechanization (loading soda ash in a dissolver, packing finished goods) and expansion the installation was reconstructed in accordance with a project of Experimental plant of НИУИФ.

2. CHARACTERISTIC OF FINISHED PRODUCTION

2.1 Disubstituted sodium phosphate 12-hydrate according to State Standard 4172-76 is produced with following parameters:

| Parameter name | Standard |  |  |
| --- | --- | --- | --- |
|  | c.p. | p.f.a. | p |
| 1. Disubstituted sodium phosphate 12-hydrate Na2HPO412H2O, % | 99-101 | 98-102 | 98-103 |
| 2.Water-insoluble substances, %, no more | 0,002 | 0,002 | 0,010 |
| 3. Total nitrogen (N), mass fraction (as nitrates, nitrites and ammonia), %, no more | 0,0005 | 0,0010 | unstandardized |
| 4. Sulphates (SO4), %, no more | 0,0005 | 0,0005 | 0,0010 |
| 5. Chlorides (Cl), %, no more | 0,001 | 0,003 | 0,005 |
| 6. Iron (Fe), %, no more | 0,0002 | 0,0005 | 0,0020 |
| 7. Potassium (К), %, no more | 0,005 | unstandardized | unstandardized |
| 8. Magnesium (Mg), %, no more | 0,001 | 0,001 | unstandardized |
| 9. Мышьяк (Аs), %, не более | 0,00004 | 0,00025 | 0,00050 |
| 10.Heavy metals (Pb), %, no more | 0,0005 | 0,0005 | 0,0005 |
| 11. рН of a 5% solution | 8,7-9,3 | 8,7-9,3 | unstandardized |

2.2 Sodium phosphate 12-hydrate is produced according State Standard 9337-79 with the following parameters:

| Parameter name | Standard |  |
| --- | --- | --- |
|  | ? | ? |
| Mass fraction of sodium phosphate 12-hydrate (Na3PO412H2O), %, no more | 99 | 98 |
| Mass fraction of a free alkali (NaOH), %, no more | 1,5 | 2,0 |
| Mass fraction of water-insoluble substances, %, no more | 0,005 | 0,01 |
| Mass fraction of nitrogen (N), as nitrates, nitrites and ammonia, %, no more | 0,001 | 0,004 |
| Mass fraction of sulphates (SO4), %, no more | 0,005 | 0,01 |
| Mass fraction of chlorides (Cl), %, no more | 0,001 | 0,005 |
| Mass fraction of iron (Fe), %, no more | 0,0005 | 0,001 |
| Mass fraction of heavy metals (Pb), %, no more | 0,0005 | 0,0005 |
| Mass fraction of arsenic (As), %, no more | 0,00005 | 0,0005 |

2.3. Disubstituted sodium phosphate represents colorless crystals soluble in water and insoluble in alcohol.

Disubstituted sodium phosphate 12-hydrate

Chemical formula – Na2HPO412H2O

Molecular weight – 358,14

Within a temperature interval from 0 to 250С disubstituted sodium phosphate crystallizes from its solutions with 12 water molecules, at temperature of 25-500С – as disubstituted sodium phosphate 7-hydrate, at temperature of 95,20С – it precipitates as dihydrate, which  starts to be dehydrated at 950С. At heating up to 1000С the salt completely loses the crystallization water, i.e. there is its full dehydration according to the reaction:

Na2HPO412H2O  Na2HPO4 + 12H2O

At temperature of 2500С sodium phosphate turns into sodium pyrophosphate:

2Na2HPO4  Na4P2O7 + H2O

At storage disubstituted sodium phosphate is weathered, loses a part of the crystallization water. Disubstituted sodium phosphate solutions have alkaline medium (рН 8,7-9,3).

Disodium phosphate solubility in water is represented in fig.1. ***

2.4 Sodium phosphate 12-hydrate represents colourless crystals weathering in dry air, easily soluble in water and insoluble in carbon bisulphide.

Chemical formula – Na3PO412H2O

Molecular weight – 380,12

At temperature 73,40С it melts in the crystallization water. Melting point of the waterless salt is 13400С.

In water solutions the salt is completely decomposed into Na2HPO4 and NaOH; in this case the solutions have strong alkaline medium (рН= -13).

Solubility of trisodium phosphate in water is represented in fig.2. ***

2.5. Application field

2.5.1 Disubstituted sodium phosphate is applied as a reactant at carrying out of laboratory experiments and also as a raw material for manufacturing reactive sodium phosphate 12-hydrate.

2.5.2 Sodium phosphate 12-hydrate is applied as a reactant in laboratory practice, in photography as an additive to alkaline developers and as a water softener.

3. CHARACTERISTIC OF INITIAL RAW MATERIALS, AUXILIARY MATERIALS AND SEMIPRODUCTS

| Name of raw materials, auxiliary materials and semiproducts | State standard, Branch standard, Technical specifications | Indices for mandatory verification |  | Regulated indices with permissible variations |
| --- | --- | --- | --- | --- |
| 3.1. Disubstituted sodium phosphate 12-hydrate production |  |  |  |  |
| Soda ash | State Standard 5100-73 | Mass fraction of a base material (Na2CO3), %, not less Mass fraction of iron (Fe), %, no more Mass fraction of chlorine (Сl), %, no more |  | 99 0,003 0,5 |
| Orthophosphoric acid (mother solution of “c.p.” orthophosphoric acid production) | State Standard 6552-80 | Density, g/cm3, not less Mass fraction of arsenic (As), %, no more Mass fraction of iron (Fe), %, no more |  | 1,66 0,0002 0,002 |
| 3.2 Sodium phosphate 12-hydrate production |  |  |  |  |
| Disubstituted sodium phosphate 12-hydrate | State Standard 4172-76 | Mass fraction of water-insoluble, %, no more Mass fraction of iron (Fe), %, no more Mass fraction of chlorine (Сl), %, no more | 0,002 0,0005 0,001 |  |
| Sodium hydroxide | State Standard 4328-77 | Mass fraction of a base material (NaOH), %, not less Mass fraction of chlorine (Сl), %, no more Mass fraction of iron (Fe), %, no more | 96 0,005 0,002 |  |

4. DESCRIPTION OF THE MANUFACTURING PROCESS

4.1 Disubstituted sodium phosphate production

Disubstituted sodium phosphate 12-hydrate is produced by neutralization of phosphoric acid by a soda ash solution according to the reaction:

Na2CO3 + H3PO4 + 11H2O = Na2HPO4∙12Р2О + CO2

A soda solution and phosphoric acid are fed to reactors at continuously working agitatos. At the neutralization carbon dioxide is liberated. For its fuller removal the solution is heated up to 1030С and boiled; thus there is sedimentation of iron and aluminium phosphates:

2Н3РО4 + Al2O3 = 2AlPO4+3H2O

2H3PO4 + Fe2O3=2FePO4 + 3H2O

The precipitate is filtered. The disubstituted sodium phosphate solution is cooled; as a result of the cooling the salt crystallizes. The disubstituted sodium phosphate crystals are separated from mother solutions by centrifugation. The mixture obtained is washed with distilled water from removal of chlorine ions. The mother solutions and washing water are used for the soda ash dissolution.

The salt is packed in polyethylene bags.

4.2 Stages of the manufacture process

4.2.1 Preparation of a soda solution.

4.2.2 Preparation of a disubstituted sodium phosphate solution.

4.2.3 Filtering the solution.

4.2.4 Crystallization of the solution.

4.2.5 Centrifugation of a suspension.

4.2.6 Packing a ready product.

4.2.1 Preparation of a soda solution

Distilled water is poured in a dissolver (pos.3) and steam is sent. Soda ash is loaded in the dissolver at continuous hashing by means of a loading auger (pos.2) from a soda bunker (pos.1).

The dissolution process is realized at temperature of 40-450С. The soda solution density is 1,28-1,29 g/cm3 (a 26-29% Na2CO3 solution).

Soda is loaded in the dissolver as the soda solution is consumed. The soda solution is pumped over in a pressure tank (pos.5) by a centrifugal pump (pos.4).

4.2.2 Preparation of a disubstituted sodium phosphate solution

The soda solution with density of 1,28-1,29 g/cm3 in quantity of 50050 l is poured in a reactor (pos.7) at a working agitator from the pressure tank, then phosphoric acid (the mother solutions formed at manufacturing the “c.p.” acid) is fed from a pressure tank (pos.6). Finishing the neutralization is determined according to a qualitative reaction of the solution with phenolphthalein (before appearance of pale pink copor). Correction of the neutralization end is performed by addition of the soda solution (if the color is absent) or the acid (in a case of the intensive color). The neutralization should be finished at alkalescent medium of the solution containing 0,5 % of Na2CO3.

After check of the neutralization reaction end the solution with density of 1,26-1,27 g/cm3 is boiled at temperature of 1030С within 40-50 minutes; the reactor is heated with the steam fed to the reactor jacket.

The solution boiling promotes coagulation of insoluble substances (basically, silicic acids and one-and-a-half oxides) and removal of the dissolved carbon dioxide. After the boiling it is necessary again to check the solution medium and in case of need to fulfill the final correction.

4.2.3 Filtration of the solution

The disubstituted sodium phosphate solution is diluted to density of 1,26-1,27 g/cm3 at temperature of 1000С and poured from the reactor through a funnel in nutsch filters (pos.9). The solution temperature at the filtration is needed to be 700С. The solution is filtered through a diagonal cloth, two filtering paper layers and a ревентух cover.

The filtration rate depends on following factors:

- the solution density;

- completeness of the coagulation of insoluble substances;

- presence of a silicic acid sol;

- the solution temperature.

The disubstituted sodium phosphate filtered solution is fed in a filtrate tank (pos.23), from which it is pumped over by a submersible pump (pos.24) in a heat exchanger (pos.15) for preliminary cooling and then to crystallization.

The filtrate tank is equipped an alarm system of the top level; switching-off the submersible pump at achievement of the bottom level is provided.

4.2.4 Crystallization of the solution

Crystallizers (pos.11) are filled by the solution at a working agitator without water cooling. Water is fed for cooling only after dissolution of the crystal crust settled on the crystallizer walls after a previous operation.

At the beginning, within 2-3 hours the solution cooling should be performed gradually giving a little water in the crystallizer jacket in order to avoid overcooling the solution by the crystallizer walls and the crust formation.

When the solution starts to crystallize (≈350C) the cooling should be more intensive.

The suspension formed need be cooled up to temperature of 20-280С within 8-12 hours. The suspension cooled is fed to centrifugation.

4.2.5 Centrifugation of the suspension

Separation of the crystals and mother solutions is realized by means of centrifugation; for this purpose the suspension is fed to a bowl shell (pos.14) of a centrifuge through a funnel at the rotating mixer of the crystallizer.

The centrifuge loading is fulfilled at on electric motor (first speed). Centrifugation of the crystals is carried out with third speed.

For fail-safe functioning of the centrifuge blocking of its cover is provided. The centrifuge cover should not be opened at the operating electric motor and rotating bowl shell.

After wringing of the crystals the salt is washed by distilled water from a tank (pos.20) for removal of chlorine ions in quantity of 100-150 l per one loading. The mother solution and washing water are given in a mother solution tank (pos.18) and pumped over by a submersible pump (pos.19) in the soda dissolver for preparation of a soda solution.

A pressure line of the submersible pump is equipped with a pressure transducer, which automatically switches off the pump at pressure drop in the pressure line.

Final mother solutions are collected in a container and used at manufacture of a salt mixture (sodium tripolyphosphate and sodium pyrophosphate).

The disubstituted sodium phosphate 12-hydrate crystals are unloaded from the centrifuge in a receiving bin (pos.15) of an inclined auger (pos.16).

The centrifuge filtering grid is washed by the hot distilled water heated in the heat exchanger established on a forcing line of the pump fed the distilled water.

4.2.6. Packing a ready product

The product from the receiving bin is fed to packing by means of the inclined auger and packed in 15 or 30 kg polyethylene bags.

The polyethylene bags with capacity of 15 kg are stacked in the boxes preliminary laid by one layer of a package paper.

The polyethylene bags with capacity of 30 kg are preliminary put into bitumized bags. The packing is realized according to State Standard 3885-73.

If the disubstituted sodium phosphate 12-hydrate is intended for production of sodium phosphate the packing is performed in 30 kg bitumized bags or other container (jute bags, rubber containers).

4.3. Sodium phosphate 12-hydrate manufacture

Sodium phosphate 12-hydrate is produced by neutralization of a disubstituted sodium phosphate solution by sodium hydroxide according to the reaction:

Na2HPO412H2O + NaOH = Na3PO412H2O + H2O

The sodium phosphate solution obtained is boiled, filtered, crystallized and centrifuged for separation of crystals from a mother solution. The mother solutions are use for the disubstituted sodium phosphate dissolution.

4.4 The technological process stages

4.4.1 Preparation of a disubstituted sodium phosphate solution

4.4.2 Preparation of a sodium phosphate solution

4.4.3 Filtration of a sodium phosphate solution.

4.4.4 Crystallization of a sodium phosphate solution

4.4.5 Centrifugation of suspensions

4.4.6 Packing a ready product

4.4.1 Preparation of a disubstituted sodium phosphate solution

A disubstituted sodium phosphate solution is prepared periodically.

Distilled water or mother solutions are loaded in a soda dissolver (pos.3) and heated up to 40-500С. Disubstituted sodium phosphate from bags or other container is put in the dissolver at continuous hashing.

Density of the sodium phosphate solution should be 1,2 g/cm3.

4.4.2 Preparation of a sodium phosphate solution

Sodium hydroxide is manually loaded in the disubstituted sodium phosphate solution at continuous hashing. The sodium hydroxide loading is realized by small portions in order to avoid the solution emission.

The neutralization process termination is determined by titration.

The ready solution with density of 1,24 g/cm3 is pumped over in a reactor (pos.7), boiled within 40-50 minutes. The heating is carried out by the steam fed to the reactor jacket. The boiling promotes coagulation of insoluble substances and transfer of iron (II) phosphates into iron (III) phosphates.

After the boiling it is necessary to check the solution medium and in case of need to fulfill the solution correction.

The ready solution is diluted in the reactor to density of 1,24 g/cm3 by distilled water from a tank (pos.20).

4.4.3 Stages of filtration (except for preliminary cooling in a heat exchanger (pos.15)), crystallization of a sodium phosphate solution, packing a ready product are similar to respective stages of the disubstituted sodium phosphate manufacture.

4.4.4 Centrifugation of a suspension

The sodium phosphate suspension is fed to a centrifuge bowl shell (pos.14) at a rotating agitator of the crystallizer. After full wringing the crystals, i.e. after full separation of a mother solution, the centrifuge is switched off; the salt is unloaded in a receiving bin (pos.15) of an inclined auger (pos.16). The mother solutions with density of 1,11-1,13 g/cm3 in quantity of 500-600 liters after each the operation are pumped over by a sinking pump (pos.19) from a mother solution tank (pos.18) in the soda dissolver and used for the disubstituted sodium phosphate dissolution.

Final mother sodium phosphate solutions in quantity of 1,5-1,8 m3 are applied for manufacturing a mixture of sodium tripolyphosphate and sodium pyrophosphate.

5. MATERIAL BALANCE

5.1 Disubstituted sodium phosphate manufacture (per 1 tonne of Na2HPO412Н2О)

Neutralization 1206

Crystallization

Centrifugation 1174

318 kg Na2CO3

Н2О

594 kg

294 kg Н3РО4

1074

1074

Н2О 100 kg

Mother solutions

Na2HPO412H2O

Losses 7 kg

Na2HPO412H2O 1000 kg

MATERIAL BALANE

(per 1 t of disubstituted sodium phosphate 12-hydrate)

Table 3

| Income |  |  | Expenditure |  |  |
| --- | --- | --- | --- | --- | --- |
| Composition, kg/t | Mass, kg | %, mass | Composition, kg/t | Mass, kg | %, mass |
| Soda ash, 100 % | 318 | 24 | Disubstituted sodium phosphate 12-hydrate | 1000 | 76,5 |
| Distilled water | 694 | 53 | Carbon dioxide | 132 | 10 |
| Orthophosphoric acid, 100 % | 294 | 23 | Mother solutions of Na2HPO412H2O | 167 | 13 |
|  |  |  | Mechanical losses | 7,0 | 0,5 |
| Total: | 1306 | 100 | Total: | 1306 | 100 |

5.2 Sodium phosphate 12-hydrate manufacture

Material flow pattern per 1 t of sodium phosphate 12-hydrate

NaOH 122 kg 100 %

1010 kg of Na2HPO412H2O

H2O 2900 kg

Neutralization

Crystallization

Centrifugation

4032 kg

Mother solutions 123 kg Na3PO412H2O

1000 kg of Na3PO412H2O

Mechanical losses 9 kg

MATERIAL BALANCE

(per 1 t of Na3PO412H2O)

Table 5

| Income |  |  | Expenditure |  |  |
| --- | --- | --- | --- | --- | --- |
| Composition, kg/t | Mass, kg | %, mass | Composition, kg/t | Mass, kg | %, mass |
| Disubstituted sodium phosphate Na2HPO412H2O | 1010 | 25 | Sodium phosphate Na3PO412H2O | 1000 | 24,3 |
| Sodium hydroxide Distilled water | 122 2900 | 3 72 | Mother solutions, including: water sodium phosphate | 3023 2900 123 | 75 72 3 |
|  |  |  | Mechanical losses | 9 | 0,2 |
| Total: | 4032 | 100 | Total: | 4032 | 100 |

TECHNOLOGICAL RATES

Table 6

| Stage | Technological parameter name |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Duration, h | Consumption | Temperature, 0С | Pressure, kgf/cm2 (kPa) | Density, g/cm3 |
| Disubstituted sodium phosphate manufacture |  |  |  |  |  |
| Preparation of a soda solution: - distilled water or mother solutions - soda ash |  | 800100 l 200350 kg | 4010 | atm | 1,28-1,29 |
| Neutralization of a soda ash solution: - a soda ash solution - phosphoric acid |  | 500 l or 160-170 kg of 100% Na2CO3 150-160 kg of 100% Н3РО4 | 6010 | atm | 1,28-1,29 1,66-1,68 |
| Boiling of a disubstituted sodium phosphate solution | 40-50 min |  | 103 | atm | 1,26-1,27 |
| Filtration |  |  | no more 70 | atm | 1,26-1,27 |
| Crystallization | no more 8-12 |  |  | atm |  |
| Centrifugation: water for washing to each a centrifuge |  | 100-150 l |  | atm | mother solutions 1,16 |
| Packing a ready product: polyethylene bags for packing in boxes polyethylene bags for packing in bags |  | 15 kg 30 kg |  |  |  |
| Sodium phosphate manufacture |  |  |  |  |  |
| Preparation of a disubstituted sodium phosphate solution: - distilled water - disubstituted sodium phosphate 12-hydrate |  | 800100 l 150-200 kg | 4010 | atm | 1,2 |
| Preparation of a sodium phosphate solution: sodium hydroxide |  | 70-73 kg of 96% NaOH | 555 | atm | 1,24 |
| Boiling of a sodium phosphate solution | 40-50 min |  | 101 | atm | 1,24 |
| Filtration |  |  | no more 70 | atm | 1,24 |
| Crystallization | 8-12 |  |  |  |  |
| Centrifugation |  |  |  | atm | Mother solutions 1,11-1,13 |
| Packing a ready product: polyethylene bags for packing in boxes polyethylene bags for packing in bags |  | 15 kg 30 kg |  |  |  |

ENVIRONMENT PROTECTION

Discharge into the atmosphere

Table 7

| Discharge name, a section. An apparatus, diameter and height of an emission source | Emission source number | Total volume of waste gases, m3/h | Cyclicity | Characteristic of emissions |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T, 0С | Composition, mg/m3 | Permissible quantity of normalized components |
| Technological waste A salt section A fan for local suction from reactore, nutsch filters, pressure tanks Emission diameter, mm, 400 Emission height, m, 4 | 1 | 3650 | Permanently | 60-80 | Water vapor, carbon dioxide 5,5 | - |

Waste water

Table 8

| A waste water source, a section, an apparatus | Discharge point | Waste water volume, m3/day | Cyclicity | Waste water characteristic |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Composition, mg/l | Permissible quantity of discharged harmful substances, kg/day |
| Drains A salt section A neutralizer | Treatment facilities collector | (1-1,5) 0,25 | 1 time per day | рН 6,5-8,5 | 36-38 (in terms of 100% NaOH) |

Solid and liquid waste

Table 9

| A waste kind, a section, an apparatus | A storing place | Waste quantity, kg/day | Cyclicity of formation | Characteristic of solid and liquid wastes |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Composition, humidity,% | Physical parameters |
| Solid wastes are absent |  |  |  |  |  |
| Liquid wastes |  |  |  |  |  |
| Sodium phosphate manufacture |  |  |  |  |  |
| Mother solutions of disubstituted sodium phosphate, centrifugation | Are pumped in a tank and used at production of a salt mixture (sodium tripolyphosphate and sodium pyrophosphate) | 606 |  | Na2HPO412H2O |  |
| Sodium phosphate manufacture |  |  |  |  |  |
| Mother sodium phosphate solutions, centrifugation | Are pumped in a tank and sent for processing at production of a salt mixture | 200 |  | Na3PO412H2O – 31,63-48,63% Fe–0,001-0,0013% Cl-0,0014-0,003% |  |

Toxic properties of raw materials, semiproducts, an end product and industrial wastes

Table 10

| Name of raw material, semiproducts, an end product and industrial wastes | Toxicity characteristic | MPC in the working air |
| --- | --- | --- |
| Soda ash | It irritates wet skin and mucous membranes; at long-term exposure it causes dermatitis, conjunctivitis. Inhalation of soda ash dust can provoke upper airways irritation. | 2 mg/m3 |
| Mother solutions of “c.p.” orthophosphoric acid | Orthophosphoric acid vapors can cause atrophic processes of mucous membranes and tooth destruction, and also inflammatory skin diseases. | 1 mg/m3 (in terms of P2O5) |
| Disubstituted sodium phosphate 12-hydrate | Its large quantities can cause irritation of mucous membranes and cutaneous covering. | - |
| Sodium phosphate 12-hydrate | The same | - |
| Sodium hydroxide | Sodium hydroxide as a solution and dust cauterizes cutaneous covering and mucous membranes. If its concentration is higher than the permissible concentration, sodium hydroxide causes burns and chronic skin diseases. | 0,5 mg/m3 |

4.3MANUFACTURE OF REACTIVE-PURITY SODIUM PYROPHOSPHATE

GENERAL INFORMATION ABOUT TECHNOLOGY

Waterless sodium pyrophosphate (tetrasodium phosphate) – Na4P2O7 – is an intermediate product of manufacturing reactive sodium pyrophosphate 10-hydrate Na4P2O7.10H2O.

The production technology of commercial and food waterless sodium pyrophosphate has been developed by НИУИФ; industrial production of sodium pyrophosphate has been introduced on the Vinnitsa chemical factory on a basis of wet-process phosphoric acid.

An advantage of the suggested technology of waterless sodium pyrophosphate production of a disodium phosphate solution is a possibility of obtaining a high-quality product using an effective combined drying-calcination apparatus.

The manufacturing circuit is characterized by minimum fuel, electric power and labor consumption at the expense of reduction of capital equipment quantity (for example, owing to combination of processes of solutions’ drying, calcination of a dry powdery disodium phosphate and cooling a ready product in one drying-calcination unit), and also application of intradepartmental transport (units of pouring a product, chutes, conveyors, etc.).

CHARACTERISTIC OF THE EXECUTED RESEARCH AND EXPERIMENTAL WORKS TAKEN AS A PRINCIPLE OF DESIGNING

A) Waterless sodium pyrophosphate production ways are similar to the sodium tripolyphosphate technology.

The equipment used in both the manufactures is also similar.

B) Waterless sodium pyrophosphate is formed as a result of disodium phosphate thermal dehydration. For the purpose of determination of optimum temperature conditions of sodium pyrophosphate production laboratory researches have been carried out.

The disodium phosphate thermal decomposition process has been studied on the installation with use of a gas chromatograph. The research heart consisted in linear heating a waterless disodium phosphate sample at variation of the heating rate from 2 to 300C/min. The heating rate was regulated by means of a special automatic device; the sample temperature was measured by a chromel-alumel thermocouple to ±10С. Water is the gaseous reaction iproduct. Quantity of the water liberated in the process was determined by an absolute calibration method, which is based on use of a dependence of chromatographic peak value and area from a substance quantity.

The researches have shown that the thermal decomposition of disodium phosphate is a complex process. The dehydration rate-temperature graphic chart has two maxima; it indicates that the moisture removal process is a two-stage one. The disodium phosphate decomposition begins with appreciable rate at 2870С; the maximum rate of the first stage is observed at 3270С. The second stage of water liberation starts at 3470С; it is characterised by considerably higher rate.

The maximum process rate at the second stage is observed at 3600С. Change of the process rate during the second stage testifies to formation of an intermediate compound. However this compound has not been identified.

The carried out researches have allowed to establish, that to increase the rate and completeness of dehydration of disodium phosphate into sodium pyrophosphate the process temperature should be not less than 380-4000С.

The process of obtaining a waterless food tetrasodium pyrophosphate on the basis of a dry-process phosphoric acid and soda ash has been studied on a half-way installation of Experimental plant НИУИФ with productivity about 100 kg/h. Drying an initial disodium phosphate solution and dehydration of a waterless dry salt were realized in a combined spraying calcining furnace at прямоточно-противоточной feed of the material and a heat-carrier. All basic parts of the installation were made of steel Х18Н9Т.

The process proceeds as follows. A commercial dry-process phosphoric acid is neutralised by soda ash; the disodium phosphate solution formed is filtered on a nutsch filter and cooled in a crystallizer. The precipitated Na2HPO4.12H2O crystals are separated from a mother solution in a centrifuge. The mother solution is returned in the technological cycle.

The disodium phosphate crystals are heated; the solution obtained by such the way with density of 1,37-1,40 g/cm3 is fed by a pump to a pressure tank from which it is given in the distributing chute located on the top floor of the furnace.

A drying-calcination unit represents a cylindrical chamber with working volume of 17 m3, internal diameter of 2000 mm, general height of 7100 mm. Height of the drying part of the chamber (from the top calcination shelf to atomizers) is 5500 mm, a distance between the shelves is 500 mm.

The solution dispersion was performed by means of compressed air from three pneumatic external-mixing atomizers located in the top part of the drying chamber. Feeding the solution from the chute in each the atomizer was carried out on a flexible rubber hose for the account of ejection. The atomizer productivity was regulated by replaceable nipples with various inlet diameters, which were put on the end of the rubber hose lowered in the chute with the solution.

The drying-calcination unit inside is lined by steel Х18Н9Т and has two calcination and one cooling shelves.

The disodium phosphate solution and the furnace gas from the top fire chamber with temperature of 860-8700С are direct-flow fed in the top part of the furnace, and the furnace gas from the bottom fire chamber with temperature of 560-6000С is fed countercurrent to the dehydrated material in the space between the calcination shelves. Waste gases were taken away in the average part of the furnace on a distance about 2 m from the top calcination shelf.

The sprayed fine-dispersed solution is dried and the material is partially dehydrated in the hot gas flow in the chamber volume in a suspended state. The dried product is collected on the top calcination shelf, fed on the second one and then on the cooling shelf. Hashing of the dry product and its removal with calcination and cooling shelves was carried out by means of the mechanical scrapers set in motion from one general vertical shaft. In the cooling shelf loop water circulated.

The cooled product from the unit was fed by an auger to grinding and sieving, and then it was packed in multilayer paper bags.

The discharge gases were given to a centrifugal cyclone; the caught product was collected in the bunker under the cyclone and lowered on the first calcination shelf through a breachblock and a chute. Having passed the dry purification the gases were fed in the wet scrubber irrigated with water by means of coarse dispersion atomizers. Then the gases were given from the scrubber in a wet cyclone for catching splashes, after which were discharged to the atmosphere by a tail fan.

The research conditions and results are represented in table 1. In the specified conditions a pure waterless tetrasodium pyrophosphate has been produced as a white fine-dispersed powder with bulk weight about 650 kg/m3. About 80% of the product is a fraction from 60 to 100µ.

The ready product composition was the following (%):

Р2О5 total - 53,0-53,5

Р2О5 ortho - 0,2-0,3

Insoluble residue - 0,004-0,006

SO4 - 0,0005

Cl - 0,02-0,05

As - 0,001-0,002

Fe - 0,002-0,003

pH of a 1% solution - 9,9-10,2

The research results have shown, that at the operation of the drying-calcination unit in the specified temperature regime about 50% of the disodium phosphate turns into sodium pyrophosphate in the drying zone during being the sprayed particles in a suspended state.

The unit efficiency during the drying and dehydration has made 11,6-16,2 kg/m3∙h.

2.03. In the Soviet Union the experimental works connected with manufacturing sodium pyrophosphate were realized in three directions. Originally the scientists of НИУИФ investigated the manufacturing circuit in which the sodium orthophosphate drying and dehydration were performed in one calcination drum. This circuit demands eightfold return, that leads to considerable consumption of fuel, electric power and capital investments. The process combining the disodium phosphate dehydration and calcination in a tube revolving furnace divided into two chambers was realized in Germany at a Pisteritsk factory.

The following stage represented separation of drying and dehydration processes: drying a disodium phosphate solution was carried out in spraying dryers, and calcination of the preliminary dried orthophosphate humidified by the initial solution was performed in tube furnaces with internal or external heating.

The third direction at the development of sodium pyrophosphate production technology provided for combination of the processes of spraying drying a sodium orthophosphate solution and dehydration of a dry powdery disodium phosphate on calcination shelves of a combined drying-calcination unit. This process has been realized at the Vinnitsa chemical factory and included drying the solution, calcination and cooling a ready product in one unit operating according to a counterflow regime.

As a result of further researches, fulfilled by scientists of the Experimental plant, the process final version has been suggested; it stipulates direct-flow feeding the material and the heat-carrier in the drying zone and counterflow feeding in the calcination zone.

FEASIBILITY STUDY OF THE RECOMMENDED PRODUCTION METHOD

The feasibility study of reconstruction and expansion of the reactive phosphorus compound manufacture was carried out by “Giprochimreaktiv” in 1975.

Further improvement of technical and economic indicators of the reactive sodium pyrophosphate manufacture is possible as a result of intensification of the operating installation and increase in its productivity in case of use of overheating an initial disodium phosphate solution before its atomization. Delivery of initial data on an intensification recommended sushilno-prokalochnogo the unit with increase in its productivity on waterless пирофосфату sodium is a subject of the subsequent stage of work.

Conditions and results of operation of the sodium pyrophosphate production installation

Table 1

| № | A name | A unit | Experiment 1 | Experiment 2 | Experiment 3 | Experiment 4 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | On-stream time (with the solution) | hour | 3,66 | 8 | 8 | 6,5 |
| 2 | Solution consumption | kg/h | 310 | 425 | 409 | 415 |
| 3 | Solution specific weight | t/m3 | 1,405 | 1,38 | 1,385 | 1,36 |
| 4 | Water content in the | % | 61,2 | 62,7 | 62,2 | 63,7 |
| 5 | Pressure of the compressed air fed for atomization | atm | 4,5 | 4,5 | 4,2 | 4,2 |
| 6 | Solution temperature | 0С | 63 | 75 | 65 | 65 |
| 7 | Ready product quantity | kg/h | - | 800 | 500 | 525 |
| 8 | Quantity of evaporated water with account of dehydration moisture | kg/h | 197,6 | 276,1 | 263,8 | 274,5 |
| 9 | A volume unit intensity according to the evaporated moisture | kg/m3∙h | 11,6 | 16,2 | 15,5 | 16,15 |
| 10 | Gas consumption for the top fire chamber | nm3/h | 16,2 | 16,0 | 17,7 | 13,8 |
| 11 | Primary air consumption for the top fire chamber | nm3/h | 260 | 219 | 210 | 219 |
| 12 | Secondary air consumption for the top fire chamber | nm3/h | 236 | 236 | 180 | 200 |
| 13 | Gas consumption for the lower fire chamber | nm3/h | 20,4 | 21,0 | 20,4 | 15,6 |
| 14 | Primary air consumption for the lower fire chamber | nm3/h | 252 | 272 | 280 | 282 |
| 15 | Secondary air consumption for the lower fire chamber | nm3/h | 280 | 290 | 320 | 320 |
| 16 | Quantity of the gases fed into the apparatus | nm3/h kg/h |  |  |  |  |
| 17 | Temperature between calcination shelves | 0С | 545 | 595 | 600 | 600 |
| 18 | Temperature on the dryer output | 0С | 220 | 200 | 195 | 175 |

TECHNICAL CHARACTERISTICS OF INITIAL RAW MATERIALS, AUXILIARY MATERIALS, MAIN AND END PRODUCTS

For dispersion of the disodium phosphate solution fed to drying the compressed air with pressure of 4-6 atm heated up to 700С is used.

PHYSICAL AND CHEMICAL CONSTANTS AND PROPERTIES OF INITIAL, INTERMEDIATE AND END PRODUCTS

The intermediate products formed at the waterless sodium pyrophosphate production are a disubstituted sodium phosphate solution and dry powder-like disodium phosphate.

Disubstituted sodium orthophosphate solubility at various temperatures is represented in a reference book A.N.Kirgintsev, L.N.Trushnikova, V.G.Lavrentyeva “Solubility of inorganic substances in water”. [L., Khimiya, 1972.-P.174.]

Disodium phosphate solution density at temperatures of 70 and 900С depending on its concentration is represented in fig.1.

Fig.2 expresses a dependence of a disodium phosphate solution’s refraction index on its concentration.

Viscosity of pure disodium phosphate solutions with concentration of 38-47% has been determined in the temperature interval of 60-900С by means of an Ostwald viscosimeter and represented in table 2.

Disodium phosphate solution viscosity

Table 2

| Solution concentration, % | Temperature, 0C | Viscosity, sentipoise |
| --- | --- | --- |
| 38,12 | 60 | 4,73 |
| 38,12 | 90 | 1,85 |
| 43,31 | 90 | 3,09 |
| 46,80 | 80 | 5,02 |
| 46,80 | 90 | 3,44 |

Dry waterless disodium phosphate represents a white powder with density of 0,7-0,8 t/m3.

The powder has an approximate composition:

| Particle size, mm | Fraction content, % |
| --- | --- |
| > 0,16 | 10 |
| > 0,10 | 25 |
| > 0,063 | 30 |
| > 0,040 | 20 |
| < 0,040 | 15 |

Crystalline disodium phosphate standard formation heat is kcal/mol (K.J.Irving, H.M.Kerol “Trans Farad Soc.”, V.63; № 12, 2913, 1967).

A disodium phosphate dehydration product is waterless sodium pyrophosphate Na4P2O7. Data about sodium pyrophosphate solubility are represented in a book “Phosphorus and its compounds” by Van Weser, M., 1962. - P.476, and also in the reference book: A.N.Kirgintsev, L.N.Trushnikova, V.G.Lavrentyeva “Solubility of inorganic substances in water”. [L., Khimiya, 1972.-P.174.]

Bulk weight of powdery sodium pyrophosphate is 0,7-0,9 t/m3. Enthalpy and average specific thermal capacity of waterless sodium pyrophosphate, determined by a calorimetric method for a temperature interval of 100-3500С, are represented in table 3.

Sodium pyrophosphate enthalpy and average specific thermal capacity

Table 3

| Temperature, 0С | 100 | 200 | 300 | 350 |
| --- | --- | --- | --- | --- |
| Enthalpy, cal/g, | 17,32 | 42,86 | 72,34 | 86,40 |
| Average specific heat capacity, cal/g∙degree | 0,230 | 0,244 | 0,263 | 0,266 |

Sodium pyrophosphate at heating undergoes a number of the polymorphic transformations occurring (according to different authors) at the temperatures:

According to our data, the polymorphic transformation temperatures are the following: 410, 435, 520, 530 and 5550С.

Sodium pyrophosphate melting temperature, according to different authors, makes 960-9900С.

Repose angle of the powder-like products has been determined in accordance with the technique described in a dissertation of A.M.Shcherbakov “Studying physicochemical properties of new mineral fertilizers for substantiation of a technological process of their dissemination”. M., 1965. - P.12. Repose angle for waterless sodium pyrophosphate makes 320 to horizon. Repose angle for waterless disodium phosphate is 260, and for the product with humidity of 7,2% it makes 290.

Caking the powdery products has been determined at temperature of 500С by the technique described in a journal “Chemistry in agriculture”, 1970, № 10. - P.27. It was established, that waterless sodium pyrophosphate at moisture content of 0-5,8% does not cake. Disodium phosphate with humidity of 0% does not cake also.

Hygroscopicity of sodium pyrophosphate and disodium phosphate has been determined by a dynamic method (I.M.Kuvshinnikov, Z.A.Tihonovich, V.A.Frolkina “Chemical industry”, 1970, № 7.-P.27). Hygroscopicity coefficient of waterless sodium pyrophosphate is 4,2 mmol/g∙h (a hygroscopic point according to Pestov is 65,5 %). Hygroscopicity coefficient for disodium phosphate is 5,6 mmol/g∙h ( a Pestov’s hygroscopic point is 60%).

Disodium orthophosphate and sodium pyrophosphate are not inclined to self-ignition. Both the compounds are fire-safe and explosion-proof.

CHEMISM, PHYSICOCHEMICAL BASES AND A SCHEMATIC DIAGRAM OF THE MANUFACTURE

The disodium phosphate dehydration process giving tetrasodium pyrophosphate takes place in a temperature interval of 280-5000С according to the reaction:

2Na2HPO4  Na4P2O7 + H2O

The researches carried out on the installation including a gas chromatograph have allowed establishing, that the dehydration process occurs in two stages with formation of an unidentified intermediate compound. By analogy with the dicalcium phosphate dehydration process it is possible to assume the following structure of the intermediate compound:

Na4P2O7∙2Na2HPO4

As it is known from literary data, impurities of some substances, for example, NaH2PO4, NaHSO4 and others, promote acceleration and completeness of the Na4P2O7 formation at the lowered dehydration temperatures. Therefore it is expedient to prepare the initial disodium phosphate solutions with the second hydrogen ion neutralization degree of 98,5-99,5 % that will provide presence of a monosodium phosphate impurity in the solution.

According to data of N.M.Dombrovsky the effective activation energy of pure Na2HPO4 dehydration makes 38,0 kcal/mol, and in the presence of 1% of NaH2PO4 – 11,6 kcal/mol.

The disodium phosphate dehydration reaction rate in the temperature range of 240-3000С to 60-90% decomposition is well described by a first-order reaction equation (N.M.Dombrovsky “Kinetics and catalysis” V.IX, № 2.-P.250-259).

Transformation degree of disodium orthophosphate into sodium pyrophosphate makes approximately 99 %.

The schematic manufacturing diagram (p.20а, fig.3)

The disodium phosphate solution, prepared in the neutralization department, is fed in a tank 28, from which pumped over by a pump 1 in a pressure tank 2. The disodium phosphate solution from the pressure tank spontaneously flows in a chute 6. Through pneumatic external-mixing atomizers 7 the fine-sprayed disodium phosphate solution is fed in the top part of the drying-calcination unit 3. The solution is atomized by means of the heated compressed air given in the atomizers.

Drying the disodium phosphate solution is realized at the expense of heat of the furnace gases formed at combustion of natural gas in a fire chamber 4. The furnace gases are fed in the apparatus by direct-flow with the sprayed solution.

The dried powder-like disodium phosphate is poured along calcination shelves 11 and dehydrated.

The kiln gases are given in space between the plates from a fire chamber 9 by countercurrent to the calcined material.

The sodium pyrophosphate formed as a result of the dehydration is cooled on cooling shelves 12.

The exhaust gases outgoing from the drying-calcination unit are purified from dust in cyclones 13, then fed by a fan 16 in a gas-washer 17 for wet cleaning and discharged into the atmosphere.

The dust from the cyclone is returned on the top calcination plate through a lock shutter 14 along a chute 15.

The gases are washed with water or a turnaround solution, which are fed in the gas-washer 17 by means of a pump 19.

The solution in the tank situated under the scrubber is concentrated to density of 1,20-1,5 g/cm3, then pumped over to the neutralization department.

The waterless sodium pyrophosphate by an auger 20 is given to dissolution in a dissolver 21; then the solution by a pump 22 is pumped over to a Na4P2O7.10H2O crystallization unit.

The given circuit provides unloading and warehousing the waterless sodium pyrophosphate in case of its production as a ready product or as a semiproduct. In this case unloading the pyrophosphate from the apparatus is realized by means of a screw 24, then along an inclined screw conveyor 25 with inclination angle to 200. Loading of soft containers 27 is performed through a chute 26 with a thread switch. Electric and auto loaders and also monorail telphers are applied for intrashop and intrafactory transportation and warehousing.

OPERATING TECHNOLOGICAL PARAMETERS

A drying-calcination unit

1. Furnace gas temperature

Input in a spraying dryer – 480-5200С

A dehydration zone – 500-5500С

Output from a drying zone – 1300С

Output from a dryer – 150-1700С

2. Solution temperature – 900С

3. Product temperature at output from a dehydration zone – 380-4200С

4. Product temperature at output from the unit – 800С

5. Compressed air pressure – 4-6 atm

6. Compressed air temperature – 700С

7. Gas consumption for the top fire chamber – 33-35 nm3/h

for the bottom fire chamber – 35-40 nm3/h

8. Disodium phosphate solution consumption – 625 kg/h

9. Solution specific weight – 1,3-1,4 g/cm3

MATERIAL BALANCE OF THE MANUFACTURE

According to data of a Kazakh research institute Giprophosphor, 625 kg/h of a disodium phosphate  solution is fed to drying, from which 165 kg/h of sodium pyrophosphate are formed.

Dust loss from the drying-calcination unit makes 25 kg/h; from which 20 kg/h is returned in the calcination zone. Dust loss to a wet purification stage is 5 kg/h.

160 kg/h of waterless sodium pyrophosphate  is fed to dissolution.

TECHNICAL CHARACTERISTIC OF BY-PRODUCTS AND REALIZABLE PRODUCTION WASTES; THEIR APPLICATION AREAS AND RECYCLING METHODS

At the manufacturing a waterless sodium pyrophosphate in accordance with the given process flowsheet by-products and realizable production wastes are not formed.

RECOMMENDATIONS AND REQUIREMENTS FOR DESIGNING AND SELECTION OF THE BASIC PROCESS EQUIPMENT

A main apparatus of the installation for manufacturing sodium pyrophosphate in a reactive phosphorus salt shop is a direct-counterflow spraying drying-calcination unit with volume of 56 m3, diameter of 3200 mm, equipped with four calcination and two cooling shelves.

Calculation of the direct-counterflow drying-calcination unit for the purpose of establishment of a possibility of the set productivity assurance (165 kg/h of a waterless sodium pyrophosphate) is represented below.

The calculations have shown that the specified unit with volume of 56 m3 can ensure the required productivity and cooling the obtained product up to temperature of 500С.

4.4POTASSIUM ORTHOPHOSPHATES PROPERTIES AND

APPLICATION ARES

Potassium phosphates are potassium salts of orthophosphoric acid: mono-, di- and tripotassium phosphates.

Monosubstituted potassium phosphate represents colorless easily soluble in water crystals generally of white color; decomposition temperature – 252,60С, thermal capacity of a crystalline substance – 0,8370 kJ/kg∙К, viscosity of water mono potassium phosphate solutions decreases with temperature increase (approximately in 2,7 times at temperature growth from 20 to 800С).

Chemical formula – КН2РО4

Molecular weight – 136,083

Disubstituted potassium phosphate represents colorless white crystals easily soluble in water; at 2820С it decomposes and at 3500С turns into potassium pyrophosphate. Viscosity of water disubstituted potassium phosphate solutions decreases with temperature increase; at temperature growth from 20 to 800С viscosity decreases approximately in 4 times.

Chemical formula – К2НРО43Н2О

Molecular weight – 228,3.

Trisubstituted potassium phosphate represents colorless white crystals easily soluble in water and insoluble in alcohol. It melts at 13400С. Viscosity of water trisubstituted potassium phosphate solutions decreases with temperature growth; at temperature increase from 20 to 800С viscosity increases approximately in 4 times.

Chemical formula – К3РО47H2О

Molecular weight – 338,39

Application area. Potassium phosphates are applied as laboratory reactants, in manufacturing pharmaceutical preparations, galvanoplasty. Monopotassium phosphate is applied in photography and to complex fertilizer production. Dipotassium phosphate is used as a buffer agent to maintenance of рН=9 in antifreezes, thereby protecting metals from corrosion and as a semiproduct for manufacturing potassium pyrophosphate. Tripotassium phosphate is used for hydrogen sulphide absorption from industrial gases, oxygen removal from gas mixtures. Monopotassium and dipotassium phosphates are applied to preparation of the electrolyte, which is used for obtaining high-pure gold coverings; effect of these salts can considerably change a porous aluminum silicate structure.

Wide use of potassium orthophosphates is caused by simplicity of their purification and recrystallization from water solutions.

Potassium orthophosphates production techniques

Raw materials for manufacturing potassium orthophosphates are mainly reactive phosphoric acid, potassium hydroxide, potassium chloride and potassium carbonate.

Potassium orthophosphates can be produced by following methods: 1) phosphoric acid neutralization KOH or К2СО3; 2) decomposition of potassium chloride by phosphoric acid at ≈ 2500С with the subsequent neutralization of the solution obtained with К2СО3 or KOH and drying in a spray-type dryer; 3) processing potassium metaphosphate under pressure in the presence of water steam or its interaction with nitric acid; 4) potassium chloride decomposition by phosphoric acid in a water-alcohol medium; 5) electrolysis of a potassium chloride solution; 6) by means of liquid phase anion exchange.

For example, obtaining alkaline metal orthophosphates is based on neutralization of phosphoric acid containing 40-60% of Р2О5 by an alkali (a 40-70% solution), evaporation of the solution obtained and crystallization of a precipitate in one or several vacuum evaporators [A Germany patent].

There is a technique of manufacturing alkaline metal mono-, di-, triphosphates or their mixtures by means of dispersion of a mixture of two solutions – phosphoric acid and an alkaline metal hydroxide [A French patent].

It should be noted a way of manufacturing an alkaline metal monosubstituted phosphate solution, which consists in mixing phosphoric acid with an alkali solution by means of a circulating pump, and the alkali solution is fed through an in-out pipe.

Dipotassium phosphate can be obtained by feeding a 10% solution of potassium hydroxide in methanol into a phosphoric acid solution to рН=8,5-9,2 to a ratio of 1:1. The formed dipotassium phosphate insoluble in methanol is crystallized with three water molecules and dried up at 40-500С within 1-2 hours.

Monopotassium phosphate can be produced at decomposition of potassium chloride by phosphoric acid at temperature of 2500С; however this process is realized only on 70-75%; to remove all chlorine it is necessary to apply a big phosphoric acid excess. The monopotassium phosphate produced by such the way contains the surplus acid, and for its neutralization it is necessary to add potassium carbonate, potassium hydroxide or ammonia, that raises the price of process.

There are other potassium orthophosphate production techniques, for example, of potassium metaphosphate in conditions of high pressure and with use of water steam or at processing of potassium metaphosphate by nitric acid, but these ways have not found industrial application for economic reasons.

A way of manufacturing potassium phosphate by means of phosphoric-acid decomposition of potassium chloride in a water-alcohol medium is also known; in the process potassium phosphate precipitates, and hydrogen chloride remains in the reaction medium.

It is suggested to obtain potassium phosphate by processing of potassium chloride with phosphoric acid at heating with the subsequent cooling and separation of a product from a solution.

Potassium phosphates can be obtained in mercury cathode electrolyzers by means of electrolysis of a saturated potassium chloride solution. Chlorine is formed on an anode and potassium amalgam – on a cathode. The potassium amalgam is subjected to decomposition at 35-1000С by water solutions of orthophosphoric or polyphosphoric acids; the process products are hydrogen, mercury, a solution and solid particles of potassium orthophosphate or polyphosphate.

A way of obtaining potassium phosphates out of brine provides for selective sedimentation of potassium as an insoluble or sparingly soluble salt. The precipitated potassium salt is turned into potassium phosphate by means of interaction of this salt with phosphoric acid during a single-phase liquid-phase anion exchange process in which an organic amine together with a liquid organic inert thinner is used as an extraction agent.

All the potassium orthophosphate production ways developed abroad are complex and expensive processes demanding additional expenses for purification from chlorine and regeneration of extraction agents.

Potassium orthophosphates have been produced at a Shymkent industrial association “Fosfor”.

The process consisted in neutralization of a potassium hydroxide solution by a reactive orthophosphoric acid. The solution obtained was evaporated and cooled; a salt was crystallized in the course of cooling.

A crystallization process is widely applied in technology of phosphoric-acid salts for separation of components from solutions and often it is one of the important technological operations influencing chemical purity and yield of a ready product.

References

- Postnikov N.N. Dry-process phosphoric acid, salts and fertilizers on its basis. M.: Khimiya, 1976. - P.297.

- Production schedule № 8. Trisubstituted potassium phosphate production. Shymkent: ShIA “Fosfor”, 1975.

- Production schedules № 50, 51. Monosubstituted potassium phosphate production. Disubstituted potassium phosphate trihydrate production.

Марина Михайловна Ескендирова

Алмагул Аккопеевна Кадирбаева

«Химия и технология реактивных кислот и солей»

учебное пособия

Редактор Есккендирова М.М.

Подписано в печать

Формат бумаги XxY 1/16

Бумага типографская. Печать офсетная. Объем  6.5 п.л.

Тираж….экз. Заказ №….

© Издание Южно-Казахстанского государственного университета

им. М.Ауезова

Издательский центр ЮКГУ им. М.Ауэзова, г. Шымкент, пр. Тауке хана, 5