Ф.7.03-03

Saribayev A.S., Umbetov A.Zh

Methodical instructions to the laboratory work

on discipline

Distribution of radio waves and antenna-feeder device

Shymkent, 2017

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

M.AUEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

Department: «Automation, Telecommunication and Control»

Saribayev A.S., Umbetov A.Zh.

Methodical instructions to the laboratory work

on discipline

Distribution of radio waves and antenna-feeder device

for students of the specialty 5В071900 – Radio engineering, electronics and telecommunications

Shymkent – 2017

УДК 658.012.011

Authors: Saribayev A.S., Umbetov A.Zh. Methodical instructions to the laboratory work on discipline: Radio wave propagation and antenna-feeder devices - Shymkent:  M.Auezov SKSU,  2017 - 84p.

Methodical instructions are made according to requirements of the curriculum and the program of discipline Radio wave propagation and antenna-feeder devices and includes all necessary data on execution of a laboratory work.

Methodical instructions are intended for students of specialty 5B071900 – Radio engineering, electronics and telecommunications

Methodical instructions contain the purpose, the contents, rules of design and the job for execution of a laboratory work.

Reviewer:

Zhukova T.A. - Ph.D., associate professor of chair «Information technology and telecommunications» Miras University

Balabekova M.O. - c.t.s., the associate professor of chair «Automation, telecommunications and control» SKSU of M.Auezov

Methodical instructions are considered and recommended for printing by chair meeting of Automation, telecommunication and management (the report № __ from __ ___ 201__) and Methodical provision of higher school “Information technology and energetic” (the report № __ from __ 201___).

It is recommended to the edition Methodical council SKSU of M.Auezov. The report № __ from __ _______ 201___.

M.Auezov South Kazakhstan State university, 2016

Responsible for release Saribayev A.S., Umbetov A.Zh.

Contents

Introduction………………………………………………………………………………. 6

Laboratory 1 - An introduction to the DATEx experimental add-in module	8

Laboratory 2 - An introduction to soft front panel control	26

Laboratory 3 - Using the Emona DATEx to model equations	37

Laboratory 4 - Amplitude modulation (AM)	43

Laboratory 5 - Double Sideband (DSBSC) modulation	53

Laboratory 6 - Observations of AM and DSBSC signals in the frequency domain	61

Laboratory 7 - AM demodulation	72

Recommended

INTRODUCTION.

The purposes and tasks of a LECTURE COMPLEX

The aim of studying the subject is achieving the following learning outcomes (on the basis of Bloom’s taxonomy): knowledge, comprehension, application, analysis, evaluation, synthesis.

Knowledge - the student should be able to reproduce received practical skills in the field of theory of transmission of electromagnetic waves. He needs to know the current state of the methods, trends and achievements in the field of hardware system design of Radio wave propagation and antenna-feeder devices.

Application - the student must apply and use creative study of the laws, procedures, standard solutions and theoretical knowledge in specific practical situations in the development of devices; presumably describe future consequences arising from the available data and the methods used to develop. The student should be able to compile, convert, defend, paraphrase, interpret and give examples on the decisions adopted within the devices.

Analysis - the student should be able to identify the relationship between facts and consequences, evaluate the significance of data and results, compare and contrast the various methods of implementation of the theory of transmission of electromagnetic waves. The student must be able to assess the reproducibility of the significance and adequacy of the data obtained during the development of Radio wave propagation and antenna-feeder devices.

Evaluation - the student should able to evaluate logic of procedure of development of devices of a broadcasting, receiving results and the written explanation of rules and spheres of its functioning and boundaries of use; to estimate the significance of application of the developed model. To be able to evaluate, criticize, discuss, justify, disputing the received results.

Synthesis - the student should be able offers the plan of carrying out experiment and other actions in case of the solution of problems of systems and devices of satellite, mobile and optical communication. To be able to combine the experimental technique and processing of their results, possessing novelty and adequacy.

Objectives of the course.

According to the listed above purposes in result of study of discipline the student shall gain theoretical knowledge, practical skills in the development area of Systems and devices of satellite, mobile and optical communication. In the course of development of the discipline the student should learn the basics of systems of systems and devices of satellite, mobile and optical communication, master the basic functions of devices, structures, methods of system design, debugging and testing methods devices. Teaching the course "Radio wave propagation and antenna-feeder devices" must meet the current state of the discipline, to form a specialist firm foundations of knowledge, high culture and practical skills that are sufficient for the successful production activity and allows it to yourself to learn new necessary knowledge and achievements in the field of radio and solve engineering problems.

As a result of studying the discipline, students should be able to develop the structure of electronic circuits of various devices, organize, depending on the requirements of the task required the device structure, to be able to choose the right methods for solving problems of radio wave propagation and antenna-feeder devices

Laboratory 1 - An introduction to the DATEx experimental add-in module

Preliminary discussion

The Emona DATEx experimental add-in module for the NI ELVIS II is used to help people learn about communications and telecommunications principles. It lets you bring to life the block diagrams that fill communications textbooks. A "block diagram" is a simplified representation of a more complex circuit. An example is shown in Figure 1 below.

Block diagrams are used to explain the principle of operation of electronic systems (like a radio transmitter for example) without having to describe the detail of how the circuit works. Each block represents a part of the circuit that performs a separate task and is named according to what it does. Examples of common blocks in communications equipment include the adder, filter, phase shifter and so on.

The DATEx has a collection of blocks (called modules) that you can put together to implement dozens of communications and telecommunications block diagrams.

The experiment

This experiment is in three stand-alone parts (2-1, 2-2 and 2-3) and each introduces you to one or more of the DATEx's analog modules. It's expected that you've completed Experiment 1 or have already been introduced to the NI ELVIS II system and its virtual instruments software.

It should take you about 50 minutes to complete experiment 2.1, another 50 minutes to complete 2.2 and about 25 minutes to complete 2.3.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

- For 2.1 only: One set of headphones (stereo)

Some things you need to know for the experiment

This box contains definitions for some electrical terms used in this experiment. Although you've probably seen them before, it's worth taking a minute to read them to check your understanding.

Two signals that are in phase with each other reach key points in the waveform (like the peaks and zero-crossing points) at exactly the same time regardless of their size.

Two signals that out of phase reach key points in the waveform at different times. An example is shown in Figure 3 below.

Phase difference describes how much two signals are out of phase and is measured in degrees (like degrees in a circle). Signals that are in phase have a phase difference of 0°. Signals that are out of phase have a phase difference > 0° but < 360°.

A sinewave is a repetitive signal with the shape shown in Figure 2.

A cosine wave is simply a sinewave that is out of phase with another sinewave by exactly 90°. A sinewave and a cosine wave are shown in Figure 3. (They're not marked because, in this case, it doesn't matter which one is which.)

2.1 - The Master Signals, Speech and Amplifier modules The Master Signals module

The Master Signals module is an AC signal generator or oscillator. The module has six outputs providing the following:

Analog	Digital

- A 2.083kHz sinewave	■ A 2.083kHz squarewave (digital)

- A 100kHz sinewave	■ An 8.33kHz squarewave (digital)

- A 100kHz cosine wave	■ A 100kHz squarewave (digital)

Each signal is available on a socket on the module's faceplate that's labelled accordingly. Importantly, all signals are synchronised.

Procedure

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to Manual.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software per the instructor's directions.

Note: If the NI ELVISmx software has launched successfully, the window called "ELVIS - Instrument Launcher" will be visible.

    - Connect the set-up shown in Figure 1 below.

This set-up can be represented by the block diagram in Figure 2 below.

    - Launch and run the NI ELVIS II Oscilloscope and set it up per the procedure in Experiment 1 (page 1-12) ensuring that the Trigger Source control is set to CH 0.

    - Adjust the scope's Timebase control to view only two or so cycles of the Master Signals module's 2kHz SINE output.

    - Use the scope's measuring function to find the amplitude (peak-to-peak) of the Master Signals module's 2kHz SINE output. Record this in Table 1 on the next page.

    - Measure and record the frequency of the Master Signals module's 2kHz SINE output.

    - Repeat Steps 12 to 14 for the Master Signals module's other two analog outputs.

| Table 1 | Output voltage | Frequency |
| --- | --- | --- |
| 2kHz SINE |  |  |
| 100kHz COSINE |  |  |
| 100kHz SINE |  |  |

Ask the instructor to check your work before continuing.

You have probably just found that there doesn't appear to be much difference between the Master Signals module's SINE and COSINE outputs. They're both 100kHz sinewaves. However, the two signals are out of phase with each other.

It's critical to the operation of several communications and telecommunications systems that there be two (or more) sinewaves with identical frequencies but out of phase with each other (usually by a specific amount). The Master Signals module's two 100kHz outputs satisfy this requirement and are 90° out of phase. The next part of the experiment lets you see this.

  - Connect the set-up shown in Figure 3 below.

Note: Insert the black plugs of the oscilloscope leads into a ground (6NC>) socket.

8. Activate the scope's Channel 1 input by checking (that is, putting a tick in) the Channel 1 Enabledbox as shown in Figure 4 below.

Note 1: When you do, you should see a second signal appear on the display that's a different colour to the Channel 0 signal.

Note 2: You may notice that the two signals don't look like the clean sinewaves that you saw earlier. Importantly, the signals haven't changed shape. The distortion tells us that we're beginning to operate the NI ELVIS II at the limits of its specification (for reasons not discussed here).

Question 1

By visual inspection of the scope's display, which of the two signals is leading the other? Explain your answer.

Ask the instructor to check your work before continuing.

The Speech module

Sinewaves are important to communications. They're used extensively for the carrier signal in many communications systems. Sinewaves also make excellent test signals. However, the purpose of most communications equipment is the transmission of speech (among other things) and so it's useful to examine the operation of equipment using signals generated by speech instead of sinewaves. The Emona DATEx allows you to do this using the Speech module.

- Deactivate the scope's Channel 1 input.

- Set the scope's Time base control to the 2ms/div position.

- Connect the set-up shown in Figure 5 below.

Note: Insert the oscilloscope lead's black plug into a ground (6ND) socket.

- Talk and hum into the microphone while watching the scope's display. Be sure to say "one" and "two" several times.

Ask the instructor to check your work before continuing.

The Amplifier module

Amplifiers are used extensively in communications and telecommunications equipment. They're often used to make signals bigger. They're also used as an interface between devices and circuits that can't normally be connected. The Amplifier module on the Emona DATEx can do both.

- Locate the Amplifier module and set its Gain control to about a third of its travel.

- Connect the set-up shown in Figure 6 below.

Note: Insert the black plugs of the oscilloscope leads into a ground (GND) socket.

This set-up can be represented by the block diagram in Figure 7 below.

Adjust the scope's Timebase control to view two or so cycles of the Amplifier module's input.

- Activate the scope's Channel 1 input.

- Measure the amplitude (peak-to-peak) of the Amplifier module's input. Record your measurement in Table 2 below.

- Measure and record the amplitude of the Amplifier module's output.

The measure of how much bigger an amplifier's output voltage is compared to its input voltage is called voltage gain (AV). An amplifier's voltage gain can be expressed as a simple ratio calculated using the equation:

Importantly, if the amplifier's output signal is upside-down compared to its input then a negative sign is usually put in front of the gain figure to highlight this fact.

Question 2

Calculate the Amplifier module's gain (on its present gain setting).

The Amplifier module's gain is variable. Usefully, it can be set so that the output voltage is smaller than the input voltage. This is not amplification at all. Instead it's a loss or attenuation. The next part of the experiment shows how attenuation affects the gain figure.

- Turn the Amplifier module's Gain control fully anti-clockwise then turn it clockwise just a little until you can just see a sinewave.

- Resize the Amplifier module's output signal on the scope's display by adjusting Channel 1's Scale control to an appropriate setting.

- Measure and record the amplitude of the Amplifier module's new output.

Table 3

| Input voltage | Output voltage |
| --- | --- |
| See Table 2 |  |

Question 3

Calculate the Amplifier module's new gain.

Question 4

In terms of the gain figure, what's the difference between gain and attenuation?

Amplifiers work by taking the DC power supply voltage and using it to make a copy of the amplifier's input signal. Obviously then, the DC power supply limits the size of the amplifier's output. If the amplifier is forced to try to output a signal that is bigger than the DC power supply voltages, the tops and bottoms of the signal are chopped off. This type of signal distortion is called clipping.

Clipping usually occurs when the amplifier's input signal is too big for the amplifier's gain. When this happens, the amplifier is said to be overdriven. It can also occur if the amplifier's gain is too big for the input signal. To demonstrate clipping:

- Turn the Amplifier module's Gain control fully clockwise.

- Resize the Amplifier module's output signal on the display by adjusting Channel 1's Scale control to an appropriate setting.

Question 5

What do you think the output signal would look like if the amplifier's gain was sufficiently large?

Ask the instructor to check your work before continuing.

24. Turn the Amplifier module's Gain control fully anti-clockwise.

Headphones are typically low impedance devices - usually around 50Q. Most electronic circuits are not designed to have such low impedances connected to their output. For this reason, headphones should not be directly connected to the output of most of the modules on the Emona DATEx.

However, the Amplifier module has been specifically designed to handle low impedances. So, it can act as a buffer between the modules' outputs and the headphones to let you listen to signals. The next part of the experiment shows how this is done.

  - Ensure that the Amplifier module's Gain control is turned fully anti-clockwise.

  - Without wearing the headphones, plug them into the Amplifier module's headphone socket.

  - Put the headphones on.

  - Turn the Amplifier module's Gain control clockwise and listen to the signal.

  - Disconnect the plugs from the Master Signals module's 2kHz SINE output and connect them to the Speech module's output.

  - Speak into the microphone and listen to the signal.

  - Disconnect the plugs from the Speech module's output and connect them to the Master Signals module's 100kHz SINE output.

  - Carefully turn the Amplifier module's Gain control clockwise and listen for a signal.

Question 6

Why is the Master Signals module's 100kHz SINE output inaudible?

  - Turn the Amplifier module's Gain control fully anti-clockwise again.

Ask the instructor to check your work before finishing.

2.2 - The Adder and Phase Shifter modules The Adder module

Several communications and telecommunications systems require that signals be added together. The Adder module has been designed for this purpose.

Procedure

    - If your equipment is still set up from the previous experiment then jump to Step 9. If not, continue on to Step 2.

    - Ensure that the NI ELVIS II power switch at the back of the unit is off.

    - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

    - Set the Control Mode switch on the DATEx module (top right corner) to Manual.

    - Connect the NI ELVIS II to the PC using the USB cable.

Note: This may already have been done for you.

    - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

    - Turn on the PC and let it boot-up.

    - Launch the NI ELVISmx software per the instructor's directions.

Note: If the NI ELVISmx software has launched successfully, the window called "ELVIS - Instrument Launcher" will be visible.

Ask the instructor to check your work before continuing.

    - Launch and run the NI ELVIS II Oscilloscope and set it up per the procedure in Experiment 1 (page 1-12) ensuring that the Trigger Source control is set to CH 0.

    - Locate the Adder module and turn its g control (for Input B) fully anti-clockwise.

    - Set the Adder module's G control (for Input A) to about the middle of its travel.

    - Connect the set-up shown in Figure 1 below.

Note: Although not shown, insert the black plugs of the oscilloscope leads into a ground (GND) socket.

This set-up page can be represented by the block diagram in Figure 2 below.

    - Adjust the scope's Timebase control to view two or so cycles of the Master Signals module's 2kHz SINE output.

    - Activate the scope's Channel 1 input (by checking the Channel 1 Enabled box) to view the Adder module's output as well as the Master Signals module's 2kHz SINE output.

    - Vary the Adder module's G control left and right and observe the effect. Question 1

What aspect of the Adder module's performance does the G control vary?

    - Use the scope's measuring function to measure the voltage on the Adder module's Input A. Record your measurement in Table 1 below.

    - Turn the Adder module's Gcontrol fully clockwise.

    - Measure and record the Adder module's output voltage.

    - Calculate and record the voltage gain of the Adder module's Input A.

    - Turn the Adder module's Gcontrol fully anti-clockwise.

    - Resize the Adder module's output signal on the scope's display by adjusting Channel 1's Scale control to an appropriate setting.

    - Repeat Steps 18 and 19.

Question 2

What is the range of gains for the Adder module's A input?

Ask the instructor to check your work before continuing.

    - Leave the Adder module's £ control fully anti-clockwise.

    - Disconnect the Master Signals module's 2kHz SINE output from the Adder module's Input A and connect it to the Adder's Input B.

    - Turn the Adder module's gcontrol fully clockwise.

    - Resize the Adder module's output signal on the scope's display by adjusting Channel 1's Scale control to an appropriate setting.

    - Measure the Adder module's output voltage. Record your measurement in Table 2 below.

    - Calculate and record the voltage gain of the Adder module's Input B.

    - Turn the Adder module's gcontrol fully anti-clockwise.

    - Repeat Steps 26 to 28.

Question 3

Compare the results in Tables 1 and 2. What can you say about the Adder module's two inputs in terms of their gain?

    - Turn both of the Adder module's Gain controls fully clockwise.

    - Connect the Master Signals module's 2kHz SINE output to both of the Adder module's inputs.

    - Resize the Adder module's output signal on the scope's display by adjusting Channel 1's Scale control to an appropriate setting.

    - Measure the Adder module's new output voltage. Record your measurement in Table 3 below.

Question 4

What is the relationship between the amplitude of the signals on the Adder module's inputs and output?

Ask the instructor to check your work before continuing.

The Phase Shifter module

Several communications and telecommunications systems require that the signal to be transmitted (speech, music and/or video) is phase shifted. Crucial to being able to implement these systems in later experiments is the ability to phase shift any signal by almost any amount. The Phase Shifter module has been designed for this purpose.

    - Locate the Phase Shifter module and set its Phase Change switch to the 0° position.

    - Set the Phase Shifter module's Phase Adjust control to about the middle of its travel.

    - Connect the set-up shown in Figure 3 below.

Note 1: Insert the black plugs of the oscilloscope leads into a ground (6ND) socket.

Note 2: The LED on the Phase Shifter module will turn on but don't be concerned by this. The LED is used to indicate that the module has automatically adjusted itself for your low frequency input.

The set-up in Figure 3 can be represented by the block diagram in Figure 4 below.

    - Adjust the scope's Scale control for both channels for signals that are a suitable size on the display.

    - Vary the Phase Shifter module's Phase Adjust control left and right and observe the effect on the two signals.

    - Set the Phase Shifter module's Phase Change control to the 180° position.

    - Vary the Phase Shifter module's Phase Adjust control left and right and observe the effect on the two signals.

Question 5

This module's output signal can be phase shifted by different amounts

- but it always leads the input signal.

- but it always lags the input signal.

- and can either lead or lag the input signal.

Ask the instructor to check your work before finishing.

2.3 - The Voltage Controlled Oscillator (VCO)

A VCO is an oscillator with an adjustable output frequency that is controlled by an external voltage source. It's a very useful circuit for communications and telecommunications systems as you'll see. The NI ELVIS II Function Generator's operation can be modified by the Emona DATEx to function as a VCO if required.

Procedure

  - If your equipment is still set up from the previous experiment then jump to Step 9. If not continue on to Step 2.

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to Manual.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software per the instructor's directions.

Note: If the NI ELVISmx software has launched successfully, the window called "ELVIS - Instrument Launcher" will be visible.

Ask the instructor to check your work before continuing.

  - Launch and run the NI ELVIS II Oscilloscope and set it up per the procedure in Experiment 1 (page 1-12) ensuring that the Trigger Source control is set to CH 0.

  - Set the NI ELVIS II Variable Power Supplies' controls (near the top right of the unit) as follows:

- Positive Voltage to the 0Vposition (that is, fully anti-clockwise)

- Negative Voltage to the 0Vposition (that is, fully anti-clockwise)

  - Launch the NI ELVIS II Variable Power Supplies' virtual instrument and check the Manual boxes to put them in to manual mode.

Note: Recall that checking these boxes hands control of the Variable Power Supplies' outputs over to the controls that you adjusted for Step 10.

  - Launch and run the NI ELVIS II Function Generator virtual instrument.

  - Type 5000 in the function generator's Hz window and press Enter on the keyboard.

Note: Once you've done this, the window's value will change to "5k" as shown in Figure 1 below.

  - Adjust the function generator for a 4Vpp output.

  - Set the function generator's Modulation Type to FM. Note: This step puts the function generator in to VCO mode.

  - Connect the set-up shown in Figure 2 below.

Note: Although not shown, insert the black plug of the oscilloscope lead into a ground (GND) socket.

  - Adjust the scope's Timebase control to view two or so cycles of the function generator's output.

  - Use the scope's measuring function to find the frequency of the function generator's output. Record your measurement in Table 1 below.

  - Modify the set-up as shown in Figure 3 below.

Before you do...

The set-up in Figure 3 builds on Figure 2 so don't pull it apart. Existing wiring is shown as dotted lines to highlight the patch leads that you need to add.

This set-up can be represented by the block diagram in Figure 4 below.

    - Activate the scope's Channel 1 input to view the function generator's DC input voltage as well as its AC output voltage.

    - Set the scope's Channel 1 Scale control to the 5V/divposition.

    - Set the scope's Channel 1 Coupling control to the DC position.

    - Increase the Variable Power Supplies' positive output voltage while watching the scope's display.

Question 1

What happens to the function generator's output when you increase its positive DC input voltage?

    - Set the Variable Power Supplies' positive output voltage to 10V.

    - Measure the function generator's new output frequency. Record your measurement in Table 2 below.

Question 2

Use the information in Tables 1 and 2 to determine the function generator's VCO sensitivity (that is, how much its output frequency changes per volt).

Note: The VCO sensitivity is different for ELVIS I and ELVIS II. Be aware of this difference if you have both in your lab. You may wish to compare the difference if you have the time. If you do, make sure that you take readings at several frequencies across the range.

Ask the instructor to check your work before continuing.

Importantly, the function generator's VCO sensitivity is different for different frequency decades.

26. Repeat this process to determine the sensitivity of the function generator's VCO at 500Hz and 50kHz. Record your measurements in Table 3 below.

Ask the instructor to check your work before continuing.

    - Modify the set-up as shown in Figure 5 below.

This set-up can be represented by the block diagram in Figure 6 below.

28. Increase the Variable Power Supplies' negative output voltage while watching the scope's display.

Question 3

What happens to the function generator's output when you increase its negative DC input voltage?

Ask the instructor to check your work before finishing.

Laboratory 2 - An introduction to soft front-panel control

Preliminary discussion

The "front-panel" of an electronics system is the face of the unit that contains most if not all of the controls that the user can adjust to vary the system's performance in some way. As an example, the NI ELVIS II front- panel is shown in Figure 1.

Over the last 20 to 30 years, digital control electronics has dramatically changed the front-panel. Multiple-pole ganged switches and potentiometers have largely given way to momentary buttons and infinite-turn rotary devices. For examples of these, think of how you change the station or volume on a car or home stereo system these days.

The digital takeover of system control has also made true remote control over systems possible. As you know, most domestic electronic devices these days can at least be turned on and off from an infrared (IR) or radio frequency (RF) remote device. In fact, for modern televisions and video recording devices there are more controls on the remote than on the unit itself. In other words, the remote control has become the front-panel.

Advances in personal computers (PCs) and digital data communications have provided for a different type of remote control for non-domestic applications such as data acquisition and industrial process control. For this type of equipment, the front-panel is either duplicated or replaced altogether by a "soft" front-panel on a computer screen that can be metres or thousands of kilometres away from the equipment being controlled. Soft front-panels have virtual buttons and knobs that, when adjusted on screen, result in changes in a system's performance as though a real button or knob had been adjusted.

You have seen this type of control before if you've attempted Experiments 1 and 2. The NI ELVIS II DMM and Oscilloscope are instruments without any hard controls. You operated them by using virtual buttons and knobs on a computer screen. The NI ELVIS II Variable Power Supplies and Function Generator and the Emona DATEx can be controlled in the same way.

The experiment

This experiment introduces you to soft front-panel control of the NI ELVIS II test equipment and the Emona DATEx experimental add-in module. It is expected that you've completed Experiment 1 or have already been introduced to the NI ELVIS II system and its virtual instruments software.

It should take you about 40 minutes to complete this experiment.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Something you need to know for the experiment

This box contains the definition for an electrical term used in this experiment. Although you've probably seen it before, it's worth taking a minute to read it to check your understanding.

When two signals are 180° out of phase, they're out of step by half a cycle. This is shown in Figure 2 below. As you can see, the two signals are always travelling in opposite directions. That is, as one goes up, the other goes down (and vice versa).

Procedure

Part A - Soft control of the NI ELVIS II Variable Power Supplies and Function Generator

- Ensure that the NI ELVIS II power switch at the back of the unit is off.

- Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

- Set the Control Mode switch on the DATEx module (top right corner) to Manual.

- Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

- Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

- Turn on the PC and let it boot-up.

1. Launch the NI ELVISmx software per the instructor's directions.

Note: If the NI ELVISmx software has launched successfully, the window called "ELVIS - Instrument Launcher" will be visible (see Figure 3).

  - Connect the set-up shown in Figure 4 below.

Ask the instructor to check your work before continuing.

  - Launch the NI ELVIS II DMM virtual instrument (VI) and click on its Run control.

  - Launch the NI ELVIS II Variable Power Supplies VI and clock on its Run control.

Note: On successfully launching these VIs the windows in Figure 5 below should be visible. Arrange them on the PC's desktop for your convenience.

  - Try adjusting the Variable Power Supplies' positive output hard Voltage control on the upper right side of the NI ELVIS II and note the DMM's reading as you do.

Note: You'll find that this has no effect of the Variable Power Supplies' positive output. This is because the default start-up condition of the VIs is for soft control over their instruments.

  - Vary the Variable Power Supplies' positive output voltage by using the mouse to "grab and turn" the soft Voltage control on-screen (see Figure 6).

Note: This time you should find that the output voltage changes. If not, call the instructor for assistance.

  - Set the Variable Power Supplies positive output voltage to exactly 7.59V by typing that value into the Vwindow as shown in Figure 7 below.

  - Preset the RESETbutton below the Vwindow and observe the effect.

  - Connect the DMM to the negative Variable DC output.

  - Repeat Steps 12 to 14 to affect the Variable Power Supplies' negative output.

Question 1

What is the advantage of being able to adjust the Variable Power Supplies using the soft front-panel?

Ask the instructor to check your work before continuing.

  - Close the Variable Power Supplies and DMM VIs.

  - Connect the set-up shown in Figure 8 below.

  - Launch and run the NI ELVIS II Oscilloscope and set it up per the procedure in Experiment 1 (page 1-12) with the following change:

■ Timebase control to the 5ms/divposition instead of 500ps/div

  - Launch the NI ELVIS II Function Generator VI and click on its Run control.

Note: On successfully launching these VIs the windows in Figure 9 below should be visible. Arrange them on the PC's desktop for your convenience.

  - Try adjusting the function generator's hard Amplitude control on the right side of the NI ELVIS II and note the scope's display as you do.

Note: For the same reasons as before, you'll find that the function generator's hard controls have no effect over its output.

  - Vary the function generator's soft Amplitude control (by grabbing and turning the knob on-screen in the same way as you did for the Variable Power Supplies) and observe the effect on the scope's display.

Note: This time you should find that the output voltage changes. If not, call the instructor for assistance.

  - Vary the function generator's soft Frequency control and observe the effect.

  - Select the other Waveform options and observe the effect.

  - Set the scope's Channel 0 Coupling control to the Deposition.

  - Vary the function generator's soft DC Offset control and observe the effect.

  - Adjust the function generator using its soft controls for an output with the following specifications:

Waveshape: Triangular Frequency: 2.5kHz Amplitude: 4Vpp DC Offset: 0.5V

Tip: To obtain exactly 2.5kHz at 4Vpp, simply type these values in the space provided below the corresponding knobs.

  - Make the appropriate adjustments to the scope's controls to display the waveform.

Ask the instructor to check your work before continuing.

An important fact about the NI ELVIS II A DATEx

On the bottom right corner of the function generator VI there is another changeable option called "Signal Route". This option defaults to "Prototyping board" meaning that the function generator's output is available on the DATEx board as demonstrated by this experiment. This is how almost all DATEx experiments access the function generator so "prototyping board" is the preferred signal route option unless otherwise specified.

For your information, the other signal route option is called "FGEN BNC". This outputs the function generator to the BNC connector marked FGEN on the top left side of the NI ELVIS II. However, this connector has a dual use and doubles as the external trigger input (TRIG) to the scope. Selecting the "F6EN BNC" signal route will cause conflicts when using the scope's external trigger option so it is best to avoid doing so.

Part В - Soft control of the Emona DATEx

  - Close the function generator's VI.

  - Connect the set-up shown in Figure 10 below.

  - Adjust the scope's Timebase control to view only two or so cycles of the Master Signals module's 2kHz SINE output.

  - Activate the scope's Channel 1 input by checking the Channel 1 Enabledbox.

  - Verify the operation of the Amplifier module by varying its hard Gain control.

Note: If the amplifier is working correctly, its output should be inverted and adjusting its Gain control should vary its amplitude.

  - Launch the DATEx soft front-panel (SFP) per the instructor's directions.

Note 1: On successfully launching the DATEx SFP, the window in Figure 11 on the next page should be visible.

Note 2: There is no need to "run" the DATEx SFP as it launches running.

Setting the DATEx SFP to the current Device number

The ELVIS unit you are using can be set up to have a particular Device number. You need to ensure that the DATEx SFP is also set to the same Device number as your ELVIS unit. The DATEx SFP can be set to numbers 1-8.

In the bottom right corner of the DATEx SFP is a Device selection box. Set it to the same device number as is your ELVIS unit. If the DATEx SFP is not set to the correct ELVIS number you will get an error dialog box. Simply dismiss the dialog, set the device number correctly and re-start the SFP.

In this example screenshot we can see the SFP set to Device number 3.

  - Adjust the positions of the DATEx SFP window and the scope's VI so that you're able to view the essential parts of both. An example is shown in Figure 12 below.

  - Switch the DATEx module's Control Mode switch (top right-hand corner) to the PC Control position.

  - Vary the Amplifier module's hard Gain control again. Note: This time it'll have no effect on the output signal.

  - Vary the Amplifier module's soft Gain control using the DATEx SFP and the mouse. Note: You should find that you now have soft control over the DATEx.

  - Use the Amplifier module's soft Gain control to set its voltage gain to as close to -2 as you can get.

If you find fine adjustments using the mouse are tricky, the DATEx SFP allows you to make changes to its soft controls using the PC's keyboard. The following instructions show you how.

  - Reposition the DATEx SFP window so that you can see all of its modules.

  - Press the keyboard's TAB key once.

Note: The Width control on the DATEx SFP's Twin Pulse Generator can now be adjusted using the keyboard and this is highlighted by a box around it.

  - Press the TAB key a few more times.

Note: Notice that each time you press the TAB key the selected control changes. Notice also that switches can be selected as wells as knobs.

  - Use the TAB key to select the Amplifier module's soft Gain control.

  - Reposition the DATEx SFP window so that you can see the scope's display.

  - Vary the soft Gain control by pressing the keyboard's left and right arrow keys.

Note: You'll have to watch the soft Gain control very closely to see it move because the adjustments are very fine.

  - Use the arrow keys to set the Amplifier module's voltage gain to as close to -2 as you can get.

Ask the instructor to check your work before continuing.

  - Connect the set-up shown in Figure 13 below.

    - Experiment with adjusting the Phase Shifter module's two soft controls while watching its input and output signals on the scope's display.

Note 1: Use the mouse and the keyboard to do this.

Note 2: See if you can work out which key on the keyboard toggles the Phase Shifter module's switch between the 0°and 180°positions.

    - Adjust the Phase Shifter module for an output signal with a phase shift that is as close to 180° as you can get.

Ask the instructor to check your work before finishing.

Loading pre-wired DATEx SFP backgrounds

The DATEx SFP background has the ability to be loaded with custom-made backgrounds. This can be used to load a "pre-wired" background which matches the experiment you are currently doing. As well, a background with notes from the instructor can be loaded if required.

As the loadable backgrounds are simply image files (jpg, bmp), any changes can be made as required by the user or instructor, using the template background file supplied.

As well, a directory of pre-wired experiment backgrounds corresponding to every wiring diagram in this manual is also supplied on the DATEX CD. Your instructor can advise you as to where these files can be found if required.

How to use pre-wired backgrounds:

"Load Wiring Diagram" : Select a background file (jpg, bmp) to load "Restore Default Background" : Load the supplied default un-wired background "Exit" : Exit the DATEx SFP "Help" : Load the Help screen (html)

Upon restart the DATEx SFP will always load the default background. A pre-wired background corresponding to Figure 13 of this chapter is shown below as Figure 14.

Figure 15 shows one of the supplied pre-wired backgrounds loaded into the DATEx SFP. In this example it is the wiring relating to Experiment 17, Figure 8 of the DATEx Lab Manual. The different colors are used to distinguish between the various stages of wiring during the experiment itself.

The loadable backgrounds can also be easily edited to include annotations the teacher may wish to add. In this example is shown a pre-wired background loaded into the SFP and including some advisory comments to students who would be doing this experiment.

4 - Using the Emona DATEx to model equations

Laboratory 3 - Using the Emona DATEx to model equations

Preliminary discussion

This may surprise you, but mathematics is an important part of electronics and this is especially true for communications and telecommunications. As you'll learn, the output of all communications systems can be described mathematically with an equation.

Although the math that you'll need for this manual is relatively light, there is some. Helpfully, the Emona DATEx can model communications equations to bring them to life.

The experiment

This experiment will introduce you to modelling equations by using the Emona DATEx to implement two relatively simple equations.

It should take you about 40 minutes to complete this experiment. Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Something you need to know for the experiment

This box contains the definition for an electrical term used in this experiment. Although you've probably seen it before, it's worth taking a minute to read it to check your understanding.

When two signals are 180° out of phase, they're out of step by half a cycle. This is shown in Figure 1 below. As you can see, the two signals are always travelling in opposite directions. That is, as one goes up, the other goes down (and vice versa).

Procedure

In this part of the experiment, you're going to use the Adder module to add two electrical signals together. Mathematically, you'll be implementing the equation:

Adder module output = Signal A + Signal В

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

Ask the instructor to check your work before continuing.

  - Launch and run the NI ELVIS II Oscilloscope virtual instrument (VI).

  - Set up the scope per the procedure in Experiment 1 (page 1-12) ensuring that the Trigger Source control is set to CH 0.

  - Launch the DATEx soft front-panel (SFP).

  - Check you now have soft control over the DATEx by activating the PCM Encoder module's soft PDM/TDM control on the DATEx SFP.

Note: If you're set-up is working correctly, the PCM Decoder module's LED on the DATEx board should turn on and off.

  - Locate the Adder module on the DATEx SFP and set its soft Gand g controls to about the middle of their travel.

  - Connect the set-up shown in Figure 2 below.

Note: Although not shown, insert the black plugs of the oscilloscope leads into a ground (GAD) socket.

This set-up can be represented by the block diagram in Figure 3 below.

  - Adjust the scope's Timebase control to view two or so cycles of the Master Signals module's 2kHz SINE output.

  - Measure the amplitude (peak-to-peak) of the Master Signals module's 2kHz SINE output. Record your measurement in Table 1 on the next page.

  - Disconnect the lead to the Adder module's B input.

  - Activate the scope's Channel 1 input by checking the Channel 1 Enabled box to observe the Adder module's output as well as its input.

  - Adjust the Adder module's soft £ control until its output voltage is the same size as its input voltage (measured in Step 15).

Note 1: This makes the gain for the Adder module's A input -1.

Note 2: Remember that you can use the keyboard's TAB and arrow keys for fine adjustment of the DATEx SFP's controls.

  - Reconnect the lead to the Adder module's B input.

  - Disconnect the lead to the Adder module's A input.

  - Adjust the Adder module's soft gcontrol until its output voltage is the same size as its input voltage (measured in Step 15).

Note: This makes the gain for the Adder module's B input -1 and means that the Adder module's two inputs should have the same gain.

  - Reconnect the lead to the Adder module's A input.

The set-up shown in Figures 3 and 4 is now ready to implement the equation:

Adder module output = Signal A + Signal B

Notice though that the Adder module's two inputs are the same signal: a 4Vp-p 2kHz sinewave. So, for these inputs the equation becomes:

Adder module output = 4Vp-p (2kHz sine) + 4Vp-p (2kHz sine)

When the equation is solved, we get:

Adder module output = 8Vp-p (2kHz sine) Let's see if this is what happens in practice.

  - Measure and record the amplitude of the Adder module's output.

Question 1

Is the Adder module's measured output voltage exactly 8Vp-p as theoretically predicted? Question 2

What are two reasons for this?

Ask the instructor to check your work before continuing.

In the next part of the experiment, you're going to add two electrical signals together but one of them will be phase shifted. Mathematically, you'll be implementing the equation:

Adder module output = Signal A + Signal В (with phase shift)

    - Locate the Phase Shifter module on the DATEx SFP and set its soft Phase Change control to the position.

    - Set the Phase Shifter module's soft Phase Adjust control about the middle of its travel.

    - Connect the set-up shown in Figure 4 below.

Note: Insert the black plugs of the oscilloscope leads into a ground (GAD) socket.

This set-up can be represented by the block diagram in Figure 5 on the next page.

The set-up shown in Figures 4 and 5 is now ready to implement the equation: Adder module output = Signal A + Signal В (with phase shift)

The Adder module's two inputs are still the same signal: a 4Vp-p 2kHz sinewave. So, with values the equation is:

Adder module output = 4Vp-p (2kHz sine) + 4Vp-p (2kHz sine with phase shift)

As the two signals have the same amplitude and frequency, if the phase shift is exactly 180° then their voltages at any point in the waveform is always exactly opposite. That is, when one sinewave is +1V, the other is -1V. When one is +3.75V, the other is -3.75V and so on. This means that, when the equation above is solved, we get:

Adder module output = 0Vp-p

Let's see if this is what happens in practice.

    - Adjust the Phase Shifter module's soft Phase Adjust control until its input and output signals look like they're about 180° out of phase with each other.

    - Disconnect the scope's Channel 1 lead from the Phase Shifter module's output and connect it to the Adder module's output.

    - Adjust Channel 1's Scale control to resize the signal on the display.

    - Measure the amplitude of the Adder module's output. Record your measurement in Table 2 below.

Question 3

What are two reasons for the output not being 0V as theoretically predicted?

Ask the instructor to check your work before continuing.

The following procedure can be used to adjust the Adder and Phase Shifter modules so that the set-up has a null output. That is, an output that is close to zero volts.

    - Use the keyboard's TAB and arrow keys to vary the Phase Shifter module's soft Phase Adjust control left and right a little and observe the effect on the Adder module's output.

    - Use the keyboard to make the necessary fine adjustments to the Phase Shifter module's soft Phase Adjust control to obtain the smallest output voltage from the Adder module.

Question 5

What can be said about the phase shift between the signals on the Adder module's two inputs now?

    - Use the keyboard to vary the Adder module's soft gcontrol left and right a little and observe the effect on the Adder module's output.

    - Use the keyboard to make the necessary fine adjustments to the Adder module's soft g control to obtain the smallest output voltage.

Question 6

What can be said about the gain of the Adder module's two inputs now?

You'll probably find that you'll not be able to null the Adder module's output completely. Unfortunately, real systems are never perfect and so they don't behave exactly according to theory. As such, it's important for you to learn to recognise these limitations, understand their origins and quantify them where necessary.

Laboratory 4 - Amplitude modulation (AM)

Preliminary discussion

In an amplitude modulation (AM) communications system, speech and music are converted into an electrical signal using a device such as a microphone. This electrical signal is called the message or baseband signal. The message signal is then used to electrically vary the amplitude of a pure sinewave called the carrier. The carrier usually has a frequency that is much higher than the message's frequency.

Figure 1 below shows a simple message signal and an unmodulated carrier. It also shows the result of amplitude modulating the carrier with the message. Notice that the modulated carrier's amplitude varies above and below its unmodulated amplitude.

Figure 2 below shows the AM signal at the bottom of Figure 1 but with a dotted line added to track the modulated carrier's positive peaks and negative peaks. These dotted lines are known in the industry as the signal's envelopes. If you look at the envelopes closely you'll notice that the upper envelope is the same shape as the message. The lower envelope is also the same shape but upside-down (inverted).

In telecommunications theory, the mathematical model that defines the AM signal is:

AM = (DC + message) * the carrier

When the message is a simple sinewave (like in Figure 1) the equation's solution (which necessarily involves some trigonometry that is not shown here) tells us that the AM signal consists of three sinewaves:

- One at the carrier frequency

- One with a frequency equal to the sum of the carrier and message frequencies

- One with a frequency equal to the difference between the carrier and message frequencies

In other words, for every sinewave in the message, the AM signal includes a pair of sinewaves - one above and one below the carrier's frequency. Complex message signals such as speech and music are made up of thousands sinewaves and so the AM signal includes thousands of pairs of sinewaves straddling carrier. These two groups of sinewaves are called the sidebands and so AM is known as double-sideband, full carrier (DSBFC).

Importantly, it's clear from this discussion that the AM signal doesn't consist of any signals at the message frequency. This is despite the fact that the AM signal's envelopes are the same shape as the message.

The experiment

For this experiment you'll use the Emona DATEx to generate a real AM signal by implementing its mathematical model. This means that you'll add a DC component to a pure sinewave to create a message signal then multiply it with another sinewave at a higher frequency (the carrier). You'll examine the AM signal using the scope and compare it to the original message. You'll do the same with speech for the message instead of a simple sinewave.

Following this, you'll vary the message signal's amplitude and observe how it affects the modulated carrier. You'll also observe the effects of modulating the carrier too much. Finally, you'll measure the AM signal's depth of modulation using a scope.

It should take you about 1 hour to complete this experiment. Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Procedure

Part A - Generating an AM signal using a simple message

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

Ask the instructor to check your work before continuing.

  - Launch the NI ELVIS II Variable Power Supplies VI and click on its Run control to activate the hardware.

  - Adjust the Variable Power Supplies negative output Voltage control for an output of about -6V (the exact value is not critical).

  - You won't need to adjust the Variable Power Supplies VI again so minimise it (but don't close it).

  - Connect the set-up shown in Figure 3 below.

Note: The NI ELVIS II DMM's inputs are on the left side of the unit.

  - Launch the NI ELVIS II DMM VI and press its Run button to activate its hardware.

  - Set up the DMM for measuring DC voltages.

  - Launch the DATEx soft front-panel (SFP).

  - Check you have soft control over the DATEx by activating the PCM Encoder module's soft POM/TOM control on the DATEx SFP.

Note: If you're set-up is working correctly, the PCM Decoder module's LED on the DATEx board should turn on and off.

  - Locate the Adder module on the DATEx SFP and turn its soft & control fully anti- clockwise.

  - Adjust the Adder module's soft g control to obtain a 1V DC output (as measured by the DMM).

  - Close the DMM VI - you'll not need it again (unless you accidentally change the Adder module's soft g control).

19. Connect the set-up shown in Figure 4 below.

Note 1: The NI ELVIS II scope inputs are on the left side of the unit.

Note 2: Insert the black plugs of the oscilloscope lead into a ground (GNC>) socket.

This set-up can be represented by the block diagram in Figure 5 below. It implements the highlighted part of the equation: AM = (DC + message) * the carrier.

    - Launch and run the NI ELVIS II Oscilloscope VI.

    - Set up the scope per the procedure in Experiment 1 (page 1-12) with the following changes:

- Channel 0 Coupling control to the Deposition instead of AC

- Channel 0 Scale control to the 500m V/divposition instead of 1V/div

- Trigger Level control to the 1Vposition instead of 0V

At the moment, the scope's display wont have a trace and there should be a message at the top stating, "***Waiting for Trigger***". This situation is normal and will be corrected when you perform the next step.

  - While watching the Adder module's output on the scope, turn its soft £ control clockwise to obtain a 1Vp-p sinewave.

Tip: Remember that you can use the keyboard's TAB and arrow keys for fine adjustment of the DATEx SFP's controls.

The Adder module's output can now be described mathematically as:

AM = (1VDC + 1Vp-p 2kHz sine) * the carrier

Question 1

In what way is the Adder module's output now different to the signal out of the Master Signals module's 2kHz SINE output?

Ask the instructor to check your work before continuing.

23. Modify the set-up as shown in Figure 6 below. Before you do...

The set-up in Figure 6 builds on Figure 4 so don't pull it apart. Existing wiring is shown as dotted lines to highlight the patch leads that you need to add.

This set-up can be represented by the block diagram in Figure 7 below. The additions that you've made to the original set-up implement the highlighted part of the equation: AM = (DC + message) * the carrier.

With values, the equation on the previous page becomes:

AM = (1VDC + 1Vp-p 2kHz sine) * 4Vp-p 100kHz sine.

    - Adjust the scope's Timebase control to view only two or so cycles of the message signal.

    - Activate the scope's Channel 1 input (by checking the Channel 1 Enabled box) to view the Multiplier module's output as well as the message signal.

    - Draw the two waveforms to scale on the graph provided below.

Tip: Draw the message signal in the upper half of the graph and the AM signal in the lower half.

Ask the instructor to check your work before continuing.

27. Use the scope's Channel 0 Position control to overlay the message with the AM signal's upper envelope then lower envelope to compare them.

Tip: If you haven't do so already, set the Channel 1 Scale control to 500mV/div. Question 2

What feature of the Multiplier module's output suggests that it's an AM signal? Tip: If you're not sure about the answer to the questions, see the preliminary discussion.

Question 3

The AM signal is a complex waveform consisting of more than one signal. Is one of the signals a 2kHz sinewave? Explain your answer.

Question 4

For the given inputs to the Multiplier module, how many sinewaves does the AM signal consist of, and what are their frequencies?

Part В - Generating an AM signal using speech

This experiment has generated an AM signal using a sinewave for the message. However, the message in commercial communications systems is much more likely to be speech and music. The next part of the experiment lets you see what an AM signal looks like when modulated by speech.

      - Disconnect the plug on the Master Signals module's 2kHz SINE output that connects to the Adder module's A input.

      - Connect it to the Speech module's output as shown in Figure 8 below. Remember: Dotted lines show leads already in place.

      - Set the scope's Timebase control to the Ims/divposition.

      - Hum and talk into the microphone while watching the scope's display.

Question 5

Why is there still a signal out of the Multiplier module even when you're not humming (or talking, etc)?

Ask the instructor to check your work before continuing.

Part С - Investigating depth of modulation

It's possible to modulate the carrier by different amounts. This part of the experiment let's you investigate this.

      - Return the scope's Timebase control to the 100ps/divposition.

      - Disconnect the plug to the Speech module's output and reconnect it to the Master Signals module's 2kHz SINE output.

Note: The scope's display should now look like your drawings on the graph paper on page 5-10.

      - Vary the message signal's amplitude a little by turning Adder module's soft & control left and right and notice the effect on the AM signal.

Question 6

What is the relationship between the message's amplitude and the amount of the carrier's modulation?

Ask the instructor to check your work before continuing.

You probably noticed that the size of the message signal and the modulation of the carrier are proportional. That is, as the message's amplitude goes up, the amount of the carrier's modulation goes up.

The extent that a message modulates a carrier is known in the industry as the modulation index (m). Modulation index is an important characteristic of an AM signal for several reasons including calculating the distribution of the signal's power between the carrier and sidebands.

Figure 9 below shows two key dimensions of an amplitude modulated carrier. These two dimensions allow a carrier's modulation index to be calculated.

The next part of the experiment lets you practise measuring these dimensions to calculate a carrier's modulation index.

      - Adjust the Adder module's soft £ control to return the message signal's amplitude to 1Vp-p.

      - Measure and record the AM signal's /'dimension. Record your measurement in Table 1 below.

      - Measure and record the AM signal's ф dimension.

      - Calculate and record the AM signal's depth of modulation using the equation below.

Ask the instructor to check your work before continuing.

A problem that is important to avoid in AM transmission is over-modulation. When the carrier is over-modulated, it can upset the receiver's operation. The next part of the experiment gives you a chance to observe the effect of over-modulation.

      - Increase the message signal's amplitude to maximum by turning the Adder module's soft & control to about half its travel then fully clockwise and notice the effect on the AM signal.

      - Set the scope's Channel 0 Scale control to 1V/divand the Channel 1 Scale control to 500m V/div.

      - Use the scope's Channel 0 Position control to overlay the message with the AM signal's envelopes and compare them.

Question 7

What is the problem with the AM signal when it is over-modulated? Question 8

What do you think is a carrier's maximum modulation index without over-modulation?

- A minus number

- 0

- 1

□	Greater than 1

42. Draw the two waveforms to scale in the space provided below.

Ask the instructor to check your work before finishing.

Laboratory 5- DSBSC modulation

Preliminary discussion

DSBSC is a modulation system similar but different to AM (which was explored in Experiment 5).

Like AM, DSBSC uses a microphone or some other transducer to convert speech and music to an electrical signal called the message or baseband signal. The message signal is then used to electrically vary the amplitude of a pure sinewave called the carrier. And like AM, the carrier usually has a frequency that is much higher than the message's frequency.

Figure 1 below shows a simple message signal and an unmodulated carrier. It also shows the result of modulating the carrier with the message using DSBSC.

So far, there doesn't appear to be much difference between AM and DSBSC. However, consider Figure 2 below. It is the DSBSC signal at the bottom of Figure 1 but with dotted lines added to track the signal's envelopes (that is, its positive peaks and negative peaks). If you look at the envelopes closely you'll notice that they're not the same shape as the message as is the case with AM (see Experiment 5 page 5-3 for an example).

Instead, alternating halves of the envelopes form the same shape as the message as shown in Figure 3 below.

Another way that DSBSC is different to AM can be understood by considering the mathematical model that defines the DSBSC signal:

DSBSC = the message * the carrier

Do you see the difference between the equations for AM and DSBSC? If not, look at the AM equation in Experiment 5 (page 5-3).

When the message is a simple sinewave (like in Figure 1) the equation's solution (which necessarily involves some trigonometry) tells us that the DSBSC signal consists of two sinewaves:

- One with a frequency equal to the sum of the carrier and message frequencies

- One with a frequency equal to the difference between the carrier and message frequencies

Importantly, the DSBSC signal doesn't contain a sinewave at the carrier frequency. This is an important difference between DSBSC and AM.

That said, as the solution to the equation shows, DSBSC is the same as AM in that a pair of sinewaves is generated for every sinewave in the message. And, like AM, one is higher than the unmodulated carrier's frequency and the other is lower. As message signals such as speech and music are made up of thousands of sinewaves, thousands of pairs of sinewaves are generated in the DSBSC signal that sit on either side of the carrier frequency. These two groups are called the sidebands.

So, the presence of both sidebands but the absence of the carrier gives us the name of this modulation method - double-sideband, suppressed carrier (DSBSC).

The carrier in AM makes up at least 66% of the signal's power but it doesn't contain any part of the original message and is only needed for tuning. So by not sending the carrier, DSBSC offers a substantial power saving over AM and is its main advantage.

The experiment

For this experiment you'll use the Emona DATEx to generate a real DSBSC signal by implementing its mathematical model. This means that you'll take a pure sinewave (the message) that contains absolutely no DC and multiply it with another sinewave at a higher frequency (the carrier). You'll examine the DSBSC signal using the scope and compare it to the original message. You'll do the same with speech for the message instead of a simple sinewave.

Following this, you'll vary the message signal's amplitude and observe how it affects the carrier's depth of modulation. You'll also observe the effects of modulating the carrier too much.

It should take you about 50 minutes to complete this experiment.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Procedure

Part A - Generating a DSBSC signal using a simple message

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Oscilloscope virtual instrument (VI).

  - Set up the scope per the procedure in Experiment 1 (page 1-12) ensuring that the Trigger Source control is set to CH 0.

  - Launch the DATEx soft front-panel (SFP).

  - Check you now have soft control over the DATEx by activating the PCM Encoder module's soft PDM/TDM control on the DATEx SFP.

Note: If you're set-up is working correctly, the PCM Decoder module's LED on the DATEx board should turn on and off.

12. Connect the set-up shown in Figure 4 below.

Note: Insert the black plugs of the oscilloscope leads into a ground (GND) socket.

This set-up can be represented by the block diagram in Figure 5 below. It implements the entire equation: DSBSC = the message * the carrier.

With values, the equation on the previous page becomes:

DSBSC = 4Vp-p 2kHz sine x 4Vp-p 100kHz sine.

    - Adjust the scope's Timebase control to view two or so cycles of the Master Signals module's 2kHz SINE output.

    - Activate the scope's Channel 1 input (by checking the Channel 1 Enabled box) to view the DSBSC signal out of the Multiplier module as well as the message signal.

    - Set the scope's Channel 0 Scale control to the 500m V/divposition and the Channel 1 Scale control to the 1V/divposition (if it's not already).

    - Draw the two waveforms to scale in the space provided below.

Tip: Draw the message signal in the upper half of the graph and the DSBSC signal in the lower half.

Ask the instructor to check your work before continuing.

17. If they're not already, overlay the message with the DSBSC signal's envelopes to compare them using the scope's Channel 0 Position control.

Question 1

What feature of the Multiplier module's output suggests that it's a DSBSC signal?

Tip: If you're not sure about the answer to the questions, see the preliminary discussion.

Question 2

The DSBSC signal is a complex waveform consisting of more than one signal. Is one of the signals a 2kHz sinewave? Explain your answer.

Question 3

For the given inputs to the Multiplier module, how many sinewaves does the DSBSC signal consist of, and what are their frequencies?

Question 4

Why does this make DSBSC signals better for transmission than AM signals?

Part В - Generating a DSBSC signal using speech

This experiment has generated a DSBSC signal using a sinewave for the message. However, the message in commercial communications systems is much more likely to be speech and music. The next part of the experiment lets you see what a DSBSC signal looks like when modulated by speech.

      - Disconnect the plugs to the Master Signals module's 2kHz SINE output.

      - Connect them to the Speech module's output as shown in Figure 6 below.

Remember: Dotted lines show leads already in place.

      - Set the scope's Timebase control to the 1ms/divposition.

      - Hum and talk into the microphone while watching the scope's display.

Question 5

Why isn't there any signal out of the Multiplier module when you're not humming or talking?

Ask the instructor to check your work before continuing.

Part С - Investigating depth of modulation

It's possible to modulate the carrier by different amounts. This part of the experiment let's you investigate this.

      - Return the scope's Timebase control to the 100ps/divposition.

      - Locate the Amplifier module on the DATEx SFP and set its soft Gain control to about a quarter of its travel (the control's line should be pointing to where the number nine is on a clock's face).

      - Modify the set-up as shown in Figure 7 below.

The set-up in Figure 7 can be represented by the block diagram in Figure 8 below. The Amplifier allows the message signal's amplitude to be adjustable.

Note: At this stage, the Multiplier module's output should be the normal DSBSC signal that you sketched earlier.

Recall from Experiment 5 that an AM signal has two dimensions that can be measured and used to calculated modulation index (m). The dimensions are denoted Pand Q. If you've forgotten which one is which, take a minute to read over the notes at the top of page 5-14 before going on to the next step.

25. Vary the message signal's amplitude a little by turning the Amplifier module's soft Gain control left and right a little. Notice the effect that this has on the DSBSC signal's P and Q dimensions.

Question 6

Based on your observations in Step 25, when the message's amplitude is varied

- neither dimensions Por Qare affected.

- only dimension Q is affected.

- only dimension Pis affected.

- both dimensions Pand Qare affected.

On the face of it, determining the depth of modulation of a DSBSC signal is a problem. The modulation index is always the same number regardless of the message signal's amplitude. This is because the DSBSC signals ф dimension is always zero.

However, this isn't the problem that it seems. One of the main reasons for calculating an AM signal's modulation index is so that the distribution of power between the signal's carrier and its sidebands can be calculated. However, DSBSC signals don't have a carrier (remember, it's suppressed). This means that all of the DSBSC signal's power is distributed between its sidebands evenly. So there's no need to calculate a DSBSC signal's modulation index.

The fact that you can't calculate a DSBSC signal's modulation index might imply that you can make either the message or the carrier as large as you like without worrying about over- modulation. This isn't true. Making either of these two signals too large can still overload the modulator resulting in a type of distortion that you've seen before. The next part of the experiment lets you observe what happens when you overload a DSBSC modulator.

  - Set the Amplifier module's soft Gain control to about half its travel and notice the effect on the DSBSC signal.

Note 1: Resize the display as necessary using the scope's Channel 1 Scale control. Note 2: If doing this has no effect, turn up the gain control a little more.

  - Draw the new DSBSC signal to scale in the space provided below.

Question 7

What is the name of this type of distortion?

Ask the instructor to check your work before finishing.

Laboratory 6 - Observations of AM and DSBSC signals in the frequency domain

Preliminary discussion

Experiments 5 and 6 use the Emona DATEx to demonstrate the differences you would see on a scope between the output signals of an AM and DSBSC modulator. To refresh your memory, Figure 1 below shows the AM and DSBSC signals that would be produced by identical inputs (for example, a 1kHz sinewave for the message and a 100kHz sinewave for the carrier).

The two signals look different because they contain different sinewaves. That is, they have a different spectral composition. The reason for this is explained by the mathematical models of AM and DSBSC. Side-by-side, it's easy to see that the equations are a little different.

AM = (DC + message) * the carrier	DSBSC = the message * the carrier

And, when the equations are solved for the inputs specified above, we find that the AM and DSBSC signals consist of the following:

As you can see, AM signals include the carrier signal whereas DSBSC signals don't.

When you think about it, a scope's display is actually a graph of time (on the X-axis) versus voltage (on the У-axis). Importantly, graphs plotted this way are said to be drawn in the time domain.

Another way of representing signals like AM and DSBSC signals involves drawing all the sinewaves that they contain on a graph that has frequency for the X-axis instead of time. In other words, they're drawn in the frequency domain. When the AM and DSBSC signals in Figure 1 are drawn this way, we get the graphs in Figure 2 below.

Frequency domain representations of complex signals are very useful for thinking about their spectral composition. They give you a tool for visualising the sinewaves that the signal is made up of. They also help you to see how much of the frequency spectrum the signal occupies. This is the signal's bandwidth and is a critical issue in communications and telecommunications.

The bandwidth of AM and DSBSC signals can be calculated in one of two ways. The frequency domain graphs in Figure 2 shows that the signals occupy a portion of the spectrum from the lower sideband up to the upper sideband. That being the case, the bandwidth can be found using the equation:

Using this equation we find that the bandwidth of the AM and DSBSC signals in Figure 2 are 2kHz. In situations where the sidebands are made up of more than one sinewave, you must solve the equation using the highest frequency in the USB and the lowest frequency in the LSB.

Now, compare the bandwidth of the signals in Figure 2 (2kHz) with the original signals used to produce them (that is, a 1kHz message and a 100kHz carrier). Notice that their bandwidths are twice the frequency of their message. This gives us the second equation for calculating bandwidth:

In situations where the message is made up of more than one sinewave, you must solve the equation using the highest frequency in the message.

The experiment

For this experiment you'll use the Emona DATEx to generate a real AM and DSBSC signal then analyse the spectral elements of the two signals using the NI ELVIS Dynamic Signal Analyzer.

It should take you about 50 minutes to complete this experiment. Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Procedure

Part A - Setting up the AM modulator

To experiment with AM spectrum analysis, you need an AM signal. The first part of the experiment gets you to set one up.

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch the NI ELVIS II Variable Power Supplies VI and click on its Run control to activate the hardware.

  - Adjust the Variable Power Supplies negative output Voltage control for an output of about -6V (the exact value is not critical).

  - Launch the NI ELVIS II DMM VI and click on its Run control to activate the hardware.

  - Set up the DMM VI for measuring DC voltages.

  - Launch the DATEx soft front-panel (SFP).

  - Check you have soft control over the DATEx by activating the PCM Encoder module's soft PDM/TDM control on the DATEx SFP.

Note: If you're set-up is working correctly, the PCM Decoder module's LED on the DATEx board should turn on and off.

  - Locate the Adder module on the DATEx SFP and turn its soft (9and gcontrols fully anti-clockwise.

  - Connect the set-up shown in Figure 3 below.

  - Connect the Adder module's output to the DMM and adjust the module's soft g control to obtain a 1V DC output.

Note 1: you must also connect the DMM's COM input to a ground terminal on the Emona DATEx.

Note 2: Remember also that you can use the keyboards tab and arrow keys for fine adjustment of DATEx soft controls.

  - Disconnect the DMM and close its VI.

  - Launch the NI ELVIS II Function Generator VI and click on its Run control to activate the hardware.

  - Adjust the function generator using its soft controls for an output with the following specifications:

- Waveshape: Sine

- Frequency: 10kHz exactly

- Amplitude: 4Vpp

- DC Offset: 0V

- you'll be using the function generator VI again later but minimise its window for now.

- Launch the NI ELVIS II Oscilloscope VI and click on its Run control to activate the hardware.

- Set up the scope per the procedure in Experiment 1 (page 1-12) with the following changes:

- Channel 0 Coupling control to the DC position instead of AC

- Channel 0 Scale control to the 500m V/divposition instead of 1V/div

- Timebase control to the 50ps/divposition instead of 500ps/div

- Trigger Level control to the 1Vposition instead of 0V

- Adjust the Adder module's soft £ control to obtain a 1Vp-p sinewave.

- Activate the scope's Channel 1 input (by checking the Channel 1 Enabled box) to view both the message and the modulated carrier.

Self check: If the scope's Scale control for Channel 1 is set to the 1 V/div position, the scope should now display an AM signal with envelopes that are the same shape and size as the message. If not, close all windows, check your wiring then repeat the process starting from Step 7.

The set-up can be represented by the block diagram in Figure 4 below. It implements the equation: AM = (1VDC + 1Vp-p 10kHz sine) * 4Vp-p 100kHz sine.

Question 1

For the given inputs to the Multiplier module, what are the frequencies of the three sinewaves on its output?

Question 2

Use this information to calculate the AM signal's bandwidth. Tip: If you're not sure how to do this, read the preliminary discussion.

Ask the instructor to check your work before continuing.

Part В - Setting up the NI ELVIS II Dynamic Signal Analyzer (DSA)

- Suspend the scope's operation by clicking on its Stop control once.

Note: The scope's display should freeze and its hardware has been deactivated. This is a necessary step as the scope and signal analyzer share hardware resources and so they cannot be operated simultaneously.

- Minimise the scope's VI.

- Launch the NI ELVIS II Dynamic Signal Analyzer VI.

Note: If the Dynamic Signal Analyzer VI has launched successfully, the instrument's window will be visible (see Figure 5).

- Adjust the signal analyzer's controls as follows: Input Settings

- Source Channel to SCOPE CH1 ■ Voltage Range to+10V

FFT Settings	Averaging

- Frequency Span to 150,000	■ Mode to RMS

- Resolution to 400	■ Weighting to Exponential

- Window to 7 Term B-Harris	■ # of Averages to 3

Trigger Settings

- Type to Edge

Frequency Display	Cursor settings

- Units to dB	■ Cursors On box unchecked (for now)

- Mode to RMS

- Scale to Auto

  - Click on the signal analyzer's Run control to activate its hardware.

Note: If the Signal Analyzer VI has been set up correctly, its display should look like Figure 6 below.

The Signal Analyzer's display needs a little explaining here. There are actually two displays, a large one on top and a much smaller one underneath. The smaller one is a time domain representation of the input (in other words, the display is a scope). Notice that it's showing the AM signal that you set up earlier and saw in Step 24.

The larger of the two displays is the frequency domain representation of the input. Notice that it looks fairly similar to the frequency domain graph for an AM signal in Figure 2 (in the preliminary discussion). The Signal Analyzer's display doesn't have single sharp lines for each of the sinewaves present in the signal because the practical implementation of FFT is not as precise as the theoretical expectation.

Part С - Spectrum analysis of an AM signal

The next part of this experiment let's you analyze the frequency domain representation of the AM signal to see if its frequency components match the values that you mathematically predicted for Questions 1 and 2.

  - Activate the signal analyzer's cursors by checking (that is, ticking) Cursors On box.

Note: When you do, green horizontal and vertical lines should appear on the signal analyzer's frequency domain display.

The NI ELVIS II Dynamic Signal Analyzer has two cursors C1 and C2 that default to the left most side of the display when the signal analyzer's VI is launched. They're repositioned by "grabbing" their vertical lines with the mouse and moving the mouse left or right.

  - Use the mouse to grab and move the vertical line of cursor Cl.

Note: As you do, notice that cursor Cl moves along the signal analyzer's trace and that the vertical and horizontal lines move so that they always intersect at Cl.

  - Repeat Step 31 for cursor C2.

Note: Fine control over the cursors' position is obtained by using the cursor's Position control in the Cursor Settings area (below the display).

The NI ELVIS II Dynamic Signal Analyzer includes a tool that measures the difference in magnitude and frequency between the two cursors. This information is displayed in green between the upper and lower parts of the display.

  - Move the cursors while watching the measurement readout to observe the effect.

  - Position the cursors so that they're on top of each other and note the measurement.

Note: When you do, the measurement of difference in magnitude and frequency should both be zero.

Usefully, when one of the cursors is moved to the extreme left of the display, its position on the X-axis is zero. This means that the cursor is sitting on 0Hz. It also means that the measurement readout gives an absolute value of frequency for the other cursor. This makes sense when you think about it because the readout gives the difference in frequency between the two cursors but one of them is zero.

  - Move C1 to the extreme left of the display.

  - Align C2 with the highest point in the AM signal's lower sideband.

Note: This is the sinewave just to the left of the largest sinewave in the display.

  - Measure the sinewave's frequency and record this in Table 1 on the next page.

  - Align C2 with the highest point in the AM signal's carrier and repeat Step 37. Note: This is the largest sinewave in the display.

  - Align C2 with the highest point in the AM signal's upper sideband and repeat Step 37. Note: This is the sinewave just to the right of the carrier.

  - Align C1 with the highest point in the AM signal's lower sideband and measure the AM signal's bandwidth.

Question 3

How do the measured values in Table 1 compare with your theoretically predicted values (see Questions 1 and 2)? Explain any differences.

Ask the instructor to check your work before continuing.

As an aside, at this point it looks as though the sidebands are nearly as large as the carrier. However, this is misleading because the vertical axis is logarithmic (that is, it's non-linear). The sidebands are actually much smaller than the carrier. This can be proven as follows:

  - Set the Signal Analyzer's Units control to Linear instead of dB.

Note: This sets the vertical axis to a simple linear voltage measurement instead of decibels.

  - Note the relative sizes of the sinewaves in the signal.

  - Return the Signal Analyzer's Units control to dB.

  - Maximise the function generator's VI and increase its output frequency to 20kHz.

  - Use the signal analyzer's cursors to find the AM signal's new bandwidth. Record this in Table 2 below.

Note: It'll take up to thirty seconds for the display to be fully up to date with the change because it's an average of three sweeps.

  - Increase the function generator's output frequency to 30kHz.

  - Find and record the AM signal's new bandwidth.

Question 4

What's the relationship between the message signal's frequency and the AM signal's bandwidth?

Ask the instructor to check your work before continuing.

  - Return the function generator's output frequency to 10kHz.

  - Wait until the signal analyzer's frequency domain display has fully updated then disconnect the banana plug to the Multiplier module's Л'input.

  - Wait until the display has fully updated then investigate the frequency of the most significant sinewave on the Multiplier module's output.

Question 5

What is this signal?

Question 6

What's missing and why?

  - Reconnect the banana plug to the Multiplier module's Л'input.

  - Disconnect the banana plug to the Multiplier module's У input.

  - Wait until the display has fully updated then investigate the frequency of the most significant sinewave on the Multiplier module's output.

Question 7

What is this signal?

Question 8

Why are the sidebands missing when there's a message?

Ask the instructor to check your work before continuing.

Part D - Setting up the DSBSC modulator

To experiment with DSBSC spectrum analysis, you need a DSBSC signal. This part of the experiment gets you to set one up.

  - Suspend the signal analyzer's operation by clicking on its Stop control once.

  - Maximise the function generator's VI and check that its output frequency has been returned to 10kHz.

  - Set the function generator's output to 1Vpp.

  - Disassemble the current set-up.

  - Connect the set-up shown in Figure 7 below.

This set-up can be represented by the block diagram in Figure 8 on the next page. It implements the equation: DSBSC = 1Vp-p 10kHz sine * 4Vp-p 100kHz sine.

59. Restart the scope and make the following checks and adjustments:

- Check that the Timebase control is set to 50ps/div

- Return the Trigger Level control to 0V

Self check: The scope should now display a DSBSC signal with alternating halves of the envelope forming the same shape as the message and is about the same size.

Question 9

For the given inputs to the Multiplier module, what are the frequencies of the two sinewaves on its output?

Question 10

Use this information to calculate the DSBSC signal's bandwidth.

Part E - Spectrum analysis of a DSBSC signal

- Close the scope's VI.

- Restart the NI ELVIS II Dynamic Signal Analyzer VI.

Note: Once the display has had time to update, you should be able to clearly see the DSBSC signal's two sidebands.

You'll also see that the signal has a carrier. However, despite appearances, this signal is very small relative to the sidebands (remember, the scale for the Y-axis is decibels which is a logarithmic unit of measurement). Design limitations in implementing DSBSC mean that there will always be a small carrier component in the DSBSC signal. That's why the second "s" in DSBSC is for "suppressed".

- Move C1 to the extreme left of the display.

- Align C2 with the DSBSC signal's lower sideband.

- Measure the sinewave's frequency and record this in Table 3 below.

- Align C2 with the DSBSC signal's upper sideband and repeat Step 64.

- Use the signal analyzer's two cursors to determine and record the DSBSC signal's bandwidth.

Question 11

How do the measured values in Table 3 compare with your theoretically predicted values (see Questions 9 and 10)?

Question 12

Compare the DSBSC signal's bandwidth with the bandwidth for the AM signal with a 10kHz message (in Table 1). What can you say about the bandwidth requirements of AM and DSBSC signals?

Ask the instructor to check your work before continuing.

67. Find the DSBSC signal's bandwidth for two other message frequencies (say 20kHz and 30kHz).

Question 13

What's the relationship between the message signal's frequency and the DSBSC signal's bandwidth?

Ask the instructor to check your work before finishing.

Laboratory 7 - AM demodulation

Preliminary discussion

If you've completed Experiment 5 then you've seen what happens when a 2kHz sinewave is used to amplitude modulate a carrier to produce an AM signal. Importantly, you would have seen a key characteristic of an AM signal - its envelopes are the same shape as the message (though the lower envelope is inverted).

Recovering the original message from a modulated carrier is called demodulation and this is the main purpose of communications and telecommunications receivers. The circuit that is widely used to demodulate AM signals is called an envelope detector. The block diagram of an envelope detector is shown in Figure 1 below.

As you can see, the rectifier stage chops the AM signal in half letting only one of its envelopes through (the upper envelope in this case but the lower envelope is just as good). This signal is fed to an RC LPF which tracks the peaks of its input. When the input to the RC LPF is a rectified AM signal, it tracks the signal's envelope. Importantly, as the envelope is the same shape as the message, the RC LPF's output voltage is also the same shape as the message and so the AM signal is demodulated.

A limitation of envelope detector shown in Figure 1 is that it cannot accurately recover the message from over-modulated AM signals. To explain, recall that when an AM carrier is over- modulated the signal's envelope is no-longer the same shape as the original message. Instead, the envelope is distorted and so, by definition, this means that the envelope detector must produce a distorted version of the message.

The experiment

For this experiment you'll use the Emona DATEx to generate an AM signal by implementing its mathematical model. Then you'll set-up an envelope detector using the Rectifier and RCLPFon the trainer's Utilities module.

Once done, you'll connect the AM signal to the envelope detector's input and compare the demodulated output to the original message and the AM signal's envelope. You'll also observe the effect that an over-modulated AM signal has on the envelope detector's output.

Finally, if time permits, you'll demodulate the AM signal by implementing by multiplying it with a local carrier instead of using an envelope detector.

It should take you about 50 minutes to complete Parts A to D of this experiment and another 20 minutes to complete Part E.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

- One set of headphones (stereo)

Procedure

Part A - Setting up the AM modulator

To experiment with AM demodulation you'll need an AM signal. The first part of the experiment gets you to set one up.

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Variable Power Supplies VI.

  - Adjust the Variable Power Supplies negative output Voltage control for an output of about -6V (the exact value is not critical).

  - Launch and run the NI ELVIS II DMM VI.

  - Set up the DMM VI for measuring DC voltages.

  - Launch the DATEx soft front-panel (SFP) and check that you have soft control over the DATEx board.

  - Locate the Adder module on the DATEx SFP and turn its soft Әand gcontrols fully anti-clockwise.

  - Connect the set-up shown in Figure 2 below.

  - Connect the Adder module's output to the DMM and adjust the module's soft g control to obtain a 1V DC output.

Note 1: Remember that you must also connect the DMM's COM input to a ground terminal on the Emona DATEx.

Note 2: Remember also that you can use the keyboards tab and arrow keys for fine adjustment of DATEx soft controls.

  - Disconnect the DMM and close its VI.

  - Launch and run the NI ELVIS II Oscilloscope VI.

  - Set up the scope per the procedure in Experiment 1 with the following changes:

- Channel 0 Coupling control to the Deposition instead of AC

- Channel 0 Scale control to the 500m V/divposition instead of 1V/div

- Trigger Level control to the 1Vposition instead of 0V

- Adjust the scope's Timebase control to view only two or so cycles of the message signal.

- Adjust the Adder module's soft £ control to obtain a 1Vp-p sinewave.

21. Activate the scope's Channel 1 input to view both the message and the modulated carrier.

Self check: If the scope's Scale control for Channel 1 is set to the 1V/divposition, the scope should now display an AM signal with envelopes that are the same shape and size as the message. If not, close all windows, check your wiring then repeat the process starting from Step 7.

The set-up in Figure 2 on the previous page can be represented by the block diagram in Figure 3 below. It generates a 100kHz carrier that is amplitude modulated by a 2kHz sinewave message.

Ask the instructor to check your work before continuing.

Part В - Recovering the message using an envelope detector

22. Modify the set-up as shown in Figure 4 below.

Remember: Dotted lines show leads already in place.

The additions to the set-up can be represented by the block diagram in Figure 5 below. As you can see, it's the envelope detector explained in the preliminary discussion.

- Adjust the scope's Scale and Timebase controls to appropriate settings for the signals.

- Draw the two waveforms to scale in the space provided below leaving room to draw a third waveform.

Tip: Draw the message signal in the upper third of the graph and the rectified AM signal in the middle third.

- Disconnect the scope's Channel 1 input from the Rectifier's output and connect it to the RCLPF's output instead.

- Draw the demodulated AM signal to scale in the space that you left on the graph paper.

Question 1

What is the relationship between the original message signal and the recovered message?

Ask the instructor to check your work before continuing.

Part С - Investigating the message's amplitude on the recovered message

- Vary the message signal's amplitude up and down a little (by turning the Adder module's soft & control left and right a little) while watching the demodulated signal.

Question 2

What is the relationship between the amplitude of the two message signals?

- Slowly increase the message signal's amplitude to maximum while watching the demodulated signal.

Question 3

What do you think causes the heavy distortion of the demodulated signal? Tip: If you're not sure, connect the scope's Channel 0 input to the AM modulator's output.

Question 4

Why does over-modulation cause the distortion?

Ask the instructor to check your work before continuing.

Part D - Transmitting and recovering speech using AM

This experiment has set up an AM communication system to "transmit" a message that is a 2kHz sinewave. The next part of the experiment lets you use the set-up to modulate, transmit, demodulate and listen to speech.

- If you moved the scope's Channel 0 input to help you answer Question 4, reconnect it to the Adder module's output.

- Return the message signal's amplitude to 1Vpp (by adjusting the Adder module's soft & control).

- Modify the set-up as shown in Figure 6 below.

- Set the scope's Timebase control to the 2ms/divposition.

- Turn the Amplifier module's soft Gain control fully anti-clockwise.

- Without wearing the headphones, plug them into the Amplifier module's headphone socket.

- Put the headphones on.

- As you perform the next step, set the Amplifier module's soft Gain control to a comfortable sound level.

- Hum and talk into the microphone while watching the scope's display and listening on the headphones.

Ask the instructor to check your work before continuing.

Part E - The mathematics of AM demodulation

AM demodulation can be understood mathematically because it is uses multiplication to reproduce the original message. To explain, recall that when two pure sinewaves are multiplied together (a mathematical process that necessarily involves some trigonometry that is not shown here) the result gives two completely new sinewaves:

- One with a frequency equal to the sum of the two signals' frequencies

- One with a frequency equal to the difference between the two signals' frequencies

The envelope detector works because the rectifier is a device that multiplies all signals on its one input with each other. Ordinarily, this is a nuisance but not for applications like AM demodulation. Recall that an AM signal consists of a carrier, the carrier plus the message and the carrier minus the message. So, when an AM signal is connected to a rectifier's input, mathematically the rectifier's cross multiplication of all of its sinewaves looks like:

Rectifier's output = carrier * (carrier + message) * (carrier - message)

If the message signal used to generate the AM signal is a simple sinewave then, when the equation above is solved, the rectifier outputs six sinewaves at the following frequencies:

- Carrier + (carrier + message)

- Carrier + (carrier - message)

- (carrier + message) + (carrier - message)

- Carrier - (carrier + message) which simplifies to just the message

- Carrier - (carrier - message) which also simplifies to just the message

- (carrier + message) - (carrier - message)

To make this a little more meaningful, let's do an example with numbers. The AM modulator that you set up at the beginning of this experiment uses a 100kHz carrier and a 2kHz message (with a DC component). So, the resulting AM signal consists of three sinewaves: one at 100kHz, another at 102kHz and a third at 98kHz. Table 1 below shows what happens when these sinewaves are cross-multiplied by the rectifier.

Notice that two of the sinewaves are at the message frequency. In other words, the message has been recovered! And, as the two messages are in phase, they simply add together to make a single bigger message.

Importantly, we don't want the other non-message sinewaves so, to reject them but keep the message, the rectifier's output is sent to a low-pass filter. Ideally, the filter's output will only consist of the message signal. The chances of this can be improved by making the carrier's frequency much higher than the highest frequency in the message. This in turn makes the frequency of the "summed" signals much higher and easier for the low-pass filter to reject.

[As an aside, the 4kHz sinewave that was generated would pass through the low-pass filter as well and be present on its output along with the 2kHz signal. This is inconvenient as it is a signal that was not present in the original message. Luckily, as the signal was generated by multiplying the sidebands, its amplitude is much lower than the recovered message and can be ignored.]

An almost identical mathematical process can be modelled using the Emona DATEx module's Multiplier module. However, instead of multiplying the AM signal's sinewaves with each other (the Multiplier module doesn't do this), they're multiplied with a locally generated 100kHz sinewave. The next part of this experiment lets you demodulate an AM signal this way.

  - Return the scope's Timebase control to its earlier setting (probably 200ps/div).

  - Disconnect the envelope detector and modify the set-up to return it to just an AM modulator with a 2kHz sinewave for the message as shown in Figure 7 below.

  - Modify the set-up as shown in Figure 8 below.

The additions to the set-up in Figure 8 on the previous page can be represented by the block diagram in Figure 9 below. The Multiplier module models the mathematical basis of AM demodulation and the RCLow-pass filter on the Utilities module picks out the message while rejecting the other sinewaves generated.

  - Compare the Multiplier module's output with the Rectifier's output that you drew earlier (see page 8-8).

Question 5

Given the AM signal (which consists of 100kHz, 102kHz and 98kHz sinewaves) is being multiplied by a 100kHz sinewave:

    - How many sinewaves are present in the Multiplier module's output?

    - What are their frequencies?

  - Disconnect the scope's Channel 1 input from the Multiplier module's output and connect it to the RCLPF's output instead.

  - Compare the RCLPF'soutput with the message and the output RCLPFsthat you drew earlier (see page 8-8).

Ask the instructor to check your work before continuing.

A common misconception about AM is that, once the signal is over-modulated, it's impossible to recover the message. However, when the AM signal is generated using an ideal or near-ideal modulator (like Figure 3) this is only true for the envelope detector.

The AM demodulation method being implemented in this part of the experiment (called product detection - though it is more accurate to call it product demodulation) doesn't suffer from this problem as it's not designed to recover the message by tracking one of the AM signal's envelopes. The final part of this experiment demonstrates this.

  - Connect the scope's Channel 0 to the AM modulator's output.

  - Set the scope's Trigger Level to 0V.

Note: The scope will lose triggering but the display will be adequate for the next steps.

  - Slowly increase the message signal's amplitude to produce a near 100% modulated AM signal by adjusting the Adder module's soft Gcontrol.

Note: Resize the AM and demodulated message signals on the screen as necessary.

  - Slowly increase the message signal's amplitude to produce an AM signal that is modulated by more than 100% while paying close attention to the demodulated message signal.

As an aside, the commercial implementation of AM modulation commonly involves a Class C amplifier for efficiency (that is, to minimise power losses). When a Class C amplifier is operated at depths of modulation above 100% the circuit's operation no-longer corresponds with the model of an AM modulator in Figure 3. Importantly, in addition to producing an envelope that is not the same as the original message, the over-modulated Class C circuit produces extra frequency components in the spectrum. This means that neither the envelope detector nor the product demodulator can reproduce the message without distortion.

Ask the instructor to check your work before finishing.

List of recommended literature

Basic

- Теория электрической снязи: Учебник для вузов/ Зюко А.Г., Кловский Д.Д., Коржик В.И., Назаров М.В. - М: Радио и связь, 1999.

- Теория электрической связи. Под редакцией профессора Д.Д.Кловского - М.: «Радио и связь», 1998.

- Скляр Б. Цифровая связь -. С-П, К., 2003

- Opadchyi U.F., Gludkin О.P., Gurov А.I. Analog and digital electronics (general course): Textbook for HEI. Under edition of О.P. Gludkin. - M.: Hot Line – Telecom, 2005.

Additional

- Баскаков СИ. «Радиотехнические цепи и сигналы», «ВШ». - М.: 2000.

- 7. Баскаков СИ. «Радиотехнические цепи и сигналы»: Руководство к решению задач, «ВШ». - М.: 2002.

- 8. Загидуллин Р.Ш. и др. System View системотехническое моделирование устройств обработки сигналов - М.: «Горячая линия - Телеком», 2005.

- Bystrov U.А. and others. Electric devices and equipment on its base: Reference book. - М.: IP RadioSoft, 2002.

- Electrical techniques and electronics in experiments and exercises: Laboratory practical work on Electronics Workbench: In 2 b. /Under edition of D.I. Panphilov - Т.2: Electronics. - М.: DODEKA, 2000

- Trevis G. LabVIEW for everyone. - М.:DMK Press; PriborKomplect, 2004.

- Page L.I., Tocholin D.А., Pollak B.P. LabVIEW for beginners and specialists. - М.: HotLine – Telecom, 2004.

Saribaev Abdushukur Satibaldievich

Umbetov Adilkhan Zhumadilovich.

Methodical instructions to the laboratory work

on discipline

Distribution of radio waves and antenna-feeder device

for students of specialty 5B071900 – Radio engineering,

electronics and telecommunications

Send for the press

Paper format XxY  1/16

Paper typographical. Offset printing. Volume 5.25 п.л.

Circulation….instance Order № …*

©  Issuing of the M.Auezov South Kazakhstan state university

Publishing center of M.Auezov SKSU, Shymkent,

Tauke Khan street, 5