Ф.7.03-03

Amirbekova A.I., Kayumova U.R., Kultas A.K.

Methodical instructions to practical works

for discipline

MULTICHANNEL TRANSMISSION SYSTEM

Shymkent, 2018 y.

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

M. AUEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

Chair: “Automation, telecommunication and control”

Amirbekova A.I., Kayumova U.R., Kultas A.K.

Methodical instructions to practical works

for discipline

MULTICHANNEL TRANSMISSION SYSTEM

for students of the specialty 5В071900 – Radiotechnics, Electronics and Telecommunication

Shymkent, 2018 y.

UDC 519.683

Authors: Amirbekova A.I., Kayumova U.R., Kultas A.K. Methodical instructions to practical works for discipline “Multichannel Transmission System”.-Shymkent: M. Auezov SKSU, 2018. – 76 p.

Methodical instructions to practical works is developed in accordance with requirements of curricula and the program of the discipline “Multichannel Transmission System” and includes all necessary data on the accomplishment of the course practical subjects.

Methodical instructions to practical works is intended for the students of 5В071900 – ”Radiotechnics, Electronics and Telecommunication” specialty.

Methodical instructions to practical works contains goals, short theoretical data, principal electrical schemes of devices, carrying out order and tasks for the working out of results, and also control questions on eight practical works.

Reviewers:

Saribayev A.S. - c.t.s., the associate professor of chair «Automation, telecommunications and control» SKSU of M.Auezov

Methodical instructions to laboratory works is discussed and recommended for publication at the meeting of the chair “Automation, telecommunication and control” (Minutes № __ “__” _____) and at the methodical committee of higher school “Information technology and energetic” (Minutes № __ “__” _____).

Methodical instructions to laboratory works is recommended for publication by Educational and Methodical Council of  M. Auezov SKSU, protocol № __ “__” _______.

M. Auezov South Kazakhstan State University, 2018.

Responsible for printing Amirbekova A.I.

Contents

| Introduction | 6 |
| --- | --- |
| A criterion for assessing the knowledge of students | 7 |
| Methodical instructions to the accomplishment of laboratory works and rules of their registration | 8 |
| Practical work № 1 | 9 |
| Practical work № 2 | 10 |
| Practical work № 3 | 13 |
| Practical work № 4 | 14 |
| Practical work № 5 | 17 |
| Practical work № 6 | 21 |
| Practical work № 7 | 24 |
| Practical work № 8 | 27 |
| Practical work № 9 | 34 |
| Practical work № 10 | 42 |
| Practical work № 11 | 51 |
| Practical work № 12 | 54 |
| Practical work № 13 | 60 |
| Practical work № 14 | 65 |
| Practical work № 15 | 70 |
| List of recommended literature | 73 |

Introduction

The purpose of teaching discipline of " Multichannel Transmission System " is formation of students knowledge on the principles of functional blocks work and typical devices of electronic technics, used for the processing of the analog and digital information at the automation of technological processes and industries.

Problems of the discipline teaching.

In the process of “Multichannel Transmission System” course study students are acquinted with basic and additional elements of electrical schemes, logic devices on the basis of elementary base, principles of these devices work and possibilities of their application, etc.

In a result of studying the discipline student must know:

- theoretical bases of electronics and physical phenomena on which principles of work of various electronic elements and devices are based;

- classification and principle of electronic devices work;

- models of electronic elements and their application;

- principles of construction and work of analog and digital schemes;

- principles of integrated and operational amplifiers and logic elements use for creation of various functional blocks.

Be able to:

- project electronic knots, understand and be able to read simple schemes of typical electronic equipment;

- use various electronic devices in electric schemes, choose types of electronic devices depending on features of their application;

- maintain electronic schemes;

- analyze work of electronic schemes, define their features in connection with carried out function and their service conditions;

- work with technical literature, directories, State Standards and normative documentation.

Practical works depending on the volume of experimental researches demand not less than 2 hours for accomplishment.

A criterion for assessing the knowledge of students

A score "A" with a digital equivalent of 4.0 and a percentage of 95-100% and A- having a digital equivalent of 3.67 and a percentage of 90-94% correspond to an "excellent" rating.

The given estimation is put in the event that the trainee has shown full mastering of a program material and has not admitted any mistakes, inaccuracies, timely and correctly carried out control and laboratory works and has handed over reports on them, has shown thus original thinking, in due time and without any mistakes he passed colloquiums and fulfilled his homework assignments, was engaged in scientific research work, independently used additional scientific literature in the study of discipline, was able to systematize programs independently the first material.

A B + score with a digital equivalent of 3.33 and a percentage of 85-89%, B having a digital equivalent of 30 and a percentage of 80-84% and B- having a digital equivalent of 2.67 and a percentage of 75 -79%.

The given estimation is put in the event that the student has mastered the program material not less than 75% and at the same time did not allow gross mistakes in the response, timely performed the control and laboratory work and passed them without fundamental remarks, performed correctly and duly delivered colloquiums and homework without any fundamental comments, used additional literature as instructed by the teacher, engaged in research work, allowed unprincipled inaccuracies or fundamental mistakes, corrected by the student himself entom, managed to systematize the program material with the help of a teacher.

The "satisfactory" estimate corresponds to estimates of C + having a digital equivalent of 2.33 and a percentage of 70-74%, C having a digital equivalent of 2.0 and a percentage of 65-69%, C- having | a digital equivalent of 1.67 and a percentage of 60-64%, D + having a digital equivalent of 1.33 and a percentage of 55 to 59% and D having an equivalent of 1.0 and a percentage of 50 to 54%

The given estimation is put in the event that the student has mastered a program material not less than on 50%, at performance of control and laboratory works, house tasks needed the help of the teacher, at delivery of colloquium admitted inaccuracies and unprincipled errors, inaccuracies, did not show activity in research work, limited only to the educational literature specified by the teacher, experienced more difficulty in the systematization of the material.

An "unsatisfactory" estimate corresponds to an estimate of Є that has a digital equivalent of 0 and a percentage of 0-49%. The given estimation is put in the event that the student has found gaps in knowledge of the basic material provided by the program, has not mastered more than half of the discipline program, in the answers he made fundamental mistakes, did not execute the separate tasks stipulated by the forms of current, intermediate and final control, did not work all the basic the literature provided by the program.

Methodical instructions to the accomplishment of  Practical works

and rules of their registration

Practical work are provided by the curriculum at studying of the “electronics” course. In the course of carrying out the laboratory works it is required: to fix theoretical material of the course, to receive practical skills in projecting the analog and digital electric schemes, the analysis of         characteristics of electronic elements, to master technics of carrying out the experimental research, and also to get skills in the estimation of experiment results.

Group is divided into two smaller groups for carrying out laboratory works. The students of these small group carry out the laboratory work under the supervision of the teacher.

The basis of laboratory practical work on any disciplines is made by a complex of the measuring apparatuses connected to laboratory models, by means of which the studied phenomena and processes are reproduced. Till the present time basically traditional measuring devices were used in educational laboratories basically. Application of the computer measuring apparatuses in the educational purposes created by means of the virtual devices technology became a modern line.

The virtual device (VD) in educational laboratory is a measuring apparatus representing, as a rule, personal computer, provided with the special applied software and various measuring modules, for example, a multipurpose board of input-output VD allowing to automate operations on gathering, processing and representation of measuring information, having convenient user interface, and its software and hardware supports realization of functions, inherent to traditional measuring apparatus, and provide representation of results on the monitor screen in the convenient form to the user. Scheme of VD, used in the laboratory practical work, is presented in Figure B.1.

The software of VD can be developed with the help of standard means, as Visual С++, Visual Basic, etc, and also with the help of program means, specially intended for the decision of gathering problems, transformation and processing of the measuring information. The applied software package LabVIEW by National Instruments company is considered nowadays as the most suitable from the specialized softwares.

The automation hardware of measuring processes and procedures presented in the market are usually completed with LabVIEW drivers. Applications programming in the given environment is conducted by visual means that doesn’t demand profound knowledge of programming from the developer.

The base laboratory stand equipped with the modern personal computer (PC), supplied with  operating system Windows 7 or older senior versions and specialized set of hardware, and also original applied software are required for carrying out the practical work.

Practical Work 1

A generalized block diagram of a multichannel transmission system (MSP). Basic requirements and definitions.

Generation of typical transmission channels is provided by a joint venture, one of the constituent parts of which is the distribution medium. Such a medium can be coaxial, symmetric or optical cables, formerly VLANs or radio links. The cost of cables is high, so you need to be able to simultaneously on one physical circuit (trunk) and independently transmit as many signals as possible. This is the first major task.

A physical circuit is one or two pairs of wires, for radio links a similar concept is the trunk. The more signals will be simultaneously transmitted over a single cable circuit, the more efficient this cable will be used. A combination of technical means for simultaneous transmission of N signals along a single physical circuit (one trunk) is called a multichannel transmission system (SMB).

The JV, apart from the propagation medium, also includes terminal and intermediate stations.

a simplified structural diagram of SMEs looks like this:

ЛИНИЯ

Σ

МN

МN

МN

Аб.

Сооб.

Первич.сигн.

С2(t)

СN(t)

С1(t)

Кан. сигн.

K1(t)

K2(t)

KN(t)

Абон.устр-во

С2(t)

ДМ2

РУ

ДМN

ДМ1

K1(t)

K2(t)

KN(t)

С1(t)

СN(t)

Сооб.

Аб.

Абон.устр-во

At the subscriber unit, the message (non-electric signal) is converted into a primary electrical signal, then the primary signals in the converter (modulator) are converted into channel signals. Each channel signal has some distinctive feature from other channel signals.

In this case, the methods of the CRM and VRK can be used:

1

2

N

. . . . . . . .

t

t1

tŃ

t1́                    t2               t2́                      tN

Tцикла=Tд

1

Tд=1fд ; fд≥2Fв (theorem Kotelnikova).

This is also a group signal, but for the DSP. The first SMEs were on 2-3 channels on the VLAN, now the DSPs provide tens of thousands of channels with the speed of Mbit / s. At the same time, the effectiveness of the use of the guiding medium (KLS, OLS, RRLS) is sharply increased.

During the transmission, the signals undergo damping, distortion and are exposed to various types of interference. Therefore, the second main task of MES technology is to provide the required range of communication while maintaining quality indicators.

For this purpose, a group signal is appropriately formed, and intermediate points are set in the line in which signals are amplified, transformed, and corrected.

By typical channels of SMEs, signals of electrical communication are transmitted:

1) telephone

2) telegraphic

3) facsimile

4) data transmission

5) audio broadcasting

6) television

Primary communication signals and their main characteristics.

For electrical signals, one of the main parameters is voltage, power and current. In the communication technique, it is convenient to use logarithmic transmission units. If decimal logarithms are used to determine these relationships, the transmission units are expressed in dB; if natural logarithms are used, then units are expressed in Hn. These transmission units are called transmission levels. There are absolute, relative and measuring levels. Transmission levels can be by power, voltage and current.

Using a logarithmic form allows:

1) simplify mathematical operations; replace multiplication, exponentiation, and division by addition and subtraction;

2) use for constructing the frequency dependences of the Bode diagram and directly apply expressions for the transfer functions of electrical circuits;

3) objectively judge the perception of certain messages (hearing and vision have a logarithmic sensitivity characteristic).

Practical Work 2.

The level diagram.

When designing and operating communication systems, level charts (DU) are often used.

DM is a graph showing the distribution of transmission levels along a path or channel. As a rule, the measurement power levels are used in the remote control. A level diagram can be constructed for either a channel or a path, or for any of the parts of a channel or path. On the RC, the ordinate axis is scaled in dB (Hp) units of transmission levels, and the abscissa axis is the conditional axis indicating the length of the channel or path.

There are external and internal controls.

External DM is the distribution of levels along the line. The characteristic points of the external control are the control points of equipment, the points of joints of network paths and channels, etc. The values ​​of the measuring levels at these points are usually normalized in the technical conditions for the equipment or in the International Recommendations.

When measuring the remote control, a signal from the generator with f = 1020 Hz with a level Pm = 0 dBm is applied to the channel input.

Internal control is the distribution of levels within different parts of the apparatus.

A telephone channel is a collection of line and station devices for transmitting one message. Let the telephone channel be a path consisting of equipment installed at the ends (OS), four amplifying sections and three intermediate amplifiers.

DU is usually built for the upper channel spectrum, because He is in the worst conditions for interference. For CLS for max t˚C soil. Equipment PMO and NUP (amplifying points) should compensate for the attenuation of the previous section.

Protection from intrinsic interference can be determined by:

Aз=pпр-pсп

pвх=0дБ, т.1 – conditional.

t.2 - the level at the end of the channel is determined in each specific case, proceeding from the concept of residual channel attenuation.

ar channel is its working attenuation when the load resistors are applied to the input and output channels, equal to 600 ohms.

ar=Σai – ΣSi=p1-p2	                                (1)

Self-attenuation is a special case of working attenuation under the assumption that all the internal parts of the four-terminal network are coordinated.

In practice, at points of the SME they tend to harmonize individual parts of the path with each other, so we can talk about the residual damping as our own, and then see the formula (1). If we increase Pper, amplifiers will be overloaded and nonlinear distortions will increase sharply. If we lower Ppr, then our own noise will increase, which is also bad.

The internal remote control is allowed for one transmitting equipment, one rack of a separate path. Fragment of internal control:

Sус

AПФ

AМ

AФНЧ

AОА

Pвх

ОА

You can always find a faulty place on the inside of the remote control.

Parameters of telecommunication signals.

To solve a number of practical problems, the following signal characteristics are usually used:

1) Effective width of the energy spectrum.

Mathematically, the frequency band characterizing the signal has a bandwidth from 0 to ∞. For each type of information, the frequency band can be limited and distortions allowed. The band (300 ÷ 3400) Hz - is the legalized frequency band of the telephone channel. The bandwidth of the television signal is from 50 Hz to 6 MHz. The signals of sound broadcasting occupy the band 15 ÷ 20000 Hz (usually it is limited from 30 to 15000 Hz). The speech signal is from 20 Hz to 10 ÷ 15 kHz. Telegraph signals and signals of low-frequency data transfer are more narrow-banded, in them the speed is measured in bauds, and the width of the spectrum depends on the speed.

2) Average signal power.

The average signal power is the power averaged during the measurement over a period of time ΔT. Most often in an hour or a minute. The smallest signal is speech. It is necessary to specify at what point the power is determined: usually measured at a point with a zero relative level (or this is the point of a two-wire input channel). Pcp = 22 mkW - average power taking into account pauses related to the transmission of the telephone signal.

Tзан

паузы

t

During a pause - we listen to the interlocutor, pauses between words, sentences (50% of pauses). In general, pauses are about 75%, Tanz is about 25%. There are PSR at the time of signal transmission.

Pcr = 88μWW - power at active sites per hour (for Figure 1). Psr = 32 mkWo - 22 mkWo actual telephone power + 10 mW - signals of control and interaction. You need to know the power to know the load of the amplifiers.

Pср=100 мкВто135 мкВто- for telegraph signals and low-speed data transmission.

All power is strictly limited so as not to overload the amplifiers.

3) Dynamic range.

The dynamic range is the possible spread of signal power at the channel input.

D=10lgPмаксPмин ; D=Pмакс – Pмин , [дБ]

Telephone signal: D = 25 ÷ 40 dB; sound broadcasting - the range is more than 96 dB, for transmission over the channel it is reduced to 55 ÷ 60 dB.

4) The peak factor (Q).

Indicates how much the maximum power differs from the average.

Q=10lgPмаксPср = Pмакс – Pср , [дБ]         Pмакс= Pмакс эквив ;

Telephone signal: Q = 14 ÷ 17 dB

5) The amount of information carried per unit of time.

It is measured in bits / s.

С=2Fвlog2(1+ PсрPш ),

Fв - the upper frequency of the spectrum (for a telephone signal Fв = 3.4 kHz, С≈7000 bps)

6) Required signal strength.

Aз=10lgPсPш=Pс - Pш ,

Pc is the average signal power.

For TSA, the requirements are strict, for DSP - the requirements are less stringent.

Practical Work 3.

Construction of a joint venture.

1) One-way four-wire.

..

N

..

N

..

N

..

N

ИО пер

ИО пр

ГО пер

ГО пр

ГО пр

ГО пр

ИО пр

ИО пер

ОП-А

ОП-В

2 узла в одном месте, но по разным парам проводов

ОП-В

ПП

ОП-А

- общий  вид

IO - individual equipment for transmission, reception.

GO - group equipment for transmission, reception.

In the line - 2 pairs of wires, one for transmission, the other for reception. The terminal stations are exactly the same. Directional amplifiers are not connected in any way.

One-way - since the multichannel signal occupies the same frequency band in the CRM during transmission and reception, but this does not prevent, because reception and transmission are carried out on different pairs of wires.

2Two-way two-wire

2̒

1

f3÷f4

f1÷f2

f3÷f4

1̒

2

f1÷f2

ФНЧ

ИО пер

ИО пр

ГО пер

ГО пр

ИО пр

ИО пер

ГО пр

ГО пер

ФВЧ

In this case, 2 frequency bands are used: upper and lower.

Нижняя полоса ч-т      Верхняя полоса ч-т

f

1                N                      1               N

ПФ

ФНЧ

ФВЧ

ЗФ

ФНЧ

ФВЧ

f2 fср f3

f

Aф

Two-wire - since one pair of wires in the line is needed. Two-way - since frequency spectra of the signal are spaced.

In such transmission systems, guide filters are used (the "plug" of the LPF and HPF filters). Sometimes they write: NF D-33 (D-throttle, LPF), NF K-33 (K-capacitor, HPF). The cut is selected in the middle of the filtering strip (f3-f2).

Practical Work 4

N

1

2

. . . . . . . . . . .

f

fн

fв

In the linear path is transmitted:

At the input of the receiving part, bandpass filters are placed, which from the entire linear spectrum separate the band of the given channel.

After demodulation, a spectrum with two lateral ones is obtained. The lower lateral should have the spectrum of the original signal. To isolate it, you need to put a low-pass filter.

To properly recover the signal, the carrier values in the transmission and reception must match.

For high-quality speech transmission, the carrier frequency divergence can reach 50 Hz, and for the transmission of non-telephone signals - 2 Hz.

Let's take a real channel:

0.3

3.4

fн1=64 кГц

НБ1

ВБ1

60.6                      63.7  fн1  64.3                     67.4

(64 кГц)

f

Spectrum at the output of the modulator:

На выходе фильтра (выделяется НБ):

f

60.6                      63.7

This signal is combined with the signals of other channels and goes on line.

At the output of the receive path filter:

f

60.6                      63.7

Such a signal is fed to the demodulator input.

At the demodulator output:

0.3                          3.4          64кГц      124.6                    127.7

f

ФНЧпр

After FNCHpr we obtain:

0.3

3.4

f

Если fн пр≠ fн1 пер , то получим:

0.4

3.5

f

All components of the signal are shifted by 100 Hz.

If the line is linearly distorted.

1) Linear distortions are: amplitude-frequency and phase-frequency (ACI and RFI).

FEC on the quality of voice transmission does not have influence, and ACI - have. We can consider the channel line as a four-terminal network:

f

fн                                 fв

K

That there was no ACI, it is necessary to have the following characteristic of the transmission coefficient:

Kл

f

fн                                  fв

α

f

А линия (кабель) вносит искажения:

After passing such a line, the amplitude of the signals that are transmitted at higher frequencies will decrease.

Linear distortion leads to a deterioration in transmission quality, but interference between channels does not occur.

2) Nonlinear distortions.

f1

t

The main parameters by which nonlinear distortions are estimated are: Kr1 - harmonic coefficient, Kni - coefficient of nonlinear distortion, Ar (n) - attenuation of harmonics.

If there is a non-linearity, then the output will be a quasi-harmonic signal.

t

t

или

In the spectrum of this signal there are new spectral components f1, 2f1, 3f1.

f1

f2

At the output in the presence of nonlinear distortion: f1, 2f1, 3f1; f2, 2f2, 3f2; nf1 ± mf2.

At the output there are new spectral components, which were not in the original signal: harmonic of the corresponding order, combinational products.

Kни = U2г2+U3г2+…U1 - the amplitudes of all products of nonlinearity at the output;

U1 – initial component

K2Г = U2гU1 - second harmonic coefficient

Harmonic distortion can lead to transient interferences between channels

U

τи

Tд

t

Practical work №5.

Tд = 1fд

At the receiving end, using the LPF, we get the original signal.

τк = τи + τзащитный , где

τк – channel gap. Without a protective interval, it is impossible, because there will be a mutual influence of the channels.

The transmission must be synchronized. If the synchronism shifts to τk, the subscriber will be connected not to his desired subscriber. Synchronization is ensured by the fact that one channel (maybe several) is engaged in the transmission of the clock signal (CC).

In the transmission systems with VRK, all the pulses are the same, therefore, the determinant of the SS is its determinacy, i.e. it will be repeated after a certain time. The remaining signals are random. Reliability is ensured by the accumulation of information about the SS.

AMI signal:

t

G

Спектр.

плотность

f

fд          2 fд         3 fд          4 fд

-спектр импульсной последовательности (спектр переносчика)

When modulating, the VBP and NBP are obtained.

Коэффициент скважности Kск = Tдτи.

For real systems, Ksk≥1. Range:

fд                2 fд           3 fд

f

G

Форма спектра канального сигнала

Исх. сигнал

ФНЧ

f

G

The carrier is modulated in amplitude, about each fd, 2fd ...... .. there will be two sidebands:

If the Kotel'nikov theorem is not satisfied, then there will be nonlinear distortions. Pulses in the line are distorted because of the limitation of the spectrum from above and below. Restrictions from above are intersignal distortions of the 1st kind. The restrictions from below are intersignal distortions of the 2nd kind.

A line limiting the signal from the top.

R

C

t

заряд C          разряд C

Это искаж. 1-го рода

U

Имп.исх.

Here the transients go fast. Impact of interference on 2, 3 subsequent pulses.

2) Limitation of the spectrum from below

R

- не пропускает постоянную составляющую

C

Момент.заряд С

А

U

B

разряд С

C

DS

t

AB = CD

эта часть будет срезана из-за помехи

These distortions (interference) are more dangerous: effect on 20 ÷ 30 pulses. All this (and 1) and 2)) are intersymbol interference of the 1st and 2nd kind. ISIs are linear distortions.

The transmitted impulse at reception takes a bell-shaped form (spreads out).

2Uэфф.помехи

U0

U0/2

U

t

Аппроксимируется трапецией

After addition with interference, the impulse blurs even more.

With AMI: low noise immunity; interference affects the height of the pulse.

With PCM: the interference only works when reaching its height ½ of the pulse height. Here noise immunity is much higher.

AMI has only official use, it is used as the initial process of signal conversion (in PCM, AMI is also used). DSPs use PCM, are based on three principles:

1) Signal time sampling based on Kotel'nikov's theorem.

2) Quantize the signal by levels. This rounding of the instantaneous values ​​of the signal to the nearest allowed values. Transition from infinite to finite set.

3) Signal coding. The instantaneous values ​​are transmitted by a set of "0" and "1". Coding is the transformation of pulses with a large number of allowed values ​​in a sequence of pulses with a small number of allowed values ​​(additions 1 and 0).

Practical Work  6.

Principles of the formation of standard groups of channels.

PG - primary standard group (12 channels), 12 channels of PM, ΔfI = 48 kHz, spectrum of primary group 60 ÷ 108 kHz.

, so that channel filters are simply implemented.ВГ – secondary group, contains 5 PGs, 60 channels of PM, ΔfII = 240 kHz, the spectrum of the secondary standard group is 312 ÷ 552 kHz.

TG - 5 VH, 300 channels of PM, ΔfIII = (1200 + 32) = 1232 kHz (32 = 8 * 4 - filtering), the spectrum of the tertiary group is 812 ÷ 2044 kHz.

I. Standard GHG.

There are 3 types of equipment with which you can get PG.

1) The method of a single conversion.

104÷108

1

108

100÷104                   12                     2        1

2                                                                          ПГ              . . . . . . . .

:                                                                        60    64            100    104     108   f

:                        104

:                                             60÷64

12                                                                                                   -   исходный сигнал (0÷4)

0.3       3.4

64

Рис. 6.7The relative width of the frequency spectrum should be. <2. In this case, the second harmonics of the spectrum do not fall into this frequency band.

NBP always has an inversion spectrum with respect to the original signal.

Because <2 => the spectrum should be selected in the higher frequency band region, but on the other hand it increases the cost of general equipment => in the lower part of the spectrum => the compromise is chosen - 60 ÷ 108 kHz - in this band the filters have sufficient uniformity and high stability of characteristics .

The complexity of the implementation is defined as follows

: , где ∆fp – the band of the distribution between the NB and the WB. The smaller the A, the more complex the filter.

1) The method of double conversion (using two conversion steps)

1) With the preliminary formation of 3-channel groups, and then 4 carriers are transferred to the standard spectrum of the primary group. There are only 7 carrier frequencies.

I. Standard secondary group (VG).

It is formed from 5 PG, contains 60 channels.

1

420

2

312,3                                     551,7     f

:                           468

Рис. 6.8

fH1 = 420 кГц

fH2 = 468 кГц

fH3 = 516 кГц

fH4 = 564 кГц

fH5 = 444 кГц

I. Standard Tertiary Group (TG).

It is formed from 5 secondary groups. It contains 300 channels.

1

2                                                                                                   . . . . . . . . .

:                                                                   512                               2044     f

:

:

5

Рис. 6.9

fH1 = 1364 кГц

fH2 = 1512 кГц

fH3 = 1860 кГц

fH4 = 2108 кГц

fH5 = 2356 кГц

Practical Work 7.

Differential system (DS).

Рic. 7.1

The main purpose of the DS is to provide a transition from a two-wire local loop to a four-wire channel. This is the 8-pole. Depending on the direction of transmission, the DS can be considered as a passive 4-pole.

3                                                                              4

1                                                                                       2                               2х-проводное окончание

2                                                                                       1

4                                                                                           3

Рic. 7.2

The channel is 4-wire. This is a closed electrical system.

When the energy is transferred from the reception path (PR) to the transmission path (PER), a closed feedback loop (parasitic OS) occurs, which leads to the following negative consequences:

1) If the OS is positive, the amplifiers in the line will become generators, i.e. the amplifier loses its stability or is self-excited. Communication on the channel will cease.

2) There are ACI in the channel.

Properties of DS.

When the DS is examined, it is assumed that it is ideal:

a) the active resistance of the windings is equal to 0;

b) the leakage inductance is 0 (there are no losses in the magnetic circuit);

c) the inductance of the windings is equal to infinity.

Usually, two groups of DS parameters are evaluated:

1) Input resistance of the DS on the side of the clamps (for alignment of directions). There should be an agreed inclusion of the DS.

- Attenuation introduced by DS in various directions.

, since this is a passive scheme.

There are also attenuations:  и .

It is necessary that , а  и .

We calculate and consider the input resistance on the side of the clamps

-1, 2-2, 3-3, 4-4.

- We denote by wa, wc, wd (рис. 7.5).

Coefficients of transformation:

;

In the general case, "m" can be any positive number, but if the DS is used as a transient device, then wc = wd, i.e. m = 1.

In this case, the DS is called the equilateral band.

When the DS balance is Z1 ≈ Z2 (that is, we select Z2).

Z1, Z2, wc, wd - shoulders of the bridge (see Figure 7.6). The condition of balancing the bridge is reduced to the equality of the ampere turns in the windings:

Define resistance by 4-4 clips. Redraw the diagram (at the transformer the ideal R).

;

А4-3

реальное

идеальное

Z2

Z1 = Z2 (m = 1)

Рис. 7.13

There is no real ideal balance, the closer Z2 to Z1, the A4-3 is bigger, but not ∞. In reality, everything is much more complicated: the resistance of the subscriber line depends on many parameters.

When the DS is included in the real channel, it is impossible to achieve a DS balance (Zb = Zl) for the following reasons:

- Zl is frequency dependent;

Zл                                                                                        волновое (характеристическое) сопротивление линии

реальное сопротивление линии с учетом неоднородностей

Рис. 7.14

1) the channel is provided to different subscribers, therefore, different subscriber lines have different parameters, and connection to the DS.

In real cases, as Zb = Z2 = 600 Ohm.

Really:

-return loss (ratio falling to the reflected wave). Abal characterizes how well balanced the AA.

Z1 = Zб                    A1-3 = 3 дБ

Zл

А4-1 = 3 дБ

Рис. 7.15

Zб=Zл   =>   А4-3 = ∞

Really it is possible to make Abal a bit more than 5 dB,

DS can also be on resistors, the circuit is also mΩ:

Standard channel PM.

The PM channel is a unit of measurement of the capacity of transmission systems and is used to transmit telephone signals, as well as data transmission, facsimile and telegraph communication.

Such a channel includes a 2-wire termination and a 4-wire path.

DS serves to switch from a 4-wire path to a 2-wire termination. Extensions have attenuation of 3.5 dB and are called transit.

The main characteristics of the channel PM.

At the channel input 0 dB; at the output of the transit extension is -3.5 dB; at the input of the 4-wire path -13 dB; at the output of the 4-wire path +4.3 dB; at the output of the transit extension is -3.5 dB; at the output of the channel -7 dB. The input and output impedances of the channel are 600 ohms.

The residual damping of the channel is a value equal to the algebraic sum of all attenuations and amplifications in the channel.

В 2х- wire parts .

Effectively transmitted frequency band channel pm (0.3 ÷ 3, 4 kHz) is a band, in extreme frequencies which residual attenuation at 8.7 DB exceeds the residual attenuation on frequency of 1.020 kHz.

Practic work  8.

Frequency converters.

Ω                                                             АМ - сигнал

(модулирующий

сигнал)                      ωО (несущая)

Рic. 8.1

Transformers are necessary to implement the principles of channel division by frequency (FDR) in the construction of SMEs.

By their function, the converters can be individual and group.

With the help of individual converters, the original signal is converted either to the final one (for a single conversion) or to an intermediate spectrum (under multiple transformations).

With the help of group converters, the multichannel group signal moves to the final spectrum or sometimes to the intermediate spectrum.

The principle of their operation is the same, but the requirements are different.

By the nature of the nonlinear elements used in the converter, passive and active frequency converters (IFs) are distinguished.

In passive IF, non-linear elements are used as diodes.

Transistors are used in active IF as nonlinear elements.

Now basically passive converters are used.

The IFs are divided according to the configuration of the circuit into:

1) single-ended inverters;

2) balanced IFs;

3) ring (double balanced) IF.

IFs in the transmitting equipment are modulators, in the receiving equipment - demodulators.

The basic requirements for the drive are:

1) must ensure the obtaining of useful total or difference frequencies of the form (- any frequency of the original complex signal).

This is a combinational frequency of the second order (total or difference). The amplitude of the component should be as large as possible and stable in time;

2) ensuring sufficient suppression of the carrier frequency;

3) the minimum of parasitic products of transformation at their minimum level;

4) if possible at the IF output there should be no combinational frequencies

as a result of the interaction of the frequencies of the original signal.

or ; - The most dangerous, because they are difficult to get rid of.

The fulfillment of these requirements is ensured by the choice of the operating mode of the inverter or the selection of the drive circuit, or both.

From existing schemes of passive inverter are known the following:

1) single-cycle (least used);

2) push-pull or balanced;

3) double push pull or ring.

When comparing the inverter, the following characteristics are used:

1) working attenuation of the IF

РΩ – power, which the source would give in a consistent load with him. - the power of one of the side bands (NB or WB).

ЕС

PΩ                Ω

ZH  = ZC                                                    ωO                       ZH

измерения делают

ZC                                                                                                                                        селективным прибором

(настроенным на частоту)

а)                                                                                  б)

Рис. 8.2

The smaller the appr, the better the IF.

1) the spectral composition at the IF output (G (t)).

The fewer parasitic products, the better the IF.

2) attenuation of the nonlinearity of the converter (AN).

The more AH, the less parasitic products, the better the IF.

Work balanced passive FREQUENCY TRANSFORMER.

ДТр1                                 Д1                 ДТр2                              Еω∑ = 0

+                              +                                                            +

UΩ                                                                                                                                  _                                     RH

-

модулирующий             -

сигнал                                               Д2

+             Uω            -      (диоды открыты)

(несущая от ГО)

Рис. 8.3

2 diode and 2 differential transformers.

In the theoretical analysis, the following assumptions are made:

1) the transformers are ideal (see DS), i.e. there are no losses in them;

2) diodes D1 and D2 operate in a key mode (they can be in one of two states)

a) the diode is open or the key is closed,

The diodes go signals from 2 sources: Ω and ωO. But Uω significantly exceeds UΩ, so state diodes manages Uω.

With AM, mAB <0.1.

You can consider 3 modes of operation of a nonlinear element:

1) the regime of large amplitudes;

2) the mode of medium amplitudes;

3) the regime of small amplitudes.

1) load resistance and diodes are pure active.

Equivalent scheme taking into account the assumptions:              Кл. 1

Кл. 1 имитирует работу Д1.

ЕΩ

H(t)                RH                    H(t) – функция, управляющая

RΩ                               Uω                                          переключением диодов (Uω).

Кл. 2

Рис. 8.5

Timing diagrams.

H (t) is the commutation function;

UBICH - signal in the load.

The INAGR repeats the signal segment UΩ in Fig. 8.6.

UΩ                                                ТΩ

t

Uω      Тω

t

H(t)

1

t

UВЫХ    Тω

t

Рис. 8.6

The diodes are closed - there are no currents. The current of the carrier will not be output; it is included in the middle points of the transformers. The currents of the carrier frequency are directed in opposite directions, they will induce compensating each other and the total EMF (Eω) = 0.

The circuit of the ring converter (double balanced circuit).

Assumptions - similar to the previous scheme.

There is no carrier at the output, the diode is open - the resistance is 0, closed - ∞.

Д1

+

Рис. 8.8

Depending on the polarity of Uω open either D1 and D2 or D3 and D4.

ТΩ

UΩ

t

Uω

t

H’(t)

1

t

UВЫХ  t                                        Рис. 8.9

Здесь H’(t) → одно положение:  Д1 и Д2 открыты;

→ другое положение: Д3  и Д4 открыты.

Апр.кольц. = 4 дБ.

Spectral composition: G(f)

. . . . . . .

ω-Ω    ω+Ω    (3ω-Ω)(3ω+Ω)                   ω

Рic. 8.10

There is no component of the original signal. The amplitude (ω + Ω) and (ω-Ω) is greater than in the previous case. Here, the original signal is suppressed, and not just the carrier.

In real conditions there will be the following differences in the parameters of the converters:

1) Apr ↑ (the attenuation of the transducers increases);

2) there remains ω (Ω). Not very well suppressed, there are remnants of the carrier;

3) additional conversion products appear in the spectrum of the output signal.

Practical work 9.

Filters in SMEs.

One of the most important hardware nodes are filters. Depending on the bandwidth filters are distinguished:

ФНЧ                                     ФВЧ

ПФ                                        РФ

Рис. 9.1

LPF - low pass filter;

High-pass filter; high-pass filter;

PF - bandpass filter;

RF - notch filter.

Attenuation characteristics of bandpass filter has the form:

Аф

ид.

реал.

f1                          f2                      f

Рис. 9.2

KF-channel filters-allocate one of the lateral bands, useful.

ПЕР              КФпер

…

М

КФпр

…                                                                 здесь выделяется канальный сигнал из группового

кан.

сигнал

Рис. 9.3

KF-solo.

Group filters of standard groups distinguish the spectrum of standard groups.

Linear filters are similar to directional filters, they must be paired, they provide separation of group signals of 2 different SPs operating along the same circuit.

One pair of wires, and 2 transmission systems work.

The linear spectrum B - 3 - 3 does not overlap the linear spectrum B - 12 - 2.

B - 3 - 3 - linear spectrum (4 ÷ 31) kHz;

B - 12 - 2 - the linear spectrum (36 ÷ 143) kHz.

In modern equipment, filters collected from inductors and capacitors (LC filters), piezoelectric filters, magnetostrictive filters or electromechanical filters are used. Usually they tend to use LC filters as widely as possible, because they are inexpensive and easy to implement, but these filters can only be used with a large enough bandwidth and bandwidth. Thus, with a bandwidth of 4 kHz, they are most effective in the frequency range from 10 ÷ 30 kHz.

In other cases, piezoelectric and magnetostrictive filters are used. Quartz resonators of such filters have acceptable dimensions in the frequency range above 60 kHz, these filters provide a great slope of attenuation.

The auxiliary filters suppress the remaining carrier frequency, other unnecessary frequencies.

Characteristics (parameters) of filters.

1) Requirement for maximum allowable attenuation in the EEPR band.

The limits of the change depend on the type of filters.

FOr КФ: Апр до 10 дБ;

НФ: Апр – десятые доли дБ.

1) The requirement for the minimum allowable attenuation in the band of effectively delayed frequencies.

In an ideal - ∞, really - not less than 40 dB. For КФ А ≈ 60 dB.

2) Admissible non-uniformity of the frequency response of the filter in the EEPH band: ΔAf - because of it, ACI occurs.

Really: tenths of a dB.

3) Input and output impedance of filters or attenuation attenuation.

Rвх

Rвых

Rн

Rсигн

Рис. 9.5

Rвх should be agreed с Rсигн, а Rвых с Rн.

АНВ – reflection attenuation on the input (usually >20 дБ).

Идеально, когда Rвх = Rс, Rвых = Rн.

Reflection coefficient.

As an example, consider the operation of 2-channel filters on a transmitting station. Assuming that the original signals occupy the spectrum 0.3 ÷ 3.4 kHz, fNES 1 = 64 kHz;

fHE2 = 68 kHz.

Duct filters should allocate NB.                                                                                                            Азад

ПФ1

0,3         3,4

f

fH1                                                             60,6    63,7  64,3    67,4

ПФ2                                              Азад

0,3         3,4

fH2                                                                                                                        f

64,6     67,7  68,3    71,4

Рис. 9.6

From Fig. 9.6 that the WB of the first channel coincides with the second channel of the second channel. Consequently, the filters should provide sufficient suppression of the VB1, otherwise it will create a transient interference in the 2nd channel. It is found that the interference affects the quality slightly, if the attenuation of the channel filter in the effectively delayed Azad band is ≥ 60 dB. Hence, we obtain the requirements for the steepness of the frequency characteristic of the channel filter attenuation.

∆fР-Strip rasfiltrovki, ∆fР = 0,6 кГц;

SФ = 0,1 дБ/Гц.

The amount of ACI in the passband largely determines the type of frequency response of the PM channel. In the transmission system equipment diagram, often 2 or more electric filters operate on a common load, i.e. are included in parallel, which violates the condition of matching the filters with the load. This leads to significant distortion of the signal and an increase in the working attenuation.

Several methods are used to eliminate this phenomenon. For example, when building a standard 12-channel group, a specially selected compensating circuit (KK) is connected in parallel to the load, which ensures the same operation of all filters.

Recently, digital methods of processing information have been widely used. Devices that implement digital linear channel filtering methods are called digital filters and have a number of advantages: high stability and accuracy, independent of the influence of external conditions (for example, temperature), the simplicity of the characteristics change, the ability to work in virtually any frequency range, linearity of phase characteristics. Because digital filters are implemented on integrated microcircuits (for example, registers, counters), they are compact, inexpensive and highly reliable devices. When implementing digital filters, microprocessors are used with great success. Because in the process of signal processing in the DF, analog-to-digital conversion of the input signal, sampling and quantization is required, then inevitably there are specific errors inherent in these operations.

Practical work 10.

Amplifying devices (CU).

Amplifiers are an integral part of terminal and intermediate stations.

- amplifier of a unilateral act.

Signs of the CU:

1) based on the element base:

(a) Electronic lamps;

b) transistors (now mostly used);

c) analog integrated circuits;

2) individual or group.

Individual - in individual equipment PER and PR.

Group amplify the signals of primary, secondary groups.

LUS is a linear amplifier, amplifies the multichannel signal in the linear spectrum of the equipment.

LUSES are used in intermediate points, in PER and PR.

i1            Zвх                                                             Zвых

Z1

i2

U1                                                  U2              Z2

источник питания

источник

сигнала                                                                    нагрузка

Рис. 10.1

The flow of energy from the signal source to the load is controlled by a relatively low power supply.

Amp settings:

1) Zin - input resistance

In most cases, the conditions must be met: harmonization

;

- -inconsistency coefficient indicates which part of the energy is reflected from the amplifier.

Sometimes take instead Z1, Z2 Rнормир. = R0.

Amplification factor in EMF or through increased

или  - voltage gain (if the number of valid).

- voltage transfer ratio.

k(f) – АЧХ; φ(f) – phase characteristic.

помеха

помеха

Р собств. помехи вых.

kМ

Рис. 10.4

At the entrance - only resistance.

Left EMF (source of noise).

Noise factor:

РСПвых.ид.ус. – There are no domestic sources of interference.

- the noise factor.

(4÷6) дБ – малошумящий; 25 дБ – много шума.

shows how much reduced protection from their own noises after passing the signal through an amplifier. To improve parameters (to increase the attenuation of nonlinearity in the first place) used deep total asset:

4 possible options:

1)  - negative OS;

2)  - positive OPERATING SYSTEM;

3)  - lack of feedback;

4)  -Confluent (critical), OS generation amplifier.

The influence of the OS on the amplifier settings.

1.

CCA reduces the gain (k ↓); PIC increases gain (k↑).

2. Если ОС глубокая, то , тогда

Т.е.  зависит только от пассивных элементов.

With a deep OS, the effect of the least stable parameter D is reduced almost to zero. Therefore, k is independent of D. The amplifier becomes much more stable.

With deep OOS it is possible to provide such conditions under which the input and output impedances of the amplifier will be equal to the passive input and output resistances and will not depend on the parameters of the active D-circuit. For this purpose, the input and output devices of the amplifier are made in the form of balanced bridges. In such circuits with the GOOS, its depth is practically independent of the load resistance, which can vary. In the passive part of this scheme, constant and variable correctors 4x-poles are introduced, which allow the adjustment of the gain and the required frequency response of it.

However, it is very difficult to ensure the stability of such amplifiers. in the non-operating frequency range of the OOS can become positive, which will lead to self-excitation. To ensure the stability of the amplifier, special elements and circuits of the high-frequency bypass are used.

To group amplifiers there are strict requirements from the point of view of stability of parameters. They can be performed only in amplifiers with deep general feedback. The stability of the amplifier is estimated by relative instability.

In the presence of OS: decreases by F times. Amplitude-frequency distortions (ACI) also decrease by a factor of F. Nonlinear distortions also decrease by a factor of F; the attenuation of the nonlinearity decreases by a factor of F.

3. Os on security from their own clutter is not affected. Disadvantages OS availability: loss of gain; the propensity of the amplifier to the self-triggering.

Automatic level control (AGC)

ARU is designed to maintain the level of outlet transfer LUZ constant when the gain plot surrounding conditions.

потери ПЗ от нелинейных помех

Рпер                                                        ∆S3

зима

Рпр                                                лето

∆SЛ                          потери помехозащищенности

от собственных помех

Рис. 10.6

To avoid ∆ S, we must adjust the gain in the course of work, automatically.

2 ways: 1) Regulation on the control signal-ARU in CZK. 2) soil temperature Regulation.

АК

терморезистор, меняет сопротивление

в зависимости от температуры грунта

Рис. 10.7

АК – amplitude correction fluid, is included in the OS circuit.

АК

Термодатчик надо выносить, он

зашунтируется тогда емкостью

кабеля – это сложно

Рис. 10.8

The imposition of the sensor due to technical difficulties. Cable shuntiruet sensor, making errors. PACKAGING-small intensity. Drawback: the sensor does not fully reflect the situation with temperature.

ARU in CZK.

основной сигнал

ЛУС

РУ                         kc

ПКК

ГКЧ

pкч

АК

Рис. 10.9

kc – control signal.

РУ – level regulator.

The PAC is the receiver of the control channel.

Depending on which direction (in the "+" or "-") the control signal changes, the adjustment is made with the help of the switchgear.

In linear amplifiers for NUP, PMO, OP, the control oscillations are allocated to the PCC, processed and controlled by appropriate regulators (flat, inclined or curved). The number fkh is determined by the type of transmission system: since systems operating on a symmetrical cable have a rather complex dependence of the fading of the line on f and tорр => three fкч. Flat fkch and oblique fkh are located on the boundaries of the spectrum, and curvilinear in the middle. In coaxial cables, a more simple relationship and apply 1 fkh in the upper part of the spectrum.

The reference frequencies are selected outside the linear frequency spectrum in order to avoid interference to channels on channels.

ауч                                         tmax                       S

ауч

tcp                                                S

tmin                                                   ауч

fH                       fB        f                                  fH                       fB        f

а)                                                                     б)

Usually on the track each amplifier is supplied with a TARU, and after 5-6 amplifiers they put AGC on KC, which increase the accuracy of the adjustment.

AGC on CC: requires the controllers to lock when the control signal disappears. Otherwise, amplification of amplifiers increases to the limit, and (residual attenuation of channels) decreases to a minimum, the channels are self-excited, the amplifiers are overloaded, the supply of kc becomes impossible.

Interference in the paths and channels of analog transmission systems.

Interference is called extraneous electrical fluctuations, which interfere with the normal reception of signals.

Depending on the nature of the impact, the interference is subdivided into additive and multiplicative.

Additive interference is a random signal that is superimposed on useful signals transmitted through channels and paths.

The effect of additive interference is described as follows

the total voltage of the channel or path output.

Multiplicative interference is due to random changes in the transmission coefficient of the channel or path as a function of time.

- результирующее напряжение.

In addition, interference can be classified as follows:

1) Depending on the scene:

- external;

- internal.

Internal: the equipment arising in the nodes: intrinsic interference, interference of non-linearity, interference due to passing flows, bad contacts at the junctions and channel switching.

External: due to the action of external sources: from linear transitions from parallel operating circuits, railways, industrial devices, etc.

1) Depending on the form of the interference:

- continuous, characterized by inequality:

;

-Pulse:

,

Where is - maximum voltage, - voltage signals.

1) Depending on the spectrum:

- solid: characterized by the distribution of power over a wide range of frequencies (for example, white noise);

- discrete or selective: its power is concentrated either on one frequency or in a very narrow band.

2) For interference, interference is subdivided into noise and transient conversation.

Noise masks the signal, thereby ↓ intelligibility.

Transient conversation can be intelligible and indistinct.

The intelligible hinders, violates the secrecy of communication, and the slurred ↑ noise in the channel.

Methods for assessing interference effects.

1) Security

;

.

Noise factor

Physically, this value shows how many times the ratio of the signal power to the power of the interference decreases when the signal of this 4-pole is included in the signal path.

3) The evaluation of interference by psofometricheskogo voltage is made when transmitting voice over the telephone channel and broadcasting channels, tk. the sensitivity of the telephone-ear system is not the same for different frequencies. The maximum is from 800 ÷ 1000 Hz. If there is interference in the channel, then its frequency components will also be perceived differently by the telephone-ear system.

Psophometric voltage (psophos noise) of interference is a voltage with a frequency of 800 Hz, which, in its interfering action, is equivalent to a real noise, it is always less than interference.

For the channel ТЧ .

Normalization of interference.

To ensure high quality of communication according to the recommendations of the MSES. The interference is normalized for the PM channels of the reference circuit. The length of the chain for cable, air and RRL is 2500 km.

According to ITU standards, in TNOU at the output of the channel of the PM of the reference cable chain, the power of all types of interference should not exceed:

Из них:

- line channel;

- Converts all stations equipment.

Т.е. на 1 км линейного тракта , а для современных систем эта норма 1пВт/км.

Own interference.

These are fluctuating noise: thermal, because of the shot effect in transistors, lamps and p / p (shot noise - due to a random deviation of the anode current).

Own interference accumulates along the signal transmission path.

To combat its own interferences:-use amplifiers; -introduce emphasis level of transfer.

Рпер                                                                     алин

f                                                                         f

а)                                                                        б)

Рис. 10.11

Interference from linear navigation.

This electromagnetic influence between pairs of cables. Symmetric cables.

на дальнем конце                                                                     на ближнем

конце                                                                                         конце

(Это влияние хуже)

Рис. 10.12

Nonlinear interference.

They arise due to the nonlinear characteristics of the nonlinear elements of the nodes of the path.

Basically, the LUS: new frequency components appear on the output of the LUS. These components can cause nonlinear transitions and nonlinear noise in the channels. Therefore, the signal voltage should not exceed the amplifier overload threshold. According to the ITU norms, the overload threshold is the output power level of the LPOS output, in which an increase in the level at the input of the LUS by 1 dB leads to an increase in the level of the third harmonic at the output of the LUS by 20

Practical work 11

The meaning of digital methods of signal transmission.

We know that telecommunication signals can be continuous and discrete.

In general, signals that reflect natural phenomena are analog signals. Analogues can be either continuous or discrete (in time) signals.

A continuous signal can be replaced by a discrete one (in time), applying Kotel'nikov's theorem.

The discrete signal can be quantized by level, rounding the instantaneous values ​​of the signal to some allowed values.

Quantization turns an analog signal into a digital one, i.e. into a signal whose instantaneous values ​​form a finite set.

The most convenient number system for digital devices is a binary system. Therefore, quantization is usually combined with coding.

When encoding, the received values ​​are written in binary code, in the form of a sequence of zeros and ones.

In general, digital signals differ in the required transmission speed (number of bits per second). A bit is a number that takes the value "0" or "1".

The transmission and processing of signals in digital form has the following advantages over the transmission of analog signals:

1) Unification of various types of transmitted information;

2) Integration of switching systems, i.e. creation of fully digital telecommunications networks;

3) Computerization of telecommunications equipment;

4) High noise immunity;

5) Stability of the channel parameters. The stability of the residual attenuation, amplitude and frequency characteristics is determined mainly by signal processing devices in analog form;

6) High technical and economic indicators.

Digital integrated circuits are used, which reduces the cost of equipment, their size, energy consumption.

The reliability of digital equipment is increasing.

Formation of a digital signal.

The digital signal is generated as a result of the analog-to-digital conversion (ADC) of the signals.

First, the continuous signal is sampled on the basis of the Kotel'nikov theorem, which states that any continuous signal limited by the frequency fc can be reconstructed from its discrete readings following the sampling frequency fd 2fv

Then quantization of discrete samples follows.

Quantization is the rounding of the instantaneous values ​​of a signal to the nearest allowed values. This operation actually turns the analog signal into a digital one.

As a result of quantization, quantization errors inevitably arise.

The smaller the distance between the nearest allowed levels, i.e. The smaller the quantization step, i.e. The larger the number of allowed levels, the smaller the quantization error.

In modern DSPs, the quantization operation is almost always combined with coding.

Coding is a very important operation. In DSP, coding is understood in a narrow sense, as a transition from a high-base code to a low-base code. Instantaneous values ​​can take allowed values ​​(for example, 256) are replaced by pulse combinations (code groups consisting, for example, of 8 pulses) that have little allowed values ​​(at least two). This increases the noise immunity of the signal.

In addition to quantization, quantization and coding, multiplexing is also implemented in the DSP - the combination of several streams into a group stream.

ADC operations on the transmission must correspond to the reverse reception operations (digital-to-analog conversion of the DAC).

As a result of the decoding, the instantaneous values ​​of the signal are restored, which after passing through the low-pass filter with a cutoff frequency Fb, become a continuous initial signal.

The reconstructed signal differs from the original one, since the signal is restored not exactly by the original instantaneous values, but by rounding to the

Practical work 12

General principles of building a DSP.

Generalized structure of the DSP.

FNCHper -limits the signal on the spectrum

AMI - time sampling of the signal

Кв - quantization by levels (or by amplitude)

Code. - the output will be a digital signal in the form of PCM

ФЦ - synchronization signals, alarms, с.с. etc. (service signals) can be constructed: 1) PCM-30, U = 2048 kbit / s 2) PCM-15, U = 1024 kbit / s (up to the dotted line)

Next: the hierarchy adopted in Europe: (PTS). The primary digital stream is 2048 kbit / s, before this stream - bcc (main digital channel) bcc - 64 kbit / s; 2048 -І (or 2 Mbit / s) 8448 -ІІ (secondary digital stream, 8 Mbit / s) 4 І

OVG - allows you to combine 4 digital streams of a lower level into a higher one

34368 - III (34 Mbit / s)

139264-IV (140 Mbps)

Next - the scope of the synchronous digital hierarchy → SDH

OVG - temporary multiplexing equipment.

After the OVG is a unipolar signal, it can not be transmitted along the linear path, it must be converted and this makes the PCPer, at the output the signal will be bipolar. The codes used to transmit on the line are many.

In Fig. 1. "7" points, the diagram.

The duration of the transmission cycle is the period of time in which one element is transmitted (element - codeword, combination code, for example) for each of the N system channels plus the necessary service information (symbols)

The transmission rate or the clock frequency determines the number of symbols transmitted in a linear path per unit time.

Fд = 8000 (Гц)

На один канал: Fд·m , для N каналов:

где m = 8 – количество разрядов в кодовой комбинации,

N – кол-во каналов;

Для ИКМ-30:

Fт = 8·8(30+2)=2048 кГц;

U=C=2048 кбит/с

Uniform quantization

The quantization of the signal by level is the main operation of the analog-to-digital conversion of the signal, as a result, the instantaneous values of the signal are rounded to the nearest allowed levels. (See Fig.3)

In Fig. 3 shows the AMI signal, eight quantization levels are marked vertically (from 0 to 7). The distances between the levels are the same, i.e. there is a uniform or linear quantization with step δ0. The arrows in the figure indicate the values ​​to which the amplitudes of the original pulses are rounded. The voltage of the 7th level corresponds to the voltage of the restriction uogr.

Obviously, when the signal is quantized, Erb errors arise (they are shaded in Figure 3), the magnitude of which is random, has a uniform distribution and does not exceed δ0 / 2. As a result, the signal after quantization is the sum of the original signal and the error signal, which is perceived as a fluctuation noise.

Such power is disturbed in a channel with a bandwidth of 0 to

The probability density of the quantization error is 1 / δ0, because the error has a uniform distribution in the interval from - δ0 / 2 to + δ0 / 2.

Рис. 4

The magnitude of the quantization step (using a symmetric code) can be expressed through the constraint voltage uor of the encoder and the number of quantization steps Nkv

А  Nкв =  , где m – разрядность двоичного кода.

The power of the quantization interference must be compared with the minimum value of the quantized signal, from which it will be possible to determine the required number of quantization steps and, consequently, the number of bits of the binary code m.

Protection from quantization noise for the weakest signals:

Where Дс - a dynamic range of a signal, Кпсоф - psofometrichesky the factor equal for channel PM 0,75.

It has been experimentally established that a sufficiently high quality is provided with an A2 kV 22 dB

The operational margin ΔA = 4,5 dB is taken and the noise immunity is reduced by 11, dB for the transit follow-up of the PM channels, we obtain

The interference immunity increases proportionally with the signal level until the Pc max corresponding to the limiting voltage is reached. The further growth of the signal will lead to an error due to the amplitude limitation of the signal

and Azkv will begin to decline due to interference restrictions (overload). This is seen in Fig. 5 for a characterization with 8-bit coding.

All these shortcomings can be eliminated by using non-uniform quantization (non-linear quantization). Its essence consists in the following: the quantization steps are reduced for small instantaneous values ​​of the signal due to the increase in steps for large values. The use of non-uniform quantization is equivalent to compressing (compressing) the dynamic range of a signal.

To avoid non-linear distortion at the reception, there must be an inverse operation - expansion (expansion) of the dynamic range.

The condition for the constancy of As for all levels of the signal is the constancy of the relative increments of the signal in the entire range of its changes, i.e.

where x and y are the normalization of the values ​​of the voltages of the input and output signals in the quantization device.

Quasilogarithmic A and μ characteristics are realized..

Рис. 5

Practical work 13

Continuous quantization of signals by level.

The quasi-logarithmic A-characteristic is described by the following expressions

а µ - characteristics of expression

In the DSP, segmental non-uniform quantization characteristics are used, since they are simply implemented on a digital basis.

In the next lecture, a characterization of type A 87,6 / 13 (positive branch, applied in the ADC of the DSP of the European hierarchy, the characteristic is symmetric with respect to zero, its positive and negative branches are each of 8 segments, each segment is divided into 16 identical steps inside the segment !!!!!!

A large number of digits in the code (m = 12) with uniform quantization leads to a complication of the equipment and an unjustified increase in the clock frequency. It can be eliminated by carrying out uneven quantization, which is used in modern DSPs. As a result, the code capacity is reduced to m = 8 (Nkv = 256), while ensuring compliance with the requirements for protection from quantization noise in a wide dynamic range of the signal Ac, which is about 40 dB.

The effect of non-uniform quantization can be obtained by compressing the DDS with subsequent uniform quantization. Compression DDs is carried out using a compressor with a nonlinear amplitude characteristic.

To restore the original DD at the reception, it is necessary to install an expander (expander), whose AX must be the inverse AX of the compressor.

General principles of signal coding in DSP.

In the encoding process, the amplitude of each quantized AMI sample is represented as a binary sequence containing m symbols (m-bit code combination). To determine the structure of a combination in the simplest case, we need to write in the binary code the AM amplitude of the NAIM sample, expressed in the quantization steps. In this case:

HАИМ

где аi = (0,1) – the status of the corresponding bit combination;      -all the corresponding bit in the conventional quantization steps.    Example: m = 5, HAIM = 26, then the code combination will have a structure

11010 , т.к.        26 = 0 · 2º + 1 · 2¹ + 0 ·2² + 1 ·2³ + 1 ·  → 11010

A sequence of m-bit combinations represents the group signal to PCM, called tact digital.

The amplitude of the counts arriving at the encoder input in this case can take values ​​in the range of NAIM = (0 ÷ 31) of the conditioned quantization steps, and at the output of the encoder a digital signal and PCMs are formed, which are a sequence of five-digit code combinations.

For telephone signals with unequal quantization, it is necessary to use the eight-bit code (m = 8, and at uniform - 12-bit (m = 12) .In practice, binary codes of the following types are found: natural binary code, symmetrical binary code, reflex binary code Gray).

1). Symmetric binary code is mainly used for encoding bipolar signals (for example, telephone signals). In this code, for all positive samples the sign symbol has a value of 1, and for negative samples equal in amplitude, the structures of the code combinations completely coincide (except for the sign discharge), i.e. the code is symmetric. For example, the maximum signal corresponds to the code 11111111, and the maximum negative value corresponds to 01111111. The absolute value of the quantization step

2). Natural binary code is mainly used for encoding unipolar signals.

The number of combinations of different structures is 256, with the minimum signal corresponding to the combination 00000000, and the maximum signal being 11111111. The absolute value of the quantization step. With the help of natural binary code, bipolar signals can also be encoded, ensuring their displacement beforehand, as shown in Fig. 1. In this case, obviously, the amplitude of the encoded samples changes, and the transition from the sample amplitude Hc, expressed in the quantization steps, when using a symmetric code to the amplitude of the same sample Hn using natural code and vice versa, can be performed as follows (Fig. 1 and 2):

Natural and symmetrical binary codes are the simplest. For both natural and symmetric code, an error in one of the symbols can lead to a significant distortion of the signal.

If, for example, in the code combination of the form 11010011 the error is in the fifth digit, i.e. a combination of 11000011 is adopted, then the amplitude of the samples will be smaller than the true value by the conditional quantization steps.

The most dangerous are the errors in the senior ranks. (P8, P7).

Linear coding is the coding of a uniformly quantized signal, and a non-uniformly quantized signal is called a non-uniformly quantized signal.

The code generated in the encoder is called parallel if the signals (1 and 0) included in the m-bit code group appear on different outputs of the encoder at the same time, with each output of the encoder corresponding to a signal of a certain bit.

The code is called sequential if all the signals included in the m-bit code group appear on the same output of the encoder alternately with a time shift (starting with the highest bit).

Lecture 14.

Guidelines for coding signals.

In the DSP, m = 8.

Most often, the encoding devices are weight-type encoders.

There are linear and non-linear encoders.

The simplest scheme of a 3-bit linear encoder.

Рис.1

СВ – subtraction scheme;

СС – Schema Compare.

Nкв=2m

Если m=3, то Nкв=8; δ – шаг квантования.

ZАИМ – the amplitude of the input reference device in steps of quantization.

1δ, 2δ, 3δ…– веса разрядов.

Доп. ZАИМ=6δ; Uэтал.1=4δ (это в (1) старшая кодовая комбинация 110).

Linear encoders are usually not used in real DSPs, but are used

Nonlinear encoders (also of the weighing type).

The difference in the magnitude of the reference stresses, another order of their inclusion.

Principles of nonlinear coding.

Characteristics of companding:

A-87.6 / 13 (in real encoders)

Implement a smooth compressor characteristic is difficult. It is approximated, as it were, by segments, chords.

We depict in scale.

δ0=2-11·uогр       xyz+ABCD

Рис.2

Quantization steps:

δi=δ0,   i=0,1δ0·2i-1,   i=2÷7

Рассмотрим δ7δ0=2-52-11=26=64 раза, т.е. шаг квантования в 7–м сегменте в 64 раза больше, чем в 0–м сегменте.

Inside the segment, δ = const.

Segments have numbers from 0 to 7 (Fig. 2), starting from the central part of the characteristic. The steps inside each segment are numbered from 0 to 15. Segments approximate the smooth curve y = f (x), which is described in the previous lecture. In the zeroth and first segments, the step is minimal and equal to Δ, and in each subsequent segment the step size is doubled relative to the previous one.

Рис.3

The beginning of the graph - the inclined line - corresponds to the zero and the first segments. This is the zone of uniform quantization, Asqv increases in proportion to the growth of pc. When you go to the second segment, Azkv jumps by 6dB, since in the second segment, the quantization step size is 2 times larger. This picture is repeated when going to each subsequent segment - this is the zone of uneven quantization. When the upper limit of the seventh segment (Uorr) is reached, an overload zone sets in, where Azkv falls sharply, but already due to the noise of the signal limitation.

Extras. we encode the segments: xyz and within the segment of 16 quantization steps → 0 ÷ 15.

ABCD - step number code.

The structure of the combinations at the output of the nonlinear encoder:

1 такт: the definition of a symbol (0 или 1)

2÷4 такты: definition of NC and the code segment number

4 такт → compare Асегм. С эталоном 116 ( 12 всех сегментов). Further: If the amplitude is greater than 1/16, then turn on the standard  14 и т.д.

5÷8 такты: standard linear encoding (4-bit linear encoder), but δkv depends on the segment number.

Practical work 15.

Principles of signal regeneration.

An important feature of the digital signal transmission method is the possibility of restoring the transmitted pulse sequence after passing through the medium introducing distortion and interference.

Distortions and disturbances distort the shape of the pulses and their duration. Therefore, in the linear path of the DSP, regenerators are installed to restore the parameters of the digital signal over certain distances.

Regenerators perform the following functions:

1) Amplification with AGC and correction of pulse shape to rectangular.

1) Strobing of pulses, as a result of which the pulses of the digital signal at the output of the regenerator are formed at strictly certain moments.

2) Decision making: comparison of the gated pulses with the threshold value Upor ±.

3) Clock synchronization - the timing of the regeneration process.

The block diagram of the regenerator is shown in Fig. 1:

Рис.1. Структурная схема регенератора

Вх. у–во – an input device that serves to match the resistance of the line with the input resistance of the regenerator (typically a linear transformer connects a symmetric cable circuit with an unbalanced input of a linear regenerator);

Cor. u-l is a correcting amplifier designed to compensate for the attenuation of the regeneration section and correction of the amplitude-frequency distortions introduced by the line, i.e. the pulse shape is corrected;

PU - threshold device. It is designed to determine the excess of a signal over interference. When the amplitude of the pulse is greater than Upor, the output of the PU will be a pulse, and when the amplitude of the pulse is less than Upor, there will be no pulse at the output of the pulse.

The pulses from the output of the PU come to the decision device.

The decision device is necessary for gating in each clock cycle of incoming pulses.

In the timing device, the clock frequency is extracted, strobe pulses are formed and they are fed to the decision device.

If, at the time of receipt of the stroking pulse, a pulse is also sent to the decision device from the output of the PU, an impulse appears at the output of the decision device. And if at the moment of gating the pulse does not come from the PU output to the decision device, then there will be no pulse at the output of the decision device.

Ex. Y-in - output device, which serves to match the output resistance of the regenerator with the line.

Fig. 2 shows time diagrams showing the principle of regeneration.

Рис.2

The linear digital signal from the output of the terminal station (Fig. 2a) enters the line where it undergoes damping, distortion and interference affect it (Fig. 2b). Then the signal goes to the correcting amplifier, at the output of which it looks like Fig. 2c. If the signal at the input of the threshold device PU exceeds the threshold voltage Upor, a pulse is applied to the input of the RU, and gating pulses are also fed to the RU. In this case, there will be an impulse at the output of the switchgear (Fig. 2d). In the reactor, the initial amplitude and duration of the pulses are restored.

It should be remembered that the frequency response of amplification of the correcting amplifier should coincide with the frequency characteristic of the decay of the regeneration site.

It is necessary to choose the optimum threshold voltage value.

The error rate will be determined as follows:

Kosh = Nosh / N,

where Nош - number of erroneous symbols;

N is the total number of characters.

The bucket should be no more than 〖10〗 ^ (- 6), and for one regenerator the requirements are more stringent Kosh <〖10〗 ^ (-11), because Errors accumulate along the trunk. At the same time, the distortion of the shape of the pulses and their change in location in time do not accumulate. As we saw above, the only deterioration in transmission quality is the appearance of errors as a result of incorrect decision-making due to interference and distortion in the regeneration area.

List of recommended literature

- Гусев В.Г., Гусев Ю.М. Электроника и микропроцессорная техника. Учебник. - 6-е изд., М.: КНОРУС, 2013. — 800 с.

- Давыдов М.В. Моделирование, компьютерное проектирование и технология производства электронных средств. -  Мн: БГУИР, 2013. – 281 с.

- Прянишников В.А.: Электроника: Полный курс лекций. – Спб.: Корона принт., 2004. – 415 с.

- A.K. Singh. Foundation of switching theory and logic design. – New age international (P) limited, Publishers. 4835/24, Ansari Road, Daryaganj, New Delhi – 2008. – 391 p.

- Anil К. Maini. Digital Electronics: Principles, devices and Applications. - John Wiley & Sons, Lid. – 2007. – 727 p.

- Hilary. D. Brewster. Digital Electronics. First Edition 2009, Oxford Book Company. - 267, 10-B-Scheme, Opp. Narayan Niwas.

- Тревис Дж. LabVIEW для всех. - М.:ДМК Пресс; ПриборКомплект, 2004.

- Пейч Л.И., Точилин Д.А., Поллак Б.П. LabVIEW для новичков и специалистов.     - М.: Горячая линия - Телеком, 2004.

- Грабовски Б. Краткий справочник по электронике. – 2-е изд.испр. – М.: ДМК Пресс, 2004. – 416 с.

- Электротехника и электроника в экспериментах и упражнениях: Практикум на Electronics Workbench: В 2 т. /Под общей ред. Д.И.Панфилова - Т.2: Электроника. - М.: ДОДЭКА, 2000.

Amirbekova A.I., Kayumova U.R., Kultas A.K.

Methodical instructions to practical works

for discipline

MULTICHANNEL TRANSMISSION SYSTEM

for students of the specialty 5В071900 – Radiotecnics, Electronics and Telemmunication

It is sent for printing

Format of paper Х*Y 1/16

Paper is typographical. Press is offset. Volume 2 items of l.

Circulation ___ copies. Order № ___

©  Edition of М. Auezov South Kazakhstan State University

Publishing center of M. Auezov SKSU, Shymkent city,

Tauke-khan avenue, 5