MINISTRY OF EDUCATION AND SCIENCE OF

THE REPUBLIC OF KAZAKHSTAN

M.Auezov South Kazakhstan State University

Chair of «Industrial, civil and road engineering»

Kunanbayeva Y.B.

Lecturenotes

on discipline «Construction machines and equipment»

for students of the specialty 5В072900 "Construction"

Shymkent, 2018

удк 621.878 (075.8)

The compiler: Kunanbayeva Y.B.

Lecture notes on discipline «Construction machines and equipment»

– Shymkent: M. Auezov SKSU, 2018 – 77 p.

Lecture notes are compiled in accordance with the requirements of the curriculum and the program of the discipline "Construction machines and equipment" and includes all the necessary information on the course.

Information is provided for solving problems in machines and equipment used in the construction of underground, aboveground parts of buildings, as well as questions on the choice of vehicles.

Lecture notes are intended for students of the specialty 5В072900 "Construction".

Reviewer: Dosaliev K.S. - Ph.D., senior teacher of the chair "Industrial, civil and road engineering"

Lecture notes has been approved and recommended at the meeting of MMC M. Auezov SKS (Protocol No.___ dated from ___.___.2018)

© M.Auezov South Kazakhstan State University , 2018

Responsible for the issue of Kunanbayeva Y.B.

CONTENT

| Introduction | 4 |
| --- | --- |
| 1.Theme 1. The requirements for construction machinery as machine-machinery, providing mechanization and automation of construction. Classification of construction machinery. Separation of machines into types and sizes. | 5 |
| 2.Theme 2.The transport vehicles. Description of construction cargo. Basic parameters, operational characteristics of transport vehicles. Purpose, design schemes, workflow and technological capabilities, basic parameters, performance characteristics of transporting machines. | 10 |
| 3.Theme 3.The classification of hoisting machines. Maintenance zones of cranes of various types. Basic parameters and indexing systems. Application area. Requirements for the device and safe operation of load-lifting machines. machines. | 13 |
| 4. Theme 4. Loading and unloading machines. Types of forklifts. The working process. | 22 |
| 5. Theme 5.The general information about machines for excavation. Classification of machines for excavation. The working organs and their interaction with the soil. | 26 |
| 6. Theme 6.Earthmoving machinery. Purpose, scope. Main technical and economic indicators. The working process. Prospects for the development of earth-moving machinery. | 29 |
| 7.Theme 7. Excavators. Classification, scope. The main types of working equipment, parameters and indexation of construction excavators. | 36 |
| 8. Theme 8.Machines and equipment for frozen soils. The use of machines with traditional organs for the development of frozen soils. Types of special machines. | 41 |
| 9. Theme 9. Drilling machines and equipment. Drilling tool. Machines for drilling holes. Crane-crane machines. Technical and operational indicators of the main types of drilling machines. Pile-driving equipment. Equipment for piling with shock and vibration methods, underwashing and screwing. | 48 |
| 10. Theme 10. The types of crushing machines and their choice depending on the strength of the crushing material and the degree of grinding. Schematic diagrams, workflow, basic parameters and productivity of crushing machines. | 54 |
| 11. Theme 11. Appointment and composition of the mixing machine. Classification of mixing machines. Purpose of the dispenser and their classification. Types, basic parameters and design schemes of concrete mixers and mortar mixers of cyclic and continuous action. | 58 |
| 12. Theme 12. Machines and equipment for the preparation and transportation of concretes and mortars. | 61 |
| 13. Theme 13. Machines for plastering and painting works. Purpose, composition, basic parameters and area of application of plaster stations and aggregates, manual trowels. | 65 |
| 14. Theme 14. General information, classification of manual machines for their intended purpose, the principle of operation, the nature of the movement of the working body, the operating mode, the type of drive and protection classes. Basic requirements for hand machines. 15. Theme 15. Acceptance and delivery of machines in operation.System of maintenance and repair of machines. | 69 71 |
| Questions for the current monitoring of students' knowledge | 76 |
| Criterion of evaluation of students' knowledge | 79 |
| List of literature | 82 |

Introduction

Modern forms of organization of construction and new methods of production of construction work require from the specialists high knowledge, skills and skills aimed at increasing the level of mechanization and automation of production. At the same time, special attention is paid to the economic feasibility of choosing one or another means of mechanization, a group of machines, devices, devices and other devices that exclude the use of manual labor in construction.

The rational formation of a fleet of construction machines in accordance with the technological processes that are being carried out, their efficient production operation and clear regulation of maintenance are important tasks facing the builders in modern conditions.

In these conditions, the role of construction machinery as an important link in the construction system at all stages of production sharply increases.

Lecture notes are compiled in accordance with the program of discipline for students of specialty 5В072900 "Construction".

Purpose and objectives of lessons

The purpose of lessons is to prepare a specialist who is proficient in the necessary information about construction machines and equipment for mechanization and automation of technological processes in construction, technological capabilities of machines, their use with the highest efficiency, depending on the characteristics of the construction site.

The task of the discipline is to study the general arrangement of machines and equipment, their parameters, work processes, basic design and performance characteristics.

As a result of studying this discipline, the student must:

know

- conditions for achieving the highest productivity;

- technological capabilities with basic and interchangeable types of working equipment,

- directions of development and perspective designs of machines;

- the order of putting machines into operation;

- the main provisions and rules of technical supervision over the use and safe operation.

be able to

- Carry out a selection of machines and equipment for efficient automation and mechanization of construction work, depending on the technology adopted and the specific volume-planning and structural characteristics of the construction projects;

- Develop technical requirements for new construction machines and other means of mechanization.

# The session content

In the materials of lecture notes, the main concepts and provisions, the order of solving problems are given, tasks are given for independent consolidation of the material and control questions for the self-examination of knowledge.

As a result of training, the specialist will present how the machine is arranged, how the main parts are calculated, from which materials it is possible to produce the appropriate parts, how to organize the operation of the machines and their complexes, and also learn how to choose the machine and the group of machines from among the typical ones industry.

Lecture №1.

Topic:The requirements for construction machines as machines - tools that provide mechanization and automation of construction. Classification of construction machinery.Separation of machines into types and sizes.

Outline of a lecture:

1. Basic concepts and requirements for machines.

2. Classification of construction machinery.

3. Indexing of construction machines.

4. Machine parameters. Nominal size and model.

1. Basic concepts and requirements for machines.

Each machine consists of assembly units (elements) that perform certain functions in its operation: power equipment (one or several engines) for obtaining mechanical energy; working equipment for direct impact on the processed material and the performance of the specified technological process; running equipment (for portable and stationary machines it is not available) for moving the machine and transferring its weight and workloads to the supporting surface; transmission mechanisms (transmission), connecting the working and running (for self-propelled machines) equipment with power; control systems for starting, stopping and changing the modes of operation of power equipment, switching on, off, reversing, regulating the speeds and braking mechanisms and the working body of the machine; carrying frame for placing and fastening on it all the nodes and mechanisms of the machine. Assembly units of many construction machines are unified.

The machine is a device that does useful work with the transformation of one type of energy into another. It consists of a number of mechanisms for various purposes, united by a common hull, frame or frame. Mechanisms include nodes in the form of complete assembly units representing jointly operating parts. The part is part of the machine, manufactured mainly from the homogeneous by name and brand of material without the use of assembly operations. They are subdivided into simple (rivet, pin, key), complex (distributive shaft, reducer and motor housing), common (bolts, shafts, cogwheels) and special purpose, used in various types of machines (crane hooks, the bucket of excavators, the pistons of pumps).

The main requirements for detail are the simplicity of their forms, cost-effectiveness (material cost, production and operation costs) and reliability (the ability to keep their performance in time). The efficiency is determined, both for individual and joint indicators of strength, wear resistance, heat resistance, rigidity, stability and vibration resistance. The values of the required indicators depend on the working conditions of the parts (for fasteners - durability, for the leadscrew - durability). However, the main indicator for most details is strength - the property of the part to resist the change in shape (destruction) under the influence of external loads.

The stresses from the loads acting on a part can be constant and variable in time. The stress variables, in turn, are divided into symmetric, asymmetric, sign-constant, alternating and pulsating.

When calculating machine parts for strength at constant or alternating stresses, the corresponding limits of strength and endurance (for tension, compression, bending and torsion) are taken as the ultimate stress, as well as the coefficients of durability according to tabular data. To determine the required dimensions of the part, design calculations are made for the voltages allowed, and then a refined verification calculation for the safety factor coefficients.

The reliability of the details depends on the manufacture (accuracy of the work), and the quality of the material used, as well as the correct choice of the types and operating modes of the parts.

2. Classification of construction machinery.

In construction, a significant number of machines are used that differ in their intended purpose, design, principle of operation, size, parameters, etc. We consider the basis for the classification of construction machinery and equipment.

By designation (technological sign), the machines are divided into transport vehicles; transporting; loading and unloading; chest-hoisting; for excavation works; for piling works; for processing and sorting of stone materials; for the preparation, transportation, packing and consolidation of concrete and mortar mixtures; for compaction of soils; for repair and maintenance of roads; for finishing works; hand machines. Each group is divided into subgroups (bulldozers, scrapers, excavators in a group of machines for excavation). Inside subgroups, machines of individual types differ in the design of knots or machines as a whole (single-bucket excavators with forward or backward shovel, trench rotary or chain, stepping, with transverse digging). Each type of machine has a number of standard sizes (models), similar in construction, but differing in individual parameters (bucket capacity, size, weight, power, capacity). When manufacturing machines of a single standard series, standard components and standardized assembly units are widely used.

According to the mode of operation (the principle of operation), machines of the periodic (cyclic) action are distinguished, performing work by periodically repeating the same alternating working and idle operations with cyclical production (bulldozers, scrapers, one-bucket excavators) and continuous operation machines, issuing or transporting products by continuous flow (multi-bucket continuous excavators, conveyors). Machines of cyclic action distinguish their versatility and adaptability to work in different production conditions, and machines of continuous action - increased productivity. There are machines and combined action (walking excavators, transverse digging excavators to form channel slopes, etc.).

By the degree of mobility, the machines are divided into portable, stationary and mobile (including in the body of vehicles, tractors and semi-trailers to trucks, tractors, tracktractors and self-propelled).

By type of running equipment distinguish cars on caterpillar, pneumatic, rail, walking and combined.

By the form of power equipment, machines are subdivided into working from electric motors and internal combustion engines. The former have great readiness to work, but depend on the availability of electricity, while the latter do not depend on energy sources and are autonomous. Many construction machines have a combined drive using hydraulic and pneumatic motors. These include diesel-electric, diesel-hydraulic (most common), diesel-pneumatic, electro-hydraulic, electro-pneumatic, and the like.

By the number of engines installed on the machine, they are different single-engine (all mechanisms are powered by a single power plant) and multi-engine (for each mechanism is pre-engineered an individual engine).

According to the control systems, the machines are divided into mechanical (handles and pedals, actuating the leverage system), hydraulic (pump-free and pump-driven), pneumatic (using compressed air), electric (with using electrical warfare) and combined (electrohydraulic, pneumo-electric, etc.).

In terms of the degree of universality, the machines are divided into universal multi-purpose vehicles, equipped with various types of quick-detachable working tools, devices and equipment for performing a wide variety of technological operations (construction single-bucket excavators, loaders) and specialized ones, having one type of working equipment and intended for performance of only one technological process (crushing machines, concrete pumps).

By the degree of automation, machines with mechanized control are distinguished, with automated control and monitoring based on microprocessor technology, with automated control at a distance, with automatic control based on microprocessors and mini-computers, construction manipulators and robots, and robotic machines and complexes.

A more detailed classification for specific machines and equipment will be given in the relevant chapters of the book.

3. Indexing of construction machines.

All the construction machines produced in our country are distributed by a single system of indexing, according to which each developer is given an index (brand) containing an alphabetic and a digital designation. The main letters of the index, located before the digits, indicate the type of machine. For example, the letter part of the index of single-bucket building excavators contains the letters EO, excavators trench rotary - ETR, chain - ETC, earth-transport vehicles - DZ, machines for preparatory work and development of frozen soils - DP, machines for compaction of soils and road surfaces - DU, crane boom self-propelled - CS, construction tower cranes - KB, equipment for diving piles - joint venture, drill and crane ma-shin - BM, machines for finishing works - CO, winches - TL, chain bucket loader - TM and loading shovel - THAT, lifts - TP, conveyors and feeders - TK, machines for cleaning and cleaning cities - KO, hand-held electric machines - IE, pneumatic - IP, vibrators - IW, etc. The digital part of the index means the technical characteristics of the machine. After the digital part, additional letters can be included in the index, which denote the orderly modernization of the machine, the type of its special execution, and so on.

In our country, all construction machines are manufactured in accordance with the state standards (GOSTs). Each state standard specifies the area of its distribution, the main parameters and dimensions, technical requirements, test methods, marking, packaging, transportation and storage of machines. The indicators and norms specified in the State Standards reflect the advanced level of technology achieved in our country and abroad.

In the technical documentation, each model of the machine is indicated by an index in which the coded form contains the full name of the machine with its main parameters. For example, in accordance with the indexing of cranes produced by the plants of the Minstroydormash, the KS-8362HL index means: a self-propelled self-propelled crane (KS) with a carrying capacity of 100 tons (eighth size group-8), a pneumatic wheel (3-code of the running gear) with a flexible rope suspension (6 - cipher flexible suspension of the boom equipment), the second model (2), in northern execution (HL). There are also other indexing systems, as, for example, given above for excavators of trenching routers (ETR).

4. Machine parameters. Nominal size and model.

The parameter of a machine is the quantitative, less often, qualitative characteristic of any significant attribute of it. There are main, basic and auxiliary parameters.

The main parameters are those that determine the technological capabilities of the machine to the greatest extent. For most machines, such parameters include: the mass of the machine, the power of the power plant or the total power of the main engines in the electric drive, productivity, etc.

The main parameters, including the main ones, include those that are necessary for the selection of machines under certain conditions of their operation. In addition to the above, these parameters include the characteristics of patency (specific pressure on the ground in working and transport modes, etc.), the maneuverability of the machine (turning radii) and other driving properties (speed of movement, maximum elevation angles etc.), effort on working bodies, dimensions of the working area, overall dimensions of the machine, etc.

All other parameters that characterize, for example, the conditions of maintenance, repair, and relocation belong to auxiliary ones.

Within the limits of each functional group, the machines are united according to the standard sizes, characterized by a single main parameter. Several models can correspond to one type of size, each of which unites machines that have identical parameters, constructive solutions and manufactured under a single working document. So, for example, the EPR250 rotary trencher models, characterized by the main parameter - the maximum depth of the 2.5 m trench, combine the ETR253 and ETR254 models, differing both in designation and design decisions. The first model - ETR253 - is designed to work in areas with seasonal freezing of soils. The second model - ETR254 - is capable of developing soils with freezing to the full depth of the trench, including permafrost.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Test questions:

1. Give the definition of a construction machine.

2. What is the machine parameter? List the categories of parameters and characterize their composition.

3. What is the size of the machine, what is the factor?

4. What is a machine model?

5. What is the machine index?

6. Which groups are divided into construction machines according to the working-process regime, the type of energy used, the ability to move around?

Lecture №2.

Topic:Transport vehicles. Description of construction cargo.Basic parameters, operational characteristics of transport vehicles.Purpose, design schemes, workflow and technological capabilities, basic parameters, performance characteristics of transporting machines.

Outline of a lecture:

1. Transport vehicles.

2. General characteristics of transportation of construction cargoes.

3. Basic parameters, operational characteristics of transport vehicles.

1. Transport vehicles.

One of the main stages of the technological process of modern industrial urban construction is the delivery to the place of production of construction materials, products, structures and equipment, carried out by transport vehicles - trucks and wheel tractors, trailed and semi-trailers specialized or general purpose vehicles. The choice of type of vehicles is determined by the nature and quantity of cargo being transported, the distance of transportation, the state of roads and the time allotted for their delivery. In addition to cargo transportation, cars, tractors and tractors are used as traction vehicles for trailer and semi-trailer construction machines, as well as a unified base of mounted construction machines - excavators, cranes, loaders, bulldozers, scrapers, drilling and piling equipment, etc. Separate units of vehicles, tractors and tractors are used in construction machines.

2. General characteristics of transportation of construction cargoes.

In construction, land, water and air transport are used to move cargoes. Over 90% of transportation to construction sites is carried out by land transport: by road, rail and pipeline. The choice of the type of vehicles is determined by the nature and quantity of cargo being transported, the range of transport and time; allocated to their delivery.

Automobile transport. This is the most mobile and mass mode of transport. With its help, construction loads are delivered without overload directly to construction projects. The share of road transport, tractors and wheeled tractors accounts for more than 82% of cargo transportation, building materials of long cargo, building structures, technological equipment and construction machinery. Expenses only for road transport make up 12...15% of the cost of construction and assembly works, reaching in some cases considerably higher values.

There are automobile transport of general purpose and specialized. General-purpose vehicles include trucks, trailers and semi-trailers with side-mounted, non-tilting open platforms, as well as truck tractors used or transported of all types of cargo except liquid, without packaging. A car or truck tractor in a vehicle with a trailer or semitrailer is called an auto-train. Specialized transport means are trucks, trailers and semi-trailers intended for the carriage of a certain type of cargo (pipes, trusses, panels, bulk containers in containers, etc.). The use of specialized transport means ensures high efficiency of transportation, preservation of quality of transported goods, introduction of advanced methods of organization and management of the transport process.

3. Basic parameters, operational characteristics of transport vehicles.

Such vehicles are adapted for the carriage of one or several homogeneous cargoes that differ in the specific conditions of their transportation and are equipped with various devices and devices that ensure the safety and quality of cargo delivered to construction sites and the integrated mechanization of loading and unloading operations. The use of specialized transport contributes to the improvement of the efficiency and quality of construction, allows reducing the cost of transportation, minimizing the loss of building materials and semi-finished products, and the damage to building products and structures, which are very significant when using general-purpose vehicles. At present, without the use of specialized transport, it is practically impossible to deliver many goods to construction sites. Most specialized vehicles are removable trailers and semi-trailers for trucks, pneumatic tractor tractors and tractors, which allows more efficient use of the base machine.

In the conditions of urban construction, automobile specialized transport is widely used. Modern specialized vehicles for construction are produced in accordance with the approved by Gosstroy "Type of specialized vehicles for construction" and are intended for the transport of soil, loose and blocky cargoes (dump trucks), liquid and semi-liquid (bitumen trucks, limestone, concrete and mortar); powder (cement carriers), small-piece and tare cargoes (container carriers), long-length cargoes (pipes, metal carriers, timber trucks), reinforced concrete structures (panel trucks, farm trucks, or slub trucks, balktrucks, block trucks), equipment and construction machinery (heavy-duty).

Dump trucks transport construction materials in metal bodies with a trough, trapezoidal and rectangular cross-sectional shape, forcibly tilted when unloading with the help of the lifting (tilting) mechanism, back to side (one or two) sides, to the sides and back. By appointment, special career and universal building dump trucks are distinguished. In the conditions of urban construction, universal dump trucks (figure 2.7) with a carrying capacity of 4...12 tons are used to transport soil, gravel, crushed stone, sand, asphalt, concrete mix, mortar, etc. Modern universal dump trucks are produced on the chassis of trucks onboard cars of general purpose (sometimes with a shortened base) and equipped with the same type of hydraulic systems that provide a fast rise and fall of the body, high reliability and safety.

The main components of such systems are an oil tank, a hydraulic pump driven by a power take-off of the vehicle, one or more (depending on the carrying capacity) of single-acting telescopic cylinders directly acting on the body, distributor or control valve, connecting pipelines and safety devices. Hydraulic cylinders of lifting mechanisms can have a horizontal, oblique and vertical arrangement and are mounted on the frame of the car under the front part of the body or on its front side (Figure 2.7, a). A divider or control cock directs the flow of hydraulic fluid from the pump to the hydraulic cylinder (or synchronous hydraulic cylinders) when the body is tilted, connects the cavities of the hydraulic cylinders to the drain tank when the body is lowered, limits the pressure in the system and ensures the body is fixed in certain positions (extreme or intermediate).

Fig. 2.1.Tipper trucks

The most common in the construction of dump trucks in a dump truck and a dump truck or a truck tractor and a semitrailer dump truck (Figure 2.7, b). The dump truck is unloaded to the sides, and the trailer-dumper is to the sides and back. Tipper trailers can have detachable (dual) bodies, the front one of which is unloaded into two (lateral), and the rear one to three (side and back) sides. Modern dump trucks and tipper trailers have unified bodies, chassis, lifting gears and are equipped with automatic opening and closing of the sides with control from the driver's cab.

For the transportation of expanded clay and other loose materials with a low density, specialized trailers and semi-trailers are used - claydite trucks with the carrying capacity of up to 12 tons, i.e. dump trucks with increased body capacity.

When transporting small-piece and tare cargoes (sanitary-technical and ventilation equipment, finishing, insulation and roofing materials, brick, window and door blocks, small in weight and sizes of prefabricated reinforced concrete structures, etc.) to the construction sites, containerization and Packing. For the delivery of containers and packages, on-board cars, general purpose trailers and semi-trailers and specialized vehicles - self-loading and container trucks - are used.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Testquestions:

1. What types of transport are used in construction?

2. What is the difference between trailers and semi-trailers?

3. Why use in the construction of trucks?

4. What is the general purpose truck?

5. Which vehicles are specialized?

Lecture №3.

Topic:Classification of hoisting machines. Maintenance zones of cranes of various types. Application area.Basic parameters and indexing systems.

Outlineofalecture:

1. Classification of hoisting machines.

2. Maintenance zones of cranes of various types. Application area.

3. Basic parameters and indexing systems.

1. Classificationofhoistingmachines.

In construction, hoisting machines are used for moving construction materials, installing structural structures, loading and unloading works in warehouses of building materials, installation and maintenance of process equipment in the process of its operation. By the nature of the working process, all lifting machines are machines of cyclic action.

According to the design and type of work performed, they are divided into jacks, winches, hoists, erection towers and cranes.

Jacks are portable lifting devices of small size and weight. They serve for lifting the load to a height of 200...500 mm, moving it horizontally and for aligning the structures during their installation. The jacks are used in construction on installation and repair work, in installations for trenchless laying of communications, in construction machines (remote supports of cranes, lifts), etc. By design jacks are divided into rack, screw and hydraulic, with manual and electric, hydraulic and pneumatic drive.

Winches are lifting devices designed to lift or move loads on construction, installation, repair and loading and unloading operations using a rope wound on a drum or pulled through a lever mechanism. They are subdivided:

by type of drive - on manual (with manual drive) and drive (with a mechanical drive);

by designation - for lifting (for lifting the load), traction (only for moving cargo along a horizontal or inclined surface);

by the number of drums - one, two-drum and without a drum (with a traction sheave) and lever.

The main parameter of the winches is the pulling force of the rope (kN).

Lifts are used for vertical movement of goods (freight lifts) and people (passenger lifts) located in cabins or on platforms. Lifts, which together with cargo can lift people, are called cargo-and-passenger.

The towers are a kind of lifts mounted on trucks.

Cranes are universal load-lifting machines. They are used for moving piece and loose loads along a spatial path of arbitrary configuration and of various lengths.

The main characteristic of a load-lifting machine is the load-carrying capacity, by which is meant the largest permissible mass of the lifted cargo, together with the mass of load-gripping devices. In addition, lifting machines are characterized by a service area, including the height of the lifting of the load, as well as the speeds of working movements.

2. Maintenance zones of cranes of various types. Application area.

The term "self-propelled cranes" combines a large group of boom cranes characterized by high transport maneuverability, independent power supply and a variety of working equipment. Maneuverability of the cranes is provided by running equipment (caterpillar or pneumatic), adapted for movement both on roads with a hard surface, and on the ground. Caterpillar trains are mainly equipped with large-capacity cranes used for large-scale assembly works with large-sized cargoes. A variety of crawler cranes are light-duty cranes on basic crawler tractors or tractor units on the basis of, inter alia, cranes, pipe layers, used in pipeline construction.

Rubber-tired moving equipment is more maneuverable than tracked, permits movement with great speed on roads with a hard coating and a moderate speed on unpaved roads, construction sites and prepared. Cranes with pneumatic wheel screwed equipment is manufactured on the basis of standard chassis truck (truck cranes) and specialized pneumatic wheel landing gear (normal or short-base) and of multi-chassis car type.

Most of the self-propelled cranes perform work in the main operating mode positively except for the pipe-laying cranes, the working mode of which includes their movement.

In order to increase the stability and conditions permissible loading pneumatic tire valves with pneumatic wheel screwed equipment mounted on a disposable at the corners I pivot outriggers as sliding beams, rotary or vertical folding arms, on the free ends of which are set based on the cell] of wooden beams screw jacks or, more often, hydraulic cylinders. In addition, for cranes with sprung chassis springs for the duration of the crane block special devices. When working on unstable soils, outriggers are sometimes used in crawler cranes.

When working with small weights (up to 50% of the load capacity of the crane) and the longitudinal position of the boom, the cranes on the pneumatic wheel can be moved and operated without outriggers. Independence of power supply for self-propelled cranes is provided by internal combustion engines (carburettorand diesel). All mechanisms of self-propelled gantry cranes are equipped with brakes. At the same time in the mechanisms of movement of automobile and pneumatic wheel cranes, as well as cranes on the special chassis of the automotive type, the controlled brakes of a normally open type are installed, together with the parking brakes. Sometimes normally open brakes are also installed in cranes with tower-boom equipment (see below). In all other cases, the brakes are normally closed, opened when the drive is turned on.

The self-propelled crane used a variety of arrows: the direct short (Figure 3.1 a.) - for cargo handling works and works with single-piece and bulk cargo, called main, which is determined by the nominal capacity of the crane on all departures; elongated inserts (Figure 3.1, b) for lifting loads to a large height; a jib equipped with auxiliary lifting hook (Figure 3.1 с.) - for reloading works with bulk general cargo, including containers, requires an increase in the size of the space under the boom. The hook is attached to the head of the boom articulately, and its second end is fastened with a flexible brace at the base of the boom.

For work on the installation work, the jibis made manageable by fixing the end of the rope pull on the winch drum. For the supply of cargo to the interior of the erected building, as well as for installation of very voluminous objects, tower-boom equipment consisting of a main boom mounted almost vertically with an inconsiderable inclination (3...50) and a horizontally mounted elongated jib (Figure 3.1, d). A self-propelled crane with tower-boom equipment for technological capabilities is similar to a tower crane. Due to the more complex device of the self-propelled crane, its operation, including the initial costs, is more expensive than the operation of the tower crane. Therefore, it is advisable to use a crane with tower-boom equipment in single-time works. For cranes of large carrying capacity (250 tons and more), use similar types of boom and tower-boom equipment, but with highly developed transverse dimensions (Figure 3.1, d).

Fig. 3.1. Arrow and tower-boom equipment of self-propelled cranes

The second type is made up of telescopic arrows, which are mainly equipped with hydraulic-driven cranes - automobile ones and special self-propelled chassis (see Figures 3.1, b and 3.2). Depending on the lifting capacity of the crane, telescopic arrows are two-three-, four- and five-section ones.

The self-propelled jib crane (Figure 3.2) consists of the chassis 1, the supporting-rotary device 2, the turntable 3 with crane equipment located on it, the boom 4 or the tower-boom working equipment, power plant, drive mechanisms and control system.

The main parameters of self-propelled gantry cranes are: crane mass, load torque, maximum lifting capacity, hook projection L, A1 and A2, respectively, relative to the axis of rotation of the turntable and the tilting rib with and without outriggers (the tipping edge is the axis, loss of stability), the maximum height of the hook H, the depth of its lowering h, the track of the trolley K, its base B, the specific ground pressure (for crawler cranes) or the load on the chassis axis (for cranes on the wheel), the speed of lifting, lowering, landing, tearing off and horizontal movement of the load, rotational speed, working and transport speeds, power plant capacity, crane performance, etc. In technical documentation and business correspondence, self-propelled jib cranes are assigned an index of the type KS-0000. For a particular model of the crane, the first zero of the digital part of the index is replaced by numbers from 1 to 9 indicating the size group (load-carrying capacity 4, 6.3, 10, 16, 25, 40, 63, 100 and more than 100 tons). The second zero is replaced by the digits denoting the type of the traveling device (1 - caterpillar with a minimal supporting surface, 2 - the same with an increased supporting surface, 3 - pneumatic, 4 - on a special chassis, 5 - automobile, 6 - tractor, 7 - trailed , 8 and 9 - reserve for other running devices). The third zero is replaced by numbers from 6 to 9, indicating the execution of boom equipment (6 - with flexible, rope, suspension, 7 - with rigid, hydraulic, suspension, 8 - telescopic, 9 - reserve). The last zero is replaced by a digit denoting the model serial number. In addition, as for tower cranes, the letters of the Russian alphabet designate the next modernization, as well as special climatic performance. An example of a designation is given in p. 2.2.

Fig. 3.2. Schematic diagram of the self-propelled gibbet crane

Crawler cranes (Figure 3.3) operate without outriggers and can move within the construction site without preliminary preparation of the track at a speed of 0.5...1 km/h, and with special preparationwith a nominal load on the hook. The carrying capacity of domestic crawler cranes is 16...250 t. High maneuverability and high payloads have caused their wide application in various construction sectors at sites with large, including dispersed, scopes for mounting large structures and technological equipment. These qualities predetermine the high competitive ability of crawler cranes in relation to special tower cranes, which require the installation of crane tracks.

In addition to cranes on its own caterpillar base, crawler cranes constructed on the basis of universal one-bucket excavators with parameters within the technical capabilities of basic machines as well as made of excavator assembly units are also used in construction. The carrying capacity of the latter is 20...63 tons.

Crawler cranes are equipped with all the above listed types of boom and tower-boom working equipment. The length of the main straight arrows is usually 10...15 m. The increase in the hook lifting height is achieved by installing up to five additional plug-in sections with a length of 5...10 m, as well as jugs of different lengths.

Fig. 3.3. Crawler Crane

Crawler cranes have, as a rule, an individual electric drive with a primary power unit - a diesel engine and an alternating three-phase current generator with a frequency of 50 Hz, 380 and 220 V, which allows operation from an external power network. The diesel generator is installed in the tail part of the turntable. The drives of all mechanisms - cargo, boom lift, swivel, chassis and others - are built according to standard schemes: electric motor - brake - reducer - working element. On light-duty cranes, mainly made on the basis of single-bucket excavators or excavator units, there is also a diesel drive with a mechanical or hydraulic transmission.

The running gear of the crawler cranes consists of a non-rotating frame, supported by two drive crawlers with multi-bearing crawler links, providing low (up to 0.1 MPa) ground pressure. Each caterpillar is driven by its own mechanism. When driving on turns, one caterpillar is braked or the caterpillar motors are turned on to move in different directions. To increase the stability of a number of models of crawler cranes, crawler tracks are disposed across the caterpillars.

Stability and associated load capacity of crawler cranes among other parameters depends on the size of the supporting contour - base and track. Cranes with a track gauge partially (only with the arrangement of working equipment across the tracks) solve the problem of more fully realizing the possibilities of using the lifting capacity. However, this measure is constructively complex, more material-intensive and expensive in comparison with the traditional design schemes of crawler cranes. In addition, the widened track reduces the maneuverability of the machine, so that it is advisable to use taps with a sliding track in a restricted area of working areas with a relatively small fraction of the movement of the entire crane during its operating cycle.

As a variant of the use of self-propelled crawler cranes, increasing their carrying capacity, is the replacement of caterpillar track equipment with a broadened railway track. This measure is effective only for cranes of large capacity, working with large departures of cargo. Rail-wheeled cranes are widely used in the installation of roofing of industrial shops with preliminary large-scale ground assembly of large areas, as well as in hydrotechnical and power engineering.

Pneumatic-wheeled cranes have the same purpose with caterpillar cranes and a similar device to the turning part, but differ in pneumatic wheeled equipment. They come with a normal base or short-base. The latter have increased maneuverability, which is especially important for work in cramped conditions, including inside production facilities. At present, pneumatic wheel cranes of types KS and MKP with lifting capacity of 16, 25, 36 and 100 tons are produced and in operation in our country. Pneumatic wheel crane of 25 t capacity is shown in Fig. 3.4.

Pneimosholesnoe running equipment can be biaxial and multiaxial (up to five axes). Short-base cranes have two axles with all rotary wheels, which significantly increases their maneuverability. The working speed does not exceed 5 km / h, and the transport speed reaches 35 km / h or more (up to 70 km/h).

Crane mechanisms and the mechanism of movement, except for short-base cranes, until recently had an individual electric drive mainly direct current, powered by a power plant consisting of a diesel engine and an electric generator. When working in a restricted area of a construction site, a three-phase electric motor is used instead of a diesel powered from an external power supply. In the drive cranes of heavy duty, usually use two generators of direct current - the main and auxiliary. The first is for driving the mechanisms of lifting and moving, and the second is for driving the rotation mechanism of the turning part and for control circuits. In the power unit of pneumatic wheel cranes, there is a hydraulic system (for controlling the rotation of wheels and outriggers) and pneumatic (for pumping pneumatic tire chambers and brake control when towing a crane by a towing vehicle). Recently in these cranes the hydraulic drive is increasingly used.

Fig. 3.4. Pneumatic wheel crane of 25 t capacity: a - general view; b - transport position; c - rotation scheme.

All lifting operations of pneumatic wheel cranes are usually performed when installed on remote hydraulic supports. Work without outriggers is allowed only with small loads, which is determined by the low maneuverability of the cranes, which is the reason for their slow development.

Promising for cramped work conditions are pneumatic wheel cranes on a short-chassis chassis with hydraulic drive of all mechanisms and all rotary wheels (Figure 3.5), providing a small turning radius of the crane. The wheels are equipped with multi-layer tires, which allow for increased loads. The boom of such cranes is carried out by telescopic telescoping, usually consisting of three sections.

Truck cranes (Figure 3.6) are called self-propelled gantry cranes on the basis of two or three-axle series-produced or reinforced chassis of trucks. In construction, they are used in carrying out loading and unloading operations and installing structures and equipment of small masses and sizes. Recently, truck cranes are widely used for carrying out lifting operations in the construction of small buildings. Equipped with a two-channel grapple, the car cranes are used for the transshipment of loose materials.

The carrying capacity of a truck crane is determined by the parameters of the base car. At present, the domestic industry produces automobile cranes with a load capacity of 4; 6.3; 10; 16; 25 and 32 t. The crane's passport carrying capacity is provided only when working on four outriggers. When working without outriggers (but with stabilizers on, blocking the spring suspension of the rear axle), the permissible carrying capacity is drastically reduced. The crane can be moved at a low speed (up to 5 km/h) with a suspended spring suspension and a load on the hook with a mass not exceeding 25...30% of the passport loads raised not more than 0.5 m by an arrow located behind the car and oriented strictly to the its longitudinal axis. Transport speed of road cranes along the roads reaches 60...70 km/h.

Fig. 3.6. Truck cranes:

a - with a lattice boom and a group drive: 1 - the chassis of the truck; 2 - power take-off box; 3 and 6 - outriggers; 4 - additional frame; 5 - the support-rotary device; 7 - rotary platform; 8 - against-arms; 9 - two-legged stance; 10- cargo and jib winches; 11 - reversive-distributing mechanism; 12 - rotation mechanism; 13 - arrow; 14 - hook suspension; b - with telescopic boom and hydraulic individual drive: 1 - boom; 2 - hydraulic cylinders for expanding boom sections; 3 - boom lifting cylinder; 4 - rotation mechanism; 5 - a load-lifting mechanism; 6 - the pump; 7 - power take-off box; 8 - a transmission; 9 - car engine

- кабиной и крановыми механизмами соединена с его неповоротной часть

By type of boom equipment distinguish cranes with rigid (non-slip) and telescopic (sliding) arrows. Rigid booms, usually latticed, are supported in the required position by the ropes of the lifting mechanism. To increase the working space under the boom and its main parameters (lifting and lifting height), the crane can be equipped with an additional elongated boom or an insert for the extension of the main boom, an extension - in single file and tower-boom equipment. The angle of inclination to the horizon of the telescopic boom of the box section, as well as its slideway, is provided by the hydraulic cylinders. In modern cranes, the telescopic boom can be moved apart with the load on the hook, thus ensuring a high accuracy of installation of the mounted elements. Recently, hydraulic cranes on the car chassis with telescopic arrows began to provide additional equipment - mounting cradles, installed instead of jibs on the upper boom section, which significantly expands the field of their application.

The crane mechanisms are driven by the diesel engine of the base vehicle through the power take-off 2 (Figure 11.16, a) (in the case of a group drive) or individually (with an individual drive) using an electric or hydraulic transmission. Kinematic scheme of the crane with a mechanical transmission allows you to combine the lifting of the load with the turn of the platform. Hydraulic and electric drives greatly simplify the kinematic scheme of the crane, expand its technological capabilities and provide deeper regulation of the speeds of working movements and remote control. At present, hydraulic transmissions are the most widespread in these cranes. Crane mechanisms with hydraulic drive are very compact, also compact telescopic boom, which quickly translates into a transported shifted position.

Cranes on the special chassis of the automobile type (Figure 11.17) are intended for construction and installation works, for installation of technological equipment of industrial enterprises, as well as for loading and unloading operations. With high mobility and high load capacity, these cranes do not require installation when installed in the working position, provide a low landing speed of the load, as well as a large height of the hook. Cranes on special chassis have lifting capacities of 25, 40, 50, 63, 100 and 250 tons. They are a continuation of a number of crane load capacities. There are known foreign cranes on special chassis with a carrying capacity of up to 1000 tons. The special multi-axle chassis (3-8 axles) of these cranes differ from the car chassis by the ability to take heavy loads from crane equipment with increased load capacity. The transport speed of such cranes is from 45 to 60 km/h.

Cranes of the type in question consist of two parts - a non-rotating (actually a special chassis) and a swivel, interconnected by a conventional type rotary-type device. The wheel formulas of domestic cranes are 6x4, 8x4, 12x6, 14x6. Undercarriage I is equipped with pneumatic dual-circuit brakes. Each circuit brakes a part of the wheels, so I, even if one circuit fails, the braking reliability is provided by the second circuit.

Cranes on special chassis are equipped with telescopic arrows consisting of 3 to 5 sections, which are pushed apart by hydraulic cylinders. Cranes of large capacity are also equipped with lattice arrows and in addition - lattice extensions, jibs or arrows. In the latter case, the working equipment of the crane turns into a tower-trill. When transporting heavy-duty cranes, the arrows are transported separately from the crane on a special cart.

Fig. 3.7. Self-propelled crane on special chassis of automotive type

The crane mechanisms also have a high-pressure hydraulic drive (up to 32 MPa). The mechanisms of the winches consist of one or two hydraulic motors, planetary gears built into drums, shoe or disc brakes. Diesels are used as primary engines. At the same time, two diesel engines are installed on the cranes of heavy lifting capacity: one on the chassis for the movement and drive of the pumps supplying the hydraulic cylinders of the outriggers; the other, on the turntable, to drive the hydraulic motors of the crane mechanisms and the boom lifting cylinders and the extension of its sections. In the crane drive, double-flow pumps are often used to ensure the combination of working movements, as well as a wide range of their speeds by combining the flow rates of liquids of two pressure lines. The power plant also includes an electric generator and a 24 V battery for starting the main engine, lighting and powering the control and safety equipment and other equipment. Control the crane from the cockpit located on the turntable. Remote control is also possible.

Cranes on special chassis operate on remote hydraulic supports, while the horizontal plane of the platform is controlled by the I automation system.

Also known are cranes of large carrying capacity (250 tons) on a non-drive chassis that are moved within a construction site at a low speed behind a crawler tractor, and when relocating on highways with a wheeled tractor.

Tractor cranes on the basis of caterpillar or pneumatic-wheeled industrial tractors are used for construction, assembly and loading and unloading operations in off-road conditions. In this case, the basic tractor is usually converted, extending its chassis, replacing the spring suspension of the frame rigid and shifting forward the power unit. On the vacant seat at the rear, a swivel part is installed, connecting it to the non-rotating part with a conventional type rotary-type device. As a power plant, a tractor engine is used, which drives the crane mechanisms through a mechanical, hydraulic or electric transmission.

Crawler tractor cranes are usually equipped with latticed or box-shaped straight and L-shaped arrows, and pneumatic-wheeled cranes are more often telescopic arrows. Cargo-lifting of domestic tractor cranes produced by domestic industry does not exceed 10 tons.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Testquestions:

1. What are the construction lifts for, what are their main types, how they are arranged and what is the principle of their operation?

2. What are the towers designed for, how are they arranged?

3. Why use cranes in construction?

4. What is the classification of tower cranes?

Lecture №4.

Topic:Loading and unloading machines Types of forklifts. The working process.

Outlineofalecture:

1. Loading and unloading machines.

2. Types of loaders. The working process.

1. Loading and unloading machines.

Construction loaders are self-propelled universal machines designed to perform loading and unloading operations with various types of cargo (bulk, lump, piece, bundled, long-length, etc.), moving goods for relatively short distances, earth-loading, assembly and auxiliary works. The versatility of loaders is ensured by the availability of a wide range of quick-release replaceable working bodies - buckets of various types and capacities, forklifts, jaws and assembly jaws, crane blockless booms, mounted rippers, drills, etc.

2. Types of loaders. The working process.

There are periodic loaders - one-bucket and fork and continuous action - multi-bucket. Single-bucket and forklift trucks perform cyclically repeated operations for loading the working organ, transporting and unloading the load separately and sequentially. For multi-bucket loaders the filling and unloading of the working element is carried out continuously and simultaneously.

One-bucket loaders are used mainly for loading, unloading, moving and storing bulk, small pieces and piece cargo, as well as for excavating and loading dump trucks (or backfill) in unearthed soils of I and II category and natural soil of the III category. The main working body of such loaders is a bucket.

Single-bucket loaders are classified:

by type of running gear - caterpillar (on the basis of tractors), pneumowires (based on special chassis and tractors) and semi-tracked;

on the arrangement of the working element with respect to the engine - with the front (most common) and rear position;

according to the method of unloading the working body - with a semi-turning, combined, reversible and front loading equipment.

In urban construction, the most common are front-end loaders on the pneumatic.

Front loaders are based on crawler, wheeled chassis and tractors and ensure unloading of the bucket forward (from the material development side) at any elevation within the specified height. Wheeled loader equipment usually has all four drive wheels, and their support frame can be rigid and articulated. Loaders with articulated frame have high mobility, maneuverability and are most effectively used in cramped conditions.

The loader (Figure 4.1) is based on a self-propelled pneumatic wheeled biaxial chassis with articulated frame 5, consisting of two half-frames, the angle of rotation in the plan of which can be ± 40°. On the front half frame mounted loading equipment and fixed front axle. On the rear semi-installed: power plant 2, hydromechanical transmission, rear axle on the balance frame and operator's cab 7. The rear axle can swing relative to the longitudinal axis of the loader, which provides high traction and coupling qualities of the machine. The forklift equipment includes: a bucket 8, a lever system consisting of an arrow 7, a rocker arm 9 and rods 77, and a hydraulic drive system. The main bucket with a capacity of 1.0 m3 has a straight cutting edge with detachable teeth. The surfaces of the cutting edges and teeth are covered with a wear-resistant alloy. Instead of the main bucket, any of the seven types of changeable working bodies can be installed: buckets of reduced and increased capacity, a double-bucket bucket, forks, jaws, a crane non-block boom.

Hydromechanical transmission of the basic chassis of the truck includes: a hydrotransformer, a hydromechanical gearbox, a power takeoff 3, propeller shafts, front and rear unified driving bridges. 4. The power take-off gear provides the transmission of torque from the engine to the gearbox and the independent drive of the hydraulic pumps of the loading equipment and hydraulic steering. The steering of the loader with follow-up hydraulic feedback includes a hydraulic steering wheel and two auxiliary hydraulic cylinders, which rotate the poles relative to each other. The hydraulic system of the loading equipment provides the control of the boom and bucket when performing work operations and includes: two gear pumps, a distributor, a hydraulic cylinder 10 of the bucket rotation, two hydraulic cylinders 6 lifting and lowering the boom. The loader is controlled from the operator's cab, in which the control panel with control devices, the steering column and the pedals are concentrated.

Fig. 4.1.Frontloader

All modern loaders are equipped with emergency light and sound alarms with electronic information display devices (UIS) on the limiting state of the monitored parameters of the engine, transmission, electric, hydraulic, braking and other systems.

Replacement work tools and attachments for single-bucket construction pneumatic wheel loaders are shown in Fig. 4.2.

Fig. 4.2. Interchangeable working and attached equipment of one-bucket loaders: 1 - bucket for rock with teeth; 2 - bucket without teeth with a straight cutting edge: 5 - the same, with a V-shaped cutting edge; 4 - skeleton bucket; .5 - load forks: 6 - correct boule blade: 7 - plow snowplow; 8 - grip for poles and piles, 9 - bucket with forced unloading; 10 - double-jaw bucket: 11 - gripper for length gauges; 12 - bucket for the distribution of concrete: AND - grip for packages; 14 - the crane; 15 - excavator: 16 - ripper; 17 - rotary snow thrower; 18 - brush cutter; 19 - rootkeeper-collector; 20 - asphalt breaker

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Test questions:

1. What are the loading and unloading machines for?

2. List the types of replaceable working organs?

3. Describe the device and work process of the front forklift truck?

Lecture №5.

Topic:General information about machines for excavation. Classification of machines for excavation.The working organs and their interaction with the soil.

Outlineofalecture:

1. General information about machines for excavation.

2. Classification of machines for excavation.

3. The working organs and their interaction with the soil.

1. General information about machines for excavation.

Machines for excavation work in civil engineering are used for loosening of dense, rocky and frozen soils, planning of construction sites, preparation of bases for roads and driveways, development of foundation pits for foundations of buildings and structures, digging open trenches for laying city communications and building underground structures, digging pits and pits, sweeping the bottom and slopes of earthworks, backfilling of excavations and trenches after the construction of foundations and laying of communications, soil compaction and etc.

Machines carry out soil development in three main ways: mechanical, in which the soil is separated from the massif by passive and driven (active) cutting bodies - knives, teeth, scrapers, wedges, cutters, cutters, etc.;

hydromechanical, in which the soil is destroyed in an open pit by a water jet directed at a pressure of up to 6 MPa or by sucking a soil previously grounded (from a water jet or a cutter) from the bottom of a river or reservoir by a ground pump;

explosive, in which the destruction of soil (rock) occurs under the pressure of expanding combustion products (gases), explosives.

Sometimes combined methods of soil development are used, for example, explosive (preliminary loosening) in combination with mechanical (the subsequent development of an excavating machine with a knife or bucket working body).

At present, about 95% of excavation work in construction is carried out mechanically. When performing excavation work using a wide range of different for the purpose, design and operation principle of machines, which are divided into: machines for preparatory work; earth-moving transport; excavators; drilling; for trenchless laying of communications; for hydromechanical soil development; for compaction of soils.

2. Classification of machines for excavation.

Machines and equipment for the development of soils are classified by purpose - earthmoving, earth-moving, drilling and equipment for hydromechanization.

Earthmovers develop the ground either positively (single-bucket excavators and rotary excavators of transverse digging), or during the movement of the entire machine (continuous excavators, except for the above-mentioned rotary excavators). The product of their activity is the developed soil. It is placed in a dump near an open pit or a quarry or is immersed in vehicles for transporting it.

Earth-moving vehicles (bulldozers, scrapers, motor graders) operate in two successive modes - earthmoving and transport. First, the machine develops the soil in the course of its movement, accumulating it in front of the blade (bulldozer) or filling it with a scoop (scraper), and then moves it along the ground - in the case of a dump or in a bucket like a transport vehicle. Dumping earth-moving machines (bulldozers, motor graders).

Drilling machines are designed for drilling wells, including holes. Usually this is a positional action machine, which is determined by the location of the wells.

Hydromechanization equipment is designed for the development of soils with the use of a high-speed head of a water jet or a water flow. They are represented as machines, and apparatuses that do not have a machine drive.

3. The working organs and their interaction with the soil.

The physical and mechanical properties of the soil, the design, the geometric parameters and the working conditions of the working organ have a significant influence on the process of interaction of the working organ of the earth-moving machine with the soil. The physical-mechanical properties of the soil, the construction, the geometric parameters and modes of operation of the working body.

The working organs of excavating machines separating the soil from the massif by mechanical means can be made in the form of a tooth on the rack (Figure 4.1, a) for loosening the developed medium, a bucket of a certain capacity with a continuous cutting edge (Fig. 4.1, d, f, z ) or equipped with teeth (Figure 4.1, b, c, d, e), dump (Figure 4.1, s), equipped in the lower part with cutting knives. Working bodies in the form of buckets are called bucket, in the form of a blade with knives - blade or knife.

Fig. 5.1. The working bodies of earth-moving machines: a - the tooth of the ripper; o-w - excavating buckets for straight and reverse drakes; loader, planner: з - scraper bucket; and - bulldozer blade

The working process of earth-moving machines with bucket and knife working bodies consists of sequentially performed operations of separating the soil from the massif, its transportation (transportation) and backfilling. The working organs separate the soil from the array by cutting and digging. Cutting is the process of separating the soil from the massif with the cutting part of the working organ. Digging is a combination of processes that involve cutting the soil, moving the cut ground along the working element and in front of it in the form of a prism of drawing, and in some machines also moving the soil inside the working organ. The resistance of the soil to digging is 1.5...2.8 times greater than the resistance of the ground to cutting.

Physical and mechanical properties of soils are characterized by:

granulometric composition - percentage by weight of particles of different sizes;

density - the ratio of mass to unit volume (for most soils - 1.5...2 t / m3);

porosity - the ratio of the pore volume to the total volume of the soil (in%);

humidity - the amount of water contained in the pores of the soil (in%);

connectivity - the ability of the ground to resist separation into separate particles under the influence of external loads;

looseness - the property of the developed soil to increase in volume with a constant mass of its own, which is expressed by the coefficient of loosening Kv, equal to the ratio of the volume of soil in the loosened and natural states (KR = 1,1...1,4);

angle of natural slope - angle at the base of the cone, which is formed when the loosened soil is scattered from a certain height;

plasticity - the ability of the ground to deform under the action of external forces and retain the resulting shape after the load is removed;

compressibility - the property of soils to decrease in volume under the influence of an external load;

strength - the ability of the ground to resist destruction under the influence of external loads;

resistance to shear - cohesion of soil particles between each other;

coefficients of friction of soil on steel (0.55...0.65) and soil on the ground (0.3...0.5);

abrasiveness - the ability of the soil (rock) to wear out (abrade) the working parts of machines interacting with it;

stickiness - the ability of the soil to adhere to the surface of the working organs.

There are non-subsurface soils (sand, sandy loam, loam, clay, etc.), collapsible-rocky (cemented clays - argalite, gypsum, chalk, limestone, etc.) and rocky (dense limestones, dolomite, marble, sandstone, etc.). Soils that have a positive temperature are called frozen (thawed), negative - frozen, if they contain ice, and frosty (chilled) if there is no ice in their composition. The presence of ice in frozen soils significantly increases their strength and hinders the work of excavating machines. Non-frozen unfrozen soils are developed by conventional earthmoving tools, rock-dismountable and frozen soils with a small depth of freezing before the development are first loosened mechanically. Rock and frozen soils with a large depth of freezing are first loosened explosively.

In some cases, frozen soils are developed specially for this purpose earthmoving machines.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Testquestions:

1. What is cutting ground?

2. What operations are the working cycles of the earth-moving machine?

3. Give the main properties of soils. What indicators do they evaluate?

4. Give a general classification of machines and equipment for soil development?

Lecture №6.

Topic:Earth-moving machinery. Purpose, scope.Main technical and economic indicators.The working process.Prospects for the development of earth-moving machinery.

Outlineofalecture:

1. Earth-moving machinery. Purpose, scope.

2. Bulldozers.

3. Scrapers.

4. Motor graders.

1. Earth-moving machinery. Purpose, scope.

Earth-moving vehicles are machines with a knife-type working organ that simultaneously perform a layer-by-layer separation from the massif and move the soil to the place of packing while it is moving. To this group of machines are: bulldozers, scrapers, motor graders. The first two types of machines, especially bulldozers, are widely used in industrial and civil construction.

Each model of an earth-moving machine has an index including alphabetic and numeric designations. The two initial letters of the DZ index denote a group of machines, the subsequent figures are the serial number of the model registration, the letters after the digital part of the index are serial modernization (ABB, ...) and climatic (northern C and CL) machine execution. In the index of modernized self-propelled scrapers, in addition to the above letters, the letters M and P can be included. In the index of bulldozers and scrapers with an automated control system, the presence of the latter is denoted by 1, following the dash for the main index numbers, and for upgraded machines - after the letters denoting the modernization . In the index of motor graders after the above numbers and letters are included through the dash numbers 1, 2. 4, 6, indicating their modifications.

2. Bulldozers.

Bulldozers are attachments to a basic crawler or pneumatic wheeled tractor (a two-axle wheel tractor), including a blade with knives, a pushing device in the form of bars or frames, and a blade control system. Modern bulldozers are constructively similar machines, the basic tractors and attachments of which are widely unified. The main parameter of bulldozers is the traction class of the basic tractor (tractor). Bulldozers are used for layer-by-level development and movement of I...IV grade soils, as well as pre-loosened rock and frozen soils. With their help they carry out the planning of construction sites, the construction of embankments, the development of excavations and excavations, the cutting of terraces on slopes, the leveling of soil spilled by other machines, digging trenches for foundations and communications, filling ditches, pits, trenches, foundation pits and sinuses of building foundations, from snow, stones, bushes, stumps, small trees, construction debris, etc. The wide use of bulldozers in the construction industry is determined by the simplicity of their construction, reliability and economy in the field exploitation, high productivity, mobility and versatility. Bulldozers classify:

by purpose - general purpose, used to perform the main types of earth-moving and auxiliary works in various soil and climate conditions, and special ones used to perform targeted works in specific ground or technological conditions (bulldozer-pushers, underground and underwater bulldozers);

depending on the traction class (nominal tractive effort) of the base machines - small-sized (class up to 0.9). lungs (classes 1.4...4), medium (classes 6...15), heavy (classes 25...35) and superheavy (class more than 35);

by type of running gear - caterpillar and pneumowheel;

on the design of the working body - with a non-turning blade in the plane permanently located perpendicular to the longitudinal axis of the base machine, and with a turning blade that can be installed perpendicularly or at an angle of up to 53 in both directions to the longitudinal axis of the machine;

by the type of the blade control system - with hydraulic and mechanical (cable-block) control.

With a cable-block control system, the blade is lifted by a gear-friction winch through a cable chain block, lowering by the force of gravity of the blade. With the hydraulic control system, the lifting and lowering of the blade is forced by one or two double-acting hydraulic cylinders. Bulldozers with mechanical control are currently not manufactured by industry.

The working cycle of the bulldozer: when the machine moves forward, the blade is buried into the ground with the help of the control system, cuts the ground layer with knives and moves the resulting ground prism in front of it along the surface of the earth to the unloading point; after dumping the soil, the blade is lifted into the transport position, the machine returns to the site of the ground set, after which the cycle repeats. The maximum possible volume of the prism of drawing modern bulldozers is typed on a section of 6...10 m in length. Economically expedient range of soil displacement does not exceed 60...80 m for crawler bulldozers and 100...140 m for pneumatic wheeled vehicles. Crawler bulldozers, which have high traction and passability, have gained the most popularity. Thehigherthehaulingclassofthemachine, thegreatertheamountofexcavationitcanperformanddevelopstrongersoils.

The main parameters of the bulldozer equipment are the height without a peak (I) and the length B of the blade (m), the radius of curvature of the blade d, the main cutting angle 5, the blade angle a, the sharpening angle p, the blade angle e and the angle of rotation (at rotary machines) of the blade in terms of y (degrees), the height of the blade lift above the bearing surface h1 and the depth of the blade lowering below the bearing surface hi (m), the pressure head T and the vertical force P on the cutting edge (kN), the lifting speed vn and dropping v0 of the blade.

The bulldozer blade is a rigid welded metal structure with a curved profile front sheet. Along the lower edge of the blade are attached replaceable two-blade cutting knives (two side and middle), welded with a wear-resistant alloy. In the middle of the upper part of the blade there is a visor that prevents soil from spilling over the top edge.

To reduce ground losses during its transportation, modern non-rotating tracked bulldozers are equipped with spherical and semi-spherical dumps.

Fig. 6.1. Schemes of devices and basic parameters of bulldozers: a - with a turning blade; b - with a non-rotating blade; c - transverse skewing of the blade

The blade 1 of the non-turning bulldozer (Fig. 6.1, b) is hinged to the pushing device in the form of two pushing bars 4 of box section, the rear ends of which are hinged to the beams of the running gear of the base machine.

The blade 1 of the rotary bulldozer (see Fig. 6.1, a) is mounted on a universal pushing frame 5, on which various interchangeable equipment with hydraulic control can be installed instead of the blade: a brush cutter, a dredger, a picker, a snow plow, etc. The swivel blade is connected with the pushing frame by means of the central ball joint 7 and the two side pushers 6, which provide a different blade position in relation to the base machine. When the bulldozer moves longitudinally with the blade turned in plan, the ground moves sideways along the blade. The ability of rotary bulldozers to move the soil to the side determines their wide use in filling up canals, ditches, trenches of communications, etc.

Fig. 6.2. Replaceable working bodies of bulldozers

The control system provides: lifting and forced lowering of the blade, its floating and fixed position with the help of hydraulic cylinders 3, turning of the blade in the plan (in the rotary bulldozers) by hydraulic cylinders 6, transverse two-sided skewing (up to 12°) of the blade in the vertical plane (Fig. 6.1, c), adjusting the blade cutting angle (average value 55°) by turning (tilting) the blade with hydraulic cylinders 2 (see Fig. 6.1, a, b) forward and backward relative to the pushing device.

Forced penetration of blade knives into the ground under the action of hydraulic cylinders, which develop a force of up to 40% or more of the weight of the tractor, allows bulldozers with hydraulic control to develop strong soils, and the possibility of setting the blade to a fixed position allows cutting a layer of soil of a given thickness. The transverse skewing of the blade improves the versatility of the machine and its operational capabilities in planning works, facilitates the development of heavy soils, etc.

Crawler bulldozers can be equipped with additional quick-detachable equipment (Figure 6.2), which significantly extends their technological capabilities: fixed or hydraulically controlled blade broadeners 1, 2, 3, front and rear loosening teeth 4, pickax 5 for cracking asphalt coatings, knives 6 for the development of frozen soils , a brush cutter, a ditch extension 7 for digging ditches, a slope with a rigid anchorage and a hydraulically operated slide-planner 8, front and rear skis 9, forks 10, a lifting hook 11, etc.

Increasingly widespread are powerful non-rotating pneumatic wheel bulldozers based on serial wheeled tractors and special chassis with an articulated frame, used mainly for the development of light and medium soils in dispersed construction sites. Such bulldozers, almost not inferior to the passability of caterpillar machines, have considerably larger (1.5...2 times) working and transport speeds, increased maneuverability and productivity.

Almost all caterpillar and wheel dozers with pulling force of 200 kN and more are equipped with single-tooth and three-tooth ripping equipment of the rear arrangement.

Automatic control of the blade provides an improvement in the quality of the ground surface treatment, increasing the productivity of the machine by reducing the number of passes of the bulldozer along the planned site, and reducing the fatigue of the driver.

3. Scrapers.

The scraper is a self-propelled or trailed (to a caterpillar or wheeled tractor, a wheeled tractor) earth-moving transport machine, the operating body of which is a bucket on pneumatic wheels, equipped with knives in the lower part for cutting the soil layer. Scrapers are designed for layer-by-layer digging, transportation, layer-by-layer deposition, leveling and partial compaction of I. IV categories in engineering preparation of the territory for construction, planning of quarters, erection of embankments, development of wide trenches and excavations for various structures and artificial reservoirs, etc. Scrapers operate on unrefined middle soils (sandy loams, loams, chernozems) that do not contain large stony inclusions. When developing scrapers of heavy soils, they are preliminarily loosened by the thickness of the cut chips. The main parameter of the scrapers is the geometric capacity of the bucket (m3), which is the basis of the standard size of these machines.

Scrapers classify:

in the capacity of the ladle - a small car (up to 5 m3), medium (5...15 m3) and large (more than 15 m3) capacity;

by the method of loading the ladle - with passive loading by the driving force of the cut layer of soil, with forced loading by means of a scraper elevator;

by the method of unloading the bucket - with forced unloading when the bucket wall is extended forward (the main way), with a free (self-dumping) unloading by tipping the bucket forward along the machine;

by the method of aggregation with traction means - trailed to caterpillar tractors and two-axle wheeled tractors; Self-propelled, mounted with uniaxial and two-axle wheeled tractors;

by the way of working control - with cable-block (mechanical), hydraulic and electro-hydraulic control.

Currently produced scrapers have a hydraulic or electro-hydraulic control system for the working element, which provides for forced lowering, lifting and unloading of the bucket, changing the depth of cut, raising and lowering the front damper of the bucket with the help of double-acting hydraulic cylinders. Forced burial of the bucket knives into the ground allows you to accurately adjust the thickness of the cuttings, shorten the time of soil picking and effectively develop dense soils.

Fig. 4.10.Operations of the working cycle of a self-propelled scraper

The scraper's working process consists of the following successively performed operations: cutting the soil and filling the ladle, transporting the soil in the ladle to the place of packing, unloading and laying the soil, reverse (idle) running of the machine into the slaughter. When reclaiming soil (Figure 4.10, a), the knives of the bucket 2 lowered onto the ground cut off a layer of soil of thickness h that enters the ladle with the movable damper 3. The ladle filled with soil on the move rises to the transport position (Figure 4.10, b), and The damper 3 is lowered, preventing the soil from spilling out of the ladle. When the bucket is unloaded (Figure 4.10, c), the damper 3 is raised, and the soil is forced out of the lowered bucket of the bucket 5, which is pushed forward, and the adjustable gap between the cutting edge of the ladle and the ground determines the thickness of the laid soil layer 4, which is leveled ) with bucket knives and partially compacted with scraper wheels. At idle, the empty bucket is raised to the transport position, and the flap is lowered. To increase the traction force of the scraper when filling the bucket in dense soils, a bulldozer-pusher 1 is usually used (see Figure 4.10, a). When filling the scoop, the speed of the scrapers is 2...4 km/h, with transport movement - 0,5...0,8 maximum speed of the tractor or tractor.

In some scraper models, an inclined scraper conveyor (elevator) is installed to reduce the resistance when working in the bucket, forcing the soil cut by the knife to be cut into the ladle and unloading it. Scrapers with elevator loading are most rationally used in loose soils when performing small amounts of work.

Depending on the type and volume of excavation works, various schemes of scraper movements are applied in terms of ellipse, figure-eight, shuttle-cross, etc. The ellipse movement pattern is used in the development of excavations and wide trenches, shuttle-transverse and eight-in the digging of shallow, but large in size pits.

Trailed scrapers to caterpillar tractors, which are highly mobile, are able to work in poor road conditions. Low transport speeds of these machines (no more than 10...15 km/h) limit the economically expedient range of soil transportation 500...800 m. Self-propelled scrapers are characterized by higher mobility, maneuverability, transport speeds (up to 50 km/h) and productivity (in 1,5...2,5 times) in comparison with trailers of the same capacity.

The range of soil transportation by self-propelled scrapers is economically effective up to a distance of 5000 m.

In construction, self-propelled scrapers with buckets of a capacity of 4.5, 8.3, 15, 16 and 25 m3 are used.

On scrapers with buckets with a capacity of 16 and 25 m3, a second additional rear engine is installed to drive the rear wheels through the hydromechanical transmission, which allows all the wheels of the machine to be driven by the leading ones. The additional engine and hydromechanical transmission control is synchronized with the control of the tractor and is driven from the driver's cab. Simultaneous operation of both engines is used when filling the bucket and transporting the soil to the unloading site: when the bucket is unloaded and the reverse run, one tractor engine.

4. Motor graders.

Motor graders are self-propelled planing and shaping machines, the main working body of which is a full-turn grader blade with knives, installed at an angle to the longitudinal axis of the motor grader and located between the front and rear axles of pneumatic wheeled equipment. When the motor grader moves, the blades cut the soil and the blade shifts it to the side.

Motor graders are used for planning and profiling operations in the construction of roads, the construction of low embankments and profile excavations, the separation of the road trough and the distribution of stone materials in it, stripping the bottom of pits, laying out areas, filling trenches, ditches, ditches and pits, as well as cleaning roads, construction sites, city highways and squares from snow in winter.

Motor graders are used on soils of III category III. The work of the motor grader consists of successive passes, during which the cutting of soil, its movement, leveling and planning of the surface of the structure is carried out.

Modern motor graders have the same design and are made in the form of self-propelled triaxial machines with a full-turn grader blade, with a mechanical and hydromechanical transmission and a hydraulic control system for working bodies.

Motor graders are classified according to their design weight, type of transmission, wheel arrangement and type of onboard gears. According to the constructive mass, motor graders are divided into light (up to 12 tons), medium (up to 15 tons) and heavy (more than 15 tons). The wheel diagram of motor graders is defined by the formula AxBxB, where A is the number of axles with steered wheels; B - the same with the driving wheels and B - the total number of axles. Wheel diagram of domestic light and medium motor graders 1x2x3, heavy type 1x3x3.

By type of transmission distinguish motor graders with mechanical and hydromechanical transmissions. Hydromechanical transmission provides an automatic and smooth change in the speed of the motor grader, mechanical - stepped. On-board transmissions are of two types - in the form of onboard reducers (for light and medium motor graders) and separate leading bridges (for heavy motor graders). Each motor grader consists of a frame, transmission, running gear, main and additional working equipment, mechanisms with a control system and driver's cab. Motor graders frames can be rigid and articulated. The presence of articulated frame provides increased maneuverability of the machine.

The main working body of motor graders is a full-turn grader blade equipped with replaceable two-blade knives. In addition to the main working body, motor graders can be equipped with additional replaceable working elements - a bulldozer dump for soil leveling, backfilling of trenches, distribution of building materials, extension of the grader dump for increasing the width of the grab, sloping piles (dredges on the dump) for planning embankment slopes, scarifier for cracking road surfaces and loosening of dense soils. Bulldozer dumps are hung from the front of the machine, pickers - both in front and behind the machine, as well as directly on the grader dump. Management of bulldozer blade and scraper is carried out by double-acting hydraulic cylinders.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Testquestions:

1. What is the purpose of earth-moving machinery? What working bodies are they equipped with?

2. What are bulldozers for? What kinds of work can they do?

3. What are the scrapers for? What operations are their work cycles?

4. What are motor graders intended for? What kinds of work can they do?

Lecture №7.

Topic:Excavators. Classification, scope.The main types of working equipment, parameters and indexation of construction excavators.

Outlineofalecture:

1. Excavators.

2. Single-bucket construction excavators.

3. Classification, scope.

4. Excavators with hydraulic drive.

5. The main types of working equipment, parameters and indexation of construction excavators.

1. Excavators.

Excavators are self-propelled excavators designed for digging and moving soil. There are single bucket excavators of periodic (cyclic) action with the main working body in the form of a bucket of a certain capacity and continuous excavators with multi-bucket, scraper and milling (beskoshovymi) working bodies. Single bucket excavators perform the work of individual repetitive cycles, during which digging and moving operations are performed separately and sequentially. In the process of working the machine periodically moves for short distances for digging of the next volumes of soil. Excavators of continuous action digging and moving of a ground carry out simultaneously and continuously. The productivity of such excavators is higher than odnokovshovyh, spending about% of the working time for moving soil and working equipment.

By designation, single-bucket excavators are divided into construction universal for earthwork and loading and unloading works in construction, career development of quarries of building materials, ore and coal deposits and stripping for the development of minerals by open method. Excavators of continuous operation, according to their design, are divided into longitudinal digging machines for digging long sections of a rectangular and trapezoidal profile - trenches for pipelines and communications for various purposes (trench excavators), channels and conduits (channel diggers), transverse digging for quarrying, planning and reclamation, radial digging for stripping and quarrying large volumes.

In urban construction, one-bucket construction and trench excavators are predominantly used.

2. Single-bucket construction excavators.

Builders are called single-bucket universal excavators with basic buckets with a capacity of 0.25.2.2 m3, equipped with various types of interchangeable working equipment. Construction excavators are designed for excavation in soils of I... IV categories. With the help of standardized replacement work equipment (up to 40 types) they can also perform loading, unloading, assembly, piling, planning, cleaning and other work.

The main parts of construction excavators are caterpillar or pneumatic wheeled chassis, rotary platform (with power plant, mechanisms, control system and driver's cab placed on it) and interchangeable working equipment. The swivel platform rests on the running device through a unified roller support and turntable and can be rotated relative to it in a horizontal plane.

The working cycle of a single bucket excavator in the development of soils consists of the following sequentially performed operations: digging of soil (filling the ladle with soil), lifting the bucket with soil from the face, turning the bucket to the place of unloading, unloading the soil from the ladle to the dump or vehicles, turning the empty bucket to slaughtering and lowering it to the starting position for the next digging operation. During the operation, individual operations of the cycle can be combined (for example, lifting or lowering the bucket with turning it into the face), which allows cutting the pr duration of the cycle.

3. Classification, scope.

Classification. Single-bucket construction excavators are classified according to the following characteristics:

by type of running gear - on caterpillar tracks with normal and increased support surface of tracks, pneumatic wheels, on a special chassis of the automobile type, on the chassis of a lorry or a tractor;

by drive type - with single-motor (mechanical and hydromechanical) and multi-motor (hydraulic and electric) drive;

for the execution of the support-rotary device - for full-turn (the angle of rotation of the working equipment is not limited in the plan) and incomplete rotation (the angle of rotation of the working equipment in the plan is limited to 270°);

by the method of suspension of working equipment - with flexible suspension on cable pulley blocks and with rigid suspension by means of hydraulic cylinders;

by the type of execution of the working equipment - with articulated and telescopic working equipment.

In addition to these features, construction excavators differ in size, weight, capacity and capacity of buckets.

The main parameters of single bucket excavators are bucket capacity, working cycle length, digging and unloading radii, digging height and depth, load height, excavator's slope, design and operating weight of the machine, average ground pressure of tracked vehicles and load per single running wheel at pneumatic wheels, track and base of the running gear.

4. Excavatorswithhydraulicdrive.

Single bucket hydraulic excavators are multi-homogenous full- and part-turn machines with rigid suspension of working equipment, in which a hydraulic three-wheel drive is used to transfer power from the engine to the operating mechanisms. Compared with mechanical hydraulics, hydraulic excavators have a wider range of replaceable working elements, the number of which is constantly increasing, a greater number of basic and auxiliary motions of the working equipment, which significantly expands their technological capabilities and provides a high level of mechanization of excavation, especially in crowded urban conditions.

Distinguished hydraulic excavators with articulated-lever (Figure 4.17, a, b) and telescopic (Figure 4.17, c) working equipment, for the retention and actuation of which use hard links - hydraulic cylinders. The main working movements of the articulated equipment are the angle of the boom angle, the turn of the handle with the bucket relative to the boom and the bucket rotation relative to the handle, the telescopic movement is the extension and retraction of the telescopic boom.

Fig. 4.17. Single-bucket hydraulic full-turn excavators with rigid suspension of working equipment:

a, b - the articulated lever; in - the telescopic; 1 - supporting and rotating device; 2 - pneumatic wheel chassis; 3 - outrigger support; 4 - rotary platform; 5 - power plant; 6, 8, 9 - boom hydraulic arrows; 7 - arrow; 10 - the handle. 11 - backhoe bucket; 12 - bulldozer blade; 13 - driver's cab; 14 - caterpillar running gear; 15 - bucket of straight shovel; 16 - telescopic boom

5. The main types of working equipment, parameters and indexation of construction excavators.

Excavators with flexible suspension of working equipment are full-turning machines (Figure 4.14) with single-motor and multi-motor (diesel-electric) drive. On the turntable platform of these machines, a two-legged support post is mounted, carrying an armored lifting block. The industry produces construction excavators with a single-motor drive 3...5 size groups, with a multi-motor drive - the 6th dimension group.

The main types of interchangeable working equipment of such excavators are forward and reverse shovels, dragline, grapple and crane. In addition to these types of excavators are also equipped with equipment for piling and doweling, layout and cleaning of sites and slopes, backfilling of trenches, stubbing stumps, loosening of frozen and rocky soils, cracking of road surfaces, destruction of old foundations of buildings and walls, etc.

Fig. 4.15. The main types of replacement working equipment for building excavators with a mechanical drive

The excavator with direct shovel working equipment (Figure 4.15, a) develops the soil in the bottom located above the parking level of the machine. The set of direct shovel equipment includes an arrow 6, a handle 4 with a saddle bearing, a ladle 3 with an opening bottom, a pressure mechanism 5 (for excavators of the 2nd and 3rd dimension groups there is no pressure mechanism), pulley blocks 1 and 2 boom and bucket lifts. Filling of the bucket occurs when lifting it with a pulley block 2 and pulling the handle towards the face with a pressure

mechanism that regulates the chip thickness. The bucket is unloaded by opening its bottom.

The excavator with backhoe equipment (Figure 4.15, b) is intended for digging trenches and small pits located below the level of its parking lot. The backhoe working equipment consists of a bucket 8, a handle 4, booms 6, a front pillar 7 and pulleys: traction 9, lifting 2 and boom 7 (to hold the front pillar). Filling of the bucket, cut into the ground under the weight of the working equipment, occurs when pulling it to the excavator with a traction pulley 9 and simultaneously loosening the tension of the hoisting pulley 2. The unloading of the soil from the bucket is performed by turning the handle from the face when the traction pulley is loosened and the lifting equipment is lifted by the lifting pulley.

The excavator with dragline equipment (Figure 4.15, c) develops the soil below the level of its parking and is used for digging foundation pits, reservoirs and trenches, as well as for the development of various excavations under water. Filling of the bucket, pressed to the slaughter by its own weight, occurs when pulling it to the excavator with a pulling cable 14. Unloading of the bucket lifted to the required height is carried out by turning it with the tension of the traction and tipping ropes weakened.

Excavator with screening equipment - excavator-crane (Figure 4.15, d) is used for various assembly and loading and unloading works. The crane equipment set includes an extended grating boom 10, boom lift 7 and cargo 2 pulleys, hook suspension or special devices for picking up loads.

Excavator with grab equipment (Fig. 4.15, d) is used for loading and unloading loose and small-sized materials, cleaning trenches and pits from collapsed soil and snow, digging wells and narrow deep excavations in light soils, and digging trenches under water. Filling of the bucket occurs as a result of the closing of its jaws with the tension of the closing and loosening of the hoisting ropes. The bucket is unloaded when the closing rope is loosened.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Testquestions:

1. What machines are called single-bucket excavators? What operations are their work cycles?

2. What is the classification of single-bucket excavators?

3. What is the purpose of hydraulic excavators with working equipment backhoe?

4. What is the purpose of hydraulic excavators with working equipment straight shovel?

Lecture №8.

Topic:Machines and equipment for frozen soils. The use of machines with traditional organs for the development of frozen soils.Types of special machines.

Outlineofalecture:

1. Machines for the development of frozen and durable soils.

2. The use of machines with traditional organs for the development of frozen soils.

3. Types of special machines.

1. Machines for the development of frozen and durable soils.

Frozen soils in comparison with non-frozen (thawed) are characterized by much greater resistance to fracture (in 15...20 times) and abrasivity (100...150 times), labor and development costs. The productivity of earth-moving and earth-moving machinery in the development of frozen soils is sharply reduced.

In modern urban construction, the development of frozen soils is mainly carried out in two ways - explosive and mechanical. The explosive method of loosening of frozen soils is usually used for large volumes of work at open sites far from the structures with a freezing depth of more than 1 m. Recently, the explosive method has been used in narrow urban conditions with the use of explosion localizers that do not allow the scattering of pieces of soil and damage to structures.

Primary distribution (more than 80% of the total volume of work) has received a highly efficient and universal mechanical method for the development of frozen soils using special machines conventionally divided into two groups: machines for preparing (preliminary loosening, cutting into blocks) of frozen soils and subsequent final development by interacting with them in a complex earth-moving machines of general purpose; machines that independently carry out the whole complex of development to a given level and evacuation of frozen ground from the face. The first group includes hinged rippers (see figure 4.2) on tractors of class 10...50. Impact machines for loosening soil with shock impulses, impact machines for tearing ground from the massif, bar and disc-milling machines for cutting cracks in frozen soils; to the second - earthmoving-milling machines and trench chain and rotary excavators, working bodies and speed regimes of which are adapted for the development of frozen soils with freezing to the full depth of the trench.

Impact machines impact destructible environments (frozen ground, hard pavement, foundation, etc.) by shock impulses of freely falling or clogged working bodies. The most common type of freely falling working bodies are wedge-hammers of cone-shaped, pyramidal and wedge-shaped forms with a mass of 0.5...4 tons.

Wedge-hammer 3 (Figure 4.40, a) is suspended to the hoist rope 2 of the freight friction winch of a self-propelled crane or a one-bucket mechanical excavator with a crane boom 1 and, when working, is pulled by a winch to the boom head and dropped from a height of 6-8 m. A freely falling wedge -milot causes non-directional shocks, which leads to high energy costs for the destruction of the soil, reduces the quality of work and contributes to the dangerous intensive dispersion of pieces of soil in the sides. The hammer-hammer can be placed in rigid guides 5 (Figure 4.40, b) and, when dropped, falls into a precisely defined location, which allows to destroy the soil with the least energy-intensive method of large cleavage and reduce the danger of fragmentation. Wedge-hammer with a guide device is usually mounted on a caterpillar or pneumatic tractor, which is equipped with a lifting gear-friction winch driven by the tractor's power take-off box. The guide device is connected to the base machine by elastic cushioning elements 4, which reduces the effect of dynamic loads on the tractor during operation.

Equipment with a clogged working organ develops frozen soils of great strength with a depth of freezing of 1...1.5 m by the most effective method of large-scale fracture. Clogging of the working element in the ground can be carried out by: a freely falling load 6 (Figure 4.40, c) suspended on the hoist rope of the base machine and moving relative to the guide 5; diesel hammers, vibration hammers; hydraulic, pneumatic and hydropneumatic hammers, used as a replacement work equipment for single-bucket construction excavators. Hydro and pneumatic hammers are currently the most common and effective equipment for the destruction of frozen soils by impact loading.

Fig. 4.40. The working organs of impact machines:

a - with non-directional blows; b - the same.with directed; c - with a clogged wedge.

During the operation of impact machines, dynamic loads are created which adversely affect both the base machine and nearby structures and communications.

2. The use of machines with traditional organs for the development of frozen soils.

In the cramped conditions of the existing construction, when working near buildings and underground utilities, hydraulic excavators with ripping and grabbing equipment are widely used, which destroys the frozen soil by the unstressed method of detaching it from the massif.

For the destruction of large volumes of frozen ground (for example, when laying linear communications in the open way), high-performance earth-cutting and earth-moving milling machines are used.

The grabbing equipment is mounted on caterpillar hydraulic excavators of the 4th and 5th size groups and is intended for loosening of frozen soils, cracking of asphalt concrete pavements, dismantling of old buildings, removal and laying of road plates, pipes, installation of wells, loading of oversized materials, etc. This equipment, produced in two versions (with one- and three-tooth ripper-gripper), is installed instead of the bucket and the handle of the back shovel. The set of single-tooth ripper (Figure 4.41, a) includes: a two-sided canine-ripper 6 with replaceable front 7 and rear 8 teeth hinged to the two-arm lever 5, a backhoe bucket 4 and a pair of hydraulic cylinders 2 with a ripper lever with respect to the handle 1, interchangeable with hydraulic cylinders 3 bucket backhoe.

Soil development is carried out by moving the handle with a canine-ripper to the excavator or by turning the canine in both directions relative to the handle by hydraulic cylinders 2 operating from the hydraulic system of the machine. The swivel joint of the canine-ripper with the lever allows the loosening of soils with the most rational cutting angles. When the soil is broken by the front tooth, the canine ripper moves to the bucket 4, supported by the teeth, cutting through the gap in the ground. The resulting forces on the teeth of the ripper and bucket are directed towards each other, which significantly reduces the transfer of load to the base machine. The back tooth of the canine-ripper, moving from the bottom up to the excavator, is used both for loosening of frozen ground, and for cracking road surfaces and loading and unloading operations.

The three-tooth ripper consists of a welded frame and three exchangeable teeth - the central and two lateral ones. The lateral teeth can be installed in three positions to obtain a different amount of loosening effort, depending on the strength of the earth being broken. The teeth of one- and three-toothed rippers are hard-alloyed.

3. Types of special machines.

Earth-cutting machines are used for cutting slots up to 0.3 m wide in homogeneous, frozen and hard-to-digest unfrozen strong soils. They are bar, chain and disc-milling equipment, which is hung on the serial chain trench excavators (instead of the main working body), on caterpillar and pneumatic-powered tractors, equipped with hydromechanical speed reducers. mechanisms of drive of working bodies and hydraulic lifting mechanisms for control of attached equipment. Chain and disk milling devices can be mounted on the same basic chassis. The main parameter of the earth-cutting machines is the maximum depth of the cut slit.

Bar working tools - chain bars from coal cutters or combine harvesters in the form of an endless chain with cutters, running around a flat frame with drive and tension sprockets. Bar trench excavators are equipped with bar-work tools, cutting through slits 0.14 m wide. The bars cut vertical longitudinal slots in homogeneous frozen soils up to a depth of 2.0 m. One basic machine can be hung, individually hydraulically controlled one, two or three bars.

One-bar machines have a central and lateral (offset) arrangement of the working body for cutting slots along the sidewalks. Bars cut an array of frozen soil into separate blocks weighing 5...10 tons that are removed from the face by winches and cranes. Sometimes the ground cut by bars is preliminarily loosened by impact machines, and its further excavation producing excavators.

The most widely used chain earth-cutting machines, which use the same type of maximally unified hinged digging equipment, consisting of four modules: a chain working organ 4 (Figure 4.42). mechanisms of drive 2 and burying 3 and hydromechanical running gears 5 of the base tractor 1.

Fig. 4.42. One-barrel cutting machine

Chain-shaped snapping organs are caterpillar chains of tractors of class 10 with cutters; consist of a guide frame, a drive sprocket mounted on the output shaft of the drive mechanism, a tension guide roller and a tension screw device.

On the links of the cutting chain, replaceable tool holders with cutters from bars of coal cutters or combines are attached. Tool holders with cutters can be installed according to the schemes providing the number of cutting lines 10, 14 and 21, which allows cutting slits with a width of 0.15, respectively. 0.21 and 0.27 mm. To improve the transport capacity of the cutting tools when cutting frozen soils and increasing the productivity of the machine, scrapers are additionally attached to the tool holders when working in melt soils.

The main advantages of chain and bar land-cutting machines are the simplicity of construction and ease of use, small metal consumption and sufficiently high productivity (up to 70 m3/h), disadvantages - high power consumption (up to 60% of all consumed) for grinding the ground and overcoming friction in chains , low durability of the working member operating in an abrasive medium.

Disk slitting machines (disk-milling machines) are cut in frozen ground with slots 80...120 mm wide to a depth of 1...2 m using one or two rotor-equipped disks (rotors) up to 3 m in diameter. These machines are also used for digging narrow trenches of a rectangular profile for power transmission and communication cables, small diameter pipelines, as well as opening of asphalt road pavements. The disk working equipment includes trench excavators and crawler tractors equipped with speed reducers and bulldozer dumps. The drive of the working element can be mechanical and hydraulic.

The disk grooving machine (Figure 4.43) is designed for digging trenches and gaps with a width of 0.28 m and a depth of 1.3 m in frozen and dense soils. The mounted excavator working equipment includes a disk rotor with hydraulic drive, frame 8 with a scraper K) and a hydraulic mechanism for raising and lowering the rotor. The rotor consists of a disk 73, on which eighteen teeth 12 are installed with the help of tooth holders 11. They develop the soil and carry it to the surface. The rotor is mounted on the frame support 14 and is driven by a high-speed hydraulic motor 6 via a gear reducer 7. The gear output gear 16 engages with a gear ring 9 rigidly attached to the rotor disk.

Fig. 4.43. Disc Shearing Machine

The working body does not have special equipment for transporting the developed soil; The tooth, removed by the teeth on the surface, moves away from both sides of the trench with plows 15 of the frame 8 and is located by a roller along the detached trench. Lifting and lowering of the working element is carried out by a hydraulic lifting mechanism including two hydraulic cylinders 5, a frame 4 and telescopic rods 5. The working speeds of the excavator when digging trenches are provided by a hydromechanical chassis and infinitely adjustable in the range 10...480 m/h.

To obtain transport speeds of the machine (2,2...9,8 km/h) a tractor gearbox is used. The drive of the pumps of the hydraulic system of the excavator and the hydraulic motor is reduced by the transfer box 2.

The main advantages of the disco-milling machines in comparison with the bar and chain ones are the reduced energy consumption of the cutting process due to the small number of rubbing surfaces of the rotor, higher productivity and durability (2...3 times) of the rigid working element; The main disadvantages are high metal consumption and limited depth of digging, which is about 0.5 diameter of the rotor.

Earthmover-milling machines (FPM) are used for layer-by-layer development (milling) of frozen soils and hard rocks during planning work, a trough for intra-quarter roads, tramway and crane tracks, as well as destroying asphalt concrete surfaces with subsequent excavation of destroyed materials with bulldozer heap.

The main parameter of the ZFM is the width of the strip milled in a single pass. ZFM are based on serial tracked bulldozers of traction class 10...15, equipped with hydromechanical speed reducers to obtain reduced working speeds, infinitely variable in the range 0...500 m/h. The designs of modern PFM have very little difference.

The working member of the ZFM is a mill with a diameter of 900...1020 mm, which is a horizontal hollow shaft with brackets welded perpendicular to its axis, which are equipped with replaceable cutting tips (tusks) with wear-resistant carbide cladding. Brackets in the number from 21 to 26 are located on the shaft along one or two helical lines diverging from the middle of the shaft. This arrangement of the brackets provides a certain sequence of operation of each tool, the minimum power consumption of the milling process, the flatness of the planned surface, and also the transportation of part of the destroyed ground to the edges of the processed strip.

Fig. 4.44. Earthmover-milling machine

The machine consists of a basic tractor of class 10 with bulldozer equipment of the working organ of milling type, power transmission for the drive of the working element, the attachment, the hydraulic drive of lifting and lowering the working element, hydromechanical walking machine, control system and counterweight.

The drive of the milling cutter 10 is carried out from the power take-off 5, through the chain drives 7 and the bevel gears 8. The drive provides one or two cutting speeds in the range 0.7...1.4 m/s and is equipped with a torque limit safety clutch 6. The operating element is hung on the base tractor with the help of a four-link hinge mechanism formed by the common housing of the power take-off reducer and the speed reducer 4, the chain link rods, the lower frame 11, and the housing of the bevel gears rigidly connected to each other by a transverse beam 9. Translation of the working element in the transport and working position and holding it at a given depth of milling are carried out by two hydraulic cylinders 12 operating from the hydroside of the base tractor 3. To counterbalance the weight of the attachments, a counterweight 2 is installed in the front of the machine.

Modern PFMs in one pass process a strip of soil with a width of 2,6...3,4 m with a milling depth of up to 0,25...0,35 m. After each passage by a milling cutter, the destroyed soil (material) is removed by a bulldozer blade 1. The productivity of PFM in the development of frozen soil is 140...400 m3/h.

The main disadvantage of earthmover-milling machines is the intensive abrasive wear of cutting elements.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Test questions:

1. What machines are used to develop frozen soils?

2. What machines and equipment are used for preliminary destruction of frozen soils?

3. What are rippers for?

Lecture №9.

Topic:Drilling machines and equipment.

Drilling tool.Machines for drilling holes.Crane-crane machines.Technical and operational indicators of the main types of drilling machines.

Pile-driving equipment.Equipment for piling with shock and vibration methods, underwashing and screwing.

Outlineofalecture:

1. Self-propelled drilling crane machines.

2. Technical and operational indicators of the main types of drilling machines.

3. Types of special machines.

4. Equipment for piling works.

5. Pile hammers.

1. Self-propelled drilling crane machines.

Self-propelled drilling and crane machines are widely used in urban construction in the construction of pile foundations of buildings and structures, bridge supports, pipelines, electricity and communication lines, wells, fences, as well as roads, trees and shrubs. They are a joint drill and special crane equipment mounted on the chassis of serial vehicles and tractors, the drive of which is carried out from the engine of the basic machine or an independent power plant. Drilling equipment is a method of mechanical rotary drilling vertical and inclined wells in melted and seasonally freezing soils, and special crane - installed in drilled well piles, poles, reinforced concrete supports, blocks of wells and other elements.

Drilling crane machines are classified according to the following main features:

by type of basic machine - to automobile and tractor;

on the principle of the operation of drilling equipment - cyclic and continuous operation;

by the type of drill drive and the crane of the equipment - with a mechanical, hydraulic and mixed (hydromechanical) drive;

by the type of the boring-crane equipment - combined (drill and crane equipment mounted on one mast) and separate (drilling equipment mounted on a mast, crane - on an arrow);

as far as possible rotation of working equipment in the plan - non-revolving and rotary;

on the location of the working equipment on the base chassis - with the rear and lateral arrangement of non-rotating machines, on the turntable - at the pivoting.

2. Technical and operational indicators of the main types of drilling machines.

The main parameter of the crane-crane machines is the maximum depth of the drilled well (in m). The main parameters are: the diameter of drilling (well), the drilling angle (the angle of inclination of the well axis to the horizon), the carrying capacity of crane equipment.

As a replacement drill tool, boring crane machines use blade, ring and auger drills fixed at the end of the drill rod, which communicates the torque and feed force.

3. Types of special machines.

The bladed drill (Figure 4.53, a) consists of a body 1 with two digging blades in the form of a two-screw screw, a drift 6 and a damper 2. The blades are equipped with replaceable cutters 5, loosening the ground and installed in the tool holders 3. The drill collar located at the end of the drill head, Creates a drill direction and holds it along the drilling axis. Damper prevents soil spillage when excavating soil from the well. The drill is fastened to the lower end of the drill rod with the help of a finger. The auger (screw) drill (Figure 4.53, b) is a tubular skeleton 10 with helical ground transporting spirals in the form of a continuous strip I. The auger has a shank 12 for attachment to the end of the drill rod. To the auger is attached a replaceable drill head 9 with incisors 8 and a drill 7.

Fig. 4.53. Drills of crane-crane machines: a - lobed; a - auger; in - ring

The annular drill (Figure 4.53, c) destroys the soil along the periphery and forms an annular gap that separates the core from the array. The drill consists of a body 13 in the form of a pipe, on the lower end part of which the cams 15 with blades 16 are uniformly arranged. The surface of the drill body is provided with screw blades 14 transporting the destroyed ground (rock) from the annular gap to the day surface. Two deflecting bars 17 cast the destroyed ground to the outer wall of the annular gap.

When drilling wells in frozen soils, cutters and drifts are used, reinforced with carbide-tipped plates. Drilling of wells is carried out with the rotation of the drilling tool with its simultaneous downward motion. During drilling, a well of the required depth is formed in several repetitive cycles, each of which includes sequentially performed drilling operations, lifting the drilling tool onto the day surface, unloading it and returning to the face.

To drill wells of different diameters, each drill-and-crane machine is equipped with a set of replaceable drilling tools. The drilling crane machine (Figure 4.54) based on the car is designed for drilling in melted and seasonal freezing grounds I...IV categories of wells with a diameter of 0.36...0.8 m to a depth of up to 3 m.

The machine consists of a base car 1, a special frame fixed to the vehicle frame, drilling equipment, hydraulic drilling rig installation mechanism, outriggers with hydraulic jacks 8, mechanical transmission, hydrosystem and electrical equipment. The crane-crane equipment is hinged on the brackets of the special frame and can be rotated in the longitudinal-vertical plane of the machine by the hydraulic cylinder 2 when the equipment is installed in the transport and working position. In the transport position, the drilling equipment is placed on the support post.

Fig. 4.54. Non-rotating drill-crane machine

The drilling and crane equipment includes a drilling mast 3 with a head, a bar with a drilling tool in the form of a bladed drill 6 with a reamer 7 and cutters, a hydraulic mechanism for feeding the drilling tool to the bottom and retrieving it from the well, a rod rotator and a single-reel winch worm gear for installation of supports in the drilled well. Feeding and extraction of the rod with the drilling tool is carried out by a double-acting hydraulic cylinder mounted inside the drilling mast. The rod moves along the piston with the rod fixed in the upper part of the drilling mast. The rotator 5 is a hypoid conical gearbox driven from the power take-off 11 of the vehicle through the transfer box 10 controlled by the friction clutch cylinder and the propshaft 9. The drive of the reversible worm winch is carried out from the transfer case. On the drum winch zap-sovan rope cargo pulley with hook clip 4. The transfer box provides three rotational speeds of the drill (1.75, 2.43 and 3.03 s-1), depending on the strength of the developed soil, as well as the reverse of the drilling tool and drum winch. At work the machine rests on two outriggers supporting the rear axle of the base vehicle. Hydraulic cylinders for the installation of the mast and the supply of drilling tools, control of the friction clutch and outriggers are serviced by a gear pump driven by the transfer case. The control of the crane-crane equipment is carried out from the control panel located in the body at the operator's workplace.

The capacity of non-stop drilling crane machines is 3.6...4.5 pph, the maximum drilling depth is 2.0...3.0 m, the drilling angle is 62...105°, the drilling diameter is 0.36...0,6 m, lifting capacity of crane equipment 1,25 t, the maximum length of installed poles, piles, supports and other elements - 10...12 m.

The crane-crane machine with turnable working equipment is mounted on the chassis of the car and is designed for drilling holes with a diameter of 0.63 m to a depth of up to 15 m in melted and frozen soils. On the frame of the base machine 3 (Figure 4.55) there are mounted a pumping station 4, remote hydraulically controlled supports 13 and a support post 2 of the mast.

Fig. 4.55. Rotary Drilling Crane Machine

On the turntable 8 with the roller bearing device 14, the crane-crane equipment, winch 5 of the lifting mechanism, the hydraulic lifting and lowering mechanism 6 of the mast, the platform turn mechanism 7, the borehole center indicator 12 and the driver's cab 10 are located. Turning in plan, the working equipment ensures quick guidance of the equipment to the drilling point and the ability to drill several wells from one position of the machine, which significantly improves its productivity. The drilling equipment of the machine includes a mast 1, hinged on a rotating platform, on which the rotator 9 is mounted, a rod with a replaceable drilling tool - screw auger 11 and a hydraulic mechanism for feeding the drilling tool to the bottom and retrieving it from the well. Lifting of the mast to the vertical (working) and lowering it into the horizontal (transport) position relative to the pivot axis is performed by two hydraulic cylinders 7.

The telescopic rod 9 (Figure 4.56), at the lower end of which is attached a replaceable screw auger 10, is passed through the rotator and is pivotally connected to the swivel 5. It serves to direct the rod. The swivel is suspended on a rope coming off the drum 3 of the winch.

The rotator provides rotation of the rod from two hydraulic motors 11 through a two-speed single-stage gearbox 8.

Fig. 4.56. Kinematic diagram of the crane-crane equipment

The forced feeding of the drilling tool into the face is made by the hydraulic mechanism of clamping and feeding the rod, the main assembly of which is a cartridge 7 suspended from the rods of two hydraulic cylinders 6. During drilling, the cartridge clamps the rod, and the hydraulic cylinders feed it into the slaughter. The rates of feed and rotation of the drill are varied with the help of the hydraulic drive steplessly, depending on the physical and mechanical properties of the developed soil.

The lifting and lowering of the rod with the drilling tool during drilling and excavation is provided by a single-drum winch, the drive of the drum 3 being driven by a high-speed hydraulic motor 1 through a single-stage planetary gear 4. The winch is equipped with a band brake 2.

The turn of the platform with the crane-crane equipment is provided in the plan by a turning mechanism including a high-torque hydraulic motor, a belt brake and a single-stage gear reducer, on the output shaft of which a pinion gear is engaged, which engages with the gear ring of the turntable.

When drilling wells, the machine is supported by outriggers, each of which is provided with a support hydraulic jack and a hydraulic cylinder for turning the support.

The hydraulic motors of the winch, the rotator and the turning mechanism, the mast lifting and lowering cylinders, the drilling tool feeder, the outriggers and the rotary gear shift are serviced by three hydraulic pumps of the pump station, driven from the transfer case of the base machine through a propshaft and a single-stage gearbox. The drive of the pumping station is switched on from the car's cab, and the drilling process and the machine are controlled from the driver's cab.

4. Equipment for piling works.

When installing pile foundations of buildings and structures for different purposes, two types of piles are used: chisel (ready-made) reinforced concrete and metal factory readiness and bored reinforced concrete piles, which are installed in vertical and steeply boreholes directly at the site of work. When erecting waterproof fences for pits, wells and trenches, use metal and reinforced concrete pile. For piling of ready piles and dowel, pile-loading aggregates, copra and koprovo equipment with pile immersions of impact, vibration, vibro-impact, indenting and vibrating action and for screwing piles are used. Some types of equipment are also used to extract from the ground previously immersed elements (pile extractors).

The technological cycle of immersing ready-made piles includes the operations of grabbing and installing piles in the design position, piling the piles by the pile-loader into the ground to the design mark, moving the pile-driving installation to the site of the next pile dipping.

5. Pile hammers.

Pile hammers consist of a massive shock part moving back and forth with respect to the guide structure in the form of a cylinder (pipe), a piston with a rod, rods, etc. The impact part of the hammer strikes alternately impacts the head of the pile and immerses the pile in the ground. The guide part of the hammer is equipped with a device for fixing and centering the hammer on the pile.

The working cycle of the hammer includes two moves - idling (raising the impact part to the extreme upper position) and working (accelerated movement of the impact part down and blow to the pile). By the drive, pile hammers are divided into mechanical (rarely used), steam-air, diesel and hydraulic. The main parameters of pile hammers are the mass of the impact part, the maximum energy of one stroke, the maximum height of the impact part, the frequency of beats per minute.

Steam-air hammers are driven by steam energy or compressed air up to 0.5...0.7 MPa. There are simple single-acting hammers whose drive energy is used only to lift the impact part, which then makes a working stroke under the influence of its own weight, and double acting hammers, whose energy drives the shock part also additional acceleration during the working stroke, resulting in increased impact energy and the working cycle is shortened.

The impact part of steam-air hammers of simple action is a cast-iron body with a mass of 1250-6000 kg, the guide is a piston with a rod resting on the head of the pile. Such hammers are simple in design, simple and reliable in operation, but due to low productivity (no more than 30 beats per minute) are used comparatively rarely. The most common are automatically working steam-air hammers of bilateral action with a frequency of impacts on the pile up to 100-300 per minute and the weight of the shock part up to 2250 kg. The drawbacks of double-acting hammers include a considerable mass of stationary parts ("dead" mass), which is 60-70% (for single-action hammers up to 30%) of the total mass of the hammer, the possibility of immersing only light dowels, wooden and reinforced concrete piles, steam or compressed air.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Test questions:

1. What is drilling?

2. What are the purpose, how are the boring-crane machines arranged and how do they work?

3. What is the main parameter of drilling machines?

Lecture №10.

Topic:Types of crushing machines and their choice depending on the strength of the crushing material and the degree of grinding. Schematic diagrams, workflow, basic parameters and productivity of crushing machines.

Outlineofalecture:

1. Machines for crushing stone materials.

1. Machines for crushing stone materials.

A large number of stone materials are consumed annually in construction: crushed stone, gravel and sand. Most of these materials are used for the preparation of concrete. The extraction of sand and gravel is carried out in natural deposits mechanically or hydraulically, and crushed stone - from natural stone by crushing rock blasting rocks. The extracted stone materials are processed at stone crushing and washing and sorting plants, and then delivered to the consumer in the form of a finished product of standard quality.

The quality of the rubble is characterized by the grain composition, the shape of the grains, the mechanical strength and the content of harmful impurities.

Depending on the grain size of the gravel, the crushed stone is divided into fractions 5...10; 10...20; 20...40 and 40...70 mm. In addition, fractions 3...10 are allowed for road construction; 10...15; 15...20 mm and for the ballast layer of the railway track 25...50 mm. For massive concrete structures, the upper limit of fineness can reach 120...150 mm. According to the shape of the grains, they are classified into breamy ones, whose width is three times or less less than the length, and cubed-shaped. Operating GOSTs do not allow the content of crushed grains in the crustal form of more than 15%. Crushed gravel is obtained by crushing gravel and boulders. The requirements for crushed stone obtained from gravel are basically the same as for crushed stone obtained from blown stone rocks. The mechanical strength of crushed stone is determined by the strength of the rocks from which it was obtained. There are rocks of low strength 30...80 MPa, medium - 80...150 and high - more than 150 MPa.

Sands by the degree of size are divided in modulus of size and other indicators into large, medium and small. In the process of processing non-metallic materials to free sand and, in some cases, rubble from clay and other harmful particles, rinsing and dehydration are used. Dehydration is performed to reduce moisture to a level that allows it to be transported, and to prevent frost in winter.

The crushing of stone materials is carried out by applying static and dynamic loads. The materials are crushed by crushing, impact destruction, abrasion, splitting, and explosion destruction. In many cases, crushing occurs with the simultaneous action of crushing and abrasion.

The degree of crushed is called the ratio of the size of the largest, loaded into the stone crusher to the size of the maximum grains in the crushing product:

The crushing is divided into a large one (Dmax = 1200...1500 mm, dmax = 100...300 mm), average (Dmax = 100...300 mm, dmax = 30...100 mm), fine (Dmax = 30...100 mm, dmax = 5...30 mm) and fine (grinding). Crushing of rocks of high and medium strength is carried out by crushing, splitting and impact; grinding - abrasion and impact.

Depending on the degree of crushing of materials, crushing machines are divided into crushers and mills. Some machines can work as crushers and as mills (for example, roller crushers, runners). According to the principle of action and constructive characteristics, the crushers are divided into jaw, cone, roll, hammer and rotor crushers; Mills - on drum, ball, runner and vibrating. Different types of crushers make it possible to obtain a certain degree of crushing inherent in this structure: jaw - 2...8; rolls - 1,5...10; conical - 3...8; hammer - 5...30; Mills - 10...20.

The choice of type of crushing equipment is carried out depending on the maximum size of the pieces of the raw material, its strength, the required degree of crushing and the required productivity.

The crushing of materials leads to one or more stages. The predominant distribution has received a stage crushing, in which the material is crushed in 2...3 doses on crushers of different types. At each stage of crushing, a material with the required dimensions of the pieces is obtained. Such pieces are screened out on a screen installed in front of crushers of different stages. Crushers of the last stages work, as a rule, in a closed cycle with a vibrating screen, while the material larger than the specified size is returned to the same crusher for repeated crushing (Figure 6.1).

In a single-stage crushing, the resulting fine grains fill the spaces between the large grains and protect them from the direct impact of the machine's crushing organs, which is accompanied by an additional grinding of the material and energy consumption.

The main indicators of the crushers are: the maximum crushing size, the degree of crushing, the specific energy consumption (kWh/m3), the productivity (m3/h or t/h).

Jaw Crushers. They are used for large and medium crushing of strong and medium strength rocks in the primary and secondary stages of crushing. By the nature of motion of the movable cheek, the jaw crushers are divided into grinders with a simple and complex swing of the cheek.

A crusher with a simple swing of the cheek (Figure 6.2, a, b) consists of a welded body 1 in which an eccentric shaft 7 with a connecting rod 8 suspended to it is installed in the bearings 8. The lower end of the connecting rod has special seats in which the ends of the spacer plates 12 are freely inserted and 13. The opposite end of the spacer plate 13 is inserted into the pocket of the movable cheek 3 suspended on the axis 5. The end of the plate 12 abuts against the wedge of the adjusting device 9. The rod 11 and the spring 10 provide the reciprocating movement of the movable cheek and prevent the spacer plates from falling out. To the stationary 2 and mobile cheeks are mounted crushing plates 4 with vertical corrugation, which are the main working organs of jaw crushers. The working surfaces of the crushing plates and the side walls of the crusher body form a crushing chamber. Crushing plates are installed so that the protrusions of one are located against the valleys of the other (Figure 6. 2, c). The crusher drive consists of an electric motor and a multi-row clincher transmission with a massive pulley-flywheel 6.

To ensure the start-up of the crushers, as well as the start-up of the crushers under the rubble, an auxiliary drive is used in the last crusher constructions (Fig. 6.3). It consists of an electric motor of lower power 4, a gear reducer 3 with a large gear ratio, an overrunning clutch 2 connected to the shaft of the main electric motor 1. The starting is carried out by an auxiliary drive. After this, the main motor is turned on, and the auxiliary drive is automatically switched off.

The operating mode of the crusher is changed by adjusting the output slit with a wedge or other design of the adjusting device. The exit slit is measured between the top and the crest of the crushing plates at the time of the largest removal of the movable cheek. The width of the discharge gap is 40...120 mm for medium crushers and 100...250 mm for large crushing.

When the eccentric shaft rotates, the movable cheek is carried out in a swing motion, like a pendulum. For one revolution of the eccentric shaft, the movable cheek, approaching the stationary one, makes the working stroke (crushing) and idling, at which the product of crushing falls through the discharge gap. For jaw crushers with a simple swing of the cheek, the most characteristic type of material is crushing, splitting and fracture. Therefore, they are used for large and medium crushing of high-strength rocks in the primary stage of crushing.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Test questions:

1. The sequence of production of non-metallic building materials and machines used for this purpose?

2.What is the degree of crushing?

3. Indicate the main performance indicators of crushers, as well as the principle of choice of crushing equipment, depending on the size of the initial product, its strength and the requirements for the grain composition of the product of crushing?

Lecture №11.

Topic:Appointment and composition of the mixing machine. Classification of mixing machines.Purpose of the dispenser and their classification.Types, basic parameters and design schemes of concrete mixers and mortar mixers of cyclic and continuous action.

Outlineofalecture:

1. Machines for the preparation of concrete and mortar mixes.

2. Classification of mixing machines.

1. Machines for the preparation of concrete and mortar mixes.

Concrete and mortar mixes are prepared by mechanically mixing their components in mixing machines - concrete mixers and mortar mixers. The quality of the mixture is determined by the accuracy of the dosage of the components and the uniformity of their distribution among themselves throughout the volume of the mixture. For the uniform distribution of the components of the mixture among themselves in the total volume of mixing, the particles of the material are informed of the trajectories of motion with the greatest possibility of crossing them. Mixing the components in a homogeneous mixture is a fairly complex process that depends on the composition of the mixture, its physical and mechanical properties, mixing time and the design of the mixing device.

The technological process of preparation of mixtures includes sequentially performed operations: loading of the detached components (binders, aggregates and water) into a mixing machine, mixing of components and unloading the finished mixture.

2. Classification of mixing machines.

Mixers are classified according to three main characteristics: the nature of the work, the principle of mixing, the method of installation.

By the nature of the work, the mixing machines of periodic (cyclic) and continuous operation are distinguished. In cyclical mixers (Fig. 6.1, a...d), the mixing of components and the delivery of the finished mixture are carried out in separate portions. Each new portion of the components of concrete or mortar can be loaded into the mixer only after the finished batch is unloaded from it. Mixers cyclic action is usually used for frequent changes in the mark of concrete mixtures or solutions. In them, you can adjust the duration of mixing.

In continuous mixers (see Fig. 6.1, d), the loading of components, their mixing and the delivery of the finished mixture are carried out simultaneously and continuously. The detached components are continuously flowed into the mixer and blended by the blades as they move from the loading hole to the discharge. The finished mixture is continuously fed into vehicles or a supply hopper. Continuous mixers are most suitable for the preparation of large volumes of concrete or mortar mix of the same brand.

The main parameter of cyclical mixing machines is the volume of finished batch (l), issued in a single cycle of operation, continuous mixers - the volume of finished products (m3) that the machine produces per hour of operation.

By the principle of mixing components distinguish machines with mixing with free falling materials (gravitational) and with forced mixing (forced action). In mixers of compulsory action, the orbits of the components are forced, in gravitational - free. The gravitational mixer rotates relative to the horizontal or inclined (up to 15°) axis of the drum with lobes on the inner surface (Figure 6.1, a).

The blades continuously pick up and lift the components of the mixture to a certain height, when they reach it they freely fall from the blades under the action of gravity; mixing occurs as a result of a collision of the falling streams of components. To avoid the occurrence of centrifugal forces that prevent the free circulation of the mixture inside the drum, the frequency of its rotation usually does not exceed 0.3...0.4 s1. In mixers with forced mixing, the components of the mixture are forcedly mixed in a stationary drum or bowl with horizontal, inclined or vertical blade shafts (Figure 6.1, b, d) or an impeller rotor (Figure 6.1, c, d) rotating inside the mixing tank. Mixers with horizontal mixing shafts are called troughs, with vertical shafts - tray.

By the method of installation mixers are divided into mobile and stationary. Mobile mixers are used for small amounts of construction and repair work on dispersed objects, and stationary ones are part of the technological lines of concrete mixing and mixing plants of medium and large capacity of concrete and mortar plants.

Fig. 6.1. Schemes for mixing materials in mixing machines

Gravity mixers are designed for the preparation of mobile mixtures and are manufactured mobile and stationary, cyclic and continuous.

Mobile gravitational cyclic mixers have a volume of ready kneading of 65, 165 and 330 liters and are used as independent machines for the preparation of mobile concrete mixes with aggregate size up to 70 mm and mortars on objects with small and medium volumes of work. Mixers with a volume of finished batch 65 l are produced on a wheeled chassis, with a volume of 165 and 330 l - on runners.

The mixer (Figure 6.2, a) with a volume of 65 liters of ready mix consists of a pear-shaped mixing drum 1, on its internal surface there are fixed three interchangeable stirring blades 2, drive 3, driving the mixing drum into rotation, frame 4 with a wheel stroke and a turning mechanism inclination) of the drum with the fixing device. The rotation of the drum is communicated from the electric motor 7 (Figure 6.2, b) through the V-belt transmission 6 and the two-stage cylinder-reduction gear 5. The drum 4 is rigidly fixed to the slow-speed shaft of the reducer and rotates at a frequency of 27 min-1.

When the components are loaded and mixed, the drum is tilted to the horizon at an angle of 12°, and when unloading the finished mixture - at an angle of 40°. The starting materials are loaded into the mixer by hand, and the finished mixture is discharged by tilting the rotating drum with the hole downward. The mixer can dispense up to 2 m3/h at 30 cycles per hour. The transfer of the drum from the loading and mixing position to the unloading position is done manually with the control handle mounted on the gear housing. In each position, the drum is fixed by the control handle pin, which enters the hole of the bracket on the mixer frame.

Fig. 6.2. Mobile gravity mixer with a volume of ready kneading of 65 liters: a - general view; b - kinematic scheme

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Testquestions:

1. What components are used for the preparation of concrete mixtures and mortars?

2. Give a classification of dispensers. How do they differ in functional and constructive features?

3. What is the classification of the mixers and what are their preferred applications?

Lecture №12. Topic:Machines and equipment for the preparation and transportation of concretes and mortars.

Outlineofalecture:

1. Machines and equipment for transportation of concrete and mortar mixes.

2. Mortar-carrying truck.

3. Ready-mix trucks.

4. Truck mixers.

1. Machines and equipment for transportation of concrete and mortar mixes.

For transporting commercial concrete and mortar mixtures for distances greater than 1 km from mixing plants and plants to construction sites, specialized vehicles are used based on the chassis of trucks - mortar-carrying trucks, ready-mix trucks and truck mixers equipped with technological equipment to prevent losses and preserve the quality of mixtures on the route. In some cases, rigid mixtures are transported in specially equipped dump trucks. At large construction sites, the mixtures are transported in bunkers, tubs, containers installed in car bodies or on railway platforms. Transportation of mixtures to the place of laying for short distances in in-house conditions is most effective by means of pipe transport - concrete and mortar pumps, concrete and mortar-carriers. When transported through pipes, the mixture is continuously moved in horizontal and vertical directions, the quality of the mixture is maintained and its loss is minimized. Pipe transport allows deliveries of the mixture to hard-to-reach places and works on their laying in cramped conditions.

The quality of mixtures transported by specialized vehicles is affected by the transportation time, the temperature of the mixture and the environment, and the condition of the road surface.

2. Mortar-carrying truck.

Mortar-carrying trucks are used to transport high-quality mortars of various mobility (5...13 cm) at a speed of up to 65 km/h with mechanical motivation in the route and batch delivery of the mixture on construction sites to the receiving capacities of mortar pumps, plaster units and stations, intermediate storage bins and tubs. Mixing of the solution in the path is ensured by auger or paddle impeller, batch dispensing of the solution by slide shutters (flaps). The drivers and cut-offs have a hydraulic drive. Mortar-carrying trucks are equipped with an on-board device for washing the tank with water heated by the exhaust gases, which facilitates the care of the tank and prevents the buildup of the skeletal residue on its walls. They operate at an ambient temperature of -20 to + 40 °C.

The main parameter of caravans is the useful capacity of the tank (the volume of the transport mixture) in m3.

Fig. 6.10.Mortar-carrying trucks: a - general view; b - tank with driver

Mortar-carrying trucks (Figure 6.10, a) consists of a set of technological equipment installed on the chassis of the car ZIL. The equipment set includes a horizontally installed tank / useful capacity of 2.5 m3 with a deployed upper generatrix, inside of which there is a single-shaft paddle driver 3 with a spiral blade 14 (Figure 6.10, b) for stirring the solution in order to avoid its delamination during transport. The tank is installed on platform 4.

The solution in the tank is loaded from above with the open folding two-leaf lids 2. The solution is unloaded through a discharge device 5 provided with an air-operated slide gate 7 and discharge chutes. An additional rotary tray is hinged to the unloading device.

The impeller shaft 12 of the drive is driven into rotation at a frequency of 5...15 minfrom the hydraulic motor 9 through a closed gear train. The drive of the hydraulic pump is carried out from the engine of the base machine 11 through the power take-off 10. When the driver's shaft is rotated clockwise, a stirring mixture is induced to prevent its separation. When rotating in the opposite direction, the driver ensures the supply of the mortar mixture to the unloading device. The blade 14 is attached to the posts 13 of the blade shaft 12 rotating in the bearings 15.

Control the operation of the driver using hydraulic control valves 8 both from the control panel 6 and from the driver's cab.

The mechanical system for unloading the tank with a controlled gate valve allows dispensing the solution in batches and servicing several building objects for one flight of the machine.

The useful capacity of the tank of mortar-carrying trucks is 2.5...4.6 m3.

3. Ready-mix trucks.

Ready-mix trucksare used to transport commercial concrete mixtures for distances up to 5...10 km. The working body of ready-mix trucksis a tilting body with a drop-shaped shape with high sides, tilted back by a hydraulic lifter when unloading at an angle of up to 90°. Ready-mix trucksare equipped with devices for washing the body, heating the body with exhaust gases, shaking the body during unloading. The main parameter of ready-mix trucksis the useful capacity of the body (the volume of the transported concrete mixture) in m3. Modern ready-mix trucksconstructively similar and the most unified.

Ready-mix truck (Figure 6.11) is mounted on the chassis / car base and is equipped with a body 3 with a useful capacity of 4 m3. The body tilts back when unloading with respect to the support frame 5 by an angle of 90° with two telescopic hydraulic cylinders. To ensure the stability of the ready-mix trucks during the lifting of the body and unloading the rear axle, the chassis of the machine is equipped with two hydraulic jacks. 4. Hydraulic cylinders and hydraulic jacks operate from the hydraulic system of the base chassis. The body is narrowed to a discharge port located above the level of the transport mixture, which virtually eliminates the loss of the mixture in the path. For complete unloading of the mixture without the use of manual labor, the body is equipped with a built-in vibrator with a hydraulic drive, shaking the body in any positions during lifting and lowering. To protect the transported mixture from the effects of atmospheric precipitation, wind and high temperatures, the body is closed from above with a lid 2, and to protect the mixture from the effects of low negative temperatures, the body has double walls between which the exhaust gases of the car circulate. The working cycle for the delivery of the mixture by the ready-mix trucksincludes the following technological operations: loading the finished mixture at the factory, closing the body with a lid, transporting itself, unloading the mixture by tipping the body, cleaning the inner surface of the body, returning it to its original position and driving after a new portion of the mixture. The mix delivered by the ready-mix trucksis unloaded directly on the site of laying or in intermediate tanks – bunkers, buckets, etc.

Fig. 6.11. Ready-mix truck

The carrying capacity of ready-mix trucks4,0...10 t, the volume of the transported concrete mix 2,5...4,0 m3, the duration of unloading of the concrete mix 1,5...2 min.

4. Truck mixers.

Truck mixers are used to prepare a concrete mixture on the way from specialized components supplied by dried components to the place of installation, preparation of the concrete mixture directly on the construction site, and also transportation of the finished quality mixture with the motivation for its transportation. They are gravity reversible concrete mixers with individual drive, mounted on the chassis of trucks.

The main parameter of truck mixers is the volume of finished batch (in m3). The technological equipment of domestic truck mixers has the same design and is maximally unified. Truck mixers operate at an ambient temperature of -30 to +40 ° C.

The truck mixer (Figure 6.12) with the volume of ready mix 4 m3 is mounted on chassis 1 of KamAZ truck. The working equipment of the truck mixer includes a frame 9, a mixing drum 4 with a loading and unloading device, a drum rotation mechanism 3, a dosing and washer tank 2, a water centrifugal pump, a control system for equipment with levers 10, 12 and instrumentation П. The mixing drum has three The reference points are inclined to the horizon at an angle of 15°. The loading and unloading device consists of a loading and unloading 6 funnels, a variable length folding tray 7 and a rotary device 8. The tray can be rotated at a horizontal angle of 180° and in the vertical plane by an angle of up to 60°. Two spiral blades 11 are mounted on the inner surface of the drum (Figure 6.13), the angle of inclination of which is adjusted in such a way that when rotating in one direction, the components of the mixture are directed to the lower part of the drum where their gravitational stirring takes place, and when rotating in the opposite direction, the finished mixture to the receiving tray connected to the rotary discharge chute. Rotation of the drum 9 is communicated from the individual diesel engine 3 through the reversible gear reducer 5 and the chain gear 6, the driven sprocket 8 of

12    11	10	9

Fig. 6.12.Truck mixers

which is rigidly attached to the spherical bottom of the drum.

The drum is supported from the front on the chassis frame by the central spigot 7, and at the rear by a smooth band 10 on the support rollers 12 mounted on the ball bearings. The drive provides two speeds of rotation of the drum in both directions during loading, mixing and unloading. The frequency of rotation during loading is selected depending on the capacity of the power plant. Preparation of the mixture in the path of follow-up is carried out at a transport range of not more than 10...15 km, while the detached components are loaded simultaneously into the mixing drum. When transporting for long distances, the drum is first loaded with dry components (cement and aggregates), and the supply of water and the preparation of the mixture is made directly at the site. The predetermined portion of water is supplied to the mixing drum from the dosing and washing tank by a centrifugal pump 1 through a nozzle in the hopper. Through the same nozzle, the drum is rinsed with water after unloading. The pump drive is carried out from the engine 3 through the propshaft 4 and the V-belt drive 2. When transporting the finished concrete mixture, in order to avoid its delamination, the drum rotates at a reduced frequency, continuously mixing the mixture.

The truck mixer with hydraulic drive and with the volume of the finished drum mix 5 m3 differs from the truck mixer with the mechanical drive system of the drum drive and power take-off, as well as the possibility of infinitely variable speed control of the mixing drum in the range 0...20 min. The rotation of the mixing drum is communicated from the reversible hydraulic motor with a working pressure of 25 MPa through the planetary reductor. The hydraulic motor is powered by an adjustable reversible hydraulic pump, which receives rotation from the power take-off through the driveshaft.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Testquestions:

1. What is the purpose of machinery and equipment for transporting concrete and mortar mixes of the machine?

2. What is the purpose of the mortar-carrying truck?

3. What are the purpose of ready-mix trucks?
4. What are the purpose of truck mixers? What kinds of work can they do?

Lecture №13.

Topic:Machines for plastering and painting works. Purpose, composition, basic parameters and area of application of plaster stations and aggregates, manual trowels.

Outlineofalecture:

1. Machines for plastering works.

2. Machines for painting works.

3. Plastering stations.

4. Manual plastering trowels.

1. Machines for plastering works.

Plastering works are performed for leveling and decorating the surfaces of building structures, improving their sanitary and hygienic qualities, as well as reducing the thermal, sound conductivity and water absorption of the enclosing structures, protecting them from atmospheric influences.

The complexity of plastering is 14...16% of the total laboriousness of erecting buildings and structures, and their cost reaches 8...10% of the total cost of construction and installation works. In modern construction, two kinds of plasters are used: monolithic and dry-cast. Monolithic plaster is used for finishing the internal and external surfaces of various structures of buildings and structures and is created by applying a solid layer of plaster mortar of a certain thickness to the surfaces to be treated. Dry gypsum plaster (gypsum plasterboards) is used for finishing the internal surfaces of buildings in rooms with dry and normal moisture conditions and is created by facing the surface to be treated with separate sheets of dry gypsum plaster of industrial production with subsequent sealing of joints. Dry gypsum plaster sheets are glued to the surfaces to be masticated or fastened with nails, screws, self-tapping screws to metal or wooden frames, previously attached to the lining surfaces. When using a dry gypsum plaster, manual machines are widely used (see Chapter 8). Monolithic plaster is performed by applying to the surface to be treated in a certain technological sequence of several layers of plaster namet: a layer of spray and one layer of soil (simple plaster); spray, a single layer of soil and a covering layer (improved plaster); spray, one or more layers of soil and the covering layer (high-quality plaster).

All technological operations for the installation of monolithic plaster (acceptance and preparation of the solution, percolation and bringing it to working mobility, transportation of the solution to the place of production, spraying of soil and the cover layer and its grouting) are mechanically integrated using mobile plaster and plaster mixer machines and units (with small amounts of plastering), high-performance mobile plastering stations (for large volumes of work).

Plastering units and stations are made on the basis of solution pumps of various types and are completed with diluents, nozzles and plastering trowels.

2. Machines for painting works.

Painting works are the final stage of all the finishing of buildings and structures (both newly constructed and reconstructed and repaired) and are performed to give them an aesthetic appearance, interior decoration, increase their durability, create normal sanitary and hygienic conditions for the operation of internal premises. Painting also includes wallpaper. All the main technological operations of painting and wallpapering in modern construction are mechanized, but their labor intensity is about 10% of the total labor costs for construction and installation work. Painting is characterized by the complexity and multi-operation of the technological process, the diversity of color compositions, materials, machines, mechanisms, equipment and devices.

The complex for the production of painting works in urban construction includes the centralized preparation of painter's and semi-finished products in specialized procurement color shops and factories, the centralized delivery of finished commodity products to construction sites and the application of painting compositions (with preliminary processing of semi-finished products) to machined surfaces by a mechanized method.

Preparation of painting compositions from semi-finished products and their supply to work places are carried out by mobile painting stations and units. The execution of painting works directly on the construction site includes the preparation of surfaces and their painting.

For mechanized coloring of various surfaces with oil, synthetic and aqueous compounds, portable and mobile painting units that operate according to the principle of spraying of color compositions are widely used. In such units, the colorful composition of the reservoir by the force of compressed air or by means of a pump is fed under pressure to a spraying device (nozzle, nozzle), crushing the composition into tiny particles and applying a thin layer on the painted surface. There are different spraying units for air (pneumatic) spraying, intended for the application of fillers, nitro-enamels, oil and glue paints, and mechanical (airless) spraying on the surface, used to paint varnishes, water-chalk and water-lime paint compositions on the surface.

3. Plastering stations.

Such stations usually work with ready-made stock solution and are used in facilities with medium and large volumes of external and internal plastering works. With the help of plaster stations, a high-performance complex-mechanized continuous feeding and application process is carried out. They are a set of equipment for receiving, inducing, sifting, pumping and applying plaster solutions mounted in a technological sequence inside a metal insulated body mounted on skids, in the body of a caravan or on a pneumatic wheel chassis. Usually the plastering stations are equipped with serial production machines. Deliver stations to facilities by trucks.

Plaster stations are distinguished according to the design and productivity of the mortar pump, the method of loading the receiving hopper, the type of motive-mixer in the receiving hopper. The main parameter of the stations is the capacity of the solution pumps installed on them. When loading receiving bunkers of stations with a solution, vehicles (autocarrier, dump truck) are located at the station parking level or on the access ramp. As an incentive-mixer, installed in the receiving bunkers of the stations, screw conveyors and rotary devices are used. Plaster stations are produced in small series.

When the solution is induced, the groove is a movable wall, forming a closed mixing chamber. In the storage tank, a stock of solution is created, sufficient for the smooth operation of the mortar pump during the feeding of the next portion of the solution from the hopper into the mixing zone. If necessary, a portion of water is added to the batch to bring the solution to readiness with a mobility of at least 7 cm. Cleaning of the mixing zone of the hopper is carried out by reversing the auger, with the waste moving to the side walls and through the hatches being removed to the outside.

The station is equipped with pressure rubber tubing sleeves with a diameter of 38 and 50 mm. The station is equipped with water supply, heating and ventilation systems, as well as fire extinguishing means. Serves the station one operator. The disadvantage of the station is the absence of a vibroactive zone on the screen grid and the absence of an impulse in the solution reservoir.

4. Manual plastering trowels.

Manual plastering trowels are used for leveling and smoothing of various plaster and other coating compositions applied to horizontal, inclined and vertical surfaces. These machines are also used for grouting cements of screeds under the floors and roofs of soft roll materials, with single-layer alignment of gypsum-concrete partitions, trowelling of surfaces in the manufacture of reinforced concrete prefabricated elements of building structures, for polishing pasted and mosaic surfaces, facing facades of buildings, etc.

Plastering trowels are produced with an electric drive, which can be integrated into the machine or rotate the working element (trowel) through a flexible shaft.

Plaster-troweling machines with built-in electric drive are produced by single-disk and double-disk - with external and internal disks. As the drive of these machines, asynchronous three-phase squirrel-cage rotor motors operating at a high-frequency current of 200 Hz at 42 V voltage are used. The machines are connected to an external AC network with a voltage of 220/380 V, a frequency of 50 Hz via a frequency converter or a special three-phase network AC with a frequency of 200 Hz and voltage of 42 V. Low voltage provides electrical safety of the operator. Machines are equipped with a current-carrying cable, which is connected to the power supply by means of a plug of the plug connection.

Fig. 7.12. Double disc plaster trowelling machine

In the two-disk machine (Fig. 7.12, a, b), the two-stage reducer 3 provides a separate drive from the electric motor 4 of the outer 1 and internal 2 trowels, which rotate in opposite directions and are rigidly connected to the output shafts of the reducer. The torque of the rotating outer and inner discs is zero, which makes the machine stable, reduces the load on the hands of the operator, thereby increasing productivity and improving the quality of work.

To improve the quality of the grouting, water can be supplied to the machines, wetting the rubbed surface. The amount of water supplied to the treatment zone under pressure of 10...30 kPa depends on the state of the surface to be rubbed and is controlled by the water supply switch. Water to the machines is supplied from the water pipe through a thin rubber sleeve. The water supply switch and the drive switch are installed at the right hand control.

The grouting of the plaster surface by the troweling machines is carried out during the setting process of the mortar, when the strength of the covering layer reaches 0.05...0.1 MPa, and must be finished before the hardening begins. When trowelling the plaster coating with a plaster trowelling machine, plastic deformation of the top layer and partial compaction occur. At the same time, the surface is leveled over the entire area by transferring the solution particles in the working plane.

When working, the plastering trowel is smoothly moved by hand, pressing the discs with a certain force to the surface to be treated.

Trowel capacity up to 50 m2 / h.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Test questions:

1. What are the purposes for, how are the plaster stations arranged and how do they work?

2. What are the use of painting units?

3. What are trowel machines used for?

Lecture №14.

Topic:General information, the classification of manual machines according to their intended purpose, the principle of operation, the nature of the movement of the working body, the operating mode, the type of drive and protection classes. Basic requirements for hand machines.

Outlineofalecture:

1. General information, classification of manual machines for their intended purpose, the principle of operation.

2. Basic requirements for hand-held machines.

1. General information, classification of manual machines for their intended purpose, the principle of operation.

Manual refers to a technological machine equipped with an integrated motor, during which the machine's mass is fully or partially perceived by the operator's hands. The main movement of the working element is carried out from the engine, and all auxiliary movements (feed, control, mode setting and operation duration) are performed manually.

The most common were hand-held machines in the construction of sanitation. finishing, assembly and repair works, as well as works on the installation of metal structures and technological equipment. The use of hand-held machines makes it possible to increase labor productivity 5 to 10 times (in comparison with manual work), significantly reduce labor input and improve the quality of the technological operations performed, and improve the working conditions of the worker.

Hand machines (PM) are classified according to the following features: by designation - machines for processing metals, wood and stone, for assembly, finishing, assembly, excavation and drilling operations;

by type of drive - electric, pneumatic, motorized (driven by an internal combustion engine), hydraulic and powder machines (gunpowder assembly guns, pyrotechnic mandrels);

by a method of transformation of energy of a food - electromagnetic, mechanical, kompressionno-vacuum and spring;

for execution and speed control - straight (the axis of the working element and the drive are parallel or coincident), angular (the axis of the working element and the drive are angled), reversible and non-reversible, single-speed and multi-speed;

by the nature of the movement of the working organ - machines with rotational, reciprocating and complex motion. In rotary machines, the force action of the working element on the object being processed is continuous.

2. Basic requirements for hand-held machines.

The electric hand-held machine is an electrically, vibro- and noise-proof portable unit consisting of a housing built into the electric drive housing, transmission mechanism, operating element, starting and regulating equipment.

All manual electric machines are divided into three classes according to the degree of protection of the operator from electric shock.

Class I refers machines to rated voltages above 42 V, in which at least one metal part accessible for touching is separated from the live parts by only one functional insulation. Class I hand-held machines are not used for construction and assembly works.

Class II refers to manual machines for rated voltages above 42 V, in which all metal parts accessible for touching are separated from live parts, double or reinforced insulation. The output of Class II machines is more than 70% of the total volume, the production of electric handheld machines in our country.

Class III includes manual machines operating at a low voltage, safe for a person up to 42 V, receiving power from an autonomous power source or from a common network via a current converter or transformer. The operation of machines II and III classes is possible without the use of personal protective equipment.

By now, fully electrically-safe hand-held machines equipped not only with double but also with so-called full electrical insulation have been created. Such machines have an all-plastic body and do not contain, in addition to the working body, the outer metal parts.

By type of drive distinguish:

electromechanical hand machines - with a rotary motor, the movement of which is communicated to the working body (tool) through the transfer device (reducer, crank-but-rod mechanism, etc.);

Compression-vacuum machines in which the transfer of energy to the working member is carried out by a drummer pneumatically connected with an intermediate conversion mechanism;

electromagnetic - with a linear electromagnetic motor of reciprocating (shock) action, informing the movement of the working tool directly.

As a drive for electric handheld machines, single-phase collector motors of the type KH II protection class with useful power 120 ... 850 W, single- and three-phase asynchronous motors with squirrel-cage rotor of useful power 120 ... 750 W with mains supply normal (50 Hz) and increased (200 Hz) frequencies.

Machines with electric motors of the KN type gained a predominant distribution in construction due to their lightness, portability, mobility and the possibility of direct connection to the mains of single-phase and three-phase normal frequency. The production of hand-operated electric machines based on lightweight high-speed collector single-phase motors with a nominal speed of 250 ... 333.3 s-1 is being mastered.

Recently, hand machines based on DC collector motors have been developed with a power source from small-sized storage batteries built into the body or the handle of the machine. In the drives of hand-held machines with collector motors, the electronic regulation of the rotational speed of the rotor shaft is increasingly used, which makes it possible to increase their productivity due to the optimal operating mode.

Three-phase asynchronous motors with squirrel-cage rotor have a simpler, reliable design (collector and brushes are absent) and operate at a normal frequency current of 50 Hz (AN type motors) and an increased frequency of 200 Hz (AP type motors). Electric motors of the AN type, having large dimensions and weight, low specific power and efficiency, are used in manual machines with a heavy duty. Electromotors of the AP type in comparison with the engines of AN have higher energy characteristics; they are portable, fast (rotor speed up to 300 s1), characterized by high power per unit mass, reliable in operation and durable. These engines of the III class of protection for 42 V are used in manual machines operating in hazardous conditions with regard to electric shock to the operator.

Manual machines with three-phase asynchronous motors are connected to the AC mains of normal frequency (50 Hz) by 220/380 V voltage through intermediate units: with AN type motors - via a step-down transformer, with AP type motors - via a frequency converter, which also performs the functions of a step-down transformer .

Machines with AP-type engines requiring the use of a frequency converter are most effectively used in stationary conditions at stable workplaces.

In the production of electric hand-operated machines, a wide unification of units and parts is used, which, if damaged, can be easily replaced.

At assembly and assembly and assembly works, machines with rotational movement of the working body are widespread: drilling, thread-cutting, and expanding. screwdrivers, impact wrenches, grinding.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Testquestions:

1. Which machines are manual?

2. What is the classification of hand machines?

3. How do manual machines index?

Lecture №15.

Topic:Acceptance and delivery of machines in operation.

System of maintenance and repair of machines.

Outlineofalecture:

1. The main provisions of the system of maintenance and repair of construction machinery.

2. Organization of maintenance and repair of construction machinery.

1. The main provisions of the system of maintenance and repair of construction machinery.

The high level of organization and technical operation and repair of construction machinery allows you to keep a fleet of machines in good condition, ensure their performance and restore the resource.

Our country has adopted a preventive maintenance system for maintenance and repair of construction machinery (the system of PPR). This system is a complex of organizational and technical activities conducted in a planned manner. In accordance with GOST 18322-78 all the work provided for by the PPR system is divided into maintenance and repair.

Maintenance is a set of operations to maintain the machine's operability when used for its intended purpose, waiting, storage and transportation. Maintenance provides for a mandatory, periodically performed according to the plan the amount of work pre-installed for this type and model of the machine. Despite the planned maintenance work, with a certain degree of wear, further use of the machine becomes technically and economically impractical or even dangerous (emergency). In this case, there is a need for repair work.

Repair is a complex of operations for restoring the working capacity of the machine and restoring the resource of the machine and its components. The scope of work on the repair of machines depends on the quality of maintenance, the conditions of use of the machine, the qualifications of the driver, in most cases it is not regulated and is established on the basis of actual demand.

In accordance with the construction rules (SNiP 111-1-76) and regulatory documents, the maintenance is provided for each year (SW). periodic technical maintenance, seasonal maintenance and planned repairs: current (T) and capital (K).

Monthly maintenance should ensure the machine is operational throughout the entire work shift. It is performed before, during or after the end of the shift. The composition of this type of service includes the lubrication of the units and parts of the machine and its preparation for transmission when changing brigades, as well as a check-up before work.

Periodic maintenance is performed through the operating time or the time interval specified in the operational documentation. This type of maintenance prevents the intensity of wear of mating machine parts due to the implementation of preventive measures. For machines mounted on the basis of a car, two maintenance operations (TO-1 and TO-2) are provided, and for machines mounted on the basis of a tractor or with a tractor engine, three maintenance (TO-1 TO-2 and TO -3). Periodical maintenance includes: external care, diagnosis, fastening and adjustment work, and lubrication of the machine.

Seasonal maintenance is carried out to prepare the machines for the autumn-winter and spring-summer operation, as well as before storage. Usually this type of service is carried out twice a year. During seasonal maintenance, the replacement of oils, fuel and coolant is carried out during the transition to the corresponding period of operation.

Current repairs are performed to ensure the machine is operational; it consists of an operation to restore and replace parts and assembly units with partial disassembly of the machine.

Major repairs are carried out to troubleshoot and restore a complete or nearly complete machine resource, including any parts of it and even basic ones.

In the system of PPR it is planned to perform the necessary actions in the car after having worked out a certain number of hours. In accordance with this, for each machine, a schedule is developed for the periodicity of performing maintenance and repairs (for the lifetime of the machine, a certain year, month). Fundamental for the development is the repair cycle, i.e., the operating time of the machine (in h) between the next capital (K) repairs. It is installed for each machine, depending on its purpose and basic parameters. The repair cycle is calculated as follows. so that each stop of the machine for repair corresponds to the time of reaching the extreme wear of a certain group of parts and assembly units. For example, for an earth-moving machine, the duration of the repair cycle (C) is 5760 hours. During this time, it is necessary to conduct four routine (T) repairs every 960 hours. In each cycle of maintenance repair, three TO-2 cycles are carried out with a cycle time of Ct-2 = 240 h. and in each Tcot-2 - three TO-1 with the cycle time Tto-i = 60 h. The intervals between individual types of impacts are always a multiple and for the given periodicity are represented by the ratio 1:4:16:96 or in hours as 60:240:960:5760. For tower cranes 5...9 size groups (М = 250...1400 тм) this ratio will be 1:3:6:72 or in hours - 200:600:1200:14400, that is, the conduct of two TO-1 and one TO-2 before the current repair and eleven current repairs in a complete repair cycle.

The technological process of maintenance of machines consists of a set of operations, each of which is part of the process carried out by one or more workers. The operation is a set of sequential actions for servicing the assembly unit (assembly or assembly) or group of assembly units. So, maintenance operations are: oil change in the engine oil sump, brake regulation, regulation of the friction clutch, etc. The maintenance of the machine consists of a large number of operations that, by their nature and execution conditions, can be combined into specific groups covering the cycle works. Practice shows that maintenance regardless of its type can be divided into the following main works: harvesting, cleaning, diagnostic, fastening, adjusting, lubricating and refueling. Such a department is conditioned, firstly, by the necessity of using the workers of the relevant specialties and qualifications in the performance of each work and, secondly, by the use of special equipment, instruments and tools at the site of work.

The listed groups of work are characterized by a certain significance. Harvesting works (external care), which precede all other types of work, allow to determine the presence of external malfunctions of construction machines, as well as determine the quality of subsequent work, their productivity and culture of implementation.

Diagnosis of construction machines, which consists in determining their technical condition without preliminary disassembly, allows for the introduction of faults and appropriate measures to prevent the increase in wear of mating parts. It is also important that, as a result of the diagnosis, it is possible to establish the residual life of the machine and its assembly units, i.e., to determine the time of normal operation of the machine prior to its overhaul.

During the implementation of practical work, the construction machine is systematically subjected to dynamic loads, which lead to a breakdown in the tightening of threaded connections. This circumstance causes the need for periodic tightening of threaded connections, which requires the performance of fastening operations. Failure to carry out fastening work late can cause an emergency defect.

Conjugate parts of assembly units are characterized by a certain location in space and the appearance between the parts of the gaps of a certain value. In the process of machine operation, the values of these gaps change and after a certain period of time their size reaches the limiting values. In order to restore the original gaps, it becomes necessary to perform adjustments.

During the operation of the machine, friction forces are generated between the friction surfaces of the mating parts, leading to an increase in the temperature of both parts and to their jamming. To reduce friction and, consequently, heat and wear, lubricants are introduced between these parts, allowing the transition from friction without lubrication to friction of the boundary lubricant, and in some cases also to the liquid friction.

Lubrication points, the type of lubricant, lubrication intervals and requirements are specified in the factory manual for each machine. The lubrication system is divided into individual and centralized. In the first case, lubricants are brought to the rubbing surfaces individually with the help of a lubricating device located next to the mating parts. In the second case, one lubricator serves a number of mating parts located in different places of the machine.

Refueling is performed in order to fill the systems of construction machines with operating materials and special fluids: fuel, oil, brake and hydraulic fluids.

Along with scheduled repairs, emergency (not planned) repairs (small, medium) are also possible, which are performed by operating and repair sites of construction organizations (mechanization departments).

2. Organization of maintenance and repair of construction machinery.

Works on maintenance and repair of construction machines are carried out centrally, partly centrally and decentralized.

The centralized form of maintenance is that all technical operations of maintenance and maintenance are performed by teams of highly skilled workers under the guidance of engineering personnel. Thus, with this form of service, machine operators are completely exempted from these works. Centralized service is one of the most progressive forms. It is aimed at the implementation of all elements of the preventive maintenance system in accordance with the technical conditions and at the lowest cost. A special feature of this form of service is the specialization of individual operations, which allows to solve the following tasks: increase the volume of mechanized works with respect to individual maintenance elements; to increase the productivity of machines in the process of exploitation due to the reduction of idle time as a result of an increase in the number of workers and the mechanization of work; improve the quality of service due to the high qualifications of workers. A kind of centralized service is the work of specialized units.

The essence of specialization is that the units carry out work only on maintenance of certain types or brands of machines, and each of the members of the link at the same time performs, with the aid of the means of mechanization allocated to it, the same type of work, the complexity of which corresponds to the category assigned to it. Specialized units should be created to perform periodic technical maintenance (ТО-1, ТО-2 and ТО-3). In operational organizations, characterized by the presence of mixed fleets of machines, it is expedient to specialize the links for servicing certain types of machines (digging, boom cranes, etc.).

Partially centralized form of the organization of maintenance is characterized by the fact that the maintenance is performed by the machinists, who are assigned to these machines, and the periodic maintenance and current repairs are the specialized workers' brigades. Disadvantages of this form of organization of maintenance are the reduction in the effectiveness of the system of PPR; reduction of the working time of the machine as a result of its downtime during maintenance; significant, against planned standards, increase labor costs for work operations due to non-mechanized performance; the difficulty of exercising due control over the maintenance work and routine maintenance.

The decentralized form of maintenance is characterized by the fact that all types of work are performed by machine operators. Specialized brigades carry out only routine maintenance of machines. The disadvantages of this form of organization of maintenance are the same as with a partially centralized form. In addition, with this form of organization, the downtime of machines during servicing is significantly increased.

It was mentioned above that the organization of maintenance and repair of construction machines is carried out by operating enterprises (mechanization departments), which include stationary and mobile workshops. Depending on the composition of the fleet of vehicles, stationary workshops are more or less developed. So, in the presence of a large number of mobile construction machines in the management of mechanization, the bulk of the maintenance and repair work is carried out in the conditions of stationary workshops, which in this case are equipped with all the necessary equipment for the performance of work. When operating small machines (excavators, bulldozers, etc.), dispersed at a considerable distance from each other, it is essential to use mobile workshops that are part of the mechanization department. Modern workshops are equipped with equipment (a compressor, a welding transformer, a hydraulic press, a steam-jet cleaner, tanks for diesel, transmission oil and for hydraulic fluid, a solidifier, machines, tools, spare parts, hydraulic crane, etc.), which ensures a high level of mechanization repair work.

For the maintenance of the industry, universal and special mobile workshops are produced. The latter are characterized by considerable variety and can be used for maintenance of one type of machine: tower cranes, excavators, etc. For mobile vehicles, diagnostic stations KI-4070A, KI-5164 and KI-13905 are used for technical diagnosis of construction machines. The stations are equipped with a set of instruments and tools, the presence of which allows diagnosing the machines with the definition of 60...130 diagnostic parameters.

Current repair of construction machinery is carried out in stationary workshops of mechanization departments and mobile workshops in the conditions of a construction site.

Major repairs are carried out by repair plants. Currently, two methods of overhaul are used: individual and aggregate-node. With an individual repair method, the repaired assembly units are installed on the same machine with which they were removed. With the aggregate-node method of repair, assembly units are depersonalized. The assembly units removed from the machine are sent for repair, and in exchange they are installed on the machines assembly units from the revolving fund, which have been repaired in advance. The aggregate-node method of repair is characterized by a number of advantages. In particular, the need to transport machines from an operating enterprise to a repair plant is excluded, in addition, the time for performing the actual repair of machinery is significantly reduced.

Literature

1. Волков Д.П., Крикун В.Я. Строительные машины.- М.Изд.АСВ, 2002-376 с.

2. Белецкий Б.Ф. «Строительные машины и оборудования». Справ.пособие. Ростов, 2002.

3. Волков Д.П. и др. «Строительные машины» М.: Высшая школа, 1988г.

Test questions:

1. What measures are included in the technical operation of construction machines?

2. What is the acceptance of machines and in what cases is it carried out?

3. What is the system of preventive maintenance and repairs?

Questions for the current monitoring of students' knowledge

- General information about construction machines and equipment.

- What are the main factors that predetermine the use of machinery in construction?

- What construction processes are called mechanized?

- Which machines belong to the technological group?

- On which groups of machines is the complex divided?

- What is the production and technical operation of a construction machine?

- Definitions of the main technical and economic indicators of construction machines.

- What is the performance of a construction machine?

- What are the main indicators of the efficiency of the drive construction machines?

- What types of transport are used in construction?

- List the types of mechanical transmissions.

- List the types of mechanical transmissions?

- What is the purpose of the running equipment of construction machines?

- What is the drive machine? What does it consist of?

- What is the power plant of the machine, what does it consist of, give an example.

- The main technical characteristics of lifting and transporting machines.

- Vehicles and loading and unloading machines.

- Determination of efforts to select a steel wire rope.

- Devices, working process and determination of the productivity of the self-propelled crane.

- Devices, workflow and determination of the productivity of the jib tower crane.

- What are the crawler and tweel propellers?

- What are the building hoists for, what are their main types, how they are arranged and what is the principle of their operation?

- What is the difference between pneumatic-wheeled crawler cranes and crawler cranes? What are their parameters?

- What are the building hoists for, what are their main types, how are they arranged and what is the principle of their operation?

- Devices, workflow and determination of the productivity of a self-propelled crane?

- Determination of efforts to select a steel wire rope?

- Devices, working process of the tower crane.

- What are the loading and unloading machines for?

- How to determine the performance of one-bucket loaders.

- What is the purpose of forklift trucks?

- What is the purpose of earth-moving machinery?

- What are bulldozers for?

- What are the scrapers for?

- Technical parameters of excavators.

- The device and mechanisms for pile driving and piling.

- What are earthmoving machines for?

- What is intended, how the trailer pneumatic wheels are designed and how they work.

- How the self-propelled scraper works and how it works.

- What are motor graders intended for?

- How the motor grader works and how it works.

- What types of work are carried out by motor graders?

- What types of piles are used for the construction of pile foundations?

- What are copras for?

- How the pile hammers work and work.

- What is intended for, how the vibrators are arranged and how they work?

- Concrete mixing plants and transporting machines.

- Devices for the working process of mixing and concrete transporting machines.

- What are the main indicators of the efficiency of the drive of construction machines?

- What is the power plant of a machine, what does it consist of?

- What types of electric motors are used in drives of building machines?

- What is the power plant of the machine, what does it consist of, give an example. What kind of machines belong to the technological group?

- What is the composition of the hydraulic drive?

- How is the hydraulic cylinder and how does it work?

- Which vehicles are specialized vehicles?

- What is the general purpose truck?

- What are the tractors for?

- What is the purpose of pneumatic tractors?

- Machines for transportation of concrete.

- Trucks.

- Machines for the preparation of concrete and mortar mixtures.

- Types of machines for finishing works and their application.

- Technical characteristics of machines for the device of floors and roofing works.

- Types of hand machines and their applications.

- Devices for the working process of machines for plastering and painting.

- Devices and operating mode of hand machines.

- Plaster aggregate machines and installations.

- What types of electric motors are used in the drives of construction machinery?

- What machines are used for gouging floors? How are they arranged and how do they work?

- What are manual trowels used for? What kind of drive do they use?

- What is the purpose of the running equipment of construction machines?

- Plaster aggregate machines and plants?

- Types of machines for finishing works.

- Which machines are manual, give a classification.

- How manual machines are indexed, how they work and what are the main parameters.

- Which machines are used for grinding materials?

- What is the automation of the construction process? What factors predetermined the effectiveness of its application in the construction of construction machines?

- What factors determine the ultimate state of the machine? What is the service life and technical resource of the machine? What is the moral wear of a machine, what is it characterized by?

- What is the size of the machine, what factor does it have that is a model of the machine? Give examples of models of the same size.

- Which groups are divided into construction machines according to the working process regime, the type of energy used, the ability to move and the type of travel devices? What factors determine the belonging of the machines to certain groups according to the indicated characteristics?

- How is the rotary trench excavator arranged and operating? How and for what the teeth on the buckets are arranged according to a special scheme?

- How is the chain trench excavator arranged and how does it work? How are the combined working elements arranged and how do they work?

- What are the main indicators of the efficiency of the drive of construction machines?

- What are the main indicators of the operation of internal combustion engines? What are their implications for diesel engines and carburetor engines?

- How is the gear ratio and efficiency determined for the transmission composed of series-connected gears?

- Describe the device and the principle of chain transfer. What types of drive chains are used in chain drives? What are the features of their work and application?

- What is the automation of construction machines? Outline the essence of automatic control of construction machines. What is automatic control? Automatic regulation?

- List the types of running equipment by the type of propulsor. Give a brief description of each type (purpose, operation characteristics).

- What are the main technical and operational indicators of running machinery equipment. What indicators do they characterize?

- Outline the crawler track equipment. What is the difference between soft caterpillars and hard ones?

- What are the advantages and disadvantages of wheeled chassis tires? How are the tires made? What are the features of their operation?

- What types of transport are used in construction? Give a brief description of each of them, the features of their use.

- What is the difference between trailers and semi-trailers?

- What types of cargo are transported through pipes? Outline the principles of pipeline transport.

- Which vehicles are special? What is their fundamental difference from other vehicles? Give a brief description of each species.

- What are the tractors for? Give them a classification and give a brief description of each type of tractors. What is the main indicator of the tractor? How is it limited?

- What is the purpose of pneumatic tractors? Bring their brief technical and operational characteristics. How are uniaxial and biaxial tractors arranged?

- What are the purpose of transporting machines and equipment? Give them a classification.

- What are the types of steel ropes? What are the characteristics of the rope? What is the characteristic of ropes?

- Why are cranes used in construction, what are their main types and structure? Name the main parameters of the cranes. What is the cargo, altitude and cargo-altitude characteristics of the cranes?

- What are the purpose, how are the single bucket excavators with working equipment of dragline working and working? What is the difference in the process of digging a soil with a bucket of dragline from digging buckets with shovels?

- What is the purpose of tower cranes and what is their predetermined widespread use in construction? Give the classification of tower cranes, the structure of their indexing, describe the device and workflows of each type.

- How are the tower cranes arranged and how do they work?

- List the types of self-propelled gantry cranes, give their general characteristics and list the main components and basic parameters. What kinds of working equipment they are equipped with?

- What are the purposes for, how are crawler cranes arranged and how do they operate? On the basis of which crawler cranes are manufactured rail-wheeled cranes, what effect is achieved by this?

- What is the difference between pneumatic-wheeled crawler cranes and crawler cranes? What are their parameters? Describe the basic arrangement of pneumatic equipment.

- What are the purpose, how are cranes constructed and operated, and the cranes on special chassis of the automobile type? What are their parameters?

- What are the purposes for, how are the pipe-laying cranes arranged and how do they work? Describe the features of their workflow as part of the pipe-laying column. What rigging devices they are equipped with?

- What is the purpose of trench excavators? What working bodies are they equipped with? What is the main parameter of a trench excavator?

- What is the purpose of continuous excavators? What working bodies are they equipped with?

- What is the purpose of one-bucket loaders? Give them a brief classification.

- How is the productivity of one-bucket loaders determined when handling bulk and unit loads?

- What operations are the working cycles of a digging machine? Describe these operations. With the help of which working organs are they performed?

- List the main types of working bodies of earth-moving machines. How are they arranged? What are the main elements of the cutting tool of the digging tool?

- What is soil digging, what is it different from cutting? Describe the force interaction between the digging tool and the soil.

- Give a general classification of machines and equipment for soil development.

- How do one-bucket excavators and continuous excavators differ in terms of material consumption, energy intensity and time use?

- What are the features of using single-bucket excavators in construction of pneumatic, caterpillar and walking walkways? What kinds of excavators are equipped with full-turn and incomplete turning devices? single-engine power plant and multi-motor drive?

- What are the main and basic parameters of single bucket excavators? What is the principle of constructing dimensional groups of universal single-bucket excavators and their indices? Give examples.

- How is the technical and operational performance of single bucket excavators determined?

- List the main and replaceable working bodies of construction hydraulic excavators. Name their main working equipment. Why do excavators have buckets of different widths?

- What are the main areas of application of excavators with pneumatic wheeled and tracked chassis devices? What are the features of their work in the excavation mode?

- What is the purpose of hydraulic excavators with working equipment backhoe? How are they arranged and how do they work?

- What is the use of the clamshell working equipment? How is it arranged and how does it work?

- What replacement working equipment is used to loosen strong soils?

- How are ropeway excavators with working equipment arranged and operated?

Criterion of evaluation of students' knowledge

| Estima-tion on alpha-betic system | Digital equiva-lent of points | % con-tent | Evaluation by the traditional system | Criteria of estimation of students' knowledge |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | excellent | A student demonstrates: - a deep and lasting assimilation of program material; - complete, consistent, competent and logically presented answers on practical exercises; - ability to solve problems freely; - correct, well-founded decisions; - skills of using information from basic and additional specialized literature in the study of discipline; - the ability to self-systematize the program material; - possession of versatile skills and methods of performing all types of tasks; - ability to work with foreign literature and information resources of the Internet; - timely and high-quality execution of all types of tasks. |
| А- | 3,67 | 90-94 |  | A studentdemonstrates: - deep assimilation of program material; - complete, consistent and logically presented answers for all activities; - ability to solve problems; - the right decisions; - skills of using special literature in the study of discipline; - the ability to self-systematize the program material; - possession of skills and techniques for performing all types of tasks; - timely execution of all types of tasks. |
| В+ | 3,33 | 85-89 | good | A studentdemonstrates: - assimilation of program material; - complete, consistent, literate, without significant inaccuracies, an account of the answers for all kinds of tasks; - correct application of theoretical knowledge; - possession of necessary skills in solving problems; - skills in using recommended literature in the study of discipline, - skills of systematization of program material; - possession of skills and techniques for performing all types of tasks; - timely execution of all types of tasks. |
| В | 3,0 | 80-84 |  | A studentdemonstrates: - assimilation of program material; - a consistent presentation of the answers for all types of tasks with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - possession of necessary skills in the calculation of practical problems; - skills in using recommended literature in the study of discipline; - skills of systematization of program material under the guidance of a teacher; - mastering the skills of performing all kinds of tasks; - the ability to independently correct mistakes made; - the timely execution of all types of tasks with the elimination of errors. |
| В- | 2,67 | 75-79 |  | A studentdemonstrates: - assimilation of program material; - ability to present answers with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - possession of methods for calculating tasks; - skills in using recommended literature under the guidance of a teacher; - skills of generalization of program material under the guidance of a teacher; - the ability to correct mistakes made with the help of a teacher; - the timely execution of all types of tasks with the elimination of errors. |
| С+ | 2,33 | 70-74 | fair | A studentdemonstrates: - mastering of the main material; - inadequate formulations when responding to all kinds of tasks; - violation of the sequence in the presentation of the program material; - difficulties in self-fulfillment of practical tasks; - possession of separate methods in the calculation of practical problems; - skills in using the literature recommended by the teacher; - skills of generalization of separate sections of program material under the guidance of a teacher; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С | 2,0 | 65-69 |  | A studentdemonstrates: - mastering of the main material; - inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - difficulties in self-fulfillment of practical tasks; - possession of separate methods in solving problems; - difficulty in using the literature recommended by the teacher; - difficulties in generalizing the individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С- | 1,67 | 60-64 |  | A studentdemonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent solution of problems; - Inadequate possession of individual methods when performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| Д+ | 1,33 | 55-59 |  | A studentdemonstrates: - mastering of separate sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of the calculation of tasks; - significant difficulties in the application of individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - difficulties in correcting the mistakes made by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| Д | 1,0 | 50-54 |  | A studentdemonstrates: - difficulties in mastering individual sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of the calculation of tasks; - inability to apply individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - inability to generalize the individual sections of the material studied; - significant difficulties in correcting the mistakes committed by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| F | 0 | 0-49 | fail | A studentdemonstrates: - ignorance of program material; - when performing all types of tasks, gross errors are allowed; - lack of skills in the use of individual methods of performing task calculations; - failure to complete certain types of tasks, provided by the forms of the current, intermediate and final control. |

Recommended reading:

Mainliterature

- Волков Д.П., Крикун В.Я. Строительные машины и средства малой механизации. - М, Изд. «Академия», 2014. - 480с.

- Белецкий, Б.Ф. Строительные машины и оборудование. - СПб.: Лань, 2012. - 608 c.

- Доценко, А.И. Строительные машины: Учебник для строительных вузов - М.: ИНФРА-М, 2012. - 533 c.

Additionalliterature

1. Строительные, дорожные и коммунальные машины. Оборудование для   производства строительных материалов. Справочник. Том IV- 9. Под общ.ред. И.П. Ксеневича. М., Машиностроение, 2005. - 736 с.

2. Кудрявцев Е.М. Комплексная механизация строительства. М., АСВ, 2005. - 416с.

3. Крикун В.Я. Строительные машины. - М., АСВ, 2005. - 314 с.

4. Домбровский Н.Г. и др. Строительные машины. - М., АСВ, 2002. - 528 с.

Signed to print ____ __________ 2018 y.

Paper size 297x210 1/16

Printing paper.Offset printing.Volume ___ p. l.

Circulation __ _ _ _ copiesOrder № ____*

© M.Auezov South Kazakhstan State University, 2018

A publication of M.Auezov South Kazakhstan State University

Publishing center wksu them. M. Auezova, Shymkent, Tauke Khan Ave., 5

*These values are set by CI