# Ф.7.03-03.

# Baizhanova A.N., Ongarova A.H., Sultanbekova P.S.

PRACTICAL INSTRUCTIONS

of the subjects

«Ecology of water resources»

# Shymkent,  2019

MINISTRY OF THE EDUCATION AND SCIENCES OF THE REPUBLIC OF KAZAKHSTAN

# M. O. AUEZOV SOUTH KAZAKHSTAN REGION UNIVERSITY

# Department «Water resources, land using and agrotechnology»

# Baizhanova A.N., Ongarova A.H., Sultanbekova P.S.

# METHODICAL INSTRUCTIONS

of the subjects «Ecology of water resources»

for students of specialty

5B080500 - Water resources and water use

# For all forms of education

# Shymkent,  2019

# UDC 631.6

Originator: Baizhanova A.N., Ongarova A.H., Sultanbekova P.S. Methodical instructions of the subjects «Ecology of water resources» Shymkent: SKSU of M. Auezova, 2019.

Methodical instructions are made according to requirements of the curriculum and the program of discipline «Ecology of water resources» and include all necessary data on performance of installation works of system of heating, water supply and the sewerage.

In methodical instructions subjects of a practical training in which are painted electro-welding operations and its types, manual gas welding, the equipment of heating systems, mounting of systems of heating and boiler rooms, an economic and drinking water supply system of cold water, the system of the sewerage the building, mounting of sanitary units and cabins, maintenance and repair of sanitary systems, etc. are specified.

Methodical instructions are intended for students of specialty 5B080500 - Water resources and water use.

Reviewer: PhD in Technology Sciences, senior lecturer of SKSU of Department «Ecology» A. Abduova

The methodical instructions practical training has been developed in accordance with the modular training  program on the subject “Ecology of water resources and approved at the meeting of the chair “Water resources, land using and agrotechnology”.

Minutes  No.__ dated from “____” _______201_

The methodical instructions practical training has been approved by the methodical commission of the Higher School of Agricultural Sciences

Minutes  No.____  dated from “_____”  ____________ 201_

The methodical instructions practical training has been approved and endorsed by the Methodical Councilor M. Auezova South-Kazakhstan Region University ______________________201__, minutes №____.

© South-Kazakhstan Region University of M. Auezova, 2019

# CONTENTS

| Introductions ………………………………………………………………........... | 6 |
| --- | --- |
| Practical classes 1 Methods for Environmental studies………………………….. | 7 |
| Practical classes 2 Influence of Environmental factors on the organism and factors…………………………………………………………………………… | 9 |
| Practical classes 3 The main water resources of Kazakhstan. Water quality indicators…………………………………………………………………………. | 11 |
| Practical classes 4 Definition of physical and chemical water quality indicators... | 13 |
| Practical classes 5 Determination of bacteriological and parasitological water quality indicators…………………………………………………………………. | 16 |
| Practical classes 6 Problems of fresh water shortage.............................................. | 17 |
| Practical classes 7 Measures of water pollution prevention and rational use……. | 19 |
| Practical classes 8 Chemicalization of agriculture and its consequences………... | 21 |
| Practical classes 9 Ecological conditions of reservoirs in Kazakhstan…………... | 24 |
| Practical classes 10 Domestic household water..…………………..…………….. | 27 |
| Practical classes 11 Composition and properties of waste water………………… | 27 |
| Practical classes 12 Pollution norms. Instability of the wastewater composition... | 30 |
| Practical classes 13 Methods for determining the irrigation suitability of treated water........................................................................................................................ | 31 |
| Practical classes 14 Determination of the amount of mixing of waste water with river water............................................................................................................... | 34 |
| Practical classes 15 Monitoring. Atmosphere and water, soil pollution control methods…………………………………………………………………………... | 35 |
| Literature................................................................................................................. | 38 |

# Introductions

The provision of clean water to the population, agriculture and irrigation of pastures in rural household and socio-cultural conditions, improving technology and increasing productivity in the industry is one of the most necessary for the development of livestock. As you know, is very useful for health, and clean water.

Currently agronets complexes and rural settlements to ensure quality drinking water "Clean water" program adopted in the design of water supply systems and much attention is paid to construction.

Design and calculation of structures and pipelines, in water supply systems of agriculture envisaged in the guidelines.

# Practical classes №1.

Topic: Methods for Environmental studies.

Environmental science field, laboratory and experimental methods of research. The method of field research – conducted in the field. The study objecter – individual, population, species, associations and other natural.

Description: history of ecology, the basic methods of creating a theoretical system analysis and modeling. And observations, comparative analysis, tarruell (field, laboratory), basic methods of monitoring is empiristic. Comparative analysis of traditional control methods and science. Using this description is done for obtaining the primary data for analysis.

Field studies – studies in populations of different species and their associations in the environment. To know the factors influence on the organism or the population of a certain field of research.

To know the mechanisms of influence of factors on the developing organism of a separate environmental and experimental techniques allows to determine through experience, through. Specificity control experiments in nature – influence on an organism of a particular factor produced in vitro.

Before any research is necessary to define the goals and objectives of the research. This, in turn, choosing research methods, collecting evidence and draw conclusions, as well as to reduce time, save money.

Field research, collecting information for example:

Edaphically, physiological, climatic features of the entry in the field diary:

District________________________________________________

Coordinates____________________________________________

Time__________________________________________________

# 1. The object of study (soil or water) ________________________

A) the features of the surface ______________________________

B) the depth of the floor __________________________________

C) pH ________________________________________________

D) temperature _________________________________________

# 2. Topography

A) the angle of the surface________________________________

B) the sea surface height__________________________________

C) relief ______________________________________________

D) water absorption _____________________________________

E) land use ____________________________________________

F) the presence of high or low water level, the height ___________

# 3. Climate

A) the air temperature ____________________________________

B) the amount of precipitation _____________________________

C) clouds on which (open sun) _____________________________

D) relative humidity _____________________________________

E) wind direction, speed __________________________________

Independent research and quality in the program, in the form of data, information, obtained by other researchers incomprehensible and knows. Therefore, this information is systematically provided:

1. Introduction: the main idea, problem, hypothesis and aim of the study, are discussed.

2. Method: the programme strategy (as) where as when use in the laboratory or in the field, the methods and features of the equipment.

3. Observations and results: table, graph, bar graph, chart, in the form of visual AIDS or any other data and information.

4. Discussion of results: the results (especially quantitative) analysis, the conclusions made based on published materials and information in the same former.

5. The importance of the discussion conclusions: when applying the evaluation methods, analysis of errors, suggestions for future research.

6. The list of references.

# The task:

Research object, select the object:

1) collection of Information lgseta description do.

2) in accordance with the programme and quality of Studies, information received in the form of data, information, show systematically.

# Control questions:

1. Orshan Orta Turala alumni damuna ser etken Andy elected ATAP toge Baladi?

2. Ecology ilimy nen serted and Andy bildenden trade?

3. Ecology alimini masati me mnde Andy?

4. Ecologially, Pavlodar destine of sipalay ber?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №2

Theme: Influence of Environmental factors on the organism and factors.

Components, conditions affecting the body of any middle or secondary are called environmental factors. Environmental factors in the life of living organisms, the number (moldyn), geographic distribution, directly or indirectly affect.

Different in nature and effect on living organisms environmental factors. Traditional classification: abiotic, biotic and anthropogenic (entropically) identifies the factors.

Ecological factor

Biotic factor                        Abiotic factor           Anthropogenic factor

Abiotic factors - are a complex of environmental conditions that affect the living organism (temperature, pressure, radiation background, humidity, atmosphere, sea and freshwater, soil composition, etc.).

Biotic factors - are the effects of life on the organisms of another organism (competition, predisposition, parasitism, etc.).

Anthropogenic factors - a set of environmental impacts of human activity (emissions of harmful substances into the atmosphere, destruction of soil layer, natural landscapes, etc.).

This classification is conditional. The reason is that temperature, as an abiotic factor, varies widely, depending on the behavior of living organisms. For example, when the air temperature drops below + 13 ° C, the moving activity of bees increases, which increases the temperature in the hole to 25-300 ° C.

In addition to the above classifications, there is a classification based on an assessment of the degree of adaptation of organisms to the effects of environmental factors (AS Monchadskii). According to this classification, all environmental factors are divided into three groups.

Ecological factor

First-order periodicity           Second-order periodicity             Without period

First of all, adaptation to periodic factors occurs in living organisms. For example, the direct rotation of the Earth and rotation of the Sun or the moon phase transitions. The cycles of persistent repetition of these factors existed before the existence of the Earth, so adaptation of organisms to these first-order periodic factors has already been established on the basis of their genetic heredity.

Factors with temperature, light, ripeness, and return factors play a significant role in many living environments. Only in the depths of the deepest ocean - in the abyssal zone, the change of the first order is insignificant. In experimental studies, it is necessary to take into account that the behavior of animals in the laboratory at regular temperature or light differs from natural conditions.

Changes in the second order periodic factors are the result of changes in primary factors. For example, air humidity is the second temperature dependent factor.

The amount of dissolved oxygen, dissolved salts for the aqueous medium, the degree of cloudiness, the flow velocity, and so on. second order periodic factors, their first order periodicity, weak dependence on factors. It has not been long before the organism has adapted to the second-order periodic factors, so the part of their adaptation can not be achieved. Examples of non-periodic factors include climatic factors - hurricanes, lightnings, fires, and human activity.

The parasite effect of the owner must be classified as secondary seasonal factors. The reason is that as a parasite habitat, the body of the owner is a normal habitat.

# The task:

Fill the table:

-  Write the following examples on the 2nd column of the table.

- Write your answer to the 3-digit column on each side of the example and write down the suggestion on the 4th column.

| The effects of human activity in nature | Examples | What kind of changes will occur in natural ecosystems? | What kind of offer do you have to improve the environmental situation? |
| --- | --- | --- | --- |
| Reversing |  |  |  |
| Irreversible |  |  |  |

1) Spontaneous generation of household wastes.

2) Extensive cultivation of agricultural crops (wheat, rice, maize, etc.).

3) cutting of forests for cultivation of crops or for carrying out construction works.

4) Air and water contamination by emission of nitrogen into atmosphere, sulfur oxide.

5) Application of pesticides.

6) Filling of polluted water with household organic substances in water tank.

7) Debris drainage or artificial reservoirs construction.

# Control questions:
1. Explain the abiotic factors?

2. Characteristics of biotic factors?

3. What is the impact of anthropogenic factors?

4. The classification of environmental factors.

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №3

Theme: The main water resources of Kazakhstan. Water quality indicators.

Water - is one of the most important natural resources, chemical compounds of hydrogen and oxygen. The simple formula of the water vapor is H2O. The molecule of liquid water is derived from the addition of two simple molecules (H2O) 2. Ice - consists of three simple molecules - (H2O) 3. Water is the only substance in the nature of all three aggregates: liquid, solid and gaseous.

The accumulation and movement of water in its various phase states are clouds, rivers, lakes, waterfalls, snow-covered forests, ice avalanches, marine wave fences, aesthetic resources of the Earth's planet.

Water resources:

1 - rivers, lakes, ponds, reservoirs, surface water resources, etc.

2 - Groundwater

3 - Glaciers

4 - The waters of the Caspian and Aral seas on the outskirts of Kazakhstan.

Water resources are systematic deployment of productive forces, and are often regarded as one of the most important factors of production. Rational and efficient use of water resources is very important for Kazakhstan. The level of water supply in the Republic is about 30 thousand cubic meters per square meter. This figure is 4 times lower than that of Ukraine, 3 times more than Russia, and 30 times less than in Georgia.

Water supply in Kazakhstan is unevenly distributed across the regions. Only East Kazakhstan Oblast is well supplied with water (290 thousand m3 / 1 km2). Atyrau, Kyzylorda and Mangistau oblasts are insufficiently supplied with water. Data on natural water resources in Kazakhstan are presented in Table 1.

Table 1. The main water resources of Kazakhstan.

| Source of water resources | Description Total number | Area and size |
| --- | --- | --- |
| Sea: Caspiy, Aral | 2 | 371000 km2 (with length of 1200 km, width of 320 km) |
| Lakes: Balkhash, Alakol Sea, Sasykkol | 50000, including in North Kazakhstan - 21500 and so on. 17500 in Central and South Kazakhstan | 15.6 thousand km2 4.6 km2 |
| Rivers: Irtysh (4500 km in Kazakhstan - 1,700 km) Syr Darya (3000 km) Uralsk (2500 km) Esil, Tobol, Ile, Shu | >7000 | Lengths > 1000 km |
| Sarysu, Nura, Turgai, Emba, Sagiz, Ilek, Bolskozen and others. |  | Lengths > 500 km |
| Groundwater |  | 7.5 trillion. m3 |
| Glaciers | 1500 | 2000 km2 (120 billion cubic meters) |
| Thermal water (30-1000 ° C) |  | 8000 thousand km2 |
| Pond and water tank | >4000 |  |

Water quality for drinking water is determined by GOST (GOST 2874-82).

The quality of natural water is determined by physical (organoleptic), chemical and bacteriological parameters. They determine the quality of water content required for different consumers.

Organoleptic (physical) properties are determined by the sensitivity or transparency of the sensory organs, the sense of taste and smell, the temperature.

The surface water sources are divided into the following types: low content of water - 50 mg / l, average 50-250 mg / l, sediments - 250-2500 mg / l; very susceptible - contains more than 2500 mg / l of sediment.

Water used as a drinking water standard may contain up to 1.5% of sediment. The water's transparency depends on its ingredients and organic matter. The viscosity of the liquid (by cross) and sedimentation is as follows:

| Transparency | 4 |  | 10 | 20 | 30 | 50 | 100 | 200 | 300 | 500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Depression, mg / l | 235 | 185 | 92 | 45 | 30 | 18,2 | 9,1 | 4,6 | 3,1 | 1,3 |

The color of water is determined by the comparison with the color standard of natural water - platinum and cobalt saline solution. Water color is calculated by the degree of platinum-cobalt scale. Due to the natural water intensity coloring, less painted - 350 and painted - more than 350. The purity of drinking water should not exceed 200 by standard.

Taste and smell of water. The dissolution of various substances in water and the existence of living organisms in it affects its taste and smell. Determine them by 5 points at a temperature of 200 ° C. By standard, "Drinking water" should not exceed 2 points. Specific aroma and taste water should not exceed 1 point after chlorination.

Water temperature. "Drinking water" and the amount of water required for cultural life should not exceed 8-120 C.

# The task:Write down the requirements for the water used in agriculture and the requirements for water used as "Drinking Water" in tables.

# Control questions:

1. Are the main water resources of Kazakhstan characterized?

2. What are the water quality indicators?

3. How does it determine the organoleptic properties of water?

4. What are the types of water on the surface of the earth's water distribution?

5. How is water transparency determined?

6. How is water color determined?

7. How is the taste and smell of water determined?

8. What is the water temperature?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №4

Theme: Definition of physical and chemical water quality indicators.

The quality of natural water is determined by physical (organoleptic), chemical and bacteriological parameters. They determine the quality of water content required for different consumers.

Dry residue - total chemical composition of water in the solution. According to the "Drinking Water" standard weighing up to 1000 mg / l. Only in some cases it is allowed to use water with a mineral content of up to 1500 mg / l.

Water hardness. The water hardness is expressed in milligrams of calcium and magnesium ions, which are 1 liter of water.

The arc is divided into time and constant.

In temporary flakes there are bicarbonates of calcium and magnesium, when boiling water, these salts become slowly soluble carbonates and precipitate.

The difference between the common and purified stiffness corresponds to constant hardness. When boiling fermented water, the walls of the steam boilers are thick. Therefore, the thermal conductivity of the boilers is reduced and the fuel consumption increases, the thickness of the skulls is affected.

The standard (temporary and permanent) water hardness should not exceed 7 mg /ekv when used in household and drinking water. In some cases, only 10 mg / well or more water is used.

According to the state standard, the correct (impressive) response rate should be pH - 6.5-8.5:

Depending on the level of pH, water can be classified into several groups:

| Indicator | рН the meaning |
| --- | --- |
| Strong acid waters | < 3 |
| Acidic waters | 3 - 5 |
| Slight acidic waters | 5 – 6,5 |
| Neutral waters | 6,5 – 7,5 |
| Low alkaline waters | 7,5 – 8,5 |
| Alkaline waters | 8,5 – 9,5 |
| Strong alkaline waters | > 9,5 |

- drinking water should not exceed 350 mg / l of chloride (salt of hydrochloric acid)

- sulfates (sulfuric acid salt) - 500 mg / l,

- iron 0.3 mg / l (in underground waters up to 1 mg / l),

- zinc - 5 mg / l;

fluorine is between 0.7-1.5 mg / l (if it exceeds 1.5 mg / l, it can cause health damage).

Natural water is divided into the following categories depending on its mineralogy:

| Water category | Minerality, g / dm3 |
| --- | --- |
| Ultra fresh (ultraviolet) water | < 0,2 |
| Freshwater waters | 0,2 - 0,5 |
| Relatively high mineral water | 0,5 -1,0 |
| Salinity (solonoid) waters | 1,0 – 3,0 |
| Salt (solenoid) | 3 - 10 |
| High salinity water | 10 - 35 |
| Salty water | > 35 |

If the content of iodine in water is higher (0,0001 mg / l), endemic goiter (sore throat), underground water will contain sulfur hydrogen (H2S), a smell of water and corrosion of pipes made of iron.

If the quality of natural water does not meet the requirements of drinking water, the quality of natural water purifies, submits them, purifies and purifies the painted species.

Task:

1. If the pH of the river is 7.7, can it be used in agriculture, your offer?

2. If a tanker's crash on the coast has resulted in thousands of tonnes of crude oil offshore, then the fish will die in oil at a concentration of 15 mg / l. Taking this into consideration, the fish count the amount of water that was killed. (V = M / C)

| Option | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Мoil | 230 | 210 | 300 | 228 | 189 | 168 | 199 | 201 | 209 | 266 | 286 |

3. Thousands of tons of oil were pumped to the sea during the tanker explosion on the shore. If the oil density (p) is 800 kg / m3, and the marine surface of the oil film is L mm, how much water (S) is polluted with an oil film?

(V = M / P S = V / L)

| Option | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Мoil | 100 | 110 | 98 | 116 | 150 | 144 | 132 | 122 | 153 | 96 | 89 |
| L | 3 | 2 | 2,6 | 3,2 | 2,1 | 3 | 2,5 | 2,4 | 2,8 | 1,6 | 1,7 |

# Control questions:

1. What is a dry residue?

2. Explain the concept of water hardness?

3. What is the total water hardness?

4. What is the pH indicator?

5. What is mineral water?

6. How to classify water depending on the level of pH?

7. What categories of natural water are divided by mineral?

8. What if the quality of natural water does not meet drinking water requirements according to GOST?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №5

Theme: Determination of bacteriological and parasitological water quality indicators.

Bacterial dehydration of water. Water contamination is expressed by the total number of bacteria, as well as by the presence of 1 liter of water in the colibaceria (intestinal tubes) or coli-index. Sticks in a single intestine (cola index) are called water in the water.

According to the standard, the total number of bacteria can not be more than 100 ml in 1 ml of water. The intestinal stick should not be more than 3 liters in water (or at least 300 ml).

Radioactivity of water depends on the presence of all types of radioactive elements and their mixtures (radium, uranium, thorium, potassium-40, radon, etc.) in all rocks.

Compared with underground water, the natural radioactivity in surface water is very small.

Radius in the water - 2 * 10-13 ÷ 4 * 10-12 g / l, and uranium - from 2 * 10-8 to 4 * 10-13 g / l. Lakes and reservoirs in the southern regions are highly radioactive, for example uranium 4 * 10-2 g / l.

The maximum concentration of radioactive substances in open water reservoirs shall be: radius - 226 ÷ 1011 curie / l and the like.

Other toxic chemicals in water should not exceed the following limit (mg / l): beryllium (Ve) - 2 * 10-3; molybdenum (Mo) - 5 * 10-1; arsenic - 5 * 10-2; selenium - 1 * 10-3; Lead - 1 * 10-1; strontium - 2; polyacrylamide - 2.0; hexamethophosphate - 3.5; tripolyphosphate - 3.5.

Task:

In order to solve the current environmental problems, please fill in the table by selecting one of the suggestions. What solution was chosen on each issue? In what case can there be several solutions?

| № | Issues | Offers |
| --- | --- | --- |
| 1 | Pathogenic bacterial diseases |  |
| 2 | Pollution of water with products of livestock farms |  |
| 3 | Water contamination with water from industrial plants |  |
| 4 | Pollution of yards and streets |  |
| 5 | Pollution of surface water |  |

Ready solutions of environmental issues:

1) Adoption of the law

2) Personal responsibility of the perpetrator and payment of penalty

3) Introduction of local taxes

4) Administrative Decree and Authority's Assistance

5) Ecological education and upbringing

# Control questions:

1. Bacteriological water quality indicators?

2. Parasitological water quality indicators?

3. Indicate the chemicals in water content?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №6

Theme: Problems of fresh water shortage.

Water is one of the most important natural resources, chemical compounds of hydrogen and oxygen. The simple formula of the water vapor is H2O. The molecule of liquid water is derived from the addition of two simple molecules (H2O) 2. Ice - consists of three simple molecules - (H2O) 3. Water is the only substance in the nature of all three aggregates: liquid, solid and gaseous.

The hydrosphere is one of the creatures of dead matter, but life on Earth is closely associated with it. There is also a living being with water. Water is an important part of any living cell. Biochemical reactions take place in the water because the viscosity of the organic compounds of biologically diverse species is dissolved in water. At the dawn of life, volatile organic compounds dissolve and dissolve in the atmosphere. Substances that did not dissolve in water, and water-soluble substances prevailed in the deep, they continued to evolve into life.

Water occupies more than 70% of the Earth's surface and 98.2% of the hydrosphere is contained in the World Ocean waters, 1.6% in ice shells, and only 0.1% in waters.

In general, Kazakhstan's water resources are insufficient, and then they are concentrated in the outskirts of the state. There are no permanent river basins in most central and western regions of Kazakhstan.

Kazakhstan's industry consists of fuel, metallurgy, machinery, metalworking, refining, petrochemicals, building materials, chemicals, light and food industries. Basic agricultural: cereals, wool and meat production. Irrigated farming in southern and eastern-eastern regions has flourished. There are rice fields (Ile, Syr Darya). Recently, irrigated farming is concentrated in eastern and cooler zones.

Major Water Management Issues: Providing water supply to industrial units, especially in the central and southern regions. Satisfaction of irrigated land needs in the southern and southeastern regions; to avoid contamination of natural water sources as the water sources are polluted by industrial wastewater (especially in eastern, central and south-eastern areas) and mesh waste water reservoirs (south and south-eastern areas). The main problem of water management in the southern, central and south-eastern regions is the protection of natural water sources (Talas, Asa, Chu, Syr Darya, etc.), especially economic, social and environmental issues in the Aral Sea and Balkhash lake - Water and Aral Sea Aral Sea Problems, discussed in Chapter 1.5) - The main water management problem in mountainous areas is the fight against floods (protection of Almaty from mud flows) have been resolved).

Watering irrigation areas in the central and western regions of Kazakhstan is a major water resource issue. Problems of pasture irrigation are mainly solved by using groundwater.

Complex watercolor. Suction (group of water structures) is a complex of general and special water-technical constructions that are combined depending on location conditions and functions.

Hydraulic structures, which are part of the watercourse, are separated into main (general and special) and auxiliary buildings. The auxiliary structures include residential, administrative, economic, housing, water and sewerage, roads and many others.

The main building blocks of the complex are usually divided into three groups. They are: shared (or shared), branch (or special) and partner (accompanying).

The general (general) assemblies are the following: all constructions of the CEC are required for construction of water structures (structures): barriers, housings; reservoirs and water regulating facilities: irrigation, disposal, etc.).

Industry (special) structures are the only hydro power stations (hydroelectric power plants); Ship conductors: gate gates, hoists (water-borne transport); fish, conductors; watering (melioration irrigation); (water supply) and so on.

Partnerships are considered to be merely one of the industries required for water supply (hydroelectric power stations); Shipbuilding: Shipyards, boats (water transport); fishery, fishing (fishery); watering (melioration irrigation); trunk and irrigation canals (water supply) and so on.

General facilities and industrial facilities are located in the water intake zone, which are integrated into a water intake unit.

Water node concentration:

- Depending on the purpose: energy, sewer, filtering, irrigation, irrigation (leakage regulation);

- Depending on location: river, canal, ocean, lake or pond;

- Depending on the nature of the tailgate (if there are any constructions), without tails;

- Depending on the scale of water supply: intermittent, integrated water networks.

Task:

Select a water scarcity zone where you can create a site for integrated water resources and show them in the scheme.

# Control questions:

1. Characteristics of water?

2. What is the problem of water deficit?

3. What is the layer of hydrosphere?

4. Describe the problem of desertification?

5. Explain the increase in water consumption for household and industrial purposes?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №7

Theme: Measures of water pollution prevention and rational use.

The diversity of water accumulation and movement in its various phase states are clouds, rivers, lakes, waterfalls, snow-covered forests, ice avalanches, marine wave fences, aesthetic resources of the Earth's planet.

Water resources:

1 - rivers, lakes, ponds, reservoirs, surface water resources, etc.

2 - Groundwater

3 - Glaciers

4 - The waters of the Caspian and Aral seas on the outskirts of Kazakhstan.

The main two ways of water use in the national economy are water use and water consumption. In the first case, the water is not taken away from waterfalls and used to solve economic needs. Hydropower, water transport, agricultural and fishery, etc., will be used by water users. Water consumption is removed from the water, some of the water is used for technological processes, and the rest is lost irrevocably. There are two water supply systems in our country:

1 - Directly flowing.

2 - Swinging around.

Rational and efficient use of water resources is very important for Kazakhstan. The level of water supply in the Republic is about 30 thousand cubic meters per square meter. This figure is 4 times lower than that of Ukraine, 3 times more than Russia, and 30 times less than in Georgia.

Water supply in the republic is different in different regions. For example, mountainous areas range from 24 to 212,000 cubic meters per square meter per square meter, and from 2,3 to 4,700 cubic meters in the plains. The main sources of water resources in the country are lakes, with an average annual flow of about 100 cubic km. and more than half of it comes from China, Uzbekistan, Kyrgyzstan, and Russia. At the beginning of 1997 the reserves of underground water in the Republic amounted to 15824.1 mln.cubic meters.

In recent years, the country has started to focus on the use and protection of water resources. Basic laws, structures, placement, chemical structure of underground water were investigated. Forecast maps of natural and operational resources are created. Scientifically justified proposals for the use and protection of water resources in various sectors of the national economy have been prepared.

However, the existing water resources in the Republic are still not used rationally. Use of groundwater in the national economy is 34-40%, and the use of groundwater does not exceed 5% -10%. The most difficult situation with water is in the Syr Darya, Ural, Ili, Shu and Talas rivers with other republics. Only in the northern Kazakhstan in the last 10-15 years, the rivers have fallen by 10 to 12%.

The use of water resources is closely linked to agricultural production. Water and agriculture will only allow for rational use of water, and water economy is a water resource research, accounting, planning, forecasting, protection of groundwater from pollution, supply to water consumers. p. one of the branches of national economy.

Many factors influence the economic level of water use. Here, first of all, the technical level of irrigation systems, irrigation techniques, irrigation mode, etc. can be called. Works on improvement of reclamation condition of irrigated lands in the Republic are underway, and implementation of complex measures to improve melioration systems. To do this, old systems must be rebuilt, new systems must be partially upgraded and equipped with additional equipment.

At present, the degree of contamination of our internal waters is likely to be very dangerous. The main ways of contamination of water:

1. Fertilizing land with toxic chemicals, flooding of residual water from reclamation into rivers and lakes. This is especially true in rice fields and cotton plantations.

2. Production remainder from the use of the field.

3. Contaminated water of oil production.

Wood wastes in the manufacture, processing and transportation of wooden materials.

5. Wastes of water and railways.

6. Waste of primary processing of technical plants.

This is one of the most dangerous sources of contamination of water resources, which is the residual waste water containing chemical toxic substances that are harmful to fish. It includes: pulp and paper factories, chemical plants, refineries, metallurgical factories, livestock products - wool, skin processing plants, etc. belonging to the Only non-ferrous metallurgy enterprises of Kazakhstan send 168.8 thousand cubic meters of contaminated water to rivers and lakes per day.

Oil, phenols, pesticides, and non-ferrous metals are particularly hazardous to water resources.

Especially, oil pollution is infinite. Because it contains many harmful things. Due to the development of new oil and gas sources, oil is not only water but also the most dangerous thing that pollutes the whole environment.

Task:

1. Make recommendations for solving the global warming problem.

2. Demonstrate protection against the occurrence of acid rain and its adverse effects

# Control questions:

1. What is the production water supply system?

2. Indicate the main ways of contaminating water?

3. Water resources and parenting?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №8

Theme: Chemicalization of agriculture and its consequences.

Agricultural chemistry. In the second half of the twentieth century, the agricultural lands were gradually reducing their quality due to the continuous turnover. These environmental conditions have become massive in many countries around the world. According to the statistics, 11.2% of all children who die every year are physically and mentally unhealthy. Of the 100,000 children born every 11-12,000 have suffered from cancer.

Support for chemistry in agriculture reached its peak in 1960-1990. If in the 1960s there were 5 kilograms of chemicals per person, and in the 1990s it was 35 kg. This indicates a dramatic increase in the use of chemicals. That is, the rate of illness rose from 0.74% in 1960 to 30% in 1990. In the 1990-1999, the use of chemistry in agriculture began to decline in our country.

The use of pesticides in the 1990s covered 20.9 million hectares of land in Kazakhstan, and in 1999 its volume decreased by 8.0 million hectares. Pesticides - all the chemical compounds used to protect cultivated plants from pests, parasites, weeds, diseases and microorganisms. The name of the pesticides is derived from the Latin alphabet; pesis - an infectious disease, caedo - killing. The use of pesticides reaches 18-20% of agricultural products. Nowadays, they are widely used, and the biosphere and humans are harmful.

They do not have the opportunity to buy them without using them because the pests are very high. Amputee cereals are quickly adapted to one type of pesticide and have the ability to give it the ability to produce it. Therefore, when using pesticides, it is necessary to choose the type of the pest.

In the chemical composition, pesticides are grouped into 5 classes:

1) chlororganic compounds - hexachlorane, DDT (hexachlorobluorocyclohexane, polychlorophene, polychlorcamphene, etc.). They can accumulate in the body, and the decay lasts for several decades. Chlororganic compounds are combined with dioxins to form stable organic compounds.

2) Phosphoric insecticides - carbophos, dihlophos, diazinon, phosphamide, metaphosus, amophos, regulators of plant growth and others. They break down rapidly in the soil and in other environments.

3) Carbamate insecticides - carbamic acid esters (Sevin). It only affects some species of caterpillars, and it does not harm animals and humans.

4) Chlorophenoxyxysulate derivatives are used as defoliants for the destruction of plants growing in reservoirs.

5) Pyrrhotoidal pesticides - trans-chrysanthemumic acid. This is a new type of insecticide that has been isolated from natural materials.

Here are some pesticides that are not harmful to humans. They are: derivatives of hydrochino I, triazines (II) and azoles (III), benzoic acid (IV) and others. The effects of pesticides on living organisms depend on the stability of the drug, its persistence and the rate of metabolic processes occurring in the body. Let's talk about the mechanism of decomposition of pesticides. The pesticides entering the environment are split into abiotic and biotic ways. The first one is photochemical, oxidation - reduction reaction and hydrolysis, the second - decomposes under the influence of enzymes. Biotic decay is faster than abiotic. The rate of decomposition depends on the concentration of enzymes and the number of microorganisms that can break down the pesticides. In agriculture, biological treatment of plant protection is sought rather than using pesticides, and pesticides are intended to produce unused environmentally friendly products. It should be used only if the pesticides are harmful to the human body, and that they are polluting the environment and polluting the environment.

Pesticides are divided into hygienic, chemical and industrial depending on their chemical nature and pathogenic properties. In the hygienic classification, the toxicity of the pesticide and its circulatory cycle are taken into account. The level of danger depends on the strength, stability, and absorption of the wound. Hereditary force is determined by its dose. The body weight is about mg / kg, mg / l or mg / m. The severity of the drug depends on the strength of the organisms. Pesticides are classified as strong, moderate, and hazardous according to their elderity. Pesticides are exposed to organisms depending on their purpose of action. For example, the organism acts through the external environment, nutrition, and respiration. When classifying pesticides, they are classified as moderate (1-6 months) and unstable (1 month), with very stable (decay-free period of 2 years) displacement (0.5-1 years).

Depending on the nature of pesticides entering the body - intestinal - contact, contact, fumigation and systemic effects. In the first, they kill the malignant by entering the body and the outer skin of the body is damaged during contact. Fumigant toxins kill the respiratory tract and, in the course of the nervous system, suffocate the entire organism. Cinccticides and acaricides are commonly used. Most are phosphororganic compounds (48%), followed by carboxylic acids (14%) and chlorine-organic compounds. All of them are used against grains, wheat, potatoes, fruits and vegetables. These chemicals are very dangerous for the environment, living organisms and the human body. They are accumulated in food and are long-term trades. And some species decompose in the natural environment. Especially thiophys is very dangerous. It was banned from banning the use of CIS countries. Carbamine products include Sevin, Circus, Zinet, and others.

Task:

1. Make a classification of pesticides. Show the need and the harmfulness in tables.

2. Analyze the use of pesticides by chemical composition in scheme.

# Control questions:

1. Classification of pesticides in relation to the subject and chemical composition of the subject?

2. Benefits and loss of chemicals for agriculture?

3. What is erosion?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №9

Theme: Ecological conditions of reservoirs in Kazakhstan.

At present, large closed water reservoirs such as the Aral Sea, Lake Balkhash are anthropogenic. As a result, the Ural is polluted, its levels decline, and eventually remains a single natural complex. This leads to fishing, climate change. This is an example of the Aral Sea.

The extent to which the Aral Sea was affected was 1066 km2, with a depth of 30-60 meters and salinity of 10-12%. At that time 50 to 150 thousand tons of fish were seized and a large amount of seagrass was seawater.

Residents of the Aral Sea region were socially-economically well-equipped until the 1970s, and 17 fish farms, 10 fish processing plants and 2 fisheries factories worked in the marine region.

In the 1960s, the development of the Aral Sea region was started. The area of ​​land in this region has increased 1.5 times from Uzbekistan and Tajikistan, Turkmenistan 2.4 times, and Kazakhstan 1.7 times. The land area of ​​1971-75 is increased to 1.1 million hectares in Fergana valley, 427 thousand in South Kazakhstan oblast and 228 thousand hectares in Kyzylorda region. The waters of the Syr Darya River were regulated by the reservoirs of Karakum, Shardara, Shapbak and Andijan in 1950-70. The population of Amudarya and Syr Darya increased 2.2 times between 1960 and 1982. With the increase in the area of ​​cultivation, the need for water increased with the increase in the population.

In this regard, the amount of water discharged into the Aral Sea has declined over the 1970-80s. Rice and cotton growing in the two rivers have grown rapidly. In the 1960-65 years, Aral Sea flow was 44,000 m3 per year, and in the 1990s it dropped twice. As a result, the sea level dropped to some 23 meters in some places, and the water surface declined by 30-200 km. The water salinity increased by 40%, and the use of fertilizers and chemicals in farms on both rivers has never reached its peak. Use of fertilizers increased 10-15 times. Such anthropogenic factors have caused an ecological catastrophe from the Aral Sea region. Every year, 2 million tons of salty dust are raised from the dried seabed.

Factors causing the Aral Sea disaster:

- ignore historical and natural features of local area;

- poor agricultural planning, non-accounting of water resources;

- To increase the rice and cotton crops, which require high water content;

- non-observance of agro technical measures for land development and water use;

- The reasons for the errors in the use of natural resources and the scientific justification of its acquisition.

Under the influence of the current ecological catastrophes in the Aral Sea region, its flora and fauna are close to disappearing. Soil salinization is very fast. Soil salinization in Uzbekistan grows by 60%, in Kazakhstan - by 60-70%. This is a major threat to the overall economy. The fishery in the Aral Sea has been restored only in recent years.

The health of the population in the Aral Sea has dropped dramatically. In this region, tuberculosis, kidney stones, serum, inflammation of the lungs, infectious diseases are higher than in other regions of the country.

There are several scientific forecasts and projects on rescue of the Aral Sea. They:

1. Turn the Siberian rivers to Kazakhstan.

2. Water reproduction by regulating Amudarya and Syrdarya rivers.

3. Partial preservation of the Arar sea.

4. Bring water from the Caspian Sea through artificial canals.

5. Use of Groundwater.

6. Waiting for the self-regulation or expansion of the Aral Sea.

Lake Balkhash - 501 thousand km², length - 605 km, width - 9-74 km. Balkhash is flowing to the Ili, Karatal, Aksu and Lepsy rivers. Because the lake is located in a desert and desert region, the water evaporation is very high. That's why the water level is changing rapidly. In recent years the ecological situation of Balkhash lake has begun to change. The reason is that the abundance of water from the rivers Karatal, Aksu, and Lepsi does not reach the lake and is used for irrigated fields. Moreover, previously Ayagoz, Biann, Sarkand, Baskan rivers poured into Balkhash and preserved its natural water level. And now these rivers do not pour into Balkhash.

For these reasons, due to the construction of the Kapshagay water reservoir built on the Ili River, the Balkhash lake ecosystem was blocked by the Bartakay Strait of the Shelek River, feeding the Ili river, with a capacity of 300,000 m3 of water. Thus, the Big Almaty canal was built. Due to the construction of the canal, the riverbed of Shelek stopped flooding.

The volume of water inflow to Balkhash has decreased by 25% in 1965-90 with maximum use of the Ili-Balkhash basin in agriculture. In the middle and lower streams of the Ili cuneiform Charyn rice, Sage vegetable, Akdala rice fields. All of this has infringed the equilibrium in the natural system of the Ili-Balkhash basin.

Changes in the Ili-Balkhash ecosystems resulted in the destruction of the Ili River, very rare in the vicinity of the glacial pastures on the riverbank and its reedbed reefs. The coast of the lake was dried up and salt dirt was rising. The air changes, droughts and wind blows.

The Balkhash lake has decreased in recent years by 20 meters. The coast of the lake is swamped, sourced and turned into ants and deserts. The vegetation and wildlife of the Ili - Balkhash region also suffer. Fishing has dropped 50 times, and spawning has even diminished. At the same time, the pesticides and mineral fertilizers that are used by the fish in the fields of slaughtered vegetables are also poisoned.

Ili - Balkhash arbor is rich with historical and natural monuments in Kazakhstan. In the region there are sturgeons on the Charyn River and its stones in the mountains of Charyn, stone sculptures and ancient burial mounds, the nature of the singer's sand, Altynemel national park, and the Kremlin.

Large environmental issues in the region include Balkhashmys Combine, Baltic Lake, Priozer, Akyek ore concentrates. The Saryshash polygon and Tekeli lead-zinc plant have a serious impact on the health of the residents of these regions.

The international environmental forum "Salvation of Lake Balkhash, its present and future" was held in 1999, where the issues of rescue of the lake were discussed and the following decisions were made:

1. Regulation of efficient water use at the Ili River.

2. Stabilization of water share from Kapshagay water reservoir to Balkhash.

3. Reduction of rice fields in Akdala and Charyn arrays.

4. Underground water use.

5. Restricting the extent of irrigated lands.

The Caspian Sea is the only waterfall in the world. Its size is 380 thousand km². The Caspian Sea covers 1030 km from north to south and 196-435 km from west to east.

Here are a number of plants and animals that are included in the "red book". As a closed pond, most of the organisms here are endemic. One of the most valuable treasures of the sea is sturgeon fishes. There are five species of these species. About 70% of the world sturgeon stocks account for this sea. The Caspian Depression is distinguished by its natural monuments, flora and fauna diversity. There are 945 plants, 64 algae, 414 of phytoplankton, over 100 species of zooplanktones.

The Caspian Sea is also rich in professional fish. 76 species of fish in the sea, 3-3.5 million birds over the coast, and 10-12 million birds per year. In recent years, the environmental situation in the Caspian Sea has been tied to sea level contamination by increasing the water level and by producing more oil products.

The rise or fall of the Caspian Sea is dependent on the natural and historical evolution of the land. Its causes can be explained by weather phenomena and anthropogenic factors.

The Volga River is about 80% of all water flowing into the sea. Therefore, sea water is closely related to the Volga River. In recent years, the sea has not been separated from the Gulf of Karabuzakol, and the inclusion of human beings into the Caspian ecosystems has been a major factor. Humanity then understands that rhythmic change of sea water is a natural phenomenon. From 1979 to 1980, the sea level began to rise independently. At the end of the 20th century, its level reached about 3 meters. More than a hundred oil wells, oil storages, oil and gas complexes, hundreds of settlements, communication networks, and industrial facilities were flooded by the rise of water. Oil and other pollutants were mixed with sea water, reaching 300,000 hectares.

The rise of the Caspian Sea causes many problems for the neighboring states. First, the rise in water levels has diminished the size of the natural pasture on the coast, and has left the birds and animals alive. Barriers damaged the water to the coastal fauna and flora. The spawning grounds of fish and the nesting density of the birds cause the problem of biological diversity diminution.

Secondly, elevated water levels lead to air pollution and soil pollution. Pollution of water with oil products has grown since the 80s. Increased water content of pesticides and heavy metals has led to the fact that sturgeon precious fish, sea squirrels, birds have been exposed to massive diseases and died. According to recent data, sturgeon fishing has decreased by 40%.

Thirdly, the rise of water began to cause huge damage to oil wells on the seaside, oil storages, oil refineries, fisheries factories and settlements.

Fourthly, attraction of foreign investors to oil production further aggravates the ecology of the Caspian Sea. Depreciation of the sea shores makes the technogenic processes faster and makes the region a desert. The main thing is to preserve the diversity of fauna and flora of the Caspian Sea. The Caspian problem is at the UN level. It is a problem not only of Kazakhstan, but also of Russia, Azerbaijan, Turkestan and Iran.

# Control questions:

1. To analyze the ecological problem of the Aral Sea and organize a rescue operation?

2. Discusses the ecological problem of the Balkhash Sea?

3. The nature of the Caspian Sea ecological problem?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №10, 11

Theme: Domestic household water. Composition and properties of wastewater.

Wastewater is divided into household, livestock, industrial and mixed species depending on its origin.

Domestic wastewater includes households (hospitals, dining rooms, baths) and dwellings. They contain various plant elements (nitrogen, phosphorus, potassium, etc.), and bacteria and helminth eggs. The chemical composition of this type of water is chloride-sulphate and bicarbonate-sulphate. If you keep them in accordance with sanitary regulations, the groundwater can be fully used. An average of 40-60 mg of nitrogen in the 1 liter household water, 10-20 mg of phosphorus, 10-30 mg of potassium and 50-80 mg of calcium. The concentration of dissolved mineral substances is 0.5-2 mg / l, and the amount of organic matter is about 200 mg / l. Another advantage of this water is that it contains no soda. This type of water can be widely used for irrigation of agricultural crops.

Cattle breeding water is generated when hydraulic culling of the stock is carried out. Due to the high content of such organic matter, it can be used as a good fertilizer. At the same time, they are very rich in nutrients needed for the plant. In particular, there is enough organic matter to increase soil fertility. In most cases this will reduce the concentration of nutrients contained in water. He adds pure water to him. These include helminths that cause disease. Therefore, livestock breeding water is also pre-treated for field irrigation.

The composition of the drain waters generated at the industrial enterprises is unstable. These chemicals are mainly based on the technology of industrial plants and the type of raw materials used. In some cases, they contain substances that cause damage to plants and livestock, so they should be checked before they can be used for irrigation.

In the case of irrigation of agricultural crops, the drinking water that is produced by the food industry has a great potential. The chemical composition of these waters is different.

Table 1. Chemical composition of drain waters of food industry enterprises.

| Enterprises | Consumption size О2, mg / l | Nitrogen, mg / l | РО, mg / l | КО, mg / l | СаО, mg / l | рН | CPC, mg / l |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dairy plants | 820 | 50 | 30 | 35 | 80 | 7,1 | 1800 |
| Meat plants | - | 290 | 102 | 142 | 160 | 7,5 | - |
| Starch Plant | - | 265 | 93 | 486 | 76 | 6,5 | 3000-10000 |
| Alcoholic spirits factories | 650-1200 | 275 | 100 | 480 | 93 | 6,0 | - |
| Yeast factories | - | 30 | 50 | 20 | 52 | 5,6 | 1500 |
| Beer factories | - | 24 | 15 | 30 | - | 6,2 | - |
| Fruit and vegetable plant | - | 15 | 17 | 50 | 30 | 6,5 | 300-1000 |
| Fish factories | - | 160 | 10 | - | - | - | - |
| Canned food factories | - | - | - | - | 50 | - | 500-1200 |

According to Table 1, the wastewater contained in the food industry contains a lot of plant nutrients, so it is very important to have them as fertilizers. However, these drainage waters are highly toxic due to their high content of organic matter, which can cause significant harm to irrigation in the water reservoirs. Therefore, cleaning of such wastewater by industrial methods is very difficult and inefficient, so it is desirable to clean them by soil absorption.

Task:

As a result of the accident, the submerged pasture containing 1000 mg of Sodium Sodium (S) containing 60 g antimony (Murmma). Water penetration depth is 0.5m (h). Given the fact that accumulation of poisonous substances in each feed chain will increase by 10 times, can it be possible to drink cow milk that is spread on this pasture? Antimicrobial anemia is 0.05 mg / kg.

- We define soil mass polluted by wastewater:
Soap = S * h * p, where p is 1000 kg / m3

- We define an antimony concentration in the soil:

- Determine the concentration of antimony in the milk by creating a feed chain circuit:

| Version | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Мсурьма | 60 | 75 | 72 | 69 | 80 | 68 | 70 | 66 | 76 | 73 | 63 |
| S | 1200 | 500 | 700 | 800 | 1000 | 1100 | 600 | 900 | 560 | 860 | 750 |
| һ | 0,5 | 0,4 | 0,35 | 0,55 | 0,56 | 0,42 | 0,48 | 0,39 | 0,6 | 0,57 | 0,45 |

# Control questions:

- Does it describe the composition of domestic waste water?

- What changes in the composition of industrial waste water varies?

- Determine the composition of mixed water?

- What is the composition of livestock water?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №12

Theme: Pollution norms. Instability of the wastewater composition.

In the sewage technique, the concept of daily pollution is commonly used to one resident. It was offered by SN Stroganov. According to his proposal, the ammonium nitrogen content in sewage is one to a person, with a short circle - from 6 to 8 g. It is aligned with the protein norm of absorption. At the same time, the consumption of salt in household water reaches 8 to 9 kg per person per day. The norm of phosphates - 1.5-1.8 g / day per person. Hereby, the calculated rates of household waste water were calculated per one person per day. Table 1 shows the pollution rates defined by SN Stroganov, embedded in SNiP 2.04.03-85.

Table 1.

| Ingredient | Mass concentration, g |  |
| --- | --- | --- |
|  | By the rules | According to S.N. Stroganov |
| Poppies | 65 | 35-60 |
| Liquid BPC5: |  |  |
| indefinitely | 54 | - |
| discolored | 35 | 35 |
| Liquid BPC5 total: |  |  |
| indefinitely | 74 | - |
| түссіздендірілген | 40 | 40-50 |
| Ammonium nitrogen | 8 | 7-8 |
| phosphates | 3,3 | 1,5-1,8 |
| Including detergents | 1,6 | - |
| Potassium | - | 3 |
| Chlorides | 9 | 8,5-9 |
| Oxidation of the parenterate | - | 5-7 |
| WUA | 2,5 | - |

Individual values in the treated water are normally calculated for pollution concentrations per inhabitant C, mg / l.

Where ɑ - the rate of pollution per person per day, d;
q - water withdrawal rate per person per day, l;
The number is 1000 - 1 milligrams.

Task:
1. How does the concentration of pollution change depending on the norm of water withdrawal?
2. As a result of the analysis, the scrubber contaminated water from the various reservoirs as shown in the table was. When evaluating the results, it is necessary to determine whether these indicators characterize the type of water.

| Water sampling | Concentration of contamination, mg / l |  |  |
| --- | --- | --- | --- |
|  | Poppies | BP Full | CPC |
| І | 180 | 215 | 260 |
| ІІ | 200 | 250 | 200 |
| ІІІ | 50 | 0 | 850 |

# Control questions:

1. To analyze the ecological problem of the Aral Sea and organize a rescue operation?

2. Discusses the ecological problem of the Balkhash Sea?

3. The nature of the Caspian Sea ecological problem?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №13

Theme: Methods for determining the irrigation suitability of treated water.

Determination of the quality of water used for irrigation is carried out in laboratory and field conditions.

1) Determination of the suitability of wastewater on the basis of the total composition of water soluble salts:

- up to 0.5 g / l - it is best to apply to all types of plants;
- from 0.5 to 1.0 g / l - good, very good for many plants;
- 1.0 to 2.0 g / l - good, can be irrigated for all crops (other than salt effect), and drainage system if appropriate measures are taken in that area;

- 2 to 3 g / l - suitable for medium-water-resistant plants irrigation, when complying with certain requirements;
- 3 to 4 g / l - irrigation water.

2) Evaluation of suitability of irrigation water for irrigation coefficient According to M.F.Budanov:

- valid

According to I. A. Intypov-Karataev

- valid

According to V.A. Primlovsky

- valid

According to Sobolev

- valid

Here Na, K, Mg, CI, Ca – unilateral, double-cationic cations and anions, mg * eq / l;

С – concentration of salts, g / l.

3) Evaluation of the suitability of wastewater by the concentration of salts allowed.

| Connections with chemical elements | USSR | USA |
| --- | --- | --- |
| Phenol | 0,001 | 0,001 |
| Fluorine | 1,0 | 1,5 |
| Copper | 1,0 | 3,0 |
| iron | 0,3 | - |
| manganese | 0,3 | 0,3 |
| chlorine | 300 | 250 |
| Nitrogen Acid | 10-40 | - |
| sulphates | 600 | 250 |
| ammonia |  | - |
| Carbon dioxide | - | - |
| lead | 0,1 | 0,1 |
| beef | 0,05 | 0,05 |
| remainder | 1000 | 1000 |

Task:
1. M.F.Budanov and I.A. Intypov-Karataev as follows determine the suitability of sewage water:

| Version | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ca (mg*equ / l) | 240 | 300 | 260 | 270 | 245 | 255 | 265 | 275 | 280 | 290 | 285 |
| Mg (mg * equ / l) | 185 | 200 | 190 | 190 | 180 | 198 | 195 | 192 | 210 | 220 | 225 |
| Na (mg * equ / l) | 190 | 195 | 185 | 187 | 191 | 185 | 185 | 190 | 195 | 185 | 187 |
| C (g / l) | 2,35 | 2,31 | 2,32 | 2,33 | 2,34 | 2,30 | 2,36 | 2,33 | 2,35 | 2,34 | 2,28 |

2. The validity of water in the composition as follows is VA Primlovsky
method:

| Version | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ca (mg * equ / l) | 240 | 220 | 223 | 231 | 245 | 255 | 215 | 210 | 208 | 219 | 218 |
| Mg (mg * equ / l) | 115 | 120 | 119 | 111 | 118 | 108 | 105 | 122 | 110 | 122 | 125 |
| Na (mg * equ / l) | 93 | 105 | 85 | 87 | 91 | 108 | 95 | 90 | 95 | 85 | 87 |
| CI (mg * equ / l | 85,8 | 85,9 | 86,8 | 87,8 | 81,9 | 83,2 | 83,7 | 84,1 | 82,5 | 85,2 | 85,3 |
| C (g / l) | 1,95 | 1,87 | 1,92 | 1,33 | 1,34 | 2,10 | 1,76 | 1,73 | 1,85 | 1,94 | 1,88 |

# Control questions:

1.Is the analysis of the composition of the wastewater?
2. What is the specific nature of the use of drainage water in agriculture?
3. Are the requirements for the use of irrigation water for irrigation?
4. How to determine whether the wastewater is irrigated?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №14

Theme: Determination of the amount of mixing of waste water with river water.

When determining the use of wastewater in sera, the dewatering indicator plays an important role.

The calculation of the required cleaning performance is determined by the amount of blasting substances, the mixing of BPC with river water.

Wastewater used for crop rotation is mixed with river water with definite coincidence. Mixing size nр is determined by the following equation:

Where: α - is the mixing ratio, which indicates how much the amount of water to be mixed with river water;
Q - waste water flow;
q - water discharge.

Determine the level of mixing of the following water with river water.

| Version | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ca (mg * equ / l) | 240 | 220 | 223 | 231 | 245 | 255 | 215 | 210 | 208 | 219 | 218 |
| Mg (mg * equ / l) | 115 | 120 | 119 | 111 | 118 | 108 | 105 | 122 | 110 | 122 | 125 |
| Na (mg * equ / l) | 93 | 105 | 85 | 87 | 91 | 108 | 95 | 90 | 95 | 85 | 87 |
| CI (mg * equ / l) | 85,8 | 85,9 | 86,8 | 87,8 | 81,9 | 83,2 | 83,7 | 84,1 | 82,5 | 85,2 | 85,3 |
| C (g / l) | 1,95 | 1,87 | 1,92 | 1,33 | 1,34 | 2,10 | 1,76 | 1,73 | 1,85 | 1,94 | 1,88 |

# Control questions:

- Is the analysis of the composition of the wastewater?

2. What is the specific nature of the use of drainage water in agriculture?
3. Are the requirements for the use of irrigation water for irrigation?
4. How to determine whether the wastewater is irrigated?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

# Practical classess №15

Theme: Monitoring. Atmosphere and water, soil pollution control methods.

The main goals and objectives of the monitoring are given in the table below.

Table 1 Goals and objectives of environmental monitoring.

| Environmental monitoring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Оbjective Тhe goal. |
| Control | Determine | Estimation | Predict | Decision making | Improving |
| Changes in environmental conditions | Causes of change in human environment are due to human action | Determine the impact of human action and control changes | Changes in Environment | Elimination of consequences of negative behavior of a person | The strategy of effective relations between the environment and the society |

In addition, monitoring technology processes can be described in the form of algorithms: Measurement analysis of the description of the correct modeling of the description.This algorithm of action is typical of any environmental monitoring.

The main types of monitoring for the ecologists are the following types of monitoring:

Biological monitoring - control of natural and anthropogenic processes in biological environments (both organisms and biocenoses) (heavy metals, pesticide accumulation). Such monitoring covers all aspects of life interactions with environmental components.

Basic monitoring - is primarily natural phenomena that are generally biospheric, that is, not only in the present, but also in the near 50-100 years.

Biosphere Monitoring - Changes in the biosphere; atmosphere dust, global water balance. Global ocean pollution, changes in biodiversity in the continental and ocean, and so on at global level.

Bio-ecological monitoring is the monitoring of the state of the environment in terms of human health. Indicators of the person's life-threatening condition - the disease, birth, lifespan and so on.

Remote monitoring is a set of aviation and space monitoring. Sometimes this concept also applies to the information that is transmitted to the information collection center, which transmits information through the devices in areas where the foot is pressed.

Marine pollution monitoring is a system for estimating, evaluating and monitoring their condition for information on the quality of water and oceans. It is necessary to use the marine water resources economically and to protect them from pollution.

Water pollution monitoring in construction - systems for forecasting, evaluating and controlling information about the state of the waters in order to use water resources economically and to protect them from pollution, drainage. Water quality indicators include temperature, mineralization, pH, color, dissolved oxygen, taste, heavy metals, petroleum products, phenols, pesticides, and, most importantly, sodium, potassium, calcium, chlorine, sulphate, carbonate and nitrate.

Monitoring of polluting sources - a system of forecasting, assessment and control of pollution levels and pollution levels of water bodies, atmospheric air, soil pollutants through polluting sources.

Regional monitoring is anthropogenic impacts of large industrial sites, cities and their surrounding areas for biosphere information.

According to the classification level, international and interregional levels of governance should be closely linked to global levels and should be linked to the national-regional level.

Monitoring of the biological monitoring of ecosystems in the ecological monitoring system plays a special role. Biological monitoring is the control of the surrounding environment by means of living organisms. The principal method of biological monitoring is bioindication, taking into account any changes in biota associated with anthropogenic factors.

Task: Understand the monitoring of water monitoring as follows.

| Version | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ca (mg * equ / l) | 240 | 220 | 223 | 231 | 245 | 255 | 215 | 210 | 208 | 219 | 218 |
| Mg (mg * equ / l) | 115 | 120 | 119 | 111 | 118 | 108 | 105 | 122 | 110 | 122 | 125 |
| Na (mg * equ / l) | 93 | 105 | 85 | 87 | 91 | 108 | 95 | 90 | 95 | 85 | 87 |
| CI (mg * equ / l) | 85,8 | 85,9 | 86,8 | 87,8 | 81,9 | 83,2 | 83,7 | 84,1 | 82,5 | 85,2 | 85,3 |
| C (g / l) | 1,95 | 1,87 | 1,92 | 1,33 | 1,34 | 2,10 | 1,76 | 1,73 | 1,85 | 1,94 | 1,88 |

# Control questions:

1.Monitoring of marine pollution?
2. What is water pollution monitoring in construction?
3. Is there an environmental monitoring system?
4. What is pollution sources monitoring?

Literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

Literature

Main literature:

1.Zholdasova M.AMethodical instruction for the students of high school of agricultural science : for first students of 5B080200, 5B080100, 5B080800, 5B080600, 5B080500  / M.A Zholdasova. - Shymkent : SKSU, 2015. - 38 с. (Шифр 81.2Англ/Z 62-347790)Экземпляры: всего:5 - Ч9(5) Держателидокумента: ОИЦЮКГУ.

2.Козачек, А.В. Экологические основы природопользования : учебное пособие для студ. СПО / А. В. Козачек. - Ростов н/Д. : Феникс, 2008. - 442 с. : ил. - ("Среднее профессиональное образование").

3.Английский язык для студентов аграрных специальностей средних профессиональных учебных заведений: Учеб. Пособие/В.А. Шляхова. – М.: Высшая школа, 2008.

4.Парахина А.В. Пособие по переводу технических текстов с английского языка на русский (для средних специальных учебных заведений). Изд. «Высшая школа», Москва, 2007.

5.Английский язык для студентов технических специальностей: учеб.пособие / Е. В. Волкова, Н. Н. Ковалева, Н. Д. Кулитова, О. Н.

6.Сорокина; под ред.: С.С.Иванов; Нижегород. гос. техн. ун-т им. Р.Е.Алексеева. Нижний Новгород, 2013.

7.Учебник английского языка для студентов технических университетов и вузов. — 6-е изд., стереотип. (Иностранный язык в техническом университете). — М: Изд-во МГТУ им. Н.Э. Баумана, 2006. — 448 с.Орловская И.В., Самсонова Л.С., Скубриева А.И.

8.Учебно-методическая разработка по английскому языку для студентов II курса всех специальностей и форм обучения «Деловая корреспонденция» / НГТУ; сост.: И.К. Бевзюк,

9.Л.И.Цветкова, Г.В.Черняева, Т.А.Поскрёбышева, Н. Новгород, 2015.

10.Троицкая Н.А. Единая транспортная система. – М.: Академия, 2009.

11.Ширяев С.А. Транспортные и погрузочно-разгрузочные средства. М.: Горячая линия – Телеком, 2010.

12.Громов А.Ю. Транспортные и погрузочно-разгрузочные средства. СПб., Изд-во СЗТУ, 2009.

Signed to the press _______________

Paper size A4 1/16

Paper typographical. Offset printing. Amount of p.p.

Circulation _____ copies. Order No. ___________

© South-Kazakhstan State University named after M.Auezov Publishing Center SKSU named after M.Auezov, Shymkent, pr. Tauke-khan, 5