Ministry of Education and Science of the Republic of Kazakhstan

M. Auezov South Kazakhstan State University

Department of Foreign Languages for Technical specialties

Ashenova A.S.

Methodical instructions for practical lessons

for 5В071700- «Heat-power engineering»

Form of study: full-time

Shymkent – 2019

UDC: 626.81

Developed by Ashenova A.S.

Methodical instructions for organization of SIWT and SIW»Shymkent: M.Auezov South Kazakhstan State University, 2019- 54 pp.

A methodical instruction for the practical lessons for the specialty 5В071700- «Heat-power engineering» is developed in accordance with the requirements of State Educational Standards for specialties and curriculum of the given discipline. This Methodical instruction comprises all necessary data on performance of the course themes.

Reviewer:

K.K. Kaldayakov -candidate of Philological Sciences, Department of «Foreign Languages for Technical specialities», M. Auezov South Kazakhstan State University.

Methodical instructions for the students of the second course Methodical instructions for practical lessons» is discussed and recommended for publication at the meeting of the Chair of foreign languages for Technical specialties (Minutes №         2019) and at the Committee on innovational technologies of training and Methodical provision of higher school “Information Technologies and Energetics”(Minutes №      2019)

Methodical instructions   is recommended for printing by the Methodical Council of M.Auezov South Kazakhstan State University, Minutes № .____ ___________ 2019

© M.Auezov South Kazakhstan State University, 2019

Responsible for printing:  Asenova A.S.

Introduction

This methodical instruction is intended for students of the Heat Power Engineering and Power Engineering Industry who study English on the basis of the multi-level approach with the focus on international standards.

The instruction includes material which helps masters to develop necessary skills to acquire the English language for professional purposes. The course is composed on the basis of the communicative competence and functioning approaches. The organization of the material allows mastering students’ communicative competence in speaking, reading and writing. In addition, each unit includes tasks to develop the linguistic competence of the students.

Unit 1-2.  Heat and temperature

Practical lesson 1

The theme of the lesson: Heat and temperature

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

- Changes of temperature and state

The two extracts below are from a basic technical training course for the customer service staff of a manufacturer of heating boilers.

As you know, Temperature is measured in degree Celsius (0С). But heat is energy, so it's measured in joules. To calculate the amount of energy needed to raise the temperature of a substance, you need to know the mass of the substance being heated, and also its specific heat capacity - in other words, -the amount of enegy in joules, required to raise the temperature of one kilogram of the substance by one degree Celsius.

Wht happens when substances change state? Well, heat energy is needed to make a solid melt and become a liquid. It's also needed to turn liquid into vapour t o take energy to make a liquid boil, so that it evaporates (or vaporisez) and becomes a gas. That’s because melting and evaporation are enrdorthermic processes. That means they take in heat energy- they need to abcorb heat from a heat source, such as a flame. And it is the opposite when substance cools. As a gas condenses to become a liquid, or as a liquid solidifies to become a solid, the process is exothermic - heat is emttted. The amount of energy absorbed or emitted while a substance changes state, in joules per kilogram, is called latent heat. During melting it is called latent heat of fusion, and during vapourising its called latent heat of vaporization.

- Heat transfer

The textbook extract below looks at heat transfer- how heat travels.

To help understand heat transfer, homes provide everyday examples. The heating systems in homes often have electric convector heaters. These heat the air and make it circulate, so that it moves in a circle- first rising, then cooling and sinking before rising again. This is called convection, where warm gas or liquid moves around and dissipates heat, transferring it to the rest of the gas or liquid. Alternatively, the heating system in a home may circulate hot water through radiators. The radiators An electric convector heater act as heat exchangers - devices that transfer heat-in this case, from the hot water inside to the cooler air outside. This happens by conduction- heat transfer through solid material. After the heat has been conducted through the metal of the radiator, the heat is dissipated by convection. The third way that heat is transferred is by radiation. This is heat that travels as electromagnetic waves. An example is the heat from the sun. So the radiators that circulate water have a misleading name, as they don't really function by radiation.

Exercise 1.Complete the sentences about water using words from A opposite. Sometimes there is more than one possible answer.

1. When the temperature of ice reaches 0 oc, it changes................... -it................................ to become water.

2. At 100 0С, water ................................ .

3. When water is ................................ to 0С  or below and ................................ to become ice, it is said to freeze.

4. In gas form, water is called................................ or .......................   ................................ .

5. Between 100 0С and 374 0С water is a................................ because it is below its critical

temperature.

6. Extremely hot water vapour is called ................................ ................................ .

Exercise 2. Match the two parts to make correct sentences. Look at A and B opposite, and a box below to help you.

Gas and vapour A gas can also be called a vapour- for example, water vapour. The definition of a vapour is a gas which is below its critical temperature. This means the gas can be condensed by putting it under pressure. Above the critical temperature (374 ac in the case of water), the gas can no longer be condensed by pressure.

Steam Water vapour is often called steam. When it is extremely hot- such as in electricity generating turbines - it is called superheated steam.

Cooling and thermal inertia Radiators are widely used in cooling systems - for example, in vehicle engines. Liquid - called coolant- is pumped around the hot engine to absorb heat, and travels through a radiator positioned at the front of the vehicle. As the vehicle moves, air flows over the radiator. The airflow cools the radiator and the coolant inside it. Without a cooling system, the engine would overheat. However, this would not happen immediately after starting the engine, due to thermal inertia- the fact that it takes time to change the temperature of a heavy mass of material (such as an engine), either when it is heated or cooled. An object with a high level of thermal inertia can be described as a heat sink.

1. A liquid pumped onto a workpiece that is being machined, to stop it overheating, is called a …….

2. The form of heat transfer that occurs with infrared heat- a form of electromagnetic wave

-is called …..

3. The metal fins (plates) around air-cooled engines, intended to maximize the surface area of the hot engine that is in contact with the cooler air, are designed to act as a ……..

4. Thick, dense, internal walls inside an energy-efficient house, which are intended to absorb heat energy during the day and store some of it to be emitted at night, function as a………

5. The soil and rocks on the surface of the earth remain warm at night in summer, due to the principle of…..

a. radiation.           b. coolant.            c. heat sink.                d. thermal inertia.

e. heat exchanger.

Exercise 3. Circle the correct words to complete the article about condensing boilers. Look at A and B opposite to help you. The first one has been done for you.

Condensing boilers are becoming increasingly popular in homes, as they use up to 40% less gas than traditional boilers. How do they work? By exploiting the fact that when a liquid condenses, due to the principle of latent heat of (1) fusion/vaporization, the process is (2) endothermic/exothermic. This means heat is (3) absorbed/emitted, and can thus be (4) circulated/conducted via the water inside the radiators in the home.

A condensing boiler burns natural gas (hydrocarbon fuel) to (5) heat/cool water, just like a conventional boiler. However, it ach ieves greater efficiency by recovering energy from water vapour. This is present in the hot, waste gas that's produced when natural gas is burned. In a traditional boiler the (6) heat/temperature energy from the gas, which is at a (7) heat/temperature of 180 0С or more, would be (8) dissipated/radiated into the atmosphere by (9) conduction/convection, and the water vapour within it would condense in the outside air. But in a condensing boiler the hot gas passes through a (1 0) heat/temperature exchanger. This allows the heat from the gas to be (11) absorbed/emitted by the cool water that's returning to the boiler after passing through the radiators in the home's ( 12) cooling/heating system - heat transfer takes place from hot gas to cool water by (13) conduction/radiation through the metal of the heat exchanger. In addition, when the temperature of the gas has fallen to a certain point, the water vapour within it (14) condenses/ solidifies. And it is this is process that enables significant amounts of heat to be transferred, due to the prin ciple of (15) latent/specific heat.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Heat and temperature”

Make up the situation

Control questions

Think about some appliances, machines or structures where heat transfer is an important consideration. How is the heat transferred, and how does this affect the design?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Heat and temperature

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 3-4. Heat power plant (HPP)

The theme of the lesson: Heat power plant (HPP)

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

A power plant is an assemblage of equipment, and apparatus used directly forgeneration of electric energy and also the buildings and structures needed for it. ... The former type includes thermal power stations (TPS), nuclear power plants(NPP), hydraulic power plants (HPP), and hydro pumped storage power plants(HPSPP).

The function of a heat power plant is to convert the energy in coal, oil or gas into mechanical or electric energy through the expansion of steam from a high pressure to a low pressure in a suitable prime mover such as a turbine or engine. A noncondensing plant discharges the steam from the prime mover at an exhaust pressure equal or greater than atmospheric pressure. A condensing plant exhausts the steam from the prime mover into a condenser at a pressure less than atmospheric pressure. The general central station plants are condensing plants because their sole output is electric energy. Industrial plants are frequently noncondensing plants because large quantities of low-pressure steam are required for manufacturing operations.

The steam-generating unit consists of a furnace in which the fuel is burned, a boiler, superheater and economizer, in which high pressure steam is generated, and an air heater in which the loss of energy due to combustion of the fuel is reduced to a minimum.

The boiler is composed of a drum, in which a water level is maintained at about the mid-point so as to permit separation of the steam from the water, and bank of inclined tubes, connected to the drum in such a manner as to permit water to circulate from the drum through the tubes and back to the drum The hot prod-ucts of combustion from the furnace flow across the boiler tubes and evaporate part of the water in the tubes. The furnace walls are composed of tubes which are also connected to the boiler drum to form very effective steam-generating surfaces. The steam which is separated from the water in the boiler drum then flows through a superheater which is in effect a coil of tubing surrounded by the hot products of combustion. The temperature of the steam is increased in the superheater to per-haps 800° to 1100° F, at which temperature the high-pressure superheated steam flows through suitable piping to the turbine.

Since the gaseous products of combustion leaving the boiler tube bank are at a relatively high temperature and their discharge to the chimney would result in a large loss in energy, an economizer may be used to recover part of the energy in these gases. The economizer is a bank of tubes through which the boiler feedwater is pumped on its way to the boiler drum.

A reduction in gas temperature may be made by passing the products of combustion through an air heater which is a heat exchanger cooled by the air re-quired for combustion. This air is supplied to the air heater at normal room tem- perature and may leave the air heater at 400° to 600° F, thus returning to the fur-nace energy that would otherwise be wasted up the chimney. The products of com-bustion are usually cooled in an air heater to an exit temperature of 275° to 400° F, after which they may be passed through a dust collector which will remove objec-tionable dust and thence through an induced-draft fan to the chimney. The function of the induced-draft fan is to pull the gases through the heat transfer surfaces of the boiler, superheater, economizer and air heater and to maintain a pressure in the furnace that is slightly less than atmospheric pressure. A forced-draft fan forces the combustion air to flow through the air heater, duct work, and burner into the fur-nace.

Coal is delivered to the plant in railroad cars or barges which are unloaded by machinery. The coal may be placed in storage or may be crushed and elevated to the overhead raw-coal bunker in the boiler room.

The coal flows by gravity from the overhead bunker to the pulverizer or mill through a feeder which automatically maintains the correct amount of coal in the mill. In the mill the coal is ground to a fine dust. Some of the hot air from the air heater is forced through the mill to dry the coal and to pick up the finely pulverized particles and carry them in suspension to the burner where they are mixed with the air required for their combustion and discharged into the furnace at high velocity to promote good combustion.

The high-pressure, high-temperature steam is expanded in a steam turbine which is generally connected to an electric generator. From 3 to 5 per cent of the output of the generator is needed to light the plant and to operate the many motors required for fans, pumps, etc., in the plant. The rest of the generator output is available for distribution outside the plant.

The condensed steam, which is normally at a temperature of 70° to 100° F, is pumped out of the condenser by means of a hot-well pump and is discharged through several feed-water heaters to a boiler feed pump that delivers the water to the economizer.

2. Answer the following questions

a. What is the main function of a heat power plant?

b. What are the components of a boiler described in this text?

c. How can the reduction in gas temperature be made according to text?

d. What is a function of coal in heat power plant?

3. Find a synonym to a given word:Convert, expansion, exhaust, permit, recover, transfer, storage

2. Nuclear power plants

Nuclear power plants use the heat generated from nuclear fission in a contained environment to convert water to steam, which powers generators to produce electricity.

Although the construction and operation of these facilities are closely monitored and regulated, accidents are possible. An accident could result in dangerous levels of radiation that could affect the health and safety of the public living near the nuclear power plant.

Local and state governments, federal agencies, and the electric utilities have emergency response plans in the event of a nuclear power plant incident. The plans define two “emergency planning zones.” One zone covers an area within a 10-mile radius of the plant, where it is possible that people could be harmed by direct radiation exposure. The second zone covers a broader area, usually up to a 50-mile radius from the plant, where radioactive materials could contaminate water supplies, food crops and livestock.

The potential danger from an accident at a nuclear power plant is exposure to radiation. This exposure could come from the release of radioactive material from the plant into the environment, usually characterized by a plume (cloud-like formation) of radioactive gases and particles. The major hazards to people in the vicinity of the plume are radiation exposure to the body from the cloud and particles deposited on the ground, inhalation of radioactive materials and ingestion of radioactive materials.

Radioactive materials are composed of atoms that are unstable. An unstable atom gives off its excess energy until it becomes stable. The energy emitted is radiation. Each of us is exposed to radiation daily from natural sources, including the Sun and the Earth. Small traces of radiation are present in food and water. Radiation also is released from man-made sources such as X-ray machines, television sets and microwave ovens. Radiation has a cumulative effect. The longer a person is exposed to radiation, the greater the effect. A high exposure to radiation can cause serious illness or death.

Nuclear power plants don't require a lot of space; they do not need a large plot like a wind farm. But they have to be built near a large body of water for cooling purposes - using the water as a heat sink. They are usually found on the coast so there is no risk to drinking water sources. Nuclear power stations do not contribute to carbon emissions - no CO2 is given out - it therefore does not contribute to global warming. Nuclear power stations do not produce smoke particles to pollute the atmosphere or emit gases that contribute to acid rain. Nuclear energy is by far the most concentrated form of energy - a lot of energy is produced from a small mass of fuel. This reduces transport costs - (although the fuel is radioactive and therefore each transport that does occur is expensive because of security implications). Nuclear power is reliable. It does not depend on weather. We can control the output from a nuclear power station to fit our needs. It is relatively easy to control the output - although the time factor for altering power output is not as small as for fossil fuel stations. It is said to have a long 'start up' time. It cannot respond immediately to demand. That is why electricity companies try to 'even out' demand by using tariff that encourages use at off peak time prides. Nuclear power produces a small volume of waste, although that waste is radioactive. Low Fuel Cost - the main reason behind the low fuel cost is that it requires little amount of uranium to produce energy. When a nuclear reaction happens, it releases million times more energy as compared to traditional sources of energy. Only about 28 gram of uranium releases as much energy as produced from 100 metric tons of coal.

1. Answer the following questions

a. What does the nuclear power plant use to generate power?

b. What is the potential danger from a nuclear power plant?

c. Why does the nuclear power plants need to be built around water?

d. Does the nuclear power plant release more energy than traditional energy sources?

2. Find a synonym to a given word

Contained, construction, emergency, exposure, unstable, respond, global

3.Translate into Russian

Nuclear power stations

1. The first industrial nuclear power station in the world was constructed in Obninsk not far from Moscow in 1954. The station was put into operation two years earlier than the British one and three and a half years earlier that the American nuclear power-stations.

2. A number of nuclear power-stations have been put into operation since 1954. The Beloyarskaya nuclear power station named after academician Kurchatov may serve an example of the peaceful use of atomic energy. The scientists and engineers achieved a nuclear superheating of steam directly in the reactor itself before steam is carried into the turbine. It is certainly an important contribution to nuclear engineering achieved for the first time in the world.

3. We might mention here another important achievement that is the first nuclear installation where thermal energy generated in the reactor is transformed directly into electrical energy. Speaking of the peaceful use of atomic energy it is also necessary to mention our nuclear ice-breakers. “Lenin” is the world’s first icebreaker with nuclear installation. Its machine installation is of a steam turbine type and steam is produced by three reactors and six stream generators.

Exercise 1. Find the correct answer

What does the scientific contribution of the nuclear power station consist of?

1. It was the first industrial nuclear power station in the world.

2. The thermal energy generated in the reactor was transformed directly into electric energy.

3. The nuclear superheating of steam was achieved directly in the reactor itself before steam is carried into the turbine.

Exercise 2. Rewrite the sentences and translate them, paying attention to the features of the translation of the degrees of comparison of adjectives and adverbs.

1. The pressure in the furnace was slightly less than atmospheric pressure.

2. The greater is the mixing of oxygen with combustible gases, the more is the increase of combustion rate.

Exercise 3. Rewrite the sentences; define in them the tense forms of the verb. Specify their infinitives and translate.

1. The substance has changed it molecular structure when exposed to sunshine.

2. A group of specialists is studying this problem.

3. The available heat drop affects the number of pressure stages in the impulse turbine. Exercise number 4. Rewrite sentences. Underline the modal verb or its equivalent in each and translate.

1. The expansion of the steam must take place in the fixed nozzle passages.

2. The products of combustion have to be cooled sufficiently before they enter the superheater tubes.

3. The temperature of the cooling water has to vary only with atmospheric conditions.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Heat power plant (HPP)”

Make up the situation

Control questions

What is the difference between a nuclear and thermal power plant?

What are the advantages and disadvantages of thermal power plants?

What is the difference between thermal power plant and thermal power station?

What is the difference between turbines used in hydro power and thermal power plants?

What is the difference between thermal power plant and steam power plant?

What is the difference between in motors of thermal and hydel power plant?

What's the difference between a thermal power plant and a thermal battery plant?

What is the difference between a gas power plant and a coal power plant?

What is the cost of a thermal power plant boiler?

What are the uses of a thermal power plant?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Heat power plant (HPP)

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 5-6

The theme of the lesson: Combined heat and power plant

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

What is Combined Heat and Power (CHP)? Power independence with up to 80% overall energy efficiency.

Combined Heat and Power (CHP) – sometimes called cogeneration or distributed energy -- is the simultaneous production of electricity and heat from a single fuel source. CHP is not a single technology but an integrated energy system that can be modified to meet the needs of an end-user.

Below is a simple high-level schematic of a CHP system.

Cogeneration equipment produces power and thermal energy from a common fuel source, generally one that is considered to be a waste product from another process.

Combined Heat and Power systems allow the production of useful heat and electricity from the same source to be utilised.  Thus, increasing the overall efficiency through recovery of the heat rejected from the inefficient energy conversion of producing electricity.  This is no new concept, with the use of CHP systems well known if under promoted.  A traditional CHP system can be seen below in figure 1 below.

The main components of a typical system are:

- the engine or prime mover that drives the generator.

- the generator that produces electricity.

- the heat recovery system that recovers the waste heat from both the engine water cooling jacket and exhaust gases.

- the exhaust system to take away the products of combustion.

- the control panel to monitor the operation.

Typical characteristics of the main CHP systems in operation are summarised below.

|  | S.I. Engine | C.I. Engine | Gas Turbine | Combined Cycle | Back Pressure Steam Turbine | Pass-out Steam Turbine |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Type | Natural gas, Biogas | Natural gas, Biogas, Oil, Heavy oils | Natural gas, Biogas, Oil, | Natural gas, Biogas, Oil, | All types | All types |
| Capacity Range (MWe) | 0.03-2 | 0.1-2 | 1 | 3 | 0.5 | 1 |
| Overall Efficiency (%) | 70-78 | 65-75 | 65-80 | 73-80 | 75-84 | 75-84 |
| Capital Costs p/kWe | 550-850 | 500-800 | 500-1500 | 500-700 | 600-2000 | 600-2000 |
| Maintenance Costs p/kWh | 0.5-0.8 | 0.4-0.8 | 0.2-0.7 | 0.2-0.7 | 0.1 | 0.1 |

Table 1 – Summary of some traditional CHP systems characteristics.

This arrangement of recovering rejected heat can be carried over to fuel cell operation.  Mainly through the development of two main types as mentioned earlier, Solid Oxide (SOFC) and Phosphoric Acid (PAFC).  For the purpose of CHP applications these two types will be the focus, although in some circumstances the Proton Exchange Membrane (PEM) cell and Molten Carbonate cell could be considered.

The reasoning for focusing on the two types is the commercially availability and interest in development of the PAFC and the high operating temperature and interest in the SOFC.  Moreover, they allow for a comparison between a relatively low operating temperature and a high operating temperature cell.

Reading

Combined heat and power (CHP) plants, also known as cogeneration, generate electricity and useful thermal energy in a single, integrated system. CHP is not a technology, but an approach to applying technologies. Heat that is normally wasted in conventional power generation is recovered as useful energy, which avoids the losses that would otherwise be incurred from separate generation of heat and power. While the conventional method of producing usable heat and power separately has a typical combined efficiency of 45 percent, CHP systems can operate at levels as high as 80 percent.

Conventional generation is inherently inefficient, only converting on average about a third of the input fuel's potential energy into usable energy. Engineers have long appreciated the tremendous efficiency opportunity of combining electricity generation with thermal loads in buildings and factories, capturing much of the energy that would otherwise be wasted. When the word "cogeneration" was coined in the 1970s to describe this practice, the dominant configuration of systems was a boiler that generated steam, some of which was used to turn a steam turbine that generated electricity. Due to the cost and complexity of these systems, they were largely confined to systems of over 50 MW, thus precluding their installation at most manufacturing facilities. However, recent advances in electricity-efficient, cost-effective generation technologies — in particular advanced combustion turbines and reciprocating engines — have allowed for new configurations of systems that combine heat and power production, expanding opportunities for these systems and increasing the amount of electricity they can produce. These CHP systems now come in many more configurations, and can even satisfy compressed air requirements by bleeding high-pressure air off the compressor stage of a combustion turbine.

New generations of turbines and reciprocating engines are the result of an intensive collaborative research by government and industry that uses advanced materials and computer-aided design techniques that have dramatically increased equipment efficiency and reliability while reducing costs and pollutant emissions.

Several barriers impede the full realization of the world’s CHP potential. These include electric utility rate structures that discourage utility investment in CHP, a lack of common and fair interconnection and net metering standards, discriminatory utility standby rates, and emissions regulations that do not recognize the improved efficiency and pollution benefits of CHP systems.

2. Find a synonym to a given word

Approach, conventional, tremendous, dominant, installation, collaborative, include

3.Discussing

Does combined heat and power fit into the future low carbon world?
Combined Heat and Power (CHP), also commonly referred to as cogeneration, is any system that generates multiple types of usable energy from a single power source. For example, a gas-fired turbine burns natural gas and outputs electricity and thermal energy such as steam or hot water. The benefit of this system approach is more efficient conversion of source fuel to usable energy. The difference in efficiency between cogeneration and two separate systems generating the same electricity and heat is around 45 percent compared to up to 90 percent.

You are likely thinking to yourself, we need to implement CHP more to reduce energy consumption and carbon emissions.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Combined heat and power plant”

Make up the situation

Control questions

1. How does the CHP generate electricity and thermal energy?

2. Why is the conventional generation inefficient?

3. What are the results of an intensive collaborative research by government and industry?

4. What kind of barriers do full realizations of CHP’s potential impede?

5. How does CHP work with your local power supply?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Combined heat and power plant

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 7-8

The theme of the lesson: District heating

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

District heating (also known as heat networks or teleheating) is a system for distributing heat generated in a centralized location through a system of insulated pipes for residential and commercial heating requirements such as space heating and water heating.

District heating (also known as heat networks) is a system for distributing heat generated in a centralized location for residential and commercial heating requirements. The heat is often obtained from a cogeneration plant burning fossil fuels but increasingly also biomass, geothermal heating, heat pumps and central solar heating are also used, as well as nuclear power. District heating plants can provide higher efficiencies and better pollution control than localised boilers. According to some research, district heating with combined heat and power (CHPDH) is the cheapest method of cutting carbon emissions, and has one of the lowest carbon footprints of all fossil generation plants. CHPDH is being developed in Denmark as a store for renewable energy, particularly wind energy, that exceeds instantaneous grid demand via the use of heat pumps and thermal stores

After generation, the heat is distributed to the customer via a network of insulated pipes. District heating systems consist of feed and return lines. Usually the pipes are installed underground but there are also systems with overground pipes. Within the system heat storages may be installed to even out peak load demands.

The common medium used for heat distribution is water or pressurized hot water, but steam is also used. The advantage of steam is that in addition to heating purposes it can be used in industrial processes due to its higher temperature. The disadvantage of steam is a higher heat loss due to the high temperature. Also, the thermal efficiency of cogeneration plants is significantly lower if the cooling medium is high-temperature steam, causing smaller electric power generation. Heat transfer oils are generally not used for district heating, although they have higher heat capacities than water, as they are expensive, and have environmental issues.

At customer level the heat network is usually connected to the central heating of the dwellings by heat exchangers (heat substations). The water (or the steam) used in the district heating system is not mixed with the water of the central heating system of the dwelling.

District heating has various advantages compared to individual heating systems. Usually district heating is more energy efficient, due to simultaneous production of heat and electricity in combined heat and power generation plants. This has the added benefit of reducing carbon emissions. The larger combustion units also have a more advanced flue gas cleaning than single boiler systems. In the case of surplus heat from industries, district heating systems do not use additional fuel because they use heat (termed heat recovery) which would be dispersed to the environment.

District heating is a long-term commitment that fits poorly with a focus on short-term returns on investment. Benefits to the community include avoided costs of energy, through the use of surplus and wasted heat energy, and reduced investment in individual household or building heating equipment. District heating networks, heat-only boiler stations, and cogeneration plants require high initial capital expenditure and financing. Only if considered as long-term investments will these translate into profitable operations for the owners of district heating systems, or combined heat and power plant operators. District heating is less attractive for areas with low population densities, as the investment per household is considerably higher. Also it is less attractive in areas of many small buildings; e.g. detached houses than in areas with a few much larger buildings; e.g. blocks of flats, because each connection to a single-family house is quite expensive.

1. Find a synonym to a given word

Distributing, provide, exceed, instantaneous, demand, commitment, benefits

2. Dicussing. How District Heating Warms Cities without Fossil Fuels

Many cities which endure cold winters are adapting district heating schemes to keep people warm without the use of fossil fuels.

Heating homes and offices without adding to the dangers of climate change is a major challenge for many cities, but re-imagined district heating is now offering an answer.

A district heating scheme is a network of insulated pipes used to deliver heat, in the form of hot water or steam, from where it is generated to wherever it is to be used.

As a way of providing warmth for thousands of homes, typically in multi-storey apartment buildings, district heating has a long history in eastern Europe and Russia. But the hot water it distributes typically comes from power stations burning coal or gas, which means more greenhouse gas emissions.

Tapping into other forms of producing hot water, from renewable energy, bio-gas or capturing waste heat from industrial production, supermarkets or IT systems, provides alternative sources of large scale heating without adding to the carbon dioxide in the atmosphere.

Sweden has pioneered the switch from fossil fuels to other ways of heating water. The Swedish Environmental Protection Agency says  the country has gone from almost exclusively relying on fossil fuels to being 90% powered by renewable and recycled heat in 2017.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “District heating”

Make up the situation

Control questions

1. Where does heat for district heating gained from?

2. What is the advantage of using steam instead of water in district heating?

3. What are the advantages of using district heating in general?

4. Where is the district heating less attractive?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of District heating

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 9-10

The theme of the lesson: Energy usage in industry

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Industry uses a large amount of energy to power a diverse range of manufacturing and resource extraction processes. Many industrial processes require large amounts of heat and mechanical power, most of which is delivered by natural gas, petroleum fuels and as electricity. In addition some industries generate waste streams that can be recovered to provide additional energy. Because industrial processes are so diverse from the cold or low temperature required process to the high temperature required process, it is impossible to describe the multitude of opportunities for energy efficiency improvements in industry. There are a number of processes and energy services that are widely used in many industries. Many depend on the specific technologies and processes in use at each industrial facility. However effective energy management in industry, irrespective of size, technology or process, will increase energy efficiency by at least 5%. To implement co-generation, decrease process heat level and amount or recover the waste heat from process or utilities provides important Energy Efficiency opportunities in many countries. Various industries co-generate steam and electricity for subsequent use within their own facilities. When electricity is generated, the heat that is produced as a by-product can be captured and used for process steam, heating or other industrial purposes. Co-generation converts up to 90% of fuel into usable energy.

Electric motors are by far the most important type of electric load in industry using about 70% of the consumed electricity. In the tertiary sector, electric motor systems use about one third of the consumed electricity. Electric motors usually run at a constant speed, but a variable speed drive allows the motor’s energy output to match the required load. This achieves energy savings ranging from 3 to 60%, depending on how the motor is used. Motor coils made of superconducting materials can also reduce energy losses. Moreover, motors can also benefit from voltage optimisation. Industry uses a large number of pumps and compressors of all shapes and sizes and for a-wide variety of applications. The efficiency of pumps and compressors depends on many factors but often improvements can be made by implementing better process control and better maintenance practices. Compressors are commonly used to provide compressed air which is used for sand blasting, painting, and other power tools. According to some investigations, optimising compressed air systems by installing variable speed drives, along with preventive maintenance to detect and fix air leaks, can improve energy efficiency by 20 to 50%.

Energy efficiency in industry has improved significantly in the last decade, and additional improvements are still possible through the implementation of best available technologies (BATs) and design guidelines for existing assets. Moreover, efficiency measures offer some of the least-cost options for reduction of carbon dioxide (CO2) emissions; however, a wider deployment of well-known, cost-effective policy instruments is needed to achieve this potential. Technologies such as carbon capture and storage (CCS), smelting reduction, separation membranes and black liquor gasification are expected to contribute massively to-meeting these challenges. Greater investment by both governments and industry is needed for research, development, demonstration and deployment of a wide range of promising new technologies and for identification and implementation of novel processes which will result in the CO2-free production of materials in the longer term; for instance, induction-(potential sectors: metallurgy, food industry), heat pumps for industry (potential sectors: food, chemistry, pulp and paper, iron and steel).

In the O&G Industry, the downstream business is strongly constrained by high crude oil costs, the required investments, increasing complexity of business processes and close monitoring of product prices, due to the huge impact of energy prices on national economies, in particular in developing countries. This can be attributed to the refining activity which has a long history of rigorous management of its operational costs, focusing on energy efficiency, maintenance practices, personnel management, etc.

In order to evaluate the energy performance of a refinery or petrochemical plant, analyse its performance trends and compare with other plants, it is important to consider performance indicators that take into account the complexity of the systems and its operations, and allow a critical analysis of its results by the manager, to bridge the gaps between the results and the proposed goals.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Energy usage in industry”

Make up the situation

Control questions

1. What are the common energy sources known worldwide?

2. Why the electric motors are considered to be the most electric load in industry?

3. What does one need to do in order to evaluate energy performance of a refinery or petrochemical plants?

4. What is the O&G business constrained with?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Energy usage in industry

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 11-12

The theme of the lesson: The steam-generating unit

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Steam generating unit - A unit to which water, fuel, and air are supplied and in which steam is generated. It consists of a boiler furnace, and fuel burning equipment, and may include as component parts water walls, superheater, reheater, economizer,air heater, or any combination thereof.

For operation at pressures below the critical pressure, a steam-generating unit consists of a boiler, superheater, air heater, and (or) economizer. The furnace walls are either partially or fully covered with boiler tubes. In general, most of the steam is generated in the furnace-wall tubes since they can absorb radiant energy from the high-temperature flame.

A typical stoker-fired steam-generating unit in the smaller size range, has a capacity of 72,500 lb of steam per hr. The gases as they leave the completely water-cooled furnace pass across the superheater surface, then the convection tubes of the boiler, then upward through a small economizer, downward through a tubular air heater, dust collector, and fan, to the chimney. The boiler is of the two-drum type without gas baffles; that is, it is a single-pass boiler. The internal baffles in the steam drum are so arranged that the last lour rows of boiler tubes in which the heat-transfer rate is quite low are downcomers. Since a major item in the cost of a boiler is the drums, as many boiler tubes as possible are placed between the drums. A large amount of surface is required to cool the gases from the temperature at which they leave the superheater to the final temperature.

Depending upon the steam pressure, the feedwater is heated in regenerative feed-water heaters to 275° F to over 600° F, depending on pressure, before being admitted to the economizer. Essentially, the economizer raises the feed-water temperature almost to the saturation temperature, the boiler supplies the latent heat, and the superheater supplies the superheat. It will be noted that, as the pressure increases, a decreasing portion of the total energy absorption occurs in the boiler and that, for pressures above the critical, there is no boiler. Supercritical-pressure steam generators essentially are economizers connected to superheaters. There is no steam drum since there is no boiling and no steam to be separated from water at a constant temperature.

At the higher pressures at which natural circulation boilers may be used, the boiler becomes a smaller part of the installation and the superheater and reheater become a larger portion of the total heat-transfer surface.

Modern high-capacity steam-generating units have been developed to the point that they can be depended upon to carry heavy loads continuously for months at a time. Their reliability is approximately equal to that of modern steam turbines. Consequently, most new central-station power plants are built on the unit system: that is, with each turbine generator supplied with steam from its own steam- generating unit. Thus, turbine-generator units in capacites up to 500,000 kw are being supplied with steam from a single steam-generating unit. One of the major reasons for this arrangement is the decreased cost per unit of capacity which results from increased size.

Discussing

Advanced steam technology

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “The steam-generating unit ”

Make up the situation

Control questions

1. What does the steam-generating unit consist of?

2. What is the capacity of the steam-generating unit?

3. What is the temperature range of heating in a steam-generating unit?

4. At what pressures do the natural circulation boilers can be used?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of the steam-generating unit

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 13-14-15

The theme of the lesson: Different types of boilers

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Boiler types

- Haycock and wagon top boilers.

- Cylindrical fire-tube boiler.

- Multi-tube boilers.

- Solid fuel firing.

- Firetube boiler.

- Superheater.

- Water tube boiler.

- Supercritical steam generator.

Refuse boilers

The standard refuse (or waste) recovery boiler incinerates solid or liquid waste products. This boiler type is not to be mixed with the recovery boilers used in pulp and paper industry. Therefore, we will always refer to refuse boilers when talking about waste and recovery boilers when we mean the specific chemical re-covery process used in the pulp and paper industry.

The combustion of waste differs radically compared to other fuels mostly due to the varying properties of waste. Also, the goal when combusting waste is not to produce energy, but to reduce the volume and weight of the waste and to make it more inert before dumping it on a refuse tip.

Waste is burned in many ways, but the main method is to combust it in a grate boiler with a mechanical grate. Other ways to burn waste is to use a fixed grate furnace, a fluidized bed for sludge or rotary kilns for chemical and problem-atic waste. Waste is usually “mass burned”, i.e. it is burned in the shape it was de-livered with minimal preparation and separation. The main preparation processes are grinding and crushing of the waste and removal of large objects (like refrigera-tors). Waste has to be thoroughly combusted, so that harmful and toxic compo-nents are degraded and dissolved.

Waste can be refined into fuel, by separating as much of the inert and inor-ganic material as possible. This is called refuse derived fuel (RDF) and can be used as the primary fuel in fluidized bed boilers or burned as a secondary fuel with other fuels. RDF is becoming more common nowadays.

Bio-energy boilers

Renewable energy production is becoming a worldwide priority as countries strive for sustainable growth and better living conditions. Many countries (e.g. EU) have already set demanding targets to increase electricity production using bio-energy resources and have introduced attractive incentives to accelerate this pro-cess. Bio-energy solutions are based on a local fuel supply and thus provide price stability, a secure supply of heat and power, and also local employment. Biofuels are increasingly becoming locally traded commodities, which will further secure fuel price stability and availability. At the same time, green certificates and emis-sion trading offer new opportunities for financing bio-energy projects.

Boilers combusting biofuels can be used to produce only electricity, but they are mostly used in combined heat and power (CHP) plants and district heating plants. These boilers are designed to operate on a wide variety of biofuels, includ-ing extremely wet fuels such as wood residues, wood chips, bark and sawdust. Smaller boilers use grate firing technology for biofuel combustion, while larger plants use fluidized bed combustion technology. Smaller grate fired plants for thermal heat production, (<10 MWth), have fire tube boilers, while larger ones are fitted with integrated water/fire tube boilers.

Packaged boilers

Packaged boilers are small self-contained boiler units. Packaged boilers are used as hot water boilers, aiding utility boilers and process steam producers. Pack-aged boilers can be both water tube and fire tube boilers. Packaged boilers can only be used with oil and gas as fuel without separate preparation devices. A packaged boiler can also be rented if there is a need for a temporary boiler solution.

The benefits of packaged boilers over common utility boilers are:

- Short installation time and low installation costs

- Small space usage

- Lower acquisition cost

- Better quality surveillance in work

- Standardized units

The drawbacks of packaged boilers are:

- Higher power consumption

- Cleaning periods more frequent.

2. Read and translate into Russian

Boilers

A gas/oil central heating boiler (heat generator) is like the engine of a car, this provides the heat that the facility needs to warm itself up. The size of the boiler is matched to the size of the facility.

If the boiler is oversized, the fuel bills will be excessive.

If the boiler is undersized, it may not generate enough heat in winter.

The ideal size for a boiler is one that just copes adequately on the coldest day of the year. Most boilers are oversized by at least 30%. This is due to the way systems used to be calculated with a card calculator. These were always over-calculated "to be on the safe side." Today, the emphasis is on energy conservation, and the fact that heat loss calculations can be done very accurately, means there is no need to oversize. This allows smaller radiators and less water in the system, which in turn, means a smaller boiler and reduced costs for both installation and fuel bills.

The boiler does not directly govern the amount of radiators fitted to the system. It is the power of the pump and circulation of the water through adequately sized pipes that determines the number of radiators you can have. But the total output of all the radiators, pipes, and cylinders determines the size of the boiler.

The boiler is not the heating system; it is only one of the parts in the global heating system. As shown in '''Figure 1''', a heating system consists of four main parts:

Boiler/burner combination (the part producing the heat)

Piping with pumps and valves (the part distributing the heat)

Radiators and convectors (the part emitting the heat to the room)

Control equipment such as room thermostat and outside temperature control (the part controlling room and water temperature)

Boiler Types and Classifications

There are two general types of boilers: ''fire-tube'' and ''water-tube''. Boilers are classified as "high-pressure" or "low-pressure" and "steam boiler" or "hot water boiler." Boilers that operate higher than 15 psig are called "high-pressure" boilers.

A hot water boiler, strictly speaking, is not a boiler. It is a fuel-fired hot water heater. Because of its similarities in many ways to a steam boiler, the term ''hot water boiler'' is used.

Hotwater boilers that have temperatures above 250° Fahrenheit or pressures higher than 160 psig are called ''high temperature hot water boilers''.

Hotwater boilers that have temperatures not exceeding 250° Fahrenheit or pressures not exceeding 160 psig are called ''low temperature hot water boiler''s.

Heating boilers are also classified as to the method of manufacture, i.e., by casting (cast iron boilers) or fabrication (steel boilers). Those that are cast usually use iron, bronze, or brass in their construction. Those that are fabricated use steel, copper, or brass, with steel being the most common material.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Different types of boilers”

Make up the situation

Control questions

1. When we talk about waste, what kind of a boiler do we refer?

2. Where can the bio-energy boilers be used?

3. How does one use packaged boilers?

4. What are the benefits of a packaged boiler over common utility boilers?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Different types of boilers

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 16-17-18

The theme of the lesson: Turbines

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

The steam turbine is prime mover in which a part of that form of energy of the steam evidenced by a high pressure and temperature is converted into kinetic energy of the steam and then into shaft work.

The basic advantage of the turbine over other forms of prime movers is the absence of any reciprocating parts. With only rotating motion involved, high speeds are attainable. Since power is directly proportional to torque times speed, an increase in the rotative speed materially decreases the value of the torque required for a given power output A decrease in the required torque permits a reduction in the size of the prime mover by reducing the length of the torque arm or the force acting on the torque arms Also, with the absence of any reciprocating parts, vibration is greatly minimized. Owing to the high rotative speeds available with relatively little vibration, the size and cost of the driven machinery, of the building space, and of the foundations are greatly reduced. These advantages are most apparent in large prime movers and permit the steam turbine to be built in sizes of over 350,000 hp in single units, and 760,000 hp in compound units.

Steam turbines may be broadly grouped into three types, the classification being made in accordance with the conditions of operation of the steam on the rotor blades. The groups are as follows:

1. Impulse. This may be divided into

| a) Simple impulse | Pressure compounded |
| --- | --- |
| b) Compound impulse | Velocity compounded |
| c) Combined impulse | Pressure velocity compounded |

2. Reaction subdivided into

a) Axial flow

b) Radial and axial flow

3. Combination of 1 and 2.

Impulse Turbines. In an impulse turbine the potential energy in the steam due to pressure and superheat is converted into kinetic energy in the form of weight and velocity by expanding it in suitably shaped nozzles.

The whole of the expansion takes place in the fixed nozzle passages. As there is no expansion in the passage between the rotor blades, the steam pressure is the same at the inlet and outlet edges of these blades. The steam impinges on the wheel blades causing the wheels to rotate. The expansion is carried out in stages referred to as “pressure stages”, each stage being separated from the next by a diaphragm with nozzle openings through which the steam passes on its way through the turbine.

a) Simple impulse. This type has a considerable number of pressure stages, a wheel in each stage having one row of blades. To obtain high economy it is necessary that the steam should flow through the turbine with high velocity. This is attained by provision of a large number of pressure stages, the greater the available heat drop, the greater the number of stages. In the simple impulse turbine a wheel of comparatively large diameter is used in the first stage which can deal efficiently with a large energy drop. This large wheel, under nozzle control of the steam can maintain a higher efficiency over a wider range of load than a small one could and is less liable to be affected by changes of steam conditions. An added advantage of a large wheel is that the maximum rating of the machine can be obtained without by-passing which results in a flat consumption curve being maintained over the whole output range.,’

b) Compound impulse. This turbine has comparatively few pressure stages, a wheel in each of them provided with two or more rows of blades. Low velocity steam is obtained by the provision of what are usually termed “velocity stages” in each of the pressure stages. In these velocity stages the steam after passing through the first row of blades on a wheel is re-directed on to the second row of blades on the same wheel, and successively on to other rows of blades on this wheel, if provided. The steam is re-directed by arranging stationary blading between each two adjacent rows of wheel blading so that the steam leaving the first row of blades on a wheel in a backwards direction, enters the first row of stationary blades where its direction is reversed ready for entering the second row of blades on the wheel and so on. This action is repeated in each pressure stage on the turbine.

c) Combined impulse. This turbine is a combination of the types a) and b). It consists of one or more pressure stages with a wheel in each of these stages provided with two or more rows of blades. In the velocity compounded impulse turbine the “carry-over” velocity and the speed of the shaft are much less than with the simple impulse machine. Each disk carrying the moving blades is perforated, thus maintaining the same pressure on both sides of the wheel. The pressure velocity compounded design is generally known as the “Curtis” type. The pressure compounded turbine has a higher efficiency since the pressure drop per stage may be arranged to give the most suitable jet velocity for a given speed of the machine.

2. Reaction Turbines. In the reaction turbines expansion takes place in both the stationary and rotating passages and the pressure at entrance to the rotor blades is therefore greater than at exit.

a) Axial flow. In a pure reaction turbine expansion should take place only as the steam passes through the moving blades, the turning effect being due to the reaction consequent on the increase in velocity which accompanies expansion. The reaction turbine has a ring of stationary blades instead of a diaphragm with nozzle passages between the blades of each pair of adjacent wheels. The steam expands in the fixed blades, increasing its velocity, which is imparted to the moving blades on the impulse principle.

Steam is supplied, direct to the blading system without expansion in nozzles and the rotation produced is chiefly due to the reaction set up by the steam between the stationary and rotating blades while expanding in them.

b) Radial flow. The Ljungstrom turbine is really a combined radial and axial flow machine. The flow of steam is radial, being admitted at the center of the blade discs and flowing outwards, the steam then being inverted to axial flow in the last stages. The turbine may be constructed for single or double motion. With the double motion design the discs rotate in opposite directions at equal speeds and the relative speed of the blades is therefore equal to twice the running speed. This design consists of one group of radial flow double rotation blading and two groups in parallel of low pressure axial flow single rotation blading, the divided flow in the final stages assisting in the reduction of the “leaving losses”. Each steam rotor is coupled to an alternator which carries half the total output.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Different types of boilers”

Make up the situation

Control questions

1. What is the turbine as described in the text above?

2. What is the main advantage of a turbine over other forms of a mover?

3. Where does the expansion take place in an impulse turbine?

4. Where does the expansion take place in a reaction turbine?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Different types of boilers

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 19-20-21

The theme of the lesson: Automation of heat power engineering process

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Over the past decade, power plant control systems have evolved from DCS-centered platforms with proprietary software, to open systems using industry standard hardware and software, and then to totally integrated plant automation systems with almost unlimited connectivity and the ability to interrogate field instruments from many different manufacturers. What’s next?

Today’s power plant control room is evolving into an almost office-like setting, typically quiet and with few staff. Gone are the large boiler-turbine generator (BTG) boards and vertical panels populated with indicators and strip chart recorders. Also gone are the numerous manual/auto control stations that allowed plant operators to individually access final control elements.

New technology has significantly changed the purpose of the control room. No longer a place where operators control, it is now just one of several portals for an integrated team of experts with the common objective of maximizing the value of the plant’s assets.

Gone are the large boiler-turbine gauge panel boards, the walls of instruments, and strip chart recorders. Decisions are made by a team of experts, most of whom will not be on site. Today’s automation systems include portals that allow people from all over the world to access information in real time.

Beyond cheap computing and advanced software applications, an automation system must be designed to optimize the economics of plant assets. Deregulation has required greater awareness of optimizing operations. Maximizing availability, efficiency, and safety are crucial roles of an automation system. Furthermore, monitoring, reporting, and controlling emissions have been elevated, in some cases to the highest corporate level, largely because of regulatory scrutiny. In sum, the current operational environment is far more sophisticated than at any time before, yet we’ve just barely scratched the surface of automation system capabilities.

Today, the operations team appears to be detached from controls processes, which puts more pressure on automation systems to not just control but also provide timely information to all parties concerned with plant operation — operations, maintenance, owners, and other specialists who can be called on to provide advice. Technology can enable personal interaction with an expert in a remote location who has real-time access to information from a process plant.

In the future, control systems will no longer control the process — they will supervise it! Plant functions such as operation, maintenance, and management will be tightly integrated across all plant functional areas, and data will be ubiquitous. The system will embrace the latest information and communication technologies (ICT), and multiple communication channels (some traditional and some personal, such as instant messaging) will be incorporated. Flexible, switchable interfaces will be at the heart of the systems. Additionally, the operator interface will evolve into a human computer interface that allows collaboration among all the interested parties, on- and off-site. This functionality, coupled with asset performance solutions, will allow all parties to participate in the operation and optimization of a plant.

This development is already taking a step forward. New "network-enabled cooperative groups," whose information is gathered from people’s private computer activities, are now available to the plant at a moment’s notice. A virtual network of experts is going to be built around production activities and will be supported by future automation systems.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Automation of heat power engineering process”

Make up the situation

Control questions

1. How did the power plant control evolve according to text?

2. Besides cheap computing and advanced software applications, what else needs to be developed to optimize plants?

3. What will control systems do with the process in the plants?

4. What will be built around production activities and will be supported by future automation systems

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Automation of heat power engineering process

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 22-23-24

The theme of the lesson: Heat and mass transfer

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Heat has been defined as energy that is being transferred across the boundaries of a system because of a temperature difference. This transfer may occur through the mechanism of conduction, convection, or radiation, either separately or in combination.

Heat is transferred by conduction through a solid, partly as a result of molecular collisions but primarily as a result of a flow of electrons which is induced by a temperature difference. Metals that are good conductors of electricity are also good conductors of heat. Poor conductors (good insulators) are solids that have low density because of the presence of large numbers of small pores or pockets containing air which reduce to a minimum the cross-sectional area of the solid material through which the electrons may flow. Conduction also occurs in liquids and gases at rest, that is, where there is no motion other than the random motion of the molecules. Since the energy is transferred as a result of random molecular collisions, the conductivity of liquids and gases is low as compared to the conductivity of solids.

Convection occurs when, either because of a difference in density or because of the operation of a fan or pump, a fluid flows across a hot or cold surface and exchanges energy with that surface. The heated or cooled fluid may then flow to some other region. Since convective heat transmission always involves a flowing fluid, the laws governing heat transfer by convection are closely related to the laws of fluid dynamics.

Radiation involves the transfer of energy through space in the form-of electromagnetic waves that are different from light waves only in their length (frequency). Since radiant energy travels in straight lines with the velocity of light and may be absorbed, reflected, or transmitted by the receiving surface in a manner similar to the action of light, the laws of optics are important in the study of radiant-energy transfer.

In general, a heat exchanger consists of a metal wall through which heat flows from one fluid to another. Heat transfer through the wall follows the laws of conduction. Heat transfer between the moving fluid and the wall involves convection, in addition to which radiation may be important at high temperatures.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Heat and mass transfer”

Make up the situation

Control questions

1. How would you describe heat?

2. How is heat transferred by conduction as stated in the text?

3. When does the convection occur?

4. What does the radiation involve?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Heat and mass transfer

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

8. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 25-26-27

The theme of the lesson: The use of solar energy

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

One of the uses of solar energy is its transformation into electric energy. Photoelectric converters operate not only aboard space vehicles. They are used to supply hard-to-reach sites, for instance, light-houses, communication facilities, etc. with electric power. Such installations can operate continuously for 20 years, and their capacity is up to 500 watts. They are reliable and do not need constantly handling by personal.

At present, mainly semiconductor silicon is used for the manufacture of photocells. Now the researchers have designed photocells on the basis of linking two materials in a single crystal-gallium arsenide and aluminum arsenide. They are most promising for the transformation of preliminary concentrated light since they continue to operate efficiently at temperature of over 200˚ C. Using heat that is re-leased in photocells one can raise the efficiency up to 30%. Their use in solar power station will greatly cut the cost of the photoelectric method of energy trans-formation.

The problem of wider uses of renewable energy sources – solar, tidal and geothermal ones is of great local importance. So far the practical use of solar energy is not very significant but the use of this energy can be profitable in many areas even now. Specialists designed water-heating installations for both seasonable and year-round operation. They have already built solar-powered homes and public buildings with hot water supply, heating and air-conditioning. Within the next few years experimental constructions will go on. After tests the best solutions will be used in standard designs.

The application of solar installations in agriculture has considerable effect. The experience in experimental solar-heating greenhouses has shown that, as compared with ordinary greenhouses that receive heat from boiler rooms, the expenses on vegetable-growing are reduced by 60% due to fuel savings alone. Solar-powered installations for drying farm products were also tested successfully.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “The use of solar energy”

Make up the situation

Control questions

1. What is the one of the uses of solar energy?

2. What is used for the manufacture of photocells?

3. What kind of a renewable energy sources are mentioned here?

4. Why does the application of solar installations in agriculture have considerable effect?

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of The use of solar energy

Recommended literature

Main literature:

1. Английский язык для инженеров: Учеб./ Т.Ю. Полякова, Е.В.Синявская, О.И. Тынкова, Э.С.Улановская. - 6- е изд., испр.- М.: Высшая школа, 2010.-

2. To score high level in reading Учебное пособие /Иванова М.А., Кузнецова Е.В., Лебедева И.С., Михеев А.И.. СПб изд. “Инфо-да”, 2011.

3. Английский для инженеров / И.П. Агабекян, П. И. Коваленко. – изд.4-е. – Ростов н/Д: Феникс, 2010. – 319.

4. Cambridge English for Scientists / Tamzen Armer, Bathany Cagnol. Cambridge University Press – 2011.

5. J.N. Miliar. English for thermal engineers. Turbines. Tomsk Polytechnic University Publishing House, 2012

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3.Орловская И.В.,Самсонова Л.С.,Скубриева А.И.Учебник английского языка для студентов технических университетов и вузов Москва 2013.

4. Kate Pickering& Jackie McAvoy Global English, Pre-Intermediate coursebook Macmillan 2010

5. Richard Acklam & AramintaCrace Total English, Pre-Intermediate Longman 2012

6. Sue Kay & Vaughan Jones. Inside Out, Pre-Intermediate Macmillan 2015

English for Electrical Engineering Course book and audio CDs (2) Cambridge University Press 2011

7. English for ICT Studies Course book and audio CDs (2) Cambridge University Press 2011

Unit 28-29-30

Modern heat insulating materials

A strong tendency of the increase requirements to modern heat insulating materials is being obviously seen nowadays. It is arisen by the resource-saving principle of the world industry development. Thus, the production of high efficient insulating materials is one of the most important task at present. The main function of insulating materials is the reduction of heat loss. Heat insulating materials must have a small factor of the thermal conductivity, a small volume mass, humidity resistance, mechanical strength, hard combustibility, frost resistance, homogeneous finely porous structure, bacterial and chemical resistance and environmental safety. Qualitative heat insulating materials are very expansive products. For example, the cost of insulating materials in refrigerators is 25-40% of the whole construction site. So, the question of economic profitability of producing material is vivid nowadays. Thus, the most important item of creating resource effective production technology arises today. The most significant role in this process is belonged to raw material resources. Existing heat insulating materials can’t meet all the above mentioned requirements. That’s why it is necessary to depict those materials which have the most optimal and economically effective characteristics. According to given estimated calculations and comparing analysis one of those materials is viralit (artificial foamed stone) . Getting possibility of viralit is produced by the nature. Under the volcano eruption the firing clinker is contacting with the earth crust and under the high temperature the thermal processing of the loams are being produced by the natural way. This natural phenomenon was noticed long ago by many famous scientists. The natural principle of loams processing has been realized for the technological base of viralit production (picture 1). The main properties of which are given in the table 1.

The Combination of high constructional strength, heat insulating properties and economical efficiency allow to build objects without using additional insulating materials and to reduce the damage and production cost price. Being developed a new power and resource-saving technology of getting porous glass-ceramic material “Viralit” has a very wide application in such industrial branches as building, power energy, nuclear power, metallurgical, chemical and others. Having used scientific principles are being assumed the further improvement of the produced technology and creating new high efficient materials of strategic meaning. “Viralit 1000” (picture 2), for example possess unique thermal physical characteristics with high strength properties in comparison with the existing thermal safety materials. Viralit’s technology allows governing by strength and heat insulating properties of being get material at the wide ranges, and modify material’s properties according to the product’s thickness, overlapping heat insulating and ornate finishing functions.

For example, the characteristic property of this technology will allow regulating the heat extraction out of the cooled melt. The process impacts on the shrinkage of the crystallized material and considerably influence the quality of the getting product in the technological processes of metallurgical industry [27-29]. “Viralit 1000” will be taken as the used material for casting molds. Taking into consideration the condition of the heat exchange under melting [30] of the viralit construction, one can considerably reduce the velocity destruction material in the case of the emergency situation at the nuclear station. The production of sandwich large block material (picture 3) allows creating building objects which are the perfecting product’s development.

Revision questions

What materials are called heat insulating?

What properties must heat insulating materials possess?

What material’s properties of heat insulating material are the most significant?

In what way do you understand the resource-saving principle?

What heat insulating materials are being regarded high-performance?

What technical areas are these materials being used?

What heat insulating materials do you know?

What is the economic efficiency of produced material?

What is the economic facet of raw material resource?

What is treating principle of raw materials being laid in the technological base

of “Viralit”?

What characteristics of material does the technology “Viralit” allow to govern?

What main functions does the material possess?

What structure does the glass ceramic material “Viralit” possess?

What industrial spheres can be the material used?

What do you mean by high performance material of strategic trend?

What the benefits does “Viralit 1000” possess in comparison?

What are the perspectives of enhancement “Viralit 1000” existing?

In what way the use of “Viralit 1000” can influence the product’s quality in the

technological process of metallurgical industry?

What properties of “Viralit 1000” do allow reducing the destruction velocity of

technological construction in the case of emergency situation at the objects of

nuclear industry?

Glossary

Adiabatic process - a process taking place without heat transfer of the working fluid to the environment.

Boiling - intensive process of vaporization of liquid (transfer of a substance from a liquid to a gaseous state) with the occurrence of phase separation boundaries (the formation of bubbles or vapor film on the heating surface, their growth and movement in the volume of liquid).

Binary cycle - thermodynamic cycle, carried out by two working substances.

Carnot cycle - a reversible cyclic process (cycle), on which the most complete conversion of heat into work (or vice versa), consisting of two isothermal processes (heating and cooling) and two adiabatic processes (compression and expansion).

Compressor - a device for increasing the pressure in the working fluid.

Closed thermodynamic system - a system that does not change with the environment substance.

Circular process (or cycle) - a set of processes that result in the working fluid periodically returns to its original state.

Diffuser - a channel in which the deceleration of the flow taking place with the pressure increasing of the working fluid.

Density - the mass per unit volume.

Effect of throttling (Joule - Thomson) – the change of the working fluid temperature at adiabatic throttling.

The first law of thermodynamics - the universal application of the law of conservation and transformation of energy to the phenomena of the interconversion of heat and work.

Entropy - a thermodynamic state function of a thermodynamic system, a change in the equilibrium process which is the ratio of the amount of heat imparted to the system, or allocated to it, to the thermodynamic temperature of the system.

Enthalpy - the sum of the internal energy and potential energy of pressure.

Environment - the body that are not in thermodynamic system.

Evaporation - vaporization from the free surface of the condensed phase (in the case of solids - sublimation or distillation).

Equilibrium process - the process of transition of a thermodynamic system from one equilibrium state to another, in which the speed of the process is much less than the rate of relaxation

Exergy heat - maximum work done with the working fluid in a heat engine, if the cold source is the receiving environment.

Exergy flow of the body - the maximum work that can be obtained in a reversible transition to a state of thermodynamic equilibrium with the environment

Equation of state - an equation expressing the relationship between the parameters of all possible equilibrium states of a thermodynamic system.

Gas turbine engine (GTE) - heat engine for converting heat of combustion into kinetic energy of the jet and mechanical work around the engine.

Heat - a special form of energy transfer, which in contrast to the work is not associated with a visible movement of the body.

Heat capacity - the amount of heat required for heating a substance by 1 degree.

Heat of vaporization - the amount of heat required to convert 1 kg of liquid heated to the boiling point in the dry saturated steam at a constant pressure (temperature).

Ideal gas - gas, which lacks the force of interaction between the molecules in the distance, and the size of the latter, is negligible compared to the mean free path.

Insulated (adiabatic) system - a system that has no the ability to exchange heat with the environment.

The internal combustion engine (ICE) - a heat engine in which fuel is burned to produce mechanical work.

Internal energy - the amount of energy of all kinds of motion and interaction of particles that make up the substance.

Inversion temperature - the temperature corresponding to a state of the working fluid, under which the process of the adiabatic throttling isn’t not changed.

Irreversible process - non-equilibrium process, which can take place only in one direction.

Isobaric process - thermodynamic process occurring at constant pressure.

Isothermal process - thermodynamic process that occurs at constant temperature.

Isochoric process - thermodynamic process that occurs at constant volume.

Jet engine - a device in which the chemical energy of the fuel is converted into the energy of the jet working substance.

Laval nozzle - combined nozzle for obtaining supersonic velocities of the working substance.

Magnetic hydrodynamic (MHD) generator - setting the direct conversion of heat energy into electrical energy by the passage of a plasma in a magnetic field.

Nozzle - channel, which is an increase in the speed of the working substance.

No equilibrium process - a process that the flow rate is greater than or comparable to the rate of relaxation.

Open thermodynamic system - a system that communicates with the environment and material, and work and energy.

Parameters of the state - the physical quantities that uniquely define the state of a thermodynamic system and change the values in the commission process.

Polytropic process - the process of changing the state of the working fluid, which during the process heat is constant.

Pressure - the force with which the gas (or vapor) acts on a unit area of its shell.

Regeneration - the use of exhaust gas heat (or steam) for heating the incoming air, water and fuel to the plant.

Reciprocating compressor - compressor, in which compressed gas is in the cylinder under the piston.

Reversible process - equilibrium process, which can occur in both the forward and back through all of the same intermediate states.

Refrigeration cycle - reverse circular process that is used to transfer heat from the less heated body to the bodies of more heated with the expenditure for this work.

Rankine cycle - theoretical thermodynamic cycle simple steam power plants using the same body of water (closed loop) and consisting of four basic operations:

1. evaporation of the liquid at high pressure

2. expansion of steam;

3. condensation

4. increased pressure condensate to the initial level.

Saturated steam (wet, humid) - vapor in thermodynamic equilibrium with the liquid or solid of the same composition.

Superheated steam - steam heated to a temperature above the boiling point at a given pressure.

Specific heat capacity - the amount of heat required for heating a unit of the substance by 1 degree.

Specific volume - the volume of a unit mass of material.

The second law of thermodynamics - sets the conditions of thermodynamic processes flow conversion of heat into work.

Temperature - a measure (or degree) of a heated body.

Thermodynamics - the science of the laws of the interconversion of heat and work, and properties of the body involved in these transformations.

Thermodynamic system - a set of material objects that are in interaction with others their bodies in the form of an exchange of energy, work and material.

Thermal efficiency - the ratio of the work performed in the cycle, and let down the heat to the working fluid.

Thermodynamic equilibrium - state is characterized by the equality of temperatures (thermal equilibrium) and pressures (mechanical equilibrium) at all points of the volume occupied by the working fluid.

Thermodynamic process - any change that occurs in a thermodynamic system and associated with the change at least one of its state variable.

The third law of thermodynamics (Nernst theorem) - unreachable absolute zero.

Throttling - the process of reducing the pressure of the working substance in overcoming the local hydraulic resistance no useful work.

Turbocharger - centrifugal or axial vane compressor for compressing and supplying of the working substance.

Vaporization - transition of a substance from a condensed phase (liquid or solid) into the gas phase.

Working substance - the substance through which the energy conversion done.

Criteria for assessing the knowledge of students

| Estimation in letters | Equivalent of scores in figures | In per cent % | Traditional assessment | Criteria for assessing the knowledge of students |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | Excellent | The student demonstrates: - a deep and lasting assimilation of program material - complete, consistent, competent and logically presented answers for all types of activities; - ability to freely cope with the tasks; - correct, well-founded decisions, - skills of using the information of basic and additional specialized literature in the study of discipline, - the ability to self-systematize the program material; - possession of versatile skills and methods of performing all types of tasks; -ability to work with foreign literature and information resources of the Internet; - timely and high-quality fulfillment of all types of tasks. |
| А- | 3,67 | 90-94 |  | The student demonstrates: - a deep assimilation of program material; - complete, consistent and logically presented answers for all activities; - ability to cope with the tasks; - correct taken decisions , - skills of using special literature in the study of discipline, - the ability to self-systematize the program material; - possession of skills and techniques for performing all types of tasks; - timely fulfillment of all types of tasks. |
| В+ | 3,33 | 85-89 | Good | The student demonstrates: - mastering the program material; - complete, consistent, literate, without significant inaccuracies, an account of the answers for all kinds of tasks; - the correct application of theoretical knowledge - the possession of necessary skills in the performance of applied tasks - the skills of using recommended literature in the study of discipline, - the skills of systematizing program material; - possession of skills and techniques for performing all types of tasks; - timely fulfillment of all types of tasks. |
| В | 3,0 | 80-84 |  | The student demonstrates: - mastering the program material; - a consistent presentation of the answers for all types of tasks with non-essential mistakes; - skills in applying theoretical knowledge under the guidance of a teacher; - possession of necessary skills in performing practical tasks - skills of using recommended literature in the study of discipline, - skills of systematizing program material under the guidance of a teacher; - mastering the skills of performing all kinds of tasks; - the ability to independently correct mistakes; - timely fulfillment of all types of tasks with the elimination of mistakes. |
| В- | 2,67 | 75-79 |  | The student demonstrates: - mastering the program material; - ability to present answers with non-essential mistakes; - skills in applying theoretical knowledge under the guidance of a teacher; - possession of skills in performing practical tasks - skills in using recommended literature under the guidance of a teacher - generalization skills of program material under the guidance of a teacher; - the ability to correct mistakes with the help of a teacher; - timely fulfillment of all types of tasks with the elimination of mistakes. |
| С+ | 2,33 | 70-74 | Good | The student demonstrates: - mastering the basic material; - inadequate formulations when responding to all kinds of tasks; - violation of the sequence in the presentation of the program material; - difficulties in self-fulfillment of practical tasks; - possession of individual techniques in the performance of practical tasks - skills of using the literature recommended by the teacher; - skills of generalization of separate sections of program material under the guidance of a teacher; - the ability to correct major mistakes with the help of a teacher; - fulfillment of all types of tasks with elimination of mistakes. |
| С | 2,0 | 65-69 | Satisfactory | The student demonstrates: - mastering the basic material; - inadequate understanding of formulations when answering all types of tasks; - lack of consistency in the presentation of the material; - difficulties in self-fulfillment of practical tasks; - possession of individual techniques in the performance of tasks - difficulties in using the literature recommended by the teacher; - difficulties in generalizing the individual sections of the material studied; - the ability to correct major mistakes with the help of a teacher; - fulfillment of all types of tasks with elimination of mistakes. |
| С- | 1,67 | 60-64 | Satisfactory | The student demonstrates: - mastering the basic material; - inadequate understanding of formulations when answering all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - inadequate possession of individual methods in the performance of tasks - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the studied material; - the ability to correct major mistakes with the help of a teacher; - fulfillment of all types of tasks with elimination of mistakes. |
| D+ | 1,33 | 55-59 | Satissatisfactory factory | The student demonstrates: - mastering of separate sections of the main material; - misunderstanding of formulations when answering all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - significant difficulties in the application of individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the studied material ; - difficulties in correcting the mistakes with the teacher; - untimely fulfillment of all types of tasks with elimination of mistakes. |
| D | 1,0 | 50-54 | Satisfactory | The student demonstrates: - difficulties in mastering individual sections of the main material; - misunderstanding of formulations when answering all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - inability to apply individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - inability to generalize the individual sections of the studied material ; - significant difficulties in correcting the mistakes with teacher; - untimely fulfillment of all types of tasks with elimination of mistakes. |
| F | 0,5 | 25-49 | Unsatisfactory | The student demonstrates: - no knowledge of the program material, - gross mistakes are made when performing all types of tasks; - lack of skills in the usage of individual methods of performing tasks; - failure to complete certain types of tasks, provided by the forms of the current, midterm and final control. |
| FX | 0 | 0-24 | Unsatisfactory | The student demonstrates: - ignorance of program material, - at performance of all types of tasks gross errors are allowed; - lack of skills in the usage of individual techniques for performing tasks; - failure to comply the tasks provided by the forms of current, midterm and final control. |

References

1. Conduction Heat Transfer, D. Poulikakos, Prentice Hall, 2011

2. Thermal Radiation Heat Transfer, R. Siegel and J. R. Howell, Taylor & Francis, 2012.

3. Heat Conduction, S. Kakac and Y. Yener, Taylor and Francis, 2013.

4. Conduction Heat Transfer, V.S. Arpaci, Addison Wesley, 1996 (Abridged edition Ginn press 1998)

5. Heat Transfer, A.J.Chapman, Macmillan, 2011

6. Duffie J. A, Beckman W. A., “Solar Engineering of Thermal Process”, Wiley, 3rd ed. 2016.

7. Khartchenko N.V., “Green Power: Eco-Friendly Energy Engineering”, Tech Books, Delhi, 2014.

8. Garg H.P., Prakash J., “Solar Energy Fundamentals and Applications”, Tata McGraw-Hill, 2015.

CONTENTS

| Introduction………………………………………………………………… | 3 |
| --- | --- |
| Unit 1-2 …………………………………………………. | 4-8 |
| Unit 3-4 …………………………………………………. | 9-15 |
| Unit 5-6 …………………………………………… | 16-21 |
| Unit 7-8...................................................................... | 22-25 |
| Unit 9-10 …………………………………………… | 26-29 |
| Unit 11-12 ………………………………………….. | 29-32 |
| Unit 13-14-15………………………………………...... | 32-37 |
| Unit 16-17-18 ………………………………………….. | 37-41 |
| Unit 19-20-21 …………………………………………… | 42-45 |
| Unit 22-23-24 ……………………………………………. | 45-48 |
| Unit 25-26-27…………………………………………… | 48-50 |
| Unit 28-29…………………………………………….. | 51-52 |
| Unit 30 ……………………………………………. | 51-53 |
| Glossary …………………………………………….. | 53-57 |
| Criteria for assessing the knowledge of students | 58-59 |
| References ……………………………………………………………….. | 60 |
| Content ………………………………………………………………. | 61 |

Ashenova A.S.

Methodical instructions for practical lessons

for 5В071700 «Heat-power engineering»

Signed for printing «__»___________ 2019

Paper format 60x 90  1/16

Typographical  paper. Offset printing.Volume 2.5 printer’s sheet.

Circulation 50 copies

© Edition of the South Kazakhstan State University

Publishing centre of the  M.Auezov South Kazakhstan State University,Shymkent