Ф.7.03-03

Abdualiyeva M.A., Tursуnbayeva K.А.

Methods for measuring physical quantities

Lecture notes for the students of specialities of

5B011000 – Physics

Ministry of Education and Science of the Republic of Kazakhstan

M.Auezov South Kazakhstan State University

dEPARTMENT of Physics

Abdualiyeva M.A., Tursуnbayeva K.А.

Methods for measuring physical quantities

Lecture notes for the students of specialities of

5B011000 – Physics

Shymkent, 2019

Abdualiyeva M.A., Tursуnbayeva K. Methods for measuring physical quantities.Lecture notes for the students of specialities of  5B011000 – Physics

-Shymkent: M.Auezov SKSU, 2019.- -----p.

Methods for measuring physical quantities.Lecture notes for the students of specialities of  5B011000 – Physics is developed in accordance with the requirement of State Educational Standarts and curriculum of the given subject.

Lecture notes for the students of specialities of  5B011000 – «Physics» is approved and recommended for publication by the department  of Physics meeting (protocol № 6 of 9.01.2019)

Lecture notes for the students of specialities of  5B011000 – «Physics»  is approved by the Committee on innovative technologies of training and methodical provision of the Natural Scientific- Pedagogical Higher School, 16.01.2019, minutes No.6.

Lecture notes for the students of specialities of  5B011000 – «Physics»   is approved and recommended for publication by Educational  Methodical Council of M. Auezov SKSU (protocol № __ of _________)

© M. Auezov SKSU, 2019

© Abdualiyeva M.A., Tursуnbayeva K.

Introduction. Problems in methods of measuring physical quantities. Purpose of the discipline.

Any measurement is connected with errors resulting from imperfection of human sense organs and measuring devices. In any measurements some errors are allowed, therefore measuring results give us not true but only approximate value of measured quantities. The stating of an admissible measuring error and interval, in which the true value of measured quantity lies is the required (necessary) condition of experiment reliability.

Measuring errors are divided into crude, systematic and random.

Crude errors are caused by the result of device (malfunction) derangement or observer’s carelessness, disarrangement of experimental technique or conditions of its carrying out. In more cases, misses are well noticeable, since measuring indications corresponding to them violently differ from those of the like ones. Measuring results containing the miss should not be taken into consideration in data processing operation - it should be just omitted.

Systematic errors appear in the result of disarrangement or inaccuracy of calibration of measuring devices, in the use for analysis of non-precise data and also are rooted from the imperfections of the experimental technique itself. These errors always influence on measuring results one-sidedly (only increase or decrease them). It is obvious that influence of systematic errors is impossible to decrease by the increase of measurement numbers. However, if the nature and the character of the systematic errors are known their influence on measuring results can be considered by introducing correction and to exclude.

Random errors are stipulated by fluctuations of the measured quantity. Their appearance cannot be prevented, therefore they may make a certain influence on individual measurements changing result to both sides that is either increase and decrease them. They obey the statistic laws, that’s why the influence of random errors on measuring results can be taken into account or considerably decrease. While applying laws of theory of probability the more probable values of measured quantities and possible deviations from these values are defined.

Assessment of accuracy (accuracy ranking) of multiple direct measuring results. Let us assume that in multiple measurements of some  physical quantity of its values were received:. In such a case arithmetical mean (average value) of all the received values, which is the most reliable, is equal to

.                    			(1)

Each individual measurement differs from the arithmetical mean by the magnitude equal to

(2)

Deviation  is called absolute error of i-th measurement.

Absolute errors  accept both positive and negative values.

It is known from the experience that if the greater the error there is the less probability of its appearance. Besides if the number of measurements is very great     (), the errors similar in value, but different in sign, will often equally occur. The rate of decrease of error occurrence probability  with its increase is characterized by dispersion. It is equal to

(3)

The less is , the less is the less probability of appearance of an error with greater magnitude , the less is the spread of individual values , i.e. the higher is  the accuracy of measurement.

In practice the number of measurements that can be carried out is often limited. Therefore n is practically chosen as the final one and as dispersion its approximate value is concerned:

(4)

Subsequently,

(5)

The square root of dispersion of measurements is called standard deviation (quadratic mean error):

(6)

Substituting instead of  in the expression (6) for its value from the equation (2), we obtain

(7)

The expression (7) includes the quantity but not. The quantity  itself is identified by some error.  The magnitude, equal to the quadratic mean error divided by square root from measuring numbers is called quadratic mean error of arithmetical mean:

,                                                            		(8)

оr

(9)

Variance estimate and are limitary, true only in, that is in great n. In small n these evaluations, which are accidental themselves, at the best case determine only the order of dispersion magnitude.

Mathematical processing of measuring results is interval determination of 	         (,), where the true value of measured quantity can be found with  probability. Interval (, ) is calledconfidential,  is confidence factor (reliability). If numbers of measurements  are great enough then the confidence factor presents the part of general number of measurements where the measured quantity makes an influence within confidence interval. For instance, if 100 measurements were done, then 95 measurements gave the results within its limits in confidence coefficient =0.95. It is clear, that the more reliability is required the more corresponding confidence interval is received, and vice versa, the more confidence interval is given it is more obvious that the measuring results will not go beyond its limits. It results from the theory of errors that in large number of measurements n (more than 100) confidence coefficient of the interval from [,] is equal to 68%, and of the interval [,] is equal to 95%. While presenting any measured quantity it is important to point out interval and confidence probability corresponding to this interval.

In cases when the number of measurements is not great then there the conditions for strict fulfillment of statistical regularity lying in the base of random errors determination are absence. It results in that the value of quadratic mean error (standard deviation)  calculated by the (9) formula is not exact and inaccuracy is more the number of measurements is less. Hence, to guarantee the true value of quantity measured with the given probability is within confidence interval, the latter should be increased. In limited number of measurements not, but absolute error

(10)

is accepted as the margin of confidence interval.

Numerical value of  depends on the quantity of confidence coefficient  and the number of measurements, and it is called the Student coefficient. Student coefficient value for various is available in the laboratories (Table 1).

Table 1.

|  | 0,6 | 0,7 | 0,9 | 0,95 | 0,99 |
| --- | --- | --- | --- | --- | --- |
| 2 | 1,38 | 2,01 | 6,31 | 12,71 | 63,66 |
| 3 | 1.06 | 1,3 | 2,92 | 4,30 | 9,92 |
| 4 | 0,98 | 1,2 | 2,35 | 3,18 | 5,84 |
| 5 | 0,94 | 1,1 | 2,13 | 2,78 | 4,60 |
| 6 | 0,92 | 1,1 | 2,02 | 2,57 | 4,03 |
| 7 | 0,90 | 1,1 | 1,94 | 2,45 | 3,71 |
| 8 | 0,90 | 1,1 | 1,90 | 2,36 | 3,50 |
| 9 | 0,90 | 1,1 | 1,86 | 2,31 | 3,36 |
| 10 | 0,90 | 1,1 | 1,83 | 2,26 | 3,25 |

Measuring result is performed in the following way:

(11)

Results of different quantity measurements cannot be compared with each other according to their absolute errors. To compare measurement accuracy of such quantities relative error, that is relevance of absolute error  to the average value of measured quantity

(12)

is introduced. It is convenient also to compare measuring results and homogenous quantities using relative error.

Physical quantities and their units. Additional physical quantities and their units. Physical Derivatives.

Theoretical introduction. Accuracy assessment of one direct measuring result: If in the process of multiple measurements the device gives one and the same indications, then the repetitions of measurements loses its significance; it’s enough to measure once. This will evidently happen in case when absolute error of working measuring device will be more than the residual errors of individual measurements. Limited absolute error either is shown in technical certificate of measuring device or is determined by the half of price of the least division of its scale.

Example.  While measuring cube sides by sliding caliper the value  was deduced. Scale division of sliding caliper is. Hence, limited absolute error of measurement is equal to. Limited relative error of measurement is equal to relative error of beam compass:

(1)

Measuring result should be written in the following way:

(2)

In the laboratory works often values of some quantities measured beforehand are given: there are physical constants,figure π, and table data. In such cases absolute error is considered to be equal to its limited value, i.e. equal to the half unit of the least category, presented in figure.  For instance, if the mass of a body is given  then, hence,  and for  we have, and etc.

Accuracy ranking of indirect measuring results. Generally, an unknown quantity is the function of one or more quantities. For example, an electric current strength I in conductor is the function of potential difference U at the conductor ends and resistance R. Therefore, for determination of such quantity at first it is necessary to hold the set of such measurement of auxiliary quantities, and then using the formulae of physical laws to calculate unknown quantity. In this case we say about indirect measurements.

We suppose that the unknown quantity A is the function of

measured quantity x:

- (3)

Absolute error of measured quantity leads to the error of, .  Expansion of the right-hand in Taylor series gives:

(4)

We will be restricted by two terms of this series, as other terms are infinitely small of higher order. Then:

(5)

According to (2.3), we have:

(6)

That is, absolute error of the function is equal to product of derivative of this function and absolute error of the argument.

Let unknown quantity A depends on n variables:

(7)

Then the differential of multivariable function can be presented as:

,				(8)

where  is partial derivatives. In calculating the latter all the arguments except, are considered as constant and the  differentiation is carried out in the way as the function of one variable.

Instead of absolute error it is often convenient to figure out relative error. Relative error can be presented as:

(9)

When the function, as it is often occurs, has monomial form, then either development of formula or calculations according to them turn out to be considerably simple, if to use the rule (9). Expression (8) can be used for error determination if to change differential sign into error sign:

(10)

or                  			(11)

Overall result is recorded as:

(12)

Mechanical, molecular physics and thermodynamic, electrical and magnetic values and their units.

There are two somewhat different meanings of the term ‘physical quantity’.

One refers to the abstract metrological concept (e.g., length, mass, temperature),

the other to a specific example of that concept (an attribute of a specific object or system: diameter of a steel cylinder, mass of the proton, critical temperature

of water). Sometimes it is important to distinguish between the two and, ideally, it might be useful to be able to do so in all instances. However little is to be gained by attempting to make that distinction in this report. The primary concern here is with symbols and terminology in general; however, gives the symbols and numerical values of specific physical constants.   A physical quantity  is expressed as the product of a numerical value (i.e., a pure number) and a unit:

physical quantity = numerical value × unit.

For a physical quantity symbolized by a, this relationship is represented in the  Form a = {a} · [a], where {a} stands for the numerical value of a and [a] stands for the unit of a. Neither the name nor the symbol for a physical quantity should imply any particular choice of unit. When physical quantities combine by multiplication or division the usual rules of arithmetic apply to both the numerical values and to the units. A quantity which arises (or may be considered to arise) from dividing one physical quantity by another with the same dimension has a unit which may be symbolized by the number 1; such a unit often has no special name or symbol and the quantity is expressed as a pure number.

Examples:

E = 200 J

F = 27 N/m 2 n = 1.55 (refractive index)

f = 3 × 10 8 Hz

Symbols for physical quantities should be single letters of the Latin or Greek alphabet with or without modifying signs (subscripts, superscripts, primes, etc.). The two-letter symbols used to represent dimensionless combinations of physical quantities are an exception to this rule. When such a two-letter symbol appears as a factor in a product it should be separated from the other symbols by a dot, by a space, or by parentheses. It is treated as a single symbol and can be raised to a positive or negative power without using parentheses. Abbreviations (i.e., shortened forms of names or expressions, such as p.f. for partition function) may be used in text, but should not be used in physical equations. Abbreviations in text should be written in ordinary roman type. Symbols for physical quantities and symbols for numerical variables should be printed in italic (sloping) type, while descriptive subscripts and numerical subscripts are to be printed in roman (upright) type.

Examples:

C g (g = gas) C p

g n (n = normal)

µ r (r = relative)

E k (k = kinetic)

e (e = electric)

It is convenient to use symbols with distinctive typefaces in order to distinguish between the components of a vector (or a tensor) and the vector (or

tensor) as an entity in itself, or to avoid the use of subscripts. The following standard conventions should be adhered to whenever the appropriate typefaces are available: (a) Vectors should be printed in bold italic type, e.g., a, A. (b) Tensors should be printed in slanted bold sans serif type, e.g., S, T. Remark: When such type is not available, a vector may be indicated by an arrow above the symbol: e.g.,

Simple mathematical operations Addition and subtraction of two physical quantities are indicated by: a + b and a − b.

Multiplication of two physical quantities may be indicated in one of the Multiplication of two physical quantities may be indicated in one of the following ways:

ab a · b a × b.

Symbols for units

The full name of a unit is always printed in lower case roman (upright) type.

If that name is derived from a proper name then its abbreviation is a one or two letter symbol whose first letter is capitalized. The symbol for a unit whose name is not derived from a proper name is printed in lower case roman type.

Examples:

Metre m, ampere A, watt W, weber Wb.

Remark: Although by the above rule the symbol for litre is l, in order to avoid confusion between the letter l and the number 1, the symbol may also bewritten L.

Symbols for units do not contain a full stop (period) and remain unaltered in Symbols for units do not contain a full stop (period) and remain unaltered in the plural.

Example:

7 cm and not 7 cm. or 7 cms

Prefixes

The prefixes that should be used to indicate decimal multiples or submultiples

of a unit are given in table 1. Compound prefixes formed by the juxtaposition

of two or more prefixes should not be used. Not : mµs , but : ns (nanosecond)

Not : kMW , but : GW (gigawatt)

Not : µµF , but : pF (picofarad)

When a prefix symbol is used with a unit symbol the combination should be considered as a single new symbol that can be raised to a positive or negative power without using brackets.

In a system consisting of a set of physical quantities and the relational equations connecting them, a certain number of quantities are regarded by convention as dimensionally independent and form the set of base quantities for

the whole system. All other physical quantities are derived quantities, defined in

terms of the base quantities and expressed algebraically as products of powers of the base quantities. In a similar way, a system of units is based on a set of units chosen by convention to be the units of the base quantities, and all units for derived

quantities are expressed as products of powers of the base units, analogous to the corresponding expressions in the system of quantities. When the derived units are expressed in terms of the base units by relations with numerical factors equal to unity, the system and its units are said to be coherent. The number of base units of the unit system is equal to that of the corresponding set of base quantities. The base units themselves are defined samples of the base quantities.

The expression of a quantity as a product of powers of the base quantities

(neglecting their vectorial or tensorial character and all numerical factors including their sign) is called the dimensional product (or the dimension) of the quantity with respect to the chosen set of base quantities or base dimensions. The powers to which the various base quantities or base dimensions are raised are called the dimensional exponents; the quantities and the corresponding units are of the same dimension.

Derived units and their symbols are expressed algebraically in terms of base units by means of the mathematical signs for multiplication and division. Some

derived units have received special names and symbols, which can themselves be

used to form names and symbols of other derived units.

Physical quantities that have as their dimension a product of powers of

the base dimensions with all exponents equal to zero are called dimensionless

quantities. The values of dimensionless quantities (e.g., relative density, refractive index) are expressed by pure numbers. The corresponding unit, which is the ratio of a unit to itself, is usually not written; if necessary it may be expressed by the number 1. Since the primary purpose of a system of units is to provide a basis for the transformation of the numerical values of physical quantities under a transformation of units, and since dimensionless quantities are invariant to such a transformation, there is no need to include quantities like plane angle and solid angle in the category of base quantities. Plane angle is usually considered to be a derived quantity, defined in terms of the ratio of two lengths, and solid angle, a derived quantity defined in terms of the ratio of an area to the square of a lenghth. Nevertheless, in some situations (notably in statistical physics, in particle transport and radiative transfer and particularly in photometry and illumination) the steradian must be treated as a base unit in order to avoid ambiguity and to distinguish between units corresponding to different quantities.

The International System of Units (SI)

The name Systeme International Unites (International System of Units) with the international abbreviation SI was adopted by the Conference Genraledes Poids et Mesures (CGPM) in 1960. The metre is the length of the path travelled by light in vacuum during a time interval of 1/299792 458 of a second. The ampere is that constant current which, if maintained in two straight parallel conductors of infinite length, of negligible circular cross-section, and placed 1 metre apart in vacuum, would produce between these conductors a force equal to 2 × 10-7 newton per metre of length.

Quantities and their units in optical, atomic and nuclear physics.

We define a physical quantity either by specifying how it is measured or by stating how it is calculated from other measurements. For example, we define distance and time by specifying methods for measuring them, whereas we define average speed by stating that it is calculated as distance traveled divided by time of travel.

Measurements of physical quantities are expressed in terms of units, which are standardized values. For example, the length of a race, which is a physical quantity, can be expressed in units of meters (for sprinters) or kilometers (for distance runners). Without standardized units, it would be extremely difficult for scientists to express and compare measured values in a meaningful way.

# There are two major systems of units used in the world: SI units (also known as the metric system) and English units (also known as the customary or imperial system). English units were historically used in nations once ruled by the British Empire and are still widely used in the United States. Virtually every other country in the world now uses SI units as the standard; the metric system is also the standard system agreed upon by scientists and mathematicians. The acronym “SI” is derived from the French Système International.

# SI Units and Symbols used in the Guide

| Subject | Physical Quantity | Symbol | Name | Unit |
| --- | --- | --- | --- | --- |
| Mechanics | Mass | m, M | kilogram | kg |
|  | Linear position Length, Distance Radius | x, r l, d R | meter | m |
|  | Time | t, | second | s |
|  | Linear angle, Angular position | , | radian | rad |
|  | Spherical angle |  | steradian | sr |
|  | Area | A | - | m2 |
|  | Volume | V | - | m3 |
|  | Moment of inertia | I | - | kg*m2 |
|  | Density |  | - | kg/m3 |
|  | Linear velocity | v, u, c | - | m/s |
|  | Angular velocity | , | - | rad/s |
|  | Linear momentum | p | - | kg*m/s |
|  | Angular momentum | L | - | kg*m2/s |
|  | Linear acceleration | a | - | m/s2 |
|  | Angular acceleration |  | - | rad/s2 |
|  | Force | F | newton | N=kg*m/s2 |
|  | Torque |  | - | N*m |
|  | Impulse | I | - | N*s |
|  | Work Energy | W E | joule | J=N*m |
|  | Power | P | watt | W=J/s |
|  | Dynamic viscosity |  | - | Pa*s |
| Electricity and Magnetism | Current | I | ampere | A |
|  | Charge | Q, q, e | coulomb | C=A*s |
|  | Current density | j | - | A/m2 |
|  | Volume charge density |  | - | C/m3 |
|  | Surface charge density |  | - | C/m2 |
|  | Linear charge density |  | - | C/m |
|  | Electric potential Voltage emf | V | volt | V=J/C |
|  | Electric field | E | - | N/C, V/m |
|  | Electric flux |  | - | V*m |
|  | Electric moment | pe | - | C*m |
|  | Resistance | R, r | ohm | =V/A |
|  | Specific resistance |  | - | *m |
|  | Capacitance | C | farad | F=C/V |
|  | Specific conductivity |  | - | (*m)-1 |
|  | Magnetic field | B | tesla | T=N/(A*m) |
|  | Magnetic flux |  | weber | Wb=T*m2=V*s |
|  | Inductance Mutual-inductance | L M | henri | H=Wb/A |
|  | Magnetic moment | pm | - | A*m2 |
|  | Polarization | P | - | C/m2 |
|  | Magnetization | I | - | A/m |
| Thermodynamics | Temperature | T | kelvin | K |
|  | Substance quantity | M | mole | mol |
|  | Pressure | P | - | Pa |
|  | Heat | Q | - | J |
|  | Heat capacity Entropy | C S | - | J/K |
|  | Specific heat | c | - | J/(kg*K) |
|  | Molar heat | cm | - | J/(mol*K) |
|  | energy flux | j | - | W/m2 |
|  | Surface tension |  | - | N/m |
|  | Stress Elasticity modulus | E | pascal | Pa=N/m2 |
| Oscillations and Waves | Wavelength |  | - | m |
|  | Wave number | k | - | m-1 |
|  | Frequency | f | hertz | Hz |
|  | Energy density |  | - | J/m3 |
|  | Energy flux | J | - | J/m2 |
|  | Intensity | I | - | J/(m2*s) |
|  | Reactance Impedance | X Z | ohm | =V/A |
| Optics | Focal length | f | - | m |
|  | Luminous intensity | I | candela | cd |
|  | Luminous flux |  | lumen | lm=cd*m2 |
|  | Illuminance | E | lux | lk=lm/m2 |
|  | Brightness | L | - | cd/m2 |
|  | Linear absorption coefficient |  | - | m-1 |
| Quantum Physics | Mass absorption coefficient |  | - | m2/kg |
|  | Radioactive activity | A | becquerel | Bq=s-1 |
|  | Absorbed dose | D | gray | Gy=J/kg |

Processing and registration of measurement results. Types of measurement errors. Random errors and their processing. Types and methods of measurement. Processing of the results of direct measurements. Processing of indirect measurement results. Accurate accuracy.

Random errors in experimental measurements are caused by unknown and unpredictable changes in the experiment. These changes may occur in the measuring instruments or in the environmental conditions.

Examples of causes of random errors are:

- electronic noise in the circuit of an electrical instrument,

- irregular changes in the heat loss rate from a solar collector due to changes in the wind.

Random errors often have a Gaussian normal distribution (see Fig. 2). In such cases statistical methods may be used to analyze the data. The mean m of a number of measurements of the same quantity is the best estimate of that quantity, and the standard deviation s of the measurements shows the accuracy of the estimate. The standard error of the estimate m is s/sqrt(n), where n is the number of measurements.

Fig. 1. The Gaussian normal distribution. m = mean of measurements. s = standard deviation of measurements. 68% of the measurements lie in the interval m - s < x < m + s; 95% lie within m - 2s < x < m + 2s; and 99.7% lie within m - 3s < x < m + 3s.

The precision of a measurement is how close a number of measurements of the same quantity agree with each other. The precision is limited by the random errors. It may usually be determined by repeating the measurements.

## Systematic Errors

Systematic errors in experimental observations usually come from the measuring instruments. They may occur because:

- there is something wrong with the instrument or its data handling system, or

- because the instrument is wrongly used by the experimenter.

Two types of systematic error can occur with instruments having a linear response:

- Offset or zero setting error in which the instrument does not read zero when the quantity to be measured is zero.

- Multiplier or scale factor error in which the instrument consistently reads changes in the quantity to be measured greater or less than the actual changes.

These errors are shown in Fig. 1. Systematic errors also occur with non-linear instruments when the calibration of the instrument is not known correctly. Examples of systematic errors caused by the wrong use of instruments are:

- errors in measurements of temperature due to poor thermal contact between the thermometer and the substance whose temperature is to be found,

- errors in measurements of solar radiation because trees or buildings shade the radiometer.

The accuracy of a measurement is how close the measurement is to the true value of the quantity being measured. The accuracy of measurements is often reduced by systematic errors, which are difficult to detect even for experienced research workers.

The measurement of an amount is based on some international standards which are completely accurate compared with others. Generally, measurement of any quantity is done by comparing it with derived standards with which they are not completely accurate. Thus, the errors in measurement are not only due to error in methods, but are also due to derivation being not done perfectly well. So, 100% measurement error is not possible with any methods. It is very important for the operator to take proper care of the experiment while performing on industrial instruments so that the error in measurement can be reduced. Some of the errors are constant in nature due to the unknown reasons, some will be random in nature,  and the other will be due to gross blunder on the part of the experimenter.

Errors in Measurement System

An error may be defined as the difference between the measured value and the actual value. For example, if the two operators use the same device or instrument for finding the errors in measurement, it is not necessary that they may get the similar results. There may be a difference between both measurements. The difference that occurs between both the measurements is referred to as an ERROR.

Sequentially, to understand the concept of errors in measurement, you should know the two terms that define the error. They are true value and measured value. The true value is impossible to find out the truth of quantity by experimental means. It may be defined as the average value of an infinite number of measured values. Measured value can be defined as the estimated value of true value that can be found by taking several measured values during an experiment.

Types of Errors in Measurement System

Generally errors are classified into three types: systematic errors, random errors and blunders.

1)GrossErrors
2)Blunders
3) Measurement Errors

Systematic Errors

- Instrumental Errors

- Environmental Errors

- Observational Errors

- Theoretical Errors

Random Errors

Types of Errors in Measurement System

1) Gross Errors

Gross errors are caused by mistake in using instruments or meters, calculating measurement and recording data results. The best example of these errors is a person or operator reading pressure gage 1.01N/m2 as 1.10N/m2. It may be due to the person’s bad habit of not properly remembering data at the time of taking down reading, writing and calculating, and then presenting the wrong data at a later time. This may be the reason for gross errors in the reported data, and such errors may end up in calculation of the final results, thus deviating results.

2) Blunders

Blunders are final source of errors and these errors are caused by faulty recording or due to a wrong value while recording a measurement, or misreading a scale or forgetting a digit while reading a scale. These blunders should stick out like sore thumbs if one person checks the work of another person. It should not be comprised in the analysis of data.

3) Measurement Error

The measurement error is the result of the variation of a measurement of the true value. Usually, Measurement error consists of a random error and systematic error. The best example of the measurement error is, if electronic scales are loaded with 1kg standard weight and the reading is 10002grams, then

The measurement error is = (1002grams-1000grams) =2grams

Measurement Errors are classified into two types: systematic error and random errors

Systematic Errors

The Systematic errors that occur due to fault in the measuring device are known as systematic errors. Usually they are called as Zero Error – a positive or negative error. These errors can be detached by correcting the measurement device. These errors may be classified into different categories.

Systematic Errors

In order to understand the concept of systematic errors, let us classify the errors as:

- Instrumental Errors

- Environmental Errors

- Observational Errors

- Theoritical

Instrumental Errors

Instrumental errors occur due to wrong construction of the measuring instruments. These errors may occur due to hysteresis or friction. These types of errors include loading effect and misuse of the instruments. In order to reduce the gross errors in measurement, different correction factors must be applied and in the extreme condition instrument must be recalibrated carefully.

Environmental Errors

The environmental errors occur due to some external conditions of the instrument. External conditions mainly include pressure, temperature, humidity or due to magnetic fields. In order to reduce the environmental errors

Observational Errors

As the name suggests, these types of errors occurs due to wrong observations or reading in the instruments particularly in case of energy meter reading. The wrong observations may be due to PARALLAX. In order to reduce the PARALLAX error highly accurate meters are needed: meters provided with mirror scales.

Theoretical Errors

Theoretical errors are caused by simplification of the model system. For example, a theory states that the temperature of the system surrounding will not change the readings taken when it actually does, then this factor will begin a source of error in measurement.

Random Errors

Random errors are caused by the sudden change in experimental conditions and noise and tiredness in the working persons. These errors are either positive or negative. An example of the random errors is during changes in humidity, unexpected change in temperature and fluctuation in voltage. These errors may be reduced by taking the average of a large number of readings.

Measurement Error Calculation

There are several ways to make a reasonable measurement error calculation such as estimating random errors and estimating systematic errors.

Estimating Random Errors

There are a number of ways to make a reasonable estimate of the random error in a particular measurement. The best way is to make a series of measurements of a given quantity (say, x) and calculate the mean and standard deviation (x ̅  & σ_x ) from this data.

The mean x ̅   is defined as

Where, Xi is the result of the i th measurements

‘N’ is the number of measurements

The standard variation is given by

If a measurement is repeated many times, then 68% of the measured valves will drop in the range  x ̅ ± σ_x

We become more positive that , is an accurate representation of the true value of the quantity x ̅ . The standard deviation of the mean σ_x is defined as

σ_(x ̅  )=σ_x⁄√N

The quantity σ_x is a good estimate of our uncertainty in  x ̅.  Notice that the measurement precision increases in proportion to √N  as we increase the number of measurements. The following example will clarify these ideas. Assume you made the following five measurements of a length:

Error Calculations

There fore, the result is 22.84±.08mm

In some cases, it is hardly useful to repeat a measurement many times. In that situation, you can estimate frequently the error by taking account of the smallest division of the measuring instrument.

For example, when using a meter stick, one can measure, perhaps a half or sometimes even a fifth of a millimeter. So, the absolute error would be estimated to be around 0.5 mm or 0.2 mm.

Thus, this is all about the various types of errors in measurement and error measurement calculation. We hope you are satisfied with this article. We express our gratitude to all the readers. Please share your suggestions and comments in the comment section below.

Instruments for measuring length, distances, areas, volumes and angles.Measurement line. Scale. Vernier. Calipers. Micrometer.

Goniometer. Vertical optimizer. Measuring microscope. Optical method

The Vernier scale is attached to the caliper’s slider and each division on this scale is made 0.05mm shorter than one main scale division of 1 mm. This means that, as the caliper jaws open, each successive movement of 0.05mm brings the succeeding vernier scale line into coincidence with a main scale line and so indicates the number of 0.05mm units to be counted (although for convenience the scale is numbered in fractions of a mm). Alternatively, one vernier division may be made 0.05mm shorter than two divisions of the main scale to make a long vernier scale. This makes the scale easier to read but the principle, and graduation, is still the same.

About Long Calipers Steel rules are commonly used to roughly measure large workpieces but if a little more accuracy is needed then a long caliper is suitable for the job. A long caliper is very convenient for its user friendliness but does require some care in use. In the first place it is important to realize there is no relationship between resolution and accuracy. For details, refer to the values in our catalog.Resolution is constant whereas the accuracy obtainable varies dramatically according to how the caliper is used. The measuring method with this instrument is a concern since distortion of the main beam causes a large amount of the measurement error, so accuracy will vary greatly depending on the method used for supporting the caliper at the time. Also, be careful not to use too much measuring force when using the outside measuring faces as they are furthest away from the main beam so errors will be at a maximum here. This precaution is also necessary when using the tips of the outside measuring faces of a long-jaw caliper.

Inside Measurement with a CM-type Caliper Because the inside measuring faces of a CM-type caliper are at the tips of the jaws the measuring face parallelism is heavily affected by measuring force, and this becomes a large factor in the measurement accuracy attainable. In contrast to an M-type caliper, a CM-type caliper cannot measure a very small hole diameter because it is limited to the size of the stepped jaws, although normally this is no inconvenience as it would be unusual to have to measure a very small hole with this type of caliper. Of course, the radius of curvature on the inside measuring faces is always small enough to allow correct hole diameter measurements right down to the lowest limit (jaw closure). Mitutoyo CM-type calipers are provided with an extra scale on the slider for inside measurements so they can be read directly without the need for calculation, just as for an outside measurement. This useful feature eliminates the possibility of error that occurs when having to add the inside-jaw-thickness correction on a single-scale caliper. Small hole measurement with an M-type caliper A structural error d occurs when you measure the internal diameter of a small hole. øD: True internal diameter ød: Indicated internal diameter d: Measurement error (øD – ød) True internal diameter (D: 5mm)

1 Potential causes of error A variety of factors can cause errors when measuring with a caliper. Major factors include parallax effects, excessive measuring force due to the fact that a caliper does not conform to Abbe's Principle, differential thermal expansion due to a temperature difference between the caliper and workpiece, and the effect of the thickness of the knife-edge jaws and the clearance between these jaws during measurement of the diameter of a small hole. Although there are also other error factors such as graduation accuracy, reference edge straightness, main scale flatness on the main blade, and squareness of the jaws, these factors are included within the instrumental error tolerances. Therefore, these factors do not cause problems as long as the caliper satisfies the instrumental error tolerances. Handling notes have been added to the JIS so that consumers can appreciate the error factors caused by the structure of the caliper before use. These notes relate to the measuring force and stipulate that  “as the caliper does not have a constant-force device, you must measure a workpiece with an appropriate even measuring force. Take extra care when you measure it with the root or tip of the jaw because a large error could occur in such cases.”

2. Inside measurement Insert the inside jaw as deeply as possible before measurement. Read the maximum indicated value during inside measurement. Read the minimum indicated value during groove width measurement. 3. Depth measurement Read the minimum indicated value during depth measurement. 4. Parallax error when reading the scales Look straight at the vernier graduation line when checking the alignment of vernier graduation lines to the main scale graduation lines. If you look at a vernier graduation line from an oblique direction (A), the apparent alignment position is distorted by X as shown in the figure below due to a parallax effect caused by the step height (H) between the planes of the vernier graduations and the main scale graduations, resulting in a reading error of the measured value. To avoid this error, the JIS stipulates that the step height should be no more than 0.3 mm. 5. Moving Jaw Tilt Error If the moving jaw becomes tilted out of parallel with the fixed jaw, either through excessive force being used on the slider or lack of straightness in the reference edge of the beam, a measurement error will occur as shown in the figure. This error may be substantial due to the fact that a caliper does not conform to Abbe’s Principle. 6. Relationship between measurement and temperature The main scale of a caliper is engraved (or mounted on) stainless steel, and although the linear thermal expansion coefficient is equal to that of the most common workpiece material, steel, i.e. (10.2 ±1) ×10-6 / K, note that other workpiece materials, the room temperature and the workpiece temperature may affect measurement accuracy. 7. Handling Caliper jaws are sharp, and therefore the instrument must be handled with care to avoid personal injury. Avoid damaging the scale of a digital caliper and do not engrave an identification number or other information on it with an electric marker pen. Avoid damaging a caliper by subjecting it to impact with hard objects or by dropping it on a bench or the floor. 8. Maintenance of beam sliding surfaces and measuring faces Wipe away dust and dirt from the sliding surfaces and measuring faces with a dry soft cloth before using the caliper. 9. Checking and setting the origin before use Clean the measuring surfaces by gripping a sheet of clean paper between the outside jaws and then slowly pulling it out. Close the jaws and ensure that the vernier scale (or display) reads zero before using the caliper. When using a Digimatic caliper, reset the origin (ORIGIN button) after replacing the battery. 10. Handling after use After using the caliper, completely wipe off any water and oil. Then, lightly apply anti-corrosion oil and let it dry before storage. Wipe off water from a waterproof caliper as well because it may also rust. 11. Notes on storage Avoid direct sunlight, high temperatures, low temperatures, and high humidity during storage. If a digital caliper will not be used for more than three months, remove the battery before storage. Do not leave the jaws of a caliper completely closed during storage.

# Classification of electric measuring instruments. The measuring mechanisms of instruments used for direct measurement. Instruments for direct measurement of a current and voltage.

The instrument used for measuring the physical and electrical quantities is known as the measuring instrument. The term measurement means the comparison between the two quantities of the same unit. The magnitude of one of the quantity is unknown, and it is compared with the predefined value. The result of the comparison obtained regarding numerical value.

## Absolute Instrument

## The absolute instrument gives the value of measures quantities regarding the physical constant. The physical constant means the angle of deflection, degree and meter constant. The mathematical calculation requires for knowing the value of a physical constant.

The tangent galvanometer is the examples of the absolute instruments. In tangent galvanometer, the magnitude of current passes through the coil determines by the tangent of the angle of deflection of their coil, the horizontal component of the earth magnetic field, radius and the number of turns of wire used. The most common applications of this type of instrument are found in laboratories.

## Secondary Instrument

In the secondary instrument, the deflection shows the magnitude of the measurable quantities. The calibration of the instruments with the standard instrument is essential for the measurement. The output of this type of device is directly obtained, and no mathematical calculation requires for knowing their value

### Deflection Type Instrument

The instrument in which the value of measuring quantity is determined through the deflection of the pointer is known as the deflection type instrument. The measuring quantity deflects the pointer of the moving system of the instrument which is fixed on the calibrated scale. Thus, the magnitude of the measured quantity is known.

The deflection type instrument is further sub-classified into three types.

- Indicating Instrument – The instrument which indicates the magnitude of the measured quantity is known as the indicating instrument. The indicating instrument has the dial which moves on the graduated dial. The voltmeter, ammeter, power factor meter are the examples of the indicating instrument.

- Integrating Instrument – The instrument which measures the total energy supplied at a particular interval of time is known as the integrating instrument. The total energy measured by the instrument is the product of the time and the measures electrical quantities. The energy meter, watt-hour meter and the energy meter are the examples of integrating instrument.

- Recording Instrument – The instrument records the circuit condition at a particular interval of time is known as the recording instrument. The moving system of the recording instrument carries a pen which lightly touches on the paper sheet. The movement of the coil is traced on the paper sheet. The curve drawn on the paper shows the variation in the measurement of the electrical quantities.

The response time of the electronic instrument is very high as compared to the electrical and mechanical device.

# Ammeter. Voltmeter and methods of extension of its full-scale reading range. Shunt, additional resistance.Range Extension of Ammeter by Shunt.

# Shunts are used for the range extension of ammeters. A shunt is a low-value resistance having minimum temperature co-efficient and is connected in parallel with the ammeter whose range is to be extended. The combination is connected in series with the circuit whose current is to be measured.
 This shunt provides a bypath for extra current because it is connected across (i.e. in parallel with) the instrument.
 These shunted instruments can be used to measure currents many times greater than their normal full-scale deflection currents.
 The ratio of maximum current (with shunt) to the full-scale deflection current (without shunt) is known as the ‘multiplying power’ or ‘multiplying factor’ of the shunt.
Emample: A moving coil ammeter reading up to 1 ampere has a resistance of 0.02 ohm. How could this instrument be adopted to read current up to 100 amperes.
 
Solution: In this case,
Full-scale deflection current of the ammeter,   Im = 1 A
Line current to be measured,    I  = 100 A
Resistance of ammeter,             Rm = 0.02 ohm
Let, the required shunt resistance = S

As seen from Figure, the voltage across the instrument coil and the shunt resistance is the same since
both are joined in parallel.
 
 Im*Rm = S*Is = S(I − Im)
 
or  S =  Im*Rm/(I – Im)
 
 =  1*0.02/(100 – 1) = 0.02/99 =  0.000202  Ans.

# Range Extension of Voltmeter by Multiplier

Multipliers are used for the range extension of voltmeters. The multiplier is a non-inductive high-value resistance connected in series with the instrument whose range is to be extended. The combination is connected across the circuit whose voltage is to be measured.

Example: A moving coil voltmeter reading upto 20 mV has a resistance of 2 ohms. How this instrument can be adopted to read voltage upto 300 volts.
Solution: In this case,
Voltmeter resistance,             Rm = 2 ohm
Full-scale  voltage of the voltmeter, ν = RmIm = 20 mV = 0.02 V
Full-scale deflection current,            Im = v/Rm = 0.02/2 = 0.01 A
Voltage to be measured,                   V = 300 V
Let the series resistance required       = R

Then as seen from figure, the voltage drop across R is V – ν
 
 R *Im = V – ν
 
 or R = (V – v)/Im
 
or R = (300 – 0.02)/0.01 = 299.98/0.01 = 29998 ohmsAns.

Shunts can not be used to extend the range of moving-iron AC ammeters accurately. It is because the division of current between the operating coil and the shunt varies with frequency (since reactance of the coil depends upon frequency). In practice, the range of moving-iron AC ammeters are extended by one of following methods:

Range Extension of Ammeter by Coil Turns

By changing the number of turns of the operating coil. For example, suppose that full-scale deflection is obtained with 400 ampere-turns. For full-scale reading with 100A, the number of turns required would be = 400 / 100 = 4. Similarly, for full-scale reading with 50A, the number of turns required in = 400/50 = 8.
 Thus the ammeter can be arranged to have different ranges by merely having a different number of turns on the coil. Since the coil carries the whole of the current to be measured, it has a few turns of thick wire. The usual ranges obtained by this method are from 0 -250 A.

## Range Extension of Ammeter by Current Transformer

For ranges above 0 – 250 A, a current transformer is used in conjunction with 0 – 5 A moving iron AC ammeter as shown in the figure. The current transformer is a step up transformer i.e. number of secondary turns is more than the primary turns.
 Usually, the primary winding of the transformer contains a single turn or at the most a few turns. The primary of this transformer is connected in series with the load and carries the load current. The AC ammeter is connected across the secondary of the transformer.

Since in figure, the current transformer ratio is 10:1, it means that line (or load) current is equal to 10 times the reading on the AC meter. Therefore, load current, IL = 3 x 10 = 30 A.

Range Extension of Voltmeter by Potential Transformer

The range of a moving-iron AC voltmeter is extended by connecting a high resistance (multiplier) in series with it.
 For ranges higher than 0 – 750 V, where power wasted in the multiplier would be excessive, a 0 – 110 V moving-iron AC voltmeter is used in conjunction with a potential transformer as shown in the figure.
 The potential transformer is a step-down transformer i.e. number of primary turns is more than the secondary turns. The primary of the transformer is connected across the load across which voltage is to be measured. The AC voltmeter is connected across the secondary.

Since in figure, the potential transformer ratio is 20:1, the load voltage is equal to 20 times the reading on the AC voltmeter.
Load voltage, VL = 100 x 20 = 2000 V.

Note that both secondaries of the instrument transformers are grounded as a safety measure.

Instruments for conversion of voltage and current. Transformer’s workEnergy considerations for transformers.

Power capacity

As has already been observed, transformers must be well designed in order to achieve acceptable power coupling, tight voltage regulation, and low exciting current distortion. Also, transformers must be designed to carry the expected values of primary and secondary winding current without any trouble. This means the winding conductors must be made of the proper gauge wire to avoid any heating problems. An ideal transformer would have perfect coupling (no leakage inductance), perfect voltage regulation, perfectly sinusoidal exciting current, no hysteresis or eddy current losses, and wire thick enough to handle any amount of current. Unfortunately, the ideal transformer would have to be infinitely large and heavy to meet these design goals. Thus, in the business of practical transformer design, compromises must be made.

Additionally, winding conductor insulation is a concern where high voltages are encountered, as they often are in step-up and step-down power distribution transformers. Not only do the windings have to be well insulated from the iron core, but each winding has to be sufficiently insulated from the other in order to maintain electrical isolation between windings.

Respecting these limitations, transformers are rated for certain levels of primary and secondary winding voltage and current, though the current rating is usually derived from a volt-amp (VA) rating assigned to the transformer. For example, take a step-down transformer with a primary voltage rating of 120 volts, a secondary voltage rating of 48 volts, and a VA rating of 1 kVA (1000 VA). The maximum winding currents can be determined as such:kVA (1000 VA). Themaximumwindingcurrentscanbedeterminedassuch:

Sometimes windings will bear current ratings in amps, but this is typically seen on small transformers. Large transformers are almost always rated in terms of winding voltage and VA or kVA.kVA.

### Energy losses

When transformers transfer power, they do so with a minimum of loss. As it was stated earlier, modern power transformer designs typically exceed 95% efficiency. It is good to know where some of this lost power goes, however, and what causes it to be lost.

There is, of course, power lost due to resistance of the wire windings. Unless superconducting wires are used, there will always be power dissipated in the form of heat through the resistance of current-carrying conductors. Because transformers require such long lengths of wire, this loss can be a significant factor. Increasing the gauge of the winding wire is one way to minimize this loss, but only with substantial increases in cost, size, and weight.

Resistive losses aside, the bulk of transformer power loss is due to magnetic effects in the core. Perhaps the most significant of these “core losses” is eddy-current loss, which is resistive power dissipation due to the passage of induced currents through the iron of the core. Because iron is a conductor of electricity as well as being an excellent “conductor” of magnetic flux, there will be currents induced in the iron just as there are currents induced in the secondary windings from the alternating magnetic field. These induced currents—as described by the perpendicularity clause of Faraday’s Law tend to circulate through the cross-section of the core perpendicularly to the primary winding turns. Their circular motion gives them their unusual name: like eddies in a stream of water that circulate rather than move in straight lines.

Iron is a fair conductor of electricity, but not as good as the copper or aluminum from which wire windings are typically made. Consequently, these “eddy currents” must overcome significant electrical resistance as they circulate through the core. In overcoming the resistance offered by the iron, they dissipate power in the form of heat. Hence, we have a source of inefficiency in the transformer that is difficult to eliminate.

This phenomenon is so pronounced that it is often exploited as a means of heating ferrous (iron-containing) materials. The photograph of (Figure below) shows an “induction heating” unit raising the temperature of a large pipe section. Loops of wire covered by high-temperature insulation encircle the pipe’s circumference, inducing eddy currents within the pipe wall by electromagnetic induction. In order to maximize the eddy current effect, high-frequency alternating current is used rather than power line frequency (60 Hz). The box units at the right of the picture produce the high-frequency AC and control the amount of current in the wires to stabilize the pipe temperature at a pre-determined “set-point.”

Methods for measuring electric resistance.Ohmmeter.

Resistance is one of the most basic elements encountered in electrical and electronics engineering. The value of resistance in engineering varies from very small value like, resistance of a transformer winding, to very high values like, insulation resistance of that same transformer winding. Although a multimeter works quite well if we need a rough value of resistance, but for accurate values and that too at very low and very high values we need specific methods. In this article we will discuss various methods of resistance measurement. For this purpose we categories the resistance into three classes

## Measurement of Low Resistance (<1Ω)

The major problem in measurement of low resistance values is the contact resistance or lead resistance of the measuring instruments, though being small in value is comparable to the resistance being measured and hence causes serious error.
        Thus to eliminate this issue small valued resistance are constructed with four terminals. Two terminals are current terminals and other two are potential terminals.

Figure below shows the construction of low resistance.

The current is flown through current terminals C1 and C2 while the potential drop is measured across potential terminals V1 and V2. Hence we can find out the value of resistance under experiment in terms of V and I as indicated in the above figure. This method helps us to exclude the contact resistance due to current terminals and though contact resistance of potential terminals still comes into picture, it is very small fraction of high resistance potential circuit and hence induces negligible error.

The methods employed for measurement of low resistances are:-

- Kelvin’sDoubleBridgeMethod

- PotentiometerMethod

- DucterOhmmeter.

### Kelvin’sDoubleBridge

Kelvin’s double bridge is a modification of simple Wheatstone bridge. Figure below shows the circuit diagram of Kelvin’s double bridge.

      As we can see in the above figure there are two sets of arms, one with resistances P and Q and other with resistances p and q. R is the unknown low resistance and S is a standard resistance. Here r represents the contact resistance between the unknown resistance and the standard resistance, whose effect we need to eliminate. For measurement we make the ratio P/Q equal to p/q and hence a balanced Wheatstone bridge is formed leading to null deflection in the galvanometer. Hence for a balanced bridge we can write

Putting eqn 2 in 1 and solving and using P/Q = p/q, we get-

Hence we see that by using balanced double arms we can eliminate the contact resistance completely and hence error due to it. To eliminate another error caused due to thermo-electric emf, we take another reading with battery connection reversed and finally take average of the two readings. This bridge is useful for resistances in range of 0.1µΩ to 1.0 Ω. Mathematical processing of measuring results is interval determination of 	         (,), where the true value of measured quantity can be found with  probability. Interval (, ) is calledconfidential,  is confidence factor (reliability). If numbers of measurements  are great enough then the confidence factor presents the part of general number of measurements where the measured quantity makes an influence within confidence interval. For instance, if 100 measurements were done, then 95 measurements gave the results within its limits in confidence coefficient =0.95. It is clear, that the more reliability is required the more corresponding confidence interval is received, and vice versa, the more confidence interval is given it is more obvious that the measuring results will not go beyond its limits. It results from the theory of errors that in large number of measurements n (more than 100) confidence coefficient of the interval from [,] is equal to 68%, and of the interval [,] is equal to 95%. While presenting any measured quantity it is important to point out interval and confidence probability corresponding to this interval.

In cases when the number of measurements is not great then there the conditions for strict fulfillment of statistical regularity lying in the base of random errors determination are absence. It results in that the value of quadratic mean error (standard deviation)  calculated by the (9) formula is not exact and inaccuracy is more the number of measurements is less. Hence, to guarantee the true value of quantity measured with the given probability is within confidence interval, the latter should be increased. In limited number of measurements not, but absolute error.

### Ducter Ohmmeter

It is an electromechanical instrument used for measurement of low resistances. It comprises of a permanent magnet similar to that of a PMMC instrument and two coils in between the magnetic field created by the poles of the magnet. The two coils are at right angles to each other and are free to rotate about the common axis. Figure below shows a Ducter Ohmmeter and the connections required to measure an unknown resistance R.

One of the coil called current coil, is connected to current terminals C1 and C2, while the other coil called, voltage coil is connected to potential terminals V1 and V2. Voltage coil carries current proportional of the voltage drop across R and so is its torque produced. Current coil carries current proportional to the current flowing through R and so is its torque too. Both the torque acts in opposite direction and the indicator come to halt when the two are equal. This instrument is useful for resistance in range 100µΩ to 5Ω.

## Measurement of Medium Resistance (1Ω – 100kΩ)

Following are the methods employed for measuring a resistance whose value is in the range 1Ω – 100kΩ –

- Ammeter-VoltmeterMethod

- WheatstoneBridgeMethod

- SubstitutionMethod

- Carey- FosterBridgeMethod

- OhmmeterMethod

### Ammeter Voltmeter Method

This is the most crude and simplest method of measuring resistance. It uses one ammeter to measure current, I and one voltmeter to measure voltage, V and we get the value of resistance as

Now we can have two possible connections of ammeter and voltmeter, shown in the figure below.

Now in figure 1, the voltmeter measures voltage drop across ammeter and the unknown resistance, hence

Hence, the relative error will be,

For connection in figure 2, the ammeter measures the sum of current through voltmeter and resistance, hence

The relative error will be,

It can be observed that the relative error is zero for Ra = 0 in first case and Rv = ∞ in second case. Now the questions stand that which connection to be used in which case. To find out this we equate both the errors

Hence for resistances greater than that given by above equation we use the first method and for less than that we use second method.

### Wheatstone Bridge Method

This is the simplest and the most basic bridge circuit used in measurement studies. It mainly consists of four arms of resistance P, Q; R and S. R is the unknown resistance under experiment, while S is a standard resistance. P and Q are known as the ratio arms. An EMF source is connected between points a and b while a galvanometer is connected between points c and d.

       A bridge circuit always works on the principle of null detection, i.e. we vary a parameter until the detector shows zero and then use a mathematical relation to determine the unknown in terms of varying parameter and other constants. Here also the standard resistance, S is varied in order to obtain null deflection in the galvanometer. This null deflection implies no current from point c to d, which implies that potential of point c and d is same. Hence

Combining the above two equations we get the famous equation –

### Substitution Method

The figure below shows the circuit diagram for resistance measurement of an unknown resistance R. S is a standard variable resistance and r is a regulating resistance.

     First the switch is place at position 1 and the ammeter is made to read a certain amount of current by varying r. The value of ammeter reading is noted. Now the switch is moved to position 2 and S is varied in order to achieve the same ammeter reading as it read in the initial case. The value of S for which ammeter reads same as in position 1, is the value of unknown resistance R, provided the EMF source has constant value throughout the experiment.

## Measurement of High Resistance (>100kΩ)

Following are few methods used for measurement of high resistance values-

- LossofChargeMethod

- Megger

- MegohmbridgeMethod

- DirectDeflectionMethod

We normally utilize very small amount of current for such measurement, but still owing to high resistance chances of production of high voltages is not surprising. Due to this we encounter several other problems such as-

- Electrostatic charges can get accumulated on measuring instruments

- Leakage current becomes comparable to measuring current and can cause error

- Insulation resistance is one of the most common in this category; however a dielectric is always modeled as a resistor and capacitor in parallel. Hence while measuring the insulation resistance (I.R.) the current includes both the component and hence true value of resistance is not obtained. The capacitive component though falls exponentially but still takes very long time to decay. Hence different values of I.R. are obtained at different times.

- Protection of delicate instruments from high fields.

Hence to solve the problem of leakage currents or capacitive currents we use a guard circuit. The concept of guard circuit is to bypass the leakage current from the ammeter so as to measure the true resistive current. Figure below shows two connections on voltmeter and micro ammeter to measure R, one without guard circuit and one with guard circuit.

Mathematical processing of measuring results is interval determination of 	         (,), where the true value of measured quantity can be found with  probability. Interval (, ) is calledconfidential,  is confidence factor (reliability). If numbers of measurements  are great enough then the confidence factor presents the part of general number of measurements where the measured quantity makes an influence within confidence interval. For instance, if 100 measurements were done, then 95 measurements gave the results within its limits in confidence coefficient =0.95. It is clear, that the more reliability is required the more corresponding confidence interval is received, and vice versa, the more confidence interval is given it is more obvious that the measuring results will not go beyond its limits. It results from the theory of errors that in large number of measurements n (more than 100) confidence coefficient of the interval from [,] is equal to 68%, and of the interval [,] is equal to 95%. While presenting any measured quantity it is important to point out interval and confidence probability corresponding to this interval.

In cases when the number of measurements is not great then there the conditions for strict fulfillment of statistical regularity lying in the base of random errors determination are absence. It results in that the value of quadratic mean error (standard deviation)  calculated by the (9) formula is not exact and inaccuracy is more the number of measurements is less. Hence, to guarantee the true value of quantity measured with the given probability is within confidence interval, the latter should be increased. In limited number of measurements not, but absolute error

In the first circuit the micro ammeter measures both capacitive and the resistive current leading to error in value of R, while in the other circuit the micro ammeter reads only the resistive current.

### Loss of Charge Method

In this method we utilize the equation of voltage across a discharging capacitor to find the value of unknown resistance R. Figure below shows the circuit diagram and the equations involved are-

However the above case assumes no leakage resistance of the capacitor. Hence to account for it we use the circuit shown in the figure below. R1 is the leakage resistance of C and R is the unknown resistance.
We follow the same procedure but first with switch S1 closed and next with switch S1 open. For the first case we get

For second case with switch open we get

Using R1 from above equation in equation for R’ we can find R.

### Megohm Bridge Method

In this method we use the famous Wheatstone bridge philosophy but in a slightly modified way. A high resistance is represented as in the figure below.

G is the guard terminal. Now we can also represent the resistor as shown in the adjoining figure, where RAG and RBG are the leakage resistances. The circuit for measurement is shown in the figure below. Mathematical processing of measuring results is interval determination of 	         (,), where the true value of measured quantity can be found with  probability. Interval (, ) is calledconfidential,  is confidence factor (reliability). If numbers of measurements  are great enough then the confidence factor presents the part of general number of measurements where the measured quantity makes an influence within confidence interval. For instance, if 100 measurements were done, then 95 measurements gave the results within its limits in confidence coefficient =0.95. It is clear, that the more reliability is required the more corresponding confidence interval is received, and vice versa, the more confidence interval is given it is more obvious that the measuring results will not go beyond its limits. It results from the theory of errors that in large number of measurements n (more than 100) confidence coefficient of the interval from [,] is equal to 68%, and of the interval [,] is equal to 95%. While presenting any measured quantity it is important to point out interval and confidence probability corresponding to this interval.

In cases when the number of measurements is not great then there the conditions for strict fulfillment of statistical regularity lying in the base of random errors determination are absence. It results in that the value of quadratic mean error (standard deviation)  calculated by the (9) formula is not exact and inaccuracy is more the number of measurements is less. Hence, to guarantee the true value of quantity measured with the given probability is within confidence interval, the latter should be increased. In limited number of measurements not, but absolute error

It can be observed that we actually obtain the resistance which is parallel combination of R and RAG. Althoughthiscausesveryinsignificanterror.

Instruments for measuring temperature and pressure.Types of thermometer.Optical method of temperature measurement.

All bodies in nature are capable of becoming electrified, i.e. acquiring an electric charge. A charged body interacts with other charged bodies.

- There are two types of observed electric charge, which we designate as positive and negative. (Two kinds of electric charges exist.)

- The smallestunit of charge known in nature is the charge of an electron or proton. It is called an elementary charge. We shall use the symbol e to denote a positive elementary charge. The unit of charge is called the Coulomb (C). The elementary charge has a magnitude of

e=1.6∙10-19C.

- An electric charge q is quantized. It is an integral multiple of e:

q=±Ne.

- The total charge of an electrically isolated system cannot change. This statement forms the law of electric charge conservation.

The magnitude of a charge does not depend on whether the charge is moving or at rest. Hence, an electric charge is relativistically invariant.

2. Coulomb’s law

- A point charge is defined as a charged body whose dimensions may be disregarded in comparison with the distances from this body to other bodies carrying an electric charge.

- The electric force exerted by a charge q1 on a second charge q2 is given by Coulomb’s law:

F12=kq1q2r2r,

where

kis the Coulomb constant, it assumed to be positive;

q1andq2 are magnitudes of the interacting point charges, separated by a distance r in vacuum;

r is a unit vector directed from q1 to q2.

The force of interaction between two stationary point charges is proportional to the magnitude of each of them and inversely proportional to the square of the distance between them. The direction of the force coincides with the straight line connecting the charges. Like charges repel each other, and unlike charges attract each other.

The magnitude of the interaction force of two charges q1and q2, which is the same for both charges, can be written in the form

F=kq1q2r2.

- In SI units, the Coulomb constant is given by

k=14πε0 ≈9∙109N∙m2/C2,

where

ε0=14πk ≈ 8.85∙10-12C2/(N∙m2).

is known as the “permittivity of free space” (or the quantity ε0 is called the electric constant).

Hence, the equation (1.1) of the law for charges in a vacuum will thus become

F12=14πε0q1q2r2r.

This modified way of writing formulas is called rationalized.

Thomson’s e/m experiment.Mass spectrometers.

Dielectrics(or insulators) are defined as substances notcapableofconducting an electric current. Ideal insulators do notexist innature. Allsubstances, even if to a negligibleextent, conduct anelectric current. Butsubstances called conductors conduct a current from 1015to 1020times betterthan substancescalleddielectrics.If a dielectricis introduced into anelectric field, then the fieldandthe dielectricitself undergoappreciablechanges.Tounderstandwhythis happens,wemusttake into accountthat atomsandmolecules contain positively charged nuclei and negatively chargedelectrons.

Amoleculeis a system witha total charge of zero. Thelineardimensionsof this systemare verysmall,ofthe orderofafew angstroms(theangstrom – A – isaunitoflength equalto 10-10 mthatis veryconvenientin atomicphysics). Weestablished in lection 2 thatthefield setupbysuchasystemisdeterminedbythe magnitudeand orientationof the dipole electric moment

p=qiri.

(summation is performed both over the electrons and over thenuclei). True,the electrons in a moleculeare in motion,so thatthis momentconstantly changes. The velocities of the electronsare so high, however, that the meanvalueof the moment(8.1) isdetectedin practice. Forthis reason in the following bythe dipolemomentof a molecule,weshall meanthe quantity

p=qiri.

(for nuclei,ri is simply taken as ri in this sum). In otherwords,weshallconsiderthat the electronsareatrest relative to the nucleiatcertainpointsobtainedbyaveragingthe positionsofthe electronsin time.

The behavior ofa moleculein an externalelectric field is alsodeterminedbyitsdipolemoment.Wecanverifythis bycalculatingthe potential energy of a molecule in an external electric field.Selectingthe originofcoordinatesinside the moleculeand takingadvantageofthesmallnessof ri, let uswritethe potentialatthepointwherethe i-th charge is in the form

φi=φ+∇φ∙ri.

whereφisthe potentialatthe originofcoordinates.Hence,

Wp=qiφi=qiφ+∇φ∙ri=φqi+∇φqiri.

Takinginto account that qi= 0 and substituting –Efor ∇φ,weget

Wp=-Eqiri=-pE=pE cosα.

Differentiatingthis expressionwith respect to α, wegetEqT=pE sinαfor the rotational moment;differentiating withrespect to x, wearrive at the force.

Fx=p∂E∂xcosα.

Thus,a moleculeis equivalentto a dipole bothwithrespect tothe field it sets up and withrespect to the forces it experiencesin an external field. Thepositivecharge ofthis dipoleequals thetotal charge ofthe nuclei and is at the "centre ofgravity" of thepositivecharges; the negativechargeequalsthe total chargeoftheelectronsand is atthe "centre ofgravity"ofthe negativecharges.

In symmetrical molecules(such as H2, O2,N2), the centre ofgravityofthe positiveandnegativechargescoincidein the absenceofanexternalelectricfield. Suchmoleculeshavenointrinsic dipolemomentandarecallednon-polar. In asymmetrical molecules (suchas CO,NH,HCI), the centreofgravityof the charges ofoppositesigns are displaced relative to eachother. In this case, the molecules have an intrinsic dipole momentand are called polar.

Underthe action of an external electric field, the charges ina non-polar moleculebecome displaced relative to one another,the positive ones in the direction of the field, the negative onesagainstthe field. Asaresult, themoleculeacquiresadipolemomentwhosemagnitude,asshown byexperiments, is proportionto thefield strength.Inthe rationalized system,the constantofproportionality is writtenin the form ε0β,whereε0 is the electric constant and β is a quantitycalled the polarizabilityof a molecule.Sincethe directions of p and E coincide, wecan writethat

p=βε0E.

Thedipole momenthas a dimension of [q], thedimensionofε0E is [q]L-2.Hence,the polarizabilityofa molecule βhas the dimension L3.

Theprocess ofpolarizationof a non-polarmoleculeproceedsasif the positiveand negativechargesofthe moleculewereboundtooneanotherbyelasticforces. Anon-polarmoleculeis therefore saidto behavein an external field like an elastic dipole.

Theactionofanexternalfield onapolarmoleculeconsists mainly in tending to rotate the moleculeso that its dipolemomentis arranged in the directionofthe field. Anexternalfield doesnotvirtually affect the magnitudeof a dipole moment.Consequently,apolarmoleculebehavesin an externalfield like a rigid dipole.

In the absence ofan external electric field, the dipolemomentsofthe moleculesofa dielectricusuallyeitherequalzero (non-polarmolecules) or are distributed in space by directions chaotically(polarmolecules).In bothcases,the totaldipolemomentofa dielectric equals zero.

Adielectric becomes polarized under the action of an externalfield. Thissignifiesthat the resultantdipolemomentofthe dielectricbecomesotherthan zero. It is quitenaturalto take the dipolemomentofa unitvolumeasthe quantitycharacterizingthe degreeofpolarization.Ifthe field orthe dielectric(or both)arenothomogeneous,the degreesofpolarizationatdifferentpointsofthe dielectricwilldiffer. Tocharacterize the polarization at a given point, wemustseparateaninfinitely small (physically) volume ∆Vcontainingthis point, find the sum ∆Vp of the momentsof the moleculesconfined in this volume, and take the ratio

P=∆Vp∆V.

ThevectorquantityPdefinedbyEq.(8.4)is calledthe polarizationof a dielectric.The dipole momentp has the dimension [q] ∙L. Consequently, the dimensionofPis [q] L-2, i.e.it coincideswiththe dimensionof ε0E.

Thepolarizationofisotropic dielectricsofanykindis associatedwiththe field strength at the same point by the simple relation

P=χε0E.

whereχis a quantityindependent ofEcalled the electricsusceptibilityofa dielectric.Itwasindicated above that the dimensionsof P and ε0E are identical. Hence,χis a dimensionlessquantity.

Fordielectricsbuiltofpolarmolecules,the orientingactionofthe externalfield is counteracted by the thermal motion of the moleculestending to scatter their dipole momentsin all directions. Asa result, a certain preferred orientationof the dipole momentsofthe moleculessets in the direction of the field. Therelevantstatistical calculations, whichagree withexperimental data, showthat the polarizationis proportionalto the field strength, i.e. The electric susceptibility of such dielectrics variesinversely withthe absolute temperature.

In ionic crystals, the separate moleculeslose their individuality. Anentirecrystalis, asit were,a singlegiantmolecule.Thelatticeof an ionic crystal can be considered as two lattices inserted intoeach other, oneofwhichis formed bythe positive, and the otherby the negative ions. Whenan external field acts on the crystalions, bothlattices aredisplacedrelative to eachother,whichleadsto polarizationofthe dielectric. Thepolarizationin this casetoo isassociated withthe field strength by. Wemustnotethatthe linearrelation between  E and P described maybeapplied only to not too strong fields.

In many capacitors there is an insulating material such as paper or plastic between the plates. Such material, called a dielectric, can be used to maintain a physical separation of the plates. Since dielectrics break down less readily than air, charge leakage canbe minimized, especially when high voltage is applied.

Experimentally it was found that capacitance Cincreaseswhen the space between the conductors is filled withdielectrics. To see how this happens, suppose a capacitor has a capacitance when there is no material betweenthe plates. When a dielectric material is inserted to completely fill the space between the plates, the capacitance increases to

C=εC0,

whereεis called thedielectric constant. Experiments indicate that all dielectric materialshave>1. Note that every dielectric material has a characteristic dielectric strengthwhich is themaximumvalue ofelectric field before breakdown occurs and charges begin to flow.

Particle accelerators and detectors.Cyclotron.Synchrotron. Collider

Cyclotron and synchrotron are two types of particle accelerators. Particle accelerators are very useful machines when it comes to the field of nuclear physics. The high-energy collisions of sub atomic particles give very good observations on the nature of the nucleus. For someone who is studying such a field, a thorough knowledge in synchrotron accelerators and cyclotron accelerators is required. In this article, we are going to discuss what cyclotron and synchrotron accelerators are, the principles these machines are based on, their similarities, applications and finally the differences between cyclotron accelerators and synchrotron accelerators.

What is Synchrotron Accelerator?

A synchrotron accelerator is a type of particle accelerator. One must understand the concept of particle accelerator first, to understand synchrotron accelerator clearly. When a charged particle is projected in to a magnetic field, it moves on a circular path. Particle accelerators are used to study the nature of atoms and sub atomic particles by making high velocity collisions of such particles and studying the collision itself and the products of the collision. A magnetic field is used in most of the cases to accelerate the particles. The practical method of obtaining high velocity collisions is by using two particle beams rotating in opposite directions. Using this method it is easy to obtain high velocity collisions with relative velocities as high as 99 percent of the speed of light. However, the theory of relativity states that there cannot be relative velocities higher than the speed of light. Therefore, a huge amount of energy is required even to accelerate the particle beam to a high velocity. A synchrotron accelerator uses a varying magnetic field and a varying electric field, which keep the particle beam on a proper circular path when the energy is increased. A particle accelerator is made of a torus with the ability to change the intensity of electric and magnetic fields inside the torus. The path of the particle beam is the circular path encased by the torus. The concept of the synchrotron accelerator was developed by Sir Marcus Oliphant. Vladimir Veksler was the first person to publish a scientific paper on synchrotron accelerators, and the first electron synchrotron accelerator was constructed by.

Cyclotron Accelerator

Cyclotron accelerator is also a particle accelerator, which is mostly used in small-scale projects. A cyclotron is a circular vacuum chamber where the acceleration of particles begins at the center. The particles take a spiral path as they are accelerated. The cyclotron uses a constant magnetic field, and a constant frequency electric field to accelerate the particles.

References:

Bronson M., (2000) Self-regulation in early childhood: Nature and Nurture New York London: Guilford Press

Bruner J., (2006) The Culture of Education(Cambridge, MA, Harvard University Press)

Chaiklin, S., (2009) Developmental teaching in upper-secondary school, in M.Hedegaard& J.

Lompscher (Eds) Learning Activity and DevelopmentOxford, Aarhus University Press

Flavell, J.H. (2008) Metacognitive aspects of problem solving, in: Resnick, L.B. (Ed) The Nature of

Intelligence(Hillsdale, NJ, Erlbaum)

Flavell, J.H., Green, F.L., and Flavell, E.R. (2015) Young children’s knowledge about thinking. Monographs of the Society for Research in Child Development, 60 (1, Serial No,243)

Forrest-Pressley, D.L., MacKinnon, G.E. & Waller, T.G. (eds) (2015)

Metacognition, Cognition & Human Performance, New York: Academic Press

Lave, J., (2008) Cognition in PracticeCambridge Cambridge University Press

Lave, J., & Wenger, E., (2010) Situated learning: Legitimate peripheral participation. New YorkCambridge University Press

Leontiev, A.N. (2011). The problem of activity in psychology. In Wertsch, J. (ed), The concept of activity in Soviet psychology. Armonk, NY: SharpeMatlin M., (1998) Cognition(Fourth Edition) Orlando: Harcourt Brace

Nunes, T.; Schliemann, A. D. &Carraher, D. W. (2013).Street Mathematics and School Mathematics. Cambridge: Cambridge University Press.

Абдуалиева М.А.

Турсынбаева К.А.

Сборник лекции  к лекционным занятиям

по дисциплине «Methods for measuring physical quantities» для студентов специальности 5B011000-Физика

Подписано в печать    ____.____.2019. Формат 60-84   1/16

Бумага типографическая. Печать офсетная. Обьем _____п.л.

Тираж 100экз. Заказ№

Издание Южно-Казахстанского государственного университета им. М. Ауэзова, г.Шымкент, пр. Тауке Хана, 5