Ф.7.03-03

Korolkov A.V.

Methodical instructions and tasks on performance of SIW and SIWT

on discipline «Electrical materials »  for bachelor students

of specialty 5B071800 «Electroenergetics»

Shymkent, 2019

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

M.AUEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

HIGHER SCHOOL: INFORMATION TECHNOLOGIES AND ENERGETIC

CHAIR « ENERGETICS AND RENEWABLE ENERGY SYSTEMS »

Methodical instructions and tasks on performance of SIW and SIWT

on discipline «Electrical materials »  for bachelor students

of specialty 5B071800 «Electroenergetics»

Shymkent, 2019

UDC 621.315

The author: Korolkov A.V.

Methodical instructions and tasks on performance of SIW and SIWT on discipline «Electrical materials»  for bachelor students  of specialty 5B071800 «Electroenergetics». - Shymkent: SKSU, 2019. – 48 p.

Methodical instructions and tasks on performance of SIW and SIWT have been drawn up in accordance with the curriculum and course program on discipline «Electrical materials» and include all necessary information on carrying-out of SIW and SIWT connected with themes.

Methodical instructions contain all instructions for performance of SIW and SIWT and individual tasks for students.

Methodical instructions of SIW and SIWT are intended for students of specialty 5B071800 «Electroenergetics».

Refs. – 8 titles.

Reviewer:  Apsemetova A.T. - Senior teacher of "Energetics and renewable energy systems" chair of M. Auezov SKSU

Methodical instructions of SIW and SIWT have been discussed at the meeting of the chair Information systems and modeling (Minutes №   9  «   28   »   02    2019y.) and they have been approved by Committee on innovational technologies of training and Methodical provision of higher school «IT&E»  (Minutes №  6  « 18 »   03   2019y.)

Methodical instructions of SIW and SIWT are recommended to the edition by Educational-methodical council of M.Auezov SKSU,

Minutes №             from  «     »         2019 y.

© M.Auezov South Kazakhstan State University, 2019

Responsible for printing: Korolkov A.V.

Content

|  | Introduction | 6 |
| --- | --- | --- |
| 1 | Aims of studying the subject | 7 |
| 2 | Organization of the bachelor students to study of the discipline | 10 |
| 3 | Recommendations for preparing for different types of activities | 12 |
| 4 | Subject of each type of SIW and SIWT on modules | 14 |
| 5 | Recommendations about SIW performance | 16 |
| 6 | Approximate list of the subjects SIW | 17 |
| 7 | Recommendations about SIWT performance | 20 |
| 8 | Approximate list of the subjects SIW | 21 |
| 9 | Tips for planning and organizing the time needed to study the discipline | 24 |
| 10 | Materials for self-checking | 25 |
| 11 | Questions for the fourth current control | 27 |
| 12 | Questions for the Ist midterm control | 28 |
| 13 | Questions for the 2nd midterm control | 31 |
| 14 | Exam questions on the disciplines | 34 |
|  | Criteria for assessing students' knowledge | 40 |
|  | List of recommended literature | 44 |

Introduction

Methodical instructions and tasks on performance of SIW and SIWT on discipline «Electrical materials» are developed on the basis of educational and scientific literature in the field of power industry.

The discipline " Electrical materials " belongs to cycle BD EC of disciplines and enters the SM module 2(G) "Electromaterials and electrotechnologies" for bachelor students of specialty 5B071800 «Electroenergetics».

The made methodical instructions pursue the aim to fix at undergraduates of knowledge of development of competences in understanding of forming of knowledge of the principles of use of electrotechnical materials in devices of electrical equipment and power industry.

The curriculum provided the following types of independent work: SIWT (independent work of the undergraduate under the leadership of the teacher), SIW (independent work of the undergraduate) including performance of homeworks for the purpose of profound studying of some sections of a course.

Tasks by these types of work (the plan of SIWT, SIW), and also terms of their performance (the schedule diagram of educational process

1. Aims of studying the subject

The aims of studying the disciplines «Electrical Materials » is an learning by basic elements and characteristics of power converting devices in professional sphere of the specialist, using skills for carrying out analysis of operation of power converters.

Methodical instructions for the study of discipline for bachelor  students represent a set of recommendations and explanations, allow bachelor students  to optimally organize the process of studying this discipline, while part of the course can be studied by bachelor students  independently.

The purpose of these methodological instructions is to help bachelor students the knowledge provided by the curriculum.

Aims of studying the subject:

-	Acquaintance of students with materials which are applied in electrical power area;

-	Assimilation by students of information on such materials as dielectrics, semiconductors, conductors and magnetic materials;

-	Forming at students of the strong base of knowledge gained in the field of electric, electrotechnical and magnetic materials;

-	Acquaintance of students with history of use of electrotechnical materials, materials of a special purpose, with history of development of superconductors;

Tasks of studying the discipline:

- - to count key parameters and characteristics of standard materials;

- - to define characteristics of materials for management of the set types of electric motors of the electric drive;

- - to development of the most common methods in the electric power industry and algorithms for research electric materials

- to study the theoretical material about electro technical materials;

- the study by students of the basic laws of electrical engineering and electrical engineering, existing and promising directions for the development of electrical devices, electrical circuits;

- - nobility and understand basic provisions concerning electrotechnical materials science;

- - knowledge about about the main physical properties of electrotechnical materials, their main operational and technical characteristics and the nature of change of these properties under the influence of external factors;

- - knowledge of the modern level of development of electrotechnical materials science, and also of the main physical and chemical processes happening in electrotechnical materials under the influence of external influences;

- - to use theoretical knowledge at the choice of electrotechnical materials for operation in the set conditions;

- - to predict properties of electrotechnical materials at long and short-term influence of constants and variation fields, mechanical loadings, chemically active environments in use.

The discipline " Electrical materials " belongs to cycle BD EC of disciplines and enters the SM module 2(G) "Electromaterials and electrotechnologies" for bachelor students of specialty 5B071800 «Electroenergetics».

As a result of development of a subject matter, students should show the following results of education:

Knowledge::

- main characteristics of electrotechnical materials;

- nobility and understand basic provisions concerning electrotechnical materials science;

- know about the key physical properties of electrotechnical materials;

- know about the main utilization and technical characteristics on materials;

- know about the nature of change of properties of materials under the influence of external factors

- know about the modern level of development of electrotechnical materials science;

- know about the main physical and chemical processes happening in electrotechnical materials under the influence of external influences

- know history of use of new electrotechnical materials;

Abilities:

- use methodical normative materials, technical and technology documentation, modern hardware components and information technologies;

- use theoretical knowledge at the choice of electrotechnical materials for operation in the set conditions;

- list types of electrotechnical materials;

- carry out comparison different by a type of electrotechnical materials on the chosen categories;

- choose the optimal solutions connected using electrotechnical materials in different devices;

- describe key properties of electrotechnical materials;

- calculate key parameters and characteristics of electrotechnical materials;

- describe the processes happening in electrotechnical materials at impact on them of electric, magnetic and other fields.

Skills:

- collecting, generalization and systematization of information on the electrotechnical materials, their features and properties applied in certain electrotechnical devices;

- determine the potential of use of certain materials at design of the electrotechnical equipment;

- analyze properties of electrotechnical materials for the purpose of their application under the set conditions;

- investigate properties of electrotechnical materials.

In the course of development of this discipline the undergraduate creates and shows the following professional competences:

- ability to make out, represent and report results of the performed work;

- ability to predict properties of electrotechnical materials at long and short-term influence of constants and variation fields, mechanical loadings, chemically active environments in use;

- ability to make the decisions connected with use of materials at design of electroprocessing equipment;

- ability to define efficiency of use of certain materials in this or that field of power;

- readiness to use modern achievements of science and advanced technology in research works;

- ability to use of theoretical knowledge at the choice of electrotechnical materials for operation in the set conditions.

2 Organization of the bachelor students to study of the discipline

A minimum level of master’s knowledge is based on the requirements defined by state standards and curriculum specialty.

At the beginning of the semester should be familiar with the weekly schedule of studying the discipline (given in the syllabus), Guidelines for its study. It must be remembered that all kinds of independent work (independent study of individual sections of the discipline) must be carried out within a specified time.

Additional recommendations connected with studying of discipline

This course introduces and explains in some depth the principles, protocols, technologies, standards at the sphere of power engineering.

Tasks are concretized with reference to a speciality (mastering and operating by terminological lexicon, phrases-cliches; specific grammatical structures and designs; construction of the statement of dialogue of special subjects etc., students 'development through problem thinking; abilities to formulate a problem, having planned possible ways of its decision: to find ways of a finding of the necessary information).

The process of learning of English language needs to be built on the basis of technology of communicative training, i.e. to provide collective interaction of all students and their inclusion in speech dialogue, the organization of studies under various organizational forms: role, business, situational games etc.

The main thing is the student's development through problem thinking; ability to formulate a problem to plan possible ways of the decision, to define necessary conditions for its decision (including to understand and to consider ways of a finding of the necessary information to master new information technology).

The aims of subject are defined by application of a foreign language (English) in professional sphere of the specialist, using skills of  four kinds of communications: auditions, readings, letters, communication in a foreign language: to find the set information, to understand the maintenance of the read:

- Reading: to read the texts with and without dictionary, to find the given information, to remember content of reading;

- Writing: to fill the blank, to write business or private letter, essay, report, annotation and etc.;

- Translating: to translate texts from foreign to native language with dictionary in accordance with translating norms;

- Auding: to understand foreign speech;

- Speaking: to enunciate own minds and to comment in a foreign language in accordance with language speech norms, to ask and to answer the questions, to support the speech in a foreign language in a studying themes volume, to use adequate communicative replications, to retell read and heard information.

As a result of studying the discipline, students should:

- Ability to own skills of perception, understanding, and also the multidimensional analysis of oral and written language in the learned foreign language.

- Use language means for achievement of the communicative purposes in a concrete situation of communication in the learned foreign language.

- Build strategy of oral and written communication in the learned foreign language according to sociocultural features of the learned language.

- Put into practice methods of rational work in classroom and out-of-class educational activity.

- country.

To this discipline is given 3 credits.

3 Recommendations for preparing for different types of activities

3.1 Lectures

The main part of the learning process in teaching the course "Electrical Materials" full-time students are lectures.

All lectures students need to take notes. In the fields of abstract recommend to write questions that arise in the study of material and require further explanation of the teacher.

3.2 Laboratory classes

Before carrying out the laboratory work should be carefully read the description of the laboratory work, to understand what are the purpose and the work task.

Theoretical information in methodological guidelines for laboratory works; contain a minimum of training material required for the preparation and implementation of laboratory work.

Rules of laboratory work. To avoid accidents and premature failure of the computing laboratory equipment master must strictly observe the following rules.

1. In the introductory lesson, master students should be familiar with the internal regulations and safety, virtual instrumentation.

2. At the opening session of each master student is assigned a permanent job for the entire semester.

3. During the classes in the laboratory is prohibited to talk loudly, leave the workplace without permission of the instructor.

The report can be submitted in electronic form, which includes all the work for the semester.

3.3 Practical classes

The main part of the learning process in teaching the course « Electrical Materials » full-time bachelor students are practical classes.

All bachelor students need to take notes. In the fields of abstract recommend to write questions that arise in the study of material and require further explanation of the teacher.

3.4 Self-Test

Each parsed on laboratory studies topic of the course master students should be fastened independently. For this purpose it is necessary to reproduce the memory determining, properties, formulations.

As part of the total hours devoted to study discipline provides the following types of independent work of students (SIW): test papers (individual homework), an independent study of theoretical material with self-control on the following issues, the study of theoretical material in preparation for the protection of laboratory work , the final repetition of the theoretical material.

For self-study discipline shall be made of the material on all topics of the discipline of self-control on control issues and the possibility of advice from a leading teacher, totalling 45hours of SIW.

The master student must first learn the theoretical material on their own respective sections. To protect the work, he should know the theoretical material and demonstrate practical knowledge.

3.5 Examination

The exam is determined by the absorption of the theoretical issues and the ability to apply the theory to solve practical problems. The exam allowed students with no debt on control points.

The final rating is determined on the basis of the master's semester exam results and rankings.

Test tasks and self-test questions to be issued at the beginning of the semester and are available in EMCD.

In the course of development of the discipline necessary to regularly refer to the list of recommendations (main and additional) literature.

On examination it appears clear mastering theoretical questions and the ability to apply the theory to solve practical problems. The exam students are allowed with no debt on the control points. The final rating of the student is determined on the basis of semester exam results and rankings.

Tests and questions for self-examination are issued at the beginning of the semester and are available in EMCD.

In the process of mastering the discipline necessary to regularly refer to the list of recommended (basic and supplementary) literature.

To pass the exam, you must have lecture notes. Preparation of lectures, where the material is given a significantly increased demand from the student considerable time and physical costs, which in the exams is never enough. Lectures should be used as a consultant on unclear issues, and as a test - at the end of each lecture, there are checklists that allow to check the degree of mastery of the material;

4. Subject of each type of SIW and SIWT on modules

For studying of discipline of "Electrical Materials" in 5B071800 Electroenergetics has allocated 3 credits of KZ and 5 credits of ECTS. It means that 30 hours, on practical or laboratory (depending on the curriculum) - are allocated 15 hours for lecture occupations. For SIW and SIWT the same hours for occupations - for 45 hours are allocated for each of them. In total for full studying of this discipline 135 hours are allocated.

All 15 weeks of training are divided into 2 big modules which are separated by midterm control on the 7th week. midterm control only 2 and midterm control control on 7 and 15 weeks them. The first module is called "Module 1. Dielectrics. Conductors" and are included into it subjects, connected by both dielectrics and conductors. The second module is called "Module 2. Semi conductors. Magnetic materials" and are included into it the subjects connected with semiconductors and magnetic materials.

The following subjects belong to the first module:

Dielectrics:

- Structure and key properties of materials.

- Concept about the zonal theory of solid bodies.

- The electric phenomena in dielectrics and their physical and chemical properties.

- Conductivity.

- Dielectric losses.

- Breakdown of dielectrics.

- Breakdown of liquid dielectrics.

- Breakdown of solidstate dielectrics.

- Physical and chemical properties of dielectrics.

- Classification of dielectrics.

- Waxy substances.

- Rubber materials.

- Vegetable oils.

- Cellulose fibrous materials and silk.

- Textile materials.

- Ceramic dielectrics.

Conductors:

- Structure and key properties of materials.

- Concept about the zonal theory of solid bodies.

- Electrophysical properties of conductors.

- Conductivity.

- Superconductivity and cryoconductivity.

- Conduction materials.

- Alloys of high resistance.

- Metals and alloys of different electrotechnical assignment.

The following subjects belong to the second module:

Semiconductors:

- History of superconductor materials

- Electrophysical properties of semiconductors.

- General characteristic of poluprovodnikovykhmaterial.

- Semi-conductor materials.

- Compound semiconductors.

- Photoconductivity in semiconductors.

- Semi-conductor materials.

- Technology of receiving semi-conductor materials

- Extrinsic type of semiconductor

- Intrinsic type of semiconductor

- Energy bands in semiconductor

Magnetic materials:

- Fundamentals of physics of magnetic materials.

- General information about magnetic properties of materials.

- Classification of magnetic materials by properties and fields of their application.

- Magnetosoft materials.

- Magnetosolid materials

- Magnetic resonance

5 Recommendations about SIW performance

By training students for performance of SIW it is necessary to look first of all at SYLLABUS on this discipline. In SYLLABUS it is shown that for obtaining rating for educational achievements it is necessary to prepare special types of works and to pass through the procedure of protection at the teacher.

On this discipline there are 2 modules and for everyone there is the corresponding task.

For the first module "Module 1. Dielectrics. Conductors" needs to execute the following types of works (the list with the indication of a form of control, a task for SIW, a deadline for completion of work is given below):

1. Preparation of the presentation in the Power Point program for subjects of lectures with 1 on 5. Protection of work has to be performance not later than 3 weeks and pass to an oral form and no more than 3-5 minutes.

2. Preparation of the report on subjects of lectures with 4 on 8. Protection of work has to be performance no later than the 5th week and pass to an oral form and no more than 3-5 minutes.

3. Preparation of the report on subjects of lectures with 6 on 12. Protection of work has to be performance no later than the 5th week and pass to an oral form and no more than 3-5 minutes.

4. Passing of Ist midterm control on subjects of lectures with 1 on 14. Passing of Ist midterm control occurs on the 7th week in the form of oral answers to control questions on earlier studied subjects.

For the second module "Module 2. Semi conductors. Magnetic materials" needs to execute the following types of works (the list with the indication of a form of control, a task for SIW, a deadline for completion of work is given below):

1. Preparation of the presentation in the Power Point program for subjects of lectures with 13 on 15. Protection of work has to be performance no later than the 9th week and pass to an oral form and no more than 3-5 minutes.

2. Preparation of the report on subjects of lectures with 16 on 22. Protection of work has to be performance no later than the 11th week and pass to an oral form and no more than 3-5 minutes.

3. Preparation of the report on subjects of lectures with 20 on 26. Protection of work has to be performance no later than the 13th week and pass to an oral form and no more than 3-5 minutes.

4. Passing 2nd midterm control on subjects of lectures with 15 on 30. Passing 2nd midterm control occurs on the 15th week in the form of oral answers to control questions on earlier studied subjects.

6 Approximate list of the subjects SIW

Module 1. Dielectrics. Conductors

- Atomic description

- Atomic description of a dielectric?

- Photoelectronic emission?

- Polarization of a dielectric?

- Thermionic emission?

- Conductor

- Dielectric

- The dielectric constant

- The dielectric strength

- Electric displacement vector

- Electrical conductivity

- Electron, proton, atom

- Emission of electrons

- Energy levels

- Coulomb’s law in dielectrics

- Electric conductivity in dielectric

- Emission electron from metals

- Energy level of atom

- Gauss’s law in dielectrics

- Random and direct motion of electron

- The displacement vector

- The electric field vector

- The polarization vector

- The electrical conductivity of doped materials

- The energy stored in the electric field

- Polar molecules

- Polarization of dielectric

- Resistivity

- Difference photoelectric emission and thermionic emission

- Difference capacitor with a dielectric and capacitor without a dielectric

- Difference between conductor and dielectric

- Difference between conductor and superconductor

- Difference between semiconductor and superconductor

- Difference between dielectric and superconductor

- Dependence of the conductivity of dielectric from temperature

- Dependence of the conductivity of metals from temperature

- Dependence of the conductivity of semiconductors from temperature

- Effect of temperature o  the electrical conductivity of metals

- Change the capacitance of capacitor

- Electron motion in electric field

- Main properties of metal

- Factors affecting the resistivity of electrical materials your know

- Place of alloys in the conductivity of metals

- Role of the dielectric in the capacitor

- A parallel plate capacitor with a dielectric between the plates

Module 2. Semi conductors. Magnetic materials

- Amalgams

- Extrinsic type of semiconductor

- Intrinsic type of semiconductor

- Magnetic materials

- Magnetic resonance

- N-type semiconductor

- Pleochroism

- P-n junction

- P-type semiconductor

- Semiconductor

- Superconductivity

- Superconductor

- Energy bands in semiconductor

- History of superconductor materials

- Superconductivity

- Superconductor materials

- The common magnetic materials

- The einstein relation

- The factors affecting permeability and hysterisis loss

- The hall effect

- The magnetic properties of materials

- The using effect magnetic resonance

- The non-linear conductors

- The the hysterisis loop

- The the magnetisation curve

- The thermal conductivity of semiconductors

- Difference between conductor and semiconductor

- Difference between Ferromagnetism and Paramagnetism

- Difference between intrinsic type and extrinsic type of semiconductor

- Difference between conductor and semiconductor band structure

- Difference between semiconductor and dielectric band structure

- Difference between superconductor and dielectric band structure

- Difference between conductor and dielectric band structure

- Difference between positive and negative current

- Difference between resistivity and conductivity

- Difference between donors and acceptor

- Difference between semiconductor and dielectric

- Difference majority and minority carriers

- Difference between Diamagnetism and Paramagnetism

- Difference between Diamagnetism and Ferromagnetism

- Difference between Diamagnetic and Anti-ferromagnetic

- Difference between Anti-ferromagnetic and Paramagnetic

- Difference between Anti-ferromagnetic and Ferromagnetic

- Difference between n-type and p-type semiconductor

- Difference between electrical conductivity and thermal conductivity of semiconductors

7 Recommendations about SIWT performance

When performing independent work of students under the leadership of the teacher (SIWT) the following four main functions are considered.

The first function assumes realization of active perception by students of information of the teacher obtained during classes in a subject matter.

The second function assumes that students independently, on the basis of recommendations of the teacher, study educational and methodical grants, references, perform homeworks, control and term papers, etc. At this stage from students knowledge of methods of work, fixing of the difficulties, self-organization and self-discipline is required.

The third function students consists in the analysis and systematization of the difficult situations, identification of the reasons of difficulties in understanding and assimilation of a training material by them, performance of other educational actions. Students transfer unsoluble difficulties to system of questions for the teacher (range them, order, make out), build own versions of answers to these questions.

The fourth function of students consists in the address to the teacher behind the corresponding explanations, councils, consultations.

8 Approximate list of the subjects SIW

Module 1. Dielectrics. Conductors

- Introduction into the physics of dielectrics.

- Permanent dipole moment.

- Induced dipole moment.

- Polarization and dielectric constant.

- Types of polarization

- Electron polarization

- Atomic polarization

- Orientation polarization.

- Ionic polarization.

- Dipole moments and electrostatic problems.

- Polarizability .

- Polarization and energy.

- Internal field Langeven function.

- Non-polar dielctrics.

- Lorentz's field.

- Clausius-Massotti formula.

- Reaction field.

- Polarization in gases

- Debye's Theories

- Polar molecules in nonpolar solvent.

- Onsager's theory.

- The dielectric properties of polar nonassociative liquids.

- Statistical-mechanical approach to dielctric theory.

- Kirkwood-Fröhlich's equation.

- The Kirkwood  correlation factor.

- Applications: pure dipole liquids, mixtures, dipolar solids.

- The modern theories of the static dielectric permittivity (Böttcher, Nienhuis and Deutch, Ramshaw, Wertheim etc.).

- The theory of linear response.

- The time dependent fields.

- The dielectric response function.

- The dielectric relaxation theory.

- Frequency and Time Domain.

- The complex dielectric permittivity.

- Dielectric losses and dispersion.

- The distribution functions of the relaxation times.

- Cole-Cole distribution.

- Cole-Davidson distribution.

- Havriliak-Nehamy and Johnsher distributions.

- The dipole correlation function.

- The relationship between the complex dielectric permittivity and the dipole correlation function.

- Short-range and long range correlation functions.

- Fulton's Theory.

- Conductivity of Metal

- Factors affecting the resistivity of electrical materials

- Motion of an electron in an electric field

- Equation of motion of an electron

- Current carried by electrons

- Mobility of electrons

- Energy levels of a molecule

- Emission of electrons from metals

- Thermionic emission

- Photo electric emission

- Field emission

- Effect of temperature on electrical conductivity of metals,

- Electrical conducting materials

- Thermal properties

- Thermal conductivity of metals

- Thermoelectric effects.

- Ferroelectricity

- Piezoelectricity

Module 2. Semi conductors. Magnetic materials

- Magnetic properties of Materials

- Classification of magnetic materials

- Diamagnetism

- Paramagnetism

- Ferromagnetism

- Magnetization curve

- The hysteresis loop

- Factors affecting permeability and hysteresis loss

- Types of Metal Alloys

- Steels

- Cast Irons

- Electrical Conduction in Solids

- Semiconductivity

- Semiconductor Devices

- Thermal Expansion

- Thermal Conductivity

- Thermal Stresses

- Basic Concepts of Magnetism

- Forms of Magnetism

- Hysteresis

- Hard and Soft Magnetic Materials

- Magnetic Storage	Superconductivity

- Electromagnetic Radiation

- Interaction of Light with Solids

- Common magnetic materials

- Magnetic resonance

- Magnetostatics

- Origin of magnetism in materials

- Magnetic domains and domain walls

- Magnetic anisotropy

- Reversible and irreversible magnetization processes

- Hard and soft magnetic materials

- Magnetic recording

- Semiconductors

- Energy band in solids

- Conductors, semiconductors and insulators

- Types of semiconductors

- Intrinsic semiconductors

- Impurity type semiconductor

- Diffusion, the Einstein relation

- Hall effect

- Thermal conductivity of semiconductors

- Electrical conductivity of doped materials.

- Basic Band Theory

- Essentials of the Free Electron Gas

- Diffraction of Electron Waves

- Energy Gaps and General Band Structure

- Periodic Potentials and Bloch's Theorem

- Band Structures and Standard Representations

- Basic Semiconductor Physics

- Intrinsic Properties in Equilibrium

- Doping, Carrier Concentration, Mobility, and Conductivity

- Lifetime and Diffusion Length

- Simple Junctions and Devices

- Elements of Advanced Theory

- Effective Masses

- Quasi Fermi Energies

- Shockley-Read-Hall Recombination

- Useful Relations

- Junction Reconsidered

9 Tips for planning and organizing the time needed to study the discipline

Before the lectures should be repeated previous lecture material. This will help in the assimilation of new material, to be ready to allow rapid survey of the lecture. Systematic repetition takes little time and in the future it will save in the process of preparation for classes, testing and examination. By repeating the lecture material is recommended to view the tutorial for the course, in which the material is considered in a broader context.

To laboratory classes should read and understand the lecture material by using the text of lectures and methodical materials on relevant topics. This will allow a meaningful approach to the implementation of the work. Making a report on laboratory work should be carried out at home. The process of registration is necessary once more to check theoretical material. Donating work should from its design, loosely and not accumulating debt.

- Criteria for readiness for practical training are the following: knowledge of the relevant literature, knowledge of research methods, the allocation of the essence of the phenomenon in the studied material, the ability to make logical constructions illustrate theoretical positions independently selected examples.

An important part of the student's work is to get acquainted with the recommended and additional literature as lecture material, with all its importance for the process of studying the discipline, contains only the minimum necessary theoretical knowledge. Higher education requires a deeper knowledge of the subject. Furthermore, it requires not only assimilate information, but also the formation of research skills. To do this, you must learn to independently analyze and articles of periodicals and Internet resources.

Students are advised as follows: to organize the time required for studying the discipline « Electrical materials »:

- The study of lecture notes on the same day after the lecture - 10 - 15 minutes;

- Repetition of the lecture the day before the next lecture - 10 - 15 minutes;

- Study of theoretical material on the recommended literature and synopsis - 1 hour per week;

- Recommended time to prepare for the laboratory work (with report design) - 2 hours;

- Preparation for the practical class – 1 hours.

10. Materials for self-checking

- Explain how you can understand the term atomic description of a dielectric?

- Explain how you can understand the term photoelectronic emission?

- Explain how you can understand the term polarization of a dielectric?

- Explain how you can understand the term thermionic emission?

- Explain the dependence of the conductivity of dielectric from temperature?

- Explain the dependence of the conductivity of metals from temperature?

- Explain the effect of temperature o  the electrical conductivity of metals?

- How change the capacitance of capacitor?

- How electron motion in electric field?

- What are main properties of metal?

- What is the factors affecting the resistivity of electrical materials your know?

- What is the place of alloys in the conductivity of metals?

- What is the role of the dielectric in the capacitor?

- What types of electric current can flow in the dielectric?

- What you can say about a parallel plate capacitor with a dielectric between the plates?

- What you can say about Coulomb’s Law in dielectrics?

- What you can say about electric conductivity in dielectric?

- What you can say about emission electron from metals?

- What you can say about energy level of atom?

- What you can say about Gauss’s Law in dielectrics?

- What you can say about random and direct motion of electron

- What you can say about the displacement vector?

- What you can say about the electric field vector?

- What you can say about the polarization vector?

- What is the different between conductor and dielectric band structure

- What is the different between positive and negative current

- What is the different between resistivity and conductivity

- Give the definition of the technical term: electrical conductivity

- Give the definition of the technical term: electron, proton, atom

- Give the definition of the technical term: emission of electrons

- Give the definition of the technical term: energy levels

- Give the definition of the technical term: extrinsic type of semiconductor

- Give the definition of the technical term: intrinsic type of semiconductor

- Give the definition of the technical term: n-type semiconductor

- Give the definition of the technical term: pleochroism

- Give the definition of the technical term: p-n junction

- Give the definition of the technical term: polar molecules

- Give the definition of the technical term: polarization of dielectric

- Give the definition of the technical term: p-type semiconductor

- Give the definition of the technical term: resistivity

- Give the definition of the technical term: semiconductor

- What is the different between conductor and semiconductor

- What is the different between electrical conductivity and thermal conductivity of semiconductors

- What is the different between intrinsic type and extrinsic type of semiconductor

- What is the different between conductor and semiconductor band structure

11.  Questions for the fourth current control

- How your can classify magnetic materials?

- What are the main properties of the metal – aluminium?

- What are the main properties of the metal – carbon  and  graphite?

- What are the main properties of the metal – copper?

- What are the main properties of the metal – iron and steel?

- What are the main properties of the metal – tungsten?

- What is the chemical properties of superconductor?

- What is the electron structure of superconductor?

- What is the physical properties of superconductor?

- What you can say about energy bands in semiconductor?

- What you can say about history of superconductor materials?

- What you can say about superconductivity?

- What you can say about superconductor materials?

- What you can say about the common magnetic materials?

- What you can say about the einstein relation?

- What you can say about the electrical conductivity of doped materials?

- What you can say about the energy stored in the electric field?

- What you can say about the factors affecting permeability and hysterisis loss?

- What you can say about the hall effect?

- What you can say about the magnetic properties of materials?

- What you can say about the magnetic resonance?

- What you can say about the non-linear conductors?

- What you can say about the the hysterisis loop?

- What you can say about the the magnetisation curve?

- What you can say about the thermal conductivity of semiconductors?

12.  Questions for the Ist midterm control

- Give the definition of the technical term: Atomic Description

- Give the definition of the technical term: conductor

- Give the definition of the technical term: dielectric

- Give the definition of the technical term: electric displacement vector

- Give the definition of the technical term: electrical conductivity

- Give the definition of the technical term: electron, proton, atom

- Give the definition of the technical term: emission of electrons

- Give the definition of the technical term: energy levels

- Give the definition of the technical term: the dielectric constant

- Give the definition of the technical term: the dielectric strength

- Explain how you can understand the term atomic description of a dielectric?

- Explain how you can understand the term photoelectronic emission?

- Explain how you can understand the term polarization of a dielectric?

- What you can say about a parallel plate capacitor with a dielectric between the plates?

- What you can say about Coulomb’s Law in dielectrics?

- What you can say about electric conductivity in dielectric?

- What you can say about emission electron from metals?

- What you can say about energy level of atom?

- What are the main properties of the metal – copper?

- What are the main properties of the metal – iron and steel?

- What are the main properties of the metal – tungsten?

- Solve  the problem::  A capacitor connected to a battery. A 12.00 pF air capacitor is connected to a 150 V battery (figure ). A piece of dielectric ( k=6.4) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor connected to a battery. A 12.00 pF air capacitor is connected to a 220 V battery (figure ). A piece of dielectric ( k=6) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor connected to a battery. A 300 pF air capacitor is connected to a 200 V battery (figure ). A piece of dielectric ( k=3.4) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor connected to a battery. A 5.50 pF air capacitor is connected to a 157 V battery (figure ). A piece of dielectric ( k=.4.6) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor connected to a battery. A 6.00 pF air capacitor is connected to a 100 V battery (figure ). A piece of dielectric ( k=5.4) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 25.0 cm2 and a plate separation of 2.00 mm, is charged by a 100 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=5.4) is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 27.0 cm2 and a plate separation of 4.00 mm, is charged by a 120 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=5.4)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 37.0 cm2 and a plate separation of 8.00 mm, is charged by a 220 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=17)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 7.0 cm2 and a plate separation of 1.00 mm, is charged by a 70 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=3.4)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

13.  Questions for the 2nd midterm control

- What is the different between Diamagnetic and Anti-ferromagnetic

- What is the different between Anti-ferromagnetic and Paramagnetic

- What is the different between Anti-ferromagnetic and Ferromagnetic

- What is the different between conductor and dielectric

- What is the different between conductor and superconductor

- What is the different between semiconductor and superconductor

- What is the different between dielectric and superconductor

- What is the different between n-type and p-type semiconductor

- What is the different between Diamagnetism and Paramagnetism

- What is the different between Diamagnetism and Ferromagnetism

- Explain how you can understand the term photoelectronic emission?

- Explain how you can understand the term thermionic emission?

- Explain the dependence of the conductivity of dielectric from temperature?

- Explain the dependence of the conductivity of metals from temperature?

- Explain the dependence of the conductivity of semiconductors from temperature?

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 7.0 cm2 and a plate separation of 1.00 mm, is charged by a 70 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=3.4)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 56.0 cm2 and a plate separation of 6.00 mm, is charged by a 380 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=7)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A potential of 127.0 V is applied to a parallel plate capacitor completely filled with a 4.50 mm piece of dielectric (k=5.4 ) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  A potential of 15.0 V is applied to a parallel plate capacitor completely filled with a 1.25 mm piece of dielectric (k=12) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  A potential of 220.0 V is applied to a parallel plate capacitor completely filled with a 0.50 mm piece of dielectric (k=8 ) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  A potential of 380.0 V is applied to a parallel plate capacitor completely filled with a 1.50 mm piece of dielectric (k=3.4 ) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  A potential of 50.0 V is applied to a parallel plate capacitor completely filled with a 2.50 mm piece of dielectric (k=5.4 ) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 1.00 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 5.00 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 11.00 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 1.50 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 4.55 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 3.85 µF. If dielectric ( k=5.4)  is placed between the plates of the capacitor what will the new capacitance be?

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 6 µF. If dielectric ( k=5.4)  is placed between the plates of the capacitor what will the new capacitance be?

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 12 µF. If dielectric ( k=12)  is placed between the plates of the capacitor what will the new capacitance be?

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 8.6 µF. If dielectric ( k=3.4)  is placed between the plates of the capacitor what will the new capacitance be?

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 8 µF. If dielectric ( k=7)  is placed between the plates of the capacitor what will the new capacitance be?

- How your can classify magnetic materials?

- What is the chemical properties of superconductor?

- What is the electron structure of superconductor?

- What is the physical properties of superconductor?

- What you can say about energy bands in semiconductor?

- What you can say about history of superconductor materials?

- What you can say about superconductivity?

- What you can say about superconductor materials?

- What you can say about the common magnetic materials?

- What you can say about the magnetic properties of materials?

- What you can say about the magnetic resonance?

- What you can say about the non-linear conductors?

- What you can say about the thermal conductivity of semiconductors?

14. Exam questions on the disciplines

I

- Give the definition of the technical term: amalgams

- Give the definition of the technical term: Atomic Description

- Give the definition of the technical term: conductor

- Give the definition of the technical term: dielectric

- Give the definition of the technical term: electric displacement vector

- Give the definition of the technical term: electrical conductivity

- Give the definition of the technical term: electron, proton, atom

- Give the definition of the technical term: emission of electrons

- Give the definition of the technical term: energy levels

- Give the definition of the technical term: extrinsic type of semiconductor

- Give the definition of the technical term: intrinsic type of semiconductor

- Give the definition of the technical term: Magnetic materials

- Give the definition of the technical term: Magnetic resonance

- Give the definition of the technical term: n-type semiconductor

- Give the definition of the technical term: pleochroism

- Give the definition of the technical term: p-n junction

- Give the definition of the technical term: polar molecules

- Give the definition of the technical term: polarization of dielectric

- Give the definition of the technical term: p-type semiconductor

- Give the definition of the technical term: resistivity

- Give the definition of the technical term: semiconductor

- Give the definition of the technical term: superconductivity

- Give the definition of the technical term: Superconductor

- Give the definition of the technical term: the dielectric constant

- Give the definition of the technical term: the dielectric strength

II

- What is the different between conductor and semiconductor

- What is the different between Ferromagnetism and Paramagnetism

- What is the different between electrical conductivity and thermal conductivity of semiconductors

- What is the different between intrinsic type and extrinsic type of semiconductor

- What is the different between conductor and semiconductor band structure

- What is the different between semiconductor and dielectric band structure

- What is the different between superconductor and dielectric band structure

- What is the different between conductor and dielectric band structure

- What is the different between positive and negative current

- What is the different between resistivity and conductivity

- What is the different between donors and acceptor

- What is the different between semiconductor and dielectric

- What is the different majority and minority carriers

- What is the different photoelectric emission and thermionic emission

- What is the different capacitor with a dielectric and capacitor without a dielectric

- What is the different between Diamagnetic and Anti-ferromagnetic

- What is the different between Anti-ferromagnetic and Paramagnetic

- What is the different between Anti-ferromagnetic and Ferromagnetic

- What is the different between conductor and dielectric

- What is the different between conductor and superconductor

- What is the different between semiconductor and superconductor

- What is the different between dielectric and superconductor

- What is the different between n-type and p-type semiconductor

- What is the different between Diamagnetism and Paramagnetism

- What is the different between Diamagnetism and Ferromagnetism

III

- Explain how you can understand the term atomic description of a dielectric?

- Explain how you can understand the term photoelectronic emission?

- Explain how you can understand the term polarization of a dielectric?

- Explain how you can understand the term thermionic emission?

- Explain the dependence of the conductivity of dielectric from temperature?

- Explain the dependence of the conductivity of metals from temperature?

- Explain the dependence of the conductivity of semiconductors from temperature?

- Explain the effect of temperature o  the electrical conductivity of metals?

- How change the capacitance of capacitor?

- How electron motion in electric field?

- What are main properties of metal?

- What is the factors affecting the resistivity of electrical materials your know?

- What is the place of alloys in the conductivity of metals?

- What is the role of the dielectric in the capacitor?

- What types of electric current can flow in the dielectric?

- What you can say about a parallel plate capacitor with a dielectric between the plates?

- What you can say about Coulomb’s Law in dielectrics?

- What you can say about electric conductivity in dielectric?

- What you can say about emission electron from metals?

- What you can say about energy level of atom?

- What you can say about Gauss’s Law in dielectrics?

- What you can say about random and direct motion of electron

- What you can say about the displacement vector?

- What you can say about the electric field vector?

- What you can say about the polarization vector?

IV

- Solve  the problem::  A capacitor connected to a battery. A 12.00 pF air capacitor is connected to a 150 V battery (figure ). A piece of dielectric ( k=6.4) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor connected to a battery. A 12.00 pF air capacitor is connected to a 220 V battery (figure ). A piece of dielectric ( k=6) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor connected to a battery. A 300 pF air capacitor is connected to a 200 V battery (figure ). A piece of dielectric ( k=3.4) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor connected to a battery. A 5.50 pF air capacitor is connected to a 157 V battery (figure ). A piece of dielectric ( k=.4.6) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor connected to a battery. A 6.00 pF air capacitor is connected to a 100 V battery (figure ). A piece of dielectric ( k=5.4) is now placed between the plates of the capacitor. Find: (a) the original charge on the plates of the capacitor, (b) the new value of the capacitance with the dielectric, (c) the new charge on the plates.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 25.0 cm2 and a plate separation of 2.00 mm, is charged by a 100 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=5.4) is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 27.0 cm2 and a plate separation of 4.00 mm, is charged by a 120 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=5.4)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 37.0 cm2 and a plate separation of 8.00 mm, is charged by a 220 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=17)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 7.0 cm2 and a plate separation of 1.00 mm, is charged by a 70 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=3.4)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A capacitor with a dielectric. A parallel plate capacitor, having a plate area of 56.0 cm2 and a plate separation of 6.00 mm, is charged by a 380 V battery. The battery is then removed. Find: (a) the capacitance of the capacitor, (b) the charge on the plates of the capacitor. A slab of dielectric (k=7)  is then placed between the plates of the capacitor. Find, (c) the new value of the capacitance, (d) the potential difference across the capacitor with the dielectric, (e) the initial electric field between the plates, (f) the final electric field between the plates, (g) the initial energy stored in the capacitor, and (h) the final energy stored in the capacitor.

- Solve  the problem::  A potential of 127.0 V is applied to a parallel plate capacitor completely filled with a 4.50 mm piece of dielectric (k=5.4 ) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  A potential of 15.0 V is applied to a parallel plate capacitor completely filled with a 1.25 mm piece of dielectric (k=12) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  A potential of 220.0 V is applied to a parallel plate capacitor completely filled with a 0.50 mm piece of dielectric (k=8 ) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  A potential of 380.0 V is applied to a parallel plate capacitor completely filled with a 1.50 mm piece of dielectric (k=3.4 ) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  A potential of 50.0 V is applied to a parallel plate capacitor completely filled with a 2.50 mm piece of dielectric (k=5.4 ) between the plates. Find the magnitude of: (a) the electric field E between the plates, (b) the electric displacement D between the plates, and (c) the polarization P between the plates.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 1.00 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 5.00 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 11.00 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 1.50 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem::  Find the effect of the dielectric on the force between one Na+ ion, and one Cl− ion, separated by a distance of 4.55 * 10^−7 cm, for the salt sodium chloride, NaCl, which is common table salt when it is placed in water.

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 3.85 µF. If dielectric ( k=5.4)  is placed between the plates of the capacitor what will the new capacitance be?

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 6 µF. If dielectric ( k=5.4)  is placed between the plates of the capacitor what will the new capacitance be?

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 12 µF. If dielectric ( k=12)  is placed between the plates of the capacitor what will the new capacitance be?

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 8.6 µF. If dielectric ( k=3.4)  is placed between the plates of the capacitor what will the new capacitance be?

- Solve  the problem:A parallel plate capacitor with air between the plates has a capacitance of 8 µF. If dielectric ( k=7)  is placed between the plates of the capacitor what will the new capacitance be?

V

- How your can classify magnetic materials?

- What are the main properties of the metal – aluminium?

- What are the main properties of the metal – carbon  and  graphite?

- What are the main properties of the metal – copper?

- What are the main properties of the metal – iron and steel?

- What are the main properties of the metal – tungsten?

- What is the chemical properties of superconductor?

- What is the electron structure of superconductor?

- What is the physical properties of superconductor?

- What you can say about energy bands in semiconductor?

- What you can say about history of superconductor materials?

- What you can say about superconductivity?

- What you can say about superconductor materials?

- What you can say about the common magnetic materials?

- What you can say about the einstein relation?

- What you can say about the electrical conductivity of doped materials?

- What you can say about the energy stored in the electric field?

- What you can say about the factors affecting permeability and hysterisis loss?

- What you can say about the hall effect?

- What you can say about the magnetic properties of materials?

- What you can say about the magnetic resonance?

- What you can say about the non-linear conductors?

- What you can say about the the hysterisis loop?

- What you can say about the the magnetisation curve?

- What you can say about the thermal conductivity of semiconductors?

Criteria for assessing students' knowledge

| Evaluation by letter system | Numeral equivalent of points | %- content | Evaluation according to the traditional system | Criteria for assessing students' knowledge |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | Excellent | The student demonstrates: - a deep and lasting assimilation of program material - complete, consistent, competent and logically presented answers for all types of activities; - Ability to freely cope with the tasks; - correct, well-founded decisions, - skills in the use of information from basic and additional specialized literature in the study of discipline, - the ability to self-systematize the program material; - Possession of versatile skills and methods of performing all types of tasks; - Ability to work with foreign literature and information resources of the Internet; - Timely and high-quality execution of all types of tasks. |
| А- | 3,67 | 90-94 |  | The student demonstrates: - Deep assimilation of program material; - complete, consistent and logically presented answers for all activities; - ability to cope with the tasks; - the right decisions, - skills of using special literature in the study of discipline, - the ability to self-systematize the program material; - Possession of skills and techniques for performing all types of tasks; - Timely execution of all types of tasks. |
| В+ | 3,33 | 85-89 | Good | The student demonstrates: - assimilation of program material; - complete, consistent, literate, without significant inaccuracies, an account of the answers for all kinds of tasks; - correct application of theoretical knowledge - Possession of necessary skills in the performance of applied tasks - skills in using recommended literature in the study of discipline, - skills of systematization of program material; - Possession of skills and techniques for performing all types of tasks; - Timely execution of all types of tasks. |
| В | 3,0 | 80-84 |  | The student demonstrates: - assimilation of program material; - a consistent presentation of the answers for all types of tasks with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - Possession of necessary skills in performing practical tasks - skills in using recommended literature in the study of discipline, - skills of systematization of program material under the guidance of a teacher; - mastering the skills of performing all kinds of tasks; - the ability to independently correct mistakes made; - the timely execution of all types of tasks with the elimination of errors. |
| В- | 2,67 | 75-79 |  | The student demonstrates: - assimilation of program material; - Ability to present answers with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - Possession of techniques in the performance of practical tasks - skills in using recommended literature under the guidance of a teacher - skills of generalization of program material under the guidance of a teacher; - the ability to correct mistakes made with the help of a teacher; - the timely execution of all types of tasks with the elimination of errors. |
| С+ | 2,33 | 70-74 | satisfactorily | The student demonstrates: - mastering of the main material; - Inadequate formulations when responding to all kinds of tasks; - violation of the sequence in the presentation of the program material; - difficulties in self-fulfillment of practical tasks; - Possession of individual techniques in the performance of practical tasks - skills in using the literature recommended by the teacher; - skills of generalization of separate sections of program material under the guidance of a teacher; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С | 2,0 | 65-69 |  | The student demonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - difficulties in self-fulfillment of practical tasks; - Possession of separate receptions at performance of tasks - difficulty in using the literature recommended by the teacher; - difficulties in generalizing the individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С- | 1,67 | 60-64 |  | The student demonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - Inadequate possession of individual methods in the performance of assignments - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| D+ | 1,33 | 55-59 |  | The student demonstrates: - mastering of separate sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - significant difficulties in the application of individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - difficulties in correcting the mistakes made by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| D | 1,0 | 50-54 |  | The student demonstrates: - difficulties in mastering individual sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - Inability to apply individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - Inability to generalize the individual sections of the material studied; - significant difficulties in correcting the mistakes committed by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| F | 0 | 0-49 | unsatisfactory | The student demonstrates: -- not having knowledge of program material, - When performing all types of tasks, gross errors are allowed; - lack of skills in applying individual tasks; - non-fulfillment of certain types of tasks provided by the forms of current, intermediate and final control. |

List of recommended literature

1 Pasynkov, V. V. Materıaly elektronnoı tehnıkı : ýcheb. dlıa výzov / V. V. Pasynkov, V. S. Sorokın. – 3-e ızd. – SPb. : Izd-vo «Lan», 2001. – 368 s. – ISBN 5-8114-0409-3.

2 Bogorodıtskıı, N. P. Elektrotehnıcheskıe materıaly : ýcheb. dlıa výzov / N. P. Bogorodıtskıı, V. V. Pasynkov, B. M. Tareev. – 7-e ızd. – L. : Energoızdat, 1985. – 304 s.

3 Jýravlёva, L. V. Elektromaterıalovedenıe : ýcheb. dlıa NPO / L. V. Jýravlёva.– 4-e ızd., pererab. ı ıspr. – M. : Akademııa, 2006. – 336 s.

4 Pererva, G. I. Elektrotehnıcheskıe materıaly / G. I. Pererva. – Gomel : Belgýt, 1999. – 80 s.

5 Spravochnık po elektrıcheskım materıalam / pod red. Iý. V. Korıtskogo, V. V. Pasynkova, B. M. Tareeva. –L. : Energoızdat, 1988. – T. 1–3.

6 Elektrotehnıcheskıe ı konstrýktsıonnye materıaly : ýcheb. posobıe dlıa SPO / V. N. Borodýlın [ı dr.] ; pod red. V. A. Fılıkova. – 2-e ızd., ster. – M. : Akademııa, 2005. – 280 s.

7 Gerasımov, V. G. Elektrotehnıcheskıı spravochnık : v 4 t. / V. G. Gerasımov. – 9-e ızd., ıspr. ı dop. – M. : Energoızdat, 2007. – T. 1: Obıe voprosy. Elektrotehnıcheskıe materıaly. – 440 s.

8 Alıev, I. I. Elektrotehnıcheskıe materıaly ı ızdelııa : spravochnık / I. I. Alıev, S. G. Kalganova . – M. : Energoızdat, 2006. – 352 s.

Korolkov A.V.

Methodical instructions and tasks on performance of SIW and SIWT

on discipline «Electrical materials »  for bachelor students

of specialty 5B071800 «Electroenergetics»

«      »         20       г.

Send for the press __________________

(date, month, year)

Paper format XxY  1/16

Printing paper. Offset printing. Volume ..…п.л.

Circulation ..….cop. Booking № ……*

© M.Auezov South Kazakhstan State University 2019 г.

Publishing center M.Auezov SKSU  ЮКГУ, Shymkent city, Tauke Khan avenue, 5, Building 5.