Ф.7-03-03

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

M.AUEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

CHAIR OF FOREIGN LANGUAGES FOR TECHNICAL SPECIALITIES

Mamekova S., Kurbanbayeva S.

Training aid

for the second year students

for practical lessons on discipline: “Professional-oriented foreign language”

for the students of 5В071900- Radio engineering, electronics and telecommunications specialities

Form of study: full-time

Shymkent 2019

UDC: 347.463

BBK 81.2 Англ.яз

Developed by Mamekova S., Kurbanbayeva S. the Training aid for practical lessons on discipline: Professional-oriented foreign (English) language for students of the 2 nd course of 5В071900- Radio engineering, electronics and telecommunications speciality of M.Auezov South Kazakhstan State University, Shymkent-2018, 83p.

The Training aid was developed in accordance with the requirements of State Educational Standards for specialties and curriculum of the given discipline. This instruction comprises all necessary information on themes performance for practical lessons of the course.

The Training aid is intended for students of the 2nd course of 5В071900- Radio engineering, electronics and telecommunications speciality studying English language. All lessons include short texts, exercises for consolidation material and lexics.

The Training aid is intended for the 2nd course students of the technical specialties.

Reviewers:

A.A. Suyuberdiyeva. - Candidate of pedagogical sciences, chair of foreign languages, South Kazakhstan State University, Shymkent

The Training aid is discussed and recommended for publicational the meeting of the Chair of foreign languages for technical specialties (minute №   «    »      2019) and at the methodical committee of the H Sc. IT&E (minute №    «    »      2019).

The Training aid is recommended for print by Academic-methodical Council of M. Auezov South Kazakhstan State University № __,  «_____»  ___________2019.

© M.Auezov South Kazakhstan State University, 2019.

Responsible for printing: S. Mamekova, S. Kurbanbayeva

Introduction

The Training aid is intended for students who have already had the base knowledge in pronunciation, grammar and a stock of words in volume of bachelor degree. This work is calculated for 30 hours and includes on the following units:

Unit 1: RADIO ENGINEERING;

Unit 2: RADIO TRANSMITTERS AND RECEIVERS;

Unit 3: BASIC PRINCIPLES OF TELEVISION;

Unit 4: BASIC PRINCIPLES TELECOMMUNICATION SYSTEMS FORMATION;

Unit 5: COMMUNICATON NETWORKS;

Unit 6 СELLULAR SYSTEMS AND STANDARTS.

The Training aid is made up for the 2nd -year students. All lessons include short texts, exercise for consolidation material and lexics and exercise for development of critical thinking skills, abilities to communicate in the professional field, ability to state and prove the opinion. There are different kinds of task in each section, like: phonetical, vocabulary-orthographical and lexical.

The Training aid is compiled in such a way that it gives the students the possibility of acquiring new grammatical and vocabulary knowledge independently and applying this knowledge for understanding the structure of typical sentences and scientific and technical terms of the Radio engineering, electronics and telecommunications speciality.

The texts’ tasks are aimed to consolidate the lexical and grammatical material and develop speaking, reading and writing skills in professional field.

The main purpose of this work is to develop and to process the basic communication skills necessary for the successful conduct of professional activities related to the search, transmission of technical information in a foreign language in professional sphere of the specialist. This Training aid will help students to improve self-work ability and expand the stock of words that will give the opportunity to read and understand texts of different levels of any complexity and teach them to state their point of view, find out the advantages and disadvantages of any discussed topic, think critically on the topic of conversation, treat the opponents in a respectful way and etc.

UNIT 1. RADIO ENGINEERING

Lessons 1-2-3-4

Radio engineering systems and channels of communication.

Organization and structural scheme of communication channels.

Radio signals and types of modulation

1. Memorize the words:

To travel – распространяться

transmitting range – дальность передачи

a receiver – приемник

a transmitter – передатчик

a high-frequency oscillator – высокочастотный генератор колебаний

an oscillatory circuit – колебательный контур

a capacitor – конденсатор

an amplifier – усилитель

a detector – детектор, следящий механизм

a rectifier – выпрямитель, детонатор

the audio frequency – звуковая частота

to couple together – соединять, спаривать

by means of a switch – с помощью переключателя (коммутатора)

means of communication – средства связи

telegraph sending key – телеграфный ключ

dots and dashes – точки и тире

the mirror galvanometer – зеркальный гальванометр

powdered carbon – порошковый углерод

a far sensitive receiver – гораздо более чувствительный приемник

wireless communication – беспроводная связь

a transmitting / receiving coil – передающая / приемная катушка

2. Read the text and explain the operation principle of radio communication: Radio communication

Radio communication is the transmission of high frequency energy from the transmitter to the receiver without wires. Radio is a device that transmits and receives signals and programs by electromagnetic waves. Since the process of radio communication includes transmission and reception of signals, the two necessary components of radio are a transmitter and a receiver.

The first component of radio, the transmitter, is a device producing radiofrequency energy. The transmitter consists of a high-frequency oscillator including an oscillatory circuit (a coil and a capacitor) and one or more amplifiers. Electric oscillations are produced in the antenna of the transmitter. They travel in all directions. Electron lamps are used to amplify currents and give greater transmitting range and better reception.

Radio waves are electric waves of very high frequency; they travel through space at a speed of light, and differ from other wave forms only n frequency (number of vibrations per second).

The second important component of radio communication is the receiver, a device that receives waves sent out by a transmitter. Radio receiver demodulates  these waves, and they are heard as speech, music or signals. To understand this process let us consider the principle of operation of these devices.

A microphone is connected to the circuit of the transmitting antenna. When we speak into the microphone its resistance varies with the audio frequency. An alternating current is established in the microphone and antenna circuits, and its frequency is the same as the audio frequency. Oscillations of the same frequency are induced in the antenna and the oscillatory circuit of a receiver. These oscillations are in fact a high-frequency current. In order to reproduce the transmitted sound, this current modulated by audio frequency should be sent through the telephone, and a detector or rectifier should be connected to the telephone circuit. The audio frequency rectified current passes through the telephone and produces oscillations. These oscillations will reproduce the sounds produced at the transmitting station. The operation of a radio set will be the better, the more energy is received by its oscillatory circuit. The oscillatory circuit is also provided with a ground. It is important for god operation of the receiver. The antenna should be grounded by means of a switch. Internet radio, also known as web radio or net radio is an audio broadcasting service transmitted via the Internet.

Internet radio services are usually accessible from anywhere in the world. This makes it popular among listeners with interests that are not adequately served by local radio stations. Internet radio services offer news, sorts, talk and various genres of music – everything that is available on traditional radio stations.

3. Look through the text and answer the questions:

1. What is radio communication?

2. What are the main components of radio?

3. What is a transmitter?

4. What does it consist of?

5. What is used for amplifying currents?

6. What happens in the microphone when we speak into it?

7. What kind of current is established in the microphone and antennas circuit?

8. What device should be connected to the circuit in order to reproduce the transmitted sounds?

9. By what means is the antenna grounded?

10. How are transmitted sounds reproduced in the receiver?

4. Find out synonyms between:

Nouns: specialist; improvement; traffic; dot; instrument; speed; transport; operation; expert; wire; point; conversation; invention; wireless; communication; tool; state; link; power; connection; perfection; possibility; prize; oscillations; bonus; capacitors; vibrations; energy; radio; feasibility; country; rate; work; conductor; discovery; speech; condenser; device.

Verbs: to provide; to produce; to establish; to demonstrate; to induce; to amplify; to involve; to build; to improve; to receive; to lay; to link; to continue; to invent; to use; to go on; to apply; to supply; to install; to intensify; to construct; to perfect; to connect; to put; to get; to advance; to include; to move forward; to excite; to show; to generate.

Adjectives: distant; fast; important; modern; several; wonderful; various; intelligible; simple; different; quick; some; far; primitive; understandable; remarkable; present-day; significant

5. Read the text and tell about developed communication systems:

Communication systems development

Long ago men found it necessary to communicate at a distance. When the alphabet was invented, they began to use papyrus and something like the modern letter appeared. The first to send letter were the ancient Egyptians. A runner delivered them. However, the Romans organized the best postal system of ancient times.

From then on until the 19-th century, there were practically no advances in the means of communication. Even when Queen Victoria began to rule England in 1837, her means of communication with distant parts of her empire were no faster than those of Julius Caesar.

The first practical electromagnetic telegraph was invented by the Russian scientist Pavel Shilling in 1828, and in 1832 he established telegraph communication between the Winter Palace and the Ministry of Transport in St. Petersburg. Shilling’s work was continued in Russia by B. Yakobi, who made several improvements in the electromagnetic telegraph and linked St. Petersburg with Tsarskoye Selo. This 25 kilometer-line was the longest in the world at that time. Yakobi invented the telegraph sending key, adopted by the American Samuel Morse. Morse, however, invented the telegraph code of dots and dashes, which is used all over the world to this day.

The first transatlantic telegraph cable from Europe to America was laid in 1858 due to the great British scientist Professor William Thomson. He also invented the mirror galvanometer, the very sensitive instrument used at first to receive signals transmitted over very long cables. Three letters could be transmitted per minute over the first transatlantic cable. The present speed of operation of telegraph cables reaches 2,500 letters per minute. The telephone is a much younger invention than the telegraph. The French mechanic Charles Boursel first suggested the idea of transmitting speech electrically.

The first telephone that found application was invented by the American Graham Bell in 1876. Russian inventors made several important improvements in the telephone. In 1879 the Russian engineer Mikhalsky made a microphone with powdered carbon, a prototype of the present-day microphone. Next year another Russian inventor, Golubitsky made a far sensitive receiver than the receiver of Bell. In 1880 the Russian military communication expert G. Ignatyev invented a device that made it possible to use the same wire simultaneously for a telephone conversation and for telegraph communication. Today the method of frequency modulation makes it possible to transmit several hundred telephone conversations over the same wire simultaneously.

The telegraph and the telephone were soon followed by an even more wonderful invention, which made possible communication without wires. Numerous scientists from different countries contributed to the appearance of wireless communication. Heinrich Hertz, constructed a primitive radio system capable of transmitting and receiving space waves through free space. In 1893, Nikola Tesla, in America, first demonstrated the feasibility of wireless communications. He proved that intelligible messages could be transmitted without wires and established a system which was composed of a transmitting coil (or conductor) and a receiving coil. At last, in 1895, the Russian scientist A.S. Popov demonstrated his first radio receiver. In March 1897 G. Marconi, an Italian inventor, transmitted wireless telegraphy signals over a distance of two miles and later he established the first transatlantic radio communication between Canada and England. For this achievement, he was awarded the Nobel Prize.

Early uses of communication were maritime for sending telegraphic messages using Morse code between ships and land. Radio was used to pass on orders and communications between armies and navies in World War I. Broadcasting became possible in the 1920s with the introduction of radio receivers in Europe and the U.S.A. Another use of radio was the development of detecting and locating aircraft and ships by the use of radar.

Today radio takes many forms, including wireless networks and mobile communications of all types, as well as radio broadcasting

6. Look through the text. Answer the questions using the information from the text:

1. When and how did the first messages appear?

2. Where were the first letters delivered?

3. Who established the first telegraph and when?

4. Who continued and improved the achievements of P. Shilling?

5. What did B. Yacobi invent?

6. What is S. Morse famous for?

7. Who first suggested the idea of transmitting speech?

8. What Russian engineers perfected the idea of telephone conversation?

9. What scientists contributed to the development of wireless communication?

10. Who was awarded the Nobel Prize and what for?

7. Translate the following word combinations:

Postal system; from then on; to establish telegraph communication; communication system development; to make several improvements; the telegraph sending key; he telegraph code of dots and dashes; to lay the cable; due to; to invent the mirror galvanometer; at first; to transmit per minute; to reach the speed of operation; the idea of transmitting speech; to find application; powdered carbon; a far sensitive receiver; to use the same wire simultaneously ; the feasibility of wireless communications; intelligible messages; a transmitting coil; at last; a radio receiver; to be awarded the Nobel Prize; to pass on orders; radio broadcasting; to become feasible; detecting and locating aircrafts and ships.

8. Memorize the words:

Information Communication Technologies (ICT) – информационно- коммуникативные технологии

the source output – исходный вывод a sequence of binary digits – последовательность двоичных цифр

a storage medium – запоминающая среда, носитель ЗУ

distinguishing characteristics – отличительные свойства

the theory of probability – теория вероятности an encoder – кодирующее устройство, шифратор

a decoder – декодирующее устройство, дешифратор

level of performance – уровень пропускной способности

random processes – беспорядочные, случайные процессы

additive noise – аддитивный (дополнительный) шум

prescribed delay – заданная задержка

software applications – прикладное программное обеспечение

installing applications – установка (размещение) прикладных программ

data management – управление (работа с) данными

networking – объединение в сеть

engineering computer hardware – конструирование аппаратного обеспечения компьютера

manipulate – управлять; обращаться с; обрабатывать; преобразовывать disseminate - распространять

stem (from) - происходить

9. Read the text and tell what the information communication technologies are:

Communication systems and information theory

Communication theory deals primarily with systems for transmitting information from one point to another. The source output might represent a voice waveform, a sequence of binary digits from a magnetic tape, he output of a set of sensors in a space probe, or a target in a radar system. The channel might represent a telephone line, a high frequency radio link, a space communication link, or a storage medium.

As it is known, in the early 1940’s a mathematical theory for dealing more fundamental aspects of communication systems was developed. The distinguishing characteristics of this theory are, first, a great emphasis on the theory of probability and, second, a primary concern with the encoder and decoder, both in terms of their functional roles and in terms of their achieving a given level of performance. In the past 20 years, information theory has been made more precise, has been extended and brought to the point where it is being applied in practical communication systems.

Much of modern communication theory stems from works of communication systems as well as from desirability of modelling both signal and noise as random processes. N. Wiener was interested in finding the best linear filter to separate the signal from additive noise with a prescribed delay. His work had an important influence on subsequent research in modulation theory.

Information technology (IT) is the study, design, development, implementation, support or management of computer-based information systems, particularly software applications and computer hardware. Information technology deals mainly with the use of electronic computers and computer software to convert, store, protect, process, transmit and securely retrieve information.

Today the term information technology includes many aspects of computing and technology and covers many fields. Information technology professionals perform a variety of duties that range from installing applications to designing complex computer networks and information databases. The duties of IT specialists may involve data management, networking, engineering computer hardware, database and software design, as well as the management and administration of the whole system. When computer and communication technologies are combined, the result is information technology, or “infotech”. Information technology describes any technology that helps to produce, manipulate, store, communicate and/or disseminate information.

Thus, Information Communication Technology (ICT) embraces all technologies for the communication of information. It includes any medium to record information (paper, pen, magnetic disc / tape, optical discs – CD / DVD, flash memory, etc.) and technology for broadcasting information – radio, television. It involves any technology for communicating through voice and sound or images – microphone, camera, loudspeaker, and telephone. At present it is apparently culminating to information communication with the help of Personal Computers (PCs) networked through the Internet, information technology that can transfer information using satellite system or intercontinental cables.

10. Revise the text and answer the questions:

1. What system does communication theory deal with?

2. When was the mathematical theory for communication systems developed?

3. What are the distinguishing features of the theory?

4. What does modern communication theory stem from?

5. What was Wienner’s contribution into the development of communication theory?

6. What is IT?

7. What does it deal with?

8. What jobs are IT experts engaged in?

9. What do you understand by “infotech”?

10. What technologies does ICT include?

Radio signals and types of modulation

1. Memorize the following word combinations:

Communication theory; primarily; the source output; a voice wave form; a sequence of binary digits; a set of sensors; a space probe; a high frequency radio link; a storage medium; distinguishing characteristics; a great emphasis on the theory of probability; a primary concern; encoder and decoder; in terms of; a given level of performance; to stem from; as well as; desirability; linear filter; additive noise; a prescribed delay; to have influence on subsequent research; implementation; support; management; particularly; software applications; computer hardware; to retrieve information securely; to cover many fields; perform a variety of duties; installing applications; to design complex computer networks and information databases; data management; networking; engineering computer hardware; database and software design; to disseminate information; to communicate through voice and sound or images; a loudspeaker; to network through the Internet.

2. Translate the following terms and word combinations into English:

Цифровая связь; технология организации дальней cвязи; методы управления; беспроводная связь; дополнительная информация; передача в режиме реального времени; заданная задержка; установка прикладных программ; сигнал от аддитивных шумов; набор датчиков; отличительные свойства; высокочастотная радиосвязь; теория вероятности; беспорядочные процессы; охватывать многие области; автоматизированные информационные системы; последовательность двоичных цифр; управлять; размещать прикладные программы; видеосвязь; уровень пропускной способности.

3. Read the text and translate it into Russian:

Radio waves

Radio waves are a type of electromagnetic radiation with wavelengths in the electromagnetic spectrum longer than infrared light. Radio waves have frequencies from 300 GHz to as low as 3 kHz, and corresponding wavelengths ranging from 1 millimeter (0.039 in) to 100 kilometers (62 mi). Like all other electromagnetic waves, they travel at the speed of light. Naturally, occurring radio waves are made by lightning, or by astronomical objects. Artificially generated radio waves are used for fixed and mobile radio communication, broadcasting, radar and other navigation systems, communication satellites, computer networks and innumerable other applications. Different frequencies of radio waves have different propagation characteristics in the Earth's atmosphere; long waves may cover a part of the Earth very consistently, shorter waves can reflect off the ionosphere and travel around the world, and much shorter wavelengths bend or reflect very little and travel on a line of sight.

To prevent interference between different users, the artificial generation and use of radio waves is strictly regulated by law, coordinated by an international body called the International Telecommunications Union (ITU). The radio spectrum is divided into a number of radio bands based on frequency, allocated to different uses.

Radio waves were first predicted by mathematical work done in 1867 by Scottish mathematical physicist James Clerk Maxwell. Maxwell noticed wavelike properties of light and similarities in electrical and magnetic observations. He then proposed equations that described light waves and radio waves as waves of electromagnetism that travel in space, radiated by a charged particle as it undergoes acceleration. In 1887, Heinrich Hertz demonstrated the reality of Maxwell's electromagnetic waves by experimentally generating radio waves in his laboratory. Many inventions followed, making the use of radio waves to transfer information through space. The study of electromagnetic phenomena such as reflection, refraction, polarization, diffraction, and absorption is of critical importance in the study of how radio waves move in free space and over the surface of the Earth. Different frequencies experience different combinations of these phenomena in the Earth's atmosphere, making certain radio bands more useful for specific purposes than others.

Radio waves travel at the speed of light in a vacuum. When passing through an object, they are slowed according to that object's permeability and permittivity. The wavelength is the distance from one peak of the wave's electric field to the next, and is inversely proportional to the frequency of the wave. The distance a radio wave travels in one second, in a vacuum, is 299,792,458 meters (983,571,056 ft) which is the wavelength of a 1-hertz radio signal. A 1-megahertz radio signal has a wavelength of 299.8 meters (984 ft).

In order to receive radio signals, for instance from AM/FM radio stations, a radio antenna must be used. However, since the antenna will pick up thousands of radio signals at a time, a radio tuner is necessary to tune in a particular signal. This is typically done via a resonator (in its simplest form, a circuit with a capacitor and an inductor). The resonator is configured to resonate at a particular frequency, allowing the tuner to amplify sine waves at that radio frequency and ignore other sine waves. Usually, either the inductor or the capacitor of the resonator is adjustable, allowing the user to change the frequency at which it resonates. The etymology of "radio" or "radiotelegraphy" reveals that it was called "wireless telegraphy", which was shortened to "wireless" in Britain. The prefix radio- in the sense of wireless transmission was first recorded in the word radio conductor, a description provided by the French physicist Edouard Branly in 1897. It is based on the verb to radiate (in Latin "radius" means "spoke of a wheel, beam of light, ray").

The word "radio" also appears in a 1907 article by Lee De Forest. It was adopted by the United States Navy in 1912, to distinguish radio from several other wireless communication technologies, such as the photo phone. The term became common by the time of the first commercial broadcasts in the United States in the 1920s. (The noun "broadcasting" itself came from an agricultural term, meaning "scattering seeds widely.") The term was adopted by other languages in Europe and Asia. British Commonwealth countries continued to use commonly the term "wireless" until the mid-20th century, though the magazine of the BBC in the UK has been called Radio Times ever since it was first published in the early 1920s.

In recent years, the more general term "wireless" has gained renewed popularity through the rapid growth of short-range computer networking, e.g., Wireless Local Area Network (WLAN), Wi-Fi, and Bluetooth, as well as mobile telephony, e.g., GSM and UMTS. Today, the term "radio" specifies the actual type of transceiver device or chip, whereas "wireless" refers to the lack of physical connections; one talks about radio transceivers, but other talks about wireless devices and wireless sensor networks.

Common types of radio waves in wireless communication

4. Translate the verbs and their derivatives:

To communicate – communication; communicative; uncommunicative; communicator.

To transmit – transmitter; transmission; transmitted; transmissible; transmitting (coil).

To receive – receiver; reception; receptive; receptivity; receiving (coil).

To follow – follower; following.

To contribute – contribution; contributor; contributory.

To invent – inventor; invention; invented.

To implement – implementation; implemented.

To retrieve – retrieval; retrievable; irretrievable.

To improve – improvement; improver; improved; unimproved; improvable; unimprovable.

To appear – to disappear; appearance; disappearance.

To establish – to disestablish; established; establishment.

To predict – predicted; prediction; predictor.

To address – addressability; addressable; addressee; addressing; addressless; addressness.

Sequence – sequent; sequential; sequencer; consequently. Function – functional; functionality; functionally/

5. Read the texts, study basic types of modulation and speak of them:

Basic types of modulation

Today vast amounts of information are communicated using radio communications systems. Both analogue radio communications systems and digital or data radio communications links are used.

However, one of the fundamental aspects of any radio communications transmission system is modulation, or the way in which the information is superimposed on the radio carrier.

In order that a steady radio signal or "radio carrier" can carry information, it must be changed or modulated in one way so that the information can be conveyed from one place to another.

There are very many ways in which a radio carrier can be modulated to carry a signal, each having its own advantages and disadvantages. The choice of modulation have a great impact on the radio communications system. Some forms are better suited to one kind of traffic whereas other forms of modulation will be more applicable in other instances. Choosing the correct form of modulation is a key decision in any radio communications system design.

There are three main ways in which a radio communications or RF signal can be modulated:

- Amplitude modulation, AM: as the name implies, this form of modulation involves modulating the amplitude or intensity of the signal.

Amplitude modulation was the first form of modulation to be used to broadcast sound, and although other forms of modulation are being increasingly used, amplitude modulation is still in widespread use.

- Frequency modulation, FM: this form of modulation varies the frequency in line with the modulating signal.

Frequency modulation has the advantage that, as amplitude variations do not carry any information on the signal, it can be limited within the receiver to remove signal strength variations and noise. As a result, this form of modulation has been used for many applications including high quality analogue sound broadcasting.

- Phase modulation, PM: as the name indicates, phase modulation varies the phase of the carrier in line with the modulating signal.

Phase modulation and frequency modulation have many similarities and are linked - one is the differential of the other. However, phase modulation lends itself to data transmissions, and as a result, its use has grown rapidly over recent years.

Each type of modulation has its own advantages and disadvantages, and accordingly they are all used in different radio communications applications.

In addition to the three main basic forms of modulation or modulation techniques, there are many variants of each type. Again, these modulation techniques are used in a variety of applications, some for analogue applications, and others for digital applications.

Angle Modulation

Angle modulation is a name given to forms of modulation that are based on altering the angle or phase of a sinusoidal carrier. Using angle modulation there is no change in the amplitude of the carrier.

The two forms of modulation that fall into the angle modulation category are frequency modulation and phase modulation. Both types of angle modulation, namely, frequency modulation and phase modulation are linked because frequency is the derivative of phase, i.e. frequency is the rate of change of phase.

Another way of looking at the link between the two types of modulation is that a frequency modulated signal can be generated by first integrating the modulating waveform and then using the result as the input to a phase modulator. Conversely, a phase modulated signal can be generated by first differentiating the modulating signal and then using the result as the input to a frequency modulator.

Signal bandwidth

One key element of any signal is the bandwidth it occupies. This is important because it defines the channel bandwidth required, and hence the number of channels that can be accommodated within a given segment of radio spectrum. With pressure on the radio spectrum increasing, the radio signal bandwidth is an important feature of any type of radio emission or transmission.

The bandwidth is governed by two major features:

- The type of modulation: Some forms of modulation use their bandwidth more effectively than others. Accordingly, where spectrum usage is of importance, this alone may dictate the choice of modulation.

- The bandwidth of the modulating signal: A law called Shannon's law determines the minimum bandwidth through which a signal can be transmitted. In general, the wider the bandwidth of the modulating signal, the wider the bandwidth required.

Modulating signal type

Apart from the form of modulation itself, the type of signal being used to modulate the carrier also has a bearing on the signal. Analogue and data are two very different forms of modulating signal and need to be treated differently. While different formats of actual modulation may be used, the type of signal being applied via the modulator also have a bearing on the signal.

Signals for high quality stereo broadcasting will be treated differently to signals that provide digital telemetry for example. As a result, it is often important to know the signal type that needs to be carried by the RF carrier.

By Ian Poole

UNIT 2 RADIO TRANSMITTERS AND RECEIVERS

Lessons 5-6-7-8-9

Classification and main characteristics of radio transmitters and receivers. Detection of oscillations. Direct amplification radio receiver. Super heterodyne receiver. Structural scheme of radio transmitter. Specification of main radio nodes

1. Memorize the words:

carrier wave – несущая волна

sine - синус

refer – отсылать; направлять; справляться; упоминать

slight – тонкий; хрупкий; легкий; незначительный

power supply – источник энергии

to amplify – усиливать

omnidirectional – действующий по всем направлениям

transverse – поперечный

conversely – наоборот

to exert – осуществлять;

оказывать back and forth – назад и вперед

2. Read the text and speak about the principle of operation of a transmitter:

Radio transmitters

A radio transmitter consists of several elements that work together to generate radio waves that contain useful information such as audio, video, or digital data.

- Power supply: provides the necessary electrical power to operate the transmitter.

- Oscillator: creates alternating current at the frequency on which the transmitter will transmit. The oscillator usually generates a sine wave, which is referred to as a carrier wave.

- Modulator: adds useful information to the carrier wave. There are two main ways to add this information. The first, called amplitude modulation or AM, makes slight increases or decreases to the intensity of the carrier wave. The second, called frequency modulation or FM, makes slight increases or decreases to the frequency of the carrier wave.

- Amplifier: amplifies the modulated carrier wave to increase its power. The more powerful the amplifier, the more powerful the broadcast.

- Antenna: converts the amplified signal to radio waves.

3. Read the text and define the role of antennas in radio communication:

Radio antennas

An antenna (or aerial) is an electrical device that converts electric power into radio waves, and vice versa. It is usually used with a radio transmitter or radio receiver. In transmission, a radio transmitter supplies an electric current oscillating at radio frequency (i.e. a high frequency alternating current (AC)) to the antenna's terminals, and the antenna radiates the energy from the current as electromagnetic waves (radio waves). In reception, an antenna intercepts some of the power of an electromagnetic wave in order to produce a tiny voltage at its terminals, which is applied to a receiver to be amplified.

Antennas are essential components of all equipment that uses radio. They are used in systems such as radio broadcasting, broadcast television, two-way radio, communications receivers, radar, cell phones, and satellite communications, as well as other devices such as garage door openers, wireless microphones, Bluetooth-enabled devices, wireless computer networks, baby monitors, and RFID tags on merchandise.

Typically an antenna consists of an arrangement of metallic conductors (elements), electrically connected (often through a transmission line) to the receiver or transmitter. An oscillating current of electrons forced through the antenna by a transmitter will create an oscillating magnetic field around the antenna elements, while the charge of the electrons also creates an oscillating electric field along the elements. These time-varying fields radiate away from the antenna into space as a moving transverse electromagnetic field wave. Conversely, during reception, the oscillating electric and magnetic. fields of an incoming radio wave exert force on the electrons in the antenna elements, causing them to move back and forth, creating oscillating currents in the antenna.

Antennas can be designed to transmit and receive radio waves in all horizontal directions equally (omnidirectional antennas), or preferentially in a particular direction (directional or high gain antennas). In the latter case, an antenna may also include additional elements or surfaces with no electrical connection to the transmitter or receiver, such as parasitic elements, parabolic reflectors or horns, which serve to direct the radio waves into a beam or other desired radiation pattern.

The first antennas were built in 1888 by German physicist Heinrich Hertz in his pioneering experiments to prove the existence of electromagnetic waves predicted by the theory of James Clerk Maxwell. Hertz placed dipole antennas at the focal point of parabolic reflectors for both transmitting and receiving.

4. Read the text and tell how radio receivers work:

Radio receivers

A radio receiver is the opposite of a radio transmitter. It uses an antenna to capture radio waves, processes those waves to extract only those waves that are vibrating at the desired frequency, extracts the audio signals that were added to those waves, amplifies the audio signals, and finally plays them on a speaker.

- Antenna: captures the radio waves. Typically, the antenna is simply a length of wire. When this wire is exposed to radio waves, the waves induce a very small alternating current in the antenna.

- RF amplifier: A sensitive amplifier that amplifies the very weak radio frequency (RF) signal from the antenna so that the signal can be processed by the tuner.

- Tuner: A circuit that can extract signals of a particular frequency from a mix of signals of different frequencies. On its own, the antenna captures radio waves of all frequencies and sends them to the RF amplifier, which dutifully amplifies them all.

Unless you want to listen to every radio channel at the same time, you need a circuit that can pick out just the signals for the channel you want to hear. That’s the role of the tuner.

The tuner usually employs the combination of an inductor (for example, a coil) and a capacitor to form a circuit that resonates at a particular frequency. This frequency, called the resonant frequency, is determined by the values chosen for the coil and the capacitor. This type of circuit tends to block any AC signals at a frequency above or below the resonant frequency.

You can adjust the resonant frequency by varying the amount of inductance in the coil or the capacitance of the capacitor. In simple radio receiver circuits, the tuning is adjusted by varying the number of turns of wire in the coil. More sophisticated tuners use a variable capacitor (also called a tuning capacitor) to vary the frequency.

- Detector: Responsible for separating the audio information from the carrier wave. For AM signals, this can be done with a diode that just rectifies the alternating current signal. What is left after the diode has its way with the alternating current signal is a direct current signal that can be fed to an audio amplifier circuit. For FM signals, the detector circuit is a little more complicated.

- Audio amplifier: This component's job is to amplify the weak signal that comes from the detector so that it can be heard. This can be done using a simple transistor amplifier circuit.

Of course, there are many variations on this basic radio receiver design. Many receivers include additional filtering and tuning circuits to better lock on to the intended frequency — or to produce better-quality audio output — and exclude other signals. Still, these basic elements are found in most receiver circuits.

5. Read the text and tell about the design and principle of operation of a super heterodyne receiver. Use the bloc diagram:

Super heterodyne receiver

In electronics, a super heterodyne receiver (often shortened to superhet) uses frequency mixing to convert a received signal to a fixed intermediate frequency (IF) which can be more conveniently processed than the original radio carrier frequency. It was invented by US engineer Edwin Armstrong in 1918 during World War I. Virtually all modern radio receivers use the super heterodyne principle. At the cost of an extra frequency converter stage, the super heterodyne receiver provides superior selectivity and sensitivity compared with simpler designs.

Design and principle of operation

The principle of operation of the super heterodyne receiver depends on the use of heterodyning or frequency mixing. The signal from the antenna is filtered sufficiently at least to reject the image frequency and possibly amplified. A local oscillator in the receiver produces a sine wave, which mixes with that signal, shifting it to a specific intermediate frequency (IF), usually a lower frequency. The IF signal is itself filtered and amplified and possibly processed in additional ways. The demodulator uses the IF signal rather than the original radio frequency to recreate a copy of the original information (such as audio).

The diagram at right shows the minimum requirements for a single conversion super heterodyne receiver design. The following essential elements are common to all super heterodyne circuits: a receiving antenna; a tuned stage, which may optionally contain amplification (RF amplifier); a variable frequency local oscillator; a frequency mixer; a band pass filter and intermediate frequency (IF) amplifier; and a demodulator plus additional circuitry to amplify or process the original audio signal (or other transmitted information).

6. Write down the translation of these two texts and entitle them:

Text A

As it is known the history of radio transmitters dates back to 1895 when a great Russian scientist A. Popov transmitted the first radiogram. Since that time many Russian and foreign scientists contributed much to the theory of radio transmitting devices.

The function of the radio transmitter is to convert the electrical power received from a primary source into radio-frequency energy modulated with a signal for transmission by means of electromagnetic waves through space.

The radio transmitter consists of two principal components: the radiofrequency section and the audio-frequency one. The radio-frequency section produces radio-frequency power of continuous waves, the audio-frequency section being concerned with modulation of radio signals.

The parameters of the radio transmitter are output power, frequency stability, efficiency and modulation. Radio transmitters are classified into many different types. When classifying them according to the service for which they are used, radio transmitters may be of communication, broadcast, radar and other types. Taking into consideration the type of transmitting signals, specialists subdivide radio transmitters into telegraph, telephone and pulse transmitters. According to the power consumed transmitters are of low power, medium power and other types. At last, they may be of fixed and mobile types. To meet the requirements of high transmission quality of radio transmitters much is being done for improving radio transmitters’ performance by developing new design of these devices.

Text B

As it is known, that Russian scientist V. Siforov worked out the theory of radio receiving devices. However, A. Popov invented and demonstrated the first radio receiving set. Since that time, radio devices have been improving and perfecting.

The receiver performs the function of converting the current in the receiving antenna into the intelligence contained in the transmission the man parameters of radio receivers are sensitivity, selectivity and fidelity. Sensitivity is a measure of the receiver’s ability to receive weak signals, as it is known that the farther an electromagnetic wave travels, the weaker is its energy. Selectivity is the ability of the receiver to reject undesirable signals. Fidelity is a measure of the receiver’s ability to reproduce clearly audio-frequency currents, which are in accordance with the modulation envelope of the received signals.

However simple the radio receiver may be, it includes an antenna, an input tuning circuit, a detector and a pair of earphones.

The principle of operation of the radio receiver is not very difficult to understand. The electromotive force is impressed upon the receiving antenna and produces a current; this current is a reproduction of the current of the transmitting antenna.

There are various types of receivers, communication and broadcast receivers being the principal types of them. Communication receivers are used in radiotelephone and telegraph service, broadcast receivers finding application for the reception of sound and visual programs. Wherever radio receivers were applied, they must meet an important requirement as reliability in operation.

7. Answer the following questions:

- What is radio communication?

- What are the main components of radio communication?

- What does a transmitter consist of?

- What is used for amplifying currents?

- What happens in the microphone when we speak into it?

- What are the advantages of Internet radio?

- What new sciences could develop due to radio communication?

- What are distinguishing features of mathematical theory dealing with communication system?

- What does the modern communication theory stem from?

- What was Wienner’s contribution into the development of communication theory?

UNIT 3 BASIC PRINCIPLES OF TELEVISION

Lessons 10-11-12-13

Basic principles of television; television sets and video technique. Image formation. Broadcasting standards. Analog-to-digital conversion. Antennas

1. Memorize the words:

capability – способности, возможности

to override the effects - устранить эффекты

appropriate – соответствующий, подходящий

to distinguish – различать

loudness – громкость

pitch (of voice) – высота

to intersect – пересекать(ся); перекрещивать(ся)

to reflect – отражать(ся)

distortion – искажение

to diverge – отклоняться

to perceive – чувствовать; ощущать

to converge – сходиться; стремиться к пределу

regardless of – не считаясь с

to emanate – излучаться

a curved mirror – искривленное зеркало

likeness – сходство

a concave mirror – вогнутое зеркало

a plane mirror - плоское зеркало

to sight – наблюдать, видеть

a lens - линза

incidental – случайный, несущественный, побочный

2. Read the texts, translate them and speak about different techniques of image formation:

A television picture

Human perception of motion.

A television system involves equipment located at the source of production, equipment located in the home of the viewer, and equipment used to convey the television signal from the producer to the viewer. The purpose of all of this equipment is to extend the human senses of vision and hearing beyond their natural limits of physical distance. A television system must be designed, therefore, to embrace the essential capabilities of these senses, particularly the sense of vision. The aspects of vision that must be considered include the ability of the human eye to distinguish the brightness, colors, details, sizes, shapes, and positions of objects in a scene before it. Aspects of hearing include the ability of the ear to distinguish the pitch, loudness, and distribution of sounds. In working to satisfy these capabilities, television systems must strike appropriate compromises between the quality of the desired image and the costs of reproducing it. They must also be designed to override, within reasonable limits, the effects of interference and to minimize visual and audial distortions in the transmission and ... (200 of 21,814 words).

Plane mirror image formation

An image is formed by a plane mirror as light emanates from an object in a variety of directions. Some of this light reaches the mirror and reflects off the mirror according to the law of reflection. Each one of these rays of light can be extended backwards behind the mirror where they will all intersect at a point (the image point). Any person who is positioned along the line of one of these reflected rays can sight along the line and view the image - a representation of the object. Thus, an image location is a location in space where all the reflected light appears to come from. Since light from the object appears to diverge from this location, a person who sights along a line at this location will perceive a replica or likeness of the actual object. In the case of plane mirrors, the image is said to be a virtual image. Virtual images are images that are formed in locations where light does not actually reach. Light does not actually pass through the location on the other side of the plane mirror; it only appears to an observer as though the light were coming from this position.

Curved mirror image formation

We have also seen how images are created by the reflection of light off curved mirrors. Suppose that a light bulb is placed in front of a concave mirror; the light bulb will emit light in a variety of directions, some of which will strike the mirror. Each individual ray of light will reflect according to the law of reflection. Upon reflecting, the light will converge at a point. At the point where the light from the object converges, a replica or likeness of the actual object is created; this replica is known as the image. Once the reflected light rays reached the image location, they begin to diverge. The point where all the reflected light rays converge is known as the image point. Not only is it the point where light rays converge, it is also the point where reflected light rays appear to an observer to be coming from. Regardless of the observer's location, the observer will see a ray of light passing through the real image location. To view the image, the observer must line her sight up with the image location in order to see the image via the reflected light ray. The diagram below depicts several rays from the object reflecting from the mirror and converging at the image location. The reflected light rays then begin to diverge, with each one being capable of assisting an individual in viewing the image of the object.

For plane mirrors, virtual images are formed. Light does not actually pass through the virtual image location; it only appears to an observer as though the light was emanating from the virtual image location. The image formed by this concave mirror is a real image. When a real image is formed, it still appears to an observer as though light is diverging from the real image location. Only in the case of a real image, light is actually passing through the image location.

Converging Lens Image Formation

Converging lenses can produce both real and virtual images while diverging lenses can only produce virtual images. The process by which images are formed for lenses is the same as the process by which images are formed for plane and curved mirrors. Images are formed at locations where any observer is sighting as they view the image of the object through the lens. Therefore, if the path of several light rays through a lens is traced, each of these light rays will intersect at a point upon refraction through the lens. Each observer must sight in the direction of this point in order to view the image of the object. While different observers will sight along different lines of sight, each line of sight intersects at the image location. The diagram below shows several incidental rays emanating from an object - a light bulb. Three of these incident rays correspond to our three strategic and predictable light rays. Each incident ray will refract through the lens and be detected by a different observer (represented by the eyes). The location where the refracted rays are intersecting is the image location.

In this case, the image is a real image since the light rays are actually passing through the image location. To each observer, it appears as though light is coming from this location.

Diverging lens image formation

Diverging lenses create virtual images since the refracted rays do not actually converge to a point. In the case of a diverging lens, the image location is located on the object's side of the lens where the refracted rays would intersect if extended backwards. Every observer would be sighting along a line in the direction of this image location in order to see the image of the object. As the observer sights along this line of sight, a refracted ray would come to the observer's eye. This refracted ray originates at the object, and refracts through the lens. The diagram below shows several incident rays emanating from an object - a light bulb. Three of these incident rays correspond to our three strategic and predictable light rays. Each incident ray will refract through the lens and be detected by a different observer (represented by the eyes). The location where the refracted rays are intersecting is the image location. Since refracted light rays do not actually exist at the image location, the image is said to be a virtual image. It would only appear to an observer as though light were coming from this location to the observer's eye.

Images of objects that do not occupy a single point

The above discussion relates to the formation of an image by a "point object" - in this case, a small light bulb. The same principles apply to objects that occupy more than one point in space. For example, a person occupies a multitude of points in space. As you sight at a person through a lens, light emanates from each individual point on that person in all directions. Some of this light reaches the lens and refracts. All the light that originates from one single point on the object will refract and intersect at one single point on the image. This is true for all points on the object; light from each point intersects to create an image of this point. The result is that a replica or likeness of the object is created as we sight at the object through the lens. This replica or likeness is the image of that object. This is depicted in the diagram below.

Now speaking about image formation, we will turn our attention to the use of ray diagrams to predict the location and characteristics of images formed by converging and diverging lenses.

In electronics, a digital-to-analog converter (DAC, D/A, D2A or D-to-A) is a function that converts digital data (usually binary) into an analog signal (current, voltage, or electric charge). An analog-to-digital converter (ADC) performs the reverse function. Unlike analog signals, digital data can be transmitted, manipulated, and stored without degradation, albeit with more complex equipment. But a DAC is needed to convert the digital signal to analog to drive an earphone or loudspeaker amplifier in order to produce sounds (analog air pressure waves).

DACs and their inverse, ADCs, are part of an enabling technology that has contributed greatly to the digital revolution. To illustrate, consider a typical longdistance telephone call. The caller's voice is converted into an analog electrical signal by a microphone, and then the analog signal is converted to a digital stream by an ADC. The digital stream is then divided into packets where it may be mixed with other digital data, not necessarily audio. The digital packets are then sent to the destination, but each packet may take a completely different route and may not even arrive at the destination in the correct time order. The digital voice data is then extracted from the packets and assembled into a digital data stream. A DAC converts this into an analog electrical signal, which drives an audio amplifier, which in turn drives a loudspeaker, which finally produces sound.

There are several DAC architectures; the suitability of a DAC for a particular application is determined by six main parameters: physical size, power consumption, resolution, speed, accuracy, cost. Due to the complexity and the need for precisely matched components, all but the most special DACs are implemented as integrated circuits (ICs). Digital-to-analog conversion can degrade a signal, so a DAC should be specified that that has insignificant errors in terms of the application.

DACs are commonly used in music players to convert digital data streams into analog audio signals. They are also used in television sets and mobile phones to convert digital video data into analog video signals that connect to the screen drivers to display monochrome or color images. These two applications use DACs at opposite ends of the speed/resolution trade-off. The audio DAC is a low speed high resolution type while the video DAC is a high speed low to medium resolution type. Discrete DACs would typically be extremely high speed low resolution power hungry types, as used in military radar systems. Very high-speed test equipment, especially sampling oscilloscopes, may also use discrete DACS.

3. Read the texts without a dictionary and retell them:

Television and telecommunication

Television is a widely used telecommunication medium for sending (broadcasting) and receiving moving images, either monochromatic (black and white) or color, usually accompanied by sound.

In its early stages of development, television included only those devices employing a combination of optical, mechanical and electronic technologies to capture, transmit and display a visual image. As it is known, all modern television systems are based on electronic technologies, however the knowledge gained from the work on mechanical-dependent systems was important in the development of electronic television.

In 1884 Paul G. Nipkow, a 20-year old university student in Germany invented the first electromechanical television system which employed a scanning disk, a spinning disk with a series of holes spiraling toward the center, for “rasterization”, the process of converting a visual image into a stream of electrical pulses. The beginning of the 20-th century brought advances in amplifier tube technology and the use of a rotating mirror-drum scanner to capture the image. Most of the 20-th century television sets depended also upon the cathode-ray tube invented by Karl Braun in 1921.

Telecommunication, transmission of signals over a distance for the purpose of communication, is an important part of modern society. In telecommunication, a communication system is a collection of individual communications networks, transmission systems, relay stations, substations and data terminal equipment usually capable of interconnection and interoperation to form an integral whole. The components of a communication system serve a common purpose, they are technically compatible, use common procedures, respond to controls and operate in unison.

Radar

The word “radar” means Radio Determination and Ranging. Radar equipment is capable of determining by radio echoes the presence of objects, their direction, range and recognizing their character. Radar detects objects at a distance by reflecting radio waves off them. The delay caused by the echo measures he distance. The direction of the beam determines the direction of the reflection. The polarization and frequency of the return can sense the type of surface.

There are several types of radar sets, all of them consisting of six essential components, namely: a transmitter, a receiver, an antenna system, an indicator, a timer and, of course, a power supply.

A radar set detects by sending out short powerful pulses of ultra-high frequency radio wave energy from a high power transmitter. The directional antenna takes this energy from the transmitter and radiates it in a beam (similar to that of a searchlight). As the transmitted energy strikes an object, a portion of it is reflected back. The receiver picks up the returning echo through its antenna and translates it into visual readable signals on a fluorescent screen. The appearance of these signals show the presence of an object within the field of view of radar.

Navigational radars scan a wide area two to four times per minute. They use very short waves that reflect from earth and stone. They are common on ships and long-distance aircraft. General-purpose radars generally use navigational radar frequencies, but modulate and polarize the pulse so he receiver can determine the type of surface of the reflector. Search radars scan a wide area with pulses of short radio waves and sometimes use the Doppler effect to separate moving vehicles from clutter. Weather radars can even measure wind speed.

4. Coordinate the words given in the left column with their interpretation in the right:

1) Radio 	a) a system for conveying speech over distances by converting sounds into electric impulses sent through a wire.

2) Computer 	b) a method, process for handling a specific technical problem

3) Internet 	c) communicating over by converting sounds or signals into electromagnetic waves and transmitting them through space.

4) Telegraph 	d) a circuit devise that determines the content of a given instruction or performs digital – to – analog conversion.

5) Telephone 	e) an apparatus or system that converts a coded message into electric impulses and sends it to a distant receiver.

6) Decoder 	f) an electronic machine which, by means of stored instructions and information, performs complex calculations

7) Networking 		g) process of development or gradual progressive change

8) Evolution 	h) a world-wide network of computers, communicating with each other by using Internet Protocol.

9) Modulation 	i) the interconnection of computer systems over communication lines.

10) Technology 	j) a variation in the amplitude frequency or phase in accordance with some signal.

MIDTERM CONTROL 1

Lesson 14

Задания для рубежного контроля включают:

перевод аутентичных  текстов научно-технического характера, имеющих профессиональную направленность.

задания рубежного контроля 1

I. Устно

1. Перевод текста 1 с английского языка на русский (объем 3 тыс. печ. знаков).

2. Выполнение заданий по содержанию текста (составление 10 вопросов)

II. Письменно

3. Составление терминологического словаря (не менее 20 терминов).

4.Выполнение лексико-грамматических заданий.

5.Написание аннотации на английском и русском языках к тексту 2.

Tasks for midterm control include:

Translation of authentic texts of a scientific and technical nature, having a professional orientation.

Tasks of midterm control 1

I. Oral form

1. Translation of text 1 from English into Russian (volume 3 thousand typed characters).

2. Tasks performance on the content of the text (compilation of 10 questions)

II. Writing form

3. Compiling a terminological dictionary (at least 20 terms).

4. Tasks performance of lexical and grammatical assignments.

5. Writing an annotation in English and Russian to the text 2.

UNIT 4 BASIC PRINCIPLES OF TELECOMMUNICATION SYSTEMS FORMATION

Lessons 15-16-17-18-19-20

Basic principles of telecommunication systems formation. Symmetric. Coaxial and optic signal transmission media. Ways of introduction and transformation of messages. Signals and interfaces, multichannel telecommunication systems (principles of formation and structural schemes).

Digital telecommunication networks (types and peculiarities).

1. Read the texts, translate them and speak about Information and Communication Technology

The Information and Communication Technology guide (Myrzabay B)

The Information and Communication Technology (ICT) Industry is one of the fastest growing and changing fields. Technology is developing rapidly and new devices and systems are constantly being created to provide faster and more efficient methods for information and communication technology. Careers in the ICT field are in high demand and the industry is expected to grow. Professionals in ICT careers may work in commercial service sectors and ICT companies.

Careers in information and communication technology fields include a variety of roles and tasks associated with planning, researching, installing and monitoring the performance of ICT systems and transmissions. ICT support technicians assist with planning and installing ICT hardware and systems and repair any damages or defects. Sales professionals in the ICT industry provide information and sell ICT hardware, systems and services on behalf of ICT companies. ICT trainers develop training programs and user guides to assist customers and clients operate systems. ICT business and systems analysts evaluate and develop technology systems to help businesses run at optimal standards and ICT managers direct and oversee the operations and facilitation of ICT services in companies and organizations.

ICT professionals should have technical expertise in computer systems, hardware and networks, and should have exceptional problem solving and analytical abilities to diagnose and repair defects or malfunctions in the system. Individuals in the ICT field should have natural interest and capabilities with ICT programs and information computer systems. They should be able to grasp large amounts of information and be able to quickly learn and adapt to new technologies and advances in the field. Individuals wanting to get into the field should complete a Bachelors degree in related ICT studies and gain hands-on experience.

2. Look through the text and answer the questions:

- What is the ICT?

2. Where students may work after graduating?

3. Do you like your own ICT professions?

4. Why you choose this profession?

3. Read the text and tell about digital communication:

Digital communication

In the design of large and complex digital systems, it is often necessary to have one device communicate digital information to and from other devices. One advantage of digital information is that it tends to be far more resistant to transmitted and interpreted errors than information symbolized in an analog medium. This accounts for the clarity of digitally-encoded telephone connections, compact audio disks, and for much of the enthusiasm in the engineering community for digital communications technology. However, digital communication has its own unique pitfalls, and there are multitudes of different and incompatible ways in which it can be sent. Hopefully, this chapter will enlighten you as to the basics of digital communication, its advantages, disadvantages, and practical considerations. Suppose we are given the task of remotely monitoring the level of a water storage tank. Our job is to design a system to measure the level of water in the tank and send this information to a distant location so that other people may monitor it. Measuring the tank’s level is quite easy, and can be accomplished with a number of different types of instruments, such as float switches, pressure transmitters, ultrasonic level detectors, capacitance probes, strain gauges, or radar level detectors. For the sake of this illustration, we will use an analog level-measuring device with an output signal of 4-20 mA. 4 mA represents a tank level of 0%, 20 mA represents a tank level of 100%, and anything in between 4 and 20 mA represents a tank level proportionately between 0% and 100%. If we wanted to, we could simply send this 4-20 milliamp analog current signal to the remote monitoring location by means of a pair of copper wires, where it would drive a panel meter of some sort, the scale of which was calibrated to reflect the depth of water in the tank, in whatever units of measurement preferred.

4. Look through the text and answer the questions:

- What’s need to design in D.C.?

- For what you should to have an account?

- What’s a job for design a system?

5. Read the text, translate it and find out new words. Find their definitions.

Symmetrical and optical signal transmission

Signal will show a delivery status for sent Signal messages and sent insecure SMS messages.  You can disable delivery reports for SMS messages through Signal > Settings > SMS and MMS > SMS delivery reports. To see more details long-press on a message bubble and select Info from the header options.

One check mark 
One check mark in your message bubble indicates that your Signal message has been sent to our servers or your insecure SMS was sent to your provider. If you see  this, there is no issue with your phone.

Two check marks
Two check marks in your message bubble indicate that the Signal message or insecure SMS has been delivered to your contact's device. This does not indicate the message was read. Your contact will only get this message if their phone is online.

Two blue check marks
Two blue check marks in your message bubble indicate that the Signal message was read. Messages are marked as read if both you and your contact have read receipts enabled.

What can I do if my Signal message was not delivered?

- You can wait until your contact has an internet connection and Signal can connect to the internet.

- You can re-type your message and long-press the send button to send it as an insecure SMS/MMS. If your contact uninstalled Signal, ask them to unregister their number.

- Make sure your contact has all the permissions enabled for iOS and has gone through this checklist for Android.

Optical signal processing

6. Answer to the questions:

- How do you know my message was delivered or read ?

- What can you say about one check marks and two check marks?

- What can you do if your Signal message was not delivered?

7. Read the text and make your own vocabulary.

Telecommunications Links

Telecommunications links may be implemented with various communication media, with a corresponding variety of characteristics. The main feature of a medium is its potential transmission speed, also known as channel capacity, which for data transmission purposes is expressed in bits per second (bps). An alternative measure of transmission channel capacity is bandwidth - the range of signal frequencies that can be transmitted over the channel.

Six potential media are employed to implement telecommunication links:

1. Twisted pair

2. Coaxial cable

3. Fiber Optics cable

4. Terrestrial Microwave

5. Satellite Transmission

6. Radio Transmission

Three of the above transmission media are classified as guided media - in which the signal moves along an enclosed path. Guided media require wiring. They include:

1. Twisted pair

2. Coaxial cable

3. Fiber optic cable

Three of the above transmission media are classified as wireless media - the signal is broadcast (radiated in many directions) over the air or space and received through an antenna. They include:

1. Terrestrial Microwave

2. Satellite Transmission

3. Radio Transmission

Characteristics of Communications Media:

Twisted Pair a communications medium consisting of a pair of wires.

Coaxial Cable a communications medium that consists of a relatively thick central conductor shielded by several layers of insulation and the second conductor just under the cable's shell

Belden 1617A RG11 Coax cable

Fiber Optics high-capacity communications medium that consists of many strands of pure glass with a data carrying core in the middle, surrounded by a reflective coating and a protective sheath.

Terrestrial Microwave long-distance telecommunications by means of microwave signals travelling on the surface of the earth.

Satellite Transmission form of microwave transmission in which the signal is transmitted by an earth station to a satellite which rebroadcasts the signal to the receiving station.

Radio Transmission wireless communications technology that transmits voice or data over the air using a lower frequency band than microwaves.

Note: Transmission speeds keep on rising, particularly in the fiber optics area. We are now moving toward a global infrastructure of gigabit-speed fiber optic links relying on digital transmission. In this multimedia environment, data, text, voice, images, and video will travel at speeds of billions of bits per second.

The overall 5G radio wireless-access solution consisting of LTE and new

8. Make up questions according to the text.

How to Reduce the Costs of Telecommunications Multiplexing and Signal Compression

With the geographical distribution of information systems, increased volumes of transmission, and the move to multimedia, the costs of telecommunications are a significant business concern. Two principal methods of reducing these costs are:

1. Multiplexing - the sharing of a high-capacity link by a number of transmissions

2. Signal Compression - using the link more efficiently by removing redundancies from the signal.

Multiplexing

Characteristics of multiplexing:

1. There are economies of scale in telecommunications systems: the higher the system capacity, the lower the unit cost of transmissions.

2. Many individual transmissions can share a physical channel through a variety of techniques collectively called multiplexing.

3. Multiplexing combines several lower-capacity transmissions into a single transmission, which is split at the receiving end.

Signal Compression

Characteristics of signal compression:

1. Signal compression is the reduction of the need for channel capacity by removing redundancies from the signal.

2. To reduce the transmission needs, we can remove the redundancies at the sender site, transmit the compressed signal, and then restore the signal at the receiving end.

3. Compression has an impressive effect on multimedia transmission needs.

UNIT 5 COMMUNICATON NETWORKS

Lessons 21-22-23-24

Communication networks and switching systems. Basic notions and definitions, types of topologies. Methods of data distribution in telecommunication networks

1. Read and speak on the given theme.

Telecommunications network, electronic system of links and switches, and the controls that govern their operation, that allows for data transfer and exchange among multiple users.

When several users of telecommunications media wish to communicate with one another, they must be organized into some form of network. In theory, each user can be given a direct point-to-point link to all the other users in what is known as a fully connected topology (similar to the connections employed in the earliest days of telephony), but in practice this technique is impractical and expensive—especially for a large and dispersed network. Furthermore, the method is inefficient, since most of the links will be idle at any given time. Modern telecommunications networks avoid these issues by establishing a linked network of switches, or nodes, such that each user is connected to one of the nodes. Each link in such a network is called a communications channel. Wire, fibre-optic cable, and radio waves may be used for different communications channels

2. Read and speak on the given theme

Components of a Telecommunications Network

Telecommunications are the means of electronic transmission of information over distances. The information may be in the form of voice telephone calls, data, text, images, or video. Today, telecommunications are used to organize more or less remote computer systems into telecommunications networks. These networks themselves are run by computers.

A telecommunications network is an arrangement of computing and telecommunications resources for communication of information between distant locations.

A telecommunications network includes the following components:

1. Terminals for accessing the network

2. Computers that process information and are interconnected by the network

3. Telecommunications links that form a channel through which information is transmitted from a sending device to a receiving device.

4. Telecommunications equipment that facilitates the transmission of information.

5. Telecommunications software that controls message transmission over the network.

3. Read and speak on the given theme

Scope of Telecommunications Networks

Two principal types of telecommunications networks can be distinguished from the point of view of their geographical scope. They are:

1. Local area networks

2. Wide area networks

Local area network (LAN): is a privately owned network that interconnects processors, usually microcomputers, within a building or on a campus site that includes several buildings.

Characteristics of a LAN:

a. LANs are the principal tool of workgroup computing

b. LANs ensure high-speed communication within a limited area and enables the users to share facilities (peripherals) connected to it.

c. Usually include a large-capacity, secondary storage device, where database and applications software are maintained, managed by a microcomputer acting as a file server that delivers data or program files to other computers.

d. Facilities (peripherals) may include jukebox optical memory and fast printers

e. Frequently, one of the facilities (peripherals) in a LAN is the gateway hardware and software that give the network users access to other networks.

f. More group members may connect to the network from remote sites using wireless telecommunications.

g. Links and equipment of LANs are owned by the user company, and these networks are generally much faster than WANs.

h. LANs are generally composed of a network of microcomputers

4. Read and speak on the given theme

Wide area network (WAN): is a telecommunications network that covers a large geographical area.

Characteristics of a WAN:

a. The information system of an entire organization may be structured as a hierarchy. The WANs system architecture looks very much like an organization chart.

b. WANs connect all the divisional minicomputers to the headquarters mainframe with a variety of local microcomputers and terminals located at remote sites connected, in turn, to the minicomputers.

c. WANs provide the backbone through which all other nodes (computers and terminals) communicate.

d. WANs often use telecommunication links and equipment provided by specialized vendors, called common carriers.

e. WANs serve to interconnect multiple LANs and can make specific resources available to a large number of workstations.

Metropolitan Area Networks (MAN) - are telecommunications networks that interconnect various local area networks within a metropolitan area, that is, within approximately a 50-mile range.

Characteristics of a WAN:

a. Purpose of MANs is to interconnect various LANs within a metropolitan area, that is, within approximately a 50 - mile range.

b. Generally, the speed of MANs is equal to that of LANs and they use similar technology.

Interorganizational Information Systems - are shared by two or more companies.

Characteristics of Interorganizational Information Systems:

a. These systems help several firms share information in order to coordinate their work, collaborate on common projects, or sell and buy products and services.

b. Internet has emerged as a global public network of networks

c. Some interorganizational systems are employed in knowledge work

d. Can be used to connect a firm's computers to the information systems of its customers, suppliers, and business partners, and are also used to execute business transactions.

5. Read and speak on the given theme

Analog and Digital Communications

Most of the lines in the telephone systems of the world at present are analog. Signals are transmitted as continuous waves. This is a satisfactory way to transmit voice, but digital data sent by computers (sequences of pulses representing 0s and 1s) must be converted into an analog signal for transmission over an analog line. The analog data must then be converted back into digital before entering the memory of the receiving computer. The conversion of data from digital form into analog for transmission and then back into digital at the receiving end is done by a pair of interface devices called modems (modulator-demodulator).

Modem-based telecommunications have created a significant bottleneck in an environment where computer and peripheral speeds have increased dramatically. The solution is end-to-end digital communications, in which signals are sent as streams of on/off pulses. Digital lines are capable of much faster communication and digital circuitry is now cheaper than analog. All the new equipment now installed in telephone networks is indeed digital.

Trend: There is a shift toward digital telecommunications is taking place throughout the world. A digital system for telecommunications, called TI carrier, is in wide use in parts of the telephone network.

Future: Integrated Services Digital Network (ISDN) - a completely digital telecommunications network standardized by an international committee. Although ISDN services are available in some areas, including most of the US, worldwide ISDN is not expected to become operational until after the year 2000.

Computer Networks

Computer networks differ in scope from relatively slow wide area networks, employed to transmit messages across vast geographic distances, to very fast local area networks that may connect computers located in the same building. System designers may select one of several arrangements for interconnecting network nodes, depending on an organization's requirement. There are several ways to establish a connection between the sender and the receiver of a message.

Network Topologies

Computers, switches, and terminals interconnected by network links are collectively called nodes. The purpose of network control is to provide a connection between nodes that need to communicate. The arrangement of nodes and links in a network is called a topology. A variety of arrangements are possible, each with its own advantages and drawbacks. Network topology has to fit the structure of the organizational unit that will use the network, and this topology should also be adapted to the unit's communication traffic patterns and to the way the databases will be stored in order to facilitate access to them.

The following topologies are the most widely used:

1. Hierarchical Network

2. Star Network

3. Ring Network

4. Bus Network

Hierarchical Network:

a. A corporate host computer (often a mainframe), divisional minicomputers or powerful workstations, and workgroup support via micros.

b. This topology matches the organizational structure of many firms and is still frequently used in WANs.

c. The user workstations may be, in turn, interconnected using one of the LAN topologies.

d. Failure of the host does not disable divisional processing, which is a fail-safe feature.

e. Cost-effectiveness of micros and the growing importance of groupwork leads some downsizing firms to move away from hierarchical networks to client/server computing.

Star Network:

a. In a star network, a hub computer or switch (such as a PBX) interconnects a number of workstations.

b. The computer at the hub acts as the network server, providing access to the shared database and software. All communications between the workstations must go through this central mode.

c. The star network is rather easy to manage and expand, since in both cases it is largely the single central node that is affected in an expansion of a processing capacity.

d. The central node is a locus of vulnerability: it may be overloaded or it may fail, disabling the entire network.

Ring Network:

a. Each node in a ring network is connected to two of its neighbours.

b. The nodes are usually close to one another; this topology is frequently used in LANs.

c. When one node sends a message to another, the message passes through each intermediate node, which restores the signal, as signals deteriorate in transmission.

d. If a node fails, the ring is out of service, unless the ring contains two channels transmitting in opposite directions.

Bus Network:

a. The nodes on a bus network are connected to a common link such as coaxial cable. This arrangement is used in LANs.

b. A failing device does not affect the rest of the network; failure of the bus itself, of course, brings the network down.

6. Read and speak on the given theme

Switching in Networks

Many users can be connected at the same time to a network of communication channels. Switching devices establish connections between nodes that need to communicate over a network. Principal techniques for switching include:

1. Circuit Switching

2. Packet Switching

3. Fast Packet Switching

Circuit Switching:

1. The circuit switching technique is employed in a telephone network.

2. Communication links are connected to switching centers, which connect to one node to another on demand.

3. The circuit is established for the entire duration of the communication

4. Circuit switching is suitable for file transfers and similar longer transmissions

Packet Switching:

1. Packet switching is of particular importance for data communication owing to its speed and its superior utilization of communication links when handling Abursty,@ intermittent, traffic. Indeed, data transmission involves short bursts of activity by a computer or a terminal when the data are sent, followed by long periods when there is no transmission.

2. Packet switching offers flexibility in connecting to a network. It is used by most of the public data networks provided by value-added carriers.

3. In packet switching, messages are divided at the source into fixed-length chunks, called packets, that also include bits identifying the receiver. Typically, a packet contains 128 bytes of data.

4. Each packet, can be transmitted independently, with routing determined at each node the packet passes through (as opposed to circuit switching, where the route is predetermined).

Fast Packet Switching:

Traditional packet switching checks each packet for errors at every node the packet passes through. Modern telecommunications equipment is far more noise-free than that for which packet switching was originally designed. To take advantage of this, two fast packet-switching technologies are being introduced:

Frame Relay: Fast packet switching that checks a packet for errors only at the entry and exit nodes of the telecommunications network, thus reducing transmission delay.

Cell Relay: (asynchronous transfer mode, or ATM) transfers very short fixed-length packets, called cells, over fast LANs or WANs.

7. Read and speak on the given theme

Local Area Networks

Organizations small and large use fast local area networks (LANs) to interconnect personal computers, and thus make a basic workgroup tool.

Local Area Network: Workplace for a Workgroup

A LAN interconnects computers within a single site, such as an office building, a manufacturing plan, or a corporate or university campus.

Characteristics of a LAN include:

1. Its scope is commonly measured in feet

2. Communication speeds are very high

3. Used as a local means of computing and communication among users in larger firms

4. Are owned by the organization

5. Afford a sense of control and the flexibility to meet the demands of the end users

A LAN gives its users the following capabilities:

1. Users can share resources, such as a fast printer or a database

2. Users can collaborate by communicating over their LAN. This collaboration may be facilitated by groupware that runs on a LAN

3. Users can access other networks within a firm or outside of it via bridges and routers

There are two principal LAN designs:

1. Peer-to-peer - peripherals are located at terminals and system administration is largely left up to the users

2. Server-based networks - shared resources are placed a dedicated server that manage a given resource on behalf of user workstations sharing the resource (file server, printer server, gateway, optical disk server). Most of the servers are dedicated to their task; using them as workstations degrades the performance of the net.

Local Networks Based on Private Branch Exchanges:

A company with a large number of telephones (from 50 to over 10,000) often elects to own a computer-based private branch exchange (PBX), an electronic switchboard that interconnects its telephones and provides connections to the public network.

Characteristics of a PBX:

1. Gives a company control over the usage of its telephone system and offers a variety of features, such as call forwarding or voice messaging.

2. Maybe employed as a switch for data communications

3. Many newer PBXs use digital technology, eliminating the need for modems, and perform conversions needed to ensure connectivity between various equipment and telecommunications links.

4. Easy to connect a new workstation to the net.

5. Speeds of PBX-based networks are limited

8. Read and speak on the given theme

Wide Area Networks

Wide area networks are the fundamental infrastructure of organizational computing. These long-distance telecommunications networks employ a variety of equipment so that the expensive links may be used effectively. The offerings of common carriers and of providers of value-added services may be combined with private networks to create an overall organizational network.

Telecommunications Equipment for Wide Area Networks

WANs include equipment that controls message transfer and makes it possible to share the links among a number of transfers.

Host Computer

A WAN has a powerful host computer. The host runs a system program, called a telecommunications monitor, which processes incoming messages, passing them to the appropriate application programs, and accepts outgoing messages from the applications in order to transmit them into the network.

Front-End Processor

Relieves the host computer of most of the tasks involved in network control. Under the control of its own software, the front-end processor accepts messages coming from the network and routes outgoing messages to their destinations. It performs the necessary code conversions, encrypts and decrypts secure messages, and performs error checking so that the host deals with Aclean@ messages.

Cluster Controller

Manages several terminals, connecting them to a single telecommunications link, and performs communication tasks for them, such as screen formatting, code conversion, and error checking. A cluster controller may also allow the terminals to share a high-speed printer and may handle electronic mail among the cluster terminals.

Multiplexor

Combines the data that terminals send to it over local low-speed links into a single stream. This stream is then transmitted over a high-speed telecommunications channel and is split by another multiplexor on the opposite end of the channel.

Concentrator

Combines transmission from several slower terminals that operate in a burst mode into a single transmission stream that requires a link of lower speed than the sum of the speeds of all the terminals combined. A concentrator stores messages from terminals and forwards them when warranted.

Switches

Establishes connections between nodes that need to communicate.

Access Terminals

Include a variety of dumb terminals, with no processing capacity and intelligent terminals with processing capacity, such as personal computers.

Where do Facilities for Wide Area Networks Come From?

Some network facilities are owned by user organizations, others can be leased by them, or simply used on a pay-as-you-go basis. Among the typical facilities owned by user firms are workstations, host computers, and front-end processors. The essential providers of telecommunications links and services are common carriers and the vendors of enhanced services on value-added networks. These include:

1. Common carriers

2. Providers of value-added networks

3. Private lines and private networks

Common Carriers

Are companies licensed by a country's government to provide telecommunications services to the public. The vast majority of common carriers provide telephone service. These carriers offer the use of a wide-area telecommunications infrastructure, that is, facilities for the transmission of voice and data messages.

Common carriers offer a service called virtual private network where a user firm can purchase guaranteed access to facilities with specified capabilities, such as transmission speed and access points.

Providers of Value-Added Networks

Value-added vendors lease facilities from the common carriers and provide telecommunications services to their own customers. These vendors add value to the basic infrastructure furnished by the common carrier. The value-added networks (VAN) provided by the vendors furnish services over and above those provided by common carriers.

Private Lines and Private Networks

Instead of using a service that has to be shared with others, a firm may lease its own private lines or entire networks from carriers. This can have economic advantages as compared with VAN use, as well as provide faster and more secure communications

UNIT 6 СELLULAR SYSTEMS AND STANDARTS

Lessons 25-26-27-28-29

Satellite radio systems. GPS and GLANASS navigation systems. GSM and SDMA cellular standards development

1. Read and speak on the given theme.

GNSS COMPONENTS

The GNSS consist of three main satellite technologies: GPS, Glonass and Galileo. Each of them consists mainly of three segments: (a) space segment, (b) control segment and (c) user segment. These segments are almost similar in the three satellite technologies, which are all together make up the GNSS. As of today, the complete satellite technology is the GPS technology and most of the existing worldwide applications related to the GPS technology. The GNSS technology will become clearer after the operation of Galileo and the reconstruction of Glonass in the next few years.

Global Positioning System: The United States Department of Defense (DoD) has developed the Navstar GPS, which is an all-weather, space based navigation system to meet the needs of the USA military forces and accurately determine their position, velocity, and time in a common reference system, any where on or near the Earth on a continuous basis. GPS has made a considerable impact on almost all positioning, navigation, timing and monitoring applications. It provides particularly coded satellite signals that can be processed in a GPS receiver, allowing the receiver to estimate position, velocity and. There are four GPS satellite signals that are used to compute positions in three dimensions and the time offset in the receiver clock. GPS comprises three main components:

- Space segment: The Space Segment of the system consists of the GPS satellites; see Picture 1. These space vehicles (SVs) send radio signals from space as shown in Picture 2.

- Control segment: The Control Segment consists of a system of tracking stations located around the world. The Master Control facility is located at Schriever Air Force Base (formerly Falcon AFB) in the State of Colorado, USA.

- User segment: The GPS User Segment consists of the GPS receivers and the user community. GPS receivers convert space vehicle (SV) signals into position, velocity, and time estimates.

Picture 1

Picture 2

The satellites are dispersed in six orbital planes on almost circular orbits with an altitude of about 20,200 km above the surface of the Earth, inclined by 55 degree with respect to the equator and with orbital periods of approximately 11 hours 58 minutes (half a sidereal day). The categories are Block I, Block II, Block IIR (R for replenishment) and Block IIA (A for advanced) and a further follow-on category Block IIF has also been planned. Picture 3 shows the main GPS segments.

Picture 3. GPS segments (Aerospace Corporation, 2003).

2. Work with the text

GPS Signals

The generated signals on board the satellites are based or derived from generation of a fundamental frequency ƒo=10.23 MHZ (Hofmann-Wellenhof et al., 2001). The signal is controlled by atomic clock and has stability in the range of 13 10− over one day. Two carrier signals in the L-band, denoted L1 and L2, are generated by integer multiplications of ƒo. The carriers L1 and L2 are biphase modulated by codes to provide satellite clock readings to the receiver and transmit information such as the orbital parameters. The codes consist of a sequence with the states +1 or -1, corresponding to the binary values 0 or 1. The biphase modulation is performed by a 180° shift in the carrier phase whenever a change in the code state occurs; see Figure 4. The clear/access code (C/A-code) and precision code (P-code) are used for the satellite clock reading, both are characterized by a pseudorandom noise (PRN) sequence. The W-code is employed to encrypt the P-code to the Y-code when Anti Spoofing (A-S) is applied. The navigation message is modulated using the two carriers (L1 and L2) at a chipping rate of 50 bps

Figure 4. Biphase modulation of carrier

It contains information on the satellite orbits, orbit perturbations, GPS time, satellite clock, ionospheric parameters, and system status messages (Leick, 2003). The modulation of L1 by Pcode, C/A-code and navigation message (D), is done using the quadrature phase shift keying (QPSK) scheme. The C/A-code is placed on the LI carrier with 90° offset from the P-code since they have the same bit transition epochs.

The signal broadcast by the satellite is a spread spectrum signal, which makes it less prone to jamming. The basic concept of spread spectrum technique is that the information waveform with small bandwidth is converted by modulating it with a large-bandwidth waveform (HofmannWellenhof et al., 2001). The generation of pseudo random sequence (PRN) in the code is based on the use of an electronic hardware device called tapped feed back shift register (FBSR). This device can generate a large variety of pseudo random codes, but in this way the generated code repeat it self after some very long time. The receiver could distinguish the signals coming from different satellites because the receiving C/A code (the Gold code), has low cross-correlation and is unique for each satellite (Leick, 2003). The navigation message consists of 25 frames with each frame containing 1500 bit and each frame is subdivided into 5 sub-frames with 300 bit. The information transmitted by the navigation message is periodically updated by the control segment.

3. Work with the text

GLONASS

The GLONASS (GLObal NAvigation Satellite System or “GLObalnaya NAvigatsionnaya Sputnikovaya Sistema”) is nearly identical to GPS. Glonass satellite-based radio-navigation system provides the positioning and timing information to users. It is operated by the Ministry of Defense of the Russian Federation. Glonass space segment is consist of 24 satellites, equally distributed in 3 orbit separated by o 120 in the equatorial plane. Satellite orbital altitude is about 19,130 km above the ground surface. This results in an orbital period of 11:15:44 corresponding to 8/17 of a sidereal day. The future of GLONASS seems uncertain due to economic problems facing the Russian Federation. The number of operational satellites was steadily decreasing over the past few years. The launch of three new GLONASS satellites in December 1998 was the first launch after a lapse of 3 years. As of January 2006, a total of 10 GLONASS satellites are operational. The oldest of the still active satellites was launched in October, 2000. According to Russian officials the GLONASS system shall again be restored by 2008.

4. Work with the text

The Signals of the GLONASS Satellites

Glonass transmit C/A-code on L1, P-code on L1 and L2. Glonass observables (code and phase) are similar to GPS. The main difference between GPS and GLONASS is that GLONASS uses Frequency Division Multiple Access (FDMA) technology to discriminate the signals of different satellites, but GPS and Galileo use (Code Division Multiple Access, CDMA) to distinguish between the satellites. All Glonass satellites transmit the same C/A- and P-codes, but each satellite has slightly different carrier frequencies. The navigation message is contained in so-called sub frames, which have duration of 2.5 minutes. Each sub frame consists of five frames with a duration of 30 seconds. The navigation message contains information, similar to GPS navigation message, about the satellite orbits, their clocks, among others. On the contrary to GPS, where the broadcast ephemeredes are defined by modified Keplerian elements, the broadcast ephemeredes of GLONASS satellites are defined by positions and velocities referred to an Earth-centered and Earth-fixed systems (PZ-90). The broadcast ephemeredes of the Glonass satellites are updated every 30 minutes.

5. Work with the text

GALILEO

GALILEO is Europe’s initiative for a state-of-the-art global navigation satellite system, providing a highly accurate, guaranteed global positioning service under civilian control. Galileo will be not too different from the other GNSS parts (modernized GPs and Glonass. It will provide autonomous navigation and positioning services, but at the same time will be interoperable with the two other global satellite navigation systems; the GPS and GLONASS. A user will be able to take a position with the same receiver from any of the satellites in any combination. By providing dual frequencies as standard, however, GALILEO will deliver real-time positioning accuracy down to the meter range. It will guarantee availability of the service under all, but the most extreme circumstances and will inform users within seconds of a failure of any satellite. This will make it appropriate for applications where safety is vital, such as running trains, guiding cars and landing aircraft. The combined use of GALILEO and other GNSS systems can offer much improved performance for all kinds of users worldwide. GALILEO is expected to be in operation by the year 2008. The first satellite of Galileo system (GIOVE A) has already been lunched in 27th December 2005. 2.4.1 Galileo segments Galileo segments are almost similar to GPS, but with some modification. The main extension of Galileo compared to GPS is the implementation of a global/ regional segment for integrity monitoring. The objective is to assist the safety critical aircraft navigation and locate and guide railway trains.

Space Segment

The space segment or the constellation features consists of 30 Medium Earth Orbiting (MEO) satellites (27 and 3 active spare satellite), distributed evenly and regularly over three orbit planes. The projected altitude is slightly larger than for GPS 23,616 km and the inclination is 56°.

Ground Segment

The Galileo ground segment is responsible for managing the constellation of navigation satellites, controlling core functions of the navigation mission such as orbit determination of satellites, and clock synchronization, and determining and disseminating (via the MEO satellites) the integrity information, such as the warning alerts within time-to-alarm requirements, at global level. The Global ground segment will also provide interfaces with service centers. The Ground Control Segment will consist of about 12-15 reference stations, 5 up-link stations and two control centers. The ground segment also will include 16-20 monitor stations, three up-link stations for integrity data and two central stations for integrity computations.

User Segment

The user segment consists of different types of user receivers, with different capabilities related to the different GALILEO signals in order to fulfill the various GALILEO services.

6. Work with the text

SIGNAL PROCESSING AND RECEIVER DESIGN

The main function of the signal processor in the receiver is the reconstruction of the carriers and extraction of codes and navigation messages. After this stage the receiver performs the Doppler shift measurement by comparing the received signal by a reference signal generated by the receiver. Due to the motion of satellite, the received signal is Doppler shifted. The code ranges are determined in the delay lock loop (DLL) by using code correlation. The correlation technique provides all components of bimodulated signals. The correlation technique is performed between the generated reference signal and the received one. The signals are shifted with respect to time so that they are optimally matched based on mathematical correlation.

Currently some geodetic type receivers are available on the market tracking GPS and Glonass satellites simultaneously on both frequencies, in particular the Ashtech Z18 receiver and the TPS (Topcon Positioning Systems) Legacy receivers. The future GNSS receiver could be designed to track the different GNSS signals and could be of many types:

• The first type could process all GNSS signals GPS L1, L2, L5 and Galileo OS, CS using L1, E5 and E6 and also Glonass L1 and L2.

• The second type uses free signal and codes, GPS L1 and L2C and Galileo OS, on L1 and E5.

• The third type uses L1 and E5.

• Forth type uses GPS L1 and L2 (which are already in the market.

• Fifth type uses GPS and Glonass signals (which are already exist)

The most common receiver types are Intermediate Frequency receiver (IF) and the software defined radio receiver (SDR). In the RF front-end receiver the signal is down converted to an intermediate frequency and then sampled, but SDR uses direct digitization, or bandpass sampling. The main components of RF-FE combined GNSS receiver are shown in Figure 8. After sampling and analog to digital conversion (ADC) of the received signal, the receiver performs parallel despreading. The received base-band signal is multiplied in parallel with the spreading codes of all visible satellites. The received signal of each satellite is multiplied in parallel with different code delay offsets. These products are then accumulated to compute the cross-correlation function.

Because BOC signals are used in Galileo, supplementary measures are necessary due to the multiple correlation peaks of the auto-correlation function. Carrier tracking is performed using a phase-locked or frequency-locked loop (PLL or FLL). Coherent correlation combined with differential or non-coherent correlation can be done for the pilot and the data channel. Multiple signals will be available at L1 within the next few years (Hein et al., 2004). Galileo will use a different modulation scheme for its signals such as BOC and QPSK, while GPS uses binary phase shift keying (BPSK) modulation for the open signals at L1 and L2. The L5 signal that will appear with the Block IIF satellites in 2006, will have quadrature phase shift keying (QPSK). The binary offset carrier (BOC) modulation scheme of Galileo provides better multipath and receiver noise performance compared to the GPS binary phase shift keying (BPSK) modulation. More complex techniques are already developed for tracking BOC signal, such as bump jump and BPSK-like.

7. Work with the text

The GSM Standard

Introduction

Global System for Mobile Communications (GSM) is the most popular mobile phone system in the world, accounting for 70% of the world’s digital mobile phones. According to a press release by the GSM Association in May 2001, there are more than half a billion GSM mobile phones in use in over 168 countries today. The phenomenal success in mobile telecommunications is due in large to GSM. One of its key strength is its international roaming capability, giving consumers a seamless service in over 168 countries.

History of GSM

In 1982, the European Conference of Post and Telecommunications Administrations (CEPT) formed a group called Group Spéciale Mobile (GSM) to develop a pan-European cellular system that would replace the many existing incompatible cellular systems already in place in Europe. In 1987, a milestone was achieved with the signing of the GSM Memorandum of Understanding (MoU) by GSM-operators-to-be, agreeing to implement cellular networks, based on the GSM specifications. While it was clear from the start that GSM would be a digital system, it was officially announced in 1987. GSM service started in 1991. In the same year, GSM was renamed to Global System for Mobile Communications from Group Spéciale Mobile. Although GSM was initially developed as a European digital communication standard to allow users to use their cellular devices seamlessly across Europe, it soon developed into a standard that would see unprecedented growth globally. Here in North America, the GSM standard is often referred to as PCS 1900 and elsewhere as DCS 1800. The number relates to the operating frequency of the system.

Key features of GSM

1) International Roaming - single subscriber number worldwide

2) Superior speech quality - better than existing analog cellular technology

3) High level of security - user’s information is safe and secure

4) Universal and Inexpensive Mobile handsets

5) Digital Convenience - talk time is doubled per battery life and digital networks can handle higher volume of calls at any one time that analog networks

6) New services - such as call waiting, call forwarding, Short Message Service (SMS), GSM Packet Radio Service (GPRS)

7) Digital compatibility - easily interfaces with existing digital networks i.e. Integrated Services Digital Network (ISDN)

8. Work with the text

GSM Architecture

Figure 1: GSM Architecture1

Mobile Station Every GSM mobile phone has a Subscriber Identity Module (SIM). The SIM provides the mobile phone with a unique identity through the use of the International Mobile Subscriber Identity (IMSI). The SIM is like a key, without which the mobile phone can’t function. It is capable of storing personal phone numbers and short messages. It also stores security related information such as the A3 authentication algorithm, the A8 ciphering key generating algorithm, the authentication key (KI) and IMSI. The mobile station stores the A5 ciphering algorithm. The SIM is removable, which allows users to travel abroad taking with them only their SIM card. They would need to inform their local provider, which countries they would be visiting, prior to their departure. At their destination, they can simply plug the SIM into a rental cellular phone and make use of the mobile unit.

The SIM can be protected with a Personal Identification Number (PIN) chosen by the subscriber. The PIN is stored on the card and if entered incorrectly thrice, the card blocks itself. At this point, you’ll have to contact your cellular provider who can unblocked your mobile phone, by entering an eight digit Personal Unblocking Key (PUK), which is also stored on the card.

Base Station Subsystem (BSS)

The role of the Base Station Subsystem (BSS) is to connect the user on a mobile phone with other landline or mobile users. The Base Transceiver Station (BTS) is in direct contact with the mobile phones via the air interface and can be thought of as a complex radio modem. The Base Station Controller (BSC) is responsible for the control of the several BTS. It monitors each call and decides when to handover the call from one BTS to another, as well as manage radio frequencies allocated for the calls through the BTS. Network Subsystem (NSS) It is a complete exchange, capable of routing calls from a fixed network via the BSC and BTS to an individual mobile station. The Mobile Services Switching Center (MSC) interconnects the cellular network with the Public Switched Telephone Network (PSTN). The MSC also serves to co-ordinate setting up calls to and from GSM users.

The Home Location Register (HLR) stores information of all subscribers belonging to an area served by a MSC. It stores permanent data such as the IMSI, services subscribed by the user, subscriber’s number from a public network, KI and some other temporary data. The HLR has to provide the MSC with all the necessary information when the call is coming from a public network.

The Visitor Location Register (VLR) contains relevant information for all mobiles currently served by a MSC. The permanent data stored in the VLR is also stored in the HLR. In addition, it also stores the Temporary Mobile Subscriber Identity (TMSI), which is used for limited intervals to prevent the transmission of the IMSI via the air interface. (See section on GSM Security: Anonymity) The VLR has to support the MSC during call establishment and authentication when the call originates from a mobile station.

The Equipment Identity Register (EIR) stores all the International Mobile Equipment Identities (IMEI) of mobile equipment and their rights on the network. The EIR maintains a white, gray and black list. Those on the white list are permitted on the network while those on the black list are blocked from the network. The gray list consists of faulty equipment that may pose a problem on the network but are still permitted to participate on the network. The IMEI reveals the serial number of the mobile station, manufacturer, type approval and country of production.

The Authentication Center (AuC) is a protective database that houses the KI, the A3 authentication algorithm, the A5 ciphering algorithm and the A8 ciphering key generating algorithm. It is responsible for creating the sets of random numbers (RAND), Signed Response (SRES) and the Cipher key (KC), though the created sets are stored in the HLR and VLR.

9. Work with the text

GSM Security

As all cellular communications are sent over the air interface, it is less secure than a wired network, as it opens the door to eavesdroppers with appropriate receivers. Several security functions were built into GSM to safeguard subscriber privacy. These include: ·

- Authentication of the registered subscribers only ·

- Secure data transfer through the use of encryption ·

- Subscriber identity protection

- Mobile phones are inoperable without a SIM ·

- Duplicate SIMs are not allowed on the network ·

- Securely stored KI

10. Work with the text

GSM Algorithms

A consequence of international roaming is the exchange of information between providers in different countries. All countries have strict regulations against the export of encryption algorithms and thus GSM works around it. When a user tries to use his phone in say another country, the local networks request the HLR of the subscriber’s home network for the RAND, SRES and KC which is sufficient for authentication and encrypting data. Thus the local network does not need to know anything about the A3 or A8 algorithms stored in the SIM.

Authentication Algorithm A3 – It is operator-dependent and is an operator option. The A3 algorithm is a one-way function. That means it is easy to compute the output parameter SRES by using the A3 algorithm but very complex to retrieve the input parameters (RAND and KI) from the output parameter. Remember the key to GSM’s security is keeping KI unknown. While it may sound odd that each operator may choose to use A3 independently, it was necessary to cover the case of international roaming.

Ciphering Algorithm A5 – Currently, there exists several implementations of this algorithm though the most commonly used ones are A5/0, A5/1 and A5/2. The reason for the different implementations is due to export restrictions of encryption technologies. A5/1 is the strongest version and is used widely in Western Europe and America, while the A5/2 is commonly used in Asia. Countries under UN Sanctions and certain third world countries use the A5/0, which comes with no encryption.

Lesson 30

MIDTERM CONTROL 2

задания рубежного контроля 2

Для выполнения рубежного контроля 2 студент должен представить

презентацию:

-используя Интернет-ресурсы по выбранной  теме, собрать необходимую информацию;

-распечатать найденную информацию;

-провести анализ и текстовую обработку отобранного материала:

-читая текст, выделять наиболее значимую информацию цветным маркером

(to highlight the text);

-представить эту информацию в виде развернутого плана;

-на основе плана написать сообщение на английском языке по теме, объемом не менее 2-3-х страниц;

-составить глоссарий или терминологический словарь по тематике специальности с толкованием терминов (definitions) –не менее 20 терминов

BRIEF REVIEW INFORMATION

Telecommunications, the Internet, and Information System Architecture

Telecommunications Networks and their Scope

The electronic transmission of information over distances, called telecommunications, has become virtually inseparable from computers: Computers and telecommunications create value together.

Components of a Telecommunications Network

Telecommunications are the means of electronic transmission of information over distances. The information may be in the form of voice telephone calls, data, text, images, or video. Today, telecommunications are used to organize more or less remote computer systems into telecommunications networks. These networks themselves are run by computers.

A telecommunications network is an arrangement of computing and telecommunications resources for communication of information between distant locations.

A telecommunications network includes the following components:

1. Terminals for accessing the network

2. Computers that process information and are interconnected by the network

3. Telecommunications links that form a channel through which information is transmitted from a sending device to a receiving device.

4. Telecommunications equipment that facilitates the transmission of information.

5. Telecommunications software that controls message transmission over the network.

Scope of Telecommunications Networks

Two principal types of telecommunications networks can be distinguished from the point of view of their geographical scope. They are:

1. Local area networks

2. Wide area networks

Local area network (LAN): is a privately owned network that interconnects processors, usually microcomputers, within a building or on a campus site that includes several buildings.

Characteristics of a LAN:

a. LANs are the principal tool of workgroup computing

b. LANs ensure high-speed communication within a limited area and enables the users to share facilities (peripherals) connected to it.

c. Usually include a large-capacity, secondary storage device, where database and applications software are maintained, managed by a microcomputer acting as a file server that delivers data or program files to other computers.

d. Facilities (peripherals) may include jukebox optical memory and fast printers

e. Frequently, one of the facilities (peripherals) in a LAN is the gateway hardware and software that give the network users access to other networks.

f. More group members may connect to the network from remote sites using wireless telecommunications.

g. Links and equipment of LANs are owned by the user company, and these networks are generally much faster than WANs.

h. LANs are generally composed of a network of microcomputers

Wide area network (WAN): is a telecommunications network that covers a large geographical area.

Characteristics of a WAN:

a. The information system of an entire organization may be structured as a hierarchy. The WANs system architecture looks very much like an organization chart.

b. WANs connect all the divisional minicomputers to the headquarters mainframe with a variety of local microcomputers and terminals located at remote sites connected, in turn, to the minicomputers.

c. WANs provide the backbone through which all other nodes (computers and terminals) communicate.

d. WANs often use telecommunication links and equipment provided by specialized vendors, called common carriers.

e. WANs serve to interconnect multiple LANs and can make specific resources available to a large number of workstations.

Metropolitan Area Networks (MAN) - are telecommunications networks that interconnect various local area networks within a metropolitan area, that is, within approximately a 50-mile range.

Characteristics of a WAN:

a. Purpose of MANs is to interconnect various LANs within a metropolitan area, that is, within approximately a 50 - mile range.

b. Generally, the speed of MANs is equal to that of LANs and they use similar technology.

Interorganizational Information Systems - are shared by two or more companies.

Characteristics of Interorganizational Information Systems:

a. These systems help several firms share information in order to coordinate their work, collaborate on common projects, or sell and buy products and services.

b. Internet has emerged as a global public network of networks

c. Some interorganizational systems are employed in knowledge work

d. Can be used to connect a firm's computers to the information systems of its customers, suppliers, and business partners, and are also used to execute business transactions.

Telecommunications Links

Telecommunications links may be implemented with various communication media, with a corresponding variety of characteristics. The main feature of a medium is its potential transmission speed, also known as channel capacity, which for data transmission purposes is expressed in bits per second (bps). An alternative measure of transmission channel capacity is bandwidth - the range of signal frequencies that can be transmitted over the channel.

Six potential media are employed to implement telecommunication links:

1. Twisted pair

2. Coaxial cable

3. Fiber Optics cable

4. Terrestrial Microwave

5. Satellite Transmission

6. Radio Transmission

Three of the above transmission media are classified as guided media - in which the signal moves along an enclosed path. Guided media require wiring. They include:

1. Twisted pair

2. Coaxial cable

3. Fiber optic cable

Three of the above transmission media are classified as wireless media - the signal is broadcast (radiated in many directions) over the air or space and received through an antenna. They include:

1. Terrestrial Microwave

2. Satellite Transmission

3. Radio Transmission

Characteristics of Communications Media:

Twisted Pair a communications medium consisting of a pair of wires.

Coaxial Cable a communications medium that consists of a relatively thick central conductor shielded by several layers of insulation and the second conductor just under the cable's shell

Fiber Optics high-capacity communications medium that consists of many strands of pure glass with a data carrying core in the middle, surrounded by a reflective coating and a protective sheath.

Terrestrial Microwave long-distance telecommunications by means of microwave signals travelling on the surface of the earth.

Satellite Transmission form of microwave transmission in which the signal is transmitted by an earth station to a satellite which rebroadcasts the signal to the receiving station.

Radio Transmission wireless communications technology that transmits voice or data over the air using a lower frequency band than microwaves.

Note: Transmission speeds keep on rising, particularly in the fiber optics area. We are now moving toward a global infrastructure of gigabit-speed fiber optic links relying on digital transmission. In this multimedia environment, data, text, voice, images, and video will travel at speeds of billions of bits per second.

Analog and Digital Communications

Most of the lines in the telephone systems of the world at present are analog. Signals are transmitted as continuous waves. This is a satisfactory way to transmit voice, but digital data sent by computers (sequences of pulses representing 0s and 1s) must be converted into an analog signal for transmission over an analog line. The analog data must then be converted back into digital before entering the memory of the receiving computer. The conversion of data from digital form into analog for transmission and then back into digital at the receiving end is done by a pair of interface devices called modems (modulator-demodulator).

Modem-based telecommunications have created a significant bottleneck in an environment where computer and peripheral speeds have increased dramatically. The solution is end-to-end digital communications, in which signals are sent as streams of on/off pulses. Digital lines are capable of much faster communication and digital circuitry is now cheaper than analog. All the new equipment now installed in telephone networks is indeed digital.

Trend: There is a shift toward digital telecommunications is taking place throughout the world. A digital system for telecommunications, called TI carrier, is in wide use in parts of the telephone network.

Future: Integrated Services Digital Network (ISDN) - a completely digital telecommunications network standardized by an international committee. Although ISDN services are available in some areas, including most of the US, worldwide ISDN is not expected to become operational until after the year 2000.

How to Reduce the Costs of Telecommunications Multiplexing and Signal Compression

With the geographical distribution of information systems, increased volumes of transmission, and the move to multimedia, the costs of telecommunications are a significant business concern. Two principal methods of reducing these costs are:

1. Multiplexing - the sharing of a high-capacity link by a number of transmissions

2. Signal Compression - using the link more efficiently by removing redundancies from the signal.

Multiplexing

Characteristics of multiplexing:

1. There are economies of scale in telecommunications systems: the higher the system capacity, the lower the unit cost of transmissions.

2. Many individual transmissions can share a physical channel through a variety of techniques collectively called multiplexing.

3. Multiplexing combines several lower-capacity transmissions into a single transmission, which is split at the receiving end.

Signal Compression

Characteristics of signal compression:

1. Signal compression is the reduction of the need for channel capacity by removing redundancies from the signal.

2. To reduce the transmission needs, we can remove the redundancies at the sender site, transmit the compressed signal, and then restore the signal at the receiving end.

3. Compression has an impressive effect on multimedia transmission needs.

Computer Networks

Computer networks differ in scope from relatively slow wide area networks, employed to transmit messages across vast geographic distances, to very fast local area networks that may connect computers located in the same building. System designers may select one of several arrangements for interconnecting network nodes, depending on an organization's requirement. There are several ways to establish a connection between the sender and the receiver of a message.

Network Topologies

Computers, switches, and terminals interconnected by network links are collectively called nodes. The purpose of network control is to provide a connection between nodes that need to communicate. The arrangement of nodes and links in a network is called a topology. A variety of arrangements are possible, each with its own advantages and drawbacks. Network topology has to fit the structure of the organizational unit that will use the network, and this topology should also be adapted to the unit's communication traffic patterns and to the way the databases will be stored in order to facilitate access to them.

The following topologies are the most widely used:

1. Hierarchical Network

2. Star Network

3. Ring Network

4. Bus Network

Hierarchical Network:

a. A corporate host computer (often a mainframe), divisional minicomputers or powerful workstations, and workgroup support via micros.

b. This topology matches the organizational structure of many firms and is still frequently used in WANs.

c. The user workstations may be, in turn, interconnected using one of the LAN topologies.

d. Failure of the host does not disable divisional processing, which is a fail-safe feature.

e. Cost-effectiveness of micros and the growing importance of groupwork leads some downsizing firms to move away from hierarchical networks to client/server computing.

Star Network:

a. In a star network, a hub computer or switch (such as a PBX) interconnects a number of workstations.

b. The computer at the hub acts as the network server, providing access to the shared database and software. All communications between the workstations must go through this central mode.

c. The star network is rather easy to manage and expand, since in both cases it is largely the single central node that is affected in an expansion of a processing capacity.

d. The central node is a locus of vulnerability: it may be overloaded or it may fail, disabling the entire network.

Ring Network:

a. Each node in a ring network is connected to two of its neighbours.

b. The nodes are usually close to one another; this topology is frequently used in LANs.

c. When one node sends a message to another, the message passes through each intermediate node, which restores the signal, as signals deteriorate in transmission.

d. If a node fails, the ring is out of service, unless the ring contains two channels transmitting in opposite directions.

Bus Network:

a. The nodes on a bus network are connected to a common link such as coaxial cable. This arrangement is used in LANs.

b. A failing device does not affect the rest of the network; failure of the bus itself, of course, brings the network down.

Switching in Networks

Many users can be connected at the same time to a network of communication channels. Switching devices establish connections between nodes that need to communicate over a network. Principal techniques for switching include:

1. Circuit Switching

2. Packet Switching

3. Fast Packet Switching

Circuit Switching:

1. The circuit switching technique is employed in a telephone network.

2. Communication links are connected to switching centers, which connect to one node to another on demand.

3. The circuit is established for the entire duration of the communication

4. Circuit switching is suitable for file transfers and similar longer transmissions

Packet Switching:

1. Packet switching is of particular importance for data communication owing to its speed and its superior utilization of communication links when handling Abursty,@ intermittent, traffic. Indeed, data transmission involves short bursts of activity by a computer or a terminal when the data are sent, followed by long periods when there is no transmission.

2. Packet switching offers flexibility in connecting to a network. It is used by most of the public data networks provided by value-added carriers.

3. In packet switching, messages are divided at the source into fixed-length chunks, called packets, that also include bits identifying the receiver. Typically, a packet contains 128 bytes of data.

4. Each packet, can be transmitted independently, with routing determined at each node the packet passes through (as opposed to circuit switching, where the route is predetermined).

Fast Packet Switching:

Traditional packet switching checks each packet for errors at every node the packet passes through. Modern telecommunications equipment is far more noise-free than that for which packet switching was originally designed. To take advantage of this, two fast packet-switching technologies are being introduced:

Frame Relay: Fast packet switching that checks a packet for errors only at the entry and exit nodes of the telecommunications network, thus reducing transmission delay.

Cell Relay: (asynchronous transfer mode, or ATM) transfers very short fixed-length packets, called cells, over fast LANs or WANs.

Communications Protocols in Computer Networks

Communication rules, called protocols, enable dissimilar hardware and software to communicate over a single network.

Network Protocols

Computer networks exist to provide connectivity among a variety of computers and access devices. To ensure orderly communication over a network, all the nodes in the network must follow a set of rules called protocols. These rules are complex. They extend from the electric connection to the network and the format of the message, all the way to the interaction between application programs that run on different nodes.

Explain to students that with the globalization of telecommunications, the International Standards Organization (ISO) has developed the OSI model in order to organizing protocols. The open system approach opens the field to a broad array of competing vendors, a situation that benefits users to ensure that they are not locked into a closed, proprietary protocol structure of a specific manufacturer.

1. Gives both users and vendors flexibility in conforming to a standard.

2. Users can select a protocol for any layer of the model, as long as the protocol performs the necessary services and provides the same interface to the adjacent layers.

3. If a layer has to be changed, only the hardware or software implementing that layer need be modified.

4. A protocol layer in one node interacts with the corresponding layer in another one.

The functions of the seven layers of protocol in the OSI model. They include:

Layer and its Function

1.Physical Provides access to the telecommunications medium and ensures transmission of the bit stream over it

2.Data Link Ensures error-free transmission of frames (blocks) of data over a network link

3.Network Routes messages (or packets) from source to destination by selecting connecting links

4.Transport Provides reliable end-to-end connection between two communicating nodes. When packet switching is used this layer breaks a message into packets

5.Session Establishes, maintains, and terminates a connection (session) between two applications running on communicating nodes. A session lasts, for example, from a long-on to a specific application to a log-off.

6.Presentation Provides any necessary conversions of the character being sent (encryption/decryption, compression/decompression, or character code conversions). Issue requests for establishing and terminating a session to the session layer

7.Application Provides services to communicating application programs; examples include file transfer, running a remote program, allocating a remote peripheral, and ensuring integrity of remote databases.

Two protocol sets have gained importance:

SNA - IBM's Systems Network Architecture.

- its functions are broken down into five layers, basically performing the functions of the five middle OSI levels.

TCP/IP - Transmission Control Protocol/Internet Protocol

- its functions are broken down into five layers. TCP provides the higher-level services in connecting the communicating applications, while IP ensures the lower-level functions of routing and addressing, guiding the packets over the Internet.

Interconnections among Networks

As communication needs increase, network connectivity becomes a major issue as users want to access a remote computer. Gateways such as routers and bridges help solve the problem.

Interconnection between two networks of the same type is accomplished by a relatively simple bridge, implemented in hardware and software. Interconnection between dissimilar networks, for example, a LAN and a WAN, is achieved through a more complex router. A router is a device that accepts messages in the format produced by one of the networks and translates them into the format used by the other.

Local Area Networks

Organizations small and large use fast local area networks (LANs) to interconnect personal computers, and thus make a basic workgroup tool.

Local Area Network: Workplace for a Workgroup

A LAN interconnects computers within a single site, such as an office building, a manufacturing plan, or a corporate or university campus.

Characteristics of a LAN include:

1. Its scope is commonly measured in feet

2. Communication speeds are very high

3. Used as a local means of computing and communication among users in larger firms

4. Are owned by the organization

5. Afford a sense of control and the flexibility to meet the demands of the end users

A LAN gives its users the following capabilities:

1. Users can share resources, such as a fast printer or a database

2. Users can collaborate by communicating over their LAN. This collaboration may be facilitated by groupware that runs on a LAN

3. Users can access other networks within a firm or outside of it via bridges and routers

There are two principal LAN designs:

1. Peer-to-peer - peripherals are located at terminals and system administration is largely left up to the users

2. Server-based networks - shared resources are placed a dedicated server that manage a given resource on behalf of user workstations sharing the resource (file server, printer server, gateway, optical disk server). Most of the servers are dedicated to their task; using them as workstations degrades the performance of the net.

Local Networks Based on Private Branch Exchanges:

A company with a large number of telephones (from 50 to over 10,000) often elects to own a computer-based private branch exchange (PBX), an electronic switchboard that interconnects its telephones and provides connections to the public network.

Characteristics of a PBX:

1. Gives a company control over the usage of its telephone system and offers a variety of features, such as call forwarding or voice messaging.

2. Maybe employed as a switch for data communications

3. Many newer PBXs use digital technology, eliminating the need for modems, and perform conversions needed to ensure connectivity between various equipment and telecommunications links.

4. Easy to connect a new workstation to the net.

5. Speeds of PBX-based networks are limited

Client/Server Computing

An important current development in organizational computing is downsizing - moving from platforms based on mainframes and minicomputers to a microcomputing environment. These architectures are based on the client/server model.

Characteristics of client/server computing:

1. The processing of a given application is split up among a number of clients - serving individual users - and one or more servers - providing access to databases and doing most of the computing.

2. Main objective of a client is to provide a graphical user interface to a user

3. Main objective of a server is to provide shared services to clients

4. In client/server computing, individual applications are actually written to run on several computer platforms to take advantage of their capabilities

5. Client/server computing is difficult to implement

Most frequently used models of client/server computing are:

1. Two-Tier Architecture

2. Three-Tier Architecture

Characteristics of Two-Tier Architecture:

1. Client performs presentation services. It displays the GUI and runs the program that determines what happens when the user selects a menu option.

2. Server manages the accesses to the database

3. Clients send remote procedure calls to activate specific applications logic on a server

Characteristics of Three-Tier Architecture:

1. An application server runs most of the application logic, with the user workstation responsible for the display at the front end and the database server providing database servers at the back end. Objective is to distribute application so as to reduce the overall hardware costs while minimizing the network traffic

Issues of client/server computing:

1. Is attractive in terms of their acquisition price as related to their performance

2. Is moving computing control out of the data centers and into the end-user areas

3. Software is complex, and is expensive to maintain

4. Generate significant traffic on the firm's backbone network that connects clients and servers

5. Can be performed in LAN and WAN environments

Wide Area Networks

Wide area networks are the fundamental infrastructure of organizational computing. These long-distance telecommunications networks employ a variety of equipment so that the expensive links may be used effectively. The offerings of common carriers and of providers of value-added services may be combined with private networks to create an overall organizational network.

Telecommunications Equipment for Wide Area Networks WANs include equipment that controls message transfer and makes it possible to share the links among a number of transfers.

Host Computer

A WAN has a powerful host computer. The host runs a system program, called a telecommunications monitor, which processes incoming messages, passing them to the appropriate application programs, and accepts outgoing messages from the applications in order to transmit them into the network.

Front-End Processor

Relieves the host computer of most of the tasks involved in network control. Under the control of its own software, the front-end processor accepts messages coming from the network and routes outgoing messages to their destinations. It performs the necessary code conversions, encrypts and decrypts secure messages, and performs error checking so that the host deals with Aclean@ messages.

Cluster Controller

Manages several terminals, connecting them to a single telecommunications link, and performs communication tasks for them, such as screen formatting, code conversion, and error checking. A cluster controller may also allow the terminals to share a high-speed printer and may handle electronic mail among the cluster terminals.

Multiplexor

Combines the data that terminals send to it over local low-speed links into a single stream. This stream is then transmitted over a high-speed telecommunications channel and is split by another multiplexor on the opposite end of the channel.

Concentrator

Combines transmission from several slower terminals that operate in a burst mode into a single transmission stream that requires a link of lower speed than the sum of the speeds of all the terminals combined. A concentrator stores messages from terminals and forwards them when warranted.

Switches

Establishes connections between nodes that need to communicate.

Access Terminals

Include a variety of dumb terminals, with no processing capacity and intelligent terminals with processing capacity, such as personal computers.

Where do Facilities for Wide Area Networks Come From?

Some network facilities are owned by user organizations, others can be leased by them, or simply used on a pay-as-you-go basis. Among the typical facilities owned by user firms are workstations, host computers, and front-end processors. The essential providers of telecommunications links and services are common carriers and the vendors of enhanced services on value-added networks. These include:

1. Common carriers

2. Providers of value-added networks

3. Private lines and private networks

Common Carriers

Are companies licensed by a country's government to provide telecommunications services to the public. The vast majority of common carriers provide telephone service. These carriers offer the use of a wide-area telecommunications infrastructure, that is, facilities for the transmission of voice and data messages.

Common carriers offer a service called virtual private network where a user firm can purchase guaranteed access to facilities with specified capabilities, such as transmission speed and access points.

Providers of Value-Added Networks

Value-added vendors lease facilities from the common carriers and provide telecommunications services to their own customers. These vendors add value to the basic infrastructure furnished by the common carrier. The value-added networks (VAN) provided by the vendors furnish services over and above those provided by common carriers.

Private Lines and Private Networks

Instead of using a service that has to be shared with others, a firm may lease its own private lines or entire networks from carriers. This can have economic advantages as compared with VAN use, as well as provide faster and more secure communications.

The Internet and Electronic Commerce

The Internet has changed the face of individual and organizational computing. Driven by the possibilities offered by the Internet and the Web, electronic commerce is expanding its reach.

Present and Future of the Internet

The Internet is the global network of computer networks without a centralized control that has become the contemporary Ainformation highway.@

Characteristics of the Internet:

1. It is run in a decentralized fashion by a number of voluntary organizations, the principal of which is the Internet Society.

2. It is a medium of communication, a source of information, and a developing means of electronic commerce.

3. A major obstacle to its development has become the limited capacity of the links interconnecting the networks.

Facilities for Communication and Information Access

The Internet provides several essential facilities that organizations can use for internal as well as interorganizational information sharing and communication.

The principal categories of Internet use include:

1. Communication

Electronic mail (E-mail) facilitates quick exchange of information and ideas, and is the Internet facility in widest use. E-mail can be used for one-to-one communications or to participate in larger communications forums (newsgroups).

2. Information Access:

The Internet provides access to the largest organized (loosely) repository of information on earth: the collection of electronic documents stored on sites all over the world. The main problem is finding the information. To help with this problem Web search engines have been developed. Examples include Gopher sites, using indexes such as Veronica, or via a WAIS (Wide Area Information Service) keyword search.

The World Wide Web

The World Wide Web (or simply, the Web) is an information service available over the Internet, providing access to distributed electronic documents via hyper links.

Characteristics of the Web:

1. Grew out of the need of scientists who wanted to share information and to collaborate from geographically dispersed locations.

2. Is a client/server system. The Web is a collection of electronic sites stored on many thousands of servers all over the world. Each site consists of a home page and often other pages stored with it. Pages contain hyperlinks to related pages, usually stored on other sites.

3. Access to the Web is through a client program, known as a browser. The browser sends out for the needed page into the Internet, interprets the formatting directions on the retrieved page, and displays the page accordingly on the screen.

4. To access a Web site, you provide the browser with the site's identifier, known as a URL (Uniform Resource Locator).

5. A search engine is a Web facility that maintains its own information about the documents available on the Web.

Electronic Commerce

Electronic commerce is sharing business information, maintaining business relationships, and conducting business transactions by means of telecommunications networks. In simple words, electronic commerce is doing business electronically, replacing most of the paper and telephone work with computer-mediated information and transaction exchange. The Internet, and the Web in particular, are emerging as the principal means for this new way of doing business.

Some potential uses include:

1. Establishing an electronic site on the Web to promote your business

2. On-line marketplaces

3. Advertising at frequented Web sites

4. Establishing newsgroups

5. Job classified

One of the biggest drawbacks for the use of the Internet for conducting electronic commerce is the lack of financial security.

The framework of electronic commerce is summarized in three levels:

1. Infrastructure

- the hardware, software, databases, and telecommunications that together deliver such functionality as the Web over the Internet, and support EDI and other forms of messaging over the Internet or over value-added networks.

2. Services

- messaging and a variety of services enabling the finding and delivery of information, as well as negotiation, transaction business, and settlement.

3. Products and structures

- direct provision of commercial services to consumers and business partners, intraorganizational information sharing and collaboration, and organization of electronic markets and supply chains.

Intranets

Using the Internet, many firms have implemented internal networks of Web sites, known as intranets. Intranets are set up on corporate LANs and WANs. An Intranet is separated from the public Internet by a facility called firewall. The firewall program runs on the server computer, preventing access to the Intranet from the public Internet, but allowing access to the Internet. Intranet is, in effect, the owner company's private Internet.

Intranets have become important business tools for:

1. Sharing information and knowledge among a company's employees

2. Accessing databases and data warehouses

3. Organizing the corporate workflow around electronic documents

4. Enabling collaboration

Information System Architecture:

The high-level design of a blueprint for the organizational information system is known as the information system architecture. This plan must support the present and future computing and communications needs of a business. Today, the architectural blueprint of many organizations rests on internetworking: interconnecting a number of local area networks with a corporate wide area network, or using the Internet connectivity. Fundamental components of an architectural plan must address the following concerns:

1. How will the processing power be distributed

2. Where will the databases be located

3. Network interconnections

4. Protocols

Using Telecommunications for Business Process Redesign and to Seek Competitive Advantage

Telecommunications give an organization the capability to move information rapidly between distant locations and to provide the ability for the employees, customers, and suppliers to collaborate from anywhere, combined with the capability to bring processing power to the point of the application. All of this offers a firm important opportunities to restructure its business processes and to capture high competitive ground in the marketplace. Through telecommunications, this value may be:

1. An increase in the efficiency of operations

2. Improvements in the effectiveness of management

3. Innovations in the marketplace

Telecommunications may provide these values through the following impacts:

1. Time compression

- Telecommunications enable a firm to transmit raw data and information quickly and accurately between remote sites.

2. Overcoming geographical dispersion

- Telecommunications enable an organization with geographically remote sites to function, to a degree, as though these sites were a single unit. The firm can then reap benefits of scale and scope which would otherwise be unobtainable.

3. Restructuring business relationships

- Telecommunications make it possible to create systems which restructure the interactions of people within a firm as well as a firm's relationships with its customers. Operational efficiency may be raised by eliminating intermediaries from various business processes.

POWER IS EVERYTHING

All satellite systems need electricity. It is usually supplied by solar panels made up of photovoltaic cells, which convert energy from photons in the Sun's rays into electrical energy.

On a satellite in low-Earth orbit, solar energy incident on the solar cells provides a power of 1,400 watts per square metre.

But solar power systems are not very efficient, since only 200 watts per square metre of this energy is retrieved and used. As a result, the cells must be turned towards the Sun all the time to gain maximum power.

Test of the solar arrays deployment. Credits: ESA/CNES/Arianespace

Certain interplanetary probes operating in deep space a very long way from the Sun have solar panels spanning 30 square metres and use nuclear radioisotope thermoelectric generators (RTGs) as a supplementary power source.

Power generated is used immediately by the satellite or stored in batteries so that systems can continue to function when the satellite passes in the Earth’s shadow or when it needs to draw extra power.

Most energy consumed is converted to heat, which does not dissipate easily. Heat exchanges therefore have to be controlled to avert overheating or excessive cooling.
The satellite’s internal temperature is regulated by reflective and insulating blankets. Excess heat is rejected to space by radiators.

DO YOU KNOW?

Heat transfer

In space, heat only propagates by radiation in the form of waves. On Earth, two other natural processes dissipate heat and regulate temperatures:

Conduction is the process by which heat is transferred by communication of kinetic energy from particle to particle

Convection is the process by which heat is transferred by displacement through a fluid

Criteria for assessing the knowledge of students

| Estimation in letters | Equivalent of scores in figures | In per cent % | Traditional assessment | Criteria for assessing the knowledge of students |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | Excellent | The student demonstrates: - a deep and lasting assimilation of program material - complete, consistent, competent and logically presented answers for all types of activities; - ability to freely cope with the tasks; - correct, well-founded decisions, - skills of using the information of basic and additional specialized literature in the study of discipline, - the ability to self-systematize the program material; - possession of versatile skills and methods of performing all types of tasks; -ability to work with foreign literature and information resources of the Internet; - timely and high-quality fulfillment of all types of tasks. |
| А- | 3,67 | 90-94 |  | The student demonstrates: - a deep assimilation of program material; - complete, consistent and logically presented answers for all activities; - ability to cope with the tasks; - correct taken decisions , - skills of using special literature in the study of discipline, - the ability to self-systematize the program material; - possession of skills and techniques for performing all types of tasks; - timely fulfillment of all types of tasks. |
| В+ | 3,33 | 85-89 | Good | The student demonstrates: - mastering the program material; - complete, consistent, literate, without significant inaccuracies, an account of the answers for all kinds of tasks; - the correct application of theoretical knowledge - the possession of necessary skills in the performance of applied tasks - the skills of using recommended literature in the study of discipline, - the skills of systematizing program material; - possession of skills and techniques for performing all types of tasks; - timely fulfillment of all types of tasks. |
| В | 3,0 | 80-84 |  | The student demonstrates: - mastering the program material; - a consistent presentation of the answers for all types of tasks with non-essential mistakes; - skills in applying theoretical knowledge under the guidance of a teacher; - possession of necessary skills in performing practical tasks - skills of using recommended literature in the study of discipline, - skills of systematizing program material under the guidance of a teacher; - mastering the skills of performing all kinds of tasks; - the ability to independently correct mistakes; - timely fulfillment of all types of tasks with the elimination of mistakes. |
| В- | 2,67 | 75-79 |  | The student demonstrates: - mastering the program material; - ability to present answers with non-essential mistakes; - skills in applying theoretical knowledge under the guidance of a teacher; - possession of skills in performing practical tasks - skills in using recommended literature under the guidance of a teacher - generalization skills of program material under the guidance of a teacher; - the ability to correct mistakes with the help of a teacher; - timely fulfillment of all types of tasks with the elimination of mistakes. |
| С+ | 2,33 | 70-74 | Good | The student demonstrates: - mastering the basic material; - inadequate formulations when responding to all kinds of tasks; - violation of the sequence in the presentation of the program material; - difficulties in self-fulfillment of practical tasks; - possession of individual techniques in the performance of practical tasks - skills of using the literature recommended by the teacher; - skills of generalization of separate sections of program material under the guidance of a teacher; - the ability to correct major mistakes with the help of a teacher; - fulfillment of all types of tasks with elimination of mistakes. |
| С | 2,0 | 65-69 | Satisfactory | The student demonstrates: - mastering the basic material; - inadequate understanding of formulations when answering all types of tasks; - lack of consistency in the presentation of the material; - difficulties in self-fulfillment of practical tasks; - possession of individual techniques in the performance of tasks - difficulties in using the literature recommended by the teacher; - difficulties in generalizing the individual sections of the material studied; - the ability to correct major mistakes with the help of a teacher; - fulfillment of all types of tasks with elimination of mistakes. |
| С- | 1,67 | 60-64 | Satisfactory | The student demonstrates: - mastering the basic material; - inadequate understanding of formulations when answering all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - inadequate possession of individual methods in the performance of tasks - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the studied material; - the ability to correct major mistakes with the help of a teacher; - fulfillment of all types of tasks with elimination of mistakes. |
| D+ | 1,33 | 55-59 | Satissatisfactory factory | The student demonstrates: - mastering of separate sections of the main material; - misunderstanding of formulations when answering all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - significant difficulties in the application of individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the studied material ; - difficulties in correcting the mistakes with the teacher; - untimely fulfillment of all types of tasks with elimination of mistakes. |
| D | 1,0 | 50-54 | Satisfactory | The student demonstrates: - difficulties in mastering individual sections of the main material; - misunderstanding of formulations when answering all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - inability to apply individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - inability to generalize the individual sections of the studied material ; - significant difficulties in correcting the mistakes with teacher; - untimely fulfillment of all types of tasks with elimination of mistakes. |
| F | 0,5 | 25-49 | Unsatisfactory | The student demonstrates: - no knowledge of the program material, - gross mistakes are made when performing all types of tasks; - lack of skills in the usage of individual methods of performing tasks; - failure to complete certain types of tasks, provided by the forms of the current, midterm and final control. |
| FX | 0 | 0-24 | Unsatisfactory | The student demonstrates: - ignorance of program material, - at performance of all types of tasks gross errors are allowed; - lack of skills in the usage of individual techniques for performing tasks; - failure to comply the tasks provided by the forms of current, midterm and final control. |

LITERATURE

1. Радовель В.А. Учебное пособие “Английский язык для технических вузов”

Москва, 2010.

2. Кожевникова Т.В « English for Communications students», М., 2003

3. Бобылева С.В., Жаткин Д.Н. «Английский язык для сферы информационных технологий и сервиса», Ростов на Дону, 2009

4. Бухина С.Б. Английский язык. Radio engineering and radio systems. Методические

указания  (для студентов  специальности  5В071900),  АУЭС, 2014г.

5. Коробейникова Л.Я. Английский язык. Telecommunication systems and networks. Методические указания (для студентов специальности), АУЭС, 2014г.

6.C. Leadbetter, S.Wainwrite  Cambridge IGCSE. “Computer  Studies” Course book, Cambridge University Press, 2010

7.Козлов  В.С., Коробейникова Л.Я.,  «Английский  язык. Developing  and Mastering  Translation  Skills  (for  radio  engineering  students). Учебное пособие, АУЭС. Алматы, 2012.

8. Fabre, Elena Marco,“Professional  English in Use: Intermediate  to  Advanced”, Cambridge University Press, 2007.9. Leabetter, Chris, “Cambridge IGCSE ICT Course book”, Cambridge University Press, 2010.

9. Пумпянский А.Л. Упражнения по переводу научной и технической

литературы с английского языка на русский и с русского языка на английский [Текст]/ А.Л. Пумпянский.-Мн.:ООО «Попурри», 1997.

10. Агатаева Б.Б. «Основы радиотехники, электроники, телекоммуникаций». Конспектлекций. Часть 1, АУЭС, Алматы,2010

11. Агатаева Б.Б.  «Основы радиотехники, электроники, телекоммуникаций». Конспект лекций. Часть2,АУЭС, Алматы 2010

12. Seidenberg  I.Telecommunications Handboo for Transportation Professionals (The Basics of Telecommunications), (электронная версия).

13. Roger L. Freeman Fundamentals of Telecommunications. –International Center for Theoretical Physics. 2005, (электроннаяверсия).

14. Salous Sana,  «Radio  Propagation  Measurement  and  Channel  Modelling», Cambridge University Press, 2013

15. Cvijetic  Milorad.,  Advanced  Optical  Communication  Systems  and  Networks  / M. Cvijetic, I.B. Djordjevic.-Boston: Artech House, 2013.

16. Palicot Jacques «Radio Engineering: From Software Radio to Cognitive Radio», London, 2011

17. Tom Ricca-McCarthy,  Michael  Duckworth,  “English  for  Telecoms and Information Technology”, Oxford University Press, 2014.

18. Virginia Evans, Jenny Dooley, Carl Taylor, “Electronics”, Express Publishing, 2014

CONTENT

| Introduction………………………………………………………………. | 3 |
| --- | --- |
| Unit 1: RADIO ENGINEERING………………………………………………………. | 4 |
| Unit 2: UNIT 2 RADIO TRANSMITTERS AND RECEIVERS……………………... | 16 |
| Unit 3: UNIT 3 BASIC PRINCIPLES OF TELEVISION…………………………….. | 23 |
| MIDTERM CONTROL 1 Lesson 14……………………………………. | 30 |
| Unit 4: UNIT 4 BASIC PRINCIPLES TELECOMMUNICATION SYSTEMS FORMATION…………………………………………………………………………… | 31 |
| Unit 5: UNIT 5 COMMUNICATON NETWORKS…………………………………... | 38 |
| Unit 6 СELLULAR SYSTEMS AND STANDARTS………………………………… | 52 |
| MIDTERM CONTROL 2 Lesson 30……………………………………. | 62 |
| BRIEF REVIEW…………………………………………………………. | 64 |
| Criteria for assessing the knowledge and skills of students in practical lessons……………………………………………………………………… | 81 |
| LITERATURE……………………………………………………………. | 83 |
| CONTENT………………………………………………………………… | 84 |