F. 7.03-20

M.M. Yeskendirova, M.Z. Yeskendirov

STUDY GUIDE

KAZAKHSTAN MINERAL RAW MATERIAL

Shymkent, 2019

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

M. AUEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

YESKENDIROVA M.M., YESKENDIROV M.Z.

STUDY GUIDE

KAZAKHSTAN MINERAL RAW MATERIAL

for students of speciality

5B072000 – Chemical technology of inorganic substances

Shymkent, 2019

UDK 622.7

Yeskendirova M.M., senior lecturer; Yeskendirov M.Z., Dc. Tech. Sci., associate professor. Kazakhstan mineral raw material / Study guide. - Shymkent: M. Auezov South Kazakhstan State University, 2019. – 125 p.

The study guide contains the information about natural and technogenic mineral raw materials of Kazakhstan. Basic kinds of Kazakhstan natural reserves, such as ores of ferrous, nonferrous, rare and noble metals, uranium, fuel resources, phosphorite, non-metallic minerals, their main deposits and date about their mining, processing and import are considered. In addition, the authors consider basic principles and directions of development of non-waste and low-waste technologies, problems of complex use of mineral resources, kinds and application areas of technogenic materials.

The study guide is intended for students of speciality 5B072000 – “Chemical technology of inorganic substances”.

Reviewers:

Satayev M.S. – Doc. Tech. Sci., professor of department “Chemical technology of inorganic substances” of M. Auezov SKSU;

Ashirov A.M. – Doc. Tech. Sci., professor of South Kazakhstan State Pedagogical University;

Kol’tsova E.M. – Doc. Tech. Sci., professor of D.I. Mendeleyev Russian Chemical Technological University.

The study guide was recommended for print by Educational Methodical Council of M. Auezov South-Kazakhstan State University, minutes #  ___________

© M. Auezov South Kazakhstan State University

CONTENTS

| Part I. Kazakhstan mineral raw material base Occurrence of chemical elements in the Earth’s crust. Kinds and a characteristic of mineral raw materials | 7 7 |
| --- | --- |
| A role of mineral raw material recourses in the innovation development of the Republic of Kazakhstan | 9 |
| Geographic distribution of minerals on the Kazakhstan territory and their characteristic | 13 |
| A share of Kazakhstan mineral raw materials in the world resources. Categories of raw material reserves | 18 |
| Characteristic of ferrous metals resources | 20 |
| Nonferrous metals | 25 |
| Gold mining in Kazakhstan | 29 |
| Production of rare-earth elements in Kazakhstan Uranium and nuclear power in Kazakhstan | 30 38 |
| Uranium in-situ leaching technology | 53 |
| Kazakhstan oil and gas reserves | 55 |
| Coal resources | 62 |
| Karatau phosphorite | 64 |
| Potassium ore reserves PART 2. Complex using of raw materials – a scientific principle of mineral resources rational exploitation Concept about a non-waste technology Basic directions of non-waste and low-waste technologies Classification of chemical wastes Complex processing of raw material Physical and chemical features of waste products Utilization of metallurgical slags | 70 76 76 81 83 85 88 95 |
| Utilization of power waste | 96 |
| Waste of phosphorus industry | 99 |
| Literature | 103 |

Part I. Kazakhstan mineral raw material base

Occurrence of chemical elements in the Earth’s crust. Kinds and a characteristic of mineral raw materials

The Earth is the third planet of the Solar system from the Sun. The Earth was formed about 4,7 billion years ago out of a trace gas-dust substance. The Galactic history of the Earth is divided into two stages: pre-Cambrian Period (first 3,9 billion years) and Phanerozoic (the last 800 million years).

The Earth consists of the geosphere (the nucleus, the Earth’s mantle, the earth crust), the hydrosphere, the atmosphere and the magnetosphere. Prevalent elements in the Earth structure are iron (34,6%), oxygen (29,5%), silicon (15,2%), magnesium (12,7%). A predominant element in the earth crust is oxygen (49,4%), then there are silicon (25,8%), aluminium (7,5%), iron (4,7%), calcium (3,4%), sodium (2,6%), potassium (2,4%), magnesium (1,9%), hydrogen (0,9-1%), titanium (0,6 %), others – 0,8%. The least widespread element is radon (10·10-16 %). The average distance from the Earth external surface to the Earth nucleus center is 6370 km.

Fig. 1 – The Earth’s structure

Mineral raw materials are minerals extracted from the Earth’s bowels.

Mineral raw materials are divided into following kinds: ore mineral raw materials (rocks → metals), nonmetallic mineral raw materials (rocks → reserves for chemical, construction industries and other kinds of nonmetallic materials) and fuel mineral raw materials (coal, oil, gas, combustible slates, etc.).

The number of the substances used by people in their practical activity is rather great and continuously grows, as new substances are opened and synthesized daily. Now there are about 3 million substances (approximately 300 ths of inorganic substances and 2,5 million of organic substances), each of them differs from others by its properties. Many of these substances are produced as the result of chemical processing.

Fig. 2 – For 200 million years, the proportions of different gases in the atmosphere have been the same as they are today: about four-fifths (78%) of nitrogen; about one-fifth (21%) of oxygen; small proportions of other gases including carbon dioxide, water vapour and noble gases.

Chemical manufactures can be divided into 2 groups: manufacture of inorganic substances and manufacture of organic substances.

Manufactures of inorganic substances includes:

- Manufacture of basic inorganic substances (acids, alkalis, salts, fertilizers, etc.).

- Manufacture of high inorganic products (reagents, rare elements, semiconductors, pharmaceutical preparations, etc.).

- Electrochemical manufactures (chlorine, alkalis, oxygen, hydrogen and other substances).

- Metallurgy (metallurgy of ferrous, nonferrous, rare, precious metals, etc.).

- Manufacture of silicates (glass, cement, ceramics, etc.).

- Manufacture of mineral paints and pigments.

The industry of organic substances includes:

- Basic (heavy) organic synthesis (alcohols, acids, ethers, processing of CH4, CO, H2, C2H2, etc.).

- Manufacture of intermediate products and dyes.

- High organic synthesis (pharmaceutical preparations, cinema and photo reagents, etc.).

- Manufacture of high-molecular substances (plastic, artificial and synthetic fibres, rubber, etc.).

- Processing of fuel (oil, coal, slates, etc.).

- Manufacture of foodstuff (sugar, fats, etc.).

Increase in efficiency of raw material and energy application at mass production of chemical substances has the exclusively great value.

A role of mineral raw material recourses in the innovation development

of the Republic of Kazakhstan

In accordance with the quantity and variety of mineral resources Kazakhstan occupies one of the leading places in the world. Explored reserves are a basis for creation of powerful oil and gas mining, uranium and coal industries, industry of mining and processing of ferrous, nonferrous and precious metal ores, various nonmetallic minerals.

Kazakhstan mining-and-metallurgical complex includes more than 200 mining and processing enterprises. Now the commodity output of these enterprises is exported in European countries, the USA, China, South Korea, Singapore, Malaysia and other countries.

The raw-material base of the fuel and energy complex in the republic is sufficient for full satisfaction of present and perspective needs. The State balance of useful minerals takes into account more than 200 deposits of hydrocarbon raw material. There are 3 oil refining and 3 gas processing factories, about 260 oil companies in Kazakhstan territory. Expected hydrocarbon resources are connected with 15 sedimentary basins, which are characterised by diversity of geological structures and a wide stratigraphic gas content range and allow us to expect the essential gain of oil and gas reserves.

Fig. 3 – A petroleum deposit

Uranium and coal branches are provided with the explored stocks on more than 100 years. Main uranium reserves are concentrated in South Kazakhstan hydrogenic deposits (deposits Mynkuduk, Karamurun, etc.) suitable for mining by a progressive underground leaching technique. The possibility to apply this method is the uniqueness of the Kazakhstan natural uranium raw-material base. Introduction of this way has radically changed all the process of extraction and processing uranium ores in Kazakhstan. Volumes of uranium output have increased at simultaneous sharp reduction of its cost price. All Kazakhstan uranium mining and processing manufactures are incorporated into NAC KazAtomProm JSC which includes a metallurgical plant on deep processing of uranium raw material. Expected uranium resources are comparable with the balance reserves and situated, mainly, in South Kazakhstan uranium provinces.

Fig. 4 – Exploitation of an iron ore deposit

Basic coal reserves are located in five largest coal basins in Central and Northern Kazakhstan (Karaganda, Ekybastuz and other), which includes more 300 deposits. Expected coal resources exceed the explored stocks in several times.

Kazakhstan has significant resources of ferrous metals. Main iron reserves are scarnified deposits in Northern Kazakhstan (deposits Sokolovskoye, Sarbayskoye, etc.) containing high-quality magnetite ores. On the base of Kazakhstan iron-ore deposits 6 large metallurgical complexes operate with 10 mines with design capacity about 80 million tonnes of an ore per a year.

All industrial manganese reserves are situated in Central Kazakhstan. These manganese ores are oxidized and carbonate-silicate-oxidized ores (deposits Ushkatyn III, Zapadnyi Karazhal, etc.). The manganese-ore branch includes 5 mining enterprises located in the territory of Karaganda oblast.

Chromites are characterized by high quality (45-50% of chromium oxide Cr2O3) and situated on a rather small area in Zapadnyi Kazakhstan (deposits Million, Almaz-Zhemchuzhina, etc.). Mining and concentration of the chromite ores is performed by transnational company “Kazchrom” which includes a ferroalloy production. Provision of mining enterprises with ferrous metals exceeds 80 years. The republic expected ferrous metal potential exceeds their explored stocks in several times.

As regards copper, lead and zinc reserves Kazakhstan concerns to the largest provinces of the world. High exploitation efficiency of nonferrous metals deposits is reached due to complex use of the ores and full extraction of lead, zinc, copper, gold, platinum and rare elements. Leading manufacturers of nonferrous metals in the republic are corporation “Kazakhmys” and company “Kazzinc” – the enterprises with a full industrial cycle starting from extraction of raw material to metallurgical processing of nonferrous metals. Widespread industrial types of copper deposits in Kazakhstan are stratiform, sulphide (pyrite), copper-porphyrite and skarn ones. A main part of the copper balance reserves is situated in East and Central Kazakhstan deposits. Supply of mining enterprises with operation-ready copper reserves is insignificant and makes about 25-30 years. At the same time the republic has potential opportunities for development of the raw-material base for copper-ore industry. A reserve for strengthening the mineral raw material base is a number of sulphide-polymetallic deposits located in East Kazakhstan (Artemyevskoye, Kosmurun, Akbastau, etc.). A copper-porphyrite deposit Nurkazgan with high quality ores in Central Kazakhstan, deposit Shatyrkol in the south of the republic and deposit Zhaman-Aybat in Zhezkazgan mining area – one of the largest in Kazakhstan – are ready for operation.

Fig. 5 – Coal mining

The republic significant potential is the largest copper-porphyrite deposits such as Aktogay, Aidarly, Koksay and Bozshakol. Problems of involvement of these deposits into development are connected with solving technological questions allowing realizing profitable processing low-grade ores. Now deposit Aktogay in East Kazakhstan containing more than 5 million tonnes of copper ores is prepared for mining by a heap leaching method.

Developed stocks of sulphide-lead-zinc ores are located, mainly, in the East Kazakhstan oblast; they are a base for operating ore dressing enterprises. Main problems of the lead-zinc branch are, on the one hand, absence of reserve deposits with profitable stocks in a field of activity of the old mining enterprises, which are provided with reserves during not more than 25 years, with another hand – the necessity of construction of processing complexes in areas of the explored deposits. At present because of bottleneck of concentrating factories and metallurgical manufactures a number of deposits in various Kazakhstan regions are not developed.

Plans of growth of the mineral raw material base in the near future stipulate the development of a large-scale deposit Shalkiya in Southern Kazakhstan, which will be a basis for construction of a new zinc factory; commissioning a karstic, nonconventional for Kazakhstan and unique on zinc content deposit Shaymerden in Northern Kazakhstan; building of mines on the basis of deposits Novoleninogorskoye, Dolinnoye and Obruchevskoye in East Kazakhstan. Real conditions for development of the copper-zinc industry have been created in Western Kazakhstan. Here there are explored copper-pyrite deposits (50 years of October, Kundyzdy, Priorsky, etc.) which can be a basis for construction of a mining and smelting plant. Expected resource of nonferrous metals in Kazakhstan is assessed highly enough. The collected data permit us to analyse the state of copper, lead and zinc reserves and are a basis for planning further regional and search works. A main body of the expected resources is situated in the main mining areas of the republic.

Fig. 6 – Powerful trucks operating on deposits

A basis of the mineral-raw-material base for aluminium production is reserves of hard-to-process bauxites in the East-Torgay bauxite area (deposits Krasnooktyabrskoye, Ayatskoye, etc.). Earlier these fields have been considered as unsuitable for manufacture of alumina according to the existing technological circuit, but as a result of development of a new manufacturing diagram by the Pavlodar aluminium factory, conditions for alumina production from low-grade bauxites have been created. All manufactures of bauxites and alumina is realized by OJSC “Aluminium of Kazakhstan”. To date prospects of development of the mineral-raw-material base for the aluminium industry are connected, first of all, with nonconventional kinds of aluminous raw material.

Kazakhstan is a large gold-bearing province. Gold-ore and gold-containing deposits are located in 16 mining areas; major of them are Kalbinsky and Rudno-Altay in East Kazakhstan (deposits Bakyrchik, Bolshevik, Ridder-Sokolnoye); Kokshetau and Zholymbet-Bestobe in Northern Kazakhstan (Vasilkovskoye, Zholymbet); Shy-Iliyisky and Dzhungarsky in Southern Kazakhstan (Akbakay, Beskempir, Arkharly); Maykainsky and North-Balhash in the Central Kazakhstan (Maykain, Boshshekul, Sayak); Zhetygarinsky and Mugojarsky in Western Kazakhstan (Komarovskoye, Varvarinskoye, Yubileynoye).

The State mineral balance takes into account gold reserves of 263 objects which major part concerns to small deposits. The basis of Kazakhstan gold raw-material base is reserves of gold-ore deposits proper. Complex sulphide-polymetallic deposits where gold is taken as an accompanying component have the essential role too. The opportunity of increase in the accompanying gold mining from these complex deposits is limited and completely depends on the development of lead-zinc and copper industries. Expected gold resources exceed the explored ones in 3-4 times.

Recently the great attention in Kazakhstan is given to the development of geologic exploration. Financing of geological prospecting is increased annually.

Geographic distribution of minerals on the Kazakhstan territory

and their characteristic

Geological exploration is unequivocally proved that an origin and a location of each useful mineral completely depend on a structure and a direction of development of the earth crust in the certain area. There are some theories about the structure of the earth crust. However, the most widespread theories are a doctrine about a geosyncline structure of the earth crust (and about a connection between formation of mineral deposits and certain stages of its development) and a doctrine about deep mobile zones. Now these two theories do not contradict to each other. Quite the contrary, they supplement each other and allow us to formulate an objective picture of mineral formation on the Kazakhstan territory.

In a genetic attitude the earth crust consists of geosynclines and platforms.

A platform is characterized by rather weak volcanic, orogenic processes. On the contrary, a geosyncline is a lot of the Earth's crust describable by a high energy level, considerable tectonic movements, wide demonstration of high intensive disjunctive and folded destructions, and great thickness of volcanic and sedimentary rocks. Formation of various volcanic minerals is connected with volcanic processes occurring inside geosynclines over a long period of time; and deposits of a sedimentary type – with products of destruction of mountain systems.

Thus, a geosyncline is a long, rather narrow and deep deflection of the earth's crust usually bounded by its fractures and filled with sedimentary and volcanic rocks; a large-scale depression in the earth's crust containing very thick deposits. As a result of a long-term and intensive tectonic deformation it turns to a complex folded structure that is a part of mountain system.

Volcanic rocks having an acid composition (quartz, orthoclase, microcline, plagioclase, mica) are prevalent in a granite-acid stratum of the earth's crust, and rocks mainly of a basic structure (basic plagioclase, pyroxene, olivine) in its basalt stratum. A sedimentary stratum of the earth's crust mainly consists of sedimentary rocks. Sedimentary rocks are rocks formed during sedimentation of a matter in a water medium or as a result of glacier activity. The sedimentation occurred by chemical, mechanical and biogenic ways. Sedimentary rocks are divided into clastic, chemical and biogenic (organogenetic) ones. Approximately ¾ of all minerals (coal, oil, combustible gases, salts, iron, manganese and aluminium ores, placers of gold, platinum, diamonds, phosphorites, building materials) are connected with the sedimentary rocks (clay, sandy and carbonate ones).

The upper mantle is of great importance. All ore minerals and the majority of non-metallic minerals extracted from the earth's crust in due time have got there from the upper mantle or gradually, or as a result of eruptions. The transformation of a matter occurs exactly in depths of the upper mantle of the Earth. For example, diamonds are formed in the upper mantle under conditions of high pressures and high temperatures.

At the development of a general plan of a deposit in any territories (including Kazakhstan) first of all it is necessary to analyze the location of already operated deposits and their connection with the structure of the earth's crust. Exactly these factors influence on the location of all commercial deposits of minerals which have arisen on the sites of geosyncline and platform development. Geosynclinal and platform structures of the earth's crust are characterized by two forms of geological structures: folded (mountain) zones and wide cavities crossing them.

The geosynclines look like the arches coincident with Kazakhstan ancient mountain ranges. There are Big, Small and Western Kazakhstan arches.

The Big folded arch (from the north to the south) includes the following ancient mountain systems: Kokchetau-Shatskiye Mountains, Karsakpay-Ulutauskiye Mountains, Karatau Mountains, and North Tien Shan Mountains.

The Small folded arch is formed by Chingiz-Akchatau-Tarbagatay Mountains, Bulattau-Atasu-Yerementau Mountains and Chu-Ilyiiskie Mountains situated in the southwest from the Lake Balkhash.

The third ancient arch is the Western Kazakhstan arch. Its beginning is situated in Zauralskiye mountains, then it disappears in the south under covering of the Northern Priaral’ye, passes presumably through the east side of Aral sea and unites with Southern Tien Shan mountains in Uzbekistan.

The extent of these three arches, or mountain ranges, is many hundreds and thousands kilometers, and their width from 100 up to 270 kilometers. Between these folded zones wide cavities and hollows are situated. They are filled in with the strata of many kilometers of the volcanic and sedimentary rocks that are younger than the rocks of the Kazakhstan ancient mountain systems.

The widest of them is the Balkhash cavity. Its width is about 270 km. Lake Balkhash is in the middle of it. Along its boards, in junctions with the Small arch there are powerful systems of faults in the earth's crust.

To the west of the Balkhash cavity, between the Big and Small arches the Chu-Sarysu-Teniz cavity is situated. It is strongly extended in the meridional direction and filled in with recent deposits.

More to the west, the enormous Turgayskiy deflection is located. In the southeast it turns into Kyzyl Kum and Fergana-Naryn cavities.

Both the systems of cavities run parallel to each other more two thousand kilometers at the width of 150-200 kilometers. To the west of the third Kazakhstan folded arch there is the Irgiz deflection; in the southeast it passes into the Bukhara-Tadjik cavity.

If to look at a metallogenic map of Kazakhstan we can see that along the internal borders of the Small folded arch there are deposits of copper, lead and zinc (Sayak, Kounrad, Gul’shad, Aksoran, Kzyl-Espe, Akzhal, Uzunzhal, Alaygyr, Kayrakhty, Uspenskoye, Akchagyl, Kodzhanchad, Tortkuduk, Boshekul, Maykain, Aleksandrovskaya group, Jusaly, Mizek, Akbastau, Kusmurun and others). All these deposits have been grouped in ore formations or families.

Ores of the majority of the deposits besides basic elements – copper, lead and zinc – contain gold and molybdenum. For this reason this ore zone is named The copper-gold-polymetallic belt.

To the north of Lake Balkhash this arched copper-gold-polymetallic belt is divided by two smaller ore belts into Uspenskaya and Tekturmasskaya polymetallic zones. The Uspenskaya zone includes: Karagailinskoye barite polymetallic deposit, Uspenskoye copper deposit and a small iron deposit Kentyube-Togay. The Tekturmasskaya zone is famous for copper and nickel-cobalt ores.

All deposits of The Belt occur in rocks of various structure and age. It’s amazing that the largest Kazakhstan coal deposits are territorially matched with The copper-gold-polymetallic belt. There are Karagandinskiy, Ekibastuzskiy, Maikubenskiy and Teniz-Korzhunkulsky coal basins.

Around The copper-gold-polymetallic ring of the Central Kazakhstan there is enormous The gold rare-metal iron manganese ring that coincides with the Small arch. Almost all endogenic ores of gold, tungsten, molybdenum, tin, bismuth, nickel, cobalt, tantalum, niobium, iron and manganese are bedded in this external belt of the Central Kazakhstan.

In addition, this external enormous belt contains exogenic deposits: sedimentary ores of iron and vanadium, phosphorites, bauxites, placers of titanium, zirconium, gold and asbestos.

In the east of Kazakhstan The gold rare-metal belt is located in the Chingiz Akchatau Tarbagatay Mountain range. To the east it includes Charskiye and Kalba-Narymskiye Mountains and borders directly with Rudny Altay polymetallic zone. All listed zones together are parts of the powerful Akchatay Chingiz Kalbinskiy gold-rare-metal ore zone.

Other half of The gold rare-metal belt is situated along Niaz-Yerementau Mountains, then – Atasu-Bulattau Mountains and further to the southeast – Chu-Iliyskiye Mountains. Gold-ore, iron-ore and rare-metal deposits are mainly placed here. They are compactly bedded in the territory of Kokchetau and Akmola Oblasts. It is The Northen Kazakhstan gold rare-metal iron-ore belt. To the south it turns into The Central Kazakhstan tungsten-molybdenum-iron-manganese belt. Iron and manganese deposits are placed within limits of this belt. In the northeast these are brown haematite of Pavlodarskoye Priyirtyshye, in the west – Atansorskaya group of iron deposits, in the south – Atasyskaya group of iron-manganous deposits. Further to the south the Central Kazakhstan tin-ore belt is placed.

The Kazakhstan part of the Big arch includes deposits of iron, manganese, bauxites, nickel, cobalt, asbestos, vanadium slates, phosphorites, tungsten, molybdenum, gold and The Western tin-ore belt. Chu-Sarysu-Teniz  and Issyk-Kul cavities, located between the Small and Big arches, contain copper-gold-polymetallic ores.

Fig. 7 – Kazakhstan minerals: 1 – hair stone (quartz) and rock crystal; 2 – jasper

The Turgayskiy Deflection. Copper and polymetallic deposits are placed here. So, at the foot of Karatau Mountains there is The lead zinc belt (deposits Shalkiya, Achisay, Mirgalimsay, Smena, Bayjansay group and others). On the southeast continuation of this belt there is other large zone – the copper-gold-polymetallic zone containing fluorspar (calcium fluoride) (Almalyk, Altyntopkan, Chalata, Kurgashinkan, Okurtau, Kansay, Kurusay, Lashkerek, Gudas and other deposits). This copper-polymetallic zone goes on the southeast up to Chatkalo-Narynskiy border deflection.

Inside the enormous zone of deflections and cavities the copper and polymetallic ores are strongly attached by fractures of border deflections and border junctions. These are deposits Karatau and Northern Tien Shan and fields of the Fergana-Nuratinsk zone.

It is necessary to note, that almost on all extent of the cavities and deflections, nearing to their border fractures, very large coal and gas-oil deposits, and also deposits of pyrite, gypsum and mineral salts are located. These are: Turgay coal belt, Bolattam pyrite area, large coal fields Angren, Uzgen, Kyzyl-Kiya, Kok-Yangak, Tash-Kumyr and others.

The Western Arch: Zauralskiye and Southern Tien Shanskiye ancient mountain systems. Zauralskiye Mountains include:

- deposits of gold, nickel and cobalt, bauxites, placers of titanium-zirconium ores and other rare metals (Jetygara, Krasny Oktyabr’, Buryktalskoye, Shevchenkovskoye, Shortandy and others);

- Main iron-ore zone of the Kustanay field. Its length is 800 kilometers, width 50 kilometers. It includes deposits of Sokolovsko-Sarbayskoy, Kacharskoy and other groups, and also Ayatskiye and Lisakovskiye brown haematites;

- Western iron-ore zone that is parallel located to the west of the Main iron-ore zone and extended on many hundred kilometers. At the southern end of this zone there is the Priaral’skaya group of brown iron ore (Kokbulak, Kutanbulak, Taldy-Espe and phosphorite deposits.

Zauralskie Mountains disappear in the northern Priaral’ye; their continuation is approximately established on the basis of results of geophysical researches. Scientists surmise the arched jointing of Zauralskiye Mountains with Southern Tien Shan Mountains. Probably, this jointing passes along the east coast of Aral Sea through the large Bel’tauskaya zone of crushing of rocks. Here deposits of chromite (Jayran, Northen Nuratau, Tamdytau), nickel ores, copper-magnetite ores are located. Tamdytau deposit contains commercial gold reserves.

Fig. 8 – Kazakhstan minerals: 1 – cozalite impregnated in quartz; 2 – calcite

Southern Tien Shan Mountains contain the same kinds of minerals:

- in the northwest: Nuratinskaya zone of molybdenum and tungsten (Lyangar, Khoytash) and gold-ore deposits (Muruntau, Sintab, Davlyat Khoja);

- in the south-east: Zirabulak-Karatyubinskaya zone of tungsten, tin and other rare metals (Ingichka, Karatyube, Karnab);

- Karatau-Turkestan tin-ore zone;

- in the east: Fergana-Kokshaal’skaya zone including tungsten and arsenic deposits, deposits of tin and molybdenum and also arsenic with cobalt and gold (Meliksu, Maykhura, Sarymat, Kaznok, Mosrif, Takfon). Numerous mercury deposits are also placed here.

Fig. 9 – Kazakhstan minerals: 1 – aragonite (Abail deposit, South Kazakhstan); 2 – malachite

In the west, Zauralskie Mountains border with Irgizskiy Deflection, and Southern Tien Shan mountains border in the south with Bukhara-Tajik cavity. Along their joints the earth's crust has deep cracks. Along these cracks there are deposits of copper, lead and zinc.

Irgizskiy Deflection has high prospects from the point of view of discovery of large copper-pyrite deposits, and Bukhara-Tajik – polymetallic and copper deposits. Inside the Deflection contains deposits of coal, oil, gas, sulfur, salts, gypsum and cuprous sandstones.

A share of Kazakhstan mineral raw materials in the world resources. Categories of raw material reserves

The Kazakhstan ore base is one of the most extensive on the global raw-materials market. Kazakhstan is included into the first six countries in the world in accordance with the quantity of kinds of extracted raw material. Only Russia, China, Australia, the USA and Brazil have the greater number of mined kinds of minerals.

Table 1 – Quantity of extracted minerals

| Country | Russia | China | Australia | The USA | Brazil | Kazakhstan |
| --- | --- | --- | --- | --- | --- | --- |
| Quantity of minerals | 49 | 45 | 42 | 42 | 41 | 39 |

Presence of sufficient and polytypic natural raw materials serves a reliable guarantee of independence of the republic in the area of mining, metallurgical, chemical industries, industry of building materials, development of the economy; it represents ample opportunities for export of raw material and finished goods to CIS countries and far abroad.

From the global explored mineral reserves Kazakhstan owns 83 % of tungsten, 25 % of uranium, 23 % of chromium-containing ores, 25 % of barite, 19 % of lead, 18 % of zinc, 10 % of copper, 9 % of gold, 6 % of silver. Kazakhstan is the first on reserves of V, W, Zn, the second - U, Cs, the third - Mn, Re, Ba, wollastonite, the fourth - Be, Nb, Mo, Pb, the fifth - Cd, the sixth - phosphorites, the seventh - vermiculite, B, Fe, Ag, the ninth - oil, coal, gold, copper.

On a commercial scale Kazakhstan possesses natural reserves of 3 ferrous metals, 29 – non-ferrous metals, 2 – precious metals, 84 kinds of industrial minerals, and also 3 hydrocarbonic energy carriers.

Now Kazakhstan is the largest manufacturer of rhenium (1-2 place in the world), beryllium (1-4 place), iron-chromic, chromic, titanium ores (2 place), ferroalloys, tantalum, niobium, astatine, gallium, thallium, arsenic (3 place), uranium (4 place), vanadium (5 place), bismuth (6 place), copper, zinc, cadmium, sulfur, boron (7 place), magnesium (8 place), alumina and silver (9 place), lead (10 place), coal, iron ore (11 place).

For objective economic-geological evaluation of deposits, designing of mining and concentrating enterprises it is necessary to estimate mineral reserves in Kazakhstan interior and to take stock of them in the state balance.

With a view of rational development of deposits it is necessary to assess the presence of other minerals that occur together with a basic element. The stock-taking is implemented according to a total mass of useful minerals and a quantity of minerals, metals, chemical elements and their compounds being in them.

Reserves of minerals are divided into the following categories:

- according to a study degree: categories A, B, С1, С2;

- according to a complexity of a geological structure;

- according to industrial value: balance and off-balance.

Category A includes the reserves quite established and probe-tested by detailed geological exploration.

Category B includes the reserves which quantity was precisely established by geological exploration, but borders of ore bodies were established insufficiently clear.

Category C includes the reserves revealed due to natural exposures and geophysical data.

The category of natural minerals is determined by the State commission of reserves. Belonging of ore reserves to A and B categories is a good reason for designing, building and operation of mining and smelting enterprises using these ores. Belonging of useful minerals to C category is a basis for substantiation of perspective industrial planning and fund allocation for the exploration work. Category C is divided into categories С1 - insufficiently studied reserves and С2 – expected (probable) reserves.

Balance reserves include the minerals which use, according to approved standards, is economically expedient at the existing or developed industrial technologies and observance of requirements on rational use of resources and environmental protection.

Off-balance reserves are the resources which use according to approved standards is now economically inexpedient or technically and technologically impracticable.

Characteristic of ferrous metals resources

Iron ores are rocks containing more than 25-30 % of elemental iron. Main minerals of iron ores are:

- magnetite (magnetic ironstone) Fe3O4 (72,4 %);

- hematite Fe2O3 (70 %);

- siderite FeCO3 (48,3 %;

- limonite (brown haematite) Fe2O3∙nH2O (48-63 %).

According to the Geological service of the USA the resource potential of iron ore in the world makes 800 billion t. The explored reserves are 150 billion t of iron ores which contains approximately 70 billion t of iron. The base of iron ores makes 330 billion t, and elemental iron in them – 160 billion t.

Basic resources of iron ores in Kazakhstan include 19 billion t. According to this parameter the country ranks the 7th place in the world to Ukraine, Russia, Australia, China, the USA, and Brazil and the 3rd to Russia and Ukraine in the CIS. Kazakhstan holds the 11th place in the world on the iron ore mining (17,7 million t per a year) (table), and the 9th place on its export. Main importing partners of Kazakhstan iron ores are Russia and China.

30 deposits of iron ores have been explored in Kazakhstan. The basic iron-ore reserves are concentrated in Karaganda and Kustanay Oblasts. The less significant resources are located in Kokchetau, Turkestan Oblasts and Northern Priaral’ye.

Fig. 10 – An iron ore

Central Kazakhstan (about 250 km to the southwest from Karaganda) contains Atasuskiy iron-manganese basin. This is a sedimentary-type deposit. It is stretched as a chain along the Zhakl’minskaya syncline. The deposit is located above the ground or at the small depth. Here iron and manganese ores are bedded jointly. The largest deposits: Western Karazhal, Eastern Karazhal, Bol’shoy Ktay. On the largest deposit – Western Karazhal (it was opened in 1951 by Nikolayev I.G.) – iron content in the ore makes 50-55 %, and manganese up to 25 %. The Bol’shoy Ktay ores are poorer (to 32 % of iron). On the basis of these deposits there are mines for mining of the ores. Deposit Western Karazhal contains 300 mil t of the ores, Bol’shoy Ktay about 25 mil t. The ores of deposit Bol’shoy Ktay contain to 36 % of iron.

Kustanay iron-ore basin is located in Bol’shoy Turgayi. About 85 % of explored resources of Kazakhstan iron ores are concentrated there. Three deposits – Kacharskoye, Sarbayskoye and Sokolovskoye contain 3,5 billion t of iron ores. In addition, Lisakovskoye and Ayatskoye – deposits of phosphorous brown haematite containing approximately 10 billion t of the mineral are also located there. Sarbayskoye and Sokolovskoye deposits were formed by volcanic sedimentary rocks; average iron content in the ores is 47-48 %. The depth of occurrence is to 400 m. Both deposits have been exploiting since 1954 by the open way. Ores of Kacharskoye deposit contain up to 45 % of iron. The depth of occurrence is 135-200 m.

Fig. 11 – A steel plant

Ayatskoye deposit of iron-phosphorous ores is located in Kustanay Oblast. The average depth of occurrence of the ore stratum is 30 m. The ore contains phosphorus, vanadium. Perspective reserves of these ores make approximately 10 billion t; the average iron content is 36 %. The ores are easy-concentrated up to 52-60 % of iron.

Lisakovskoye deposit is less than Ayatskoye deposit according to the reserves, but the ore quality is higher (37 % of Fe). The general reserves of the deposit (expected) make 6 billion t, and the industrial ones – 2,9 billion t. The bed formation thickness is 20 m and its extent is approximately 100 km. The ore stratum everywhere outcrops (thickness of overburden is 3 m). On the base of this deposit the Lisakovskiyi ore dressing plant was constructed. The ore is complex, contains phosphorus (to 0,5 %) and vanadium.

Korsakpayskiy iron-ore basin is located in Ulutau region of Karaganda Oblast. The ore type is ferriferous quartzite. The basin includes 14 deposits with general resources approximately of 500 mil t. The largest deposits are Keregetas and Balbraun.  The ores of these deposits contain 37-42 % of iron and 30 % of SiO2. The established reserves are 200 mil t. The ores can be concentrated to 56-60 % of Fe.

Information about other Kazakhstan iron-ore deposits is represented in the table 2.

Table 2 – Kazakhstan iron-ore deposits

| Deposit | Location | Resources | Fe content, % |
| --- | --- | --- | --- |
| Loschinovskoye | Priaral’ye | Perspective – 4,5 billion t | 35-37 |
| Kokbulak | Northen Priaral’ye | Explored – 1,3 billion t | 38-39 |
| Kentyube | Karaganda Oblast | Explored – 0,15 billion t | 55-60 |
| Atassor | Kokchetav Oblast | Explored – 0,1 billion t | 42-46 |
| Irisu | South Kazakhstan Oblast | Explored – 0,58 billion t | 47-49 Cu – 0,8-1 % |
| Abail | South Kazakhstan Oblast | Explored – 0,05 billion t | 47-48 |
| Tlegen | Akmola Oblast | Expected – 0,2 billion t | 36-38 |

Besides iron, manganese Mn, chrome Cr and vanadium V are also considered ferrous metals.

Manganese. The main property of manganese, which provides its wide application in metallurgy, is its ability to add viscosity, forgeability and hardness to steel. Besides, addition of manganese at melting of ores promotes the extraction of harmful impurities in a slag (a waste product). Modern metallurgy is inconceivable without application of manganese. 90-95 % of all extracted manganese ores are used in metallurgy. Other part of manganese (5-10 %) is used mainly in chemical industry.

Kazakhstan has large deposits of manganese ores. The largest deposits of manganese are Atasusky and Zhezdinsky located in the Central Kazakhstan. Manganese content in these ores reaches 27 %. Manganese deposits also have been found out in Sary-Arka.

Kazakhstan manganese ores are basically ores of an iron-manganese type. Sedimentary iron-manganese ores of the Atasuskaya group (Karaganda Oblast) are of exceptional interest. The Atasuskaya group of iron-manganese deposits contains million tons of industrial iron-manganese ores. This ores have the same content of iron as iron ores of Krivoy Rog and Ural.

A significant quantity of manganese (more than 90 %) is used at production of manganese steel. Each ton of the steel contains 7-9 % of manganese. In this case manganese is a deoxidizer and an alloying element. Kazakhstan holds the 3rd place in the world on resources of manganese ores (429 mil t) after the Republic of South Africa (1,05 billion t) and Ukraine (650 mil t). The general world reserves of manganese ores make 23 billion ton. At present Kazakhstan holds the 5th place in the world on extraction of manganese ores (after Ukraine, China, the Republic of South Africa, Australia). Properties of Kazakhstan manganese ores is represented in table 3.

Table 3 – Properties of Kazakhstan manganese ores

| Deposit | Mn content, % | Resources, mil t |
| --- | --- | --- |
| West Karazhal | 21-29 | 285 |
| Eastern Karazhal | 22-32 | 13,2 |
| Bol’shoy Ktay | 23-26 | 2,8 |
| Ekutort | 19-22 | 7,0 |
| Tur | 21-25 | 10,4 |
| Ushkatyn III | 29-40 | 105,0 |

There are 19 deposits of manganese ores in Kazakhstan.

Fig. 12 – A manganese ore

Chrome. Reserves of chrome ores in Kazakhstan are 316 mil t; it makes 19 % of the world reserves. On the mining of chrome ores (2,4 mil t) Kazakhstan is the 3rd in the world after the Republic of South Africa (6,1 mil t) and India (2,8 mil t). The basic part of chromium-containing ores is used for production of ferrochromium applied at steel making.

Chromium in the ferrous metallurgy is used as an alloying (improving) additive for production of high-grade steels. This metal adds viscosity, raises their hardness and anticorrosive properties (corrosion-proof, acid-resistant, heat-resistant, high-speed steels).

The unique natural chromium-containing ore is mineral chromite; this mineral is found almost exclusively among serpentines and serpentinites – rocks of dark green color.

For the first time chromite was started to be applied in 1820 in Norway as a fire-resistant material. The main application area of chromite is metallurgy (about 50 %). Besides, chromite is applied in the chemical and fire-resistant industries. Application of chromium allows us to produce its valuable alloys with nickel, cobalt, aluminum, it is a component of special alloys – stellites (chromium with cobalt and tungsten or molybdenum); it is applied as a covering (chromium plating).

Chromite (in its natural kind or in artificial products) is a fire-resistant material. It is used as a cover in Martin furnaces and furnaces for smelting of nonferrous metals.

In the chemical industry chromite serves as a raw material for manufacturing paints and a tanning agent (potassium bichromate, chromic alum).

99 % of chrome deposits are found in Mugodzharsky Mountains. The largest Kazakhstan chromite deposit is Donskoye (Aktyubinsk Oblast). Owing to high content of chromium oxide these chromites are the best ones in the world. Chromium oxide content in the ores of Donskoye deposit makes 29-52 % (North Kempirsay group) and 20-63 % (South Kempirsay group). Here, on the base of the chromite ores the Aktyubinsk factory of ferroalloys operates, which produces high-quality alloyed steels.

The Donskoye deposit borders upon the Central group of chromite deposits (Kustanay Oblast). The ores of this group are characterized by a lower content of chromium oxide and high content of alumina therefore they are hardly applicable for manufacturing ferroalloys. However, they can be used for potassium bichromate and chromic alum production.

Vanadium – is a plastic metal of a silvery colour; it is one of the ferrous metals. Its basic advantage is: vanadium improves steel quality. Only one kilogram of this metal added to one tonne of steel turns it into alloyed steel. Vanadium is a necessary component for manufacture of chromium-vanadium steel, titanium-vanadium-based heat-resistant alloys. It is a constituent of aluminum-vanadium alloys applied in shipbuilding and aircraft construction. In a word, the spectrum of vanadium application in ferrous metallurgy is very wide. Vanadium in a pure state practically is not applied. Even in the modern, so-called vanadic accumulators this metal is used as its oxide.

Vanadic ores are represented by complex titanium-magnetite, sedimentary brown haematite-leptochlorite, placer (ilmenite-rutile and magnetite-zircon-ilmenite), infiltration and copper-lead-zinc deposits. In addition, vanadium ores form native deposits in black shale strata. The titanium magnetites (Mugodzhary, Zaural’ye) contains 0,1-0,6 % of V2О5, the vanadium oxide content in the brown haematite (Ayatskoye, Lisakovskoye and other deposits) is lower. Vanadium-bearing black shale (coaly-siliceous-clay) strata have been found in Northwest Karatau, Talassky Alatau, Ulutau, Dombralytau in Western Balkhash. Vanadium-bearing strata in the Talassky Alatau concern to the Cambrian coaly-siliceous-shale formation. The vanadium-bearing strata of the Talassky Alatau are a part of the Cambrian coaly-siliceous-clay formation. Content of V2О5 in these rocks reaches 3,64-3,93 %. In the Balasauskandyksky ore field (Northwest Karatau) the V2О5 content in some interlayers of the coaly-siliceous-clay shales of the vanadium-bearing strata reaches 1,22 %. Vanadium reserves of the Cambrian shales in Kazakhstan are very considerable, but they are not involved in operation.

In the Karatau Mountains in Shiyeli region the LP “Balausa” has been developing the deposit Balasauskandyk from 2006. The deposit was opened during the Soviet time; at the very beginning of the ninetieth the manufacture of ammonium metavanadate has begun, but the deposit was closed after the first delivery to a consumer to the Ural. Till 2006 this largest Kazakhstan deposit of vanadic ores has been forgotten. Now the mentioned LP develops this deposit. These ores are hard-concentrating ores; therefore they were concerned to off-balance reserves. At present the ores on the given deposit are extracted using heap leaching.

Vanadium contains also in bauxite deposits and ilmenite-magnetite ores of the Velihovsky-Southern deposit. The vanadium content in the ores of these deposits is sufficiently low and the possibility of its extraction is insignificant. At the same time, the Kurumsak vanadium-containing shale deposit (the vanadium content is 0,9 %-1,0 %) was explored in Southern Kazakhstan. Involving of these fields in operation is inhibited by the absence of a comprehensible benefication technology; in this connection now these reserves are considered as off-balance resources.

Vanadium is widely used in ferrous metallurgy, transport and heavy engineering industry, agriculture, medicine, textile, glass and ceramic industries.

Nonferrous metals

Nonferrous metals are divided into subgroups.

Table 4 – Classification of nonferrous metals

| Nonferrous metals |  |  |  |
| --- | --- | --- | --- |
| Heavy nonferrous metals | Light nonferrous metals | Noble metals | Rare metals |
| Basic: copper Cu, lead Pb, zinc Zn, nickel Ni, tin Sn + mercury Hg, cobalt Co, antimony Sb, cadmium Cd, bismuth Bi, arsenic As | Basic: aluminium Al, magnesium Mg + lithium Li, beryllium Be, sodium Na, potassium K, silicon Si, calcium Ca, barium Ba, rubidium Rb, caesium Cs | Gold Au, silver Ag, platinum Pt, osmium Os, iridium Ir, rhodium Rh, ruthenium Ru, palladium Pd | Refractory metals: tungsten W, molybdenum Mo, titanium Ti, tantalum Ta, niobium Nb, zirconium Zr, hafnium Hf Rare-earth metals: scandium Sc, yttrium Y, lanthanum La, lanthanides (17 elements) Trace-rare metals: gallium Ga, indium In, thallium Tl, germanium Ge Radioactive metals: radium Ra, uranium U, polonium Po, thorium Th, actinium Ac and actinoides, trans-uranium elements |

Copper ores

Copper was known to mankind in ancient times and was widely applied in a pure state and in alloys.

Copper is applied in electro industry and mechanical engineering, at making of power lines, in chemical industry, to manufacturing of steam boilers, refrigerators, coils, pipes, in shipbuilding, for stamping of coins (as copper-nickel alloy), for various household purposes as alloys with tin and aluminium, with zinc and nickel (nickel silver and German silver), with zinc (brass).

Copper ores are developed if they contain 1-2 % of copper and more, but for the very large deposits of, so-called, impregnated ores the minimal copper content acceptable for the development is 0,5 %.

There are more than 150 minerals containing copper. Copper sulfides are ranked as the most important copper mineral having the main industrial value.

Kazakhstan possesses huge resources of copper ores. Main industrial types of the ores are cuprous sand rocks (71 %) and copper porphyritic ores (24 %). The largest deposit of cuprous sand rocks is Jezkazgan.

Kazakhstan ranks ninth in the world on copper reserves (14 mil t). First 8 countries are located in the line: Chile (88 million ton), the USA (45 million ton), Poland (20 million ton), Russia (20 million ton), Peru (19 million ton), Indonesia (19 million ton), China (18 million ton), Mexico (15 million ton). Extraction of copper ores in Kazakhstan makes approximately 38 million ton per a year (the seventh in the world). 92 % of copper are exported to foreign countries.

The basic part of copper is concentrated in Central Kazakhstan. 4 main deposits are located here: Jezkazgan, Kounrad, Boschekulskoye and Sayakskoye. The ores are complex. The Kazakh word “jez-kazgan” means “there, where copper is dug out”.

Information about Kazakhstan copper deposits are represented in Table 5.

Table 5 – Information about Kazakhstan copper deposits

| Deposit | Content | Bedding depth | Ore type |
| --- | --- | --- | --- |
| Jezkazganskoye | Cu 1,5 – 1,6 % Pb 0,4 – 0,5% Zn 0,4 – 0,7 % | 500 m | Copper-lead sulphide |
| Kounradskoye | Cu 0,6 - 1,1 % | 1 km | Copper-molybdenum sulphide |
| Boschekulskoye | Cu 0,5 – 1,6 % Mo 0,2 – 0,3 % |  | Copper-molybdenum sulphide |
| Sayakskoye | Cu 0,8 – 1,2 % |  | Copper sulphide |

Jezkazgan copper deposit represents thin strata of a rich copper ore overlying one after another. The copper ores are prevalent up to the depth of 200 meters; deeper ore strata contain lead, silver and other metals. Ores of Kazakhstan copper deposits lie superficially. It enables to apply the open-cast mining.

Kounrad deposit. The deposit is placed in the stone-clay semidesert to the north of Lake Balkhash in 18 kilometers from Balkhash town. A large mine for extraction of the ore from open pits is located there.

Boschekulskoye deposit. In the northeast part of Kazakhstan between Astana and Pavlodar a geologist Borukayev R.А in 1930 opened the hugest deposit of impregnated copper ores Boschekul. It is situated quite near to the Ekibastuz coal deposit. This is a positive factor for copper manufacturing. Except for copper the given ores contain cobalt, gold, silver, arsenic, a plenty of pyrite and other elements.

Other Kazakhstan copper deposits:

Akbastau and Kusmurun – copper-pyrite-gold-polymetallic ores (Au - 10-14 g/t)

Nikolaevskoye – a copper-zinc-pyrite deposit (Cu:Zn:Pb = 1:2,2:0,11)

Orlovskoye – a copper-pyrite-polymetallic deposit

Irisuyskoye – a copper-magnetite deposit (0,3-0,9 % of Cu).

Other two main nonferrous metals - lead and zinc - are usually found together forming, so-called, polymetallic ores. Except for lead and zinc, polymetallic ores contain copper, gold, silver, sometimes antimony, arsenic, bismuth, cadmium, indium, gallium and germanium.

Lead is mainly applied in accumulator and cable industry, as a component of typographical and bearing alloys, defense industry, X-ray equipment.

Zinc is applied in the alloys (brass, bronze) used in mechanical engineering, for covering of iron (zinc plating), for manufacture of zinc whitewash.

Usually the deposits which ores contain 3-5 % of lead and more are subjected to development. However, at present in connection with deficiency of the metal, large deposits of complex polymetallic ores containing not less than 1 % of lead and not less than 2-3 % of zinc also are of industrial interest.

On zinc reserves Kazakhstan ranks first in the world (34,7 million ton), and on zinc extraction – seventh.

Table 6 – Reserves and extraction of zinc ores in the world

| Country | Kazakhstan | China | Australia | USA | Canada | Peru | Mexico |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zn reserves, mil t | 34,7 | 34 | 32 | 25 | 11 | 8 | 6 |
| Zn extraction, mil t | 0,4 | 1,5 | 1,1 | 0,74 | 1,0 | 1,1 | 0,47 |

According to Geological service of the USA, Kazakhstan on lead reserves ranks fourth (5 mil t) to Australia (15 mil t), China (11 mil t), the USA (8,1 mil t). In accordance with the lead extraction countries are in the following order:

- Australia - 715 thousand ton

- China - 650 thousand ton

- The USA - 450 thousand ton

- Peru - 295 thousand ton

- Mexico - 140 thousand ton

- Canada - 100 thousand ton

- Morocco - 80 thousand ton

- KDPR (Korean Democratic People’s Republic) - 60 thousand ton

- Poland - 55 thousand ton

- Ireland - 55 thousand ton

- Spain - 50 thousand ton

- Kazakhstan - 44 thousand ton

- Sweden - 36 thousand ton

- Greece - 30 thousand ton

Basic resources of lead and zinc are located in the Eastern Kazakhstan, South Kazakhstan, Karaganda and Almaty Oblasts and concentrated in 66 deposits. Main lead-zinc deposits:

- Eastern Kazakhstan Oblast – Zyryanovskoye, Leninogorskoye, Tishinskoye;

- Taldy-Kurgan Oblast – Tekeliyskoye;

- Kyzyl-Orda Oblast –  Shayakshinskoye;

- South Kazakhstan Oblast – Bayjanasay, Achisay;

- Central Kazakhstan – Bestyubinskoye, Alaygyrskoye, Zhayremskoye, Kayraktinskoye, Karagaylinskoye, Uzunzhalskoye.

Fig. 13 – Tin stripping operations will begin in May at Syrymbet, Central Asia’s largest and Kazakhstan’s only tin deposit with an annual capacity of two million tonnes of ore

Nickel. Considerable nickel resources are concentrated in Mugojary. More than 40 large deposits are located in the northeast of Saryarka (Astana surroundings) and in the Turgaysky deflection (Amangeldinsky group).

Aluminium. A foundation of the mineral raw-material base of aluminium industry is reserves of difficult-to-process bauxites of the East Torgay bauxite-bearing basin (deposits Krasnooktyabrskoye, Ayatskoye, etc.). Firstly these bauxites were considered to be unsuitable for manufacture of alumina in accordance with the existing technological scheme, but engineers of the Pavlodar aluminium factory developed a new technological scheme that allowed them to use the low-grade bauxites for production of alumina. Improvement of the processing technology of poor-quality bauxites will permit us to expand the mineral raw-material base of aluminium industry at the expense of explored deposits. Manufacture of bauxites and marketable aluminium production (such as alumina) fully belongs to OJSC “Aluminium of Kazakhstan”. Prospects of development of the raw-material base of aluminium industry are now connected, first of all, with nonconventional kinds of aluminous raw materials. Nepheline-leucite and nepheline rocks and also alunite secondary quartzites are of the greatest interest to this purpose.

Gold mining in Kazakhstan

Total Kazakhstan gold resources are approximately estimated in 800 t. The State balance of the Republic of Kazakhstan takes into account gold resources of 237 deposits, from which 122 – primary deposits, 81 – complex deposits and 34 – placer deposits. Gold deposits have been found in all regions of Kazakhstan. The Eastern, Northern and Central Kazakhstan hold a leading position. Gold-ore and gold-containing deposits are located in 16 mining areas.

Main deposits:

- Kalbinsky and Rudno-Altaysky in the East Kazakhstan (deposits Bakyrchik, Bol’shevik, Ridder-Sokol’noye and others);

- Kokshetausky and Zholymbet-Bestobe in the Northern Kazakhstan (deposits Vasil’kovskoye, Zholymbet, Bestobe and others);

- Shu-Iliysky and Jungarsky in the South Kazakhstan (Akbakay, Beskempir, Arharly);

- Maykalinsky and Northern Balhashsky in the Central Kazakhstan (Maykayin, Boshekul, Sayak IV, Dolinnoye and others);

- Zhetygarinsky and Mugojarsky in the Western Kazakhstan (Zhetygara, Komarovskoye, Yubileynoye and others).

Kazakhstan gold-ore deposits include 2 unique deposits, 1 large deposit, 11 average deposits, other – fine.

In comparison with the gold-ore resources of the world, complex deposits in Kazakhstan are of importance both in the gold reserves and in the gold mining; and gold-ore deposits proper are found very seldom. In addition, under the conditions of the unfavorable world gold conjuncture the presentation of more rigid requirements to qualitative and quantitative parameters of the deposits is inevitable.

Basic problems of the raw-material base of the gold-ore industry in Kazakhstan:

- absence of large reserve gold-ore deposits proper;

- more than 50 % of active ore reserves are complex (hard-dressing) and contain harmful impurities (arsenic, antimony);

- available gold reserves do not provide the predicted level of gold production;

- in case of preservation of the unstable condition of the world gold market and further drop in prices the development of some deposits, including the large deposits, will be unprofitable;

- the opportunity of expansion of the gold raw material base due to complex deposits is limited by absence of prepared reserve deposits.

Solving of these problems is impossible without search and investigation of new gold-containing deposits and competitiveness in the modern conditions.

At favorable conjuncture and preferential taxation some additional quantity of gold can be extracted on numerous fine Kazakhstan deposits.

Geological industrial types of Kazakhstan gold deposits

1. Quartz vein type – high-concentrated easy-dressing ores which are a basic object of the gold mining.

2.  Network type – average gold content in such ores is 3,6-7,9 g/t; the ores are characterized by various dressability depending on gold forms (sulphide, carbonate).

3. Type of mineralized zones – gold is mainly connected with sulfides, free gold is found seldom, the ores are hard-dressing, gold content is 4,8-9,4 g/t.

4. Complex gold-containing deposits (pyrite-yellow pyrite, pyrite-yellow pyrite-sphalerite-galenite ores).

The basic part of gold reserves and gold mining falls on the deposits of Rudno-Altaysky basin (Ridder-Sokol’noye, Tishinskoye, Belousovskoye, Nikolayevskoye).

The part of exogenic gold resources (placers and gold-bearing residual soils) is only 2 % of active reserves. Placers are mainly located in mountain ranges of the Eastern and Southern Kazakhstan (Southern Altay, Kalba, Tarbagatay, Jungarsky and Zailiysky Alatau, Karatau). They are characterized by small sizes and reserves; average content of gold is 600-700 mg/m3. Deposits with gold-bearing residual soils are readily available for their mining and can be developed by the open way using simple technologies of gold mining (gravitation and heap leaching). Deposits of such the type are located in Semipalatinskoye Priirtyshye: Suzdal, Zherek, Zhanan, Mirazh, Zhetygarinsky region (Elevatornoye, Akhkarginskoye, Komarovskoye). Gold occurrence depth is 40-50 m, its content 1,5-10 g/t.

The raw-material base of Kazakhstan gold mining includes basically fine (to 25 t of gold) and average (from 25 to 100 t) deposits; 70 % of the gold produced now in the Republic was mined on such the deposits.

Gold production

Today Kazakhstan is not a big manufacturer of gold as China, Russia and Uzbekistan; however, at present many small and average gold-containing Kazakhstan deposits are of quite interest to foreign investors who are capable to provide necessary technologies and capital.

Production of rare-earth elements in Kazakhstan

Rare-earth elements. Lanthanides (from La № 57 to Lu № 71), which are situated in one square of the Mendeleev periodic table, have the same structure of outermost electron shells that is a reason of similarity of their properties and their joint presence in natural minerals. Separation of lanthanides demands application of special high technological processes (repeated crystallization of salts, ion exchange, extraction, etc.). Rare-earth metals are more often produced by means of electrolysis of melted salts or metallothermic reduction. Scandium and yttrium have the similar properties and they are also referred to rare-earth metals.

Fig. 14 – Dependence of REE abundance on an atomic number

Rare-earth elements are of primary importance at the manufacture of materials for hi-tech spheres such as electronic and electro-optical branches, information technologies, biomedicine, protection of the environment, energy saving. They are used for manufacture of luminophors, industrial ceramics, catalysts for oil-refining and motor industry, superconductors, constant magnets, high-quality glass, fiber optics, oxygen sensor, lasers, and storage batteries with a long operation life for automobiles, cinema- and photo equipment. In addition, rare-earth elements have wide application in metallurgy.

Basic sources of rare-earth elements are minerals:

- bastnaesite – a yellow to brown mineral consisting of a fluoride and carbonate of cerium and other rare earth metals;

- turnerite (monasite) – a salt of orthophosphoric acid containing cerium and lanthanum;

- loparite – a grey-black to black mineral, chemical formula – (Na, REE)Ti2O6; from the Russian name for the Lapps, “Lopar”, inhabitants of the Kola peninsula, Russia;

- phosphorite – a sedimentary rock containing a high proportion of calcium phosphate. It is used in the manufacture of fertilizers;

- apatite – a widely occurring pale green to purple mineral, consisting of calcium phosphate with some fluorine, chlorine, and other elements. It is used in the manufacture of fertilizers.

In the world there are only a small number of the deposits of these minerals profitable for the development. Large deposits of turnerite are located in Australia, Brazil, China, India, Malaysia, the Republic of South Africa, Sri Lanka, Thailand and the USA. However, extraction and processing of turnerite were considerably reduced owing to both the presence of other kinds of raw materials and ecological reasons (because turnerite contains radioactive). Now bastnaesite deposits are the most important. The greatest part of the world bastnaesite resources is concentrated in China and the USA.

Fig. 15 – Arrangement of rare earth elements in Periodic Table

China possesses the largest resources of rare-earth elements (more than 40 % of the world reserves). According to data Industrial Minerals the CIS ranks second in the world on resources of rare-earth raw materials. Rare-earth elements in Russia are bedded on a loparite deposit of Kola Peninsula. Besides, rare-earth elements in Russia (particularly, yttrium) are located on Katuginskoye deposit. Deposits of rare-earth elements are available in Kirghizia on deposit Ak-Tyuz. The USA ranks third in the world on natural resources of rare-earth elements. A large bastnaesite deposit is Mountain Pass in California (now it is not used).

Kazakhstan carries out the works connected with studying of an element structure and assessment of a geochemical spectrum on more than 30 deposits. Received results testify that Kazakhstan uranium deposits are characterized by high content of rare-earth elements, rare metals and gold. The uranium deposits mainly contain easy lanthanides (lanthanum, cerium), rare metals (especially hafnium and antimony). High content of heavy lanthanides (ytterbium, lutecium) is observed on deposits Zaozernoye and Tastykkolskoye, which are a part of the Koksengir ore zone. High concentrations of samarium, lutecium, scandium, hafnium, antimony, zirconium, niobium, yttrium and europium have been found in some regions of the Grachevsky basin. Deposits of the Balhash zone also contain big quantities of rare-earth elements and rare metals: tantalum, hafnium and partly europium, ytterbium and lutecium. Shokkaragayskiye deposits contain a specific set of rare metals and rare-earth elements. Lanthanum, cerium, samarium, europium, ytterbium, lutecium, scandium, hafnium, sometimes gold have been found in the tin-ore zones of deposits Donetsk and Syrymbet, and samarium, ytterbium, scandium, hafnium and uranium on deposit Losevskoye. According to data of Institute of geological sciences, rare-earth elements are deposited on the titanium-rare-earth deposit Kundybay, rare-metal deposits Verhneye Kayrakty, Zhanet, Southern Zhaur, Koktenkol, Verhneye Espe, Yosor, Karasu, in the phosphorite massif Karatau and carbonatite deposits Dubravsky, Krasnomaysky and Barchinsky.

Fig.16 – Global REE production in 1950–2000

Taking into account that natural ore accumulation of any metals are not reproduced, the problem of effective and economic using of technogenic waste products of mining and smelting and chemical industries, which are characterized by high content of unrecovered valuable components, is very important. The wide application of technogenic waste products in Kazakhstan will allow us to increase resource-saving due to economy of expenses for investigation and mining of mineral raw materials, and also partly to solve the environmental problem. Among the technogenic waste products containing rare-earth elements it is necessary to note:

- phosphogypsum – the waste product formed at acid processing of phosphorite and apatite at production of mineral fertilizers;

- dump cakes formed at tungsten (wolfram) manufacture;

- products of uranium ore processing.

Content of rare-earth elements in these technogenic waste products makes 0,5-0,6 % that corresponds to some kinds of ore raw materials. In comparison with mineral raw materials a technogenic raw material is opened much easier due to its changed phase structure. For this reason, technologies of production of rare-earth concentrates from industrial wastes do not demand high capital expenses and can be rather simple realized.

Now the production of rare-earth elements in Kazakhstan is practically absent though there are all necessary preconditions for its creation and development. The Irtysh chemical metallurgical plant used to be a manufacturer of rare-earth metals in the Republic. Since 1999 the plant was divided into two parts and sold as separate lots: LP Irtysh Rare Earth Co. Ltd, which processes imported concentrates and produce rare-earth metals, and JSC “Irtysh Chemical Metallurgical Plant» producing niobium. However, LP Irtysh Rare Earth Co. Ltd operates with underloading in connection with absence of manufacture of rare-earth concentrates in Kazakhstan.

Now in the republic the production technologies of rare-earth concentrates from natural and technogenic raw materials are actively developed.

Kazakhstan, possessing the significant raw reserves containing rare metals, is a supplier of some intermediate products, concentrates, technical pure metals on the world market, but these deliveries make no more than 8-10 % of its potential.

Fig. 17 – These rare-earth oxides are used as tracers to determine which parts of a drainage basin are eroding

*** The potential of rare and rare-earth elements available in the country allows the Kazakhstan companies to solve the question about their supply with domestic raw materials.

In the republic over 600 ore occurrences of tantalum, niobium and also rare-earth elements of the yttrium and cerium groups, connected with magmatic intrusive and sedimentary rocks, have been found.

Tantalum, niobium. According to a ratio of these two elements all the deposits are subdivided into three groups: tantalum, tantalum-niobium with REE, zirconium and hafnium and niobium proper. The deposits connected with rare-metal granite pegmatites and also with hatchettolite carbonatites cover the first group. A ratio of the pentoxides in these ores is different. So, Nb varies from 3:1-1:1 to 1:5. Concentrates contain from 5-8 to 60-65 % of Та2О5, respectively. Though in the ores of tantalum-niobium deposits niobium predominates over tantalum, but to extract the latter is technologically and economically expedient together with niobium. These are the deposits connected with subalkaline granitoids, selvage alkaline metasomatites (albitites) and stratified plutons of agpait nepheline syenites. The ratio of tantalum pentoxide to niobium pentoxide in these ores makes 1:5-1:20, and concentrates contain from 0,5 to 5 % of Та2О5.

The third group includes niobium deposits proper. Niobium in the ores predominates over tantalum (the Ta and Nb pentoxides ratio is less than 1:20); extraction of tantalum from them is unprofitable. Practically always such the ores contain chalcolamprite, and the deposits are connected with carbonatites and partly with albitites of nepheline syenites. Besides tantalum and tantalum-niobium deposits proper, tantalum can be extracted as a accompanying element from ores of some types of tin and tungsten deposits. Lately tantalum and niobium are extracted from the waste sand formed at tantalum production (Australia).

The state balance of Kazakhstan takes into account 15 deposits including 8 with balance reserves: Belogorskoye, Verhne-Baymurzinskoye, Bakennoye, Yubileynoye, Kvartsevoye, Akhmetkino, Kalay-Topkan (East Kazakhstan), Karaobinskoye (greisen) in the Central Kazakhstan.

Five among the listed deposits – Belogorskoye, Verhne-Baymurzinskoye, Bakennoye, Yubileynoye, Kvartsevoye – are depleted; the major part of their reserves was extracted almost for thirty years of operation. Extraction of the residue is not profitable. Tantalum from the Karaobinskoye deposit was not extracted.

Rare-earth elements. There are the following types of rare-earth deposits: magmatic, cabonatite, albitite, greisen, skarn-greisen and developed along them residual soils, hydrogene ores and coals. Endogenous REE-containing deposits are usually characterised by a complex multicomponent structure. Rare-earth elements in them are found, as a rule, in a close association with thorium, niobium, tantalum, zirconium, rarely – with uranium, molybdenum, beryllium. The state balance of the republic takes into account 8 rare-earth deposits.

Four of them are located in Mangistau Oblast on uranium deposits Melovoye, Tokhmak, Taybogar and Tasmuryn. Deposits Akbulak and Kundybay are located in Kostanay Oblast. On the deposits situated in the South Kazakhstan Oblast – Moinkum and Akdala only off-balance REE resources have been estimated. Five deposits are reserve.

In Soviet time REE in Kazakhstan were produced at Near-Caspian mining and chemical complex as accompanying elements at extraction of uranium and Irtysh chemical metallurgical plant from the raw material of Solikamsk magnesium factory. Now a little quantity of oxides of the cerium group is produced by LP Irtysh Rare Earth Co. Ltd. using imported raw materials.

For stable and reliable work of the enterprises in modern conditions it is necessary to recreate a mineral raw material base of rare metals and rare earths. Preconditions for this purpose in the republic are available.

On intensity of the ore-generating ability, petrochemical and geochemical features the tantalum-niobium and rare-earth ore occurrences, suitable for industrial development, are predicted in the mentioned below regions of the country.

Western Kazakhstan. The major factors allowing to predict the presence of endogenous rare-earth and tantalum-niobium deposits are, first, their spatial and genetic affinity with magmatic intrusions of acid and alkaline compositions, and secondly, a linkage of the rare metal occurrences with zones of deep submeridional ruptures and the high-order tectonic movements associated with them. Similar intrusive complexes are widespread in the Western Kazakhstan region. Their spatial arrangement has a clearly defined regularity – formation of three meridional belts passing throught all Mugodzharsky region. Large magmatic massifs – Berlinsky, Kairaktinsky, Ashchelysaysky, Beskopinsky, Shotinsky, etc. – are located along the meridional zones of deep breaks. More ancient (Caledonian) intrusions include small massifs of plagiogranites and gabbroids. In the east part of this structure, along the East Mugodzharsky breaks a number of narrow massifs of ultrabasite was mapped. In Mugodzhary there are five formations of rare metal and rare earth occurrences. Three of them are connected with granitoid magmatism. These intrusions, as a rule, are polyphase, contain dykes and pegmatites (tungsten-molybdenum mineralization). Rare-earth pegmatites were also found in Olataldyksky and Balataldyksky ore fields.
Rare-metal granitoids are located on separate sites – Borsyksaysky, Karasyrsky, Priozernoye, Uimola, etc.

The fourth formation – alkaline feldspar metasomatites (albitites) and the residual soils connected with them (Verhne-Irgizskoye, Tsentral’noye, Kyzylsay, Utesh-auly, etc.). In the world the largest and rich complex deposits of tantalum, niobium, zirconium, beryllium, rare earth elements with fluorite and cryolite are genetically connected with the given formation.

Now similar deposits are known in Ukraine, the Eastern Siberia (Katuginskoye, Hailominskoye), Polar Urals Mountains (Taikeusskoye) and other areas of Russia. At last, the fifth formation is rare-metal residual soils and placers.

Northern Kazakhstan. The area includes two regions with rare-metal and rare-earth mineralization: Kokshetausky Middle Massif and Ural Metallogenic Belt (Zhetygarinsky ore area). The Kokshetausky Middle Massif consists of three endogenous ore formations of different ages, which have tantalum-niobium and rare-earth specializations, and the residual soils and placers connected with them. The earliest formation is represented by Krasnomaysky alkaline-ultrabasic carbonatite complex.

In the world three formations are connected with them.

1. The formation of alkaline ultrabasic rocks and the residual soils associated with them. This formation includes the world's largest niobium deposits (Araksa, Tapira, etc.). The phlogopite-apatite deposit located near Lake Barchinskoye is also a representative of this formation.  The given apatite contains a raised concentration of REE of the yttrium group.

2. The columbite-bearing low-alkalinity granite formation (Та/Nb – 1:10) (Orlinogorsky, Zolotonoshensky, Elenovsky complexes).

3. The lithium-fluoric muscovite-albite granite formation it is practically unexplored. It is represented by small intrusions of lithium-fluoric granites (Losevskoye, Orlinogorskoye), dykes of amazon-stone albite granites (Totoguz, Prodol’ny, Zolotonoshensky massif) and albite and granite massifs of Syrymbetsky ore basin. The rich tantalum deposits of the world (Orlovskoye in Russia; Abu-Dabbab, Nuveybi in Egypt; Tor-Lake in Canada; Hald-zyan-buregteg in Mongolia) are connected with granitoids of this type. A number of rare-metal granite pots with complex mineralization of tantalum, niobium, lithium, tin, which, in the scientists’ judgment, are rather perspective, were found on the Kokshetausky Middle Massif.

Within the Ural metallogenic zone (Zhetygarinsky ore basin) from the west to the east there are three submeridional structural zones and respective metallogenic zones – Kusokanskaya, Troitskaya and Denisovskaya ones, which are separated from each other with complex deep breaks. All known gold-ore, rare-earth and rare-metal deposits (Kundybay, Smirnovskoye, Drozhilovskoye, etc.) are contained in their limits.

Central and Eastern Kazakhstan. Rare-metal and rare-earth mineralization of the central region of the republic, found in ancient Precambrian massifs, is complex and it is connected with fault alkaline metasomatites (albitites). These are Arganatinskaya and Karsakpayskaya structural metallogenic zones. The Arganatinskaya zone includes three ore areas: Ashchitastinsky (deposit Akbulak), Tobolgasaysky (Talayryk), Mayatassky. At present the given areas of rare-metal mineralization are unexplored.

Eastern Kazakhstan is a rare-metal and rare-earth geological province where the tin-tantalum-niobium and zirconium-tantalum-niobium mineralization predominates. Scientists single out two ore areas: Kalbinsky rare-metal pegmatite belt and Chingiz-Tarbogataysky rare-earth zone.

The pegmatite belt of Kalba is characterised by high density of tantalum occurrences; it contains a variety of ore formations including rare-metal granite pegmatites containing the major part of tantalum (deposits Ognevsko-Bakennoe, Asu-Bulakskoye, Belogorsko-Baymurzinskoye, Kvartsevoye); rare-metal granites (Karasu, Orgymbay, Diabazovy) and lithium-fluoric granites (Samsonovka, Amazonitovy); rare-metal complex tin-tantalum placer deposits (Asu-Bulakskaya, Kvartsevaya, Taintinskaya) and raw materials of a technogenic origin (old tails of concentrating factories of former Belogorsky ore dressing and processing enterprise).

Geological exploration established that the open spaces here make 14 %, semienclosed – 52 %, closed – 34 %. All known commercial deposits and large mineralizations have been found due to their outcrops. After the 70th of the last century the new deposits of tantalum connected with rare-metal pegmatites have not been found, that is the stock of easy-to-open deposits is practically up in the region.

Prospects of finding rare-metal pegmatites in the region are connected with their search on the closed and semienclosed areas. Blind, barely perceptible bodies of rare-metal pegmatites (Kenebay-Zhilandinskaya area, Proletarsky massif) were already revealed on some of them. Considerable resources of tantalum and niobium are available on deep levels and flanks of used deposits.

In the late eighties – the beginning of 90th years of the last century the lithium-fluoric granites, which can be considered as a new perspective source of tantalum-niobium raw materials were found out in Narym and Central Kalba. The tantalum quantity in them exceeds the reserves of the rare-metal pegmatites in tens times.

The obtained new data about the metallogeny of tantalum and niobium in the region allow us to draw a conclusion that the rare-metal mineralization is polygene and polychronic; in this connection it is necessary to continue its further studying and assessment. Also it is recomended to carry out exploration of complex deposits of the rare-earth tantalum-niobium formation in the joint of the Chingiz-Tarbagatayskaya zone and the Central Kazakhstan break (Kent-Akbastau area).

Southern Kazakhstan. It is a region that is characterized by a feebly marked rare-earth and tantalum-niobium mineralization type. Vanadium, phosphate and hydrogene uranium deposits with raised content of rare-earth elements (Ak-dala, Balasauskandyk, Kurumsak) have practical importance. In addition, the REE of the yttrium and cerium groups are established as accompanying components in the rare-metal deposits Kairagaily-Aktas, Boguty, the ore occurrence Khorgos. Isolated REE mineralization points have been found in the Central Dzungariya in the Tekeliysky Mountains.

Summarizing the above-stated, it is necessary to notice, that the available potential of rare and rare-earth elements allows us to solve the problem of supply of the domestic enterprises producing rare and rare-earth metals with Kazakhstan raw materials. For this purpose it is necessary to develop a complex program (tantalum, niobium and REEs) within the next few years to coordinate the geological prospecting works.

As the top-priority objects of prospecting and assessment works Kazakhstan scientists offer the following ones: the Verhne-Irgizskoye deposit (tantalum, niobium, REEs), the average content is from 50 to 2 000 g/t; the ore occurrence Priozernoye (REEs, beryllium, tantalum, niobium), the average REEs content of 8-100 g/t.

In addition, deposits Kadyrsky (tantalum, niobium, beryllium, lithium) with the average tantalum pentoxide content from 80 to 200 g/t, Prodol’ny of the Zolotonoshensky massif (tantalum, niobium, REEs) with the average tantalum pentoxide content of 200 g/t are also perspective for investigation and development.

It should be noted the ore occurrence Novosvetlovskoye (tantalum, niobium, REEs; the average content of yttrium REEs is approximately 6 000 g/t) and also the apatite deposit Barchinskoye (the yttrium REEs content in the apatite is 60 % and more).

Uranium and Nuclear Power in Kazakhstan

- Kazakhstan has 12% of the world's uranium resources and in 2018 produced about 21,700 tU.

- In 2009 it became the world's leading uranium producer, with almost 28% of world production, then 33% in 2010, rising to 41% in 2014, and 39% in 2015 and 2016.

- A single Russian nuclear power reactor operated from 1972 to 1999, generating electricity and desalinating water.

- Kazakhstan has a major plant making nuclear fuel pellets and aims eventually to sell value-added fuel rather than just uranium. A fuel fabrication plant is being built with 49% Chinese equity.

- The government is committed to a high level of uranium exports, and plans in 2018 to complete a feasibility study for a reactor, probably to be sited at Kurchatov.

Kazakhstan has been an important source of uranium for more than 50 years. Over 2001 to 2013 its production rose from 2022 to about 22,550 tU per year making Kazakhstan the world's leading uranium producer. Capacity is about 25,000 tU/yr, but in October 2011 Kazatoprom announced a cap on production of 20,000 tU/yr, which was evidently disregarded. Of its 17 mine projects, five are wholly owned by Kazatomprom and 12 are joint ventures with foreign equity holders, and some of these are producing under nominal capacity. In 2017, 12,093 tU was attributable to Kazatomprom itself – 21% of the world production, putting it ahead of Cameco, followed by Orano and Uranium One. In January 2017 Kazatomprom said that the production would be reduced by about 10%, due to low prices; in December 2017 Kazatomprom announced that the reduction would be 20% from 2017 levels, enacted over a period of three years. The announced cuts amount to a production deferral of 11,000 tonnes of natural uranium. In 2018 production was reduced to 21,699 tU, from 23,321 tU in 2017. However in February 2019 the country's energy ministry announced that the 2019's production target was set about 5% higher, at 22,742 tU.

Fig. 18 – Flag of NAC Kazatomprom

Kazakhstan has northern and southern electricity grids with some connection, and links to Russia and Kyrgystan and Uzbekistan, respectively. Electricity production was 107 TWh in 2016, 71 TWh (66%) from coal, 22 TWh (21%) from natural gas, 12 TWh (11%) from hydro and 0.4 TWh from wind. There were virtually no imports or exports of electricity in 2016. At the end of 2016 capacity was 22 GWe with 18.8 GWe available. In 2012 the government's energy system development plan had 150 TWh/yr productions in 2030, with 4.5% of this from nuclear and 10% from renewables. The government planned investment in electricity production and grid of $7.8 billion by 2015, and foresees $64 billion by 2030.

Future electricity demand will depend to some extent on the country’s role from 2019 in the Eurasian Economic Community energy market. Also the State Grid Corporation of China (SGCC) is planning transmission links from China. The state-owned Kazakhstan Electricity Grid Operating Company (KEGOC) was set up in 1997. The question of nuclear power in Kazakhstan has been discussed for many years, notably since 2006 with Russia, and a national nuclear strategy is expected in 2018.

Fig.19 – Yellow cake

Kazatomprom is the national atomic company set up in 1997 and owned by the government. It controls all uranium exploration and mining as well as other nuclear-related activities, including imports and exports of nuclear materials. It announced in 2008 that it aimed to supply 30% of the world's uranium by 2015 (it produced 39% in fact), and through joint ventures: 12% of the uranium conversion market, 6% of enrichment, and 30% of the fuel fabrication market by then.

Prior to Kazatomprom’s establishment, other arrangements pertained for uranium development.  One of these was with Canada-based World-Wide Minerals Ltd (WWM), under a 1989 bilateral investment treaty between Canada and the USSR.*

* WWM invested heavily in the country over 1996-97, upgrading and operating the Tselinny (TGK) uranium mining and processing facilities at Stepnogorsk, with an option to acquire 90% equity in them as well as developing additional mines. WWM and subsidiaries entered into agreements with the Kazakh government, but claims that the government frustrated its endeavours, leading to a loss of more than $50 million and its exit from the country. In January 2016 an international arbitral tribunal upheld WWM’s claims under investor-state arbitration and dismissed Kazakh objections. WWM is seeking $5 billion settlement.

## International collaboration

## Kazatomprom has forged major strategic links with Russia, Japan and China, as well as taking a significant share in the international nuclear company Westinghouse. Canadian and French companies are involved with uranium mining and other aspects of the fuel cycle.

## Russia

## In July 2006 Russia and Kazakhstan (Kazatomprom) signed three 50:50 nuclear joint venture agreements totalling US$ 10 billion for new nuclear reactors, uranium production and enrichment. The first joint venture (JV) with Atomstroyexport is JV Atomniye Stantsii for development and marketing of innovative small and medium-sized reactors, starting with OKBM's VBER-300 as baseline for Kazakh units. Russia's Atomstroyexport expected to build the initial one.

Fig.20 – LP “Mangistau atomic energy plant – Kazatomprom”

## The second JV with Tenex, confirmed in 2008, is for extending a small uranium enrichment plant at Angarsk in southern Siberia (this will also be the site of the first international enrichment centre, in which Kazatomprom has a 10% interest). It will eventually be capable of enriching the whole 6000 tonnes of uranium production from Russian mining JVs in Kazakhstan. See Fuel Cycle section below.

## The uranium exploration and mining JVs Akbastau and Karatau with Tenex started with Budenovskoye in the Stepnoye area of south Kazakhstan, which commenced production in 2008. These complemented the Zarechnoye JV 250 km to the south which was set up in June 2006. However, in 2009 and 2010 the 50% ARMZ equity in these three was traded for an eventual 51% share of Canadian-based Uranium One Inc, which subsequently became wholly-owned by ARMZ. Uranium One Holdings (U1H) is now the holding company for all Russian uranium mining interests in Kazakhstan (and its equity in an acid plant).

## In March 2011 Russia and Kazakhstan (Kazatomprom) signed stage II of this 2006 integrated cooperation program, involving uranium exploration and a feasibility study for a Kazakh nuclear power plant. Under this, and following JV development at Angarsk, Kazatomprom bought a 25% share of Russia's Novouralsk enrichment plant in 2013. (Separately, Kazatomprom has a 10% share in the International Uranium Enrichment Centre – IUEC – at Angarsk.)

## At the end of May 2014 several agreements were signed between Rosatom and NAC Kazatomprom. One was a MOU for construction of a nuclear power plant using VVER reactors and with capacity up to 1200 MWe. It also involved fuel fabrication and nuclear waste management. A second agreement related to uranium mining at Kharasan-1, Akdala and South Inkai, where ARMZ has equity through Uranium One. A third agreement was a Comprehensive Development Program for Russia-Kazakhstan Cooperation in the Peaceful Uses of Atomic Energy, for nuclear power and fuel cycle matters.

## Japan

## In April 2007 a number of high-level agreements on energy cooperation were signed with Japan. These included some relating to uranium supply to Japan, and technical assistance to Kazakhstan in relation to fuel cycle developments and nuclear reactor construction. A further agreement on uranium supply and Japanese help in upgrading the Ulba fuel fabrication plant was signed in may 2008. Kazatomprom is keen to move from being a supplier of raw materials to selling its uranium as fabricated fuel assemblies. It said that it aimed to supply 40% of the Japanese market for both natural uranium and fabricated fuel from 2010 – about 4000 tU per year. Negotiations then commenced for a bilateral nuclear cooperation agreement between Kazakhstan and Japan. In May 2011 a high-level intergovernmental agreement on developing nuclear energy was signed.

## In August 2006 The Japan Bank for International Cooperation had signed an agreement with Kazatomprom to support and finance Japanese firms in developing Kazakh uranium resources to supply Japan's power generation. In April 2007 several Japanese companies – the Energy Asia consortium – bought 40% of the whole Kharasan mine project. Initially, Energy Asia comprised Marubeni 55%, Tepco 30%, Chubu 10% and Tohoku 5%. When Toshiba agreed to sell part of Westinghouse to Kazatomprom, it agreed to buy 9% of Kharasan from Marubeni (i.e. 22.5% of the Japanese stake). Then Kyushu Electric Power Co bought 2.5% of the Japanese stake, leaving Marubeni with 30%. The Energy Asia consortium share involved with both JVs (Kyzylkum and Baiken-U) became: Marubeni 30%, Tepco 30%, Toshiba 22.5%, Chubu 10%, Tohoku 5% and Kyushu 2.5%. In 2018 Kazatomprom bought a 40.05% share in Energy Asia (BVI) Ltd and a 16.02% share of Khorosan-U.

## In March 2009 three Japanese companies – Kansai, Sumitomo and Nuclear Fuel Industries – signed an agreement with Kazatomprom on uranium processing for Kansai plants. In March 2010 a joint venture with Sumitomo was set up: Summit Atom Rare Earth Company, and in June, Kazatomprom and Toshiba Сorp. agreed to set up a rare earth metals joint venture.

## In September 2010, based on an April 2007 agreement, Japan Atomic Power Co, Toshiba and Marubini signed a technical cooperation agreement with the National Nuclear Centre (NNC) to study the feasibility of building nuclear power capacity. A further agreement to this end was signed in February 2013, between Japan Atomic Power Co (JAPC) and Marubini Utility Services, with NNC (see section below). At the same time an agreement between NNC and the Japan Atomic Energy Agency (JAEA) with JAPC concerned mining and processing of uranium and rare earth minerals. In October 2015 a further agreement between Kazatomprom, JAPC and Marubini Utility Services was signed, to develop cooperation on construction and financing of a nuclear power plant, involving consultations, exchange of experience in public communications, organization of technical workshops and meetings of experts on security and the training of staff.

## In June 2012 and February 2013 R&D agreements between NNC and JAEA were signed relating to the design, construction and operation of the Kazakhstan high-temperature gas-cooled reactor (HTR) of about 50 MW at Kurchatov. This is flagged as a national project.

## In June 2015 an agreement was signed between NNC and JAEA for stage 3 of a project to investigate sodium-cooled fast reactors in Kazakhstan.

## China

## In December 2006 China Guangdong Nuclear Power Group (now China General Nuclear Corporation - CGN) signed a strategic cooperation agreement with Kazatomprom, in May 2007 an agreement on uranium supply and fuel fabrication, and in September 2007 agreements on Chinese participation in Kazakh uranium mining joint ventures and on Kazatomprom investment in China's nuclear power industry. This is a major strategic arrangement for both companies, with Kazatomprom to become the main uranium and nuclear fuel supplier to CGN (accounting for a large share of the new reactors being built in China). In October 2008 a further agreement was signed covering cooperation in uranium mining, fabrication of nuclear fuel for power reactors, long-term trade of natural uranium, generation of nuclear electricity and construction of nuclear power facilities. In December 2014 a further agreement was signed with similar scope but focused on establishing a joint venture in Kazakhstan for the production of 200 t/yr of fuel assemblies. In December 2015 a further agreement was signed on the fuel fabrication project, to be at the Ulba Metallurgical Plant. A CGN subsidiary, Sino-Kazakhstan Uranium Resources Investment Co, has invested in two Kazakh uranium mines, Irkol and Semizbai, through the Semizbai-U LLP joint venture. In 2015 CGN Mining Co bought the 49% Chinese equity in Semizbai-U.

## A framework strategic cooperation agreement was signed with China National Nuclear Corporation (CNNC) in September 2007 and this was followed in October 2008 with another on "long-term nuclear cooperation projects" under which CNNC was to invest in an uranium mine. Late in 2007 Kazatomprom signed an agreement with both GCNPC (now CGN) and CNNC for them to take a 49% stake in two uranium mine joint ventures and supply 2000 tU per year from them. In February 2011 CNNC signed a contract to buy 25,000 tU.

## Early in 2009 Kazatomprom signed an agreement with CGNPC for establishment of a specialized company for the construction of nuclear power plants in China, since Kazakh plans to work with Russia's Atomstroyexport developing and marketing innovative small and medium-sized reactors had been put on hold. In mid-2009 a feasibility study on this joint CGNPC project was underway, but no more has been heard since. In December 2015 both governments announced the establishment of a $2 billion fund for bilateral projects within the framework of the 'New Silk Road', now Belt & Road Initiative (BRI), the new Chinese investment program. CGN is working with Kazatomprom to build the Ulba-FA fuel fabrication plant.

## In mid-2014 Kazatomprom said that 55% of Kazakh uranium production was exported to China.

## At the end of August 2015, among $23 billion of China-Russia deals, JSC Samruk-Kazyna, the national holding company owning NAC Kazatomprom, signed deals worth $5 billion with Chinese companies and Kazatomprom agreed on transit of its products via China to North America.

## In 2013 China agreed to a $5 billion stake in the new Kashagan oil project, trumping a bid from India, and underlining China’s Central Asian resource aspirations.

## The Samruk-Kazyna Wealth Fund, with assets of over $64 billion, plans initial public offerings in 2018, including one for Kazatomprom, to reduce its holdings in several state enterprises.

## India

## In January 2009 Kazatomprom signed an agreement with India's Nuclear Power Corporation (NPCIL) to supply 2100 tonnes of uranium to India and undertake a feasibility study on building Indian PHWR reactors in Kazakhstan. NPCIL said that it represented "a mutual commitment to begin thorough discussions on long-term strategic relationship." Under this agreement, 300 tonnes of natural uranium will be supplied by Kazatomprom in the 2010-11 year.

## In April 2010 Kazakhstan signed a nuclear cooperation agreement with South Korea, paving the way for export of Korean SMART 100 MWe nuclear reactors and for joint projects to mine and export Kazakh uranium.

## In addition Kazakhstan has signed intergovernmental agreements on nuclear energy cooperation with the USA and Euratom.

## South Korea

## The Kazakh Industry and Trade Ministry has held talks with South Korea's KEPCO, (Korea Electric Power Corporation) on uranium mining and nuclear power plant construction in Kazakhstan, apparently on KEPCO's initiative.

## Toshiba

## At the corporate level, in 2007 Kazatomprom purchased a 10% share in Westinghouse. Toshiba had bought the company from BNFL for $5.4 billion early in 2006, and The Shaw Group then took 20% and IHI Corp. 3%. Toshiba originally envisaged holding only 51%, and this deal reduced its holding to 67%. The Kazatomprom link strengthened the company's upstream links for fuel supplies, and was to enhance its marketing of nuclear reactors (the vendor usually supplies the first core for a new reactor, and ongoing fuel services may be offered in addition). It also brought Kazatomprom more fully into the industry mainstream, with fuel fabrication in particular. However, in 2017 Kazatomprom exercised its option to require Toshiba to buy its share for $522 million

## The Westinghouse link led to a decision to set up with Toshiba a nuclear energy institute in the northeastern town of Kurchatov, near Semipalatinsk, which is already a centre of R&D activity. This was announced by Kazatomprom and the Kazakh prime minister in September 2008 and was to focus on skills development in all aspects of the nuclear fuel cycle as well as reactor technology. Other Japanese companies such as Toyota and Marubeni were expected to support the institute, especially in its rare earth metals department which aims to utilise present waste materials as the basis of a billion-dollar high-tech export industry. Three research reactors are operated by the Institute of Atomic Energy at Kurchatov.

## Canada, Cameco

In May 2007 Canada's Cameco Corporation signed an agreement with Kazatomprom to investigate setting up a uranium conversion plant, using its technology, and also increasing uranium production at its 60% owned Inkai mine.

In May 2016 Cameco and Kazatomprom agreed to restructure the Inkai JV, extending it to 2045 but with Cameco becoming a minority owner, with 40%. Production is to be ramped up to 4000 tU per year by about 2021. The restructuring took effect from 1 January 2018.

In June 2008 Cameco and Kazatomprom announced the formation of a new company – Ulba Conversion LLP – to build a 12,000 t/yr uranium hexafluoride conversion plant at the Ulba Metallurgical Plant (UMZ) in Ust-Kamenogorsk. Cameco would provide the technology and hold 49% of the project. A preliminary feasibility study was undertaken jointly by Kazatomprom and UMZ, then the project was put on hold. In mid-2013 Cameco announced that subject to a feasibility study, construction of 6000 t/yr capacity would start in 2018, for 2020 operation. A May 2016 agreement grants Kazatomprom a five-year option to license Cameco's uranium conversion technology for constructing and operating a uranium conversion plant in Kazakhstan.

In December 2013, a prefeasibility study (PFS) for a uranium refinery in Kazakhstan was completed, to produce UO3for further processing in Canada. The project will require government approvals for the transfer of Cameco’s proprietary uranium refining technology from Canada. In January 2014 the government referred to the proposed plant as a ‘strategic goal’, and in May 2016 the two companies agreed to complete a feasibility study on a uranium refinery producing 6000 tU per year as UO3. This would initially be owned 71.67% by Kazatomprom and 28.33% by Cameco, and Cameco's interest in JV Inkai would increase from 40% to 42.5% on commissioning of the refinery. Should the refinery be built, Kazatomprom will also be given an option to obtain UF6 conversion services at Cameco's Port Hope facility and to receive other commercial support from Cameco, whose equity in both operations might increase slightly as a result.

In November 2013 Canada and Kazakhstan signed a nuclear cooperation agreement.

### USA, Centrus, Converdyn

In October 2015 Kazatomprom signed an agreement with Centrus Energy to help market Kazakh uranium in the USA.

In April 2016 Kazatomprom signed an agreement with US-based Converdyn so that Kazatomprom could offer uranium for sale in the form of natural UF6, and access new markets with an “integrated product offering”, giving customers “increased supply options”. This is simply a marketing alliance, complementary to the Cameco investment in Ulba.

Also in April a Kazakh-US energy partnership agreement was signed, related to nuclear security and in particular the “conversion of Kazakhstan's research reactors and enforcement of physical nuclear security.”

### Areva

In June 2008 Areva signed a strategic agreement (MOU) with Kazatomprom to expand the existing Katco joint venture from mining 1500 tU/yr to 4000 tU/yr (with Areva handling all sales), to draw on Areva's engineering expertise in a second JV (49% Areva) to install 1200 tonnes per year fuel fabrication capacity at the Ulba Metallurgical Plant, and in a third JV (51% Areva) to market fabricated fuel.

In October 2009 the two parties signed another agreement to establish the IFASTAR joint venture (Integrated Fuel Asia Star - 51% Areva) to establish the feasibility of marketing an integrated fuel supply for Asian customers (ie selling the enriched and fabricated fuel, not simply Kazakh uranium or Areva front-end services), and of building a 400 t/yr nuclear fuel fabrication line at the Ulba plant. IFASTAR is to be based in Paris, and would market the fuel.

In October 2010 an agreement was signed to set up a joint venture company (51% Kazatomprom) to build a 400 t/yr fuel fabrication plant based on an Areva design at the Ulba Metallurgical Plant, starting operation by 2014. In November 2011 a further agreement was signed in relation to the plant. In March 2016 Areva was awarded a contract for equipment and support to the Ulba-FA plant for the fabrication of 200 t/yr of fuel assemblies for China General Nuclear Corp (CGN) reactors. The plant is being built by a Kazatomprom-CGN joint venture.

In April 2017 a further agreement with Areva was signed to give the Katco joint venture a new long-term perspective, with the development of the South Tortkuduk project.

### Iran

In March 2017 Kazatomprom contracted to supply 950 t of uranium concentrate to Iran over three years, subject to agreement by the UN Security Council.

Mines

At a corporate and project level in mining, the following table summarises international equity links:

Table 7 – International equity links

| Company, project or mine | Foreign investor and share | Value of share or project if known |
| --- | --- | --- |
| Inkai JV (Inkai mines) | Cameco 40% |  |
| Betpak Dala JV (South Inkai, Akdala mines) | Uranium One 70% | $350 million for 70% in 2005 |
| Appak JV (W.Mynkuduk) | Sumitomo 25%, Kansai 10% | $100 million total in 2006 |
| JV Karatau (Budenovskoye 2 deposit) | Uranium One 50% (bought from ARMZ in 2009) | 117 million Uranium One shares (giving 19.9% ownership) + $90 million |
| Akbastau JSC (Budenovskoye 1, 3, 4 deposits) | Uranium One 50% (bought from ARMZ in 2010) |  |
| Zhalpak | CNNC 49% |  |
| Katco JV (Moinkum, Tortkuduk mines) | Areva 51% | $110 million in 2004 |
| Kyzylkum JV now Khorosan-U (Kharasan 1 mine) | Uranium One 30%, Energy Asia (Japanese + 40.05% Kazatomprom) 20% | $75 million in 2005 for 30%, $430 million total in 2007 (both mines) |
| Baiken-U JV (Kharasan 2 mine) | Energy Asia (Japanese + 40.05% Kazatomprom) 47.5% | $430 million total in 2007 (both mines) |
| Semizbai-U JV (Irkol, Semizbai mines) | CGN 49%, also CNEIC |  |
| Zarechnoye JSC (Zarechnoye & S.Zarechnoye mines) | Uranium One 49.67% (bought from ARMZ in 2010), Krygyzstan 0.66% | ARMZ paid $60 million total |

Early in 2012 Kazatomprom announced that it would increase its share in mining activities nationally from 46% to 51% by buying out Japanese (and possibly some Uranium One) equity in the Kyzylkum and Baiken-U JVs, where it held 30% and 5% respectively. Both JVs are mining the Kharasan deposit in the western part of Syrdarya province. In 2014 Khorosan-U was set up by Kazatomprom to take over from Kyzylkum the mining of Kharasan 1 in JV with Energy Asia (36%) and Uranium One (30%). In 2018 Kazatomprom's ownership of the Baiken-U, Kyzylkum and Khorosan-U joint ventures became 52.5%, 50% and 50% respectively after it bought a 40.05% share in Energy Asia (BVI) Ltd and a 16.02% share of Khorosan-U.

In 2009 investigations were launched into how, and at what prices, certain Kazakh entities came to hold title to particular mineral deposits before those rights were sold to international investors, particularly some of those above. In June 2009 Kazatomprom reassured its foreign joint venture and equity partners in uranium mining, from Japan, Russia, Canada, France and China that existing arrangements with foreign partners would not be changed, despite criminal charges being laid against former Kazakh executives.

The transfer to Uranium One of ARMZ's half shares in Akbastau and Zarechnoye (valued at US$ 907.5 million) in 2010 involved payment by ARMZ of US$ 610 million in cash (at least US$ 479 million of which would be paid directly to shareholders – other than ARMZ – as a change of control premium) and ARMZ increasing its shareholding in Uranium One from 23% to at least 51.4% through a share issue. It subsequently took over the whole company.

### Uranium trading

In April 2017 Kazatomprom announced the formation of a Swiss-based trading subsidiary TH Kazatom, to bring greater liquidity to the uranium market from late in the year. It will buy and sell on the spot market as part of its corporate transformation to align its pricing mechanism with “the way our customers want to buy”, especially in European and US markets.

## Uranium mining

Uranium exploration started in 1948 and economic mineralisation was found in several parts of the country and this supported various mines exploiting hard rock deposits. Some 50 uranium deposits are known, in six uranium provinces. Reasonably Assured Resources plus Inferred Resources to US$ 130/kgU were 679,000 tU in 2013.

In 1970 tests on in situ leach (ISL) mining commenced and were successful, which led to further exploration being focused on two sedimentary basins with ISL potential.

Up to 2000 twice as much uranium had been mined in hard rock deposits than sedimentary ISL, but almost all production is now from ISL. Uranium production dropped to one-quarter of its previous level 1991 to 1997, but has since increased greatly.

Table 8 – Kazakh uranium production

| Year | 2006 | 2007 | 2008 | 2009 | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tonnes U | 5281 | 6637 | 8521 | 14,020 | 17,803 | 19,450 | 21,317 | 22,451 | 23,127 | 23,607 | 24,586 | 23,321 | 21,705 | 22,742 (planned) |

Source: Kazatomprom

In 2009 Kazakhstan became the world's leading uranium producer, with almost 28% of world production, then 33% in 2010 rising to 41% in 2014, and 39% in 2015 and 2016.

In January 2017, Kazatomprom announced a 10% cut in planned production for 2017. Then, in December 2017, it announced a 20% cut in planned production over three years, starting from January 2018. The company said that the revised production plan was to better align with demand. However in February 2019, the Kazakhstan Ministry of Energy announced that production would increase by 5% in 2019.

The last surviving underground mines at Grachev and Vostok in the Northern province had been operating since 1958 and are now rather depleted. KazSabton operated them, having taken over from Tselinny Mining & Chemical Co (TGK) in 1999. It treated the ore at the Stepnogorsk mill, yielding some 250 tU per year. Production from the Stepnogorsk Mining & Chemical Complex plant at some 300 tU/yr has been the only non-ISL production, but the mine was reported to be shut down in 2015. The Semizbai ISL project is also in the Northern province, Akmola region, and Semizbai-U was formed in 2006 to mine it.

Table 9 – Kazakh uranium production by mines (tonnes U)

| Province and Group | Mine | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chu-Sarysu, Eastern | Tortkuduk & northern Moinkum (Katco) | 3661 | 3558 | 4322 | 4007 | 4003 | 3519 |  |
|  | Southern Moinkum & Kanzhugan (Taukent/GRK) | 1075 | 1129 | 1174 | 873 | 781 | 616 |  |
| Chu-Sarysu, Northern | Uvanas & Eastern Mynkuduk (Stepnoye-RU/GRK) | 1234 | 1192 | 1154 | 1341 | 1222 | 974 |  |
|  | Central Mynkuduk (Ken Dala.kz) | 1622 | 1800 | 1790 | 1770 | 1953 | 1898 |  |
|  | Western Mynkuduk (Appak) | 1003 | 998 | 870 | 880 | 1004 | 901 |  |
|  | Inkai 1, 2, 3 (Inkai) | 1701 | 2047 | 1922 | 2418 | 2413 | 2202 | 2654 |
|  | Inkai 4 (South Inkai) | 1870 | 2030 | 2002 | 2007 | 2058 | 2037 | 1617 |
|  | Akdala (Betpak Dala) | 1095 | 1020 | 1007 | 1042 | 1000 | 900 | 800 |
|  | Budyonovskoye 1, 3 & 4 (Akbastau) | 1203 | 1499 | 1594 | 1630 | 1778 | 1941 | 1546 |
|  | Budyonovskoye 2 (Karatau) | 2135 | 2115 | 2084 | 2064 | 2108 | 2359 | 2081 |
| Syrdarya, Western | North and South Karamurun (GRK) | 1000 | 1000 | 941 | 956 | 1015 | 718 |  |
|  | Irkol (Semizbai-U) | 750 | 750 | 700 | 781 | 700 | 678 |  |
|  | Kharasan 1 (Kyzylkum, now Khorosan-U) | 583 | 752 | 858 | 1095 | 1354 | 1564 | 1601 |
|  | Kharasan 2 (Baiken-U) | 603 | 888 | 1135 | 1503 | 1838 | 1762 |  |
| Syrdarya, Southern | Zarechnoye (Zarechnoye) | 942 | 931 | 876 | 800 | 817 | 802 | 776 |
| Northern, Akmola region | Semizbay (Semizbai-U) | 470 | 411 | 400 | 440 | 542 | 450 |  |
|  | RU-1 (Vostok, Zvezdnoye) | 370 | 331 | 298 | 0 | 0 | 0 |  |
| TOTAL |  | 21,317 | 22,451 | 23,127 | 23,607 | 24,586 | 23,321 |  |

In the Balkhash province some mining of volcanogenic deposits occurred during the Soviet era. In the Ili province east of this there is some uranium in coal deposits.

In the Caspian province the Prikaspisky Combine operated a major mining and processing complex on the Mangyshalk Peninsula in the 1960s and this led to the founding of Aktau. It was rivatized as Kaskor in 1992 and operations were ceased in 1994.

All except one of the operating and planned ISL mine groups are in the 40,000 square kilometre Chu-Sarysu province in the central south of the country and controlled by the state corporation Kazatomprom. Mines in the Stepnoye area have been operating since 1978, some in the Tsentralnoye area since 1982 – both in the Chu-Sarysu basin/uranium district, which has more than half the country's known resources. It is separated by the Karatau Mountains from the Syrdarya basin/uranium district to the south, where mines in the Western (No.6) area have operated since 1985. All have substantial resources.

Fig.21 – Map from Kazatomprom, 2007

The ISL mines and projects in the two central southern provinces are in four groups. Production costs from these are understood to be low. Mining is at depths of 100-300 metres, though some orebodies extend to 800 metres. Uranium One in September 2007 was quoting "cash cost" figures of $8.00 to $10.50/lb for three mines it is involved with, though these may not include wellfield development and current figures are quoted below. A further feature of Kazakh uranium mining is that Kazatomprom plans to establish new mines in three years, compared with twice this time or more in the West, due to regulatory hurdles.

Inkai is the largest ISL mine and Cameco's description of its operation is: Uranium occurs in sandstone aquifers as coatings on the sand grains at a depth of up to 300 metres. Uranium is largely insoluble in the native groundwater which is not potable due to naturally high concentrations of radionuclides and dissolved solids. Using a grid of injection and production wells, a mining solution containing an oxidant (sulfuric acid) is circulated through the orebody to dissolve the uranium. The uranium-bearing solution (generally containing less than 0.1% uranium) is then pumped to a surface processing facility where the uranium is removed using ion exchange resin. The water is re-oxidized and re-injected into the orebody. The uranium is stripped from the resin, precipitated with hydrogen peroxide and then dried to form the final product, U3O8. This process is repeated to remove as much uranium as is economically feasible. When mining at the site is complete, the groundwater will be restored to its original quality.

This is a closed loop recirculation system since the water from the production well is reintroduced in the injection wells. Slightly less water is injected than is pumped to the surface to ensure that fluids are confined to the ore zones intended for extraction. Monitor wells are installed above, below and around the target zones to check that mining fluids do not move outside a permitted mining area.

### Health and environment

Kazatomprom said that its enterprises in 2014 continued to ensure ecological safety at its mines, and 23 of the company's affiliates and subsidiaries have ecological management standards certification. More than KZT 1.19 billon ($11 million) was spent in 2014 on measures to reduce the environmental impact of uranium mining, including efficiency improvements to dust and gas collecting installations and water purification units.

Occupational safety and security at uranium production sites is monitored and in 2014 the number of detected violations of occupational and industrial safety requirements dropped by 28% compared with 2013.

## Fuel cycle

The internationally-significant Ulba Metallurgical Plant (UMP) at Oskomen also known as Ust Kamenogorsk in the east of the country was commissioned in 1949. It has a variety of functions relevant to uranium. (It also produces beryllium, niobium and tantalum.)

In June 2008 the formation of a new company – Ulba Conversion LLP – was announced, to build a 12,000 t/yr uranium hexafluoride conversion plant here, with Cameco providing the technology and holding 49% of the project. Ulba has produced HF since 1952, and the new conversion subsidiary would fit in with Russian JV enrichment arrangements. In May 2013 Cameco said that it expected construction of the plant with 6000 t/yr capacity to commence in 2018, with first production in 2020, subject to a feasibility study from 2014. This plant will produce UO3 for conversion to UF6 at Port Hope in Canada.

Kazatomprom has a JV with Russia's TVEL for uranium enrichment, (agreed with Tenex in 2006 and set up in 2008). Initially this envisaged adding to the enrichment plant at Angarsk in southern Siberia where Russia has its main conversion plant and a small enrichment plant now being expanded to 4.2 million SWU/yr. Kazatomprom and Tenex agreed to finance a 5 million SWU/yr increment to this. Each party would contribute about US$ 1.6 billion and Kazatomprom would hold 50% equity. When this looked uneconomic due to surplus enrichment capacity, in March 2011 Russian equity in the JV was transferred from Tenex to TVEL and the Kazatomprom-TVEL JV Uranium Enrichment Centre (Closed Joint Stock Company UEC) was offered a share in the Urals Electrochemical Combine (Open Joint Stock Company UECC) which has a 10 million SWU/yr plant at Novouralsk instead. The Kazakh share in UEC would be 50%, related to the need to enrich 6000 tU/yr, and estimated to cost up to $500 million (though amount not disclosed). In the event the joint venture CJSC UEC took up a 25% share of UECC in September 2013 and became entitled to half its output – 5 million SWU/yr. In 2014 the UEC CJSC share was 4.99 million SWU, and in 2015 it was 5.11 million SWU. This is distinct from the International Uranium Enrichment Centre (IUEC).

In September 2007 the joint stock company Angarsk International Uranium Enrichment Centre (IUEC) was registered with 10% Kazatomprom ownership and the balance Techsnabexport (Tenex). This share is being sold down to other partners – Ukraine confirmed 10% share in 2008, and Tenex is to hold only 51% eventually.

Since 1973 Ulba has produced nuclear fuel pellets from Russian-enriched uranium which are used in Russian and Ukrainian VVER and RBMK reactors. Some of this product incorporates gadolinium and erbium burnable poisons. Other exports are to the USA and Asia. Ulba briefly produced fuel for submarines (from 1968) and satellite reactors. Since 1985 it has been able to handle reprocessed uranium, and it has been making fuel pellets incorporating this for western reactors, supplied through TVEL. It is a major supplier to China.

Fig.22 – A worker poses with a uranium fuel pellet at Ulba (Kazatomprom)

Ulba Metallurgical Plant (UMP) is majority owned by Kazatomprom and 34% by Russia's TVEL and has major new investment under way. It has secured both ISO 9001 and ISO 14001 accreditation. In 2007 a technological assistance agreement was signed with Japan apparently in line with government announcements that it would move towards selling its uranium as fabricated fuel or at least fuel pellets rather than just raw material. (One agreement is on fabrication of nuclear fuel components, between Kazatomprom, Kansai Electric and Sumitomo Corp.) In 2010, UO2 powder for Japan was certified by Japan's Nuclear Fuel Industries, and fuel pellets for China by CNNC's China Jianzhong Nuclear Fuel.

Kazatomprom has said that it aims to supply up to one third of the world fuel fabrication market by 2030, with China to be an early major customer. In June 2008 Areva signed a memorandum of understanding to provide engineering expertise to build a 1200 t/yr fuel fabrication plant as part of the Ulba complex, utilising fuel pellets from it. In October 2010 and November 2011 further agreements focused on a dedicated 400 t/yr line (51% owned by Kazatomprom, 49% Areva) specifically for fuel for French-designed reactors, including those in China. Another 800 t/yr line would be wholly owned by Kazatomprom. These plans with Areva appear to have stalled.

In December 2014 Kazatomprom signed an agreement with China General Nuclear Corp (CGN) focused on establishing the Ulba-FA joint venture to build a fuel fabrication plant for the production of 200 t/yr of fuel assemblies at the Ulba Metallurgical Plant. In December 2015 and September 2016 further agreements on this were signed, with UMP to have 51% and CGN-URC 49% shares in the $147 million project. In March 2016 Areva was awarded a contract for equipment and support to the Ulba-FA plant for the fabrication of AFA 3G fuel assemblies for CGN reactors. In December 2016 construction of the plant by Kazatomprom and CGN began. It is due for completion in 2018, and first shipments to China are expected in 2020, at 200 t/yr (about 400 fuel assemblies).

Kazatomprom was also negotiating technology transfer agreements to enable it to supply fabricated fuel for Westinghouse reactors, related to its 10% stake in Westinghouse.

Uranium in-situ leaching technology

In-situ leaching (ISL), also called in-situ recovery (ISR) or solution mining, is a mining process used to recover minerals such as copper and uranium through boreholes drilled into a deposit, in situ. In situ leach works by artificially dissolving minerals occurring naturally in a solid state.

The process initially involves the drilling of holes into the ore deposit. Explosive or hydraulic fracturing may be used to create open pathways in the deposit for solution to penetrate. Leaching solution is pumped into the deposit where it makes contact with the ore. The solution bearing the dissolved ore content is then pumped to the surface and processed. This process allows the extraction of metals and salts from an ore body without the need for conventional mining involving drill-and-blast, open-cut or underground mining.

In-situ leach mining involves pumping of a lixiviant into the ore body via a borehole, which circulates through the porous rock dissolving the ore and is extracted via a second borehole.

The lixiviant varies according to the ore deposit: for salt deposits the leachate can be fresh water into which salts can readily dissolve. For copper, acids are generally needed to enhance solubility of the ore minerals within the solution. For uranium ores, the lixiviant may be acid or sodium bicarbonate.

Fig.23 – Diagram of in-situ leaching for uranium

In-situ leach for uranium has expanded rapidly since the 1990s, and is now the predominant method for mining uranium, accounting for 45 percent of the uranium mined worldwide in 2012.

Solutions used to dissolve uranium ores are either acid (sulfuric acid or less commonly nitric acid) or carbonate (sodium bicarbonate, ammonium carbonate, or dissolved carbon dioxide). Dissolved oxygen is sometimes added to the water to mobilize the uranium. ISL of uranium ores started in the United States and the Soviet Union in the early 1960s. The first uranium ISL in the US was in the Shirley Basin in the state of Wyoming, which operated from 1961-1970 using sulfuric acid. Since 1970, all commercial-scale ISL mines in the US have used carbonate solutions. ISL mining in Australia uses acid solutions.

Ion exchange resin beads

In-situ recovery involves the extraction of uranium-bearing water (grading as low as 0.05% of U3O8). The extracted uranium solution is then filtered through resin beads. Through an ion exchange process, the resin beads attract uranium from the solution. Uranium loaded resins are then transported to a processing plant, where U3O8 is separated from the resin beads and yellowcake is produced. The resin beads can then be returned to the ion exchange facility where they are reused.

The advantages of this technology are:

- Reduced hazards for the employees from accidents, dust, and radiation

- Low cost, no need for large uranium mill tailings deposits.

In-situ leaching is the purest and cheap way of uranium mining. As opposed to traditional – opencast and underground – mining, at in-situ leaching there is no lifting of ores on the surface. Acid solutions work instead of miners on the depth from 100 to 900 meters under the ground depending on a type of a deposit. A special acid mixture through drilled holes is pumped in uranium-ore strata and dissolves the uranium rock. Then the solution saturated with uranium is pumped on special pipes in sorption columns where the uranium is extracted, and the acid is again injected into the ground for the subsequent work. It is so-called a closed cycle.

In-situ leaching is the most effective way of uranium mining. On power consumption and capital expenses it is cheaper in ten times than other ways. First, it does not change a geological structure of the earth interior; the ground on the surface is not taken; only uranium is extracted. Only 5 % of radio-activity is lifted on the surface. At traditional extraction technologies all 100 % of radio-activity is taken out on the surface together with the ground. Consequently, uranium-mining enterprisers are reluctant to bear additional costs for organization of protective measures on safety of workers and recycling of the ground. For comparison, employees of “Kazatomprom” working on the mines receive the irradiation doze, which does not exceed the level of natural background (5 mZv per a year). This parameter is twice below the norms established by radioactive safety regulations for Kazakhstan enterprisers – 20 mZv per a year.

The manufacturing technology on ISL mines does not include construction of storages for high radioactive waste products. It is unequivocally established, that the natural environment on the uranium deposits of Southern Kazakhstan has the unique ability to gradual self-restoration. The used acid changes only acidity of the environment. The acid mixtures developed by “Kazatomprom” dissolve only uranium and does not form additional radioactive waste products.

Kazakhstan oil and gas reserves

Origin. Kazakhstan has a group of sedimentary basins which differ in a geological structure, a stratigraphic range of the platform mantle, and oil and gas potentials.

The sedimentary basins found in the western regions of the country include the Precaspian basin, which has a thick Mesozoic-Cenozoic and Upper Paleozoic platform mantle; and the Mangyshlak-Usturt basin, which consists primarily of Jurassic-Paleocene sediments and contains a transitional Lower Mesozoic (Upper Paleozoic) structural floor.

The southeastern and eastern regions of Kazakhstan include the Priaralsky, Syrdarinsky and South Turgai Jurassic-Paleocene sedimentary basins, as well as a group of intermontane basins of different ages (Shu-Sarysu, Zaisan, Alakol and Ili basins).

The northern regions cover only a part of the west Siberian sedimentary basin.

To date, economic oil and gas reserves have been delineated only in the Precaspian, Mangyshlak-Usturt, Shu-Sarysu and South Turgai sedimentary basins while prospecting works have started in the prospective Priaralsky region.

A stratigraphic range of proven oil and gas potentials in the Precaspian basin covers the Middle and Upper Devonian; the Lower, Middle and Upper Carboniferous; the Lower and Upper Permian; the Triassic, Jurassic, Cretaceous, Paleocene and Neogene. Productive formations in the north Usturt basin are Middle to Upper Jurassic and Paleocene in age, while similar formations found in the Buzachi are of Jurassic and Lower Cretaceous ages. Within the limits of the south Mangyshlak basin, oil and gas have been found in the Triassic; in the Lower, Middle and Upper Jurassic; in the Cretaceous, Paleocene, and partially Paleozoic. Oil-and-gas-bearing formations in the South Turgai basin are Jurassic and Lower Cretaceous in age, and are locally restricted to the weathering crusts of the Paleozoic basement. Oil fields are absent in the Shu-Sarysu basin, while gas deposits are confined the Upper Devonian, Lower Carboniferous and Lower Permian.

Looking at the above, it implies that Kazakhstan sedimentary basins have different geological conditions and a wide stratigraphic range of oil and gas, especially in the Precaspian basin.

Using the adopted scheme of oil and gas geological zonation, these sedimentary basins have been grouped into the following oil-and-gas-bearing (OGP) and gas-and-helium-bearing (GHP) provinces: Precaspian, Northen Caucasus-Mangyshlak, Turan, Central Kazakhstan, East Kazakhstan and, partially, West Siberian provinces.

The Precaspian (GHP) province is associated with the Precaspian basin. Its south-southeastern boundary has been established in the zone where the reflections of the seismic horizon П3 become extinct; this has accounted for why the northern part of the Buzachi uplift and the boundaries of Northern Usturt have been recognized as belonging to the Precaspian basin.

The North Caucasus-Mangyshlak (OGP) province includes the Mangyshlak-Buzachi, South Mangyshlak and Central Caspian oil-and-gas-bearing districts within the limits of the republic.

The Turan (OGP) province covers the North Usturt, South Turgai and Central Aral oil-and-gas-bearing and potential oil-and-gas-bearing districts as well as the Sredne-Syrdarinsky basin which offers little prospects for oil and gas exploration.

The Central Kazakhstan gas-and-helium-bearing province encompasses the Shu-Sarysu gas-and-helium-bearing district as well as the Tengiz basin that has little exploration potentials.

The quoted scheme of oil and gas geological zonation is based on available geological-geophysical materials and, despite the controversies over the boundaries of a number of provinces, may be used in resolving practical issues related to oil and gas exploration in the immediate future.

Based on the division of Kazakhstan’s territory into oil-and-gas-bearing zones, the West Kazakhstan district includes two oil-and-gas-bearing zones: the Northwestern edge zone where oil and gas are found in subsaline and intrasaline layers; and the Central Precaspian internal zone where oil and gas as well as potential zones are associated with supersaline formations. The Northwestern oil-and-gas-bearing regions (the External near-edge, the Edge and the Internal near-edge zones) where the following zones of oil-gas accumulation have been established: Chinarev, Prigranichnaya, Karpov; Teplov-Tokarev, Darin-Derkul, Mezhstupnaya; Karashiganak-Koblandinskaya.

Zones of oil-and-gas accumulations are associated with specific lithologic-stratigraphic complexes and structural elements, and play different role on the oil and gas balance of the region. The largest hydrocarbon (HC) reserves have been delineated in the Internal oil-and-gas-bearing near-edge zone within the limits of the Karashiganak-Koblandinsky zone of oil-gas accumulation where they are associated with Karachiganak – the only large oil-gas-condensate field delineated here so far.

Reserves. Among the countries of the former Soviet union, Kazakhstan possesses the largest deposits of liquid hydrocarbons after Russia. According to data of the State commission on mineral reserves of the Republic of Kazakhstan, mineable reserves of oil in the country are evaluated in 4,1 billion tons (30 billion barrels) including deposits on land (more than 4 billion tons). Gas condensate reserves are assessed in 300 million tons.

According to the data of Ministry of oil and gas of the Republic of Kazakhstan the proved reserves of oil and gas condensate in the country reach 39,8 billion barrels (approximately 5,3 billion tons). At preservation of the current production level and invariable volume of the reserves the oil and gas extraction in the country can last within 70 years.

According to data of Oil&Gas Journal, in January, 2014 Kazakhstan on oil reserves was the 12th in the world: 30 billion barrels, or 4,1 billion tons that makes 1,8 % of world's reserves.

According to data of Ministry of oil and gas of the Republic of Kazakhstan the reserves of hydrocarbons in Kazakhstan can double at the expense of prospecting of deep-laying horizons of the Near-Caspian hollow. At present, the geological prospecting and investigation were carried out only in, so-called, pre-edge zones of this basin.

Kazakhstan has 15 sedimentary basins; five of them are used for commercial purposes (they contain more than 65 % of the oil extracted in the country).

The subsequent increase in the Kazakhstan resource base can be realized due to development of shelf deposits. The traditional oil-producing regions of the country cannot compensate the cutback in production in the future.

Kazakhstan oil companies

There are many oil companies in the Kazakhstan territory – from large transnational corporations to small private companies. The largest companies are TengizShevroil, Karachaganak, Petroleum Operating, NC KazMunayGas, CNPC AktubeMunayGas and others.

Oil mining in Kazakhstan in 2008 made 70 mil tones including Tengiz – 17,3 mil, Karashaganak – 11 mil, Uzen’ – 6,5 mil, Zhanazhol – 6,3 mil, Kumkol’ – 5,8 mil, Kalamkas – 4,2 mil, Karazhanbas – 2 mil, Kenkiyak-nadsolevoy – 2 mil, Severnoye Buzachi – 1,8 mil, Zhetybay – 1,2 mil, Karakuduk – 1,2 mil, Alibekmola – 0,4 mil, Kozhasay – 0,3 mil, Asar – 0,2 mil, Tasbulat – 0,1 mil and others.

A share of oil companies on the Kazakhstan market

KazMunayGas is a Kazakhstan national oil-gas company on mining, exploration, processing and transportation of petroleum.   The credit side of KazMunayGas includes exploration and mining of KazMunayGas, KazakhOil-Aktyube (67 %), MangistauMunayGas (50 %), TengizShevroil (20 %), KazMunayTeniz (20 %), Kashagan (16,81 %), PetroKazakhstan (33 %) and others. The KazMunayGas total oil mining in Kazakhstan in 2008 made 18,7 mil t.

Kazakhstan oil companies – Caspian Tristar (Myortvy Kultuk – 50 %), Aday Petroleum (Aday), ZhalgiztyubeMunay (Zhalgiztyube), Aral (Arys), TolkynNefteGas (Tolkyn), KazPolMunay (Borankol), KhazarMunay (Severnoye Pridorozhnoye), ANAKO (Kyrykmyltyk – 81 %), YemirOil (Yemir), Aydan Munay (Blinovskoye), NC Kol’zhan (Tuzkol’, Northwest Kyzylkiya), Galaz&K (Northwest Konys), Aktau Tranzit (Zhangurshi, Tyubejik), KazNefteChim-Kopa (Tagan Yuzhny) and others.

Russian oil companies in Kazakhstan are Lukoil and RosNeft. The credit side of Lukoil: Tengiz (2,5 %), Karashyganak (15 %), Severnoye Buzachi (25 %), Karakuduk (62,5 %), Arman (50 %), Kumkol’ Severny (33 %), Kozhasay (33 %), Alibekmola (50 %), Tyub-Karagan (50 %), Atashskiy (50 %), Zhambay Yuzhny (12,5 %).

RosNeft: Kurmangazy (50 %).

The total petroleum extraction of all Russian companies in Kazakhstan in 2008 made 4,166 mil t.

American and European companies in Kazakhstan: Chevron (Tengiz – 50 %, Kashagan – 16,81 %, Karashyganak – 20 %); Eni (Karashyganak – 25 %, Kashagan – 16,81 %); Total (Kashagan – 16,81 %); ExxonMobil (Tengiz – 25 %, Kashagan – 16,81 %); Royal Dutch Shell (Kashagan – 16,81 %, Arman – 50 %); British Gas (Karashyganak – 20 %); Repsol YPF (Zhambay Yuzhny – 25 %); Petrom (Tasbulat, Aktas, Turkmenoy); Maersk Oil (Dunga); Jupiter Energy (Vostochny Akkar); LP “Kor-Tazh” (Northwest Zhetybay); Lancaster Petroleum (Kumsay, Kokzhide).

The share of American and European companies on the Kazakhstan petroleum market in 2008 was 42 %.

Chinese companies started to work on the Kazakhstan market in 1997 when CNPC bought AktyubeMunayGas (Zhanazhol, Keniyak), then in 2001 CNPC bought 50 % of Buzachi Operating LTD (Severnoye Buzachi), in 2006 CNPC bought PetroKazakhstan, and the Chinese group CITIC bought 50 % of KarazhanbasMunay (Karazhanbas). In 2009 CNPC bought 50 % of MangistauMunayGas. In 2013 CNPC bought a share of ConoccoPhillips (16,8 %) of KazMunayGas in Kashagan.

The total oil mining of all Chinese companies in Kazakhstan in 2008 made 17,2 mil t.

Asian companies in Kazakhstan: Inpex (Kashagan – 7,56 %), Mittal Investments (Karakuduk – 37,5 %, Severnoye Buzachi – 25 %, Zhambay Yuzhny – 12,5 %), JNNK (Kulandy – 50 %), ONGG Videsh Ltd (Satpayv – 25 %), KazakhturkMunay (Saztyube, Yelemes – 49 %).

Oil extraction

The general 17-year-old oil extraction of the independent Kazakhstan has made 696,4 million tons. On the end of 2012 British BP has estimated oil reserves in Kazakhstan at level of 30 billion barrels or 3,9 billion tons that makes 1,8 % of the world's reserves.

Oil extraction at Kashagan was stopped in September, 2013, in two weeks after the beginning, because of gas leak. Almost right after the extraction resumption in October, 2013 other leakage was found out. The analysis, which was implemented within several months, has revealed presence of numerous microcracks in the pipeline formed as a result of effect of the oil gas with high sulphur content on the metal. The operator of the project, consortium North Caspian Operating Co (NCOC), in April, 2014 confirmed the necessity of full replacement of the gas pipeline and the oil pipeline on the deposit which general range makes about 200 kilometres.

Geological reserves of Kashagan are estimated in 4,8 billion tons of oil. The general oil reserves make 38 billion barrels including mineable reserves – about 10 billion barrels, reserves of natural gas – more than 1 billion cubic metre.

In accordance with the results of 2017 the minister of national economy has declared about the record volume of oil extraction for all history of Kazakhstan – 86,2 million tons.

Oil processing

Now there are three oil refining plants (Atyrau, Pavlodar, Shymkent) in Kazakhstan). Total capacity of three basic Kazakhstan oil refining plants on oil processing in 2019 makes 16,6 million tons per year.

Table 10 – Information about Kazakhstan oil-refining plants

| ORP | Controlling shareholder | Processing capacities, mil t | Volume of processing (2015), mil t | Depth of processing (2015), % | Oblast | Start-up |
| --- | --- | --- | --- | --- | --- | --- |
| Atyrau | KazMunayGas | 4,9 | 4,868 | 59,2 | Atyrau Oblast | 1945 |
| Pavlodar | KazMunayGas | 6,0 | 4,811 | 72,5 | Pavlodar Oblast | 1978 |
| Shymkent | PetroKazakhstan | 5,2 | 4,493 | 75 | Turkestan Oblast | 1985 |

Prospects of the Kazakhstan oil branch

The government of Kazakhstan pins its hopes on the North Caspian project, namely with Kashagan. After the beginning of industrial extraction of Kashagan oil Kazakhstan will be included into 5 largest oil-producing countries of the world.

The second prospect of Kazakhstan oil branch is Kurmangazy. Unlike the neighbouring deposit Kashagan, Kurmangazy is an over-salt deposit. It will considerably facilitate the development of Kurmangazy as its exploitation will not demand the hole making through a salt layer.

Fig.24 – Petroleum extraction

Also, there are some prospects in Middle Caspian Sea: it is the Block-N. The Block-n is a group of oil and gas structures in Middle Caspian Sea – Nursultan, Sary-Arka, Akmola, Al-Farabi, Ulytau, Samal, Akbota and Ketik.

The Aral region is also prospective. These are structures of Zapadny Kulandy and Vostochny Kulandy.

Share of the Kazakhstan gas resources in the world reserves

Resource industrial potential. A share of Kazakhstan in the world gas reserves makes approximately 1,1 % and the country is the 15th in the general rating. The explored and estimated resources of natural gas (taking into account the opened new deposits in the Caspian shelf) make about 3,3 bln cubic m, and potential resources reach 6-8 bln cubic metre. A share of the republic in the world volume of natural gas extraction is 0,9 %. A considerable part of the gas produced in the republic – more than 40 % – is extracted at the oil gas condensate deposit Karachaganak. Oil of practically all large developed oil and gas deposits in Kazakhstan contains dissolved gas with raised content of hydrogen sulphide and other sulphurous compounds. And the share of associated petroleum gas reaches 50 % and continues to increase together with the quickly growing petroleum industry. One of the basic problems of development of the resource industrial potential of the branch is removal of sulphur compounds from extracted oil and gas with the subsequent recycling of obtained sulphur, its purification and selling on foreign product markets. One of variants of solving this problem is return pumping of sulphur-containing gases in producing horizons of the deposits for maintenance of strata pressure and increase in efficiency of extraction of liquid components of hydrocarbon raw materials.

Fig.25 – Petroleum deposit

Reserves. A feature of the explored gas reserves in the republic is that practically all the deposits and, especially, new largest deposits extract natural gas jointly with oil and condensate.

Fig.26 – Karachaganak deposit

Resources of industrial categories of free gas (in a gas cap) are explored in more than 70 gas, gas-condensate and oil-gas deposits in volume of 2 billion cubic metre. About 30 deposits are now developed. A considerable part of the natural gas resources is situated in Atyrau Oblast (about 43 %), Mangistau Oblast (29 %), Zapadny Kazakhstan Oblast – 19 %, Aktyubinsk Oblast – 5 %. The major part of natural gas resources (more than 60 %) is concentrated in deposits Karachaganak and Tengiz. Expected Kashagan gas reserves are estimated in 227 billion cubic metres.

Coal resources

The coal industry plays the extremely important role in the national economy. There is no a manufacture where coal would not be applied. Coal is necessary for ferrous metallurgy, railway transportation, thermal power stations, for all branches of the national economy.

Coal is not only a fuel. It serves as raw material for the chemical industry. Aromatic oils for preparation of alcohols, aspirin, salol and other medicines are produced at processing of products of coal distillation.

A fragrant vanillin powder, benzene used for manufacture of explosives, and aniline paints for colouring fabrics are also produced from coal.

The iron and steel industry widely applies coke – an artificial solid fuel produced from baking bituminous coal. Coke is obtained at heating of special grades of coals (so-called, coking coals) in coke furnaces at high temperature (up to 700oС) without access of air. Coking is implemented in special coke-oven batteries consisting of 45-60 coke furnaces connected together.

At coking a number of auxiliary products (coke gas, pitches, ammonia, benzene) are formed; they represent a mixture of different hydrocarbons, mainly aromatic hydrocarbons. Then benzene is fed to rectification and forms new products – aviation gasoline, toluene, xylol, solvents.

Coke is used for melting cast iron in blast furnaces. The coal pitches formed during the coking process finally give about 200 new chemical substances. Among them: artificial liquid fuel, synthetic gasoline, medicines, solvents, motor fuel, lubricants and many other things. Distillation of coal permits us to produce ammonium chloride, ammonia, saltpeter, paints, sugary substances, naphthalene, salicylic acid and other products.

Artificial gasoline can be obtained at hydrogenation of coal (saturation of its molecules with hydrogen), mineral wax – at dissolution of coal.

Plastics, which have wide application in technics and household use, are produced from coal using complex physical and chemical processes. Acetylene, formed at decomposition of solid mineral fuel, when its further processing gives an artificial fibre – nitrilactone, which is applied for manufacture of fabrics. During this process, varnishes, motor fuel, plastic, artificial rubber are also formed. The ability of coal to form diversified products at its processing, which are absolutely dissimilar to the initial material is almost boundless.

It is necessary to note, that the chemical products of coal distillation are frequently better and much cheaper, than expensive substances of natural origin, for example, artificial aniline colours, which are applied instead of expensive and rare natural painting substances.

One of the major methods of coal processing is its burning directly under the ground, so-called, in-situ coal gasification. For the first time this way was offered by D.I. Mendeleev in 1888. He proved the practical opportunity of coal transformation directly under the ground into combustible gases, which then can be transported on the ground surface.

Fig.27 – Coal formation scheme

The coal burning is carried out using the air fed in a mine. The gases, formed as a result of this process, can be used as a fuel for metallurgical furnaces, boilers of power stations and also as the initial raw material for its subsequent processing into valuable chemical substances.

Kazakhstan coal resources makes 3,3 % of the world ones. On the general reserves of bituminous and brown coals, Kazakhstan is the 9th in the world (the 3rd in the CIS) and one of the  largest manufacturers of coal in the world (11th place). The general Kazakhstan coal reserves make 162 bn t; reserves of bituminous and brown coals are approximately identical. More 300 coal deposits are explored now in the territory of Kazakhstan. They are non-uniformly situated in the Kazakhstan huge territory. The main coal reserves are in five largest coal basins of Central and Northern Kazakhstan (Karaganda, Ekibastuz, Pavlodar, Kustanay, etc.) consisting of the deposits of different ages and structures. The explored coal resources exceed the expected ones in some times.

The Karaganda coal basin has an area of more than 3600 square kilometers. It is a main coal Kazakhstan base. The Karaganda coal is coked; that is its quality is very high. 80 coal strata have been explored. Their total bed thickness is 120 m. The general coal reserves are 45 bn t. The second important coal basin – Ekibastuz – is situated between Saryarka and Priyertysskaya plain.

Table 11 – Characteristic of Kazakhstan coal basins

| # | Deposit | Oblast | Combustible mineral |
| --- | --- | --- | --- |
| 1 | Karaganda basin | Karaganda | Bituminous coal, brown coal |
| 2 | Ekibastuz basin | Pavlodar | Bituminous coal |
| 3 | Ubagansky basin | Kustanay | Brown coal |
| 4 | Maykyubensky basin | Pavlodar | Brown coal |
| 5 | Teniz-Korzhunkulsky basin | Karaganda | Bituminous coal |
| 6 | Yaysansky basin | Aktyubinsk | Brown coal |
| 7 | Kendyrlyk deposit | Semipalatinsk | Bituminous coal, brown coal and combustible slates |
| 8 | Suzak basin | Turkestan | Brown coal |
| 9 | Kuuchekinskoye deposit | Karaganda | Bituminous coal |
| 10 | Kaynaminskoye deposit | Pavlodar | Bituminous coal |
| 11 | Batyrshasorskoye deposit | Pavlodar | Bituminous coal |
| 12 | Jamantuzskoye deposit | Karaganda | Bituminous coal |
| 13 | Koksengirskoye deposit | Kokchetav | Bituminous coal |
| 14 | Lenger deposit | Turkestan | Brown coal |

Fig.28 – Types of coal

Karatau phosphorites

According to various sources potential world reserves of phosphorite and apatite ores are estimated from 84,5 to 174 billion tons. Phosphorite reserves in Kazakhstan amount to approximately 9-10 bn t.

The Karatau basin of fine-grained phosphorites, located in the north-eastern branches of the Maly Karatau Mountain Bridge in South Kazakhstan, is the largest basin both in the Republic of Kazakhstan and in the world. Width of the Karatau phosphorite field is 20-25 km.

Expected reserves of micro granular phosphorite in the Karatau basin make 3 billion t. (in term of P2O5 – 740 million t.). The Aktyubinsk basin of nodular phosphorites according to prediction contains 5 billion t of phosphorite (in term of P2O5 – 500 million t).

The Karatau basin includes 45 fields on the area of 2,5 ths sq.km with the production center in Zhanatas. Main reserves (≈ 80 % of phosphorite) are concentrated in six largest deposits of the South Kazakhstan mining basin – Zhanatas, Koksu, Kokjon, Aksay, Chulaktau and Akzhar – and make about a third of the explored resources in the territory of the CIS. P2O5 content in the ores is 24-26 %. The main part of the phosphorite represent grains with a size of 0,1-0,2 mm. The phosphate raw materials of Karatau have different structures: from Ca10P6O24F2 to Ca10P4C2O22(FOH4). A basic mineral of the deposits is fluorcarbonateapatite – Ca10P5CO23(F,OH)3. Resources of phosphate raw materials in Kazakhstan represent sedimentary phosphorites.

Fig.29 – Phosphorite deposits of Karatau Basin

Besides the Karatau deposits, large resources of phosphate raw materials are available in the Aktyubinsk phosphorite-bearing basin located in the Northwest Kazakhstan where industrial strata consist of nodular phosphorites. The most studied deposits are Chilisaysky, Kandagagsky, Bogdanovsky and New Ukrainian ones. Main minerals of the phosphorite ore are quartz and phosphate.

In accordance with technical requirements to the phosphate raw materials used for acid and thermal processing, the Karatau ores are subdivided into three main types: rich, ordinary and poor ones. Rich phosphorite ores are bedded in surface zones of deposits of Zhanatas, Kokjon and Koksu, contain on average 28÷30 % of P2O5, 7÷10 % of SiO2, 4÷8 % of CO2, up to 2,5÷3,5 % of MgO, 1÷1,5 % of Fe2O3, up to 1% % of Al2O3. They are suitable for directly acid processing. They represent high-quality phosphorites and include an insignificant quantity of chalcedony, quartz and carbonates. The phosphate component in these ores contains in fluorcarbonateapatite such as franсolite.

According to MgO content the rich ores are divided into low-magnesia, magnesia and high-magnesia ores, which contain less than 1,9 %, 1,9÷2,4 % and more than 2,4 % of MgO, respectively.

The ordinary phosphorite contain 23÷25 % of P2O5. Quantitative ratios of other components in them are not constant. Kokjon and Aksay ores are characterized by increased content of carbonates (to 8÷10 % of CO2) and rather low content of silicon dioxide (10÷15 % of SiO2). Chulaktau, Zhanatas, Koksu deposits contain carbonate- siliceous, siliceous and pelitomorphic-siliceous ores, which are characterized by high content of insoluble residue (15÷25 %) and lower carbonate content (5÷7 % of CO2). A feature of these ores is fine interpenetration of phosphate and siliceous minerals.

Fig.30 – Map of Kazakhstan phosphorite deposits

The poor phosphorite ores contain 15÷23 % of P2O5 and represent siliceous and phosphate rocks containing 18÷23 % of P2O5, 25÷30 % of SiO2, 5÷7 % of CO2, 2÷2,5 % of Fe2O3, 1÷2 % of A12O3. Siliceous and slate ores contain 15÷21 % of P2O5 and 30÷40 % of SiO2. The poor (in accordance of P2O5 and impurity content) phosphorites of Zhanatas and Koksu deposits are considered as off-balance reserves. On the Zhanatas deposit they lie in the bottom of the top phosphorite stratum and in the top of the bottom one; in the northwest part of the Koksu deposit they constitute the top phosphorite stratum in full.

Fig.31 – The operating phosphorite deposit

The Karatau basin has been developing by the open way since 1946. The peak of its development has fallen in the 80th years of the XX century when there was the steady growth of phosphoric industry. During this period the top layers enriched with P2O5 have been exhausted. In the southeast and the northwest of the Zhanatas field the selective extraction of rich ores of a surface zone has been organized; these ores are subjected to the acid processing without concentration. By the beginning of the 90th the top phosphorite packs on the majority of the fields has been generally used up, i.e. there was gradual exhaustion of the reserves of rich and ordinary ores composing industrial layers. At the same time, lower powerful phosphorite strata of Zhanatas and Koksu deposits, which are composed of poor ores, were not extracted.

The lower stratum of Zhanatas field includes the phosphate and siliceous pack; its southeast part contains the quartz-bearing phosphorites containing 22-26 % of P2O5 and 28-33 % of SiO2. The northwest part of the pack is composed of pelitomorphic and siliceous phosphorite with thin interlayers of phosphatized and carbonated flints containing 15-20 % of P2O5 and to 35-50 % of SiO2. All deep part of the ores is characterized by increased content of carbonates.

Packs, which are a part of the lower layer of the Koksu deposit, contain phosphorite, phosphate and siliceous slates. Poor ores of Koksu deposit have increased content of silicates.

Now, Karatau phosphorites are relatively widely involved in the production. At the same time, there are many new technologies suggested for processing of low-grade phosphate raw materials with low content of P2O5 and significant content of impurities: 21-24 % of P2O5, 2,5-4,0 % of MgO, 2,3-3,0 % of R2O3, 10-11 % of CO2.

Fig.32 – Novodzhambulsky phosphorus plant in Taraz (Kazfosfat)

After the deep industrial recession in the middle of 90th, associated with the violation of economic relations as a result of the collapse of the USSR and insolvency of many industrial enterprises, the Government of Kazakhstan since 1999 adopted a policy of development of the phosphorus industry. As a result of attraction of investments LLP “Kazphosphate” has been organized, which united all enterprises of phosphate industry into one complex. At present, due to continuous operation of the mining complex “Karatau” and the mine “Zhanatas”, Novodzhambulsky phosphorus plant and Mineral fertilizer plant stably operate and produce products. In connection with the exhaustion of top rich phosphate strata the mining complex “Karatau” would be aimed, first of all, on the development of lower layers consisting of poor ores.

Fig.33 – Yellow phosphorus (Novodzhambulsky phosphorus plant)

Besides a low P2O5 and high SiO2 content the composition of balance ores is also characterized by a high content of magnesium oxide and sesquioxides; for this reason such the raw material is unsuitable for the acid processing. However, the low-grade phosphate rock due to its low cost and large reserves is the most promising and cost-effective one. Thus, application of the non-standard phosphorites for manufacturing phosphorus-containing products is one of the most important tasks of the phosphorus industry.

Characteristics of the productive horizons for major deposits of Karatau Basin are represented in Table 12.

Zhanatas deposit, situated in 90 km to the north-west of Karatau town, is the largest producing field of the Karatau.

Table 12 - Characteristics of deposits of Karatau Basin

| Deposit | Horizon thickness, m |  |  |  | Efficient horizon |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low-magnesium lime | Silicic | Efficiency | Ferrousmagnesium lime | The number of industrial mine | Average capacity of mine, m | Average Р2О5 content, % |
| Chulaktau | 5-8 | 7-8 | 15-20 | to 1 | 1-2 | 3-10 | 24 |
| Aksay | 7 | 4-5 | to 40 | to 1 | 1-2 | 13-15 | 23-24 |
| Koksu | 6-10 | 4-5 | more 30 | - | 2-3 | 4-10 | 24,5-25 |
| Zhanatas | 2,5-3 | 7-18 | more 60 | - | 2 | 7-17 | 24-25 |
| Kokdzhon | 2-20 | До 2,5 | more 25 | 3-5 | 1 | to 14,4 | 25-27 |

Phosphorites of Zhanatas deposit are identical in whole. These are dense gray, dark gray and black micro granular rocks with density of 2.5-3 g/cm3. The main mineral is fluoro carbonate apatite. Other phosphorus-containing minerals are chalcedony, quartz and, sometimes, feldspar.

According to content of the basic substance – P2O5 (% mass) there are four main types of phosphate ores: rich (more than 28.7 %), ordinary (23-28,7 %), poor (15-23 %), phosphate-siliceous (10-15 % of P2O5, 40-55 % of SiO2). In turn, the rich ores in accordance with magnesium content (% mass) are subdivided into low magnesia (2), magnesia (2-2.5) and high magnesia (more than 2.5) phosphorites. The rich ore is used to produce ground phosphate rock and for acid processing, the ordinary ones – for electro-refining, the poor ores are applied in a mixture with rich ores.

Akzhar deposit is, currently, one of the largest one according to its reserves. Akzhar phosphates are low-quality raw materials. They are characterized by a low P2O5 content (15-19 %) and presence of a considerable quantity (to 40 %) of impurities, in particular SiO2. Furthermore, they are characterized by strong mutual intergrowth of minerals included in their composition, it does not allow adequately efficiently and cost effective to concentrate the raw material using flotation and other methods.

Phosphate rocks of Kokdzhon field stretch in length and are divided into 4 deposits: Kistas, Kesiktobe, Atkum and Araltyube. There are phosphorites of two types: the first pack – a phosphate-containing shale (P2O5 from 5 to 21 % and insoluble residue from 20 to 70 %) and the second pack composed of high-quality and carbonate phosphorite (P2O5 content is 20-30 %, insoluble residue 3-15 %).

Fig.34 – Operating phosphate deposit

Koksu is a promising field of the Karatau basin intended for development. The thickness of the phosphate sediments includes 6 packs.

The phosphate shale pack contains phosphorites interbedded with clay-siliceous shales; P2O5 content is from 7 to 20-33 %.

The slate pack is composed of thin clay-siliceous shales with interbedded siliceous phosphorite; P2O5 content is 7-8 %, insoluble residue – 65-67 %.

The average pack of the phosphate rock is composed of strata of alternating clay-siliceous shales; P2O5 content is 13-14 %, insoluble residue 50 %.

The top phosphorite pack consists of granular carbonate and carbonate-siliceous phosphorites with interlayers of phosphorite agglomerate. Content of P2O5 makes 24-30 %, insoluble residue – 9-23 %.

POTASSIUM ORE RESERVES

Nowadays, more than ever, there is a long-felt need to increase world of potassium fertilizers. Potassium salts, according to some experts, have become an original new "rare earths". Today they are consumed by the whole world. This market, unlike most others, has been developing steadily, without significant downturns for several decades. More than 95 % of potassium salts mined in the world is used to obtain potassium fertilizers, and the rest part is used for production of detergents and various chemicals.

Although potassium enterprises traditionally belong to the chemical industry and fertilizer production, in their essence and technology they are more similar to mining enterprises, because their main production assets are mines, and their main wealth is deposits of potash and other salts. In the global potash industry, Kazakhstan traditionally plays an important role, occupying a special place both in potassium salts’ mining and in the amount of explored reserves and forecast resources. At present, there is no production of potassium fertilizers and salts in Kazakhstan, which leads to a high demand for potassium salts. Another fact testifies the large potential of Kazakhstan’s deposits: salts are deposited quite close to the surface, so that open extraction methods can be used for their mining.

The potash ore reserves in Kazakhstan amount to about 6 billion tons and are represented by three largest deposits located in the west of the country: the Zhilyansk polyhalite ore deposit in the Aktobe region, the Chelkar and Satimola carnallite and sylvinite ore deposits in the West Kazakhstan region (Fig.35). The Chelkar potassium ore deposit is located 98 kilometers on south-east of Uralsk and about 160 kilometers north of another deposit, Satimola; it covers an area of about 779 square kilometers. In terms of orography, the territory is located in the northern part of the Caspian Caspian Sea region lowland and belongs to the typical steppe zone, with all its inherent features. The relief of the Chelkar uplift has an asymmetrical structure. The highest heights of the upland surface are located in its northern part. To the south, its surface gradually decreases and imperceptibly merges with the flat steppe spaces. The northern slope of the uplift by a rather steep ledge ends in the direction of the lake basin and is markedly dissected; its highest point, Mount Sassai, has an absolute height of 94 m, and the surrounding uplifting steppes have the form of a flat plain with the surface height of the order of 18-20 m. In the lake area, the dome most clearly appears in the relief in the form of a ridge-hilly upland, framing the lake from the south. To the south, east and west the relief gradually flattens and turns into a plain. The Basin of Chelkar Lake, as already noted, is an inter-dome compensatory downwarping.

Fig. 35 – Reserves of potash ores in Kazakhstan

The lake water with a normal mark of +16.7 m, is slightly mineralized. Its water surface occupies about 200 km2 with a length from north to south of more than 18 km and a width of 15 km. The depth in some places reaches 13.5 m. In the 1950s and 1960s, geological exploration at this field was carried out under the conditions of common economic space of the USSR.

Fig.36 – Shelkar Lake (Kokshetau, Kazakhstan)

Revealed as a part of the salt strata of Chelkar salt dome structure of rich boron-bearing rocks, layers of high-quality sylvinites, as well as carnallite and almost monomineralic bischofite rocks, allowed to promote the Chelkar uplift to a number of very promising objects of complex mineral-salt raw materials. Of the salt minerals known in the Chelkar uplift, the main ones are halite, anhydrite, sylvine, carnallite, kizerite, bischofite; in the borate group there is boracite, which is especially widespread, as well as calibroite, sulfoborite, hilgardite, ginorite, preobrazhenskite, hydroboracite. The boreholes in the salt layer of the Chelkar uplift revealed a diverse complex of rocks, among which the main ones are rock salt, anhydrite rock, sylvinite, carnallite rock, kieserite rock, and bischofite rock. Large concentrations of borates, confined mainly to rock salt, kieserite rock, and anhydrite rock and less often to sylvinite, are also encountered here. The described types of salt rocks are met on the Chelkar uplift with numerous wells in various areas. It should be noted that there are various transitional rocks – sylvinite-carnallite, halite-carnallite, halite-carnallite-bischofite, etc., which are a layered alternation of carnallite, bischofite and other rocks. As already noted, in the process of study, more than 600 wells were drilled to a depth of 1,200 meters on the uplift area. Nevertheless, the factual material on which all constructions can be based to get representation about ​​the geological structure of the Chelkar uplift, mode of occurrence of salt rocks, the mineralogical composition and quality of potassium and boron bearing horizons is not enough. Since they are harmless and the performance of any production with the use of potassium salts is much higher, the need for the production of this fertilizer is relevant.

Presently the company LLP “Batys Kaliy” is working on the industrial development of this deposit. “Kazakhstan Potash Corporation”, which owns in Kazakhstan subsoil use rights for three potash deposits - Zhilyansk, Chelkar and Satimola - also intends to develop the potassium fertilizer production. Since 2011, “Kazakhstan Potash Corporation” has invested $ 631.6 million in Kazakhstan. Taking into account the life term of projects and the situation in the global potassium fertilizer market, investors expect that the business will be fairly stable. In 2016, the import volume of potassium fertilizers in Kazakhstan amounted to $ 14 million (Fig. 37).

Fig.37 – Import of potassium fertilizers

To achieve leading positions in tough competition, it is necessary to have a well-developed infrastructure, railway and sea transport complex and corresponding logistic systems.

Belarus plans to help Kazakhstan to set the development of these minefields. This was announced at the opening of the Belarus-Kazakhstan Economic Forum in Minsk. In addition to a large-scale project for the development of potassium salt deposits, there are about 30 joint Belarusian-Kazakh projects at various stages of development, including the creation of productions for quarry and agricultural machinery and food industry products.

The task of the research is to choose ways of processing carnallite from the Chelkar deposit and to search for new technologies of potassium fertilizers to meet the needs of both the domestic and foreign markets.

One of the main minerals of Chelkar ore is carnallite. Carnallite is a salt consisting of two types of compounds, potassium and magnesium chlorides, which is defined with the formula KCl∙MgCl2∙6H2O. It is a white crystalline product with a pinkish or greyish tinge; it is incombustible, fire and explosion proof. All existing methods of carnallite processing are based on greater solubility of magnesium chloride in comparison with solubility of potassium chloride; they are divided into methods of complete and incomplete dissolution. The first methods consist in the complete dissolution of carnallite ore by hot circulating liquor, which contains a significant amount of MgCl2 and little amount of KCl. When the obtained solution is cooled, most part of the potassium chloride crystallizes (mixed with NaCl). The mother liquor is evaporated and cooled; at that carnallite crystallizes. Production of potassium chloride by the flotation enrichment of potash ores is based on the flotation or flotation-and-gravity separation of water-soluble ore minerals in a saturated salt solution. In this case the sylvin or other potassium minerals are usually foam products. The main stages of the manufacturing process include the following operations:

- Dry and wet grinding of the ore;

- “Attritioning” of clay impurities from ore particles in special mixers;

- Separation of the clay sludge by mechanical desliming in hydrocyclones and hydroseparators;

- Treatment of salt pulp with special reagents including depressors in order to suppress sludge foaming and sorption of flotation reagents on the surface of clay particles (the most common treatment is sludge combining mechanical desliming of the main mass of clay particles with subsequent processing of the pulp with reagents-depressors);

- Treatment of saline pulp with flotation reagents, which are fed into special mixers, partly, directly into flotation machines. The flotation reagents include: the main component — the collector, in which water soluble aliphatic amines, depressants and deactivators (silicic acid, polyacrylamide, etc.) and foaming agents (pine oil, alcohols, dodecylamines, etc.) are used in the sylvite flotation.

Considering the deficit of potassium fertilizer production in China, estimated at 5-6 million tons of potassium chloride, Chinese companies with serious government support are involved in almost all potash projects in Laos, Thailand, Kazakhstan and Uzbekistan, setting up dumping prices for the subsequent sale of potassium fertilizers to their country in advance at a price of 250-320 dollars. The chemical industry is a basic and socially significant sector of the economy, having a significant impact on the formation of macroeconomic indicators of the Republic of Kazakhstan. However, there are certain difficulties in developing new deposits. Thus, the production of potassium salts is characterized by a complex combination of factors.

On the one hand, there is a constantly growing population of the planet and an increase in the consumption of mineral fertilizers, on the other hand, there is growing competition among fertilizer producing countries. On the one hand, the cost of raw materials is still low, for example, compared to oil, on the other hand, the market is saturated with potassium chloride fertilizers.

On the one hand, there is a high country concentration of deposits of potassium salts, on the other hand - the active development of their own and "foreign" deposits of countries-importers of potassium fertilizers.

Further development of the industry can be carried out in several directions:

- increasing the complexity of raw material processing to obtain new types of export products through the introduction and development of new innovative technologies and the development of scientific research and technologies;

- involvement in the processing of carnallite ores of the Chelkar deposit and technogenic mineral formations, which will expand the raw material base and increase the level of research and development and design work;

- creation of industries aimed at potassium fertilizer production for the needs of the domestic market with the subsequent focus on foreign markets. Considering that the competition in world markets on the processing the raw materials is very high, and the profitability of raw materials supply is growing, government intervention is necessary to avoid technical backwardness;

- in the absence of demand in the domestic market, government policies should promote exports to foreign markets; at the same time, create conditions for ensuring the consumption of products in the domestic market;

- in the case when products will be in great demand in foreign markets, a restriction should be envisaged for the export of raw materials by introducing differentiated export duties in order to create knowledge-intensive industries in the country;

- it is necessary to constantly adjust the development plans of large companies and enterprises, taking into account the needs of the internal and external markets;

- provision of protection of companies’ rights and, in general, entrepreneurship in the development of the chemical complex, for investing in projects and long-term investments in research and developmental works.

The performed review of the potash ore reserves in Kazakhstan and known methods of processing carnallite ores showed that the explored Chelkar deposit has not yet moved to the mining stage. This market in Kazakhstan is free; there is a need for domestic production of potassium chlorine-free fertilizers in default of such production, for refusing from export of these products. It has been substantiated that the Chelkar deposit development based on introduction of innovative technologies for the mining and processing the salts will make it possible to cover the needs for potassium fertilizers of both domestic and foreign markets.

PART 2. Complex using of raw materials – a scientific principle of mineral resources rational exploitation

Concept about a non-waste technology

Zero waste is a philosophy that encourages the redesign of resource life cycles so that all products are reused. Any trash sent to landfills and incinerators is minimal. The process recommended is one similar to the way that resources are reused in nature. A working definition of zero waste, often cited by experts in the field originated from a working group of the Zero Waste International Alliance in 2004.

“Zero Waste is a goal that is ethical, economical, efficient and visionary, to guide people in changing their lifestyles and practices to emulate sustainable natural cycles, where all discarded materials are designed to become resources for others to use. Zero Waste means designing and managing products and processes to systematically avoid and eliminate the volume and toxicity of waste and materials, conserve and recover all resources, and not burn or bury them. Implementing Zero Waste will eliminate all discharges to land, water or air that are a threat to planetary, human, animal or plant health.”

Fig. 38 – Industrial enterprise – a source of air, soil and water pollution

In industry this process involves creating commodities out of traditional waste products, essentially making old outputs new inputs for similar or different industrial sectors. An example might be the cycle of a glass milk bottle. The primary input (or resource) is silica-sand, which is formed into glass and then into a bottle. The bottle is filled with milk and distributed to the consumer. At this point, normal waste methods would see the bottle disposed in a landfill or similar. But with a zero-waste method, the bottle can be saddled at the time of sale with a deposit, which is returned to the bearer upon redemption. The bottle is then washed, refilled, and resold. The only material waste is the wash water, and energy loss has been minimized (see container deposit legislation).

Zero waste can represent an economical alternative to waste systems, where new resources are continually required to replenish wasted raw materials. It can also represent an environmental alternative to waste since waste represents a significant amount of pollution in the world.

In industry this process involves creating commodities out of traditional waste products, essentially making old outputs new inputs for similar or different industrial sectors. An example might be the cycle of a glass milk bottle. The primary input (or resource) is silica-sand, which is formed into glass and then into a bottle. The bottle is filled with milk and distributed to the consumer. At this point, normal waste methods would see the bottle disposed in a landfill or similar. But with a zero-waste method, the bottle can be saddled at the time of sale with a deposit, which is returned to the bearer upon redemption. The bottle is then washed, refilled, and resold. The only material waste is the wash water, and energy loss has been minimized (see container deposit legislation).

Zero waste can represent an economical alternative to waste systems, where new resources are continually required to replenish wasted raw materials. It can also represent an environmental alternative to waste since waste represents a significant amount of pollution in the world.

The movement gained publicity and reached a peak in 1998-2002, and since then has been moving from "theory into action" by focusing on how a "zero waste community" is structured and behaves. The website of the Zero Waste International Alliance has a listing of communities across the globe that have created public policy to promote zero-waste practices.

The tension between zero waste, viewed as post-discard total recycling of materials only, and zero waste as the reuse of all high level function remains a serious one today. It is probably the defining difference between established recyclers and emerging zero-wasters. A signature example is the difference between smashing a glass bottle (recovering cheap glass) and refilling the bottle (recovering the entire function of the container).

The tension between the literal application of natural processes and the creation of industry-specific more efficient reuse modalities is another tension. Many observers look to nature as an ultimate model for production and innovative materials. Others point out that industrial products are inherently non-natural (such as chemicals and plastics that are mono-molecular) and benefit greatly from industrial methods of reuse, while natural methods requiring degradation and reconstitution are wasteful in that context.

Biodegradable plastic is the most prominent example. One side argues that biodegradation of plastic is wasteful because plastic is expensive and environmentally damaging to make. Whether made of starch or petroleum, the manufacturing process expends all the same materials and energy costs. Factories are built, raw materials are procured, investments are made, machinery is built and used, humans labor and make use of all normal human inputs for education, housing, food etc. Even if the plastic is biodegraded after a single use, all of those costs are lost so it is much more important to design plastic parts for multiple reuse or perpetual lives. The other side argues that keeping plastic out of a dump or the sea is the sole benefit of interest.

Fig.39 – Waste dump close by a human settlement

Many dumps are currently exceeding carrying capacity. This is often, mistakenly used as a justification for moving to Zero Waste. Others counter by pointing out that there are huge tracts of land available throughout the USA and other countries which could be used for dumps. This is no more of an argument against the need for Zero Waste than is the former an argument for Zero Waste. The underlying need to move to a society designed along Zero Waste principles arises from the huge waste of resources that is inherent in poorly made, short-lived articles and production processes. The locus of the most egregious wasting takes place as articles are built and processes are run wastefully. The actual placing of a now useless item in a dump is barely the icing on the cake, in terms of the waste it represents. Poorly conceived proposals, that appear with a dismaying regularity on the Internet, to blithely destroy all garbage as a way to solve the garbage problem, make use of the common delusion that it is the garbage itself which is the problem. These proposals typically claim to convert all or a large portion of existing garbage into oil and sometimes claim to produce so much oil that the world will henceforth have abundant liquid fuels. One such plan, called Anything Into Oil was promoted by Discover Magazine and Fortune Magazine in 2004, even though it absurdly claimed to be able to convert a refrigerator into "light Texas crude" by the application of high pressure steam. Zero Waste analysis, which is long on scientific results and short on spectacular claims, receives no such promotion by the media.

Fig.40 – The waste sent to landfills may be harvested as useful materials, such as in the production of solar energy or fertilizer for crops. It may also be reused and recycled for something that we can actually use

Zero Waste is a goal, a process, a way of thinking that profoundly changes our approach to resources and production. Zero Waste is not about recycling and diversion from landfills but about restructuring production and distribution systems to prevent waste from being manufactured in the first place. The materials that are still required in these re-designed, resource-efficient systems will be reused many times as the products that incorporate them are reused. Deconstruction can be described as construction in reverse. It involves carefully taking apart a building to maximize the reuse of materials, thereby reducing waste and conserving resources. Deconstruction can capture materials and some components from the millions of buildings that are existing and that were poorly designed for high level reuse but it is not a favored approach from a Zero Waste point of view. Zero Waste favors the design of buildings as assemblages of high level components, not their creation from rough materials such as lumber, cement or plaster. The details are not worked out yet but to the extent that entire rooms, entire walls, roofs or floors or entire utility systems can be pre-built and installed as completed components, that will be the goal of Zero Waste design. Until buildings are built as components capable of later dismantling, deconstruction is a stop-gap process that the United States can use to minimize the waste of building materials.

Fig.41 – Gas emissions of an industrial enterprise

Market-based, legislation-mediated campaigns like Extended Producer Responsibility (EPR) and the Precautionary Principle are among numerous campaigns that have a Zero Waste slogan hung on them by means of claims they will ineluctably lead to policies of Zero Waste. At the moment, there is no evidence that EPR will increase reuse, rather than merely moving discard and disposal into private-sector dumping contracts. The Precautionary Principle is put forward to shift liability for proving new chemicals are safe from the public (acting as guinea pig) to the company introducing them. As such, its relation to Zero Waste is dubious. Likewise, many organizations, cities and counties have embraced a Zero Waste slogan while pressing for none of the key Zero Waste changes. In fact, it is common for many such to simply state that recycling is their entire goal. Many commercial or industrial companies claim to embrace Zero Waste but usually mean no more than a major materials recycling effort, having no bearing on product redesign. Examples include Staples, Home Depot, Toyota, General Motors and computer take-back campaigns. Earlier social justice campaigns have successfully pressured McDonald’s to change their meat purchasing practices and Nike to change its labor practices in Southeast Asia. Those were both based on the idea that organized consumers can be active participants in the economy and not just passive subjects. However, the announced and enforced goal of the public campaign is critical. A goal to reduce waste generation or dumping through greater recycling will not achieve a goal of product redesign and so cannot reasonably be called a Zero Waste campaign.

Re-shaping people’s resource use pattern is a challenge beyond the scope of piecemeal and reactive laws.  Policy instrumentalism reflects policymakers tendency to build on what already exists and is characterized by minimal disturbance of the current state.  This cannot achieve the targets of Zero Waste at a national level. What is needed is a shift from government (formal organizations and procedures of the public sector) to governance (array of governmental and non-governmental institutions).  The government alone does not have the cognitive breadth to determine how to reach the ultimate target of Zero Waste which requires the involvement of businesses, NGOs, the public and the state in governing. Governance is “ultimately concerned with creating the conditions for ordered rule and collective action” and to achieve Zero Waste, a departure from waste management based simply on waste disposal ideology is necessary. The role of the state is to act as central steering mechanism and governance networks that bring together government; public and market actors are viewed as important to achieve Zero Waste similar to many other environmental goals. The Johannesburg World Summit on Sustainable Development emphasized these partnerships between different stakeholders in the environment and the establishment of Public-Private partnerships is seen as the best way to achieve sustainability.

Fig.42 – Unregulated wastes discharge

BASIC DIRECTIONS OF NON-WASTE AND LOW-WASTE TECHNOLOGIES

“A non-waste technology represents such a production method when all raw materials and energy are used the most efficiently and fully in a cycle: raw material resources – manufacture – use – secondary resources, and any effects on the environment do not violate its normal functioning».

Development of non-waste manufactures is a rather complex and long process; an intermediate stage is a low-waste manufacture. A low-waste manufacture is such manufacture which effect on the environment doesn’t exceed the level admissible by sanitary-and-hygienic norms, i.e. maximum permissible concentration MPC. Thus according to technical, economic, organizational or other reasons a part of raw materials can turn into waste and is fed to a long storage or burial.

Principles of non-waste technologies

- A basic principle is a systemacy principle. According to it each separate process or manufacture is considered as an element of a dynamical system, i.e. all industrial production in a region and at a higher level as an element of an ecological-economic system in whole, including both the material production and other economic activities of people, and the environment (populations of live organisms, atmosphere, hydrosphere, lithosphere, biogeocenosises, landscapes), and also a person and its habitat.

- Other major principle of creation of a non-waste manufacture is complexity of application of resources. This principle demands maximum use of all components of raw materials and a potential of energy resources. It is known, practically all raw materials are complex, and at the average more than a third of its amount is accompanying elements which can be extracted only at its complex processing.

- A basic way of this purpose achievement is development of new technological processes and manufactures and improvement of existing ones. One of the examples of such approach to organisation of a non-waste manufacture is recycling pyrite cinders – sulfuric acid production wastes. Now pyrite cinders are completely applied in cement manufacture. However, the most valuable components of pyrite cinders – copper, silver, gold, and most of all iron are not used.

Requirements to non-waste manufacture

To improve existing technological processes and develop essentially new technological processes it is necessary to observe the following requirements:

- realisation of production processes at a minimum possible number of technological stages (apparatuses) as waste formation and raw materials loss take place on each a stage;

- application of continuous processes allowing to use raw materials and energy most effectively;

- increasing a unit capacity of apparatuses (to the optimum value);

- intensification, optimisation and automation of production processes;

- development of energotechnological processes. Combination of energetics with technology allows us more completely to use energy of chemical transformations, to save power resources, raw materials and other materials and to increase productivity of units. An example of such manufacture is a large-capacity manufacture of ammonia according to an energotechnological scheme.

Mining industry

In the mining industry it is necessary:

- to introduce technologies of complete recycling wastes formed both at the open-cast mining and at the underground one;

- to wider apply geotechnological methods of development of mineral deposits, striving for extraction only target components on the earth surface;

- to use non-waste methods of concentration and processing of natural raw materials on the place of its mining;

- to wider apply hydrometallurgical methods of ores processing;

- at mining and processing of ferrous and nonferrous metals ores to introduce using large-tonnage dump solid waste of mining and concentrating manufactures as building materials, filling of worked-out areas in mines, roadway coverings, wall blocks etc. instead of specially extracted mineral resources;

- complete processing of all blast-furnace and ferroalloy slags, and also essential increase in scales of processing of steel-smelting and nonferrous metallurgical slags;

- sharp reduction of fresh water consumption and decrease in waste water volume at the expense of further development and introduction of waterless technological processes and zero-discharge water supply systems;

- increase in efficiency of existing and new developed processes of catching by-components from waste gases and waste water.

Nonferrous metallurgy:

- acceleration of introduction of resource-saving autogenous processes including melting in a liquid bath; it will allow not only to intensify the processing of raw materials and reduce energy consumption, but also considerably improve the atmosphere around operating enterprises at the expense of sharp reduction of a waste gas volume and produce high concentrated sulphur-containing gases that can be applied for sulfuric acid and elemental sulphur manufacture;

- rapid development and introduction of new progressive low-waste and non-waste processes including direct ore-reduction and non-coke processes of steel manufacture, powder metallurgy, autogenous processes in nonferrous metallurgy and other perspective technological processes aimed at reduction of emissions in the environment;

- expansion of application of microelectronics, automated management systems in metallurgy with the view of economy of energy and materials, and also the control of waste formation and their reduction.

Chemical and petroleum-refining industry

- to wider use the following technological processes: oxidation and reduction with application of oxygen, nitrogen and air; electrochemical methods, membrane technology for separation of gas and fluid mixtures; biotechnology, including manufacture of a biogas from organic products residues.

Classification of chemical wastes

Wastes obtained in chemical industry can be classified or according to an aggregate state or methods of their recycling. The most generally accepted and clear  is classification in accordance with an aggregate state.

Solid wastes are the wastes produced in a form of powders, dust, ingots or indurations. These are cinders, ash, dust and soot particles, wastes of plastic and rubber, mineral metal-containing residues after concentration of ores, organic decomposing residues.

Liquid wastes are the wastes almost completely consisting of a liquid phase and containing dissolved in water or other solvents salts, alkalis, acids, and also impurities of suspended particles. A content of suspended particles does not exceed a level at which there is their sedimentation. Liquid wastes are mobile also should be pumped over easily by the pumps used for transportation of liquids in the chemical industry. Concentration of soluble substances should not  exceed a solubility limit at which there is their crystallization from a solution under usual conditions. First of all, there are the industrial sewage which can be dumped in the sewerage without preliminary processing at a factory; the sewage polluted by toxic and poisonous compounds and demanding the special processing (the water containing acids, alkalis, chlorides, fluorides, bromides, dissolved metals, toxic organic  compounds etc.); fat organic solvents and organic toxic liquids (manufacture of pesticides).

Gaseous wastes are the gas emissions of industrial  furnaces, drying equipment, gas emission of chemical processes etc. To them concern various smokes, the gases possessing a smell, containing disperse solid or liquid particles in the form of a fog, and also containing NO*, SO2, НСl, HF, etc., steams of organic substances, the steam-air mixes polluted by toxic impurities.

In a separate category it is necessary to mark out wastes  received in a kind of slimes which can be ranked neither to liquid, nor to solid wastes. Usually they represent amorphous or fine-crystalline masses containing from 20 to 80 mass % of water and badly giving in to transportation without preliminary processing (drying, freezing etc.). Here concern: residues of filtration and sedimentation processes; slimes received at neutralization or special processing of liquid wastes; slimes received at clearing of faecal sewage; pitches, sour and viscous tars, the residual oil products  received in organic synthesis.

Considering the system of chemical wastes classification it is necessary to note such their important characteristic, as  toxicity. According to this parameter chemical wastes can be subdivided on harmless, toxic and especially toxic. The concept "toxicity" means a degree of influence of a chemical waste on wildlife. First of all, it concerns  a person, and then animals and vegetation. Practically all chemical wastes are toxic, and their influence depends on a substance dose to which a person or natural sphere adjoins. Besides, many chemical substances possess ability to accumulate both in an organism, and in environment and by that to raise the toxic action with the course of time. Warehousing and burial of chemical toxic wastes leads to ingress of toxic components at evaporation and washing away in environment where there is their circulation. Obviously, classifying the chemical wastes, it is necessary to specify degree of their toxicity for which definition it is necessary to know a chemical composition, already available concentration of these substances in environment, ability to accumulation and biological degradation. Toxic and especially toxic wastes should be carried to a special category of the “specific” wastes requiring special methods of neutralization before their dump or burial.

Sources of wastes formation

Manufacture of a ready chemical product usually consists of numerous consecutive operations, since  processing of raw materials and finishing conditioning of a received  product. For this aim physical, mechanical, chemical and physicochemical processes are used. As a rule, at each stage  of a product manufacture wastes or residues in the form of dust, gases, liquids, slimes, and – frequently - in the form of raw material particles are formed. At crushing of ores, for example,  the formed dust can be taken by means of filters and is returned in process; the same concerns to a stage of drying and conditioning of firm products. Different wastes, polluting an atmosphere and reservoirs, are formed in various branches of the national economy.

The chemical industry concerns to one of those  branches of the national economy in which great volumes of various wastes are formed.

Now from 4 million identified chemical  substances about 30 thousand is produced in the industrial  scale. Thus in each separate technological process the certain type of wastes is formed which in most cases cannot be used in a place of their formation. The technology of inorganic substances plays a special role in  the chemical industry as it is made the most large-tonnage products used in many  branches of a national economy. Scales of manufacture of the basic chemical industry on many products exceed millions of tons.

Along with finished goods the basic chemistry also  makes significant amounts gaseous, liquid, solid and slime wastes. So, acid manufacture allocates in atmosphere sulfur oxides, nitrogen oxides, and fluoric compounds; at manufacture of sulfuric acid from iron pyrite a cinder is formed. In ammoniac manufactures drains and the gases  containing NH3, dissolved ammoniac salts etc. are formed. At preparation of raw materials and drying of finished goods (for example, mineral fertilizers) the considerable quantity of dust is formed. At manufacture of potash fertilizers more than 75 % of raw materials go over in wastes in the form of clay slimes and halite wastes.

Thus, it is possible to draw a conclusion, that the basic chemical  industry, making the large-tonnage products, in the same time emits in the environment huge quantities of the various liquid, gaseous and solid wastes which recycling is necessary both from the point of view of complex  use of natural resources, and from the point of view of protection of a person and a nature against their negative influence.

Complex processing of raw material

Chemical, mining, extractive, metallurgy industries produce the considerable quantity of solid waste. Dumps reduce the areas of ground fund and poison soil and water sources. The agricultural industry is a source of pollution by fertilizers and pesticides.

One of the most large-tonnage wastes of the chemical industry is a pyrite cinder formed at the sulfuric acid manufacture. More than 40 million tons of pyrite cinder was accumulated on sulfuric acid enterprises. To available stocks 7 million tons is added annually. The pyrite cinder consists of iron oxides, sulphides and oxides of other metals, quartz, aluminosilicates and pyrite. The pyrite cinder contains about 58 % of iron, 2 % of copper, small amounts of silver, gold and other valuable metals. Pyrite cinder can be used in the cement and glass industry. Complex processing of pyrite cinders with the high content of copper, zinc, lead, silver and sulphur by the chloridizing roasting is most perspective, because allow to take and to use rare and nonferrous metals. After extraction of valuable metals by chloridizing roasting the pyrite cinder represents raw materials for iron production.

As a chloridizing agent can be apply calcium chloride solution, formed at soda ash manufacture. Pyrite cinder is mixed with calcium chloride solution and granulated; the received granules are roasted at 1500˚С. There is a sublimation of nonferrous metals and formation of strong agglomerates - raw materials for the blast-furnace cast iron manufacture.

The significant amount of solid waste is formed in manufacture of potash fertilizers from sylvinite (mixes of sodium and potassium chlorides). On 1 ton of potassium chloride 1,8-2,6 tons of so-called halite waste (sodium chloride with impurity of potassium chloride and other salts) is formed. Warehousing of halite waste demands the big areas and result in salinization of soils, increasing of a content of mineral salts in underground waters. Halite waste can be processed on table salt. Now at factories of potash fertilizers more than 500 million tons of halite wastes are accumulated and 50 million tons annually increases. Their use makes no more 5,6 million tons in a year. Sometimes burial of halite wastes spend in the exhausted mines, but to solve a problem in such a way is impossible.

In manufacture of phosphoric fertilizers at the concentration of phosphoric raw materials by flotation the considerable quantity of concentration tailings is formed – 1,7-2 tons on 1 ton of phosphorus. Complex processing of this waste is necessary from the ecological point of view, and with the purpose of extraction of non-ferrous and rare metals (aluminum, titanium, vanadium, gallium, indium), and also valuable nonmetallic products, such as soda, potash, cement.

Other of the most large-capacity waste of the chemical industry is phosphogypsum dumped by the enterprises producing phosphoric acid and phosphoric fertilizers.

Enterprises of the nitric industry, manufacture of polymeric materials and many other chemical manufactures dump the considerable quantities of solid waste. Wastes of the chemical industry in the increasing degree are utilized as secondary raw materials. However a number of the chemical plants constructed without requirements of ecology, before their transfer into the closed production cycle still long time will increase capacity, and will dump increasing quantity of solid wastes. Even now at any chemical enterprise wastes cannot be hidden. It is necessary to utilize wastes completely in short terms, return them in economic circulation as secondary raw materials. It is necessary to neutralize and bury nonutilizable firm wastes. The arable and forest lands occupied with firm industrial wastes need to recultivation, i.e. to restoration of natural landscapes. For preservation of lithosphere it is necessary to make recultivation lands round the industrial enterprises. It is necessary to create the artificial ecological systems preventing migration of toxic substances in environment. Lands which are polluted at overburden operations of an earth surface for the purpose of mining, at laying of gas and oil pipelines also require restoration of an environment.

There are following methods of neutralization and burial of firm industrial wastes: biological oxidation; thermal processing; warehousing of wastes on an earth surface; burial of especially harmful wastes on the sites which do not have the economic value: ravines, open-cast mines, holes, trenches, chinks.

Biological oxidation is applied to neutralization of firm wastes, including the deposits formed in systems of biological clearing.

Thermal processing is the most reliable, but the outdated way of neutralization and recycling of firm wastes. Burning is carried out in high-temperature chemical reactors which represent the furnaces providing 1) good hashing for development of a contact surface of phases and for acceleration of external and internal diffusion of oxygen for the purpose of the maximum oxidation of an organic part of wastes; 2) the high temperature, sufficient for full neutralization of a toxic part of wastes. More often apply drum-type and chamber furnaces, but also cyclonic furnaces and furnaces with the fluidized bed of a solid metal. In drum-type furnaces wastes passes some temperature zones: preliminary drying, preparation for burning, ignition, burning, after burning. In the preliminary drying zone, apply exhaust kiln gases with temperature 800-1000˚С. In the preparation zone, partial distillation of volatile organic products occurs; the temperature of formed pitch reaches 300˚С – an ignition temperature of separate components of wastes. Burning of solid mass (in all types of furnaces) begins at 600˚С. Temperature in the burning zone is supported in limits 1100-1500˚С. Burning in cyclonic furnaces and fluidized bed furnaces occurs most full and intensively thanks to active hashing of a firm material with air.

A rather perspective way is thermal processing of solid wastes by the pyrolysis method which products can serve as a power carrier, and also as raw materials for organic synthesis. Pyrolysis is spent in vertical cylindrical furnaces. Heating is provided by means of an electric arch, high frequency current or application of solid heat-carriers – melts of salts, products of solid fuel pyrolysis (semicoke), etc. Pyrolysis is carried out at 300-900˚С depending on required structure of gaseous products. The structure of gases depends on structure of a processed waste and the oxygen content in a pyrolysis zone. For prevention of formation of soot and toxic products water steam is entered into a reactionary zone. The solid pyrolysis residue can be utilized as filler for plastics and rubbers, as a sorbent.

Solid waste is processed under a high pressure. Under its action the compacted, caked materials are formed which can use in the industry of building materials. This method is used for processing of wood waste, ash dumps, and waste of concentration of mineral raw materials.

In addition to the list can be use the modern leaching methods, low-temperature catalytic hydration, fire method with explosive combustion, electrical catalytic oxidation.

Burial of solid industrial wastes in superficial storehouses is the most widespread (while) way their isolation and partial neutralization. Such way leads to alienation of the big land sites and pollution of superficial and underground water. The basic type of superficial storehouses is slime storages which are built according to a cascade principle. Slime storages include a pool, banks, a dam and the drainage system. Drainage system protects soils under the storage from filtrational deformations and drains polluted waters from the storehouse for neutralization.

Burial of industrial toxic wastes on the sites which do not have economic value, is spent after their stabilization by processing by binding or cementing substances - liquid glass, cement mortars, bitumens. Received solid-phase wastes in the form of blocks are placed in open-cast mines, chinks, holes and other, natural and artificial hollows in earth’s surface. Such way is applied to wastes containing compounds of mercury, arsenic, cyanides, and also slightly radioactive wastes. Thus frequently toxic metals and substances continue to migrate in an environment. All the ways conservation and burial of solid wastes are unsafe, conduct to alienation of useful areas and are connected with considerable expenses. It is necessary to replace these methods with full recycling of solid wastes first of all as secondary raw materials. Methods of leaching and hydration successfully solve this problem.

Until recently many industrial wastes were subjected to liquidation. Such approach to solve this problem is obsolete. Only those waste products should be subjected to liquidation that cannot be practically used or processed into target products using modern methods. The majority of waste products are processed by means of physical and chemical techniques to extract useful components from them. As a result of it new solid residues or sludges are formed. Further processing of these waste products is inexpedient. Thus, a final stage of the process can be liquidation of a high toxic part of waste products or a final deposit; extraction of useful components from it is practically impossible.

In our country and abroad the following liquidation methods of waste products are practically used: dumps and sludge storages, sanitary earthen covering, burial or pumping in deep underground chinks, burning and pyrolysis.

Physical and chemical features of waste products

Metallurgical slags. A great bulk of waste products of metallurgical processes is formed in the form of slags.

Slags are products of a high-temperature interaction of initial materials’ components (fuel, an ore, fluxing agents and a gas medium). Their chemical composition and structure change depending on a dead rock composition, a kind of a melted metal, features of a metallurgical process, conditions of cooling, etc. Slags can be formed as a result of the following processes: at burning of low-calorie kinds of fuel without formation of a melt and at aluminothermic processes; at the partial fusion of initial components at fuel burning; at the full fusion of initial components (in the majority of metallurgical processes). In the latter case slags have almost homogeneous structure and contain a glassy phase. A reducing medium in metallurgical furnaces promotes formation of protoxide compounds of iron, manganese, and also sulphidic sulphur in slags.

Metallurgical slags are divided into slags of ferrous and nonferrous metallurgy.

Depending on a process’s character and a type of a furnace the ferrous metallurgy slags are subdivided into the following kinds: blast-furnace slag; steel-smelting slag (martin, converter, Bessemer and Thomas, electrosmelting); ferroalloys manufacture; cupola slags. Blast-furnace slag has the greatest output, it makes 0,6-0,7 t per 1 t of blast iron. At melting the output of slag is reduced: at the martin way – 0,2-0,3 t, Bessemer and Thomas methods – 0,1-0,2 t; at melting of steel in electric furnaces – 0,1-0,04 t (per 1 t of a product).

Amount of slags of ferroalloys manufactures and cupola slags is rather insignificant.

Slags’ output in the nonferrous metallurgy depends on an extracted metal’s content in an initial charge. At the fusion of copper concentrates with copper content 10-15 % in reflective furnaces a slag output makes 10-20 t per 1 t of metal, at the fusion of copper ores with copper content 1-2 % in mine furnaces – 50-100 t, at the mine fusion of oxidised nickel ores – 100-200 t.

A chemical composition of blast-furnace slags is basically composed of four oxides: СаО (29-30 %), MgO (0-18 %), А12O3 (5-23 %) and SiO2 (30-40 %). Also they contain small amount of iron oxides (0,2-0,6 %) and manganese oxides (0,3-1 %), and also sulphur (0,5-3,1 %). Steel-smelting slags are characterised by higher content of iron oxides (to 20 %) and manganese oxides (to 10 %).

A characteristic of the nonferrous metallurgy slags is a low content of СаО + MgO (7-13 %) and a high content of FeO (21-61 %). Except the basic components the nonferrous metallurgy slags can contain small amounts of residual (unrecovered) metals – copper, zinc, lead, nickel, etc.

At estimation of slags as raw materials for manufacturing building materials an important characteristic of their chemical composition is a lime factor (a basicity module) – a basic and acid oxides ratio.

A chemical composition considerably influences physical properties of slag melts, structure and properties of hardened slags. So, the increasing of calcium oxide content in slags causes rise in melting points and decrease in fluidity.

At a high temperature (more than 1300°С) presence of СаО reduces viscosity of a melt, and at a low temperature - sharply raises. MgO, MnO, FeO, SO3 reduce viscosity of a slag melt if their content is in certain limits. Increase of silica content above 40 % and also increase of aluminium oxide content lead to increase of melts' viscosity. Gas impurities result to decrease of melts’ viscosity.

The oxides containing in the slags form various minerals. According to analysis results, silicates, aluminosilicates, aluminates and ferrites hold a leading place.

Basic minerals of slow-cooled acidic blast-furnace slags are anorthite CaO∙Al2O3∙2SiO2, diopside CaO∙MgO∙2SiO2, in neutral and basic slags – helenite 2CaO∙Al2O3∙SiO2, okermanite 2CaO∙MgO∙2SiO2, mervinite 3CaO∙MgO∙2SiO2, dicalcium silicate 2CaO∙SiO2, solid solutions of okermanite and halenite – melilites, etc. A phase structure of steel-smelting slags is more complex than one of blast-furnace slags. Such components of slags as iron and manganese oxides, sulphur, etc. form solid solutions with the basic minerals, and at the considerable content they can be separated in the form of independent phases - ferriferous, sulphidic, manganous compounds.

At the slow cooling of slags along with formation of minerals their polymorphic transformations can occur that leads to disintegration and spontaneous transformation of slag’s pieces into a powder. There are silicate, ferriferous and other kinds of slags’ disintegration.

Silicate disintegration is a consequence of the polymorphic transformation of P-2CaO∙SiO2 into Y-2CaO∙SiO2 at temperature of below 525°С, accompanied by increase in the volume approximately by 10 %. This form of disintegration is observed at the content of calcium oxide 44-46 % in the slags. This form of disintegration can be prevented by fast cooling of slags and their granulation.

Ferriferous and manganous disintegration is caused by a volume increase at the interaction of iron or manganese sulphides with water and hydroxides formation. So the slags containing more 3 % of FeO and 1 % of sulphidic sulphur disintegrate. Crumbling of slags is possible as a result of free СаО and MgO hydration (limy and magnesia disintegrations).

Practically all metallurgical slags along with the crystallisation products contain in this or that quantity a glassy phase. The glass amount in dump slow-cooled basic slags is insignificant, and in granulated blast-furnace slags the glass content reaches 98 %. The glass is a thermodynamically unstable phase; it appreciably defines a chemical activity of slags. It is established, that slag glasses interact with water much more intensively, than crystals of minerals.

From all kinds of metallurgical slags the blast-furnace slags are most widely applied in the manufacture of building materials that is caused by their leading position in the general slags’ balance and also similarity of their structure to cement mixtures, ability to get hydraulic activity at fast cooling, etc. Great bulks of blast slags are received at melting of conversion and foundry pig-iron.

Fig.43 – Metallurgical slag dump

Blast-furnace slags are products of interaction of fluxing agents (calcium and magnesium carbonates) with a dead rock of an iron ore and a coke ash. Differences in structures of iron ores and coke in different regions of the country are caused the corresponding differences in the slags composition.

The most widespread way of slags’ processing is granulation which essence consists in quenching (fast cooling) of slag melts by water, steam or air and formation of glassy grains in a size to 10 mm as a result of it. Apply two ways of granulation: wet and semidry.

Fig.44 – Slag tipping

Wet granulation consists in the quenching of fused slag usually in ferroconcrete tanks in volume to 800 м3, filled with water, and its dispersion by formed steam, and also the gases escaping from the melts. Wet granulation installations are simple, have a high efficiency, and performance of a technological process demands low labour inputs. However the wet granulation slags have a high humidity (10-30 %) that leads to freezing during winter time, increase of cost of transportation, causes of considerable heat consumption for their drying.

The semidry granulation based on combined cooling of slags is more effective: at first slag is cooled by water, and then – by air. Thus a final humidity of granulated slag reaches 4-7 %.

From numerous semidry granulation installations hydro channel coolers are the most progressive now. They can be constructed both outside of a blast-furnace shop, and direct near a blast furnace. In such installations firstly a melt is given in a loading bath, and then – on a shoot where it is cooled by the water submitted under the pressure 0,7-0,8 MPa. Granulated slag is thrown by water on a distance to 20 m and taken out in a pulp receiver, and then it is fed on pipelines in a system of dehydrating bunkers, whence moves on a warehouse. Granulation of slags by the semidry way is possible also with using drum-type, hydraulic shock installations and granulation mills.

The maximum content of a glassy phase is observed in the wet granulation slags obtained on pool-type and channel cooling installations from high overheated melts (temperature above 1600°С). Such slags have also the highest chemical activity. At the wet granulation of slag’s melts there is a slow cooling of melts with the corresponding reduction of glass amount and chemical activity. Chemical (hydraulic) activity of slags is characterized by СаО amount in mg, absorbed by 1 g of slag during 28 days. The chemical activity of granulated blast slags can usually reach about 100 mg of СаО on 1 g of slag.

A way of processing and a cooling mode of slags influence on their physicomechanical properties.

Smaller mechanical durability of granulated slags in comparison with dump slags is the reason for their high grindability. Fine grinding of granulated slags require in 1,3-1,5 times less energy, than on grinding of dump slags.

In the majority of the countries granulate basically blast slags. The great bulk of granulated blast-furnace slags is applied in the slag-portland cement manufacture. They are also used to production of local non-clinker binding agents, slag basic concrete, mineral cotton, slag glassceramics products, as filler in cement and asphalt concrete.

Steel-smelting slags are characterized by a high lime factor, therefore at their cooling they are practically completely crystallized and almost don’t contain some glass. These slags are not granulated. They are poured in dumps and slowly cool.

Martin slags usually contain metal inclusions (1-3 %), therefore they are not granulated, and are poured in dumps. Characteristic of martin slags is a high lime factor that promotes their full crystallization. This kind of slags has a dense or porous structure.

The basic slags, being in dumps, fall to pieces owing to limy disintegration. Durability of martin slags is 80-150 MPa, they stand more than 200 cycles of frost resistance tests. Converter and electrosmelting slags usually contain decomposition products. Lumpy slags have a dense structure; an average density is 3100-3400 kg/m3. Compressive resistance is 60-130 MPa.

Cupola slags are formed at melting of foundry pig-iron in cupola furnaces. Its lime factor is low and, as a rule, does not exceed 0,8. These slags both in a granulated form and dump one contain glass.

Among slags of ferroalloy manufacture the slags formed at melting of ferrochromium and ferrovanadium with a high lime factor (1,6-1,8), and also acidic ferromolybdenum slags are of the greatest interest. Basic ferroalloys slags contain calcium orthosilicate of y-modification, and at cooling they are usually crumbled up in a slag powder. Slags of ferroalloys contain residual quantities of some elements (Сг, V, Мо, etc.) in the form of metals, oxides or other compounds.

Martin slags make more than 60 % of steel-smelting slags and slags of the converter manufacture make more than 35 %. About 30 % of steel-smelting slags and also slags of ferroalloys manufacture are processed for production of rubble, slag powder, and also they are used as mineral fertilizer. Ferrochromium slag is applied for deoxidation and alloying of steel. This slag represents a powder-like material with a high dispersion owing to disintegration at the polymorphic transformation of dicalcium silicate. Research has shown that using ferrochromium slag as an aluminosilicate and coloring component of a raw charge allows us to produce a Portland cement clinker of green colour, that essentially reduces a cost price of colored cement.

Among the nonferrous metallurgy slags copper-smelting and nickel slags have the greatest value for the building industry. Dump copper-smelting slags have black colour. They are not subjected to disintegration. The average density of slags makes 3300-3800 kg/m3, water absorption is 0,1-0,6 %, ultimate compression strength is 120-300 МPа. Nickel slags possess the same high indicators of physicomechanical properties, as well as copper-smelting slags. According to a chemical composition they concern to the acid slags. The nickel granulated slags, despite a glassy structure, practically do not possess hydraulic activity.

Aluminium (secondary) slags are received at the remelting of aluminium alloys. Their chemical composition is the following, %: КС1 – 38-59, NaCl – 11,4-34,1, СаСl2 – 3,0-4,2, MgO – 6,2-7,2, Аl203 – 6,5-12,6, SiO2 – 1,8-3,5. Water-soluble compounds in the slag make 75-85 % of weight. At the long stay of slags in water the water-soluble compounds are extracted. Average density of slags is 1800-2000 kg/m3. Their ultimate strength is 40-45 МPа.

For the present slags of the nonferrous metallurgy are applied in a small amount in the cement manufacture as a ferriferous component and an active mineral additive, and also at the production of mineral cotton wool and cast products. Potentially the nonferrous metallurgy slags are a perspective base of various building materials. Their output is 10-25 times as much an output of nonferrous metals.

Slime by-products. At the manufacture of aluminium and some other metals in considerable quantities wastes in the form of water suspensions of disperse particles, so-called slimes, are formed. For the building materials manufacture nepheline, bauxite, sulphate, white and mono calcium slimes have an industrial value. A volume of nepheline slimes makes more than 7 mil t per a year. According to a content of СаО, SiO2, Аl2O3, Fe2O3 they hold the intermediate place between Portland cement, blast-furnace slag and aluminous cement.

A mineralogical composition of slimes, except mono calcium slime, is characterized by prevalence of dicalcium silicate (50-90 %), and also presence of calcium aluminates and calcium ferrites. A significant amount of water in slimes leads to the partial hydration of minerals and formation of hydrosilicates, hydroaluminates and hydroferrites.

Nepheline (belite) slime is received at the extraction of alumina from nepheline rocks. Nepheline mineral represents sodium aluminosilicate (KNa3[AlSiO4]4). At the processing a nepheline concentrate is burned in rotary furnaces in a mix with limestone at temperature nearby 1300°С. Formed cake, consisting from phosphorus dicalcium silicate and alkaline aluminates, is subjected to crushing and extraction. At this process alkaline aluminates are separated from a deposit and are processed in alumina (a half-finished product for the aluminium manufacture) and soda products. The deposit after washing represents a coarse-dispersed suspension – nepheline slime, which chemical composition is the following (%): SiO2 – 26-30; Аl2O3 – 2,2-6,5; Fe2O3 – 2,1-5,5; СаО – 2-59; MgO - 0,2-1,8; Na2O + K2O - 1-2,5; burning loss – 1-5,5. From minerals a nepheline slime contains partially hydrated belite 2CaO • SiO2 (80-85 %), in a small amount dicalcium ferrite 2СаО • Fe2O3, tricalcium hydroaluminate, calcium and sodium aluminosilicates and calcium carbonate. At the extraction of alumina from nepheline rocks, each ton of finished goods gives 7-8 t of nepheline slime being a valuable raw material for the manufacture of Portland cement and nepheline cement, and also products of autoclave hardening.

Bauxite (red) slime is a waste product of the processing of a basic raw material for aluminium-bauxite manufacture. Ore bauxite minerals are aluminium hydroxides, and basic impurities – are silica SiO2, iron and titanium oxides. Alumina from bauxite is received by the wet alkaline way or by the sintering way. Both ways consist in the sodium aluminate preparation Na2O•Аl2O3 which is hydrolyzed in water with sedimentation of a crystal precipitate А1(OH)3.  The precipitate is washed out, dried and calcinated for the hydration water removal and the pure alumina production.

A characteristic of bauxite slimes is a high content of iron and aluminium oxides. A chemical composition (%): SiO2 – 10-18; Аl2O3 – 10-18; Fe2O3 – 24-45; СаО – 15-40; Na2O – 4-7. As opposed to nepheline slimes, bauxite slimes contain much less belite, but include larger quantity of calcium ferrites and alumoferrites, aluminates, aluminosilicates and sodium ferrites. Now the great bulk of bauxite slimes is discharged in dumps. A high content of iron oxides does not allow applying them as the basic raw materials to cement manufacture. Bauxite slimes are applied as a corrective ingredient at the Portland cement clinker manufacture, and also as an active mineral additive. Technologies of the ceramic and silicate brick production with the using bauxite slimes were developed. Dry bauxite slime is used as a filler of paints, mastics, plastic. Sulphate slimes are received at the replacement of soda by sodium sulphate Na2SO4 in the alumina manufacture by a sintering way. They are characterized by presence of the compounds containing sulphur of various oxidation degrees.

As well nepheline slimes, sulphate slimes can be applied as components of Portland cement raw mixes, for the manufacturing local slime binding agents and materials of autoclave hardening.

At the production of alumina from aluminosilicate rocks by a method of a high alkaline charge sintering mono calcium slime is formed as waste. 1 t of alumina produces nearby 4 t of such slime. In mono calcium slimes СаО content is much less, than in belite slimes (35-38 %), thus a ratio CaO : SiO2 is close to unity. This kind of by-products can be considered as a siliceous component of a raw mix in the Portland cement clinker manufacture.

Metallurgical works gives a significant amount of various iron-containing dusts and slimes. They with success can be applied as a ferriferous corrective ingredient in the Portland cement clinker manufacture. Ferriferous additives are also used at claydite (expanded clay) for the improvement of clay raw materials blowout and sintering.

Fine-dispersed wastes of the ferrosilicon production more than on 90 % consist of the amorphous silicon dioxide particles with a diameter less than 1 micron. This dust at the introduction it in concretes in a combination with a plasticizing agent allows to increase essentially durability or accordingly to lower the cement consumption. Efficiency of the addition of ferrosilicon production wastes at the manufacturing silicate bricks and porous concretes was also proved.

Utilization of metallurgical slags

Copper production plants. At present a volume of old slags on copper production plants makes 100 mil t. At the average the slags contain: Zn – 2 %, Cu – 0,37 % and Fe – 30 %. Slag dumps of copper production plants contain about 2 mil t of zinc, 335 ths t of copper and 27 mil t of iron.

Lead production plants. Now dumps of lead plants contain more than 10 mil t of the slag, which includes 8,7 % of Zn; 0,66 % of Сu; 1,46 % of Рb; 28 % of Fе. More than 900 ths t of zinc, 70 ths t of copper, 150 ths t of lead and about 3 mil t of Fe has been collected in these dumps.

Nickel production plants. More than 130 mil t of the slag containing 0,15 % of Ni, 0,026 % of Cd and 18,4 % of Fе are kept now in dumps of nickel production plants.

According to a chemical structure the slags can be divided on three types:

1. Slags of lead and tin melts and very small amount of slag of copper melts, containing light components – zinc, lead, tin – in such quantities, that their extraction is economically profitable without a complex processing of slag and extraction of all ingredients.

2. Slags of copper melts containing below 5 % of zinc and lead, with a low content of copper and a high content of iron. Processing of such slags can be expedient only at the complex extraction of volatile components, iron and silicate compounds.

3. Nickel slag and a part of copper slag in which a content of iron and non-ferrous metals is insufficient for their extraction, and their direct processing for utilization of the silicate part is possible.

Physical properties of metallurgical slags and their elemental composition play a relevant role both at melting of metals and at subsequent processing. At fusion of ores or concentrates we specify such a composition of slag that its formation and fusion require minimum fuel consumption and provides for minimum loss of metals with it. A technological scheme of slag processing also depends on the slag composition.

Phase composition and structure of dump slags

Copper slag. In the great majority of cases the copper slags contain a large quantity of iron and silicon and present alloys of ferrosilicates with insignificant additives of other oxides. The basic part of these slags is a binary mixture FеО-5SiO2.

A detailed study of 27 samples of slags of copper concentrates’ electric fusion using phase chemical, microscopic and X-ray methods shows the following structure: 30-36 % of SiO2 , 50-55 % of FеО, 4,6 % of А12O3, 0,3-0,6 % of Сu,  1,5 % of S. Acidity factor changes from 1,2 to 1,5.

In all samples (85 %) the basic mineral is fayalite including 90 % of Fе5SiO4. Except fayalite, the slag contains magnetite, glass, sulphide and elemental copper. Content of magnetite is 5 %, glass – 10 %, the total of sulphides (troilite, bornite, yellow copper ore) is less than 5 %. Free copper meets in the form of strings in copper globules.

At refrigeration of the samples the sulphides containing in the melts precipitate as microscopic corpuscles; 95 % have a size of 0,2-5 microns and 3 % - 20 microns (emulsion corpuscles). The corpuscles with a size of more than 20 microns represent globules.

Iron in the slags enters into the composition of fayalite and magnetite.

Lead slags. A basis of lead slags is a triple system FeO-5SiO2-CaO which contains 20-25 % of SiO2; 35-38 % of FеО; 8-12 % of СаО; 12-46 % of ZnО; up to 3 % of CrО3; 1,5 % of Сu; 2,5 % of Рb and 4 % of S. Phase analyses shows the presence of oxides (3 % of zinc spinel and to 8 % of vustite and magnetite), silicates (54 % of fayalite, 20 % of melilite and 9 % of soda-lime glass), zinc sulphides, bornite, halkozine, yellow copper ore and pirrotite.

Utilization of power waste

According to a physical state all waste products are divided into liquid, solid, gaseous, and also paste-like and vaporous waste products. Groups of  waste products are also classified according to a chemical composition (for example, flue cinder of thermal power plants (ТЭС) are subdivided into inactive, low-active and active cinders) and according to physical-mechanical properties in connection with variety of their properties and a wide range of changing their quantitative characteristics.

Fuel waste products of thermal power plant (TPP)

TPP use basically ash-rich coals, slates and peats, burning them up to a dust-like condition.  Fuel waste products in the form of flue cinder and slags are transported and stored in dumps. Ash volumes of thermal power stations reach 20-40 million m3 and occupy areas from 500 to 1000 hectares and more. Their average remoteness from ТPP is 7-9 km and more. On this distance it is necessary to lay pipelines for transportation of ash dump pulp, and these pipelines demand replacement each 5-7 years. About 1 % of electric power, generated by TPP, is spent for the ash and slag transportation. Capital expenses for construction of an ash dump removal system make 6-10 tg/ kW installed capacity. Annual expenses for the operation of existing ash dump removal systems and construction of new systems make 120-150 million tg. A level of using ash and slag in the country is insignificant and makes about 4 % of their annual output.

Stone coal-sedimentary rocks have a non-uniform structure which is defined by various combinations of organic products and mineral impurities, and also variety of organic mass. The highest ash level arises at the allochthonous accumulation of a vegetative material as flowing water simultaneously with vegetative residues brings the argillo-arenaceous material, polluting a vegetative material’s congestion in an accumulation place.

Fig.45 – Gas emissions of a thermal power plant

The Ekibastuz coal concerns to the most ash-rich coals, the average ash level in this coal makes 40-50 %, reaching to 52 % that defines a considerable content mineral impurities. Fused ash particles form slag which in a solidified condition represents a vitreous mass, coloured in a dark colour by iron oxide. Ash particles, which are removed by smoke gases from a combustion area, refer to a fly ash. The particles consisting of a fly ash and unburned fuel, taken out by smoke gases from boilers refer to flue.

According to a building classification, an ash is a product with a size less 0,25 mm, formed at the fuel burning. It is settled in fly-ash collectors (cyclones, electro filters) from furnace gases. A granular material concerns to slag with a size of particles more 0,25 mm, formed at the fusion of a mineral part of fuel. It is obtained at the separation of ash-slag dumps or at the processing of ash-slag mixture from dumps.

Ashes and slags are divided on flue ash and slags of the dry catching and removal, and also ash-slag materials of the wet catching and removal. Fuel waste products of ТPP are applied in various areas of the national economy; however one of the most perspective directions of their using is construction.

Flue ashes and slags are subdivided depending on:

- fuel’s kind and quality (coal, slate, peat). Coal ashes are divided on subgroups: anthracite, stone, brown-coal.

- a kind of preparation and conditions of fuel burning: ashes of dust-like and layer burning. Coal is usually burnt after crushing and grinding, and sometimes – without preliminary processing. In the first case the solid furnace wastes contain fine-dispersed ashes. These ashes of the dust-like burning contain less unburned fuel and differ by more uniformity of chemical composition, than at lumpy burning of fuel.

- according to a place of sedimentation of ashes in a boiler unit:

- ash blockage obtained at the burning of fuel on lattices;

- ash fell down on a hearth;

- sediments on a tube of a boiler;

- wall sediments;

- smoke dust carrying away by waste gases in a smoke pipe.

Fig.46 – Ash-and-slag wastes storage

The major part of smoke flue ashes is caught at the cleaning of furnace gases with using of special apparatuses. It is a flue ash. Smoke pipe ashes unlike the others have the most stable structure.

Dust collection method:

- a dry way – in electro filters and cyclones;

- a wet way – in wet cleaning cyclones.

Basic properties of ashes are defined by mineralogical and chemical structures, average density, density, dispersion, etc. A chemical composition of various ash-slag waste products includes over 100 minerals, however approximately only 40 minerals are more or less constant.

Basic ash-slag minerals can be subdivided into three groups:

1) minerals with constant hydraulic properties;

2) minerals which are activated at water processing and thermal processing;

3) inert minerals (incapable to hydrolysis or interaction with activators in temperature interval of 20-250°С and pressure of  0-4 MPa).

The first and the second groups have the greatest interest for builders:

- calcium and magnesium silicates (slags and ashes from burning of combustible slates, brown coals, peat);

- oxides (presence of free iron oxides  is characteristic for lumpy fuel slags). Slate, peat and some brown coal ashes contain Аl2O3.

Considering a mineralogical composition of ashes in whole, it is possible to notice, that they contain quartz of various modifications, alumina, mullite, various compounds of iron, sulphurous compounds and other substances in smaller quantities. The chemical composition characterizes activity of ashes. Ash’s activity is the ability to hardening in the presence of additives containing lime, or without them.

Activity of ashes was studied insufficiently. Literature data differently explain the ash activity. More often the reasons of ash high activity are connected with the presence of burnt clay minerals in their structure.

Waste of phosphorus industry

Kazakhstan is a republic with the developed chemical industry based on high-grade deposits of phosphorites, salts and other chemical raw material, the best part of which go to wastes. For example, ten million tonnes of phosphate-siliceous ores on the open-cast mine Zhanatas are stored in dumps. The dumps are increased with every year.

Waste of the phosphate ore flotation concentration is annually raised on 6 million tones. These wastes represent fine silica sand which can be used in the manufacture of lime-sand brick and glass, as well as a filler material for concrete products.

The most bulky waste product of the phosphorus production is a slag which is processed for obtaining a granulated slag. Complex studies have been carried out to determine the availability of phosphorus slag for production of building materials. One of the directions of phosphorus waste utilization is production of bonding concretes on the basis of a granulated phosphorus slag and nonconforming liquid glass. An effective direction of using the phosphorus slag is glass-ceramics manufacture. Complete processing of phosphorus slag requires considerable capital investments.

The phosphoric acid production is accompanied a large quantity of wastes too. The most significant waste product is phosphogypsum. Phosphogypsum is used in the cement industry. Combined processing of phosphogypsum with nepheline can be one of the ways of phosphogypsum application. A method of complex processing of a nepheline-limestone mixture is also known. As a result of this process the following products can be produced: alumina, cement, soda, as well as sulfur dioxide for sulphuric acid production.

Fluorine from a wet-process phosphoric acid can be separated as sodium silicon tetrafluoride and applied in the glass industry.

Now a technology of joint complex processing of vanadium ores and Karatau phosphorite is developed which is intended for manufacturing white phosphorus by an electro-thermal method. This method includes melting of phosphorites with coke and quartzite. As a result of this process the white phosphorus and slag are obtained, and also a by-product – ferrophosphorus; it contains 24-26 % of phosphorus. According to the suggested technology the replacement of quartzite (which is applied as a flux) by a vanadium ore allows us to use not only the basic component of the ore - silica, but to concentrate vanadium in the ferrophosphorus without some capital expenses.

Processing of ferrophosphorus, sludge, slag and waste gases

formed at phosphorus production

For clear understanding the processes taking place at phosphorus electro thermal reduction and sublimation, a reaction space of an ore-thermal furnace is divided on 5 zones.

The first zone is a zone of solid-phase processes. Its lower bound is defined by the beginning of melting the charge components – 1630K.

The secondary zone is a zone of solid-phase melting – 1570-1670К.

The third zone is a zone of a melt and solid coke particles chemical interaction.

The fourth zone is a zone of a melt with a stable chemical composition (zone of molten slag).

The fifth zone is a zone of liquid ferrophosphorus.

In the first zone the following processes take place: heat exchange between the gases formed in zone 3, heating of a charge, removal of crystallization and hydration water, decarbonization of minerals, chemical interaction of carbon dioxide with coke, condensation of alkaline metals sublimates and others.

Phosphorite melted in the second zone (its melting point is 1620-1670К) dissolves other mineral impurities and flows in bottom parts of a furnace. Zones I-III are the zones of all thermo chemical processes.

6Ca5(PO4)3F + 45C + 30SiO2 = 9P2 + 45CO + 29CaSiO3 + CaF2 + SiF4

Heat-exchange and mass-exchange processes take place in zones IV-V.

Depending on a composition of basic materials, 9-12 tonne of slag is formed per 1 tonne of phosphorus. The slag structure is complex. It contains oxides of Mg, Mn, Al, Si, Ca, Fe, alkaline metals and other elements. Basic components of the slag are CaO – 40-50 %; SiО2 – 35–40 %. The slag released from a furnace is processed by different ways. The major part of slag released from slag-sublimation tap holes is granulated and cooled by water directly in the furnace. For this purpose the slag from tap holes in the liquid state (1400-1450оС) is given on a slag-sublimation trunk entered in a granulation trunk, where it is mixed with water in the ratio 1:20. Slag pulp, formed in the granulation trunk, flows in a bunker. Separation of water from the slag takes place in the bunker. Granulated and dried slag from evaporation bunkers is loaded up to consumers. The granulated slag is used as an additive to cement clinker.

Fusibility and enthalpy of slags. Metallurgical slags consist of metal oxides. At melting the breakage of chemical bonds in a compound takes place not in a moment, but in a temperature interval between the beginning of softening and complete liquefaction of the slag. This interval for acid slags is 200-250оС, for alkaline slags – 30-80оС. Slag melting point is a temperature, at which its crystal phase disappears, but the total slag mass becomes dissolved-liquid.

Besides melting point at the process carrying out, calculation of fuel cost and heat balance the enthalpy of melting products (including the slag enthalpy) is used.

Slag viscosity. Viscosity characterizes a structure of slag and influences considerably on losses of non-ferrous metals with slags, kinetics of the melting process and a rate of fusion products separation.

There are some general regularities of the slag viscosity change which depend on its composition and temperature: in all cases, the viscosity of silicate melts rises with the increase of complex oxides content. The viscosity of melted slag decreases with the temperature rise.

Slag conductivity. Study of the slag’s conductivity allows us to obtain additional information about a structure and a character of bonds in melted slag. The slag conductivity value is very important for correct exploitation of ore-thermal furnaces and choice of the optimum operation mode.

The melted slag has, mainly, an ionic structure, and consequently, it has electrolytic conductivity. Ion mobility and conductivity are defined by dimensions of cations and anions and a degree of their interaction, consequently, conductivity of melted slag is determined, first of all, by its composition. Ion mobility considerably depends on temperature; with the temperature increase the melt conductivity rises as opposite to the metal conductivity. A FeO-SiO2-СаО system is a basis of the slag in case of electrosmelting copper and lead. The pure FeO conductivity is 304, 299 и 262 Оm-1∙сm-1 at 1500, 1450 and 1400оС. Electric conductivity of the iron silicate containing 20 % of SiO2 at 900оС is 3 Оm-1∙сm-1 and gradually rises to 4,5 Оm-1∙сm-1 at the temperature growth to 1300оС.

Conclusions:

- under conditions of reduction-sulphide processes the copper containing in melt can  be separated as matte (matte is an intermediate product used for production of nonferrous metals from their sulphides);

- considerable quantity of lead containing in slag as silicates can be extracted only at their decomposition, and small inclusions of lead sulfides and lead metal can be extracted from the slag only by distillation;

- zinc, dissolved in fayalite and as other forms, can be extracted only by distillation.

Use of a silicate part of the slag for the building materials production. Production of cast slag’s products.

Basic ways of application of waste slags to produce building materials include the following directions:

- production of makes, tubes, slabs, large block and so on from the molten slag;

- manufacturing heat-insulating materials;

- production of high-porous aggregates;

- manufacturing binding substances for high-density and high-grade concretes;

- production of slag glass-ceramic.

Ferrophosphorus is a by-product of phosphorus production (100-200 kg of ferrophosphorus is formed per 1 tonne of phosphorus). Ferrophosphorus is used in metallurgy for steel deoxidation and manufacture of special steel.

At the electro-thermal processing of phosphorite about 30 % of the charge incoming into a furnace is transformed in gaseous products. 2500-3500 m3 of gases or 3,5-4,5 tonnes of gaseous products is liberated at the processing of 1 tonne of phosphorus. The gases have temperature 280-500oС.

Phosphorus sludge is a by-product formed at the phosphorus condensation. It is a difficult-to-separate mixture of phosphorus, water and solid particles.

Vocabulary

A

Abrasion – истирание

Accept – принимать

Accompanying (associated) component – сопутствующий, попутный компонент

Accompanying mining – попутная добыча

Accordingly – соответственно

Accumulation – накопление

Achieve – достигать

Actuate – приводить в действие

Activator – активатор

Adhere – слипаться

Adhesion – слипание

Adit – штольня

Advance (or lead) – опережение

Aeration – аэрация

Aerial photograph – аэрофотоснимок

Agglomeration – окускование

Ahead – вперед

Allow – позволять

Alternately – попеременно

Armouring – армирование

Aperture – отверстие

Appear – появляться, возникать

Approach – сближение

Approbation – опробование, апробация

Approximation – приближение

Apron conveyor – пластинчатый конвейер

Arc – дуга, арка

Arch – дуговой

Asbestine – асбестовый

Asbestos – асбест

Ascending stream – восходящий поток

At once – сразу

Attract – притягивать

Auger – шнек

Auger drilling rig – шнековая буровая установка

Auger type – шнековый тип

Auto truck – автокар

Auxiliary – вспомогательный

Axial channel – осевой канал

Axis – ось

B

Bandage – бандаж, бинт

Bar – колосник

Bar grate – колосниковая решетка

Bar screen – колосниковый грохот

Bath – ванна

Batching – дозирование

Basic – опорный, основной

Basin – бассейн

Bauxite area – бокситоносный район

Beam – луч

Bearing – подшипник

Bed – постель

Belt – лента

Belt conveyor – ленточный конвейер

Bench – уступ

Benefit – выгода, польза, прибыль, эффект

Bin – бункер

Binding agent – вяжущее

Bit – коронка

Bit of short drilling – коронка дробового бурения

Bitumen – битум

Bituminous coal – каменный уголь

Blast-hole – шпур

Blast furnace – доменная печь

Blast-furnace dust – колосниковая пыль

Blending – усреднение

Blower – воздуходувка

Board – борт

Bog – болото

Border – граница

Borehole – скважина

Bore-hole pump – глубинный насос

Boron – бор

Bottom horizon – нижний горизонт

Bowels – недра

Bowl – чаша

Box – короб

Branch pipe – патрубок

Brass – латунь

Break (broke, broken) down – отбивать

Breakage face – очистной забой

Breaking – отбойка

Briquette – брикет

Briquetting – брикетирование

Briquetting factory – брикетная фабрика

Broken down ore – отбитая руда

Brown coal – бурый уголь

Bubble – пузырек

Bucket – ковш

Bucket loader – ковшовый погрузчик

By gravity – самотеком

By-product – побочный продукт

Bulk (collective) flotation – коллективная флотация

Bulk-selective flotation – коллективно-селективная флотация

Bulky – громоздкий

C

Cake – кек, уплотняться

Caking – спекание

Calibrated aperture – калиброванное отверстие

Capacity – производительность

Capital mining operations – горнокапитальные работы

Carbonaceous steel – углеродистая сталь

Carboxylmethilcellulose – карбоксиметилцеллюлоза (КМЦ)

Carry-over – унос, уносить

Case – корпус

Casing – кожух, обшивка

Casing pipe – обсадная труба

Cast-iron – чугун

Catch basin – водосборный колодец

Caving – обрушение

Cavity – полость

Cell – ячейка

Centrifuge effluent – фугат

Centrifuging – центрифугирование

Cerussite – церуссит

Chalcopyrite – халькопирит

Chamber – камера

Chamber-and-pillar way – камерно-столбовой способ

Channel – канал

Charge – заряд, заряжать; шихта; расход

Charging – заряжание

Chilled cast iron – закаленный чугун

Chip – стружка

Chisel – долото

Chloridation – хлоридовозгонка

Churn drilling rig – канатно-ударная буровая установка

Chute – желоб

Cinder – огарок

Circle – окружность

Circuit – схема

Circular – круговой

Clamp – зажим

Clarify – осветлять

Clarifying – осветление

Classifier – классификатор

Classifying – классификация

Clay – глина

Cleavage – спайность

Coal-tar pitch – каменноугольный пек

Coarse – крупный

Coarse screening – грубое грохочение

Coat – покрывать

Cogged – зубчатый

Coincide – совпадать

Coke – кокс

Coking – коксование

Collector – собиратель

Colliery – карьер

Collision – столкновение

Column pipe – колонковая труба

Combustion – горение

Commodity product – товарный продукт

Compact – уплотнять, трамбовать

Compress – сжимать

Compulsory – принудительный, обязательный

Completeness – полнота

Concentrate – концентрат

Concentration – обогащение

Concentrating factory – обогатительная фабрика

Condensation – сгущение

Conductor – проводник

Cone – конус

Cone (cone-type) crusher – конусная дробилка

Conformity – соответствие

Conic – конический

Consecutive – последовательный

Consumer – потребитель

Consumption – расход

Continuously – непрерывно, постоянно

Convenience – удобство

Conveyor loader – конвейерный погрузчик

Copper-porphyrite deposit – меднопорфировое месторождение

Core – керн, стержень

Corner – угол

Correspondence – соответствие

Corrugated – рифленый

Counterflow separation – противоточная сепарация

Covelline – ковеллин

Cover – крышка

Crack – трещина, растрескиваться

Cracking – растрескивание

Crop out – выходить на (поверхность)

Cross-country chassis – гусеничное шасси

Cross-section – поперечный

Crude – черновой (материал, концентрат и т.д.)

Crusher – дробилка

Crushing – дробление

Crushing-sorting factory – дробильно-сортировочная фабрика

Cube – куб

Cupreous iron pyrite – медистый колчедан

Cuprite – куприт

Current – ток

Cutting shoe – режущий башмак

Cyanic – цианистый

Cycle – цикл

Cyclonic – циклонный

Cylinder of not broken rock – цилиндр неразрушенной горной породы

Cylinder screen – грохот барабанного типа

D

Damage – ущерб

Damp – влажный

Dead rock – пустая порода

Decrypitation – декрипитация

Deduster – пылеуловитель

Degradation – ухудшение

Degree – степень

Dehydrating – обезвоживающий

Delete – удалить

Deliver – доставлять

Demountable – съемный

Demountable bit – съемная коронка

Depend on – зависеть от

Deposit – отложение

Depressor – депрессор

Design – конструкция

Design capacity – проектная мощность

Destination – назначение

Destruction – разрушение

Detain – задерживать

Detonating cord – детонирующий шнур

Detonation – взрывание

Developed reserves – разрабатываемые ресурсы

Development – вскрытие месторождения

Development operations – горноподготовительные работы

Deviate – отклоняться

Dewatering, water pumping – водоотлив, обезвоживание

Dextrin – декстрин

Diamond bit – алмазная коронка

Dielectric – диэлектрический

Diesel machine tool – дизельный станок

Direct – прямой

Direct-flow – прямоточный

Disappear – исчезать

Disk mill – дисковая мельница

Disintegrator – стержневая дробилка, дезинтегратор

Dispersion – диспергирование

Distance – удаление

Distinction – различие

Distinguish – различать

Distribution – распределение

Disturb – нарушать

Ditch – канава

Disposable, reusable – одноразовый, многоразовый

Division – деление

Downpour – ливень

Downwards – вниз, внизу

Dragline excavator – драглайн

Drain – сток, слив, осушать воду

Drainage – дренаж, осушение, дренирование

Drain line – водоотвод

Drain surface – поверхность стока

Drawing fan – всасывающий вентилятор

Draw off – отсасывать, отводить (воду) с помощью канала, дренажных канав

Dressing – обогащение

Drier – сушилка

Drifting – проходка (траншей)

Drill – бурить

Drill bit – буровая коронка

Drilling – бурение

Drill-and-fire breaking – буровзрывная отбойка

Drilling rig – буровая установка

Drilling tool – буровой снаряд

Drill chuck – буродержатель

Drilling vessel – буровое судно

Drive – привод, двигатель

Driving – проходка

Drilling – бурение

Drill-and-fire operations – буровзрывные работы

Drill-and-fire way – буровзрывные работы

Droplet – капелька

Drying – сушка

Duct – короб

Dump – сбрасывать

Dumper – самосвал

Durability – прочность

Dustproof – пыленепроницаемый

Dust – пыль

Dust removal – обеспыливание

Dust separation, dust collection – пылеулавливание

Dynamic load – динамическая нагрузка

E

Earth’s crust – земная кора

Eccentric – эксцентричный, эксцентрично расположенный

Edge – порог

Effluent – выброс

Elasticity – упругость

Electric conduction – электропроводность

Electric hammer (perforator) – отбойный молоток (перфоратор)

Electroflotation – электрофлотация

Electroignition – электрозажигание

Elevator – элеватор

Embodiment – воплощение

Emerge – всплывать

Emit – испускать

Encapsulate – закупоривать, капсулировать, герметизировать

Enclosing rock – вмещающая порода

End face – торец

End-product – конечный продукт

Enter – вводить

Envelop – обволакивать

Essence – сущность

Estimation – оценка, оценивание

Evenly – равномерно

Excavating machine – экскаватор

Exceed – превышать

Excessive – излишний, чрезмерный

Excitation – возбуждение

Exclude – исключать

Expansion – расширение

Expedient – целесообразный

Expensive – дорогой

Exploitation – разработка (месторождения полезных ископаемых)

Exploratory trench – разведочная траншея

Explored reserves – разведанные запасы

Explosive, explosive – взрывать, взрывчатое вещество

Explosive work – взрывные работы

Expose – подвергать

Extend – вытягивать

Extraction degree – степень извлечения

Express – выражать

Extrusive rocks – изверженные породы

F

Fabric – ткань

Fan – вентилятор

Face – забой

Face front – фронт забоя

Fail – разрушаться

Fall – падать

Falling – падение

Fall rate – скорость падения

Fall through – проваливаться

Fasten – крепить, прикреплять

Favorable – благоприятный

Feeder – питатель

Feldspar – полевой шпат

Felling area – вырубка леса

Ferroalloy industry – ферросплавное производство

Ferro-concrete – железобетон

Fiber – волокно

Fibre – волокно

Fill – заливать, заполнять

Filling – заполнение

Film flotation – пленочная флотация

Finding – нахождение

Fine-grained – мелкозернистый

Fine grinding material – тонкоизмельченный материал

Fines – мелочь (например, угольная мелочь)

Fire-resistant – огнеупорный

Fix – устанавливать, закреплять

Fixing – закрепление

Flat – плоский

Float – флотировать

Floating particle – флотирующая частица

Float-and-sink separation – обогащение в тяжелых суспензиях

Float up – всплывать

Flotation – флотация

Flow – сток

Flowability – сыпучесть, текучесть

Flow down – стекать

Fluctuation – изменение, колебание

Fluidity – текучесть

Fluidization – псевдоожижение

Fluidized bed drier – сушилка кипящего слоя

Fluoridation – фторидовозгонка

Fluted – желобчатый

Flux – флюс

Fluxed sinter – офлюсованный агломерат

Fly away – отлетать

Foam flotation– пенная флотация

Foam generator – пенообразователь

Force – сила, нагнетать

Fracture – излом

Fragile – хрупкий

Frame – рама

Frequency – частота

Friable deposit – рыхлое отложение

Friable overburden – рыхлые вскрышные породы

Friction – трение

Frost resistance – морозостойкость

Frother – пенообразователь

Furnace – печь

Further – дальнейший

G

Gain – прирост, усиление

Gangue – пустая порода

Garnet – гранат

Gate – затвор

Gather – собирать

Geologic exploration – геологоразведка

Grade – сорт

Grain – зерно

Granular material – зернистый материал

Granulometric composition – гранулометрический состав

Great bulk – основная масса

Gravel – гравий

Grinding – измельчение

Gross output, total output – валовая продукция

Ground electrode – заземленный электрод

Ground waters – грунтовые воды

Grout – цементировать

Gunpowder – порох

H

Hammer crusher – молотковая дробилка

Hammering – забивание

Hand sorting – породовыборка

Hard alloy – твердый сплав

Hardening – затвердевание, застывание, упрочение

Hardening agent – упрочняющее вещество

Hardness – твердость

Hashing – перемешивание

Hazardous – опасный

Haulageway – откаточная выработка

Heap leaching method – метод кучного выщелачивания

Heats – высокие температуры

Heat treatment – теплообработка

Hillside ditch – нагорная канава

Hold – держать

Hole – скважина, дырка

Hollow – полый, пустой, пустотелый

Homogeneous – однородный, гомогенный

Humidity –влажность

Hydraulic drive – гидропривод

Hydroclassifying – гидроклассификация

Hydrophilic – гидрофильный

Hydrophilic property – гидрофильность

Нydrophobic – гидрофобный

I

Ignition – зажигание

Ignition hearth – зажигательный горн

Ignoring – без учета

Immovable – неподвижный

Impact – удар

Impact crusher – дробилка ударного действия

Impregnation – вкрапленность

Inclined working – наклонная выработка

Indispensable – непременный

Induction – индукция

Inferred (prospective, probable) reserves – прогнозные ресурсы

Infiltration – просачивание

Infinite – бесконечный

Information-dispatching system – информационно-диспетчерская система

In spite of – несмотря на …

In such a manner – таким образом

Intend – предназначать

Immersed spiral – погруженная спираль

Immersion – погружение, опускание (в жидкость)

Improve – улучшать

Impurity – примесь

Inadmissible – недопустимый

Inch – дюйм

Inclination – наклон

Increment – приращение

Independent – самостоятельный

Input – вход

Install – устанавливать, монтировать

Intensity – напряженность, интенсивность, мощность

Interconnect – связывать

Interconnection – взаимная связь

Intend – предназначать

Interface – граница раздела

Intergrowth – срастание, прорастание

Intermediate product – промежуточный продукт

Interval – промежуток

Intractable bauxite – трудноперерабатываемые бокситы

Irradiation – облучение

Issue – выдача, выпуск (продукции), выпускать

J

Jaw crusher – щековая дробилка

Jaw, jaw plate – щека (дробилки)

Jet – струя

Jet-type mill – струйная мельница

Jewel – драгоценный камень

Jigging – отсадка

Joint – совместный

K

Kapron – капрон

Known (actual, positive, explored) reserves – достоверные запасы

L

Labour-consuming – трудоемкий

Ladle – ковш

Lamellar – пластина, пластинчатый

Lamellar feeder – пластинчатый питатель

Lamp material – кусковой материал

Lateral – боковой

Lattice – решетка

Let out – выпускать

Lift – подъемник

Like-charged – одноименно заряженный

Limiting depth – предельная глубина

Line – линия, черта

Line – футеровать

Linear – линейный

Lining – футеровка

Link – звено

Load – загружать

Loading – загрузка

Lock – замок

Loess – лесс

Longitudinal – продольный

Lot – участок

Loose – сыпучий

Loosen – разрыхлять

Loosening – разрыхление

Lower – понижать

Lump – кусковой

Lye – щелок

M

Machine tool – станок

Machining – механическая обработка

Magnet – магнит

Maintenance – обеспечение, поддержание

Making – составляющая

Malfunction – сбой, неисправная работа

Mantle – мантия

Manual – ручной

Manually – вручную

Manual way – ручной способ

Mechanical shovel – механическая лопата

Melt – расплав, плав

Mergence – слияние, объединение

Mesh – сетка, сетчатый

Mesh number – число меш

Metallurgical complex – металлургический комбинат

Mica – слюда

Mill – мельница

Mill scale – прокатная окалина

Mine – рудник

Mine water – шахтные воды

Mining – добыча

Mineral raw material – минеральное сырье

Mining-and-metallurgical complex – горно-металлургический комплекс

Mixer – смеситель, мешалка

Mobility – подвижность

Mode – режим, модель

Moisten – увлажнять, смачивать

Moisture – влага

chemically connected – химически связанная

hygroscopic – гигроскопическая

adhesive – адгезионная

capillary – капиллярная

gravitational – гравитационная

adsorbed – адсорбированная

Motor transport – автотранспорт

Movable screen – подвижный грохот

Movement – движение

Moving – перемещение, передвижение

Multichamber – многокамерный

Mutual – совместный, взаимный

N

Narrow – узкий

Non-core drilling – бескерновое бурение

Non-electroconductive – неэлектропроводный

Nonmagnetic – немагнитный

Non-saturated – ненасыщенный, непредельный

Non-uniform – неоднородный

Nonwettability – несмачиваемость

O

Oblong – продолговатый, удлиненный

Observe – наблюдать

Obvious – очевидный

Occurrence – распространенность

Oil-and-gas industry – нефтегазовая промышленность

Oil flotation – масляная флотация

Oil refining and gas processing plants – нефтеперерабатывающие и газоперерабатывающие заводы

One above the other – один над другим

Open-cast flank – борт карьера

Open-cast mining – открытая разработка

Opening – вскрытие, раскрытие

Open-hearth sinter – мартеновский агломерат

Open worked-out area – открытое очистное пространство

Operating force – действующая сила

Operational – эксплуатационный

Opposite flotation – обратная флотация

Oppositely charged – разноименно заряженный

Ore body – рудное тело

Ore sorting – рудоразборка

Output – выход (продукта)

Overall dimensional – габаритный (размер)

Overburden rock – вскрышные породы

Overcoming – преодоление

Overgrind – переизмельчать

Overgrinding, overmilling – переизмельчение

Overload – перегружать

Overrolling – перекатывание

Oversize – негабарит

Overshoot – проскок

Oversize product, plus material – надрешетный продукт

Overturned – опрокинутый, перевернутый

Overflow – слив

Overflow edge – сливной порог

Oxidizer – окислитель

P

Packing – насадка

Paddle – гребок

Pallet – поддон

Pan – корыто

Partition – перегородка

Pebble – галька

Pellet – окатыш

Pelletizer – окомкователь

Pelletizing – окомкование

Penetration – проникание

Permanent trench – капитальная траншея

Permittivity – диэлектрическая проницаемость

Piece – кусок

Pile – штабель

Pillar – целик

Pine oil – сосновое масло

Pipe-drier – труба-сушилка

Piston – поршень

Place in layers – укладывать слоями

Placer – россыпь

Placer ore – рассыпная руда

Plane – плоскость

Plate – пластина

Plenty – большое количество, множество

Plunger – плунжер

Pole – полюс

Pore – пора

Porosity – пористость

Porous – пористый

Possess – обладать

Power-intensive – энергоемкий

Pre-existing – ранее существующий

Preliminary – предварительный

Preparatory – подготовительный

Pressing – прессование

Pressing force – вдавливающая сила

Prevail – преобладать

Prevent – препятствовать, предотвращать

Previous – предыдущий

Primary – первичный

Primary crusher – первичная дробилка

Priming explosive – инициирующее взрывчатое вещество

Private – частный

Probability – вероятность

Product – произведение

Profile – профиль

Profitable (or economic) reserves – рентабельные запасы

Prospecting works – разведочные работы

Provide – обеспечивать

Promote – способствовать

Promotion – продвижение

Pulp – пульпа

Pulse – пульсировать

Punch – перфоратор

Punched – перфорированный

Put out – выносить

Pyrrhotine – пирротин

Q

Qualitative variety – качественная разновидность

Quartz – кварц

Quiet – тихий

R

Radiation – излучение

Rake – гребок

Rate – скорость

Rattler – барабанная мельница

Reduction – 1) снижение, сокращение; 2) восстановление

Receipt – поступление

Reception bunker – приемный бункер

Reckon on – рассчитывать на, ожидать

Reconcentration – дообогащение

Recovery – возврат

Rectangular – прямоугольный

Reducer – восстановитель

Reextraction – доизвлечение

Reflect – отражать

Regeneration – регенерация, восстановление

Regrinding – доизмельчение

Regular – геометрически правильной формы

Rehumidification – переувлажнение

Reinforce – армировать

Reliability – надежность

Remain – оставаться

Removal – удаление

Render – оказывать

Repair – ремонт

Repartition (processing) of nonferrous metals – передел (переработка) цветных металлов

Repeated – многократный, повторяющийся

Repel – отталкиваться

Replace – замещать, заменять

Replaceable – сменный

Replacement – замена

Repurification – переочистка

Reservoir – водоем

Retreating – доводка

Reuse – повторное использование

Rhythm – ритм

Ripper – рыхлитель

Ripping – рыхление

Rise – подъем

Roasting – обжиг

Chloridizing – хлорирующий

Oxidizing – окислительный

Reducing – восстановительный

Sulphatizing – сульфидирующий

Rock mass – массив горных пород

Roll – валок

Roll crusher – валковая дробилка

Roller – ролик

Roll-ring mill – ролико-кольцевая мельница

Roof – кровля

Rope – веревка

Rotary crusher – роторная дробилка

Rotary drilling – вращательное бурение

Rotary drilling rig – вращательная буровая установка

Rotary lever – поворотный рычаг

Rotation – вращение

Rough – шероховатый, грубый

Rubbing – протирка

Rubble – щебень

Runners – полозья

S

Saleable – кондиционный

Sample – образец, проба

Sands – пески

Sandy-clay mix – песчано-глинистая смесь

Saturate – насыщать

Saturated – насыщенный, предельный

Scale – окалина

Scarnified (or scarn) deposit – скарновое месторождение

Scrapper – скрепер

Scratch – царапать

Scratching – царапание

Screen – отсеивать

Screen, sizing screen – грохот

Screening (screen sizing, sizing) – грохочение

Screw – шнек, навинчивать

Screw installation – шнековая установка

Scrubber – скруббер

Seam – пласт

Section – профиль, сечение

Sediment bowl – отстойник, осадительная башня

Sedimentary rocks – осадочные породы

Sedimentary seam – осадочный пласт

Sedimentation analysis – седиментационный анализ

Selection of a core – отбор керна

Selective – избирательный

Selective flotation – селективная флотация

Self-grinding – самоизмельчение

Semicircular – полукруглый

Semi-cylindrical – полуцилиндрический

Separation – разделение

Sequence of detonation – последовательность взрывания

Set – совокупность

Set module – заданный модуль, установленный модуль

Set sequence – определенная последовательность

Sewage – сточные воды

Shaft – ствол (шахты), вал

Shake (up) (shook, shaken) – трясти, встряхивать

Shaker – встряхиватель

Shaking horizontal screen – качающийся горизонтальный грохот

Sharpen – точить, заострять

Shearing – срезывание

Sheet mica – листовая слюда

Sheet – лист

Sheet deposit – пласт, пластовое месторождение

Sheet sieve – листовое сито

Shift – смена

Shine – блеск

Shock rod – ударная штанга

Shrinkage – магазинирование

Sieve analysis, mesh analysis, screen analysis – ситовый анализ

Sieve’s scale, screening scale – шкала сит

Sieving – просеивание

Sifting – отсев, просеивание

Similar – подобный, похожий

Simplification – облегчение, упрощение

Simultaneous – одновременный

Sink – тонуть

Sinter – агломерат

Sintering – агломерация

Six-side drill – шестигранный бур

Size class, size – класс крупности

Sizing screen – грохот

Slabby material – плитняковый материал

Slag – шлак

Slaked lime – гашеная известь

Slide – скользить

Slime – шлам

Slime removal – удаление шлама, обесшламливание

Slip – сползать

Slope – склон

Slot – щель

Sludge – шлам

Sluice – шлюз

Small deposit – мелкое месторождение

Smelting – выплавка

Smooth – гладкий

Solder – припаивать, паять

Spark – искра

Sphalerite – сфалерит

Spiral classifier – спиральный классификатор

Spiral-blade classifier – спирально-лопастной классификатор

Split – разделение, расщепление

Splitting – раскалывание

Spring – пружина

Spring floods – паводок

Squash – давить, раздавливать

Squashing – раздавливание

Stack – штабель

Starch – крахмал

Stationary (fixed) – неподвижный

Stationary sea platform – стационарная морская платформа

Steel – сталь

Steepness – крутизна

Step – ступень

Sticking – залипание

Stick – застрять, застревать

Storage – складирование, склад, хранилище, хранение

Storehouse – склад

Stowing by waste rock – закладка пустой породой

Stratiform deposit – стратиформное месторождение

Stream – поток

Strictly supervise – строго контролировать

Strike – отбивать

Striker – ударник

String – нить

Stripping – вскрышные работы, разрез

Strong overburden – крепкие вскрышные породы

Stumping out – корчевка пней

Subject – подвергать

Submission – подача

Subsequent – последующий

Subsurface management – недропользование

Sulphurizing – сульфидизация

Sulphurous – сернистый

Sump – зумпф

Suitable – подходящий

Sulphide (or pyrite) deposit – колчеданное месторождение

Superficial – неглубокий, поверхностный

Supervising personnel – контролирующий персонал

Supply situation – обеспеченность

Support – обеспечение, поддержка, крепь, опора

Suppress – подавлять

Suppressor – подавитель

Surface tension – поверхностное натяжение

Susceptibility – восприимчивость

Suspend – вешать, подвешивать

Suspended – взвешенный

System with open face – система с открытым забоем

T

Tailings – хвосты обогащения

Take out – выходить на (поверхность)

Tank – бак, емкость, чан

Tape – лента

Tape conveyor – ленточный конвейер

Tense – натянутый

Termination – окончание

Terrain – рельеф

Test – испытывать

Testing – проверочный

Thereof – вследствие этого

Thermal drilling – термическое бурение

Thickener – сгуститель

Thickness – толщина, толща

Throughput – пропускная способность

Throw out – отбрасывать, откидывать

Top – верх, вершина

Towards – навстречу

Tower with a power plant – вышка с силовой установкой

Trace gas-dust substance – рассеянное газопылевое вещество

Track-mounted drill – буровая каретка

Transfer – перенос, перевод

Transformation – превращение

Transition – переход

Trapeze – трапеция

Trapezoid – трапециевидный

Treatment – переработка, обработка

Trench – траншея

Triboelectric effect – трибоэлектрический эффект

Trolley – тележка

Trough – корыто

Truncated – усеченный

Tub – чан

Turbulization – турбулизация

Turn – оборот

Turnaround – оборотный

Turned – перевернутый

U

Uncharged – незаряженный

Underlying – нижележащий, нижерасположенный, подстилающий (слой)

Undersize product, minus material – подрешетный продукт

Undesirable – нежелательный

Uniform – равномерный, однородный

Unit – агрегат

Unjustified – неоправданный

Unload – выгружать

Unproductive sediment – непродуктивное отложение

Until recently – до недавнего времени

Upwards – вверх

V

Valuable – ценный

Vary – изменяться

Vein – жила

Vertical well bore – вертикальный ствол (шахты)

Vessel – емкость

Vibrate – колебаться

Vibration exciter – вибровозбудитель

Violation – нарушение

Viscous – вязкий

Vitriol – купорос

Volatize – улетучиваться

W

Washing – промывка

Washing drum – промывочный барабан

Washing parameter – коэффициент промывистости

Wash out – промывать

Waste – отходы

Waste gases – отходящие газы

Water absorption – водопоглощение

Water-bearing stratum – водоносный слой

Waterproof – водонепроницаемый

Waterproof textile braiding – непромокаемая текстильная оплетка

Wear – износ

Wear out – изнашиваться

Wear resistance – износостойкость

Wear-resisting steel – износостойкая сталь

Weighing – дозирование, взвешивание

Weighting compound – утяжелитель

Weld – сваривать

Wet – влажный

Wet mine – обводненная шахта

Wettability – смачиваемость

Wetting – смачивание

Wheel-type trenching machine – многоковшовый роторный экскаватор

Width – ширина

Wire – проволока

Wire rope anchor – тросовый анкер

Wooden ramrod – деревянный шомпол

Working – выработка

Working trench – разрезная траншея

Worsen – ухудшать

X

Xanthogenate – ксантогенат

Y

Yield – выход, давать результат, приводить к чему-то

Literature

- Фосфаты в ХХI веке: Монография/Под ред. Ю.А. Кипермана. – Алматы-Тараз-Жанатас, Гылым, 2006. – 208 c.

- Бишимбаев В.К., Ресслер Л.Б., Жекеев М.К. Состояние и перспективные направления развития основной химической промышленности Республики Казахстан. – Шымкент, Ғылым, 2006. – 290 с.

- Щащанова Р.Б. Минеральное сырье Казахстана. Учебно-методический комплекс дисциплины. – Караганда, КарГУ им. Е.А. Букетова, 2012. – 82 с.

- Шарипов Т.В. Переработка фосфоритов Каратау в гексафторосиликат натрия. Автореферат на соискание ученой степени кандидата технических наук. – Башкирский государственный университет, Казань, 2014. – 24 с.

- Голик В.И. Разработка месторождений полезных ископаемых. Учебное пособие. – М.: НИЦ Инфра-М, 2014. – 136 с.

- Михайлов Ю.В. Подземная разработка месторождений полезных ископаемых. – М.: Academia, 2008. – 320 с.

- Абрамов А.А. Переработка, обогащение и комплексное использование твердых полезных ископаемых. В 3-х томах. – М.: «Издательство Московского государственного горного университета», 2004. – 480 с.

- Переработка некондиционных фосфоритов Каратау и техногенных отходов на удобрения /текст/ Ч.Ж.Джунбеков, Р.М.Чирникова, Т.М.Ошакбаев, Г.О.Нургалиева. Инс-т Хим-х наук. им. Л.Б.Бектурова. – Алматы, Ғылым, 2000. – 132 с.

- Химическая технология неорганических веществ. У.Қ. Бишимбаев и др. Фосфор и технология фосфорной кислоты. -Алматы: Изд. Книга, 2006. – 240 с.

- Химическая технология неорганических веществ. У.Қ. Бишимбаев и др. Технология минеральных удобрений. - Алматы: Изд. Книга, 2007. – 265 с.

Марина Михайловна Ескендирова

Марат Заханович Ескендиров

«Kazakhstan mineral raw material»

Учебное пособие

Редактор Ескендирова М.М.

Подписано в печать________

Формат бумаги 80х64  1/16

Бумага типографская. Печать офсетная. Объем 7,8 п.л.

Тираж ________. Заказ № ___________

©  Издание Южно-Казахстанского  государственного университета

им. М. Ауезова

Издательский центр ЮКГУ им. М.Ауезова, г. Шымкент, пр. Тауке хана, 5