Ministry of Education and Science of the Republic of Kazakhstan

M. Auezov South Kazakhstan State University

Department of Foreign Languages for Technical specialties

Ashenova A.S.

Methodical instruction for practical lessons on Professional-oriented English language

for 6B07150- “Electrical power engineering”

Form of study: full-time

Shymkent – 2020

UDC: 626.81

Developed by Ashenova A.S.

Methodical instruction for practical lessons on Professional-oriented English language. Shymkent: M.Auezov South Kazakhstan State University, 2020- 54 pp.

A methodical instruction for the practical lessons for the specialty 6B07150- “Electrical power engineering is developed in accordance with the requirements of State Educational Standards for specialties and curriculum of the given discipline. This Methodical instruction comprises all necessary data on performance of the course themes.

Reviewer:

- A.A. Suyberdieva- Candidate of Pedagogical Sciences, head of the Department of «Foreign Languages for Humanitarian specialities», M. Auezov South Kazakhstan State University.

Methodical instructions for the students of the second course Methodical instructions for practical lessons» is discussed and recommended for publication at the meeting of the Chair of foreign languages for Technical specialties (Minutes №         2020) and at the Committee on innovational technologies of training and Methodical provision of higher school “Information Technologies and Energetics”(Minutes №      2020)

Methodical instructions   is recommended for printing by the Methodical Council of M.Auezov South Kazakhstan State University, Minutes № .____ ___________ 2020

© M.Auezov South Kazakhstan State University, 2020

Responsible for printing:  Asenova A.S.

Introduction

This methodical instruction is intended for students of Power Engineering Industry who study English on the basis of the multi-level approach with the focus on international standards.

The instruction includes material which helps masters to develop necessary skills to acquire the English language for professional purposes. The course is composed on the basis of the communicative competence and functioning approaches. The organization of the material allows mastering students’ communicative competence in speaking, reading and writing. In addition, each unit includes tasks to develop the linguistic competence of the students.

Practical lesson 1

The theme of the lesson: Electric generating stations

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information: individual work

Types of power plants for energy generation

Nuclear power plants

Using a nuclear fission reaction and uranium as fuel, nuclear power plants generate high amount of electricity.

As nuclear power plants emit low greenhouse gas emissions, the energy is considered environmentally friendly.

When compared to renewable sources of energy such as solar and wind, the power generation from nuclear power plants is also considered more reliable.

Though the investments required to set up nuclear power plants are huge, the costs involved in operating them are low.

Nuclear energy sources also have higher density than fossil fuels and release massive amounts of energy.

Due to this, nuclear power plants require low quantities of fuel but produce enormous amounts of power, making them particularly efficient once up and running.

Hydroelectric power plants

Hydroelectricity is produced by harnessing the gravitational force of flowing water.

Compared to fossil fuel-powered energy plants, hydroelectric power plants emit fewer greenhouse gases.

However, the construction of hydroelectric power plants and dams requires huge investment.

According to the International Hydropower Association’s 2017 Hydropower Status Report, an estimated 31.5 GW of hydropower capacity was put into operation in 2016, bringing the world’s cumulative installed capacity to 1,246 GW.

China alone accounted for almost one third of global hydropower capacity and added around 11.74 GW of new capacity that year.

Coal-fired power plants

According to World Coal Association, coal-fired power plants currently account for 41% of global electricity, with almost half of that coming from China alone.

Coal-fired power plants use steam coal as source to generate electricity and consequently emit a significant amount of harmful gases into the atmosphere.

In a bid to reduce greenhouse gas emissions, many developed nations have already announced plans to phase out coal-fired power plants.

In November 2016, the Canadian government had announced plans to phase out its coal-fired power plants by 2030.

In the same month, the UK government had outlined plans to phase out coal-fired power plants by 2025, with a plethora of other European countries following suit.

Diesel-fired power plants

With diesel as fuel, this type of power plant is used for small scale production of electric power.

They are installed in places where there is no easy availability of alternative power sources and are mainly used as a backup for uninterrupted power supply whenever there are outages.

Diesel plants require only a small area to be installed and offer higher thermal efficiency compared to coal-fired power plants.

Due to high maintenance costs and diesel prices, the power plants have not gained popularity at the same rate as other types of power generation plants such as steam and hydro.

Geothermal power plants

The three main types of geothermal plants include dry steam power stations, flash steam power stations and binary cycle power stations, all of which use steam turbines to produce electricity.

As of May 2015, 24 countries were home to a combined geothermal power capacity of 12.8GW, according to a report by Geothermal Energy Association.

Geothermal power plants are considered environmentally friendly and emit lower levels of harmful gases compared with coal-fired power plants.

Combined-cycle power plants

Using both gas and steam turbines, combined-cycle power plants produce higher amounts of electricity from a single fuel source compared to a traditional power plant.

They capture heat from the gas turbine to increase power production and are also found to release low amounts of harmful gases into the atmosphere.

Solar power plants

Solar energy plants convert energy from the sun into thermal or electrical energy using one of the cleanest and most abundant renewable energy sources.

They generally do not require high maintenance and last for about 20-25 years.

The International Energy Agency (IEA) projected in 2014 that by 2050 solar photovoltaic and solar thermal would contribute about 16 and 11%, respectively, to the worldwide electricity consumption and solar would be the world’s largest source of electricity.

However, initial costs involved in setting up solar power plants are high and the installation of solar power systems requires a lot of space.

Solar thermal power plants

Solar thermal is a system of giant mirrors arranged in such a way so as to concentrate the sun’s rays on a very small area to create significant amount of heat, thus creating steam to power a turbine that creates electricity.

Wind power plants

In recent years, there has been a rapid growth in the number of wind farms across the world, underpinned by technological advancements.

As wind is naturally occurring source of energy, there are no limitations to harness its power.

Operational costs involved in maintaining wind power plants are low after the erection of wind turbines and they are generally considered cost effective.

Wind farms can also be built on agricultural lands, without causing any interruption to cultivation activities.

However, maintenance of wind turbines may vary, as some need frequent check, and wind power projects typically require huge capital expenditure.

Tidal power plants

Tidal energy is generated from converting energy from the force tides into power and its production is considered more predictable compared to wind energy and solar power.

Despite this, tidal power is still not exploited widely even as the world’s first large-scale tidal power plant became operational in 1966.

However, increased focus on generating power from renewable sources is expected to accelerate the development of new methods to exploit the tidal energy.

Though the development of the tidal power is at the nascent stage, it is estimated to have vast potential globally.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Types of power plants for energy generation”

Make up the situation

Wind power plant,solar thermal power plant, Combined-cycle power plants,tidy energy, potential globally, Geothermal power plants

Control questions

- What are the 5 sources of electricity?

The three major categories of energy for electricity generation are fossil fuels (coal, natural gas, and petroleum), nuclear energy, and renewable energy sources. Most electricity is generated with steam turbines using fossil fuels, nuclear, biomass, geothermal, and solar thermal energy

- How is electricity generated at a typical power station?

Electricity is produced at a an electric power plant. Some fuel source, such as coal, oil, natural gas, or nuclear energy produces heat. The heat is used to boil water to create steam. ... The spinning turbine interacts with a system of magnets to produce electricity.

- What is the most efficient way to generate electricity?

That actually results in more energy produced than fossil fuels put in. Wind, the most efficient fuel for electricity, creates 1164% of its original energy inputs when converted into electricity; on the other end of the efficiency spectrum, coal retains just 29% of its original energy

- What are the different types of power stations?

Types of power plants

- Hydroelectric power plants. Hydroelectric Dam. Hydroelectric power plants generate power using the force of water to turn generators. ...

- Thermal power plants. Coal being pushed into coal power plant. ...

- Solar power plants. Solar field. ...

- Wind power plants. Wind power plant.

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of types of power plants for energy generation

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Practical lessons 2-3

The theme of the lesson: The nature of electricity. Basic facilities of Thermal Power Plant (TPP)

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information: individual work

A power plant is an assemblage of equipment, and apparatus used directly forgeneration of electric energy and also the buildings and structures needed for it. ... The former type includes thermal power stations (TPS), nuclear power plants(NPP), hydraulic power plants (HPP), and hydro pumped storage power plants(HPSPP).

A thermal power station is a power station in which heat energy is converted to electric power. In most places the turbine is steam-driven. Water is heated, turns into steam and spins a steam turbine which drives an electrical generator. After it passes through the turbine the steam is condensed in a condenser and recycled to where it was heated. This is known as a Rankine cycle. The greatest variation in the design of thermal power stations is due to the different heat sources; fossil fuel power generation, though nuclear heat energy, solar heat energy, biofuels, and waste incineration are also used. Some prefer to use the term energy center because such facilities convert forms of heat energy into electrical energy. Certain thermal power stations are also designed to produce heat for industrial purposes, for district heating, or desalination of water, in addition to generating electrical power.

A power station, also referred to as a power plant and sometimes generating station or generating plant, is an industrial facility for the generation of electric power. Power stations are generally connected to an electrical grid.

Many power stations contain one or more generators, a rotating machine that converts mechanical power into three-phase electric power. The relative motion between a magnetic field and a conductor creates an electric current.

The energy source harnessed to turn the generator varies widely. Most power stations in the world burn fossil fuels such as coal, oil, and natural gas to generate electricity. Cleaner energy sources include nuclear power, and an increasing use of renewables such as solar, wind, wave and hydroelectric.

How does a power plant work?

A power plant's a bit like an energy production line. Fuel feeds in at one end, and electricity zaps out at the other. What happens in between? A whole series of different steps, roughly along these lines:

Fuel: The energy that finds its way into your TV, computer, or toaster starts off as fuel loaded into a power plant. Some power plants run on coal, while others use oil, natural gas, or methane gas from decomposing rubbish.

Furnace: The fuel is burned in a giant furnace to release heat energy.

Boiler: In the boiler, heat from the furnace flows around pipes full of cold water. The heat boils the water and turns it into steam.

Turbine: The steam flows at high-pressure around a wheel that's a bit like a windmill made of tightly packed metal blades. The blades start turning as the steam flows past. Known as a steam turbine, this device is designed to convert the steam's energy into kinetic energy (the energy of something moving). For the turbine to work efficiently, heat must enter it at a really high temperature and pressure and leave at as low a temperature and pressure as possible.

Cooling tower: The giant, jug-shaped cooling towers you see at old power plants make the turbine more efficient. Boiling hot water from the steam turbine is cooled in a heat exchanger called a condenser. Then it's sprayed into the giant cooling towers and pumped back for reuse. Most of the water condenses on the walls of the towers and drips back down again. Only a small amount of the water used escapes as steam from the towers themselves, but huge amounts of heat and energy are lost.

Generator: The turbine is linked by an axle to a generator, so the generator spins around with the turbine blades. As it spins, the generator uses the kinetic energy from the turbine to make electricity.

Electricity cables: The electricity travels out of the generator to a transformer nearby.

Step-up transformer: Electricity loses some of its energy as it travels down wire cables, but high-voltage electricity loses less energy than low-voltage electricity. So the electricity generated in the plant is stepped-up (boosted) to a very high voltage as it leaves the power plant.

Pylons: Hugh metal towers carry electricity at extremely high voltages, along overhead cables, to wherever it is needed.

Step-down transformer: Once the electricity reaches its destination, another transformer converts the electricity back to a lower voltage safe for homes to use.

Homes: Electricity flows into homes through underground cables.

Appliances: Electricity flows all round your home to outlets on the wall. When you plug in a television or other appliance, it could be making a very indirect connection to a piece of coal hundreds of miles away!

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Thermal power plant (TPP)”

Make up the situation

Power station, power plant , generating station or generating plant, industrial facility ,an electrical grid,magnetic field, an electric current, generator, fossil fuels , coal, oil, natural gas , generate, electricity. renewables , solar, wind, wave , hydroelectric.

Control questions

- What is the fuel used in thermal power plant?

Almost all coal, petroleum, nuclear, geothermal, solar thermal electric, and waste incineration plants, as well as many natural gas power stations are thermal. Natural gas is frequently combusted in gas turbines as well as boilers

- How is electricity produced in thermal power plant?

Thermal generators or specially designed furnaces produce this wonderful thermal electricity. Thermal power plant burn fuels to boil water and make steam. The steam is then used to spin a turbine which is connected to a generator that weaves electricity

- Which power plant has highest efficiency?

First 9HA Gas Turbine Installed

And when it comes to efficient natural gas-fired plants, there are none better than EDF Bouchain. The 605-MW unit has been recognized by Guinness World Records as the most-efficient combined cycle power plant in the world, with a third-party verified net efficiency of 62.22%.

- How does a thermal plant work?

The way energy from fuel gets transformed into electricity forms the working of a power plant. In a thermal power plant a steam turbine is rotated with help of high pressure and high temperature steam and this rotation is transferred to a generator to produce electricity.

- Who invented power plants?

In 1882 the world's first coal-fired public power station, the Edison Electric Light Station, was built in London, a project of Thomas Edison organized by Edward Johnson. A Babcock & Wilcox boiler powered a 125-horsepower steam engine that drove a 27-ton generator.

- What are the advantages and disadvantages of thermal power plants?

Disadvantages of thermal power plant are:

The running cost of thermal power station is more as compared to hydro power station. It pollutes the atmosphere due to production of large amount of smoke and fumes. Maintenance cost is more. Skilled persons are required for erecting and maintaining the power station

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Heat power plant (HPP)

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Practical lessons 4-5

The theme of the lesson: Practical methods for calculation and design. Power supply

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information:individual work

A power supply is an electrical device that supplies electric power to an electrical load. The primary function of a power supply is to convert electric current from a source to the correct voltage, current, and frequency to power the load. As a result, power supplies are sometimes referred to as electric power converters. Some power supplies are separate standalone pieces of equipment, while others are built into the load appliances that they power. Examples of the latter include power supplies found in desktop computers and consumer electronics devices. Other functions that power supplies may perform include limiting the current drawn by the load to safe levels, shutting off the current in the event of an electrical fault, power conditioning to prevent electronic noise or voltage surges on the input from reaching the load, power-factor correction, and storing energy so it can continue to power the load in the event of a temporary interruption in the source power (uninterruptible power supply).

All power supplies have a power input connection, which receives energy in the form of electric current from a source, and one or more power output connections that deliver current to the load. The source power may come from the electric power grid, such as an electrical outlet, energy storage devices such as batteries or fuel cells, generators or alternators, solar power converters, or another power supply. The input and output are usually hardwired circuit connections, though some power supplies employ wireless energy transfer to power their loads without wired connections. Some power supplies have other types of inputs and outputs as well, for functions such as external monitoring and control.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Energy usage in industry”

Control questions

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Energy usage in industry

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Combined heat and power plant

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Practical lessons 7-8

The theme of the lesson: Energy saving in modern conditions. Electric currents and their properties

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Efficient energy use, sometimes simply called energy efficiency, is the goal to reduce the amount of energy required to provide products and services. For example, insulating a home allows a building to use less heating and cooling energy to achieve and maintain a comfortable temperature. Installing LED lighting, fluorescent lighting, or natural skylight windows reduces the amount of energy required to attain the same level of illumination compared to using traditional incandescent light bulbs. Improvements in energy efficiency are generally achieved by adopting a more efficient technology or production process or by application of commonly accepted methods to reduce energy losses. There are many motivations to improve energy efficiency. Reducing energy use reduces energy costs and may result in a financial cost saving to consumers if the energy savings offset any additional costs of implementing an energy-efficient technology. Reducing energy use is also seen as a solution to the problem of reducing greenhouse gas emissions. According to the International Energy Agency, improved energy efficiency in buildings, industrial processes and transportation could reduce the world's energy needs in 2050 by one third, and help control global emissions of greenhouse gases. Another important solution is to remove government-led energy subsidies that promote high energy consumption and inefficient energy use in more than half of the countries in the world. Energy efficiency and renewable energy are said to be the twin pillars of sustainable energy policy and are high priorities in the sustainable energy hierarchy. In many countries energy efficiency is also seen to have a national security benefit because it can be used to reduce the level of energy imports from foreign countries and may slow down the rate of energy at which domestic energy resources are depleted.

Translate and write questions to the text

Industry uses a large amount of energy to power a diverse range of manufacturing and resource extraction processes. Many industrial processes require large amounts of heat and mechanical power, most of which is delivered by natural gas, petroleum fuels and as electricity. In addition some industries generate waste streams that can be recovered to provide additional energy. Because industrial processes are so diverse from the cold or low temperature required process to the high temperature required process, it is impossible to describe the multitude of opportunities for energy efficiency improvements in industry. There are a number of processes and energy services that are widely used in many industries. Many depend on the specific technologies and processes in use at each industrial facility. However effective energy management in industry, irrespective of size, technology or process, will increase energy efficiency by at least 5%. To implement co-generation, decrease process heat level and amount or recover the waste heat from process or utilities provides important Energy Efficiency opportunities in many countries. Various industries co-generate steam and electricity for subsequent use within their own facilities. When electricity is generated, the heat that is produced as a by-product can be captured and used for process steam, heating or other industrial purposes. Co-generation converts up to 90% of fuel into usable energy.

Electric motors are by far the most important type of electric load in industry using about 70% of the consumed electricity. In the tertiary sector, electric motor systems use about one third of the consumed electricity. Electric motors usually run at a constant speed, but a variable speed drive allows the motor’s energy output to match the required load. This achieves energy savings ranging from 3 to 60%, depending on how the motor is used. Motor coils made of superconducting materials can also reduce energy losses. Moreover, motors can also benefit from voltage optimisation. Industry uses a large number of pumps and compressors of all shapes and sizes and for a-wide variety of applications. The efficiency of pumps and compressors depends on many factors but often improvements can be made by implementing better process control and better maintenance practices. Compressors are commonly used to provide compressed air which is used for sand blasting, painting, and other power tools. According to some investigations, optimising compressed air systems by installing variable speed drives, along with preventive maintenance to detect and fix air leaks, can improve energy efficiency by 20 to 50%.

Energy efficiency in industry has improved significantly in the last decade, and additional improvements are still possible through the implementation of best available technologies (BATs) and design guidelines for existing assets. Moreover, efficiency measures offer some of the least-cost options for reduction of carbon dioxide (CO2) emissions; however, a wider deployment of well-known, cost-effective policy instruments is needed to achieve this potential. Technologies such as carbon capture and storage (CCS), smelting reduction, separation membranes and black liquor gasification are expected to contribute massively to-meeting these challenges. Greater investment by both governments and industry is needed for research, development, demonstration and deployment of a wide range of promising new technologies and for identification and implementation of novel processes which will result in the CO2-free production of materials in the longer term; for instance, induction-(potential sectors: metallurgy, food industry), heat pumps for industry (potential sectors: food, chemistry, pulp and paper, iron and steel).

In the O&G Industry, the downstream business is strongly constrained by high crude oil costs, the required investments, increasing complexity of business processes and close monitoring of product prices, due to the huge impact of energy prices on national economies, in particular in developing countries. This can be attributed to the refining activity which has a long history of rigorous management of its operational costs, focusing on energy efficiency, maintenance practices, personnel management, etc.

In order to evaluate the energy performance of a refinery or petrochemical plant, analyse its performance trends and compare with other plants, it is important to consider performance indicators that take into account the complexity of the systems and its operations, and allow a critical analysis of its results by the manager, to bridge the gaps between the results and the proposed goals.

1. What are the common energy sources known worldwide?

2. Why the electric motors are considered to be the most electric load in industry?

3. What does one need to do in order to evaluate energy performance of a refinery or petrochemical plants?

4. What is the O&G business constrained with?

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Electric currents and their properties”

Control questions

- What are some examples of energy efficiency?

Energy efficiency, means using less energy to provide the same level of energy. It is therefore one method to reduce human greenhouse gas emissions. For example if a house is insulated, less energy is used in heating and cooling to achieve a satisfactory temperature

- What are some examples of energy efficiency?

Energy efficiency, means using less energy to provide the same level of energy. It is therefore one method to reduce human greenhouse gas emissions. For example if a house is insulated, less energy is used in heating and cooling to achieve a satisfactory temperature

- What is energy efficient building?

Energy efficient buildings (new constructions or renovated existing buildings) can be defined as buildings that are designed to provide a significant reduction of the energy need for heating and cooling, independently of the energy and of the equipments that will be chosen to heat or cool the building

- What is the importance of energy efficiency?

Why Energy Efficiency. Energy efficiency — delivering the same (or more) services for less energy — helps protect the environment. When we use less energy, the less energy we need to generate at power plants, which reduces greenhouse gas emissions and improves the quality of our air

- What are energy saving devices?

Although energy-saving light bulbs are the clear winners when it comes to saving energy, there are other electricity-saving devices for homes include energy-saving plugs and rechargeable batteries

- What is the definition of energy efficiency?

Energy efficiency simply means using less energy to perform the same task – that is, eliminating energy waste. Energy efficiency brings a variety of benefits: reducing greenhouse gas emissions, reducing demand for energy imports, and lowering our costs on a household and economy-wide level

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Energy efficiency

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Practical lessons 9-10

The theme of the lesson: Alternatives & Renewables. Ways of automotive elimination of damages and abnormal regimes in the power system

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

What is renewable energy?

Broadly speaking, the world's energy resources (all the energy we have available to use) fall into two types called fossil fuels and renewable energy:

Fossil fuels are things like oil, gas, coal, and peat, formed over hundreds of millions of years when plants and sea creatures rot away, fossilize, and get buried under the ground, then squeezed and cooked by Earth's inner pressure and heat. Fossil fuels supply about 80–90 percent of the world's energy.

Renewable energy means energy made from the wind, ocean waves, solar power, biomass (plants grown especially for energy), and so on. It's called renewable because, in theory, it will never run out. Renewable sources currently supply about 10–20 percent of the world's energy.

What's the difference between fossil fuels and renewable energy?

In theory, fossil fuels exist in limited quantities and renewable energy is limitless. That's not quite the whole story, however.

The good news is that fossil fuels are constantly being formed. New oil is being made from old plants and dead creatures every single day. But the bad news is that we're using fossil fuels much faster than they're being created. It took something like 400 million years to form a planet's worth of fossil fuels. But humankind will use something like 80 percent of Earth's entire fossil fuel supplies in only the 60 years spanning from 1960 to 2020. When we say fossil fuels such as oil will "run out," what we actually mean is that demand will outstrip supply to the point where oil will become much more expensive to use than alternative, renewable fuel sources.

Just as fossil fuel supplies aren't exactly finite, neither is renewable energy completely infinite. One way or another, virtually all forms of renewable energy ultimately come from the Sun and that massive energy source will, one day, burn itself out. Fortunately, that won't happen for a few billion years so it's reasonable enough to talk of renewable energy as being unlimited.

What are the different types of renewable energy?

Almost every source of energy that isn't a fossil fuel is a form of renewable energy. Here are the main types of renewable energy:

Solar power

For as long as the Sun blazes (roughly another 4–5 billion years), we'll be able to tap the light and heat it shines in our direction. We can use solar power in two very different ways: electric and thermal. Solar electric power (sometimes called active solar power) means taking sunlight and converting it to electricity in solar cells (which work electronically). This technology is sometimes also referred to as photovoltaic (photo = light and voltaic = electric, so photovoltaic simply means making electricity from light) or PV. Solar thermal power (sometimes called passive-solar energy or passive-solar gain) means absorbing the Sun's heat into solar hot water systems or using it to heat buildings with large glass windows.

Wind power

Depending on where you live, you've probably seen wind turbines appearing in the landscape in recent years. There are loads of them in the United States and Europe, for example. A turbine is any machine that removes kinetic energy from a moving fluid (liquid or gas) and converts it into another form. Windmills, based on this idea, have been widely used for many hundreds of years. In a modern wind turbine, a huge rotating blade (similar to an airplane propeller) spins around in the wind and turns an electricity generator mounted in the nacelle (metal casing) behind. It takes roughly several thousand wind turbines to make as much power as one large fossil fuel power plant. Wind power is actually a kind of solar energy, because the winds that whistle round Earth are made when the Sun heats different parts of our planet by different amounts, causing huge air movements over its surface.

Hydroelectric power

Hydro means water, so hydroelectricity means making electricity using water—not from the water itself, but from the kinetic energy in a moving river or stream. Rivers start their lives in high ground and gradually flow downhill to the sea. By damming them, we can make huge lakes that drain slowly past water turbines, generating energy as they go. Water wheels used in medieval times to power mills were an early example of hydro power. You could describe them as hydromechanical, since the water power the milling machines used was transmitted by an elaborate systems of wheels and gears. Like wind power, hydroelectric power is (indirectly) another kind of solar energy, because it's the Sun's energy that drives the water cycle, endlessly exchanging water between the oceans and rivers on Earth's surface and the atmosphere up above.

Using more by using less

The easiest way to save the planet is to use resources more wisely. If you can't find a way to use more renewable energy, you can still try to use less conventional energy (from fossil fuels). Being more efficient is surprisingly quick and easy and often costs nothing at all. It costs nothing, for example, to share your car with a friend and getting a bus or a train often saves you money, as well as saving energy. Heat insulating your home is another good way of saving energy (and money) at little or no cost, while turning down your thermostat (and putting on an extra layer of clothing) is something anyone can do without spending so much as a cent. Try switching to energy-efficient light bulbs (LEDs are more efficient than CFLs and now just as cheap) and use energy monitors to help you measure and cut the cost of your other appliances. You can save money in your car too by giving some thought to fuel efficiency.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Alternatives & Renewables”

Make up the situation

Control questions

- Is alternative and renewable energy the same?

Renewable energy sources are not exactly the same as alternative energy sources. Alternative energy is a broader category encompassing all non-fossil-fuel-based energy sources and processes, of which renewable energies are only a part.

- Is alternative and renewable energy the same?

Renewable energy sources are not exactly the same as alternative energy sources. Alternative energy is a broader category encompassing all non-fossil-fuel-based energy sources and processes, of which renewable energies are only a part.

- Are all alternative energy sources renewable?

Renewable energy generated from natural resources—such as sunlight, wind, rain, tides and geothermal heat—which are renewable (naturally replenished). ... Nuclear power is an alternative to fossil fuels that is non-renewable, like fossil fuels, nuclear ones are a finite resource

- What is the difference between alternative energy sources and renewable energy sources?

Although alternative energy and renewable energy both work to cut down on carbon emissions, there is a stark difference between the two. ... Examples of renewable energy include biomass resources, solar energy, wind energy, geothermal and hydro resources. The most abundant of these resources is solar energy

- What's the difference between renewable and sustainable?

Renewable Energy, meanwhile, is a method of energy production that does not exhaust resources for its energy generation because it uses resources that can be renewed continually. While sustainability is a mindset and renewable energy is a method, both are environmentally-focused

- What are the 7 alternative sources of energy?

Commonly known alternative energy sources:

Hydroelectric Energy.

Solar Energy.

Wind Energy.

Biomass Energy.

Geothermal Energy.

Tidal Power.

- What is the best form of alternative energy?

The Best Examples of Alternative Energy Sources

Solar Power. When most people think of alternative energy sources they tend to use solar power as an example.

Nuclear Power. ...

Hydroelectric Energy. ...

Wave Energy. ...

Biofuels. ...

Natural Gas. ...

Geothermal Power. ...

Wind Energy.

- Why is alternative energy important?

Renewable energy provides reliable power supplies and fuel diversification, which enhance energy security, lower risk of fuel spills, and reduce the need for imported fuels. Renewable energy also helps conserve the nation's natural resources

- Which is the most environmentally friendly form of energy?

Types of Eco Friendly Energy Sources

- Geothermal Power. Geothermal energy utilizes the Earth's internal heat. ...

- Solar Energy. Solar energy is one of the most accessible renewable resources today. ...

- Wind Energy. Energy from wind is harnessed with the use of wind turbines and windmills. ...

- Hydropower. Water is another source of strong, renewable energy

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of renewable

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 11-12-13

The theme of the lesson: Non-traditional and renewable energy sources

Development of wind power in the world

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Nontraditional renewable sources include solar, wind, geothermal, biomass, tidal, wave, and ocean energy and energy from small rivers. However, the tidal, wave, and ocean energies are not now widely used. Sometimes, heat pumps are also counted as nontraditional energy sources.

Application of non-conventional & renewable energy sources

Renewable energy sources also called non-conventional energy, are sources that are continuously replenished by natural processes. For example, solar energy, wind energy, bio-energy - bio-fuels grown sustain ably), hydropower etc., are some of the examples of renewable energy sources A renewable energy system converts the energy found in sunlight, wind, falling-water, seawaves, geothermal heat, or biomass into a form, we can use such as heat or electricity. Most of the renewable energy comes either directly or indirectly from sun and wind and can never be exhausted, and therefore they are called renewable. However, most of the world's energy sources are derived from conventional sources-fossil fuels such as coal, oil, and natural gases. These fuels are often termed non-renewable energy sources. Although, the available quantity of these fuels are extremely large, they are nevertheless finite and so will in principle 'run out' at some time in the future Renewable energy sources are essentially flows of energy, whereas the fossil and nuclear fuels are, in essence, stocks of energy

Various forms of renewable energy

Solar energy

Wind energy

Bio energy

Hydro energy

Geothermal energy

Wave and tidal energy

Solar Energy

Solar energy is the most readily available and free source of energy since prehistoric times. It is estimated that solar energy equivalent to over 15,000 times the world's annual commercial energy consumption reaches the earth every year. India receives solar energy in the region of 5 to 7 kWh/m2 for 300 to 330 days in a year. This energy is sufficient to set up 20 MW solar power plant per square kilometre land area. Solar energy can be utilised through two different routes, as solar thermal route and solar electric (solar photovoltaic) routes. Solar thermal route uses the sun's heat to produce hot water or air, cook food, drying materials etc. Solar photovoltaic uses sun's heat to produce electricity for lighting home and building, running motors, pumps, electric appliances, and lighting.

Solar Thermal Energy Application In solar thermal route, solar energy can be converted into thermal energy with the help of solar collectors and receivers known as solar thermal devices.

The Solar-Thermal devices can be classified into three categories:

Low-Grade Heating Devices - up to the temperature of 100°C.

Medium-Grade Heating Devices -up to the temperature of 100°-300°C

High-Grade Heating Devices -above temperature of 300°C

Low-grade solar thermal devices are used in solar water heaters, air-heaters, solar cookers and solar dryers for domestic and industrial applications.

Wind energy

Wind energy is basically harnessing of wind power to produce electricity. The kinetic energy of the wind is converted to electrical energy. When solar radiation enters the earth's atmosphere, different regions of the atmosphere are heated to different degrees because of earth curvature. This heating is higher at the equator and lowest at the poles. Since air tends to flow from warmer to cooler regions, this causes what we call winds, and it is these airflows that are harnessed in windmills and wind turbines to produce power. Wind power is not a new development as this power, in the form of traditional windmills -for grinding corn, pumping water, sailing ships - have been used for centuries. Now wind power is harnessed to generate electricity in a larger scale with better technology.

Wind Energy Technology

The basic wind energy conversion device is the wind turbine. Although various designs and configurations exist, these turbines are generally grouped into two types:

1. Vertical-axis wind turbines, in which the axis of rotation is vertical with respect to the ground (and roughly perpendicular to the wind stream),

2. Horizontal-axis turbines, in which the axis of rotation is horizontal with respect to the ground (and roughly parallel to the wind stream.)

Bio Energy

Biomass is a renewable energy resource derived from the carbonaceous waste of various human and natural activities. It is derived from numerous sources, including the by-products from the wood industry, agricultural crops, raw material from the forest, household wastes etc. Biomass does not add carbon dioxide to the atmosphere as it absorbs the same amount of carbon in growing as it releases when consumed as a fuel. Its advantage is that it can be used to generate electricity with the same equipment that is now being used for burning fossil fuels. Biomass is an important source of energy and the most important fuel worldwide after coal, oil and natural gas. Bio-energy, in the form of biogas, which is derived from biomass, is expected to become one of the key energy resources for global sustainable development. Biomass offers higher energy efficiency through form of Biogas than by direct burning

Advantages

Some of advantages of biomass briquetting are high calorific value with low ash content, absence of polluting gases like sulphur, phosphorus fumes and fly ash- which eliminate the need for pollution control equipment, complete combustion, ease of handling, transportation & storage - because of uniform size and convenient lengths.

Application

Biomass briquettes can replace almost all conventional fuels like coal, firewood and lignite in almost all general applications like heating, steam generation etc. It can be used directly as fuel instead of coal in the traditional chulhas and furnaces or in the gasifier. Gasifier converts solid fuel into a more convenient-to-use gaseous form of fuel called producer gas.

Hydro energy

The potential energy of falling water, captured and converted to mechanical energy by waterwheels, powered the start of the industrial revolution. Wherever sufficient head, or change in elevation, could be found, rivers and streams were dammed and mills were built. Water under pressure flows through a turbine causing it to spin. The Turbine is connected to a generator, which produces electricity. In order to produce enough electricity, a hydroelectric system requires a location with the following features: Change in elevation or head: 20 feet @ 100 gal/min = 200 Watts. 100 feet head @ 20 gal/min gives the same output. In India the potential of small hydro power is estimated about 10,000 MW. A total of 183.45 MW small Hydro project have been installed in India by the end of March 1999. Small Hydro Power projects of 3 MW capacity have been also installed individually and 148 MW project is under construction.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Non-conventional & renewable energy sources”

Make up the situation

Control questions

- What do you mean by renewable energy

- Explain working of solar water heater?

- List few applications of low temperature water heaters in domestic and industrial use

- What are the two methods by which energy can be recovered from solar radiation

- How can the performance of solar collectors be improved?

- Explain any two applications of concentrated solar energy?

- What do you mean by photovoltaic?

- Explain the terms cell, module and array as applicable to photovoltaic.

- What are the typical applications of photovoltaic power?

- What are the criteria for selection of wind mill installation?

- What ere the incentives available for wind mill installation?

- Explain the bio-energy potential in India and its applications.

- What are the various methods by which power can be generated from biomass?

- Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of non-conventional & renewable energy sources

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Practical lessons 15-16

The theme of the lesson: Three-phase transformers. Features of physical processes in three-phase transformers with separate and common magnetic systems

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Three-phase voltage transformations

Three-phase voltage transformations can be accomplished by using three phase transformers, which are single devices with all windings constructed on a single iron core. They also can be accomplished by using three single-phase transformers that are connected externally to form a three-phase bank.

While three-phase devices are usually the more cost-effective option, the single-phase option provides more versatility and can be attractive from a reliability and maintenance standpoint. If several identical transformers are needed at one location, the single-phase option can include the purchase of a spare unit to reduce outage time in the event of a failure.

This practice often is seen with critical autotransformer banks and generator step-up transformers because loss of the transformer for an extended period has very significant impacts.

The connections discussed in this article will be implemented using single-phase units.

Easy understanding of 3-phase transformer connections - Delta–Delta, Wye–Wye, Delta–Wye and Wye–Delta (on photo: Jefferson Electric transformer)

The two most commonly used three-phase winding configurations are delta and wye, named after the Greek and English letter that each resembles. In a delta configuration, the three windings are connected end-to-end to form a closed path. A phase is connected to each corner of the delta.

Although delta windings are often operated ungrounded, a leg of the delta can be center tapped and grounded, or a corner of the delta can be grounded. In a wye configuration, one end of each of the three windings is connected to form a neutral. A phase is connected to the other end of the three windings. The neutral is usually grounded.

The following paragraphs describe three-phase transformers which utilize the delta and wye connections.

Delta-Delta

Wye-Wye

Delta-Wye

Wye-Delta

Next part of this article will discuss three-phase transformers using the open-delta and open-wye connections, where one of the single-phase transformers making up the three-phase bank is omitted. The leg of the transformer with the missing transformer is referred to as the phantom leg.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Three-phase voltage transformations”

Make up the situation

Control questions

- What is a three phase transformer?

A three-phase transformer is made of three sets of primary and secondary windings, each set wound around one leg of an iron core assembly. ... Three phase transformer core has three sets of windings. Those sets of primary and secondary windings will be connected in either Δ or Y configurations to form a complete unit.

- How many windings does a 3 phase transformer have?

The three-phase transformer actually has 6 windings (or coils) 3 primary and 3 secondary. In power applications, these windings are usually connected in one of two configurations: Delta or Wye. In a Delta connection, there are three phases and no Neutral.

- What happens if one phase of a 3 phase transformer fails?

When a three phase motor looses one of the phase supply voltage then it is called Single Phasing. Single Phasing a case when any one phase out of the three phases fails. ... The motor will run but would not be able to drive rated load. The uneven torque results in abnormal noise and vibration in motor

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Three-phase voltage transformations

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Practical lessons 17-18

The theme of the lesson: Winding circuit and groups. Features of idling of three-phase transformers

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

A transformer is a passive electrical device that transfers electrical energy from one electrical circuit to one or more circuits. A varying current in any one coil of the transformer produces a varying magnetic flux, which, in turn, induces a varying electromotive force across any other coils wound around the same core. Electrical energy can be transferred between the (possibly many) coils, without a metallic connection between the two circuits. Faraday's law of induction discovered in 1831 described the induced voltage effect in any coil due to changing magnetic flux encircled by the coil. Transformers are used for increasing alternating voltages at low current (Step Up Transformer) or decreasing the alternating voltages at high current (Step Down Transformer) in electric power applications, and for coupling the stages of signal processing circuits. Since the invention of the first constant-potential transformer in 1885, transformers have become essential for the transmission, distribution, and utilization of alternating current electric power. A wide range of transformer designs is encountered in electronic and electric power applications. Transformers range in size from RF transformers less than a cubic centimeter in volume, to units weighing hundreds of tons used to interconnect the power grid.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “3 phase transformer”

Make up the situation

Control questions

- What is a 3 phase transformer?

A three-phase transformer generally has the three magnetic circuits that are interlaced to give a uniform distribution of the dielectric flux between the high and low voltage windings. The exception to this rule is a three-phase shell type transformer

- Can you use a 3 phase transformer for single phase?

A 3 phase transformer can work as a single phase transformer also. ... Theoretically, it is possible to parallel the three phase connections at both primary and secondary side to convert it into single phase transformer. The flux in core remains sinusoidal and produces sinusoidal secondary supply voltage

- Why there is no neutral in 3 phase?

The delta wiring of the primary side of that supply transformer means that no neutralis needed in the high voltage side of the network. If the supply neutral of a three-phase system with line-to-neutral connected loads is broken, generally the voltage balance on the loads will no longer be maintained

- Why are transformers rated in KVA?

The Copper loss(I2R) occurs due to the flow of the current in the transformerwinding and the Iron or core loss occurs due to the voltage. These losses do not depend on the power factor so that is why the transformer rating in KVA not KW. These are the Three Reasons Why Transformer is Rated in KVA

- What is phase voltage?

Voltage and Current Values in Three-Phase Systems

With the above circuit, the line voltage is roughly 208 volts. Phase voltage refers to the voltage measured across any one component (source winding or load impedance) in a balanced three-phase source or load.

- What is the advantage of 3 phase over single phase?

Comparing with single phase motor, three phase induction motor has higher power factor and efficiency. Three phase motors are very robust, relatively cheap, generally smaller, have self-starting properties, provide a steadier output and require little maintenance compared with single phase motors

- What is phase voltage and phase current?

by "phase" you mean 1 phase of a multi-phase line. Line voltage is the voltageseen on the AC line, typically single phase. Line current is the current that results when that voltage is applied to a load. If you have a 3 phase line, then each of the 3phases (a separate wire) has it's own voltage, phase voltage

- Why neutral has no current?

So the voltage of the neutral wire is always zero... ... If it's not small enough to be ignored, it means that the neutral wire is too small (diameter) for the amount ofcurrent that it's carrying. current flows because of voltage. Well, yes, but no

- What are the two main types of transformer?

The primary may be connected phase to ground or phase to phase. The secondary is usually grounded on one terminal. There are three primary types of voltagetransformers (VT): electromagnetic, capacitor, and optical. The electromagnetic voltage transformer is a wire-wound transformer.

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of 3 phase transformer

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 19-20-21

The theme of the lesson: Constant current generators. Parallel operation of generators, conditions for switching to parallel operation

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Constant voltage sources

An electrochemical cell is a device capable of either generating electrical energy from chemical reactions or facilitating chemical reactions through the introduction of electrical energy. A common example of an electrochemical cell is a standard 1.5-volt cell meant for consumer use. This type of device is known as a single Galvanic cell, so an obsolete name for steady current was galvanic current. A battery consists of two or more cells, connected in either parallel or series pattern.

A homopolar generator is an electrical generator comprising an electrically conductive disc or cylinder rotating in a plane perpendicular to a uniform static magnetic field. A magnetohydrodynamic generator directly extracts electric power from moving hot gases through a magnetic field, without the use of rotating electromagnetic machinery.

AC generators can be used as sources of steady current with a rectifier and a good ripple filter. Pulsed DC generators can be used as sources of steady current with a good ripple filter.

Constant current power supplies

In electronics, a constant current system is one that varies the voltage across a load to maintain a constant electric current. When a component is indicated to be driven by a constant current, the driver circuit is, in essence, a current regulator and must appear to the component as a current source of suitable reliability.

An important usage of constant current power supplies is with LEDs. While a high series resistance is sufficient to light an LED, sometimes the design needs to guard against high current (or risk burning out the LEDs).

Another use is in fluorescent lamps which have very dynamic electrical resistance and are optimally operated within a short range of currents. Other uses include shielded metal arc welding and gas tungsten arc welding.

Constant Current Source Applications for LEDs

Reliability Testing

Device Burn-in Test

HASS and IESNA LM-80 Applications

Energy Star Certification

Calibration Lamp Drive

DC Driven Lamps

Resistor Aging

Laser Diode DC Drive

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Constant current generator”

Make up the situation

Control questions

- What is constant current generator?

A constant current source is a power source which provides a constant current to a load, even despite changes and variance in load resistance. ... You can see that the current is constant all throughout despite changes in voltage or resistance

- What is constant current control?

Constant Current. 'Constant current' refers to the ability to vary the voltage across an electronic circuit to maintain a constant electric current. ... Constant current mode: Delivers same current regardless of resistance changes. Compensates for part thickness changes.

- Is constant current the same as direct current?

Once the current is established, it is thus also a constant. Direct current (DC) is the flow of electric charge in only one direction. It is the steady state of a constant-voltage circuit. Most well-known applications, however, use a time-varying voltage source.

- Why does a current source have infinite resistance?

When the current source has infinite internal resistance, the output current from the source will not change despite changes in resistance value from the load. The current will always want to escape out of its infinite resistance to a lower resistance

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of constant current generator

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Unit 22-23-24

The theme of the lesson: Converters of radiant energy into electricity

Natural Resources and Energy

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

In a photovoltaic process, radiant sunlight is converted to electricity. ... The energyfrom the sun invigorates the photovoltaic material at the molecular level, causing electrons to break away, creating an electrical current.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “Converters of radiant energy into electricity”

Make up the situation

Control questions

- Can radiation be converted into electricity?

Electricity is usually made using nuclear power by heating steam to rotate turbines that generate electricity. ... Now US researchers say they have developed highly efficient materials that can convert the radiation, not heat, from nuclear materials and reactions into electricity

- How is energy converted into electricity?

Hydroelectric turbines use water to spin turbine blades, and wind turbines use the wind. Electricity generators that do not use turbines include solar photovoltaic cells, which convert sunlight directly into electricity, and fuel cells, which convert fuels, such as hydrogen, into electricity through a chemical process

- What are some examples of energy conversions?

Some examples of energy conversion are:

A vehicle moving is an example of chemical energy being converted into kinetic energy.

Electricity being produced with water is an example of potential energy being converted into kinetic energy

- How can radiant energy be transformed?

Solar power harvests radiant energy carried by the light from our sun by converting it into electricity. ... Plants are able to harness and use light energy in a process called photosynthesis. They absorb radiant energy from sunlight and transform it into useful chemical energy contained in molecules within their cells

- What are 10 energy transformations examples?

Examples of Day-to-Day Energy Transformations

Chemical Energy is converted to Electrical Energy (stove), Kinetic Energy (car), Electricity (power plant), and Mechanical Energy (space shuttle)

- Which of the following is a device that converts electrical energy into mechanical energy?

A generator converts mechanical energy into electrical energy, while a motor does the opposite - it converts electrical energy into mechanical energy. Bothdevices work because of electromagnetic induction, which is when a voltage is induced by a changing magnetic field

- What is an example of mechanical energy to electrical energy?

A lightning strike on a tree is an example of electrical energy being transformed into heat or thermal energy. ... Mechanical energy can also be transformed intoelectrical energy. For example, a hydroelectric plant uses the mechanical energyof flowing water to generate electrical energy

- Which is the best example of electrical energy being converted into heat energy?

Examples of Electrical Energy to Thermal Energy Conversions: 1. A toaster drawselectric current (electrical energy) from a wall outlet and converts these movingelectric charges into heat (thermal energy) in the filaments that turn red hot tocook your toast

- How energy is transformed in electric flat iron?

Then the person must click the on button once this occurs the electrical energy is transformed into thermal energy. There are other types of flat irons such as using water to create steam to straighten the hair or using a chemical called butane this is chemical energy which is later transformed into thermal energy

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of Converters of radiant energy into electricity

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Practical lessons 24-25-26

The theme of the lesson: Geothermal power systems. World Energy Resources. Unlimited and affordable energy sources

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

One of the uses of solar energy is its transformation into electric energy. Photoelectric converters operate not only aboard space vehicles. They are used to supply hard-to-reach sites, for instance, light-houses, communication facilities, etc. with electric power. Such installations can operate continuously for 20 years, and their capacity is up to 500 watts. They are reliable and do not need constantly handling by personal.

At present, mainly semiconductor silicon is used for the manufacture of photocells. Now the researchers have designed photocells on the basis of linking two materials in a single crystal-gallium arsenide and aluminum arsenide. They are most promising for the transformation of preliminary concentrated light since they continue to operate efficiently at temperature of over 200˚ C. Using heat that is re-leased in photocells one can raise the efficiency up to 30%. Their use in solar power station will greatly cut the cost of the photoelectric method of energy trans-formation.

The problem of wider uses of renewable energy sources – solar, tidal and geothermal ones is of great local importance. So far the practical use of solar energy is not very significant but the use of this energy can be profitable in many areas even now. Specialists designed water-heating installations for both seasonable and year-round operation. They have already built solar-powered homes and public buildings with hot water supply, heating and air-conditioning. Within the next few years experimental constructions will go on. After tests the best solutions will be used in standard designs.

The application of solar installations in agriculture has considerable effect. The experience in experimental solar-heating greenhouses has shown that, as compared with ordinary greenhouses that receive heat from boiler rooms, the expenses on vegetable-growing are reduced by 60% due to fuel savings alone. Solar-powered installations for drying farm products were also tested successfully.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “The geothermal power plant”

Make up the situation

Control questions

- What are the 3 types of geothermal power plants?

There are three basic types of geothermal power plants:

Dry steam plants use steam directly from a geothermal reservoir to turn generator turbines. ...

Flash steam plants take high-pressure hot water from deep inside the earth and convert it to steam to drive generator turbines

- What are the different types of geothermal resources?

Geothermal power plants, however, use steam produced from reservoirs of hot water found a couple of miles or more below the Earth's surface. There are three types of geothermal power plants: dry steam, flash steam, and binary cycle. Dry steam power plants draw from underground resources of steam

- What are some examples of geothermal energy current use?

A lava fountain is an example of the amount of heat stored in the Earth. Geysers, lava mountains and hot springs are all natural examples of geothermal energy. In addition, geothermal energy is now a lot more common in homes and businesses, using geothermal heat pumps to cool and heat buildings

- How much of the world's energy is geothermal?

It is considered possible to produce up to 8.3% of the total world electricity withgeothermal resources, supplying 17% of the world population

- What is the source of energy for geothermal power stations?

Geothermal power stations are similar to other steam turbine thermal power stations in that heat from a fuel source (in geothermal's case, the Earth's core) is used to heat water or another working fluid. The working fluid is then used to turn a turbine of a generator, thereby producing electricity.

- What are the disadvantages of geothermal energy?

Geothermal Energy Disadvantages

Potential emissions – Greenhouse gas below Earth's surface can potentially migrate to the surface and into the atmosphere. ...

Surface Instability – Construction of geothermal power plants can affect the stability of land

- Do Geothermal power plants produce pollution?

Geothermal power plants do not burn fuel to generate electricity, so the levels of air pollutants they emit are low. Geothermal power plants emit 97% less acid rain-causing sulfur compounds and about 99% less carbon dioxide than fossil fuel power plants of similar size.

- How do you convert geothermal energy to electricity?

Water or working fluid is heated (or used directly incase of geothermal dry steampower plants), and then sent through a steam turbine where the thermal energy(heat) is converted to electricity with a generator through a phenomenon called electromagnetic induction

- Which state uses the most geothermal energy?

Most geothermal energy is produced in four states--California, Nevada, Utah, and Hawaii. Today the total installed capacity of geothermal power plants in the UnitedState is 3,200 megawatts (MW) That's the energy equivalent of three nuclear power plants.

- Which country produces 30 of the world's geothermal generated electricity?

Iceland, with 27 percent of its electricity needs met by harnessing the earth's heat, is number one in the world in the share of its electricity generated from geothermalenergy

What country uses the most geothermal energy?

Geothermal energy is generated in over 20 countries. The United States is the world's largest producer, and the largest geothermal development in the world is The Geysers north of San Francisco in California

- Does geothermal use a lot of electricity?

While electric baseboards are 100% efficient, geothermal heat pumps are 400% efficient. This means that for every 1 watt of electricity you use, you get 4 watts of heat energy for your home

How geothermal energy works step by step?

Geothermal Power Plants

Hot water is pumped from deep underground through a well under high pressure.

When the water reaches the surface, the pressure is dropped, which causes the water to turn into steam.

The steam spins a turbine, which is connected to a generator that produces electricity.

- What is the largest geothermal power plant in the world?

The Geysers Complex, located in the Mayacamas Mountains, 72 miles north of San Francisco, California, USA, is the largest geothermal field in the world. The complex, and its 22 geothermal power plants therein, has a combined installed capacity of 1,520 MW

- Do Geothermal power stations use fossil fuels?

Geothermal power stations are similar to other steam turbine thermal power stations in that heat from a fuel source (in geothermal's case, the Earth's core) isused to heat water or another working fluid. The working fluid is then used to turn a turbine of a generator, thereby producing electricit

Taks for SIW

1. Give the main idea of the text.

2. Find the information describing this text.

3. Read the text and find characteristics of geothermal power plant

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

Practical lessons 28-29-30

The theme of the lesson: Nuclear power engineering. Nuclear energy, nuclear technology

Lesson plan: to extend vocabulary from the fields of engineering and general technical vocabulary; to improve the skills evaluation, viewing and reading of professional character of the texts;

The purpose and objectives of the lesson, skills and competences: to apply English in professional sphere of the specialist, using skills of four kinds of communications: auditions, readings, letters, and communication in a foreign language; to use logical and critical thinking for problems

Brief theoretical information

Nuclear power engineering. Nuclear energy, nuclear technology

Discuss and write questions to the text

Nuclear technology is technology that involves the nuclear reactions of atomic nuclei. Among the notable nuclear technologies are nuclear reactors, nuclear medicine and nuclear weapons. It is also used, among other things, in smoke detectors and gun sights.

The Many Uses of Nuclear Technology

The first power station to produce electricity by using heat from the splitting of uranium atoms began operating in the 1950s. Today most people are aware of the important contribution nuclear energy makes in providing a significant proportion of clean electricity.

The applications of nuclear technology outside of civil electricity production in power plants are less well-known.

Radioisotopes, nuclear power process heat and non-stationary power reactors have essential uses across multiple sectors, including consumer products, food and agriculture, industry, medicine and scientific research, transport, and water resources and the environment.

Radioisotopes

Isotopes are variants of a given chemical element that have nuclei with the same number of protons, but different numbers of neutrons. Some isotopes are referred to as 'stable' as they are unchanging over time. Others are 'unstable' or radioactive since their nuclei change over time through the loss of alpha and beta particles. The attributes of naturally decaying atoms, known as ‘radioisotopes’, give such atoms several applications across many aspects of modern day life.

The first practical application of a radioisotope was made by a Hungarian man named George de Hevesy in 1911. At the time de Hevesy was a young student working in Manchester, studying naturally radioactive materials. Not having much money he lived in modest accommodation and ate his meals with his landlady. He began to suspect that some of the meals that appeared regularly might be made from leftovers from the preceding days or even weeks, but he could never be sure. To try and confirm his suspicions de Hevesy put a small amount of radioactive material into the remains of a meal. Several days later, when the same dish was served again, he used a simple radiation detection instrument – a gold leaf electroscope – to check if the food was radioactive. It was, and de Hevesy's suspicions were confirmed.

History has forgotten the landlady, but George de Hevesy went on to win the Nobel prize in 1943 and the Atoms for Peace award in 1959. His was the first use of radioactive tracers – now routine in environmental science.

Recommendations on the preparation of students for classes

Students should understand technical texts of varying difficulty.

to search for more information contained in the text.

to generalize (synthesize) the received information from several texts;

-to find sentences, paragraphs, which can be used to argue their point of view during the discussion, the conversation;

comments contained in the text of the facts and events;

-to select information to report .

Tasks and order of work in the classroom:

to read texts with the extraction of complete and accurate information:

to search for the information you need, annotate and abstracted texts:

to keep the conversation in English language within the studied theme:

to do exercises

Technical aids and tools: tape-recorder, Course book and audio CDs (2) Professional English in use. Technical English for professionals. Mark Ibbotson. Cambridge University Press

List and types of homework:

Prepare the report on the theme: “The geothermal power plant”

Make up the situation

Control questions

- What is nuclear energy technology?

Nuclear power is a type of nuclear technology involving the controlled use ofnuclear fission to release energy for work including propulsion, heat, and the generation of electricity. ... The turbine is used to generate electricity and/or to do mechanical work.

- What are 3 different uses for nuclear technology?

Other Uses of Nuclear Technology

Agriculture and Food. In many parts of the world, agricultural workers use radiation to prevent harmful insects from reproducing. ...

Medical. Nuclear technologies provide images inside the human body and can help to treat disease. ...

Space Exploration. Nuclear technology makes deep space exploration possible. ...

Water Desalination.

- What is nuclear power used for?

Nuclear power is the use of nuclear reactions that release nuclear energy to generate heat, which most frequently is then used in steam turbines to produce electricity in a nuclear power plant

- Is nuclear energy the same as nuclear power?

A nuclear reactor produces electricity in much the same way other power plantsdo. The chain reaction produces the energy, which turns water into steam. ... In anuclear energy plant, heat is produced from splitting atoms – a process callednuclear fission

- Why is nuclear technology important?

The importance of the nuclear power industry to the nation's energy security is becoming increasingly clear. ... Nuclear power plants prevent the release of significant quantities of emissions that would be created by burning fossil fuels to generate the same amount of electricity

- Who is the world's top producer of nuclear power?

Nuclear power plants currently operate in 31 countries. Most are in Europe, North America, East Asia and South Asia. The United States is the largest producer of nuclear power, while France has the largest share of electricity generated bynuclear power.

- How does nuclear energy affect the environment?

Nuclear power plants use uranium as fuel. The process of mining uranium releases high amounts of carbon dioxide into the environment. Carbon dioxide is also released into the environment when new nuclear power plants are built. Finally, the transport of radioactive waste also causes carbon dioxide emissions

- How does nuclear energy affect human health?

As radioactive material decays, or breaks down, the energy released into the environment has two ways of harming a body that is exposed to it, Higley said. It can directly kill cells, or it can cause mutations to DNA. ... But it does not concentrate in one part of the body the way radioactive iodine does

Revision questions

Recommended literature

Main literature:

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

Additional literature

1. R.Murphy “English Grammar in Use” a self-study reference and practice book for elementary students of English, and audio CDs (2)  third edition; Oxford University Press. 2011

2. R. Murphy “English Grammar in Use” a self-study reference and practice book for intermediate students of English, and audio CDs (2) third edition); Oxford University Press. 2011

3. A Guide to the Exams.Cambridge Preliminary English Test (Pet), Cambridge First Certificate in English (FCE), Cambridge certificate in advanced English (CAE), Cambridge Certificate in Proficiency in English (CPE). Cambridge University Press 2011.

A brief history of energy

- ~1–2 million years ago: Making energy using fire is invented in Mesopotamia (a region of the Middle East now occupied by Iraq and Syria). Fire releases the energy locked in fuels such as wood, coal, gas, and oil.

- ~3500 BCE: The wheel is invented in Mesopotamia (a region of the Middle East now occupied by Iraq and Syria). Wheels are "simple machines" that magnify force or speed, helping people to use energy more efficiently.

- ~600 BCE: Ancient Greek philosopher Thales (c.624–546 BCE) discovers static electricity.

- ~400 BCE: Ancient Greeks invent gears. A gear is a pair of wheels with teeth around the edge that mesh together to magnify the force or speed of a machine, helping it to use energy more effectively.

- ~27 BCE: Water wheels are developed in ancient Rome by an engineer named Vitruvius. They are an early example of turbines: machines that harness the kinetic energy in moving water or air.

- 1712: English engineer Thomas Newcomen (1663/4–1729) makes the first practical steam engine at Dudley, England. James Watt (1736–1819) later makes it much more efficient. Steam engines greatly increase the demand for coal.

- 1800: Italian physicist Alessandro Volta (1745–1827) develops the first electricity-storing battery, which is called a Voltaic pile. His work owes much to the research of another Italian scientist, Luigi Galvani (1737–1798), who shows that electricity can make a frog's legs move.

- 1840s: James Prescott Joule (1818–1889) shows that energy cannot be created or destroyed, an idea that becomes known as the conservation of energy.

- 1860s: Early gasoline engines are developed by French engineers Jean Joseph Etienne Lenoir (1822–1900) and Alphonse Beau de Rochas (1815–1893) and German engineer Nikolaus August Otto (1832–1891). Gasoline engines create a huge demand for oil in the 20th century.

- 1881: Jacques d'Arsonval (1851–1940), a French physicist, describes how heat energy can be extracted from the oceans.

- 1882: Prolific American inventor Thomas Edison (1847–1931) opens the world's first major electricity producing power plant in Pearl Street, New York City.

- 1884: British engineer Charles Parsons (1854–1931) develops the steam turbine, a machine for turning the energy in steam into electricity.

- 1890s: German engineer Rudolf Diesel (1858–1913) develops the diesel engine.

- 1956: The world's first nuclear power plant opens at Calder Hill in Cumbria, England. It is later renamed Windscale, then Sellafield.

- 1969: The world's first solar power station (a solar furnace used for scientific research) opens at Font-Romeu-Odeillo-Via in France.

- 1982: The world's current largest hydroelectric plant opens at Itaipu on the border of Brazil and Paraguay.

- 2012: The world's largest hydroelectric plant is completed on the Yangtze river in China. (Work commenced in 1994.)

- 2015: Lockheed Martin opens a large-scale OTEC power plant in Hawaii that can successfully extract heat energy from the oceans.

The system of evaluating the results of educational achievements of education receivers'

| Letter Grade | Points in numbers | %-percentage | Assessment in traditional system | Criterion of education receivers’ knowledge assessment |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | excellent | Education receiver’s demonstration: - deep and lasting assimilation of program material - complete, consistent, competent and logically stated answers for all types of classes; - the ability to freely cope with the tasks; - correct, informed decisions made, - the skills of using the information of the main and additional specialized literature in the study of the discipline, - the ability to self-systematize program material; - possession of versatile skills and techniques for completing all types of tasks; - the ability to work with foreign literature and information resources of the Internet; - timely and high-quality implementation of all types of tasks. - understanding of the communicative intentions of the partner, the authors of the texts at this level; - the ability to adequately express their own communicative intentions at this level; - the choice of the form and type of speech / communication at a given level with an adequate logical type of speech; - sufficient completeness of the communicative intention, using real facts, references to authoritative opinion |
| А- | 3,67 | 90-94 |  | Education receiver’s demonstration: - deep assimilation of program material; - complete, consistent and logically stated answers for all types of activities; - ability to cope with tasks; - correct decisions made, - skills of using special literature in the study of discipline, - the ability to self-systematize program material; - possession of skills and techniques for completing all types of tasks; - timely completion of all types of tasks. |
| В+ | 3,33 | 85-89 | good | Education receiver’s demonstration: - assimilation of program material; - complete, consistent, competent, without significant inaccuracies, statement of answers for all types of tasks; - the correct application of theoretical knowledge - possession of the necessary skills in performing applied tasks - the skills of using the recommended literature in the study of discipline, -the skills to systematize program material; - possession of skills and techniques for completing all types of tasks; - timely completion of all types of tasks - a common understanding of the communicative intentions of the partner using clarifications; - the ability to adequately express their own communicative intentions with a fairly correct consideration of their compliance with the socio-cultural standards of the language being studied; - the choice of the form and type of speech / communication with insufficient adequate type of speech logical construction; -insufficient completeness of communicative intent with a sufficiently high degree of evidence using certain facts and references; |
| В | 3,0 | 80-84 |  | Education receiver’s demonstration: - assimilation of program material; - consistent presentation of answers for all types of tasks with minor errors; - the skills in applying theoretical knowledge under the guidance of a teacher; - possession of the necessary skills in performing practical tasks - the skills of using the recommended literature in the study of discipline, - the skills in organizing program material under the guidance of a teacher; - possession of skills for completing all types of tasks; - the ability to independently correct the mistakes made; - timely completion of all types of tasks with the elimination of errors |
| В- | 2,67 | 75-79 |  | Education receiver’s demonstration: - assimilation of program material; - the ability to present answers with minor errors; - the skills in applying theoretical knowledge under the guidance of a teacher; - the skills in performing practical tasks - the skills of using recommended literature under the guidance of a teacher - the skills of generalization of program material under the guidance of a teacher; - the ability to correct mistakes made with the help of a teacher; - timely completion of all types of tasks with the elimination of errors. |
| С+ | 2,33 | 70-74 |  | Education receiver’s demonstration: - assimilation of the basic material; - insufficiently correct wording when answering all types of tasks; - violation of the sequence in the presentation of program material; - difficulties in the independent implementation of practical tasks; - mastery of individual techniques in carrying out practical tasks - the skills of using the literature recommended by the teacher; - the skills of summarizing individual sections of program material under the guidance of a teacher; - the ability to correct gross errors with the help of a teacher; - execution of all types of tasks with the elimination of errors. |
| С | 2,0 | 65-69 | satisfactorily | Education receiver’s demonstration: - assimilation of the basic material; - lack of understanding of the wording of the answers for all types of tasks; - lack of consistency in the presentation of the material; - difficulties in the independent implementation of practical tasks; - possession of certain techniques in the performance of tasks - difficulties in using the literature recommended by the teacher; - difficulties in summarizing the individual sections of the studied material; - the ability to correct gross errors with the help of a teacher; - execution of all types of tasks with the elimination of errors. - only an approximate understanding of the communicative intentions of the partner; - a fairly adequate expression of their own communicative intentions using a limited amount of lexical units; - the choice of the form and type of speech does not always correspond to the communicative plan; - insufficient completeness of the communicative intention, the use of certain facts is distorted due to interference; - inability to enter into discussion at the required level - speech behavior is communicative and cognitively insufficiently expressed |
| С- | 1,67 | 60-64 |  | Education receiver’s demonstration: - assimilation of the basic material; - lack of understanding of the wording of the answers for all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - Inadequate mastery of certain techniques in the performance of tasks - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the synthesis of individual sections of the studied material; - the ability to correct gross errors with the help of a teacher; - execution of all types of tasks with the elimination of errors. |
| Д+ | 1,33 | 55-59 |  | Education receiver’s demonstration: - assimilation of individual sections of the basic material; - misunderstanding of the wording of the answers for all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - significant difficulties in the application of certain techniques for completing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the synthesis of individual sections of the studied material; - difficulties in correcting gross errors made by the teacher; - untimely execution of all types of tasks with the elimination of errors. |
| Д | 1,0 | 50-54 |  | Education receiver’s demonstration: - difficulties with the assimilation of certain sections of the main material; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - significant difficulties in using the literature recommended by the teacher; - inability to generalize certain sections of the studied material; - significant difficulties in correcting gross errors made by the teacher; |
| FX | 0,5 | 25-49 | unsatisfactory | Education receiver’s demonstration: - misunderstanding of the wording of the answers for all types of tasks; - inability to use individual techniques for completing tasks; - untimely execution of all types of tasks with the elimination of errors. |
| F | 0 | 0-24 |  | Education receiver’s demonstration: - ignorance of program material, - when performing all types of tasks gross errors are allowed; - lack of skills in the application of individual techniques for completing tasks; - non-fulfillment of tasks stipulated by the forms of current, intermediate and final control. |

References

1. The typical curriculum, non-profit JSC “Almaty University of Power Engineering and Telecommunication”, minutes №2 dated from 30.06.2016.

2. Ашимова Т.С., Мусалиева М.О. «Future energy» 5B071800 – Электр энергетикасы мамандығының 2 курс студенттеріне арналған ПС ӘН, ОҚМУ Шымкент 2017

3. Campbell Simon. English for the Energy Industry. Oxford Business English. Express series. Oxford University Press, 2009

4. Телесник Г.П. Power Engineering. Методические указания к практическим занятиям, СРСП, СРС по английскому языку для студентов бакалавриата специальностей «Теплоэнергетика», «Электроэнергетика» Усть-Каменогорск, 2010

CONTENTS

| Introduction………………………………………………………………… | 3 |
| --- | --- |
| Unit 1-2 …………………………………………………. | 4-8 |
| Unit 3-4 …………………………………………………. | 9-15 |
| Unit 5-6 …………………………………………… | 16-21 |
| Unit 7-8...................................................................... | 22-25 |
| Unit 9-10 …………………………………………… | 26-29 |
| Unit 11-12 ………………………………………….. | 29-32 |
| Unit 13-14-15………………………………………...... | 32-37 |
| Unit 16-17-18 ………………………………………….. | 37-41 |
| Unit 19-20-21 …………………………………………… | 40-41 |
| Unit 22-23-24 ……………………………………………. | 41-43 |
| Unit 25-26-27…………………………………………… | 43-45 |
| Unit 28-29…………………………………………….. | 45-46 |
| Unit 30 ……………………………………………. | 46-47 |
| Criteria for assessing the knowledge of students | 48-52 |
| References ……………………………………………………………….. | 53 |

Ashenova A.S.

Methodical instruction for practical lessons on Professional-oriented English language

for 6B07150- “Electrical power engineering”

Signed for printing «__»___________ 2020

Paper format 60x 90  1/16

Typographical  paper. Offset printing.Volume 2.5 printer’s sheet.

Circulation 50 copies

© Edition of the South Kazakhstan State University

Publishing centre of the  M.Auezov South Kazakhstan State University,Shymkent