MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC

OF KAZAKHSTAN

M. AUEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

R.R.Yakubova, Zh.K.Dzhanmuldayeva, G.M. Seytmagzimova

STUDY GUIDE

ON THE DISCIPLINE

«ENERGY TECHNOLOGICAL SYSTEMS IN TECHNOLOGY

OF INORGANIC SUBSTANCES»

for Students of Specialty

5B072000 – Chemical Technology of Inorganic Substances

Shymkent, 2020

UDK 621.039

BBK

Yakubova R.R., Dzhanmuldayeva Zh.K., Seytmagzimova G.M. Study Guide on the Discipline «Energy Technological Systems in Technology of Inorganic Substances» for students of specialty 5В072000 – Chemical Technology of Inorganic Substances – Shymkent, M. Auezov South Kazakhstan State University, 2020.  – 101 p.

Study guide represents of the Study Guide on the Discipline «Energy Technological Systems in Technology of Inorganic Substances». In the first part of the study guide is considered the basics of technical thermodynamics and the principles of operation of thermal power plants used in the chemical industry. The second part of the study guide is presented basic information on energy technology, as well as thermodynamic and exergy analysis of energy technology systems in the technology of inorganic substances.

The study guide is designated for students of specialty 5В072000 – Chemical Technology of Inorganic Substances of any form of training.

Reviewers: - Orymbetov E.M., Acting Professor of the department  "Technology of pharmaceutical production" South Kazakhstan Medical Academy, associate professor;

- Zh.A.Shingisbayeva, Head of the Department "Ecology" M. Auezov South Kazakhstan State University, associate professor

The study guide is recommended for print by Study-Methodical Council of M. Auezov South Kazakhstan State University (protocol #____ dated from «___» _____ 2020).

© M. AUEZOV South-Kazakhstan State University, 2020

CONTENT

|  | PREFACE | 5 |
| --- | --- | --- |
|  | Introduction | 6 |
|  | Section 1 General information about thermodynamic analysis of systems | 9 |
| 1.1 | Basic concepts and definitions | 9 |
| 1.2 | Thermodynamic system | 9 |
| 1.3 | State parameters | 10 |
| 1.4 | Equation of state and thermodynamic process | 11 |
| 1.5 | Universal equation of an ideal gas state | 12 |
| 1.6 | The First Law of Thermodynamics | 13 |
| 1.6.1 | Heat and work | 13 |
| 1.6.2 | Internal energy. Enthalpy | 13 |
| 1.6.3 | The first law of thermodynamics | 14 |
| 1.7 | The heat capacity of the gas | 15 |
| 1.8 | The Second Law of Thermodynamics | 16 |
| 1.8.1 | The main provisions of the second law of thermodynamics | 16 |
| 1.8.2 | Enthropy | 17 |
| 1.8.3 | Cycle theory | 18 |
| 1.9 | Compressor plants | 23 |
| 1.9.1 | Displacement compressor | 23 |
| 1.9.2 | Multi-stage compressor | 26 |
| 1.10 | Thermodynamic cycles of heat power plants | 28 |
| 1.10.1 | Cycles of steam turbine plants (STP) | 28 |
| 1.10.2 | Cycles of internal combustion engines (ICE) | 29 |
| 1.10.3 | Cycles of gas turbines (GT) | 32 |
|  | Section 2 General information about energy technology and energy technological systems | 36 |
| 2.1 | The concept of energy-chemical-technological systems (ECTS) | 36 |
| 2.2 | Thermodynamic analysis of chemical-technological systems | 39 |
| 2.3 | The main provisions of the exergy method | 43 |
| 2.4 | Exergy, its types and components | 44 |
| 2.5 | Exergy method of thermodynamic analysis | 47 |
| 2.6 | Exergy balance, exergy efficiency | 52 |
| 2.7 | Types of exergy losses | 58 |
| 2.8 | Exergy performance and power | 62 |
| 2.9 | The relationship between energy and exergy characteristics of thermodynamic systems | 63 |
| 2.10 | The relationship of the overall performance of ECTS with the characteristics of its individual items. Features of exergy loss in ECTS | 65 |
| 2.11 | Exergy efficiency of ECTS and its elements. Grassmann-Shargut diagram | 67 |
| 2.12 | Analysis of individual processes of chemical technology | 70 |
| 2.13 | Exergy analysis of the compressor | 76 |
| 2.14 | Exergy analysis of chemical-technological systems | 79 |
| 2.15 | Analysis and thermodynamic optimization of ECTS | 83 |
| 2.16 | Secondary energy resources (SER) of the chemical industry | 89 |
| 2.17 | Use of secondary energy resources | 91 |
| 2.18 | Utilization plants | 94 |
| 2.19 | Energy-technology combination in chemical technology | 98 |
| 2.20 | The main directions of increasing the efficiency of the use of raw materials and fuel and energy resources | 104 |
|  | REFERENCES | 107 |

PREFACE

The discipline "Energy Technological Systems in the Technology of Inorganic Substances" refers to the cycle of major disciplines (PD) and is included in the M35 module of the mandatory component for students of the specialty 5В072000 - Chemical Technology of Inorganic Substances.

The purpose of the discipline "Energy Technological Systems in the Technology of Inorganic Substances" is the study by students of the thermodynamic analysis of the work of the main heat-utilizing installations of the chemical industry, to study the main ways of saving energy, energy technology systems.

In accordance with this goal, the main objectives of the discipline "Energy technological systems in the technology of inorganic substances" are the acquisition by students of knowledge on:

- the theoretical foundations of the laws of thermodynamic processes of ideal and real gases;

- thermodynamic and thermophysical properties of substances for the calculation of the main parameters of heat-using devices of chemical technology;

- thermodynamic analysis of the operation of the main heat-utilizing installations of the chemical industry;

- the question of choosing a rational technological scheme of production in their technical and economic analysis;

- solving ways to reduce fuel and energy consumption in chemical technology apparatuses;

- heat and power analysis of the main technological processes.

After studying the discipline, the student must

know:

- thermodynamic parameters and thermophysical properties of working fluids;

- thermodynamic features of real gases;

- methods of heat transfer in space;

- the current state and the main directions of improvement of heat-using installations of the chemical industry.

be able to:

- calculate material and heat flows in heat-using devices;

- apply thermodynamic and thermophysical properties of substances to calculate the main parameters of heat-using devices of chemical technology;

- identify ways to reduce fuel consumption and energy in the apparatus of chemical technology;

- to evaluate the main directions of improvement of heat-using installations of the chemical industry.

INTRODUCTION

An assessment of the potential for energy conservation indicates the ability of industry to reduce the need for energy as a result of increased efficiency in their use.

This training manual outlines the basics of technical thermodynamics in the amount that is necessary to understand the principles of the device and the operation of heat-using devices and installations. The theoretical foundations of thermodynamic analysis and optimization of the operation of technological plants are also shown, the fundamentals of the energy technology of chemical industries are described, and specific schemes for recycling secondary energy resources are considered.

The scientific analysis of all energy transformations that occur in the chemical industry is very important and necessary for energy conservation in technological processes. .This analysis should be carried out not only for primary energy resources and raw materials and intermediate products, but also when using secondary energy resources and waste in terms of environmental impact [11].

The basis for such an analysis is modern thermodynamics. Modern thermodynamics is based on the thermodynamic analysis of technological systems and has the following objectives: to take into account the degree of perfection of thermodynamic processes in power plants and save fuel and electricity. Thermodynamic perfection should be understood as the ideal effect of thermal processes. Since all real processes are imperfect, a concept such as the degree of perfection of a process or installation is accepted. The degree of perfection is characterized by the degree of approximation of the real process to the ideal process. With an increase in the degree of perfection, for example, thermal power plants, the specific fuel consumption decreases. The thermodynamic method for the analysis of chemical-technological systems has two varieties: the method of exergy flows (exergy method) and the method of subtracting exergy losses (entropy method) [11].

The exergy method involves the determination of all exergy flows, including those that are closed by themselves, take into account both an increase and a decrease in energy flows in the working processes of the plants.	The entropy method assumes that there is no need to study all exergy flows. Knowing the primary exergy (exergy at the entrance to the unit), it is sufficient to consider only exergy losses in the future. These losses are summed up and make it possible to consider the products of any power plant (electricity or heat exchanger supplied to consumers) as the difference between primary exergy and exergy losses.

Currently, there are two approaches to the study of energy transformations in technical and technological systems. In the first case, the analysis of direct and reverse cycles based on the first and second laws of thermodynamics is carried out. This allows you to find the relationship between heat and work, as well as system parameters. The energy balance of the system is drawn up, as a result of which the thermodynamic efficiency is found.

In this case, it is possible to determine in this system the total loss of work due to the irreversibility of processes. In this case, it is possible to determine in this system the total loss of work due to the irreversibility of the processes.

Losses of work can be due to imperfection of processes, which are internal losses. Another part of the losses is external losses, which are associated with the fact that the system interacts with external sources and receivers of energy - the environment. This is an entropy method for analyzing a technological system.

In the second case, when analyzing the processes of energy conversion in systems, the use of thermodynamic potentials is used. Thermodynamic potentials allow you to directly find the value of any type of work in certain conditions. With their help, it is possible to assess the performance of energy or substance flows at any point of the system under consideration, regardless of its structure and complexity.

Thus, to assess the efficiency of the flow of a substance or energy, not only the parameters of the processes within the system are important, but also their relationship with the environment. A measure of the energy resources of a system that determines the performance of a substance or energy, including environmental parameters, was called exergy.

Exergy, in contrast to the energy associated with the fundamental properties of matter, is a particular concept that characterizes the suitability of energy under given environmental conditions.

An exergy method is a research method based on the analysis of the loss of efficiency in thermodynamic processes. The exergy method of analysis is widespread due to its versatility.  It allows you to compare with each other any kind of energy and on this basis to determine the effectiveness of various processes of its transformation. Exergy indicators are directly related to the technical and economic characteristics of the equipment, which makes it possible to find the economically most advantageous parameters and dimensions of the designed heat and power equipment.

Chemical-technological process is a set of operations that allow to obtain the target product from the feedstock.

The chemical technological process as a whole is a complex system consisting of single interconnected processes (elements) and interacting with the environment.

Elements of the chemical-technological system are heat and mass transfer processes, hydromechanical, chemical and others. They are considered as single processes of chemical technology.

Analysis of individual processes, their mutual influence allows us to develop a technological mode.

A technological mode is a set of technological parameters (temperature, pressure, concentration of reagents, etc.) that determine the operating conditions of the apparatus or system of devices (technological scheme).

The optimal process conditions are a combination of the main parameters (temperature, pressure, composition of the initial reaction mixture, etc.), which allows to obtain the highest yield of the product at high speed or to ensure the lowest cost, subject to the conditions of rational use of raw materials and energy and minimize possible damage environment.

Single processes occur in various apparatuses - chemical reactors, absorption and distillation columns, heat exchangers, etc. Separate devices are connected in the technological scheme of the process.

The technological scheme is a rationally constructed system of single devices connected by various types of connections (direct, reverse, sequential, parallel), which allows to obtain a given product of a given quality from natural raw materials or semi-finished products.

Technological schemes can be open, closed, may contain bypass (bypass) flows and recycles, which can increase the efficiency of the functioning of the chemical-technological system as a whole.

The development and construction of a rational technological scheme is an important task of chemical technology.

Section 1 General information about thermodynamic analysis of systems

1.1 Basic concepts and definitions

In any phenomenon of nature, many different bodies are involved, one way or another connected with each other. In the thermodynamic study of any phenomenon, a group of bodies or a single body, or even its individual parts, is singled out as an object of study. The object of study is called the thermodynamic system, and everything that lies outside its boundaries is the environment. A thermodynamic system is a set of macroscopic bodies exchanging energy both with each other and with the surrounding (external) medium [1, 6,7,8,10,15,17,20].

If the thermodynamic system has no interactions with the environment, then it is called an isolated, or closed, system.

If the external conditions in which the thermodynamic system is located change, then the state of the system will also change.  The combination of changes in the state of the thermodynamic system during the transition from one equilibrium state to another is called a thermodynamic process.

The equilibrium state of the body is understood to mean that at all points of its volume, pressure, temperature, specific volume, and all other physical properties are the same.

The process of changing the state of the system can be equilibrium and non-equilibrium. If the process proceeds through equilibrium states, then it is called equilibrium.

An equilibrium process can be carried out at an infinitely slow change in external conditions or when changes in the parameters characterizing the state of the system are infinitely small compared with the values ​​of the parameters themselves. Consequently, real processes, being nonequilibrium, can only approach to equilibrium ones to one degree or another, never exactly coinciding with it.

If the state of all bodies entering into the thermodynamic system does not change for a long time, then they say that the system is in thermodynamic equilibrium.

The change in the state of the thermodynamic system when interacting with the environment occurs as a result of the thermodynamic process [1, 6,7,8,10,15,17,20].

1.2 Thermodynamic system

Technical Thermodynamics examines patterns of mutual transformation of heat into work. It establishes a relationship between the thermal, mechanical and chemical processes that occur in the heating and cooling machines, studies the processes in gases and vapors, as well as the properties of these bodies at different physical conditions [1, 6,7,8,10,15,17,20].

Thermodynamics is based on two fundamental laws (principles) of thermodynamics:
         The First law of Thermodynamics – the law of energy conservation and conversion and The Second Law of Thermodynamics sets the conditions and the direction of the flow of macroscopic processes in systems consisting of a large number of particles.

Technical Thermodynamics, applying the basic laws of the processes of heat conversion into mechanical work and back, makes it possible to develop the theory of heat engines, to investigate the processes occurring in them, etc.
        The object of study is the thermodynamic system, which may be a group of bodies, body or body part. That is out of the system, called the environment. Thermodynamic system is a set of macroscopic bodies, exchanging energy with each other and the environment. For example: a thermodynamic system is a gas in a cylinder with a piston, and the environment is a cylinder, piston, air, walls of the room.
     Isolated system is a thermodynamic system which does not interact with the environment.
     Adiabatic (thermally insulated) system is a system with adiabatic shell, which excludes the exchange of heat with the environment.

Homogeneous system is a system that has the same composition and physical properties in all its parts.

Homogeneous system is a system in composition and physical structure within which there is no interfaces (ice, water, gas).

Heterogeneous system is a system consisting of few homogeneous parts (phases) with different physical properties that are separated one from another by partition visible surfaces (ice and water, water and vapor).

In heat engines (engines) mechanical work is done by working fluids: gas, steam [1, 6,7,8,10,15,17,20].

1.3 State parameters

The parameters ​​that characterize the physical condition of the body, are called the thermodynamic parameters of the state. These parameters are specific volume, absolute pressure, absolute temperature, internal energy, enthalpy, entropy, concentration, heat capacity, etc. In the absence of external force fields (gravitational, electromagnetic, etc.) thermodynamic state of the single-phase body can be identified by 3 parameters: the specific volume (v), temperature (T), pressure (P)[1,6,7,8,10,15,17,20].
 	Specific volume is a parameter determined by the ratio of the volume of a substance to its mass:

v=Vm ,   m3 /kg                                            (1.1)

The density of substance is a parameter determined by the ratio of mass to volume of the substance.

=mV,   kg/m3                                           (1.2)

v=1,    =1v,  v∙  =1                                    (1.3)

Pressure – in terms of the molecular-kinetic theory – is an average result of blows of gas molecules which are in continuous random motion, to the wall of the vessel in which the gas is enclosed.

P=F/S                                                 (1.4)

There is excessive and absolute pressure.

Excessive pressure (Pex) is a difference between the pressure of fluid and the ambient pressure.

Absolute pressure (Pabs) is a pressure measured from absolute zero pressure or absolute vacuum. This pressure is a thermodynamic parameter of state.
      Absolute pressure is:

- At a pressure of vessel greater than atmospheric:

Pabs= Pat+ Pm                                          (1.5)

- At a pressure of vessel less than atmospheric:

Pabs= Pat- Pvak                                        (1.6)

where Pat  is atmospheric pressure;

Pm  is manometric pressure;                            
          Pvak is vacuum pressure.

Temperature characterizes the degree of heating of the body, a measure of an average kinetic energy of the translational motion of the molecules. The more the average speed, the higher body temperature. For thermodynamic parameter of the system can be the thermodynamic temperature (T), i.e. absolute temperature. It is always positive. At a temperature of absolute zero (T = 0) thermal motions cease and this temperature is the origin of the absolute temperature.

1.4 Equation of state and thermodynamic process

The basic thermodynamic parameters of state P, v, T of homogeneous depend on each other and are mutually connected a mathematical equation, which is called the equation of state

fP, v, T=0                                               (1.7)

Equilibrium state is the state of the body, in which all points of P, v and T and all the other physical properties are the same.

The set of changes in the state of a thermodynamic system during the transition from one state to another is called as thermodynamic process. Thermodynamic processes can be equilibrium and nonequilibrium. I f the process goes through the equilibrium state, it is called the equilibrium I n real cases, all the processes are nonequilibrium. If in any thermodynamic process the change of state parameter does not depend on the type of process, and by the initial and final states, the state parameters are called state function. These parameters are the internal energy, enthalpy, entropy, etc.

Intensive parameters are parameters which do not depend on the parameters of the mass of the system (pressure, temperature).

Additive (extensive) parameters are parameters that are proportional to the mass of the system (volume, energy, entropy, etc.) [1, 6,7,8,10,15,17,20].

1.5 Universal equation of an ideal gas state

Ideal gas is a gas that has no force of mutual attraction and repulsion between molecules and molecular size is neglected. All real gases at high temperatures and low pressures can be practically considered as ideal gases [1, 6,7,8,10,15,17,20].

The equation of state for ideal gases as for real are described by three parameters of the equation (1.7).

The equation of state of an ideal gas can be deduced from the molecular-kinetic theory or from the joint consideration of the laws of Boyle-Mariotte and Gay-Lussac. This equation was derived in 1834 by French physicist Clapeyron and for 1 kg of gas is given by:

P∙v=R∙T                                              (1.8)

wherein: R is gas constant and represents the work of one kg of gas in the process at constant pressure and temperature changes by 1°.

Equation (1.8) is called the thermal equation of state or the characteristic equation. For an arbitrary number of gas of mass m the equation of state:

P∙V=m∙R∙T                                              (1.9)

In 1874, Mendeleev, based on the law of Dalton ("In equal volumes of different ideal gases at the same temperature and pressure the same number of molecules contains"), offered a universal equation of state for 1 kg of gas, which is called the Clapeyron –Mendeleev

P∙v= R ∙T/                                              (1.10)

where    is molar (molecular) gas mass, kg / kilomole;

R  = 8314,20 J/mole (8,3142 kJ/kilomole) is a universal gas constant and represents the work of 1 mole of ideal gas in the process at constant pressure and temperature change by 1 degree.

Knowing R, it is possible to find the gas constant R = R/.
For an arbitrary mass of gas Clapeyron-Mendeleev equation would be:

P∙V=m∙R∙T/                                 (1.11)

1.6 The First Law of Thermodynamics

1.6.1 Heat and work

The bodies involved in the flow of the thermodynamic process exchange by energy. The transfer of energy from one body to another occurs in two ways.
The 1st method is implemented by direct contact of bodies with different temperatures, by an exchange of kinetic energy between molecules of bodies in contact or radiant transfer of internal energy of radiating objects by electromagnetic waves. In this case, energy is transferred from a warmer to a less heated [1, 6,7,8,10,15,17,20].
     	The amount of energy transferred by the 1st method from one body to another is amount of heat Q (J), and the method is the transfer of energy in the form of heat.

The 2nd way is associated with the presence of force fields, or external pressure. To transfer energy in this manner body must either move in force field, or change its volume under the influence of external pressure, i.e. energy transfer occurs under the condition of movement of the body or its part in the space. The number of transmitted energy is called work  L (J), and a way of transferred energy in the form of work.

The amount of energy produced by body in the form of the work, called a work done on the body, and transferred energy is a work of body.
The amount of heat produced (Given) by body and work done (spent) on the body depend on the conditions of the transition of the body from the initial to the final state, i.e., depend on the nature of the thermodynamic process.

1.6.2 Internal energy. Enthalpy

In general, the internal energy is a collection of all kinds of energy contained in a body or system of bodies. This energy can be represented as the sum of the individual types of energy: kinetic energy of molecules (translational and rotational motion of the molecules), the vibrational motion of the atoms in the molecule itself; energy of electrons, nuclear energy, the energy of interaction between the nucleus and the electrons of the molecule, the potential energy of the molecules.
     	The technical thermodynamics deals only with those processes that change the kinetic and potential internal components of energy. In this case, knowledge of the absolute values ​​of the internal energy is not required. Therefore, the internal energy of an ideal gas is the kinetic energy of the molecules, and the energy of the vibrational motion of the atoms in the molecule, and for real gases additionally the potential energy of the molecules is included [1, 6,7,8,10,15,17,20].

The internal energy (U) is a function of two main parameters of state of the gas, i.e., U = f (P, T), U = f (v, T), U = f (P, v). Each state of the working body (system) corresponds to a definite value of state parameters, each state of the gas is characterized by its own single-value, certain parameter of internal energy U. That is, U is a function of the gas state. And the difference between the internal energies of the two states of working body or system of bodies will not be affected by the transition from the first state to the second.

Four quantities called "thermodynamic potentials" are useful in the chemical thermodynamics of reactions and non-cyclic processes [1, 6,7,8,10,15,17,20]. They are internal energy, the enthalpy, the Helmholtz free energy and the Gibbs free energy. Enthalpy is defined by

H=U+P∙V                                                       (1.12)

where P and V are the pressure and volume, and U is internal energy. Enthalpy is then a precisely measurable state variable, since it is defined in terms of three other precisely definable state variables.

1.6.3 The first law of thermodynamics

The first law of thermodynamics is the basis of the thermodynamic theory and is of great practical importance in the study of the thermodynamic processes [1, 6,7,8,10,15,17,20]. This law is the law of conservation and conversion of energy: "Energy does not disappear or recurs, it only moves from one form to another in a variety of physical processes".

For thermodynamic processes the law establishes the relationship between heat, work and the change in internal energy of a thermodynamic system: "The heat supplied to the system, consumed on the change of the energy of the system and the work".

The equation of the first law of thermodynamics is:

Q=U2- U1+L                                             (1.13)

where Q is the amount of heat to the system;

L  is the work done by the system (on the system);

U2- U1 is the change of internal energy in the process.
If   Q > 0 then the heat is supplied to the system.

If    Q < 0 then the heat is removed from the system.

If    L > 0 then work is done by the system.

If    L < 0 then work is done on the system.
      For a unit mass of substance the equation of the first law of thermodynamics is:

q=Qm=u2- u1+l                                           (1.14)

The First law of thermodynamics states that for obtaining the useful work (L) in a continuous heat engine it is necessary to supply the heat (Q).  "The engine constantly producing work and do not consume any energy is called perpetual motion machine of I kind".

From this we can make the following definition of the First law of thermodynamics: "Perpetual motion machine of the first kind is not possible".

1.7 The heat capacity of the gas

The real heat of the working fluid is determined by the ratio of heat supplied to the working fluid in the thermodynamic process caused by these changes in body temperature [1, 6,7,8,10,15,17,20].

C= dQ/dT                                           (1.15)

Heat capacity depends on external conditions or the nature of the process, at which the supply or removal of heat occurs.

There is the following specific heat capacity:

- mass heat capacity  с = С/m ,   J/(kg .K)                                                        (1.16)

- molar heat capacity   с = С/v,  J/(mole .K)                                                   (1.17)

where v is the amount of substance, mol.

- volume heat capacity   с/ = С/V = с· ,   J/(m3 .K)                                        (1.18)

where   = m /V is the density of the substance.
      The link between this specific heat capacity:

с = с/ · v = с /                                                           (1.19)

where  v = V/m is specific volume of the substance, m3/kg;

= m /ν is molar (molecular) weight, kg/mol.

The specific heat of gases largely depends on the conditions under which the process of heating or cooling is realized. There is the specific heat at constant pressure (isobaric) and constant volume (isochoric).

Thus, there are the following specific heats:

ср, сv is mass isobaric and isochoric heat capacity;

сp, сv is molar isobaric and isochoric heat capacity;

с/p, с/v is volume isobaric and isochoric heat capacity.
        Between isobaric and isochoric heat capacity there is the following relationship Mayer equation determined by the formula:

cp- cv=R                                                      (1.20)

cp- cv=R                                                    (1.21)

The heat capacity depends on the temperature, which is given in reference books in the form of a table as an average specific heat in the temperature range from 0 to tх. To determine the average heat capacity in the temperature range of t1 to t2 it is possible to use the following formula:

с|t2t1 = (с|t20 t2 – с|t10 t1) / (t2 – t1)                                      (1.22)

1.8 The Second Law of Thermodynamics

1.8.1 The main provisions of the second law of thermodynamics

The first law of thermodynamics states that heat can be converted into work, and work into the heat and it does not establish the conditions under which these transformations are possible [1, 6,7,8,10,15,17,20].

The transformation of work into heat is always fully and unconditionally. The reverse process of converting heat into work with its continuous transition is possible only under certain conditions and not completely. The heat itself can move from the hotter body to the colder.  Transfer of heat from the cold body to a heated is not happening itself. To do this, it is necessary to expend extra energy.
     	Thus, for a complete analysis of phenomena and processes it must be additional regularity other than the first law of thermodynamics even more regularity. This law is the Second Law of Thermodynamics. It establishes if the process is possible or impossible, the direction in which the process takes place, when the thermodynamic equilibrium is reached and under what conditions it is possible to get maximum performance [1, 6,7,8,10,15,17,20].

The formulations of the second law of thermodynamics.
      For the existence of a heat engine 2 sources of heat are required: hot source and a cold source (environment). If the heat engine runs only from a single source, it is called perpetual motion of 2nd kind.

1 formulation (Ostwald): "Perpetual motion machine of the 2nd kind is impossible".
    	 Perpetual motion machine of the 1st kind is a heat engine, which L > Q1, where Q1 is the supplied heat. The first law of thermodynamics "allows" the possibility of a heat engine, completely transforms the supplied heat Q1 to the work of L, that is L = Q1. The second law imposes greater restrictions, and claims that the work should be less than supplied heat (L < Q1) on the amount of removed heat Q2, i.e. L = Q1 – Q2.

Perpetual motion machine of the 2nd kind can possible if the heat Q2 transferred from a cold to a hot source. But for this heat must pass spontaneously from a cold to a hot body, which is impossible. It follows 2nd formulation (Clausius): "The heat cannot spontaneously pass from a colder to a warmer body".
    	 For the operation of a heat engine 2 sources of heat – hot and cold – are required.

Third formulation (Carnot): "Where there is a temperature difference, the work can be carried out".

All these formulations are interconnected, one formulation can be obtained from another.

1.8.2 Enthropy

One of the functions of the state of a thermodynamic system is entropy. Entropy is a value defined by the expression:

dS=dQ/T                                                (1.23)

or for the specific entropy:

ds=dq/T                                               (1.24)

The entropy of a single-valued function of the state of the body, having a definite value for each state. It is extensively (depending on the mass of material) state parameter and any thermodynamic process completely determined by the initial and final state of the body and does not depend on the path of the process.
Entropy can be defined as a function of the basic parameters of the state:

S= f1P, V;  S= f2P, T;   S= f3 V, T                     (1.25)

or for the specific entropy:

s= f1P, v;  s= f2P, T;   s= f3v, T                     (1.26)

Since entropy is independent of the process and determined by the initial and final states of the working body, there is only the change in the process which can be found by the following equations:

s= cv∙lnT2T1+R∙lnv2v1                                    (1.27)

s= cp∙lnT2T1- R∙lnP2P1                                    (1.28)

s= cv∙lnP2P1+ cp∙lnv2v1                                    (1.29)

If the entropy of the system increases (s > 0), the system is supplied with heat. If the entropy of the system decreases (s < 0), then the system removes heat.
If the entropy of the system does not change (s = 0, s = const), then the system is not supplied and removes heat (adiabatic process) [1, 6,7,8,10,15,17,20].

1.8.3 Cycle theory

Historically, the second law of thermodynamics has emerged as a working hypothesis of thermal machines, which sets the conditions of the conversion of heat into work from the point of view of the maximum of this transformation, i.e. obtain maximum efficiency of a heat engine [1, 6,7,8,10,15,17,20].

Theoretically, heat engines operate in circular thermodynamic processes or cycles.

The process in which the working fluid, coming from some initial state, and having undergone a number of changes, it returns to the same state, called a circular process or cycle.

The design of cycle requires three elements: a heater or a heat receiver with T1, the fridge T2 < T1 and the working fluid, which consistently engaging in heat exchange with the heater (or heat receiver) and a refrigerator transfers energy from one to the other. Cycles are reverse and direct, reversible and irreversible.

From the definition of the cycle, it follows that in all the cycles of ΔU, ΔS, Δh to zero.

It follows that the first law of thermodynamics for cycles is mathematically represented as

qc= lc                                                 (1.30)

where qcand lc  is the algebraic sum of heat and work of all the thermodynamic processes of which the cycle consists of.

Direct cycle is a cycle of heat engine in which the conversion of heat into work realizes.

l c= l1- l2  0                                         (1.31)

q c= q1- q2  0                                         (1.32)

P

D

V

A

C

F

E

B

lc=l1-l2

l1

l1

S

T

2

f

e

c

a

в

d

qc= lc

Figure 1. PV- and TS- diagrams of the direct cycle

The heat of the heater cannot be completely converted into work. The formulation of Carnot.

In the direct cycle it must be that the maximum of heat supplied to working fluid turns into work. The effectiveness of direct cycle is evaluated by thermal efficiency of the cycle.

d

V

P

в

D

a

с

S

Т

в

a

с

Т1

Т2

ΔS

Isotherm

Adiabatic

Figure 2. PV- and TS- diagrams of the direct ideal Carnot cycle

t= lcq1= qcq1= q1- q2q1= 1- q2q1                               (1.33)

It is not possible to create a heat engine, the thermal efficiency of which would be equal to unit is the formulation of Carnot.

The ideal cycle of heat engine is a direct reversible Carnot cycle.

lc=l1-l2=q1-q2=T1-T2ΔS                             (1.34)

ηtc=lcq1=T1-T2ΔST1ΔS=+T1-T2T1=1-T2T1                              (1.35)

The thermal efficiency of the Carnot cycle for gas does not depend on its nature and is uniquely determined by the heater and cooler temperatures.

Reverse cycle is a circular process of cooler and heat pump, spent work from the outside in order to transfer heat from the refrigerator to the heat receiver.

P

B

V

A

C

F

E

D

S

T

2

f

e

c

a

d

в

Figure 3. PV- and TS- diagrams of the reverse cycle

In expanding the working fluid, it is supplied with the amount of heat q1 from the fridge T2. At subsequent compression of the working fluid heat quantity q1 greater q2 is removed into the heat receiver with temperature T1  T2.

Thus, qc=q1-q2  0,  lc=l1-l2  0.

The First Law of Thermodynamics      q2-q1=- lc  ,  q1=q2+lc,

i.e. heat removed by working fluid into heat receiver is equal to sum of heat received by it from the fridge, and heats which are equivalent to work spent on cycle.

The effectiveness of the reverse cycle assesses by cooling coefficient:

=q2lc=q2q1-q2                                                 (1.36)

Thus, the implementation of the reverse-cycle without costs of works from the outside is not possible.

Heat cannot come from a cold body to a warmer itself (without the *) is Clausius statement.

The warmth of the coldest body in the system cannot be the source of the work is the formulation of W. Thomson.

The ideal cycle of cooler and heat pump is reversible reverse Carnot cycle.

=q2q1-q2=ΔSTxΔST1-ΔSTx=TxT1-Tx                                (1.37)

Adiabat

2

V

P

4

2

1

3

S

Т

4

1

3

Т1

Т2

ΔS

Isotherm

Figure 4. PV and TS diagrams of the inverse ideal Carnot cycle

The analytical expression of II law of thermodynamics. For any reverse cycle:

ABCDAqT= ABCDAdS=0                                      (1.38)

dq=TdS                                               (1.39)

For an irreversible cycle:

ABCDAqT ABCDAdS=0                                       (1.40)

qT   dS                                                             (1.41)

The irreversibility of the process is associated with an increase in entropy:

q=TdS

Example. The air in the counter-current heat exchanger is heated from the temperature t1 = 40 0С, and the gases are cooled from the temperature t3 = 450 0С to the temperature t4 = 200 0С. The heat loss of the heat exchanger is 20 % of the heat released by the gas. Determine the loss of working capacity per 1 kg of passing gas due to irreversible heat transfer. Gas and air are considered ideal gases possessing air properties. The heat capacity of air and gases is assumed to be constant. The ambient temperature is t0 = 25 0С.

We determine the temperature t2, to which air is heated, from the heat balance equation:

0,8∙cpt3- t4= cpt2- t1

whence

t2=0,8∙t3- t4+ t1= 0,8∙450- 200+ 40=240  0C

Thermal losses per 1 kg of passing gas are equal:

q=0,2∙t3- t4= 0,2∙1,0∙450- 200=50,0 kJ/kg

where cpis specific heat of gas, cp = 1,0 kJ/(kg.К).

Decrease in working capacity is determined from equation:

l0= T0ssys

where T0 is the ambient temperature, and ssys.  It is composed of changes in the entropy of gas s1, air s2  and environment s3 due to the transfer of heat to the environment (in the form of losses).

The change of the entropy of the gas:

s1 = cp∙lnT4T3 = 1,0 . 2,3 lg(473/723) =  – 0,425 kJ/(kg.К).

The change of air entropy:

s2  = cp∙lnT2T1 = 1,0 . 2,3 lg(513/313) =  + 0,495 kJ/(kg.К).

The change of the entropy of the environment:

s3 = q/T0 = 50/298 =  + 0,168 kJ/(kg.К).

The change of the entropy of the system:

ssys= s1+ s2 + s3 = – 0,425 + 0,495 + 0,168 = 0,238 kJ/(kg.К).

The loss of the working capacity of the installation per 1 kg of passing gas is

l0= T0ssys= 298 . 0,238 = 71 kJ/kg.

1.9 Compressor plants

1.9.1 Displacement compressor

Compressed gas, usually air, produced by compressors are widely used in a number of sectors of the economy, such as metallurgical industry (blast furnace), the chemical industry, gas turbines and jet engines, pneumatic tools, etc.

According to the principle of the device and work all compressors are divided into two groups: the volume and blade. Volume compressors are divided into reciprocating and rotary, blade: centrifugal and axial. Thermodynamic principles of operation are similar [1, 6,7,8,10,15,17,20].

Single-stage reciprocating compressor

Compressor of static compression that occurs as a result of reducing the volume where the gas is enclosed.

V

P

E

F

0

1

2''

2

2'

C

D

P2

P1

Vh

BDC

TDC

S

Т

1

2''

2

2'

P2

P1

a is scheme of work; b is a theoretical indicator diagram;

c is the compression process in the TS diagram.

Figure 5. Single stage piston compressor circuit

Figure 5 shows a schematic diagram of a single stage reciprocating compressor. Crankshaft of compressor is driven by an electric motor or piston of internal combustion engine.

When the piston moves from TDC to BDC gas is sucked into the cylinder with chilled cover through an automatic shut-off valve A from the environment. The discharge valve В is closed under the pressure of gas in the tank, there is no more atmospheric. At the return moving of the piston from BDC to TDC gas begins to compress, its pressure increases and the suction valve closes. The compression process continues until such time as the pressure in the cylinder becomes equal to the pressure in the tank. Then the valve is opened and the process of injecting compressed gas into the tank begins until the piston comes to TDC.

At the return moving of the piston from BDC to TDC gas begins to compress, the pressure increases and the suction valve closes. The compression process continues until the pressure in the cylinder becomes equal to the pressure in the tank. Then the valve B opens, the discharge process starts of the compressed gas in reservoir starts until the piston comes to TDC.

Vh is worker (useful) volume of the cylinder; Р1 is ambient pressure; Р2 is the gas pressure in the reservoir.

D – 1 is suction, 1 – 2 is compression, 2 – С is injection.

The most useful for the compression process on work from the outside to drive the compressor was an isothermal process 1–2. In this case, the ideal conditions for preservation of lubricating oil (viscosity, etc.) are observed.

However, the isothermal compression of gas in the compressor is practically impossible, and the compression curve is located between the isotherm and the adiabatic n = 1,2–1,25.

The most useless from the point of view of the high values ​​T2 adiabatic process 1–2" was.

A0=Asys+A0-Asuc=V1V2PdV+P2V2-P1V1                     (1.42)

For an isothermal process:

AprТ=P1V1lnP1P2                                                   (1.43)

For adiabatic process:

Aprad=1к-1P1V1-P2V2                                       (1.44)

For a polytropic process:

Aprpol=1n-1P1V1-P2V2                                                 (1.45)

A0Т=P1V1lnP1P2+P2V2-P1V1=P1V1lnP2P1                        (1.46)

as                                                           P2V2=P1V1

A0pol=1n-1P2V2-P1V1+P2V2-P1V1=nn-1P2V2-P1V1=

=P2V2-P1V1+P2V2-P1V1=nn-1P1V1P2V2P1V1-1=

=nn-1P1V1P2P1n-1n-1                                   (1.47)

A0ad=kk-1P1V1P2P1k-1k-1                                 (1.48)

VsusVh=ηvol   is volumetric efficiency of the compressor

P

2

P2

C

D

P1

V

V0

TDC

BDC

Vвс

Vh

Figure  6. PV- diagram of the presence of "harmful" space

1.9.2 Multi-stage compressor

Compression of gas is performed sequentially in several cylinders (up to seven) with an intermediate cooling after each cylinder in special refrigerators [1, 6,7,8,10,15,17,20].

As an example, let’s consider the work of a three-stage piston compressor, scheme of which is shown in Figure 7, and the workflow – in the PV-coordinates (for the ideal compressor) – on Figure 8.

1

6

5

Х2

Х1

4

3

2

III

II

I

Figure 7. Schematic diagram of the three-stage reciprocating compressor

V1

P

4

6

7

P6

P1

6'

5

2

3

1

V

P4= P5

P2= P3

Figure 8. The theoretical indicator diagram of a three-stage compressor

The work of a multi-stage compressor is to be organized so as to ensure the following three conditions:

1)  cooling of the gas in all the refrigerators, i.e. the gas temperature is adjusted to an initial temperature T1 which it had at the entrance of the first stage

Т1= Т3 = Т5

2) the similar final temperature of the gas compression at all levels, providing all cylinder’s reliable lubrication conditions in all cylinders

Т2= Т4 = Т6

3) same polytropic compression in all cylinders, i.e.

nI = nII = nIII

At these conditions’ differential pressures (the ratio of the final to the initial pressure) at all stages are similar

Р2/Р1 = Р4 /Р3 = Р6/Р5

To determine the total work done in the drive of a multistage compressor, it is necessary to sum the work done in compressing the gas for individual steps. It is easy to show that at these three conditions, LI0, LII0, LIII0 are to be the same.
In accordance with (1.8) we have:

L0I= n(n-1) P1v1P2P1n-1n-1 = n(n-1) RT1P2P1n-1n-1                (1.49)

L0II= n(n-1) P3v3P4P3n-1n-1 = n(n-1) RT3P4P3n-1n-1               (1.50)

L0III= n(n-1) P5v5P6P5n-1n-1 = n(n-1) RT5P6P5n-1n-1             (1.51)

Since the right-hand sides are equal, then L0I = L0II = L0III. Then the theoretical work m of stage compressor expended in compression of 1 kg of gas is determined by the product m·L0.

Theoretical capacity N0 (W) expended on the compressor drive may be determined by the equation:

N0=M∙m∙ L0                                      (1.52)

where M is compressor capacity kg/sec;

L0 is theoretical work on compression of 1 kg of gas in one stage J/kg;

m is the number of compressor stages.

In order to determine the real (effective) power Ne required to drive the compressor, it is necessary to know the loss of work to overcome the resistance of valves and pipelines and the friction in the adjoining parts of the compressor, which are taken into account by the mechanical efficiency:

Ne=N0m=M∙m∙ L0/m                                       (1.53)

For reciprocating compressor m= 0,8–0,9.

1.10 Thermodynamic cycles of heat power plants

1.10.1 Cycles of steam turbine plants (STP)

Steam turbine is the basis of modern thermal and nuclear power plants. Working body in such installations is any liquid vapor (steam). The main cycle in the steam turbine plant is the Rankine cycle [1, 6,7,8,10,15,17,20].

Schematic diagram of the STP is shown in Figure 9, and the process of getting works conducts in the following way. In the steam boiler 1 and the superheater 2 combustion heat is transferred to the water. The received vapor enters the turbine 3, wherein the heat is converted into mechanical and then electrical energy in the electric generator 4. Exhaust steam enters the condenser 5, where it gives off heat to cooling water. The received condensate is sent to the nutrient tank 7 by pump 6, where condensate is compressed by the supply pump 8 till to a pressure of the boiler, and is supplied through the pre-heater 10 to the steam boiler 1.

Figure 9. Schematic diagram of the steam turbine plant

Let’s consider the Rankine cycle for saturated steam. Plant scheme differs from the previous scheme in that in this case there will be no superheater. Therefore, the turbine will be supplied by saturated steam. On Figure 10, the Rankine cycle TS-diagram is shown.

Processes:
           3–1 is heat supply from the source in water q1, consists of two processes:

3–3/ is boiling water in the boiler;
3/–1 is evaporation of water to steam at a constant pressure;
1–2 is in the turbine steam expands adiabatically;
2–2/ isvapor condenses and releases heat q2 to cooling water;
2/–3 is condensate is adiabatically compressed.

Thermal efficiency of Rankine cycle is determined by the equation:

t= q1- q2/q1                                   (1.54)

as                                           q1= h1- h3;  q2= h2- h2                                  

that

t=h1- h2- h3- h2h1- h3=l/q1                               (1.55)

Useful work of cycle is equal to the difference between the turbine and the pump works:

l= lt- lp

where:                                   lt= h1- h2 ; lp= h3- h2

Basically  lt >> lp, while assuming h3 = h2, we can write:

t= h1- h2/h1- h3                                  (1.56)

The theoretical capacity of the turbine is calculated using the formula:

Nm= h1- h2∙D3600,  W                                    (1.57)

where: D = 3600·m is consumption per hour, kg/h;
            m is second flow rate, kg/sec.

Rankine cycle with superheated steam is used to increase the thermal efficiency of STP cycle. For this purpose, in front of superheater 2 the turbine is installed (Fig.9), which increases the temperature and pressure of steam. At this the average temperature of the heat medium in the cycle increases. Cycle diagram is shown in Figure 10b. The formula for calculating l, t, Nt remain unchanged.

Figure 10. Rankine cycle on saturated (a) and superheated (b) steam

    - Cycles of internal combustion engines (ICE)

Cycles of reciprocating internal combustion engines are divided into three groups: with supply of heat at constant volume (gasoline engines); with supply of heat at constant pressure (compressor diesel engines); with mixed supply of heat at constant volume.

The main characteristics or parameters of any heat engine cycle are the following dimensionless values:
- compression ratio (ratio of the specific volume of the working fluid at the beginning and end of compression)

= 1/2                                              (1.58)

- pressure ratio (pressure ratio at the end and the beginning of isochoric heat supply process)

= P3/P2                                              (1.59)

- degree of pre-expansion or isobaric expansion degree (ratio of the specific volume at the end and the beginning of isochoric heat supply process)

= 3/2                                              (1.60)

1) Let’s consider cycle of engine with supply of heat at constant volume on the example of four-stroke engine.

The diagram shows the actual engine on Figure 11.

a–1 (1 stroke) – into the cylinder through the intake valve a mixture of air and fuel vapor (no thermodynamic process) comes;

1–2 (2 stroke) – adiabatic compression (temperature increases);

2–3 – combustion of gas mixture, the pressure rapidly increases at constant volume (heat supply q1);

3–4 (3 stroke) – adiabatic expansion (workflow, useful work);

4–a – the exhaust valve opens and the exhaust gases leave the cylinder, pressure drops (heat q2 is removed);

1–a (4 stroke) – expulsion of the remaining gases in the cylinder. The process repeats.

Figure 11. Internal combustion engine cycle with constant heat input

Described process is irreversible (friction, chemical reactions in the working fluid, finite piston velocity, heat at a finite temperature difference, etc.).
          For the analysis of the theory of  heat engines thermodynamics considers ideal cycles as reversible cycles. Ideal process the internal combustion engine is shown in Figure 12.

Figure 12.  Ideal process the internal combustion engine

This diagram shows the formula for thermal efficiency cycle with a supply of heat at constant volume, which is as follows:

t=1-1/k-1                                          (1.62)

where t is the compression ratio (the main parameter of the engine, the higher the , the higher the efficiency of internal combustion engines);

k is adiabatic parameter.

= v1/v2 – compression index of engine.

2) The ideal cycle of internal combustion engine with mixed supply of heat

at constant volume. Cycle is shown in Figure 13.

Figure 13. Mixed heat combustion engine cycle

1–2 – clean air with temperature T1 is compressed to a temperature T2 which is greater than the ignition temperature of fuel. At this moment fuel is injected into the cylinder through the nozzle under pressure);
2–3 – fuel-air mixture ignites and heat q1/ is supplied to the working fluid, the pressure increases to P3);
3–4 –  the piston moves back,  supply and combustion of fuel continues at constant pressure and heat q1// is supplied);
4–5 – the piston continues to move to the bottom dead point, the pressure drops (adiabatic expansion);

5–1 – process of removing heat q2 at constant volume (through the exhaust valve exhaust gases leave).

Cycles of engines with heat supply at a constant pressure have not found wide application, since these cycles have very high compression ratio.

1.10.3 Cycles of gas turbines (GT)

The main disadvantages of reciprocating internal combustion engines are the limitations of their power and the inability to adiabatic expansion of the working fluid to atmospheric pressure, which are absent in the gas turbines in GT working body is the products of combustion of liquid or gaseous fuels [1, 6,7,8,10,15,17,20].
     	On Figure 14 there is a diagram of a simple gas turbine with the combustion of fuel at constant pressure. By fuel pump 5 and the compressor 4 the fuel and air through the nozzles 6 and 7 are supplied to the combustion chamber 1. From the combustion chamber products of combustion are directed to the nozzle 2, where they extend, and come to a gas turbine blade 3.

Figure 14. Diagram of a simple gas turbine with the combustion of fuel at constant pressure.

On Figure 15 and Figure 16 the ideal cycle of gas turbine plant at PV and TS diagrams presents.

Figure 15. The ideal cycle of gas turbine plant at PV diagrams

1–2 – adiabatic compression to a pressure P2;
2–3 – heat supply q1 at a constant pressure P2 (combustion);
3–4 – adiabatic expansion to the initial pressure P1;
4–1 – cooling of the working fluid at a constant pressure P1 (heat removal q2)
      Cycle characteristics are:
pressure ratio is    = Р2/ Р1 ;
degree of isobaric expansion is   = V3 /V2 .

Figure 16. The ideal cycle of gas turbine plant at TS diagrams

Work of the turbine is:

lt= h3- h4                                            (1.64)

Work of the compressor is

lc= h2- h1                                            (1.65)

Useful work of GTP is equal to the work difference between the gas turbine and compressor:

LGTP= lt- lc                                            (1.66)

Thermal efficiency of gas turbine cycle is the following:

t=1-1/(k-1)/k                                          (1.67)

The theoretical capacity of the gas turbine, compressor and plant are:

Nt= lt∙D3600= h3- h4∙D3600                               (1.68)

Nc= lc∙D3600= h2- h1∙D3600                               (1.69)

NGTP= lGTP∙D3600= h3- h4-h2- h1 ∙D3600              (1.70)

The actual cycle of gas turbine differs from the theoretical by existence of friction and vortex formation in the turbine and compressor. Effective methods of increasing the efficiency of gas turbines are: heat recovery, stage compression and expansion of the working fluid, etc.

Control questions and tasks for individual work

- Thermodynamic system. Thermodynamic processes.

- The first law of thermodynamics. Internal energy. Heat. Enthalpy. Word.

- The second law of thermodynamics.  Entropy.

- The concept of entropy and aspects of the disclosure of this concept.

- Circular thermodynamic processes and cycles. Heat balance and thermal efficiency cycles.

- Entropy method for studying the thermodynamic system

- That allows evaluating the thermodynamic analysis of the system when creating and improving chemical processes?

- The compressor compresses 100 m3 / h of air with a temperature of t1 = 27 0C from a pressure of p1 = 0.098 MPa to p2 = 0.8 MPa. Determine the power required to drive the ideal compressor. Compression is carried out according to isothermal, adiabatic and polytropic processes with a polytropic index of n = 1.2.

Section 2 General information about energy technology and energy technological systems

2.1 The concept of energy-chemical-technological systems (ECTS)

Energy technology is the section of energy, which studies the patterns of interconnection and interdependence of technological and energy processes of a given production in order to save fuel and energy resources and create virtually waste-free production of materials and heat. With the greatest economic effect, primary and secondary resources are used in such industries, in which the share of energy consumption in the cost of products is relatively large [11,15].

Real energy-consuming installations are far from ideal and consume a significant amount of external energy, and with a low efficiency.

Until now, the chemical industry enterprises are large consumers of primary energy resources (fuel, heat and electricity) received from outside. With the right development of the energy technology production scheme, it is possible not only to significantly reduce the consumption of primary energy resources, but even to completely abandon the consumption of heat and electricity from outside.

In the chemical industry, only 27 % of the fuel is used as raw material, and 73 % goes to energy needs. In the cost of production of certain types of products of the chemical industry, the share of energy costs accounts for from 10 to 60 % [11,15]. Such high energy costs are obtained due to improper organization of the process of energy conversion in power equipment, significant energy losses in the process, a small degree of use of secondary energy resources (SER) [11,15].

The energy efficiency of plants can be improved in two fundamentally different ways. The first way involves reducing the output of secondary energy resources by returning (regenerating) a part of their technological process (internal energy use). The second way is considered the most promising, efficient and involves the creation of energy technology units (ETU), in which energy equipment (steam generators, heat pumps, turbines, etc.) is directly connected to technological equipment, making up a single system [11].

The advantages of combined energy use over the separate production of technological and energy products are clearly identified by the exergy method of thermodynamic analysis, which allows you to determine the ultimate capabilities of the processes, sources of losses and ways to eliminate them, to increase the efficiency of the considered energy-technological units and their elements [11].	Energy-technological combination allows us to solve the problem of optimizing the technological process on the basis of rational energy consumption at all stages of the basic technology. This opens up broad prospects in creating advanced heat-engineering processes and, based on non-waste or low-waste technologies, allows for the full use of energy and material resources.

Energy technology is studying the most effective methods to reduce the cost of fuel and energy resources while increasing the technological parameters of energy technology units.

Using the heat of reaction is an important area of ​​chemical engineering. Currently, chemical reactors in most large-scale industries are combined with heat exchangers, which serve to heat the starting materials to the reaction temperature while cooling the conversion products or to produce water vapor in waste heat boilers due to the heat of exothermic processes. At the same time, heat exchangers form an energy-chemical unit together with the reactors. Accordingly, the transformation of chemical technology into energy technology [11].

The creation of non-waste industries solves the environmental problem in a comprehensive manner and reduces the cost of production due to the full use of all components of raw materials. One of the most rational ways of organizing production, which is approaching waste-free production, is the circulation of the reaction mixture and coolants (water, air) in individual processes and reactors, and in particular the creation of circulating chemical-technological systems (CTS) of the whole production. Non-waste technology can include technological schemes, where special devices are used for purification of exhaust gases and wastewater [11].

It is considered the most promising to create ECTS, in which power equipment (heat and steam generators, heat-using devices, refrigeration units, heat pumps and thermal transformers) is in direct connection with chemical process equipment, making up a single system. In such ECTS, any change in the parameters of chemical technology must be accompanied by corresponding changes in the energy parameters and vice versa. Thus, in ECTS, a close relationship and interdependence between technological and energy production stages is created.

When developing energy-technological schemes for chemical production, it is necessary, first of all, to provide for the maximum use of industrial waste for the generation of by-products and only if this is not possible, should the combustible industrial waste be incinerated in order to obtain heat. With this approach to the creation of ECTS, you can successfully solve the problem of creating waste-free technology, both in terms of energy and materials.

The main task in developing ECTS is to find the most effective methods for reducing the costs of fuel and energy resources while simultaneously increasing technological indicators.

It is impossible to create ECTS without its thermodynamic analysis. This analysis has two objectives, namely:

1) to get a picture of the energy transformations occurring in the system, which will allow to obtain reliable information about ECTS (values ​​of the system efficiency and its individual elements, distribution and nature of losses in the system, the relative weight of each element of the system, characteristics of the connections between the elements, interaction with medium, etc.);

2) to be able to carry out optimization of various parameters of elements of ECTS in order to obtain maximum thermodynamic and economic efficiency.

The basis of any ECTS is a thermodynamic cycle. Thus, in the production of ammonia, the most widely used energy-technology scheme with a steam-power cycle. A distinctive feature of this scheme is energy autonomy, which completely eliminates the consumption of water vapor from the side and minimizes the consumption of electricity. The power equipment built into the process (utilization boilers, steam and gas turbines, etc.) makes it possible to fully satisfy the need of chemical production, both in process and energy steam. In this case, water vapor allows replacing the electric drive of compressors and pumps with steam (driven from steam turbines). The scheme with a steam-power cycle has the advantage that a multi-stage scheme for the integrated use of the heat of exothermic technological processes and partial combustion of natural gas is used to produce water vapor. Here, natural gas simultaneously serves as feedstock and fuel, and from a thermodynamic point of view, the steam-power cycle approaches the generalized (regenerative) Carnot cycle, thereby increasing the relative thermal efficiency of the energy technology system. Great savings can be obtained when operating the ECTS of ammonia production in the steam-gas cycle. According to this scheme, the furnace of a tubular furnace operates under a pressure of 3.0 MPa. The resulting flue gases are used as a working fluid in a gas turbine. Thus, in this energy-technological scheme, a gas turbine is used along with a steam generator and a steam turbine. According to the calculations, a scheme with a steam-gas cycle is more economical than a scheme with a steam-power cycle by 5 ... 10 % [15].

Greater economic effect can be obtained if the energy technology scheme with a vapor-gas cycle is complemented by a gas (air) refrigeration unit cycle. According to this scheme, the combustion products of the fuel are converted into a refrigerant with a temperature of - (60 ... 80) 0C. In ECTS operating under this scheme, it is possible to carry out (by lowering the temperature of the combustion products to the condensation temperature of their component carbon dioxide), the energy technological use of fuel is not only for its intended purpose, but also for obtaining marketable products of solid carbon dioxide [15].

The main task in developing ECTS is to find the most efficient methods for reducing the costs of fuel and energy resources while simultaneously increasing technological indicators.

To develop ways of saving various energy resources of existing or new technological processes in the chemical industry, the following main steps are proposed:

1. The calculation of the material balance of the technological process and all stages of the process

2. Calculation of the energy (thermal) balance of the technological system and all stages of the process

3. The calculation of chemical and phase equilibria for all stages of the process

4. An exergy analysis of the process and all stages of the process. Determination of exergy efficiency, calculation of exergy losses in the technological process and at various stages of the process.

5. Carrying out the optimization of the technological process in terms of exergy analysis.

6. Development and proposal of technical solutions for the implementation of optimal process parameters in production.

When carrying out such actions, it is necessary to use appropriate computer programs for modeling and optimization of chemical-technological systems.

2.2 Thermodynamic analysis of chemical-technological systems

When analyzing chemical-technological processes from the point of view of energy efficiency, it is necessary to know the laws of thermodynamics. This allows you to conduct thermodynamic calculations, which make it possible to determine the boundaries of pressure and temperature in the process, to calculate the equilibrium composition of substances.

When conducting a thermodynamic analysis of chemical-technological systems, it is possible to determine the characteristics that affect the energy and economic efficiency of energy-chemical-technological systems in general and their elements [3].

In the chemical industry, the exergy method of thermodynamic analysis of energy-chemical-technological systems in general and their elements is of great importance [3].

This method is based on the use of the concept of “exergy” for the study of industrial processes. The term "exergy" was introduced by Z. Rant in 1956. The term consists of two parts: the Greek word "erg" - work, strength and prefixes "ex", meaning "from outside." [3]

Thermodynamic analysis of chemical-technological systems is very important in the design of technological processes. It allows you to make a conclusion about the direction of the process (right or left), pre-select the conditions of the process, determine the equilibrium composition of the reaction products, as well as calculate the thermal effects.

The values ​​characterizing the state of thermodynamic systems are called thermodynamic parameters. These include: temperature, pressure, specific volume, density, specific internal energy, etc.

Thermodynamic parameters are divided into extensive and intensive. Parameters that are proportional to the mass of the system are called extensive: this is volume, internal energy, enthalpy, entropy. Extensive parameters have the property of additivity (they can be folded).

Intensive parameters do not depend on the mass of the thermodynamic system: these are temperature, pressure, and also extensive values ​​related to the unit mass or volume of the system (specific heat capacity, density, etc.).

The change of intensive parameters in order to accelerate chemical-technological processes is called intensification.

The thermodynamic analysis of chemical-technological systems is based on the laws, concepts and laws governing the transfer of mass and energy.

The law of conservation of mass must be observed for the system as a whole and for individual components.

For a chemical system, the material balance of income and consumption can be written in the general form [16]:

Nin= Nout                                        (2.1)

The mass of incoming (Nin) substances includes the input raw materials containing the component to be processed, and auxiliary substances with which the process proceeds. The mass of outgoing substances (Noyt) is a mixture of the products of the main reaction, parallel and secondary interactions, as well as the non-recyclable part of the feedstock and auxiliary substances. The quantitative ratios between the components of the production mixture are determined by the excess for each starting reagent and the degree of conversion for each reaction, calculated from thermodynamic data, the loss of raw materials and the resulting substances [16].

The practical degree of conversion of the starting materials to the final ones is always less than 100 %. Thermodynamic calculations establish the theoretically possible yield of the product (Kteor). Commodity yield (Kpract) is less by the sum of chemical and mechanical losses (Kchem + Kmech) [16]:

Kpract= Kteor- Kchem+ Kmech                                     (2.2)

The coefficient of chemical losses calculated by thermodynamic data or set experimentally. These losses of the target component are due to the incomplete course of the main reaction, as well as the presence of side (parallel or secondary) reactions of the original or target, already formed substance. Mechanical losses are the losses of the target substance as a result of spilling, spilling, gas discharge, incomplete extraction of the target component during filtration and washing, settling, condensation, absorption, precipitation, degassing, etc. They are determined by the imperfection of technology and equipment, the ratio of technological parameters and environmental parameters, the stability of the technological process, the physicochemical properties of technological masses, and the insufficient organization of production [16].

Thermodynamic analysis of various transformations in a chemical-technological system is based on the laws of thermodynamics, analysis of the thermodynamic properties of substances and chemical systems.

In thermodynamic analysis, there is a concept of intensive and extensive state parameters. Intensive state parameters are temperature, pressure, chemical potentials of system components, etc. Intensive state parameters are independent of the mass of the system and have constant values in this phase. Extensive state parameters are volume, internal energy, entropy, enthalpy, etc. Extensive state parameters are equal to the sum of the corresponding parameters of all parts of the system, i.e. proportional to the mass of substances. In the case of the assignment of extensive parameters to a unit of mass and to one pray, they acquire the properties of intensive parameters.

According to the first law of thermodynamics (the law of conservation of energy), if a certain amount of heat Q is transferred from the environment to the chemical system, then part of it is spent on changing the internal energy dU, and the rest is spent on doing work L:

Q=dU+ L                                            (2.3)

According to the second law of thermodynamics, the heat exchange of a chemical system with the environment leads to a change in the entropy of the system:

dS= Q/T                                                (2.4)

In this case, all spontaneous irreversible processes of mass and energy transfer are accompanied by an increase in the entropy of the system, and in an isolated system in the equilibrium state, the entropy reaches its maximum value.

The concept of entropy can be revealed in the following three aspects.

1) Entropy is a measure of the loss of system performance due to the irreversibility of real processes. The loss of work from the irreversibility of the process is directly proportional to the increase in entropy.

2) Entropy is a measure of efficiency, technological efficiency (value) of heat.

3) Entropy is a measure of the disorder, degradation and disorganization of the system. If heat is removed from the system, then its entropy decreases.

Knowledge of the exact value of the absolute value of entropy is necessary for processes proceeding with a change in the mass of the initial substances and the formation of new ones from them (for example, during chemical reactions).

Chemical-technological processes proceed with an increase in the chemical potential of a substance or phase, if considered under equilibrium conditions.

In thermal equilibrium, there is no heat transfer, and the temperatures are the same. If the pressure is the same between the phases, then there is an equilibrium in the system.

In case of equality of chemical potentials of substances in all phases, chemical equilibrium is observed, characterized by the absence of directional transfer of matter and energy within the phases and between the phases [16].

Table 1 compares the differential forms of the characteristic functions, their change with one constant state parameter and the total integral forms.

When creating and improving chemical processes, thermodynamic analysis allows us to estimate:

1) The probability of the process in a given direction.

2) The depth of conversion of the starting materials.

3) The value of the thermal effect of the process.

Table 1 – Characteristic functions and their variables

| Characteristic function | State options | Differential shape | Accepted integral form |
| --- | --- | --- | --- |
| Internal energy U (isochoric-isentropic potential) | S, V | dV = TdS – PdV | At S = const US = –RT .ln(V2/V1) At V = const UV = T(S2 – S1) U = F + TS |
| Enthalpy H (isobaric-isentropic potential) | S, P | dH = TdS + VdP | At S = const HS = RT .ln(P2/P1) At P = const HP = T(S2 – S1) H = U + PV |
| Helmholtz energy F (isochoric-isothermal potential) | V, T | dF = –PdV – SdT | At V = const FV = Qобрln(T1/T2) At T = const FT = UT F = U – TS |
| Gibbs energy G (isobaric-isothermal potential) | P, T | dG = VdP – SdT | At P = const GP = Qобрln(T1/T2) At T = const GT = RT .ln(P2/P1) G = F + PV |

Based on these estimates, the technologist can make a conclusion about the degree of use of raw materials, the presence of waste (pollution), and energy dependence on external sources. The probability of the process is determined by the sign of change in the Gibbs energy G (this is a property of a substance that characterizes its ability to react with other substances; it depends on temperature, pressure and other parameters). If G  0, then the process is possible in the right direction, if G  0, then the process proceeds in the opposite direction, if G = 0, then an equilibrium state is established. A quantitative indicator of equilibrium is the equilibrium constant K, defined as the ratio of the rate of the direct reaction to the rate of the reverse at equilibrium.

Knowledge of the laws of thermodynamics is necessary for an engineer not only to carry out thermodynamic calculations, but also to evaluate the energy efficiency of chemical-technological processes. The value of the analysis of energy and economic efficiency lies in the fact that often at the earliest stages of researching new proposals, it is possible to foresee their economic feasibility or, conversely, their complete futility. Thermodynamic analysis makes it possible to establish this before drawing up a project. In recent years, the exergy method of thermodynamic analysis has been developed. This method is based on the use of the concept of exergy for the study of industrial processes.

2.3 The main provisions of the exergy method

An exergy method for analyzing chemical-technological systems is a method in which flows of substances (water, steam, chemical products) or flows of energy (for example, heat) are estimated from the useful work that they have done as much as possible with respect to the environment.  [3]

«The maximum ability of the system to perform work, taking into account interaction with the environment, the parameters of which do not depend on the impact of the system in question, is called technical performance, or exergy» [3].

The environment is a medium that is characterized by parameters such as temperature T0 = 293 K, pressure p0 = 0.1 MPa and chemical composition. Energy in the form of heat or work that has passed into the environment becomes the internal energy of the environment [3].

n this case, the parameters of the environment do not change.

The environment is a source of starting materials and a receiver of products that has minimal free energy and therefore cannot be a source of work [3].

A technological system that is in thermodynamic equilibrium with the environment, i.e. Since the state parameters of a substance or system are equal to the environmental parameters, the system cannot perform useful work, the practical value of the system or substance from the energy point of view is zero.

Therefore, the state of thermodynamic equilibrium of the chemical-technological system with the environment is equal to zero. The smaller the difference between the temperature of the heat source and the ambient temperature, the lower the energy utility of the system.

External sources of mass and energy (for example, chemical raw materials, fuel), the parameters of which differ significantly from the parameters of the medium, are considered separately from the medium [3].

2.4 Exergy, its types and components

Exergy is a property of a thermodynamic system or energy flow, characterized by the amount of work that can be obtained by an external energy receiver in a reversible transition from a given state to a state of equilibrium with the environment [11].

The system that is considered in exergy analysis can be either very simple (for example, a certain amount of a working fluid in a closed volume or flow), or very complex (a large energy or other aggregate, a complex of aggregates, an industry).

The environment is characterized in that its parameters are independent of the parameters of the system in question. A technological system can interact with the environment in an ideal process (reversible), and in a real process (irreversible). In a reversible (ideal) process, work is obtained that is equal to exergy. In a real process, the resulting work will be less than exergy. In this case, as a result of the irreversibility of the real process, part of the exergy disappears. This is one of the significant differences between exergy and energy [11].

Exergy of the substance in a closed volume. Consider a substance with parameters p, v, T, u, and s. By “substance” is meant not only an individual substance, but also a mixture of any components. The parameters characterizing the state of equilibrium with the environment are p0, v0, T0, u0 and s0.

The specific exergy of a substance in a closed volume is calculated by the formula:

exv=u- T0s+ p0v                                           (2.5)

The difference of exergies in two states of the system is determined by the formula:

exv=u- T0s+ p0v                                       (2.6)

Exergy of the substance in the stream. Consider the steady-state flow of a working fluid with parameters p, v, T, u, and s. The parameters characterizing the state of equilibrium with the environment are p0, v0, T0, u0 and s0.

The specific exergy of a substance in a stream is calculated by the formula:

ex=h- T0s                                                 (2.7)

The difference of exergies in two states of the system is determined by the formula:

ex=h- T0s                                            (2.8)

The exergy of the heat flux Q is determined by the equation:

exq=Q- T0s                                             (2.9)

Environmental condition for all occasions p0 = 0,1 МРа; T0 = 298,15 К.

The essence of the exergy method is that any flows of energy carriers (water, steam, chemical products) or energy (electricity, heat) are evaluated for the maximum useful work that they can bring. The maximum ability of the system to perform work, taking into account the interaction with the environment, whose parameters do not depend on the impact of the system in question, is called technical efficiency or exergy.

At the same time, the environment is understood as an environment of practically unlimited size, characterized by relatively constant parameters - temperature T0, pressure p0 and chemical composition. Energy released to the environment in the form of heat or work becomes the internal energy of the environment. The environment serves as an accumulator of energy that has large dimensions; the parameters of the state of the environment, despite the receipt of energy, do not change. Thus, the environment is a source of raw materials and a receiver of products, which has minimal free energy and therefore cannot be a source of work. A system that is in thermodynamic equilibrium with the environment loses its ability to do useful work. If the parameters of the state of a substance correspond to the parameters of the components of the environment, the practical energy suitability of the substance is zero. Therefore, the state of its thermodynamic equilibrium with the environment is taken as the zero level when calculating the practical energy suitability of this substance.  Heat has a lower practical suitability, the smaller the difference between the temperature of the heat source and the ambient temperature. External sources of mass and energy in the environment, whose parameters differ significantly from those of the environment, are considered separately from the environment.

Based on the second law of thermodynamics, we establish a quantitative relationship between the work that could be accomplished by the system under the given external conditions in the case of the occurrence of equilibrium processes in it, and the actual work performed under the same conditions under non-equilibrium processes.

Consider an isolated system consisting of a hot source with a temperature T1, a cold source (environment) with a temperature T0, and a working fluid performing a cycle.

The efficiency (or exergy) of heat Q1 taken from a hot source with a temperature T1 is the maximum useful work that can be obtained due to this heat, provided that the cold source is an environment with a temperature T0.

The maximum useful work Lmax of heat Q1 is the work of the Carnot equilibrium cycle, carried out in the temperature range T1 – T0:

Lmax= tQ1                                            (2.10)

where    t=1- T0/T1.

Thus, the exergy of heat Q1

Lmax= Q11- T0/T1                               (2.11)

Those efficiency of heat is greater, the smaller the ratio T0/T1.

When T1 = T0, it is zero.

Useful work obtained from the heat Q1 of a hot source can be represented as L1 = Q1  –  Q2, where Q2 is the heat given off in a cycle to a cold source (environment) with temperature T0.

If Scol denotes the entropy increment of a cold source, then Q2= T0Scol, then

L1= Q1- T0Scol                                   (2.12)

If in the isolated system under consideration only equilibrium processes proceeded, then the entropy of the system would remain unchanged, and the increase in entropy of the cold source Scol would be equal to the decrease in entropy of the hot source Shot. In this case, due to the heat of Q1, it would be possible to get the maximum useful work.

Lmax= Q1- T0Shot                                     (2.13)

which follows from equation (2.12).

The actual amount of work done under the same conditions, but with non-equilibrium processes, is determined by equation (2.12).

Thus, the loss of heat efficiency can be written as

L=Lmax- L= T0Scol- Shot

but the difference Scol- Shot is the change in the entropy of the isolated system considered, therefore

L= T0Ssys                                              (2.14)

The value of L determines the loss of work due to the dissipation of energy due to the disequilibrium of the processes occurring in the system. The greater the non-equilibrium of processes, the measure of which is the increase in the entropy of an isolated Ssys the less work is done by the system.

Equation (2.14) is called the equation of Huy-Stodola by the name of the French physicist who obtained this equation in 1889, and the Slovak heat engineer A. Stodola, who first applied this equation.

The decrease in the efficiency of an isolated system, in which irreversible processes take place, is equal to the product of the increment of the system's entropy by the absolute temperature of the heat sink. This loss of working capacity is heat that is useless to the environment. All irreversible processes in an isolated system are accompanied by the devaluation of energy, which is transferred from a more useful form to a less useful one. Energy dissipation and degradation occur. The entropy of the system increases.

Example. Determine the exergy of 1 kg of carbon dioxide at a pressure p = 1 MPa and a temperature T = 600 K with respect to the environment with parameters р0 = 0,1 MPa and Т0 = 293 K.

E=i -i0-T0s-s0=cpT-T0-T0cplnTT0-R∙lnpp0=           =0,97600-293-2,3 ∙∙2930,97∙lg6002   93-8,31444∙lg10,1 =211,8 kJ/kg

2.5 Exergy method of thermodynamic analysis

The exergy method of thermodynamic analysis of ECTS is based on the wide use of exergy.

The exergy of a substance is the maximum work that it can do in a reversible process with the environment as a source of free heat, if at the end of this process all forms of matter participating in it go into a state of thermodynamic equilibrium with all components of the environment.

All real processes are irreversible, and in each case, irreversibility is the reason for the decline in the perfection of the process. This is not due to the loss of energy, but due to a decrease in its quality, since in irreversible processes the energy does not disappear, but is devalued. Thus, every irreversible phenomenon is the cause of irretrievable loss of energy [3].

In general terms, the formula for determining the exergy of a substance flow is as follows [3]:

E= Ek+ Ep+ 0E+ Ech+ En+…                     (2.15)

where Ek and Ep are the kinetic and potential exergy, respectively; 0E is physical exergy; Ech is chemical exergy; Enis nuclear exergy.

The most important components of formula (2.15) for chemical technology processes are physical and chemical exergy. The sum of these components is called thermal exergy Et:

Et= 0E+ Ech                                        (2.16)

Physical exergy of a substance is determined by the formula:

0E= H- T0S                                          (2.17)

where H is the difference in enthalpies at the beginning and end of the process; S is the change in entropy during the transition from the starting materials to the reaction products at ambient temperature.

Chemical exergy is determined by the equation:

Ech= j=1njnj                                       (2.18)

When calculating exergy according to formula (2.17), it is necessary to have data on enthalpies and entropies. These data can be taken from directories or calculated by the formulas:

specific enthalpy

i1- i0= T0T1cpdT                                           (2.19)

specific entropy

s1- s0= T0T1cpdTT                                           (2.20)

where cp is the specific heat capacity.

If average heat capacities in the working temperature ranges are known, then formulas (2.19) and (2.20) turn into expressions, respectively:

i1- i0= ср (Т1- Т0)                                             (2.21)

s1- s0= ср  lnT1T0                                                  (2.22)

If substances during the process undergo a change in the state of aggregation, the formula for calculating the specific entropy takes the form:

s1- s0=T0T1cpdTT+ qjTj                                    (2.23)

where qj is the thermal effect of the phase transformation; Tj is the temperature at which the phase transformation occurs, accompanied by a change in enthalpy by qj.

Example. Determine the exergy of molten phosphate (m = 1000 kg) at a temperature of T1 = 1720 K. The melting point of phosphate is Tm = 1670 K, the heat of fusion q = 400 kJ/kg, the heat capacity of solid phosphate at a temperature of 293–1670 K cp = 1.2 kJ/(kg.K), the heat capacity of the melt at 1670–1720 K cp = 1.2 kJ/(kg.K) [3].

In accordance with formulas (14) – (18), phosphate exergy is determined by the formula:

E=me=mi1- i0- T0s1- s0=

=mcpTm- T0+q+ cpT1- Tm- T0cplnTплT0+ qTпл+cplnT1Tm

E=1001,21670- 293+400+ 1,41720- 1670 - 2931,2ln1670293+ 4001670+1,4ln17201670=10002122-293∙2,37== 1,43∙106 kJ

Here e is the specific exergy.

The maximum useful work is equal to the change in Gibbs energy if a chemical reaction proceeds at constant pressure p = const [3]:

Amax.w= G= H-T S                               (2.24)

The value of the maximum work, which is determined by the Gibbs energy (2.24), differs from the difference in exergies E (2.17The quantities G and E have the same values only when the temperature and pressure parameters approach the environmental parameters: at T = T0 and p = p0.

Gibbs energy G is equal to maximum work only when temperature and pressure have the same value both at the beginning and at the end of the process.

The exergy difference E is equal to the maximum work when the technological system interacts with the environment in a reversible process. Gibbs energy G takes into account only internal processes that occur in the technological system, while exergy E allows you to determine all the energy interactions outside the technological system [3].

In the exergy method of thermodynamic analysis of ECTS, each of its elements is considered as an independent thermodynamic system. The performance of each element of ECTS is estimated by comparing exergy at the entrance to this element with the loss of efficiency in it, i.e. loss of exergy as a result of irreversible processes occurring in this element, and due to both internal and external irreversibility. Thus, when determining the exergy losses in each element of the investigated ECTS, the causes of imperfection of the processes occurring in them are identified and quantified, which gives information about the possibility of improving perfection in all elements and allows you to create the most perfect ECTS. This is the main purpose of the exergy method of analyzing ECTS.

The thermal efficiency of the cycle t= lc/q1, defined as the ratio of useful work to all the heat supplied to the working fluid, does not give an idea of ​​the true cost-effectiveness of turning heat into work. This is explained by the fact that work can be completely transformed into heat, and the heat in a cycle, according to the second law of thermodynamics, can never fully go into work. Therefore, the thermal efficiency is actually a ratio of qualitatively different quantities and characterizes only their quantitative ratio [17].

It is known that the work will be maximal when the processes of change in the state of the working fluid are thermodynamically reversible, and at the end of the process the working fluid will come into balance with the environment, i.e. its parameters will take values ​​equal to the parameters of this medium p0 and T0. Naturally, after this, the working fluid must be removed from the installation, since further energy production will no longer be possible.

This maximum useful work performed by the unit of the working fluid during a reversible transition from a state characterized by the parameters p1, T1, s1, h1 to a state of equilibrium with the environment, characterized by the parameters p0, T0, s0, h0, (heat transfer occurs at T0) is called exergy, or technical performance unit of the working fluid, and is denoted by e.

Mathematically, the expression for exergy can be written from the equation of the first law of thermodynamics for a gas flow [17].

10dq=h0-h1+l0                                              (2.25)

and equations for entropy change in a reversible process

10dqT=s0-s1                                                     (2.26)

Multiplying equation (2.26) by T0 and subtracting the results from equation (2.25), transforming, we get:

10T-T0Tdq=h0-h1+l0-T0(s0-s1)                     (2.27)

In this equation, l0 is the work done by the unit of the working body as it passes through the system. As mentioned, this work reaches a maximum value of e if the process in the system proceeds thermodynamically reversible and reaches an equilibrium state with the environment.

Therefore, the maximum value of l0 can be represented by

l0max=e=h1- h0- T0(s1- s0)                       (2.28)

The resulting equation shows that exergy (as well as energy) is a function of the state, but its value, unlike energy, also depends on environmental parameters. The peculiarity of exergy is that it can be considered as a function of the state of a no equilibrium system from the environment and the source of work in the form of a stream. This is explained by the fact that in most thermal power and refrigeration units we deal with a continuous flow of working fluid (products of combustion of fuel, steam, air, refrigerants). In principle, one can consider exergy. Not only flow, but also closed volume.

Exergy is divided into two main types, namely: exergy of energy types, not characterized by entropy, for which it is equal to the energy itself e = W (mechanical, electrical, etc.), and exergy of energy types, characterized by entropy e  W, e  W (internal energy, radiation energy, thermomechanical, etc.). The exergy of the latter types of energy is subdivided into the exergy of a substance in a closed volume, the exergy of the flow of matter and the exergy of the flow of energy.

The concept of exergy is convenient to use to analyze the degree of thermodynamic perfection of a thermal unit.

Consider, for example, a turbine into which a working fluid flow enters with parameters p1 and T1, and comes out with parameters p2 and T2; inside the turbine, this stream did useful work работуLus. If the process inside the unit is irreversible, then it loses the efficiency of the ΔL flow, which will be equal to:

ΔL=(e1-e2)-Lus                                               (2.29)

where e1, e2 is exergy flow at the entrance to the unit and at the exit of the unit;

L is loss of performance due to friction and heat exchange with the environment.

In the absence of a loss of efficiency (ΔL = 0), the flow would perform the maximum useful work, i.e. Lusmax=e1-e2, which in accordance with equation (2.28) is equal to:

e1-e2=(h1-h2)+T0(s2-s1)                               (2.30)

But for a reversible adiabatic flow

Lusmax=e1-e2=h1-h2,

then the loss of working capacity due to the irreversibility of the process will be equal t

ΔL=T0(s2-s1)                                           (2.29a)

This is the main idea of ​​the exergy method of calculation: the working body enters the unit with exergy e1 and, having done useful work Lus, leaves the unit with exergy e2; at the same time, the loss of working capacity due to the irreversibility of the processes inside the aggregate is determined by equation (5). Thus, this method allows us to judge the degree of reversibility of the processes inside the unit by its external characteristic – the difference between exergues at the entrance to the unit and at the exit from it [17].

By analogy with the exergy of the flow of the working fluid, the concept of exergy of the heat flux eq is introduced. From the formula for thermal efficiency with a reversible Carnot cycle we have:

T1-T0T1=Lusmaxq

or

Lusmax=eq=q(1-T0T1)                                    (2.31)

where T1, T0 is the temperature of the substance or system and the environment.

Equation (2.31) shows that the working capacity of heat, called exergy of heat eq, is greater, the greater the T1 and the smaller T0.

2.6 Exergy balance, exergy efficiency

To assess the economic efficiency of technological processes using the equations of material and energy (thermal) balances.

Based on the calculation of the energy balance, the productivity of the apparatus, the coolant flow rate and the efficiency, that is, quantitative energy data, are determined. Qualitative data on energy resources are not taken into account. Therefore, material and energy (thermal) balances in the calculation of chemical-technological systems are supplemented with an exergy balance [3].

The exergy balance equation in general has the following form:

E=E +D                                        (2.32)

where E  and E   are the incoming and outgoing exergy flows, respectively; D is loss of exergy.

The terms “energy loss” and “exergy loss” have a fundamentally different meaning. As you know, energy cannot disappear, and energy losses mean losses for a given system or losses for a given purpose. Losses of exergy mean its complete disappearance, destruction associated with the dissipation of energy. Hence the designation of exergy losses through D (dissipation) [3].

A comparison of energy and exergy balances in the form of flow diagrams clearly illustrates the difference between energy and exergy (see Figure 17).

For example, two energy flows into the system: W1  and W2 and three come out: W1, W2 and W3, and W3     represents losses, if W2 is heat flow [3].

Due to the law of conservation of energy

W1 + W2 = W1 + W2 + W3                                      (2.33)

or in general case                         Wi= Wj

The value of energy efficiency is determined by the formula:

= Wn /W                                                  (2.34)

where Wn is amount of useful energy; W is total energy cost.

In Figure 17b, the incoming exergy flows Е1 and Е2 and the outgoing flows Е1, Е2  and Е3 correspond to energy efficiency. The flows Е1 and W1 are shown in the diagram (Figure 18) with the same thickness (Е1= W1).  This corresponds to the case if W1 represents electricity. Electricity refers to unlimitedly convertible forms of energy, i.e. fully functional.

The flow Е2 is shown to be smaller than W2 (by W2 heat flow is meant). The value of Е will always be less than Е by the amount of losses Di from irreversibility inside the system (Di = Е  – Е ), which does not reflect the energy balance. There are external exergy losses De, which are associated with the exergy of unused exiting flows Е2.

Exergy losses caused by irreversibility are determined according to the Guy – Stodola formula:

W1, W2 and Е1, Е2– at the entrance to the system; W1 , W2 , W3 and Е1 , Е2, Е3 – at the exit from the system; De and Di   – external and internal exergy losses

Figure 17.  Diagram of energy flows W (a) and exergy E (b)

Di= T0S                                              (2.35)

where S is the entropy increment of an isolated system.

Figure 18 shows the curves of changes in the state of two streams in thermal contact with each other. Area F1256 corresponds to the exergy of the flow, which is its source and changes its state along the curve 1– 2 E1- E2. Area F3475 corresponds, in turn, to the exergy of the heated stream, which changes its state according to curve 3 – 4 E4- E3 [3].

The difference between the arrival and consumption of exergy is the loss of exergy due to irreversible heat transfer:

Di= E1- E2- E4- E3                                (2.36)

These losses can be calculated without determining the exergy of the flows, knowing only the entropy of the coolant:

Di= T0S= T0S4- S3-S1- S2                                  (2.37)

Figure 18. Diagram of exergetic losses caused by irreversible heat transfer

Thus, the hatched area in Figure 18 represents the loss of exergy.

For exergy, the general conservation law is inapplicable: the sum of exergy of all elements of the system during the process decreases. If there is no loss of exergy, for example, in a reversible process, then the amount of exergy does not change from the beginning of the process to its end. In irreversible processes, exergy is converted into energy.

Formula (2.38) is called exergy efficiency, which is always less than one [3].

e= EnE= E- Di +DeE     1                                      (2.38)

Exergy efficiency shows the degree to which the system is close to ideal. For an ideal process where there is no exergy loss, e = 1, or 100 %. If the failed exergy is completely lost in the process, then e = 0.

All real processes are irreversible. Exergy losses are irreversible energy losses. Exergy losses increase and decrease the work that is done by the chemical-technological system as a result of real processes. And this entropy increases. Exergy losses are the most important task of thermodynamic analysis of chemical-technological systems [3].

For a quantitative assessment of the degree of thermodynamic perfection of the unit, exergy efficiency is used.

ηex=Lusein  -  еout                                          (2.39)

When reversible processes in the aggregates Lus=Lusmax  and ηэкс=1. For thermal units that do not produce useful work, ηex=еinеout and is a measure of the irreversibility of the processes occurring inside the unit. In expressions for ex, еin means the sum of exergies of the flow of working fluid and the flow of heat.

It is easy to establish that, for example, for a turbine, ex = 0, and for a thermal power installation as a whole, the exergy efficiency is the same as the effective relative efficiency of the installation. For a boiler room installation and its furnace device, the exergy efficiency is significantly lower than the thermal efficiency (for a boiler unit 35–40 against 85–90 %, and for furnace 65–70 against 96–98 %), i.e. according to the exergy balance, boilers and fireboxes are imperfect devices.

When calculating exergy efficiency, the following actions can be performed:

- compare processes that are designed for one purpose; choose the most efficient process from an exergy point of view;

- determine whether to improve the process;

- determine what methods can be applied to improve various process indicators.

The exergy method of analysis of chemical-technological systems reveals the main exergy losses, how to reduce these losses, to find in which direction to move. To create more efficient technologies, that is, to determine the degree of thermodynamic perfection of a given chemical-technological process  [3].

The literature provides the results of an exergy analysis of many industries: methanol, nitric acid, ammonia. So, for example, in an enthalpy analysis of an ammonia synthesis unit in an ammonia synthesis column, a water heater, and heat exchange equipment, the energy loss is zero. In an exergy analysis, the exergy losses are highest in the synthesis column (22.6% of all losses), i.e. higher than in the turbine (20%) and compressor (16%). Large losses of exergy are present in the synthesis column, the water heater. Half of all exergy losses are in heat exchange equipment. Exergy analysis shows that exergy losses can be reduced by changing the technological mode of ammonia synthesis. If you apply the temperature increase in the column of ammonia synthesis, this allowed us to obtain steam with a higher temperature and pressure. This makes it possible to more efficiently use the heat of reaction. [3].

Example. Carry out an exergy analysis of the ammonia synthesis unit at a pressure of 30 MPa [3, 16].

Table 1 –  Unit operating mode

| № | Indicators | Unit of measurement | Value |
| --- | --- | --- | --- |
| 1 | Ammonia performance | t / day | 1360 |
| 2 | as consumption: - common, through the synthesis column, - fresh nitrogen-hydrogen mixture, | m3/h m3 / 1t NH3 | 820000 2700 |
| 3 | The mass of the gas mixture at the inlet to the column | kg / 1 t NH3 | 4070 |
| 4 | Volume fraction of NH3 in gas and gas temperature: - at the entrance to the synthesis column, - after the catalytic stage, - after the water cooler, - after the evaporative condenser | %, К | 3,4 and 308 15,7 and 783 9,45 and 298 – 308 7,1 and 240 |

When compiling the exergy balance, the environment at T0 = 298 K and p0 = 0.1 MPa was taken as a reference point.

The total exergy balance of the NH3 synthesis unit is determined by the formula:

Еnhm+ Еsh.p.+ Еsl.p.+ Еwin+ Еe= ЕNH3+ Еsm.p.+ Еp.g+ Еwout+ Еcond+ D

where Enhm is the exergy of the initial nitrogen-hydrogen mixture; Еsh.p. and Еsl.p.  is steam exergy, respectively, of high and low pressure at the inlet; Еwin and Еwout is water exergy respectively at the inlet and outlet of the water heater; Еe is exergy of the supplied electric power; ENH3 is ammonia exergy; Еsm.p. is exergy of medium pressure steam at the outlet; Еp.g is exergy of purge and tank gases; Еcond is exergy of condensate exiting the condensation stage of the turbine; D are losses from irreversibility of energy transformations.

The results of calculating the exergy balance are given below.

Table2  –  Exergy Flow E, 10-6 kJ / h

| Input flow | Output flow |
| --- | --- |
| Еnhm = 1533,2 | ЕNH3 = 1272,9 |
| Еsh.p. = 462,2 | Еsm.p. = 276,2 |
| Еsl.p. = 41,9 | Еp.g = 154,9 |
| Еwin = 11,4 | Еwout = 69,5 |
| Еe = 13,7 | Еcond = 0,9 |
| Total Е = 2062,4 | Total Е = 1774,4 |

Exergy loss  D = 2062,4 . 106 – 1774,4 . 106 = 288. 106 kJ/h.

Exergy coefficient of performance is determined by the formula:

e=  ЕЕ = 1774,4∙1062062,4∙106=0,86

The performed exergy analysis indicates that there are more resources for intensifying the synthesis of ammonia than this follows from the enthalpy analysis [3,16].

2.7 Types of exergy losses

Exergy losses in technological systems are divided into internal losses and external losses. Internal losses Di exist inside the technological system, which are caused by hydraulic resistance, friction in plants, heat and mass transfer, etc. [11, 15].

External losses De. associated with the conditions of interaction of the technological system with the environment. External losses in the technological system are caused by the temperature difference between the heated and cooled devices and the temperature of the working fluid, thermal insulation, etc. External losses include combustion products, cooling water, heat losses with end products [11].

Total exergy losses are obtained by summing the internal and external losses:

D= Di+ De= Ex- Ex                              (2.40)

where Ex, Ex is the sum of the exergy flows entering and leaving the system, respectively.

Exergy losses are divided into internal and external losses because they have different ways to reduce them. Internal losses Di are associated with the imperfection of the equipment that is part of the technological system, as well as the processes in its elements. External losses De are determined by the fact that there is a mismatch between the process and external conditions  [11].

The sum of the internal losses Difor all elements of n gives the total internal loss in the system, the sum of the external losses Deof the elements m gives the total external loss. Finally, the equation of exergy losses for a stationary process has the form:

1nDi,i + 1mDe,j= Ex- Ex                             (2.41)

Thus, a complete quantitative characterization of losses in the installation and its individual parts can be obtained, associated with both internal and external factors.

Exergy losses can be calculated using the following formula:

D= T0S                                            (2.42)

The exergy losses in the heat exchanger Dhe generally represent the sum of four losses caused by the final temperature difference Dt, hydraulic resistance Dh, heat exchange with the environment Den and thermal conductivity along the heat exchanger [15].

Dhe= Dt+ Dh+ Den                                (2.43)

Loss of exergy in the heat exchanger from the final temperature difference can be determined by the formula:

Dt= EqB- EqA= mBeqB- mAeqA= qe=Qe             (2.44)

where EqB and EqA are exergy of heat fluxes of heat carriers B and A, respectively; mB and mAare the mass of coolants B and A, respectively; eqB and eqA are the specific exergues of heat fluxes of heat carriers B and A, respectively.

For heat exchangers in which heat exchange occurs at constant temperatures of TV and SL, for example, in condenser boilers, the losses Dm can be calculated by the formula:

Dt= EqB- EqA= QeB- QeA=QeB- eA=Q1-T0TB-                       - 1-T0TA=QT01TA-1/TB                                               (2.45)

From the formulas it follows that the smaller the temperature difference in the heat exchanger, the less the loss from the final temperature difference occurs, the less the loss of exergy.

Loss of exergy from hydraulic resistance Dh arise due to the movement of coolants in the apparatus. If there was no friction, then there was no hydraulic resistance Dh = 0. Losses of exergy from hydraulic resistances are equal to the operation of pressure devices (compressors and pumps) and are determined by the formula:

Dh= LA+ LB                                   (2.46)

where LAand LB are the technical work of the supercharger A and the heat carrier B, respectively.

Losses of exergy from heat exchange with the environment Doc depend on the imperfection of the thermal insulation of the equipment and are determined by the formula:

Den= mAeqA- mBeqB= mAqAeA- mBqBeB=mAqA1-                                 - T0/TinA+ mBqB1- T0/TinB                                    (2.46)

where qA and qB are the densities of heat fluxes from heat carriers A and B through thermal insulation to the environment or vice versa; qB and TinB are the average integral temperatures of external thermal insulation of heat carriers A and B, respectively.

The loss of exergy in the reactor from the flow of a chemical reaction in its Dc.r. can be calculated from the formula:

L= T2sнеобр

Consider an exothermic reaction proceeding at constant temperature and pressure [15]. The heat of reaction is transferred to anybody that is at the same temperature. Then the entropy change in the chemical reactor will be:

sc.r.= s+ sT                                   (2.47)

where s and sT  is the change in the entropy of the system in which the reaction takes place and of the other body, respectively.

If work due to a change in the volume of a system at constant pressure is accomplished only as a result of a reaction, then the heat transferred to another body will be equal to the decrease in the enthalpy of this system, i.e. (– Н) and therefore the above equation can be written as:

sc.r.=- s+HT=-Ts+ HT= Z/T                  (2.48)

and irreversibility losses, the so-called exergy losses:

Dc.r.= T0sc.r.= T0Z/T                                 (2.49)

Consequently, the entropy created in a chemical reaction that occurs at constant temperature and pressure is equal to the change in the isobaric-isothermal potential divided by the temperature at which the reaction occurs, and the exergy loss of this process to the product of entropy and the ambient temperature.

Exergy losses in heat engines, compressors and pumps are directly from the exergy balance made up for this machine.

Example. In the regenerative air heater of the gas turbine, air is heated from Tair = 425 K, and the exhaust gases are cooled from Tgas = 650 K to Tgas = 500 K. Assuming that the gas has air properties and the ambient temperature T = 290 K, and neglecting the losses, determine: a decrease in the exergy of gases; increase in air exergy; exergy efficiency air heater. The specific heat of air is cp = 1,005 kJ / (kg.K).

The air temperature is determined from the heat balance equation: Тa-Тa = Тg-Тg; Тa = 650 – 500 + 425 = 575 К.

Reduction of the exergy of gases:

-∆Eg=E1-E2=i1-i2-T0s1-s2= cpТg-  Тg- T0CplnТg Тg== 1,005650-500- 2,3∙293∙1,005∙lg650500=74,3 kJ/kg

Increase of air exergy:

∆Ea=i2-i1-T0s2-s1= cpТa- Тa- T0CplnТaТa== 1,005575-425- 2,3∙293∙1,005∙lg575425=63,0 kJ/kg

Loss of working efficiency of thermodynamic systems:

D = Eg-Ea = 74,3 – 63,0 = 11,3 kJ/kg.

Exergy efficiency of air blower:

э=1-DEg=1-11,374,3=0,848.

2.8 Exergy performance and power

The fact that exergy makes it possible to estimate the fraction of flows of all types of energy that are included in the energy balance of any ECTS allows us to obtain its generalized characteristics. Such generalized characteristics are exergy performance and power ECTS [15].

The algebraic sum of all types of exergy Eeffwhich determines the effect given by ECT, is called the exergy performance of ECT. The value of Eeffper time называют is called the exergy power of the ECTS:

Nin=Eeff= Eeff                                  (2.50)

Consider specific expressions of exergy power for power plants encountered in chemical technology [15].

As is known, heat-electric generating plant is designed for the simultaneous generation of electricity and heat. Consequently, the exergy power of the heat-electric generating plant is determined by the formula:

Nin= Nel+ mnieqni- mkieqki                        (2.51)

where Nel is the electrical power of the heat-electric generating plant net;

mniand eqni is the amount of steam bent on heat supply and its exergy, respectively;

mki  and eqkiare the amount of condensate returned to the heat-electric generating plant and its exergy, respectively.

The compressor is designed to increase the pressure of the gas (steam), and, consequently, its performance should be evaluated by increasing the exergy of газа gas (steam) in the compression process. Therefore, the exergy power of the compressor:

Nin= miei                                             (2.52)

if the heat of compression is useful is not used and

Nin= miei+ mbeb                                         (2.53)

if this heat is used for technological purposes.

In these formulas: mi is the amount of compressible gas (vapor); eb  is increase in exergy of the cooling medium (steam, air, gas) in refrigerators; mb  is the flow rate of the cooling medium [15].

The exergy power of the combined unit for obtaining heat and cold, consisting of a heat pump and a refrigeration unit, will obviously be equal to:

Nin= Eqi+ Eqxi                                  (2.54)

where Eqi and Eqxi is the sum of the exergy capacities of hot and cold streams.

Since heat and cold flows are usually transmitted to the consumer through heat carriers and refrigerants flowing through heat exchangers at constant pressure, then Eqi and Eqxican be expressed through changes in the exergy of coolant and coolant flows, i.e.

Nin= miei+ mxiexi

where ei and mi are the change in the exergy of the coolant flow and its flow, respectively; exi and mxi are change in exergy of refrigerant flow and its consumption, respectively.

The exergy power makes it possible to determine the exergy efficiency. ECTS and is used as one of the criteria for optimizing ECTS [15].

2.9 The relationship between energy and exergy characteristics of thermodynamic systems

Thermal power installation can be characterized by the following efficiency:

energy t=L/Q1  and exergy ex=EeffEus= Aeff/Aus [15]. The beneficial effect of a thermal power plant is expressed by electrical or mechanical work L and, therefore, Aeff= L. The costs are determined by the exergy of heat:

Aus= Eus= Eq= Q1e                                 (2.55)

Consequently,          ex=L/Q1e                                                                    (2.56)

Given the formula (2.56), you can write that the energy efficiency of thermal power plant

t=LQ1= exQ1eQ1= exe                                 (2.57)

The refrigeration unit can be characterized by a coefficient of cooling коэффициентом  = Qx/L  and exergy efficiency ex= Aeff/Aus. In a refrigeration unit, the value Aeff   is an exergy of refrigeration flows, i.e. Аeff = Eqx, and the cost Aus = L,  therefore,

ex=EqxL=Qxex/L                                 (2.58)

In the formula (2.58)      Qx/L =   and, therefore,

= ex/ex                                           (2.59)

If the refrigeration unit is operated due to the heat flow Q1, then Aus = Eqx =  Qxex  and, therefore,

ex=AeffAus= Qxex/Q1e                           (2.60)

In a heat pump installation, the Aeff value is an exergy of the heat flux Q2 withdrawn from the installation to the consumer, i.e. Aeff = Q2e2.

Since the exergy of heat taken from the environment (at a temperature T0) is zero, then Aus = L, and, consequently, exergy efficiency of heat pump installation:

ex=AeffAus= Q2e2/L                                             (2.61)

If the heat pump installation works by heat flow Q1, then Aus = Eq1 =  Q1e1  and, therefore,

ex= Q2e2/Q1e1                                             (2.62)

Heat pump installation is characterized by thermal coefficient = Q2/L                                             and exergy efficiency ex=AeffAus.

In the formula  (2.61) = Q2/L   therefore, ex= e2   or

= ex/e2                                         (2.63)

When heat driven thermal coefficient of heat pump installation  = Q2/Q1. In this case, the formula takes the form [15]:

ex= e2/e1                                         (2.64)

or             = exe1/e2

Example. In a boiler plant at an ambient temperature of t0 = 25 0С, steam with a temperature of t2 = 550 0С is generated. The temperature of the combustion products in the furnace is t1 = 1727 0С, (calorific value of fuel QpH = 42,000 kJ / kg). Find the exergy of the heat of the flue gases, the steam produced and the exergy efficiency.

The exergy of flue gases will be:

e1=Q1-T0T1=420001-2982000=35742 kJ/kg

The exergy of the resulting steam:

e2=Q1-T0T2=420001-298823=26792 kJ/kg

Loss of exergy per 1 kg of consumed fuel will be:

е = е1 – е2 = 8950 kJ/kg

Loss of exergy by the Gyu-Stodoly equation:

∆e= T0∆S= T0QT2-QT1= 8950 kJ/kg

Exergy efficiency of boiler plant without heat loss will be:

е = е2/е1= 26792/35742 = 0,75

Taking into account the effective efficiency of boiler installation t = 0.9 for the exergy of the heat produced by the steam, we will have:

е2= t∙е2=0,9∙26792=24113 kJ/kg

Thus, exergy efficiency taking into account the heat losses will be:

е  = 24113/42000 = 0,574

2.10 The relationship of the overall performance of ECTS with the

characteristics of its individual items. Features of exergy loss in ECTS

This relationship is essential in the thermodynamic analysis of ECTS. If Хе is an exergy characteristic related to the efficiency of the ECTS (D, ηtx, Е) as a whole, and хеi is the same characteristic of the i-th element of this system, then the value [15]:

z1= Xe/xeiy=idem                                        (2.65)

will show the effect of changes in the characteristic in the i-th element on the characteristic of the ECTS as a whole, where y is the parameters of other elements ECTS; у = idem does not mean that they will be unchanged.

It only means that under the action of a change in xei, all other quantities not associated with xeiwill remain unchanged. The greater the value of z1, the greater the influence of the change in the characteristic of the i-th element on the performance of the ECTS as a whole. Therefore, when optimizing and improving this ECTS, the main attention should be paid to its elements with the largest value z1. Thus, the coefficient xe characterizes the internal connections of the system and is determined by its structure. So, if ECTS is a sequentially located elements, processes in which flow without exergy supply and removal from the outside ηexECTS  and ηexii  without its circulation between elements, then the connection between and will be the simplest [15].

Since in this case the exergy at the exit of the i-th element Еi is always equal to the exergy at the entrance to the (1 + i)-th element, the exergy efficiency is ECTS will be equal to the product exergy efficiency of all its elements, i.e.

ηexECTS = i=1i=nηexi                                          (2.66)

Using this system as an example, it is possible to visually establish the peculiarities of exergy losses in various elements of ECTS. Let in the system consisting of three elements (Figure 19), D1 = D2 = D3 = D. Since in the direction from the first element to the third the exergy at the entrance to the element Еi will continuously decrease, in this case it will also continuously decrease and ηelemi, because ηelemi = (Еi – D)/Еi. Therefore, the reduction of losses is especially important at the final stages of the technological process. This circumstance should be taken into account when improving the ECPS and its optimization [15].

Figure 19. Towards a demonstration of loss features in an ECTS

2.11 Exergy efficiency of ECTS and its elements.

Grassmann-Shargut diagram

Exergy efficiency of ECTS, machines and devices are reduced to a single formula in which the numerator is a useful exergy effect, and the denominator is the cost of exergy [15].

In the general case, for ECTS as a whole and for its individual elements, the exergy efficiency is can be calculated by the formula:

ηex=E1-DiE1i=Eeff,iEus,i=АeffАus                                  (2.67)

The Grassmann – Shargut diagram of fluxes and exergy losses provides substantial assistance in exergy analysis of ECTS. In this diagram, each exergy stream of the analyzed ECTS is represented by a strip whose width is proportional to the value of the exergy [15].

Consider the schematic diagram of the compression heat pump installation (Figure 20a). Compressor II compresses the low-boiling heat carrier vapor, after which it enters condenser III. Here, coolant vapors are cooled and condensed at high pressure, with the release of heat Q, which is then used for heating.  From apparatus III, condensate enters choke IV, where as a result of choking, its temperature decreases. Then the cooled condensate enters the evaporator V, where due to the heat Q  Q, supplied from the environment, it completely evaporates. The coolant vapors formed in the evaporator enter the compressor II, which completes the heat pump installation cycle [15].

Figure 20b shows the Grassman-Schargut diagram of the considered heat pump compressor installation. Figure 20b shows the Grassman-Schargut diagram of the considered heat pump compressor installation. Figure 20b shows all exergy losses in the plant elements, the magnitude of which is represented by the corresponding bandwidth and is conventionally represented by a shaded triangle that goes into a curved arrow Di (exergy loss in the i-th element of the installation) [15].

An exergy EI, equal to the electric power of the electric motor I, is supplied to the installation, since the exergy of electrical energy is not characterized by entropy. In the electric motor there is a loss of exergy Dm, equal to the sum of the loss of electrical energy in the car and drive. Consequently, the exergy at the exit of the motor is EI= EI- Dm

The exergy at the entrance to the compressor is  EII= EI + EV

where ЕV  is the exergy of coolant vapor exiting the evaporator V.

The total exergy of EII is equal to the exergy of compressed coolant vapors.

Exergy at the outlet of the compressor is determined by the formula:

EII= EII- Dc

where Dc is exergy losses in the compressor, and Dc  Dm.

The exergy at the input to the capacitor is   EIII= EII

a)                                                                b)

Figure 20. Schematic diagram (a) and the Grassman-Shargut diagram (b)

for compression heat pump installation

In a condenser, the loss of exergy De is associated with heat transfer at a finite temperature difference between the coolant and the external heat receiver.

The exergy at the output of the capacitor is   EIII= EIII- De

Exergy at the inlet to the throttle IV is   EIII- Eq= EIV

When throttling the coolant there is a loss of exergy from the irreversibility of the Dthr process.

Exergy at the outlet of the throttle is   EIV= EIV- Dthr

Throttle evaporator inlet exergy is   EV= EIV + Eq

where Eqis exergy is the exergy of the heat flux supplied to the evaporator from the environment;

Eq= Q1- T0/T1  0

as        Т1  Т0.

The exergy at the outlet of the evaporator is EIV= EV

Figure 21 shows a diagram of energy conversion in the process of fuel combustion and the use of combustion products in the form of a diagram of exergy flows [15]. Typically, the total exergy at the inlet E1,  is equal to the sum of the exergy of the fuel ET,  and the oxidizer EOX. In those cases when the fuel and the oxidizing agent are heated in the heater I before combustion due to a part of the exergy of the combustion products E5, their exergy increases to the value E1  Е1. The heating process in the heater is accompanied by loss of exergy D1. Further, the heated fuel and oxidizer with the exergy E2 = E1 enter the combustion chamber II, where the process of converting the exergy of the fuel E2 and the oxidizer into high temperature combustion products is carried out. The process of fuel combustion in the combustion chamber II is accompanied by a loss of exergy D2.

Figure 21. The Grassman-Shargut diagram for the combustion process

The products of combustion with exergy E3 = E2  enter the III element of the ECTS, which is either a steam generator, or a heat generator, or a gas turbine. The process of using exergy of combustion products in element III is accompanied by exergy losses D3, the nature of which depends on the type of element III. Residual exergy E4 can partially be used to heat the fuel and oxidizer (E5) in the heater or in other heat-using plants. The exergy of the combustion products E6  = D4  emitted into the atmosphere (in the absence of a heater). Another part of the exergy of E7 after element III is supplied for further use.

After calculating the exergy of the fuel and oxidizer e1, as well as the combustion products ер.с., we find the exergy losses d1 associated with the transition of the chemical energy of the fuel into the exergy of combustion products:

d1= e1- ec.p.

Exergy of combustion products is determined by the formula:

ec.p.= cpc.p. Tteor- T0- T0cpc.p.lnTteorT0- Rc.p.lnpc.p.p0- Riln1ri= Hteor- T0cpc.p.lnTteorT0- Rc.p.lnpc.p.p0- Riln1ri

where Hteor is theoretical enthalpy of combustion products, kJ/ kg;

Tteor is theoretical combustion temperature, К;

cpc.p. is average heat capacity of combustion products, kJ / (kg. K);

Rc.p. is gas constant of combustion products, kJ / (kg. K);

Ri и ri are gas constant and volume fractions of the components of the products of combustion, respectively.

Then the exergy efficiency of the combustion process is determined by the formula:

ex=ec.p.e1=(e1- d1)/e1

The exergy efficiency of the combustion process varies from 0.45 (for industrial boiler rooms and furnaces) to 0.7 (for modern steam generators).

2.12 Analysis of individual processes of chemical technology

In the analysis of technological processes of chemical technology, it is necessary to approach comprehensively, taking into account the problems of resource conservation. When conducting research, you must adhere to the following rules:

1. At all points of the technological equipment at any time, the driving force of the technological process should be equal to zero.

2. To save energy resources and capital costs, the driving force must be uniform.

3. If you do not know how to save energy, then slow down the process, and then gradually remove the braking.

The conversion of energy and substances that occur in technological systems consist of repetitive processes, such as: compression and expansion of gases and liquids; heat transfer from one body to another due to heat transfer; mixing and separation of substances; chemical reactions.

The methodology for considering the processes of all these groups is general and corresponds to the main provisions of the exergy method. Consider some of the processes characteristic of a gas turbine installation (GTI) [11].

Compression of gases and liquids. The process of compressing gases and liquids to increase their pressure or to carry out movement through pipelines is one of the most important technical processes. In industrialized countries, the total drive power of various pumps and compressors reaches 25 % of the installed capacity of power plants [11].

Almost all circular processes (ICE cycles, gas turbine engines, etc.) are associated with compression processes. Therefore, the energy perfection of the compression process is of substantial practical importance.

Energy transformations during the compression process are shown schematically in Figure 22.

Figure 22. Diagram of the energy (a) and exergy (b) balances of the compression process

A flow of the working fluid (point 1) under pressure р1 enters through the control surface of the compression device (for example, a compressor) and leaves it with pressure р2  р1 (point 2). The pressure increase is provided by the approach of work L. If the compression is not adiabatic, then it is accompanied by the removal of heat Q, characterized by the corresponding exergy Exq.

For all compression processes, the equations of energy and exergy balances are valid:

LK= H2- H1+Q 	                               (2.68)

LK= Ex2- Ex1+Exq+ D                          (2.69)

The work LK, spent in the compressor or pump, is spent on increasing the exergy of the working fluid Ex= Ex2- Ex1, on the heat exhaled from the gas (for example, the exergy of cooling water) Exq and partially lost on losses D.

The thermodynamic perfection of the compression process is evaluated by the exergy efficiency of the compression device:

Ex= Ex2- Ex1+ExqLK                                    (2.70)

In many cases, exergy Exq (cooling water) is not used, but discharged into the environment. Then this exergy can be considered an external loss, and the exergy efficiency is determined by the equation:

Ex= Ex2- Ex1LK                                       (2.71)

Exergy flow diagrams for compression processes are shown in Figure 23 (the electric drive and, accordingly, the losses in it are located outside the surface and therefore are not shown).

In the case of using the heat flux diverted from the compressor, for example, for heating water, the loss D  decreases slightly (diagram a), the exergy efficiency of the compressor increases [11].

- b)

Figure 23. Diagram of exergy flows for the compression process using (a) and without using (b) exergy of the heat removed

Expansion of gases and liquids. Expansion in high-temperature processes (for example, in a turbine) is intended to obtain mechanical (electrical) operation of LT due to a decrease in the enthalpy of the working fluid (H1 – H2) [11].

The energy and exergy balances of expansion processes in general are shown in Figure 24. The corresponding balances are:

H1- H1= LT+Q                                         (2.72)

а)                                             b)

Figure 24. Schemes of energy (a) and exergy (b) balances of expansion

processes (turbines)

where Q may have different signs depending on whether heat is supplied or removed.

Ex1- Ex2= LT+Exq+ D                             (2.73)

Figure 25. Exergy flow diagram for expansion process

The exergy efficiency of the turbine, depending on whether or not the exergy of the heat of the working fluid leaving the turbine Exq is used, will be calculated by the formulas:

Ex= LTEx1- Ex2-Exq                                       (2.74)

Ex= LTEx1- Ex2                                            (2.75)

Figure 25 shows an exergy flow diagram for expansion processes.

Heat exchange. Heat transfer processes are widely used in modern technology. Almost all fuel exergy is realized through heat exchange. Heat transfer can be divided into two groups [11]:

- 1) Heat transfer, which occurs due to an increase in the exergy of one working fluid (or system) with a decrease in the exergy of another working fluid (or system). In this case, the heat transfer losses are influenced by hydraulic resistance in the equipment, as well as the final temperature difference of the working fluid (or system). In this regard, losses are own and technical.

- Heat transfer due to heat loss to the environment due to the thermal conductivity of the equipment material, or thermal insulation.

The task of thermodynamic analysis is to reduce heat loss and increase the exergy efficiency of heat transfer in a process or system.

Other types of losses during heat transfer (from hydraulic resistance, longitudinal thermal conductivity and heat transfer through insulation) relate only to technical losses.

An exergy analysis of the heat transfer process allows you to choose thermodynamically more favorable temperature differences of the coolant, based on the allowable losses during heat transfer [11].

The exergy balance of the heat exchanger can be written in the form of the equation:

Ex1= Ex2+ D                                        (2.76)

where Ex1= Ex1- Ex1 is change (decrease) in the exergy of the hot coolant; Ex2= Ex2- Ex2  is  change (increase) in the exergy of the coolant; D is the sum of the exergy losses in the heat exchanger.

The exergy efficiency of the heat exchanger is calculated by the formula:

Ex= Ex2Ex1= Ex2- Ex2Ex1- Ex1=Ex1+Ex2Ex1+ Ex2                             (2.77)

A diagram of the exergy fluxes of the heat transfer process is shown in Figure 26.

Exergy losses in the process of heat transfer are determined by the formula:

D= T0S2- S1= T0S                               (2.78)

Figure 26. Diagram of exergy flows of the heat transfer process

Burning fuel. Fuel combustion is a group of chemical reactions. In most technical devices, fuel combustion reactions proceed under isobaric conditions [11].

Exergy flows through the adiabatic combustion chamber are shown in Figure 27. Assume that the supplied fuel energy consists entirely of exergy ExT.

By its nature, this energy can be converted into any other type of energy, for example, into mechanical work. Due to the irreversibility of the transfer of part of the exergy to anergy, the combustion products leaving the chamber contain less exergy ExПС  less than that (ExT+ ExB ) from the outside. The exergy balance of the combustion process has the form:

ExT+ ExB = ExПС+ D 	                              (2.79)

Figure 27. Diagram of exergy flows of the fuel combustion process

The exergy efficiency of the fuel combustion process is determined from the formula:

Ex= ExCPExТ- ExВ                                          (2.80)

It is possible to increase the exergy efficiency of the combustion process only by increasing the combustion temperature.

Specific ways to improve energy and exergy indicators for different industries and processes are different [10]. The most common and effective way is energy recovery. The essence of regeneration is the transfer of energy from the flows exiting the unit to the incoming. For example, many large heating and smelting furnaces are equipped with heat exchangers in which air blast (and sometimes gaseous fuel) is heated by exhaust gases (Figure 28).

At the same time, regeneration allows not only to utilize the heat of the exhaust gases, but also reduces fuel consumption and improves the operation of the furnace itself. Since the combustion temperature rises when using heated air. The conditions and completeness of fuel combustion are improved, and heat transfer by radiation between the gas flow and the heated products is enhanced.

1 – the working volume of the furnace; 2 – heated parts;

3 – air heater; 4 – burner

Figure 28. Diagram of a heating furnace with heat recovery

at walking gases

Sometimes it is advisable to use heat recovery on low-temperature flows. For example, the heat of ventilation emissions can warm the flow of air supplied to the room, while reducing the energy consumption for heating.

2.13 Exergy analysis of the compressor

As an object for disclosing the essence of the thermodynamic optimization of the system and its individual elements, we consider a compressor installation, since it is widely used in chemical technology. From the exergy capacity of the Nex compressor unit, it is possible to obtain generalized characteristics of this unit, which allow analyzing and optimizing the energy indicators of existing compressors, as well as compare them.

The main exergy characteristic of the compressor is the power consumption Nx per unit of exergy power Nex, i.e.

m= Nx/Nex                                          (2.81)

This value characterizes the operation of the compressor, taking into account the amount of compressed gas and its parameters.

Analysis of the compressors based on the indicator m shows that:

– for most compressors, m = 1.5 ... .2 or = 0.65 ... 0.5, since ηex= 1/m.

Centrifugal machines have the best energy performance, and vertical V-shaped and horizontal machines have the worst. The indicator m can serve as an estimate of the energy efficiency of the machine for the preliminary calculation of the energy consumption for its drive.

Indicator m allows you to give an objective assessment of the energy indicators of compliance of the compressor achieved world level;

– piston machines without cylinder lubrication can have energy indicators no worse than machines with lubrication;

– the indicator m allows you to compare the performance of this compressor in different modes and thereby identify the optimal conditions for its operation.

Determine the exergy of the working fluid at the entrance to the compressor by the formula:

E1=H1-H0-T0S1-S0

or

Е1=G⋅сp⋅T1-T0-T0⋅G⋅сp⋅lnT1T0-R⋅lnP1P0,  kW,

where G is the mass flow rate of the working fluid, kg / s;

ср is the heat capacity of the working fluid at P = const, kJ /(kg .K);

Р0, Т0 is pressure and ambient temperature;

Р1, Т1 is pressure and temperature of the working fluid.

Determine the exergy of the working fluid at the outlet of the compressor according to the formula:

Е2=G⋅сp⋅T2-T0-T0⋅Gсp⋅lnT2T0-R⋅lnP2P0

Determine the effective drive power of the compressor by the formula:

Nе=NТη0i                                                   (2.82)

where  is the relative internal efficiency of the compressor, η0i=0.8÷0.82.

Determine the power of electricity supplied to the compressor motor by the formula:

Nel=Nеηem                                            (2.83)

where ηem is the electromechanical efficiency of the compressor, ηem=0,85÷0,95.

Determine the loss of exergy in the motor by the formula:

Del=Nel⋅1-ηem=Еel⋅1-ηem

Determine the loss of exergy in the compressor by the formula:

Dc=Eel⋅ηel+Е1-Е2

The loss of exergy in the compressor mainly consists of two components: the exergy removed by the cooling agent and the exergy lost due to the irreversibility of the processes of compression, friction and heat transfer.

The exergy removed by the cooling agent is determined by the formula:

DQ=Q⋅τе

where Q is the amount of heat removed from the compressor;

е is coefficient of performance of the heat removed, determined by the formula:

e=1- T0T2

Exergy lost due to irreversible processes of compression, friction and heat transfer is determined by the equation:

Dirr=DC-DQ

Determine the exergy efficiency of the compressor by the formula:

ηl=ЕinЕout=Е2Еel⋅ηel+Е1

The diagram of the exergy balance of the compressor is shown in Figure 29.

Figure 29. Exergy balance of the compressor

2.14 Exergy analysis of chemical-technological systems

When carrying out the process in optimal conditions, heat is supplied or removed excess heat. To do this, reduce the use of energy or use the heat of chemical reactions. To solve this problem, an analysis of the energy and exergy balances of the chemical-technological system is carried out [16].

The term “exergy” represents “technically suitable energy”, “operability”, “a measure of the suitability of any type of energy for energy in a reversible process of interaction with the environment” [16].

Exergy E characterizes the quality of energy. Unlike energy, exergy depends not only on system parameters, but also on environmental parameters. A system can have high energy, but its exergy can be small or equal to zero.

If the energy of a body or system has the same temperature as the environment, then this energy cannot be used.

If the pressures in the system and in the environment are equal, then the exergy is zero. Similarly, this applies to all other forms of energy [16].

An exergy complex is subjected to exergy analysis, including a system in which energy transformations occur, the environment as a whole and the source or receiver of energy in it. Conducting exergy analysis from thermodynamic positions allows us to establish the degree of perfection of the beneficial use of energy and the source of undesirable losses.

Transformations of such types of energy as electric, magnetic or mechanical into its other forms are not characterized by a change in entropy [16].

The conversion coefficient  is equal to the ratio of the sum of the allocated energy flows W to the sum of the summed energy flows Wand is determined by the formula:

= W/W                                      (2.84)

W= W+ W

The transformation of various types of energy into other forms does not take place completely and corresponds to a change in the entropy S.

In this case, in the formula (2.84) WSshould be subtracted from the sum of the withdrawn flows:

S= W- WS/W                               (2.85)

W= W+ W+ WS

The table 3 compares the properties of energy and exergy [16].

Table 3 – The properties of energy and exergy

| Energy of system W | Exergy of system E |
| --- | --- |
| 1. Energy depends only on system parameters and does not depend on environmental parameters. | 1. Exergy depends on the parameters of the system and the environmental parameters |
| 2. Energy always has a value other than zero, and is equal to mc2 in accordance with the Einstein equation | 2. Exergy may have a value of zero (in complete equilibrium with the environment) |
| 3. Energy is subject to the law of conservation in any processes and cannot be destroyed | 3. Exergy obeys the conservation law only in reversible processes; in real irreversible processes it is partially or completely destroyed (Guy's Law-Stodola) |
| 4. The transition of some forms of energy to others, including for reversible processes, is limited by the conditions of the second law of thermodynamics | 4. The transition of some forms of exergy to others for reversible processes is not limited by the conditions of the second law of thermodynamics |

The difference between the properties of energy and exergy is shown here. If the process is ideal, then in this case W is equal to the exergy of E. If there is a change in the state of the system, then a change in the work W is equal to a decrease in the exergy of the system Е.

In the real process, W  Е, which indicates the loss of part of the exergy, since energy scattering (dissipation D) arises in irreversible processes. It follows that the degree of thermodynamic perfection of the process is higher, the smaller the loss D.

The exergy balance and exergy coefficient of performance Е are represented by the following equations:

E  E+ E                                              (2.86)

0Ech= E- E/E                                           (2.85)

D=E -E+ E  0                                  (2.86)

For chemical-technological systems, exergy is a thermal (thermomechanical) ЕТ part of total exergy, which is determined by the formula:

ET= 0Eph+ 0Ech                                       (2.87)

where 0Eph and 0Ech are the physical and chemical components of total exergy, respectively; 0 indicates that environmental parameters are taken as a reference.

In exergy transformations in the case of constant chemical transformations (0Ex = 0), the difference in exergy at the output E2 and input E1 is determined by the formula:

ET= EF= E2- E1= I2- I1- Q0                            (2.88)

If the output parameter values ​​coincide with the environmental parameter values: I2= I0 and S2- S0.

Therefore, when counting from the state parameters in the environment (T0, P0, I0) to the input parameters, the change in physical exergy will be:

0Eph= 0I- T0S= I1- I0- T0S1- S0                       (2.89)

If the pressure in the system differs from the ambient pressure, then the physical exergy is determined by the formula:

0Eph= RT0∙lnP/P0+ I1- I0- T0S1- S0                       (2.90)

According to the Guy – Stodola law (the law of exergy reduction), the exergy losses for an installation are defined as the sum of the exergy losses Ei, calculated separately for all stages of the process:

Ei= T0Si                                                (2.91)

where Siis the algebraic sum of the entropy increments of all components of the chemical-technological process.

The exergy efficiency of the process as a whole for all stages is determined by dividing the sum of all exergy flows at the input E2 by the sum of all incoming exergy flows E1 according to the formula:

E= E2/E1                                                    (2.92)

If we compare the exergy efficiency of the same technological processes, then for a process with lower exergy efficiency it is necessary to find ways to increase the degree of thermodynamic perfection. This can be done by making better use of the heat of substances.

The exergy efficiency of a technological process or system can be defined as the ratio of the sum of the exergy increments of all heated working bodies E to the corresponding sum of the exergy increments of all heated working bodies E:

E= E/E                                                   (2.93)

Here, the exergy loss is Ej = E– E,  and the efficiency value is will be:

E= 1-Ej/E                                              (2.94)

If there are m nodes (stages) in the chemical-technological scheme in the value of total efficiency take into account the proportion of each node (stage) Ym, calculated through the ratio of exergy consumption (Е)m                                                                                  at this stage to the sum of exergy expenses at all stages of the scheme:

Ym= Em/mEm                                          (2.95)

It is obvious that mYm = 1.

Sometimes, the fraction of the increment (return) of thermal energy for each stage 0Qm is estimated with respect to the total increment (decrease) of heat 0Qm:

Ym= 0Qm/m0Qm                                      (2.96)

Respectively  mYQ,m = 1.

After some transformations of the formula, given the average temperature of the node or stage Tm, we obtain the formula:

E=1-T0m(1TmYQ,m)/[1- T0m(1TmYQ,m)]                         (2.97)

For one stage with one heated and one cooled heat carriers:

E=1-T0/T/T0/T                               (2.98)

The average temperatures entering into formula (2.97) with an admissible error can be calculated from the equations:

Tt= ∆0Qm/∆0Sm                                            (2.99)

Тev= m(∆0Qm)/m(∆0Sm)                             (2.100)

Тm= T+ T/2      -     for liquids and suspensions                            (2.101)

Тm= Тv           -       for vapor, liquids and solids                                        (2.102)

In formulas (2.97) – (2.102) m(∆0Qm)   and m(∆0Sm)  are increments of heat and entropy throughout the technological scheme; ∆0Qmand ∆0Smare the increments of heat and entropy at the Tav and Tt  are average temperatures for the scheme as a whole and for one stage; Tand Tare the coolant inlet and outlet temperatures either for the scheme as a whole or for the stage; Tv is the vapor saturation temperature of a single-component liquid, a solid, and also a volatile component of a complex solution (at constant total pressure).

The average temperatures for the scheme as a whole are calculated using the relations:

- the output of the heated coolant outlet heat carrier

1/Tev = m(δ/Tm)                                         (2.103)

- output of the cooled coolant heat carrier

1/Tev = m(δ/Tm)                                         (2.104)

where m= Qm/Qm  is the fraction of heat fluxes at each stage.

2.15 Analysis and thermodynamic optimization of ECTS

An energy-technological system in the chemical industry is a chemical-technological system, which includes an energy unit. In this case, fuel consumption occurs, the use of heat-chemical reactions, energy production to maintain the technological regime in the system [15].

The amount of heat that must be supplied at different stages of the chemical process is determined by its mode. Energy costs can be reduced by recovering energy between stages of the process and using the potential of the flows in the process itself.

The energy required for conducting of the chemical-technological process (CTP) can be generated by consuming fuel or using the heat of exothermic reactions.

In the general case, the combination of energy and technology makes it possible to more fully utilize the energy of chemical reactions and energy resources, improve the quality of the product, and improve the performance of energy technology units.

The use of energy technology schemes can reduce the consumption of electricity, steam, fuel. It also becomes possible not to use external energy, to use the additional amount of energy that the technological system itself produces within production.

For example, the introduction of energy technology schemes in the production of urea leads to a 40% reduction in steam consumption. In the production of diluted nitric acid, energy consumption is reduced several times and steam is generated and transferred to other consumers.

Exergy analysis of technical systems represents a further step compared to the analysis of individual processes. Thermodynamic analysis of technical systems can be carried out at three levels [15].

At the first level, the internal losses Di from irreversibility in the system and its elements, the external losses De, as well as the causes that cause these losses are determined.

At the second level, the values of exergy flows of all types are determined.

In this case, the exergy efficiency of the technological system and its elements can be determined.

The third level of thermodynamic analysis of systems makes it possible to find factors whose variation allows one to reach the thermodynamic optimization of the system.

The most efficient energy technology systems in the chemical industry are large-scale production systems for ammonia, weak nitric acid and urea. As a result of the use of these systems in the production of ammonia, the specific energy consumption decreased by almost 8 times (from 6840 to 900 MJ / kg). In urea production plants, the consumption of steam received from outside was reduced by 40%, specific capital investments decreased by 35–40%, and production costs decreased by 10%. In the production of weak nitric acid, electricity consumption was reduced several times, and more than 5 GJ of heat energy is generated, which can be transferred to other consumers [15].

1 – loading pipe; 2 – a melting bath; 3 – bottom burner device; 4 – device for granulation and cooling of the melt; 5 – separation section; 6 – recuperator; 7 – steam generator; 8 – basin sump; 9 – cooling tower of the system of granulation and cooling of the melt; 10 – converter cooling tower; 11 – pumps; 12 – pressure tank emergency cooling system

Figure 30. General scheme of an energy-technological melting converter plant for producing defluorinated phosphates

Figures 30 and 31 show the diagrams of a cyclone energy technology plant for producing defluorinated fused phosphates and energy vapor [15]. Figure 32 shows the flow diagram of the exergy balance of this plant. Unit productivity: for the main product (defluorinated phosphate with a temperature of 1723 K) – 10 t / h; for an auxiliary product (superheated water vapor with a pressure of 10.6 MPa and a temperature of 783 K) – 15 t / h; feed water temperature 488 K, fuel – natural gas Qlw = 35.8 MJ / m3; air heating temperature 623 K; flue gas temperature 323 K.

The calculation of the exergy balance was carried out without taking into account the zero exergy of the combustion products, as a result of which the deviations of the obtained results from the exact ones are  3 %. As can be seen from Figure 32, for the entire installation ex = 16,6 + 30,6 = 47,2 %.

With the separate production of phosphate and a pair of the same parameters, the overall efficiency of the installation will be approximately 36 %. The decrease in efficiency is explained by the fact that during the joint development of these products, the main exergy losses (from irreversibility during heat transfer) are significantly reduced. Since at the beginning of the technological chain, where the temperature is maximum, a high-temperature technological process is placed - the production of phosphates in a cyclone.

I – cyclone, II – radiation chamber, III – superheater, IV – economizer, V – air heater

Figure 31. Scheme ECTS obtain fluorine-free phosphate

According to the energy balance, the efficiency of the installation is t = 27,4 + 63,5 = 90,9 %, where 27.4% is the enthalpy removal with phosphate and 63.5% with steam.

Figure 32. Flow diagram of the exergy balance in the ECTS of obtaining fluorine-free phosphate

According to the energy balance, a picture of a highly efficient installation is obtained, but this is not true. A comparison of the results obtained from the exergy and energy balances shows that the energy balance does not reveal the essence of energy transformations in ECTS and does not provide a correct assessment of both the installation itself and its elements [15].

Consider the possibilities of energy technology in the chemical industry using the example of ammonia production. Powerful turbocompressors requiring a high-speed drive are used to compress and circulate the nitrogen-hydrogen mixture at the synthesis stage. A steam turbine is used as such a drive. Steam of high parameters is usually obtained at the cogeneration plant. To exclude dependence on cogeneration plants, use an energy technology system.

The flue gases after the methane conversion tube furnace have a temperature of more than 950 ° C. Such flue gases can be used to produce high-quality steam, but they are not enough to drive a steam turbine. The lack of energy is made up for by burning additional fuel in the flue gas duct after the tube furnace, i.e., by installing an energy unit as an element of the energy technological scheme.

In the installation of the heat of the process gas, flue gases and an additional burner as an energy unit, it is enough to abandon the energy consumption (a pair of high parameters) from the outside. In addition, the tube converter is equipped with a convection chamber, passing through which flue gases preheat natural gas, air, and steam. Next, the flue gases are sent to a recovery boiler to produce steam. Thus, the production of ammonia when creating an energy technological scheme becomes autonomous in energy.

Substantial advances in chemical energy technology have been achieved in the bound nitrogen industry. A joint solution of the problems of chemical technology and the use of energy resources made it possible to create rationally working nodes for the absorption of nitrogen oxides by water (to produce nitric acid) and the catalytic reduction of nitrogen oxides contained in the exhaust gases [15].

Increasing the degree of extraction of nitrogen oxides in the absorber leads to an increase in the dimensions and weight of the apparatus, hydraulic resistance of the column, to an increase in the energy consumption supplied to the compressors.	A joint analysis of the conditions of the chemical-technological process and its energy characteristics allowed us to find the conditions for the rational organization of this chemical-technological unit.

Another example of an energy technological scheme is a contact sulfuric acid production scheme.

In the production of sulfuric acid, the total amount of energy released in the form of heat is from 5000 to 8000 MJ per 1 ton of acid. If you use only 5% of the power of the output heat flux, then the energy costs for the production of sulfuric acid are fully compensated. The remaining energy is used to produce high potential steam.

1, 10 − economizers; 2 − gas turbine; 3 − compressor; 4 – installation for burning sulfur;

5 − waste heat boiler; 6 − gas filter; 7, 9 − contact devices; 8 − heat exchanger;

11, 12 − absorbers; 13 − drying tower; 14 − air filter

Figure 33. Energy technology scheme for the production of sulfuric acid

The rest of the energy should be used to produce high-grade steam. Figure 33 shows a diagram of the energy technology system for the production of sulfuric acid under pressure.

The cleaned and dried air in the apparatus 14 and 13 is compressed to a pressure of 0.5 MPa in the compressor 3 and enters the installation for the combustion of sulfur 4. Sulfur dioxide containing 11.5−12.5 % ​​of sulfur dioxide with a temperature of about 1470 K, is fed to the waste heat boiler 5, generating steam with a pressure of 4 MPa and a temperature of 710 K. Steam (750–1000 kg per 1 ton of acid) is used to generate electricity in cogeneration plants and can provide steam turbines to drive the compressor and feed pumps [15]..

The gas cleaned in the filter 6 enters the first stage of contact, successively in contact devices 7. The degree of oxidation of SO2 to SO3 after the first stage is 94–95 %. After intermediate absorption carried out in the absorber 11, the gas mixture is heated in heat exchangers and fed to the second stage of contact in the reactor 9 and then through the economizer 10 enters the absorber 12. The overall oxidation state of SO2 to SO3 reaches 99.85 %. The system is completely independent of external power source. The gas turbine 2 is driven by overpressure and heat from the absorber 12 gases, which provide 60–70 % of the required power. The rest of the energy comes in the form of electrical, for the generation of which is applied superheated steam. Receiving steam is carried out due to utilization of heat released during the combustion of sulfur. The feed water for the recovery boiler is heated in economizers 1 and 10, using the thermal energy of the gas mixture leaving the second stage of contact and from the gas turbine. With fully autonomous system operation, an additional 700 kg of steam is produced per 1 ton of sulfuric acid [15]..

2.16 Secondary energy resources (SER) of the chemical industry

The main direction in increasing the energy efficiency of chemical industries is to reduce their energy intensity through the use of internal resources of each chemical technological system (CTS). This is facilitated by the creation of large-capacity units, the transfer of production to continuous technology, the use of the fuel potential of combustible waste, and the rational organization of energy-technological units. So, when creating large-capacity units, specific heat losses are sharply reduced: for continuous production, the heat generation of stabilized parameters and a constant amount of energy released per unit time are typical. Waste incineration should be organized so that it does not require additional fuel consumption, but is itself a source of thermal energy.

The rational organization of energy technology units in the chemical production system requires studying the relationship of chemical processes and the processes of energy release or absorption accompanying them. Based on the analysis of the data obtained, it is possible to build a rational chemical-technological process with optimal conditions for the use of energy resources of the chemical-technological process.

In a real industrial facility, an additional fuel resource may be the waste of certain types of packaging that cannot be reused, as well as combustible filter materials and various combustible materials in the form of waste from industrial activity of an industrial facility.

The tasks of energy technology are directly related to issues of hardware and technology design of chemical-technological schemes of large-capacity industries.

Secondary energy resources (SER) are waste, by-products and intermediate products, high temperature exhaust gases, which are formed during the technological process but are not used. Such resources can be used in the future to supply energy to other equipment. [3].

So, for example, in the production of ammonia, the secondary energy resources are liquid hydrocarbons, purge gases, physical heat of the flue gases of furnaces and heaters, physical heat of synthesis gas, etc. [3].

The physical heat of the flue gas is used to heat natural gas, the air that enters the conversion; also used in a recovery boiler for overheating water vapor.

Secondary energy resources must be used rationally, since they help to increase the saving of fuel and energy resources in obtaining industrial products.

Fuel that is used to produce electric and thermal energy is called energy. The fuel that is used in industry in various plants in the process is called process [3].

2.17 Use of secondary energy resources

By type of energy, secondary energy resources (SER) are divided into three groups [3]:

- combustible (fuel) secondary energy resources - this is the chemical energy of the waste that is obtained in the process by processing raw materials, exhaust gases from furnaces. Such waste is not used in the future for processing.

- thermal secondary energy resources - this is the physical heat of the exhaust gases of technological equipment, waste from the main production, hot water and steam that have been spent in thermal installations;

- secondary energy resources of overpressure are the energy of gases and liquids that came out of technological equipment with overpressure.

Depending on the types and parameters of the working fluid, there are four main areas of use of secondary energy resources [3]:

- fuel – the direct use of combustible components as fuel. Some combustible waste from chemical industries can simultaneously be both energy and material resources.

- thermal - the use of heat, for example, in utilization plants (in waste heat boilers, heat exchangers), as well as in absorption refrigeration units;

- power – the use of mechanical and electrical energy generated in utilization plants (stations) due to secondary energy resources;

- combined – the use of heat, electrical or mechanical energy, simultaneously generated from secondary energy resources.

A large potential for saving primary energy resources lies in the efficient use of secondary energy resources (SER): physical heat of furnace and process gases, waste fluids, heat of combustion of chemical production wastes, energy of overpressure of products and raw materials of chemical industries. In all chemical-technological systems (CTS), minimization of the use of primary energy resources and, conversely, to the maximum use of SER should occur without any reduction in the quality of the products obtained.

Energy conservation in chemical technology is an improvement in technology with a more complete use of primary energy resources, as well as recycling of secondary energy resources.

All major industries (chemical and oil refining industries, metallurgy, production of building materials) have secondary energy resources.

Utilization of thermal secondary energy resources is most used. Thermal secondary energy resources include the heat of waste and process gases, as well as products of chemical reactions.

For utilization of the heat of the exhaust, flue gases of high-temperature processes (coking, steel production, etc.), regenerators are used − chambers filled with a nozzle of refractory bricks; the nozzle is periodically heated by passing hot gases through it, and then, due to the heat of the nozzle, the cold source reagents, air or gas, are heated. Several regenerators working in parallel ensure continuity of the process.

Improving technological processes in chemical production involves the search for more rational types of raw materials and methods for its preparation, as well as the integrated use of raw materials. It is necessary to organize energy-technological installations and systems, the use of energy-saving equipment.

For example, using natural gas instead of brown coal in the production of synthetic ammonia as a raw material saves energy. In modern installations that use natural gas as a raw material, the energy expenditure coefficient is 60 - 100 kWh / t N2. [3].

The use of more active catalysts allows the process to be carried out at lower temperatures and pressures, that is, at lower energy costs. Including reduces operating costs.

In the production of aromatic hydrocarbons, the use of new, more active and resistant catalysts lowered the energy consumption per ton of product by 0.26 tons of reference fuel. In catalytic reforming, an increase in catalyst activity of 1 % leads to a reduction in energy costs by 2.9 %. [3].

Energy technological systems are being developed and implemented based on the integrated use of fuel, in particular, on the basis of deep technological processing of low-grade coals.

Very promising is the possibility of using thermal pumps. The principle of operation and the device of heat pumps are similar to refrigerators, but they are designed to generate heat. Heat pump stations that already exist in a number of countries take away the warmth of sea and river (as well as sewage) water and heat objects that require a moderate temperature not higher than 60–80 0C. Heat pumps do not pollute the environment and are economical because they use small amounts of electricity [3].

In SER utilization installations, water vapor, hot water, electric power, high temperature coolants (high removal fluid, brine, etc.), chilled water, hot air, mechanical energy for direct driving of machines are produced. Depending on the role of SER in the main technological process in which they are formed, the installations can be energy-technological and utilization. The power technological installations include installations, without which the main technological process cannot proceed or the regime undergoes significant changes when they fail. These include forced cooling systems of technological units, whose cooling coolant, such as, for example, high removal fluid, is used in other processes, gas utilization turbines, and also waste heat boilers for cooling production flows.

Utilization installations include installations, without which the main technological process can proceed [3].

2.18 Utilization plants

To improve technological processes in industry, it is necessary to find reserves of secondary energy resources and use them both in production and for domestic purposes. This improves the efficiency and effectiveness of the industry. The use of secondary energy resources depends on the amount of secondary energy that is generated as a result of the technological process, the constancy of its production, as well as the temperature of this secondary energy  [3].

Secondary energy resources can be used directly as fuel, as well as converted by recycling plants into other energy carriers. Equipment for the use of thermal secondary energy resources, as well as secondary energy resources of overpressure, are waste heat boilers, dry coke extinguishing plants, gas utilization non-compressor turbines, and absorption refrigeration machines.

In waste heat boilers, steam is produced due to the heat of the flue gases of industrial furnaces. In water economizers, the boiler feed water is heated; in air heaters, air is heated, also due to the heat of the exhaust flue gases. The heat of the secondary energy resources is used in absorption and steam ejector refrigerators, drying plants, etc. [3].

Waste heat boilers provide greater fuel savings through the use of energy or process steam of secondary energy resources, as well as water heating due to the use of secondary heat.

For example, when firing sulfur-containing raw materials and processing sulfur dioxide into sulfuric acid, a large amount of heat is released 7.1 million kJ when 1 ton of H2SO4 is obtained from pyrites, 5.77 million kJ from sulfur and 8.03 million kJ from hydrogen sulfide) [3].

The gas temperature at the outlet of the combustion furnaces reaches 1073–1273 K. In the utilization unit used in the production of sulfuric acid (Fig. 34), the main technological apparatus is a fluidized bed furnace 2, into which pyrites come from hopper 1The waste heat boiler has a drum separator 5, steam-water emulsion from cooling devices 6, 8 is sent to it. Then the steam is fed to the superheater 3, where it is heated to a temperature of 430-450 0С and then sent to the consumer. Hot water that enters the drum separator 5 is pre-cleaned, heated and deaerated. In such recovery boilers, through the use of the heat of combustion of sulfur pyrite in fluidized bed furnaces, up to 1.5 tons of steam are obtained by burning 1 ton of pyrite.

1 – hopper feeder; 2 – fluidized bed furnace; 3 – superheater;

4 – waste heat boiler; 5 – drum separator; 6, 8 – cooling elements; 7 – circulation pump

Figure 34. Schematic of the recovery boiler

Consider a recovery boiler (Fig. 35). The recovery boiler is used to produce nitric acid. The superheater of the recovery boiler is used at a temperature of nitrous gases of 880–1000 0С.The evaporative part of the boiler is a tubular heat exchanger, through tubes 4 of which nitrous gases occur. Water is in the annulus. The gas pressure is 0.68 MPa, water – 1.27 MPa. Wet gas is collected in the steam collector 6 and through the tubes 7 enters the superheater 8, from where it leaves at a temperature of 230 0С [3].

When using the heat of the reaction of ammonia synthesis in waste heat boilers, 0.8–0.9 tons of steam per 1 ton of ammonia can be obtained (Fig. 36).

1 – input camera; 2 – manholes; 3 – output camera; 4 – evaporation tubes;

5 – safety valve; 6 – steam collector; 7 – tubes of saturated steam; 8 – superheater

Figure 35. Heat recovery boiler

1 – synthesis column; 2 – waste heat boilers; 3–circulation pumps; 4 – remote heat exchanger

Figure 36. Double-circuit (a) and single-circuit (b, c) schemes for utilization of the heat of reaction of ammonia synthesis

The transfer of synthesis columns to work using the heat of reaction significantly improves the technical and economic indicators of synthesis units [3].

Contact economizers are used for heating feed water in boilers, heating and air conditioning, producing hot water for process and domestic water supply. [3].

Excessive exhaust gas pressure is used when generating electricity in gas recovery turbines [3].

A diagram of an energy-technology installation in which through the use of secondary energy resources it is possible to obtain electricity, steam and compressed air is shown in Figure 37.

1 – absorption column; 2 – heater; 3 – contact device; 4 – waste heat boiler;

5 – steam turbine; 6 – air compressor; 7 – gas turbine

Figure 37.  Diagram of installation of a catalytic gas purification

from nitrogen oxides

The gases leaving the absorption column 1, into the heater 2. There they are heated to a temperature of 110-145 0C. Then they are mixed with a reducing gas (natural gas) and then enter the contact apparatus 3. Gas cleaning occurs at a temperature of 760 0C. Then the purified gases are sent to the recovery boiler 4 and fed to the gas turbine 7. The steam from the recovery boiler 4 is sent to the steam turbine 5. The energy generated by the turbines due to the heat of the exhaust gases and steam is used to operate the compressor 6. The compressor supplies compressed air to production [3].

2.19 Energy-technology combination in chemical technology

Improving the efficiency of fuel consumption in industry requires the use of integrated energy-technological methods of fuel processing. This makes it possible to use a combination of energy and various technological processes, that is, to include all the main processes in a common energy-chemical-technological system. This allows you to increase the utilization of heat of fuel [3].

Also, in modern chemical technology, the creation of energy-technological processes and systems in which the heat of chemical reactions can be used is of great importance.

Energy-technological combination in industry provides for the creation of new technological processes and plants. Energy-technological heat use primarily solves the problems of optimizing the technological process in combination with its high energy efficiency. At the same time, the technological and energy elements of the installation are inseparable [11].

In combined energy technology installations, along with a possible reduction in the consumption of process fuel and resulting fuel economy as a result of more efficient use of secondary energy resources. Technological advantages are also achieved that are aimed at improving the technological and energy performance of the entire installation [11].

The energy-technology combination in chemical technology is carried out both in the ECTS and in the facilities for the utilization of secondary energy resources [3].

Figure 38 shows the power technology unit SETA-C-100-1, designed to produce sulfuric acid from elemental sulfur or hydrogen sulfide, while the heat of combustion of sulfur is used to produce water vapor. It is a single drum water-tube boiler with natural circulation, it works under pressure in closed rooms. The body of the unit is an all-welded cylindrical vertical ¬ with a horizontal cyclone furnace 1, from which the combustion products of sulfur enter the radiation chamber 2. The entire unit is sheathed with sheet steel; between the shell of the boiler and the cyclone air is circulated to the burning of sulfur [3].

Figure 39 shows the scheme of ECTS of production of weak nitric acid under pressure of 0.716 MPa. Liquid ammonia enters the ammonia evaporator 4, where it evaporates due to the heat of cooling water (this produces a by-product – cooled water. The resulting ammonia gas then enters the superheater 6 and from there into the mixer 7.

Atmospheric air through the cleaning unit 1 enters the turbocharger 2a, where it is compressed to a pressure of 0.716 MPa, and then enters the air heater 5 and further into the mixer 7. Here the gas is mixed ammonia with air, after which the ammonia-air mixture, having passed the paronite filter 8, enters the ammonia oxidation reactor 9. The heat of formation of nitrous gases is used in the waste heat boiler KUN-22/13 (pos. 10) to generate water vapor.

Figure 38. Power-engineering unit SEТА-Z-100-1

Figure 39. Scheme of an energy technological installation for the production of weak nitric acid

After passing through oxidizer 11, nitrous gases from the waste-heat boiler are successively cooled in an air heater 5 and a water cooler 12, after which they enter the absorption column 13. Finished production of weak nitric acid is removed from the bottom of the column, and gases. After passing through the separator 14 and the catalytic purification reactor 3 (which is simultaneously the “combustion chamber” of the gas turbine), the gas enters the gas turbine 26. Expanding in it from a pressure of 0.7 MPa to atmospheric pressure, the “tail” gases transfer their overpressure energy to the air compressed in the turbo-compressor 2a. Exhaust gases in the turbine flow to the utilization of their physical heat into the KUG-66 waste heat boiler (pos. 15), after which they are emitted into the atmosphere.

The initial start-up of the gas turbine is carried out using the start-up “combustion chamber” 2в.

Figure 40 shows the scheme of utilization of the physical heat of the process streams in the production of sulfuric acid from pyritic sulfur. In the furnace of the "fluidized bed" (FB) 5 an exothermic oxidation reaction of pyritic sulfur occurs in order to obtain sulfuric anhydride. The heat of reaction is removed from the "fluidized bed" with water circulating with the help of a circulation pump 8 according to the scheme: separator 7 – circulation pump 8 – coil 2 of the "fluidized bed" furnace – separator. In coil 2, a steam-water mixture is formed, which in separator 7 is divided into steam and water. The physical heat of the gaseous reaction products is used to produce steam in the recovery boiler 16. It has two independent evaporation circuits with forced circulation 14, 15 and a superheater 6. In the first circuit, the water circulates according to the scheme: feed tank 9 – pump 10 – coil 15 – separator 7. In the second circuit, water circulates according to the scheme: separator 7 – circulation pump 8 – coil 14 – separator 7. Produced in two coils in the contours of the steam-water mixture, as well as the steam-water mixture of the coil, in the separator 7 is divided into water and steam, which then enters the superheater.

The energy technology system can be considered using the ammonia synthesis process as an example. The functional diagram of the ammonia synthesis process is shown in Figure 41. It shows only the main stages of the chemical transformation of the feedstock and there are no nodes to create an energy-technology production [3].

At the first stage of the processing of raw materials, the steam conversion of methane in a tube furnace at a temperature of 800 ° C is shown.

The second stage is the steam-air conversion of methane by atmospheric oxygen in a mine converter; it is also carried out at a high temperature 1000 0C. The third stage of the process is the vapor conversion of carbon monoxide at a temperature of 450 0C.

The nitrogen-hydrogen mixture is cleaned of impurities in the absorber under a pressure of 2.6 MPa, then it is compressed in a compressor and fed to a contact column for the synthesis of NH3. Reagents that exit the synthesis column are cooled in the refrigerator, while ammonia is separated. Next, the nitrogen-hydrogen mixture is returned to the synthesis column for re-conversion [3].

1 – blower; 2 – coil; 3 – feeder; 4 – FB furnace receiving hopper; 5 – FB furnace;

6 – steam superheater; 7 – separator; 8 – circulation pump; 9 – nutrient tank;

10 – circulation pump; 11 – cyclone; 12 – electrostatic precipitator; 13 – screw;

14, 15 – waste heat boiler coils; 16 – waste heat boiler; 17 – cyclone

Figure 40. Scheme of utilization of physical heat of process streams in the production of sulfuric acid

Figure 41. Functional diagram of the synthesis of ammonia

1– tube methane converter; 1a – convection chamber of a tube converter;

2 – mine methane converter; 3, 5, 11– waste heat boilers; 4 – carbon monoxide converter;

6 – absorber; 7 – compressor; 8 – steam turbine; 9 – synthesis column;

10 – heat exchanger

Figure 42. Schematic diagram of the synthesis of ammonia

Figure 42 shows a flow chart of ammonia production. In a methane converter 1, methane is converted at high temperature to steam. The converter is heated by the combustion of natural gas. The heat of the exhaust flue gas is used in the future. To heat natural gas, which is sent for steam conversion to converter 1; air that enters the steam-air conversion in the mine Converter 2; steam that is fed to the steam reforming of methane (converter 1) and carbon monoxide (converter 4). Next, hot flue gases are sent to a waste heat boiler 11 to produce steam [3].

The reagents that leave the CH4 2 steam reforming and CO 4 steam reforming reactors also have high temperatures and are sent to waste heat boilers 3 and 5, where saturated steam is obtained. Then saturated water vapor is overheated and sent to one of the steam turbines. Steam is supplied to a steam turbine 8 mounted on the same shaft with a turbo-compressor 7, which compresses the nitrogen-hydrogen mixture entering the ammonia synthesis column 9 and then passing along with the reaction products through the cooling unit 10, where condensed ammonia is separated. This reduces the energy consumption for compression of the nitrogen-hydrogen mixture.

An example of the regeneration of mechanical energy is a unit for purifying a nitrogen-hydrogen mixture from carbon dioxide CO2. Cleaning is carried out in the absorber 6 under a pressure of 2.6 MPa with an aqueous solution of monoethanol amine. The liquid exiting the absorber under pressure enters the turbine, which is located on the same shaft as the pump [3].

2.20 The main directions of increasing the efficiency of the use of raw materials and fuel and energy resources

To increase the efficiency of economic development, rational use and saving of raw materials and fuel and energy resources are of no small importance.

Research and development in the field of chemical technology and chemical engineering are developing in two main directions [3]:

- developments to improve existing technological processes aimed at reducing the specific consumption of raw materials, materials, energy consumption, as well as improving the quality and environmental friendliness of products and technological processes;

- developments of fundamentally new types of chemical and petrochemical products competitive in the world market, as well as methods for their production.

Work is underway on the integrated use of natural raw materials. New progressive technological processes are being developed, such as electrochemical, membrane, photochemical, plasmachemical, biochemical, radiation, etc. [3].

The main trend in changing the structure of the energy balance is the increasing use of atomic energy. Only 1 kg of U–235 during splitting emits 24 million kW of energy, which corresponds to the calorific value of 3 thousand tons of coal. The splitting of 1 kg of uranium gives almost 3 million times more energy than burning 1 kg of hard coal, and 7 million times more than burning 1 kg of brown coal.

The consumption of raw materials and energy in society should be carried out in accordance with three conditions:

• the rate of consumption of renewable resources should not exceed the rate of their recovery;

• the rate of consumption of non-renewable resources should not exceed the sustainable rate of use of their alternative replacement;

• the intensity of emissions and discharges of pollutants should not exceed the possibility of their absorption by the environment without deterioration of its quality.

Recent data indicate that to date, the main sources of primary energy are oil and oil products (40.5 %), coal (25 %), gas (24 %), nuclear energy (8 %) and renewable energy sources (2.5 %)  [3].

Efficient energy-using equipment is being introduced in industry. Technological processes and equipment are developed and implemented that have a high technical level and low costs of both raw materials and energy resources. The combined production of electric and thermal energy in industry is used.

The quality of thermal insulation of technological equipment and pipelines increases, which contributes to energy saving. In the chemical industry, more powerful energy-technological installations and equipment are being introduced [3].

The modernization and reconstruction of existing equipment, the optimization of the operation of energy and technological installations, which leads to a reduction in the consumption of raw materials, fuel, heat, electricity, are carried out.

In various industries, modern automation tools are used to record, control and optimize fuel consumption, heat and electric energy in technological processes and installations.

When using secondary material and fuel and energy resources, utilization and recovery of various types of heat in power and technological installations, large savings of raw materials and energy are achieved.

For this purpose, reactor designs for various exothermic and endothermic processes are created and improved, cost-effective plants are created that ensure the use of low potential heat of technological flows of chemical plants [3].

One of the most important areas of rational use of energy of various types is the creation of combined energy technology installations.

Membrane methods with high efficiency and low energy consumption are being developed. The separation of mixtures by membrane methods (ultrafiltration, reverse osmosis, electrodialysis) is carried out without phase transformations and usually at ambient temperature.

Such methods can solve many problems with a high economic effect, create technological processes and equipment that conserve raw materials and energy, and improve the environment. [3].

The use of non-traditional renewable energy sources (solar, deep heat of the Earth, wind and tidal energy, biomass) is a promising way to obtain energy.

Control questions and tasks for individual work

- What is exergy?

- What is the Energy Technology?

- What are the possibilities and goals of the exergy analysis of technological processes?

- How to calculate the exergy of the reaction flow?

- What is the main difference between the concepts of "energy loss" and "loss of exergy"?

- What is the purpose of using exergy efficiency in technology?

- What are the ways to increase exergy efficiency?

- What are the main types of energy resources? Which of them are the most perspective?

- List renewable and non-renewable energy resources.

- What are secondary energy resources? How are they classified by type of energy? What is the role of secondary energy resources in saving fuel and energy?

- Give examples of the use of the heat of chemical reactions in energy systems.

- Formulate the main directions of increasing the efficiency of the use of raw materials and fuel energy resources.

- Determine the increase in the exergy of 1 kg of air, which is in equilibrium with the environment, having parameters p0 = 0,1 MPa, T0 = 293 K, if it is adiabatically compressed to T = 500 K.

- Determine the loss of exergy in the throttling of CO2 from the pressure p1 = 2,0 MPa to p2 = 1,0 MPa. The ambient temperature is T0 = 285 K.

- Determine the exergy efficiency of the regenerative heat exchanger of a gas turbine in which air is heated from Тa= 400 K to Тa  = 535 K, and the exhaust gases are cooled from Тg = 615 K to Тg = 480 K. For gases and air, take the heat capacity cp = 1,005 kJ / (kg.K). The ambient temperature is T0 = 275 K.

REFERENCES

- Рындин В.В. Теплотехника: Учебное пособие / ОмГУ, 2012 – 460 с.

- Бесков В.С. Общая химическая технология: Учебник для вузов – М.: ИКЦ «Академкнига», 2005. – 452 с.

- Кутепов А.М., Бондарева Т.М., Беренгартен М.Г. Общая химическая технология: Учеб. для вузов – М.: ИКЦ «Академкнига», 2004. – 528 с.

- Гюнтер Цербе, Гернот Вильгельмс. Техническая термодинамика. Теоретические основы и практическое применение. – Изд-во Фолиант, 2015.

- Цветков Ф.Ф. Тепломассообмен: Учебник. – Изд. МЭИ, 2011. – 562 с.

- R.E.Sonntag, C.Borgnakke, G.J.Van Wylen. Fundamentals of Thermodynamics. – 2009. – 794 p.

- Теплотехника: Учебник для вузов. /Луканин В.Н., Шатров М.Г., Камфер Г.М. – М.: Высшая школа, 2000. – 671 с.

- Исаев С.И. Термодинамика: Учебник для вузов – М.: Изд-во МГТУ им. Н.Э.Баумана, 2000 – 416 с.

- Бесков В.С., Сафронов В.С. Общая химическая технология и основы промышленной экологии: Учебник для вузов. – М.: Химия, 1999 – 472 с.

- Теплотехника: Учеб.для вузов/ А.П.Баскаков, Б.В.Берг, О.К.Витт и др.; под ред. А.П.Баскакова. – М.: Энергоатомиздат, 1991. – 224 с.

- Латыпов Р.Ш., Шарафиев Р.Г. Техническая термодинамика и энерготехнология химических производств; Учебник для вузов. – М.: Энергоатомиздат, 1998. – 344 с.

- Бродянский В.М., Фратшер В., Михалек К. Эксергетический метод и его приложения/ Под ред. В.М.Бродянского. – М.: Энергоатомиздат, 1988.

- Лейтес И.Л., Сосна М.Х., Семенов В.Л. Теория и практика химической энерготехнологии. – М.: Химия, 1986. – 280 с.

- Вторичные энергетические ресурсы и энерготехнологическое комбинирование в промышленности: Учебник для вузов/ Н.А.Семененко, Л.И.Куперман, С.А.Романовский и др. – Киев: Виша школа, 1986.

- Чечеткин А.В., Занемонец Н.А. Теплотехника – М.: Высшая школа,1986. – 344 с.

- Позин М.Е., Зинюк Р.Ю. Физико-химические основы неорганической технологии: Учеб.пособие для вузов. – Л.: Химия, 1985. – 384 с.

- Лариков Н.Н. Теплотехника: Учебник для вузов. – М.: Стройиздат, 1985.

- Молдабеков Ш. Технология синтеза аммиака: Учебное пособие/ Ш. Молдабеков, Б. А. Альмаханов, Б. Ш. Молдабеков, В. К. Бишимбаев; ЮКГУ. – 2004

- Амелин А.Г. Технология серной кислоты: Учебное пособие для студ. хим- тех. спец. вузов/ А. Г. Амелин. – М.: Химия, 1971.

Раиса Ривкатовна Якубова

Жаныл Кемаладиновна Джанмулдаева

Галина Мануиловна Сейтмагзимова

Учебное пособие

по дисциплине «Энерготехнологические системы в технологии неорганических веществ»

для студентов специальности

5B072000 – Химическая технология неорганических веществ

всех форм обучения

Редактор Якубова Р.Р.

Подписано в печать

Формат бумаги XxY 1/16

Бумага типографская. Печать офсетная. Объем  6,25 п.л.

Тираж     экз. Заказ №     .

© Издание Южно-Казахстанского государственного университета им.М.Ауезова

Издательский центр ЮКГУ им.М.Ауезова, г.Шымкент, пр.Тауке хана, 5