Ф.7-03-03

Bakirzhankyzy A.,Toshboltayeva N.N., Turlybekova G.E., Orazalieva R.N.

Practical Physics

"Methodical instructions for laboratory work on Physics"

for students of technical specialties

Shymkent, 2020

Ministry of Education and Science of

the Republic of Kazakhstan

M.AuezovSouthKazakhstanStateUniversity

Departmentof High mathematics and Physics for TS

Bakirzhankyzy A., Toshboltayeva N.N., Turlybekova G.E., Orazalieva R.N.

Practical Physics

Shymkent 2020

УДК 378.147.53(076.5)

ББК 74.265.1

Bakirzhankyzy A.,Toshboltayeva N.N., Orazalieva R.N., Turlybekova G.E. Practical physics (for students of the technical specialties). -Shymkent: M.Auezov SKSU, 2020.-94 p.

Practical physics has been developed in accordance with thetypical program in the discipline on the technical specialities.

Reviewers:

N.S. Saidulaeva - doctor of physical-mathematical sciences

The study guide has been approved and recommended for publication by the High mathematical and Physics for TS Department meeting

(minutes №8from 13.03. 2020)

The study guide has been approved and recommended for publication by Mechanical and Petroleum engineering methodicalcommission

(minutes№8 from26.03. 2020)

The study guide has been approved and recommended for publication by Methodical Council of M. Auezov SKSU (minutes №__ from  __.__.2020)

© M. Auezov SKSU, 2020

© Bakirzhankyzy A.

Contents

| Introduction………………………………………………………………...... | 3 |
| --- | --- |
| Safety rules at performance of laboratory works in physics………………… | 4 |
| Students’ duties..…………………………………………………………….. | 5 |
| MECHANIC |  |
| Laboratory work №1. Mathematical Treatment of Measuring Results |  |
| 1.1. Processing of Multiple Direct Measuring Results………………………. | 6 |
| 1.2. Processing of Indirect Measuring Results | 10 |
| Laboratory work №2. Definition of Coefficient of aRollingFriction……… | 15 |
| Laboratory work № 3. The Study of Rotational Motion Laws of a Solid Body. | 18 |
| Laboratory work №4. The study of Laws of Oscillatory Motion by Physical Pendulum…………………………………………………………………… | 22 |
| MOLECULAR PHYSICS AND THERMODYNAMICS |  |
| Laboratory work №5. Determining Viscosity Coefficient of the Liquid by Stokes Method………………………………………………………………... | 27 |
| Laboratory work №6. Determination of Reynolds number ………………….. | 31 |
| Laboratory work №8. Determination of fluid friction force by Stokes formula Laboratory work №9 Determination of a fluid surface tension coefficient by a breaking method of ring…………………………………………………... | 35 39 |
| ELECRICITY AND MAGNETISM |  |
| Laboratory work №9. Modeling of Electrostatic Field…………………… | 43 |
| Laboratory work №10. Method of Determining Electric Resistance………… | 46 |
| Laboratory work №11. Determination of Capacitor Capacitance………… | 49 |
| Laboratory work №12.Determination of RC Time Constant………………….. | 54 |
| Laboratory work №13. Studying of Hall Effect in Semiconductors…………… | 59 |
| Laboratory work №14. Establishment of Normal Magnetization Curve of Ferromagnetic …………………………………………………………………. | 64 |
| Laboratory Work №15. Determination of the Coil Inductance………………… | 70 |
| OPTICS Laboratory Work №16. Checking Malus’s law………………………………... | 74 |
| Laboratory Work № 17.Determining refraction index of liquid by refractometer | 78 |
| Laboratory Work №18. Study of Photoeffect Basic Law……………………… | 83 |

Introduction

Practical physics demonstrates the purposive and critical approach that should be made to all experimental work in physics. It does not describe a systematic course of experiments, but is intended as a companion to any undergraduate course of practical work.

This book is intended to help you to do practical physics at university:  its aim is to make the laboratory work more useful and profitable. We may start by asking what is the object of laboratory work in a university physics course. There are several possible objects. Laboratory work may serve

(a) to demonstrate theoretical ideas in physics,

(b) to provide a familiarity with apparatus,

(c) to provide a training in how to do experiments.

Let us consider each of these in turn.

Seeing something demonstrated in practice is often a great help in understanding it. For example, interference in light is not an intuitive concept. The idea that two beams of light can cancel each other and give darkness takes a little swallowing, and most people find it helpful to be given a visual demonstration. Ademonstration is useful for another reason - it gives an idea of orders of magnitude. The interference fringes are in general close together, which indicates that the wavelength of light is small compared with everyday objects. But the demonstration is no substitute for a proper explanation, which goes into the details of geometry and phase relations. So the first object, the demonstration of theoretical ideas, has a definite but limited usefulness.

The second object is perhaps more important, but it is necessary to say exactlywhat is meant by 'apparatus' in this context. In any practical course you will handle a number of instruments, such as oscilloscopes, timers, thermometers, and so on, and the experience you gain from using them should prove useful. However, if you eventually do scientific work of some kind, the range of instruments you could conceivably work with is enormous. No practical course could possibly teach you to use them all. What the course should do is to train you to use instruments in general. There is a certain attitude of mind that an experimenter should adopt when handling any instrument, and this the course should try to instill. But this is part of the third object which is the most important of all.

The phrase 'how to do experiments' may sound vague, so let us try to be more specific. The primary object - or set of objects - of practical physics is to train you to

(a) plan an experiment whose precision is appropriate to its purpose,

(b) be aware of and take steps to eliminate systematic errors in methods and instruments,

(c) analyze the results in order to draw correct conclusions,

(d) estimate the precision of the final result,

(e) record the measurements and calculations accurately, clearly, and concisely.

All this adds up to saying that the main object of a course in practical physics is to train you to be a competent experimenter. But the course can do still more. It can show the way physics works.

Physics is one of the natural sciences, that is to say, it is part of our attempt to understand the natural world. When we are confronted by a situation in the natural world, the way we proceed in physics is to select what we think are the essential features. For example, the Greeks saw that a moving body came to rest and they therefore said that a force is necessary to keep a body moving. Galileo and Newton observed the same phenomenon, but they said that the coming to rest of the body is not an essential feature of the situation. In depends on friction; in the absence of friction a body keeps moving. If we try to do an experiment to test this view, we find that we cannot completely eliminate friction or other retarding forces, but we can make such forces small, and the smaller we make them the farther the body goes before coming to rest. So it is reasonable to believe that in the limiting case of zero friction the motion will remain unchanged as stated in Newton's first law.

This is the way physics works. We select what we think are the essential features in an actual physical situation. From them we make a generalization, or theory, and from the theory, deductions. We test a deduction by doing an experiment. But the deduction refers to an idealized or theoretically simple situation. In order to test it we have to create this simple situation in the messy, complicated, natural world, which is often a difficult thing to do.

In lectures you are taught the theory of the subject. The physical world is described in terms of the features which the current theories say are essential. These features tend to be the only ones you hear about, and you may well come to feel that they constitute the entire world, instead of a specially selected part of it.

Moreover, everything fits together so smoothly that you can easily lose sight of the genius and effort that went into creating the subject. The most effective antidote to this is going into the laboratory and seeing the complications of real life.

By doing practical physics, then, you learn at first hand some of the obstacles to testing a theory, to measuring what you want to measure and not something else, and you learn how to overcome them. But above all you get an insight into physics as a whole, into the way experiment and theory interact, which is the very essence of the subject.

Safety rules at performance of laboratory works in physics



- Do not work in outer clothing.

- Do not work without the teacher’s permission. To start studies only having admission for work before.

- Study the theoretical introduction and the work sequence carefully before doing any work.

- Do not admit ultimate loads of measuring devices during experiences.

- See to device accuracy. Do not touch and not bend over rotating device parts.

- Use wires with strong insulation at assembly of electric circuits.

- Turn on power supply at the end after checking the precision of the assembled circuit.

- Do not touch the stationary electrics housing and clips of even the switched - off capacitors.

- Use electric tools with insulating handles.

- Switch off power supply after work completing and disassemble a circuit.

- Switch off power supply immediately having found out malfunction in the electric device alive and inform about it to the teacher or the laboratory assistant.

- t is necessary to keep in order and cleanliness of laboratory. Put to rights the workplace after finishing work.

- It is necessary to remember that students to account for used implements and devices.

- Checkup whether (two-pin) plugs are switched - off before leaving laboratory.

Students’ duties

- Acquaint with safety rules and protection of labor.

- Acquaint with the aim of work, problems and tasks before doing work.

- Study theoretical material necessary for digestion of knowledge related to work.

- Obtain a permit from the teacher for doing work.

- Do work according to the task and methodical instructions.

- Drawing up of work results and speak in support of it at the teacher.

MECHANIC

Laboratory Work №1

Mathematical Treatment of Measuring Results

1.1. Processing of Multiple Direct Measuring Results

The aim of the work: to study the processing technique of multiple direct measuring results; to define best probable value of measured quantity, confidence interval at given (in desired) reliability and relative error.

Materials: mathematical pendulum, stop-watch.

Theoretical introduction. Any measurement is connected with errors resulting from imperfection of human sense organs and measuring devices. In any measurements some errors are allowed, therefore measuring results give us not true but only approximate value of measured quantities. The stating of an admissible measuring error and interval, in which the true value of measured quantity lies is the required (necessary) condition of experiment reliability.

Measuring errors are divided into crude, systematic and random.

Crude errors are caused by the result of device (malfunction) derangement or observer’s carelessness, disarrangement of experimental technique or conditions of its carrying out. In more cases, misses are well noticeable, since measuring indications corresponding to them violently differ from those of the like ones. Measuring results containing the miss should not be taken into consideration in data processing operation - it should be just omitted.

Systematic errors appear in the result of disarrangement or inaccuracy of calibration of measuring devices, in the use for analysis of non-precise data and also are rooted from the imperfections of the experimental technique itself. These errors always influence on measuring results one-sidedly (only increase or decrease them). It is obvious that influence of systematic errors is impossible to decrease by the increase of measurement numbers. However, if the nature and the character of the systematic errors are known their influence on measuring results can be considered by introducing correction and to exclude.

Random errors are stipulated by fluctuations of the measured quantity. Their appearance cannot be prevented, therefore they may make a certain influence on individual measurements changing result to both sides that is either increase and decrease them. They obey the statistic laws, that’s why the influence of random errors on measuring results can be taken into account or considerably decrease. While applying laws of theory of probability the more probable values of measured quantities and possible deviations from these values are defined.

Assessment of accuracy (accuracy ranking) of multiple direct measuring results. Let us assume that in multiple measurements of some  physical quantity  of its values were received:. In such a case arithmetical mean (average value) of all the received values, which is the most reliable, is equal to

.                    			(1)

Each individual measurement differs from the arithmetical mean by the magnitude equal to

(2)

Deviation  is called absolute error of i-th measurement.

Absolute errors  accept both positive and negative values.

It is known from the experience that if the greater the error there is the less probability of its appearance. Besides if the number of measurements is very great     (), the errors similar in value, but different in sign, will often equally occur. The rate of decrease of error occurrence probability  with its increase is characterized by dispersion. It is equal to

(3)

The less is , the less is the less probability of appearance of an error with greater magnitude , the less is the spread of individual values , i.e. the higher is  the accuracy of measurement.

In practice the number of measurements that can be carried out is often limited. Therefore n is practically chosen as the final one and as dispersion its approximate value is concerned:

(4)

Subsequently,

(5)

The square root of dispersion of measurements is called standard deviation (quadratic mean error):

(6)

Substituting instead of  in the expression (6) for its value from the equation (2), we obtain

(7)

The expression (7) includes the quantity but not. The quantity  itself is identified by some error.  The magnitude, equal to the quadratic mean error divided by square root from measuring numbers is called quadratic mean error of arithmetical mean:

,                                                            		(8)

оr

(9)

Variance estimate and are limitary, true only in, that is in great n. In small n these evaluations, which are accidental themselves, at the best case determine only the order of dispersion magnitude.

Mathematical processing of measuring results is interval determination of 	         (,), where the true value of measured quantity can be found with  probability. Interval (, ) is calledconfidential,  is confidence factor (reliability). If numbers of measurements  are great enough then the confidence factor presents the part of general number of measurements where the measured quantity makes an influence within confidence interval. For instance, if 100 measurements were done, then 95 measurements gave the results within its limits in confidence coefficient =0.95. It is clear, that the more reliability is required the more corresponding confidence interval is received, and vice versa, the more confidence interval is given it is more obvious that the measuring results will not go beyond its limits. It results from the theory of errors that in large number of measurements n (more than 100) confidence coefficient of the interval from [,] is equal to 68%, and of the interval [,] is equal to 95%. While presenting any measured quantity it is important to point out interval and confidence probability corresponding to this interval.

In cases when the number of measurements is not great then there the conditions for strict fulfillment of statistical regularity lying in the base of random errors determination are absence. It results in that the value of quadratic mean error (standard deviation)  calculated by the (9) formula is not exact and inaccuracy is more the number of measurements is less. Hence, to guarantee the true value of quantity measured with the given probability is within confidence interval, the latter should be increased. In limited number of measurements not, but absolute error

(10)

is accepted as the margin of confidence interval.

Numerical value of  depends on the quantity of confidence coefficient  and the number of measurements, and it is called the Student coefficient. Student coefficient value for various is available in the laboratories (Table 1).

Table 1.

|  | 0,6 | 0,7 | 0,9 | 0,95 | 0,99 |
| --- | --- | --- | --- | --- | --- |
| 2 | 1,38 | 2,01 | 6,31 | 12,71 | 63,66 |
| 3 | 1.06 | 1,3 | 2,92 | 4,30 | 9,92 |
| 4 | 0,98 | 1,2 | 2,35 | 3,18 | 5,84 |
| 5 | 0,94 | 1,1 | 2,13 | 2,78 | 4,60 |
| 6 | 0,92 | 1,1 | 2,02 | 2,57 | 4,03 |
| 7 | 0,90 | 1,1 | 1,94 | 2,45 | 3,71 |
| 8 | 0,90 | 1,1 | 1,90 | 2,36 | 3,50 |
| 9 | 0,90 | 1,1 | 1,86 | 2,31 | 3,36 |
| 10 | 0,90 | 1,1 | 1,83 | 2,26 | 3,25 |

Measuring result is performed in the following way:

(11)

Results of different quantity measurements cannot be compared with each other according to their absolute errors. To compare measurement accuracy of such quantities relative error, that is relevance of absolute error to the average value of measured quantity

(12)

is introduced. It is convenient also to compare measuring results and homogenous quantities using relative error.

Work sequence

As an instance let us consider measuring results processing of oscillation period of mathematical pendulum (mathematical pendulum is the object suspended on weightless and not unstretchable thread the length of which is much more of its sizes).

- To measure the time  of N pendulum oscillations by a stop-watch. Repeat measurement  times (N and  is given by a teacher).

- To calculate oscillation period according to the formula

(I3)

- To determine arithmetic mean of the period  according to the formula (9).

- To calculate standard deviation of the quantity by the formula (9).

- To find the value of Student coefficient  corresponding the reliability given by the teacher from the table (table of Student coefficients can be found in laboratory).

- To figure out absolute error   by received  and  using the formula (I0).

- To calculate relative error according to the formula (I2).

- To register in the table 1 measuring and figuring results.

Table I

| № n | N | ,s | ,s | ,s | ,s | ,s |  | , s | ε |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Control questions

- What is mathematical treatment of multiple direct measuring results?

- What kind of measurements do you know? What kind of errors do you know?

2. 	 How is absolute error defined?

3. 	What is the Student coefficient introduced for?

4. 	 What does confidence interval present? What does confidence probability present?

5. 	 Why is it necessary to calculate relative error?

6. 	 What is mathematical pendulum?

7. 	 What is the period of oscillation?

References

1. Zaydel' A.N. Elementarnyye otsenki oshibok izmereniy. –M .: Nauka, 1974.

2. Kassandrova O. N., Lebedev V.V. Obrabotka rezul'tatov nablyudeniy. - M .: Nauka .1970.

3. Agekyak T.A. Osnovy teorii oshibok dlya astronomov i fizikov.-M .: Nauka, 1972.

1.2. Processing of Indirect Measuring Results

The aim of the work: acquaintance with errors determination methods of indirect measurements of physical magnitudes.

Materials: drawing scale, sliding caliper, micrometer, weighbridge, and set of objects of geometric regular shaped.

Theoretical introduction. Accuracy assessment of one direct measuring result: If in the process of multiple measurements the device gives one and the same indications, then the repetitions of measurements loses its significance; it’s enough to measure once. This will evidently happen in case when absolute error of working measuring device will be more than the residual errors of individual measurements. Limited absolute error either is shown in technical certificate of measuring device or is determined by the half of price of the least division of its scale.

Example.  While measuring cube sides by sliding caliper the value  was deduced. Scale division of sliding caliper is. Hence, limited absolute error of measurement is equal to. Limited relative error of measurement is equal to relative error of beam compass:

(1)

Measuring result should be written in the following way:

(2)

In the laboratory works often values of some quantities measured beforehand are given: there are physical constants,figure π, and table data. In such cases absolute error is considered to be equal to its limited value, i.e. equal to the half unit of the least category, presented in figure.  For instance, if the mass of a body is given  then, hence,  and for  we have, and etc.

Accuracy ranking of indirect measuring results. Generally, an unknown quantity is the function of one or more quantities. For example, an electric current strength I in conductor is the function of potential difference U at the conductor ends and resistance R. Therefore, for determination of such quantity at first it is necessary to hold the set of such measurement of auxiliary quantities, and then using the formulae of physical laws to calculate unknown quantity. In this case we say about indirect measurements.

We suppose that the unknown quantity A is the function of measured quantity x:

(3)

Absolute error of measured quantity leads to the error of, .  Expansion of the right-hand in Taylor series gives:

(4)

We will be restricted by two terms of this series, as other terms are infinitely small of higher order. Then:

(5)

According to (2.3), we have:

(6)

That is, absolute error of the function is equal to product of derivative of this function and absolute error of the argument.

Let unknown quantity A depends on n variables:

(7)

Then the differential of multivariable function can be presented as:

,				(8)

where  is partial derivatives. In calculating the latter all the arguments except, are considered as constant and the  differentiation is carried out in the way as the function of one variable.

Instead of absolute error it is often convenient to figure out relative error. Relative error can be presented as:

(9)

When the function, as it is often occurs, has monomial form, then either development of formula or calculations according to them turn out to be considerably simple, if to use the rule (9). Expression (8) can be used for error determination if to change differential sign into error sign:

(10)

or                  			(11)

Overall result is recorded as:

(12)

Thus, for definition of error of indirect measuring results we should:

1. 	 take the logarithm of calculating formula;

2. 	 take total differentiation of both parts of equality:

3. 	 replace the  d-symbol with  the symbol , all the symbols of  minus with plus ones:

4. 	 find relative error ε by the (11) formula,

5. 	determine absolute error, knowing the ε:

(13)

6. 	 write the result in the form of (12).

Absolute error of measuring result should be rounded off to first significant figure, andmeasuring result should be expressed in round numbers to the order where the error starts.

As an example, we consider the calculation of spherical volume. It is equal to

(14)

where D is the diameter of sphere. D is directly measured and its volume V is measured indirectly.

Let D be measured by beam compass which minimal scale division is equal to 0.05mm. Then the limiting error in measuring diameter is equal to. Result of direct measurement taking into account errors has the following value:

(15)

Let us calculate the volume V.  It is equal to. Further we defined the error  by the formula:

(16)

As for the error, it can be done as little as possible, if to take with sufficient amount of signs. Then it (error) can be ignored. For the considered example we have, . Eventually

(17)

, .

Absolute error of result should be certainly round off till one-valued figure, and measuring result (in considered example V) should be rounded off till the category where the error starts. In the given case they should be rounded off till hundreds. Final result of measurements will be recorded in the following way:

(18)

Work sequence

It is necessary to calculate mass density of object of geometric shape (rectangular parallelepiped, cylinder, and sphere).

- To define mass of object (parallelepiped, cylinder, sphere). To write with the account of measurement accuracy.

- To measure the size of object by beam compass (the same objects). To determine the errors.

- To calculate the mass density of  objects measured by the following formulae:

а) For rectangular parallelepiped

(19)

where m is mass, his height, b is width, l is length of parallelepiped:

b) For the cylinder

(20)

where m is mass, D is diameter of the base, h is height of the cylinder:

c) For  sphere

(21)

where m is mass, D is diameter of the sphere.

1. 	 According to the formula (11) to calculate relative errors, i.e.

а) for rectangular parallelepiped

(22)

b) for cylinder

(23)

c) for  sphere

(24)

- By the formula (13) to figure out absolute error. To round off received results with measurement accounts.

- To record final result as (12)

- To register results of measurements and calculations in the table 1

Table 1

|  |  |  |  |  |  |  |  |  |  | ,% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Parallelepiped |  |  |  |  |  |  |  |  |  |
| 2 | Cylinder |  |  |  |  |  |  |  |  |  |
| 3 | Sphere |  |  |  |  |  |  |  |  |  |

Control questions

- How are instrumental errors defined? What is precision of device indication? Point out division value of nonius.

2. 	 How are absolute errors of physical constants with infinite nonperiodic fractions fixed?

3. 	 How is the accuracy of indirect measurements assessed?

4.   Present that relative error of measurements (ε) depends on geometric shape of the object.

5.  Work out the formulae (22)-(24)

6.  Work out the formulae of relative errors assessment for the function of the type

.

Here  are some constant figures.

References

1. Zaydel' A.N. Elementarnyye otsenki oshibok izmereniy. –M .: Nauka, 1974.

2. Kassandrova O. N., Lebedev V.V. Obrabotka rezul'tatov nablyudeniy. - M .: Nauka .1970.

3. Agekyak T.A. Osnovy teorii oshibok dlya astronomov i fizikov.-M .: Nauka, 1972.

Laboratory work №2

Definition of Coefficient of aRollingFriction

The aim of the work: Experimental studying of the main regularities arising at a rolling friction. Definition of coefficient of a rolling frictionfor a number of materials.

Materials:Inclined pendulum FPM-07,  universalmillisecond timer FPM-14,  balls and samples.

Theoretical introduction: Frictional forces appear during the interchange of contacting bodies or parts relatively to each other. The friction, arising during relative motions of two contacting bodies, is called external. If these bodies are motionless relatively to each other, it is a rest friction, and during their relative motions it is called a sliding friction.

The friction arising at a rollingof a wheel (cylinder) along the plane without sliding is called a rolling friction. The friction between parts of the same continuous body (for example, liquid or gas) is called an internal friction. It is characteristic for an external friction an existence of force of a rest friction, defined as limiting tangential force under the influence of which relative motion of contacting bodies begins.

Frictional force depends on a friction coefficient f. The quantity of friction coefficient isn't constant, and depends on a material and a condition of surfaces of contacting bodies.

For experimental studying of the main regularities arising at a rolling friction, usually, a ball suspended on a thread, which leans on a solid sample, is used. If we deprive a ball its equilibrium position, it will start rolling along a sample. The motion of a ball has nature of the fluctuations damping mainly under the influence of a rolling friction, arising at a ball rolling on a surface of the test specimen. Appearance of rolling frictioncan be explained by formation the deformations of a ball and a sample surface. Thus both elastic and plastic deformations can arise. Because of deformation of surfaces the line of action of reaction force Q (Fig.1.) doesn't coincide with the line of action of normal pressure, i.e. with ball weight.

| Figure 1. | Figure 2. |
| --- | --- |

The normal component Qn of this reaction force to the plane is almost equal to the applied normal load , and the horizontal component represents a frictionforce.

If the ball moves on the plane without acceleration, the rule of equality of the moments of forces should be carried out:

(1)

i.e. QnN, that from (1) we have:

(2)

where R is ball’s radius. Thus, force of a rolling frictionis proportional to force of normal pressure and is inversely related to ball’s radius.

The quantity f is called coefficient of a rolling friction. The coefficient of a rolling frictionrepresents an arm of force and has dimension of length.

For n of cycles of fluctuations the pendulum loses its energy, which is equal to work of friction force on a way of S, where  is the loss in altitude by the ball’s center of gravity.

Thus

(3)

It is seen from the figure 2 that . Considering that force of normal pressure N=mgcos, the equation (3) will look like:

(4)

From the last equation at burn-out loss of a deviation we find coefficient of a rolling friction:

(5)

Figure 3. Inclined pendulum FPM-07

Installation description: To research a process of a friction the inclined pendulum FPM-07 (Fig.3) is used in this work. To the basis, with four legs with adjustable height, the millisecond timer is attached. In the basis the pipe (3), on which the case (4) with a worm gear is built in, is fixed. The worm gear is connected with the supporting arm (5) by axis. On the supporting arm (5) the scale 1 (6) and a scale 2 (7) is attached. In the supporting arm the column (8), on which the ball with a rod (9) is suspended, is fixed. Samples (10) are put into the supporting arm (5) along the directions. For an inclination of a pendulum the handle (11) is used. The photo-electric sensor (12) is screwed to the supporting arm. The ball is replaced by unscrewing it from a rod and a screwing on a new one.

Work sequence

- Regulating the heights of legs of the device a pendulum thread should be located over against a zero scale division.

- To measure a ball radius by calliper (micrometer).

- To fix a test ball to a rod, and into the directions – a sample.

- To incline an arm of the device at the angle  on a lateral scale.

- To switch a cord into an electric network and switch on by pressing a power button.

- Deflect the ball from equilibrium position at an angle  or ,  and write down in the table, having converted degrees into radians.

- To release a ball without a push, thus a stop watch will count the time of a rolling down and number of fluctuations.

- To count 10-15 fluctuations and measure approximately a deviation angle  of a pendulum on-scale (write down into the table, having converted into radians).

- To repeat points 4-8 at other provisions of an angle (set by the teacher).

- To repeat points 2-8 with other balls and samples.

- To calculate coefficient of a rolling frictionby the formula (5).

- To estimate errors of measurements using the formula:

Table

| № |  |  |  |  |  |  | , (m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | grad. | rad. | grad. | rad. |  |  |
| 1. |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |
| …. |  |  |  |  |  |  |  |  |

Control questions

1. What types of a friction do you know?

2. How does the internal friction differ from the external?

3. How is the maximum rest friction defined?

4. Can a friction force be more than external force applied to the body?

5. How is the force of a rolling frictiondefined?

6. How is the coefficient of a rolling frictiondefined? Specify dimension of coefficient of a rolling friction.

References

1. I.V.Savel'yev. Kurs obshchey fiziki, t.1, M .: Nauka, 1987 str. 66-69.

2. D A. Detlaf, B.M.Yavorskiy, L. B. Milkovskaya. Kurs fiziki, t.1, M .: Vysshaya shkola, 1973 g., str. 97-99.

3. S.P.Strelkov, Mekhanika. M .: Nauka, 1975 g., str. 263-267.

Laboratory work № 4

The Study of Rotational Motion Laws of a Solid Body

The aim of the work: study of rotational motion laws of a solid body, determining the moment of inertia of Oberbek’s pendulum by experimental and computational method.

Materials: Oberbek pendulum apparatus, loads, beam compass.

Installation description and derivation of formula for calculation. Oberbek Pendulum apparatus, hereafter referred to as device, is a complex maintaining verification of laws of dynamics of a solid rotational motion.

Figure 1.

Pendulum apparatus, used in the device, is an inertial disk in a cross form (Fig.1). The load  can change its position on the four mutually perpendicular rods. On horizontal axis there is two stage disks with radii Rm and Rb, where the thread is winded.

One end of the thread is fixed with the disc, and on the other end of the thread the load is hung P (mass is m). Under the influence of the falling load P the thread is unwinded from the disc and causes rotational uniformly accelerated motion of the cross.

If falling of the load will go on with acceleration a, then pendulum apparatus will rotate with angular acceleration ε. Measuring the time t during of which the load P will pass the distance h from the rest state, it is possible to find a load acceleration:

(1)

which is connected with angular acceleration ε by means of correlation

(2)

where R is disc radius.

If to indicate straining force of the thread by means of T then rotational moment of the force influencing on the disc, will be defined according to the formula:

(3)

The force T can be found in the equation of P load translational motion:

(4)

Hence

(5)

Substituting (5) into (3) we receive calculating formula for the moment of force

(6)

Having constructed a graph of M forces moment law from angular acceleration ε moment of inertia of pendulum apparatus with JL loads will be got. Compare derived value of the second moment with theoretical one, which can be calculated according to the following formula:

(7)

where J0 is total inertia of two-stage disc, axis and plug of the cross (moment of inertia of the system without loads m0), its value is shown on the device; L is the distance from the axis of rotation to the centre of load m0; m0 is the mass of one moving load; l is the forming cylinder, the form of which m0 loads have; r0, rc are internal and external radii of the cylinders.

Work sequence

  - To displace movable supporting arm to the height h pointed by the teacher. To install it in the way that the loads falling freely could pass through the mid of the working door of photoelectric sensor.

  - To install moving loads on the rods in divisions pointed by the teacher and determine the distance L from the axis of rotation to the center of the load.

  - To measure diameters of two-stage disc with a beam compass and define radii of RL and Rb discs.

  - To switch on line cord into power system. Press the key ‘Start’

  - To press the key ‘Power’, checking whether all the indicators of measuring device show zero, and whether indicators of both photoelectric sensors illuminate.

  - To fix up with the thread, winded on the disc of Rm radii, an initial load with the mass m.

  - To move the load to the upper position, winding the thread on the chosen disc set up lower edge of the load exactly with the frame line of upper photoelectricsensorand push up the key “Start”. To check whether there is locking.

  - To press the key “Reset”. To press the key “Start”.

  - To count out the measured load falling time on the way h.

  - To press the key “Reset”.

  - To repeat measurement 3-5 times and determine average value of h pass way time of the load according to the formula:

(8)

where n is amount of carried out measurements;ti is value of the time, measured in i – that measurement.

  - To calculate linear acceleration a according to the formula

(9)

and angular acceleration t according to the formula

(10)

where R is a disk radius.

  - To calculate moment of force according to the formula

(11)

  - To repeat points 6-13 for 2-3 loads of other mass.

  - To draw a graph of law ε=f(M) according to calculated values ofMiand εi. Along axis of ordinates the value ε, along axis of abscissa moment of force are singled out. However, make sure that the received dependence is linear (Fig.2).

  - To define from the graph moment of inertia of the system  and  moment of frictional force along the axis of rotation and methods of determination shown in the figure 5.

  - To repeat the points 6-16 for block with the radius Rb. Choosing the scale for the values M and ε, it is possible to plot a graph of dependence ε=f(M) for small and large blocks in one system of coordinates and be convinced that they are on one straight line.

  - To define the system of moment of inertia by calculating according to the formula

(12)

  - The value of moment of inertia J0 of pendulum without loads is pointed on the device J0=0.265∙10-3kg∙m3

Figure 2.

  - Check up the convergence of experimentally and theoretically received values of  system inertia moments according to the proportion:

(9)

  - To include all the data into the table 1

  - Table 1.

| № | R, m | h, m | m, kg | ti, s | tav, s | a, m/s2 | M, N•m | ε, rad/s2 | JE, kg·m2 | JT, kg·m2 | К, % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Rm= |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Rb= |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |

Control questions

- What is the rotational motion of the body?

- Write the formula of angular velocity and angular acceleration. Point out its direction and units.

- What is the relation between the linear and angular velocity? What is the direction of linear velocity?

- What is the rotational inertia of a point mass and of a continuous object? Whatdoesitcharacterize?

- Formulate the Steiner theorem. Write the formula.

- What is the moment of the forces? What force does create rotational moment? What is the direction of the moment of the forces?

- Write and explain the basic law of dynamics for rotational motion.

- Describe the installation used in this work. What part of the system moves translationally and what part of the system is rotated?

- Is the graph  straight? Why?

References

1. Irodov I.Ye. Osnovnyye zakony mekhaniki. - M .: Vysshaya shkola. 1985g.

2. Savel'yev I.V. Kurs obshchey fiziki, kniga 1. Mekhanika. - M .: Nauka, 1998g.

3. Strelkov S. Mekhanika. - M .: Nauka. 1975g. 4. Trofimova T.I. Kurs fiziki. - M .: Vysshaya shkola. 1990g.

Laboratory work №5

The study of Laws of Oscillatory Motion by Physical Pendulum

The aim of the work: to study of laws of oscillatory motion in terms of physical pendulum; to determine the acceleration of gravity by reversible pendulum.

Materials: reversible pendulum, stop-watch register SSU-M; stop watch, fulcrum (support prism), beam compass (sliding calipers).

Theoretical introduction: A physical pendulum is a rigid body oscillating by the gravity around a horizontal axis not passing through its centre of gravity. Assume the rigid body (Fig.1) with mass of m freely rotates around the horizontal axis O perpendicular to a plane of the drawing. Distance from the mass centre C up to the axis O is l. At a deviation of the pendulum from equilibrium position on an angle  there is an angular momentum of the gravity aspiring to return the pendulum to its equilibrium position. As the moment of a force  and the angular displacement  are directed along an axis O, but to the opposite directions, the fundamental equation of dynamics of oscillatory motion has the next form

0

φ

l

L

С

К

mg

Figure 1

, (1)

where   = the angular acceleration;

I = the moment of inertia of the pendulum relative to an axis passing through the point of suspension O;

= the moment of gravity relative to the same axis.

At small angles of a deviation of a pendulum, then the formula (1) we can rewrite as:

or

.                                                     (2)

Introducing the next designation

(3)

We transform the equation (2) as follows:

.                                              	(4)

Thus, small oscillations of the physical pendulum are described by the differential equation (4) which solution is:

, 					(5)

where = the amplitude of oscillation, i.e. the maximum angle of deviation of the pendulum from the equilibrium position;

= circular frequency (number of oscillations in seconds);

= a phase of oscillation;

= a constant that represents the value of a phase at the moment of time t=0 and it is called an initial phase.

Hence, at small oscillations the angular deviation of the pendulum varies according to the harmonious law (by the laws of a sine or cosine). The basic property of oscillations is their periodicity, i.e. repeatability every Т time interval called a period. The phase of oscillations receives an increment in this Т time interval:

(6)

From formulae (6) and (3) it follows that at small deviations from the equilibrium position the period of a physical pendulum relative to an axis of rotation is defined by expression

,            		(7)

or                      	,                                               		(8)

where   =the effective length of a physical pendulum.

According to the Steiner theorem:

, 				(9)

where  = the moment of inertia of a body relative to an axis passing through the centre of gravity C (see Fig. 1) and the parallel axis O;

l = a distance between these axes. Then, the effective length of a physical pendulum will be equal to

(10)

Evidently from (10) we getL › l. If we measure along a line of OS a length ОК (Fig. 1) equal to the effective length of a physical pendulumL the obtained point K is calledthe center of oscillation. Then the pendulum with the weight concentrated at a point K (such pendulum is mathematical), will oscillate with the same period as the given physical pendulum.

The point of suspension O and the centre of oscillation K have property of reciprocity: at oscillating of a pendulum around the K pointanew the centre oscillation becomes the point O, at that the period of oscillations of a physical pendulum will not change. The definition of the gravitational acceleration by a reversible pendulum is based on this property.

Apparatus specification andreceivingcalculating formula. A reversible pendulum is a steel rod where fulcrumsO and O′ (Fig.2) and a steel load B placed between them are fixed rigidly. Another load A is at one of the ends of rod, it can move along the rod by the scale and be fixed in necessary position. Pendulum oscillations are observed suspending it by turns for the fulcrums. By replacing of the movable load A, coincidence of oscillation periods of the pendulum can be reached when points of suspension are edges of the fulcrums O and O′.  Let us write the value of the periods T1 and T2 in oscillation on the fulcrums O and O1:

,	(11)

,   	(12)

and       		,        	(13)

where Т=Т1=Т2; and ,  i.e. in this case effective length of the physical pendulum will be equaled to the distance between supports O and O′.

Actually it is difficult to get the precise equality of periods. Therefore the formulae (11) and (12) with the account of (9) will be written in the form:

А

В

l1

l2

0

0/

С

Figure 2

,         		(14)

.         (15)

Using these equations we obtain the next:

.

Whence    ,       (16)

where        .    (17)

The error of  can be in the result of the correlation

,                (18)

where .

Work sequence

- To introduce with the construction of a reversible pendulum;

- To fix the load A at the end division of the scale (2cm) from the part of the fulcrum O (Fig.2)  and suspend the pendulum for this support prism;

- To descend the pendulum from its equilibrium position on the small angle no more than 5 degrees (upper end of the rod must easily cross the beam of sensor device of counter) and after 3-5 complete oscillations turn on the stop watch register SSU-M and  manual stop-watch simultaneously.

- To measure the time t of  40 complete oscillations (according to the stop watch register 80 cycles) and determine  period of oscillations by the formula:

(19)

- Moving the load A along the scale in 2-3 cm find out subsequently oscillation periods for every position for every load as it is in the points 2-4 (no less than 6-8 positions of the load).

- To turn over the pendulum and hang it on the second prism O′; to repeat the experiments in accordance with the points 3-5, displacing the load A in the same positions.

- Plot diagrams of dependence of the oscillation period Т on the scale division appropriate to the A load position. To find the point of crossing of curves that corresponds to the A load position with the most approximate values of the periods at oscillating relative to both supports. Tabulate the results of measurements into the table 1.

Table 1.

| n | d, cm | Oscillating on О |  | Oscillating onО/ |  |
| --- | --- | --- | --- | --- | --- |
|  |  | t, s | T, s | t, s | T, s |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 … |  |  |  |  |  |

- To fix the load A in the position found by the diagram. To define the distances  and .  Here put the pendulum horizontally on the special fulcrum and get its equilibrium. To determine this equilibrium point and measure unknown distances  and (Fig.2);

- Suspending the pendulum for one and another prisms (not changing the A load position), to determine corresponding oscillation periods Т1і and Т2і.To determine each periodno less than 3 times and within 100 complete oscillations (200 cycles);

- To figure out the gravitational acceleration by the formula (16). Т1 and Т2are average arithmetic values of three measurements in this formula;

- To calculate error of measurements by (18). For determination of  it is necessary to use standard deviation and confidence probability α, i.e.

,                                 		(20)

where  is Student coefficient. Standard deviation is equal to

.                                   (21)

- To include all measuring and calculating results into the table 2.

Table 2.

| n | d, cm |  |  | Oscillating onО |  |  | Oscillating onО/ |  |  | m/s2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | t1,s | T1,s | ,s | t2 ,s | T2,s |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

Control questions

- What is oscillation?

- Give definitions of amplitude, phase, period, frequency, cyclic frequency of oscillations.

- What is harmonic oscillator?

- What body is physical pendulum?

- Give definition of the moment of forces and the moment of inertia.

- Specify direction of the moment of gravities and turning angle.

- Write the equation of motion of physical pendulum.

- At what simplifying assumptions is the formula of the period of physical pendulum (3.9) derived?

- What is the period of oscillations of physical pendulum equal to?

- What quantity is the effective length of physical pendulum called? What physical meaning has the effective length?

- Formulate the Steiner theorem.

- What is the essence of the method of reversible pendulum?

- Tell a construction of the reversible pendulum used in the given work.

References

1. Irodov I.Ye. Osnovnyye zakony mekhaniki. - M .: Vysshaya shkola. 1985g. 2. Savel'yev I.V. Kurs obshchey fiziki, kniga 1. Mekhanika. - M .: Nauka, 1998g.

MOLECULAR PHYSICS AND THERMODYNAMICS

Laboratory work №6

Determining Viscosity Coefficient of the Liquid by Stokes Method

The aim of the work: determining liquid viscosity by velocity of ball falling (Stokes method).

Materials: viscosimeter; stop-watch; micrometer; collection of steel balls (with the diameter of about 1mm); measuring line.

Theoretical introduction. Viscosity or internal friction is the property of real liquids to offer resistance to displacement of one part of a liquid relative to another.

L2

L1

mg

FA

Ffr

Fig.1

The cause of resistance due by the liquid viscosity is explained as follows. We imagine two plates A and B (Fig.1) divided into a plane-parallel liquid layers. We will consider what happens if to start displacing the top plate A relative to the bottom B in the direction shown by the arrow. Mentally we divide a liquid into the thinnest layers. Liquid molecules the nearest to the movable plate A stick to it, because of it they start to move together with this plate with the same velocity. These molecules, in their turn, carry the following layers, and etc.

The layer of molecules, directly adjoining to the bottom motionless plate B, remains at rest, but other layers move sliding each on another and their velocities the greater, the greater their distance from the bottom layer. The viscosity is developed by the force influence hampering the relative shift of the adjoining liquid layers, hence, and the shift of plates relative to each other.

The resistance caused by viscosity depends on the difference of velocities between its layers and the distance between them. The change of the liquid velocity at transition from layer to layer the greater, the greater the viscous resistance.

To characterize the velocity change we determine the difference of velocity of two liquid layers  and the distances between these layers counted on normal to direction of the velocity. The limit of ratio of these quantities is the velocity gradient:

(1)

The velocity gradient  shows how velocity quickly varies at transition from one layer to another in a direction of  perpendicular to the direction of layers motion.

At laminar flow (i.e. the flow without turbulences) the module of the internal frictional force is proportional to the velocity gradient

,							(2)

or

(3)

where F = internal frictional force;

S =area of a surface of liquid layers sliding each on another;

f =internal frictional force acting on unit area of the surface of a layer (a tangential exertion);

η=constant of proportionality depending on the liquid nature named the dynamic viscosity (or viscosity).

From Eq.(3) (the Newton’s law) we see that the dynamic viscosity equals an internal frictional force acting on unit area of the surface of a layer at the velocity gradient equal to unit.

Unit of viscosity in SI is pascal-second.

Viscosity depends on temperature, and the character of this dependence for liquids and gases is various (for liquids it is decreased with the increasing temperature, in gases, on the contrary, it is increased), that points to the distinction of mechanisms of the internal friction in them. Especially, the viscosity of oils depends on temperature very much. For example, the castor oil viscosity in the range of 18-400C falls twice.

Installation description. Viscosimeter represents glass cylindrical vessel installed vertically that is used for determining liquid viscosity by Stokes’ method (Fig.1).

Vessel is filled up with studied liquid (glycerine, oil). Vessel diameter and its length are given on the installation. On the walls of the vessel two marks (A and B), in the distance of l from each other, are written. The upper mark (A) is placed lower of the fluid level in the way that the ball velocity manages to be fixed to the moment of passing this mark.

We choose some metal balls with known density and measure time of balls passing of the distance between marks A and B at falling in a liquid.

The ball in a liquid (Fig.3) is acted by forces of:

the gravity of

(4)

the Archimede’s law of

(5)

the internal frictional force of

,							(6)

where is the density of  a ball material;  is a density of a liquid; is a radius of a sphere;  is a dynamic viscosity; is a velocity of the established ball motion in a liquid.

In the established motion, when the acceleration of a ball is equal to zero:

(7)

Projecting all forces on a vertical axis and substituting in (7) expressions for forces, we obtain

(8)

The velocity of a ball can be defined over time  of a path passing

(9)

and then we have

(10)

or it is written down in diameter of a ball as:

(11)

Work sequence

- To check vertical position of viscosimeter installation

- To pick over 5-10 balls, measure their average diameters  with the help of micrometer [For each ball are recommended to be measured several various diameters and identify their average value. Such averaging is reasonable, as the balls, the form of which may differ a bit from spherical ones, are used in the process of work].

- To measure the distance l between the marks A and B

- Having taken g, ρb, ρf from the handbook, find out C according to the formula (11)

- To measure the time  of the first ball passing the distance. For this purpose to turn on a stop watch at the moment of passing the mark A by ball and turn off a stop watch at the moment of its passing by the mark B.

- To calculate auxiliary parameter  by the formula

(12)

- To repeat the experiment (4-6 points) for other balls.

- To find average arithmetic value of auxiliary parameter

(13)

- To write down the formula (11) through auxiliary parameter

(14)

and calculate the average value of liquid viscosity, where the free falling acceleration is .

- To determine relative error according to the formula

(15)

where =an error of  a ball density; =an error of a liquid density; =an error of parameter.

The  magnitude is necessary for determining as standard error of multiple direct measurements of parameter

(16)

where = Student coefficient at

- To figure out absolute error of viscosity according to the formula

(17)

- To write the final result of unknown quantity with showing of confidential interval as:

(18)

- To enter result of measurements and calculations into the table 1

Control questions

  - What does the phenomenon of internal friction consist in? Write the Newton’s formula.

  - What is the velocity gradient?

  - Explain physical meaning of constant of internal friction (viscosity). Specify its unit.

  - Explain dependence of viscosity on temperature

  - What does the essence of the Stokes ‘methods consist in?

  - What motion is laminar?

  - What forces act on a ball moving in liquid? Specify direction of these forces and development of their formulae

References

1. Irodov I.Ye. Osnovnyye zakony mekhaniki. - M .: Vysshaya shkola. 1985g.

2. Savel'yev I.V. Kurs obshchey fiziki, kniga 1. Mekhanika. - M .: Nauka, 1998g.

3. Strelkov S. Mekhanika. - M .: Nauka. 1975g.

4. Trofimova T.I. Kurs fiziki. - M .: Vysshaya shkola. 1990g.

Laboratory work №7

Determination of Reynolds number

The aim of the work: determination of Reynolds number at a laminar flow of a moving sphere.

Materials: a glass cylindrical vessel with a fluid; a stop-watch; a measuring ruler; a micrometer; a set of steel spheres.

Installation description: For measurement the glass cylindrical vessel filled with a fluid (glycerin, oil) and vertically set is used (Fig. 1). On vessel walls there are two mark points (A and B), on distance l from each other. The top mark point (A) is located below level of a fluid with such calculation, that sphere velocity by the moment of travelling of this mark reaches limit (ultimate) velocity.

Theoretical introduction: For description of sphere motion in a viscous fluid it is convenient to introduce Reynolds number

Re=ρflvrη							(1)

where η is the coefficient of the internal friction or the viscosity of the fluid (in brief the viscosity), ρfl is the density of the fluid, v is the velocity of the sphere relative to the fluid, r is the radius of the sphere. At a laminar flow of the sphere by the fluid (at low Reynolds numbers, Re<1) resistance force Fs is defined by Stokes’ law

Fs=6πηrv							(2)

| Figure 1. |  |
| --- | --- |

As is known resistance force Fs of the medium is directed against velocity of the body motion v. On a sphere falling in a fluid Archimedes’ force FA and gravitational force FG also act. Archimedes’ force is equal to mass of the displaced fluid and is directed upwards against free fall acceleration g

FA=ρflVg							(3)

where  V=43πr3 is the volume of the sphere. The gravitational force acting on a sphere, is defined by expression

Fg=43πr3ρgρ						(4)

Where ρ the density of the sphere is (43πr3ρ=Vρ=m is the mass of the sphere). According to Newton's second law the vector sum of these forces is equal to product of mass m of a sphere and its acceleration (a=dυdt)

ma=Fg+FA+FS					(5)

We take a projection of the equation (5) on an axis directed vertically downwards on g, and taking into account of the equations (2) - (4) we rewrite the formula (5) in the form of

43πr3ρdυdt=43πr3ρ-ρflg-6πηrv		(6)

The first summand on the right in the equation (6) is called as effective weight of a sphere in a fluid. Velocity of a sphere increases until increasing resistance force is equal to effective weight of a sphere. Thus acceleration becomes equal to zero and growth of velocity stops. This maximum value of velocity υstis called as the limit (ultimate velocity) velocity. It can be found from the equation (6), taking dυdt=0 (the sphere moves uniformly):

υst=29gr2ρ-ρflη=118d2ρ-ρflηg			 (7)

The exact solution of the equation (6) under a condition υ0=0 (that is usually done with sufficient accuracy) has a form of

υt=υlim1-e-tτ,					(8)

where

τ=118d2ρη.							(9)

Velocity of a sphere υt increases from 0 at t=0 to limit value of υlim by exponential law. Therefore it is possible that velocity reaches limit value during time (exponent in the formula (8) becomes equal 1). The quantity is called as relaxation time, settling time υlim. Limit velocity of the sphere υlim is defined, measuring distance between marks l using a ruler, and falling time t using a stop-watch

υlim=lt						(10)

Knowing relaxation time , it is easy to calculate a relaxation path Sr. Integrating (8), we obtain

St=τυlimtτ-1+e-tτ.			(11)

Evidently from (11), at t = 

Sr=τυlime=lτet,					(12)

where eis the exponent. I.e., the distance between a surface of a fluid and the top mark (A) is defined by an inequation

SA≫Sr=lτet					(13)

In experiment it is necessary to check up how well the given inequation is fulfilled.

From the equation (7) with the account (10) we obtain expression for viscosity

η=g(ρ-ρfl)d2t18l					(14)

The calculating formula for determination of Reynolds number after substitution of equations (10) and (14) in equation (1) is given by

Re=Cdt2,						(15)

where

C=9ρfll2g(ρ-ρfl).					(16)

Values of quantities g, ρ, ρflare taken from the directory; diameter of a sphere d is measured by a micrometer. It is necessary to underline, that the technique of Reynolds number determination described above is correct only in the field of application of Stokes formula (Re<1).

Work sequence

- Select 5 - 10 spheres and using a micrometer measure their average diameters. For each sphere it is recommended to measure various diameters and to calculate average value. Such averaging is feasible, as in work the spheres which form can differ a little from the spherical one are used.

- Measure distancel between marks A and B and distance SA between border of a fluid and a mark A.

- Find C by the formula (16), taking g, ρ, ρflfrom the directory.

- To measure time t1of travellingof distancel by the first sphere. For this purpose to turn on a stop-watch at the moment of travelling of a mark A by a sphere and to turn off at the moment of travelling of a mark B by a sphere.

- To calculate by the formula (15) Reynolds number.

- To define a relative error  by the formula

ε=ΔReRe=D+Δdd2+2Δtt2,			(17)

where

D=2Δll2+Δgg2+Δρflρfl2				(18)

Then by the formula

ΔRe=Re∙ε					(19)

define an absolute error. Specify confidential interval Re±ΔRe.

- By the formula (9) to define relaxation time τ.

- Define the ratioSA/Sr(calculate Srby the formula (12)).

- To repeat experience (point 4) and calculations (points 5-8) for other spheres.

- To write the results of measurements and calculations in table 1.

Table 1

| № | l | SA | C | di |  | ti |  | Re |  |  |  | Re±ΔRe |  |  | Sr | SA/Sr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Control questions

- Specify types of fluidflows and characteristic of flows.

- Physical meaning of Reynolds number.

- Formulate laws of hydrodynamic similarity.

- Under the action of what forces does the sphere move in a fluid?Give determination of these forces.

- Explain occurrence of limit velocity at motionof a sphere in a fluid.

- Deduce the formula for determination υlim and calculate it by the formula (7).

- Give determination of the phenomenon of viscosity. Write the fundamental law of a viscous friction.

- What is the gradient of velocity?

- Physical meaning of viscosity coefficient, and a measure of viscosity.

- Molecular-kinetic theory of the phenomenon of viscosity.

References

1. Savel'yev I.V. Kurs fiziki. Tom 1. Izd. M .: Nauka, 1998

2. Dzhankoli D. Fizika. Tom 1. Izd. M .: Mir, 1989

3. Radchenko R .. Molekulyarnaya fizika. M. Vysshaya shkola, 1976

4. Kikoin I.K. Kikoin A.K .. Molekulyarnaya fizika.-M .: GIFM, 1983.

5. Telesnin R.V. Molekulyarnaya fizika.-M .: Vysshaya shkola, 1980.

6. Fizicheskiy praktikum pod rukovodstvom Iveronovoy V.I. M., Nauka, 1968

7. Maysova N.N. Praktikum po kursu obshchey fiziki. M., «Vysshaya shkola», 1970 8. Sivukhin D.V. Termodinamika i molekulyarnaya fizika. Tom 2. Izd. M .: Nauka, 1975

9. Trofimova T.I. Kurs fiziki.-M .: Vysshaya shkola, 1990.

10. Feynman R. i dr. Feynmanovskiye lektsii po fizike. Tom 7. Izd. M .: Mir, 1977 11. Kabylbekov K.A. Molekulyarnaya fizika i elektrichestvo. Izd. Kar.GU, 2002.

Laboratory work №10

Determination of fluid friction force by Stokes formula

The aim of the work: to define force of a fluid (viscous) friction at body motion with a limit velocity.

Materials: a glass cylindrical vessel with a fluid; a stop-watch; a measuring ruler; a micrometer; a set of steel spheres.

Installation description. For determination of a viscous friction force the glass cylindrical vessel filled with a fluid (glycerin, oil) and set vertically is used (Fig.1). On vessel walls two marks (A and B), on distance l from each other are marked. The top mark (A) is located below level of a fluid with such calculation, that velocity of a sphere by the moment of travelling of this mark reaches limit velocity.

Figure 1.

Theoretical description. Resistance force acts on a body moving in the fluid. It depends on the shape of a moving body and the property of a fluid named viscosity. For the description of body motion in a viscous fluid it is convenient to introduce Reynolds number

(1)

where η is the internal friction coefficient or fluid viscosity (in brief the viscosity), ρflis the fluid density, υ is the velocity of a sphere relative to the fluid, r is the characteristic quantity of an body (for example, radius for a body of the spherical shape). At low Reynolds numbers (Re <1, i.e. at small velocities of motion and small r) for spherical bodies  a resistance force of fluid friction FS it is defined by Stokes formula

FS=6πηrυ=3πηdυ, 					(2)

where d is the diameter of a sphere. As is known resistance force FSof environment is directed against velocity of body motion υ. On a sphere falling in a fluid, Archimedes force FAand gravity force FG also act. Archimedes force is equal to mass of the forced out fluid and is directed upwards against free fall acceleration g

FA=ρflVg					(3)

whereV=43πr3  is the volume of the sphere. The gravity force acting on a sphere, is defined by expression

FG=43πr3ρg						(4)

where ρ is the density of the sphere (43πr3ρ=Vρ=m is the mass of the sphere). According to Newton's second law the vector sum of these forces is equal to product of mass (m) of a sphere on its acceleration (a=dυdt)

FS+FA+FG=ma					(5)

We take a projection of the equation (5) on an axis directed vertically downwards along g, and taking into account of equations (2) - (4) we rewrite formula (5) in the form of

(6)

The first item on the left in the equation (6) is called as effective mass of a sphere in a fluid. Velocity of a sphere increases until increasing resistance force is equal to effective mass of a sphere. Thus acceleration becomes equal to zero and growth of velocity stops. This maximum value of velocity υlim is called as the limit velocity. It can be found from equation (6), taking dυdt=0 (the sphere moves further uniformly):

(7)

The exact solution of the equation (6) at condition of υ0=0 (that is usually fulfilled with sufficient accuracy) looks like

(8)

Velocity of a sphere υ(t) increases from 0 at t =0 to limit value υlim in exponential law. Therefore it is possible to decide, that velocity reaches limit value for a period of time τ(exponent indicator in the formula (8) becomes equal to1). The quantity τis called as relaxation time, setting time υlim. Limit velocity of sphere υlim  is defined, measuring distance between marks (l) with a ruler, and falling time (t) with stop-watch

(10)

Knowing relaxation time τ, it is easy to calculate a relaxation path Sp. Integrating (8), we obtain

(11)

Evidently from (11), att =τ

(12)

where eis the exponent. Thus the distance between a surface of a fluid and a top mark (A) is defined by an inequation

SA≫Sp=lτet							(13)

In experiment it is necessary to check up how well the given inequation is fulfilled.

From equation (7) taking into account (9) we obtain the calculation formula for viscosity coefficient

η=Cd2t, 							(14)

where

C=g(ρ-ρfl)18e						(15)

The values of quantities g, ρ, ρflareundertaken from the directory, the diameter of a sphere (d) is measured by a micrometer. The technique of determination of viscosity described above is correct only in the field of applicability of Stokes formula (1). If the calculated values of viscosity (η) by the formula (14) do not depend on sphere radius it specifies in applicability of Stokes formula.

Work sequence

- To check up vertical position of installation of viscosimeter.

- Select 5 - 10 spheres and using a micrometer measure their average diameters. For each sphere it is recommended to measure some various diameters and to calculate average value. Such averaging is expedient, as in work the spheres which shape can differ a little from spherical are used.

- Measure distance l between marks A and B.

- By the formula (15), taking g, ρ, ρflfrom the directory, find C.

- To measure time t1 of passed distance l by the first sphere. For this purpose to turn on a stop-watch at the moment of passing by a sphere of a mark Aand to turn off at the moment of passing by a sphere of a mark B.

- To calculate by the formula (14) viscosity coefficient of the fluid.

- By the formula (8) to define relaxation time τ. Then using a stop-watch to define time of passing Т by a sphere of distance from a surface of a fluid to a mark A and to be convinced, that Т is at least several times more than τ.

- To repeat experience (points 5-7) for other spheres.

- To find average arithmetic value of viscosity of the fluid and to define a relative error ε by the formula

(16)

where (n is the number of experiences). 			(17)

ΔК is a parameter K error, we define as an absolute error of multiple direct measurements of parameter K

(18)

where n (the number of experiences) and α (the confidential probability)is given by the teacher.

- To calculate an absolute error of viscosity by the formula

(19)

and to specify a confidential interval of the given quantity ηav±Δη.

- To write the results of measurements and calculations in table 1.

| № | l | C | di | ti | ηi | ηav | Ki | Kav | ηav±Δη | ε | τi | Ti |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |

Control questions

  - Give determination of the phenomenon of viscosity. Write the fundamental law of a viscous friction.

  - What is the gradient of velocity?

  - Physical meaning of viscosity coefficient, and a viscosity unit of measure.

  - Molecular-kinetic theory of the phenomenon of viscosity.

  - Characteristics of flow. Determination of a laminar flow.

  - Physical meaning of Reynolds number.

  - Deduce the equation of a sphere motion in the fluid.

  - Deduce the formula for determination ϑlim.

  - The reasons of occurrence of frontal resistance of an body, moving in a fluid.

- Practical application of Stokes method.

References

1. Savel'yev I.V. Kurs fiziki. Tom 1. Izd. M .: Nauka, 1998

2. Dzhankoli D. Fizika. Tom 1. Izd. M .: Mir, 1989

3. Radchenko R .. Molekulyarnaya fizika. M. Vysshaya shkola, 1976

4. Kikoin I.K. Kikoin A.K .. Molekulyarnaya fizika.-M .: GIFM, 1983.

5. Telesnin R.V. Molekulyarnaya fizika.-M .: Vysshaya shkola, 1980.

6. Fizicheskiy praktikum pod rukovodstvom Iveronovoy V.I. M., Nauka, 1968

7. Maysova N.N. Praktikum po kursu obshchey fiziki. M., «Vysshaya shkola», 1970 8. Sivukhin D.V. Termodinamika i molekulyarnaya fizika. Tom 2. Izd. M .: Nauka, 1975,

Laboratory work № 11

Determination of a fluid surface tension coefficient

by a breaking method of ring

The aim of the work: Studying of the phenomenon of a surface tension of a fluid; determination of surface tension coefficientof a fluid by force of breaking method of ring

Materials: metal rings; weigh-scales with set of weights; a sliding calipers; examined fluid

Figure 1. A molecule at the surface of a fluid is attracted into the bulk fluid, which tends to reduce the fluid’s surface area.

Theoretical introduction.The fluid surface is in special conditions in comparison with its rest mass, characterized by excess potential energy. These special conditions arise because of nonequivalence of intermolecular interactions of the molecules which are at a surface of a fluid and in its interior. For example, molecule A, being in a fluid (Fig.1), is surrounded by molecules of the same fluid from different sides. Forces with which neighboring molecules act on a molecule A, are directed in different sides, therefore theresulting acting one on a molecule in a fluid from other molecules is equal to zero. Otherwise it deals with other molecules (molecule B and C), being in a surface layer of a fluid. In this case the molecules which are in a thin layer near to a surface of a fluid, are surrounded by molecules of the same fluid not from all sides, i.e. a part of co-acting molecules are molecules of gas over a fluid. As concentration of molecules in located over a gas fluid is small in comparison with their concentration in a fluid resultantF, applied to each molecule of a surface layer, is not equal to zero and is directed in a fluid.

Total energy of fluid particles is the sum of kinetic energy of their heat motion and the potential energy caused by forces of intermolecular interaction. For moving of a molecule from interior of a fluid in a surface layer it is necessary to do work against acting forces in this layer. This work is done by a molecule due to kinetic energy storage and goes on increase in its potential energy. Therefore molecules of a surface layer of a fluid possess large potential energy, than particles in a fluid. This higher energy which molecules in the fluid surface layer possess, called surface energy, it is proportional to the area of a layer∆S:

(1)

where δ is the surface tension coefficient defined by additional energy of unit of a surface layer.

Because of presence of surface energy the fluid surface aspires to be reduced. Therefore for increase in a fluid surface due to transition of a quantity of molecules from interior of a fluid in a surface layer, it is necessary to do work against forces of a surface tension. The increase in a surface of a fluid is accompanied by negative work, and reduction isby positive. Therefore

∆A=-Fl∙∆S						(2)

where Flis a surface tension force acting on line of unit length of a fluid surface.

This work is done simultaneously with increase in surface energy, i.e.

∆A=∆E-Fl∙∆S					(3)

From comparison of expressions (1) and (3) it is visible, that

δ=Fl							(4)

i.е. the surface tensioncoefficientδ is equal to a surface tension force measuredper unit length, bounding a surface. Forces of a surface tension are tangential to the surface but perpendicular to the line on which they act. Unit of a surface tension coefficient in SI unit isnewtonper metre (N/m), or joulesper square metre (J/m2).

The majority of fluids at temperature 300К have a surface tension of an order  of 10-2-10-1N/m. The surface tension with raise in temperature decreases, as average distances between fluid molecules increase. The surface tension also essentially depends on the mixtures which are available in fluids. It speaks as follows. Surface tensionδ therefore also surface energy of a fluid depend onintermolecular forces which in turn are defined by a structure of molecules. The minimum of fluid surface energy is reached not only by reduction of its surface, but also by surface layer completion by such molecules that quantity δ was minimum. The substances adsorbed in a surfacearea with corresponding decrease of surface energy, are called as surface-active. Their addition even in small concentration considerablydecreases a surface tension. Molecules of such substances first of all enter in a surface layer and only after it is filled with them densely enough, get into the basic volume of a fluid.

Molecules of organic substances havingoblong shape (for example, –ОН, -СООН, -NH2) usually belong to surface-active substances. The most known example of surface-active substance is soap.

There are also substances (salt, sugar) which addition to water raises its surface tension.

If the length l limiting the fluid surface, surface tension forces are defined from the following ratio:

(5)

Installation description and obtaining of the calculation formula. The essence of breaking method of ring consists that forces are measured, which are necessary for putting to tear apart a thin metal ring from a fluid surface.breaking method of ring

Experimental installation represents technical weigh-scales (Fig. 2) which have one beam, except a pan, is horizontally hung up (weighed) by the thin aluminium ring. The vessel with the fluid is located on a special little table under the ring.

If to bring a vessel under the ring so that its bottom to touch a fluid surface, the ring as though sticks. For a separation of a ring from a fluid surface to apply force F. The break occurs on two circles which diameters D and d are equal to external and internal diameters of a ring (Fig.3). The total length of break lines is equal:

(6)

| Figure 2 Figure 3 |  |
| --- | --- |

Indicating a thickness of ring walls through h, we have:

d=D –2h						(7)

Substituting the last in expression (6) instead of d, we find:

(8)

On the basis of (5) taking into account (8) we obtain the equation by which we calculateδ:

,						(9)

where F=mg is equal to set of weights, necessary for ring separation.

Work sequence

1. To measure by a trammel external diameter D and a thickness of ring walls h.

2. To hang up a ring on the left arm and to balance weigh-scales by set of weights (the mass of the set of weights is not included into calculations).

3. To place under a ring a vessel with the fluid. To put a fluid on a ring touchitsbottom in a fluid surface area. Strictly to observe horizontal position of a ring.

4. To load the right cup of weigh-scales cautiously with set of weights until the ring is separated from fluid surface. The mass of the set of weightsР is equal to force of a surface tension:

F=P=mg						(10)

Where m is the  mass of set of weights, necessary for ring break, g=9.8 m/s 2 is the free fall acceleration.

5. To calculate  a surface tension coefficient of the fluid by the formula (9).

6. To repeat experience (points 1-5) for 2-3 rings of other diameters with the same fluid.

7. To calculate an average arithmetic value of coefficientof a surface tension:

δcp=1Ni=1Nδi=1Nδ1+δ2+…+δN		(11)

8. To calculate a relative error of coefficient of a surface tension under formulas

(12)

- To calculate absolute error∆δ by a ratio

(13)

- To write results of measurements and calculations in table 1.

Table 1.

| № | D, m | h, m | , m | , kg | , kg | ,N | δ, N/m | , N/m | , % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  |  |  |

Control questions

- Explain, what circumstances cause the phenomenon of a surface tension of the fluid?

- What is a surface tensioncoefficien? Explain its physical meaning. Specify unit of coefficient of a surface tension?

- How are the forces of a surface tension directed?

- Write the expression for the force acting on a line.

- How does the coefficient of a surface tension depend on temperature and surface-active substances?

- How is the coefficient of a surface tension defined in the given work?

- Tell the conditions of ring separation.

- What phenomena are spoken about a surface tension of a fluid?

References

1. Savel'yev I.V. Obshchiy kurs fiziki. t.1,2. Almaty, Mektep, 2002.

2. Radchenko R .. Molekulyarnaya fizika. M. Vysshaya shkola, 1976

3. Kikoin I.K. Kikoin A.K .. Molekulyarnaya fizika.-M .: GIFM, 1983.

4. Telesnin R.V. Molekulyarnaya fizika.-M .: Vysshaya shkola, 1980.

5. Sivukhin D.V. Obshchiy kurs fiziki. M .: Nauka, 1979.

6. Trofimova T.I. Kurs fiziki.-M .: Vysshaya shkola, 1990.

ELECRICITY AND MAGNETISM

Laboratory work №12

Modeling of Electrostatic Field

The aim of the work: study of electrostatic field, finding of equipotential surfaces of the field.

Materials: special installation, galvanometer, the direct current source

Theoretical introduction: It is well known the electric field is around any charge or charged body. In this field the Coulomb’s force acts on the other entered charge.

The electric field is characterized at every point by the next magnitudes: a vector of the  electric strength and the  potential.  is the force characteristics and  is energy one.

The electric field strength is the physical magnitude which numerical equal and coinciding with direction of the force  acting from the side of field on a positive point charge  placed at the considered point

(1)

The electric field potential at point is the scalar magnitude numerically equal to potential energy  of a unit positive point charge  placed at the considered point

(2)

Relation between the electric field potential and the strength is defined by the next equation

(3)

We remind you that a gradient of any scalar  quantity is the vector, and its direction coincides with a direction of the rapid increase of the quantity. Its module equals to the  difference at a replacement on a unit length in direction of the rapidest  difference. In (3) formula “-” sign shows that the electric field strength is directed to the potential decreasing side.

To represent the electric field strength at any point and the field potential distribution visually we use concepts of the force lines and the equipotential surfaces.

The force lines are lines the tangents of which at every point define the electric field strength direction. The number of that force lines crossing a unit area normal to that lines, determines the electric strength magnitude at the center of this area. The electric force lines are unclosed. They begin from a positive charge and finish at a negative charge.

Locus of the field whose potentials are the same is called equipotential surfaces.Equipotential surfaces are plotted in that way that a potential difference for two adjacent surfaces is the same everywhere. It permits by density of the equipotential surfaces to get a physical meaning of the field strength magnitude.

The electric field strength vectors, and consequently, the force field lines are normal to the equipotential surfaces. In figure 1 the force lines of a field are imagined by smooth line, and the equipotential surfaces are by dash lines.

It is known that the experimental and theoretic studies of the electric field potential distributions are simpler than the field strength definition. Therefore, in this work the potential distributions of not varying with time electrostatic field are defined. The force lines of a researched field can be drawn as curves orthogonal to experimentally received surfaces of equal potentials.

The potential distributions of an electrostatic field can be studied by the method of probes consisting in the following: an additional electrode –probe, arranged so that it marginally destroys the investigated field by its present and accepts a potential of the field point where  was placed, is placed at researched point. At these conditions the appliance connected with probe by a conductor will show good drawing of potential distributions in the researched field.

Complexities of this work with probes, and generally, the difficulties of the electrostatic measurements led to a special method development of the electrostatic field studying by an artificial reproduction of their structures in the conducting medium through which continuous current is passed. By such way the direct study of an electrostatic field is replaced by a study of its precise and more convenient model. It is known that in a condition of the potential distribution in the medium through which current is passed between the arranged (set) electrodes can be changed by the potential distribution between the same electrodes when the field in a vacuum or in a homogeneous dielectric is between them. Then the study of the electrostatic field between systems of charged conductors can be changed by the study of the electrostatic field of a continuous current between the same systems of conductors.

The described investigation method which is called the electrostatic field modeling has a wide application in practice.  This method is useful for a study of complex electrostatic fields; precise calculation is difficult because of boundary conditions (multielectrode radio valves, electrostatic lenses, photoelectron multipliers and etc.).

Installation description:   The scheme of experimental installation is shown in                       Fig.2. It presents a bridge scheme consisting of the electrodes A and C, there is investigated field between those electrodes, rheochord R, null galvanometer G conducted with the probe Z and current source - rectifier B. Voltage upon the electrodes A and C is given from the rheochord R connected on the rectifier output B.

Current-conducting paper, contour lines of which are shown in the figure 2 with dashed lines, serve as environment allowing to model electrostatic field.

Principles of the installation work can be explained from the next considiration: moving of the rheochord’s slider, this slider can be given different potential values relative to the electrodes. Availability or absence of current in a circuit depends on that at what point the probe is placed. If it is at the field point where potential equals to fixed potential of rheochord’s slider then the current is absent in the galvanometer circuit.

The locus of a field when there is no current in the galvanometer circuit current forms the equipotential surface in the investigated field. The  potential of this surface relative to the C electrode can be defined as follows

(4)

where  is the voltage upon the output, is the number of the rheochord scale divisions,  is scale divisions on which arranged slider.

The installation is supplied by a special device - coordinator allowing by a rotation of coordinator’s handles to replace the probe, and also to count coordinates of found equipotential surface points out.

Work sequence

- To check the scheme of experimental installation.

- To cut rectifier in a network 220 V.

- To fix the reachord’s slider in the distance of 8-10cm from one of the terminals of rheachord. To determine potential of the slider according to the formula (4).

- Replacing the probe by rotation of the pen of coordinator to find the field points for which galvanometer current equals zero. Amount of these points must be enough for drawing the lines of equipotential surface (7-10 points). Coordinates of these points are counted out by the coordinator line.

- To plot the found points on the millimeter paper and connect them by smooth line. To write its potential along line.

- To repeat 4, 5 points having changed rheochords position. The amount of experimental repetition is  4-5 times

- To plot the field lines using found equipotential surface lines. Here it should be taken into consideration that at the points of crossing of lines the equipotential surfaces and the force lines must be perpendicular to each other.

Control questions

- What quantities is the electric field characterized by? Give the definitions of these quantities and write these units.

- What relation is between the electric field potential and the strength totally and in the case of a uniform field?

- What concepts do we use for visual imagination of the electric field?

- Draw the force lines and the field equipotential surfaces of a point charge, two unlike charges?

- What experimental methods are used for study of an electrostatic field?

- What method is a field studied in this work by?

Laboratory work №13

Method of Determining Electric Resistance

The aim of the work: to study methods of determining electric resistance by measuring voltage and the current strength with the help of a bridge –scheme.

Materials: ammeter with shunt, voltmeter, rheochord, standard and unknown resistances

Theoretical introduction: an electric current intensity I flowing through a conductor conforms to the Ohm’s law:

(1)

where  is a potential difference at the ends of the conductor,  is resistance of the conductor.

The resistance offering on a current by a metallic conductor is determined by interaction of the free electrons – carriers of the current with positive ions of crystal lattices at ordered electron motion under an action of an electric field. As the unit of the resistance is accepted the resistance of such conductor in what it is established the current of 1A at the potential difference of 1 V. This unit is called Ohm ():  1=

The quantity of the conductor resistance can be defined by direct measuring of a voltage and a current in a conductor according to the (1) expression.  In practice for measurement of resistance one applies installations based on the Wheatstone bridge property, the scheme of which is shown in figure 1. The main property of this bridge is as follows: under certain ratio between   resistances the galvanometer current  is absent. Using the second Kirchhoff’s law for this we write equations connecting the quantities of resistances and currents for the ACD and BCD sections of a circuit. It states that in any closed circuit the algebraic sum of products of current  and resistance in various sections of the circuit is equal to the algebraic sum of the electromotive forces in this circuit.  In symbols

(2)

Then for the ACD section of the circuit at

,						(3)

and for the BCD section

(4)

From the Eqs. (3) and (4) considering the equalities of  at  we’ll obtain the condition of the Wheatstone bridge equilibrium:

,  						(5)

Installation description:  The scheme of the Wheatstone bridge used in the exercise 2 of the given work is shown in figure 3. Here the R0 resistor with known resistance is connected to the scheme instead of R1, and resistor Rx expected to define is connected instead of R2. Resistors R3 and R4 are replaced with rheochord AB that is homogeneous wire with a great resistance fixed on the line. Changing the position of the rheochord slider D it can be changed resistance correlation of rheochord arm AD and BD.Their resistances are proportional to lengths of arms. Then, using the equilibrium condition (5) for the given installation, it is possible to write

,  					(6)

Work sequence

Exercise 1.Determination of unknown resistance by measuring voltage and current

- To connect the following electric scheme (Fig.4): connect R1 resistor with unknown resistance in this circuit.

- To determine scale divisions of ammeter and voltmeter.

- To turn on the  source of current  in the circuit, to close the key K

- Step by step changing the voltage given to R1 with the help of rheostat P rheostat, to write in the 1 table 6-8 values of voltage and corresponding current.

Table 1

| № | U, V | I, A | R, |
| --- | --- | --- | --- |

- Plotting the diagram of dependence to be assured that this dependence has linear character. To determine the resistance value by slope of received line

(7)

Exercise 2.Determination of the electric resistance by a bridge scheme

- To fig up electric bridge scheme (Fig.3): connect in circuit R1 resistor instead of Rx.

- To turn on the current source in the circuit.

- To close the K key. Replacing the rheochord slider D slowly to find its position where the current of galvanometer can be equal to zero.  To count out the lengths of  arms. To write down the given values of the resistance R0, the lengths of  arms in the table 2. To calculate resistance value according to the formula (6), to include into the table 2. To disconnect the K key.

Table 2.

| № | R0, | l2, m | l1, m | Rx, |
| --- | --- | --- | --- | --- |

- To include R2 resistor into this scheme. To implement the point 3 for this resistor.

- To connect in parallel R1 and R2 resistors in this scheme. To repeat the third point again for this case

- To connect the R1 and R2 resistors in series in this scheme. Also do the third point again for this case

- To calculate  and  using the measured values of  R1 and R2 resistors

;     				(8)

and compare them with obtained values of R1 and R2  in the points 5-6.

- To define the measuring errors for one-two unknown resistances by formula:

(9)

Admit the value of  equal to.

Control questions

- Explain by what is resistance to current in metals conditioned (the theory of metal electric conductivity)?

- How does the resistance of a conductor depend on its dimensions, temperature?

- What property of the Wheatstone bridge can be used for measuring of a resistance?

- Formulate the first and second Kirchhoff’s laws

- Obtain the formula for determination of the electric resistance of conductors connected parallelly and in series

References

1. Zisman G.A., Todes O.M. Kurs obshchey fiziki, t.2, 1974 §§15,17

2. Kalashnikov S.G. Elektrichestvo. 1977, §57-60,145.149

3. Savel'yev I.V. Kurs obshchey fiziki, t.2. 1978, §78

Laboratory work № 14

Determination of Capacitor Capacitance

The aim of the work:to learn the methods of measurement of electric capacitance of capacitor C.

Materials:voltage generator, multimeter instruments, miniblocks «Current integrator», "Key", "Capacitor" of standard capacitance, "Capacitor" of unknown capacitance.

Theoretical introduction: Measurement of capacitance of the capacitor can be carried out by various ways. Into a basis of measurement of capacitance the ratio between charge Q of a capacitor, its capacitance C and potential differences U on the capacitor plates is set in this work:

Q=CU						(1)

The method of measurement of capacitance of the capacitor includes graduation of a current integrator, determination of unknown capacitance by two methods, control of accuracy of result of graduation by capacitance measurement C of the battery from two capacitors of known capacitance.

In this work for measurement of a charge the current integrator is used. Thus the charge quantity which has passed through it is proportional to the voltmeter readingUinteg:

Q=γUinteg						(2)

where γ is a calibration constant of the integrator.

Calculating formula for the measuring capacitance we will find, using equations (1) and (2):

C=γUinteg/U						(3)

Definition of a calibration constant (graduation of device) is carried out also by a formula (3), carrying out measurements for the standard capacitor with known capacitance of Cst. Thus expression

γ=CstUst/Uintegst						(4)

allows to calculate quantity γ (quantity with an index “st” belong to measurements with the standard capacitor).

To check of accuracy of graduation of the device it is necessary to carry out measurement of any known capacitance with its help. For this purpose it is possible to use the capacitance received by connecting of two capacitors of Cst and Cx, previously having measured unknown capacitance of Cx. Comparing the measured value of capacitance of the connected capacitors Cexpwith calculated by known formulas for parallel Cp=i=1NCiand serial 1Cs=i=1N1Ci connections, we check calibration accuracy.

Installation description: The electric circuit diagram is presented on Fig.1, connection diagram- on Fig. 2.

|  | Figure 1.Electric circuit diagram: 1-controlled source of constant voltage “0...+15V”; 2-switch key; 3- miniblock minibox “Key”; 4-studied capacitor C; 5- damping key; 6-current integrator; 7-miniblok minibox «Current integrator»; 8, 9 - multimeter instruments. |
| --- | --- |
| For a capacitor charging the switch key 2 is set in the position "A", and a damping key 5 is closed (position "Reset"). The capacitor is charged to the voltage U (no more than 2 B), controlled by the voltmeter 9. Before measurement a damping key 5 is disconnected, and the switch key 2 is put into the position "B". Thus the charge which is available on capacitor plates will pass through the current integrator and will be fixed by the voltmeter 8 (the Uinteg voltmeter indication). Further owing to leaks of the voltage, fixed by the voltmeter 8, can change. Figure 2.Connection diagram: 3 - miniblock “Key”; 7 - miniblok «Current integrator»; 8, 9 – multimeter instruments.. |

Work sequence

- Assemble an electric circuit according to the scheme provided on fig. 2, having connected capacitors of Cx and Cexp in parallel. Write down value of standard capacitance Cst into table1.

- Using the buttons “Network” switch on the power supply of  voltage generators and block of multimeters. Press the button «Initial installation».

3. Charge capacitors, for this purpose:

а)	set a damping key 5 close in a position "Reset";

b)	set the switch key 2 (toggle-switch) into position «А»;

c) changing the charging voltage of capacitor with the buttons of voltage setting «0...+15V», set it not more than 2V (counting on the multimeter (multimeter reading 9).

- Discharge the charged capacitor via integrator, for that:

а)	open the damping key 5;

b)	change the switch key 2 into position «В». If the overload indicator at the current integrator lights up, reduce voltage of a charging of capacitors. Remember the multimeter readings 8 directly after the capacitor discharge.

- Repeat points 3 and 4 several times, having picked up such voltage of a charging (the multimeter readings 9) Up, at which voltage of a discharge (the multimeter readings 8) Uintp are 8-10V (the quantity which is proportional to a charge of the capacitor). Write down this voltage reading in table 1and further during laboratory work don't change it.

| № | Standard capacitor Cst (μF) | Determination of capacitance |  |  |
| --- | --- | --- | --- | --- |
|  |  | Unknown capacitor | Connection of capacitors |  |
|  |  |  | parallel | serial |
|  | Ust, (V) | Ux,(V) | Up,(V) | Us,(V) |
|  | Uintst, (V) | Uintx, (V) | Uintp, (V) | Uints, (V) |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| Average |  |  |  |  |

- Without changing a charging voltage Up , execute 5 measurements of Uintp, writing down values in table 1.

- Connect Cxand Cstin series. Leave voltage of Us equal toUp . Execute 5 measurements of Uints and write down results into table 1.

- Carry out measurements of quantityUintstseparately for the standard capacitor Cstand the quantity Uintxfor the capacitor of unknown capacitance Cx. Values Ust and Ux  remain equal to Up  and Us . Results of measurements are registered in table 1.

- To check accuracy of graduation by means of multimeter instrument measure unknown capacitance of the capacitor, write down the result Cmx into table 2, precision of measurement of a multimeter of δC=5%.

- Switch off the “Power” buttons of a power supply of voltage generators and multimeters.

Processingof measurementresults:

- Using the data from  table 1, calculate a calibration constant γ (Eq. 4)

γ=CstUst/Uav.intst.

- Calculate capacitances of the unknown capacitor by Eq.(3)

Cexpx=γUav.intxUx,

and capacitance of parallel connected capacitors

Cexpp=γUav.intpUp,

capacitance of serial connected capacitors

Cexps=γUav.intsUs.

Write down results of calculations into table 2.

Table 2

| Unknown capacitor Cx, (μF) |  | Connection of capacitorsC, (μF) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Parallel |  | Series |  |
| ExperimentalCexpx | TheoreticalCthx | ExperimentalCexpp | TheoreticalCthp | ExperimentalCexps | TheoreticalCths |
|  |  | δCp= |  | δCs= |  |

- Estimate a relative error of value Cx:

δCx=δCst2+δUav.intst2+δUav.intx2 ,

where δCst is a relative error of standard capacitance (it is set on the miniblock), δ Uav.intst=δ Uav.intx=1.2% is accuracy (a relative error) measurements by a voltage multimeter.

- Make calculations using values of capacities Cst and Cexpx, by formulas for parallel and serial connections of capacitors.

Cthp=Cst+Cexpx,

Cths=Cst∙CexpxCst+Cexpx.

Write down results of calculations into table 2.

- Find a relative deviation of experimental values from theoretical ones in %:

δCp=Cexpp-CthpCthp∙100%,

δCs=Cexps-CthsCthp∙100%.

Write down the received results of calculations into table 2.

6.	Comparing these deviations with a relative error of measurements δCx, make aconclusion about accuracy of measurements.

7.	Compare results of measurement of unknown capacitance (CthxиCexpx).

Control questions

- Give the determination of values of capacitance of the conductor and the capacitor.

- On what values do the conductor and capacitor capacitance depend?

- How will the capacitor capacitance change if there is a change of dielectric permittivity εr or the distance between plates d? in case:

а) the capacitor is disconnected from a current source; b) without shutdown?

- Write down formulas for capacitance calculation at serial and parallel connections of capacitors. How will the capacitance in connection change in comparison with capacitance of one capacitor?

- Compare parameters (a charge, voltage) of one capacitor and the battery of the capacitors connected: а) in series; b) in parallel.

- What value is measured by the integrator? On what do the voltage readings U of the device depend?

- What does the value of a calibration constant γ show?

- Which values should be measured for device graduation?

- What formulas are used to define a calibration constant γ, capacitance of the unknown capacitor Cxand capacitances of connections of capacitors Cexp and Cth?

References

1.Detlaf A.A., Yavorskiy B.M. Kurs fiziki. - M .: Vysshaya shkola, 1989. - §§

16,2, 16,3.

2. Kalashnikov S.G. Elektrichestvo. - M .: Nauka, 1977. - §§ 31, 32.

Laboratory work №15

Determination of RC Time Constant

The aim of the work:to study the law of variation of voltage at the capacitor discharge, to define a RC time constant and its resistance R.

Materials: controlled  constant voltage source, stop-watch, multimeter instrument, “Key” and “Capacitor” miniblocks.

Figure 1. Capacitor discharge

Theoretical introduction: The alternating electric current flows in the circuit if there is charging or discharging of a capacitor (Fig.1). If this current I changes not too quickly and so that during every moment of time the value I is identical at all points of a loop, then the laws of a direct current are fairforvalues of instantaneous current I.Such slowly changing currents are called quasisteady.

Current in a loop of length l will be quasisteady if it is set almost instantly, i.e. if time of its setting equal lc (c=3∙108msis propagation speed of an electromagnetic field), which is much less, than τ  time of a current change that is characteristic for this circuit. In case of current change according to the exponential law the τ is called a time constant of a circuit - time during which the current changes е = 2.71 times, and in case of electric oscillations the condition of a quasisteady currents is lc≪T, where T is an oscillation period.

Methods of measurements: In this work measurement of a RC time constant is based on studying of process of the capacitor discharge through resistance R. Thus the electric current flows in a circuit (Fig.1)

I=-dQdt							(1)

Here Q is the charge of a capacitor that is proportional to voltage U on its plates,

Q=CU							(2)

where C is capacitor capacitance.

According to the Ohm law a quasisteady current

I=UR.								(3)

Using ratios (1), (2), (3), we make the differential equation describing the change of a capacitor voltage eventually with time t:

dUdt=-URC.

Dividing variables in this equation and solving it by integrating from the initial moment of t =0 (the voltage U0) to the current time t (the voltage U):

U0UdUdt=-1RC0tdt

we receive dependence of the capacitor voltage on time (Fig.2):

U=U0e-tRC,  or  U=U0e-tτ,  				(4)

where τ is a time constant of the circuit containing capacitance and resistance (RC-circuit),

τ=RC.							(5)

We linearize dependence (4) taking the logarithm:

lnU=lnU0-tτ						(6)

The diagram of this linear dependence is presented on figure 2.

Thus, investigating dependence of voltage on the capacitor on time, it is possible to define experimentally: a RC time constant, resistance R of a circuit if the capacitance C, capacitor capacitance, resistance R, through which it is discharged, are known.

| Figure 2. Dependence of voltage on the capacitor and its natural logarithm on time |  |
| --- | --- |

Installation description: The electric circuit diagram of installation is shown on figure 3, connection diagram - on figure 4.

Fig. 1.Electric circuit diagram: 1-controlled source of constant voltage “0...+15V”; 2-switch key; 3- miniblock minibox “Key”; 4-minibox “Capacitor”; 5- multimeter instrument.

The capacitor C is charged to voltage U0 from a source of constant voltage 1. Then a key 2 is disconnected, and the capacitor starts discharging through the voltmeter 5 connected to it having big input resistance R. With the help of a voltmeter 5 it is possible to watch the current value of voltage on the capacitor which changes by the law (4) established above.

Figure 4. Connection diagram: 3, 4, 5 see figure 3

Work sequence

- Assemble an electric circuit according to the scheme provided on figure 4 .

- Using the buttons “Power” switch on the power supply of voltage generators and block of multimeters. Press the button «Initial installation».

- Set on the capacitor a voltage from 10 to 15V (according to the teacher’s task) by the buttons of installation of voltage «0... 15 V».

- Disconnect the capacitor from a controlled constant voltage source having changed a key to the position "B", and in every 5 seconds write down voltmeter’s reading into the table as far as the capacitor discharges.

- Connect the second capacitor C2to the capacitor of C1in parallel and repeat similar measurements for parallel connected capacitors Cp.

- Switch off power supply of voltage generator and block of multimeters with “Power” buttons.

Table

|  | t, s | C1, μF |  | Cp, μF |  |
| --- | --- | --- | --- | --- | --- |
|  |  | U1, V | lnU1 | Up, V | lnUp |
|  | 0 5 10 15 20 25 30 35 40 45 50 55 |  |  |  |  |
| Midpoint |  |  |  |  |  |

Processing of measurement results

- Calculate and write down into the table the capacitor capacitance Cpby the formula for parallel connection:

Cp=C1+C2

2.	Draw a diagram of dependences U1=ft and lnU1=f(t)on one space of drawing for the capacitor C1 and on other one Up=ft and lnUp=f(t)for capacitor Cp, placing an axis of voltage U  on the left side, and the lnU axis - on the right.

3. Determine angular coefficients (slope factors) K1 and Kp of linear dependences and time constants of a circuit τ1=-1K1 and τp=-1Kp for each value of capacitance according to the diagrams lnU1=f(t)and lnUp=ft.

4. Calculate the resistance of the voltmeter R by the formula (5) for each value of capacitance:

R1=τ1C1, Rp=τpCp

5.Estimate a relative error of the measured values:

δR1=δC12+δK12≅δK1 and  δRp=δCp2+δKp2≅δKp, ,

where δK1 and δKp are errors of angular coefficients.

6. In a conclusion of work make the analysis of the received experimental dependences and estimate the accuracy of a method for determination of a time constant.

Control questions

- Which current is called quasisteady? Formulate a statement of a quasisteady electric current.

- What physical laws and formulas are used to obtain dependences of voltage on time at the capacitor discharge?

- According to which law are varied with the time at discharge of a capacitor following values: 1) charge of plates, 2) the capacitor voltage, 3) current in RC-circuit?

- What value is called a time constant of a circuit and what does it characterize?

- How is the RC time constant connected with parameters of this circuit?

- Specify a multimeter operating mode at carrying out measurements: the measured quantity, a mode and inputs for device connection.

- What functionality does the power supply in the studied electric circuit have?

- For what purpose is the diagram of dependence U=f(t) plotted in this work?

- How is it possible to show, that empirical relation is exponential?

- How is a time constant of the RC circuit defined experimentally?

11 By which method is a resistance of the voltmeter measured in this work?

12. What formulas are used in this work to define the following quantities:

а) time constant of a circuit,

b) resistance of a multimeter instrument?

13. How are errors of determining of a time constant estimated in the work?

References

1. Detlaf A.A., Yavorskiy B.M. Kurs fiziki. -M .: Vysshaya shkola, 1989. §§19.1,27.3.

2. Kalashnikov S.G. Elektrichestvo. -M .: Nauka, 1975. - §§ 73, 74.

Laboratory work № 16

Studying of Hall Effect in Semiconductors

The aim of the work: Acquaintance with the Hall phenomenon, to define a hall-effect constant and concentration of electronic carriers in the semiconductor.

Materials:the controlled source of constant voltage «0... 15V», the stabilized source of constant voltage «+15V», the miniblock «Hall Effect», multimeters

Figure 1. Hall effect

Theoretical introduction: Hall Effect is the appearance of voltage U in perpendicular direction to vectors j and B (Fig.1) in the conductor (or in the semiconductor) with density of a current j  placed in a magnetic field B. The effect was opened in 1879 by the American physicist E.G.Hall who experimentally establish the following expression for voltage:

U=RIBd,

where R iscoefficient of proportionality (Hall coefficient), its value and a sign depend on a chemical composition of the conductor, on temperature and a charge of  current carriers; I is a current force in a sample; B is an induction of a magnetic field; d is the thickness of a sample.

Let's consider the elementary theory of Hall Effect. Let carriers of a current are electrons (for example, in metals and extrinsic n-type semiconductor). If there is a current with the density j in a sample, the electrons have a drift rateυ↑↓j. If the conductor with a current is placed into the magnetic field, Lorentz's force affects the electrons:

FL=-eυ,B						(2)

causing charge redistribution Y-direction (see Fg.1). Electrons will deviate to one of sides, leaving on opposite one uncompensated positive charge. As a result the electric field Ey along Y axis appears that affects the electron with force Fel=-eEy,which is directed opposite to Lorentz's force. To a stationary condition there corresponds a condition

Fel=FL  or   eFy=eυB,				(3)

as in this case a velocity vector is υ⊥B.

From the equation (3) we find strength of Hall field:

Ey=υB							(4)

and a potential difference between sides 1 and 2 (Hall voltage):

U=0bEydy=υBb					(5)

where b is the size of a sample along the direction of a Hall field.

Let's express velocity of electrons of υ through current strength by formulas:

I=jbd;    j=enυ;     υ=Ienbd,   			(6)

where n is concentration of electrons of conductivity in a sample.

According to the equation (5) taking into account a formula (6) we will write down Hall voltage in the form of

U=1en∙IBd							(7)

We will receive hall-effect constant (m3/C), having compared expressions (1) and (7):

R=1en							(8)

More rigorous theory considering interaction of current carriers (electrons) with a crystal lattice, gives the hall constant:

R=ren							(9)

where r is the Hall-factor; its size r>1 depends on a magnetic field, temperature and properties of a material of a sample. For lightly doped germanium at room temperature r=3π/8.

According formula (8) the sign of Hall coefficient is defined by a sign of a charge of current carriers. For metals and n-type semiconductors R<0, and for semiconductors with hole-type conductivity (p-type) R>0.

The method of studying of Hall Effect: Hall voltage, according to the formula (1), depends linearly on magnetic induction B and on the current which flows in sensor. The installation allows to receive dependences U(B) and U(I) and to determine Hall constant R by angular coefficient of an experimental direct straight. In case of linear dependence of a type U=KB according to a formula (1) the value of angular coefficient is

K=RId							(10)

As a source of a constant magnetic field it is possible to use an electromagnet. The quantity of an induction of a magnetic field B in an electromagnet gap depends nonlinear on a magnetization current Iem in its coil. However on the curve of magnetization of the coreB (Iem) almost linear region can be allocated for which the next formula is true

B=μ0IemNh,							(11)

where μ0=4π∙10-7H/m isa magnetic constant; N isturn number of electromagnet; h is thickness of a gap of an electromagnet.

The task of measurement of Hall voltage becomes complicated that at carrying out experiment in a plate there is not only Hall field Ey but also the electric field E creating a current in the sensor (which is passed to observe the effect). These mutually perpendicular fields and position of conductors (2'-2'), soldered to the sensor to measure of the Hall voltage, are shown on figure 4.

As it is seen on figure 4a, in case the line of metering contacts (2'-2') is displaced from ideal (2-2), which should be strictly normal to the line of current I(field E), the potential of a point 2' will be less, than potential of a point 2. Thus the measured quantity U1 will be on ∆U less, than the Hall voltage (at shift of 10 microns of ∆U≅5mV): U1=(U-∆U).

Figure 4. Measuring of Hall voltage

Having addressed to figure 4b, we see that it is enough to change only the direction of an induction of a magnetic field B in order to measure the voltage on line (2'-2'): U2=(U+∆U).

Using U1and U2 values, we exclude an unknown imprecision∆U:

U=U1+U22.							(12)

For experimental realization of this technique the current direction Iem in an electromagnet coil is changed. According to the figure 4b, Hall voltage polarity is also changed. But quite often it appears that the value ∆U≫U; thus measured voltage U1and U2 have an identical sign, and their values U1=(U-∆U)иU2=U+∆Uallow to find the Hall voltage U=U2-U12.

Installation description: The electric circuit of installation is shown on figure 2, connection diagram - on figure 3. Test sample, Hall sensor (D), is thick plate of germaniumpossessing hole-type conductance at room temperatures. The sensor is placed in a gap of an electromagnet coil 6 and is connected to steady voltage source 10 through switch key 8. The direction of a current Iem in electromagnet coil can be changed by switch key.Digital voltmeter 5 is intended to measure Hall voltage.

|  | Figure 2. Electric circuit diagram: (1-1)-power circuit of electromagnet 6; 8-switch key; 9- multimeter; (2-2)-measuring circuit of hall voltage; 5- multimeter; (3-3) -power circuit of hall sensor D; 4- stabilized voltage source “+15V”; 7-minibox of “Hall effect”; 10-controlled steady voltage source “0…+15V”. |
| --- | --- |

Work sequence

1. Write down into the table turn number of electromagnetN, width of a gap h and thickness of the sensor d (specified on the miniblock «Hall Effect»).

2. Connect an electric circuit by the connection diagram given on figure 3.

3. Switch on power supply of voltage generators by the “Power” button. Press the button «Initial installation».

Figure 3. Connection diagram

Table

| Installation parameters:I=5mA, N= turns, h= mm, d= mm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Quantity |  |  |  |  |  |
| № | Iem, mA | U1, mV | U2, mV | U3, mV | B, mT |
| 1 | 10 |  |  |  |  |
| 2 |  |  |  |  |  |
| … | … |  |  |  |  |
| 10 | 100 |  |  |  |  |
|  |  | Midpoint: |  |  |  |

4. Set a current in an electromagnet circuit Iem≈10 mА by the buttons of voltage setting«0... 15 V».

5. Measure the voltage U1 (at this direction of a current in an electromagnet winding) by a multimeter 5.

6. Change the current direction Iem and measure voltage U2by the switch key 8.

7. Increasing a current in an electromagnet windingIem, by the buttons of voltage setting«0... 15V» measure voltages U1and U2 on subitem 5, 6 every ≈10 mА. Write down results of measurements into table.

8. Switch off power supply of voltage generator and block of multimeters by “Power” buttons

Processingofmeasurement results:

- Calculate the values of a magnetic induction B for each value Iem, using a formula (11). Write down results of calculations into the table.

- Calculate Hall voltage U for each value of a magnetic induction B by formula (12). Write down results of calculations into the table.

- According to the received data plot the diagram of dependence of U=f(B).

4. Draw a straight line through an middle point and zero one: it is known that U=0 at B=0.

5. Define angular coefficient K of an experimental straight line and its relative errorδK.

6. Using formula (10), find value of a Hall constant for the studied semiconductor:

R=KdI

7. Estimate a relative error of quantity R: δR=δK+δI.

8. Calculate concentration of holes n in studied semiconductor on the basis of expression (8) or (9):

n=rRe

9. Find a confidential interval of quantity n, accepting that a relative errorδn=δR.

10. In a conclusion of work point out what regularities of Hall Effect are investigated and specify possible applications of Hall device.

Control questions

- WhatisHallEffect?

- What conditions are necessary for observation of the Hall phenomenon?

- Specify the reason of appearance of Hall voltage.

- How is Lorentz's force, affecting moving electron, directed?

- Show the direction of vectors E and B for electric and magnetic fields in a semiconductor plate at observation of Hall Effect on a given drawing.

- Between what sides of a plate does the Hall voltage appear? Specify the position of sides in relation to a current I and to a magnetic field B.

- For measurement of what quantities are: а) milliamperemeter, b) digital voltmeter used in this work?

- To what sides of the Hall sensor are connected the following devices: а) multimeter, b) digital voltmeter?

- What quantities and parameters of the Hall phenomenon will change if to change:

а) value and direction of a working current of the sensor, б) value and the current direction in an electromagnet winding?

- What size of a semiconductor plate is necessary for calculation of the Hall constant?

- By what way is the Hall voltage measured?

- Write down formulas which are used in this work for calculation of concentration of electrons of conductivity in a semiconductor.

References

1. Detlaf A.A., Yavorskiy B.M. Kurs fiziki. -M .: Vysshaya shkola, 1989. §§23.1,27.3.

2. Kalashnikov S.G. Elektrichestvo. -M .: Nauka, 1975. - §§ 50.

Laboratory work №17

Establishment of Normal Magnetization Curve of Ferromagnetic

The aim of the work:plotting of the normal curve of magnetization B(H) and graph of magnetic permeability as a function of magnetic field strengthμrHof ferromagnetic.

Materials:an controlled source of the constant voltage, the stabilized sources of constant voltage, multimeters, miniblocks of “Ferromagnet”, «Current integrator», "Resistance".

Theoretical introduction:The magnetic induction of a field Bin ferromagnetic (iron, cobalt, nickel, special alloys), placed in a magnetic field, not linearly depends on its intensity H (Fig.1). And, the quantity B in ferromagnetic substance depends not only on this field, but also on the previous magnetic state of a sample.

| Figure 1. Magnetic induction B –strength H diagram | Figure 2.Domains |
| --- | --- |

If the sample was demagnitization previously, thus when magnetizing the B–H diagram is represented by curve 0-1 and is called the normal magnetization curve. At reduction of a field strength H (see a curve 1-2) change of a magnetic induction B in a ferromagnetic will lag behind change of H (a magnetic hysteresis).

Ferromagnetics have such feature because there is an existence of areas of spontaneous (spontaneous) magnetization, called domains, inside of them. In the degaussed state of ferromagnetic the magnetic moments of various domains are oriented so that the resultant vector of magnetization of a sample is equal to zero. One of possible domain structures of such state can be found on figure 2, where the directions of magnetization of domains are shown by the arrows.

When placing the ferromagnetic into an external magnetic field the borders of domains are shifted so that domains grow firstly, the magnetic moments of which make an acute angle with a vector H, i.e. the domains having the most advantage energetic state. At the following stage with growth of H there is a turning of the magnetic moments of all domains towards the field direction.

Thus magnetization of a sample J (the magnetic moment of unit volume) reaches saturation (Fig.3), and the induction of a field B continues to increase slightly at the expense of increase in strength H of an external magnetic field:

B=μ0H+J=μ01+χH=μ0μrH,		(1)

where  μ0=4π∙10-7H/m  is magnetic constant, χ is magnetic susceptibility.

The domain structure of ferromagnetics is also the reason of that the quantity of relative magnetic permeability μrcan change over wide range (from 1 to 106) and with complicated way depends on strength of an external magnetic field (Fig.4).

| Figure 3 | Figure 4 |
| --- | --- |

When heating of a ferromagnetic above Curie temperature the thermal motion destroys its domain structure and together with that all features of a ferromagnetic state disappear: the substance pass into a paramagnetic state.

Figure 5

The method of measurements: The complete study of dependence B(H) was first carried out in 1871-1872 by the professor of the Moscow university A.G Stoletov. It showed also that a field strength inside ferromagnetic, strongly depends on a shape of a sample and only in case of the toroidal-shaped sample, magnetized by a ring winding, it coincides with strength of an external magnetic field.

In this work to get the normal magnetization curve B(H) a method offered by A.G.Stoletov is used. It is as follows. On a ring from a ferromagnetic material two windings (Fig.5) are located: primary (magnetizing) contains N1 turns, and secondary (with number of coilsN2) is intended to measure the quantity of a magnetic induction B.

Magnetic field strength which is created in the ring core in the process of flowing in primary winding of a current I1, is possible to calculate by a formula

H=I1N1l=K1I1						(2)

where K1=N1/l; l is length of the middle axial line of the core.

This field magnetizes a ring. A magnetic flux in cross-section of a ring with the area S

Φ=BS

At its change in a secondary winding e.m.f of induction appears:

εi=-N2dΦdt,

where dΦdt is the rate of change of a magnetic flux.

Change of a magnetic flux is reached by change of the direction of a current I1. Thus value of field strength H remains the same, and only the direction of field lines changes. As a result the flux of a magnetic induction changes from value +Ф to - Ф, and an increment of a flux is

ΔΦ=-2Φ=-2BS					(3)

If the circuit of a secondary winding is closed, the current will begin to flow in it

I2=εiR=-N2R∙dΦdt,

where R is a circuit resistance of a secondary winding.

In a process of flowing of an induction current I2 in this circuit the charge Q, the value of which can be calculated using formulae (3), (4), is transferred:

Q=0tI2dt=+Φ-Φ-N2R∙dΦdtdt=-N2RΔΦ=2N2RBS.

Expressions from this equation a magnetic field induction can be received

B=QR2N2S, 							(5)

where R, N2 and S are quantities, constants for this installation.

The charge Q is measured by the current integrator connected to a secondary winding. Thus the value of the charge which has passed through the integrator, is proportional to the voltmeter reading n:

Q=γn,						(6)

where γ is an integrator constant; n is multimeter reading.

The quantity of constantγ characterizes the sensitivity of the integrator and is equal to a charge (Q/n) which causes the singular reading of a multimeter.

Using the expressions (5) and (6), it is possible to write down a calculating formula for a magnetic field induction in the studied ring core:

B=γnR2N2S=K2n,						(7)

where K2=γR2N2S.

Installation description:The electric circuit installation is shown on figure 6, connection diagram - on figure 7.

| Figure 6. Electric circuit: 1-source of controlled constant voltage «0... +15V»; 2-a multimeter; 3-“Resistance” block, R0=100Ω; | 4- the switch key of the direction of a current in primary winding; 5-a toroid with primary N1 and secondary N2 windings; 6- ‘Ferromagnetic” block; 7- damping key; 8- current integrator; 9- «Current integrator» block; 10-a multimeter |
| --- | --- |
| Figure 7. Connection diagram |  |

Primary N1 and secondary N2 of a winding are coiled on the ring core which is made of a studied ferromagnetic material. Primary winding is used for magnetization of magnetic and according to its parameters the strength H of a magnetizing field is defined. The switch key 4 serves to change the direction of a current in primary winding for the purpose of core alternating magnetization. The resistor Ro restricts a current in a winding.

The secondary winding of N2 is intended to define an induction of a magnetic field B in the core. The integrator of a current 8 in a circuit of a secondary winding serves to measure a charge Q fixed by a multimeter 10, proportional to the measured value B.

As the core is made of soft magnet ferromagneticwith small value of a residual magnetic induction B, thus to establish the normal magnetization curve there is no need to carry out a preliminary demagnitization of the core.

Work sequence

- Write into the table parameters of the installation and a studied sample:

N1 and N2arenumber of turns of primary and secondary windings;

l is length of the middle axial line of the core;

S is area of cross-section section of the core;

R is resistance of a circuit of a secondary winding;

γ is an integrator constant.

- Connect an electric circuit according to the connection diagram given on figure 7.

- Switch on power supply of voltage generators and the block of multimeters by the “Power”.  Press the button «Initial installation». Set damping key 7 of the integrator of a current in the position "Reset". Set the switch key 4 in the bottom position.

- Set I1≈6.0mAin primary winding by the buttons of installation of voltage «0... 15V» .

- Open a damping key 7. Change the switch key 4 to the top position, notice the maximum reading U1 of a multimeter 10 and write down it into the table. Change a key 7 to the "Reset" position.

- Open a damping key 7. Change the switch key 4 to the bottom position, notice thus the maximum reading U2 of a multimeter 10 also write down it into the table. Change a key 7 to the "Reset" position.

Table

| Installation parameters | I1, mA | U1, V | U2, V | Uav, V | H, A/m | B, mT | μr |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N1=100 N2=200 l=37.7 mm S=25 mm2 R=2.5 кΩ γ=23.710-8C/V |  |  |  |  |  |  |  |

- For each current I measure maximum readings U of multimeter 10 according to 5, 6 subitems setting consistently values of a current with a step ≈0.2 mА to 14 mА, and then with a step ≈10 mА to 100 mА. Write down results of measurements U1, U2 and a current I1 into the table.

- Switch off power supply of voltage generators and the block of multimeters by the “Power” buttons.

Processingofmeasurement results:

- Calculate the average value Uav value for each current:

Uav=U1+U22

- Calculate the constants of K1 and K2, necessary for calculations of values H and B by formulas (2) and (7), using values of parameters of the installation and a studied sample:

K1=N1l

K2=γR2N2S

- Calculate the values H and B for each value of a current by formulae:

H=K1I1

B=K2Uav

- Calculate magnetic permeability μrby formula (1) for each value H:

μr=Bμ0H

Write down results of calculations into the table.

- According to the table data plot the normal curve of magnetization B(H) and the μrHdependence diagram.

- In a conclusion of work point out features of a shape of experimentalcurves:

а) compare a run of a curve of magnetization with the provision of a maximum on graphics μrH;

b) compare the received curves to known theoretical and experimental dependences.

Control questions

1. Show a form of the normal curve of magnetization В(Н) and μrHdependence diagram (relative magnetic permeability - field strength) for ferromagnetic:

а) at Т<Тc; b) atТ>Тc; (ТcisCurie temperature).

2. How does the main magnetization curve of ferromagnetic differ from a similar   dependence В(Н)for non-degaussed sample?

3. Name characteristic properties of ferromagnetic and features of their magnetization.

4. Describe changes of domain structure of ferromagnetic in the process of its magnetization (in process of growth of a field strength H).

5. On what quantities do:

а) strength H of a toroid magnetic field;

b) an induction B of a magnetic field of a toroid with the ferromagnetic core;

c) magnetic permeability μrof the core of toroid depend?

- Which formulas (from mentioned in the work description) show the dependence of characteristic of magnet field ВandНon othervalues?

- On what and how does the charge Q, measured with the help ofcurrent integrator, depend? What current flows in the device?

- Define the functionality of current integrator.

- For what is the switch key 4, while measuring used?

10.	 Which measured values and formulas are used to define the following quantities:

а)	magnetic field strength Нin core;

b)	magnetic induction В;

c)	magnetic permeability μr of core material?

References

1. Detlaf A.A., Yavorskiy B.M. Kurs fiziki. -M .: Vysshaya shkola, 1989. §§24.

2. Kalashnikov S.G. Elektrichestvo. -M .: Nauka, 1975. - §§ 109-111, 119.

Laboratory Work №18

Determination of the Coil Inductance

The aim of the work: the inductance coil defining and phase shift between current and voltage in the coil.

Materials:coil inductance; steel core; voltmeter; ammeter; rheostat; direct current source; alternating current source.

Theoretical introduction:If the current strength changes in the circuit with current,  then magnetic flux penetrating this curciut which is created by the current changes. According to the law of electromagnetic induction it leads to electromotive force (EMF) of self-induction occurrence:

(1)

Evidently, EMF of self-induction is proportional to current strengthrate of change  in the circuit. This expression is true in the event that, when the coefficient of self-induction (inductance) L remains constant in the current strength change.  Minus in the formula (1) means that inductive current is directed to buck the origin causing it.

According to the expression (1), the inductance is numerical equal the EMF quantity arising in the conductor in the current strength change at the unit for the time unit.  The SI inductance unit is called henry (H). Inductance depends on the forms and dimensions of the conductor as well as magnetic permeability of the environment.

Let the coil resistance to direct current equal R. This resistance is often called active. If such coil is turned on the ac circuit, then as a result of periodic current strength change in it EMF of self-induction occurs opposing applied voltage change. It results in coil resistance becoming more than active.  In the other words, the coil inductance will have not only active but also inductive (reactive) resistance XL. As far as the coil resistance to alternate current is more than direct, then it is possible to see it as connected in series resistances R and  XL. However, common resistance will not be equal the algebraic sum R  and  XL. It arises from the following reasons.

The alternating voltage applied to coil changes with time according to the principle

U= U0 sin ωt                                                  		(2)

A

V

K

P

Rx

We will define the current strength in the circuit. Then the coefficient, concatenating voltage and current strength, produces full resistance quantity.  Applied voltage is equal the voltage drop amount

at active resistance      and reactive

(3)

Obtained equation represents differential equation relative to current strength. We will find its decision as

Figure 1.

(4)

where φ- angular phase difference between the current in circuit and applied voltage (Figure1).

Putting the expression (4) in equation (3), we will have:

or

(5)

Equating the coefficients in соsωt    and   sinωt   in both parts of equality (5), we will have:

(6)

(7)

The first of these equalities after division of its members for    gives

(8)

Raising these equalities (6) and (7) to square and summing, we will have:

(9)

from here

J0=                                                        (10)

We will analyze the obtained results. The quantity

z=                                        (11)

plays the role of full resistance. We will find from here:

L=	(12)

Ohm's law for direct current circuit part is particular case of the expression (10).

Indeed, for direct current ω=0 and  . If alternating current circuit consists of resistance with epsilon squared value of self-induction coefficient (L=0), as well as

it is possible to write  .  From the formula (11) it is clear that full resistance of circuit Z  is more than active R. If, then it is possible to disregard the quantity under the root. Then

(13)

From the last expression it is seen that inductive resistance is equal, i.e. inductive resistance is equal to product of self-induction coefficient for frequency of alternating current.

Applied voltage  and the current in the circuit do not coincide with each other by phase (Figure 1). It means that, when voltage in the coil reaches the maximal value, current strength is not maximal and vice versa.

Angular phase difference φ can be defined from the formula (8), self-induction coefficient  L by the formula (11). But amplitude value of current and voltage appear in the expressions (8) and (11). In measuring the voltmeter and ammeter show not amplitude but effective values, and , which are connected with amplitude values   and   following correlations

,   	(14)

Then, it is possible to write

(15)

From equation (12) it is clear that, for defining the coil self-induction coefficient, it is necessary to know it full resistance  Z, active resistance R  and cyclic frequency of alternating current

(16)

where    ν is alternating current frequency.

Figure 2.

Installation represents the electric circuit (Figure 2), consisting of coil inductance L, rheostat Р, ammeter А and voltmeter V connected in parallel with coil. For defining the coil active resistance to dots A and B schemes, the direct current source (DCS) are connected, and for defining the full resistance - audio-frequency oscillator (AFO). For defining the coil inductance with core, steel core is inserted to coil. It is necessary to note here that, in this case it is possible to speak about a certain average value of coil inductance with core, defined for the given interval of current strength change.

Work sequence

- Collect electric scheme of installation connecting to scheme of the direct current source (DCS).

- Define the ammeter and voltmeter division value.

- Turn on the current source to circuit 220 V.

- Changing the current strength in circuit by rheostat Р, write down three values of current strength   and voltage .

- Find the resistance R for each pair I and U, and then ,   for three measurements. Define the absolute error    and  ,

- Connect the audio-frequency oscillator to the scheme.

- Repeat the item 4 for intended frequency of alternating current (20 Hz <ν<200 Hz  according to the teacher’s instruction).

- Find three corresponding values of full resistance Z by the formula (15), and then , for three measurements. Define absolute error     and  ,

- Define the coil inductance , putting found values  ,   and   to the formula  (12).

- Calculate the absolute error  ∆L:

∆L=L·                           (17)

11.Define the phase displacement between the current and voltage by the formula (8).

12. Repeat the items 6-11, inserting the core to coil.

13. Write down the results of the experiments to table 1.

Table 1

| № |  |  | A |  | , |  | A |  |  | L Гн | Гн | tg |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. 2 . 3. | Without the core |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Average value |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. 2. 3. | With the core |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Average value |  |  |  |  |  |  |  |  |  |  |  |  |

Control questions

1. Wherein lies the self-induction phenomenon?

2. What is the physical sense of self-induction coefficient?

3. What method defines the coil inductance in the present work?

4. How is the phase displacement between current and voltage defined?

5. What is called the inductive resistance of conductor? What quantities does it depend on?

6. What is called the effective value of current strength, voltage?

References

1. Savel'yev I.V. Fizika. Obshchiy kurs. Elektrichestvo i magnetizm. Volny. Optika. - M .: Nauka, 1989.

2. Abdikasova A.A., Niyazova SH.V., Uteulina K.A. i dr. Elektrichestvo i magnetizm. Metodicheskoye ukazaniye k laboratornym rabotam. - Ural'sk: TOO «Nauka», 1996.

3. Matveyev A.N. Elektrichestvo i magnetizm. Fizicheskiy praktikum. - M .: Vysshaya shkola, 1983.

OPTICS

Laboratory work №19

Checking of Malus’s law

The aim of the work: acquaintance with the phenomena of polarization of light and checking of  Malus’s law  by experimental method.

Materials: polarizers, source of light, photo-cell, galvanometer.

Theoretical introduction. Light wave is transversal and represents distribution mutually perpendicular oscillations of the intensity vectors of an electric field  and a magnetic field, here the vector  is called a light vector.

The light beam is natural if various directions of the vector  in a field are equiprobable to a direction of a wave distribution of a plane. In natural light the oscillations are various and directed quickly and randomly replace each other (Fig.1)

Light is polarized if the directions of oscillations of the vector are ordered and submit to some law.

Figure 1

Light is linear or plane-polarized if the light vector  oscillates only in one direction (Fig.2).

Figure 2

Light is polarized on a circle or an ellipse if the light vector  rotates, not varying on magnitude or varying on magnitude correspondingly (Fig.2).

At linear polarization the plane containing the vector is called the plane of oscillation of polarized light, and a plane where the magnetic vector  is allocated, is a plan of polarization.

Malus’s law. Visually a polarized light does not differ from natural. The device, allowing them to distinguish, is called the analyzer. The principle of operation of the analyzer is similar to polarizer’s one.

An amplitude oscillation in a plane forming an angle with a plane of a polarizer, is possible to decompose on two oscillations with their amplitudes (Fig.3)

АІІ= Асоs φ;       AІ= A sin φ				(1)

The first oscillation passes through the device, but the second is detained.  Первое колебание пройдет через прибор, а второе будет задержано. Since

I~A2								(2)

Then,

= A2cos2φIІІ = Icos2φ			(3)

In natural light all values of φ are equiprobable. Therefore the portion of light passed through a polarizer, will be equal to average value of cos2 φ, i.e. 0,5.  Hence, I=0,5I0, where I0 is the light intensity incident on a polarizer.

Let's place on a way of a natural beam two polarizers which planes form an angle φ. From the first polarizer a plane-polarized light will leave which intensity will make half of intensity of natural light 0,5I0. According to the Malus’s law from the second polarizer the light with intensity I×cos2 φ leaves. Thus, the light intensity passed through two polarizers is equal to

I=0,5I0 cos2φ						(4)

From this formula we obtain Iтах=0,5 at  φ=0 and I min=0 atφ=90o, i.e. crossed polarized don’t let pass through a light.

Installation description. The device for checking Malus’s lawis shown in Fig.5 (1 is the source of the natural light, 2 is the condenser  lens, 3 is polarizer, 4 is analyser, 5 is depolarizer (opal glass), 6 is photo-cell, 7 is milliammeter).

All parts of installation are fixed in special riders (bars) that they could be moved along an optical bench. The polarizer and the analyzer can rotate around of an axis. For the account of angles in the polarizer and the analyzer there are available special limbs with the scale factor. There are used polarizers as the polarizer and the analyzer in the given work. Each of them is a thin film from celluloid in which are interspersed equally oriented thin needle of the herapathite crystal. At passing a natural light ray through a crystal it become double on the ordinary and the extraordinary rays, polarized in two mutually perpendicular planes.

The herapathite crystal has one property named dichroism. At thickness of this crystal of 0.1-0.3mm on the ray path all ordinary rays of visible part of a spectrum completely are displaced and leave the crystal as linearly polarized light. The value of a photocurrent depends not only on a direction of a plane of the E vector oscillation in it. Therefore before a photo cell it is necessary to establish depolarizer D, i.e. a matte glass plate. Under these conditions the photocurrent strength i can be considered proportionally to intensity of incident light i ~ ІА.

CL

P

А

D

2

3

4

5

6

1

7

К

Figure 5

Work sequence

1. To fix all parts of installation on the optical bench as shown in figure.

2. To turn on the source of light and regulate along height all parts of installation.

3. To turn on the source of light, to close the circuit of photocell and to write down the values of milliammeter i0 in the table 1. Presence of weak current i0 is due to influence of outside light sources in the classroom on photocell.

4.   To fix limb on zero-mark on a scale, to turn on the source of light. Smoothly rotating the polarizer to achieve the maximum deviation of the microammeter’s pointer im and wrote down in the table 1. Obviously it’s known that the planes of polarization of polarizer and analyzer are parallel to each other, i.e. φ angle between them is zero.

5. Not changing more the polarizer’s position to rotate analyzer within the limits of 00- 3600 writing every 100 down every angle φ and corresponding the photocurrent strength iк values of measurements in the table1.

Table 1

| № | φ(0C) | i0 | cosφ | cosφ | i-i |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 2 3 .. .. 37 | 0 10 20 .. .. 360 |  |  |  |  |  |

6. Considering the photocurrent is proportional to the incident light intensity we can write the next:

;

Hence, getting relation  , we obtain  =cosφ. This dependence is shown in the Fig.6. For checking the Malus’s law we have to draw in the figure 6 experimental values of   and  cosφ. It is possible to judge of the Malus’s law by that how well the experimental points cover theoretical dependence.

Control questions

1. Сan polarization of longitudinal waves be observed?

2. What difference of light natural from polarized is in?

3. Why is intensity of light proportional to a square of amplitude of a vector?

4. Deduce Malus’s law.

5. Will be light reflected from a matte surface (from a white paper) polarized?

References

1. Savel'yev I.V. Kurs obshchey fiziki. t.3. M .: Nauka, 1982, 304 s.

2. Yavorskiy B.M., Detlaf A.A. Kurs fiziki. ch.3.M .: Vysshaya shkola.1975-458 s.

3.Trofimova T.I. Kurs fiziki. Uchebnoye posobiye dlya vuzov. M .: «Vysshaya shkola», 1990.

5.Fizicheskiy praktikum pod redaktsiyey Iveronovoy M., «Vysshaya shkola», 1956

6.Laboratorskiy praktikum po obshchey fizike pod redaktsiyey Gershenzona Ye.M. i Malova N.N ..- M .: «Prosveshcheniye», 1985 g.

7. Yevgrafova N.N., Kagan V.L. Rukovodstvo po laboratornym rabotam po fizike M .: «Vysshaya shkola», 1970.

8. Butikov Ye.N. Optika.-M .: Vysshaya shkola, 1987.

9. Landsberg G.S. Optika.-M .: Nauka, 1976.

Laboratory Work № 20

Determining refraction index of liquid by refractometer

The aim of the work:  1. Acquaintance with the definition method of refraction index of substances, the device and principle of operation refractometer, the technical data and the device’s operational performance, acquiring of skills of work with refractometer. 2. Determining refraction index of liquid solution (sucrose, glycerin) and concentration of substances

Materials: Refractometer, a set of  with sucrose and glycerin solutions of the different concentration, a distilled water, a thermometer, a cotton wool.

air

water

Figure 1

Theoretical introduction. Refractometer is the device intended for determining refraction index of light rays in solid, liquid, gaseous bodies.

The determining method of refraction index of investigated material is put in a basis of refractometer’s operation by a limiting angle of refraction or total internal reflection.

At transiting a light ray from optically denser medium in optically less dense one, for example, from water in air, it is possible to observe the phenomenon of the total reflection. At total reflection the refracted ray slides on boundary of two media and the refraction angle will be 900 (Fig.1).

Here the angle i is a limiting one of total internal reflection. It is obvious that the light rays path obeys the reciprocity principle.

The basic parts of refractometer  are two prisms АВС and А′В′С ′ made of a glass with the big refraction index (Fig. 2).

Between these prisms it is possible to drop a researched liquid (n1) which uniformly flow in layer of thickness of 0.02 mm.

The upper prism side А′В ′ is made matte in order that rays going from the А′В′С ′ prism (lighting prism) disperse in liquid in every possible directions and fall on the lower (measuring) prism side АВ at different angles.

Let's choose a SO ray falling at an angle i to a normal of a surface of the lower prism and we’ll write down the light refraction law for it:

(1)

(2)

Where  i = an angle of incident ray on  investigated liquid layer;

r′ = a refraction angle of this ray in measuring prism on the BC side;

r= an incident angle of ray in measuring prism on the BC side;

i’= a refraction angle of the ray (in air) leaving the prism;

n1=refraction index of an investigated liquid;

n2 =refraction index of measuring prism;

nair=1 is the refraction index of ray in air.

For the ОDЕ triangle an external angle is DЕК=α=r+r′.  Then

sin ί′=nsin r'=n2sin(α-r')(3)

sin ί′=n2 sinα∙cosr–n2 cosα∙sinr'

Substituting (1) and (2) in (3) equation and intending the next we obtain

(4)

For rays going at the angle of i=900 and sliding along the measuring prism sides there are

i=i′

(5)

It is possible to show that at any other incident angles i< 900 the light rays leave the prism at the angles i bigger than i2. Thus, the i0 angle is limiting and the optical axis of a spyglass coincides with a direction of the limiting ray. Then all rays parallel to limiting will give the image in a focal plane on an optical axis of the spyglass.

The parallel rays going at angles big parallel, will give the image more to the right. Thus, one half (right) a field of vision will be light, since the parallel rays going at the angles smaller limiting corresponds,  other (left) half of field of vision will be dark. Position of border of a treatment of light and shade is determined by magnitude of a limiting angle. So the border of a treatment of light and shade is limiting rays. One can see from the formula (5) that the anglei depends only on the refraction index of an investigated liquid n1, since α and n are constants for the given device. Thus, position of the chiaroscuro boundary shade fixed on a scale also is determined a refraction index of a researched liquid.

Device’s Optical Scheme

Figure 3. 1-source of light; 2,3- condensers; 4-lightening prism; 5-measuring prism; 6-prism of direct vision; 7- catoptric prism; 8- objective spyglass; 9-power circuit; 10 –scale; 11,12-ocular spyglass.

Researched solution place between planes of 4 and 5 prisms. Light from a source 1 through the condenser 2,3 goes on an entrance side of a lighting prism 4, then passes a thin layer of a researched liquid and refracts on border of a researched solution to a plane of a measuring prism and leaves on a prism of direct sight 6. Then they are focused by an objective 8 telescopes. For fixing position of the chiaroscuro boundary concerning a motionless scale 10, the telescope rotates concerning an axis. Through an eyeglass 11 telescopes it is observed: the chiaroscuro boundary, grid crossing 9 and scales 10.

The scale is designed for the basis of the formula 5 and the miscalculation of a course of the beams which have left a measuring prism of beams. Two prisms 6 direct sights rotating concerning an axis of a telescope serve for indemnification of a dispersion of the beams which have left a measuring prism.

By rotation of these prisms are established spectral colorings around of border of a treatment of light and shade. Structurally the device consists of the following basic parts given in fig.

Work sequence

1. Set refractometer in convenient measurement position to switch on the network.

2. Draw dawn (Fig.3) the handle of the turned mechanism 14 and turn counter-clockwise the handle 11.

3. Check up the installation of refractometer’s zero - point state on distilled water (at research of distilled water the refractive index  should be equal to 1.3299 in n scales and 0% dried substances.)

4. Take a sample from the top chamber window, bottom windows should be closed.

5. Open the top chamber and wash out measuring and lighting prism surface by distilled water and wipe dry b a clean napkin.

6. Drip on plane of a measuring prism 1-2 drops of distilled water by stick end and close the top chamber.

7. Direct the light ray to the top chamber window 6 displacing lighter 4.

8. Enter in field of vision a chiaroscuro boundary moving the handle 11 with an eye-glass along a scale up and down

9. Set sharpness of the chiaroscuro boundary,scale lines and grid crossing by the observer’s eye byturning the eye-glass screws 10.

10.	Lead to the center of grid crossing the chiaroscuro boundary moving the handle 11; if at moving with the grid crossing centre it has gone through the scale mark n=1,33299 and 0 % of dried substances scale, zero -point state is established rightly. If it is not so the zero - point state setting is produced by the laboratorian or the teacher.

11. Measuring n researched liquids and percent of dry substances by saccharose make similarly to measurement of distilled water at the zero – point state setting.

Make the account by the refraction index scale and percent of dry substances by saccharose after matching (overlapping) the chiaroscuro boundary with grid crossing. Make measurements 3 times.

Average arithmetic of 3 measurements is final result.

Task 1

1. Determine the refraction index of water, glycerin, acetone.

2. Find dependence of the refraction index of various solutions of saccharose on percent (%) of dry substances.

3. Analyze the dependence of the refraction index.

4. Determine the refraction index of water solutions of glycerin of various given concentration. Plot a (calibrating) diagram of dependence n оn percent (%) of glycerin.

5. Determine the refraction index and defined the concentration of an unknown solution of glycerin with the help the (calibrating) diagram.

6. Register in the table measuring results.

The table of scale translation of percent (%) of dry substances by sucrose.

Table 1.

| № | Investigated material | Refraction Index | Arithmetical mean |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |

Control questions

- What is absolute refraction index?

- What is relative refraction index of a medium?

- How is refraction law expressed in this case?

- What phenomenon is put in basis of refractometer’s operation?

- In what case the total internal reflection is observed?

- Why are surfaces of prism made lustreless?

- How is concentration of the glycerin solution (Сх) determined?

References

1. Sivukhin D.V. Obshchiy kurs fiziki.-M .: Nauka, 1977-1986, t.1-5.

2. Savel'yev I.V. Kurs obshchey fiziki. t.3. M .: Nauka, 1982, 304 s.

3. Yavorskiy B.M., Detlaf A.A. Kurs fiziki. ch.3. M .: Vysshaya shkola. 1975.– 458 s.

4. Trofimova T.I. Kurs fiziki. Uchebnoye posobiye dlya vuzov. M .: «Vysshaya shkola», 1990.

5. Fizicheskiy praktikum pod redaktsiyey Iveronovoy M., «Vysshaya shkola», 1956 g.

6. Laboratornyy praktikum po obshchey fizike pod redaktsiyey Gershenzona Ye.M. i Malova N.N ..- M .: «Prosveshcheniye», 1985 g.

7. Yevgrafova N.N., Kagan V.L. Rukovodstvo po laboratornym rabotam po fizike M .: «Vysshaya shkola», 1970.

8. Butikov Ye.N. Optika.-M .: Vysshaya shkola, 1987.

9. Landsberg G.S. Optika.-M .: Nauka, 1976.

10.Gadzhayev N. Optika.-M: Vysshaya shkola, 1979.

Laboratory work №21

Study of Photoeffect Basic Law

The aim of the work: 1.	Investigate the photocurrent dependence on brightness of photo cathode. 2.	Investigate the photocurrent dependence on amount of applied voltage to photocell.

Materials: Optical bench with located on cursors by photoelectric cell and incandescent lamp, battery eliminator, rheostat, micro ammeter, voltmeter.

Theoretical  introduction. Phenomenon of electron tearing from solid under the influence of light is called photoeffect.   Electron tearing from body surface under the influence of light is often called extrinsic photoeffect. In intrinsic photoeffect, electrons relinquished by light remain inside of body. At the same time the body conductivity changes. Laws of extrinsic photoeffect were determined by A.G.Stoletov (1888 -1890).

Investigation of extrinsic photoeffect is produced in the scheme of figure 1. Monochromatic flow of current is directed to examined bar K, placed in the evacuated vessel B. Bar K is accepted to call photoelectric cathode. Electrons shed due to photoeffect, traverse under the influence of electric field to anode A. Current appearing in the circuit is measured by galvanometer.  Electrons released by light are accepted to call photoelectrons and electric current arising in the device under the influence of light is accepted to call photocurrent.  It is possible to change the voltage between anode and cathode with the help of potentiometer R.

А

К

+

–

R

Figure 1.

V

А

Light

Main features of photoeffect can be clarified with the help of voltage-current characteristic – curve, expressing the photocurrent dependence on potential difference between anode and cathode.  Figure 6.2 shows curve, corresponding to different brightness of cathode Е1 and Е2, Е2>Е1, (light frequency in both cases is identical).

Is2

Is1

Е2

Е1

Uз

U

I

Figure 2.

2 features are characteristic to curve:

1) In increase of promoting difference of potential current U reaches the saturation;

2) There is such a sense of stopping potential difference U3 in which the current is shut.

Saturation current is amounted when all the electrons torn by light reach anode:

(1)

where, е– electron charge; n – number of photoelectrons, outcoming from cathode for 1 second.

Existence of photocurrent in the area of negative voltages from 0 up to U3 is explained that photoelectrons knocked out by light from cathode have nonzero initial kinetic energy.   Due to decrease, the electron energy can perform the work against strength of stopping electric field and reach anode. It is evidently that maximal basic speed   of photoelectrons is connected with U correlation:

(2)

where, е and m – absolute value of charge and electron mass.

Particular interest represents the photoelectron dependence velocity from light frequency. The experience gives the dependence of stopping potential from light frequency: velocity dependence from frequency can be found on the base of correlation (2). Numerous measures have shown that U3 is linear function from frequency,

(3)

where, k  and 0  are direct, at that the sense  k  does not depend on the type of material. Data obtained by Milliken are given on the figure.6.3. As stopping potential U is connected with velocity of electrons correlation  (6.2), then from  (6.3) it is followed: photoelectrons velocity is more than frequency of absorbing light. Correlation (6.3) can be given by the other type putting instead of  U3 its sense via   at   (2)

(4)

Figure 3.

From   (4) it is followed: initial kinetic energy of photoelectrons increases linear with frequency ν.

From correlations (2) and (3) one more consequence comes: in order electrons to be able to break away light from body, the frequency must satisfy the inequality

.

Value   is called  "red border" of photoeffect:

The following basic laws of extrinsic photoeffect were established by experience.

1. Number of photoelectrons and, torn from cathode for unit time is proportional to light intensity (saturation photocurrent is proportional to brightness Е)

2. Maximal initial velocity of photoelectron is defined by light frequency and does not depend on its intensity.

3. Photoeffect “red border” exists for each substance, in other words, minimal frequency of v0 light, in which extrinsic photoeffect is possible some more.

Value v0 depends on chemical nature of substance and its surface condition.

Photoeffect laws conflict with concepts of wave theory of light. According to these concepts, under the influence of electromagnetic light wave, substance electrons have to execute forced oscillations with amplitude proportional to wave amplitude. In enough intensity of oscillation, the connection with electron substance can be disturbed and electrons can escape out with velocity, the value of which must depend on amplitude of impinging light (in other words, on its intensity).  Indeed, there is no such dependence; electrons’ velocity depends only on frequency of impinging light.  .

Explanation of quantitative patterns of photoeffect can be given on the base of quantum concepts about light. Einstein had shown that all the patterns of photoeffect can be easily explained, if we suppose that light is absorbed by such portions – quantum, according to Plank it is shed.   These quanta of electromagnetic emission with frequency were called photons. For monochromatic emission with frequency v all the photons have the same energy which is equal hv. According to Einstein’s idea, the energy obtained by electron is delivered to it as quantum hv, which is fully adopted.

Let’s consider the extrinsic photoeffect in metals from quantum point of view. It is known, to exit from metal electron must execute the work of exit А. The electron obtains energy hv as a result of photon absorption. If hv>А, then electron can execute the work of exit from metal.  Energy excess is kept in it in the form of kinetic energy.

(5)

Correlation (5) is called Einstein equation for extrinsic photoeffect.

Equation (5) easily explains all the laws of extrinsic photoeffect in metals. Really from (5) maximal kinetic energy depends not on intensity but on light frequency. According to Einstein “red border” of photoeffect happens from condition. Consequently, frequency corresponding to “red border” is equal:

(6)

It depends only on value of electron exit work, in other words on chemical nature of substance and its surface condition.

At last, total number of photoelectrons escaping for 1 second is proportional to number of photons n′, impinging on the metal surface at the same time. For flat cathode even lightening by monochromatic light with frequency ν

where, Е is the brighness proportional to light intensity, in other words, in accordance with the first law of photoeffect, the number of photoelectrons escaping from cathode for unit time is proportional to light intensity.

Installation description.Vacuum photoelectric cell with cesium-antimonide cathode is used in the work. This photoelectric cell represents glass evacuated specific balloon, anode is located in its center. Thin layer of antimony is deposited on one the half of inner surface, and thin layer of cesium by means of consequent steam condensation of these metals in vacuum. Formed connection of antimony and cesium serve as photocathode of photoelectric cell. Red border of photoeffect for this cathode is located at visible part. Principal scheme of turned on photoelectric cell is given in the figure. 6.4. Photoelectric cell is placed in the sun-protecting case. Light access is closed by shutter, (shutter is opened only at the moment of measuring).

Figure 4.

.

+

–

R

V

А

Work sequence:

Exercise  1

Study of Photocurrent Strength Dependence on Photocathode Brightness

Essence of this exercise is to change the photocathode brightness according to a definite law and conduct the measuring of current strength parallel.

1)	Lamp is placed at a distance 40cm from photoelectric cell.

2)	Voltage about 150 W falls on photoelectric cell with the help of rheostat, voltage-divider in order to provide its work in the regime of current saturation in advance.

3)	It makes sure that in closed shutter of photoelectric cell through micro ammeter does not exceed 1-2 scale division in these cases.

4)	Condition of photoelectric cell on the optical bench, voltage on photoelectric cell is written after installation of work regime.

5)	Shutter of photoelectric cell is opened and the micro ammeter reading is written.

6)	Removing by stages for 5 cm each time, photoelectric cell from lamp up to optical bench end, the micro ammeter reading is written for each photoelectric cell condition.

7)	Then the same measurements are repeated returning the photoelectric cell to initial condition. Measurementsarewritteninthetable 1.

Table 1

| № | U | Rm | Rn |  | I1 | I2 | Iav |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

8)	Average results of executed measurements are developed by the following way. Accepting the lamp to exact source of light, it is possible to consider that photocathode brightness changes back proportionally to distance square of lamp from photoelectric cell. May Е1 is photocathode brightness in minimal removing from photoelectric cell, then brightness of photocathode in some conditions of photoelectric cell will be equal:

where Rm  – maximal photoelectric cell distance from lamp, Rn – distance of  п - condition.

9)	Laying by abscissa axis of rectangular coordinate system quantity are proportional to , and by ordinate axis  - photocurrent quantities, corresponding to senses , and connecting by continuous line,  obtained in that way dots, we obtain graphic photocurrent dependence on relative photocathode.

Exercise 2

Obtaining the volt-ammeter characteristics of vacuum photoelectric cell

Current strength dependence on the value of laid on the photoelectric cell (in constant photocathode brightness) of voltage is called volt-ammeter characteristics of photoelectric cell.

The following measurements are executed for each lamp condition placing a lamp consequently at distances 35, 45, 60 cm from photoelectric cell.

1)	Zero voltage is fixed at photoelectric cell.

2)	Change the value of supplied voltage for photoelectric cell   gradually by stages.

3)	Micro ammeter readings for each sense of fixed voltage are written. Voltage at photoelectric cell is increased in that way to the point of obtaining the photocurrent saturation evident effect.

4)	Obtaining saturation current, the voltage is decreased in the process of voltage increase stage by stage. Measurementsarewritteninthetable 2.

Table 2.

| R1=55 сm |  |  |  | R2 = 45 сm |  |  |  | R3= 60 сm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U | I1 | I2 | Iav | U | I1 | I2 | Iav | U | I1 | I2 | Iav |

5)	Measurement results are presented as graphics, describing photocurrent dependence on value of applied voltage at photoelectric cell    (all the three graphics are constructed on the same coordinates).

Control questions

  - Wherein lies the photoeffect phenomenon?

  - What types of photoeffect are distinguished?

  - What are the main laws of photoeffect?

  - Can the photoeffect phenomenon be explained by light wave theory? Where is the contradiction of light wave theory with photoeffect laws?

  - How does the quantum theory explain the photoeffect phenomenon?

  - What are the uses of photoeffect in engineering?

References

1. SivukhinD.V. Obshchiy kurs fiziki.-M .: Nauka, 1977-1986, t.1-5.

2. Savel'yev I.V. Kurs obshchey fiziki. t.3. M .: Nauka, 1982, 304 s.

3. Yavorskiy B.M., Detlaf A.A. Kurs fiziki. ch.3. M .: Vysshaya shkola. 1975 g. - 458 s.

4. Trofimova T.I. Kurs fiziki. Uchebnoye posobiye dlya vuzov. M .: «Vysshaya shkola», 1990.

5. Fizicheskiy praktikum pod redaktsiyey Iveronovoy M., «Vysshaya shkola», 1956 g.

6. Laboratornyy praktikum po obshchey fizike pod redaktsiyey Gershenzona Ye.M. i Malova N.N ..- M .: «Prosveshcheniye», 1985 g.

7. Yevgrafova N.N., Kagan V.L. Rukovodstvo po laboratornym rabotam po fizike M .: «Vysshaya shkola», 1970.

8. Butikov Ye.N. Optika.-M .: Vysshaya shkola, 1987.

9. Landsberg G.S. Optika.-M .: Nauka, 1976.

10. Gadzhayev N. Optika.-M: Vysshaya shkola, 1979.

Applications

List of Physical Constants

Prefixes for Powers of Ten                   Approximate Coefficients of Friction

Wavelengths of Light in a Vacuum               Resistivities at 20°C

Absolute Indices of Refraction                 Circuit Symbols

Heat Constants

Approximate Coefficients of Friction

Bakirzhankyzy A.,Toshboltayeva N.N., Turlybekova G.E., Orazalieva R.N.

Practical Physics

Signed for publication 2019.

Format Х*Y 1/16

Offset paper. Volume 6  p.p.

Edition 92 pcs.

©South Kazakhstan State University named after M.Auezov

M. Auezov South Kazakhstan State University Publishing Center, Shymkent.

Tauke khan 5 street.