Ф.7.03.-03

ARKHABAYEV A.B., UMBETOV A.Zh., MAKHANBETOV A.Zh.

METHODICAL INDICATIONS

for laboratory workon discipline

«АUTOMATED CONTROL SYSTEMS FOR MEANS AND SCIENTIFIC RESEARCHES IN COMMUNICATION SYSTEMS»

Shymkent - 2020

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

M.AUZEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

Chair of: «Automation, telecommunication and control»

Methodical indications

for laboratory workon discipline

«АUTOMATED CONTROL SYSTEMS FOR MEANS AND SCIENTIFIC RESEARCHES IN COMMUNICATION SYSTEMS»

for students of the specialty 5В071900 (6B06210) – «Radio technology,electronics and telecommunication»

Shymkent, 2020

UDC 681.5.01

Methodical indications for laboratory work on discipline «Automated control systems for means and scientific researches in communication systems» for students of the specialty 5В071900 (6B06210) – «Radio technology, electronics and telecommunication». Shymkent:  M. Auezov SKSU,  2019 -76 p.

Methodical instructions are made according to requirements of the curriculum and the program of discipline “Theory of Automatic Control” include all necessary data on performance of Laboratory work of a course.

Methodical instructions are intended for students of specialty 5В071800 – «Power Engineering».

Methodical instructions contain the purpose, themes and tasks for performance of Laboratory work.

Reviewers:

Arystanbaev K.E. - cand.tech.sci., the associate professor of M.Auezov SKSU

Are considered and recommended for printing by chair meeting of Automation, telecommunication and management (the report № __ from __ ___ 2020) and Methodical provision of higher school “Information technology and energetic” (the report № __ from __ 2020).

It is recommended to the edition Methodical council SKSU of M.Auezov. The report № __ from __ _______ 2020.

M.Auezov South Kazakhstan State University, 2020

Responsible for release: Arkhabayev A.B.

Contents

Laboratory work 1 "Introduction to Laboratory program complex"	8

Laboratory work  2 "Study and computer modeling of connecting processes in cascade automatic system of the regulation"	28

Laboratory work 3 "Study and computer modelling connecting processes in multifunction automation system regulations"	35

Laboratory work  4 "Study and computer modelling connecting processes in digital automatic system of the regulation"	39

Laboratory work №5 «A calculation and investigation of algorithms of filtration and starting	49

data transformation in ASCTP signals»	49

Laboratory work №6 «Research and computer simulation of algorithms of optimum allocation of restricted resources between parallel switched on technological objects in ASCTP»	59

Criteria for assessing students' knowledge	71

INTRODUCTION

Regularity or adjustable parameter is the parameter of the technological process that can be constantly maintained or changed according to certain law. The value of the regulated value is the value at that time.

The value of the adjustable value of any measuring device at the time of its consideration is called the measurement value. Control object (object of regulation) is a tool that is used with the special mode of operation required by the controller. Management - Adaptation of controls to ensure that the operating mode is required. Adjustment is a particular case of management, which should ensure that the output of the BoD is stable. Automatic control - ensuring the control without any involvement of the person. Input Effect (X) is the effect of input to the system or instrument. Output Effect (Y) is the effect of the system or instrument output. External Impact - Influence of the external environment on the system. Customer Impact (such as X Income) is the effect that determines the law of the required change in the regulatory value of the system. Managing Impact (u) is the effect of the controller on the control object. Controller (CG) is a controller for controlling the operating mode of the BD. Scarcely exposed (f) - the magnitude of the regulation and the required magnitude between the functional impairment and the tactile effect. Management error (e = x - y) is the value of the adjusted value (x) and (y) the difference between the actual values. Regulator (P) is a set of tools that are connected to an adjustable object and that automatically adjust the required adjustment or automatically change it under certain law. The automatic control system (ARS) is a closed closed circuit system, where (u) the control is generated as a result of the comparison of the given x value with the actual value.

The optional feedback link (CB), which is targeted at the input from the portion of the site considered in the ARS structural scheme. Feedback can be positive or negative.

The discipline «The theory of automatic control of linear systems» teaches the principles of automatic control applied in engineering, types of automatic control systems, mathematical apparatus of investigation of non-linear ARS, basic elements and characteristics of ARS, methods of analysis of stability and management quality, correction of properties of dysfunctional ARS.

Laboratory work 1 "Introduction to Laboratory program complex"

Introduction information about complex Laboratory work. The problem questions: types of the mathematical models plant and managerial system and ways of radiation and transformation; the methods of the processing experimental given on removing the dynamic features plant control; the mathematical models automatic controller; the methods and algorithms of simulation modeling of the connecting processes in system subjects of control; the staff and application of the department and student fund about the using of software for personal computer on discipline ATTP; the requirements to order of the registration of the report.

1.1 Goal of the work.

Information about the Laboratory work which is considered general principles of the using the  personal computer during the  study of the sections of the disciplines ATTP, LSA, ATS in accordance with calculation and study of the automatic systems of the regulation (ASR):

- types used in mathematical models  of the technological plant control (TBC) and ASR, ways of their radiation and transformations;

- methods of the processing experiment given on removing the dynamic features plant control;

- mathematical models of automatic controller;

- methods and algorithms simulation modeling the processes in managerial system;

- composition and use of the student software (FSPO) for personal computer;

- requirements to order of the registration of the reports;

For performing the work, it is required 4 hours of Laboratory lessons and 4 hours of the individual lessons with teacher.

1.2 Task for Laboratory work

- study the contents of the methodical instructions;

- get acquainted with the main terminology and conditional importance, doing algorithms and Laboratory work about the using  of software of the discipline;

- using program LAB5, imitating work (valid) with the object, remove the curves of the runaway on channel: the main, additions and indignations;

- define the parameters transmission function on the channels;

- check the adequacy of the mathematical models;

- arrange and protect the report on Laboratory work.

1.3 Main positions

1.3.1 General information

The Component part of discipline ATTP, LSA, ATS is a calculation and study of mono contour and multi contour analogue and figural control system.

The main problems are:

- reception of the identical mathematical models of the speakers’ think;

- reception of the identical mathematical models of the static TBC

- calculation of the parameter controllers or micro-controller, realizing algorithms of the regulation;

- reception of the mathematical models of connected ASR on given channel for mono contour and multi contour ASR;

- choice and realization (or use) of concrete method of  simulation modeling of the connecting processes, most comfortable realized on computer;

- estimation quality of the connecting processes in connected ASR.

As for the instrument of relieving decision of the tasks of the calculation on construction site and simulation modeling of the connecting processes in ASR is a program designed FSPO discipline, the main composition, which is brought in table 1 below. There are descriptions of the programs on pulpit and algorithm which are considered as their base. Besides, these algorithms are studied in lecture course of discipline and maintain in description of this and the following Laboratory work.

1.3.2 Recommended and used conditional marks.

1.3.2.1 Objects, controllers and systems as a whole.

U- input (or vector input) of the plant or systems, Control parametres;

UР - an output (or vector outputs) controllers, adjusting influences;

X - an output (or vector output) of the plant or systems, State variables;

V - an indignations (or vector of the indignations); X* - a tasks;

E, e - an error, unbalance (E = X - X*)

W, W(p) - Plant  transfer function;

W*, W*(p) - a transmission function of the plant  disregarding  slow;

W1, W1(p) and etc - a transmission functions of the plant  in multi contour ASR;

W, W(p) - a transmission function of the connected ASR; i - a serial number W, R, element vector U, X, V;

m - an order of the multinomial in denominator W(p) of the type (1)-(3), (6); n - an order of the multinomial in numerator W(p) of the type (1)-(3), (6), (n>m); ai - a factors of the multinomial in numerator W(p) of the type (1)-(3), (6), i=1,2, n; bi - a factors of the multinomial in denominator W(p) of the type (1)-(3), (6), i=1,2, m; R, R(p) - a transmission function of the controller;

R1, R1(p) and etc - a transmission functions of the controller in multi contour ASR;

SП; SИ; SД - P, I, D parameters of the adjusting the analog controller;

S1; S0; S2 - P, I, D parameters of the adjusting the controller, used in[1, 2] referred to.(15); KP - a factor of the transmission;

TI - constant time of the integration (time of isodrom);

TD - constant time of the differentiation (time anticipate the thread); TF - constant time of the aircushion (the filter); C - a gaining of the  factor of the object;

D - a gaining of the  factor of  integral formation for W(p) of the type (6) T - constant time for equation of the type (5), (7);

T1, T2, T3 - constant time for equation of the type (4); TБ - constant time for W(p) without self leveling type (7); τ	- Delay time

1.3.2.2 Frequency features.

ω;   ω*;   ωраб	- frequency; critical frequency; valid frequency; ω1 ; ωк ; ∆ω	- starting and final frequencies; frequency step; N  	- an amount of points of frequency;

ϕ ;  ϕоб ;  ϕрег	- phase (PhFC); PhFC of the object; PhFC of the controller; A ;  Aоб ; Aрег – amplitude (AFC); AFC of the object; AFC of the controller;

J  - Imaginary unit ( −1); P  - operator of Laplas;

W(jω) ; R(jω) 	 - AFC of the object; AFC of the controller; Im  ; Re 	- Imaginary; valid part;

ImW  ; ReW 	- Imaginary ; valid part of the AFC object;

ImR; ReR- Imaginary; valid part of AFC controller;

ImЗн  ; Re den 	- Imaginary; valid part of AFC indicator of the transferable function; ImЧис  ; Re den;   - Imaginary; valid part of AFC denominator of transfer function;

ϕзн ;    A den 	- PhF; AFC of the denominator of transfer function; ϕчис  ; A den 	- PhFC; AFC of the denominator of transfer function; ϕзс  ; A den 	- PhFC; AFC of connected ASR; m  	- degree of waving for espand AFC; 1.3.2.3 Digital models and systems.

j - a number of the point on time;

N - an amount point on time;

Xj; Uj - importances of the output and input at the moment of time j; Tо- period of the sensor functioning; the step(the tact) quanting on time;  about a frequency of the questioning of the sensor ( =2 /T); A; B - a factors recurrence equations of models of the object;

D - slow, equal integer count; calculate; list tact of quanting on time (d = /Tо); QП; QИ; QД - P, And, D parameters of the adjusting the digital controller;

1.3.2.4 . Connecting processes and their quality

T  	-  current time;

T1 ; Tк ; ∆T 	-  time of beginning; final time; step in time;

N  	- point amount of time;

NФ		- maximum number of the ,member of decomposition into row of Furie and method

of Akulishin;

∆U 		- amplitude of step like indignation at the input; xст		- static (set) mistake;

ψ	- degree of fading;

ТР	- time of regulation, length of transferable process; η	- over regulation.

1.4 Mathematical models of plant control.

1.4.1 Mathematical models of  speakers sink.

The Mathematical models speakers TBC are central points for decision of the tasks of the analysis and syntheses ASR. Their types and ways of the reception are considered in discipline "Theoretical bases of automation" (TBA), "Identification plant and control system" (IOCS), "Theory of the auto control" (TA).

The following types of transmission functional purpose are used during the Laboratory work: For plant with self leveling in general type:

| . |  |  |  |
| --- | --- | --- | --- |
| W(p) = C⋅ b1 + b2 ⋅p ++ bm+1 ⋅pnm ⋅exp(−p⋅τ) . a1 + a 2 ⋅p ++ a n+1 ⋅p Using of W(P) «method of squares» [3-8] (b1=a1=1): |  |  | (1.1) |
| W(p) = C ⋅ b1 + b2 ⋅ p 2 ⋅exp(−p⋅τ) , or a1 + a2 ⋅ p + a3 ⋅ p |  |  | (1.2) |

W			(1.3)

| Other type may also be used, for example: |  |  |  |  |
| --- | --- | --- | --- | --- |
| W(p) = Cn⋅exp(−p⋅τ)ит.п. (1 T p) |  |  |  | (1.4) |

∏ +	⋅

i

Simple expression of a periodic position with slow is used in Laboratory works №3 и №4 :

W(p) =	C	⋅exp(−p⋅τ)						(1.5)

1+T ⋅ p

For the objects without leveling we may use W(p) in general case:

W(p) = Dp −C⋅ ba11++ba22 ⋅⋅pp++++bamn++11 ⋅⋅ppnm ⋅exp(−p⋅t)		(1.6)



Or simple expressions:

W(p) =	C	⋅exp(−p⋅τ)				(1.7)

p⋅TБ⋅(1+T⋅p)

or:

W(p) =	C ⋅exp(−p⋅τ)					(1.8)

T⋅p

The Concrete type W(P) is chosen from condition of the provision adequacy (refer to.[6, 7]) and comfort of the calculations. The Raw data for output W(p) can be analytical expressions, describing processes of the accumulation material and energy, for instance the differentials of the equation material and heat balances [1, 8]. Experimental ways of mathematical modeling speakers are basically used in given LaboratoryLaboratory work. The are based on processing of the  result experiment on removing the crooked runaway [2-8]. The Procedure of the reception of the crooked runaway was described in section 1.4.3 below and in [2, 5, 6, 7]. The Methods to identifications were provided in [2-8] in Laboratory works, except  3 and  4 factors, falling into (2) graphically analytical way, as this is shown on Figure  2.4 in [10] and on Figure  1.49 in [1]. Importance gain factor with the attitude is defined as the importance of output-ranks to amplitude outraging influences at the input U. In Laboratory work 3 and  4 types and parameters W(p) are defined by the "method of the areas" [3-8].

The Design value of the leaving the plant under step-like indignation at the input, necessities for check of adequacy, possible and applicable to expression W(p) conversion Laplasa, for instance, using program complex Mahtcad [13-14] or applicable formula of the decomposition Hevisayda [6, 11]. The Root natures of the equation required for this, not difficult get, using complex Mahtcad (refer to. [12-14]). The Good results for calculation of the leaving the plant gives use the method of the material frequency features with using the complex Mahtcad (refer to nor- and application

1). For equation of the type (5) output is easy defined on formula analytically:

X∧ (t) = C⋅∆U⋅1−exp(− t T− τ)					(1.9)



а для W(p) вида (4) при n=2 ( [8], pg. 112):

X∧ (t) = C⋅∆U⋅1− T1 T−1T2 ⋅exp(− tT−1τ) + T1T−2T2 ⋅exp(− tT−2τ) (1.10)



but for W(p) of the type (4) under n=2 (refer to. [8], p. 112):

If importance , found on the following formula is not risen than 3-7%, that model is considered identical:

100%	N X

δ =	N	⋅∑i=1	iXi	i			              (1.11) where N - an amount used point of the crooked runaway.

The Ways of the calculation of the leaving the plant for the other type W(p) and about-верки to adequacy refer to.[2-8].

1.4.2 Receptions of the mathematical models of the static

The Mathematical models of the static TBC at calculation and investigation ASR are used less. Their are more often got statistical about the results of  active or passive experiment. In the discipline, these tasks are considered enough in detail [9, 6, 7]. The Raw data for mathematical modeling one hundred-teaks usually are tabular or graphic dependencies between the input and output TBC in formed mode. This is a reception significant factor linear or nonlinear algebraic equations or systems of the equations, identical considered regularity. Mathematical models of static are often used in the manner of multinomial: length of the row Tailor, in which decomposes the unknown function [9] (the event with 1 you-move and with To input):

k	k	k

x = a 0 + ∑ai ⋅ui + ∑ai ⋅ui ⋅ul + ∑aii ⋅ui2 + (1.12)

i,l=1	l,i=1	i=1 i≠l

At type of the equation to models can be others, for instance reflecting physical essence considered by pouring. The Factors to mathematical model and their value are defined at the methods of regressive, correlating analysis and others.[6, 7, 9]. The Program ALAB10 FSPO suggests in mode of the dialogue quickly to define the factors to un variable model of the static of the different type and graphically demonstrated about the  result of modeling graphic. Besides, for decision of the tasks of mathematical modeling of the static possible to use pulpit such as specialized program, as Statistica, SPSS, Graphit 99 Pro, SuperGraph, CurvePlot, SigmaPlot and others, as well as systems Mahtcad,. MAHTLAB, Matematica.

1.4.3 Reception of the raw data for Laboratory work.

The Source curves of the runaway, being central to determination transmission function plant are got by use the program LAB5 (refer to. [6]). After start the program necessary you-take the menu item a REGISTRATION. It is necessary to assign the correct number of the credit books and to check the correctness of the determination factor W(p). The Numerals must be exactly six. If number consists of five numerals then before the first numeral it is necessary to add the numeral 9. In step of EXPERIMENT first it is necessary to choose the channel of the fixing of the indignation, having used keys of cursor control and having pressed cue-вишу "ENTER". Necessary consecutively to conduct the experiment on removing the crooked runaway on three channels (refer to. Figure 1):

" main (main) channel regulations - an input U, output X (for determination W(p));

" auxiliary channel - an input U, output X1 (for define-thread W1(p));

" channel of the indignation - an input V, output X (for determination W2(p)).

The Indignation at the input possible to assign alike on all channel, for instance 10% move RO. Hereon possible proceed with experiment that. "Start - a halt of" experiment is realized by key "GAP". Time is on the right higher part of the screen showing current time of the experiment. Importance you-move (the ordinates point crooked runaway) are displayed on meter instrument, conditionally expressed in lower part of screen. After halt of the move of the experiment it is necessary to write importance of the output and time for the current point of the crooked runaway. It is Enough to remove 15-20 points of the crooked runaway. Type of the typical screen is shown On ris.1. about-grams LAB1 after termination of the experiment. For the following about-работки result experiment are used only importance from-bending the input and output from initial formed with-standings. So necessary to bring the result in table 1.1.  The Table 1.1.

| Time | Output | Output refuse |
| --- | --- | --- |
| t1 | x1 | 0 |
| t2 | x2 | x2 - x1 |
| t3 | x3 | x3 - x1 |

Figure 1.1 - the Instance of operation program LAB5

1.5 Mathematical models analogue controller.

In practice automatic controllers are used with distinguishing algorithm work, as well as optimization controllers without hard structure and others.[1]. However mostly, the controllers are used in realization standard P, PI, PD, PID - regulations. The Equation PID - a controller as functions of time possible to write:

TP

Up (t) = S ⋅E(t)+S ⋅ ∫E(t)⋅dt +S ⋅ dEdt(t)	(1.13) that corresponds to the transmission function:

.  			R(p) = SП+SИp+SД⋅р	(1.14)

Sometimes [1,2] use other indications, for instance:

R(p) = −(S1 +S0 p+S2 ⋅р)  	(1.15)

The Sign minus in (1.15) emphasizes that controller influences on principle to negative feedback. To use the type R(p) with sign plus, then no need to change general taken expressions for transmission function of connected systems with feedback.

Often applicable other form R(p), so called standard "scholastic" analog PID - a controller:

R(p) = KП⋅(1+	1	+	TД⋅p ) (1.16)

TИ⋅p	1+ TФ⋅p

where TФ=TД/NФ, where TФ=3..10.

We shall Note that in law of the regulation for real industrial of the systems are used other indications for parameter on-construction site such as KP, TI, TD, TF and others. They correspond to define the structural scheme of controller and management body in designs of the controller. Knowing the importance of SP; SI; SD and real functions it is  not difficult to do the recalculation of importance coefficient  KP, TI, TD. Importance of the TF usually find the special methods by method or by selection. Need of the entering in R(p) of the filter, for instance with transmission function in the manner of a periodic link:

WФ(p) =	1				(1.17)

1+ TФ⋅p

it is caused that that operation of the differentiation much feeling-flesh to hindrance.

Are they Below brought transmission functions controller some industrial systems.  The Controller RBA systems AKESR-M:

R(p) = K П+ K П	+ TД⋅p	(1.18) Controller systems START-2:

TИ⋅p	1+ TФp

R(p) = KП+	1	+ K П⋅TД⋅p 	(1.19)

TИ⋅p

The Controller R-17 systems KASKAD-2:

R(p) = 1 KTФП p ⋅1+ TИ1⋅p +TД⋅p (1.20)

+	⋅	

Digital PID - a controller:

Uj= Uj-1 + Qп E j+ Qи Ej-1 + Qп Ej-2 (1.21)

1.6 Simulation modeling of the connecting processes in ASR.

1.6.1 Simulation modeling and mathematical models of connected ASC.

Simulation modeling of the connecting processes is conducted for check of correctness and acceptability result is executed in calculation of ASR and estimations quality regulations. For the realization of simulation modeling it is necessary to:

- get the mathematical model, describing speaker of the closed system (for instance, in the manner of differential equation, transmission function, frequency nature of statistic, differences of the equation etc;

- choose the method of the decision of mathematical model of the closed system under initial condition;

- define the parameters required for realization of used methods of the decision. So for mono contour connected ASR on channel of the indignation:

WЗАМ (р) =	W(p)

1+ W(p)⋅R(p)  	(1.22)

if we take input U for output X (i.e. indignation is inflicted on the part of RO, but controller must

provide return the output X to task X*), or on channel of the task of the type:

WЗАМ (p) =	W(p)⋅R(p)	,

1+ W(p)⋅R(p)	                                                                      (1.23)

If we take input X  for output X, (i.e. task H is changed* the controller and it must provide the achievement of importance of the output H equal new importance H*). For cascade connected ASR on channel of the indignation:

WЗАМ (p) =	W(p)

1+ W(p)⋅R(p)⋅R1(p) + W1(p)⋅R1(p) (1.24)

and on channel of the task:

WЗАМ (p) =	W(p)⋅R(p)⋅R1(p)

1+ W(p)⋅R(p)⋅R1(p) + W1(p)⋅R1(p)           (1.25)

For multifunction Closed ASR on channel of the indignation:

WЗАМ(p) = ⋅[W1(p) + W(p)⋅R1(p)]

	                                           (1.26)

The Student must know how to remove the concrete expression WЗАМ(r) for under investigation ASR.

From large number of the known methods of the finding of the decision mathematic model of the closed system (i.e. determinations accounting-go importance of the output closed ASR) we stop on frequency method, as having sufficient accuracy and easy realized numerically.

1.6.2 Buildings of the connecting process by method Akulishin .

One of the uncomplicated methods of the building of the connecting process in automatic system of the regulation is a method Akulishin . This approximate method requires the knowledge of AFH and FCHH closed ASR and is founded on use the decomposition in row Furie input and output signal of the system [2, 6].The Known that periodic function can be distributed in row Furie. On ris.2 are shown three first harmonicas F1, F2, F3 decompositions to functions of the type rectangular wave with amplitude A=1 and period 6.28. It provides with enough amount of count; calculate; list members of the decomposition can be reached any accuracy to source periodic function by amount of the harmonicas.

Essence of the method Akulishin consists in that, in the beginning write expression of the decomposition in row Furie input influence (27)

Then with provision for that that for linear systems of the output signal will change only amplitude and phase for input signal, is removed sought expression for leaving the system(28).

The Output value x(t) shall consider as periodic function since period To in 4-6 once big time of the connecting process in system. Herewith conditionally consider that in current time nor smaller, than To/2 amplitudes of the indignation + U, but in current VWV tell, the menu is greater lied, than To/2 amplitudes of the indignation - U. On Figure  3. are shown such input influence and reaction on it. Range of time must be used For calculation from 0 before point "a". Starting  from point "a" before point "b" is got mirror (artificial) of the reflection graphics connecting process, as reaction on - U. Reeked-live such input influence a row Furie:

u	⋅ ∑ sin(i⋅ω0 ⋅t)  	i=1,3,5,...		(1.27) where i=1, 3, 5, ,N - increasing pleased uneven numerals (N=19 23).

Since considered system suppose linear, that waving of the output value possible to present the length of the row:

x(t) = 2⋅∆U ⋅ =∑Nф	АЗС (ω0 )⋅sin(2i⋅⋅iω−01⋅t + ϕЗС (ω0 )), 	(1.28)

π	i 1,3,5,...

Figure 1.3. The Graph, explaining building of the connecting processes on method Akulishin :T0 - a period to functions, made from transferable processes ASR;   T0/2 - duration of the connecting process

The Calculation of the connecting process produces using formula (1.28) ZS - FCHH and AZS - ACHH closed ASR must be computed for frequencies 0, 3 0, 5 0 and t. d., for instance before 21 0. Importance 0 definition as rab/6, where slave is defined at calculation adjustment [2].

1.6.3. The Buildings of the connecting process by method material  of the frequency features In base of this method as one should known from course "Theory automation of control" [5] expression, applicable, for instance, in the. "method trapezoid" for determination of the connecting feature of the firm system:

X	2⋅∆U ωS ReWЗАМ (j⋅ω)sin(ω⋅t)dω	(1.29)

ω

The Main difficulties of the use of this method at calculation manually:

- difficulty of the analytical output Re WЗАМ - a material frequency feature (VCHH) closed systems;

- difficulty of the calculation of the integral under different levels;

- problem of the determination of importance S.

If use, for instance, system Mahtcad is not essential (refer to application B). The Upper case integration on frequency S should  be chosen such to at frequency S graph VCHH faded after 2-4 periods of the fluctuations. This is well seen from graphics VCHH in application 2. We shall Remind that characteristic VCHH such a that initial part her(its) graphics defines the final part ("tail") graphics connecting process, but a part VCHH under greater frequency initial part graphics x(t). So if small importance S is taken then at the beginning initially graphics x(t) can appear distortion and not-accuracy.

- 7.Short descriptions of the rules of the work with program which are used in discipline.

1.7.1.Program ALAB1. "Building of the connecting processes in ASR frequency methods" In program is used method of Akulishin , type of the structure ASR and importance her(its) parameter, as a whole, free.

After start about-grams necessary to execute the following actions:

- At appearance main window of the program, choose in menu point Section.

- Enter the expressions transmission function plant W(p) and controller R(p); importance factor, falling into W(p) and adjusting the controller S, S, S,; the values outraging influence (dU), final time (Tкон) and worker often-that * as defined in sample on screen). Press Ok.

- Choose the point a menu Calculation/Graph. If final  time is chose wrong, that follows to close the window with graph and choose the point a menu Section, enter new meaning Tкон, press Ok and repeat p.

4.

- In point menu Calculation/Table possible to look figural importance x(t).

- At presence of the printer possible to print the graph or table person of the connecting process.

Example type several screens is shown On Figure  1.4 at calculation of the connecting process for checking example.

Figure  1.4 Example of the functioning program ALAB1

1.7.2.Program  ALAB2 "Calculation common and extended of the frequency features of the linear systems with slow under free type of the transmission function ASR". The Raw data for calculation of the frequency features are:

- structure and type to transmission function of the under investigation system;

- factors transmission function plant and controller;

- range of the frequencies and degree wave

- The Choice necessary menu or substitution - a keystroke "ENTER". Output from menu or substitution - a keystroke "ESС".

1. 	The Order of the work with program following:

- in point Section will enter the transmission functions and coefficient all section and the general transmission function system;

- the parameters of the range of the frequencies assign in point Parameters.

- move over to point menu Calculation, press "ENTER" - will is made calculation given for building.

- in point Building choose necessary to characterize and find out (it) on the screen, having pressed "ENTER".

- for viewing the table of importance fall into point Table and choose where remove given (the File| Screen).At output file table.txt s in file in the current directory.

For contextual help press "F1", for the general - "Alt F1";

The Operations with window.  "Alt F10". Roll up "Alt F9". The Following window "AlT  F6".Move the window "Alt F7". The Size window "Alt F8". Close the window "ESC". Restore the window "Alt F5".

In point Section expressions transmission function section and systems are written as a whole arbitrarily, for instance, in following types:

с=5; tau=6;

w=c*exp(-tau*p)/(1+5*p+8*p^2);

T=4;w1=exp(-6*p)/(T*p+1);

Sp=5;Si=4;Sd=3;   R=Sp+Si/p+Sd*p;

w*w1/(w+w1*R);

The Last expression in text is considered general transmission function. In expressions possible to use before 40 names section. The Following predestined name it is impossible change:

p = -mw + iw; pi - a number pi; sin(), cos (), exp() - a functions.

Small and capital symbols differ i.e. T and t - different variable.

Parameters of the range of the frequency features are entered at points of parameters. The Initial frequency must be zero and menu must be final frequent. The Number point graphics must be within the range of 1..200. THE REMARK: for section of the type 1/p under initial frequency equal zero will protrude the error "to Division by zero". To avoid this, will enter very small frequency (for instance, 1е-5) , not equal zero.

If message is given in point menu Calculation about error, it is carried  as possible untrue given parameters of the system in point Section or pair meters of the range of the frequency features.

In point Building graphs are scaled depending on sizes window. To change the size a window, refer to brought above description to operations with window and in the main help "Alt-F1". In window graphics are removed sizes of the cell of the coordinate net and limits on bases of the abscissas and ordinates. THE REMARK: if on screen inscription "ERROR GIVEN" appears instead of data, that this signifies that for given transmission function under given parameter it is impossible races-consider the frequency feature.

When we work at point of table of output of the table in file table.txt; which is possible applied apart. Under each new use, all old given in this file disappear. For moving on table use the keys of cursor control. Possible change given straight in table. For this simply we take type; gather and fill  numbers in range. All changes are reflected on graph. ZAMECHA-NIE: at большом amount point (>50) filling the table can be slowed. In this case better find out the table in file and about-see her(it) there.

1.7.3 Program ALAB3 Calculation and study connected ASR (Kombinir.exe) For execution of this functioning to necessary transmission functions of the plant of the regulation on channel of the regulation W(p) and on channel of the indignation W1(p). After start the program necessary to execute the following actions:

- Choose the point a menu Section.

- Enter the transmission functions of the plant of control on ka-raid of the regulation and channel of the indignation. Then press on button Ok 3) Choose the point a menu Calculation.

- Copy adjusting the controller, ideal and real compensator.

- Start the program ALAB1 - a Building of the connecting processes.

- In her to choose the point a menu Section.

- In field of the editing to enter the transmission functions of the object, controller, real compensator and general transference function.

- Enter the critical frequency *. Press on button Ok.

- Choose the point a menu Calculation/Graph.

- If required they are brought back into point menu Section

- Choose the point a menu Calculation/Graph and from table fart write importance a column Time (t) and Process (X(t)).

1.7.4 Program ALAB4 Calculation and study cascade ASR (Kask_asr.exe)

For execution of this functioning to necessary transmission functions of the plant of the regulation on channel of the regulation and on additional channel. After start the program necessary to execute the rallyblowing actions:

- Choose the point a menu Section.

- Enter the transmission functions of the plant of control on ka-raid of the regulation and auxiliary channel. Then on-reap on button Ok 3) Choose the point a menu Table.

- Copy adjustment a controller (the last iteration).

- Start the program ALAB1 - a Building of the connecting processes.

- In her to choose the point a menu Section.

- In field of the editing to enter the transmission functions of plant and controller.

- Will Enter the critical frequency *. Press on button Ok.

- Choose the point a menu Calculation/Graph.

- If required they are brought back into point menu Section

- Choose the point a menu Calculation/Graph and from table fart-write importance a column Time (t) and Process (X(t)).

1.7.5 ALAB5 "Calculation and study mono contour ASR" Description refer to in methodical instruction to Laboratory functioning  2.

1.7.6 ALAB6 "Calculation and study mono contour  numeral  ASR" Description refer to in methodical instruction to Laboratory functioning  .9

1.7.7 ALAB11 "Complex analysis of the linear systems and technological  plant (the calculation of the object; one-  two- and three impulse ASR".

The Program ALAB11 is intended for calculation in mode on personal computer of the dialogue element and systems automatic of the regulation of the unceasing technological processes. After start the program ALAB11 necessary to type the password tes-26. Hereinafter actions of the user are defined appearing on screen with-contacts.

The Program allows to solve the complex of the tasks, referring to  wide-spread mono contour, double-circuit and three impulse system regulations applicable in industry.

The Program allows:

- approximate experimental got transitional of the feature plant regulations by analytical expression in the manner of transmission function;

- get the tables and graphs of the frequency features objects and systems of the regulation;

- get the tables and graphs of the connecting features object, processes of the regulation at indignations on main and additional channel;

- define the borders of the areas to stability and given for-pas to stability of the closed sidebar with

And - a controller;

- calculate the best values a parameter adjusting the controller;

- choose the structure and define importance a factor compensating device and etc.

As criterion optimal at determination of important parameter dynamic adjustment adjusting equipments in program is used minimum linear integral criterion (the area under crooked connecting process ) with restriction of the value of the factor wave m, under the most heavy step-like indignation, acting on any channel of the object.

The Connecting processes pay by means of transformation Furie AFCHH systems, formed in accordance with is chosen system channel regulations.

The Input program data are an actions of the user at the option from proposed menu number specified tasks or others of the messages and shaping hereunder condition of the solved task, as well as entering of necessary importance factor to transmission function of the plant and if necessary, controller and desired catch of the calculation of the required features.

As plant of the regulation can be used as objects with or without self level, as well as equivalent object, which transmission function is formed in the manner of relation or  two transmission functions object.

Importance factor transmission function , in accordance with expression, taken out on screen, can be entered in the manner of integer or material figures and alongside with usual form record can use the significant form of the numeration (for instance 0.0006 = 1Е-3).

In many events, at decision that or other tasks program gives on screen of the message in the manner of helps about supposed on important parameter or range of the change the argument at calculation of the required features and calls these importance beside user acknowledgement.  If user wants to enter their own importance, that their necessary to print if no, that striking keyboard "ENTER" saves importance, proposed by program.

When shaping the condition of the task by means of menu necessary to enter importance of the selected points menu. Keystroke "ENTER" instead of numeral brings about choice of the first point menu.

The Output program data are a tables of importance transference and complex frequency features plant and system (open or closed on chosen channel), borders about fields to stability and given spare to stability, as well as relevant graphs.

After output of necessary information on result of the decision of the task on screen are removed questions, answer which user can return the condition of the task in different points of the shaping. So, for instance, at will user possible to change the initial conditions of the task: importance factor transmission function objects and controllers, range and step of the argument, type of the plant and controller etc.

At will user possible to return to menu of the choice of the type of the system of the regulation or do with the solution tasks or conduct the decision of the test example.

1.8 Composition of the fund of student software (FSPO) of the scholastic process, used in discipline ATP, CAD, SAU and IOSU. Table 1.2 Name

programs Name program P-1-IOSU

(LAB1) Removing the steady-state features of the object. experimental data by method least square.

P-2-IOSU

(LAB2) Study plant at mono factor of the analysis of variance P-3-IOSU

(LAB3) Steady-state estimation parameter and checking the hypothesis P-4-IOSU

(LAB4) Building to models of the static of the plant of control by experimental method. The Factorial experiment. P-5-IOSU

(LAB5) Study speakers plant to automations. experimental determination of the dynamic features P1-SAU (ALAB1) Building connecting processes in ASR frequency methods P-2-SAU (ALAB2) Calculation common and extended frequency characteristic linear systems with slow under free type to transmission function ASR P-3-SAU (ALAB3) Calculation and study multifunction ASR (Kombinir.exe) P-4-SAU (ALAB4) Calculation and study cascade ASR (Kasrad.exe) P-5-SAU

(ALAB5) Calculation and study of mono contour analog ASR P-6-SAU

(ALAB6) Calculation and study mono contour digital ASR P-1-ATP

(ALAB7) Study algorithm of the primary processing information in ASUTP P-2-ATP

(ALAB8) Optimum distribution resource in technological system with parallel structure P-3-ATP

(ALAB9) Optimum distribution resource in technological system with consequent structure P-7-SAU

(ALAB10) Approximation and building of the steady-state features plant P-4-ATP

(ALAB11) Calculation and study unceasing ASR (MEI) (the calculation of the object; one- two-and three impulse ASR) (password: tes-26) P-5-ATP (ALAB12) Complex analysis of linear systems and technological plant (MEI)

1.9.The order of  performing the work are following:

- to get acquainted with methodical instructions and quoted literature, to copy main theoretical positions;

- to get the curves of the runaway on three channels (refer to section 1.4.3);

- to remove the transmission functions on three channels, using graphically analytical method, but for the main of the channel in addition and method of the areas (refer to section 1.4.1);

- to check adequacy all got mathematical models (refer to section 4.1 and formula(1.11)), build the graphs source and accounting crooked runaway;

- to test functioning enumerated above programs;

- to arrange the report and be prepared to answer to checking questions; The Contents of the report:

- production to purposes of the study;

- thumb nail sketch used mathematical models and calculation; 3. description result calculation;

- graphs and dependencies.

- analysis result and findings;

- list of the used literature.

1.10 Recommended literature

- Auto control in chemical industry: training-drooped for high school. Under ред. Dudnikova

E.G. -M.: Chemistry, 1987. -368с.

- The Laboratory work on automation and managerial system production-ными process: Scholastic allowance for high school/ Under ред. I.M. Mas-lennikova. - M.: Chemistry, 1986. -336с.

- System Designing the checking and automatic регулирова-thread of the metallurgical processes: Scholastic allowance for high school/ Glinkov G.M. and others 2-e izd. - M.: Metallurgy, 1986,

-352с.

- Borisov V.V., Plyutto V.P. The Laboratory work on theories of the autocontrol chemisttechnological process. Analog systems: -2-e izd. - M.: Chemistry, 1987, -152с.

- Makarov I.M., Menskiy V.M., Linear automatic sistemy. - M.: Mashinostroenie,1977, - 464 s., (2-e publishing, 1982.).

- Mihelev V.M., Inkov A.M., Dmitriev O.L. Modeling объек-тов and managerial system. The Methodical instructions to undertaking the Laboratory functioning student to professions 36.03. SHymkent, YUKTU, 1997 g.-44 p., tab., silt.

- The Building of the mathematical models chemist - a technological processes. L.: Chemistry, 1970 - 312 s.

- Balakirev V.S., Dudnikov E.G., Cirlin A.M. The Experimental determination of the dynamic features industrial объек-тов of control. M.: Energy, 1967.

- Ahnazarova S.L., Kafarov V.V. Methods to optimization of the experiment in chemical technology: Scholastic allowance for vuzov. - 2-e изда-ние, pererab. and dopolnennoe. -M.: High school, 1985. -327с.

- Plyutto V.P. The Laboratory work on theories automatic управле-thread chemist-technological process. Digital sistemy. -M.: Chemistry, 1989,-168с.

- The Guide to mathematician. (for scientific workman and инже-неров) G.KORN and

E.KORN. M.: Science, -1973. 832с.

- Inkov A.M. The Systems to automations and control. "Issledo-вание on COMPUTER quality of the connecting processes in одноконтурной ASR when adjusting the controller on method Ciglera-Nikolisa". The Methodical instructions to Laboratory functioning   2 and  3 for student of the professions 36.03. SHymkent, YUKGU, 2002 - 14 p., 1таб., silt.

- The Spectacles V. Mathcad 7 Pro for student and engineer. M.: "Lump-пьютер Press", 1998

- The Deacon V.P. Mathcad 2001: scholastic publishing. SPB: PETER, 2001 624с.

Laboratory work  2 "Study and computer modeling of connecting processes in cascade automatic system of the regulation"

2.1 Goal of work

The Purpose scholastic-exploratory Laboratory functioning is a study and computer modeling quality connecting processes in analog cascade automatic system regulation (ASR) and Laboratory mastering the methods of the calculation of the adjustment cascade ASR [1,2].

Laboratory functioning is executed with use personnel of the computer and standard programs Alab1, Alab2 and Alab4 for IBM PC, valid in mode of the dialogue (refer to method of Laboratory work  1 [ ] ). The program of the own development or system Mahtcad use Instead of this possible.

For execution of each functioning is required 8 hours classroom occupation and 4 hours of the independent functioning. 2.2 Task

The Long-run objective of the study is an analysis quality transfer of the processes in closed ASR with P, PI and PID-controller. As criterion quality (refer to.[4], p. 59) use:

- time connecting process;

- degree of wave;

- over travel;

- integral factor quality.

Steady-state accuracy of the regulation student chooses itself.

2.3 Virtual model cascade ASR

The Plant to automations in LaboratoryLaboratory work is heat changer. The Purpose to automation is a maintenance of the  temperature of warm water.

The Simplified scheme of the plant is shown on Figure  2.1, where indications are shown: U - controlling influence (input) - a consumption pair;

X - a controlled parameter (output), the leading index effectiveness of  functioning(valid) the plant - a current temperature of hot water;

X* - given importance of the controlled parameter - required importance of the temperature of hot water;

X1 - intermediate (auxiliary) regime parameter -a consumption pair; V - an indignation - a consumption of cool water.

Figure 2.1 -  Plant to automations.

Often for automation like plant is used one contour ASR(refer to Figure  2.2) with influence on regulative part  of the indignation i.e. signal from sensor X enters in controller R. The Controller depending on errors of the unbalance E=X-X* in accordance with PI, PID or the other law of the regulation work of  the control signal Up. This signal is given on regulating device at the input object, which changes importance of the races-move pair U. If adjusting the controller are determined it is correct then on certain time T is fixed required signification X equal X* under a certain given possible value one hundred  of the error of the unbalance Emin=X-X*. However in practice enough often appears the situation, when time T or value Emin do not meet the requirements quality of the regulation, or when ingeneral can be not reached firm connecting process.

One their reasons this can be too big slow on channel U -> X. In such event one of the variant provide; ensure ;b)supply-thread given purposes to automations is an use cascade ASR (one of the possible variant refer to Figure  2.3).

Figure 2.2 – Mono contour ASR теплообменника. X* - a task (set) for the main of the controller, given importance of the output.

Figure 2.3 -- Cascade ASR hearter. X1* - a task for auxiliary controller R1.

Under such way of the way of the regulation is used additional controller R1, input for he serves the signal from intermediate sensor X1, but task is an output signal X1* main (leading) of the controller R. The Advantage cascade ASR is that speed such ASR vastly rising since channel of the influence U -> X1 has vastly smaller slow, than channel U -> X. However adjustment such ASR pack; fold-it since necessary to calculate adjustment already two controllers R1и R.

As example of the plant of control on installation over work gas condensate and oil product emerges heater T-801 (refer to Figure  2.4). The Particularity such heater is presence big slow in channel regulation: the temperature oil product - a consumption pair. In this case usual mono contour ASR do not provide the sufficient quality of the regulation.

Figure 2.4 -. Example cascade ASR

2.4 Raw data

As source necessary to have a parameters transmission function:

On the main channel U -> X:

W(p) =	C ⋅exp(−p⋅τ)

1+T ⋅ p (2.1) and on intermediate (auxiliary) channel U -> X1:

W1(p) =	C1

,  			1+T1 ⋅ p ⋅exp(−p⋅τ1) 		(2.2)

I.e. necessary to know importance poles parameterС, τ, TиС1, τ1, T1.. Their importance student defines on result of the removing individually and mathematical processing the crooked runaway of the plant on is rolled-bark U -> X and U -> X1 in LaboratoryLaboratory work on discipline MIOU or using program LAB5 (refer to above)

2.5 Theoretical introduction.

As stated above the main difference cascade ASR is presence two controllers - R main (external) of the controller, rally-reaping for stabilization of the main leaving the plant y, and additional flesh (internal) of the controller R1, intended for regulation auxiliary parameter X1. The Task for auxiliary of the controller R1 serves the output signal of the main controller X1*. Herewith, the auxiliary controller R1 supports importance of the auxiliary parameter X1 on determined level, which is necessary for stabilization of the main parameter X [6].

Thereby, calculation cascade ASR is reduced to determination adjustment main R and auxiliary R1 controller under is given importance dynamic feature of the plant of control on the main and auxiliary channel. Since adjustment main and auxiliary controller interdependent, calculation on-construction site conduct the method an iteration.

R

1

в

спо

могательный

регул

я

тор

R

г

лавный

регул

я

тор

U

e

1

1

X

*

1

X

X

*

U

e

Объект управления

W

2

инерционный

канал

W

1

малоинерц

и

о

н

ный

канал

X

On each step of the iterations calculate brought mono contour ASR, in which one of the controller conditionally pertains to equivalent plant (refer to Figure  2.5).

R

W

Э

X

X

*

1

R

1

W

2

Э

X

2

-

X

*

2

U

Figure 2.5 - a Structured schemes equivalent mono contour system subject of the regulation with the main (but) and auxiliary (b) controllers.

On each step of the iterations pay brought mono contour ASR, in which one of the controller conditionally pertains to equivalent object. This it is necessary to understand so that for each scheme but) and b) of importance all factor must be known. The Calculation adjustment R1и R produce usually one of the simple methods, on-example by method of Ciglera-Nikolisa [1].

The Transmission function of the equivalent object, which is used for calculation main controller R has a following type:

W Э=	R1(p)⋅W(p)

1−W1(p)⋅R1(p)

where falling into this expression transmission function additional flesh controller R1 in the manner

of:

or more often in the manner of,

R1(p) = SП1 + SИ1  p + S Д1 ⋅ р

R1(p) = SП1 + SИ1p		(2.3)

The Transmission function of the equivalent object, which is used for calculation of the auxiliary controller R1 is of the form of:

W1Э=W1(p)−W(p)⋅R(p)  	(B) where falling into this expression transmission function main of the controller R in the manner of:

R(p) = SП+ SИp			(2.4)

The Calculation adjustment controller R and R1 (the formulas (2.4) and(2.3)) usually produce the integration method, essence which shall consider on example, when calculation produce, as from determinations adjustment main of the controller R i.e. using expression (But).

Since while else importance parameter adjustment R1 falling into (But) not yet known, that them it is necessary or what- or image be assigned. Usually take the admission about that that worker frequency main of the controller much less, than auxiliary, then possible (But) simplify before type (A1):

WЭ0 = W(p)	(A1)

W1(p)

not containing R1, with the known parameter W(p) and W1(p).

In this case if calculation begin with the main of the controller R, using method Ciglera-Nikolisa, that consecutively executes following.

" on the first step of the calculation is used expression (A1) and are defined first approximations adjusting the controller R;

" on the second step calculation is used expression (B) and defined the first approximations an adjusting the controller R1 under found on the first step brandy R;

" on the one third step calculation is used already expression (But) and are defined first approximations adjusting the controller R under found on the second step brandy R1.

The expressions consecutively use On the following step:

(In) -> (But) -> (In) ->(But) ->(In) ->(But) ->(In) and so on defining on each of these step of the adjustment controller R1 and R.

These are  calculations stop, when found on every step brandies Sп, Sп1, Sи, Sи1 stop greatly changing. Usually for this is required undertaking 10-15 steps iteration.

Thereby, calculation optimum adjustment continue until adjustment controller R1 and R, found in two iteration, do not comply with given by accuracy.

2.6 Algorithm calculation cascade ASR Algorithm calculation cascade ASR consists of the following step:

1. 	Entering transmission function plant of control on the main and auxiliary channel (2.1) and

- 2);

  - The Calculation to transmission function of the equivalent plant WЭ0(p) formula (A1);

  - The Calculation adjusting the main controller R;

  - The Calculation to transmission function of the equivalent plant W1Э(p) formula (In);

  - The Calculation adjusting the auxiliary controller R1;

  - The Calculation to transmission function of the equivalent plant WЭ(p) of the advantagesmule (But);

  - The Calculation adjusting the main controller R;

  - If importance new adjustment R do not comply with importance R, found on previous step, that return on point 4;

  - Output R and R1 1

  - Compare the connecting processes in mono contour and connected ASR on indignation and do the findings.

The Algorithm of the program "Cascade (ALab4) " (Kask_asr.exe) Process calculation is conducted on the following step:

  - The window appears At choice of the point menu "Section", in which not necessary consecutively enter the transmission functions on main and auxiliary channel.

  - The calculation cascade ASR is Realized on algorithm calculation, presented in N 1.

  - The Output on screen result calculation.

2.7 Order of the performing the functioning

After starting the program "Cascade" necessary to choose the point a menu "Section" and in appeared window to enter the transmission functions of the plant on the main and auxiliary channel of the regulation.

Then necessary to press on button "Ok" and choose the point a menu "Table". The table will appear on screen with result calculation. Most best values adjustment controller are found In the last line of the table.

For graphing of the connecting process possible get use of  the program LD049.exe (or PEREHODNYY_PROCESS.EXE), which allows to build such graphs for ASR any structures. For graphing connecting process cascade ASR necessary to enter following:

" transmission functions on the main and auxiliary channel of the plant of control;

" transmission functions both controller with adjustment in program "Cascade"; " transmission function of the closed plant of control trace type: For cascade connected ASR on channel of the indignation:

WЗАМ(p) =	W(p)

1+W(p)⋅R(p)⋅R1(p)+W1(p)⋅R1(p) (2.5)  and on channel of the task:

WЗАМ(p) =	W(p)⋅R(p)⋅R1(p)

. 	1+W(p)⋅R(p)⋅R1(p)+W1(p)⋅R1(p) 		(2.6)

Then necessary to enter the critical frequency auto waving and press on button "Ok".

At choice of the point menu "Calculation/Graph" graph of the connecting process it will appear on screen. At presence of the printer there is carried to  remove the graph on print.

2.8 Description of the checking example

Calculate adjustment a controller in cascade ASR if transfer functions of the plant of control on the main and additional channel accordingly are: W(p) = 0,5 ? e-4?p / (4 ? p + 1)

W1(p) = e-0,4?p / (1,2 ? p + 1)

The Results calculation on iteration were provided in table 2.1.  Table 2.1

| w* | Sп* | Sп | Sи | w*1 | Sп*1 |
| --- | --- | --- | --- | --- | --- |
| 0,728 | 4,636 | 2,086 | 0,290 | 3,790 | 4,091 |
| 0,601 | 4,298 | 1,934 | 0,220 | 3,809 | 4,218 |

2.9 Recommended literature

1 .Automatio control in chemical promyshlennosti./uchebnik for high school. Under ред. E.G.DUDNIKOVA. -M.:Chemistry, 1987,-368 s.

2. The Laboratory work on automation and managerial system production processami./uchebnoe allowance for high school. Under ред. I.M.MASLENNIKOVA.-M.: Chemistry, 1986, - 336 s.

3.Борисов V.V.,Plyutto V.P.PRAKTIKUM on theories автоматическо-go control chemisttechnological processami. analog systems:-2-e izd.-M.:Himiya,1987.-152с.

4.Анисимов I.V. Bases of the autocontrol technological process petrochemical and oil promyshlennosti.-L.:Himiya,1967.-408с.

Laboratory work 3 "Study and computer modelling connecting processes in multifunction automation system regulations"

3.1 Goal of work.

The Purpose scholastic-exploratory Laboratory functioning is a study and computer modeling quality connecting processes in analog multifunction automatic system of the regulation (ASR) and Laboratory mastering the methods of the calculation on-construction site ASR [1,2].

Laboratory functioning is executed with use personnel of the computer and standard programs Alab1, Alab2 and Alab4 for IBM PC, valid in mode of the dialogue (refer to methodical ediction to Laboratory functioning  1 [ ] ). The program of the own development or system Mahtcad use Instead of this possible.

3.2 Task.

The Long-run objective of the study is an analysis quality fart-ходных of the processes in multifunction ASR with P, PI and PID-controller. As criterion quality (refer to.[4], p. 59) use:

1) time connecting process;

- degree of wave;

- overtravel;

- integral factor quality.

Steady-state accuracy of the regulation student chooses itself.

3.3 Virtual model multifunction ASR

Either as in the event of cascade ASR (refer to previous Laboratory functioning) by plant to automations in LaboratoryLaboratory work is heater. The Purpose to automations is to give temperature warmed water. The Simplified scheme of the plant is shown on Figure  2.1.

The Multifunction systems of the regulation use under automation object, subject to action essential controlling indignations. On Figure  3.1 is shown variant connected ASR.

Figure 3.1 -. Multifunction ASR heater. Output main controller R and compensator R2 after summation  on regulative organ.

The Multifunction systems of the regulation possess following-mi particularity: presence two channels of the influence on output coordinate of the plant and use two sidebars of the regulation - closed (through controller R) and open (through compensator R2). Introduction correcting pulse on the most strong indignation allows greatly to reduce the dynamic control error at condition of the right choice and calculation dynamic device, forming law of the change of this influence.

The Base of the calculation of the similar systems is a principle to invariance: deflection of the output coordinate of the system from given importance must be an identical equal zero under any assigning or outraging influence.

For performing the principle to invariance necessary two conditions: ideal compensation all outraging influence and ideal reproducing the signal of the task. Obviously that achievement to absolute invariance in real system of the regulation Laboratoryly impossible. Are they Usually limited by partial invariance to the most dangerous indignations. We shall Consider the condition to invariance open and multifunction systems of the regulation to one outraging influence:

R2 =−W(p)			(3.1)

W2(p)

Thereby, for provision of invariance of the system regulation to some indignation necessary to install the dynamic compensator, transmission function mounted-swarm is an attitude transmission function plant on channel of the indignation and regulations, taken with inverse sign.

The Conditions physical realization invariant ASR. On of the first  from the main problems, appearing at building invariant of the systems of the regulation, is their physical realization i.e. realization compensator, meets condition (3.1).

Unlike usual industrial controller, the structure which is given and required only calculate their adjustment, structure of the dynamic compensator is completely defined by correlation of the dynamic features of the plant on channel of the indignation and regulation and can turn out to be very complex, but under disadvantage correlation of these features physically not realized.

"Ideal" compensators physically not realized in two events.

- If time clean slow on channel of the regulation more, than on channel of the indignation. In this case ideal condenser must contain the section of the predictions future-go conditions.

- If in transmission function of the compensator degree multinomial in numerator more, than degree of the multinomial in denominator. In this case compensator must contain ideal differentiated  section.

The Technical realization invariant ASR. Under practice to realization opened and multifunction ASR usually obtained drawn near invariance of the system to races indignation in the most dangerous range of the frequencies. Herewith, the real compensator choose from number most easy realized dynamic section, parameters which counts quarterdeck from condition of vicinity of the frequencies.

The Choice of the structure of the real compensator is dictated frequency feature of the ideal compensator within the range of frequencies from well-la before frequency of the cut. Usually, the compensators choose as combination the most simplest linear section: a periodic first-order and real differentiating. In table 1.1в [1] are brought by dynamic of the feature of the most wide-spread types compensators.

Thereby, calculation multifunction partly invariant ASR includes the following stages:

- calculation adjusting the controller and determination worker frequencies in mono contour to system of the regulation;

- output to transmission function of the ideal compensator from moustache-caught invariance and analysis his(its) realization;

- choice real compensator and determination his(its) parameter from condition drawn near invariance in the most essential for system range frequencies.

  - Raw data

As source necessary to have a parameters transmission function:

On the main channel U -> X:

W(p) =	C ⋅exp(−p⋅τ)

1+T ⋅ p (3.2) and on channel of the indignation on the part of presenting of cool water V -> X1:

,  			W	(3.3)

I.e. necessary to know importance poles parameter С, ?, T and S2, ?2, T2. Their importance student defines on result of the removing individually and mathematical processing the crooked runaway of the plant on is rolled-bark U -> X and U -> X1 in LaboratoryLaboratory work on discipline MIOU or using program LAB5 (refer to above)

  - Order of the performing the functioning

The Detailed examples of the calculation of the multifunction systems regulation is provided in example 1 and 2 in [1] on page 38-40.

- To Define the equivalent transmission function of connected ASR on channel of the indignation (3.1) of  structural  scheme.

- Define the transmission function of the ideal compensator W from condition of absolute invariance W = 0. The Controlled value is invariant for indignations if she absolutely will not respond to indignation.

- Value physical realization compensator.

- Build AFH, ACHH and FCHH ideal compensator (for building is used complex Mathcad).

- Select model of the real compensator (refer to examples in [1 and 2]

- Build the connecting process of the regulation in connected ASR with reality by compensator using program LAB49.

7 Compare the connecting processes in mono contour and connected ASR on indignation and do the findings.

3.6 Recommended literature

1 .Automation control in chemical industries for high school. Under ред. E.G.DUDNIKOVA. M.:Chemistry, 1987,-368 s.

2. The Laboratory work on automation and managerial system production processami./uchebnoe allowance for high school. Under ред. I.M.MASLENNIKOVA.-M.: Chemistry, 1986, - 336 s.

3.Борисов V.V.,Plyutto V.P.Laboratory work on theories автоматическо-go control chemisttechnological processami. analog systems:-2-e izd.-M.:Himiya,1987.-152с.

4.Анисимов I.V. Bases of the autocontrol technological process petrochemical and oil promyshlennosti.-L.:Himiya,1967.-408с

Laboratory work  4 "Study and computer modelling connecting processes in digital automatic system of the regulation"

4.1 Purpose of the functioning.

The Purpose scholastic-exploratory Laboratory functioning is a study on COMPUTER quality of the connecting processes in digital mono contour to automatic system of the regulation (ASR) and Laboratory mastering the methods of the calculation of their adjustment.

Laboratory functioning is executed with use personal of the computer and standard programs Alab5 and Alab6 for IBM PC, valid in mode of the dialogue (refer to methodical instructions to Laboratory to functioning [9]). Instead of this possible use the program of the own development or system  Mathcad,

MATLAB.

For execution of each functioning is required 6 hours classroom occupation and 6 hours of the independent functioning.

4.2 Statement of the problem the study and raw data

In the course of functioning is conducted study mono contour ASR, having structured scheme, shown on Figure  1. i.e. same, as Laboratory functioning  2. However, in change from analog ASR is here considered digital ASZ. In her controller marketed different in the manner of algorithm of the type (7), used usually presently in industrial controller. The Mathematical model of the plant regulation is also written digitally (5). In the course of work of the Laboratory functioning  2 on discipline ATS one of the tasks was a calculation adjustment mono contour ASR, using analog controller. The Contents of these functioning was shown:

- reception of the identical mathematical models speakers sink;

- calculation  parameter controller, realizing different algorithms of the regulation;

- reception to mathematical model closed ASR on is given-ному channel;

- choice and realization (or use) of the concrete method of simulation modeling of the connecting processes, comfortable realized on computer;

- estimation quality connecting processes in connected ASR.

Figure 4.1 - a Structured scheme mono contour ASR. U - an entering the system (the control parameter); X - a leaving the system (the parameter with-standings, result of the measurement); X*- task controller; Q- adjusting the controller (QП; QИ; QД); RO - a regulative organ; D- sensor (the measuring converter).

In particular, in LaboratoryLaboratory work  2 are received by trace results.

The Transmission function of the plant of control is determined in the manner of:

W( p) =	C	⋅exp(−p⋅τ)

1+T ⋅ p          (4.1) on this base is calculated adjustment P, PI and PID controller

.

R(p)= SP + SI p + SD ⋅ p               (4.2)

Founded importance  C,T,τ,SP ,SI ,SD , are brought in table 4.1, they are a raw data for calculation digital ASR in is given to Laboratory functioning.

Table 4.1 - a Raw data for calculation   зачетной of the book For regulation of Plant Controller

| № of credit book | Law of regulati on |  | Object |  |  | Controller |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C | T | τ | SП | SИ | SД |
|  | P |  |  |  |  |  |  |
|  | PI |  |  |  |  |  |  |
|  | PID |  |  |  |  |  |  |

P PI PID

The Purposes are:

- calculation adjusting the digital controller - a determination of the period of the slicing on time(the period of the questioning); the determination on-construction site QП, Qи, QД, providing given quality regulation;

- computer modeling digital mono contour ASR - build the graphs to imitations of the connecting processes in ASR;

- study of the influence upon quality of the connecting processes.

At fixing of the indignation at the input plant , controller for time of the connecting process must provide the deflection a current  importance of the leaving the plant H(t) from given by H* not more, than required steady-state error HST., which importance is chosen by student.

Modeling of the connecting processes in ASR is produced deliver in-thread, as well as with use own or standard about-grams (Alab6) for PC.

4.3 Theoretical introduction

4.3.1 General information

Using the digital computing machinery in system automatic of control allows to change the unceasing controllers and analog device of the transformation signal digital. At, the signals enter in system of the regulation in discrete moments of time with some interval TO; the signals discrete and on level, since number bit presentations these signal in CVM is limited. However transition to discrete signal, as a rule, does not contribute the significant changes to calculation of the systems of the regulation, on accuracy presentations signal to digitally usually much more accuracy measuring device, and inaccuracy to sampling signal on level small. As to sampling on time then in view of big inertia of technological processes interval  on time for them  so small that features of the system with discrete regulation and with unceasing controller Laboratoryly do not differ if parameters of the discrete system are chose with provision for interval of the slicing and are coordinated with parameter corresponding to lasting of the system.

The Digital systems of the collection and information handling have a row of the particularities:

- digital way of the operation UVM conditions entering and output given code in figure; since output signals sensor usually present itself unceasing functions of time, them before entering in UVM subject to analogue digital transformation;

- digital methods of the calculations provide high, but final accuracy calculation; the row computing algorithm may be marketed only with the help of UVM;

- as a rule, carried in machine information must be worked out quickly to possible for using results for well-timed reaction on change check; control  parameter technological process;

- UVM processes only numeric importance of the measured values, units of the measurements are taken into account under scale and are assigned final result.

The Functional components input and output interface of the computer are shown on Figure  2 - 3.

The Signal worked out by sensor, must be filtered from all stranger of the frequencies before that, as he will is processed computer program. In particular, necessary to avoid the radio-frequency noise, which is usually directed in cable at transmission of the signal. Filtered  measuring signals are going to in multi device. This device, which has several inputs and one output. The Main purpose of the multi device - reduce overall value of the system to account of the using only one device of the processing (in this instance - controlling computer), which usually greatly cherish multi device for all input signal. The Transformation of the analog signal in digital occurs in lectern figural converter (ACP). The Scheme of the sample and keeping remembers quick importance of one signal in beforehand installed moments of time and holds his(its) constant on output during interval of the sampling. Before the further processing in computer importance signal to measuring information necessary in addition about-believe to be certified in that that it acceptable and makes sense in context of the physical process.

Before performing the Laboratory functioning necessary to study the subject "Mathematical models plant in digital ASR", "Mathematical  models controller in digital ASR and methods of the calculation their adjustment", "Algorithms of digital control and their realization"[1-8].

Figure  4.2 Digital system

Figure 4.3 -. Scheme of the entering/output in system "process-control computer"

Figure 3 - a Structure digital mono contour ASR (one channel).

1- sink; 2- ACP; 3 - a fixative and commutate; 4 - PRK and algorithms of control; 5 - a fixative; 6 - a commutate.

For processing in computer managerial system (refer to. Figure 4.2 - 4) data must be presented in discrete type. This results from quant on time in resulting that periodic questioning sensor since period THAT and analog-to-digital transformation (ACP). With the help of ACP is produced quant on level with the result that importance of the signal numeric code introduces for moment of time j - an integer number step of quant size:

∆ = X MAX2N−−X1MIN				(4.3)

where and accordingly most least signify-thread of the range of the measurement of the sensor, N - charge ACP (usually 8..16). Under it is correct chosen by N influence quant on accuracy of the presentation Laboratoryly invisibly. If final  importance of the variable then after ACP this:

X J* = intX∆J ⋅∆			(4.4)

means taking the whole part from expression in square bracket, for instance.

4.3.2 Mathematical models plant in digital system

The Mathematical models in digital managerial system are written in the manner of recurrence different of the equations (refer to. [ 3 - 8]).

If mathematical model TOU is presented transmission function of the type (1) and is used fixative of the zero order, that leaving the plant in figural  type for moment of time definition as:

X j = A⋅ X j−+ B⋅U j−D		                        (4.5)

A = −exp(−TO /T)       B = (1− A)⋅CD = [τ/TO ]    (4.6)

The Equation (4.5) is differentiated  by equivalent of the unceasing equation of the plant (3) for discrete moments of time j = 1, 2, 3, .

D - a number, rounded before nearest greater integer, define slow TOU, expressed as a whole count; calculate; list period of the questioning. For j < D:

X j = A⋅ X j−1  		(4.5А) 10

- a leaving the plant at moment of time - i.e. on past step of the questioning.

- controlling influence (leaving the controller) at moment of time -D.

4.3.3 Algorithms of digital control and their realization From the known expression of the analog algorithm of the regulation:

Up(t )= SП⋅E(t )+ SИ⋅T∫0PE(t )⋅dt + SД⋅ dEdt(t ) (4.1А)

using operations numerical integration and differentiation possible get expression for digital PID - a

controller:

U j =U j−1 +QPE j +QI E j−1 +QDE j−2                                            (4.7) Ej = X j − X*

(4.8)

On the most first step of the function (valid) of the algorithm and unknown, and instead of (7) use:

U j =U j−1 + QPEj + QI Ej−1                                    (7А)            U j =U j−1 + QPEj

(7Б)

In digital controller for filtering the hindrances often use connection of the exponential filter with transmission function

W(p) =	1

this in figural type corresponds to to expression:

X~

j =γ⋅ X j + (1−γ)⋅ X~ j−1 		(10)

where and - filtered and measured importance of the leaving the plant for the current step j;  - a result

to filtering on previous step; ? - a constant factor (adjusting the filter, , ).

The Fragment of the algorithm PID - a controller with filter for j-oho step of control possible to present in the manner of:

- questioning the sensor: receptionX j

- filtering from hindrances:     X~ j =γ⋅ X j + (1−γ)⋅ X~ j−1

- calculation of the error:                     E j = X~ j − X *

- calculation of the output:                        U j =U j−1 +QPE j +QI E j−1 +QDE j−2

(and only for the most first step )U j =U j−1 +QP E j )

The Algorithm to realization digital mono contour ASR for PI controller without accumulation of all importance , and ,   - a step algorithm:

    - Only for the most first step of the questioning: U1=0; E1=0

    - Questioning the sensor of the input signal - a reception X

    - Determination of the error of the unbalance: E=X-X*

    - Calculation of the leaving the controller: U=U1+QПE+QИE1

    - Keeping for use on following step: E1=E

    - Keeping for use on following step: U1=U

    - Influence on executive mechanism

    - Transition to  1 Here raw data are , , ,. Here, the index j corresponds to the moment of time i.e.

persisting moment time-nor, index j-1 corresponds to the moment of time - , index j-2 relates the moment of time -2.

Thereby algorithm of modeling of the connecting processes in digital ASR can be recorded in following type:

    - step - a determination of importance of time: =j;

    - steps - a calculation of the leaving the plant for moment of time on the form-le (5) or (5А);

    - steps - a calculation of the error (the unbalances) on formula (6);

    - steps - a calculation of the leaving the controller for moment of time on formula (7) or (7А) or (7Б);

    - steps - a calculation of importance of time = +;

    - steps - a similar step 2, but for moment of time;

    - steps - a similar step 3, but for moment of time; and etc while importance will be fixed and will not become less given statistic control error Hst.

On this algorithm not difficult to produce and manual calculation, his(its) results snowdrift in table 2.

Table  4.2

| step j | T Time j | X Output of result j | E Mistake j | U Output of controller j |
| --- | --- | --- | --- | --- |
| 1 | 0 | X * | 0 | ∆U |
| 2 | 1*TO |  |  |  |
| 3 | 2*TO |  |  |  |
| М | T М* O |  |  |  |

The Algorithm to realization digital mono contour systems regulation for PI controller without accumulation of all importance , and:

The Stage 0. Only for the most first step of the questioning we take:

U1=0,  E1 =0.

The Stage 1. Questioning the sensor of the input signal - a reception X.

The Stage 2. The Determination of the error of the unbalance: E=X-X*, where X* - a task. The Stage 3. Calculation of the leaving the controller:

U = U1 +   E +   E1.

The Stage Storing errors E for use on following step: E1=E.

The Stage 5. Storing control U for use on rally-blowing step: U1=U.

The Stage 6. The Influence on executive mechanism. The Stage 7. Transition to stage 1.

3.4 Calculation adjustment controller digital mono contour ASR.

In detail this is described in [3 - 8]. Adjustment , , possible, for instance, approximate to define on formula:

QD = 3/C

QP = QD ⋅(1+0.22⋅T /τ) 			(11)

QI = −2⋅QD ⋅(1+0.1⋅T /τ)

SP , SI , SD :

QD = SD /TO

QP = SP + SI ⋅TO /2+QD ) 				(4.12)

QI = −(SP − SI ⋅TO /2+ 2⋅QD )

4.4 Order of the performing the functioning

    - It Is Described output to discrete model of the digital controller.

    - It Is Described output to discrete model TOU for the main channel.

    - The period of the slicing is Chosen on time That.

    - It Is Conducted calculation adjustment manually, with detailed description of the move of the calculation in correspondence to

    - The Results of the calculation are reduced in table, the form which is regarded in task.

    - Computer simulation modeling ASR is Conducted.

    - The analysis of simulation modeling ASR is Described.

    - The algorithm of modeling digital ASR is Written.

    - The results of modeling connecting processes are Described.

  - Analysis result and findings.

The findings are Done about influence of the value of the period of the slicing and method of  determinations adjustment on quality of the connecting processes.

  - Report must contain the following sections.

    - Production to purposes of the study.

    - The General information about digital ASR; the mathematical models TOU and controller; the methods of the determination adjustment digital controller; the thumbnail sketch used source is given data, mathematical models and methods calculation.

    - The Output of the discrete models TOU for the main of the channel.

    - The Choice of the period of the slicing on time (in accordance with methods, brought on

str.53 and 46 in [5]);

    - .Результаты calculation adjustment manually, with detailed descriptions of the move of the calculation for three different for each of rally-blowing three methods:

but) on formula brought below, and on p.53 in [5]:

QД= 3/СQП= QД⋅(1+0.22⋅T /τ)

QИ= −2⋅QД⋅(1+0.1⋅T /τ)

b) formula brought below and on str.47 in [5] coming from on-construction site , , , determined

above:

QД= S Д/TОQП= SП+ SИ⋅TО/2+QД

QИ= −(SП− SИ⋅TО/2+ 2⋅QД)

in) on method Ciglera in accordance with methods, brought on p. 29-30 in [2] and in [4-5].

    - The Results of the analytical transformations and calculations.

    - The Description result functioning on COMPUTER.

    - Write the algorithm of modeling digital ASR.

    - .Построить graphs of the connecting processes in ASR for all results of the calculation X(t), and in one scale and value their quality (importance and for time of the termination transition of the process under given importance Hst), where[4]:

M	M

∑E2j	∑U 2j

SE2 = j=M1	SU2 = j=M1	            where M - a gross amount step.

    - Do the findings about influence of the value That and method of the determination adjustment on quality of the connecting processes.

    - The Graphs used and got dependencies.

    - The Analysis result and findings.

The Results all calculation are reduced in table of the following form:

Table 4.3 - a Results of the calculation А In D

| ТО | А | В | D | QП | QИ | QД | SE2 | SU2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - The List of the used literature.

4.7 Advisable sources

    - Inkov A.M. The Automation of the technical systems and automation standard technological processes. Introduction to LaboratoryLaboratory work for student of the professions 050702. Shymkent, YUKGU, 2008 - 29 p.

    - Inkov A.M. The Automation of the technical systems. The Study on COMPUTER quality of the connecting processes in mono contour ASR when adjusting the controller on method Ciglera-Nikolisa. Methodical  of the instruction to Laboratory functioning   2 and  3 for student of the professions 050702. SHymkent, YUKGU, 2008 - 14 p., 1таб., silt.

    - Autocontrol in chemical industry: textbook for high school. Under ред. Dudnikova E.G. -M.: Chemistry, 1987.-368с.

    - Izerman R. Digital managerial system: Per. with engl. -M.: World, 1984, -541с.

    - Plyutto V.P. The Laboratory work on theories automatic управле-thread chemist-technological process. Digital siste-my. - M.: Chemistry, 1989, -168с.

    - B. Kuo. Theory and designing the digital systems upravle-niya. -M: Machine building, 1986.-448с.

    - Rotach V.YA. The Laboratory work on course MELT. A Part I. Calculation SAR with analog regulyatorami. -M: MEI, 1992. -52с.

    - Rotach V.YA. The Laboratory work on course MELT. A Part II. Calculation SAR with digital regulyatorami. -M.: MEI, 1992. -52с.

    - Inkov A.M. The Automation of the technical systems and automation standard technological processes. Introduction to LaboratoryLaboratory work for student of the professions 050702. Shymkent, YUKGU, 2008 - 29 p.

Laboratory work №5 «A calculation and investigation of algorithms of filtration and starting

data transformation in ASCTP signals»

1 Goal of work.

The goal of the given Laboratory work is an electronic investigation of the effectiveness of algorithm filtration and smoothing of the using in real industrial ASCTP and investigation of program means of algorithm realization for ASCTP

Program complexes such as IBM PC ALAB7 are used in doing Laboratory work "Investigation of algorithm of smoothing in temporal orders".

6 hours are required for Laboratory lessons and 6 hour of individual work 5.2. Theoretical introduction.

5.2.1. General information about Laboratory work.

For development of the system of automation control, applications of micro process devices and sets electronic sets are broadly used now.  Implementation of program and controlling functions in such devices are realized by means of  program equipment . More flexible and real algorithms are widely used in information data and control systems. At the same time, micro process control devices have no memory storage and productivity. This is the reason of problem of achieving the theory of control and identification. So, instead of doing the tasks of optimal algorithm synthesis of control system, micro controllers are widely used in the system of regulation. Not algorithms of optimal filtration but simple recurrent algorithms of  smoothing and medium smoothing are used in micro process regulative devices [2-12].

In the way of doing the given task, following filter and smoothed algorithms are realized by means of computer devices.

- Sliding medium.

- Smoothing by three points.

- Smoothing by five points.

- Smoothing  by four points.

- Smoothing by means of extrapolitional polinom

- Reduction of information data

- Exponsial smoothing

Methods above don’t need a big amount of calculation expenses and they may be used in the real time systems

Program of filtration and smoothing includes complex programs of dialogue regime of IBM PC ALAB7 "Investigation of algorithms  of smoothing in temporary levels" and allows to realize the following:

1) imitation modeling of smoothing process on the base of algorithms named above and analysis of their effectiveness;

2)formation of testing levels with necessary characteristics of noise (normal distributed or equally distributed);

- light of the program episode on the screen which contains text of filter algorithms;

- the evident graphical reflection of the investigational results on the screen of the display or printing device.

5.2.2. Mathematical setting of the task.

Frequent problem of signal exposition in the presence of occasional noises appears in the task doing.  The methods of filtration are used in doing the tasks. They help to share signals from noises.

X(t)= U(t) + E(t)         (2.1)

Noise is short active and compounding device. Signal is vise versa as a rule long active compounding information data Otherwise consistent meaning of the signal is correlated, as the noise in one point doesn’t depend on the noise of near by points as signal from one point to another doesn’t change its meaning on several points and achieves the meaning of the signal.

Meaning of X(i) and U(i) not their informational signals are used in ASCTS of concrete discrete points of time which are set by Dt steps. For convenience, Dt step is considered constant.   The choice of Dt dimension is characterized by such factors as the sensor of question, an amount of temporal questioning sensors, the speed of measured parameters an etc. transfer from X(t) to X(i) is realized according the formula:

X(i)= X(i-1)+Dх  ,  (2.2)

где   i = 1,2,...,N,

N – an amount of discrete meaning of X.

Formula (1.1) takes view of:

X(i) = U(i) + E(i)  , (2.3)  where  i  may take the meaning of 1 to N.

The consistency of the marks Y(i) forms more flexible curve than final consistency X(i). That is why this is know as smoothing of data or filtration  The task of filtration is to share signal X(i) from signal Y(i). Dimension of filtration method of the use depends on: D = Y(i) - U(i)    (2.4) may be different, we can judge about dispersion meaning relating to U.

In general task setting it is necessary to define some parameters of filtration which supply minimal meaning of D

We set the task to investigate comparative effectiveness of work named in part 2.1  of algorithms of filtration and smoothing

Methods and algorithms of filtration and smoothing are described below. Their mathematical models allow to define Y(i) according the results of dimension X(i).

5.2.2.1. Method of sliding medium.

This method is one of the simple tasks. Sliding meaning Y(i) calculated by formula:

Y(i)*=S*X(i)/m   ,    (2.5)

where S - sum X(J) in J changing from(i-к) to (i+к), к=(m-1)/2, m – length of the interval (or amount of points) of calculation of the medium meaning.

Number м should be even as the getting mark Y(i) relates to the central point. Calculation of Y(i) comprises about м-1/2 meaning with every sides from the appreciated point.  That is why м-1/2 point is lacked in smoothing consistency to м-1 less.

If smoothed interval м is great then procedure brings to considerable shortage of the consistency.

Learn the text of the program:

Index of smoothing consistent Y(i) is characterized in level 400; cycle of I is formed, i refers to (1, Nm+1); as an amount of meanings of Y(i) will be to m-1 less than X(i). 410 and  470 numbers are calculated with the help of information output.

5.2.2.2. Smoothing by means of three points.

Smoothing meaning Y(i)  is calculated according formula:  Y(i)=(X(i-1)+X(i)+X(i+1))/3    ,  (2.6)  i=2,3,...,(N-1) ; for final points:

Y(1)=(5*х1+2*х2-х3)/6 ,    (2.7)

Y(N)=(X(N-2)+2*X(N-1)+5*X(N))/6    (2.8)

Calculation of final members is conducted according the formulas (2.7) and (2.8) in program levels of

120 and 160. Calculation i-х members occurs in cycle, passages 130-155.N should be equal to  3

2.2.3. Smoothing by means of five points.

Smoothing meaning Y(i) is calculated according the formula:

Y(i)=(-3*X(i-2)+12*X(i-1)+17*X(i)+12*X(i+1)-3*X(i+2))/35  (2.9)  i = 3,4,...,(N-2), for final points:

Y(1)=(69*х1+4*х2+6*х3+4*х4-х5)/70    (2.10) Y(2)=(2*х1+27*х2+12*х3-8*х4+2*х5)/35    (2.11)

Y(N-1)=(-8*X(N-4)+12*X(N-3)+2*X(N-2)+27*X(N-1)+2*X(N))/35  (2.12) Y(N)=(4*X(N-4)-6*X(N-3)-X(N-2)+4*X(N-1)+69*X(N))/70    (2.13)

Realization of the formulas (2.10 - 2.13) occurs in the passages of the program 210,215, 231,235.

calculation according the formula (2.9) occurs in the cycle of the passages 217-225.

5.2.2.4. Smoothing of four level.

Smoothing meaning Y(i)  is calculated according the formula:  Y(i)=X(i)-(3/35)*D4    (2.14)  where D4 -  fourth central difference in I point. For final points:

Y(1)=X(1)+(1/5)*D3+(3/35)*D4    (2.15)

Y(2)=X(2)-(2/5)*D3-(1/7)*D4    (2.16)

Y(N-1)=X(N-1)+(2/5)*D3-(1/7)*D4    (2.17)  Y(N)=X(N)-(1/5)*D3+(3/35)*D4    (2.18)

At the beginning of the procedure (passages 315-325) is calculated by the first difference in the passages 335-355. Realization of the formulas (2.14) and (2.15-1.18)  is done in passages 365-390, here the result conclusion is also done.

5.2.2.5. Smoothing of three level polynom.

This method helps to smooth the level of Y(i)  according the given X(i) and constant smoothing @.Except all, till the end of smoothing of the coefficient  A,B,C are defined for the expression:

A+B*T+(C*T*T)/2    (2.19)

This expression may extrapolated  smoothing level for T periods of time before.

This program has two levels for  i=1,2,...N, which begins at A,B,C, or given by the user, or calculated by the mechanism automatically: definition Y(i)  to one step before:

Y(i)=A,B,+0.5*C    (2.20)

Transformation of coefficient  A,B,C about formula:

A=X(i)+(1-@)**3*(Y(i)-X(i))  ;    (2.21)

B=B+C-1.5*(@**2)(2-@)*(Y(i)-X(i));    (2.22)  C=C-(@**3)*(Y(i)-X(i))    ,    (2.23)  where @ - constant smoothing (0<=@<=1) given by the user.

If coefficients A,B,C  are not equal to 0, then given dimensions are taken for final meanings.  If A=B=C=0, then starting meanings A,B and C are formed by the following formulas:

C=X(1)-2*X(2)+X(3)    ,    (2.24)  B=X(2)-X(1)-1.5*C    ,    (2.25)  A=X(1)-B-0.5*C    .    (2.26)

Description of program parameters:

X – output level, N- length,

A1 – constant of smoothing - B1,B2,B3 – coefficient of equality:

B1+B2*T+(B3*T*T)/2

(instead of A,B and C relatively).  Last transformed coefficients of the dimensions Y – smoothing level from – elements care contained in the output of the program.

5.2.2.6. Reduction of data by means of not equal meanings.

On of the methods of the reduction of data is the method of Tuky. This method repeats method of smoothing medium twice (see. 2.2.1). method is described at p.97 in  [3].

Program realization of algorithm is realized at passages of the program 600-750.

5.2.2.7. Exponsial smoothing.

Requirements of the calculation technique are given to use filter exponsial smoothing: Y(t)=@*ехр (@*t)    (2.27)  where @ - filter parameter.

Every discrete filter is described by different equalities:

A(N)*X(i-N)+A(N-1)*X(i-N+1)+...+A(0)*X(i)=B(M)*D(i-M)+ +B(M-1)*D(i-M+1)+...+B(0)*D(i)                    (2.28)

X(i) – discrete output signal D(i) – discrete outlet signal

This suggests to get expression for transfer function of filter in the following ways:

D(Z)    A(N)*Z**(N)+...+A(0)

Y(Z)= ------ = ----------------------    (2.29)

X(Z)    B(M)*Z**(-M)+...+B(0)

For filter of exponsial; smoothing:  Y(Z)=@*Z/(Z+@-1)    (2.30)

Expression is got  for the realization of the electronic file:

D(N)=X(N)+E(N)+(1-@)*(X(N-1)+E(N-1)+...+(1-@)**(N-1)*(X(1)+E(1)+ +(1-@)**N*(X(0)+E(1)) ,

(2.31)

where X(N) -  meaning of input signal at the moment of time T=T(N),

T – an interval of the discreteness (step on time), E(i) – meaning of the noise at the moment of time (i), @ - parameter filter (0<=@<=1).

Expression of the recurrent form (2.31) has the following:

D(N)=@*Z(N)+(1-@)*D(N-1)  ,    (2.32)

Z(N)=X(N)+E(N)    ,    (2.33)  where

X(N)- regular compounding process, E(N)- noise.

Method of smoothing medium is frequently used in this case for marking the current meaning of X(N).   Formula of smoothing level has the view of: Y(N)=(1/N)*S+(1-1/N)*Y(N-1)    (2.34)

where S - sum X(i) and i is changeable from 1 to N, calculation is realized at 700 - 799 passages of

program.

5.2.2.8.  Generation of artificially closed temporal level and mark of the smoothing quality.

Using the standard function RND, we can provide an imposing of the noise onto determinated level for example U(i) at the beginning of the recommended program whish is to be realized in generation of 10 first occasional numbers (by means of receiving the cycle in Q=RND(1)) as first generated numbers are close to zero very much.

Normal (Gaussovo) spreading of the noise may be achieved through the use of the method of marsaly and brey [12]. With the help of function RND two occasional figures are generated V1=2*RND(1)-1 and  V2=2*RND(1)-1. S equal sum is calculated to the sum of the squares of these figures if S>1 or S=1, then numbers V1 и V2 are generated; when  S<1, we calculate:

S1=V1*SQR(-2*LN(S)/S)    (2.35) S2=V2*SQR(-2*LN(S)/S)    (2.36)  where SQR и LN – relatively exposition of square root and taking of the natural logorithm.

We get normal devised occasional number from a gross of normally devised numbers with mathematical waiting and avarage square refusal of S0:

S1=е+S0*S1    (2.37)

Generating closed temporal level X(i) we will use expression X(i)=U(i)+S1*S0    (2.38)

where U(i) – meaning of test function or determinating level at the moment i,  and as S0 we shall use

refusal from U(i).

Formula is used providing the noise if necessary as a consistency of impulses in the corridor of width р relatively to U(i):

X(i)=U(i)-S0*(1-2*RND(1))    (2.39)

For example, if U(i)=SiN(Z(i)) and  S0=0.1*SiN(Zi). Regeneration of the noise at the corridor of 0.1 is provided according to the graphic SiN(Zi). This case it is comfortable to take S0=0.1*SiN(пи/2)=0.1.(гдепи - number пи). Described algorithms are realized in 4000 - 4500 passages of the program.

For marking the quality of smoothing, we shall use the following relations:

(1/(N-1))*SUM1 < (1/(N-1))*SUM2    (2.40)  where suм1 and SUм2 relatively the sum (Y(i+1)-Y(i))**2 and (X(i+1)--X(i))**2  i is changeable from  1 to N-1.

In case if the inequality (2.40) is right and the smoothing is done.

5.2.3. Description of the  program supply in use and control sample

Programs are reliable to the learning of the algorithms of filtration and smoothing of the temporary level. The possibility to use test sample is also considered [6].

Input data for the program is temporary level of X(I) which is introduced into the dialogue. Except all it is necessary to introduce an amount of points of the medium м<N taking into account the loss of smoothing level of м-1 points in output; Al- smoothing constant is also considered as one of the methods of smoothing the extrapolation polinom during input data (0<=а<=1) and the coefficient of polinom: F(t)=B1+B2*T+B3*T*T/2 .    (2.42)

If we take B1, B2,B3 as equal to zero then program produces calculation individually.

Analysis of the work of algorithms of filtration and smoothing was done on test sample which suggests part of sinusoid with imposed signal normally distributed among noise.

Main signal U=Sin(i) imposed on distribution to 20% of noise. Three graphics were taken where  X – signal with noise , U – main signal, Y – smoothing signal.

Numeral criteria of the results of smoothing were taken: disperses х relating to U. According to this criteria this we may consider that how much meaning of closed and smoothing signal differs from that of ideal one But the given criteria doesn’t give an imagination about self smoothed signal.

That is why, for the rating of smoothing the fact, that the difference between nearby members of the closed and smoothed levels should be subdued to such inequality as:

(X(i+1)-X(i)) > (Y(i+1)-Y(i))    (2.43) or

(1/(N-1))*SUм2 > (1/(N-1))*SUм1    ,    (2.44)  где SUM2 и SUM1 relatively the sum (X(i+1)-X(i))**2 и (Y(i+1)--Y(i))**2  i changeable from 1 to  N-1.

It was chosen for the program in our case:

S8=(1/(N-1))*SUM1 ;    (2.45) S9=(1/(N-1))*SUM2 ,    (2.46)

where SUM1 and SUM2 relatively the sums (Y(i+1)-Y(i))**2 и (X(i+1)--X(i))**2  i changes from 1 to N-1.

Relying to the criteria, we may judge about the effectiveness or non effectiveness of the methods for the convenient  of the results of the investigation. See the table.

Table  2.1.

Methods of smoothing |(U(i)-X(i))**2/(U(i)-|S9/S8 ||- Y(i))**2    |    |

- three points                                                 |    1.82    | 4.69 |

- five points                                                  |    1.38    | 2.24 |

- smoothing of fourth level                   |    1.50    | 3.23 |

- sliding medium                                     |    0.82    | 6.52 |

- reduction of in equal data                        |    0.65    | 7.66 |

- exponsial smoothing                            |    3.24    | 7.73 |

------------------------------------------+---------------------+------+

In the first column the methods are indicated, in the second one  - relation of disperse (U(i)-X(i))**2 and (U(i)-Y(i))**2, in third – relation S9/S8 from (2.42) we can see that if  S9/S8 > 1    (2.46)  then smoothing occurs if

(U(i)-X(i))**2/(U(i)-Y(i))**2 > 1    (2.47)

But smoothing signal is lost less than closed.

Table 2.1  suggests that more effective method is the method of exponsial smoothing.

Methods of sliding medium and reduction of in equal data also conduct smoothing but they loses their signal.

5.3 The order of the work doing

Initial data is done by the teacher is generated by the program according the number of the credit book The order of the work is following:

3.1 To learn the methodical instructions and references and to copy main theoretical indications. 3.2 To conduct the work on computer  we should :

1). Load the program. Text is shown on the screen of the display which contains short information about the loading the program. We should press ENTER after learning it;

2). Further, menu with the list of filtration and smoothing methods and questions about method selection are also shown on the screen of the display. Pointing the light, we should select the number of the method from 1 to 7 and press ENTER;

- question is asked about using of the test. If yes press the button 1 and ENTER and if no press the button 0 and press ENTER;

- the list of tests is shown on the screen if you want to choose the work with the test. Choose one of them pressing ENTER

Further program starts to work and an initial time number of machine memory isn’t need in introducing. We should introduce only data about methods named above. There are some requirements to use. We should press ENTER to introduce them.

- you should ask your own data about time number if you refuse to work with the test. N- is an amount of the numbers and members of the time number is  X(i). Press ENTER after introduction of any meaning;

- calculation of the program test starts to work after introduction of data;

- the smoothed number and set graphics are introduced on the display after the calculation;

- the question like “need in copy?” is asked after their introducing on the display. If yes press the buttons 1 and ENTER, if no, press the buttons 0 and ENTER;

- the question like “do you want go on investigation?” is asked after finishing of the calculation. Answer the same way as indicated before. The list of the methods is introduced after answering the questions.

  - To process the investigating time number manually according to the descriptions of the algorithms named above.

  - To do the construction of the graphics of the initial time number, manually smoothed number and smoothed computer number.

  - To analysis the quality of the smoothing which provides every method and calculating expenses for every algorithm and to do the conclusion.

  - To report and to be ready for the control test questions.

5.4 Control questions

1 Give the goal and make an analysis of the investigational results. 2 Write expressions of the described algorithms:

- sliding average.

- smoothing on three points.

- smoothing on five points.

- smoothing on four points.

- smoothing of the extra politional polinom -  reduction of the improbable data.

- exponsial smoothing.

3 Give the examples about the using of the investigational algorithms in ASCTP.

5.5 The content of the report  1 To set the goal of the investigation.

- to make short description about the use of mathematical methods of the calculation.

- To make he results of the analytical transformations and calculations using computer programs.

- To describe the results of computer work.

- To show the graphics of the using and receiving dependences.

- To analyze about the results and make conclusion.

- To make the list of references.

5.6 The list of the references

- Сейдж Э.,Мелc Дж. Теoрия oценивания и ее применения в cвязи и управлении М.; Связь, 1976.-469 c.

- Балакирев В.С., Дудникoв Е.Г., Цирлин А.М. Экcперементальнoе oпределение динамичеcких характериcтик прoмышленных oбъектoв управления М.: Энергия, 1967. -232 c.

- Изерман Р. Цифрoвые cиcтемы управления М.: Мир, 1984. -541 c.

- Отнеc Р. прикладнoй анализ временных рядoв М.: Мир, 1982, -482 c. 5. Практикум пo автoматике и cиcтемам управления прoизвoдc- твенными прoцеccами М.: Химия, 1986. -336 c.

- Бoкc Дж.,Дженкиc Г. Анализ временных рядoв. прoгнoз и управление вып.1 и 2 М.:Мир,1974 197 c. и -406 c.

- Дэвиc Д. Статика и анализ геoлoгичеcких данных М.:Мир, 1977-568 c. 8. Зенкoв В.В. и др. Сравнение ряда алгoритмoв перерабoтки измерительнoй инфoрмации М. Инcтитут прoблем управления, 1971 -69c.

9 .Ицкoвич Э.Л. Контрoль прoизвoдcтва c пoмoщью вычиcлительных машин М.: Энергия, 1975, 416 c.

10. Сбoрник научных прoграмм на Фoртране.  выпуcк 1 М.: Статиcтика, 1974 -315 c.

11.Фритч в.  применение микрoпрoцеccoрoв в cиcтемах управления М.; Мир, 1984, -464 c.

12.Шеннoн Р. Имитациoннoе мoделирoвание cиcтем-иcкуccтвo и наука м. мир, 1978, -418 c

Laboratory work №6 «Research and computer simulation of algorithms of optimum allocation of restricted resources between parallel switched on technological objects in ASCTP»

6.1 Goal of work.

The goal of study research Laboratory work is mastering of setting of the task and methods of the decision of the task of optimum allocation of restricted loading between parallel switched on technological objects in the automated management systems of  technological processes (ASCTP). [1-5].

During the Laboratory work, the program dialogue complex for IBM PC ALAB89 and the programs which have been developed by students are used.

6 hours of Laboratory lessons and 6 hours of independent work are required for the performance.

2. The task for work performance.

The initial data for the task decision is loaded by means of the  characteristics of aggregates (the equation (6.1 see more low)) in a graphic and analytical type, restrictions (6.2) and value S (6.3). It is necessary for work performance:

- to define experimentally tabular values of loaded characteristics of aggregates with usage of a program complex of simulation of system operation of parallel connected aggregates ALAB89;

- To set the value S and restrictions on an admissible range of input controlling influences xi;

- To find analytical expressions of the equations of loaded characteristics of aggregates for the linear and non-linear models;

- To prepare the initial data for the task decision in the assumption of the linear and non-linear models of plant to select a technique of the decision of the task of optimum allocation;

- To formulate setting of the task of optimum allocation of loading;

- To solve the task of optimum allocation of total loading between parallel switched on aggregates for the linear and non-linear models of objects the methods which have been described below; 7) To issue and to report about the  Laboratory work.

  - Theoretical introduction.

6.3.1. Substantive provisions.

There is a necessity of parallel switching on of the objects which are united by the general collector (input) on a resource (on raw materials, energy, financing, capital investments, etc.) in technological and other processes. Such collector circuits allow to raise productivity of technological process and to provide raised reliability of functioning of manufacture as in this case of failure of one of aggregates. It does not lead to the full violation of functions of system. Besides it, switching off of a part of objects allows to work in remain in the most economic mode.

Let's speak about collector circuits further. Parallel switched on aggregates with the general given restricted input resource S (the expenditure of raw materials, capacities, energies, means, etc. on the general input of system of aggregates) are considered when we speak about collectors. It is required to define composition of operating aggregates and to arrange restricted resource S between aggregates so that the general productivity (a total output of all aggregates) is maximum as for the optimization. The decision of such tasks is labor-consuming enough. Besides, the decision should be received operatively and repeatedly in flow of days in the real valid conditions normally implemented in ASCTP. Laboratory determination of optimum allocation of resources is produced by means of special software in ASCTP. It is realized on the basis of the typical algorithms which have been studied in given Laboratory work.

We consider about the system which is parallel switched on aggregates (the Picture 6.1 see). The aggregate is used in part of resource S and an amount (an aggregate input) and overall performance of this aggregate are defined by the function of an aggregate output:

, 			yi = Fi (xi )		(6.1)

It is told about the load (the static load) of the following characteristics such as linear or non-linear, convex or non convex.

Restrictions are imposed on inputs xi:

, 			xi ≤ xi ≤ xi i =1,n (6.2)

According to this, minimum and maximum admissible values of input expenditures and resource of  i-m of the aggregate are defined by technological singularities of object.

If some j-е aggregates are disconnected, there can be type restrictions xi=0.

All circuit from n parallel switched on aggregates (the Picture 1) is in  process on the conditions of restricted and constant general input resource S. Amount S of this resource is set, thus:

n

S = ∑xi				 (6.3)

i=1

Efficiency of all system is defined by value of function of the purpose formed of characteristics of aggregates as the following:

(6.4)

n

I = ∑Fi (xi )

i=1

: ……………………

Агрегат 1

Агрегат 2

Агрегат

n

Picture 6.1. System of n- parallel switched on the aggregates.

6.3.2. Setting of the task of optimum allocation of resources in the system of n- parallel switched on aggregates.

It is required to find such x1, x2..., xn from set D selected with ratios (2-3) and (6.2). Function I reaches a maximum:

n 	max I = max	F (x ) 		(6.5)

The method of the decision of the task depends on a type of characteristics of aggregates (1). We consider the characteristic cases.

6.3.3 Decision of the task at the linear load characteristics. In this case the mathematical model (1) looks like:

Fi (xi ) = ai +bi ⋅ xi		 (6.6)

Then the allocation task is formulated as following:

Maximizing of the linear form is found:

n 	max I = max	(a + b ⋅ x )  	(6.7)

Under conditions (6.3) and (6.2).

This task can be solved by the methods of the linear programming [4] (application 1 see).  However, singularities of structure of the task are allowed to solve by the methods described below [2].

Let's try  to number aggregates in ascending order according to their characteristics, i.e. values. As the coefficient b is characterized by the aggregate "return", i.e. a productivity increment of magnification expenditure and  raw materials are united.  It is comfortablr to set the maximum expenditure of raw materials and aggregates with big “b”.

Let's arrange loadings between aggregates in the following order:

1 = x1;x2 = x2;...x j−1 = xj−1	(6.8) x

j−1 n x j = S − ∑xi − ∑xi (6.9)

i=1	i= j+1

		x j+1 = x j+1;...xn = xn		(6.10)

Select the condition of (6.4) using the  equalities of numbers such as (6.8-6.10) of the aggregate j. The aggregate works with the intermediate level loading.  We show the load of  distribution which is found on the equations of (8-10). It is optimal [2]. For this purpose, we transfer a part of loading from much loaded aggregate to the aggregate l with a minimum loading (k> j; l <j). Thus productivity is changed on value:

∆l(∆x) = ∆x⋅(bl −bk ). 	(6.11)

i.e. productivity is decreased. Thus, allocation of (6.8-6.10) isn’t locally improved. It is optimal

owing to the convexity of the task (6.7).

Thus, optimum allocation of loadings between aggregates with straight-line characteristics is defined by the following rules:

For optimum allocation, it is necessary to load aggregates  as much as possible. They should be inclined by means of great characteristics of  reduction. They are loaded on the aggregates which have been inclined due to small characteristics of reduction.

This rule is generalized on a case of aggregates with convex partly straight-line characteristics [3].

Let's sum up a design procedure on this simple method:

- experimentally, it is found on the loading characteristics (6.1) of each aggregate. If they are explicitly linear, we define tilt angles for each of them at non-linear loaded characteristics (1), we lead to the linearization, for example a method of the least squares (p. 128-130 in [6] and application 2 see) is received by value of selective coefficient of correlation. It is closed as united linearization is admissible and value is defined under formula IV.29 on p. 129 in [6] (application 2 see also).

- it is found under formulas for the aggregates with the essential great values of optimal loadings

(6.8);

- it is found under formulas (6.10) for aggregates with essentially small values of optimal loadings;

- it is found under the formula for (6.9) the aggregate with average value of optimal loading;

- we shall find the  total value S using formula (6.3). It should be corresponded according to the resource S;

- we define the value I using formula (6.4);

- we choose the  adjacent average value of appropriate optimal loading. It is defined using formula

(6.9);

- we define the value I using formula (6.4).

- if the value I isn’t essentially changeable the calculation is ended.

6.3.4. The task decision at the convex non-linear load characteristics should be set analytically.

Let's start from a case, when restrictions (6.2-3) are not considered. As Fi (xi) - convex functions, their total number (6.4) is also convex functionally. Convexity of criterion function I and linearity of communication (6.4) are guaranteed by the uniqueness of the decision. Thus, for search of an extreme of function of many variables are effectively applied by the method of indefinite factors of Lagrange [4]. We use function of Lagrange. This case looks like:

n	n

L = ∑Fi (xi ) +λ⋅(∑xi − S) 		(6.12)

i=1	i=1

Where λ - an indefinite factor of Lagrange.

We receive system from (n+1) the equations  differentiating L and xi and equating derivatives to zero,:

dFdxii = −λ	i =1,n∑i=n1 xi − S = 0  	(6.13)

The decision of this system gives optimal value of loading and value of the multiplier of Lagrange λ.

If to eliminate λ from the equations containing derivatives of characteristics of aggregates, the following condition of an optimality  will be known as «a rule of equal increases» turning out:    		dF1 = dF2 = ... = dFn	 (6.14) dx1	dx2	dxn

In the presence of independent restrictions on xi, some loadings can be accepted  as limiting values. In this case the first of conditions (6.13) looks like:

;, 	dFdxii ⋅∆xi ≤ −λi =1,n			(6.13а)

We receive Δxi-admissible variation of loading I of the aggregate in a point xi i. If, Δx i> 0 and from (6.13а)

dFi (xi ) ≤ −λ		 (6.13б)

dxi

Similarly if  Δxi £ 0 and (5.81а) follows:

dFi (xi ) ≥ −λ		 (6.13в)

dxi

However, it is almost convenient in the case of the restricted loadings and it is analytically given by the characteristics to use the same conditions (6.13). Thus, the algorithm of calculation is following [2]:

1st step. We have the restriction of (6.2). It is calculated as xi for all aggregates of conditions as (6.13).

2nd step. We count the total excess D + as the total of differences for aggregates of the first group, it is appeared in. Similarly we count the total shortage D - of all aggregates of the second group.

3rd step. It is finished if D + = D - to all aggregates of the first group are set with maximum loading and minimum loading to aggregates of the second group.

4th step. If for D +> D we fix loadings of aggregates of the first group on a maximum, we subtract the general loading and residual loading which are arranged between remaining aggregates (1st and 2nd steps see).

5th step. If D + <D, we fix on loadings of aggregates of the second group, we subtract the S totally, we reallocate residual loading between all aggregates which aren’t existed at the second group.

Such processes are continued until boundary or intermediate values of optimal loadings for all aggregates. Each cycle of algorithm is  at least reliable to one of the optimal loading aggregate It is appropriated and eliminated  for further reviewing. The total number of such cycles does not exceed the item

6.3.5. The task decision at the convex non-linear load characteristics has been set graphically

In the case, when characteristics of aggregates are presented in the form of schedules, the graphic method [2-4] can be used in the form of schedules for the decision of system of the equations (6.13). Dependences of productivity on loading Fi (xi) and their derivatives are represented in the upper part of the schedule (the Picture 6.2) dFdxi (ixi ) = βi (xi )

Convex functions of dependence are monotonically decreased.

Picture 6.2. Load characteristics of aggregates and their derivatives (a graphic method of the decision) [2-5].

If it is necessary to consider restrictions of type (6.2), dependences should be predetermined as following:

dFi (xi )

dxi

dFdxi (ixi ) = βi (xi )  xi ≤ xi ≤ xi

dFi (xi ) = Mxi ≤ xi                               (6.15)

dxi

dFi (xi ) = −Mxi ≥ xidxi

n

“M” is  a great number. See Picture 6.2  about constructs of  total dependence. β⋅(∑ xi )

i

The method of the decision of the task is clear from creations on the Picture 6.2. On axes of

n

∑xi ordinates, value b is postponed, for axes of abscissas, the total of the values is corresponded to “b”. i

n

β⋅(∑ xi )

i

Point S is recovered by a perpendicular to intersection from a curve in A. then the point is followed by a horizontal line through a point of  intersection with curves. It is defined as appropriate value. As it is appeared on the creations on fig. 2, productivity derivatives on loading are equal to a point of optimum allocation (the rule of "equal increases» is observed), and total loading is equal to S. Productivity Fi (xi) is defined under the upper schedules of Picture 6.2. Owing to a mono tonicity of functions, the system of the equations (6.13) has been formed by the unique decision.

6.3.6 Decision of the task at the non convex non-linear load characteristics which is set analytically [2].

Load characteristics of all  parts of aggregates can be non convex [4]. It complicates the decision of the task of allocation of resources. We mark that it is frequent for the loading (statically) characteristics and (mathematically) including discontinuity, irregularity. Such situations are generally spoken in real technical systems which are typical to liquidated filtration, approximation, elimination of a trend, smoothing of original signals, etc.) . For example, the method of representation of non convex function by its convex jacket which is quite applied at removal inevitable in technical systems, but almost insignificant variable in the dependences (6.1). It is necessary to mean that at approximation of the experimental dependences of polynomials is explicitly convex by the dependences and analytical representation in some ranges. Especially it can be noticeable at approximation of the experimental dependences. Load characteristics of  polynomials of high levels are resulted in lower technique. It is applied in a case of inevitably and  explicitly non convex load characteristics.

dFdxi (ixi ) = βi (xi )

n

β⋅(∑ xi )

i

dFdxi (ixi ) = −λS = ∑i=n1 xi   (i =1,n)     	(6.16)

Therefore optimality conditions

Select the decision which isn’t unique (х *; l *) and a dial-up of "applicants" for the decision, each of

n which satisfies to conditions (6.16). I = ∑Fi (xi ) Strictly speaking, it is necessary to find all these

i=1 decisions and substitute them in task criterion function.  However the task decision can be facilitated. We should solve it completely passing to average setting. Besides, such setting often is represented with independent interest. It allows to raise the productivity of the circuit by passage of switching modes.

6.3.7. The average task of allocation of resources [2].

Earlier we supposed that the variables in which values are found in result of the decision of the optimal task are not changeable  in time. It is possible to speak about the optimal installed operation mode of the collector circuit shown in a picture 6.2., entering average indexes of its functioning to admit the possibility of available or specially created cyclical changes of consumption of resources

It requires period Т to arrange resource S between п parallel aggregates that is averaged in overall performance of the collector circuit for this maximum period.

Thus, it is admitted to spend a resource bigger or smaller than average intensity S/T during each moment of time if only Т expenditure is equal  to S, i.e.

T	n

∫∑xi (t)dt = S	(6.17)

0 i=1

It is a question of average restriction on intensity of consumption of a resource:

∑n xi	1 T	n	dt	(6.18)

i=1

The efficiency index also has been averaged in character:

I	1 T	n	n	(xi ) 	(6.19)

An average of the total task of allocation equally to the resource set of registers:

n

n ∑i=1 xi = Sxi ≤ xi ≤ xii =1,n    (6.20) max I = max F (x )

Let's solve the task (6.20) in two stages. At the first stage, the device is defined by an optimal mode assuming that average consumption of a resource is known and is equal to Si, i.e. we solve the task

Fi (xi ) →max,     еслиxi = Sixi ≤ xi ≤ xi	(6.21)

The characteristics of the aggregate look like  as in picture 3. Dependence on the set of admissible values xi  is named under condition and convex jacket of function Fi (xi). It represents minimum convex of function in which any xi is equal to Fi.

Picture 6.3. The characteristic of  the aggregate and its convex jacket.

Let's designate it and as Si accepts the same values, as xi it is possible to consider a convex jacket also as function Fi (xi).

If S, lies on a segment  to the optimal decision there corresponds a switching mode at which loading xi accepts values (basic), and in each of them stays during shares from the general time of a cycle g1 and g2 =1 - g1. These are changed at Si. On a section where the convex jacket is coincided with Fi (xi), g1 =1, a g2 =0.

At the second stage for п aggregates, we set average value of the total of loadings

n	n

Or 		∑xi = S∑Si = S	(6.22)

i=1	i

Thus, the average task of allocation can be treated as a choice of such average loadings of aggregates for which the total of averages is maximum. At preset value of  S, the optimal mode of the aggregate should  be corresponded to the decision of the task (6.21), and appropriated productivity 

F(xi ) = Fi (Si ) 		 (6.23)

Passing to the normal task of allocation of loadings of a type:   Under a condition

n 	n

∑Fi (Si ) →max∑Si = S	(6.24)

i=1	i

The role of load characteristics is played by convex jackets of functions Fi (xi). This task is convex and conditions of its optimality are given by means of the unique decision. It is necessary to define basic modes of xi and bi for each aggregate and a share from the general time of a cycle g1 and g2. Loading i- of the aggregate is accepted by the appropriate values.

One of basic values which is coincided with a minimum loading. It is important that number of these values is no more than two, and they are corresponded to those points of characteristics of aggregates for which these characteristics are coincided with their convex jackets.

It aggregates the work in a static mode, and decisions of the average and not averaged tasks are coincided. Thus, it is possible to receive the decision of the task with non convex characteristics, using methods of the decision of much more simple task, in which characteristics are replaced by their convex

jackets. Fi	*) = Fi (Si*) (Si

If for one or several aggregates (we unite them in group)  and the optimal mode are appeared in switching, it is possible to state that their convex jackets on the sections which are including in values of  Si *, are linear, and inclinations of these characteristics are identical. Then it is possible to change Si *, increasingly loading for one of aggregates of group and reducing for another so that their total number is not changed. Thus allocation will remain optimal. Therefore for all aggregates of group except one, we change (we reduce or we increase) all loadings Si* to the values corresponding to the boundaries of sections Those sections are convex strictly by more loaded characteristics. Thus, only one aggregate will work in a switching mode. As this aggregate (we assign it number v) is selected as the aggregate with a minimum difference between a convex jacket and a load characteristic from a point xi = Si*. Then it is possible to state that assignment of all aggregates, except v, static loadings x*i = Si* and accept of loading v aggregate are constant and equal to Sv.  We receive a static mode of productivity of all circuit which is differentiated from extreme value but it is not exceeded     ∆v = F(x*) − Fv (xv*)  (6.25)

v	v

If this difference is insignificant, it is possible to consider the found allocation of approximately optimal non convex characteristics of aggregates.

6.4. The order of work performance.

- To familiarize the methodical instructions and the references, to summarize the main theoretical positions.

- To remove dependence of productivity on the expenditure of raw materials (load characteristic) for each aggregate using an imitative dialogue program complex for personal computers ALAB7_8 (enough 10 - 15 values for each characteristic in all range). To be set by value S and restrictions (6.2).

- Using program ALAB10, system Mathcad (application 3 see) or another way to define analytical expressions of load characteristics for linear (6.6) and non-linear models.

- To solve the task of optimum allocation of restricted loading between parallel switched on technological objects of a method which has been described in section 6.3.3.

- To solve the task of optimum allocation of restricted loading between parallel switched on technological objects of a method which has been described in section 6.3.4.

- To solve the task of optimum allocation of restricted loading between parallel switched on technological objects of a method which has been described in section 3.5.

- To solve the task of optimum allocation of restricted loading between parallel switched on technological objects of a method  which has been described in section 6.3.6.

- To solve the task of optimum allocation of restricted loading between parallel switched on technological objects of a method which has been described in section 6.3.7. 9. To be prepared to control questions (see below)

    - To issue the report.

    - To protect the report.

6.5 Content of the report

    - Introduction. Setting of the goal of the research; the short description of the problem of optimum allocation of restricted loading between parallel switched on technological objects.

    - The Description of the initial data. The obtaining method is described and the table with the experimental data is received by the computer (10 - 15 values for each characteristic in all range); the choice of value S and restrictions (2) are justified.

    - Approximation of results of experiments. The technique and results of determination of analytical expressions of load characteristics for linear (6) and non-linear models are described; numerical results of handling, including values of coefficients of the received dependences and correlation coefficients are resulted; schedules of the initial and approximated dependences are applied.

    - Results of the decision of the task concerning the research sources (for each of the methods which are described in sections 3.3-3.7). Theoretical bases of each method and results of the decision of the task are described.

    - Conclusion.

    - The List of the references.

    - Applications.

6.6 Control questions

1 formulate the purpose and analyze results of researches.

  - Write down

  - Result examples of usage of researched methods.

6.7 Recommended literature

    - Automatic control in the chemical industry: the textbook for high schools. Under the editorship of Dudnikova E.G. - TH.: Chemistry, 1987.-368s.

    - The Laboratory work on automatics and management systems productions: the Manual for high schools / Under the editorship of I.M.Maslennikov. - М: Chemistry, 1986.-336s.

    - Minsker I.N.operative of control of himiko-technological complexes. М: Chemistry, 1972 221 with.

    - Tsirlin A.M.optimum control technological processes: the Manual for high schools. - TH.: Энергоатомиздат, 1986.-400s.

    - Balakirev V. S, Volodin V. M, Tsirlin A.M., Optimum control of processes of chemical technology. - TH.: Chemistry, 1978-383s.

    - Ahnazarova S.L., Kafarov V.V.Metody of optimization of experiment in chemical technology: the Manual for high schools. - 2 issuing, - TH.: the Higher school, 1985.-327s.

    - Bojarinov V.Metody of optimization in chemistry and chemical technology - 3 issuing, - TH.: Chemistry, 1991.-527s

Criteria for assessing students' knowledge

| Estimation in letters | Equivalent of scores in figures | In per cent | Traditional assessment | A criterion for assessing the knowledge of students |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | Excellent | The student demonstrates: - a deep and lasting assimilation of program material - complete, consistent, competent and logically presented answers for all types of activities; - Ability to freely cope with the tasks; - correct, well-founded decisions, - skills in the use of information from basic and additional specialized literature in the study of discipline, - the ability to self-systematize the program material; - Possession of versatile skills and methods of performing all types of tasks; - Timely execution of all types of tasks. |
| А- | 3,67 | 90-94 |  | The student demonstrates: - Deep assimilation of program material; - complete, consistent and logically presented answers for all activities; - ability to cope with the tasks; - the right decisions, - skills of using special literature in the study of discipline, - the ability to self-systematize the program material; - Possession of skills and techniques for performing all types of tasks; - Timely execution of all types of tasks. |
| В+ | 3,33 | 85-89 | Good | The student demonstrates: - assimilation of program material; - complete, consistent, competent, without significant inaccuracies, an account of the answers for all types of tasks; - correct application of theoretical knowledge - Possession of necessary skills in the performance of applied tasks - skills in using recommended literature in the study of discipline, - skills of systematization of program material; - Possession of skills and techniques for performing all types of tasks; - Timely execution of all types of tasks. |
| В | 3,0 | 80-84 |  | The student demonstrates: - assimilation of program material; - a consistent presentation of the answers for all types of tasks with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - Possession of necessary skills in performing Laboratory tasks - skills in using recommended literature in the study of discipline, - skills of systematization of program material under the guidance of a teacher; - mastering the skills of performing all kinds of tasks; - the ability to independently correct mistakes made; - the timely execution of all types of tasks with the elimination of errors. |
| В- | 2,67 | 75-79 |  | The student demonstrates: - assimilation of program material; - Ability to present answers with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - Possession of techniques in the performance of Laboratory tasks - skills in using recommended literature under the guidance of a teacher - skills of generalization of program material under the guidance of a teacher; - the ability to correct mistakes made with the help of a teacher; - the timely execution of all types of tasks with the elimination of errors. |
| С+ | 2,33 | 70-74 |  | The student demonstrates: - mastering of the main material; - Inadequate formulations when responding to all kinds of tasks; - violation of the sequence in the presentation of the program material; - difficulties in self-fulfillment of Laboratory tasks; - Possession of individual techniques in the performance of Laboratory tasks - skills in using the literature recommended by the teacher; - skills of generalization of separate sections of program material under the guidance of a teacher; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С | 2,0 | 65-69 | Satisfactory | The student demonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - difficulties in self-fulfillment of Laboratory tasks; - Possession of separate receptions at performance of tasks - difficulty in using the literature recommended by the teacher; - difficulties in generalizing the individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С- | 1,67 | 60-64 |  | The student demonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of Laboratory tasks; - Inadequate possession of individual methods in the performance of assignments - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| Д+ | 1,33 | 55-59 |  | The student demonstrates: - mastering of separate sections of the main material; - lack of understanding of the formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of Laboratory tasks; - significant difficulties in the application of individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - difficulties in correcting the mistakes made by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| Д | 1,0 | 50-54 |  | The student demonstrates: - difficulties in mastering individual sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of Laboratory tasks; - Inability to apply individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - Inability to generalize the individual sections of the material studied; - significant difficulties in correcting the mistakes committed by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| FX | 0 | 25-49 | Unsatisfactory | The student demonstrates: - not knowledge of program material, - when performing all types of tasks, gross errors are allowed; - lack of skills in the use of individual methods of performing tasks; - Failure to complete certain types of tasks, provided by the forms of the current, intermediate and final control. |
| F | 0 | 0-24 |  | The student demonstrates: - not knowledge of program material, - when performing all types of tasks, gross errors are allowed; - lack of skills in the use of individual methods of performing tasks; - Failure to complete certain types of tasks, provided by the forms of the current, intermediate and final control. |

ARKHABAYEV A.B., UMBETOV A.Zh., MAKHANBETOV A.Zh.

METHODICAL INDICATIONS

for laboratory workon discipline

«АUTOMATED CONTROL SYSTEMS FOR MEANS AND SCIENTIFIC RESEARCHES IN COMMUNICATION SYSTEMS»

for students of the specialty 5В071900 (6B06210) – «Radio technology,electronics and telecommunication»

It is sent for printing

Format of paper Х*Y 1/16

Paper is typographical. Press is offset. Volume 2 items of l.

Circulation ___ copies. Order № ___

©  Edition of М. Auezov South Kazakhstan State University

Publishing center of M. Auezov SKSU, Shymkent city,

Tauke-khan avenue, 5

1+TF ⋅ p			(9),