Ф.7.03-03

Botayeva S.B., Botayev B.B., Umarova Zh.R.

Methodical guidelines to SIW on discipline «Computer System Architecture»

for students of specialty 5В070300 – Information systems

Form of training – daily

Shymkent, 2019

MINISTRY OF EDUCATION AND SCIENCE OF REPUBLIC OF KAZAKHSTAN

M.AUEZOV SOUTH KAZAKHSTAN STATE UNIVERSITY

Chair «Information Systems and modeling»

Botayeva S.B., Botayev B.B., Umarova Zh.R.

Methodical guidelines to SIW on discipline «Computer System Architecture»

for students of specialty 5В070300 – Information systems

Form of training – daily

Shymkent, 2019

UDC 681.5 (075.8)

The author: Botayeva S.B., Botayev B.B., Umarova Zh.R. Methodical guidelines to SIW on discipline «Computer System Architecture» for students of specialty 5В070300 – Information systems. Shymkent: SKSU, 2019. – 60 p.

The Methodical guidelines to SIW has been drawn up in accordance with the curriculum and course program on discipline «Computer System Architecture» and includes all necessary data concerning the execution of themes of SIW.

Reviewer:

Kurakbayeva S. – c.t.s., associate professor of the chair «Information Systems and modeling» of M.Auezov SKSU.

The Methodical guidelines to SIW have been discussed at the meeting of the chair “Information Systems and modeling” (Protocol № ____ « ____ » ______ 2019) and they have been approved by Committee on innovational technologies of training and Methodical provision of Higher school “Information Technologies and Energetics” (Protocol № ____ « ____ » ______ 2019)

The Methodical guidelines to SIW are recommended to the edition by Educational-methodical council of M.Auezov SKSU,

Protocol №___ from «___ »____ 2019.

© M.Auezov South Kazakhstan State University, 2019

Responsible for printing: Botayeva S.B.

CONTENT

| Introduction …………………………………………………………………... | 6 |
| --- | --- |
| SIW 1. Logic Design. ………………..…………………………………….…. | 7 |
| SIW 2. Combinational logic ……….................................................................. | 13 |
| SIW 3. Sequential Circuits, Memory, and State................................................ | 21 |
| SIW 4. Representing instructions in the Computer .......................................... | 32 |
| SIW 5. Constructing an ALU--the fun begins................................................... | 38 |
| SIW 6. Simulating Combinatorial Circuits at the Gate Level. ……………….. | 41 |
| Recommended references ……………………………………………………. | 57 |

Introduction

Text is Hennessy and Patterson « Computer System Architecture».

- Available in bookstore.

- The main body of the book assumes you know logic design.

- I do NOT make that assumption.

- We will start with appendix B, which is logic design review.

- A more extensive treatment of logic design is M. Morris Mano «Computer System Architecture», Prentice Hall.

- We will not need as much as mano covers and it is not a cheap book so I am not requiring you to get it. I will have it put into the library.

- My treatment will follow H&P not mano.

- Most of the figures in these notes are based on figures from the course textbook. The following copyright notice applies.

«All figures from Computer Organization and Design: The Hardware/Software Approach, Second Edition, by David Patterson and John Hennessy, are copyrighted material. Figures may be reproduced only for classroom or personal educational use in conjunction with the book and only when the above copyright line is included. They may not be otherwise reproduced, distributed, or incorporated into other works without the prior written consent of the publisher»

### Homeworks and Labs

I make a distinction between homework and labs.

Labs are

- Required

- Due several lectures later (date given on assignment)

- Graded and form part of your final grade

- Penalized for lateness

Homeworks are

- Optional

- Due beginning of Next lecture

- Not accepted late

- Mostly from the book

- Collected and returned

- Can help, but not hurt, your grade

Upper left board for assignments and announcements.

SIW 1. Logic Design

## Gates, Truth Tables and Logic Equations

Homework: Read B2 Digital ==> Discrete

Primarily (but NOT exclusively) binary at the hardware level

Use only two voltages -- high and low

- This hides a great deal of engineering

- Must make sure not to sample the signal when not in one of these two states.

- Sometimes it is just a matter of waiting long enough (determines the clock rate i.e. how many megahertz)

- Other times it is worse and you must avoid glitches.

- Oscilloscope traces shown below

  - Vertical axis is voltage; horizontal axis is time.

  - Square wave--the ideal. How we think of circuits

  - Sine wave

  - Actual wave

    - Non-zero rise times and fall times

    - Overshoots and undershoots

    - Glitches

Since this is not an engineering course, we will ignore these issues and assume square waves.

In English digital (think digit, i.e. finger) => 10, but not in computers

Bit = Binary digIT

Instead of saying high voltage and low voltage, we say true and false or 1 and 0 or asserted and deasserted.

0 and 1 are called complements of each other.

A logic block can be thought of as a black box that takes signals in and produces signals out. There are two kinds of blocks

- Combinational (or combinatorial)

  - Does NOT have memory elements

  - Is simpler than circuits with memory since it is a function from the inputs to the outputs

- Sequential

  - Contains memory

  - The current value in the memory is called the state of the block.

  - The output depends on the input AND the state

We are doing combinational now. Will do sequential later (few lectures).

### TRUTH TABLES

Since combinatorial logic has no memory, it is simply a function from its inputs to its outputs. A Truth Table has as columns all inputs and all outputs. It has one row for each possible set of input values and the output columns have the output for that input. Let's start with a really simple case a logic block with one input and one output.

There are two columns (1 + 1) and two rows (2**1).

In  Out

0   ?

1   ?

#### How many are there?

How many different truth tables are there for one in and one out?

Just 4: the constant functions 1 and 0, the identity, and an inverter (pictures in a few minutes). There were two `?'s in the above table each can be a 0 or 1 so 2**2 possibilities.

OK. Now how about two inputs and 1 output.

Three columns (2+1) and 4 rows (2**2).

In1 In2  Out

0   0    ?

0   1    ?

1   0    ?

1   1    ?

How many are there? It is just how many ways can you fill in the output entries. There are 4 output entries so answer is 2**4=16.

How about 2 in and 8 out?

- 10 cols

- 4 rows

- 2**(4*8)=4 billion possible

3 in and 8 out?

- 11 cols

- 8 rows

- 2**(8**8)=2**64 possible

n in and k out?

- n+k cols

- 2**n rows

- 2**([2**n]*k) possible

Gets big fast!

### Boolean algebra

Certain logic functions (i.e. truth tables) are quite common and familiar.

We use a notation that looks like algebra to express logic functions and expressions involving them.

The notation is called Boolean algebra in honor of George Boole.

A Boolean value is a 1 or a 0.
A Boolean variable takes on Boolean values. 
A Boolean function takes in boolean variables and produces boolean values.

- The (inclusive) OR Boolean function of two variables. Draw its truth table. This is written + (e.g. X+Y where X and Y are Boolean variables) and often called the logical sum. (Three out of four output values in the truth table look right!)

- AND. Draw TT. Called log product and written as a centered dot (like product in regular algebra). All four values look right.

- NOT. Draw TT. This is a unary operator (One argument, not two like above; the two above are called binary). Written A with a bar over it (I will use ' instead of a bar as it is easier for my to type).

- Exclusive OR (XOR). Written as + with circle around. True if exactly one input is true (i.e. true XOR true = false). Draw TT.

Homework: Consider the Boolean function of 3 boolean vars that is true if and only if exactly 1 of the three variables is true. Draw the TT.

Some manipulation laws. Remember this is Boolean ALGEBRA.

Identity:

- A+0 = 0+A = A

- A.1 = 1.A = A

- (using . for and)

Inverse:

- A+A' = A'+A = 1

- A.A' = A'.A = 0

- (using ' for not)

Both + and . are commutative so don't need as much as I wrote

The name inverse law is somewhat funny since you Add the inverse and get the identity for Product or Multiply by the inverse and get the identity for Sum.

Associative:

- A+(B+C) = (A+B)+C

- A.(B.C)=(A.B).C

Due to associative law we can write A.B.C since either order of evaluation gives the same answer.

Often elide the . so the product associative law is A(BC)=(AB)C.

Distributive:

- A(B+C)=AB+AC

- A+(BC)=(A+B)(A+C)

- Note that BOTH distributive laws hold UNLIKE ordinary arithmetic.

How does one prove these laws??

- Simple (but long) write the TTs for each and see that the outputs are the same.

- Do the first dist laws on the board.

Homework: Do the second distributive law.

Let's do (on the board) the examples on pages B-5 and B-6. Consider a logic function with three inputs A, B, and C; and three outputs D, E, and F defined as follows: D is true if at least one input is true, E if exactly two are true, and F if all three are true. (Note that by if we mean if and only if.

Draw the truth table.

Show the logic equations

- For E first use the obvious method of writing one condition for each 1-value in the E column i.e.
(A'BC) + (AB'C) + (ABC')

- Observe that E is true if two (but not three) inputs are true, i.e.,
(AB+AC+BC) (ABC)' (using . higher precedence than +)

The first way we solved part E shows that any logic function can be written using just AND, OR, and NOT. Indeed, it is in a nice form. Called two levels of logic, i.e. it is a sum of products of just inputs and their compliments.

DeMorgan's laws:

- (A+B)' = A'B'

- (AB)' = A'+B'

You prove DM laws with TTs. Indeed that is ...

Homework: B.6 on page B-45

Do beginning of HW on the board.

With DM we can do quite a bit without resorting to TTs. For example one can show that the two expressions for E on example above (page B-6) are equal. Indeed that is

Homework: B.7 on page B-45

Do beginning of HW on board.

### GATES

Gates implement basic logic functions: AND OR NOT XOR Equivalence

Show why the picture is equivalence, i.e (A XOR B)' is AB + A'B'

(A XOR B)' =

(A'B+AB')' =

(A'B)' (AB')' =

(A»+B') (A'+B») =

(A + B') (A' + B) =

AA' + AB + B'A' + B'B =

0   + AB + B'A' + 0 =

AB + A'B'

Often omit the inverters and draw the little circles at the input or output of the other gates (AND OR). These little circles are sometimes called bubbles.

This explains why inverter is drawn as a buffer with a bubble.

Homework: B.2 on page B-45 (I previously did the first part of this homework).

Homework: Consider the Boolean function of 3 boolean vars (i.e. a three input function) that is true if and only if exactly 1 of the three variables is true. Draw the TT. Draw the logic diagram with AND OR NOT. Draw the logic diagram with AND OR and bubbles.

We have seen that any logic function can be constructed from AND OR NOT. So this triple is called universal.

Are there any pairs that are universal? Could it be that there is a single function that is universal? YES!

NOR (NOT OR) is true when OR is false. Do TT.

NAND (NOT AND) is true when AND is false. Do TT.

Draw two logic diagrams for each, one from definition and equivalent one with bubbles.

Theorem A 2-input NOR is universal and A 2-input NAND is universal.

Proof

We must show that you can get A', A+B, and AB using just a two input NOR.

- A' = A NOR A

- A+B = (A NOR B)' (we can use ' by above)

- AB = (A' OR B')'

Homework: Show that a 2-input NAND is universal.

Can draw NAND and NOR each two ways (because (AB)' = A' + B')

We have seen how to get a logic function from a TT. Indeed we can get one that is just two levels of logic. But it might not be the simplest possible. That is we may have more gates than necessary.

Trying to minimize the number of gates is NOT trivial. Mano covers this in detail. We will not cover it in this course. It is not in H&P. I actually like it but must admit that it takes a few lectures to cover well and it not used so much since it is algorithmic and is done automatically by CAD tools.

Minimization is not unique, i.e. there can be two or more minimal forms.

Given A'BC + ABC + ABC' 
Combine first two to get BC + ABC'
Combine last two to get A'BC + AB

Sometimes when building a circuit, you don't care what the output is for certain input values. For example, that input combination is cannot occur. Another example occurs when, for this combination of input values, a later part of the circuit will ignore the output of this part. These are called don't care outputs situations. Making use of don't cares can reduce the number of gates needed.

Can also have don't care inputs when , for certain values of a subset of the inputs, the output is already determined and you don't have to look at the remaining inputs. We will see a case of this in the very next topic, multiplexors.

### An aside on theory

Putting a circuit in disjunctive normal form (i.e. two levels of logic) means that every path from the input to the output goes through very few gates. In fact only two, an OR and an AND. Maybe we should say three since the AND can have a NOT (bubble). Theorticians call this number (2 or 3 in our case) the depth of the circuit. Se we see that every logic function can be implemented with small depth. But what about the width, i.e., the number of gates.

The news is bad. The parity function takes n inputs and gives TRUE if and only if the number of input TRUEs is odd. If the depth is fixed (say limited to 3), the number of gates needed for parity is exponential in n.

SIW 2. COMBINATIONAL LOGIC

Homework: Read B.3.

Generic Homework: Read sections in book corresponding to the lectures.

### Multiplexor

Often called a mux or a selector

Show equiv circuit with AND OR

Hardware if-then-else

if S=0

M=A

else

M=B

endif

Can have 4 way mux (2 selector lines)

This is an if-then-elif-elif-else

if S1=0 and S2=0

M=A

elif S1=0 and S2=1

M=B

elif S1=1 and S2=0

M=C

else -- S1=1 and S2=1

M=D

endif

Do a TT for 2 way mux. Redo it with don't care values.
Do a TT for 4 way mux with don't care values.

Homework: B-12. Assume you have constant signals 1 and 0 as well.

### Decoder

- Note the «3» with a slash, which signifies a three bit input. This notation represents three (1-bit) wires.

- Takes n signals in produces 2^n signals out

- View input as «binary n», the output has n'th bit set

- Implement on board with AND/OR

#### Encoder

- Reverse "function" of encoder

- Not defined for all inputs (exactly one must be 1)

Sneaky way to see that NAND is universal.

- First show that you can get NOT from NAND. Hence we can build inverters.

- Now imagine that you are asked to do a circuit for some function with N inputs. Assume you have only one output.

- Using inverters you can get 2N signals the N original and N complemented.

- Recall that the natural sum of products form is a bunch of ORs feeding into one AND.

- Naturally you can add pairs of bubbles since they «cancel»

- But these are all NANDS!!

#### Half Adder

- Inputs X and Y

- Outputs S and Co (carry out)

- No CarryIn

- Draw TT

Homework: Draw logic diagram

#### Full Adder

- Inputs X, Y and Ci

- Output S and Co

- S = #1s in X, Y, Ci is odd

- Co = #1s is at least 2

Homework:

- Draw TT (8 rows)

- show S = X XOR Y XOR Ci

- show Co = XY + (X XOR Y)Ci

How about 4 bit adder ?

How about n bit adder ?

- Linear complexity

- Called ripple carry

- Faster methods exist

### PLAs--Programmable Logic Arrays

Idea is to make use of the algorithmic way you can look at a TT and produce a circuit diagram in the sums of product form.

Consider the following TT from the book (page B-13)

A | B | C || D | E | F

--+---+---++---+---+--

O | 0 | 0 || 0 | 0 | 0

0 | 0 | 1 || 1 | 0 | 0

0 | 1 | 0 || 1 | 0 | 0

0 | 1 | 1 || 1 | 1 | 0

1 | 0 | 0 || 1 | 0 | 0

1 | 0 | 1 || 1 | 1 | 0

1 | 1 | 0 || 1 | 1 | 0

1 | 1 | 1 || 1 | 0 | 1

- Recall that there is a big OR for each output

- We can see that there are 7 product terms that will be used in one or more of the ORs (in fact all seven will be used in D, but that is special to this example)

- Each of these product terms is called a Minterm

- So we need a bunch of ANDs taking A, B, C as inputs (and their complements A', B', and C')

- This is called an AND plane and the collection of ORs mentioned above is called a OR plane.

- Convert to sum of product forms (only NOTs on vbles)

Here is the circuit diagram for this truth table.

Here it is redrawn

- This figure shows more clearly the AND plane, the OR plane, and the minterms.

- Rather than having bubbles (i.e., custom and gates that invert), we simply invert each input once and send the inverted signal all the way accross.

- AND gates are shown as vertical lines; ORs as horizontal.

- Note the dots to represent connections

Finally, it can be redrawn in a more abstract form.

When a PLA is manufactured all the connections have been specified. That is, a PLA is specific for a given circuit. It is somewhat of a misnomer since it is notprogrammable by the user

Homework: B.10 and B.11

Can also have a PAL or Programmable array logic in which the final dots are specified by the user. The manufacturer produces a «sea of gates»; the user programs it to the desired logic function.

Homework: Read B-5

### ROMs

One way to implement a mathematical (or C) function (without side effects) is to perform a table lookup.

A ROM (Read Only Memory) is the analogous way to implement a logic function.

- For a math function f we start with x and get f(x).

- For a ROM with start with the address and get the value stored at that address.

- Normally math functions are defined for an infinite number of values, for example f(x) = 3x for all real numbers x

- We can't build an infinite ROM (sorry), so we are only interested in functions defined for a finite number of values. Today a million is OK a billion is too big.

- To create the ROM for the function f(3)=4, f(6)=20 all other values undefined just have the ROM store 4 in address 3 and 20 in address 6

- Consider a function defined for all n-bit numbers (say n=20) and having a k-bit output for each input.

  - View each n-bit input as n 1-bit inputs.

  - View each k-bit output as k 1-bit outputs.

  - Since there are 2^n inputs and each requires a k 1-bit output, there are a total of (2^n)k bits of output, i.e. the ROM must hold (2^n)k bits.

  - Imagine a truth table with n inputs and k outputs. The total number of output bits is again (2^n)k (2^n rows and k output columns).

- Thus the ROM implements a truth table, i.e. is a logic function.

Important: The ROM is does not have state. It is still a combinational circuit. That is, it does not represent «memory». The reason is that once a ROM is manufactured, the output depends only on the input.

A PROM is a programmable ROM. That is you buy the ROM with «nothing» in its memory and then before it is placed in the circuit you load the memory, and never change it.

An EPROM is an erasable PROM. It costs more but if you decide to change its memory this is possible (but is slow).

«Normal» EPROMs are erased by some ultraviolet light process. But EEPROMs (electrically erasable PROMS) are faster and are done electronically.

All these EPROMS are erasable not writable, i.e. you can't just change one bit.

A ROM is similar to PLA

- Both can implement any truth table, in principle.

- A 2Mx8 ROM can really implment any truth table with 21 inputs (2^21=2M) and 8 outputs.

  - It stores 2M bytes

  - In ROM-speak, it has 21 address pins and 8 data pins

- A PLA with 21 inputs and 8 outputs might need to have 2M minterms (AND gates).

  - The number of minterms depends on the truth table itself.

  - For normal TTs with 21 inputs the number of minterms is MUCH less than 2^21.

  - The PLA is manufactured with the number of minterms needed

- Compare a PAL with a PROM

  - Both can in principle implement any TT

  - Both are user programmable

  - A PROM with n inputs and k outputs can implement any TT with n inputs and k outputs.

  - A PAL does not have enough gates for all possibilities since most TTs with n inputs and k outputs don't require nearly (2^n)k gates.

### Don't Cares

- Sometimes not all the input and output entries in a TT are needed. We indicate this with an X and it can result in a smaller truth table.

- Input don't cares.

  - The output doesn't depend on all inputs, i.e. the output has the same value no matter what value this input has.

  - We saw this when we did muxes

- Output don't cares

  - For some input values, either output is OK.

    - This input combination is impossible.

    - For this input combination, the given output is not used (perhaps it is «muxed out» downstream)

Example

- If A or C is true, then D is true (independent of B).

- If A or B is true, the output E is true.

- F is true if exactly one of the inputs is true, but we don't care about the value of F if both D and E are true

Full truth table

A   B   C || D   E   F

----------++----------

0   0   0 || 0   0   0

0   0   1 || 1   0   1

0   1   0 || 0   1   1

0   1   1 || 1   1   0

1   0   0 || 1   1   1

1   0   1 || 1   1   0

1   1   0 || 1   1   0

1   1   1 || 1   1   1

Put in the output don't cares

A   B   C || D   E   F

----------++----------

0   0   0 || 0   0   0

0   0   1 || 1   0   1

0   1   0 || 0   1   1

0   1   1 || 1   1   X

1   0   0 || 1   1   X

1   0   1 || 1   1   X

1   1   0 || 1   1   X

1   1   1 || 1   1   X

Now do the input don't cares

- B=C=1 ==> D=E=11 ==> F=X ==> A=X

- A=1 ==> D=E=11 ==> F=X ==> B=C=X

A   B   C || D   E   F

----------++----------

0   0   0 || 0   0   0

0   0   1 || 1   0   1

0   1   0 || 0   1   1

X   1   1 || 1   1   X

1   X   X || 1   1   X

These don't cares are important for logic minimization. Compare the number of gates needed for the full TT and the reduced TT. There are techniques for minimizing logic, but we will not cover them.

### Arrays of Logic Elements

- Do the same thing to many signals

- Draw thicker lines and use the "by n" notation.

- Diagram below shows 32- bit 2-way mux and implementation with 32 muxes

- A Bus is a collection of data lines treated as a single logical (n-bit) value.

- Use an array of logic elements to process a bus. For example, the above mux switches between 2 32-bit buses.

32-bit mux

SIW 3. Sequential Circuits, Memory, and State

Why do we want to have state?

- Memory (i.e. ram not just rom or prom)

- Counters

- Reducing gate count

  - Multiplier would be quadradic in comb logic.

  - With sequential logic (state) can do in linear.

    - What follows is unofficial (i.e. too fast to understand)

    - Shift register holds partial sum

    - Real slick is to share this shift reg with multiplier

    - We will do this circuit later in the course

Assume you have a real OR gate. Assume the two inputs are both zero for an hour. At time t one input becomes 1. The output will OSCILLATE for a while before settling on exactly 1. We want to be sure we don't look at the answer before its ready.

## Clocks

Frequency and period

- Hertz (Hz), Megahertz, Gigahertz vs. Seconds, Microseconds, Nanoseconds

- Old (descriptive) name for Hz is cycles per second (CPS)

- Rate vs. Time

Edges

- Rising Edge; falling edge

- We use edge-triggered logic

- State changes occur only on a clock edge

- Will explain later what this really means

- One edge is called the Active edge

  - The edge (rising or falling) on which changes occur

  - Choice is technology dependent

  - Sometimes trigger on both edges (e.g., RAMBUS memory)

#### Synchronous system

Now we are going to add state elements to the combinational circuits we have been using previously.

Remember that a combinational/combinatorial circuits has its outpus determined by its input, i.e. combinatorial circuits do not contain state.

Reading and writing State Elements.

State elements include state (naturally).

- i.e., memory

- state-elements have clock as an input

- can change state only at active edge

- produce output Always; based on current state

- all signals that are written to state elements must be valid at the time of the active edge.

- For example, if cycle time is 10ns make sure combinational circuit used to compute new state values completes in 10ns

- So state elements change on active edge, comb circuit stablizes between active edges.

- Can have loops like this.

- Think of registers or memory as state elements.

- A loop like the above is a cycle of the computer

## Memory Elements

We want edge-triggered clocked memory and will only use edge-triggered clocked memory in our designs. However we get there by stages. We first show how to build unclocked memory; then using unclocked memory we build level-sensitive clocked memory; finally from level-sensitive clocked memory we build edge-triggered clocked memory.

#### Unclocked Memory

S-R latch (set-reset)

- «Cross-coupled» nor gates

- Don't assert both S and R at once

- When S is asserted (i.e., S=1 and R=0)

  - the latch is Set (that's why it is called S)

  - Q becomes true (Q is the output of the latch)

  - Q' becomes false (Q' is the complemented output)

- When R is asserted

  - the latch is Reset

  - Q becomes false

  - Q' becomes true

- When neither one is asserted

  - The latch remains the same, i.e. Q and Q' stay as they were

  - This is the memory aspect

### Clocked Memory: Flip-flops and latches

The S-R latch defined above is not clocked memory. Unfortunately the terminology is not perfect.

For both flip-flops and latches the output equals the value stored in the structure. Both have an input and an output (and the complemented output) and a clock input as well. The clock determines when the internal value is set to the current input. For a latch, the change occurs whenever the clock is asserted (level sensitive). For a flip-flop, the change only occurs during the active edge.

#### D latch

The D is for data

- The left part uses the clock.

  - When the clock is low, both R and S are forced low

  - When the clock is high, S=D and R=D' so the value store is D

- Output changes when input changes and the clock is asserted

- Level sensitive rather than edge triggered

- Sometimes called a transparent latch

- We won't use these in designs

- The right part is the S-R (unclocked) latch we just did

In the traces below notice how the output follows the input when the clock is high and remains constant when the clock is low. We assume the stored value is initially low.

#### D or Master-Slave Flip-flop

This was our goal. We now have an edge-triggered, clocked memory.

- Built from D latches, which are transparent

- The result is Not transparent

- Changes on the active edge

- This one has the falling edge as active edge

- Sometimes called a master-slave flip-flop

- Note substructures with letters reused having different meaning (block structure a la algol)

- Master latch (the left one) is set during the time clock is asserted. Remember that the latch is transparent, i.e. follows its input when its clock is asserted. But the second latch is ignoring its input at this time. When the clock falls, the 2nd latch pays attention and the first latch keeps producing whatever D was at fall-time.

- Actually D must remain constant for some time around the active edge.

  - The set-up time before the edge

  - The hold time after the edge

  - See diagram below

Note how much less wiggly the output is with the master-slave flop than before with the transparent latch. As before we are assuming the output is initially low.

Homework: Try moving the inverter to the other latch What has changed?

This picture shows the setup and hold times discussed above. It is crucial when building circuits with flip flops that D is stable during the interval between the setup and hold times. Note that D is wild outside the critical interval, but that is OK.

Homework: B.18

#### Registers

- Basically just an array of D flip-flops

- But what if you don't want to change the register during a particular cycle?

- Introduce another input, the write line

- The write line is used to «gate the clock»

  - The book forgot the write line.

  - Clearly if the write line is high forever, the clock input to the register is passed right along to the D flop and hence the input to the register is stored in the D flop when the active edge occurs (for us the falling edge).

  - Also clear is that if the write line is low forever, the clock to the D flop is always low so has no edges and no writing occurs.

  - But what about changing the write line?

  - Assert or deassert the write line while the clock is low and keep it at this value until the clock is low again.

  - Not so good! Must have the write line correct quite a while before the active edge. That is you must know whether you are writing quite a while in advance.

  - Better to do things so the write line must be correct when the clock is high (i.e., just before the active edge

  - An alternative is to use an active low write line, i.e. have a W' input.

- Must have write line and data line valid during setup and hold times

- To do a multibit register, just use multiple D flopss.

#### Register File

Set of registers each numbered

- Supply reg#, write line, and data (if a write)

- Can read and write same reg same cycle. You read the old value and then the written value replaces this old value for subsequent cycles.

- Often have several read and write ports so that several registers can be read and written during one cycle.

- We will do 2 read ports and one write port since that is needed for ALU ops. This is Not adequate for superscalar (or EPIC) or any other system where more than one operation is to be calculated each cycle.

To read just need mux from register file to select correct register.

- Have one of these for each read port

- Each is an n to 1 mux, b bits wide; where

  - n is the number of registers (32 for MIPS)

  - b is the width of each register (32 for MIPS)

For writes use a decoder on register number to determine which register to write. Note that 3 errors in the book's figure were fixed

- decoder is log n to n

- decoder outputs numbered 0 to n-1 (NOT n)

- clock is needed

The idea is to gate the write line with the output of the decoder. In particular, we should perform a write to register r this cycle providing

- Recall that the inputs to a register are W, the write line, D the data to write (if the write line is asserted) and the clock.

- The clock to each register is simply the clock input to the register file.

- The data to each register is simply the write data to the register file.

- The write line to each register is unique

  - The register number is fed to a decoder.

  - The rth output of the decoder is asserted if r is the specified register.

  - Hence we wish to write register r if

    - The write line to the register file is asserted

    - The rth output of the decoder is asserted

    - Bingo! We just need an and gate.

Homework: 20

#### SRAMS and DRAMS

- External interface is below

- (Sadly) we will not look inside. Following is unofficial

  - Conceptually like a register file but can't use the implementation above for a large memory because there would be too many wires and the muxes would be too big.

  - Two stage decode

  - Tri-state buffers instead of mux for SRAM. Hence I was fibbing when I said that signals always have a 1 or 0. However, we will not use tristate logic (will use muxes the way we build them

  - DRAM latches whole row but outputs only one (or a few) column(s)

    - So can speed up access to elts in same Row

    - Merged DRAM + CPU a modern hot topic

- Error Correction (Omitted)

Note: There are other kinds of flip-flops T, J-K. Also one could learn about excitation tables for each. We will not cover this material (H&P doesn't either). If interested, see Mano

#### Counters

A counter counts (naturally). The counting is done in binary.

Let's look at the state transition diagram for A, the output of a 1-bit counter.

We need one flop and a combinatorial circuit.

The flop producing A is often itself called A and the D input to this flop is called DA (really D sub A).

Current      Next ||

A    I R   A   || DA <-- i.e. to what must I set DA

------------------++--      in order to

0    0 0   0   || 0      get the desired Next A for

1    0 0   1   || 1      the next cycle

0    1 0   1   || 1

1    1 0   0   || 0

x    x 1   0   || 0

But this table (without Next A) is the truth table for the combinatorial circuit.

A I R  || DA

-------++--

0 0 0  || 0

1 0 0  || 1

0 1 0  || 1

1 1 0  || 0

x x 1  || 0

DA = R' (A XOR I)

How about a two bit counter.

- State diagram has 4 states 00, 01, 10, 11 and transitions from one to another

- The circuit diagram has 2 D flops

To determine the combinationatorial circuit we could preceed as before

Current      Next ||

A B   I R  A B  || DA DB

------------------++------

This would work but we can instead think about how a counter works and see that.

DA = R'(A XOR I)

DB = R'(B XOR AI)

Homework: READ chapter 1. Do 1.1 -- 1.26 (really one matching question)
Do 1.27 to 1.44 (another matching question),
1.45 (and do 7200 RPM and 10,000 RPM), 1.46, 1.50

Read sections 3.1 3.2 3.3

SIW 4. Representing instructions in the Computer (MIPS)

Register file

- We just learned how to build this

- 32 Registers each 32 bits

- Register 0 is always 0 when read and stores to register 0 are ignored

Homework: 3.2

### R-type instruction (R for register)

op    rs    rt    rd    shamt  funct

6     5     5     5      5      6

The fields are quite consistent

- op is the opcode

- rs,rt are source operands

- rd is destination

- shamt is the shift amount

- funct is used for op=0 to distinguish alu ops

  - alu is arithmetic and logic unit

  - add/sub/and/or/not etc.

Example: add $1,$2,$3

- Machine format: 0--2--3--1--0--21

- op=0, alu op

- funct=21 specifies add

- reg1 <-- reg2 + reg3

- the regs can be the same (doubles the value in the reg)

- Do sub by just changing the funct

- If regs are the same, clears the register

### I-type (why I?)

op    rs    rt   address

6     5     5     16

- rs is a source reg

- rt is the destination reg

- Transfers to/from memory, normally in words (32-bits)

  - But the machine is byte addressible!

  - Then how come have load/store word instead of byte?

  - Ans: It has load/store byte as well, but we don't cover it.

  - What if the address is not a multiple of 4:

  - Ans: An error (MIPS requires aligned accesses).

lw/sw $1,addr($2)

- machine format is: op 2 1 addr

- $1 <-- Mem[$2+addr]
$1 --> Mem[$2+addr]

RISC-like properties of the MIPS architecture

- Instructions of the same length

- Field sizes of R-type and I-type correspond.

- The type (R-type, I-type, etc.) is determined by the opcode

- Note that rs is the ref to memory for both load and store

- These properties will prove helpful when we construct a MIPS processor.

### Branching instruction

#### slt (set less-then)

- R-type

- Slt $3,$8,$2

- reg3 <-- (if reg8 < reg2 then 1 else 0)

- Like other R-types: read 2nd and 3rd reg, write 1st

#### beq and bne (branch (not) equal)

- I-type

- beq $1,$2,L

- if reg1=reg2 then goto L

- bne $1,$2,L

- if reg1!=reg2 then goto L

#### blt (branch if less than)

- I-type

- blt $5,$8,L

- if reg5 < reg8 then goto L

- *** WRONG ***

- There is no blt instruction.

- Instead use

- stl $1,$5,$8

- bne $1,$0,L

#### ble (branch if less than or equal)

- There is no «ble $5,$8,L» instruction.

- Note that $5<=$8 <==> NOT ($8<$5)

- Hence we test for $8<$5 and branch if false

- stl $1,$8,$5

- beq $1,$0,L

#### bgt (branch if greater than>

- There is no «bgt $5,$8,L» instruction.

- Note that $5>$8 <==> $8<$5

- Hence we test for $8<$5 and branch if true

- stl $1,$8,$5

- bne $1,$0,L

#### bge (branch if greater than or equal>

- There is no «bge $5,$8,L» instruction.

- Note that $5>=$8 <==> NOT ($5<$8)

- Hence we test for $5<$8 and branch if

- stl $1,$5,$8

- beq $1,$0,L

Note: Please do not make the mistake of thinking that

stl $1,$5,$8

beq $1,$0,L

is the same as

stl $1,$8,$5

bne $1,$0,L

The negation of X < Y is not Y < X

Homework: 3.12-3.17

### J-type instructions (J for jump)

op   address

6     26

#### j (jump)

- J type

- Range is 2^26 words = 1/4 GB

#### jr (jump register)

- R type, but only one register

- Will it be one of the source registers or the destination register?

- Ans: This will be obvious when we construct the processor

#### jal (jump and link)

- Used for subroutine calls

- J type

- return address is stored in register 31

### I type instructions (revisited)

- The I is for immediate

- These instructions have an immediate third operand, i.e., the third operand is contained in the instruction itself.

- This means the operand itself and not just its address or register number are contained in the instruction

#### addi (add immediate)

addi $1,$2,100

Why is there no subi?
Ans: Make the immediate operand negative.

#### slti (set less-than immediate)

slti $1,$2,50

Handout Lab #1 and supporting sheets

- Due in two weeks

#### lui (load upper immediate)

- How can we get a 32-bit constant into reg since we can't have a 32 bit immediate?

  - Load the word

    - Have the constant placed in the program text (via some assembler directive).

    - Issue lw to load the register

    - But memory accesses are slow and this uses a cache entry

  - Load shift add

    - Load immediate the high order 16 bits (into the low order of the register).

    - Shift the register left 16 bits (filling low order with zero)

    - Add immediate the low order 16 bits

    - Three instructions, three words of memory

  - load-upper add

    - Use lui to load immediate the high order 16 bits into the high order bits and clear the low order

    - Add immediate the low order 16 bits.

    - lui $4,123 -- puts 123 into top half of reg4
addi $4,$4,456 -- puts 456 into bottom half of reg4

Homework: 3.1, 3.3-3.7, 3.9, 3.18, 3.37 (for fun)

Homework: Read 4.1-4.4

Homework: 4.1-4.9

Signed and Unsigned Numbers

MIPS uses 2s complement (just like 8086)

To form the 2s complement (of 0000 1111 0000 1010 0000 0000 1111 1100)

- take the 1s complement

- That is complement each bit (1111 0000 1111 0101 1111 1111 0000 0011)

- Then add 1 (1111 0000 1111 0101 1111 1111 0000 0100)

Need comparisons for signed and unsigned.

- For signed a leading 1 is smaller (negative) than a leading 0

- For unsigned a leading 1 is larger than a leading 0

#### sltu and sltiu

Just like slt and slti but the comparison is unsigned.

## Addition and subtraction

To add two (signed) numbers just add them. That is don't treat the sign bit special.

To subtract A-B, just take the 2s complement of B and add.

#### Overflows

An overflow occurs when the result of an operatoin cannot be represented with the available hardware. For MIPS this means when the result does not fit in a 32-bit word.

- We have 31 bits plus a sign bit.

- The result would definitely fit in 33 bits (32 plus sign)

- The hardware simply discards the carry out of the top (sign) bit

- This is not wrong--consider -1 + -1

- 11111111111111111111111111111111   (32 ones is -1)

- + 11111111111111111111111111111111

- ----------------------------------

- 111111111111111111111111111111110   Now discard the carry out

- 11111111111111111111111111111110   this is -2

- The bottom 31 bits are always correct.
Overflow occurs when the 32 (sign) bit is set to a value and not the sign.

- Here are the conditions for overflow

- Operation  Operand A  Operand B  Result

- A+B       >= 0        >= 0      < 0

- A+B        < 0         < 0     >= 0

- A-B       >= 0         < 0      < 0

- A-B        < 0        >= 0     >= 0

- These conditions are the same as
CarryIn to sign position != CarryOut

> I have a question about the first lab; I'm not sure how we

> would implement a mux, would a series of if-else

> statements be an acceptable option?

No.  But that is a good question.  if-then-elif...-else

would be a FUNCTIONAL simulation.  That is you are

simulating what the mux does but not HOW it does it.  For a

gate level simulation, you need to implement the mux in

terms of AND, NOT, OR, XOR and then write code link

Fulladder.c

The implementation of a two way mux in terms of AND OR NOT

is figure B.4 on page B-9 of the text.  You need to do a 3

way mux.

Homework: (for fun) prove this last statement (4.29)

#### addu, subu, addiu

These add and subtract the same the same was as add and sub, but do not signal overflow

## Logical Operations

#### Shifts: sll, srl

- R type, with shamt used and rs not used

- sll $1,$2,5
reg2 gets reg1 shifted left 5 bits

- Why do we need both sll and srl, i.e, why not just have one of them and use a negative shift amt for the other?
Ans: The shift amt is only 5 bits and need shifts from 0 to 31 bits. Hence not enough bits for negative shifts.

- Op is 0 (these are ALU ops, will understand why in a few weeks).

#### Bitwise AND and OR: and, or, andi, ori

No surprises.

- and $r1,$r2,$r3
or $r1,$r2,$r3

- standard R-type instruction

- andi $r1,$r2,100
ori $r1,$r2,100

- standard I-type

SIW 5. Constructing an ALU--the fun begins

First goal is 32-bit AND, OR, and addition

Recall we know how to build a full adder. Will draw it as

With this adder, the ALU is easy.

- Just choose the correct operation (ADD, AND, OR)

- Note the principle that if you want a logic box that sometimes computes X and sometimes computes Y, what you do is

  - Compute X and also compute Y

  - Put both X and Y into a mux

  - Use the «sometimes» condition as the select line to the mux

32-bit version is simple.

- Use an array of logic elements for the logic. The logic element is the 1-bit ALU

- Use buses for A, B, and Result.

- «Broadcast» Opcode to all of the internal 1-bit ALUs. This means wire the external Opcode to the Opcode input of each of the internal 1-bit ALUs

First goal accomplished.

How about subtraction?

- Big deal about 2's compliment is that
A - B = A + (2's comp B) = A + (B' + 1)

- Get B' from an inverter (naturally)

- Get +1 from the CarryIn

1-bit ALU with ADD, SUB, AND, OR is

For subtraction set Binvert and Cin.

32-bit version is simply a bunch of these.

- For subtraction assert both B-invert and Cin.

- For addition de-assert both B-invert and Cin.

- For AND and OR d-assert B-invert. Cin is a don't care

SIW 6. Simulating Combinatorial Circuits at the Gate Level

Write a procedure for each logic box with the following properties.

- Parameters for each input and output

- (Local) variable for each (internal) wire

- Can only do AND OR XOR NOT

  - In the C language & | ^ ~

  - Other languages similar

- No conditional assignment; the output is a FUNCTION of the input

- Single assignment to each variable.
Multiple assignments would correspond to a cycle

- Bus (set of signals) represented by array

- Testing

  - Exhaustive possible for 1-bit cases

  - Cleverness for n-bit cases (n=32, say)

Handout: FullAdder.c and FourBitAdder.c.

Lab 1: Do the equivalent for 1-bit-alu (without subtraction). This is easy. Lab 2 will be similar but for a more sophisticated ALU.

Extra requirements for MIPS alu:

- slt set-less-than

  - Result reg is 1 if a < b
Result reg is 0 if a >= b

  - So need to set the LOB (low order bit, aka least significant bit) of the result equal to the sign bit of a subtraction, and set the rest of the result bits to zero.

  - Idea #1. Give the mux another input, called LESS. This input is brought in from outside the bit cell. That is, if the opcode is slt we make the select line to the mux equal to 11 (three) so that the the output is the this new input. For all the bits except the LOB, the LESS input is zero. For the LOB we must figure out how to set LESS.

  - Idea #2. Bring out the result of the adder (BEFORE the mux)

Only needed for the HOB (high order bit, i.e. sign) Take this new output from the HOB, call it SET and connect it to the LESS input in idea #1 for the LOB. The LESS input for other bits are set to zero.

- Why isn't this method used?

- Ans: It is wrong!

- Example using 3 bit numbers (i.e. -4 .. 3). Try slt on -3 and +2. True subtraction (-3 - +2) give -5. The negative sign in -5 indicates (correctly) that -3 < +2. But three bit subtraction -3 - +2 gives +3 ! Hence we will incorrectly conclude that -3 is NOT less than +2. (Really, it signals an overflow. unless doing unsigned)

- Solution: Need the correct rule for less than (not just sign of subtraction)

Homework: figure out correct rule, i.e. prob 4.23. Hint: when an overflow occurs the sign bit is definitely wrong (so the complement of the sign bit is right).

  - Overflows

    - The HOB is already unique (outputs SET)

    - Need to enhance it some more to produce the overflow output

    - Recall that we gave the rule for overflow: you need to examine

      - Whether add or sub (binvert)

      - The sign of A

      - The sign of B

      - The sign of the result

      - Since this is the HOB we have all the sign bits.

      - The book also uses Cout, but this appears to be an error

  - Simpler overflow detection

    - An overflow occurs if and only if the carry in to the HOB differs from the carry of of the HOB

  - Zero Detect

    - To see if all bits are zero just need NOR of all the bits

    - Conceptually trivially but does require some wiring

  - Observation: The initial Cin and Binvert are always the same. So just use one input called Bnegate.

The Final Result is

Symbol for the alu is

What are the control lines?

  - Bnegate (1 bit)

  - OP (2 bits)

What functions can we perform?

  - and

  - or

  - add

  - sub

  - set on less than

What (3-bit) values for the control lines do we need for each function?

| and | 0 | 00 |
| --- | --- | --- |
| or | 0 | 01 |
| add | 0 | 10 |
| sub | 1 | 10 |
| slt | 1 | 11 |

Adders

  - We have done what is called a ripple carry adder.
The carry "ripples" from one bit to the next (LOB to HOB).
So the time required is proportional to the wordlength.
Each carry can be computed with two levels of logic (any function can be so computed) hence the number of gate delays is 2*wordsize.

  - What about doing the entire 32 (or 64) bit adder with 2 levels of logic?

    - Such a circuit clearly exists. Why?

    - Ans: A two levels of logic circuit exists for any function.

    - But it would be very expensive: many gates and wires.

    - The big problem: The AND and OR gates have high fan-in, i.e., they have a large number of inputs. It is not true that a 64-input AND takes the same time as a 2-input AND.

    - Unless you are doing full custom VLSI, you get a toolbox of primative functions (say 4 input NAND) and must build from that

  - There are faster adders, e.g. carry lookahead and carry save. We will study the carry lookahead.

#### Carry Lookahead adders

We did a ripple adder

  - The delay proportional to # bits.

  - In detail, each bit the CarryOut uses two levels of logic after the CarryIn is stable.

  - So the delay for calculating an n-bit sum is 2n gate delays.

  - For 4 bits the delay is 8

  - For 16, delay is 32

  - For 32, delay is 64

  - For 64, delay is 128

We will now do the carry lookahead adder, which is much faster, especially for many bit (e.g. 64 bit) addition.

For each bit we can in one gate delay calculate

generate a carry    gi = ai bi

propogate a carry   pi = ai+bi

H&P give a plumbing analogue for generate and propogate.

Given the generates and propogates, we can calculate all the carries for a 4-bit addition (recall that c0=Cin is an input) as follows

c1 = g0 + p0 c0

c2 = g1 + p1 c1 = g1 + p1 g0 + p1 p0 c0

c3 = g2 + p2 c2 = g2 + p2 g1 + p2 p1 g0 + p2 p1 p0 c0

c4 = g3 + p3 c3 = g3 + p3 g2 + p3 p2 g1 + p3 p2 p1 g0 + p3 p2 p1 p0 c0

Thus we can calculate c1 ... c4 in just two additional gate delays (where we assume one gate can accept upto 5 inputs). Since we get gi and pi after one gate delay, the total delay for calculating all the carries is 3 (this includes c4=CarryOut)

Each bit of the sum si can be calculated in 2 gate delays given ai, bi, and ci. Thus 5 gate delays after we are given a, b and CarryIn, we have calculated s and CarryOut

So for 4-bit addition the faster adder takes time 5 and the slower adder time 8.

Now we want to put four of these together to get a fast 16-bit adder. Again we are assuming a gate can accept upto 5 inputs. It is important that the number of inputs per gate does not grow with the size of the numbers to add. If the technology available supplies only 4-input gates, we would use groups of 3 bits rather than four.

We start by determining «supergenerate» and «superpropogate» bits. The super propogate indicates whether the 4-bit adder constructed above generates a CarryOut or propogates a CarryIn to a CarryOut

P0 = p3 p2 p1 p0          Does the low order 4-bit adder

propogate a carry?

P1 = p7 p6 p5 p4

P2 = p11 p10 p9 p8

P3 = p15 p14 p13 p12      Does the high order 4-bit adder

propogate a carry?

G0 = g3 + p3 g2 + p3 p2 g1 + p3 p2 p1 g0        Does low order 4-bit

adder generate a carry

G1 = g7 + p7 g6 + p7 p6 g5 + p7 p6 p5 g4

G2 = g11 + p11 g10 + p11 p10 g9 + p11 p10 p9 g8

G3 = g15 + p15 g14 + p15 p14 g13 + p15 p14 p13 g12

C1 = G0 + P0 c0

C2 = G1 + P1 C1 = G1 + P1 G0 + P1 P0 c0

C3 = G2 + P2 C2 = G2 + P2 G1 + P2 P1 G0 + P2 P1 P0 c0

C4 = G3 + P3 C3 = G3 + P3 G2 + P3 P2 G1 + P3 P2 P1 G0 + P3 P2 P1 P0 c0

From these C's you just need to do a 4-bit CLA since the C's are the CarryIns for each group of 4-bits out of the 16-bits.

How long does this take, again assuming 5 input gates?

  - We calculate the P's one gate delay after we have the p's. Since the pj's are determined 1 gate delay after we are given the a's and b's. So we have all Pi 2 gate delays after we start.

  - The G's are determined 2 gate delays after we have the g's and p's. So the G's are done 3 gate delays after we start.

  - The C's are determined 2 gate delays after the P's and G's. So the C's are done 5 gate delays after we start.

  - Now the C's are sent back to the 4-bit adders, which have already calculated the p's and g's. Hence the c's are calculated in 2 more gate delays (7 total) and the s's 2 more after that (9 total).

  - So a 16-bit CLA takes 9 cycles instead of 32 for a ripple carry adder.

Some pictures follow.

Take our original picture of the 4-bit CLA and collapse the details so it looks like.

Next include the logic to calculate P and G

Now put four of these with a CLA block (to calculate C's from P's, G's and Cin) and we get a 16-bit CLA. Note that we do not use the Cout from the 4-bit CLAs.

Note that the tall skinny box is general. It takes 4 Ps 4Gs and Cin and calculates 4Cs. The Ps can be propogates, superpropogates, superduperpropogates, etc. That is, you take 4 of these 16-bit CLAs and the same tall skinny box and you get a 64-bit CLA.

Homework: 4.44, 4.45

As noted just above the tall skinny box is useful for all size CLAs. To expand on that point and to review CLAs, let's redo CLAs with the general box.

Since we are doing 4-bits at a time, the box takes 9=2*4+1 input bits and produces 6=4+2 outputs

  - Inputs

    - 4 generate bits from the previous size (i.e. if now doing a 64-bit CLA, these are the generate bits from the four 16-bit CLAs). Let's call these bits Gin0, Gin1, Gin2, Gin3.

    - 4 propogate bits from the previous size Pin0, Pin1, Pin2, Pin3.

    - The Carry in Cin

  - Outputs

    - Four carries C1, C2, C3, and C4 to be used in the previous size

      - Cin is also called C0 and is used in the previous size as well as in this box.

      - C4 is also called Cout. It is the carry out from this size, but is not used in the next size

    - Gout and Pout, the generate and propogate to be used in the next size

  - Formulas

  - C1 = G0 + PO Cin

  - C2 = G1 + P1 G0 + P1 P0 Cin

  - C3 = G2 + P2 G1 + P2 P1 G0 + P2 P1 P0 Cin

  - C4 = G3 + P3 G2 + P3 P2 G1 + P3 P2 P1 G0 + P3 P2 P1 P0 Cin

  - Gout = G3  +  P3 G2  +  P3 P2 G1  +  P3 P2 P1 Go

  - Pout = P3 P2 P1 P0

  - Picture

A 4-bit adder is now

What does the «?» box do?

  - Calculates Gi and Pi based on ai and bi

    - Gi = ai bi

    - Pi = ai + bi

  - Calculate s1 based on ai, bi, and Ci=Cin (normal full adder)

  - Do not bother calculating Cout

Now take four of these 4-bit adders and use the identical CLA box to get a 16-bit adder

Four of these 16-bit adders with the identical CLA box to gives a 64-bit adder.

#### Shifter

  - Just a string of D-flops; output of one is input of next

    - Input to first is the serial input.

    - Output of last is the serial output.

  - We want more.

    - Left and right shifting (with serial input/output)

    - Parallel load

    - Parallel Output

    - Don't shift every cycle

  - Parallel output is just wires.

  - Shifter has 4 modes (left-shift, right-shift, nop, load) so

    - 4-1 mux inside

    - 2 control lines must come in

  - We could modify our registers to be shifters (bigger mux), but ...

  - Our shifters are slow for big shifts; «barrel shifters» are better

Homework: A 4-bit shift register initially contains 1101. It is shifted six times to the right with the serial input being 101101. What is the contents of the register after each shift.

Homework: Same register, same init condition. For the first 6 cycles the opcodes are left, left, right, nop, left, right and the serial input is 101101. The next cycle the register is loaded (in parallel) with 1011. The final 6 cycles are the same as the first 6. What is the contents of the register after each cycle?

#### Multipliers

  - Recall how to do multiplication.

    - Multiplicand times multiplier gives product

    - Multiply multiplicand by each digit of multiplier

    - Put the result in the correct column

    - Then add the partial products just produced

  - We will do it the same way ...
... but differently

    - We are doing binary arithmetic so each «digit» of the multiplier is 1 or zero.

    - Hence «multiplying» the mulitplicand by a digit of the multiplier means either

      - Getting the multiplicand

      - Getting zero

    - Use an «if appropriate bit of multiplier is 1» stmt

    - To get the «appropriate bit»

      - Start with the LOB of the multiplier

      - Shift the multiplier right (so the next bit is the LOB)

    - Putting in the correct column means putting it one column further left that the last time.

    - This is done by shifting the multiplicand left one bit each time (even if the multiplier bit is zero)

    - Instead of adding partial products at end, keep a running sum

      - If the multiplier bit is zero add the (shifted) multiplicand to the running sum

      - If the bit is zero, simply skip the addition.

  - This results in the following algorithm

product <- 0

for i = 0 to 31

if LOB of multiplier = 1

product = product + multiplicand

shift multiplicand left 1 bit

shift multiplier right 1 bit

Do on board 4-bit addition (8-bit registers) 1100 x 1101

What about the control?

  - Always give the ALU the ADD operation

  - Always send a 1 to the multiplicand to shift left

  - Always send a 1 to the multiplier to shift right

  - Pretty boring so far but

    - Send a 1 to write line in product if and only if LOB multiplier is a 1

    - I.e. send LOB to write line

    - I.e. it really is pretty boring

This works!
but, when compared to the better solutions to come, is wasteful of resourses and hence is

  - slower

  - hotter

  - bigger

  - all these are bad

The product register must be 64 bits since the product is 64 bits

Why is multiplicand register 64 bits?

  - So that we can shift it left

  - I.e., for our convenience. 
By this I mean it is not required by the problem specification,
but only by the solution method chosen.

Why is ALU 64-bits?

  - Because the product is 64 bits

  - But we are only adding a 32-bit quantity to the product at any one step.

  - Hmmm.

  - Maybe we can just pull out the correct bits from the product.

  - Would be tricky to pull out bits in the middle because which bits to pull changes each step

Recommended references

- Hennessy, J. L., and D. A. Patterson. Computer Architecture: A Quantitative Approach, 3rd ed. San Mateo, CA: Morgan Kaufman, 2012. ISBN: 1558605967.

- Patterson, D. A., and J. L. Hennessy. Computer Organization and Design: The Hardware/Software Interface, 3rd ed. San Mateo, CA: Morgan Kaufman, 2014. ISBN: 1558606041

- Вычислительные машины, системы и сети. Под редакцией П.А. Пятибратова. – М: «Финансы и статистика», 2010г.

- Мансурова М.Е. Компьютерное окружение: Учебное пособие. – Алматы: Издательство ТОO “Print S”, 2014. 146с.

- Сергеев Н.П., Вашкевич Н.П. Основы вычислительной техники. – М: Высшая школа, 2014.

- А. Гук. Процессоры фирмы Intel  от 8086 до Pentium II. – СПб: Питер-Пресс, 2013.

- М.Гук. Аппаратные средства IBM PC. – СПб: Питер, 2010.

- Пухальский Г.И., Новосельцева Т.Я. Цифровые устройства: Учебное пособие для втузов. – СПб.: Политехника, 2013. -885с.

- К.Г. Самофалов, И.В. Корнейчук, В.П. Тарасенко. Цифровые электронные вычислительные машины. – Киев: ВИЩА ШКОЛА, 2013.

- М. Гук, В. Юров. PENTIUM 4 Athlon и Duron. – СПб.: Питер, 2011г.

- Томпсон Р.Б., Томпсон Б.Ф. Железо ПК: Энциклопедия. – СПб.: Питер, 2013г.

Botayeva S.B., Botayev B.B., Umarova Zh.R.

Methodical guidelines to SIW on discipline «Computer System Architecture»

for students of specialty 5В070300 – Information systems

Form of training – daily

Подписано в печать «            »                   2019

Формат бумаги

Бумага типографская. Печать офсетная. Объем  4 п.л.

Тираж 50 экз. Заказ  №                 .

© M.Auezov South Kazakhstan state university, 2019.

Printing centre of M.Auezov SKSU, Shymkent, Tauke-khan  av., 5