Ф.7.03-03

Zhilkibayeva S.K., Abzalova D.A., Abshenov Kh.A.

LECTURE NOTES

Course title «Mathematical Methods of Experiment Planning»

for students of the specialty «5В071200 – Machine Engineering» and of the educational program «6B07120 – Machine Engineering»

Shymkent, 2021

MINISTRY OF EDUCATION AND SCIENCE OF REPUBLIC OF KAZAKHSTAN

M. AUEZOV SOUTH KAZAKHSTAN UNIVERSITY

Department «Mechanics and Engineering»

Zhilkibayeva S.K., Abzalova D.A., Abshenov Kh.A.

LECTURE NOTES

Course title «Mathematical Methods of Experiment Planning»

for students of the specialty «5В071200 – Machine Engineering» and of the educational program «6B07120 – Machine Engineering»

Shymkent, 2021

UDC 621.81

Compilers: Zhilkibayeva S.K., Abzalova D.A., Abshenov Kh.A. – Lecture notes on the course «Mathematical Methods of Experiment Planning» - Shymkent:

M. Auezov SKU,2021.-P. 92.

Lecture notes for the course «Mathematical Methods of Experiment Planning» are compiled in accordance with the requirements of the working curriculum and the curriculum of the elective component course, the working curriculum (syllabus) of the course «Mathematical Methods of Experiment Planning» and includes all the necessary information on the implementation of lecture topics.

Lecture notes for the course «Mathematical Methods of Experiment Planning» for full-time and distance learning students of the specialty «5В071200–Machine engineering» and of the educational program «6B07120–Machine Engineering» should be considered as a summary of the programmatic issues of the course, facilitating the assimilation of educational material and preparation for exams.

Reviewer:

Moldagaliyev A.B. – candidate of technical sciences, docent, M. Auezov SKU

Ibragimova Z.A. – PhD, docent, M. Auezov SKU

Considered at the meeting of the Department "Mechanics and Engineering" (Protocol no. ___ from "________" 202_) and

Methodological commission of the Faculty «Mechanical and Petroleum Engineering»

(Protocol no. ___ from "________" 202_)

Recommended for publication by the educational and methodological Council of M. Auezov SKU, Protocol no.____ from "________" 202_)

©М.Auezov South Kazakhstan University, 2021

Responsible for the release: Zhilkibayeva S.K.

Introduction

The course «Mathematical Methods of Experiment Planning» is a general professional, providing basic knowledge in the assimilation of special disciplines studied in the future.

This lecture notes cover the basic topics in experimental design and is intended for undergraduates. The lecture notes outline all the statistical methods used in the selection of factors, the construction of mathematical models of various types, experimental optimization.

The course objective is to learn how to plan, design and conduct experiments efficiently and effectively, and analyze the resulting data to obtain objective conclusions. Both design and statistical analysis issues are discussed. Opportunities to use the principles taught in the course arise in all aspects of today’s industrial and business environment. Applications from various fields will be illustrated throughout the course. Computer software packages (JMP, Design-Expert, Minitab) will be used to implement the methods presented and will be illustrated extensively.

All experiments are designed experiments; some of them are poorly designed, and others are well-designed. Well-designed experiments allow you to obtain reliable, valid results faster, easier, and with fewer resources than with poorly-designed experiments. You will learn how to plan, conduct and analyze experiments efficiently in this course.

Upon successful study of the lecture notes, students should be able to: 1. Apply the tools and techniques of engineering experimentation in real life industrial environment. 2. Think logically to design the new experiments, philosophy, principles, theories which will enhance the efficiency and effectiveness of the industries. 3. Face the challenges of the industrial environment by using engineering experimentation.

This lecture summary is compiled in full accordance with the programs of the course «Mathematical Methods of Experiment Planning» and is intended for students of all specialties of full-time and distance learning, for which such courses are provided according to the curriculum.

  - Strategy of experimentation.

  - Some typical applications of experimental design.

  - Basic principles.

  - Guidelines for designing experiments.

  - A brief history of statistical design.

  - STRATEGY OF EXPERIMENTATION

Experiments are performed by investigators in virtually all fields of inquiry, usually to discover something about a particular process or system. Literally, anexperimentisatest. More formally, we can define an experiment as a test or series of tests in which purposeful changes are made to the input variables of a process or system so that we may observe and identify the reasons for changes that may be observed in the output response (Then an example of an experiment with metallurgical engineer must be discussed). Inanyexperiment, the results and conclusions that can be drawn depend to a large extent on the manner in which the data were collected. In general, experiments are used to study the performance of processes and systems. The process or system can be represented by the model shown in Fig. 1.1.

Fig. 1.1 – General model of a process or system

We can usually visualize the process as a combination of machines, methods, people, and other resources that transforms some input (often a material) into an output that has one or more observable responces.  Some of the process variables  are controllable, whereas other variables  are uncontrollable (although they may be controllable for purposes of a test).

The general approach to planning and conducting the experiment is called the strategy of experimentation. The strategy of experimentation, which is called the best-guess approach, is used frequently in practice by engineers and scientists. Another strategy of experimentation that is used extensively in practice is the one-factor-at-a-time approach. The major disadvantage of the one-factor-at-a-time strategy is that it fails to consider any possible interaction between the factors.

The correct approach to dealing with several factors is to conduct a factorial experiment.

  - BASIC PRINCIPLES

References:

- D.C. Montgomery. Design and analysis of experiments. Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

- Спиридонов А.А. Планирование эксперимента при исследовании технологических процессов. –М.: Машиностроение, 1981. -184с., ил.

2-1.	Simple comparative experiments.

2-2.	Basic statistical concepts.

2-3.	Sampling and sampling distributions.

2-1	SIMPLE COMPARATIVE EXPERIMENTS

An engineer is studying the formulation of a Portland cement mortar. He has added a polymer latex emulsion during mixing to determine if this impacts the curing time and tension bond strength of the mortar. The experimenter prepared 10 samples of the original formulation and 10 samples of the modified formulation. We will refer to the two different formulations as two treatments or as two levels of the factor formulations. When the cure process was completed, the experimenter did find a very large reduction in the cure time for the modified mortar formulation. Then he began to address the tension bond strength of the mortar. If the new mortar formulation has an adverse effect on bond strength, this couldimpact its usefulness. The tension bond strength data from this experiment are shown in Table 2.1 and plotted in Figure 2.1. The graph is called a dot diagram. Visual examination of these data gives the impression that the strength of the unmodified mortar may be greater than the strength of the modified mortar. This impression is supported by comparing the average tension bond strengths, for the modified mortar and for the unmodified mortar. The average tension bond strengths in these two samples differ by what seems to be a modest amount. However, it is not obvious that this difference is large enough to imply that the two formulations really are different. Perhaps this observed difference in average strengths is the result of sampling fluctuation and the two formulations are really identical. Possibly another two samples would give opposite results, with the strength of the modified mortar exceeding that of the unmodified formulation. A technique of statistical inference called hypothesis testing can be used to assist the experimenter in comparing these two formulations. Hypothesis testing allows the comparison of the two formulations to be made on objective terms, with knowledge of the risks associated with reaching the wrong conclusion. Before presenting procedures for hypothesis testing in simple comparative experiments, we will briefly summarize some elementary statistical concepts. Each of the observations in the Portland cement experiment described above would be calleda run. Notice that the individual runs differ, so there is fluctuation, or noise, in the observed bond strengths. This noise is usually called experimental error or simply error. It is a statisticalerror, meaning that it arises from variation that is uncontrolled and generally unavoidable. The presence of error or noise implies that the response variable, tension bond strength, is a random variable. A random variable may be either discrete or continuous. Ifthe set of all possible values of the random variable is either finite or countably infinite, thenthe random variable is discrete, whereas if the set of all possible values of the random variable is an interval, then the random variable is continuous.

2-2	BASIC STATISTICAL CONCEPTS

If the data are fairly numerous, the dots in a dot diagram become difficult to distinguishand a histogram may be preferable. Figure 2.2 presents a histogram for 200 observations on themetal recovery, or yield, from a smelting process. The histogram shows the central tendency,spread, and general shape of the distribution of the data. Recall that a histogram is constructedby dividing the horizontal axis into bins (usually of equal length) and drawing a rectangle overthe jth bin with the area of the rectangle proportional to nj, the number of observations that fallin that bin. The histogram is a large-sample tool. When the sample size is small the shape of the histogram can be very sensitive to the number of bins, the width of the bins, and the starting value for the first bin. Histograms should not be used with fewer than 75–100 observations. The box plot (or box-and-whisker plot) is a very useful way to display data. A boxplot displays the minimum, the maximum, the lower and upper quartiles (the 25th percentile and the 75th percentile, respectively), and the median (the 50th percentile) on a rectangular box aligned either horizontally or vertically. The box extends from the lower quartile to the upper quartile, and a line is drawn through the box at the median. Lines (or whiskers) extend from the ends of the box to (typically) the minimum and maximum values. Figure 2.3 presents the box plots for the two samples of tension bond strength in the Portland cement mortar experiment. This display indicates some difference in mean strength between the two formulations. It also indicates that both formulations produce reasonably symmetric distributions of strength with similar variability or spread. Dot diagrams, histograms, and box plots are useful for summarizing the information in a sample of data. To describe the observations that might occur in a sample more completely, we use the concept of the probability distribution.

2-3	SAMPLING AND SAMPLING DISTRIBUTIONS

Random Samples, Sample Mean, and Sample Variance.The objective of statistical

inference is to draw conclusions about a population using a sample from that population. Most of the methods that we will study assume that random samples are used. A randomsample is a sample that has been selected from the population in such a way that every possible sample has an equal probability of being selected. In practice, it is sometimes difficult to obtain random samples, and random numbers generated by a computer program may be helpful.

Statistical inference makes considerable use of quantities computed from the observations in the sample. We define a statistic as any function of the observations in a sample that does not contain unknown parameters. For example, suppose thatrepresents a sample. Then the sample mean

and the sample variance

are both statistics. These quantities are measures of the central tendency and dispersion of thesample, respectively. Sometimes , called the sample standard deviation, is used as a measure of dispersion. Experimenters often prefer to use the standard deviation to measure dispersion because its units are the same as those for the variable of interest y.

This result states essentially that the sum of nindependent and identically distributed

random variables is approximately normally distributed. In many cases, this approximation is good for very small n, say n< 10, whereas in other cases large n is required, say n >100. Frequently, we think of the error in an experiment as arising in an additive manner from several independent sources; consequently, the normal distribution becomes a plausible model for the combined experimental error. An important sampling distribution that can be defined in terms of normal random variables is the chi-square or distribution. If are normally and independently distributed random variables with mean 0 and variance 1, abbreviated NID(0, 1), then the random variable

.

References:

1.	D.C. Montgomery. Design and analysis of experiments.Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

2.	Грачев Ю.П. Математические методы планирования экспериментов: Учеб. пособие для вузов. – М.: ДеЛи принт, 2005. – 294 с.

Experiments with a single factor: the analysis of variance. Analysis of the fixed effects model.

An Example

References:

1.	D.C. Montgomery. Design and analysis of experiments. Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

2.	Сидняев Н. И. Введение в теорию планирования эксперимента: учеб. пособие . М. : Изд-во МГТУ им. Н. Э. Баумана, 2011. – 463 с.

The randomized complete block design. Statistical analysis of the RCBD.

There is a potentially serious problem with a completely randomized experiment in this design situation. If the metal coupons differ slightly in their hardness, as might happen if they are taken from ingots that are produced in different heats, the experimental units (the coupons) will contribute to the variability observed in the hardness data. As a result, the experimental error will reflect both random error and variability between coupons. We would like to make the experimental error as small as possible; that is, we would like to remove the variability between coupons from the experimental error. A design that would accomplish this requires the experimenter to test each tip once on each of four coupons. This design, shown in Table 4.1, is called a randomized complete block design (RCBD). The word “complete” indicates that each block (coupon) contains all the treatments (tips). By using this design, the blocks, or coupons, form a more homogeneous experimental unit on which to compare the tips. Effectively, this design strategy improves the accuracy of the comparisons among tips by eliminating the variability among the coupons. Within a block, the order in which the four tips are tested is randomly determined. Notice the similarity of this design problem to the paired t-test of Section 2.5.1. The randomized complete block design is a generalization of that concept.

The RCBD is one of the most widely used experimental designs. Situations for which the RCBD is appropriate are numerous. Units of test equipment or machinery are often different in their operating characteristics and would be a typical blocking factor. Batches of raw material, people, and time are also common nuisance sources of variability in an experiment that can be systematically controlled through blocking.1 Blocking may also be useful in situations that do not necessarily involve nuisance factors. For example, suppose that a chemical engineer is interested in the effect of catalyst feed rate on the viscosity of a polymer. She knows that there are several factors, such as raw materialsource, temperature, operator, and raw material purity that are very difficult to control in the full-scale process. Therefore she decides to test the catalyst feed rate factor in blocks, where each block consists of some combination of these uncontrollable factors. In effect, she is using the blocks to test the robustness of her process variable (feed rate) to conditions she cannot easily control. For more discussion of this, see Coleman and Montgomery (1993).

References:

1.	D.C. Montgomery. Design and analysis of experiments.Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

2.	Спиридонов А.А. Планирование эксперимента при исследовании технологических процессов. –М.: Машиностроение, 1981. -184с., ил.

Basic definitions and principles of factorial design.

The two-factor factorial design

The 2k factorial design.The 22 Design.The 23 Design.

The General 2k Design.A single replicate of the 2k Design.

Reactantconcentration,A

We define the interaction effect AB as the average difference between the effect of A at the high level of B and the effect of A at the low level of B. Thus,

References:

1.	D.C. Montgomery. Design and analysis of experiments.Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

2.	Спиридонов А.А. Планирование эксперимента при исследовании технологических процессов. –М.: Машиностроение, 1981. -184с., ил.

Blocking a replicated 2k factorial design. Confounding in 2k factorial design.

References:

1.	D.C. Montgomery. Design and analysis of experiments.Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

2.	Спиридонов А.А. Планирование эксперимента при исследовании технологических процессов. –М.: Машиностроение, 1981. -184с., ил.

Factorial treatment structure

We have been working with completely randomized designs, where g treat-ments are assigned at random to N units. Up till now, the treatments have had no structure; they were just g treatments. Factorial treatment structure ex-ists when the g treatments are the combinations of the levels of two or more factors. We call these combination treatments factor-level combinations or factorial combinations to emphasize that each treatment is a combination of one level of each of the factors. We have not changed the randomization; we still have a completely randomized design. It is just that now we are con-sidering treatments that have a factorial structure. We will learn that there are compelling reasons for preferring a factorial experiment to a sequence of experiments investigating the factors separately.

8.1	Factorial Structure

It is best to start with some examples of factorial treatment structure. Lynch and Strain (1990) performed an experiment with six treatments studying how milk-based diets and copper supplements affect trace element levels in rat livers. The six treatments were the combinations of three milk-based diets (skim milk protein, whey, or casein) and two copper supplements (low and high levels). Whey itself was not a treatment, and low copper was not a treatment, but a low copper/whey diet was a treatment. Nelson, Kriby, and Johnson (1990) studied the effects of six dietary supplements on the occur-rence of leg abnormalities in young chickens. The six treatments were the combinations of two levels of phosphorus supplement and three levels of calcium supplement. Finally, Hunt and Larson (1990) studied the effects of sixteen treatments on zinc retention in the bodies of rats. The treatments were the combinations of two levels of zinc in the usual diet, two levels of zinc in the final meal, and four levels of protein in the final meal. Aga in, it is the combination of factor levels that makes a factorial treatment.

We begin our study of factorial treatment structure by looking at two-factor designs. We may present the responses of a two-way factorial as a table with rows corresponding to the levels of one factor (which we call factor A) and columns corresponding to the levels of the second factor (factor B). For example, Table 8.1 shows the results of an experiment on sprouting barley (these data reappear in Problem 8.1). Barley seeds are divided into 30 lots of 100 seeds each. The 30 lots are divided at random into ten groups of three lots each, with each group receiving a different treatment. The ten treatments are the factorial combinations of amount of water used for sprouting (factor with two levels, and age of the seeds (factor B) with five lev els. The response measured is the number of seeds sprouting.

We use the notation yijk to indicate responses in the two-way factorial. In this notation, yijk is the kth response in the treatment formed from the ith level of factor A and the jth level of factor B. Thus in Table 8.1, y2,5,3 = 25. For a four by three factorial design (factor A has four levels, factor B has three levels), we could tabulate the responses as in Table 8.2. This table is just a convenient representation that emphasizes the factorial structure; treatments were still assigned to units at random.

Notice in both Tables 8.1 and 8.2 that we have the same number of re-sponses in every factor-level combination. This is called balance. Balance turns out to be important for the standard analysis of factorial responses. We will assume for now that our data are balanced with n responses in every factor-level combination. Chapter 10 will consider analysis of unbalanced factorials.

Table 8.1: Barley sprouting data.

|  |  | Age of Seeds (weeks) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ml H2O | 1 | 3 | 6 | 9 | 12 |
| 4 | 11 | 7 | 9 | 13 | 20 |
|  | 9 | 16 | 19 | 35 | 37 |
|  | 6 | 17 | 35 | 28 | 45 |
| 8 | 8 | 1 | 5 | 1 | 11 |
|  | 3 | 7 | 9 | 10 | 15 |
|  | 3 | 3 | 9 | 9 | 25 |

Table 8.2: A two-way factorial treatment structure.

|  | B1 | B2 | B3 |
| --- | --- | --- | --- |
|  | y111 | y121 | y131 |
| A1 | . | . | . |
|  | . | . | . |
|  | . | . | . |
|  | y11n | y12n | y13n |
|  | y211 | y221 | y231 |
| A2 | . | . | . |
|  | . | . | . |
|  | . | . | . |
|  | y21n | y22n | y23n |
|  | y311 | y321 | y331 |
| A3 | . | . | . |
|  | . | . | . |
|  | . | . | . |
|  | y31n | y32n | y33n |
|  | y411 | y421 | y431 |
| A4 | . | . | . |
|  | . | . | . |
|  | . | . | . |
|  | y41n | y42n | y43n |

8.2	Factorial Analysis: Main Effect and Interaction

When our treatments have a factorial structure, we may also use a factorial analysis of the data. The major concepts of this factorial analysis are main effect and interaction.

Consider a two-way factorial where factor A has four levels and factor B has three levels, as in Table 8.2. There are g = 12 treatments, with 11 degrees of freedom between the treatments.  We use i and j to index the levels of factors A and B. The expected values in the twelve treatments may be denoted ij , coefficients for a contrast in the twelve means may be denote d wij (where as usual   ij wij  = 0), and the contrast sum is    ij wij   ij .  Similarly, yij• is the observed mean in the ij treatment group, and yi•• and y•j• are the observed means for all responses having level i of factor A or level j of B, respectively. It is often convenient to visualize the expected values, means,

and contrast coefficients in matrix form, as in Table 8.3.

For the moment, forget about factor B and consider the experiment to be a completely randomized design just in factor A (it is completely randomized in factor A). Analyzing this design with four “treatments,” we may compute a sum of squares with 3 degrees of freedom. The variation summarized by this sum of squares is denoted SSA and depends on just the level of factor A. The expected value for the mean of the responses in row i is + αi, where we assume that i αi = 0.

Table 8.3: Matrix arrangement of (a) expected values, (b) means, and (c) contrast coefficients in a four by three factorial.

|  | (a) |  |  |  |  |  |  | (b) |  |  |  |  |  |  | (c) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 11• |  |  | 12• |  |  | 13• |  | w11 | w12 | w13 |
| 11 | 12 | 13 |  |  | y |  |  | y |  |  | y |  |  |  |  |  |
|  |  |  |  |  |  | 21• |  |  | 22• |  |  | 23• |  | w21 | w22 | w23 |
| 21 | 22 | 23 |  |  | y |  |  | y |  |  | y |  |  |  |  |  |
|  |  |  |  |  |  | 31• |  |  | 32• |  |  | 33• |  | w31 | w32 | w33 |
| 31 | 32 | 33 |  |  | y |  |  | y |  |  | y |  |  |  |  |  |
|  |  |  |  |  |  | 41• |  |  | 42• |  |  | 43• |  | w41 | w42 | w43 |
| 41 | 42 | 43 |  |  | y |  |  | y |  |  | y |  |  |  |  |  |

Now, reverse the roles of A and B. Ignore factor A and consider the ex-periment to be a completely randomized design in factor B. We have an ex-periment with three “treatments” and treatment sum of squar es SSB with 2 degrees of freedom. The expected value for the mean of the responses in column j is + βj , where we assume that j βj = 0.

The effects αi and βj are called the main effects of factors A and B, respectively. The main effect of factor A describes variation due solely to the level of factor A (row of the response matrix), and the main effect of factor B describes variation due solely to the level of factor B (column of the response matrix). We have analogously that SSA and SSB are main-effects sums of squares.

The variation described by the main effects is variation that occurs from row to row or column to column of the data matrix. The example has twelve treatments and 11 degrees of freedom between treatments. We have de-scribed 5 degrees of freedom using the A and B main effects, so there must be 6 more degrees of freedom left to model. These 6 remaining degrees of freedom describe variation that arises from changing rows and columns si-multaneously. We call such variation interaction between factors A and B, or between the rows and columns, and denote it by SSAB .

Here is another way to think about main effect and interaction. The main effect of rows tells us how the response changes when we move from one row to another, averaged across all columns. The main effect of columns tells us how the response changes when we move from one column to an-other, averaged across all rows. The interaction tells us how the change in re-sponse depends on columns when moving between rows, or how the change in response depends on rows when moving between columns. Interaction between factors A and B means that the change in mean response going from level i1 of factor A to level i2 of factor A depends on the level of factor B under consideration. We can't simply say that changing the level of factor A changes the response by a given amount; we may need a different amount of change for each level of factor B.

References:

1.	D.C. Montgomery. Design and analysis of experiments.Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

2.	Спиридонов А.А. Планирование эксперимента при исследовании технологических процессов. –М.: Машиностроение, 1981. -184с., ил.

A Closer Look at Factorial Data

Analysis of factorially structured data should be more than just an enumer-ation of which main effects and interactions are significant . We should look closely at the data to try to determine what the data are telling us by under-standing the main effects and interactions in the data. For example, reporting that factor B only affects the response at the high level of factor A is more informative than reporting that factors A and B have signific ant main effects and interactions. One of my pet peeves is an analysis that just reports sig-nificant terms. This chapter explores a few techniques for ex ploring factorial data more closely.

9.1	Contrasts for Factorial Data

Contrasts allow us to examine particular ways in which treatments differ. With factorial data, we can use contrasts to look at how specific main ef-fects differ and to see patterns in interactions. Indeed, we have seen that the usual factorial ANOVA can be built from sets of contrasts. Chapters 4 and 5 discussed contrasts and multiple comparisons in the context of single factor analysis. These procedures carry over to factorial treatment structures with little or no modification.

In this section we will discuss contrasts in the context of a three-way factorial; generalization to other numbers of factors is straightforward. The factors in our example experiment are drug (one standard drug and two new drugs), dose (four levels, equally spaced), and administration time (morning or evening). We will usually assume balanced data, because contrasts for balanced factorial data have simpler orthogonality relationships.

We saw in one-way analysis that the arithmetic of contrasts is not too hard; the big issue was finding contrast coefficients that add ress an interest-ing question. The same is true for factorials. Suppose that we have a set of contrast coefficients wijk. We can work with this contrast for a factorial just as we did with contrasts in the one-way case using the formulae in Dis-play 9.1. These formulae are nothing new, merely the application of our usual contrast formulae to the design with g = abc treatments. We still need to find meaningful contrast coefficients.

Pairwise comparisons are differences between two treatments, ignoring the factorial structure. We might compare the standard drug at the lowest dose with morning administration to the first new drug at the l owest dose with evening administration. As we have seen previously with pairwise com-parisons, there may be a multiple testing issue to consider, and our pairwise multiple comparisons procedures (for example, HSD) carry over directly to the factorial setting.

A simple effect is a particular kind of pairwise comparison. A simple effect is a difference between two treatments that have the same levels of all factors but one. A comparison between the standard drug at the lowest dose with morning administration and the standard drug at the lowest dose with evening administration is a simple effect. Differences of main effects are averages of simple effects.

The structure of a factorial design suggests that we should also consider contrasts that reflect the design, namely main-effect contr asts and interaction contrasts. In general, we use contrasts with coefficient pat terns that mimic those of factorial effects. A main-effect contrast is one where the coefficients depend only on a single index; for example, k for a factor C contrast. That is, two contrast coefficients are equal if they have the s ame k index. These coefficients will add to zero across k for any i and j. For interaction contrasts, the coefficients depend only on the indices of fac tors in the inter-action in question and satisfy the same zero-sum restrictions as their corre-sponding model terms. Thus a BC interaction contrast has coefficients wijk that depend only on j and k and add to zero across j or k when the other subscript is kept constant. For an ABC contrast, the coeffici ents wijk must add to zero across any subscript.

We can use pairwise multiple comparisons procedures such as HSD for marginal means. Thus to compare all levels of factor B using HSD, we treat the means y•j•• as b treatment means each with sample size acn and do mul-tiple comparisons with abc(n − 1) degrees of freedom for error. The same approach works for two-way and higher marginal tables of means. For exam-ple, treat y•jk• as bc treatment means each with sample size an and abc(n −1) degrees of freedom for error. Pairwise multiple comparisons procedures also work when applied to main effects—for example, βj —but most do not work for interaction effects due to the additional zero sum restrictions. (Bonferroni does work.)

Please note: simple-effects, main-effects, and interaction contrasts are examples of contrasts that are frequently useful in analysis of factorial data; there are many other kinds of contrasts. Use contrasts that address your ques-tions. Don't be put off if a contrast that makes sense to you does not fit into one of these neat categories.

| Factorial contrasts | Example 9.1 |
| --- | --- |

Let's look at some factorial contrasts for our three-way drug test example. Coefficients wijk for these contrasts are shown in Table 9.1. Suppose that we want to compare morning and evening administration times averaged across all drugs and doses. The first contrast in Table 9.1 has coeffic ients -1 for

| Table 9.1: Example contrasts. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Morning versus Evening |  |  |  |  |  |  |  |  |
|  |  |  | Dose |  |  |  |  |  |  | Dose |  |  |
| Drug |  | 1 | 2 | 3 | 4 | Drug |  |  | 1 | 2 | 3 | 4 |
| 1 |  | 1 | 1 | 1 | 1 | 1 |  |  | -1 -1 -1 -1 |  |  |  |
| 2 |  | 1 | 1 | 1 | 1 | 2 |  |  | -1 -1 -1 -1 |  |  |  |
| 3 |  | 1 | 1 | 1 | 1 | 3 |  |  | -1 -1 -1 -1 |  |  |  |
|  |  |  | Dose |  | Linear in Dose |  |  |  |  | Dose |  |  |
| Drug |  | 1 | 2 | 3 | 4 | Drug |  |  | 1 | 2 | 3 | 4 |
| 1 |  | -3 -1 |  | 1 | 3 | 1 |  |  | -3 -1 |  | 1 | 3 |
| 2 |  | -3 -1 |  | 1 | 3 | 2 |  |  | -3 -1 |  | 1 | 3 |
| 3 |  | -3-113 |  |  |  | 3 |  |  | -3-113 |  |  |  |
|  |  | Linear in Dose by Morning versus Evening |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dose |  |  |  |  |  |  | Dose |  |  |
| Drug |  | 1 | 2 | 3 | 4 | Drug |  |  | 1 | 2 | 3 | 4 |
| 1 |  | -3 -1 |  | 1 | 3 | 1 |  |  | 3 | 1 | -1 -3 |  |
| 2 |  | -3 -1 |  | 1 | 3 | 2 |  |  | 3 | 1 | -1 -3 |  |
| 3 |  | -3-113 |  |  |  | 3 |  |  | 3 | 1 | -1 -3 |  |
|  |  | Linear in Dose by Morning versus Evening |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | by Drug 2 versus Drug 3 |  |  |  |  |  |  |  |  |
|  |  |  | Dose |  |  |  |  |  |  | Dose |  |  |
| Drug |  | 1 | 2 | 3 | 4 | Drug |  |  | 1 | 2 | 3 | 4 |
| 1 |  | 0 | 0 | 0 | 0 | 1 |  |  | 0 | 0 | 0 | 0 |
| 2 |  | -3 -1 |  | 1 | 3 | 2 |  |  | 3 | 1 | -1 -3 |  |
| 3 |  | 3 | 1 | -1 -3 |  | 3 |  |  | -3 -1 |  | 1 | 3 |
|  |  |  |  | Linear in Dose for Drug 1 |  |  |  |  |  |  |  |  |
|  |  |  | Dose |  |  |  |  |  |  | Dose |  |  |
| Drug |  | 1 | 2 | 3 | 4 | Drug |  |  | 1 | 2 | 3 | 4 |
| 1 |  | -3 -1 |  | 1 | 3 | 1 |  |  | -3 -1 |  | 1 | 3 |
| 2 |  | 0 | 0 | 0 | 0 | 2 |  |  | 0 | 0 | 0 | 0 |
| 3 |  | 0 | 0 | 0 | 0 | 2 |  |  | 0 | 0 | 0 | 0 |

evening and 1 for morning and thus makes the desired comparison. This is a main-effect contrast (coefficients only depend on administ ration time, factor C). We can get the same information by using a contrast with coefficients (1, -1) and the means y••k• or effects γk.

The response presumably changes with drug dose (factor B), so it makes sense to examine dose as a quantitative effect. To determine the linear effect of dose, use a main-effect contrast with coefficients -3, -1, 1, and 3 for doses 1 through 4 (Appendix Table D.6); this is the second contrast in Table 9.1. As with the first example, we could again get the same informat ion from a

contrast in the means y•j•• or effects βj using the same coefficients. The simple coefficients -3, -1, 1, and 3 are applicable here becau se the doses are equally spaced and balance gives equal sample sizes.

A somewhat more complex question is whether the linear effect of dose is the same for the two administration times. To determine this, we compute the linear effect of dose from the morning data, and then subtract the linear effect of dose from the evening data. This is the third contrast in Table 9.1. This is a two-factor interaction contrast; the coefficients add t o zero across dose or administration time. Note that this contrast is literally the elementwise product of the two corresponding main-effects contrasts.

A still more complex question is whether the dependence of the linear effect of dose on administration times is the same for drugs 2 and 3. To de-termine this, we compute the linear in dose by administration time interaction contrast for drug 2, and then subtract the corresponding contrast for drug 3. This three-factor interaction contrast is the fourth contrast in Table 9.1. It is formed as the elementwise product of the linear in dose by administration time two-way contrast and a main-effect contrast between drugs 2 and 3.

Finally, the last contrast in Table 9.1 is an example of a useful contrast that is not a simple effect, main effect, or interaction contrast. This contrast examines the linear effect of dose for drug one, averaged across time.

The interaction contrasts in Example 9.1 illustrate an important special case of interaction contrasts, namely, products of main-effect contrasts. These products allow us to determine if an interesting contrast in one main effect varies systematically according to an interesting contrast in a second main effect.

We can reexpress a main-effect contrast in the individual treatment means

ijk• in terms of a contrast in the factor main effects or the factor marginal means. For example, a contrast in factor C can be reexpressed as

| wijk |  | ijk• = |  | w11k |  |  |  |  | ijk• |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | y |  |  |  |  |  |  | y |  |  |
| ijk |  |  | k |  |  |  | ij |  |  |  |
| = |  |  |  | wk |  | ••k• |  |  |  |  |
|  |  |  |  |  | y |  |  |  |  |  |
|  |  |  | k |  |  |  |  |  |  |  |
| = |  |  |  | wk γk , |  |  |  |  |  |  |
|  |  |  | k |  |  |  |  |  |  |  |

where wk = abw11k . Because scale is somewhat arbitrary for contrast coef-ficients, we could also use wk = w11k and still get the same kind of informa-tion. For balanced data, two main-effect contrasts for the same factor with coefficients wk and wk⋆ are orthogonal if

wk wk⋆ = 0  .

k

We can also express an interaction contrast in the individual treatment means as a contrast in marginal means or interaction effects. For example, suppose wijk is a set of contrast coefficients for a BC interaction contras t. Then we can rewrite the contrast in terms of marginal means or interaction effects:

| wijk |  | ijk• = | wjk |  | •jk• |
| --- | --- | --- | --- | --- | --- |
|  | y |  |  | y |  |
| ijk |  |  | jk |  |  |
| = |  |  | wjk βγjk |  |  |
|  |  |  | jk |  |  |

where aw1jk = wjk. Two interaction contrasts for the same interaction with coefficients wjk and wjk⋆ are orthogonal if

wjk wjk⋆ = 0  .

For balanced data, the formulae in Display 9.1 can be simplifi ed by re-placing the sample size nijk by the common sample size n. The formulae can be simplified even further for main-effect and interaction c ontrasts, because they can be rewritten in terms of the effects or marginal means of interest in-stead of using all treatment means. Consider a main-effect contrast in factor C with coefficients  wk ; the number of observations at the kth level of factor

C is abn. We have for the contrast	:

Expected value

Variance

Sum of squares

Confidence interval

F-test

k wk γk

- wk2 σ2/(abn)

- k wk y••k•)2

k wk2/(abn)

- wk y••k•±

| t | E/2,N −abc | M SE | k | w2 | /(abn) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | k |  |

- k wk y••k•)2

| M SE | k | w2 | /(abn) |
| --- | --- | --- | --- |
|  |  | k |  |

The simplification is similar for interaction contrasts. Fo r example, the BC interaction contrast jk wjk y•jk• has sum of squares

- jk wjk y•jk•)2 jk wjk2 /(an)

(an is the “sample size” at each	jk combination).

9.2	Modeling Interaction

An interaction is a deviation from additivity. If the effect of going from dose 1 to dose 2 changes from drug 2 to drug 3, then there is an interaction between drug and dose. Similarly, if the interaction of drug and dose is different in morning and evening applications, then there is a three-factor interaction between drug, dose, and time. Try to understand and model any interaction that may be present in your data. This is not always easy, but when it can be done it leads to much greater insight into what the data have to say. This section discusses three specific models for interaction; th ere are many others.

9.2.1	Interaction plots

We introduced interaction plots in Section 8.4 as a method for visualizing interaction. These plots continue to be important tools, but there are a few variations on interaction plots that can make them more useful in multi-way factorials. The first variation is to plot marginal means. If , for example, we are exploring the AB interaction, then we can make an interaction plot using the means yij••. Thus we do not plot every treatment mean individually but instead average across any other factors. This makes for a cleaner picture of the AB interaction, because it hides all other interactions.

A second variation is to plot interaction effects rather than marginal means. Marginal means such as yij•• satisfy

- ij•• =+ αi + βj + αβij  ,

so they contain main effects as well as interaction. By making the interaction

plot using αβij instead of yij••, we eliminate the main effects information and concentrate on the interaction. This is good for understanding the nature of the interaction once we are reasonably certain that interaction is there, but it works poorly for diagnosing the presence of interaction because interac-tion plots of interaction effects will always show interaction. So first decide whether interaction is present by looking at means or by using ANOVA. If interaction is present, a plot of interaction effects can be useful in understand-ing the interaction.

9.2.2	One-cell interaction

A one-cell interaction is a common type of interaction where most of the ex-periment is additive, but one treatment deviates from the additive structure. The name “cell” comes from the idea that one cell in the table o f treatment means does not follow the additive model. More generally, there may be one or a few cells that deviate from a relatively simple model. If the devia-tion from the simple model in these few cells is great enough, all the usual factorial interaction effects can be large and statistically significant.

Understanding one-cell interaction is easy: the data follow a simple model except for a single cell or a few cells. Finding a one-cell interaction is harder. It requires a careful study of the interaction effects or plots or a more sophis-ticated estimation technique than the least squares we have been using (see Daniel 1976 or Oehlert 1994). Be warned, large one-cell interactions can be masked or hidden by other large one-cell interactions.

One-cell interactions can sometimes be detected by examination of in-teraction effects. A table of interaction effects adds to zero across rows or columns. A one-cell interaction shows up in the effects as an entry with a large absolute value. The other entries in the same row and column are mod-erate and of the opposite sign, and the remaining entries are small and of

|  | Table 9.2: Data from a replicated four-factor experiment. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All factors have two levels, labeled low and high. |  |  |  |  |  |  |  |
|  | A | B | C |  |  | D |  |  |
|  |  |  |  |  | Low |  | High |  |
|  | low | low | low | 26.1 | 27.5 | 23.5 | 21.1 |  |
|  | low | low | high | 22.8 | 23.8 | 30.6 | 32.5 |  |
|  | low | high | low | 22.0 | 20.2 | 28.1 | 29.9 |  |
|  | low | high | high | 30.0 | 29.3 | 38.3 | 38.5 |  |
|  | high | low | low | 11.4 | 11.0 | 20.4 | 22.0 |  |
|  | high | low | high | 22.3 | 20.2 | 28.7 | 28.8 |  |
|  | high | high | low | 18.9 | 16.4 | 26.6 | 26.5 |  |
|  | high | high | high | 29.6 | 29.8 | 34.5 | 34.9 |  |

the same sign as the interacting cell. For example, a three by four factorial with all responses 0 except for 12 in the (2,2) cell has interaction effects as

| follows: |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | -3 | 1 | 1 |  |
| -2 | 6 | -2 | -2 |  |
| 1 | -3 | 1 | 1 |  |
| Rearranging the rows and columns to put the one-cell interaction in a corner |  |  |  |  |
| emphasizes the pattern: |  |  |  |  |
| 6 | -2 | -2 | -2 |  |
| -3 | 1 | 1 | 1 |  |
| -3 | 1 | 1 | 1 |  |
| One-cell interaction |  |  |  | Example 9.2 |

Consider the data in Table 9.2 (Table 1 of Oehlert 1994). These data are responses from an experiment with four factors, each at two levels labeled low and high, and replicated twice. A standard factorial ANOVA of these data shows that all main effects and interactions are highly significant, and analysis of the residuals reveals no problems. In fact, these data follow an additive model, except for one unusual treatment. Thus all interaction in these data is one-cell interaction.

The interacting cell is the treatment combination with all factors low (it is about 12.5 units higher than the additive model predicts); casual inspection of the data would probably suggest the treatment with mean 11.2, but that is incorrect. We can see the one-cell interaction in Figure 9.1, which shows an

|  |  |  |  |  | 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 35 |  |  |  |  |  | 4 |
|  | 4 |  |  |  |  |  |  |
|  | 30 |  | 4 |  | 2 |  | 2 |
|  |  |  |  |  | 3 |  |  |
|  | 1 |  |  |  |  |  | 3 |
| y | 25 |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |
|  | 3 |  | 32 |  | 1 |  |  |
|  | 20 |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 1 |
|  | 15 |  |  |  |  |  |  |
|  |  |  | 1 |  |  |  |  |
|  | 1 | 1.5 | 2 | 2.5 | 3 | 3.5 | 4 |
|  |  |  |  | A/B combinations |  |  |  |

Figure 9.1: Interaction plot for data in Table 9.2, using MacAnova. Horizontal locations 1 through 4 correspond to (A low, B low), (A high, B low), (A low, B high), and (A high, B high). Curves 1 through 4 correspond to (C low, D low), (C high, D low), (C low, D high), and (C high, D high).

interaction plot of the treatment means. The first mean in the line labeled 1 is too high, but the other segments are basically parallel.

9.2.3	Quantitative factors

A second type of interaction that can be easily modeled occurs when one or more of the factors have quantitative levels (doses). First consider the situation when the interacting factors are all quantitative. Suppose that the doses for factor A are zAi, and those for factor B are zBj . We can build a polynomial regression model for cell means as

a−1	b−1	a−1 b−1

ij = θ0 +	θArzAir +	θBszBjs +	θArBszAirzBjs	.

r=1	s=1	r=1 s=1

Polynomial terms in zAi model the main effects of factor A, polynomial terms in zBj model the main effects of factor B, and cross product terms model the AB interaction. Models of this sort are most useful when relatively few of

the polynomial terms are needed to provide an adequate description of the response.

A polynomial term zAirzBjs is characterized by its exponents (r, s). A term with exponents (r, s) is “above” a term with exponents (u, v) if r ≤ u and s ≤ v; we also say that (u, v) is below (r, s). The mnemonic here is that in an ANOVA table, simpler terms (such as main effects) are above more complicated terms (such as interactions). This is a little confusing, because we also use the phrase higher order for the more complicated terms, but higher order terms appear below the simpler terms.

A term in this polynomial model is needed if its own sum of squares is large, or if it is above a term with a large sum of squares. This preserves a polynomial hierarchy. We compute the sum of squares for a term by looking at the difference in error sums of squares for two models: subtract the error sum of squares for the model that contains the term of interest, and all terms that are above it from the error sum of squares for the model that contains only the terms above the term of interest. Thus, the sum of squares for the term zAi2zBi1 is the error sum of squares for the model with terms zAi, zAi2 , zBi and zAizBi, less the error sum of squares for the model with terms zAi,

zAi2, zBi, zAizBi, and zAi2 zBi1.

Computation of the polynomial sums of squares can usually be accom-plished in statistical software with one command. Recall, however, that the polynomial coefficients θ depend on what other polynomial terms are in a given regression model. Thus if we determine that only linear and quadratic terms are needed, we must refit the model with just those terms to find their coefficients when the higher order terms are omitted. In part icular, you should not use coefficients from the full model when predicti ng with a model with fewer terms. Use the full model M SE for determining which terms to include, but use coefficients computed for a model including just your se-lected terms.

For single-factor models, we were able to compute polynomial sums of squares using polynomial contrasts when the sample sizes are equal and the doses are equally spaced. The same is true for balanced factorials with equally spaced doses. Polynomial main-effect contrast coefficients are the same as the polynomial contrast coefficients for single-fac tor models, and polynomial interaction contrast coefficients are the eleme ntwise products of the polynomial main-effect contrasts.

| Amylase activity, continued | Example 9.3 |
| --- | --- |

Recall the amylase specific activity data of Example 8.10. Th e three factors are analysis temperature, growth temperature, and variety. On the log scale,

the analysis temperature by growth temperature interaction (both quantitative variables) was marginally significant. Let us explore the ma in effects and interactions using quantitative variables. We cannot use the tabulated contrast coefficients here because the levels of analysis temperatur e are not equally spaced.

Listing 9.1 gives the ANOVA for the polynomial main effects and in-teractions of analysis temperature (at) and growth temperature (gt). The M SE for this experiment was .00546 with 64 degrees of freedom. We see that linear, quadratic, and cubic terms in analysis temperature are significant, but no higher order terms. Also the cross products of linear in growth tem-perature and linear and cubic analysis temperature are significant. Thus a succinct model would include the three lowest order terms for analysis tem-perature, growth temperature, and their cross products. We need to refit with just those terms to get coefficients.

This example also illustrates a bothersome phenomenon—the averaging involved in multi-degree-of-freedom mean squares can obscure some inter-esting effects in a cloud of uninteresting effects. The 7 degree-of-freedom growth temperature by analysis temperature interaction is marginally signif-icant with a p-value of .054, but some individual degrees of freedom in that 7 degree-of-freedom bundle are rather more significant.

There can also be interaction between a quantitative factor and a non-quantitative factor. Here are a couple of ways to proceed. First, we can use interaction contrasts that are products of a polynomial contrast in the quantitative factor and an interesting contrast in the qualitative factor. For example, we might have three drugs at four doses, with one control drug and two new drugs. An interesting contrast with coefficients (1, -.5, -. 5) compares the con-trol drug to the mean of the new drugs. The interaction contrast formed by the product of this contrast and linear in dose would compare the linear effect of dose in the new drugs with the linear effect of dose in the control drug.

Second, we can make polynomial models of the response (as a function of the quantitative factor) separately for each level of the qualitative factor. Let ij be the expected response at level i of a quantitative factor with dose zAi and level j of a qualitative factor. We have a choice of several equivalent models, including:

a−1

ij	=	θj +	θArj zAir

r=1

and

|  |  |  | a−1 |  |  | a−1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ij | = θ | 0 | +β+θ | Ar0 | zr | +θβ zr | , |
|  |  |  | j |  | Ai | Arj Ai |  |
|  |  |  | r=1 |  |  | r=1 |  |

where θj = θ0 + βj , θArj = θAr0 + θβArj , and the parameters have the zero sum restrictions j βj = 0 and j θβArj = 0.

In both forms there is a separate polynomial of degree a − 1 in zAi for each level of factor B. The only difference between these models is how the regression coefficients are expressed. In the first version t he constant terms of the model are expressed as θj ; in the second version the constant terms are expressed as an overall constant θ0 plus deviations βj that depend on the qualitative factor. In the first version the coefficients for power r are expressed as θArj ; in the second version the coefficients for power r are expressed as an overall coefficient θAr0 plus deviations θβArj that depend on the qualitative factor. These are analogous to having treatment means i written as + αi, an overall mean plus treatment effects.

Suppose again that we have three drugs at four doses; do we need sepa-rate cubic coefficients for the different drugs, or will one o verall coefficient suffice? To answer this we can test the null hypothesis that al l the θA3j 's equal each other, or equivalently, that all the θβA3j 's are zero. In many statistics packages it is easier to do the tests using the overall-plus-deviation form of the model.

References:

1.	D.C. Montgomery. Design and analysis of experiments.Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

2.	Спиридонов А.А. Планирование эксперимента при исследовании технологических процессов. –М.: Машиностроение, 1981. -184с., ил.

Further Topics in Factorials

There are many more things to learn about factorials; this chapter covers just a few, including dealing with unbalanced data, power and sample size for factorials, and special methods for two-series designs.

10.1	Unbalanced Data

Our discussion of factorials to this point has assumed balance; that is, that all factor-level combinations have the same amount of replication. When this is not true, the data are said to be unbalanced. The analysis of unbalanced data is more complicated, in part because there are no simple formulae for the quantities of interest. Thus we will need to rely on statistical software for all of our computation, and we will need to know just exactly what the software is computing, because there are several variations on the basic computations.

The root cause of these complications has to do with orthogonality, or rather the lack of it. When the data are balanced, a contrast for one main effect or interaction is orthogonal to a contrast for any other main effect or interaction. One consequence of this orthogonality is that we can estimate effects and compute sums of squares one term at a time, and the results for that term do not depend on what other terms are in the model. When the data are unbalanced, the results we get for one term depend on what other terms are in the model, so we must to some extent do all the computations simultaneously.

The questions we want to answer do not change because the data are unbalanced. We still want to determine which terms are required to model the response adequately, and we may wish to test specific null hypotheses about model parameters. We made this distinction for balanced data in Sec-tion 8.11, even though the test statistics for comparing models or testing hy-potheses are the same. For unbalanced data, this distinction actually leads to different tests.

Our discussion will be divided into two parts: building models and test-ing hypotheses about parameters. We will consider only exact approaches for computing sums of squares and doing tests. There are approximate meth-ods for unbalanced factorials that were popular before the easy availability of computers for doing all the hard computations. But when you have the computational horsepower, you might as well use it to get exact results.

10.1.1 Sums of squares in unbalanced data

We have formulated the sum of squares for a term in a balanced ANOVA model as the difference in error sum of squares for a reduced model that excludes the term of interest, and that same model with the term of interest included. The term of interest is said to have been “adjusted for” the terms in the reduced model. We also presented simple formulae for these sums of squares. When the data are unbalanced, we still compute the sum of squares for a term as a difference in error sums of squares for two models, but there are no simple formulae to accomplish that task. Furthermore, precisely which two models are used doesn't matter in balanced data so long as they only differ by the term of interest, but which models are used does matter for unbalanced data.

Models are usually specified as a sequence of terms. For examp le, in a three-factor design we might specify (1, A, B, C) for main effects, or (1, A, B, AB, C) for main effects and the AB interaction. The “1” de notes the overall grand mean that is included in all models. The sum of squares for a term is the difference in error sums of squares for two models that differ only by that term. For example, if we look at the the two models (1, A, C) and (1, A, B, C), then the difference in error sums of squares will be the sum of squares for B adjusted for 1, A, and C. We write this as SS(B|1, A, C).

Example 10.1	Unbalanced amylase data

Recall the amylase data of Example 8.10, where we explore how amylase activity depends on analysis temperature (A), variety (B), and growth tem-perature (C). Suppose that the first observation in growth te mperature 25, analysis temperature 40, and variety B73 were missing, making the data un-balanced. The sum of squares for factor C is computed as the difference in error sums of squares for a pair of models differing only in the term C.

Here are five such model pairs: (1), (1, C); (1, A), (1, A, C); (1 , B), (1, B, C); (1, A, B), (1, A, B, C); (1, A, B, AB), (1, A, B, AB, C). The sums of squares for C computed using these five model pairs are denote d SS(C|1),

- (C|1, A), SS(C|1, B), SS(C|1, A, B) and SS(C|1, A, B, AB), and are shown in following table (sum of squares ×106, data on log scale):

| SS(C¦1) | 2444.1 |
| --- | --- |
| SS(C¦1, A) | 1396.0 |
| SS(C¦1, B) | 3303.0 |
| SS(C¦1, A, B) | 2107.4 |
| SS(C¦1, A, B, AB) | 2069.4 |

All five of these sums of squares differ, some rather substant ially. There is no single sum of squares for C, so we must explicitly state which one we are using at any give time.

The simplest choice for a sum of squares is sequential sums of squares. This is called Type I in SAS. For sequential sums of squares, we specify a model and the sum of squares for any term is adjusted for those terms that precede it in the model. If the model is (1, A, B, AB, C), then the sequential sums of squares are SS(A|1), SS(B|1, A), SS(AB|1, A, B), and (C|1, A, B, AB). Notice that if you specify the terms in a different order, you get different sums of squares; the sequential sums of squares for (1, A, B, C, AB) are SS(A|1), SS(B|1, A), SS(C|1, A, B), and SS(AB|1, A, B, C). Two models that include the same terms in different order will have the same estimated treatment effects and interactions. However, models that in-clude different terms may have different estimated effects for the terms they have in common. Thus (1, A, B, AB, C) and (1, A, B, C, AB) will have the same αi's, but (1, A, B, AB, C) and (1, A, B, C) may have different αi's.

10.1.2	Building models

Building models means deciding which main effects and interactions are needed to describe the data adequately. I build hierarchical models. In a hierarchical model, the inclusion of any interaction in a model implies the inclusion of any term that is “above” it, where we say that a fa ctorial term U is above a factorial term V if every factor in term U is also in term V. The goal is to find the hierarchical model that includes all terms that must be included, but does not include any unnecessary terms.

Our approach to computing sums of squares when model-building is to use as the reduced model for term U the largest hierarchical model M that does not contain U. This is called Type II in the SAS statistical program. In two-factor models, this might be called “Yates' fitting cons tants” or “each adjusted for the other.”

Consider computing Type II sums of squares for all the terms in a three-factor model. The largest hierarchical models not including ABC, BC, and C are (1, A, B, C, AB, AC, BC), (1, A, B, C, AC, AB), and (1, A, B, AB), respectively. Thus for Type II sums of squares, the three-factor interaction is adjusted for all main effects and two-factor interactions, a two-factor inter-action is adjusted for all main effects and the other two-factor interactions, and a main effect is adjusted for the other main effects and their interactions, or SS(ABC|1, A, B, C, AB, AC, BC), SS(BC|1, A, B, C, AB, AC), and

- (C|1, A, B, AB). In Example 10.1, the Type II sum of squares for growth temperature (factor C) is 2069 × 10−6 .

  - It is important to point out that the denominator mean square used for testing is M SE from the full model. We do not pool “unused” terms into error. Thus, the Type II SS for C is SS(C|1, A, B, AB), but the error mean square is from the model (1, A, B, C, AB, AC, BC, ABC).

    - Testing hypotheses

In some situations we may wish to test specific hypotheses abo ut treatment means rather than building a model to describe the means. Many of these hypotheses can be expressed in terms of the factorial parameters, but recall that the parameters we use in our factorial decomposition carry a certain amount of arbitrariness in that they assume equally weighted averages. When the hypotheses of interest correspond to our usual, equally weighted factorial parameters, testing is reasonably straightforward; otherwise, special purpose contrasts must be used.

Let's review how means and parameters correspond in the two-factor sit-uation. Let ij be the mean of the ijth treatment:

- ij =	+ αi + βj + αβij

- Listing 10.2: SAS output for data in Table 10.1.

|  | General Linear Models Procedure |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Dependent Variable: Y |  |  |  |  |  |
|  |  | Sum of | Mean |  |  |
| Source | DF | Squares | Square | F Value | Pr > F |
| Model | 3 | 2876.88041 | 958.96014 | 8.53 | 0.0022 |
| Error | 13 | 1460.78900 | 112.36838 |  |  |
| Corrected Total | 16 | 4337.66941 |  |  |  |
| Source | DF | Type I SS | Mean Square | F Value | Pr > F |
| A | 1 | 2557.00396 | 2557.00396 | 22.76 | 0.0004 |
| B | 1 | 254.63189 | 254.63189 | 2.27 | 0.1561 |
| A*B | 1 | 65.24457 | 65.24457 | 0.58 | 0.4597 |
| Source | DF | Type II SS | Mean Square | F Value | Pr > F |
| A | 1 | 485.287041 | 485.287041 | 4.32 | 0.0581 |
| B | 1 | 254.631889 | 254.631889 | 2.27 | 0.1561 |
| A*B | 1 | 65.244565 | 65.244565 | 0.58 | 0.4597 |
| Source | DF | Type III SS | Mean Square | F Value | Pr > F |
| A | 1 | 499.951348 | 499.951348 | 4.45 | 0.0549 |
| B | 1 | 265.471348 | 265.471348 | 2.36 | 0.1483 |
| A*B | 1 | 65.244565 | 65.244565 | 0.58 | 0.4597 |

- ①

- ②

- ③

- ④

  - Example 10.4	Unbalanced data puzzle, continued

- Let us continue Example 10.3. If we wish to test the null hypothesis that αi ≡ 0 or βj ≡ 0, we need to use Type III sums of squares. This is shown at ④ of Listing 10.2. None of the null hypotheses about main effects or interaction is anywhere near as significant as the overall mo del; all have p-values greater than .05.

- How can this be so when we know that there are large differences be-tween treatment means in the data? Consider for a moment the test for factor A main effects. The null hypothesis is that the factor A main effects are zero, but no constraint is placed on factor B main effects or interactions. We can fit the data fairly well with the αi's equal to zero, so long as we can manipulate the βj 's and αβij 's to take up the slack. Similarly, when testing factor B, no constraint is placed on factor A main effects or AB interactions. These three tests of A, B, and AB do not test that all three null hypotheses are true simultaneously. For that we need to test the overall model with 3 degrees of freedom.

- When we test the null hypothesis that a contrast in treatment effects is zero, we are testing the null hypothesis that a particular linear combination

- Contrast SS are	of treatment means is zero with no other restrictions on the cell means. This

- Type III	is equivalent to testing that the single degree of freedom represented by the contrast can be removed from the full model, so the contrast has been ad-justed for all other effects in the model. Thus the sum of squares for any contrast is a Type III sum of squares.

| Unbalanced amylase data, continued | Example 10.5 |
| --- | --- |

- Continuing Example 10.1, the Type III ANOVA can be found at ② in List-ing 10.1. The Type III sum of squares for growth temperature is .0025845, different from both Types I and II. If you compute the main-effect contrast in growth temperature with coefficients 1 and -1, you get the res ults shown at ③ in Listing 10.1, including the same sum of squares as the Type III analysis. This equivalence of the effect sum of squares and the contrast sum of squares is due to the fact that the effect has only a single degree of freedom, and thus the contrast describes the entire effect.

- The only factorial null hypotheses that would be rejected are those for the main effects of analysis temperature and variety and the interaction of growth temperature and variety. Thus while growth temperature and variety jointly act to influence the response, there is no evidence that the av erage responses for the two growth temperatures differ (equally weighted averages across all analysis temperatures and varieties).

Criteria for evaluating students' knowledge:

Knowledge, skills, skills and competencies of students in all types of control are determined by the scores of the point-rating letter system, which have a directly proportional ratio.

The "excellent" rating corresponds to the grades A, which has a digital equivalent of 4.0 and a percentage of 95-100%, and A -, which has a digital equivalent of 3.67 and a percentage of 90-94%.

This assessment is made if the student showed complete assimilation of the program material and did not make any mistakes, inaccuracies, timely and correctly completed the tasks in the classroom, showed original thinking, passed all the tasks in a timely manner and without any errors, completed homework, independently used additional scientific literature in the study of the discipline, was able to independently systematize the program material.

The rating" good " corresponds to the ratings B+, which has a digital equivalent of 3.33 and a percentage of 85-89%; B, which has a digital equivalent of 3.0 and a percentage of 80-84%; B -, which has a digital equivalent of 2.67 and a percentage of 75-79%; C+, which has a digital equivalent of 2.33 and a percentage of 70-74%.

This assessment is made if the student has mastered the program material by at least 75% and at the same time has not made any gross mistakes in the answer, has completed all the tasks in a timely manner and passed them without fundamental comments, has correctly completed and passed the homework assignments in a timely manner without fundamental comments, has used additional literature on the instructions of the teacher, has made unprincipled inaccuracies or fundamental errors corrected by the student himself, has managed to systematize the program material with the help of the teacher.

The rating" satisfactory " corresponds to the ratings C, which has a digital equivalent of 2.0 and a percentage of 65-69%; C -, which has a digital equivalent of 1.67 and a percentage of 60-64 %; D +, which has a digital equivalent of 1.33 and a percentage of 55-59%; D, which has an equivalent of 1.0 and a percentage of 50-54%.

This assessment is made if the student has mastered the program material by at least 50%, when performing control and laboratory work, homework, needed the help of a teacher, when passing the colloquium, made inaccuracies and unprincipled errors, inaccuracies, did not show activity in research work, was limited only to the educational literature specified by the teacher, experienced more difficulties in systematizing the material.

The rating "unsatisfactory" corresponds to the rating FX, which has a digital equivalent of 0.5 and a percentage of 25-49% , and F, which has a digital equivalent of 0 and a percentage of 0-24%.This assessment is made if the student found gaps in the knowledge of the main material provided by the program, did not master more than half of the discipline program, made fundamental mistakes in the answers, did not complete individual tasks provided for by the forms of the current, intermediate and final control, did not work through all the main literature provided by the program.

References:

1.	D.C. Montgomery. Design and analysis of experiments. Eighth edition. –John Wiley & Sons, Inc, 2013. ISBN 978-1-118-14692-7.

2.	Сидняев Н. И. Введение в теорию планирования эксперимента: учеб. пособие . М. : Изд-во МГТУ им. Н. Э. Баумана, 2011. – 463 с.

3. Спиридонов А.А. Планирование эксперимента при исследовании технологических процессов. –М.: Машиностроение, 1981. -184с., ил.

4. H.J.Seltman. ExperimentaldesignandAnalysis.2018. – P. 428.

5. Gary W. Oehlert.A First Course in Design and Analysis of Experiments.Library of Congress Cataloging-in-Publication Data.2010. – P. 679.

Additional:

6. К. Хартман, Э. Лецкий, В. Шефер. Планирование эксперимента в исследовании технологических процессов. М.: Мир, 1977. – 545 с., ил.

7. A. A. Спиридонов. Планирование эксперимента при исследовании технологических процессов. М.: Машиностроение, 1981. – 184 с., ил.

CONTENT

| Introduction | 5 |
| --- | --- |
| Strategy of experimentation. | 6 |
| Some typical applications of experimental design. | 7 |
| Basic principles. | 9 |
| Guidelines for designing experiments. | 9 |
| A brief history of statistical design. | 12 |
| Simple comparative experiments. | 13 |
| Basic statistical concepts. | 14 |
| Sampling and sampling distributions. | 15 |
| Experiments with a single factor: the analysis of variance. Analysis of the fixed effects model. | 17 |
| The randomized complete block design. Statistical analysis of the RCBD. | 22 |
| Basic definitions and principles of factorial design. The two-factor factorial design | 24 |
| The 2k factorial design.The 22 Design.The 23 Design. The General 2k Design.A single replicate of the 2k Design. | 27 |
| Blocking a replicated 2k factorial design. Confounding in 2k factorial design. | 30 |
| Factorial treatment structure | 33 |
| A closer look at factorial data | 37 |
| Further Topics in Factorials | 52 |
| Unbalanced Data | 52 |
| Building models | 54 |
| Testing hypotheses | 55 |
| Unbalanced data puzzle, continued | 56 |

Zhilkibayeva S.K., Abzalova D.A., Abshenov Kh.A

Lecture notes

for the course «Mathematical Methods of Experiment Planning»

Signed to the press

Paper format Х*Y   1/16

Typographic paper.Offset printing. Volume 4 p.sh.

Circulation ______ copies. Order №

© Edition of M.Auezov South Kazakhstan University, 2020

Publishing center of M.Auezov South Kazakhstan University, Shymkent, Tauke-khan ave., 5

*These values are set by the CI