Ф. 7.03-11

KOCHEROVA A.N.

THEORY OF ELECTRICAL COMMUNICATION

Lecture notes

For students of specialty

6В06210 "Radio Engineering, electronics and telecommunication"

Shymkent, 2021

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

SOUTH KAZAKHSTAN UNIVERSITY them. M.O.UEZOVA

|  | Non-profit joint stock Company "M.Auezov South Kazakhstan University" |
| --- | --- |

KOCHEROVA A.N.

THEORY OF ELECTRICAL COMMUNICATION

Lecture notes

For students of specialty

6В06210 "Radio Engineering, electronics and telecommunication"

Shymkent, 2021

UDC 658.012.011

Kocherova A.N. Theory of electrical communication  /The collection of lectures - Shymkent:  M.Auezov SKU,  2021 - 64p.

ISBN ______________

Short annotation

The collection of lectures is made according to requirements of the curriculum and the program of discipline Theory of electrical communication and data transmission and includes all necessary data on discipline. Methodical instructions contain the purpose, the contents, rules of design.

Theory of electrical communication. The collection of lectures is intended for students of specialty 6B06210 – Radio engineering, electronics and telecommunications

Reviewer:

Musabekov A.A.. - cand.tech.sci., the associate professor of M.Auezov SKU

Smilov N.K. – Assoc.Professor Department of Ethics Kazntu named after K.I. Satpayev

Radio wave propagation and antenna-feeder devices. The collection of lectures is recommended to the edition Methodical council SKSU of M.Auezov. The report № __ from __ _______ 202___.

ISBN ______________

M.Auezov South Kazakhstan university, 2021

Contents

Lecture 1 - The general data on telecommunication. A role and telecommunication place in a national economy. Communication as an information transmission medium. Examples of systems of an information transfer………………………….. …………….9

Lecture 2 - The information, messages, signals. Messages, their sources and addressees. A signal as the message carrier…………………………………………………...…….12

Lecture 3  - The message and the information. Casual character of messages and signals. Transfer of continuous messages. Modulation as operation of transformation of the message in a signal.	…………13

Lecture 4 - Key parametres of signals: duration, width of a spectrum and a dynamic range. Examples: speech (telephone), broadcasting, television,    cable signals,     data transmission signals	…………14

Lecture 5 - Classification of messages, signals and hindrances. The determined and casual processes, their mathematical models. Direct and indirect models of processes.	…………15

Lecture 6  - Functional spaces and their bases. Representation of messages and signals in various metric and topological spaces	…………18

Lecture 7  - Random processes (RP) and their basic characteristics. The stationary and non-stationary RP. Ergodic property of the stationary RP…………..	…………19

Lecture 8 - Features of non-stationary processes. Functions of correlation and their property. The Gauss RP. A spectrum of density of capacity  and its   communication with correlation function...	…………20

Lecture 9  - Function of correlation of "white" noise with the limited spectrum. Effective width of a spectrum.	…………21

###### Lecture 10  - Formation and detecting of amplitude modulation (AM)…...………...…23

Lecture 11 – Let’s consider amplitude modulated signals (AM)...	…………23

Lecture 12  - Principles of reception of the signal with AM	…………24

###### Lecture 13  - Detecting ……………………………………………….…...………...…27

Lecture 14 - Transformation of signals to linear and nonlinear communication channels. Transformation of the determined signals to the determined linear channels....	…………29

Lecture 15  - Transformation of power characteristics of the determined signals. Transformation of casual signals to the determined linear channels. Transformation of casual signals to the determined nonlinear channels.	…………30

###### Lecture 16  - Passage of signals through casual communication channels. Casual linear channels and their characteristics, features wire and radio channels, fading signals.…...…………………………………………………………………………..…30

Lecture 17  - Additive hindrances in the channel. Fluctuating, the concentrated and pulse hindrances, their likelihood characteristics. Quantum noise………...………...…30 Lecture 18  -  Models of continuous channels. The ideal channel without hindrances. The channel with additive гауссовым noise. The channel with an uncertain phase of a signal and additive Gaussian noise. The channel with an intersymbolical interference and additive noise…………………………………………………...............…………31

Lecture 19  - Models of discrete channels. Some models of discrete channels with memory. Model of discretely continuous channel…………………………..…………33

###### Lecture 20  - The general data on digital transmission of continuous messages. Problem statement about the optimum demodulator (receiver) of discrete messages. Criteria of quality and a rule of reception of discrete messages……………………………...……………………………….…...………...…36

Lecture 21 - Optimum algorithms of reception at completely known signals (coherent reception). The optimum receiver with the matched filter (MF). Synthesis of algorithms and schemes of optimum receivers (the correlation receiver, the matched filter)....	..…39

Lecture 22  - A noise stability of optimum coherent reception. A potential noise stability at precisely known set of signals. Probability of an error of reception for binary system of signals at Gaussian white noise (GWN).       ……...……………….…...………...…42

Lecture 23 - Reception of signals with an uncertain phase (not coherent reception). Optimum reception at an uncertain phase and amplitude of a signal……….....	..…45

Lecture 24 -  Processor Viterbi. Comparison of a potential noise stability of coherent and not coherent reception. ……...………………………..………….…...………...…48

Lecture 25 - A problem of support of as much as high fidelity of transfer of discrete messages in channels with hindrances……...……………….…...…………..……...…51

Lecture 26 -  Potential possibilities of discrete communication channels. Definition of a source of discrete messages, a discrete communication channel, coding and decoding. The basic conceptual device of the theory of the information. Theorems of coding Shannon for a discrete communication channel……...……………….…...………...…54

Lecture 27 -  Classification of methods of coding. Constructive methods of coding of sources of messages. ……............................................……………….…...………...…57

Lecture 28 -  Noiseproof (channel) coding, probability of an error of optimum decoding for codes with the fixed length of blocks(exhibitors of probabilities of errors) ...........................................................................……….............................................…60

Lecture 29 - Codes with the guaranteed detection and correction of errors. .……....…61

Lecture 30 -Code distance. Linear binary codes for detection and correction of errors.62

Introduction

The aim of studying the subject is achieving the following learning outcomes (on the basis of Bloom’s taxonomy): knowledge, comprehension, application, analysis, evaluation, synthesis.

Knowledge - the student should be able to reproduce received practical skills in the field of theory of transmission of electromagnetic waves. He needs to know the current state of the methods, trends and achievements in the field of hardware system design of theory of transmission of electromagnetic waves.

Application - the student must apply and use creative study of the laws, procedures, standard solutions and theoretical knowledge in specific practical situations in the development of devices; presumably describe future consequences arising from the available data and the methods used to develop. The student should be able to compile, convert, defend, paraphrase, interpret and give examples on the decisions adopted within the devices.

Analysis - the student should be able to identify the relationship between facts and consequences, evaluate the significance of data and results, compare and contrast the various methods of implementation of the theory of transmission of electromagnetic waves. The student must be able to assess the reproducibility of the significance and adequacy of the data obtained during the development of the theory of transmission of electromagnetic waves.

Evaluation - the student should able to evaluate logic of procedure of development of devices of a broadcasting, receiving results and the written explanation of rules and spheres of its functioning and boundaries of use; to estimate the significance of application of the developed model. To be able to evaluate, criticize, discuss, justify, disputing the received results.

Synthesis - the student should be able offers the plan of carrying out experiment and other actions in case of the solution of problems of systems and devices of satellite, mobile and optical communication. To be able to combine the experimental technique and processing of their results, possessing novelty and adequacy.

Tasks of studying the subject - According to the listed above purposes in result of study of discipline the student shall gain theoretical knowledge, practical skills in the development area of Systems and devices of satellite, mobile and optical communication. In the course of development of the discipline the student should learn the basics of systems of systems and devices of satellite, mobile and optical communication, master the basic functions of devices, structures, methods of system design, debugging and testing methods devices. Teaching the course " Technology of digital communication " must meet the current state of the discipline, to form a specialist firm foundations of knowledge, high culture and practical skills that are sufficient for the successful production activity and allows it to yourself to learn new necessary knowledge and achievements in the field of radio and solve engineering problems.

As a result of studying the discipline, students should be able to develop the structure of electronic circuits of various devices, organize, depending on the requirements of the task required the device structure, to be able to choose the right methods for solving problems the theory of transmission of electromagnetic waves.

Lecture1. The general data on telecommunication. A role and telecommunication place in a national economy. Communication as an information transmission medium. Examples of systems of an information transfer.

Communication systems play the increasing role in a life of people, uniting and pulling together the separate countries, continents and objects of space. Last years are noted not only intensive development of wire and optiko-fiber communication systems, but also appreciable development of systems of a radio communication. Besides traditional relay and satellite systems of a radio communication networks of mobile digital cellular systems of a radio communication quickly develop.

System engineering of communication of last time use not only possibilities of modern technologies, but also achievements of the modern theory of the communication, allowing to raise not only volumes of the transferred information, but also quality of message transfer (fidelity of communication).

The modern theory of communication uses both the determined models of signals, and likelihood models for the transferred messages corresponding to them of signals and hindrances (noise) in the channel. The likelihood approach considers casual (for the addressee) character of message transfer and hindrances in the channel and allows to define optimum intakes (providing the greatest possible quality) and limiting indicators of systems of message transfer (communication systems).

Concepts of the information and the message are used often enough. These near on sense of concept are difficult and to make their exact definition through more simple hardly. The word the information occurs from Latin informatio - an explanation, acquaintance, awareness. Usually as the information understand set of the data given about any events, the phenomena or subjects. We live in the information world. Everything, that we see, we hear, we remember, we know, we worry, - all is various forms of the information. Set of the data, given becomes knowledge only after their interpretation with the account of value and the maintenance of these data. Consequently, the information in a broad sense can be defined as set of knowledge of the world surrounding us. In such understanding the information is the major resource of scientific and technical and social and economic development of a society. Unlike material and power resources, the information resource does not decrease at consumption, collects in due course, is rather easy and simple by means of means is processed, stored and transferred to considerable distances.

For transfer or information storage use the various signs (symbols), allowing to express to (present) it in some form. Words and phrases in human speech, gestures and drawings, forms of fluctuations, mathematical signs can be these signs. The sum total of the signs displaying this or that information, name the message. So, by cable transfer by the message the text of the telegramme representing sequence of separate signs, letters and figures.

Message transfer (and consequently, and information) on distance is carried out by means of any material carrier (a paper, a magnetic tape etc.) or physical process (sound or electromagnetic waves, a current etc.). The physical process displaying (bearing) transferred message, is called as a signal.

As a signal it is possible to use any physical process changing according to the transferable message. In modern систмах managements and communications use electric signals more often. In the physical size defining such signal, the current or pressure is. Signals are formed by change of those or other parametres of the physical carrier according to the transferred message. It is accepted to name this process (change of parametres of the carrier) modulation.

If the signal represents function х (t), accepting only certain discrete values х (for example, 1 and 0) it name discrete or discrete on level (amplitude). If the signal (or the message) can accept any levels in some interval they are called as continuous or analogue.

On fig. 1.1 various kinds of signals are visually illustrated.

Fig. 1.1. Kinds of signals: (a) a continuous signal; (b) discrete signal on time; (c) a signal, quantized on level; (d) a digital signal

Casual character of messages, signals, and also hindrances has caused the major value of probability theory in construction of the theory of communication. As it will be shown in the subsequent heads, likelihood properties of signals and messages, and also environments in which the signal is transferred, allow to define quantity of the transferred information and its loss.

On fig. 1.2 the block diagramme of the elementary single-channel systems communications is represented. We will consider appointment of separate elements of this scheme. A various sort of the device (the automatic machine, the computer etc.) can be a source of messages and the addressee in one communication systems. The device, the reformative message in a signal, name transferring, and the device, reformative the accepted signal in the message, - to receptions. By means of the primary converter in a sending device the message and, which can have any physical nature (the image, sound fluctuation, etc.), will be transformed to a primary electric signal b (t).

In the transmitter the primary signal b (t) (usually low-frequency) turns to a secondary (high-frequency) signal u (t), suitable for transfer on the used channel. It is carried out by means of modulation.

Fig. 1.2. The Block diagramme of the elementary single-channel communication system

Lecture 2.3.4. The information, messages, signals. Messages, their sources and addressees. A signal as the message carrier. The message and the information. Casual character of messages and signals. Transfer of continuous messages. Modulation as operation of transformation of the message in a signal. Key parametres of signals: duration, width of a spectrum and a dynamic range. Examples: speech (telephone), broadcasting, television, cable signals, data transmission signals

As the communication line is called the physical environment and set of the hardware used for signalling from the transmitter to the receiver. In systems of electric communication is first of all a cable or a wave guide, in radio communication systems - space area in which electromagnetic waves from the transmitter to the receiver extend. By transfer the channel signal and (t) can be deformed and on it hindrances n (t) can be imposed. The Intake processes the accepted fluctuation z (t) = s (t) + n (t), representing the sum of the come deformed signal s (t) and hindrances n (t), and restores on it the message a which with some error displays the transferred message a.

Set of means for message transfer from a source to the consumer is called as a communication system. These means are the sending device, the communication line and the receiver.

As communication channel is called set of the means providing a signal transmission from some point A system to a point B (fig. 1.3).

Fig. 1.3. The communication channel Block diagramme

In a communication system presented on fig. 1.1, message transfer is carried out in one direction from a source to the addressee. Such mode of communication is called as simplex. The mode at which possibility of simultaneous message transfer in a direct and return direction is provided, is called as duplex. The half-duplex mode when the exchange of messages is carried out serially is possible also. The communication system is called as multichannel if it provides transfer of several messages on one general communication line.

1.3. HINDRANCES AND DISTORTIONS IN THE CHANNEL

In the real channel the signal by transfer is deformed also the message is reproduced with some error. The reason of such errors are as the distortions brought by the channel, and the hindrances influencing a signal. Frequency and time characteristics of the channel define so-called linear distortions.

As hindrance is called any casual influence on a signal which worsens fidelity of reproduction of transferred messages.

z (t) = s (t) + n (t), (1.1) hindrance is called as additive. If the operator can be presented in the form of product z (t) = k (t) u (f), (1.2) that a hindrance name multiplicate. Here k (t) - casual process.

1.4. CODING AND MODULATION

Transformation of the discrete message to a signal is usually carried out in the form of two operations - codings and modulations. Coding is represented by transformation of the message to sequence of code symbols, and modulation - transformation of these symbols to signals, suitable for transfer on the channel. By means of coding and modulation the source of messages sogla-is put with the channel.

At coding there is a process of transformation of elements of the message in numbers corresponding to them (code symbols). To each element of the message certain set of code symbols which is called as a code combination is appropriated. Set of the code combinations displaying discrete messages, forms a code.

Decoding consists in message restoration on accepted code symbols. The devices which are carrying out coding and decoding, name accordingly the coder and the decoder. As a rule, it is logic units. On fig. 1.5 the block diagramme of system of transfer of discrete messages is represented, and on fig. 1.6 process of transformation of the discrete message in a signal is explained. The transferred message is designated by the letter ак the coded message (or a primary digital signal) - bd (t), its components bl (i) (l - number of consistently transferred symbol, і-number of a position of a code, i=0, m-1). The signal arriving in the communication line is designated and (і), the accepted fluctuation - z (t), the restored sequence of code symbols - bd (t) and the decoded (restored) message - av.

In modern systems of transfer of discrete messages it is accepted to distinguish two groups concerning independent devices: codecs and modems. As the codec are called devices, reformative the message in a code (coder) and a code in the message (decoder), and the modem - devices, reformative a code in a signal (modulator) and a signal in a code (demodulator). Channel devices (strip amplifiers of the transmitter and the receiver, proof-readers, etc.) together with the communication line form the continuous channel, and last together with the modem - the discrete channel.

Fig. 1.5. The Block diagramme of system of transfer of discrete messages

Fig. 1.6. Process of transformation of the discrete message in a signal and a signal in the discrete message

Lecture 5. Classification of messages, signals and hindrances. The determined and casual processes, their mathematical models. Direct and indirect models of processes.

The general principle of modulation consists in change of one or several parametres of bearing fluctuation (переносника) f (t, α, β...) according to the transferred message. So if as a carrier harmonious fluctuation f (t)=Ucos((ω0t + φ) it is possible to form three

kinds of modulation is chosen: amplitude (AM), frequency (FM) and phase (PМ).

If a carrier is the periodic sequence of impulses  that at the set form of impulses v (t) it is possible to form four principal views of pulse modulation: pulse-amplitude modulation (PAM), time-pulse modulation (TPM), pulse-frequency modulation (PFM), pulse-duration modulation (PDM) or pulse-width modulation (PWM).

Duration of a parcel of a primary signal bu (t) by discrete transfer defines speed of transfer of parcels (technical speed or speed of modulation). This speed ν is expressed by number of parcels, передавасмых for a time unit. Technical speed in Бодах is measured. One Бод is a speed at which for 1 with one parcel is transferred. If duration of parcel T is expressed in seconds, speed of modulation ν = 1/T in Бодах. If a strip of frequencies to limit to the third harmonic, width of a spectrum of primary sig-cash F = l, 5 ν Hz.

Fig. 1.7. Forms of signals at a binary code for various kinds of discrete modulation

1.5. DEMODULATION AND DECODING

The transferred message in the receiver usually is restored in such sequence. At first a signal демодулируется. In systems of transfer of continuous messages as a result of demodulation the primary signal displaying the transferred message is restored. This signal then arrives on reproducing or a recorder. In broadcasting by such device there can be a loudspeaker or the tape recorder. In systems of transfer of discrete messages usually as a result of demodulation the sequence of elements of a signal turns to sequence of code symbols. Then on them the messages which are given out to the addressee are restored. Last transformation is called as decoding.

1.6. DIGITAL CODING OF CONTINUOUS MESSAGES

Now wide application is found by digital systems of transfer (ЦСП) in which continuous messages are transferred by discrete signals. Transformation of the continuous message to the digital form is carried out by digitization and quantization operations.

The error size ε = b (tk) - bкв (tk) does not surpass half of step of quantization ∆b and can be shown to admissible level. The error ε is stochastic function and is shown on an exit as additional noise (quantization noise), imposed on the transferred message. Sampling allows to transform continuous messages in discrete (in time) a signal which after quantization turns to the digital. Advantage of digital ways of transfer is possibility of application of codes, both for noise stability increase, and for reduction of redundancy of a source. Now the greatest application is found by system with pulse code modulation (ИКМ). In this system the continuous message at first is exposed to sampling and quantization on level, and then received sequence L of levels (figures) is coded (usually binary code). Thus to each level the code combination consisting from п of symbols 1 and 0 is appropriated. The received sequence of binary symbols is transferred on a communication channel to one of methods of discrete modulation. Modulation is usually used frequency (IKM-CHM) or phase (IKM-FM).

2.1. CLASSIFICATION OF MESSAGES, SIGNALS AND HINDRANCES

The mathematical models displaying the basic properties of messages, signals and hindrances from the point of view of the decision of problems of electric communication, are base ТЭС. As messages, signals and hindrances first of all are represented by the electric fluctuations varying in time their base mathematical model is some function (time process) x (t) which value varies in time. On the physical and mathematical properties processes x (t) share on determined and casual.

Or regular such processes х (t) which current in time can be predetermined completely in advance are called as determined. In other words, for any

beforehand set moment of time t value of function x (t) can be unequivocally defined. The determined functions are widely applied at studying of electric chains. So, at the analysis of transients in linear chains often use the determined test signals ν (t, ∆) and η (t, ∆) which forms are shown on fig. 2.1, and and 6, If parametre ∆→ 0, the signal ν (t, ∆) passes in individual function of inclusion

Fig. 2.1. Test signal (a), Rectangular impulse with the individual area (b), δ impulse

Lecture 6. Functional spaces and their bases. Representation of messages and signals in various metric and topological spaces. Representation of digital signals by vectors of space Hamming. Decomposition of signals in generalised Fourier series. Decomposition of functions in orthogonal numbers but to basic functions of space of signals. Decomposition of an analogue signal in basis Wolsh. The basic parities between elements of functional linear spaces. Generalized Fourier series. Spectral and time representation of signals. Digitization of signals in time. Contelnyks theorem

Transfer and information storage, i.e. information carrying over to space and time, is carried out by means of signals. In communication systems usually use an electric signal. In the physical size defining such signal, voltage (or a current), changing in time under the law displaying the transferred message is.

Signals can be classified on a number of signs. Signals share on discrete and continuous which, in turn, can be determined or casual. Also distinguish signals simple and difficult.

The signal is called as determined or regular if its mathematical representation is the set function of time. From the information point of view it designates, that the regular signal is understood as such signal which corresponds to the known message. Such signals do not carry the information.

Casual signal we will name a signal which mathematical description is time stochastic function. In systems of transfer of discrete messages each code symbol is transmitted by a piece of a signal of certain duration τ0. Such piece of a signal is called as an elementary (simple) signal. In the majority a case the elementary signal represents a parcel of a direct current or a piece of harmonious fluctuation. The elementary signal carries the elementary information, for example, "yes" or "is not present", "plus" or "minus". A signal representing set of elementary signals, name compound or difficult (fig. 1.1).

The concept of base of a signal - ν is entered into communication theories. It is defined as the doubled product of a strip of frequencies of a signal on its duration:

. (1)

For simple signals ν≈ 1, and for difficult ν »1. On this basis simple signals name often narrow-band, and difficult - broadband.

At last, it is possible to allocate reference or trial signals. Them concern:

harmonious signal  ; (2)

individual function (inclusion function)

(3)

an individual impulse (delta - function)

 . (4)

Lecture 7.8.9. Random processes (RP) and their basic characteristics. The stationary and non-stationary RP. Ergodic property of the stationary RP. Features of non-stationary processes. Functions of correlation and their property. The Gauss RP. A spectrum of density of capacity and its communication with correlation function. Function of correlation of "white" noise with the limited spectrum. Effective width of a spectrum.

Fig. 1.2. Realisations of casual process

t

t

t

t

t1

t1

t1

t1

ξ4

ξ3

ξ2

ξ1

In real conditions at message transfer the signal in a reception point is in advance unknown and consequently cannot be described certain function of time. The same it is possible to tell and about the hindrances which occurrence in the channel can be caused the most various and more often unknown persons for us the reasons. Thus, real signals and hindrances represent casual processes. Casual process is described by the stochastic function which values at any value of argument are random variables. For electric signals argument usually is time. Under invariable conditions of experience casual process ξ (t) can accept this or that concrete form ξk (t) (fig. 1.2). These possible forms of casual process are called as its realisations. Set of all possible realisations {ξk (t)} casual process ξ (t) is called as ensemble. Realisations ξk (t) already are not casual, but the determined functions.

If to consider not each realisation separately, and set of their big number it is possible to define likelihood characteristics of casual process. Such characteristics are laws of distribution which can be received theoretically or on the basis of experimental data. Let ξ (t) there is a casual process. During some fixed moment of time t1 various realisations of process will have various values ξ1 (t1), ξ2 (t1), …, ξn (t1) (fig. 1.2). Value ξ (t1) is a random variable.

Fig. 1.2. Realisations of casual process

One-dimensional function of distribution or the integrated law of distribution is defined as probability of that the random variable ξ (t1) will not exceed some value х1 . (1.5)

Private derivative  (1.6) is called as one-dimensional density of probability of casual process {ξk (t)} for t=t1.

The two-dimensional integrated law of distribution F2 (x1, t1; x2, t2) casual process {ξk (t)} is called probability of that during the moment t1 function ξ (t) will not exceed some value х1, and during the moment t2 - values х2, i.e.

. (1.7)

The two-dimensional density of probability is defined as a private derivative (if it exists) second order

. (1.8)

The fullest characteristic of casual process is n - the measured law of distribution, i.e. value distribution ξ (t) for n any way chosen moments of time. Function

. (1.9)

Is called n - as the measured integrated law of distribution, and private derivative  (1.10) is called n - as measured density of probability.

Casual process is set, if it n - measured distribution is known for any number n any way chosen moments of time t1, t2, …, tn.

Distribution laws are full enough characteristics of casual process. However they are difficult and demand for the definition of processing of the big experimental material. Besides, such detailed description of process not always happens it is necessary. The nobility suffices For the decision of many practical problems more simple (though also less full) the characteristic of casual process. Such characteristics are average value and function of correlation of casual process.

Average value, or population mean, casual process Х (t) is defined under the formula: , (1.11) where line means averaging on set. Average value defines itself some average function about which every possible realisations of stochastic function are grouped. Average value of square  is similarly defined. (1.12)

The dispersion of casual process is defined as a population mean of a square of a deviation of process from the average value

. (1.13)

The dispersion is a measure of disorder of values of stochastic function about average value. At дисперсия coincides with a population mean of a square of casual process: .

Average value and dispersion characterise behaviour of casual process during the separate moments of time.

Statistical communication between values of casual process X (t1) and X (t2) which it accepts during the moments of time and, is described by correlation function. This function is defined as a population mean of product of these values:

. (1.14)

Statistical communication of two casual processes X (t) and Y (t) during the moments t1 and t2 is described by function of mutual correlation

. (1.15)

If stochastic functions X (t) and Y (t) are statistically independent, the two-dimensional density of probability breaks up to product of two one-dimensional: , and function of mutual correlation .

For some processes average values and correlation function are exhaustive characteristics. The process having normal distribution of probabilities, in particular, concerns such processes.

In some cases correlation function (correlation factor) is entered нормированная

. (1.16)

The extensive class of practically important casual processes is made by stationary processes. Casual process is called stationary in a broad sense if its population mean does not depend on time, i.e. , and correlation function depends only on a difference t2 - t1 = τ:

. (1.17)

For stationary processes in the majority of practically important cases the theorem according to which on ensemble it is possible to replace averaging with averaging on time, i.e. with the probability as much as close to unit is fair ergodic, it is possible to consider:

. (1.18)

Property ergodicity has the big practical value. It allows to consider at research of statistical properties of process not set of its realisations, and only one realisation enough the big duration.

The major characteristic ergodic process is its correlation function. The basic properties of correlation function are:

autocorrelation function эргодического process is function even, i.e. Bx (τ) =Bx (-τ);

value of correlation function эргодического process at τ =0 coincides with average capacity of this process Bx (0) =x2 (t) = σ2x;

any value of correlation function cannot exceed value of this function at τ =0, i.e. Bx (0) ≥ Bx (τ);

нормированная correlation function does not surpass in unit module ρх (τ) ≤1;

if autocorrelation function of process satisfies to conditions:

,

That it means, that between values x (t) and x (t + τ) there is no communication. Such processes name purely casual;

if эргодический process does not contain the determined component its correlation function beyond all bounds decreases with increase τ, i.e. dependence between values x (t) and x (t + τ) weakens and in a limit (at τ→∞) they become independent. Presence of a constant component in process x (t) = ξ (t) +x0 leads to that

On fig. 1.3 the typical curve of correlation function ergodic the process, illustrating the listed properties of this function is resulted.

Lecture .10.

###### Formation and detecting of amplitude modulation (AM)

If the message х (t) can accept a number of discrete values carrier parameters at modulation will change jump. By means of modulation one of carrier parameters changes under the law defined by a code. By direct transfer by a carrier there can be a direct current, changing which parameters are the size and a direction. Usually as a carrier, as well as at continuous modulation, the alternating current (harmonious fluctuation) is used. In this case it is possible to receive amplitude modulation (АМ).

For signaling on the big distances, for carrying over of a spectrum of signals to the necessary range of frequencies and for increase in efficiency of use of communication channels apply modulation.

Change of one of parameters carrying according to the law transmitted messages is called as modulation.

High-frequency fluctuation which carries fluctuation, is called as carrying.

That parameter of a modulated signal in which change the transmitted message contains, is called as information.

Most often as the carrying use high-frequency harmonious fluctuation, in some cases use sequence of impulses (saw-tooth, rectangular, or trapezoidal).

In separate necessary cases use narrow-band random processes - noise-type signals or carrying.

###### Lecture 11.

LET'S CONSIDER AMPLITUDE -MODULATED SIGNALS (AM)

- Carrying  - a modulating signal.

As a result of modulation we will receive a signal:

(1) - formula AM of a signal at modulation by one tone with частотой

At т>1 there is a overmodulation, correctly to restore the message form in the reception end to become it is impossible.

Let's consider spectrum AM of a signal at modulation by one tone:

Spectral diagramme AM of a signal at modulation by one tone, and width of its spectrum is equal . The real message depending on the

communication system designation has a certain spectrum of frequencies.

At modulation by the difficult message:

SАМ (t) =U (1 + Mt) cost, where M - a resultant depth of modulation.

At modulation by the difficult message the width of spectrum AM of a signal will be equal to the doubled value of as much as possible modulating frequency:

Economy on capacity:

1. Cleans carrying after have received a signal, hence capacity of the carrying goes on capacity of lateral making (strips).

2. With one lateral strip (it is possible to leave a part carrying).

3. Instead of carrying it is possible to use пилосигнал.

###### Lecture 12.

PRINCIPLES OF RECEPTION OF THE SIGNAL WITH AM.

The amplitude modulator the single-cycle diode.

The module of resistance of the parallel contour which has been adjusted on frequency

If for frequencies ,   resistance of contour  and for other frequencies  then:

If amplitude carrying and amplitude of the modulating keep within on a site volt-ampere characteristic VAC which it is possible to approximate a polynomial 2ой degrees modulation occurs without distortion. If amplitude carrying and amplitude of the modulating do not keep within on a site VAC approximation is carried out by polynomial 3 of degree, there is a distortion.

Let's consider a principle of work of the balancing amplitude modulator (BМ) which consists of two single-cycle diode amplitude modulators (BМ).

1. On exit BМ is absent carrying and its harmonics ().

2. Is absent 2,

The further improvement of a spectrum of a signal can be received, using the scheme of the ring modulator.

The ring modulator (converter) represents 2 БМ, connected among themselves in such a way in which result its target voltage will be equal to the sum of target voltage of everyone БМ separately.

The ring converter (RC) - is widely applied in equipment of multichannel communication with frequency division of channels. RC it is possible to use as: 1. The modulator. 2. The detector. 3. The frequency converter.

RC - the detector, hence carrying remains, on an input moves , UАМ (t).

RC - the frequency converter: carrying over of a spectrum of a signal from one range of frequencies in another without change of parities between spectral making (spectrum transposing).

###### Lecture .13.

DETECTING

Detecting is a process, return modulations in which result from ВЧ the modulated fluctuation the law of change of information parameter is allocated.

ФНЧ

НЭ

ПЭ

Д

SАМ(t)		u(t)	Sмод(t) 					  u(t)

The basic characteristic of detectors is their detector characteristic. For the amplitude detector:

UΩ = I0 Rn

Uω

Let's consider detecting AM of a signal:

U(t)

i

SАМ(t)

Cн     Rн

U(t)

The basic condition of a choice of loading:

Lecture 14.15.16.17.

Transformation of signals to linear and nonlinear communication channels. Transformation of the determined signals to the determined linear channels. Transformation of power characteristics of the determined signals. Transformation of casual signals to the determined linear channels. Transformation of casual signals to the determined nonlinear channels. Passage of signals through casual communication channels. Casual linear channels and their characteristics, features wire and radio channels, fading signals. Additive hindrances in the channel. Fluctuating, the concentrated and pulse hindrances, their likelihood characteristics. Quantum noise.

1. LINEAR AND NONLINEAR MODELS OF COMMUNICATION CHANNELS

For construction of the theory of electric communication it is necessary to have first of all the mathematical model of the channel describing the real channel with the necessary degree of approach. Such model (if not to be interested in internal processes in system) is reduced to the task of mathematical model of signals for an input and a channel exit (or electric chains forming it) and communications between them. We will consider only purely time channels, setting communication of signals (generally multidimensional, vector) on an input x(t) and an exit y(t) (the last name also the response or system reaction) the system operator

y(t)=L{x(t)}

Completely to define a problem solved in the theory of communication, it is necessary to set area Vx of some functional space which is called as area of admissible entrance influences. The instructions of this area describe character of entrance signals which can be continuous, discrete, digital determined or casual. Area Vy of admissible target signals should be similarly defined. In the present chapter systems with analogue signals (continuous on levels and time) on an input and an exit (continuous channels) are considered mainly.

Mathematical model of system (channel) name set of system operator L and areas of admissible signals Vx and Vy. Classification of systems (channels) can be spent on the basis of characteristic properties of their mathematical models. It is said that the system stationary if its response to an exit does not depend on during what moment of time arrives an entrance signal. Stationary systems name systems with constants in time in parametres. If properties of system depend on during what moment of time the entrance signal arrives, system name non-stationary (system with variables in time in parametres or parametrical system).

2. TRANSFORMATION OF THE DETERMINED SIGNALS TO THE DETERMINED LINEAR CHANNELS

Using filtering property δ-functions, we will write down        (4.3)

Expression (4.3) can be considered as the dynamic model of a signal showing a course of its development from - ∞ with to a point t.

As δ (t - τ) = 0 at τ> t (4.3) it is possible to write down as  (4.4)

Let's enter into consideration pulse characteristic (PH) g (t) the linear stationary channel as its response at the moment of time t on δ a-impulse submitted during the moment of time 0. Then the response of such channel to elementary influence  is equal , and the response to a signal (4.3) according to a superposition principle

t in integral it is possible to replace the top variable on ∞, as from a condition of a realizability of system (the system response cannot appear before influence) g (t - τ) = 0, at t - τ <0 or τ> t

The formula (4.5) has evident physical sense: the linear stationary system carries out operation of the weighed summation of all its instant values existing in the past at ∞ <τ <t over an entrance signal. The role of weight function plays PH.

3. TRANSFORMATIONS OF POWER CHARACTERISTICS OF THE DETERMINED SIGNALS

Let's consider transformations of power characteristics of the determined signals of duration Т to linear stationary systems (channels). Considering definition СПМ (spectral density of power), it is possible to express communication between these characteristics on an exit and an input of the determined linear stationary channel:

Average powers of signals on an input and a system exit are defined by parities:

CF (correlation function) Bx (τ) a signal x (t) and it SDP Gt (f) are connected by pair transformations Fourier (Фурье). If to enter into consideration CF for PH system (channel):  that, using generalised formula Rayleigh (Рэлея) and a parity (4.38), it is possible to receive

CF the system characteristic g (f) and factor of transfer of system on power of K2(t) are connected by pair transformations Fourier. Using a spectral parity (4.38), it is possible to assert, that CF a target signal y (t) it is defined by convolution CF of an entrance signal x (t) and CF the pulse characteristic of system

Control questions?

1. Whether the linear stationary channel has pulse characteristic Обладает the channel stability?

Lecture 18

Models of continuous channels. The ideal channel without hindrances. The channel with additive гауссовым noise. The channel with an uncertain phase of a signal and additive Gaussian noise. The channel with an intersymbolical interference and additive noise.

4.4. MODELS OF CONTINUOUS COMMUNICATION CHANNELS

To give the mathematical description of the channel, it is necessary and to specify enough set of signals which can be submitted on its input, and for any admissible entrance signal to set casual process (signal) on its exit. To set casual process it is possible in this or that form of its distribution of probabilities. So, in the continuous channel it is necessary to set aprioristic density (multidimensional) w [u] entrance process U (t) on an interval of analysis Тa and multidimensional transitive density w [z/u], i.e. density of realization of accepted casual fluctuation Z (t) (a signal + noise) under condition of transfer of realization U (t).

The exact mathematical description of any real channel usually rather difficult. Instead use the simplified mathematical models which allow to reveal all major laws of the real channel if at model construction the most essential features of the channel are considered and the minor details a little influencing a course of communication are rejected.

Let's consider the most simple and widely used mathematical models of channels, having begun with continuous channels as they in many respects predetermine also character of discrete channels.

4.4.2. THE CHANNEL WITH ADDITIVE GAUSSIAN NOISE

Signal on an exit of such channel

Where N (t) – Gaussian additive noise with a zero population mean and the set correlation function. To a bowl of all it is considered white Gaussian noise (БГШ) or quasiwhite (квазибелый) (with uniform spectral density in a strip of a spectrum of a signal s (t)). Often at the analysis it is possible τ not to consider, that corresponds to change of a reference mark of time on a channel exit. Some complication of model (4.48) turns out, if transfer factors γ and delays τ to consider as known functions of time:

Such model well describes many wire channels, radio channels at communication within direct visibility, and also radio channels with the slow general замираниями at which it is possible to predict reliably values γ and τ.

4.4.3. THE CHANNEL WITH THE UNCERTAIN PHASE OF THE SIGNAL AND ADDITIVE NOISE

This model differs from model (4.48) that in it delay is a random variable. For narrow-band signals with the account (4.48) at constant γ and casual τ it is possible to present (4.44) expression in a kind

Where  - Hilbert's transformation from u (t); θ=-ω0 τ a casual phase. Distribution of probabilities G is supposed set, more often uniform on an interval from 0 to 2 π. This model well describes the same channels, as previous if the signal phase in them fluctuates. Such fluctuation is caused by little changes of extent of the channel, properties of environment in which there passes a signal, and also phase instability of basic generators.

One-beam Gaussian the channel with the general fading (fluctuations of amplitudes and signal phases) also is described by the formula (4.48), but a multiplier γ, as well as a phase θ, are considered as casual processes. Differently, casual will be quadrature components

4.4.4. THE CHANNEL WITH THE INTERSYMBOLICAL INTERFERENCE (МСИ) AND ADDITIVE NOISE

This model is a special case when G (t, τ) from t does not depend (or varies very slowly) so dispersion on frequency practically is not observed. The intersymbolical interference is caused by signal dispersion in time at its passage on a communication channel. In particular, on an exit of the multibeam channel the useful signal appears deformed so, that simultaneously there are responses of the channel to the pieces of the entrance signal concerning by remote enough moments of time. By transfer of discrete messages it leads to that at reception of one symbol on an intake input responses to earlier (and sometimes influence also and later) symbols, which in these cases can (at not optimum methods of reception) to prove as a hindrance.

The intersymbolical interference is caused by nonlinearity PFC of the channel and limitation of its pass-band. In radio channels reason МСИ is multibeam distribution of radio-waves) more often.

Let the transmitter transfers synchronously with clock interval Т sequence of the elementary signals corresponding to a chain of symbols b-Q, b - (Q-1)..., b, u bot bl..., bD-1, bD, and each of sequence symbols

Choose from a possible set for the given code 0, 1..., m - 1 (m - the code basis).

CONCLUSIONS

1. Communication channels and electric chains realizing them can be divided on character of the signals operating on an input and an exit, on: continuous, discrete (digital) and is discrete-continuous (digital -continuous) or is continuous-discrete (is continuous-digital).

2. Classification of channels (systems, chains) is based on properties of the system operators connecting an input and an exit. Distinguish the linear and nonlinear, stationary and non-stationary, concentrated systems and systems with the distributed parameters.

3. As integrated system characteristics of linear chains use the pulse characteristic and transfer function the systems connected in pair of transformations Fourier more often.

4. The spectral density on Fourier or Laplace a signal on an exit of linear stationary system is defined by product of spectral density of an input and transfer function of system. The target signal can be found on its spectral density return transformation Фурье or Лапласа (a spectral method of the analysis). It can be found convolution of an entrance signal and the pulse characteristic of system (a time method of the analysis).

5. Calculation of passage of narrow-band signals through narrow-band systems (channels) essentially becomes simpler if to take advantage of concepts complex bending around an entrance signal and a low-frequency equivalent of transfer function of system.

QUESTIONS

1. To what signs it is possible to classify communication channels?

2. What channel is called as continuous, discrete and is discrete-continuous?

3. How pulse and transfer characteristics of linear system are connected among themselves?

Lecture 19.

Models of discrete channels. Some models of discrete channels with memory. Model of discretely continuous channel.

4.5. MODELS OF DISCRETE COMMUNICATION CHANNELS

It is useful to remind, that in the discrete channel the continuous channel always contains. Transformation of the continuous channel to the discrete carries out the modem. Therefore basically it is possible to deduce mathematical model of the discrete channel from models of the continuous channel at the set modem. Such approach often is fruitful, however it leads to difficult models.

The constant symmetric channel without memory is defined as the discrete channel in which each transferred code symbol can be accepted wrongly with the fixed probability р and is correct with probability 1-р, and in case of an error instead of the transferred symbol bi any other symbol can be accepted with equal probability. Thus, the probability of that the symbol bj is accepted if has been transferred bi

The term "without memory" means, that the probability of erroneous reception of a symbol does not depend on background, i.e. from what symbols were transferred to it and as they have been accepted. Further, for reduction, instead of "probability of erroneous reception of a symbol" we will speak "probability of an error".

The constant symmetric channel without memory with deleting differs from previous what the alphabet on a channel exit contains additional (m+1) a symbol often designated by a sign "?". This symbol appears when the first solving scheme (demodulator) cannot reliably identify the transferred symbol. The probability of such refusal of the decision or symbol deleting рс in the given model is constant and does not depend on a transferred symbol. For the account of introduction of deleting it is possible to lower considerably probability of an error, sometimes it even consider equal to zero. On fig. 4.4 probabilities of transitions in such model are schematically shown.

Fig. 4.4. Transitive probabilities in the binary symmetric channel with deleting

4.5.1. SOME MODELS OF DISCRETE CHANNELS WITH MEMORY

If without memory the conditional probability of erroneous reception (i+r) a symbol provided that i a symbol is accepted wrongly, equal in the constant symmetric channel to unconditional probability of an error in the channel with memory it can be more or less than this size.

The deviation of distribution of errors from binomial (the channel without memory) in real channels is caused by the various reasons. So, discrete display of the majority of radio channels is the channel with memory owing to замираний which we have considered above. Other reason atmospheric and mutual hindrances can be. The elementary model of the binary channel with memory is Markovian, defined a matrix of transitive probabilities

Where P1 - conditional probability to accept (i+1) a symbol wrongly if the previous is accepted correctly; 1-P1 - conditional probability to accept (i+1) a symbol correctly if the previous is accepted correctly; Р2 - conditional probability to accept (i+1) a symbol wrongly if the previous is accepted wrongly; 1-P2 - conditional probability to accept (i+1) a symbol correctly if the previous is accepted wrongly.

4.5.2. MODEL OF THE IS DISCRETE-CONTINUOUS CHANNEL

The is discrete-continuous channel with independent symbols bi on an input and a continuous signal z (t) on an exit is described by aprioristic probabilities of entrance symbols P (bi) and transitive (conditional) density w [z|bi] accepted realization z (t) (on set interval T) under condition of symbol transfer bi. This density name credibility function (see гл. 5). Instead of credibility functions the is discrete-continuous channel can be described posteriori probabilities P (bi|z) symbol transfers bi, at fixing on fluctuation reception z (t). According to formula Bayes

Where density of accepted fluctuation

Control questions?

1. The two-beam channel has pulse characteristic Найти the module of its transfer function (АЧХканала). On what frequencies АЧХ has zero values at γ1 = γ2 = γ?

2. Values density of function of credibility.

Lecture 20.

The general data on digital transmission of continuous messages. Problem statement about the optimum demodulator (receiver) of discrete messages. Criteria of quality and a rule of reception of discrete messages. Optimum reception in the is discrete-continuous channel without distortions in the presence of additive white noise. Criterion of a maximum of average probability of correct reception. The solving scheme constructed by a rule of maximum a posteriori of probability. The credibility relation.

5.1. PROBLEMS OF SYNTHESIS OF OPTIMUM DEMODULATORS

Systems and methods of transfer of discrete messages take the basic place in modern systems of electric communication that speaks not only presence of set of sources of discrete messages (data from the COMPUTER exit, cable sources, etc.), but also message transfer of continuous sources (speech, music, TV, facsimile transfer, etc.) more noiseproof discrete digital methods.

On a modulator exit in изохронных transfer systems there is a transformation of a digital signal bd (f) t considered in § 3.6, in sequence u (t) elementary signals carrying f (t-kT, bk) the set form:

Where Т - a clock interval of transfer in the channel, defining technical speed of transfer V = 1/T, measured in baud; bк → B - set of elements of a code.

5.2. CRITERIA OF QUALITY AND THE RULE OF RECEPTION OF DISCRETE MESSAGES

Let's consider at first widespread criterion Котельникова or criterion of the ideal observer according to which quality of the demodulator estimate unconditional (average) probability of correct reception of a symbol. We will believe at first, that the space of transferred and accepted signals is конечномерным евклидовым. It can be, for example, space финитных the signals represented by the final trigonometrical sum. Further this restriction will be removed.

In n-dimensional space the casual signal z (t) is characterized by n-dimensional density of probabilities of a vector z: w (z). It can be considered as density of probability of factors of decomposition z (t) on any orthonormalized to basis. If some symbol bi is transferred i.e. the signal si (t) it is possible to define conditional n-dimensional density of probability w (z|bj) = w (z|si) - credibility function i-й hypotheses (i=0, m-l) is accepted.

Let on an input of the demodulator during a piece [0, T] some element of a signal z (t) comes. We will assume, that the demodulator makes thus the decision that the symbol bi is transferred, i.e. gives out an estimation bi. Probability of that this decision is correct, obvious, is equal to conditional probability P (bi|z (t)) that the symbol bi under condition of arrival of realisation of an element of a signal z (t) was really transferred. It name posteriori probability of a symbol bi, (i.e. the probability defined after experience, consisting in supervision and the signal analysis z (t)).

THE SOLVING SCHEME CONSTRUCTED BY THE RULE OF MAXIMUM a POSTERIORI OF PROBABILITY. THE CREDIBILITY RELATION.

It is obvious, that the probability of correct reception will be

The sum reaches a maximum, if for each concrete realization z (t)

- It is maximum.

Here P (bi|z) posteriori probability of transfer bi, a P (bi|z) in aggregate form solving rule of the demodulator. As

, the best solving rule will be such, at which P (bi|z) = 1 for i corresponding maximum P (bi|z).

For construction of the solving scheme by a rule (5.10) it is necessary to know aprioristic probabilities of symbols P (bi), also properties of the modulator and the channel, defining conditional density w (z|bi) - credibility functions.

The rule (5.10) can be written down differently. The decision that the symbol bi was transferred, should be accepted, if for all j≠i are carried out m-1 inequalities

The relation in the left part of this inequality is called as the relation of credibility of two hypotheses: that the symbol bi and that the symbol bj was transferred was told. It designate .

In a case when all т symbols are transferred equiprobably, i.e. P (bi) = 1/т, the rule (5.12) becomes simpler:

Sometimes enter into consideration besides т hypotheses about transfer of symbols  still an additional ("noise") hypothesis that no signal was transferred, i.e. z (t) = n (t) - a pure hindrance. The relation of credibility обычно designate simply . Then the rule (5.13) can be written down so:

Or is shorter: . Such rule of a maximum of credibility realises criterion of the ideal observer provided that all symbols are transferred equiprobably.

For binary system the rule (5.14) is reduced to inequality check

As it was marked, the criterion of the ideal observer is not unique reasonable criterion of an optimality of the solving scheme. The matter is that in many cases various errors lead to various consequences. For example, in system of the automatic fire alarm system it is more dangerous not to find out a signal about a fire, rather than to declare "false alarm" when actually the fire is not present.

Control questions?

1. To find the pulse characteristic of an integrating chain (dynamic system of the first order) a spectral method. At calculation of return transformation Фурье from spectral density of the response of system to take advantage of the theorem of deductions, believing about a complex variable (ω =a+jb) and closing a contour of radius R in the top semiplane.

2. Concept a posteriori probabilities

Lecture 21. Optimum algorithms of reception at completely known signals (coherent reception). The optimum receiver with the matched filter (MF). Synthesis of algorithms and schemes of optimum receivers (the correlation receiver, the matched filter).

Let's assume, that all distortions in the channel are strictly determined also casual is only gaussian additive noise n (t) which is believed in the beginning white, with spectral density N0, It means, that by symbol transfer bi (i = 0, 1..., m-1) accepted fluctuation can be described model (4.48):  (5.21). Where все are known. Realisation of a hindrance and a position (an index i) really transferred signal which the solving scheme should define is not known only.

Let's consider also, that all si - are finite the signals, which duration T. It takes place, if transferred signals ui (t) finite and have identical duration (system synchronous), and in the channel are not present either multibeam distribution, or the linear distortions causing increase of duration of a signal (or they are corrected).

(5.25). Where  - energy of an expected signal si (t). Expression (5.25) defines those operations (algorithm of reception) which the optimum receiver over entrance fluctuation should make. For binary system the algorithm (5.25) is reduced to check of one inequality. At performance of inequality  (5.26) the symbol "1", otherwise "0" is registered. The device directly calculating scalar product (or correlation integral).  (5.27) name the active filter or the correlator, therefore the receiver realising algorithm (5.25), name correlation. On fig. 5.2 the block diagramme of the intake working according to (5.26) is shown. Here blocks "х" - remultipliers; Г0, Г1 - generators of basic signals s0 (t), s1 (t); ∫ - integrators; " - "- subtracting devices; SD - the solving device defining during the moments of time, multiple Т (at key short circuit), number i - branches with the maximum signal.

(i=0,1). At m> 2 in the scheme fig. 5.2 and other mentioned below schemes grows accordingly number of branches of processing of the signal, getting on SD.

Scalar product (5.27) can be calculated not only by means of  the active filter (correlator) described in the previous paragraph, but also by means of a passive linear filter with constant parametres. If on a filter input to give an accepted signal z(t), voltage on an exit of the filter at the moment of time  where g(τ) - the pulse characteristic of the filter. We will choose its such that in moment t=T to receive

value at (T), equal to scalar product (5.27). It is easy to see, that it will be executed at the following coordination g(T-τ) =si(τ) or  (5.31). In more general case the

matched filter for a signal s (t) name the linear passive filter with constant parametres and them  (5.32), where a, t0 - constants. Function g (t) is mirror display s (t) concerning an axis lead through a point t0/2 (fig. 5.7).

According to (5.31) at the moment of time T voltage on an exit of the matched filter to proportionally signal on an exit of the integrator of the active filter in the scheme fig. 5.2. Therefore the demodulator realising algorithm (5.26), can be executed and on the basis of the matched filters. The block diagramme of such demodulator for binary system is shown on fig. 5.8, where MFi, - the filter matched (MF) with a signal si (t).

Really, by means of the scheme shown on fig. 5.9, it is possible to synthesise with the set accuracy any signal s (t), presented by truncated number Котельникова (2.61):  where ак=s (k ∆); ∆ =1 / (2F); F - width of a spectrum of a signal. As follows from in § 2.4, such signal can be received on an exit ideal LFF (low-frequency filter) with pass-band F, submitting on its input sequence δ-impulses with scales а'к=ак ∆ through time intervals ∆.

It is carried out with known approach in the scheme fig. 5.9. If on a line input during the initial moment one short impulse approximating δ-function from taps the same impulses carried on intervals ∆, which act in film moves, having passed through

weighing blocks ак, arrive serially on input LFF (low-frequency filter). Weighing blocks contain attenuators or amplifiers with strengthening factor [a'k], an also at negative a'k - inverters. The scheme fig. 5.9 represents a linear filter named transversal, with pulse reaction s (t). It is easy to see, that if an entrance impulse a tax not in a point And, and in a point In the signal representing mirror display s (t) will be synthesised. Therefore the same scheme with an input in a point In appears the matched filter with s (t).

Control questions?

1. What mainframes are contained by the correlation scheme of optimum coherent reception in the channel with additive stationary Gaussian white noise?

2. What understand as the matched filter? During what moment of time on an exit of Council of Federation the maximum relation a signal-noise and to that it is equal is provided?

Lecture 22. A noise stability of optimum coherent reception. A potential noise stability at precisely known set of signals. Probability of an error of reception for binary system of signals at Gaussian white noise (GWN).

Let's define a potential noise stability for binary system with additive GWN in the channel when at reception both expected signals are precisely known: s1 (t) and s0 (t), believing, that aprioristic probabilities of these signals are identical. The coming signal z(t) is casual as, first, realisation of a transferred signal in advance is not known, secondly, it contains casual hindrance N(t).

In this case according to (5.26) algorithm of optimum reception  (5.44)

At performance of an inequality (5.44) optimum receiver registers a symbol 1 corresponding to a signal s1 (t), otherwise - a symbol 0 corresponding to a signal s0(t). If the symbol 1, z(t) =s1 (t) +N (t) is really transferred. Thus the probability of an error is defined by probability of that the inequality (5.44) is not executed, i.e. probability of performance of return inequality  (5.45) At the set intensity of hindrance No the potential noise stability of binary system depends only on so-called equivalent energy of signals  (5.50) which is equal to a distance square between alarm points in Hilbert's space. A noise stability above (the probability of an error is less) at that system, at which more equivalent energy of used signals, irrespective of the form of used signals. The last, in particular, can be as simple (pieces of a sinusoid with small base), and difficult (noise-type, with the big base).

The parity (5.50) allows to carry out an optimum choice of signals s1 (t) and s0 (t) or accordingly u1 (t) and u0 (t), providing the greatest possible noise stability at the set energy of signals Е. Really, for such optimum system size Еэ should be maximum provided that  (5.51).

It is possible to write . For reception of a maximum of this expression it is necessary to make Е1 and E0 probably big, and integral in the right part - as it is possible smaller. Greatest possible values Е1 and Е0 will turn out, if, considering conditions (5.51) to put Е1=E0=Е (5.52).

Integral  accepts only non-negative values, therefore its minimum is equal to zero and is reached under a condition s1 (t) =-s0 (t) which does not contradict a condition (5.52). Thus, in the binary channel with constant parametres and additive GWN optimum there is a system with opposite signals. To this condition satisfy, for example, two-polar impulses, signals of binary phase modulation (PM), if a difference of phases of signals ∆φ = π, etc.

For all such systems Ee=4E and probability of error  (5.53) where N2=E/N0 the relation of energy of a signal on a demodulator input to spectral density of power fluctuating hindrances.

In binary system with a passive pause, believing s0 (t) =0 and  it is received for the minimum probability of error  (5.55). From here it is visible, that at transition from system AM to system FM the power prize on the maximum power is equal 2, and at transition to system FM is equal 4. If message comparison not on peak, and on average power transition from AM to FM does not give a power prize as at FM average power is equal maximum, and at AM - twice less maximum (if si and s0 are transferred with identical probability).

The effective method of elimination of this phenomenon has been found by transition to the relative methods of modulation offered by N.T.Petrovich. They are reduced to modulation of information parametre of a transferred parcel of an element of a signal concerning the same parametre of a previous parcel. At relative phase manipulation (RPM) the message contains not in absolute value of a phase of an element of a signal, and in differences of phases of two next elements, thus a symbol 1 the symbol 0 realisation with a return phase, or on the contrary is transferred by repetition of that realisation of a signal which took place as the previous element, and.

Let's define probability of an error in system OFM at the account fluctuating hindrances in the channel at coherent reception. It is obvious, that erroneous registration of a symbol at reception by a comparison method polarity is possible as a result of one of two not joint events:

The sign on the given element is accepted wrongly, and a sign previous - is true; the sign on the given element is accepted truly, and a sign previous - it is erroneous.

For orthogonal system of signals with equal energy (the system appears also equidistance - all alarm points are on identical distance from each other) probability of an error pm (identical by transfer of any symbol) it is expressed by the following integral:  (5.57)

In the determined not deforming channel with GWN optimum (an error providing the minimum probability at preset value А2) it appears equidistance the system which alarm points form a multidimensional simplex (they lay on identical distance from each other in (m-l) measured hypersphere). For this optimum system of signals it is possible to define probability of an error also in the field of small errors the formula (5.57).

Control questions?

1. To find transfer function of the filter matched with a rectangular video impulse and on its basis to construct the filter block diagramme.

2. The filter is matched with a pack from N periodically transferred (with period T) impulses. The first impulse has spectral density S1 (j ω). To find transfer function of Council of Federation and on its basis to construct the filter block diagramme.

Lecture 23. Reception of signals with an uncertain phase (not coherent reception). Optimum reception at an uncertain phase and amplitude of a signal

If the phase of an accepted signal fluctuates so slowly, that by its measurement it is possible to predict reliably enough, optimum reception basically is realised the same as at precisely known signal. Such situation is characteristic for many channels wire and radio communications are more rare. However quite often the phase fluctuates quickly enough, and its exact estimation to receive it is not possible. Besides, the estimation of a phase demands sometimes application of difficult devices. Therefore even when it is essentially possible to estimate an initial phase of a coming signal, at times it refuse and use the algorithm constructed in the assumption, that the initial phase of a coming signal is not known and can accept any value on an interval (0,2π). Such method of reception is called as not coherent.

For binary system of signals the rule of optimum not coherent reception is expressed by inequality  (5.71), at performance of this inequality is registered 1, otherwise 0.

N0 - Unilateral spectral density of power quasiwhite and white noise

- The relation of energy of an element of a signal on a demodulator input to unilateral spectral density of power of white noise;  - modified function Bessel of a zero order.

,  where z (t) - realisation of the sum of a signal and an additive hindrance on an input of the receiver (detector); ui (t) - realisation of a casual signal on a modulator exit; γ - channel broadcast factor; θ - phase shift, current dimensionless time.

Sizes  can be received at the moment of readout Т on an exit of the active filter with the basic signals equal accordingly . With the account told clearly construction on the basis of active filters of the scheme named quadrature and realising algorithm (5.71) (fig. 5.15). Here Г0, Г1 - accordingly generators of basic signals u0 (t), u1 (t);

φ-π/2 - the phase shifter of all alarm a component on - π/2 (Hilbert's converter); БОМ - the block of definition of the module of vector on orthogonal components; НУ - nonlinear inertialess devices with characteristic

The algorithm (5.71) can be realised and on the basis of the agreed filters, as is shown in fig. 5.16. The ideal detector allocates bending around voltage on an exit of the agreed filter. For binary system with a passive pause, believing, that the symbol 0 is transferred by a signal u0 (t) = 0 and it is possible to write down in the form of V1> λ, where

threshold level , and function х=f(y) inverse functions у=lnI0 (. At performance of inequality V1>λ (excess V1 over a threshold) is registered a symbol 1, otherwise a symbol 0.

At a conclusion of a rule of the decision (5.71) it was supposed, that the casual initial phase is distributed in regular intervals on an interval (0,2π). However in some cases distribution of an initial phase non-uniformly, and is even more often this distribution at communication system construction is not known. In these conditions two approaches are possible: construction adaptive quasicoherent systems in which by the analysis of an accepted signal the approached estimation of a phase used instead of missing aprioristic data (the aprioristic information) is defined; decision-making in the assumption, that the initial phase represents some unknown parameter which the same as and a transferred symbol, it is possible to estimate on a credibility maximum.

Let's notice, that one of actual problems of the theory of communication is search of algorithms of the decision for the demodulator, applicable at the insufficient aprioristic information on the channel and on a message source, for example about aprioristic probabilities of various signals, about distributions of probabilities of amplitudes and phases, about some parameters entering into the description of a signal.

Control questions?

1. What explain practical advantages of communication systems with identical energy of signals at coherent and not coherent reception?

2. At optimum coherent reception of binary signals by the correlation scheme generators of basic signals have admitted phase detune Δφ. To find size detune at which power loss in relation to the scheme with ideal phase coherent does not exceed 1,1 (increase of power of the transmitter makes 10 %).

3. Find power loss of binary system, orthogonal in the strengthened sense, in the channel with Rayleigh fading in relation to the channel without fading at probability of an error р=103.

4. In what separation you know sense of the carried reception of signals and what kinds?

Lecture 24. Processor Viterbi. Comparison of a potential noise stability of coherent and not coherent reception.

The considerable quantity of algorithms of reception is developed for channels from an intersymbolical interference (МСИ). Many of them are based on idea of measurement of the characteristic of the linear channel g(t,τ) with the subsequent correction of the channel. As it is necessary to make it in the conditions of varying properties of the channel such proof-readers name adaptive. Even if the satisfactory estimation of the system characteristic of channel  (where - g(t, τ) - the determined pulse characteristic of a linear chain is received; K(jω) - the frequency characteristic), adaptive proof-readers not always qualitatively work in multibeam channels with selective fading, points characterized by presence on an axis of frequencies with zero values of the module of transfer function. The linear proof-reader (equalizer), aspiring to add the frequency characteristic of the channel to ideal, gives a factor "infinity" that leads to characteristic splashes in noise on these frequencies. Therefore we will consider the approach to construction of the optimum demodulator for channels with МСИ.

In the early seventies D.Forni has suggested to use for demodulation of signals in channels with МСИ algorithm Viterbi (АВ) which provides about the same noise

stability, as algorithm Klovsky-Nikolaeva (АКН). Difference АВ from АКН consists in quantity of readout of the entrance signal used for removal of the decision about b0(i) and, as consequence, in a delay of the decision. It is visually illustrated by means of the directed count fig.5.14 constructed for an on-off code (m=2) and memory of the channel, stretched on 2 elements of a signal (Q=2).

On fig. 5.14 the trajectories (way) displaying formation сигналов by transfer of sequence of binary symbols (chain) b0, bi are shown. Transfer 1 is displayed by movement in the corresponding knot (it is designated by a point) lattices downwards, and transfer of a symbol 0 - movement upwards. Lattice edges have duration of clock interval T. We will consider, that during the moment t=0 symbol transfer b0 begins. Knots A, B, C, D in section 2T, knots А1, В1, С1, D1 in section 3T etc. define separate conditions of the channel with memory. Their number is equal in our example L=mQ=22=4.

The concept a channel condition is directly connected with МСИ and characterizes its concrete display. If are fixed Q the elements previous transfer  of an element bk it means, that by the moment of the beginning of the response of the channel on bк the channel is in one of L=mQ conditions (on number of possible realisations of chains on Q clock  intervals). At the known system characteristic of the channel (the pulse characteristic or transfer function of the channel) it means also, that the support (signal ) on which the response on bк starts to develop is known. After the lapse of one clock interval Т the message element bk-Q ceases to influence a course of analyzed process z(t), but the new element bk starts to influence. The number of conditions of channel L=mQ thus remains.

According to АВ in section 3T it is considered, that in each condition (А1, В1, С1, D1) conduct m the trajectories defined by various symbols, bк (i). One of them has the minimum metrics. It also should be left ("survives") for the further choice of decisions (estimations). The others (m-1) trajectories are rejected. On fig. 5.14 rejected trajectories are noted on an interval (2Т, 3T) by a shaped line. (We will notice, that in section 3T points 1 and 1 ' mean the same knot ("condition") if not to consider an element b0, the same it is possible to tell about points 2-2 ', 3-3 ', 4-4 ').

In section (n-1) Т again from m the trajectories conducting in a new condition (Аn-Q-1, Вn-Q-1, Сn-Q-1, Dn-Q-1) leave one, having the greatest weight or minimum metrics . The survived trajectories can differ certain symbols b0. Therefore process of search of the best decision in АВ proceeds until on some step (on fig. 5.14 in section nТ) the escaped trajectories will not merge on a symbol b0 (conditionally it is displayed by an arrangement of knots (Аn-Q, Вn-Q, Сn-Q, Dn-Q) above a shaped line. On this step the decision (on fig. 5.14 b0=0) also is defined. Similarly process of demodulation on АВ proceeds and further: on some step trajectories on a symbol b1 (on it the decision is made) etc. Generally АВ will merge is characterized by a variable delay of the decision and various depth of decision-making for various elements of a signal.

Objective comparison of various systems of transfer of discrete messages at optimum reception and the set model of the continuous channel probably if to take advantage of concept of equivalent probability of an error pэ, the entered L.M.Finkom. Under pэ understand probability of an error in the symmetric binary channel without memory with constant parameters, in which the system with primitive coding appears to equivalent considered system (transfers the equal quantity of the information I bit and provides identical probability of correct reception of the block of the symbols carrying this information in the set channel).

From definition follows, that probability of correct reception of the block of symbols , where pошбл - probability of erroneous reception of the block of symbols.

In the conditions of qualitative communication pошбл «1, having taken advantage of the formula of a binomial of Newton , we receive:  Thus, pэ approximately defines (in any channel) probability of an error on bit. With other things being equal that system is better, for which pэ it is less.

Control questions?

1. How to be considered communication if in a strip of frequencies of a useful signal there is no concentrated hindrance or at its occurrence amplitude of a useful signal exceeds amplitude of a hindrance?

2. How to be considered communication if on a time interval of the analysis of a signal there is no pulse hindrance or at its occurrence energy of signal Е exceeds energy of a hindrance?

3. To find expression for probability of qualitative communication if the pulse hindrance has the form of a rectangular radio impulse of duration τи with amplitude Uп distributed on exponential to law

Lecture 25. A problem of support of as much as high fidelity of transfer of discrete messages in channels with hindrances

The probability of an error m a symbol (defining fidelity of reception) is minimized at a choice of system  of signals in a kind m a simplex. (Actually at enough big size т almost same probability of an error will be given by system from т mutually orthogonal signals.) Both for simplex, and for  orthogonal signals the size of probability of an error at optimum reception is unequivocally defined by number of these signals т and parameter h2=E/N0, where Е - energy of an element of a signal, a N0 - spectral density of white noise on positive frequencies. From here, apparently, it is possible to draw unequivocally  a conclusion, that unlimited increase of fidelity of message transfer (i.e.  decrease of probability of an error p to zero) can be received only for the account  of unlimited increase of power parameter h2. In particular, if spectral density of power of white noise N0, and also average power of signal Pc are set, it can occur only at aspiration to zero of speed of transfer (symbols) ν, as E=PcT, and T=1 / ν.

Question on, whether it is possible to provide in such channel p→0 at N0=const, Pc=const, ν =const> 0, it is represented therefore, at first sight, not having  sense. Nevertheless it appears, that the answer to it is positive, however  only at performance of certain conditions. The key moment of the decision  of this problem is the increase in number possible simplex (or orthogonal) signals m.

Let is mutual-orthogonal signals and their quantity m=2k, where k - the whole positive number are used. Then by means of each such orthogonal signal of duration T it is possible to transfer k binary signals or (if each of them carries a maximum quantity of the information of 1 bit) k bit. If any binary source of messages is connected to such modulator it can to transfer without a delay growing in time  these messages with information speed ν =k/T bit per second. Using additive border for probability of an error orthogonal m signals, we receive:

(6.1), where

It is possible to show, that for any х ≥ About fairly following inequality:

(6.2)

Substituting (6.2) in (6.1) and making elementary transformations, we find (at 2k »1), that

(6.3)

Analyzing (6.3), we receive, that p→0 at N0=const, Pc=const, ν =const, T →∞ if the condition of negativity of an indicator exhibitors is satisfied, that is equivalent to a following inequality:

(6.4)

So, we have absolutely strictly deduced a sufficient condition for the positive answer to the question formulated earlier, that at first sight it seemed impossible.

The first person who has "surprised" engineers-signalmen by such possibility - it is absolutely reliable to transfer the information on communication channels with hindrances not for the account of increase in power of a signal or reduction of speed of transfer, and for the account of complication of methods of modulation-demodulation, there was American scientist Claude Elvud Shennon. It has made it in the remarkable work "the Mathematical theory of communication", published in 1948г. We have in essence proved "strongly weakened" variant of the so-called theorem of coding Шеннон as have found only sufficient conditions and and only for one model of a continuous communication channel - with an unlimited strip of frequencies as at T →∞ and ν =const we have k →∞. Hence, the quantity of signals and the strip of frequencies occupied with such signals, too always aspire to with. Шеннон has found necessary and sufficient conditions of decrease of probability of an error to zero. Besides, it has made it for various models of channels, in particular with the limited strip of frequencies, for the fixed number of forms of signals m, for sources with not equiprobable or with dependent symbols (so-called superfluous sources), for sources of continuous messages, for criteria of fidelity of reception of the discrete messages which are distinct from probability of an error of a symbol p, etc. All it and has made essence of the theory of the information.

Let's notice, that the information theory is also some kind of communication philosophy. The concepts entered within the limits of this theory, have found application and out of communication problems, for example in art and medicine. However for the engineer-signalman it is important to understand not so much definition of the theory of the information, how many to be able to use if necessary theorems of coding which define potential possibilities of the communication channels set, in turn, by some restrictions. The subsequent sections of this chapter and basically will be focused on the given problematics.

Control questions?

1. If in m to a communication system with orthogonal signals average power of signals and spectral density of noise are constant, that it is necessary to make to provide aspiration to zero of probability of an error at preservation of constant information speed of transfer?

Lecture 26. Potential possibilities of discrete communication channels. Definition of a source of discrete messages, a discrete communication channel, coding and decoding. The basic conceptual device of the theory of the information. Theorems of coding Shannon for a discrete communication channel.

Formally source of discrete messages is completely defined by alphabet A = (а0, a1 … ak-1) used symbols of the set volume K and distribution of probabilities P (a), set on sequences a = (а0, a1 … an), A = (а0, a1 … ak-1) произвольной lengths n, made of symbols of given alphabet A. If the source has the fixed speed, unlike a source with the adjustable speed defined by the coder speed of a source νи as number of the symbols generated by such source for 1сек is in addition set.

The source has no memory if symbols consistently given out to it are mutually independent from each other. For the full description of such source it is enough to set only distribution of probabilities to separate symbols, i.e. P (ai),  if the source is stationary, i.e. distributions P (a) do not depend on time shifts.

As a simple example of a source of messages it is possible to consider the printing text in any language. Then the volume of alphabet K will coincide with volume of the alphabet of the chosen language (26 - for English, 32 - for Russian), and distribution of probabilities will be defined by probabilities of various word-combinations. It is obvious, that it is an example of a source with memory as letters in any language do not follow anyhow, and are subordinated the determined or likelihood dependence. So, in English language always after the letter "q" follows "u", in Russian "ь" cannot go after a vowel, and three "е" meet only in enough rare words "длинношеее" successively.

Let's define formally coding in the discrete channel as unequivocal display of the sequences made of symbols of a source, in the sequences made of entrance symbols of a communication channel. Decoding we will name unequivocal display of the sequences made of target symbols of the channel in sequence, made of source symbols. Here we believe, that alphabets of symbols of a source and the addressee of the information coincide. Lengths of sequences on the channel input, corresponding to various sequences of symbols of a source, can be, generally speaking, various. Then coding is called as non-uniform.

As example of non-uniform coding the known Morse code can serve. Really, in this code to the letter "е" there corresponds sequence from one point ".", and the letter "ш" - sequence from four dashes "-----". We will notice, that the Morse code is a coding not with the binary entrance alphabet (a point and a dash) as can seem at first sight, and with the figurative alphabet: a point, a dash and a blank.

Definition of private quantity of the information i (a), containing in this message, proceeding from following requirements:

1. The quantity of the information i (a) should be additive function, i.e. for pair mutually independent messages a1, a2 it should be equaled to the sum of quantity of the information of each of them, i.e. i (a1, a2) =i (a1) +i (a2)

2. The quantity of the information containing in the authentic message (having probability P (a) =1), is equal to zero.

3. The quantity of the information should depend only on probability of the transferred message, i.e. i (a) =f (P (a)).

4. The quantity of the information should be continuous function from P (a).

Concepts of entropy is average self-descriptiveness a source one symbol, defining "unexpectedness" or "unpredictability" of messages given out to it. Completely determined source which is giving out only one, in advance known sequence, possesses zero self-descriptiveness. Entropy of a source of messages, we will designate through H (A):

(6.11). The basic properties concept of entropy:

1. H (A) ≥0, and H (A) =0 in only case when when one of sequences has individual probability, and all the others - zero

2. For any stationary source сообщений (6.12). As expression in the right part (6.12) is an entropy of a source without memory the given property means, that memory reduces entropy of a source.

3. For any stationary source of messages  (6.13) and equality takes place when the source has no memories and all its symbols are equiprobable.

Theorems of coding Шеннона for a discrete communication channel:

The theorem 1. About source coding. There is a way of coding, at which average length of sequence of the channel symbols , falling to one symbol of a source of messages,  (6.46)

The theorem 2. About coding in the channel without hindrances. There is a way of coding and decoding in the channel without hindrances at which for length n sequences of the channel symbols falling to one symbol of a source, the parity (6.46) will be carried out, and the probability of an error does not surpass any as much as small size δ> 0.

The theorem 3. Basic Shannon theorem. If productivity of source Н ' (A) is less than throughput C ' in unit of time the discrete channel with hindrances at any δ> 0 there is a way of coding and source and channel decoding at which messages are transferred to the addressee with probability of an error smaller, than δ, and on the average without growing time lags. If Н ' (A)> C ' such way of coding does not exist.

Control questions?

1 That such coding and decoding in discrete and continuous communication channels? What at them the general and than they differ from each other?

2 What kinds of modulation - demodulations lead to model of the binary symmetric channel, and what to model of the asymmetrical channel?

3 Properties of entropy, conditional entropy, quantity of the information can be explained intuitively. However if it was not so, whether that the basic results of the theory of the information have remained?

4 Than the source of the information possessing the big entropy is good or bad?

Lecture 27. Classification of methods of coding. Constructive methods of coding of sources of messages

Classification of methods of coding is resulted on fig. 7.1. According to the destination coding is subdivided on primitive, economical and noiseproof. Primitive coding is applied to the coordination of the alphabet of a source and the channel alphabet.

Tab. 7.1 shows, how messages of a discrete source with volume of alphabet K=4 can be transformed for transfer on the discrete binary channel. Distinctive property of primitive coding consists that redundancy of the discrete source formed by an exit of the primitive coder, is equal to redundancy of a source on a coder input.

Primitive coding is used also with a view of enciphering of the transferred information and increase of stability of work of system of synchronisation. In the latter case the coding rule gets out so that probability of occurrence on an exit of the coder of the long sequence consisting only from zero or only from units, was minimum. The similar coder is called also as the scrambler (from an English word "scramble" - to mix). Features of systems of enciphering and synchronisation are studied in special rates here

again not considered.

Economical coding, or compression of data, is applied to reduction of time of an information transfer or a demanded memory size at its storage. Distinctive property of economical coding consists that redundancy of the source formed by an exit of the coder, less, than redundancy of a source on an input of the coder.

Noiseproof coding is applied to detection and correction of the errors arising by transfer on the discrete channel. Distinctive property of noiseproof coding consists that redundancy of the source formed by an exit of the coder, more than redundancy of a source on a coder input.

That the used code possessed property unequivocal decodable, it, obviously, should satisfy to some conditions. Unequivocal decoding will be provided, if any code word is not the beginning of other code word. Such codes are called prefix.

There is a set of ways of coding of sources of messages and some algorithms of construction of non-uniform codes with prefix property. Among them algorithm Shennona-Fano is optimum. Symbols of the alphabet of a source register as it should be not increasing probabilities. Then they are divided on two parts so that the sums of probabilities of the symbols entering into each of such parts, were approximately identical.

To all symbols of the first part the zero, and to symbols of the second part - unit is attributed as the first symbol of a combination of a non-uniform code. Then each of these parts (if it contains more than one message) shares in turn on two, whenever possible equiprobable parts and to them the same rule of coding is applied. This process repeats until in each of the received parts does not remain under one message.

Example. Let alphabet A of a source consists of six symbols a0, a1, a2, a3, a4, a5 with probabilities P (a0) =0,4; P (a1) =0,3; P (a2) =0,1; P (a3) =0,08; P (a4) =0,07; P (a5) =0,05. Procedure of construction of a non-uniform code without alphabet integration (n=1) is set tab. 7.2. At the first stage division into two sets to a0 and a1, a2, a3, a4, a5 is made; on the second - a1 and a2, a3, a4, a5; on the third - a2, a3 and a4, a5; on the fourth - a2, and a3, a4 and a5. It is easy to check up, that the given code appears prefix and average length of code combination , that less than on 2 % exceeds the entropy of the given source equal 2,16.

Let's consider short a method of compression of the similar type, known as algorithm Ziva-Lempela and widely applied in the COMPUTER for archiving files. The general idea of data swept an ode consists that the sequence of symbols of a source breaks into the shortest distinguishable chains which did not meet earlier, and then these chains are coded by a uniform code. For example, if the sequence looked like 1011010100010 it will be broken into chains as follows: 1, 0, 11, 01, 010, 00, 10. At such way of splitting all prefixes of the given chain can be only to the left of it. In particular, the chain which differs from given only in last symbol (bit), always will settle down at the left. If c (n) means number of various chains of splitting for the given sequence of length n, it is necessary log2c (n) bit to code prefix number to the given chain and one more bit for the description of last element of this chain. So, for considered above sequence and its splittings we will receive the following uniform code: (000, 1) (000, 0) (001, 1) (010, 1) (011, 0) (010, 0) (001, 0) where first three bits define prefix number to the next chain, and last bit gives value of last symbol of this chain. It is obvious, that decoding of such code is made by unequivocal image.

The considered example gives actually not compression, and "stretching" of messages as instead of 13 initial bats we receive 28 bits. However for long sequences of messages efficiency of algorithm increases, and in asymptotic, i.e. at n →∞, compression of messages comes nearer to extremely possible, defined by the coding theorem in the channel without hindrances, namely is proved, that for any stationary ergodic a source of messages  we Will notice, that actually for compression of files the modified algorithm named algorithm of compression Ziva-Lempela-Velcha is used, on the basis of this algorithm the most effective programs archiving files, such as PKARC, PKZIP are constructed, etc.

Control questions?

1 In what the method of integration of the alphabet consists?

2 Explain distinction between uniform and non-uniform coding.

3 Make definition prefix a code.

4 As by means of a code tree to check up, whether the code prefix is?

Lecture 28.29.30. Noiseproof (channel) coding, probability of an error of optimum decoding for codes with the fixed length of blocks (exhibitors of probabilities of errors). Codes with the guaranteed detection and correction of errors. Code distance. Linear binary codes for detection and correction of errors.

Let's name channel (noiseproof) block a code V any set from M various sequences (combinations, words) x1, x2, x3..., xM lengths n which each position can accept any of m values of the entrance alphabet X if M <mn (7.6)

Such code name also superfluous. At performance of equality M=mn the code is called as primitive. We will name speed of a code size  (7.7) Obviously, superfluous codes have R <1, and for primitive code R=1. The theorem of coding Шеннона, asserts, that there is such sequence block superfluous codes with the fixed speed R <C/log2m where C - throughput of a discrete communication channel, that at unlimited increase in lengths of these blocks n the probability of an error after optimum decoding in the set channel will aspire to 0 zero.

Probability of an error of optimum decoding for codes with the fixed length of blocks (exhibitors of probabilities of errors). Let there is some, a communication channel which is described by conditional transitive probabilities P (y|x),  where X and Y - its entrance and target alphabets, and Xn and Yn mean every possible sequences of length n from alphabets X and Y accordingly. We will designate through pOД (V, S) probability of erroneous decoding in such channel at use of some code V consisting from M of combinations, and algorithm of decoding on a credibility maximum if message S is transferred, 1≤S≤М. Enough general top border for pOД (V, S) at the best choice of a code V has been received by R.Gallagerom.

The theorem. Exists block a code of V length n consisting of M combinations, for which by transfer of any message S, 1≤S≤М,  (7.8), where P (y|x) - transitive conditional probability for blocks of length n in the set communication channel; Q (x) - any likelihood distribution to entrance blocks of length n.

Codes with the guaranteed detection and correction of errors. Let some is set block a code of length n, consisting of M combinations (blocks, words, vectors) x1, x2, x3..., xM. For simplification of definitions and proofs we will consider everywhere, that entrance X and day off Y channel alphabets coincide. Generally the channel can have memory and be set by probabilities p (y|x) transitions of entrance blocks x during week-end y.

Definition 1. Distance of Hamming ρ (x, x ') between two combinations  we will name number of positions of these combinations in which separate code symbols x and x ' do not coincide. It is obvious, that 1 ≤ρ (x, x ') ≤n for any x ' ≠x and that ρ (x, x) =0 for any .

Definition 2. The sample of an error e we will name the binary block of length n which has units in those positions in which symbols transferred x and accepted y blocks differ from each other, and zero - in other positions.

Definition 3. Weight of Hamming |x | the block (vector) x (it is similar for y and e) we will name number of nonzero symbols of these blocks.

Definition 4. Frequency rate of the sample of an error e (or is shorter - frequency rate of an error) we will name its weight Хэмминга |e |. (In essence this number of errors which has occurred by transfer x.)

Decoding in the set communication channel on a credibility maximum is a decision-making on transfer of such code block  for which conditional probability P (y|xi) is maximum where y - the accepted block. This rule of reception of an estimation can be written down in the following compact form:

Such rule leads to the greatest possible average probability of correct reception of code blocks at an equiprobable parcel of these blocks on a communication channel. (If last condition is not carried out, optimum decoding should correspond to a rule maximum a posteriori to probability.)

Definition 11. As a linear block binary code of length n is called any set of binary sequences of length n which contains purely zero sequence, and for each pair of the sequences belonging to this set, their digit-by-digit sum on mod 2 also is an element of this set. - it is possible to name last property isolation concerning digit-by-digit addition on mod 2.

Example. The set of sequences of length 5: 00000, 11101, 01010, 10111, forms a linear code that is checked directly. The digit-by-digit sum on mod2 2nd and 3 combinations gives 4th combination, the sum 3 and 4 gives 2nd, and the sum 2 and 4 gives 3rd combination. It is obvious, that the linear code satisfies to definition linear subspace V for space Vn of everything, binary sequences of length n. It is a subgroup for group of all binary n - sequences concerning group operation of digit-by-digit addition on mod 2. Therefore binary linear codes are called - as group.

From algebra it is known, that everyone k - measured linear subspace n - the measured space consisting of final number (2 ") elements, contains basis, i.e. set k linearly independent elements of which by digit-by-digit summation on mod 2 it is possible to form any elements of data subspace, i.e. in this case combinations of a linear code. It is proved, that all bases such subspace consist of the same number of elements k which defines dimension subspace. The general number of combinations of linear code M=2k as the general number of the possible digit-by-digit sums from to basis elements will be such, and among them cannot be two identical sums in view of linear independence of elements of basis.

Control questions?

1. Explain distinction between a linear and nonlinear code.

2. Make definition of a regular code.

3. What is the redundancy of a noiseproof code?

4. What code name superfluous?

5. What code is called as primitive?

Main literature

- Hilary. D. Brewster. Digital Electronics First Edition 2011, Oxford Book Company 267, 10-B-Scheme, Opp. Narayan Niwas,

- Proakis, John, and Masoud Salehi. Communication Systems Engineering. 2nd ed. Upper Saddle River, NJ: Prentice Hall, 2009.

- Brindley, К Starting Electronics; 3d ed. Oxford, England: Newnes, 2010.

- Бойко В.И., Гуржий А.Н., Жуйков В.Я. и др. Схемотехника электронных устройств. Микропроцессоры и микроконтроллеры. - СПб.: БХВ - Петербург 2014. - 464 с.

- Long, L. Personal Computing Demystified New York: McGraw-Hill, 2014.

- Greene, N., M. Ramalho, and B. Rosen, RFC 2805: Media Gateway Control Protocol Architecture and Requirements. 2011.

- Угрюмов Е.П. Цифровая схемотехника - СПб.: БХВ - Санкт-Петербург, 2010. 528с.

- Haykin, Simon. Communication Systems. 5th ed. New York, NY: Wiley, 2012.

Additional literature

- Джон Д.Рули и др. Сети WIN NT 4.0. Пер.с англ.–Киев:Изд.Гр.ВНV,2010

- Cutcher, D. Electronic Circuits for the Evil Genius. New York; McGraw-Hill, 2012.

- Калибеков Б.А. Цифровые устройства и микропроцессорные системы. 2изд., Горячая линия Телеком, 2013. 336 с.

- Пухальекий Г.И. Проектирование микропроцессорных систем. Учебное пособие для вузов, -СПб.: Политехника, 2011. - 544 с.

- “Telecommunications Breakdown”, C. R. Johnson, Jr., W.A. Sethares, Prentice Hall, 2014, ISBN: 0-131-43047-5.

- “Algorithms for Communications Systems and their Applications”, N. Benvenuto, John Wiley & Sons Inc. 2013, ISBN 0-470-84389-6.

Kocherova A.N.

Theory of electrical communication

The collection of lectures

It is sent for printing _______2021

Format of paper Х*Y 1/16

Paper is typographical. Press is offset. Volume 2 items of l.

Circulation – 50 copies. Order № ___

©  Edition of М. Auezov South Kazakhstan University

Publishing center of M. Auezov SKU, Shymkent city,

Tauke-khan avenue, 5