F.7.03-20

MINISTRY OF EDUCATION AND SCIENCE OF REPUBLIC OF KAZAKHSTAN

M.AUEZOV SOUTH KAZAKHSTAN UNIVERSITY

Chair «Energetics and renewable energy system»

Korolkov A.V.

Case on theme « Materials with high electric conductivity »

on discipline « Electrotechnical Materials»

for students of EP  “6В07150-Electroenergetics” and “6В07151-Electric Power Supply”.

Shymkent, 2021

MINISTRY OF EDUCATION AND SCIENCE OF REPUBLIC OF KAZAKHSTAN

M.AUEZOV SOUTH KAZAKHSTAN UNIVERSITY

Chair «Energetics and renewable energy system»

Korolkov A.V.

Case on theme « Materials with high electric conductivity »

on discipline « Electrotechnical Materials»

for students of EP  “6В07150-Electroenergetics” and “6В07151-Electric Power Supply”.

Shymkent, 2021

UDK 621.315

The authors:  Case on theme «  Materials with high electric conductivity » on discipline « Electrotechnical Materials » for students of EP  “6В07150-Electroenergetics” and “6В07151-Electric Power Supply”. Shymkent: SKU, 2021. –_32_ p.

Case on theme « Case on theme «  Materials with high electric conductivity » on discipline « Electrotechnical Materials »   have been drawn up in accordance with the curriculum and course program on discipline « Electrotechnical Materials » and includes all necessary information on carrying-out of lessons connected with themes.

The case includes the following sections: the general definition and characteristics of metals, their alloys and compounds, in particular aluminium and its alloys, copper and its alloys, silver and iron. The developed case will be useful to students when studying discipline of " Electrotechnical Materials " as they include all the necessary information on the theme « Materials with high electric conductivity ».

Reviewer:  Markabayeva D.B., Lecturer of " Automation, telecommunications and control" chair of M. Auezov SKU

Сase  have been discussed at the meeting of the chair «Information and communication technology»   (Minutes №         from  «        »        2021y.) and they have been approved by methodical commission of the faculty  « Information Technologies » (Minutes №       from  «     »       2021y.)

Collection of lectures  are recommended to the edition by Educational-methodical council of M.Auezov SKU,

Minutes №              from  «          »             2021 y.

© M.Auezov South Kazakhstan University, 2021

Responsible for printing:  Korolkov A.V.

CONTENT

| INTRODUCTION…………………………………………………………… | 6 |
| --- | --- |
| 1. Requirements for Materials with High Conductivity | 8 |
| 2. Copper and Alloys of Copper | 9 |
| 3.Aluminum and Alloys of Aluminum | 11 |
| 4.Silver | 14 |
| 5. Iron and Alloys of Iron | 16 |
| Write an extended answer to the questions | 19 |
| ASSESSMENT QUESTIONS | 20 |
| CONCLUSION……………………………………………………………… | 23 |
| Criteria for assessing the knowledge and skills of students in Case work…... | 24 |
| LIST OF REFERENCES…………………………………………………..... | 29 |

Introduction

The electrical conduction is the phenomenon of passing of the electric current through a material when it is submitted to the action of an electric field. The electric current of conduction is defined by the ordered movement of free electrical charges (electrons or/and ions) under the action of the electric field. The  metals  and their  alloys  fall  under  the class  of  the  conductive  materials, having the conductivity of the order 106 – 108 1/(Ωm).

The metals are simple substances, solid at normal temperature (excepting the mercury which is liquid at this temperature), crystallized in compact lattice and which differ from the rest of the substances by a series of properties such as: metallic luster, the property of light absorption, the insolubility in common dissolvent but dissoluble in metals  with  which  they  form  alloys.  Metals  present  special  mechanical,  thermal, electrical, and magnetic properties.

The metals are made up of atoms, which have a reduced number of electrons on the  last  electronic  layer  (maximum  4  electrons,  excepting  bismuth  Bi,  that  has  5 electrons on the last electronic layer). 
The atoms of metals have the tendency of giving up electrons and transforming

into positive ions (cations):

M0 – n e− → Mn+, 
where M0 is the atom of a metal and Mn+ is its ion.

The metallic character is founded in:

-  Alkaline metals, which are placed in the 1st group of the periodic table,

-  Earth metals, placed in the 2nd group of the periodic table of elements,

-  Transitional metals, placed from the 3rd group to the 13en group. 
The  properties  of  metals  are  determined  by  a  special  type  of  bond  that establishes between the atoms of the crystal lattice - the metallic bond.

The high electric conductivity of metals has been explained since 1900 by P. Drude who formulates a classical microscopic theory of electric conduction, and it was completed later by H. A. Lorentz (1916).

This theory considers that the crystal lattice of metals is made up of the metallic cations  through  which  the  electrons  move  freely.  In  the  metallic  lattice,  thus,  the metallic ions are “sank” in a fluid of free electrons, named electron gas. The interaction between  the  metallic  cations  and  the  electron  gas  represents  exactly  the  metallic bond.

The theory, known under the name of “the theory of the electron gas” explains some specific properties of metals such as: the metallic luster, the opacity, the thermal conductivity,  the  electric  conductivity,  the  density,  the  malleability,  the  ductility,  the chemical reactivity, etc.

The presence of the free (extremely mobile) electrons, which move disorderly in all directions, under the action of the thermal agitation, doesn’t permit the establishment of a permanent electric current inside the metal.

Applying  an  electric  field,  over  each  quasi-free  electron,  an  electric  force operates, and over the movement of the thermal agitation, an oriented motion of the electrons is developed, which represents in fact the electric current.

The  electric  current  through  metals  doesn’t  transport  substance,  being  an electronic  current  (the  mass  of  the  electron  is  very  small),  unlike  the  electrolytes, where  the  electric  current  transports  substance  (the  ions  have  an  important  mass compared to that of the electron).

The  electric  conductivity  of  the  electrolytes  increases  concurrently  with  the growth of the temperature (the dissociation and the mobility of ions increase). The variation of the metal conduction with temperature has at its base from the variation of the frictional forces that operate on the free electrons. The frictional forces are the result of the oscillations of the ions from the edge points of the crystalline lattice and of the interaction of electrons with these particles.

A  specific  characteristic  of  the  metals  is  that  the  electric  and  thermal conductivities  increase  simultaneous  with  the  temperature  decreasing.  At  low temperatures,  the  oscillations  are  more  reduced  so,  the  frictional  forces  are  also reduced, that is why the electric conductivity becomes higher.

At 0 K some metals don’t show resistance against the passing electric current anymore, they become superconductors.

1. Requirements for Materials with High Conductivity

The  materials  with  high  conductivity  have  the  function  of  conducting  the electrical current, due to the low or negligible resistance opposing the electrical current.

In order to use a material as an electrically conductive material, it is necessary to fulfill the following requirements:

-  low electrical resistivity,

-  the skin effect should be neglected,

-  high admitted current density,

-  high thermal conductivity,

-  appropriate elasticity,

-  high mechanical strength,

-  high resistance at chemical corrosion,

-  easy processing through rolling,

-  easy sticking, soldering and durable contacts,

-  adequate technologies of obtaining, and recycling possibilities,

-  low costs.

The main requirement is to have high electrical conductivity. Table 2.4 presents the electrical conductivity, respectively the electrical resistivity of some metals at normal environmental temperature.

Table 1.1. Conductivity and electrical resistivity of different metals.

| Metal | Ag | Cu | Au | Al | W | Zn | Ni | Fe | Pb | Mn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| σ, *107[1/Ωm] | 6.21 | 5.8 | 4.55 | 3.65 | 1.89 | 1.69 | 1.43 | 1.02 | 0.48 | 0.07 |
| ρ, *107[1/Ωm | 1.61 | 1.72 | 2.20 | 2.74 | 5.3 | 5.92 | 7.0 | 9.8 | 21.0 | 139 |

Materials with high conductivity are: silver, copper, gold, aluminum, and iron.

These  metals  meet  almost  completely  the  requirements  imposed  for  their  use  as conductors. However, among all gold and silver are rarely used being precious metals, their performances will be presented as it follows.

2. Copper and Alloys of Copper

Its name comes from the island of Cyprus where there was discovered a large amount of copper ores. Known for more than 6000 years, since the age of bronze (as alloy of Cu-Sn), it has been used as material for tools, guns and jewels. Only in the 19th  Century  the  copper  begin  to  be  used  for  its  electro-conductive  properties.  The copper is the metal used especially in the electrotechnic industry, as electrolytic copper of 99.6 - 99.9% purity, and as alloy element.

Natural state

The  copper  is  found  under  the  form  of  compounds, in  minerals,  usually  polymetallic ones: Cu2S, Cu2O, Cu2CO3(OH)2, CuS, CuO, Cu2(CO3)2(OH)2 , CuFeS2, etc.

Obtaining

The copper is obtained through sulfurous ores and oxidized by pyro- or hydrometallurgical reducing. For obtaining it, the ore is enriched (ore processing), by gravity and flotation processes. The pyro- or hydro-metallurgical reducing methods are applied according to the ore character and its copper content. The obtained copper contains, in variable quantities, S, Fe, Ni, Zn, Co, Sn, Pb.

Purifying the copper is realized by affinage and refinement. By affinage it can obtain  pure  copper  99.99 %  and  by  refinement,  copper  of  high  purity  99.9999 %. The refinement can be done pyrometalurgically or electrically.

Pyrometallurgical  refinement is realized by  melting  the  solid  copper at  1100-1200єC  in  a  refinement  oven,  where  compressed  air  is  blown.  In  this  process,  the impurities transform into oxides, As2O3, Sb2O3, SO2, volatile oxides, and the metal oxides react with SiO2 from the coating of the oven forming cinders (FeSiO3, ZnSiO3, NiSiO3).

When the cinders are put away in order to de-oxide the copper, partially oxided (Cu2O),  green  birch  trees  are  introduced  in  the  melting,  they  decompose  at  the cupboard  temperature  and  form  water  vapors,  H2,  CO,  which  agitates  the  melting, enhancing the departure of volatile compounds and reducing Cu2O to metallic copper. From the refined copper, the electrolytic copper is obtained.

The electrolytic refinement of the copper is realized in concrete basins coated with walls of lead. The electrolyte is a solution of copper sulfate and sulfuric acid with sodium sulfate. In the absence of H2SO4 in the electrolyze process it will result variable quantities of Cu2O.

In  the  electrolyze  basin  there  are  alternatively  arranged  anodes  from pyrometalurgically refined copper and cathodes from pure copper. The reactions are:

CuSO4=  Cu2+ + SO42−

CuO + H2SO4 = CuSO4 + H2O

which re-enters the process:

Cu2O + 2 H2SO4 + O2 = 2 CuSO4 + 2 H2O

HOH = H+ + HO−

Fig. 1. Copper and Alloys of Copper

Usage

According  with  Romanian  standards  STAS  643-69  and  STAS  270/1-74,  the

copper sorts are:

- Sorts obtained through thermal refinement: Cu 0 (99.00%), Cu 5 (99.50%), Cu 9

(99.9%)

- Sorts obtained through electrolytic refinement: CuE (99.99%), for industrial use.

3.Aluminum and Alloys of Aluminum

The name of aluminum derives from the Latin alumen, the name of the stone, which has double sulphide of aluminum and potassium – used from ancient times as mordant in dyeing.

Natural state

The aluminum is the most encountered metal on the earth, after the oxygen and silicon. Among its compounds, the most important ones are:

-  Al2O3 - corundum, with impure and colored variations used as precious stones:

* ruby - red (unpurified corundum with Cr2O3)

* topaz – yellow (unpurified corundum with MnO)

* sapphire - blue (unpurified corundum with TiO2, Fe2O3),

-  Al2O3⋅nH2O – bauxite,

-  Na3[AlF6] – criolite.

Fig. 2. Aluminum and Alloys of Aluminum

There are over 250 minerals, which contain aluminum, as silicates of aluminum (feldspars, argyle etc.).

Obtaining

The  aluminum  is  obtained  by  electrometallurgical  reducing  process.  The aluminum metallurgy implies two important stages:

a) obtaining alumina Al2O3 from bauxite,

b) obtaining aluminum Al from alumina.

The refinement is realized through electrolyze in melting state at 700 - 750єC. The anode is made out of impure aluminum, the cathode from pure aluminum and the electrolyte is a mixture of AlF3, NaF, BaCl2.  The process of zonal melting is applied in electronics.

Physical properties

Both  in  compact  and  powder  state,  the  aluminum  is  silver-white.  It  is  a  soft metal, having the density dm = 2700 kg/m3, its hardness is 2,7 (Mohs), easily fusible, its melting point is Tt = 660єC, plastic, malleable and ductile, a good conductor of heat and electricity.

From the electrical and thermal point of view, the aluminum follows the copper. It is much lighter and cheaper than copper, but with an inferior mechanical strength; it offers a high processing possibility and a good resistance at electrochemical corrosion.

The types of electroconductive aluminum contain maximum 0.5% additions, the main ones being the iron Fe and the silicon Si. Figure 3 indicates the influence of the impurities on the aluminum electric conductivity.

Fig.3 . Dependence of aluminum electrical conductivity on the impurity percentage.

Chemical properties

The  aluminum  is  a  metal  with  high  reactivity.  The  form  of  stable  and characteristic valence is III, being situated in the periodical table of the elements at the period 2, group III A, with the atomic number Z = 13 and the electronic structure of the atom: 1s2 2s2 2p6 3s2 3p1.

Thus, with the normal reducing potential ε 0Al3+ / Al 0 =−1.66V , has a high reducing character. The aluminum is destroyed in contact to technical metals. That is why the connecting clamps between the conductors of the air conductor networks from Cu - Al are special.

The corrosion resistance of aluminum is high because the aluminum is covered with a film of Al2O3, which protects it. It resists at water, organic substances, ammonium solutions, but it can be easily attacked by chloride, halogen, seawater and organic acids.

Usage

In electrotechnics the aluminum is used for:

⵼  coil conductors and transportation lines;

⵼  armatures for capacitors (malleability and high ductility);

⵼  obtaining the semiconductive materials;

⵼  obtaining the magnetic alloys, the alloys of high resistivity etc.;

⵼  housings, massive parts (specific low weight).

The main alloys of aluminum, with use in electrotechnics are:

- duraluminum, (90% Al, 3-5% Cu, 1-2% Mg, 1% Mn, 0.2-1% Si) STAS 7608 - 71, which presents superior mechanical properties to that of the aluminum, but with a lower  resistance  against  corrosion,  it  is  used  with  a  protective  layer  or  pure aluminum;

- siluminum, (Al + 10-13% Si) STAS 201/1 - 71, is used for molding various devices or frames with complex geometrical forms having a very low molding contraction coefficient, being superior to the aluminum by its higher resistance to corrosion;

- alduro, (Al + 0.7% Mg + 0.6% Si + 0.25% Fe), also known as aldrey, is used in the air  conductors  industry  for  transporting  the  electrical  energy,  having  a  relatively good electrical conductivity and a high mechanical strengths.

The  aluminum  alloys  present  low  density,  high  hardness,  high  electrical  and thermal  conductivity,  and  high  chemical  resistance  to  air,  water  and  acids.

Other uses of the aluminum and its alloys:

-  in metallurgy in order to obtain other metals,

-  in anticorrosive protection,

-  in packing food products and pharmacy,

-  in explosive mixtures,

-  as salts,

-  as alloys it is used to make airships, vehicles, navies, tanks used in transporting the nitric acid, nuclear reactors, etc.

4.Silver

Silver  has  special  applications  in  electrotechnics,  being  the  metal  with  the highest electrical conductivity.

The name of silver comes from the Greek argentum. It is known from ancient times  because  it  could  be  found  in  pure  state.  The  Phoenicians,  famous  navigators during Antiquity, have found in Spain large amounts of silver and as they had no more space in their ships, to load it, they have replaced the wood anchor with massive silver ones. In Antiquity, silver was used for trade.

Natural state

Silver  can  be  found  free in  nature,  most  of  the  time  it is  associated  with gold, platinum metals, mercury, copper or antimony. There are known some other minerals of silver: Ag2S, Ag3AsS3, AgCl.

Obtaining

Native  silver  is  extracted  by  amalgam  process  and  the  technical  cooper  from compounds by hydrometallurgical reducing process. Pure silver is obtained by electrochemical refinement of raw silver. The anode is formed  by  raw  silver,  the  cathode  from  pure  silver  and  the  electrolyte  solution  is  of AgNO3.

Physical properties

In compact state, silver has a bright white color and as powder it is gray. It is a heavy metal dm=10500 kg/m3, soft regarding hardness, its melting point is Tt = 960.5єC. It presents special mechanical properties, it is very malleable and ductile, good heat and electricity conductor as well as diamagnetic.

Chemical properties

Silver is a metal with a low chemical reactivity:  ε A0g+ / Ag 0 =+0.799V , it is a noble metal.

It presents stability in the air with no hydrogen sulphide. In its presence, silver is covered with a black layer of silver sulphide.

4 Ag + 2 H2S + O2 = 2 Ag2S + 2 H2O

from air       black

At  normal  temperature  it  reacts  with  halogens,  with  diluted  nitrogen  acid  and concentrated with alkaline cyanide solutions.

3 Ag + 4 HNO3 = 3 Ag NO3 + NO + 2 H2O

diluted

Ag + 2 HNO3 = Ag NO3 + NO2 + H2O

concentrated

4 Ag + 8 KCN + 2 H2O + O2 = 4 K [Ag(CN)2] + 4 KOH

At high temperature it also reacts with sulfuric acid.

Usage

Silver  is  used  in  alloys  to  make  coins,  jewels,  plugs,  etc.  It  is  used  for anticorrosive  protection  or  for  decoration,  to  fabricate  mirrors  with  a  high  power  of  reflection,  to  make  laboratory  tools,  to  produce  cathodes  in  unconventional  cells,  to produce electrical conductors, for photoelectrical films.

Silver and its compounds present anti-bacterium properties. The colloidal silver is used as pharmacy products.

5. Iron and Alloys of Iron

In certain cases, to fabricate conductors, distribution lines, instead of copper or aluminum, iron is often used, due to its lower costs, its superior mechanical resistivity and due to its good malleability.

The name of the iron derives from the Latin ferrum. The Iron Age followed the Bronze Age, when the iron was known especially as meteoric iron. It was very rare and needed a complex technology.

At Louvre, there is displayed an iron amulet engraved in gold, from 1900 B.C., which shows the fact that the iron was a very expensive metal in a period of time, if it was engraved in gold.

Natural state

The iron is found in free state as meteoric iron (in fact, an alloy of the iron with 5-20% Ni)  and  telluric  iron,  with  a  maximum  amount/content  of  2% Ni,  0.3% Co, 0.4% Cu and 0.1% Pt.

Large amounts of iron can be found as compounds:

- Fe3O4 - magnetite

- FeS2 - pyrite (crystallized in cubic system)

- Fe2O3⋅H2O - limonite

- FeS2 - marcasite (crystallized in rhombic system)

- Fe2O3 - hematite

- FeCO3 - siderite

Iron could be also encountered in the composition of silicates, thus all argyles are unpurified with the iron compounds.

Obtaining

Pure iron is obtained by reducing its oxides using hydrogen or aluminum

Fe2O3 + 3 H2 = 2 Fe + 3 H2O

Fe2O3 + 2 Al = 2 Fe + Al2O3

FeO + H2 = Fe + H2O.

Practically, the iron alloys, cast iron and steel, are used more commonly. Cast iron is an alloy of Fe-C which contains 1.7-5% carbon, and steel is an alloy of Fe-C which contains 0.1-1.7% carbon. Cast iron is obtained in furnaces, where the reducer is the carbon. By partial oxidation of the cast irons, steel can be obtained.

Physical properties

In compact state, the iron is silver-gray, and in powder it is gray. It is a heavy metal, dm=7860 kg/m3, hardly fusible, with its melting point Tt = 1536єC, it is malleable and ductile.

Chemical properties

From the value of normal reducing potentials

it can be stated that iron is an active metal and in chemical reactions it behaves as a reducer. It presents valence II and III, mostly III.

The divalent iron compounds are easily oxidized even with the oxygen from the air. In humid air, the iron rusts.

Fe + 2 H2O = Fe(OH)2 + H2

white rust

4 Fe(OH)2 + O2 = 4 FeO(OH) + 2 H2O

brown rust

Fe(OH)2 + 2 FeO(OH) = Fe3O4 + 2 H2O

black rust

It reacts, at normal temperature, with diluted acids (HCl, H2SO4, HNO3) and it becomes passive with concentrate acid HNO3:

Fe + 2 HCl = FeCl2 + H2

Fe + H2SO4 = FeSO4 + H2

4 Fe + 10 HNO3 = 4 Fe(NO3)2 + NH4NO3 + 3 H2O

At  high  temperature,  it  reacts  with  halogens  (X),  with  oxygen,  sulfur,  water, hydrogen,  sulfured  hydrogen,  forming  the  corresponding  compounds:  FeX3,  Fe3O4, FeS, Fe2S3.

Usage

Large  quantities  of iron  are  used  as  cast  iron  and  steel.  In  small  quantities  of 4-6.5%, it is added to brasses and bronzes for increasing the elasticity, hardness and antifriction properties.

Iron is used for communications lines, for distribution lines of low power, and for electrical lines which have to support high mechanical charges. The iron is mostly used as a conductive material, having the form of a rope, as a core for bimetallic conductors Fe-Al.

The technical iron of Armco type, which contains less than 0.2% impurities, is mostly used for the lines of continuous current. In alternative current, the steel with 0.1-0.15% C is used as conductors, having ρ = 1.3 ⋅ 10-7 Ωm, but, due to the skin effect (equivalent to an energy loss at the surface of the conductor) the resistance in alternative current increases, that why, the steel type armco iron is recommended for DC applications.

Indeed,  it  is  well  known  that  the  report  between  the  resistance  in  alternative current and the one in the continuous current is expressed according to the depth of penetration of the electromagnetic field, given by the relation (2.36). In the iron, the skin effect is very intense, because the depth of penetration of the magnetic field depends on the electrical conductivity, magnetic permeability and the frequency of the inductive magnetic field.

The  telecommunication  lines  are  often  made  by  iron  with  zinc-covered conductors to protect them from corrosion.

Write an extended answer to the questions.

1. What does the electrical conductivity of ideal and real metals depend on?

2. How do you determine the resistivity of a metal at any temperature?

3. What is the fundamental difference between ρ and α ρ variation of Cu-Au and Cu-Ni alloys?

4. How do ρ and αρ depend on the thickness of a thin metal film? What is the resistance of a square?

5.  Give the main high conductivity materials and their applications.

6. How do impurities affect the properties of copper and aluminium?

7.  List the major noble metals, their properties and applications.

8.  Name the main refractory metals, the features of their properties and applications.

9. Which materials are used for fixed contacts?

10. What materials are used for low current rupture contacts?

11.  What materials are used for high load breaking contacts?

12. What material is used to make the collector plates of electrical machines?

13.  What kind of electric brushes are best used for high-powered low-voltage electric motor; low-powered electric motor and high-speed electric machines?

ASSESSMENT QUESTIONS

1. What is a material parameter?

- A material property

- A physical quantity that can be associated to each material property

- A chemical property

2. Which of the quantities is a material parameter?

- Electrical conductivity

- Electrical suscep

- Thermal elongation

3. What is the electrical conductivity measured by?

- 1/Ωm

- Ωm

- N/m2

4. What is the thermal conductivity measured in?

- Ωm

- W/m K

- W/m2

5. What is the linear dilation coefficient measured in?

- N/m2

- W/k

- 1/K

6. What is the classification criterion of gaseous, liquid and solid materials?

- State of aggregation

- A specific property

- The domain in which they are used

7. According to electrical conductibility materials are divided into:

- Conductive materials, Semiconductive materials, Electroinsulating materials

- Ferromagnetic materials, Ferrimagnetic materials

- Crystalline materials, Amorphous materials, Mezomorphous materials

8. What material parameter contains the law of elasticity?

- The friction coefficient

- The Young module

- The specific heat

9. The law of electrical conductivity is mathematically expressed by:

- E=f(I)

- P=f(E)

- J=f(E)

10. Quantity J represents :

- The intensity of the electrostatic field

- The electric current density

- Electrical resistivity

11. Which is the correct order of the following materials if they are ordered increasingly

according to the electrical resistivity values?

Gallium arsenic, amber, paper, silver, manganese  a

2.2.1 §Silver, manganese, gallium arsenic, paper, amber  b

14

Amber, gallium arsenic, silver, manganese, paper

12. In which materials takes place the ionic conductivity?

- Conductive materials, semiconductive materials, electroinsulating materials

- Conductive materials, electroinsulating materials

- Semiconductive materials, electro-insulating materials

13. Decide to which category of materials belongs, if its electrical resistivity is 10 Ωm.

- Semiconductive materials

- Electroinsulating materials

- Electrically conductive materials

14. The temporary electrical polarization is measured in:

- V/m

- Ω m

- C/m2

15. What is the correct order of the following materials if they are ordered increasingly according to their relative electrical permittivity?

- Barium titanate, paper, transformer oil, water, air

- Air, water, paper, barium titanate, transformer oil

- Air, transformer oil, paper, water, barium titanate

16. The quantity marked by the χe symbol represents:

- Electrical permittivity

- Electrical susceptivity

- Magnetic permeability

17. The temporary magnetization Mt  is measured in:

- A/m

- H/m

- F/m

18. The quantity marked by the µ symbol represents:

- Magnetic susceptibility

- Absolute electrical permittivity

- Absolute magnetic permeability

19. Decide to which category of materials the bakelite belongs?

- Electrically conductive materials

- Semi-conductive materials

- Electroinsulating materials

CONCLUSION

The case includes the following sections: This paper outlines all the material students need to study the material. Students can study the theoretical material on this topic, namely conductive materials, aluminium, copper, silver, iron and their alloys. They are also provided with test and answer questions to which the student should not simply rewrite sentences and texts but analyse the material studied, paraphrase and write down the answer in their own words.

The developed case will be useful to students when studying discipline of " Electrotechnical Materials " as they include all the necessary information on the theme « Materials with high electric conductivity ».

Criteria for assessing students' knowledge

| Evaluation by letter system | Numeral equivalent of points | %- content | Evaluation according to the traditional system | Criteria for assessing students' knowledge |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | Excellent | The student demonstrates: - a deep and lasting assimilation of program material - complete, consistent, competent and logically presented answers for all types of activities; - Ability to freely cope with the tasks; - correct, well-founded decisions, - skills in the use of information from basic and additional specialized literature in the study of discipline, - the ability to self-systematize the program material; - Possession of versatile skills and methods of performing all types of tasks; - Ability to work with foreign literature and information resources of the Internet; - Timely and high-quality execution of all types of tasks. |
| А- | 3,67 | 90-94 |  | The student demonstrates: - Deep assimilation of program material; - complete, consistent and logically presented answers for all activities; - ability to cope with the tasks; - the right decisions, - skills of using special literature in the study of discipline, - the ability to self-systematize the program material; - Possession of skills and techniques for performing all types of tasks; - Timely execution of all types of tasks. |
| В+ | 3,33 | 85-89 | Good | The student demonstrates: - assimilation of program material; - complete, consistent, literate, without significant inaccuracies, an account of the answers for all kinds of tasks; - correct application of theoretical knowledge - Possession of necessary skills in the performance of applied tasks - skills in using recommended literature in the study of discipline, - skills of systematization of program material; - Possession of skills and techniques for performing all types of tasks; - Timely execution of all types of tasks. |
| В | 3,0 | 80-84 |  | The student demonstrates: - assimilation of program material; - a consistent presentation of the answers for all types of tasks with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - Possession of necessary skills in performing practical tasks - skills in using recommended literature in the study of discipline, - skills of systematization of program material under the guidance of a teacher; - mastering the skills of performing all kinds of tasks; - the ability to independently correct mistakes made; - the timely execution of all types of tasks with the elimination of errors. |
| В- | 2,67 | 75-79 |  | The student demonstrates: - assimilation of program material; - Ability to present answers with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - Possession of techniques in the performance of practical tasks - skills in using recommended literature under the guidance of a teacher - skills of generalization of program material under the guidance of a teacher; - the ability to correct mistakes made with the help of a teacher; - the timely execution of all types of tasks with the elimination of errors. |
| С+ | 2,33 | 70-74 | satisfactorily | The student demonstrates: - mastering of the main material; - Inadequate formulations when responding to all kinds of tasks; - violation of the sequence in the presentation of the program material; - difficulties in self-fulfillment of practical tasks; - Possession of individual techniques in the performance of practical tasks - skills in using the literature recommended by the teacher; - skills of generalization of separate sections of program material under the guidance of a teacher; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С | 2,0 | 65-69 |  | The student demonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - difficulties in self-fulfillment of practical tasks; - Possession of separate receptions at performance of tasks - difficulty in using the literature recommended by the teacher; - difficulties in generalizing the individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С- | 1,67 | 60-64 |  | The student demonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - Inadequate possession of individual methods in the performance of assignments - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| Д+ | 1,33 | 55-59 |  | The student demonstrates: - mastering of separate sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - significant difficulties in the application of individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - difficulties in correcting the mistakes made by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| Д | 1,0 | 50-54 |  | The student demonstrates: - difficulties in mastering individual sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - Inability to apply individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - Inability to generalize the individual sections of the material studied; - significant difficulties in correcting the mistakes committed by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| F | 0 | 0-49 | unsatisfactory | The student demonstrates: -- not having knowledge of program material, - When performing all types of tasks, gross errors are allowed; - lack of skills in applying individual tasks; - non-fulfillment of certain types of tasks provided by the forms of current, intermediate and final control. |

List of references

- Helerea Elena, Calin Marius Daniel. Materials In Electrical Engineering, Publishing House Transilvania University Brasov, Romania, 2015. — 378 p. — ISBN 978–606–19–0717–5.

- Ulyeva G.A. Material science : tutorial / G.A. Ulyeva. - Almaty : TechSmith, 2019. - 252 с

- Zhumamukhambetov N.Ғ. Electro-technical material : Oku kuraly / N. Ғ. Zhumamukhambetov, V. A. Yashkov, M. A. Zhanalieva. - Almaty : SSK, 2017. - 304 с.

- Neverov A.S. Corrosion and protection of materials : textbook for students of technical specialties of universities / A.S. Neverov, D.A. Rodchenko, M.I. Tsyrlin. - M. : Forum : INFRA-M, 2015. - 224 с.

- Taukebayeva K.S. Resistance of materials: textbook for researchers and students of technical universities / K.S. Taukebayeva, A.Zh. Talipov. - Taraz : IP "Beisenbekova A.Z.", 2015. - 200 с.

- Zhakishev B.A. Thermal insulation materials, thermal conductivity and strength calculation of building structures of steam and hot-water boilers : tutorial / B. A. Zhakishev ; Ministry of Agriculture of RK, KazGAU named after S. Seifullin. - Almaty : Evero, 2011. - 84 с.

- Understanding electric power systems : an overview of the technology and the marketplace / Jack.

- Casazza, Frank Delea.-Published by John Wiley & Sons, Inc., Hoboken, New Jersey, 313p.

- Shinibekova R.A. Electrical Engineering Materials: Electrical Power Engineering Managers / R. A. Shinibekova. - Shymkent : 0 KUMU, 2011. - 79 с.

- Ignatov A. N. Optoelectronics and Nanophotonics: Textbook / A. N. Ignatov. - 2nd edition, revised and supplemented. Lan', 2017. - 596 с. -

- Muratova K.A. Zhylu Energetikadaғy konstruklyk materialy : 5B071700 - "Zhyluenergetika" man. student dirister zhinagy / K. A. Muratova. - Shymkent : KMMU, 2017. - 108 с.

- Muratova K.A. "Constructing Material" under 5B071700 - "Energy and Power Engineering" student үshin student үsındіk zhumysın ınalgа arnalgаn әdіstemelіk nұқau / K. A. Muratova. - Shymkent : KMMU, 2014. - 24 с.

Korolkov A.V.

Case on theme « Materials with high electric conductivity »

on discipline « Electrotechnical Materials»

for students of EP  “6В07150-Electroenergetics” and “6В07151-Electric Power Supply”.

Signed to print  «__» ___2020y.

Paper size

Typographical paper. Offset printing. Volume 2 p.sh

Circulation ех. Оrder  №__

© M.Auezov South Kazakhstan University, 2021

Publishing center SKU by M.Auezov, Shymkent, Tauke khan 5