F.7. 03 – 03

Zheleuova Zh.S.,Utebaeva A.A., Koshtayeva G.E.

Lecture notes on the discipline

“Structural and mechanical properties of food products”

Shymkent, 2022

MINISTRY OF EDUCATION AND SCIENCE OF THE REPUBLIC OF KAZAKHSTAN

|  | Non-profit joint stock company «M. Auezov South Kazakhstan university» |
| --- | --- |

HIGHER SCHOOL  "TEXTILE AND FOOD ENGINEERING"

CHAIR “Technology and Safety of Food Products”

Zheleuova Zh.S.,Utebaeva A.A., Koshtayeva G.E.

Lecture notes on the discipline

«Structural and mechanical properties of food products»

for 6B07241 – Technology of fats and vegetable oils

Shymkent, 2022

UDK 664.562 (075.5)

BBK 30.607

Authors: Zheleuova Zh.S.,Utebaeva A.A., Koshtayeva G.E.

Lecture notes on the discipline «Structural and mechanical properties of food products» are intended for students on the educational program 6B07241 – «Technology of fats and vegetable oils» - Shymkent: SKU, 2022. -   p.

Lecture notes are composed according to syllabus and standard program of discipline « Structural and mechanical properties of food products» and include all information needed on carrying out of practicum course subjects.

Lecture notes are developed for the students of specialty for students on the educational program 6B07241 – «Technology of fats and vegetable oils»

Reviewer: Kaldybekova Zh.B – M. Auezov South Kazakhstan university, c.t.s., associate professor

Berdembetova A.T. – M. Auezov South Kazakhstan university, senior lecturer

It is considered and is recommended for printing by chair meeting «Technology and safety of food products» (Minutes #     «__»_____20__) and by thehigh school T&F Emethodical Board (Minutes # ___   «___»________20__  )

Recommended by session of Methodical council of M. Auezov South Kazakhstan University    (#,      «___» ______20 )

© M. Auezov South Kazakhstan University, 2022

Content

| Introduction…………………………………………………………………………. Lecture 1. Subject and objectives of the discipline………………………………….. Lecture 2. Rheology as the science of deformation and flow of materials………….. Lecture 3. Dispersion types. Types of structures and their classification…………… Lecture 4. Basic terms and definitions of rheology, axioms of rheology…………….. Lecture 5. Rheological models of simple «ideal» bodies. Basic equations of stresses and deformations of «ideal» bodies…………………………………………………. Lecture 6. Rheological models of complex real bodies. Basic equations of complex rheological bodies…………………………………………………………………… Lecture 7. Flow curves as a tool for describing the rheological properties of materials……………………………………………………………………………… Lecture 8. Structural and mechanical characteristics of food products as an objective indicator of impact………………………………………………………… Lecture 9. Compression and surface properties of food grade materials……………. Lecture 10. Measurement methods……………………………………………………. Lecture 11. Classification of methods and instruments for measuring the structural and mechanical properties of food…………………………………………………… Lecture 12. Instruments for measuring shear properties of products………………... Lecture 13. Instruments for measuring the compression properties of products……….. Lecture 14. Instruments for measuring the surface properties of products………….. Lecture 15. Relationship between structural and mechanical properties and organoleptic assessing the quality parameters of finished products…………………. A system for assessing the students' results of educational achievements…………... List of literature……………………………………………………………………… | 6 7 10 13 18 21 23 27 29 32 37 39 40 51 57 65 71 74 |
| --- | --- |

INTRODUCTION

The food industry of the XXI century is developing rapidly, the profitability of the food industry has more than tripled in recent years, which made it possible to make it competitive by world standards. Many enterprises are paying more and more attention to the problems of economic efficiency and the quality of their products due to with increasing competition in this area. As you know, different food products have not only a variety of chemical composition, but also different properties, which affects the quality of products. When designing machines and apparatus for the production of food products, it is necessary to take into account the various properties of certain products, which is not always fulfilled.

Rheology (from the Greek rheos - flow, and logos - word, doctrine) - science of deformation and fluidity of matter. Considers the processes associated with irreversible residual deformations and the flow of various viscous and plastic materials, stress relaxation phenomena, etc.

Rheology is closely related to hydromechanics, the theories of elasticity, plasticity, and creep; viscometry methods are widely used in it. Problems of rheology in many fields of technology, in the physics of polymers in the study of friction mechanisms, in the physics of dispersed systems, in biophysics, etc. For technologists of the meat and dairy industry, knowledge of the rheological characteristics of food products is a prerequisite for successful work. Raw materials or obtained finished products, the rheological characteristics of which are unsatisfactory, cannot be realized, because the appearance and content of the products will not correspond to high quality.

The objective of this course is to master rheological concepts, methods and skills for applying the knowledge gained in the food industry. Rheological phenomena are manifested in many natural processes and in a large number of technological ones. This direction is relevant and promising for young specialists in the study, for the application of knowledge in production and in research activities.

Depending on the task, studied rheological characteristics can be used to determine the quality of the finished product, regulate the parameters of the technological process of production, serve as initial data in the design of technological equipment, etc.

Lecture 1

Subject and objectives of the discipline

Lecture plan:

1. The main tasks of the discipline «Rheology of food products»

2. The role of food rheology in ensuring the control, regulation and management of the quality of raw materials and finished products

The purpose of the lecture: to study the theoretical foundations of the rheology of food products as an integral part of the science of physical and chemical mechanics of food production

- The main tasks of studying the discipline «Rheology of food products».

The study subject of the discipline «Rheology of food products» are food materials of animal and plant origin (meat, fish, milk, grain, vegetables, fruits, etc.), as well as food compositions consisting of two or more components with the addition of various additives or without them (minced meat and liver mince, pates, semi-finished products, dough, confectionery masses, etc.).

The main objectives of studying the discipline «Rheology of food products» are:

- study of the theoretical foundations of the food products rheology as an integral part of the science of physical and chemical mechanics of food production;

- study of the foundations of the structure formation of food masses (products) and the behavior of food materials in the processes of technological processing;

- study of the main structural and mechanical characteristics of food products and their impact on the quality of finished products;

- study of mechanical models and their practical application for modeling in the development of new real food products;

- study of methods and devices for determining the structural and mechanical characteristics of food products;

- know rheological calculations of individual elements of technological equipment;

- study of the main directions in the field of optimization, control and management of technological processes in order to obtain high quality products.

The most complete picture of some essential aspects of product quality can be given by a group of physical properties, which is dependent on the biological and chemical composition (recipe) and internal structure (product structure). Small changes in these defining characteristics should cause significant changes in the values ​​of the properties that are recorded by the instruments. At the same time, the characteristics of raw materials predetermine the main indicators of finished products. Structural and mechanical (rheological) properties are one of the groups of such indicators.

2. The role of rheology in ensuring the control, regulation and management of the quality of raw materials and finished products. Nowadays the production of food products, which includes a large number of different technological processes, is impossible without using of instrumental (objective) measurement methods and instrumental technology for monitoring, regulating and managing the quality of raw materials and finished products. So, an important and responsible role is assigned to rheology, as a science dealing with the structure formation of food materials, the study of structural and mechanical properties, the development of methods and instruments for their determination.

The processing of various food materials is accompanied by complex physicochemical, biological and mechanical processes, the study of which makes it possible to organize effective and objective rheological control and management of technological production cycles. Most of the processes in the food industry are associated with the processing of dispersed systems, suspensions, colloidal solutions, various elastic-plastic-viscous materials. Rheological research allows a deeper understanding of the physics of the phenomenon that occurs during the processing of food materials.

Rheological properties can be used in calculating the processes that must be performed when creating new designs of machines and reconstructing existing ones, as well as for choosing the most rational modes of operation of equipment and optimal technological schemes of production, used as controlled parameters when creating automated control systems for machines, units, production sites, with automated product quality control. Rheology allows you to control the structure and quality of products by adding additives, changing modes and methods of mechanical and technological processing.

The structure formation of food materials, the study of structural and mechanical (rheological) properties, the development of methods and devices for their determination is the subject of rheology. The devices developed and proposed for introduction into the industry allow them to be installed locally or directly in technological equipment, including them in the technological process of processing raw materials. Operational collection of information on the quality of raw materials at the stages of their receipt, processing and release of finished products based on rheological characteristics using instruments - all this provides the ability to control, regulate and manage the quality of raw materials and finished products.

The most complete picture of the quality of a product is given by the properties determined by its structure. At the same time, it is important that small changes in the structure of the product cause significant fluctuations in those characteristics that are determined with the help of instruments. These requirements are best met by the structural and mechanical (rheological) properties of the product.

Structural and mechanical (rheological) properties of raw materials - shear (shear stress, viscosity, penetration, etc.), compression (adhesion, stickiness, plasticity, etc.) and surface (friction force, coefficient of external friction, etc.)

Using devices, it is possible to measure individual indicators (characteristics) of the structural and mechanical (rheological) properties of raw materials before, during and after a technological operation or a technological process. Further, comparing the obtained data with the optimal (reference) values ​​of the indicators, they are analyzed, determining the deviation, or to control. In cases of significant deviation of the indicators from the specified ones, perform operative intervention in changing the technological parameters of working off.

When using devices that allow measuring the controlled indicators directly in the technological process, it is possible to control the change in these indicators, and their changes to reach the required values, that is, to regulate them directly during the execution of the technological process.

The finished product quality depends not only on the used raw materials quality, but also on the quality of all technological processes at the stages of its production. This is possible with the installation of devices capable of monitoring changes in the parameters of raw materials in the flow, directly at all technological operations of the product production.

Collecting information on raw materials indicators from all operations of the production process and their further comparison with the specified (reference) indicators of finished products allow, in general, to monitor, regulate and manage these indicators. As a result of all the activities carried out, it becomes possible to obtain products that meet all the indicators of the standard, that is, to produce high-quality standard products.

At present, based on the results achieved in science, technology and engineering rheology, it becomes possible to create automated complexes, systems for the production of food products with standard quality indicators.

The rheology role of food products is that, using the structural and mechanical properties of products and instrumental (objective) methods and devices for operational control as control indicators, it becomes possible to ensure control, regulation and management of the quality of raw materials and finished products.

Control questions

1. The main tasks of the discipline «Rheology of food products»

2. The role of food rheology in ensuring the control, regulation and management of the quality of raw materials and finished products

Profitability- доходность

Economic efficiency - экономическая эффективность

Designing machines and apparatus – проектирование машин и аппаратов

Creep – ползучесть

Rheology – реология

Deformation – деформация

fluidity of matter – текучесть вещества

irreversible – необратимый

stress relaxation phenomena – явления релаксации напряжений

Liver mince – печеночный фарш

Pates – паштеты

Disperse system – дисперсная система

Suspension – суспензия

Colloidal solution –коллоидный раствор

Elastic-plastic-viscous materials - упруго-пластично-вязкиематериалы

Structure formation of food materials – структурообразования пищевых материалов

Shear – сдвиг, сдвиговые

Shear stress – напряжение сдвига

Viscosity - вязкость

Penetration- пенетрация

Compression- компрессионные

Adhesion - адгезия

Stickiness - липкость

Plasticity - пластичность

Surface- поверхностные

Frictionforce- сила трения

Coefficient of external friction - коэффициентвнешнеготрения

Operativeintervention - оперативное вмешательство

Deviation – отклонение

Lecture 2

Rheology as the science of deformation and flow of materials

Lecture plan:

1. Stages of development of food rheology

2. Components of food rheology

3. Nutritional materials as a subject of rheology study

The lecture purpose: to study theoretical issues of the science of deformation and flow of various bodies.

1. Stages of development of food rheology. Rheology is the science of deformation and flow of various bodies. The word «rheology» comes from the Greek «rheo», which means «flow». Deformation is understood as the relative displacement of the particles of a material body, in which the continuity of the body itself is not disturbed. If, under the action of finite forces, the deformation of the body increases in time continuously and irreversibly, then this means that the material is flowing. Deformation usually results in a change in the shape or size of the body. The subject of rheology in general is the study of various types of deformation, depending on the accompanying stresses.

In defining what is rheology, there are two views on the subject of rheology.

Rheology, in a broad sense, is the science of the deformation and flow of various materials.

Rheology, in a narrow sense, is the science of deformation and flow of materials that exhibit significant deviations in properties from classical models - Newt's liquids, Hooke's solid and St. Venant's plastic bodies.

The following stages are distinguished in the development of engineering rheology.

Classical rheology is the science of flow and deformation of real bodies, the task of which is to study the properties of existing products and develop methods for calculating their flow processes in the working bodies of machines.

Controlling rheology is a science whose task is to study and substantiate a combination of various types of influences, which ensure a given level of rheological characteristics throughout the entire technological process.

Physicochemical mechanics is the science of methods and patterns of formation of structures of dispersed systems with predetermined properties, the tasks of which include: establishing the essence of the formation and destruction of structures in dispersed and native systems, depending on the combination of physicochemical, biochemical, mechanical and other factors; research, substantiation and optimization of ways to obtain structures with predetermined technological properties.

Engineering physical and chemical mechanics of food production is a science about deformation and flow of products in working bodies of machines and methods of forming structures of dispersed systems with predetermined technological characteristics and develops on the basis of physical and chemical mechanics and rheology.

Rheology as the science of flow and deformation of real bodies was formed in connection with the study of materials, the mechanical properties of which are intermediate between the properties of elastic bodies and viscous liquids. Such materials also include food products - meat and meat products, milk and dairy products, fish and fish products, etc. Rheology also studies the flow of various other viscous-plastic materials, such as clay pastes and suspensions, greases, resins, mortars, etc.

The founder of physical and chemical mechanics is a Soviet scientist, academician P.A. Rebinder. A significant contribution to the development of physical and chemical mechanics and rheology was made by many domestic and foreign scientists M.P. Volorovich, G.V. Vinogradov, V.E. Gul, M. Reiner and many others. A.V. Gorbatov, V.D. Kosoi in the confectionery, bakery and other branches of the food industry Yu.A. Machikhin, S.A. Machikhin, B.A. Nikolaev and others.

2. Components of food rheology. Rheology of food products as a science includes a wide range of issues, a theoretical basis, various methods and instruments for measuring the rheological characteristics of food materials, methods of engineering calculations of parameters, development for optimization, intensification of technological processes, as well as for control and management of product quality.

In order to systematize the material, the rheology of food production is conventionally divided into four parts:

- Rheology of food materials - contains basic information on the theoretical part of the engineering rheology of food materials;

- Rheometry of food materials - is devoted to methods of measuring the rheological characteristics of food materials, instrumentation and presentation of measurement results;

- rheodynamic calculations - is devoted to the presentation of the obtained experimental data in the form of mathematical formulas, graphs (rheograms), mechanical models that quantify the results of changes in food materials as a result of the impact of the working organs of machines, as well as engineering calculations for technological equipment;

- rheological foundations of optimization, intensification, control and management of product quality - is devoted to the use of the results obtained in the course of experimental research on the development of issues: optimization of technological parameters, intensification of technological processes of production of products, quality control of products, as well as the development of issues of quality management of products.

Macro-rheology considers all materials in the form in which they appear to the observer when viewed superficially with the naked eye, i.e. as homogeneous and linear structures. However, only pure liquids and perfect crystals are phenomenologically homogeneous.

Microrheology examines the rheological behavior of two - and multiphase systems depending on the rheological properties of their components.

3. Food materials as a subject of rheology study. In scientific and industrial circles among specialists, as well as in the study of special disciplines in universities, technical schools, etc. The term «food materials» is widely used, which means real food products - raw materials of plant and animal origin (meat, milk, fish, vegetables, fruits, etc.), semi-finished products, mixtures, compositions (minced meat, minced fish, flour dough, confectionery masses, etc.) and finished products (sausages, dairy products, bread, pasta and confectionery, etc.). This term is used only to indicate that food, as an object of study, is also considered as a material, but only as a food material.

The purpose of considering this issue is (clarification) of what science food materials belong to from the point of view of their study. There is no definite answer to this question.

Food products or food materials, such as raw materials, semi-finished products, finished products, are mostly complex compositions consisting of two, three or more components in various proportions. Moreover, each component has an effect on the properties of the product to one degree or another and it is impossible not to take them into account or abstract from them. From a scientific point of view, food products (materials) belong to dispersed systems, the object of study of which is the engineering physicochemical mechanics of food production.

Food products (materials) differ significantly from others in that they are materials of plant and animal origin. In addition, they cannot be preserved for a long time in the form in which they were originally. This is especially true for meat and meat products. The study of such materials is engaged in the engineering physicochemical mechanics of food production.

Food products (materials), like others, have a complex of different properties, among which the group of physical properties gives the most complete picture of the structure of the product. The structures of the product, its change are determined by structural and mechanical properties, which are usually expressed by such characteristics (indicators) as elasticity, viscosity, plasticity, stickiness, etc. If we assume that the material has only one of these properties, for example, elasticity, viscosity or plasticity, then the study of such materials (see table 1.) is engaged in theoretical mechanics, resistance of materials, hydraulics. The behavior of such materials under the influence of external forces is described by the equations of Newton, Hooke, Saint-Venant and have a linear dependence.

Table 1

Classification of real bodies

| Perfectly solid body, Euclidean | Elasticbody, Gukovo | Plasticbody, Saint-Venant | Rheological body | True viscous fluid Newtonian | Idealliquid, Passkal |
| --- | --- | --- | --- | --- | --- |
| For any normal shear stresses, the deformation is zero | Stress proportional to deformation | When the ultimate shear stress is reached, plastic deformations begin | Linear - composite of bodies in columns 2, 3, 5 and nonlinear - empirical | Voltage is proportional to the velocity gradient to the first power | Zero viscosity and compressibility |
| Theoretical Mechanics |  | Strength of materials | Rheology | Hydraulics | Hydraulics |

Based on the studies carried out, it was found that most food materials when exposed to external loads have a slightly different behavior. Moreover, the nature of the change in elasticity, viscosity and plasticity has a non-linear relationship. At the same time, food materials can simultaneously have elasticity, viscosity and plasticity. This type of material is studied by rheology (see table 1.), therefore food products (materials) are the subject of rheology study.

Control questions

1. Stages of development of food rheology

2. Components of food rheology

3. Nutritional materials as a subject of study of food products

Relative displacement – относительное смещение

Particle – частица

Classical rheology- классическая реология

управляющая реология

Controlling rheology -	реология пищевых материалов

Rheometry of food materials -	реометрия пищевых материалов

Rheological foundations of optimization – реологические основы оптимизации

Intensification – интенсификация

Control and management of product quality –контроль и управления качеством продукции

Macrorheology - макрореология

Microrheology– микрореология

Lecture 3

Dispersion types. Types of structures and their classification

Lecture plan:

1. Types of dispersions.

2. Types of structures and their classification.

The purpose of the lecture: to get acquainted with the general classifications of real bodies

1. Types of dispersions. The subject of study of engineering rheology is food materials, which are considered as real bodies and occupy a specific place in the sections of technical mechanics of a continuous medium. The general classification of real bodies is presented in Table 1.

According to the classification, food products can be presented in various combinations of the main so-called «primary» bodies - elastic (Hooke's body), ideally viscous (Newton's body) and ideally plastic (Saint-Venant's body) bodies, which can be combined to simulate deformation characteristics lots of real food products. In this case, the behavior of rheological bodies is described not by simple ones, but by more complex ones - nonlinear and empirical equations, most often by power equations, connecting deformations and strain rates.

With the help of the main «primary» bodies, it is possible in various combinations and combinations to simulate the deformation characteristics of any real food products.

Types of dispersions. In engineering physicochemical mechanics, on which engineering rheology is based, the classical objects of study are dispersed systems.

A dispersed system is a system consisting of two or more phases, in which it is conventionally accepted to call: a continuous (layered) phase - a dispersion medium, and another, fragmented, consisting of particles not in contact with each other - a dispersed phase. From this point of view, food products in most cases, consisting of two or more components, are dispersed systems. The difference between a dispersed system and a homogeneous one is that in the first, the particles of the dispersed phase do not consist of individual molecules, but of conglomerates that have the thermodynamic properties of the phase. The main condition for the existence of dispersed systems is the insolubility or low solubility of a dispersed phase substance in a dispersion medium.

Dispersed systems for the purpose of their systematization are classified according to a number of essential features and for this they use one or another criterion reflecting one specific property. The most famous and widely used is the classification of dispersed systems, proposed by V. Oswald, which is based on the difference between the state of aggregation of the dispersed phase and the dispersed medium, is presented in Table 2. This classification allows you to cover almost all dispersed systems of meat products.

In addition to the above classification, dispersed systems are divided according to their structure: into free-dispersed and connected-dispersed systems. Free dispersed systems include dispersed systems in which phase particles can move freely in a dispersion medium (suspensions, emulsions, sols), and cohesive dispersed systems are dispersed systems in which one of the phases is structurally fixed and cannot move freely (emulsions, gels, pastes, jellies).

Table 2

Classification of dispersed systems

| Dispersion medium | Dispersed phase | System | System examples |
| --- | --- | --- | --- |
| Gas | solid liquid gaseous | aerosols aerosols-mists atmosphere | Smoke, dust, dispersion of blood, milk; atmosphere of earth |
| Liquid | solid liquid gaseous | suspension emulsion foam | broth, minced sausage, pates; blood, fat in water, milk; cream, whippedprotein |
| Solid | solid liquid gaseous | solid suspension - alloy capillary systems, solid emulsion porousbodies, rigidfoams | frozen muscle tissue; frozen butter, native muscle tissue, fluid in porous bodies; bone, cheese, insulating material, whipped and coagulated melange |

Depending on the particle size of the dispersed phase, systems are subdivided into coarse and highly dispersed.

Table 3.

Classification of food products by rheological properties

| Disperse system | Product (including raw materials, semi-finished product) | Typical rheological properties |
| --- | --- | --- |
| Pure liquid | Water, alcohol, oil | Newtonian viscosity |
| Pure melt | Melted fats (cocoa butter), melted sugar | Predominantly Newtonian viscosity |
| Truesolution | Saline and sugar solutions, extracts, beer, drinks | The same |
| Colloidalsolution | Protein solutions, cloudy fruit and berry juices | Newtonian viscosity, viscoelasticity, thixotropy possible |
| Liquid | Suspensions (cocoa, fruit and vegetable juices, soups), emulsions (milk, cream, mayonnaise) | Newtonian and non-Newtonian viscosity, thixotropy, viscoelasticity |
| Pasty | Fruit puree (apple mousse), nut mousse, cottage cheese, minced meat | Non-Newtonian viscosity, thixotropy, rheopexy, viscoelasticity |
| Bound soft | Butter, foam, jelly, dough, yoghurt, soup, pate, mashed potatoes | Plastic viscosity, reversible and irreversible thixotropy, elasticity, viscoelasticity |
| Bound semi-solid | Bread crumb, boiled sausage, boiled potatoes | Elasticity, plasticviscosity, viscoelasticity |
| Firm | Fresh apples, pears, potatoes, cucumbers, meat, shelf-stable bakery products, chocolate, sweets | Elasticity, plasticviscosity, viscoelasticity |
| Solid | Caramel, grain, nut kernels, pasta, carrots | Elasticity, hardness, high fluidity and strength, fragility |

2. Types of structures and their classification. To represent the material and as well as to carry out similarity or separation with other materials, the concept of structure is introduced.

Structure - the internal structure of a product (material) and the nature of the interaction between its individual elements (particles), which is determined by: chemical composition, biochemical indicators, temperature, dispersion, state of aggregation and a number of technological factors.

Academician P.A. Rebinder, applying a phenomenological approach to dispersed systems, conditionally divided all materials (masses) into two types, into sols - substances that behave like liquids and gels - substances with the properties of solids.

According to the classification proposed by academician P.A. Rebinder, food products structures are divided: into coagulation and condensation-crystallization.

Coaculation structures are formed in dispersion systems by the interaction between particles and molecules through interlayers of a dispersed medium due to van der Waals cohesion forces. In such structures, the particles are linked together by molecular cohesion forces, forming a continuous network or framework. The strength of the framework depends on the presence of a solvation (hydration) layer on the surface of the particles. Distinctive features of such structures are: relatively low bond strength between particles; the ability to spontaneously restore the structure after destruction, i.e. thixotropy; possess elasticity, plasticity, viscosity and other properties. When dehydrated, their strength increases, and after a certain limit they cease to be reversible thixotropic. Typical representatives of systems with a coagulation structure are raw meat and liver sausages, etc.

Condensation-crystallization structures.They are inherent in natural products (meat and meat products with a preserved cellular structure, etc.), but can be formed from coagulation when the dispersion medium is removed or particles of the dispersed phase grow together during heat treatment (coaculation or denaturation of proteins), as well as when melts are cooled or the concentration of solutions is increased. Distinctive features of this type of structures are: greater strength in comparison with coagulation; lack of thixotropy and irreversible destruction; high fragility and elasticity due to the rigidity of the skeleton of the structure; the presence of internal stresses arising during the formation of phase contacts and subsequently entailing recrystallization and a spontaneous decrease in strength up to discontinuity, for example, cracking during drying.

Subsequently, on the basis of research on meat and dairy products, the classification was supplemented by A.V. Gorbatov with two more transitional types of structures, which were named: coagulation-crystallization and coagulation-condensation.

Thus, the type of product structure determines its quality and technological parameters and behavior in deformation processes. To describe the deformation behavior of structures, flow curves are used - rheograms, which relate stress and strain rate (deformation) to each other. The nature of rheograms, as a rule, makes it possible to refer a real product to one or another type of rheological bodies.

Control questions

1. Name the types of dispersions.

2. List the types of structures and their classification.

Dispersion - дисперсия

Elastic (Hooke's body) body -упругое (телоГука) тело

Ideally viscous (Newton's body) body - идеально-вязкое (телоНьютона) тело

Ideally plastic (Saint-Venant's body) body - идеально-пластичное (телоСен-Венана) тело

Nonlinear - нелинейный

Empirical equation - эмпирическиеуравнение

Particles - частицы

Continuous - непрерывный, постоянный

Dispersionmedium - дисперсионнаясреда

Dispersedphase - Дисперснаяфаза

Residual - остаток, остаточный

Shearstress - напряжение сдвига

Lecture 4

Basic terms and definitions of rheology, axioms of rheology

Lecture plan:

1. Axioms of rheology

2. Types of deformations.

The purpose of the lecture: to get acquainted with the axioms of rheology

1. Axioms of rheology.Rheology, like any other science, relies on a number of axioms, assumptions, assumptions and other restrictions, which are caused by the need to absolve from secondary issues.

First axiom. Under the action of all-round (isotropic) pressure, all materials behave in the same way - like ideally elastic bodies.

This means that an all-round uniform pressure will have the same effect on a ball made of steel or a ball made of minced meat (dough). The density of both balls will increase without distorting the shape. The change in diameters will occur due to elastic deformation, i.e. when the pressure is removed, the diameter and density of these bodies will return to their initial values. Differences in rheological properties appear only during deformation that changes the shape of the body - deformation of shape change.

Second axiom.Each material has all the rheological properties, albeit to varying degrees. The main rheological properties are elasticity, plasticity, toughness and strength.

This means that one and the same material, depending on its state and loading conditions, can manifest to a greater or lesser extent different rheological properties. For example, such a viscous-plastic material as pasta dough, under an instantaneous action of the load, basically behaves like an elastic body, and plastic deformation and viscous flow are almost absent. Under other loading conditions, plastic and viscous properties are of greater importance. Therefore, first of all, it is necessary to find out which properties of the investigated material under the given conditions are the main ones.

Let's consider the basic physical, mechanical and mathematical concepts used in rheology.

2. Types of deformations. When an external load is applied to the material, it is subjected to stress, which is expressed in a change in its size and shape. These material changes are commonly referred to as deformation. Depending on the application of the load, the deformations are fundamentally divided into two types: the first are deformations of volumetric (linear) tension-compression and the second are shear deformations. In the first case, only the volume (linear size) of the material changes, and its shape does not undergo noticeable changes. With shear deformation, the shape of the material changes, but its volume remains the same. There is a close relationship between these types of deformations, which is determined by the Punch's ratio. The ability to deform under the influence of external forces is the main property of the materials of all real bodies.

Deformation is a change in the shape and (or) linear dimensions of a body under the influence of external forces, with a change in humidity, temperature, etc., in which particles or molecules move relative to one another without disrupting the continuity of the body. The magnitude and nature of the deformation depend on the method of application of external forces, the properties of the material of the body and its shape.

As you know from the course «Resistance of Materials», deformation is divided into two types: a) reversible (elastic), which disappears after the cessation of the force; b) plastic (viscous), which does not disappear after removing the load. During elastic deformation, part of the mechanical energy is converted into heat. In addition, there is another division of deformations. For example, by the type of loading, they can be shear (Fig. 1 a), uniaxial (linear) (Fig. 1. b), as well as biaxial (flat) and volumetric. In this case, the deformations are calculated by the equations:

; .						(1.1)

τ

а

Δ

γ

Δl/2

Δl/2

l

F

F

а)							б)

Picture 1. Material loading schemes:

- shift; b) stretching

If the deformations change in time t during an unsteady process, then the deformation rate (rate gradient) is taken into account; with a steady deformation process, the change in deformation per unit time is constant. All this is described by the concept of «strain rate» - [s – 1], in tension - compression and [s – 1], in shear:

; .				(1.2)

If the shear strain under the action of finite forces increases continuously and continuously, then the material begins to flow. The steady-state flow regime is characterized by a velocity gradient, which is similar in meaning to the strain rate (Fig. 1.2):

dy

u

y

du

Picture 1.2. Steady-state material flow diagram

τ

0

,				(1.3)

Where: τ – shear stress, Pa;

u – linear velocity of an elementary layer, m / s;

y – layer coordinate along the normal to the velocity vector, m.

In rheology, two types of flow are distinguished: 1.viscous flow - is realized in truly viscous, Newtonian fluids at any arbitrarily small shear stressesτ. This flow is described by Newton's equation:

or,				(1.4)

Where:η – coefficient of dynamic or absolute viscosity, which characterizes the magnitude of the forces arising between two elementary layers of a liquid during their relative displacement, Pa ∙ s;

F – resistance force between two elementary layers, Н;

A –surface area of the resistance of these layers, m2;

2. Plastic flow - flow at stress τ equal to the yield pointτТ.

Stress is a measure of internal forces F [N] arising in a body under the influence of external influences on a unit area A [m2] normal to the force application vector:

, Pa.					(1.5)

Stress at the point of the loaded body:

.					(1.6)

Formally, shear or tangential stresses (τ), normal stresses (σ), pressure, or hydrostatic pressure (p), and adhesion, or sticking pressure (p0), are described by dependence (1.5) where the force and area will have the corresponding physical meaning.

The study of various types of deformations, depending on the accompanying stresses, is the subject of rheology.

Control questions

1. What are the rheology axioms?

2. List the types of deformations.

3. What rheological properties of food do you know?

Axioms of rheology –аксиомы реологии

Elasticity - упругость, эластичность

Plasticity - пластичность, гибкость

Toughness -прочность

Strength - сила, прочность

State -состояние

Pasta Dough – макаронное тесто

Change in size and shape – изменение размеров и формы

Tension-compression-растяжения-сжатия

Shear deformations – деформация сдвига

Punch's ratio – коэффициент Пуансона

Reversible (elastic) deformation - обратимая (упругая) деформация

Lecture 5

Rheological models of simple «ideal» bodies.

Basic equations of stresses and deformations of «ideal» bodies

Lecture plan:

1. Mechanical model of an «ideal» elastic body.

2. Mechanical model of an «ideal» viscous body.

3. Mechanical model of an «ideal» plastic body.

The purpose of the lecture: to get acquainted with rheological models and basic equations of stresses and deformations of «ideal» bodies

In rheology, two mutually exclusive concepts are distinguished: «solid ideal elastic body» and «inviscid fluid». The first is understood as such a body, the equilibrium shape and tension of which are achieved instantly. The fluid is called the residual, i.e. if the fluid is unable to create and maintain shear stresses. Between the limiting states of bodies - ideally elastic solids and non-viscous liquids - in nature there is a huge variety of bodies of an intermediate nature.

Let us consider the basic models that can be encountered in the study of the rheological properties of food masses. At the same time, it is necessary to point out that exact mathematical regularities were obtained only for Newtonian fluids, for all non-Newtonian flows, only approximate formulas were obtained.

There are three intermediate models of idealized materials (table 1.3): ideal elastic body (Hooke); ideal viscous fluid (Newton); ideal plastic body (Saint-Venant).

1.Hooke's perfectly elastic body. In an ideally elastic body (the model is a spring), the energy spent on deformation is accumulated and can be returned during unloading. Hooke's law describes the behavior of crystalline and amorphous solids at small deformations, as well as liquids at isotropic expansion - compression.

2.Ideally viscous liquid of Newton. An ideal-viscous fluid is characterized by the fact that the stresses in it are proportional to the rate of deformation. A viscous flow occurs under the action of any forces, no matter how small they are; however, the deformation rate decreases with decreasing forces, and when they disappear, it vanishes. For such fluids, the viscosity, which is constant, is proportional to the shear stress.

Newton's Law describes the behavior of many low molecular weight fluids in shear and longitudinal flow. The mechanical model of a Newtonian fluid is a damper consisting of a piston that moves in a cylinder with fluid. When the piston moves, liquid flows through the gaps between the piston and the cylinder from one part of the cylinder to another. In this case, the resistance to movement of the piston is proportional to its speed (see table 1.).

3.The ideally plastic body of Saint-Venantcan be represented as an element consisting of two plates pressed against each other. With the relative displacement of the plates, a constant friction force arises between them, which does not depend on the force compressing them. The body of Saint-Venant will not begin to deform until the shear stresses exceed a certain critical value - the yield point τТ (ultimate shear stress), after which the element can move at any speed.

Table 1.

Rheological models of simple idealized bodies

| Model | Model types | Flowgraphs | Equation | Symbols |
| --- | --- | --- | --- | --- |
| σ σ τ γ α Hook |  |  |  | –shear and normal stresses, Pa; –angular and linear deformations; G, E–elastic moduli at angular and linear deformation, Pa. |
| τ α τ τ Newton |  |  |  | –shear rate, s–1; –shear viscosity, s–1; –longitudinal flow velocity, s–1; –longitudinal flow viscosity (Truton), Pa · s. |
| τ τТ τ τ Saint-Venant |  |  | atτ<τТno deformation; atτ = τТ flow | –shearyieldstress, Pa. |

In practice, mechanical models of «ideal» bodies and their basic equations are used to describe the behavior, properties of real food materials, liquids, which are quite close in properties to them. However, in most cases this is impossible due to the fact that food materials are rather complex compositions that can simultaneously have two, three or more properties.

Control questions

1. Mechanical model of an «ideal» elastic body.

2. Mechanical model of an «ideal» viscous body.

3. Mechanical model of an «ideal» plastic body.

Lecture 6

Rheological models of complex real bodies.

Basic equations of complex rheological bodies

Lecture plan:

1. Mechanical model of a viscoelastic body with relaxation of deformations

(Maxwell's bodies).

2. Mechanical model of a viscoelastic body with stress relaxation

(Voigt-Kelvin bodies).

3. Mechanical model of Shvedov-Bingham viscous-plastic body.

The purpose of the lecture: to get acquainted with rheological models and basic equations of complex rheological bodies

In order to describe the rheological behavior of a complex body, depending on the properties of its components, it is possible to combine in various combinations the models of the simplest ideal bodies considered above, each of which has only one physical and mechanical property. These elements can be combined in parallel or in series.

The main complex models are: elastic-plastic body; visco-elastic bodies of Kelvin - Foig and Maxwell; viscous-plastic bodies of Bingam, Shvedov and Shvedov - Bingam).

The model of an elastic-plastic body (Fig. 2., a) is obtained by sequentially connecting an elastic Hooke element with a modulus of elasticity G and a Saint-Venant plastic element with a yield point τt. At τ<τt, elastic deformation of the material occurs, and at τ = τt, plastic flow.

The viscoelastic Kelvin - Voigt body is represented by a mechanical model obtained by parallel connection of Hooke's elastic element with elastic modulus G and Newton's viscous element with viscosity η (Fig. 2., b). The tensile force lengthens the spring and moves the piston through the fluid. This movement of the piston is associated with the viscous resistance of the fluid, so that the full extension of the spring does not occur immediately. When the load is removed, the spring compresses to its original length, but this takes time due to the viscous resistance of the fluid.

To write a mathematical model of the Kelvin - Voigt body, the fact is used that with parallel connection of elements, the deformation of a complex body γKF is equal to the deformation of each element, and the stress of the total element τKF is equal to the sum of stresses in individual elements τG and τN. Basedonthis, wehave a systemofequations:

We will use the previously written mathematical models for the Hooke (G) and Newton (H) elements:

Having considered together, we get the final mathematical rheological model of the Kelvin - Voigt body:

where: G - shear modulus, Pa;

γ - angular deformation;

η - Newtonian viscosity, Pa ∙ s.

G

τТ

а)

G

η

б)

G

η

в)

η

τТ

г)

д)

η

τТ

G

е)

η

τТ

G

G

Figure 2. Mechanical models of rheological materials a) a model of an elastic-plastic body; b) the Kelvin-Foig model c) Maxwell's model; d) Shvedov-Bingham viscous-plastic body model; e) Bingham model; f) Shvedov's model

The Kelvin - Voigt body model reflects the phenomenon of elastic aftereffect, which is a change in elastic deformation in time, when it either constantly increases to a certain limit after the application of the load, or gradually decreases after it is removed.

The mechanical model of Maxwell's viscoelastic relaxing body (Fig. 2, c) is a series connection of Hooke's elements with elastic modulus G and Newton's with viscosity η. Both elements are affected by the same voltage τ.

Maxwell's body behaves as elastic or viscous, depending on the ratio of the relaxation time of the material to the duration of the experiment. So, if, under the action of an instantaneous force, the spring is stretched, and then immediately the load is removed, then the piston does not have time to move and the system behaves like an elastic body. However, on the other hand, if the tension of the spring is kept constant, it gradually relaxes, moving the piston upward, and the system behaves like a Newtonian fluid. The rheological equation of Maxwell's body is:

.

The two-element mechanical model of the Shvedov-Bingham visco-plastic body (Fig. 2, d) consists of Newton's elements with a viscosity η and Saint-Venant with a yield point τТ connected in parallel. If τ ≤ τТ, then the system behaves like an absolutely rigid undeformed body. The rheological equation of this body at τ>τТ has the form:

.

There are materials in nature that, as a first approximation, can be regarded as the body of Saint-Venant. They begin to flow when the shear stress reaches its ultimate value. If there is no viscous resistance, then the flow rate of the material will become arbitrarily high. This shows that such materials can only be considered as the bodies of Saint-Venant in a first approximation. In the second approximation, they should have a viscosity. All this leads to the postulation of the ideal Bingham body, combining elasticity, viscosity and plasticity.

Bingham's mechanical model (Fig. 2, e) consists of Hooke's elements with elastic modulus G, Newton's with viscosity η, and Saint-Venant's with yield point τТ. The elements of Newton and Saint-Venant are mutually connected in parallel, and together - in series with the element of Hooke.

Under stress τ<τТ, the Bingham model has only elastic deformation. The rheological equation of this model at τ>τТ has the form:

.

Shvedov's mechanical model consists of Hooke's elements with elastic modulus GН, Saint-Venant with yield point τТ and Maxwell elements with elastic modulus GM and viscosity η (Fig. 2, f). In the 80s of the 19th century F.N. Shvedov studied relaxation processes in colloidal solutions and for the first time discovered their elasticity and viscosity. The model of this body differs from the Bingham model in that the Maxwell model is attached parallel to the Saint-Venant model, and the Newton element is attached to the Bingham model.

At τ ≤ τТ, the deformation of the Shvedov model occurs only due to the Hooke element. At τ>τТ, all elements of the model are deformed. The rheological equation of the Shvedov model in differential form is:

.

The desire of researchers to more accurately represent the behavior of food materials under load has led to the creation of complex models, which significantly increased the complexity of the calculations. Models with a small number of elements rarely give satisfactory convergence of experimental data with those calculated from equations. At the same time, an increase in the number of elements over four does not lead to a significant qualitative change in the model, since models containing up to four elements, inclusive, exhaust the entire variety of mechanical behavior of a given material.

Modeling the formation behavior of food materials can be carried out not only on the basis of mechanical models, but also electrical ones. In this case, the voltage is compared with the emf. electrical circuit, the rate of deformation - with an electric current, the modulus of elasticity - with the reciprocal of the capacitance, and the viscosity - with resistance. A series connection of elements of a mechanical model is equivalent to a parallel connection of elements of an electrical circuit, and a parallel connection in a mechanical model is equivalent to a series connection in an electrical one. Electrical modeling makes it possible to use modeling computers in the study of the elastic-visco-plastic properties of food materials, as well as in the calculation of the processes of their processing.

Control questions

1. Retell the mechanical model of a viscoelastic body with relaxation of deformations (Maxwell's body).

2. Retell the mechanical model of a viscoelastic body with stress relaxation (Voigt-Kelvin body).

3. Retell the mechanical model of the Shvedov-Bingham viscous-plastic body.

Utimate shear stress - предельное напряжение сдвига

A constant friction force – постоянная сила трения

Shear modulus - модуль упругости при сдвиге

Angular deformation – угловая деформация

Newtonian viscosity – ньютоновская вязкость

Voltage – напряжение

Yield point – предел текучести

Electrical circuit – электрическая цепь

Electric current – электрический ток

Capacitance – емкостное сопротивление

Resistance - сопротивление

Flow curves – кривые течение

Lecture 7

Flow curves as a tool for describing the rheological properties of materials

Lecture plan:

1. Curves flow

2. Thixotropic and rheopex systems

The purpose of the lecture: to get acquainted with flow curves as a tool for describing the rheological properties of materials

The simplest method for studying the structural and mechanical properties of food materials is to construct curves of the kinetics of deformation (flow curves). Based on these curves, one can find seven independent deformation characteristics of the material: moduli of instantaneous elasticity and elastic aftereffect; viscosity of relaxation (flow) and elastic aftereffect; limits of elasticity, yield and strength. The ultimate strength value is not invariant, since it depends on the mechanical deformation mode. The listed constants make it possible to explain the deformation behavior of the material and to characterize its structural and mechanical properties rather fully. Obtaining such characteristics is possible in the process of studying the rheological properties of food masses, i.e. when studying the process of their flow under the influence of constant voltage.

The flow curves (rheograms) of Newtonian fluids are a straight line 1 passing through the origin. All flow curves (2 - 5) that deviate from a straight line are called non-Newtonian fluids. In this case, curve 2 characterizes the dilatant flow, which is characteristic mainly of concentrated dispersed systems, in which, with an increase in the deformation rate, “shear hindered” occurs; there is an increase in viscosity; curve 3 describes a pseudoplastic flow, which is characteristic of «shear softening» due to the destruction of the structure with an increase in the strain rate; curve 4 shows the nonlinear plastic flow characteristic of most plastic bodies after reaching the limiting shear stress θ0. Linear dependence 5 is characteristic of Bingham bodies and corresponds to an ideal plastic flow after reaching the limiting shear stress θ0.

The flow curves (Fig. 1) graphically depict the laws of behavior of various materials, i.e. dependencies of the form.

5

4

3

2

1

τ

τ0

τ0

Fig.1. Flow Curves:1 – Newtonian fluid; 2 - dilatant liquid; 3 - structurally viscous liquid; 4 - nonlinear plastic body; 5 - linear plastic body

Thixotropic systems are characterized by isothermal restoration of the structure after destruction, as well as its continuous destruction (up to a certain limit) during deformation (Fig. 2, a). Rheopex systems are able to structure, i.e. to form contacts between particles as a result of orientation or weak turbulization under mechanical action with small velocity gradients (Fig. 2, b).

τ

θ

б)

τ

а)

Fig.2  Curves of flow characterizing: a) thixotropic systems; b) rheopex systems

A feature of many pseudoplastic and plastic-viscous structured dispersed systems of the coagulation type is the presence of hysteresis loops (Fig. 2) during loading and unloading. The area of ​​the rheogram between the curve and the y-axis represents (on the appropriate scale) the specific power (per unit volume in W / m3). It consists of the power of the Newtonian flow and the power required at the same velocity gradient to achieve a given degree of destruction of the structure. The power proportional to the area between the two curves forming the hysteresis loops characterizes the degree to which the structure approaches the equilibrium state.

In many processes, the product is exposed to intense mechanical stress (pumps, mixers, etc.), i.e. its structure reaches partial or almost extreme destruction. Therefore, when using the results of rheological studies for practical calculations, one should at least approximately choose the flow curve that corresponds to a given degree of destruction. In accordance with this, when calculating various processes, it is necessary to use the characteristics determined in the corresponding range of stresses and strains. A qualitative assessment of the product must also be carried out according to the characteristics that are most essential for this process.

Control questions

1. Application of rheological models to describe the properties of real food masses (products)

2. Mechanical models of real food materials

Pseudoplastic flow – псевдопластическое течение

Dilatant flow – дилатантное течение

Hysteresis loops - петли гистерезиса

Plastic (viscous) deformation - пластическая (вязкая) деформация

Shear stress-напряжение сдвига

Shear or tangential stresses - сдвиговые или касательные напряжения

Sticking pressure – давление прилипания

Science –наука

A viscous-plastic material-вязко-пластичныйматериал

External load – внешняя нагрузка

Volume-объем

Humidity-влажность

Deformation rate –скорость деформации

Lecture 8

Structural and mechanical characteristics of food products as an objective indicator of impact

Lecture plan:

1. Basic structural and mechanical properties of food materials

2. Shear properties of food materials

The purpose of the lecture: to get acquainted with the structural and mechanical characteristics of food

The main structural and mechanical properties of food materials.Product quality is understood as the totality of product properties that determine its suitability to meet certain needs in accordance with its purpose. The most complete picture of the quality of a product is given by the properties determined by its structure.

Structure - the internal structure of the product and the nature of the interaction between its individual elements (particles), which is determined by the chemical composition, biochemical indicators, temperature, dispersion, state of aggregation and a number of technological factors.

To conduct a general assessment of the quality of a product, a complex of its properties is used: chemical, biological, physical, electrophysical, optical, etc. Among them, a complex of physical properties, the so-called structural-mechanical, predetermine the behavior of products in a wide variety of technological processes and energy fields. They are a qualitatively external expression of the inner essence of objects, i.e. determine the state of aggregation, dispersion, structure, structure and type of interactions within the product. In quantitative terms, structural and mechanical properties are presented in the form of characteristics, i.e. values ​​of the corresponding physical quantities in the form of the units of measurement adopted for them. Numerical values ​​are represented as the ratio of the value of a physical quantity to the unit of its measurement, i.e. dimensionless numbers.

Structural and mechanical characteristics (SMC) qualitatively and quantitatively determine the behavior of a product under stress conditions and allow one to relate stresses, deformations, or strain rates during the application of forces. They are not “pure” material constants and depend on the shape and size of the body, loading rate, surface condition, environmental impact, temperature, structure, and many other factors. With known values ​​of the characteristics, it is possible to calculate the values ​​of stresses and strains and, as a result, obtain the necessary parameters of the process or apparatus, perform strength and technological calculations. That is, the structural and mechanical characteristics of food materials act as an objective indicator of any impact. In addition, the property of a product as an objective reality allows us to characterize its quality.

Basic structural and mechanical properties of food materials. Structural and mechanical properties, according to the type of force (load, stress) applied to the product, are divided into three related groups: shear, compression and surface.

The main shear rheological properties of materials include - ultimate shear stress, Pa s, effective viscosity, Pa s, plastic viscosity, Pa s, relaxation period, s. They represent a group of properties that most fully reflect the inner essence of the material (object) and therefore they are considered to be basic. With their help, the flow of materials in technological pipelines, working bodies of machines and apparatus is calculated, and the necessary efforts to move the product are determined. In addition, they allow you to judge the quality of the product and the degree of its processing, i.e. make it possible to substantiate the optimal technological and mechanical conditions of the process, and the instrumentation allows them to control and regulate, ensuring constant and stable quality.

The main compression (volumetric) properties of materials include: modulus of elasticity of the first kind , Pа; modulus of elasticity of the second kind G, Pa, equilibrium modulus , Pа; strain relaxation period at constant stress ; s, relative deformation ; volumetric strain ; densityρ, kg/m.

These characteristics are used to calculate the processes of extrusion, molding, dosing, transportation through pipelines, etc., as well as to assess the quality of products.

The main surface properties are - adhesion Pа, stickiness , Pа, external friction coefficient .

They characterize the force during the interaction of materials between the contact surfaces (adhesion) during normal pull-off or shear, which is determined by the pull-off method. At the same time, the separation of food materials from each other can be adhesive, cohesive and adhesive-cohesive (mixed).

2. Shear properties of food materials. There is also a group of systems with time-varying properties: thixotropic, in which the shear stress and effective viscosity decrease during mechanical action, and rheopex, in which the shear stress and effective viscosity increase with time in the case of action on the system of shear stresses at a constant shear rate gradient ...

P.A. Rebinder and N.V. Mikhailov proposed to divide rheological bodies into liquid and solid, depending on the nature of the ηEF (τ) curve (Fig. 1.3) and on the relaxation period (the relaxation period is the time during which the stress in the loaded body decreases by e = 2.7 times). Liquid-like bodies include Newtonian fluids and structured systems that do not have a static limiting shear stress (θ0 ST = 0), i.e. such systems flow when an arbitrarily small external influence is applied.

Solid-like bodies include elastic-plastic and other bodies with static and dynamic ultimate shear stress. The dependence of effective viscosity on stress or shear rate is considered the main characteristic of the structural and mechanical properties of dispersed systems, since the effective viscosity is the final characteristic describing the equilibrium state between the processes of destruction and restoration of the structure in a steady flow. In general, the flow curve has an S-shaped character and cuts off a segment on the abscissa axis, within which the stresses acting on the body cause only elastic or elastic deformations.

The most important shear properties of structured systems are plastic ηPL and effective viscosity ηEF (τ) and relaxation period τР (τ); the highest viscosity (η0) of the undestroyed structure during «sliding» of the contact points and the viscosity of the ultimately destroyed structure (ηm); shear moduli (G); yield limits conditionally static (τST) and dynamic - ultimate shear stress (θ0); structural strength at elastic-brittle or elastic rupture (τm) and elastic-ductile fracture (τr). These characteristics are shown in Fig. 1.3. In addition to the characteristics listed above, the following zones can be distinguished on the curve: 0A - elastic deformation zone; AB - zone of the beginning of the flow with the highest effective and plastic viscosity; ВС - the beginning of the zone of avalanche-like destruction of the structure; СD - zone of avalanche-like destruction of the structure (flow with the lowest plastic viscosity); E and above - zone of Newtonian flow with constant viscosity of the extremely destroyed structure.

Control questions

1. Basic structural and mechanical properties of food materials

2. Shear properties of food materials

3. Shear characteristics of solid meat products

Chemical composition –химический состав

Biochemical indicators – биохимические показатели

Ultimate shear stress- предельное напряжение сдвига

Effective viscosity-эффективная вязкость

Plastic viscosity-пластическая вязкость

relaxation period-период релаксации

modulus of elasticity-модуль упругости

equilibrium modulus-равновесный модуль

constant stress-постоянные напряжение

relative deformation-относительная деформация

volumetric strain- объемная относительная деформация

density-плотность

extrusion-шприцевание

molding-формование

dosing-дозирование

transportation through pipelines- транспортирования по трубопроводам

Lecture 9

Compression and surface properties of food grade materials

Lecture plan:

1. Compressive properties of food masses.

2. Surface properties of food materials

The purpose of the lecture: to get acquainted with the compression properties of food materials

Volumetric or axial compressions, as well as axial tension are the main types of mechanical deformation of products, which they undergo in a number of machines, syringes, tops, presses, dispensers, etc. In this case, the products are at elevated pressures, which change their initial volume and density. At the exit from the machine, the volume and density may also differ. These changes are determined based on the compression characteristics.

1. Compression properties are used to calculate the working bodies of machines and devices and to assess the quality of the product, for example, in tension - compression. These include the lateral pressure coefficient, Punch's ratio, elastic moduli, etc. In addition, a number of mechanical models given above (Maxwell, Kelvin, etc.) describe the behavior of the product under axial or volumetric deformation.

Density, as one of the compression properties, is an essential characteristic when calculating a number of machines and apparatus and when assessing the quality of a product. The average density ρ, kg / m3, for a relatively small volume is determined from the ratio:

ρ = М / V,					(1.24)

where: М – product mass, kg;

V – product volume, m3.

The true density is equal to the limit of the ratio of mass to volume, when the latter tends to zero.

There is a simple relationship between density ρ and specific gravity γ [N / m3]:

γ = ρ ∙ g,						(1.25)

where: g – free fall acceleration, m / s2.

The density of a mixture of several components, when they do not interact, in which the composition or volume of the mixture changes, can be calculated from the dependence:

or,		(1.26)

where: сi – concentration of one of the mixture components, kg per 1 kg of the mixture;

ρi – component density, kg / m3;

i – number of components.

2. Surface properties of food materials.

A special place among the structural and mechanical properties is occupied by surface properties (adhesion, cohesion, coefficient of friction). They characterize the force of interaction between the working surfaces of the equipment and the processed product during separation or shearing.

In the process of technological processing, food materials are in contact with the surfaces of various working bodies of machines, transporting devices, etc. The mass flow nature through the channels of various types of forming machines (screw, roller, gear, etc.), as well as through the process pipelines is determined by its structural and mechanical properties and by the forces of adhesion to the contact surfaces.

Adhesion is the adhesion of the surfaces of two dissimilar materials. This phenomenon is often found in nature and is widely used in technology. Cohesion is understood as the cohesion of particles inside the body under consideration. Food materials are characterized by different types of tearing (Fig. 1.): a) adhesive; b) cohesive; c) mixed - adhesive-cohesive.

In some cases, it is difficult to establish the destruction boundary for two or more phase systems. After peeling off, the surface of the plate can be moistened with a dispersion medium or covered with the thinnest film of a finely dispersed fraction of the investigated product.

а)				б)				в)

Fig. 1.1. Types of material tearing:

a) adhesive; b) cohesive; c) mixed - adhesive-cohesive

There is still no general quantitative theory of adhesion, although attempts to comprehensively explain adhesion on the basis of various interaction mechanisms are very fruitful. In this sense, the fundamental works of Academician P.A. Rebinder on the adsorption and surface activity of thin films. As a result of witty and subtle experiments V.A. Pchelin established surface tension, dielectric constant, surface potential, etc. for solutions of protein substances. In the phenomenon of adhesion of protein substances, as follows from the theoretical concepts of B.V. Deryagin, in addition to van der Waals forces of attraction, electrostatic forces due to the appearance of a double electric layer on the surface are involved.

It is customary to characterize the amount of adhesion of two bodies: by the force of separation; specific work of separation per unit area; the time required to break the bond between the substrate and the adhesive under a certain load. Themostcommontestmethodsare:

a) uneven separation, which makes it possible to identify changes in the values of adhesion strength in certain areas of the test specimen;

b) uniform separation, at which the value of the force required to separate the adhesive from the substrate is measured simultaneously over the entire contact area;

c) the shift of one material relative to another.

Adhesion is often characterized by the minimum force required to pull off. This value is called adhesion strength, adhesion pressure (stress), adhesion pressure or specific adhesion.

The formation of an adhesive bond is greatly influenced by the rheological properties of the adhesive, the cleanliness of the substrate surface and its topography, the duration of contact between the adhesive and the substrate, the contact pressure, the temperature of the adhesive and the substrate, and the rate of separation from the substrate.

During the operation of equipment, as well as in the design and creation of new machines, it is necessary to take into account the adhesive phenomena in order to choose the right material for parts or coatings, and to establish the optimal operating mode. For example, in the production of soft varieties of sweets from praline masses, butter fudge and a number of others, depending on the purpose of certain organs of the machine, it is necessary either to increase their adhesive interaction, or to achieve minimal adhesion. So, if in the feeding zone of the forming machine the adhesion of the mass to the walls should be the least, then in the screw chamber - the greatest. The surface of the screw, in contrast to the above, should be smooth, made of a material that adheres least to the mass. The roller blower is characterized by the maximum increase in the forces of interaction of the mass with the surface of the rolls, which increases the efficiency of the machine.

Although until now the nature of adhesion has not been identified, there are several theories that explain the physicochemical essence of adhesion phenomena:

1) according to the adsorption theory of Debreun and McLaren, adhesion is associated with the action of intermolecular forces: physical - van der Waals or chemical, for example, covalent ionic;

2) according to the electrical theory of B.V. Deryagin and N.A. Krotova - with a potential difference at the border of dissimilar bodies, i.e. with the appearance in the contact zone of a kind of electric molecular capacitor caused by a double electric layer;

3) in the electromagnetic field - with electromagnetic interaction, i.e. radiation and absorption of electromagnetic waves by atoms and molecules, which can be realized in condensed bodies;

4) according to the electro-relaxation theory of N.M. Moskvitina - with a double electric layer and a tear-off rate, the measurement of which causes the appearance of a deformation component of the force or work of destruction associated with the rate of relaxation processes in the broken joint;

5) according to the diffusion theory of S.S. Voyutsky and B.V. Deryagin - with diffusion of the ends of macromolecules through the boundary of the initial contact, as a result of which, in the limiting case, the phase boundary may disappear; similar to this mechanical theory, according to which the adhesive contact is formed due to the mechanical engagement of molecular or supramolecular formations with microroughnesses of the surface;

6) according to the thermodynamic theory - with surface tension, which causes, according to Dupre's rule, the work of replacing the «solid - liquid» interface with the «solid - gas» surface, which is realized when the disk is detached from the product.

Formally, adhesion is defined as the specific force of normal separation of the plate from the product:

р0 = F0 / A0,

where: F0 – breakout force, Н;

A0 – geometric area of the plate, m2.

It is difficult to separate the forces of friction and adhesion arising from the relative displacement of the contacting surfaces of two bodies. The relationship between friction and adhesion forces is determined by the Deryagin equation:,

where: F – external friction force, Н;

μ – true coefficient of friction;

N – normal load, Н;

А0 – true contact area, m2;

р0 – specific adhesion acting on areas of the area A0, Pa.

External friction is the interaction between bodies at the boundary of their contact, which prevents their relative movement along the contact surface. It depends on the effort of normal contact and stickiness, is determined by the two-term law of B.V. Deryagin:

FТР = f ∙ (FК + р0 ∙ A0),

where: f – true coefficient of external friction;

FК – force proportional to the shear surface.

If the values in equation (1.20) are divided by the area of true contact A0, then we get the following equations:

FТР = f ∙ (FК + р0), orFТР = f ∙ FК + θ*0.

In some cases, it is more convenient to operate with the effective coefficient of external friction, which is calculated from the dependence:

fЭФ = FТР / FК,

and may be related to the value of the true coefficient of friction:

fЭФ = f ∙ (1 + р0 / рК).

The equation can be applied to calculate both the net friction and the friction on the surface film of the material. Then θ * 0 can be identified with the limiting shear stress in the film, f ∙ FK is the shear stress caused by external pressure FK; the coefficient of friction f characterizes the sliding in the sheared film.

Control questions

1. Compression characteristics of minced meat under volumetric compression.

2. Strength characteristics of whole tissues of meat and meat products

3. Surface properties of food materials

Adhesion - адгезия

Stickiness -липкость

external friction coefficient – коэффициент внешнего трения

Normalpull-off –нормальный отрыв

Pull-off method – метод отрыва

Adhesive -адгезионный

Cohesive -когезионный

Adhesive-cohesive - адгезионно-когезионный

Thixotropic - тиксотропные

Rheopex - реопексные

Yield limits conditionally static – пределы текучести условно-статический

Dynamic - ultimateshearstress- динамический – предельное напряжение сдвига

Structural strength at elastic-brittle or elastic rupture – прочность структуры при упруго-хрупком или эластичном разрыве

Elastic-ductile fracture - эластично-вязкое разрушение

Lecture 10

Measurement methods

Lecture plans:

1. A set of objects of rheological research

2. Classification of methods for measuring rheological characteristics

1.The set of objects of rheological research in the food industry covers a wide range of states of aggregation of materials, from solids to low-viscosity liquids. This, as well as the variety of objectives of rheological research, requires the use of an exceptional variety of devices, both in terms of the principle of operation and in terms of design features.

The methods of physicochemical mechanics are increasingly being used in various fields of food production. Determination of the structural and mechanical properties (SMS) of food masses is associated with the need for constant technological control of production. Significant deviations of SMS from the accepted norms can affect not only a decrease in the quality of finished products, but also a decrease in the productivity of equipment during certain technological processes. Therefore, the main task of rheometry is to determine changes in material properties that occur under the influence of certain factors.

Methods for measuring SMS are very numerous, which is due, on the one hand, to a huge variety of real liquids and dispersed systems, and on the other hand, to a wide range of practical conditions for their flow.

According to the type of the measured value, rheological methods are divided into four groups (table 2.1). This classification is to a certain extent arbitrary, since some devices allow you to vary two values ​​with a constant third.

The first two methods are most widely used, especially in viscometry. The first method, the constant shear rate method,  is usually implemented using an electromechanical or hydraulic drive. In this case, the force of resistance of the medium is measured by various dynamometers.

The second method - the constant load method, or - is structurally much simpler, since the speed of movement or rotation of the measuring body can be easily measured with an ordinary stopwatch or recorded on a chart tape.

Table 2.1. Classification of methods for measuring rheological characteristics

| Measuredvalue | ConstantValues | Instrumentexample |
| --- | --- | --- |
| Dynamic (force, moment, tension) | Geometric, kinematic | Viscometer «Reotest»; VNIIMP universal device; Simonyan's shear meter |
| Kinematic (time, speed) | Dynamic, geometric | Viscometers: RV - 8, Ostwald, Ubbelohde, Heppler |
| Geometric (length, area, volume) | Dynamic, kinematic (time) | Penetrometers (conical plastometer KP - 3, etc.) |
| Energy (power) | Geometric, kinematic | FarinographBrabender, Bolshakov-Fomin device; devices giving a force-distance diagram |

In the third measurement method, the constant loading force is due to the constant mass of the moving part of the device. The measurement time is usually constant (180 - 300 s) and is assumed to be slightly longer than the relaxation period. The instruments measure the depth of immersion into the material of an indenter of a special shape at a decreasing speed, which reaches zero in the limit.

The fourth method makes it possible to determine the deformation energy from the area of ​​the diagram obtained in the «force - displacement» coordinates, and the ordinate on the diagram shows the effort. In addition, in devices of this group, energy can be calculated from power, if the device is equipped with a self-recording or indicating wattmeter or counter.

According to the classification of M.P. Volarovich, rheological research methods and devices can be divided into integral ones, which make it possible to determine the total effect of the flow, and into differential ones, which make it possible to directly observe the deformation in time at each point of the dispersed system during its flow.

Differential methods make it possible to observe the change in the stress and strain field over time in the study of complex cases of the flow of structured systems. In this case, observations can be carried out visually, as well as with the help of fluoroscopy, microcinema, etc. Differential methods allow you to obtain only a qualitative characteristic of the phenomenon under study. For example, when studying the flow of a viscoplastic material in the gap between two cylinders, M.P. Volarovich and his co-workers applied aluminum powder to the surface of the material before the study. During deformation (during the rotation of the inner cylinder), two zones were clearly identified on the surface of the material: a) the zone of plastic flow; b) the zone of elastic state (Figure 2.1).

б

а

ω

Fig. 2.1.Viscous-plastic deformation scheme

Integral methods allow one to observe the total effect of the flow. Depending on the research conditions, using model analysis in each individual case, the number of independent characteristics of the mechanical properties of the system under study is determined, which are necessary to solve the problem. The most advanced methods are capillary and rotational viscometry, cone penetration, longitudinal displacement of the plate, as well as ball viscometers.

Lecture 11

Classification of methods and instruments for

measuring the structural and mechanical properties of food

Lecture plan:

1. Classification of methods and devices for measuring the structural and mechanical properties of food

The purpose of the lecture: to get acquainted with the Classification of methods and devices for measuring the structural and mechanical properties of food.

1. Classification of methods and devices for measuring the structural and mechanical properties of food. Devices used to measure the structural and mechanical properties of food (materials) are classified according to the following criteria.

By destination. Devices are divided into four groups: industrial, laboratory, devices for in-depth research in laboratory conditions and devices for scientific purposes. Devices of the first group are installed directly on technological machines and register the properties of materials in the flow. The devices of the second group, in the production laboratories of enterprises, are designed to carry out accelerated control over technological processes. Devices of the third group are intended for in-depth testing and are installed in the laboratories of the head enterprises. Devices of the fourth group are used in research centers and are used to test materials under various conditions. They are distinguished by high measurement accuracy and rather long test duration.

By the expression of the measurement result. Devices are divided into absolute, relative and conditional. Devices of the first group show the numerical values ​​of properties in the absolute system of units, based on the geometric dimensions of the working body and the conditions of the experiment. Devices of the second group require preliminary calibration on a reference material, as a result, dimensionless, relative indicators are obtained, which are then recalculated into absolute values ​​using calibration charts. Devices of both groups are theoretically justified. On devices of the third group, the values ​​of the measured quantities are expressed in conventionally accepted units or points, the obtained values ​​are unsuitable for calculations and are used mainly for comparing any qualitative indicators in a narrow range.

Depending on the display of the measurement result, the devices are conventionally divided into integral and differential.

Functionally. The classification of devices on this basis was proposed by B.M. Azarov and V.A. Aret, according to which they are divided into 40 types.

The above classifications of methods and devices can be further supplemented and modified.

Modern production of meat, fish, dairy and other branches of the food industry, which includes many different technological processes, require the use of devices for measuring the properties of raw materials at the stages of their receipt and processing in order to control, regulate and manage the indicators of raw materials and finished products.

Control questions

1. Methodology.

2. Classification of methods and devices for measuring the structural and mechanical properties of food

Lecture 12

Instruments for measuring shear properties of products

Lecture plan:

1. Rotational viscometers

2. Capillary viscometers

The purpose of the lecture: to get acquainted with devices for measuring the shear properties of products.

Shear properties characterize the behavior of the volume of the product when it is subjected to shear, tangential stresses.

To measure the shear properties of food materials, devices are used to determine the resistance force inside the material with a relative displacement of its layers. According to the principle of operation, devices for measuring the shear properties of food materials are divided into the following groups: capillary, rotary, penetrometers, devices with plane-parallel displacement of plates, etc.

Rotational viscometers

Rotary viscometers are widely used in many branches of the food industry in technological laboratories of enterprises, in research organizations. Viscometers are used to control the quality of raw materials, semi-finished products and finished products, as well as to control technological processes.

The idea of ​​rotational viscometers is that a measure of resistance to shear flow can be determined by measuring the torque and angular velocity during relative rotation, for example, of coaxial (coaxial) cylinders, in the gap between which there is a viscous fluid. A diagram of a rotary viscometer with coaxial cylinders is shown in Fig. 2.2, where, for definiteness, the inner cylinder is stationary, and the outer one rotates with an angular velocity ω.

In viscometers with rotating cylinders, especially with a small gap between them, the nature of the product flow is close to a simple shear, which simplifies the processing of experimental data. The range of materials whose properties are monitored on rotary viscometers is quite wide: syrups, milk, canned milk, curd masses, mash, creams, chocolate and candy masses at elevated temperatures, minced meat, etc.

L

r

RВ

М, ω

τв

Fig. 2.2.Flow diagram in a rotary viscometer

with coaxial cylinders

When the coefficient  of dynamic viscosity is determined by the Margules formula:

,

where: RВ – radius of the inner cylinder, m;

RН – radius of the inner cylinder, m;

М – torque applied to the outer cylinder, rpm;

ω – angular velocity of the outer cylinder, rad – 1;

L – layer height between cylinders, m

Whenthe calculation formula of the viscometer is:

,

where: α – geometric simplex.

.

There is the following connection between formulas (2.1 and 2.2):

.

Rotary instruments with systems are distinguished by the shape of the measuring surfaces: coaxial cylinders, spheres or hemispheres; two cones, two plane-parallel plates, two flat rings or two conical rings; cylinder - disk; cylinder - hemisphere; cone - disc; cylinder - cone; cylinder - cone - disc.

Some of them are shown in Fig. 2.3.

RH

RВ

RH

RB

ω

ω

ω

ω

ω

а)					б)					в)

г)						д)

Fig. 2.3.Diagrams of measuring surfaces of rotary viscometers:

a) coaxial cylinders; b) two hemispheres; c) two cones,

d) two plane-parallel plates, e) two flat rings

The shape of the sensing organ (rotor) depends on the type of material under study (Newtonian or non-Newtonian) and the range of measured viscosity values. The most widespread in the food industry are viscometers with coaxial-cylindrical measuring surfaces.

There are two main variants of the device with coaxial cylinders. The first is as follows: the test substance is placed in an outer cylinder driven in a rotational uniform motion, i.e. at a constant shear rate (), and the torque on the inner cylinder transmitted through the test material is measured by the twisting of the elastic element on which this cylinder is suspended (Couette, McMichel viscometer, etc.).

In the second version of the device: the outer cylinder is stationary, and the inner cylinder is mounted on an axis mounted on ball bearings and is driven into rotation under the action of a constant torque ().The angular velocity of the cylinder ω is measured, depending on the viscosity of the liquid (viscometer M.P. Volarovich).

Thus, the devices implement, respectively, two research methods:

а)strain rate constant method  = const (fig. 2.4,а);

b) constant torque method М = const (fig. 2.4, b).

The combination of both methods is very effective in rheological studies. It is advisable to combine the methods in such a way that the viscoelastic properties of materials with an intact structure are studied by the M = const method, and the fracture processes and the steady-state flow regime are studied by the  =const method.

The method for calculating the rheological characteristics has specific features for each of the two main areas of the state of the product structure.

In the area of ​​the intact structure, the elastic moduli, the highest viscosity and the nature of the development of deformations are determined. Measurements begin after thixotropic restoration of the structure. The deformation values ​​are measured according to the readings of the device. The experiment is carried out with efforts less than the ultimate shear stress, with a deformation recording interval of 10 - 20 s.

When passing to the region of avalanche destruction of the structure, the static τС and dynamic θ0 limiting shear stresses, plastic viscosity ηPL, and the dependence of the effective viscosity ηEF on the velocity gradient or shear stress τ are determined from the flow curve. The results are calculated using the equilibrium flow curve passing through all points. The rotation of the rotor causes the appearance of internal stresses in the product, which is located between the rotor and the glass. These shear stresses are proportional to the shear forces, therefore, graphical and mathematical processing of experiments can be carried out in consistent variables (τ), or using the primary dependencies obtained directly from the experiment.

Shear elastic moduli G [Pa] are determined using Hooke's law and graphical dependences of the relative deformation ε on the time of action of constant stress τ:

,

where: τ – shear stress, Pa;

ε is the relative deformation of the product;

;

δ - deviation of the instrument arrow, m;

ΔR is the thickness of the product layer between the glass and the rotor, m;

l - arrow length, m;

ε / τ - relative deformation reduced to shear stress unit, 1 / Pa.

The highest effective viscosity [Pa · s], corresponding to the beginning of plastic-viscous flow, is found by the Newton equation:

,

or

,

where: Δt - time interval, s;

Δε / Δt is the strain rate for a straight section of a curve or a curved section approximated to a straight line with a small convexity.

The calculation of the Newtonian and effective viscosity ηEF for any voltage can be carried out using formula (2.8), but with a large number of experiments it is more convenient to use the dependence proposed by M.P. Volarovich, when using a viscometer working according to the method :

,

where: K - the constant of the device;

N - the rotor speed of the viscometer, s – 1;

m - the mass of the weights rotating the rotor, kg (minus the value that compensates for the friction in the bearings).

The most widespread rotary viscometer working with the = const method is the Reotest viscometer (Germany) and its modifications. In addition to the basic set of cylindrical measuring elements, this instrument is equipped with a cone-plane device designed to measure viscosity at high shear rates for medium and high viscosity products. The angle between the plane and the generatrix of the cone is 0.3 °. The device allows measuring the shear rate from 0.56 to 4860 s – 1, the shear stress τ - from 40 to 2.2 · 105 Pa. The value of the measured viscosity η is in the range from 8 to 40 · 107 MPa · s.

The «Reotest» viscometer is shown in fig. 2.4. A synchronous electric motor is installed inside the frame 1 of the device, connected to a 12-speed gearbox, which makes it possible to change the rotational speed of the inner cylinder 2 from 0 to 1500 s – 1. The torque from the gearbox is transmitted to the drive shaft 6 and then through the spiral spring 5 to the driven shaft 4, which is connected to the inner cylinder 2 by a clutch. Outer cylinder 3 is attached to the body of the viscometer with a special clamp. Thedevicehas a thermostaticvessel.

The magnitude of the torque is measured on the scale of the device 8, and the shear rate - on the pointer 9. The torsion-type torque meter with ohmic sensors works on the principle of converting mechanical forces into electrical impulses. The readings of the device 8 are directly proportional to the torque, as well as the shear stress and viscosity of the material under study. The speed of rotation of the synchronous motor and, therefore, of the inner cylinder 2, depends on the frequency of the current in the network. Deviations from the normal frequency of 50 Hz are recorded by the device 7 and are taken into account by a special design factor.

Viscosity measurement range: from 10–2 to 104 Pa · s; shear rates: from 0.1667 to 1.458 · 103 s – 1; shear stress: from 12 to 3 · 103 Pa; temperature: from - 30 to 150 ° C. Measurement error ± 3% (with respect to Newtonian fluids).

Despite its apparent simplicity, there are a number of problems in rotational viscometry. These are, first of all, various effects that reduce the measurement accuracy, namely:

1. Turbulization of the flow. One of the conditions for the accuracy of measurements in rotary instruments is the laminarity of the deformed flow, which is characterized by the Reynolds number (Re), which is a dimensionless criterion, the excess of which causes flow turbulization.

2. Thermal effects. The very principle of rotational viscometry implies the performance of work on the material located in the gap of the device. This leads to the generation of heat and a change in the temperature of the measured medium, which in turn causes a change in viscosity. A large number of works have been devoted to solving this problem, the essence of which boils down to the introduction of correction factors.

3. Weissenberg effect. K. Weissenberg found that during the flow of elastic fluids under simple shear conditions, not only tangential, but also normal stresses arise that are orthogonal to the shear stress. An elastic fluid, the deformation state of which is characterized by axial symmetry, is pulled together by normal stresses opposing gravity and centrifugal forces and squeezed out of the viscometer gap.

4. The phenomenon of elastic turbulence in the movement of elastic fluids. When elastic fluids flow in capillaries at high velocities, it was found that the fluid jet begins to deform and various disturbances appear on it. And at very high deformation rates, the stream of material sometimes even breaks up into individual grains. Despite the fact that the phenomenon has been detected in capillary viscometers, it can also be observed during the operation of a rotary viscometer. In this case, spontaneous fluctuations of the measured parameter can be observed.

5. End effects. When rotary viscometers are in operation, the torque is transmitted to the measuring element not only through the side (working) surfaces, but also from the cylinder bottoms. Since the mathematical description of the stress and shear rate fields arising in the gaps formed by the cylinder bottoms is very difficult, the calculation formulas for rotary instruments are derived without taking into account the influence of end effects, which introduces certain errors in measurements.

Capillary viscometers

Viscometers of this type are used to measure the viscosity characteristics of materials with relatively low viscosity. The idea of capillary viscometers is that by measuring the flow rate, pressure drop, length and diameter of the capillary through which the material flows, it is possible to calculate the measure of the material's resistance to shear flow, i.e. viscosity. This problem is especially simple for Newtonian fluids, the laminar flow of which in a capillary is described by the Poiseuille equation:

,

where: η - dynamic viscosity coefficient, Pa · s;

R is the radius of the capillary, m;

Δр - pressure drop over the length of the capillary, Pa;

L - capillary length, m;

Q is the volumetric flow rate of the material through the capillary, kg / m3.

The flow diagram of the liquid in the viscometer capillary is shown in Fig. 2.5. The theory of capillary viscometry is based on the fact that the flow in the device is laminar, there is no slip on the wall, and the shear rate at a point depends on the loading at the same point.

L

r

R

τ

τ

x

Р1

Р2

Fig. 2.5.Flow diagram in a viscometer capillary

Along the way, it should be noted that slippage is possible in some cases, which has been repeatedly noted by many researchers. However, sufficiently reliable methods of accounting for this phenomenon have not yet been developed. In experiments on capillary viscometers, the dependence of the mass flow rate Q on the pressure drop Δp is obtained. If the experiments are carried out on one capillary, then the shear stress on the capillary wall τ, Pa, is calculated by the formula:

,

where: n – correction for end effects.

To exclude the influence of end effects on the accuracy of viscosity measurements, the method of two capillaries of the same radius, but different lengths, is used. Then τ isdeterminedbytheformula:

.

The shear rate, s – 1, (also on the capillary wall) is calculated by the formula:,

where:                                      .

Schematic diagrams of capillary viscometers for Newtonian and non-Newtonian products are shown in Fig. 2.6. Common to all devices of this type is the presence of a capillary, a device for measuring the flow rate or volume of a liquid, and a system that ensures the creation of hydrostatic pressure. As a capillary, calibrated tubes with a diameter ranging from fractions to 2 - 3 mm are used to measure the viscosity of Newtonian and not very viscous non-Newtonian fluids. The results obtained are usually invariant, i.e. do not depend on the tube diameter. For high-viscosity non-Newtonian fluids and plastic-viscous systems, the diameter of the «capillary» can reach several tens of millimeters, and the measurement results often depend on its diameter, i.e. are not invariant. In the two named cases, the capillary diameter is, of course, included in the theoretically obtained formulas for the corresponding body models. However, in the case of plastic-viscous bodies moving through tubes, the flow model can depend on the diameter, i.e. at different diameters, the same product has different flow models, which is rather difficult to take into account.

The simplest, traditional and at the same time universal capillary viscometers Ostwald (Fig. 2.6, a) and Ubelode (Fig. 2.6, b) have a capillary 1 and two hollow balls for liquid 2 and 3. The driving force of the outflow process is the pressure difference. In the Ostwald viscometer, it is due to the difference in the heights of the liquid, in the Ubbelohde viscometer - by vacuum or pressure in one elbow of the tube. During measurements, the instruments are usually placed in a water bath to thermostat the test liquid. Thermostating of the investigated liquid in the device takes 10 - 30 minutes, which is determined by its volume. During this time, its temperature reaches the temperature of the liquid from the thermostat, i.e. conditions are created that are identical to the previous measurement. With short-term thermostating, temperature errors give significant deviations from its true values.

When measuring with Ubbelohde viscometers, it is necessary to create pressure or vacuum on one side. For this, an installation is used, consisting of an air pump, which can operate as a compressor or a vacuum pump; monostat bottles with a volume of 8 - 10 liters with a stopper; U-shaped liquid pressure gauge or micromanometer; a vessel that traps the liquid ejected from the pressure gauge, and a three-way valve for regulating the air supply. Before starting the measurement, the pressure in all containers must be equalized. Excessive pressure during the flow of liquid through the capillary creates the possibility of turbulization of the flow, therefore, the check for laminarity according to the Reynolds criterion should be carried out especially carefully.

1

2

3

1

3

2

А

В

а)						b)

Fig.2.6. Capillary viscometers: a) Ostwald; b) Ubbelohde: 1 - a ball vessel for measuring the volume of the liquid flowing through the capillary; 2 - capillary; 3 - ball vessel for collecting liquid

General conditions in the method of work on all devices are the layout of the stand and strict horizontal or vertical installation of the device; filling a dry, thoroughly washed device with a test product and thermostating it at a selected temperature; measurement of the flow time of a certain volume, which is due to the viscosity and the difference in heights or hydrostatic pressures. Before working measurements, the device is calibrated against a reference liquid - distilled water, sugar solution or castor oil. For absolute devices that do not require calibration, the constants are determined by the size of the working bodies in accordance with the Poiseuille equation:

,

where: p - hydrostatic pressure, Pa; d — inner diameter of the capillary, m;

l - capillary length, m; η - dynamic viscosity, Pa · s; w - fluid velocity, m / s.

It should be borne in mind that geometric measurements must be carried out with extremely high accuracy. Therefore, it is preferable to calibrate capillary viscometers against a reference liquid.

The Ostwald viscometer is used as a relative instrument. Calculation formula is:

,

where: KT is the water constant of the device at the measurement temperature, m2 / s2;

ρ is the density of the liquid at the pouring temperature, kg / m3;

t - expiration time, s.

The Ubbelohde viscometer can be used both relative and absolute. In the first case, it is calibrated against the reference liquid; in the second, constants are determined in accordance with the Poiseuille equation; they can be determined in a complex manner, based on the data of preliminary calibration and direct geometric measurements. The viscosity is calculated by following formula:

,

where the subscript «b» refers to the data obtained when calibrating the device against water or any other liquid.

By varying the outlet pressure pV, one can construct calibration rheograms that allow calculating constants for working measurements. Similar graphs are plotted for the measurement results of the investigated liquid. Of significant importance are the pt = f (p) plots, which give horizontal lines for each temperature in the area of ​​the Poiseuille law. When turbulization begins and Poiseuille's law ceases to objectively reflect the process, the lines deviate upward. If the fluid has a viscosity anomaly, then the straight lines may have partial curvature or not leave the origin, cutting off on the abscissa a segment proportional to the limiting shear stress θ0.

In conclusion, we note the conditions necessary to achieve a given measurement accuracy with capillary viscometers:

a) provide a laminar flow regime;

b) eliminate the influence of end effects, for example, when using two capillaries of different lengths;

c) check the absence of the wall slip effect, for example, by the coincidence of the flow curves in consistent variables for capillaries of different diameters;

d) when measuring the flow rate by the gravimetric method, it must be remembered that many food materials tend to be compacted under pressure. This issue requires special studies of the material under conditions of all-round compression.

Capillary viscometry, due to its peculiarities, has very limited application and is rarely used in industrial conditions.

Ball viscometers

Ball viscometers belong to heterogeneous research methods and are widely used when working with homogeneous Newtonian or semi-structured (non-Newtonian) fluids. The viscosity is determined by the time the ball passes the measuring section of a certain length. Devices are designed according to two main schemes (Fig. 2.7):

a) a ball freely falling in a vertical cylinder;

b) a ball rolling in an inclined cylinder.

The disadvantage of scheme a) is that when the ball is lowered in a weakly structured liquid, it may deviate from rectilinear motion, i.e. the so-called «hovering» of the ball is manifested, which can lead to distortion of the measurement results.

Therefore, rolling ball viscometers have become more widespread. A diagram of such an example - a Heppler viscometer - is shown in Fig. 2.8. Ball 2 moves in inclined tube 1, forming a narrow crescent-shaped slit with its wall. In a structured fluid, when passing through a slit, the structural bonds are destroyed, therefore, there may not be an accurate reproduction of the results in two successive measurements. The device is calibrated for measuring the viscosity of Newtonian liquids in the range from 3 · 10–4 to 60 Pa ∙ s. The passport error of measurements of Newtonian liquids does not exceed 0.5% for the laboratory model. The device is not suitable for systems with ultimate shear stress.

To calculate the viscosity, use the formula obtained on the basis of Stokes' law:

η = К(ρШ – ρ)t,

l

а)

l

б)

α = 10о

Fig. 2.7.Capillaryviscometercircuits

6	   7

8

9

10

11

5

4

3

2

1

12

Fig. 2.8.Ball viscometer

Гепплера

where: К - device constant, m2 / s2; ρШ is the density of the ball material, kg / m3;

ρ - the density of the liquid at the measurement temperature, kg / m3; t - the time of movement of the ball in the section h, s.

The difficulty of using ball viscometers for rheological studies of structured liquids lies in the fact that to obtain a flow curve, it is necessary to carry out a series of measurements using balls of different diameters, and when studying thixotropic liquids, large errors are possible due to the uneven velocity of the ball along the length of the measuring section.

One of the difficulties in using ball viscometers is that when a falling ball acts on a product, the rheological properties of the latter change, as a result of which the result of each subsequent observation differs from the previous one. With strict compliance with the requirements of GOST, the end of the measurement is judged by the achievement of the difference between the two subsequent observation results of no more than 1 s. For example, for condensed milk, this is achieved after 10 to 15 observations, which indicates the duration of the test.

Due to these shortcomings, ball viscometers have not found wide application in the food industry.

Lecture 13

Instruments for measuring the compression properties of products

Lecture plan:

1.Methods for measuring the compression properties of products

2. A device for measuring the compression characteristics of products

The purpose of the lecture: to get acquainted with devices for measuring the compression properties of products

- Methods for measuring the compression properties of products

Along with shear, volumetric or axial compression as well as axial tension are the main types of mechanical deformation of products. In a number of machines, such as tops, syringes, dosage-forming, presses, dispensers, etc., the products are at elevated pressures, which changes their initial volume and density. At the exit from the machine, the volume and density may also differ from the original. These changes in the product are determined based on the compression characteristics.

а      b                               c

Figure 24.Methods for measuring compressive properties of products

The main methods of measuring (Figure 24.) the compression characteristics of products are: axial compression (a), axial tension (b), one-sided volumetric compression (c) and two-sided volumetric compression.

A device for measuring the compression characteristics of products (Figure 25.). The device is designed to determine the deformation properties of products under axial compression.

The device consists of a housing (1), consisting of a lower and an upper part. A retractable table (2) is installed inside the upper part, on which a strain gauge beam (3) is located in a fixed position on the prisms, with strain gauges (4) fixed to it. On the strain gauge beam in its middle part, a boss is rigidly fixed for installing and fixing the bottom plate (5). The upper and lower plates (5) can be made of the same or different materials. The movable rod-nut (6) receives a reciprocating uniform movement from the screw (7), rigidly connected to the electric motor (9) through a gearbox and a step variator, which allows you to create different speeds of movement of the rod-nut. In the lower part of the rod-nut, a cross-bar plate (10) is attached, which serves as a support for the indicator leg, and an upper working plate (5). A switch and a reverse switch are installed on the body of the device. The device is equipped with a set of round plates of different diameters with an area of ​​5, 7, 10, 15 cm2.

Along with shear, volumetric or axial compression and axial tension are the main types of mechanical deformation of products during processing and packaging. The main methods for measuring axial tension-compression are shown in Fig. 2.13.

а)

1

2

F

1

2

г)

F

F

1

2

3

б)

F

1

2

3

1

в)

F

F

Schematic diagrams of measuring instruments compression characteristics:

a) axial compression: 1 - plates; 2 - product;

b) one-sided volumetric compression; c) bilateral volumetric compression: 1 - piston; 2 - product; 3 - cylinder;

d) axial tension: 1 - clamps; 2 - product

The measurement technique is practically the same for all instruments. Structural schemes differ in the way the plate is driven: from below and from above, from an electric motor or directly from a load.

The device for following materials in tension (Fig. 2.14) consists of a bath 1, into which a solution is poured 2. The ends of the sample 4 floating on the surface are fixed in clamps 3, one of which is motionless. The measuring part is a ruler 9, fixed on the guide. The loading mechanism consists of a rod 7, weights 11, a thread 10 of rollers 5 and 8, brackets 6. The concentration of the salt or sugar solution is selected so that the sample (for example, dough) floats freely on the surface. The device is intended for research purposes.

Compression strain gauges are used to determine elastic-plastic properties.

B.A. Nikolaeva and A.S. Shpigelglas for measuring uniaxial compression strains between parallel planes (Fig. 2.15) consists of a bed 1, in which the table support moves. The material to be tested is placed between the parallel planes of the disks 2 and 3. The upper of them can move under the action of the load 4, at the same time it is connected to the rack 8, which meshes with the gear 7, on the axis of which the arrow 6 is fixed, showing the deformations on the scale 5 The lower disc 2 has the ability to move in the vertical plane with the help of the eccentric 9. The device described can be used to determine the instantaneous elastic deformation, the elastic modulus, as well as the viscosity and ultimate shear stress.

9	 8   	 7

1 2     3    4    5         6

10

11

Fig. 2.14.Tensiletestapparatus

3

8

7

6

5

4

2

1

9

Fig. 2.15.Compressiontester

2.5 Instruments and methods for studying surface tension

Devices and methods for measuring adhesion are based on the destruction of the adhesive seam by applying an external force (Fig. 2.16). According to the method of applying the force, the methods of separation (uniform and uneven) and shear are distinguished.

Most often, in adhesiometers for food products, the scheme is implemented according to Fig. 2.16, a. It can differ in the ways of applying force and registering the tearing force. For example, in Nikolaev's device «instantly» the upper disc comes off with the help of a lever mechanism; in Kulman's device, the lower disc gradually comes off when the container suspended from it is filled with water. Separation of contacting bodies can occur in three ways: adhesive - along the product - wall interface, cohesive - along the product layer - and mixed.

The external friction of foods is influenced by stickiness and a number of other factors. Friction can be static - prior to displacement and dynamic - as the product moves across the surface.

For the theoretical substantiation of external friction, molecular-kinetic, mechanical, physical and other theories similar to the theories trying to explain adhesion have been proposed. However, the existing theoretical developments have not yet been implemented for the analysis of external friction of food products.

The friction coefficients of food materials, depending on the rheological properties, the state of the friction surfaces and the sliding speed, are determined by various methods. The classic type of device for measuring the force of external friction is a pair of bodies in contact with flat surfaces, the area of ​​which can be from fractions of a square millimeter to tens of square millimeters. In this case, one of the bodies is displaced relative to the other. The force of displacement (friction) is measured by strain gauges, torque gauges, or some other sensor. For low speeds of friction, a rectilinear displacement of the surface is realized. I.V. Kragelsky divided the known methods for determining the coefficient of friction into four groups, based on the geometric and kinematic principles (Figure 2.17).

а)			б)

в)

г)

д)

е)

Fig. 2.16.Schematic diagrams of devices for measuring adhesive strength:

- uniform separation; b) eccentric stretching; c) bending of sheet materials; d) methods of delamination of materials; e) methods of shear fracture; f) methods of shear failure in torsion

α

а)			б)				в)			г)

д)					е)						ж)

Fig. 2.17.Methods for measuring external friction

The first group of methods includes those in which one plane moves progressively relative to another plane (Fig. 2.17, a, b, c). According to scheme a, the product moves along a moving test surface, while the friction force is measured with a dynamometer of any type. This, the most common method, determines the friction of solid products: crackers, bread, meat, fish, grain, etc. According to scheme b, the product is moved with the help of a load along the investigated fixed surface. The friction force in this case is equal to the minimum mass of the load required for uniform movement of the product. This method determines the coefficient of friction of such solid products as cheese, fish, confectionery. According to the pattern, the product slides on an inclined surface. The coefficient of friction is determined by the minimum angle of inclination α of the surface of the structural material under study, over which the food product slides. This method made it possible to determine the static coefficient of friction of cheese, rusks, bulk products. Determination of the coefficient of friction by the angle of inclination of the plane, at which the product begins to move along the surface, is not accurate enough, since the angle of inclination of the plane at the moment of movement is determined approximately, moreover, it is impossible to establish the change in the coefficient of friction with a change in the sliding speed. The second group includes such methods in which one of the friction pairs makes a rotational movement. According to scheme d, the coefficient of friction of the candy masses is determined. The product lies on the rotating disc, the structural material to be examined slides over the rotating product. The frictional force is determined from the torque transmitted from the disc to the sample. According to scheme d, the product, fixed to a non-elastic beam, slides along the rotating surface of the material under study. This method is used to determine the coefficient of friction of meat, fish, coffee and tea tablets, rusks, tin bread.

The third group includes methods in which the generatrix of the cylinder is in contact with the plane. Scheme e - a beam performing oscillatory movements on two rollers rotating towards each other.

The fourth group includes methods in which one cylindrical or flat flexible surface moves along a cylindrical surface (scheme g).

At the same time, the change in pressure is allowed by the methods shown in all diagrams, except for w; surface resizing - methods a - e; a change in the sliding speed is allowed by all methods, except for c.

External friction is influenced by the same factors as adhesion. Consequently, the static friction must be greater than the dynamic one. During the initial shift, part of the effort is spent on the release of particles or molecules of one body from the irregularities or molecular wells of another, the rest - on breaking molecular and other contacts, which do not have time to fully recover during movement. A sufficiently low speed of the relative motion of the rubbing bodies can cause jumps, i.e. discontinuity of movement: the body moves from one static position to another, where contacts with the sliding surface are again formed. At high sliding speeds and a constant state of the sliding surface, such effects are usually not observed.

Lecture 14

Instruments for measuring the surface properties of products

Lecture plan:

1. Methods for measuring adhesion

2. Requirements for adhesiometers

3. Instruments for determining the coefficients of friction.

The purpose of the lecture: to get acquainted with devices for measuring the surface properties of products

1. Methods for measuring adhesion. Devices and methods for measuring adhesion are based on the destruction of the adhesive seam by applying an external force (Fig. 2.16). According to the method of applying the force, the methods of separation (uniform and uneven) and shear are distinguished.

Most often, in adhesiometers for food products, the scheme is implemented according to Fig. 2.16, a. It can differ in the ways of applying force and registering the tearing force. For example, in Nikolaev's device «instantly» the upper disc comes off with the help of a lever mechanism; in Kulman's device, the lower disc gradually comes off when the container suspended from it is filled with water. Separation of contacting bodies can occur in three ways: adhesive - along the product - wall interface, cohesive - along the product layer - and mixed.

Instruments and methods for measuring adhesion are based on the destruction of the adhesive bond by applying an external force. Determination of the adhesion properties of products, depending on the method of application of force, is carried out: by the method of separation and by the method of shear. For these methods, various methods of their implementation can be used, which are schematically shown in Figure 30, where the food product (adhesive) is shown dark, and the material with which it contacts (substrate) is light.

а)			б)

в)

г)

д)

е)

Fig. 2.16.Schematic diagrams of devices for measuring adhesive strength:

- uniform separation; b) eccentric stretching; c) bending of sheet materials; d) methods of delamination of materials; e) methods of shear fracture; f) methods of shear failure in torsion

Separation (detachment) of the contacting bodies, depending on their nature and technological conditions, can be along the contact boundary (adhesive detachment), along the product layer (cohesive detachment) or mixed (adhesive-cohesive detachment). Given these circumstances, before testing, it is necessary to carefully prepare samples of the test product (adhesive) and the material (substrate) with which it will contact. With adhesive detachment, external bonds between the material and the product are violated, which characterize the energy of the free surface. In the case of cohesive separation, internal bonds in the product are violated, which depend on the interaction energy between the structural elements under conditions of a volumetric stress state. For food products, pure adhesion detachment is rarely observed, therefore, in experiments, the force of detachment of the material (substrate) is measured, often without specifying its type.

The value of adhesion is influenced by: technological properties of the product itself (temperature, humidity, composition, etc.); structural material grade; roughness of the material surface; measurement conditions (geometric, kinematic and dynamic parameters of the device).

2. Requirements for adhesiometers. Requirements for adhesiometers:

- instrument readings must be primary, i.e. do not need preliminary calibration on any reference material and be expressed in an absolute system of units;

- before measurement, the product must be pressed against the substrate to establish contact and remove air bubbles from it;

- the application of a product of a given thickness on the plates of the device should be carried out using a template in order to exclude the effect of thickness on its adhesion properties;

- the devices should provide for the possibility of replacing the plates in order to determine the influence of the brand of the plate material and the degree of its processing (roughness) on the strength of molecular contacts;

- Devices should allow for the possibility of varying the speed of application of the pull-off or shear force in a wide range, in order to establish the influence of the pull-off force on the distribution of deformations and forces between the plates and the product.

The characteristic of the adhesion capacity of the product can be the following values: the force of the separation, referred to the contact area; work of separation, referred to the contact area; the time it takes to break the bond between the substrate and the adhesive under a given load. Specific minimum force is otherwise called adhesive strength, adhesive stress (pressure), adhesion pressure or specific adhesion.

Devices for determining the adhesive characteristics by the method of applying the load are divided into: adhesion testers with a constant pull-off, adhesiometers with instant pull-off, as well as shear meters. With uniform separation, the load is applied perpendicular to the plane of the substrate, while adhesion is characterized by normal force per unit contact area, i.e. normal voltage. In shear, shear stresses are determined that arise when the adhesive layers are relatively displaced relative to the substrate. To study the adhesion of food-grade viscous-plastic materials, devices based on the normal tear-off method are used.

B.A. adhesiometerNikolaev (Figure 31). The device is simple: it is designed on the principle of a beam balance and is designed to study the adhesion of viscous food products. It consists of a measuring plate (substrate) (1), made in the form of a disk, a two-arm lever (2), to which the substrate is suspended on one side, and on the other - a weight (3) and the base of the device (5). An adhesive is applied between the disc and the base (4). During the test, gradually increasing the mass of the load (3), the disc (1) is pulled off the adhesive (4). The tackiness of the material is calculated from the measurement results.

AdhesiometerYu.V. Klapovsky (Figure 31 b). The device is designed to determine the adhesive properties of viscous food materials, in particular candy masses. It is simple in design and reliable in operation.

The device consists of a lever (7), a vertical rod (8) is hinged to it, located in the guides, to which a plate (10) is attached through an elastic measuring ring (9). A stop screw (5) and a stop (4) are provided to create a preliminary contact. Breakoutforceismeasured

using strain gauges (6) glued to the elastic measuring ring (9). The movement of the plate (10) is measured using a photodiode (3), while the shutter (2) overlaps a part of the light beam of the illuminator (1), changing the illumination of the photodiode. The test product is placed in a special container (11), which has a lid with a hole, into which the substrate plate enters with a small gap.

Figure 31 Adhesiometers:

a) - B.A. Nikolaev's adhesion meter; b) - Yu.V. Klapovsky adhesion meter.

During the tests, the readings of the strain gages and the photodiode after amplification are recorded on a loop oscilloscope, allowing the change in time of the pull-off force and the movement of the substrate relative to the adhesive to be recorded. Universal adhesion meter MTIMMPa (MAPB). The device (Figure 32) is designed to measure the adhesion properties of products and allows changing geometric, kinematic and dynamic and other measurement parameters in a wide range.

The device has a body (1), a lifting table (2) with a micrometric screw for fixing the bottom plate (substrate) (3) and a device for loading the product and tearing off the top plate. The upper plate is fixed by a holder, which is mounted on a strain gage (4), and has a shank to ensure the alignment of the moving part of the system. In the upper part of the device, on the transverse beam of the body, there is a bushing (8), into which a cross-bar (7) with a shank is inserted, at the ends of which the shackles (6) are fixed. A drive shaft with a set of pulleys (10) of different diameters is installed on the top of the housing. Vertically upward, the strain gage is intermixed with earrings, which end in prisms and are hinged on a crosspiece with a shank. The latter moves in the sleeve by means of a thread (9) and a set of pulleys, which are driven in rotation by an electric motor through a gearbox (not shown in the figure). To create a preliminary contact, weights are installed on the strain girder (5). In each series of experiments, the strain gauge is calibrated, obtaining the dependence: the magnitude of the load - the deviation of the «spot» on the oscillogram.

Figure 32. Universal adhesion meter MTIMMPa

Device parameters: pressure of preliminary contact from 300 to 3.5 · 10 Pa; duration of preliminary contact - 3; 60; 180; 300; 600 s; the speed of separation of the top plate from the product - 0.00167; 0.00358; 0.0093;

0.0193 m / s; plate area - 0.0005; 0.0007; 0.0010; 0.0015 m; plate material - steel St. 3, stainless steel 1X18H9T, duralumin, cast iron, fluoroplastic-4, brass; product height - from 0.00003 to 0.003 m; relative error of the device - ± 4%.

Adhesiometer AM-1. On the device, the determination of the adhesion properties of products is based on the shear method and is intended for the study of viscous-plastic materials, in particular for dough.

The device consists of a frame (1), a retainer (2), a rod (3), a bushing (4), an elastic element (5) with strain gauges fixed on it, a weight (6), a matrix (7) with a workpiece and a platform (8) ...

During testing, a die with a baked dough piece is attached to a vertical rod, then the piece is crimped with two semi-conical pads. When the platform is rotated around the vertical axis, the workpiece is shifted relative to the stationary matrix. Lifting the die with a weight allows you to determine the breakout force.

3. Instruments for determining the coefficients of friction. Devices used to determine the coefficients of friction of food products are called tribometers.

Figure 33. AM-1 adhesion meter

The coefficient of external friction of food materials, depending on the rheological properties, the state of the friction surfaces and the sliding speed, is determined in different ways. The classic type of device for measuring the force of external friction is a pair of bodies in contact with flat surfaces, the area of ​​which can be from fractions of a square millimeter to tens of square centimeters. In this case, one of the bodies is displaced relative to the other. The force applied to displace (friction) one body relative to another is measured by strain gauges, dynamometric or some other sensors.

I.V.Kragelsky, based on the geometric and kinematic principles, proposed to divide the well-known methods for determining the coefficient of friction into four groups, which are shown in the form of diagrams in Figure 34.

The first group - methods in which one plane moves progressively relative to another plane (Figure 35. a, b, c). According to scheme a, the product moves along a moving test surface, while the friction force is measured with a dynamometer of any type. This most common method is used to determine the friction of solid materials: meat, fish, bread, etc.

According to scheme b, the product is moved with the help of a load along the investigated stationary surface. The friction force in this case is equal to the minimum mass of the load required for uniform movement of the product. This method is used to determine the coefficient of friction of products such as cheese, fish, confectionery, etc.

In pattern c, the product slides on an inclined surface. The coefficient of friction is determined from the minimum angle of inclination of the surface of the structural material under test on which the food product slides. This method allows you to determine the static coefficient of friction for cheese, rusks, bulk products. The method is considered insufficiently accurate and has not received widespread use.

α

а)			б)				в)			г)

д)					е)						ж)

Figure 35. Methods for measuring sliding friction force:

a), b), c) - with the translational movement of the planes;

d), e) - during rotational movement of one of the friction pairs;

f) - when the generatrix of the cylinder comes into contact with the plane;

g) - when moving a cylindrical or flat flexible surface over a cylindrical surface

The second group - methods in which one of the friction pairs makes a rotational movement. According to scheme d, the product, fixed on an elastic beam, slides along a rotating surface of the material under study. This method determines the coefficient of friction of meat, fish, etc. According to scheme d, the product lies on a rotating disc, the structural material under study slides over the rotating product. The frictional force is determined from the torque transmitted from the disc to the sample.

The third group - methods in which the generatrix of the cylinder is in contact with the plane.According to the scheme e, a beam performing oscillatory movements on two rollers rotating towards each other.

The fourth group - methods in which one cylindrical or flat flexible surface moves along a cylindrical surface (Figure 36 g). Tribometer with a trolley (Figure 35. a). The device allows you to determine the friction force and calculate the true and effective coefficients of external friction. It consists of a device (1) for measuring the friction force (strain gauge), a device (2) for creating a contact pressure, a frame with a test product (3), a trolley with a plate (4) fixed on it, and an electric drive (5) with a pulley.

During testing, the test product is placed in a frame, placed on the flat surface of the trolley plate, and the electric motor is turned on. The thread, winding up on the pulley, begins to move the trolley, and with it the product, enclosed in a frame, from which, through the thread, is transferred to the strain beam, which, under the action of force, is bent from its original position. With further movement of the trolley, the forces between the frame with the product and the strain gauge increase and, having reached a critical value, the frame with the product begins to slide over the surface of the trolley plate. In this case, the deflection of the tensor beam will remain constant at the same level. The friction force arising between the product and the plate is recorded by the deflection of the strain gage, the value of which is transmitted to the oscilloscope tape with the help of strain gauges attached to it. To change the speed of the carriage, there is a four-stage pulley on which the thread is wound. The device allows changing geometric (size of the contact surface area, surface finish, etc.), kinematic (trolley displacement speed, preliminary contact time, etc.) and dynamic (preliminary contact pressure, force growth rate, etc.) factors. The device is equipped with a set of replaceable plates made of different materials.

Figure 36.Tribometers:

a) - tribometer with a trolley; b) - tribometer with parallel displacement of the plates; c) - a tribometer with a movable drive part from hydraulic cylinders.

Tribometer with parallel displacement of the plates (Figure 36 b). The device is designed to determine the adhesion characteristics of solid products, such as cheese. It consists of a loading device (1) for measuring the friction force, a device (2) for creating contact pressure, a plate (4), below and above which, a layer of the investigated product (3) is located, a cargo drive (5) and a device for measuring displacement in the form of a dial indicator (6). The plate is driven by a load with a pre-pressing of the samples with a screw mechanism. The displacement of the plate is measured with a dial indicator. The device can be used to measure both external friction and viscosity at low velocity gradients.

A tribometer with a moving part driven by a hydraulic drive (Figure 36 c). The device is designed to determine the friction force with a small contact area and large normal forces. It consists of a strain gauge (1), a device (2) for creating a preliminary contact, a plate (4) and a hydraulic drive. When measuring large frictional forces, the hydraulic drive system has an advantage over the electric drive, in addition, the hydraulic drive provides infinitely variable speed control.

At present, for many meat products, the nature of adhesion has not been clarified, although a large number of studies have been devoted to this issue and several hypotheses have been proposed to explain the physicomechanical essence of adhesion phenomena.

Control questions

1. Methods for measuring adhesion

2. Requirements for adhesiometers

3. Instruments for determining the coefficients of friction.

Lecture 15

Relationship between structural and mechanical properties and organoleptic assessing the quality parameters of finished products

Lecture plan:

1. Evaluation of food quality

2. Optimization of technological processes based on engineering rheology

The purpose of the lecture: to get acquainted with the relationship between the structural and mechanical properties of a product and sensory quality assessment

Evaluation of the quality of food products can be carried out in two ways. The first method is instrumental measurement of structural and mechanical characteristics, expressed in certain physical units. The second method is sensory (organoleptic) quality assessment, i.e. subjective sensory assessment of the resistance, deformation and consistency of the product. The development of rheology made it possible to substantiate the quality indicators of products and develop some mathematical principles for constructing generalized quality indicators.

Sherman proposed a classification of the structural characteristics of products, which can be used to establish the relationship between the rheological characteristics of a product and an organoleptic assessment (consistency). The characteristics are divided into three classes:

- primary, associated with analytical and geometric properties (content of air, moisture, fat and other components; size and shape of particles);

- secondary, associated with the main rheological properties (elasticity, viscosity, adhesion);

- tertiary, associated with mechanical processes of grinding and chewing the product.

Tertiary characteristics are the terms most commonly used by consumers when evaluating various foods. There are the following types of tertiary characteristics: for solid products - friable, brittle, powdery, wet, sticky, dry, soft, hard, porous, tender, rough (chocolate, biscuits, vegetables, fruits, meat, cheese, bread, butter); for semi-solid products - pasty, crumbling, sticking, wet, dry, lumpy, tender (processed cheese, pasta, minced meat, cream, jam); for liquid products - liquid, watery, thick, greasy, sticky (melted ice cream, mayonnaise, sauces, soups).

Thus, according to viscometric data at a certain shear rate, it is possible to predict an adequate human response to sensory viscosity assessment. However, conclusions based on the study of only the ratios of instrumental and sensory assessments cannot be simply mechanically used for broad generalizations about product quality. In this case, the following should be taken into account: the values ​​of the correlation coefficients; characteristics measured by this device and experimental conditions; the homogeneity of the tasting product samples; methods for decoding sensory assessments. thus, to assess the quality of each product, it is necessary to carry out a series of rheological measurements with parallel tasting by specialists to identify practical ranges of the relationship of the measured rheological characteristics with the consistency of the product.

Control rheology

When creating mechanized production lines, it is necessary to provide for continuous quality control of semi-finished products. The existing chemical and organoleptic methods of assessment do not allow determining the quality state of the bakery dough directly in the process of technological processing. The most promising are rheological methods, since relationships can be established between the quality of the dough and its properties, for example, viscosity, which can be used not only for quality control, but also for its regulation in the production process.

It should be taken into account that food materials are complex organic multicomponent systems, and it is not enough to measure any one parameter to control their condition. Therefore, it is advisable to use a multiparameter computational method.

Determination of a generalized indicator of the quality of a multicomponent product can be reduced to measuring a small number of integral parameters (density, viscosity, pH, electrical conductivity, etc.) and solving the regression equation. The measurement of individual parameters is carried out using appropriate sensors, information from which in analog or discrete form is sent to a device for solving the regression equation - a calculating device. The result of the solution is issued in analog or discrete form for further use in automatic control and regulation systems. For example, for bakery dough, the following parameters can be selected: viscosity, acidity and temperature.

DХ1

DХ2

DХ3

ВП1

ВП2

ВП3

Пр

Calculatingdevice

Result

DХ1

DХ2

DХ3

ВП1

ВП2

ВП3

Пр

Calculatingdevice

Result

Fig.4.1. Block diagram of the automated

test quality control

The dependence allows us to propose an automatic quality control system, the block diagram of which is shown in Fig. 4.1. Sensors of acidity DX1, viscosity DX2 and temperature DX3 transmit signals of measuring information about the current values ​​of the main parameters of the test to the secondary devices VP1, VP2 and VP3. Then the signals go to the calculating device (SRU). Since the automatic viscosity sensor DX2 generates pneumatic signals, a pneumo-electric converter Pr is installed behind the secondary device VP2, which converts the received signals and then sends them to the calculating device, which gives the result in a form convenient for direct perception.

The rheological properties of semi-finished products can be successfully used to assess the performance of individual processes. Technological machines of various designs, intended for the implementation of the same process, differ in the degree of impact on the processed mass, which leads to the production of end products with different structural and mechanical properties, and the production of finished products of different quality. The study of the influence of individual machines on the properties of semi-finished products allows you to choose the most appropriate designs for the production of a given type of bread and select the optimal processing modes.

One of the most urgent tasks of increasing the efficiency of bakery production is the automation of dough kneading. The process of continuous kneading of the dough is carried out on kneading units, including a continuous kneading machine, dispensers for flour and liquid components, and control and measuring equipment.

In the process of operation, the kneading machine is affected by various disturbances, under the influence of which the controlled value of the pressure takes on a value different from the set one, therefore, the task of automatic control is to return the value of the controlled parameter to the stabilized value. To do this, the regulator makes a control action of a certain value on the control object until the pressure is equal to the preset one. The regulating parameter can be the dosage amount of one of the components coming from the dispensers. For the correct choice of a regulating component, it is necessary to take into account the conditions of its dosage and the change in the rheological properties of the dough depending on the different percentage of this component, provided that the dosage of the remaining components required by the recipe is constant.

Studies show that the consistency of the dough is more influenced by the ratio of the dosage of flour and yeast solution. It is advisable to choose a yeast solution as a regulating component, since it can be dosed by an automatic dosing station.

To carry out the process of adjusting the consistency of the kneaded dough, you need to know the function:

∆р = ∆р (∆η),

where: ∆р is the deviation of the outlet pressure from the stabilized value corresponding to the set value of the outlet pressure,%;

∆η - deviation of viscosity from the value corresponding to the specified value of the outlet pressure,%.

In turn, ∆η can be associated with the required control action - changing the dosage of the yeast solution ∆GДР, i.e. find dependency:

∆η = ∆η(∆GДР).

To identify the latter dependence, it is necessary to conduct appropriate experiments. For a number of bakery products, the results are available in the relevant literature.

Using the method of calculating the flow in channels of complex shape, it is possible, given a different content of yeast solution, to determine the corresponding values of the working pressure at the location of the sensor of the automatic control system.

∆Gдр, %

∆р, %

Fig. 4.2. Static

object characteristic

regulation ∆η = ∆η (∆Gdr)

Based on the calculation results, a graph ∆η = ∆η (∆GДР) was built (Fig. 4.2). The obtained linear dependence in relation to the kneading machine as a control object is a static characteristic of the control system. The tangent of the slope of the resulting straight line determines the transmission coefficient of the BER object. For the case under consideration, the BER = 2.6. The value of pressure at normal dosage of all components differed from the average value obtained experimentally by 12%.

To the course of the automatic regulation process in time, i.e. to the dynamics of regulation, certain requirements are imposed. When choosing a regulator and adjusting it, it is necessary to take into account the properties of the controlled object, characterized by the ratio t / T (t is the transport delay time, s; T is the time constant of the object, s).

The value of t is determined from the acceleration curves by measuring the time interval from the moment the disturbance is applied to the moment the consistency sensor responds to this disturbance. The transport lag time is determined by the average speed of the dough moving through the kneading container. The time constant characterizing the intensity of the consistency change is also determined from the acceleration curves.

For a given control object, consisting of a kneading machine and a dosing station, an isodromic control law is applicable. The isodromic, or proportional-integral (PI) controller has two tuning parameters: the coefficient of transmission of the KR regulator and the reset time TIZ.

The transfer ratio of the regulator is:

КР = КС / КОБ,

where: КС - transmission coefficient of the regulation system;

BER is the transfer factor of the object.

The block diagram of automatic control is shown in Fig. 4.3. The signal from the sensor D, proportional to the value of the working pressure in the outlet pipe of the machine, enters the electronic regulator. In the measuring unit of the IB controller, the circuit of which depends on the type of sensor, the electronic signal is multiplied by a constant coefficient and compared with the specified one. The setting of the setpoint of the controlled parameter is carried out by a dial (usually a wirewound potentiometer). The measuring unit is an AC bridge, the output of which is connected to a phase-sensitive amplifier.

In case of deviation from the set value, the unbalance signal is sent to the electronic unit of the electronic unit, which amplifies the unbalance signal, forms the corresponding regulation law and acts through the output element of the SE on the PU starting device (usually a magnetic starter) of the IM actuator. In addition, the electronic unit receives signals from a non-contact position sensor, which provides feedback (feedback) on the position of the regulator.

The main elements of the electric actuator mounted at the VNIIHP dosing station are an electric motor, a gearbox, an output device for mechanical connection with the RO regulator and additional devices that provide self-braking when the electric motor is turned off, stopping the mechanism in extreme positions, etc. The regulating body is a piston that occupies a certain level in the glass of the yeast dispenser, depending on the value of the working pressure in the outlet of the machine. The scheme also provides an indicator of the position of the regulatory body UP.

Control questions

1. Evaluation of food quality

2. Optimization of technological processes based on engineering rheology

A system for assessing the students' results of educational achievements

| Estimation in letters | Equivalent of scores in figures | In per cent | Traditional assessment | Criteria for evaluation students knowledge |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | excellent | The student demonstrates: - a deep and strong assimilation of the program material - complete, consequent, competent and logically stated answers for all types of classes; - ability to cope with tasks freely; - correct, reasonable decisions, - skills of using information of the main and additional special literature in the study of the discipline, - the ability to self-systematization of program material; - possession by various skills and techniques of performance of all types of tasks; - ability to work with foreign literature and Internet information resources; - timely and high-quality performance of all types of tasks. |
| А- | 3,67 | 90-94 |  | The student demonstrates: - deep assimilation of program material; - complete, consequent and logically stated answers for all types of classes; - ability to cope with the tasks; - the right decisions, - skills of using special literature in the study of the discipline, - the ability to self-systematization of program material; - knowledge of skills and techniques for performing all types of tasks; - timely execution of all types of tasks. |
| В+ | 3,33 | 85-89 | good | The student demonstrates: - mastering the program material; - complete, consequent, competent, without significant inaccuracies, the presentation of answers to all types of tasks; - correct application of theoretical knowledge - possession by the necessary skills in the performance of applied tasks - skills of using the recommended literature in the study of the discipline, - skills of systematization of program material; - knowledge of skills and techniques for performing all types of tasks; - timely fulfillment of all types of tasks. |
| В | 3,0 | 80-84 |  | The student demonstrates: - mastering the program material; - consistent presentation of answers to all types of tasks with minor errors; - skills of applying theoretical knowledge under the guidance of a teacher; - possession of the necessary skills in the performance of practical tasks - skills of using the recommended literature in the study of the discipline, - skills of systematization of the program material under the guidance of a teacher; - proficiency in performing all types of tasks; - ability to self-correction of mistakes; - timely fulfillment of all types of tasks with the elimination of mistakes. |
| В- | 2,67 | 75-79 |  | The student demonstrates: - mastering the program material; - ability to present answers with minor errors; - skills of applying theoretical knowledge under the guidance of a teacher; - possession of techniques in the performance of practical tasks - skills of using the recommended literature under the guidance of a teacher - skills of generalization of the program material under the guidance of a teacher; - ability to correct mistakes with the help of a teacher; - timely fulfillment of all types of tasks with the elimination of mistakes. |
| С+ | 2,33 | 70-74 |  | The student demonstrates: - mastering the basic material; - not enough correct wording in the answers to all types of tasks; - violation of the sequence in the presentation of the program material; - difficulty in independent performance of practical tasks; - possession of individual techniques in the performance of practical tasks - skills of using the literature recommended by the teacher; - skills of generalization of individual sections of the program material under the guidance of a teacher; - ability to correct gross mistakes with the help of a teacher; - execution of all types of tasks with the elimination of mistakes. |
| С | 2,0 | 65-69 | satisfactory | The student demonstrates: - mastering the basic material; - lack of understanding of the wording in the answers to all types of tasks; - lack of consistency in the presentation of the material; - difficulty in independent performance of practical tasks; - possession of individual techniques in the performance of tasks - difficulties in using the literature recommended by the teacher; - difficulties in summarizing the individual sections of the studied material; - ability to correct gross mistakes with the help of a teacher; - fulfillment of all types of tasks with the elimination of mistakes. |
| С- | 1,67 | 60-64 |  | The student demonstrates: - mastering the basic material; - lack of understanding of the wording in the answers to all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in independent implementation of practical tasks; - lack of knowledge of individual techniques in the performance of tasks - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of certain sections of the studied material; - ability to correct gross mistakes with the help of a teacher; - execution of all types of tasks with the elimination of mistakes. |
| Д+ | 1,33 | 55-59 |  | The student demonstrates: - assimilation of separate sections of the main material; - misunderstanding of the wording in the answers to all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in independent implementation of practical tasks; - significant difficulties in the application of certain methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of certain sections of the studied material; - difficulties in correcting the mistakes made by the teacher; - untimely fulfillment. |
| Д | 1,0 | 50-54 |  | The student demonstrates: - difficulties with the assimilation of individual sections of the main material; - misunderstanding of the wording in the answers to all types of tasks; - lack of consistency in the presentation of the material; - significant difficulties in independent implementation of practical tasks; - inability to use certain methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - inability to generalize certain sections of the studied material; - significant difficulties in correcting the mistakes made by the teacher; - untimelyfulfiilment of all types of tasks with the elimination of mistakes. |
| FX | 0.5 | 25-49 |  | The student demonstrates: |
| F | 0 | 0-49 | unsatisfactory | The student demonstrates: - absenceofknowledge of the program material, - rude errors are made at performing all types of tasks; - lack of skills in the use of individual methods of performing tasks; - failure to perform certain types of tasks provided by the forms of current, intermediate and final control. |

List of literature

Basic literature

- Косой, В.Д. Инженерная реология биотехнологических сред: учебное пособие для студентов вузов, обучающихся по направлению "Технология сырья и продуктов животного происхождения / В. Д. Косой, Я. И. Виноградов, А. Д. Малышев. - СПб. : Гиорд, 2012. - 648с.

- Косой, В.Д. Инженерная реология в производстве колбас : Учебное пособие для студ. вузов обуч. по направ. "Технология сырья и продуктов животного происхождения" / В. Д. Косой, А. Д. Малышев, С. Б. Юдина; Рекомендовано УМО по образованию в области технологии сырья и продуктов животного происхождения. - М.: Колос С, 2012. - 264 с.: ил. - (Учебники и учебные пособия для студ. вузов).

- Мачихин, Ю.А.Реология пищевых продуктов : Учебное пособие / Ю.А. Мачихин, Ю.К. Берман ; Ю. А. Мачихин, Ю. К. Берман. - М. : МГУПП. – 2010 г., 250с.

- Физические методы обработки пищевых продуктов / Рогов И.А., Горбатов А.В. - М. : Пищевая промышленность, 2011. - 583 с.

Additional literature

1. Шамбулова, Г.Д. Конспект лекции для студентов спец. 5В072700- "Технология продовольственных продуктов" по дисциплине "Реология продовольственных продуктов" / Г. Д. Шамбулова. - Шымкент: ЮКГУ, 2013. - 84 с.

2. Шамбулова, Г.Д. Методические указания для студ. спец. 5В072700- "Технология продовольственных продуктов" к лабораторным занятиям по дисциплине "Реология продовольственных продуктов" / Г. Д. Шамбулова, Н. У. Камалова. - Шымкент : ЮКГУ, 2013. - 68 с.

3. Скуратовская, О.Д.  Контроль качества продукции физико-химическими методами [Текст] / Скуратовская,О.Д. - М. : Дели-принт. – 2001  : Сахар и сахарные кондитерские изделия. - 122 с.

Zheleuova Zh.S., Utebaeva A.A., Koshtayeva G.E.

Lecture notes on the discipline

“Structural and mechanical properties of food products”

Signed in print __________________________

(day.month.year)

Paper size XxY  1/16

Typographic paper.Offset printing. Volume ...... printed sheets.

Circulation ....... copies. Order No. …….*

©  Publication of the M.Auezov South Kazakhstan University

Publishing Center of the M.Auezov SKU, Shymkent, Tauke Khan Ave., 5

*These values are set by IC