F.7.03-13

Korolkov A.V.

Case on theme « Circuit models for the semiconductor diode »

on discipline « Digital and Microprocessor Technology »

for students of EP  «6В07150- Electroenergetics

MINISTRY OF SCIENCE AND HIGHER EDUCATION OF THE REPUBLIC OF KAZAKHSTAN

M.AUEZOV SOUTH KAZAKHSTAN UNIVERSITY

Chair «Energetics and renewable energy system»

Korolkov A.V.

Case on theme « Circuit models for the semiconductor diode »

on discipline « Digital and Microprocessor Technology »

for students of EP  «6В07150- Electroenergetics

Shymkent, 2022

UDK 621.315

The authors:  Case on theme «Circuit models for the semiconductor diode» on discipline « Digital and Microprocessor Technology » for students of EP  «6В07150- Electroenergetics Supply . Shymkent: SKU, 2022. –_32_ p.

Case on theme « Circuit models for the semiconductor diode » on discipline « Digital and Microprocessor Technology »  have been drawn up in accordance with the curriculum and course program on discipline « Digital and Microprocessor Technology » and includes all necessary information on carrying-out of lessons connected with themes.

The case includes the following sections: the ability to explain the i-v characteristic of a semiconductor diode (or of a pn junction); the ability to use the ideal, offset, and piecewise linear diode models in simple circuits. This case shows how it is possible to use the i-v characteristics of the semiconductor diode to construct simple yet useful circuit models.

The developed case will be useful to students when studying discipline of " Digital and Microprocessor Technology " as they include all the necessary information on the theme « Circuit models for the semiconductor diode ».

Reviewer:  Berdalieva A.A. – Candidate of Engineering Sciences, Associate Professor of department of “Pharmaceutical manufacturing technology”, South Kazakhstan Medical Academy.

Сase  have been discussed at the meeting of the chair «Energetics and renewable energy systems»   (Minutes №  3     from  «   27     »  10      2022y.) and they have been approved by methodical commission of the faculty  « Information Technologies » (Minutes №  3     from  « 30    »  11     2022y.)

Collection of lectures  are recommended to the edition by Educational-methodical council of M.Auezov SKU,

Minutes №              from  «          »             2022 y.

© M.Auezov South Kazakhstan University, 2022

Responsible for printing:  Korolkov A.V.

CONTENT

| Introduction…………………………………………………………… | 6 |
| --- | --- |
| 1. Large-Signal Diode Models | 7 |
| 1.1. The principle of operation of the diode | 7 |
| 1.2 Ideal Diode Model | 9 |
| 1.3. Offset Diode Mode | 14 |
| 2. Small-Signal Diode Models | 15 |
| 2.1. Piecewise Linear Diode Model | 17 |
| Questions for self-control | 19 |
| Conclusion……………………………………………………………… | 20 |
| Criteria for assessing the knowledge and skills of students in Case work…... | 21 |
| List of references…………………………………………………..... | 26 |

Introduction

From the viewpoint of a user of electronic circuits (as opposed to a designer), it is often sufficient to characterize a device in terms of its i-v characteristic, using either load-line analysis or appropriate circuit models to determine the operating currents and voltages. This case shows how it is possible to use the i-v characteristics of the semiconductor diode to construct simple yet useful circuit models. Depending on the desired level of detail, it is possible to construct large-signal models of the diode, which describe the gross behavior of the device in the presence of relatively large voltages and currents; or small-signal models, which are capable of describing the behavior of the diode in ﬁner detail and, in particular, the response of the diode to small changes in the average diode voltage and current. From the user’s standpoint, these circuit models greatly simplify the analysis of diode circuits and make it possible to effectively analyze relatively “difﬁcult” circuits simply by using the familiar circuit analysis tools.

1 Large-Signal Diode Models

1.1. The principle of operation of the diode

To understand the operation of the semiconductor diode intuitively, we make reference to a very common hydraulic device that ﬁnds application whenever one wishes to restrict the ﬂow of a ﬂuid to a single direction, and prevent (check) reverse ﬂow. Hydraulic check valves perform this task in a number of ways. We illustrate a few examples in this box.

Figure 1

Figure 1 depicts a swing check valve. In this design, ﬂow from left to right is permitted, as the greater ﬂuid pressure on the right side of the valve forces the swing “door” to open. If ﬂow were to reverse, the reversal of ﬂuid pressure (greater pressure on the right) would cause the swing door to shut.

Figure 2 depicts a ﬂapper check valve. The principle is similar to that described above for the swing check valve. In Figure 2, ﬂuid ﬂow is permitted from left to right, and not in the reverse direction. The response of the valve of Figure 2 is faster (due to the shorter travel distance of the ﬂapper) than that of Figure 1.

Figure 2

You will ﬁnd the analysis of the diode circuits in this chapter much easier to understand intuitively if you visualize the behavior of the diode to be similar to that of the check valves shown here, with the pressure difference across the valve oriﬁce being analogous to the voltage across the diode, and the ﬂuid ﬂow rate being analogous to the current through the diode.

Figure 3

Figure 3 depicts the diode circuit symbol. Current ﬂows only from left to right whenever the voltage across the diode is positive, and no current ﬂows when the diode voltage is reversed. The circuit element of Figure 3 is functionally analogous to the two check valves of Figures 1 and 2.

1.2 Ideal Diode Model

Our ﬁrst large-signal model treats the diode as a simple on-off device (much like a check valve in hydraulic circuits).

Figure 4 illustrates how, on a large scale, the i-v characteristic of a typical diode may be approximated by an open circuit when vD < 0 and by a short circuit when vD ≥ 0 (recall the i-v curves of the ideal short and open circuits).

The analysis of a circuit containing a diode may be greatly simpliﬁed by using the short-circuit–open-circuit model. From here on, this diode model will be known as the ideal diode model. In spite of its simplicity, the ideal diode model (indicated by the symbol shown in Figure 4) can be very useful in analyzing diode circuits

Figure 4 Large-signal on-off diode model

Consider the circuit shown in Figure 5, which contains a 1.5-V battery, an ideal diode, and a 1-kOhm resistor. A technique will now be developed to determine whether the diode is conducting or not, with the aid of the ideal diode model.

Assume ﬁrst that the diode is conducting (or, equivalently, that vD ≥ 0). This enables us to substitute a short circuit in place of the diode, as shown in Figure 5, since the diode is now represented by a short circuit, vD = 0. This is consistent with the initial assumption (i.e., diode “on”), since the diode is assumed to conduct for vD ≥ 0 and since vD = 0 does not contradict the assumption.

Figure 5 Circuit containing ideal diode

The series current in the circuit (and through the diode) is iD = 1.5/1,000 = 1.5 mA. To summarize, the assumption that the diode is on in the circuit of Figure 5 allows us to assume a positive (clockwise) current in the circuit. Since the direction of the current and the diode voltage are consistent with the assumption that the diode is on (vD ≥ 0,iD > 0), it must be concluded that the diode is indeed conducting.

Figure 5 Circuit of Figure 4, assuming that the ideal diode conducts

Suppose, now, that the diode had been assumed to be off. In this case, the diode would be represented by an open circuit, as shown in Figure 6. Applying KVL to the circuit of Figure 6 reveals that the voltage vD must equal the battery voltage, or vD = 1.5 V, since the diode is assumed to be an open circuit and no current ﬂows through the circuit.

Figure 6 Circuit of Figure 4, assuming that the ideal diode does not conduct

Equation 1 must then apply.

1.5=vD+1000*iD=vD				(1)

But the result vD = 1.5 V is contrary to the initial assumption (i.e., vD < 0). Thus, assuming that the diode is off leads to an inconsistent answer. Clearly, the assumption must be incorrect, and therefore the diode must be conducting.

This method can be very useful in more involved circuits, where it is not quite so obvious whether a diode is seeing a positive or a negative bias. The method is particularly effective in these cases, since one can make an educated guess whether the diode is on or off and solve the resulting circuit to verify the correctness of the initial assumption. Some solved examples are perhaps the best way to illustrate the concept.

One of the important applications of the semiconductor diode is rectiﬁcation of AC signals, that is, the ability to convert an AC signal with zero average (DC) value to a signal with a nonzero DC value. The application of the semiconductor diode as a rectiﬁer is very useful in obtaining DC voltage supplies from the readily available AC line voltage. Here, we illustrate the basic principle of rectiﬁcation, using an ideal diode—for simplicity, and also because the large-signal model is appropriate when the diode is used in applications involving large AC voltage and current levels

Consider the circuit of Figure 8.20, where an AC source, vi = 155.56 sin ωt, is connected to a load by means of a series ideal diode.

Figure 7.

The diode will conduct only during the positive half-cycle of the sinusoidal voltage—that is, that the condition vD ≥ 0 will be satisﬁed only when the AC source voltage is positive—and that it will act as an open circuit during the negative half-cycle of the sinusoid (vD < 0). Thus, the appearance of the load voltage will be as shown in Figure 8.21, with the negative portion of the sinusoidal waveform cut off. The rectiﬁed waveform clearly has a nonzero DC (average) voltage, whereas the average input waveform voltage was zero. When the diode is conducting, or vD ≥ 0, the unknowns vL and iD can be found by using the following equations:

(2)

and

(3)

The load voltage, vL, and the input voltage, vi, are sketched in Figure 8. From equation 3, it is obvious that the current waveform has the same shape as the load voltage. The average value of the load voltage is obtained by integrating the load voltage over one period and dividing by the period:

(4)

The circuit of Figure 7 is called a half-wave rectiﬁer, since it preserves only half of the waveform. This is not usually a very efﬁcient way of rectifying an AC signal, since half the energy in the AC signal is not recovered. It will be shown in a later section that it is possible to recover also the negative half of the AC waveform by means of a full-wave rectiﬁer.

Figure 8 Ideal diode rectiﬁer input and output voltages

1.3. Offset Diode Mode

While the ideal diode model is useful in approximating the large-scale characteristics of a physical diode, it does not account for the presence of an offset voltage, which is an unavoidable component in semiconductor diodes. The offset diode model consists of an ideal diode in series with a battery of strength equal to the offset voltage (we shall use the value Vγ = 0.6 V for silicon diodes, unless otherwise indicated). The effect of the battery is to shift the characteristic of the ideal diode to the right on the voltage axis, as shown in Figure 9. This model is a better approximation of thelarge-signal behavior of a semiconductor diode than the ideal diode model.

According to the offset diode model, the diode of Figure 9 acts as an open circuit for vD < 0.6 V, and it behaves like a 0.6-V battery for vD ≥ 0.6V.The equations describing the offset diode model are as follows:

(5)

The diode offset model may be represented by an ideal diode in series with a 0.6- V ideal battery, as shown in Figure 9. Use of the offset diode model is best described by means of examples.

Figure 9 Offset diode as an extension of ideal diode model

2. Small-Signal Diode Models

As one examines the diode i-v characteristic more closely, it becomes apparent that the short-circuit approximation is not adequate to represent the small-signal behavior of the diode. The term small-signal behavior usually signiﬁes the response of the diode to small time-varying signals that may be superimposed on the average diode current and voltage.

Figure 10 depicts a close-up view of a silicon diode i-v curve. From this ﬁgure, it should be apparent that the short-circuit approximation is not very accurate when a diode’s behavior is viewed on an expanded scale. To a ﬁrst-order approximation, however, the i-v characteristic resembles that of a resistor (i.e., is linear) for voltages greater than the offset voltage.

Thus, it may be reasonable to model the diode as a resistor (instead of a short circuit) once it is conducting, to account for the slope of its i-v curve. In the following discussion, the method of load-line analysis  will be exploited to determine the small-signal resistance of a diode.

Figure 10 Offset diode as an extension of ideal diode model

Consider the circuit of Figure 11, which represents the Theґvenin equivalent circuit of an arbitrary linear resistive circuit connected to a diode. Equations 6  and 7 describe the operation of the circuit:

(6)

arises from application of KVL, and

(7)

Although we have two equations in two unknowns, these cannot be solved analytically, since one of the equations contains vD in exponential form.

Figure 11 Diode circuit for illustration of load-line analysis

Two methods exist for the solution of transcendental equations of this type: graphical and numerical. In the present case, only the graphical solution shall be considered. The graphical solution is best understood if we associate a curve in the iD-vD plane with each of the two preceding equations. The diode equation gives rise to the familiar curve of Figure 8.8. The load-line equation, obtained by KVL, is the equation of a line with slope −1/R and ordinate intercept given by VT /RT .

(8)

The superposition of these two curves gives rise to the plot of Figure 8.30, where the solution to the two equations is graphically found to be the pair of values (IQ, VQ).

The intersection of the two curves is called the quiescent (operating) point, or Q point. The voltage vD = VQ and the current iD = IQ are the actual diode voltage and current when the diode is connected as in the circuit of Figure 8.29. Note that this method is also useful for circuits containing a larger number of elements, provided that we can represent these circuits by their Theґvenin equivalents, with the diode appearing as the load.

Figure 12 Graphical solution of equations 6 and 7

2.1. Piecewise Linear Diode Model

The graphical solution of diode circuits can be somewhat tedious, and its accuracy is limited by the resolution of the graph; it does, however, provide insight into the piecewise linear diode model. In the piecewise linear model, the diode is treated as an open circuit in its off state, and as a linear resistor in series with Vγ in the on state. Figure 13 illustrates the graphical appearance of this model. Note that the straight line that approximates the “on” part of the diode characteristic is tangent to the Q point. Thus, in the neighborhood of the Q point, the diode does act as a linear small-signal resistance, with slope given by 1/rD, where

(9)

That is, it acts as a linear resistance whose i-v characteristic is the tangent to the diode curve at the operating point. The tangent is extended to meet the voltage axis, thus deﬁning the intersection as the diode offset voltage. Thus, rather than represent the diode by a short circuit in its forward-biased state, we treat it as a linear resistor, with resistance rD. The piecewise linear model offers the convenience of a linear representation once the state of the diode is established, and of a more accurate model than either the ideal or the offset diode model. This model is very useful in illustrating the performance of diodes in real-world applications.

Figure 8.35 Piecewise linear diode model

Questions for self-control

- Give the definition of the semiconductor

- Draw a diagram of forward-biased and reverse-biased pn junctions and explain it

- Draw a diagram of semiconductor diode i-v characteristic and explain it. Draw a semiconductor diode circuit symbol

- Explain the large-signal diode models. Draw the appropriate diagrams and graphs

- Explain how to determine the conductivity states of the ideal diode

- Explain the using the offset diode model in a half-wave rectiﬁer

- Explain the small-signal diode models. Draw the appropriate diagrams and graphs

CONCLUSION

Semiconductor materials have conductive properties that fall between those of conductors and insulators. These properties make such materials useful in the construction of many electronic devices that exhibit nonlinear i-v characteristics. Of these devices, the diode is one of the most commonly employed.

The semiconductor diode acts like a one-way current valve, permitting the ﬂow of current only when biased in the forward direction. Although the behavior of the diode is described by an exponential equation, it is possible to approximate the operation of the diode by means of simple circuit models. The simplest model treats the diode as either a short circuit or an open circuit (the on-off, or ideal, model). The ideal model can be extended to include an offset voltage (usually 0.2 to 0.7 V), which represents the contact potential at the diode junction. A slightly more realistic model, the piecewise linear diode model, accounts for the effects of the diode forward resistance. With the aid of these circuit models it becomes possible to analyze diode circuits using the DC and AC circuit analysis techniques

One of the most important properties of the semiconductor diode is that of rectiﬁcation, which permits the conversion of AC voltages and currents to DC voltages and currents..

Criteria for assessing students' knowledge

| Evaluation by letter system | Numeral equivalent of points | %- content | Evaluation according to the traditional system | Criteria for assessing students' knowledge |
| --- | --- | --- | --- | --- |
| А | 4,0 | 95-100 | Excellent | The student demonstrates: - a deep and lasting assimilation of program material - complete, consistent, competent and logically presented answers for all types of activities; - Ability to freely cope with the tasks; - correct, well-founded decisions, - skills in the use of information from basic and additional specialized literature in the study of discipline, - the ability to self-systematize the program material; - Possession of versatile skills and methods of performing all types of tasks; - Ability to work with foreign literature and information resources of the Internet; - Timely and high-quality execution of all types of tasks. |
| А- | 3,67 | 90-94 |  | The student demonstrates: - Deep assimilation of program material; - complete, consistent and logically presented answers for all activities; - ability to cope with the tasks; - the right decisions, - skills of using special literature in the study of discipline, - the ability to self-systematize the program material; - Possession of skills and techniques for performing all types of tasks; - Timely execution of all types of tasks. |
| В+ | 3,33 | 85-89 | Good | The student demonstrates: - assimilation of program material; - complete, consistent, literate, without significant inaccuracies, an account of the answers for all kinds of tasks; - correct application of theoretical knowledge - Possession of necessary skills in the performance of applied tasks - skills in using recommended literature in the study of discipline, - skills of systematization of program material; - Possession of skills and techniques for performing all types of tasks; - Timely execution of all types of tasks. |
| В | 3,0 | 80-84 |  | The student demonstrates: - assimilation of program material; - a consistent presentation of the answers for all types of tasks with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - Possession of necessary skills in performing practical tasks - skills in using recommended literature in the study of discipline, - skills of systematization of program material under the guidance of a teacher; - mastering the skills of performing all kinds of tasks; - the ability to independently correct mistakes made; - the timely execution of all types of tasks with the elimination of errors. |
| В- | 2,67 | 75-79 |  | The student demonstrates: - assimilation of program material; - Ability to present answers with non-essential errors; - skills in applying theoretical knowledge under the guidance of a teacher; - Possession of techniques in the performance of practical tasks - skills in using recommended literature under the guidance of a teacher - skills of generalization of program material under the guidance of a teacher; - the ability to correct mistakes made with the help of a teacher; - the timely execution of all types of tasks with the elimination of errors. |
| С+ | 2,33 | 70-74 | satisfactorily | The student demonstrates: - mastering of the main material; - Inadequate formulations when responding to all kinds of tasks; - violation of the sequence in the presentation of the program material; - difficulties in self-fulfillment of practical tasks; - Possession of individual techniques in the performance of practical tasks - skills in using the literature recommended by the teacher; - skills of generalization of separate sections of program material under the guidance of a teacher; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С | 2,0 | 65-69 |  | The student demonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - difficulties in self-fulfillment of practical tasks; - Possession of separate receptions at performance of tasks - difficulty in using the literature recommended by the teacher; - difficulties in generalizing the individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| С- | 1,67 | 60-64 |  | The student demonstrates: - mastering of the main material; - Inadequate understanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - Inadequate possession of individual methods in the performance of assignments - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - the ability to correct major mistakes made with the help of a teacher; - execution of all types of tasks with elimination of errors. |
| D+ | 1,33 | 55-59 |  | The student demonstrates: - mastering of separate sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - significant difficulties in the application of individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - significant difficulties in the generalization of individual sections of the material studied; - difficulties in correcting the mistakes made by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| D | 1,0 | 50-54 |  | The student demonstrates: - difficulties in mastering individual sections of the main material; - misunderstanding of formulations when answering all types of assignments; - lack of consistency in the presentation of the material; - significant difficulties in the independent implementation of practical tasks; - Inability to apply individual methods of performing tasks; - significant difficulties in using the literature recommended by the teacher; - Inability to generalize the individual sections of the material studied; - significant difficulties in correcting the mistakes committed by the teacher; - untimely execution of all types of tasks with elimination of errors. |
| FX | 0,5 | 25-49 | unsatisfactory | The student demonstrates: - lack of skills in applying individual tasks; - non-fulfillment of certain types of tasks provided by the forms of current, intermediate and final control. |
| F | 0 | 0-49 | unsatisfactory | The student demonstrates: -- not having knowledge of program material, - When performing all types of tasks, gross errors are allowed; - lack of skills in applying individual tasks; - non-fulfillment of certain types of tasks provided by the forms of current, intermediate and final control. |

List of references

- Taranov A. V. Basics of electronics / Taranov A. V., Neshina Y. G., Mekhtiyev A. D., Kalinin A. A., Bulatbayeva Y. F., Balandin V. S., Alkina A. D., 2020. - 176 c.  https://elib.kz/ru/search/read_book/7167/

- ZHarmakin B. K. Cifrlіk elektronika elementterіn arduino platformasynda қҧrastyru zhәne baғdarlamalau / ZHarmakin B. K., 2018. - 193 c. https://elib.kz/ru/search/read_book/3252/

- Manukovskij, A. V.     Mikroprocessornye sredstva i sistemy upravleniya [Tekst] : uchebnoe posobie / A. V. Manukovskij, A. ZH. Sarinova. - Almaty : CyberSmith, 2019. - 308 s.  http://www.lib.ukgu.kz/cgi-bin/irbis64r_01/cgiirbis_64.exe

- Ettel' V. A. Cifrovaya skhemotekhnika / Ettel' V. A., Sinkevich N. N., 2020. - 113 c. https://elib.kz/ru/search/read_book/7105/

- Burambaeva, N. A.     Mikroprocessorlyқ құraldar zhәne zhүjeler [Tekst] : zerthanalyқ zhұmystarғa әdіstemelіk nұsқaular / N. A. Burambaeva. - Almaty : TekhnoErudit, 2018. - 88 s.  http://www.lib.ukgu.kz/cgi-bin/irbis64r_01/cgiirbis_64.exe

- Manukovskij A. V. Mikroprocessornye sredstva i sistemy upravleniya / Manukovskij A. V., Sarinova A. ZH., 2019. - 309 c. https://elib.kz/ru/search/read_book/5157/

- ZHilkishbaeva G.S., Sұlbatekhnika: oқu құraly. - ZHK "LP-Zhasulan". - Almaty, 2019. - 108 b.https://aknurpress.kz/reader/web/2029

- ZHұryntaev ZH.Z. Sұlbatekhnika : ҚR Bіlіm zhәne ғylym min. zhoғary oқu oryndaryndaғy tekhnikalyқ mamand. stud. arnalғan  oқulyқ  / ZH. Z. ZHұryntaev. - Almaty : Dәuir, 2011. - 288 s. - ("ҚR ZHoғary oқu oryndarynyң қauymdastyғy"). (SHifr 6P2.154/ZH 89-158978)

- Balabekova M. O. CIFROVYE USTROJSTVA I MIKROPROCESSORY / Balabekova M. O., 2020. - 165 c. https://elib.kz/ru/search/read_book/6625/

- Kazagachev V. N. Osnovy mikroprocessornoj tekhniki: uchebnoe posobie. 3 tom / Kazagachev V. N., 2020. - 165 c. https://elib.kz/ru/search/read_book/6563/

- Kazagachev V. N. Osnovy mikroprocessornoj tekhniki: uchebnoe posobie. 2 tom / Kazagachev V. N., 2020. - 393 c. https://elib.kz/ru/search/read_book/6562/

- Kazagachev V. N. Osnovy mikroprocessornoj tekhniki 1 tom / Kazagachev V. N., 2020. - 293 c. https://elib.kz/ru/search/read_book/5503/

- Burambaeva N. A. Mikroprocessorlyқ құraldar zhәne zhүjeler / Burambaeva N. A., 2020. - 89 c. https://elib.kz/ru/search/read_book/3354/

- Zarubin M. YU. Skhemotekhnika / Zarubin M. YU., 2020. - 221 c. https://elib.kz/ru/search/read_book/1448/

- Balabekova, M. O.     Cifrovoe ustrojstva i mikroprocessory [Tekst] : uchebnoe posobie dlya stud. spec. 5V071900- "Radiotekhnika, elektronika i telekommunikacii" , "Cifrovaya tekhnika i mikrokontrollery", 5V070200- "Avtomatizaciya i upravlenie" / M. O. Balabekova. - Almaty : Epigraf, 2019. - 164 s.  http://www.lib.ukgu.kz/cgi-bin/irbis64r_01/cgiirbis_64.exe

- Berdalieva, A. A.     Mikroprocessornye sredstva upravleniya v elektroenergetike [Tekst] : konspekt lekcij dlya magistrantov special'nosti 6M071800 - "Elektroenergetika" / A. A. Berdalieva, R. M. Il'yasov, V. A. Ovchinnikov. - SHymkent : YUKGU, 2018. - 184 s. .http://www.lib.ukgu.kz/cgi-bin/irbis64r_01/cgiirbis_64.exe

- SHermantaeva, ZH. U.    Avtomatika құrylғylary men elementterі [Tekst] : oқu құraly / ZH. U. SHermantaeva. - SHymkent : TOO "Evrika-media", 2017. - 200 s.http://www.lib.ukgu.kz/cgi-bin/irbis64r_01/cgiirbis_64.exe

- Tazhibaeva B. T. «Elektronika» / Tazhibaeva B. T., 2021. - 80 c. https://elib.kz/ru/search/read_book/7239/

- Taranov A. V. Osnovy elektroniki / Taranov A. V., Neshina E. G., Mekhtiev A. D., Kokkoz M. M., Ignatov A. N., Bulatbaev F. N., Al'kina A. D., 2020. - 200 c. https://elib.kz/ru/search/read_book/7169/

- Қabasova ZH. Қ. ӨNDІRІSTІK ELEKTRONIKA Oқu құraly / Қabasova ZH. Қ., Erқoңyr Ә. K., 2020. - 129 c. https://elib.kz/ru/search/read_book/6548/

- Ettel' V. A. Cifrovaya skhemotekhnika / Ettel' V. A., Sinkevich N. N., 2020. - 113 c. https://elib.kz/ru/search/read_book/7105/

- Bulatbaev F. N. Osnovy elektroniki i nanoelektroniki / Bulatbaev F. N., Neshina E. G., Mekhtiev A. D., Al'kina A. D., 2019. - 205 c. https://elib.kz/ru/search/read_book/5311/

Korolkov A.V.

Case on theme « Circuit models for the semiconductor diode »

on discipline « Digital and Microprocessor Technology »

for students of EP  «6В07150- Electroenergetics

Signed to print  «__» ___2022y.

Paper size

Typographical paper. Offset printing. Volume 1 p.sh

Circulation ___ ех. Оrder  №__

© M.Auezov South Kazakhstan University, 2022y

Publishing center SKU by M.Auezov, Shymkent, Tauke khan 5