MINISTRY OF SCIENCE AND higher EDUCATION OF THE REPUBLIC OF KAZAKHSTAN

|  | Non-profit joint stock company «M. Auezov South Kazakhstan university» |
| --- | --- |

- Saparbekova

Principles of waste management in biotechnological industry

textbook

for undergraduate students in the specialty/ educational program

7M05123-Biotechnology

Shymkent 2023

UDC 581.19 (075.8)

A. Saparbekova, Principles of waste management in biotechnological industry /textbook-   Shymkent: M. Auezov South Kazakhstan university, 2023, - 167   pages.

The textbook, intended for undergraduate students in the specialty 7M05123-Biotechnology, includes 10 main chapters. The textbook describes the main methods of waste management, defines industrial waste, and presents the main environmental legislation.    Waste management will not only save natural resources, biodiversity and human lives, but will also positively impact the economy as more jobs will be created to run an efficient waste management system. In developed countries, waste is not thrown into landfills, while in developing countries, waste usually ends up on streets and wastelands. When these untreated wastes are exposed to air, they pose a risk to the environment. Over time, waste management has become more and more important, not only for the protection of the environment, but also as a growing branch of the economy.

Textbook are prepared for undergraduate students in the specialty/ educational program 7M05123-Biotechnology.

Reviewers:

Aimenova Zh.E. PhD, Head of the Phytochemistry laboratory, M. Auezov south Kazakhstam, M.Auezov South Kazakhstan university

Torlanova B.O. - candidate of Pharmaceutical sciences, Associate Professor of the Department "Technology of pharmaceutical manufacture", South Kazakhstav  Medical Аcademy

Korazbekova K.U. senior lrcture of the Department of Biology, South Kazakhstav State Pedagogical University

© M. Auezov South Kazakhstan university, 2023

Introduction

Waste management or waste disposal includes the processes and actions required to manage waste from its inception to its final disposal.  This includes the collection, transport, treatment and disposal of waste, together with monitoring and regulation of the waste management process and waste-related laws, technologies, economic mechanisms.

Waste management practices are not uniform among countries (developed and developing nations); regions (urban and rural areas), and residential and industrial sectors can all take different approaches.

Proper management of waste is important for building sustainable and liveable cities, but it remains a challenge for many developing countries and cities. A report found that effective waste management is relatively expensive, usually comprising 20%–50% of municipal budgets. Operating this essential municipal service requires integrated systems that are efficient, sustainable, and socially supported.

Waste can be solid, liquid, or gases and each type has different methods of disposal and management. Waste management deals with all types of waste, including industrial, biological, household, municipal, organic, biomedical, radioactive wastes. In some cases, waste can pose a threat to human health. Health issues are associated throughout the entire process of waste management. Health issues can also arise indirectly or directly. Directly, through the handling of solid waste, and indirectly through the consumption of water, soil and food. Waste is produced by human activity, for example, the extraction and processing of raw materials. Waste management is intended to reduce adverse effects of waste on human health, the environment, planetary resources and aesthetics.

Waste management in Kazakhstan is an important concern within the country, considering the billions of tons of industrial waste produced yearly, the currently less-than-optimal state of solid waste management, and existing toxins remaining from both pollutants and Kazakhstan's historical position as the USSR's testing grounds for rockets and nuclear weapons. Kazakhstan has very few services for recycling solid waste, and waste management is currently dealt with using regional programs.

The years 2022-2023 were the years of the worst environmental situation in Asia and Africa.

China is one of the largest countries in the world, with the enormous population – almost 1.4 billion people. To provide such a huge number of people with the necessary agricultural products and industrial goods, huge production capacities are required, which also supply more than half of the world with cheap products. In general, they have a terrible impact on the environmental conditions of the country.

India is another highly populated country, the number of inhabitants in which is slightly lower than in China. There 70% of population live in dirty slums and have access only to the basic benefits of civilization (but not always). Cities and areas around them are a huge dump; almost all the rivers in the country are heavily polluted.

Bangladesh has the same problems as India. About 170 million people live on the territory of Bangladesh (while, in Kazakhstan the population is only 18 million). However, unlike India, the country has a very low potential for economic development, because they do not have enough land even to establish the production of agricultural and industrial products in the required amount for the local population.

However, Kazakhstan and Russia are also included in the list of unfavorable countries in terms of the environment. (Source: https://visasam.ru /emigration/vybor/ekologiya-v-stranah-mira.html).

Nowadays, the environmental situation in the Republic of Kazakhstan is characterized by significant violations of the environment which cause negative impacts.

Environmental problems in the national policy of the Republic of Kazakhstan have not yet acquired an independent status, the prevailing view in the public mind is still that the achievement of short-term economic benefits is better than the interests of environmental safety. The sources of high environmental hazards include industrial companies producing emissions of toxic and harmful substances into the environment, agricultural and forestry enterprises, chemicals,  and motor vehicles.

Nowadays, the national environmental problems are: the Aral Sea and Semipalatinsk regions, which are declared zones of ecological disasters as in these regions owing to unfavorable environmental conditions natural ecosystems were destructed, flora and fauna degraded, and population suffered from significant physiological and moral harm.

Nuclear weapons tests conducted over 40 years at the Semipalatinsk nuclear test site, have caused irreparable damage to human health and the environment resulting in an increase in overall morbidity and mortality.

The main sources of air pollution in the Republic of Kazakhstan are the following negative factors:

1. Emissions of transport, i.e. cars – a chemical factory on wheels. "It’s conscience" the sourсe of 60% of all pollutants in urban air. The exhaust gases contain aldehydes, formaldehyde, lead dioxide, and nitrogen. Annually cars release in the air more than 1 million tons of harmful substances, i.e. one vehicle per year emits 200 kg of carbon dioxide, 60 kg of nitrogen oxides and 40 kg of hydrocarbons.

2. Industrial emissions, which include:

- Emissions from the chemical industry;

- Emissions from the production of cement and building materials;

- Plant emissions petroleum and petrochemical industries;

- Emissions of non-ferrous metallurgy.

Huge amounts of industrial emissions into the atmosphere cause acid rains, which lead to soil erosion, disruption of building structures, and appearance of certain skin diseases.

3. Agriculture cattle-breeding and poultry farms.

Pollution is caused by violation of pesticides storage conditions,  by industrial complexes on production of meat, and emission of evil-smelling gases, ammonia and carbon sulfur;

4. Destructions of ozone layer. One of the facts is emergency cases at starts of space rockets into space from the Baikonur spaceport.

They not only cause pollution of atmospheric air and surrounding environment, but also dramatically increase the number of cancer and skin diseases due to ultraviolet rays and radiation. Annually Baikonur spaceport launches 15 space rockets, 5 of which cause huge damage to the environment and health of the public.

5. Global climate change. It is expressed in gradual increase in average annual temperature. Scientists connect it with accumulation of "greenhouse gases" – carbon dioxide, methane, chloro-fluoro carbon, and nitrogen oxide in the atmosphere. The greenhouse effect is caused by global technogenic pollution of atmosphere.

This leads to a complete or partial loss of natural resources, reduction in the productivity of natural landscapes, depletion of soils, rangelands, deterioration of water systems and human environment. This happens on the background of the threat of global processes of desertification (which have a particularly adverse effect in Kazakhstan), the greenhouse effect, acid rains, depletion of the ozone layer and increase in the number of cancers among the population of Kazakhstan.

The current ecological state of the air environment in the Republic of Kazakhstan, despite the huge number of measures aimed at improvement of the state of the air environment, continues to remain negative as well as around the world.

The industry of the republic is focused on production of ferrous and non-ferrous metallurgy products. A half of annual emissions of lead in the former Soviet Union (more than 4 million tons) belonged to the share of Kazakhstan. According to the experts in ecology, emissions of gaseous, liquid and solid wastes in Kazakhstan resulted in appearance of anthropogenin biogeochemical provinces: lead-zinc-arsenic – in the Eastern Kazakhstan region, lead-phosphorus – in the Southern Kazakhstan area, chromic – in the Western Kazakhstan region. Air and soil reservoirs are also polluted; the adverse situation is preserved in Semipalatinsk, Karaganda, and Pavlodar areas.

The most important problems in Kazakhstan regions are:

1. A heavy ecological situation was developed in the East Kazakhstan region where the main part of the enterprises of non-ferrous metallurgy with concentrated harmful emissions, most dangerous to the environment is located. About 45% of lead, approximately 50% of zinc and 90% of rare metals are processed in this region. Of 226 local industrial enterprises having about 7000 sources of emissions of polluting substances, only 63% are equipped with complexes for purification of flue gases, and they catch only solid waste.

2. Central Kazakhstan is polluted with soil harmful substances of mining and metallurgical industry. There are large enterprises of power industry, metallurgy, coal-pits in the Karaganda region. The main pollutant is the join-stock company Ispat-Karmet.

3. Western Kazakhstan region is susceptible to the high level of pollution because of intensive development of oil and gas fields with a high content of toxic components (sulphurous anhydride, carbon monoxide, hydrocarbons, oxides of nitrogen, sulfur, dust). In Aktobe region, plants producing ferroalloys and chromium compounds have a negative impact on the environment.

4. In Southern Kazakhstan the main environmental pollutants are lead-phosphorus, pharmaceutical, cement and oil refineries.

5. The reasons of high level of air pollution are:

1) Outdated production technologies;

2) Inefficient treatment facilities;

3) Poor quality of the applied fuel;

4) Insufficient usage of non-conventional power sources.

The use of biotechnology for solving ecological problems is not a new idea. Mixed bacterial populations have been used for wastewater treatment for over a hundred years. All living organisms (animals, plants, bacteria, and others) in their life cycle engulf and digest nutrients and secrete products of vital activity into the environment.

Various organisms require different nutrients to maintain life. Some bacteria can absorb chemical compounds containing wastes, others feed on toxic chemicals such as methylenechloride, detergents and creosote.

The use of the power of biotechnology helps to transform the agricultural feedstock, i.e., the biomass (plant stems), fats (oils) and proteins into products of advanced technologies. We can use biocatalysts and biological processes for transforming these materials into fuel, chemicals, solvents, monomers and polymers, adhesives and other substances required by new economy.

The following basic methods are used successfully in practice of ecobiotechnology:

• biological purification of sewage;

• biological purification and deodorization of gases;

• restoration of surface layer and properties of soils;

• recycling and utilization of organic wastes.

Today, the bank of ecologically beneficial types of microorganisms exists. Preparations based on them are characterized by a complex action. They have a stimulating effect on plant growth, suppress a number of diseases, improve the mineral nutrition of plants, enhance soil fertility, and significantly reduce the pesticide load. Application of a new generation of biological products not only increases plant productivity but also allows us to get earlier products improving their safety.

Waste management will not only save natural resources, biodiversity, and human life, it will also impact the economy in a positive way as there will be more jobs created for running an efficient waste management system. In developed countries waste is disposed into garbage disposal sites while in developing economies the waste usually ends up on streets and vacant areas.

Chapter 1

# Industrial waste. Environmental Legislation. The Earth is getting Warmer and detrimental effect on human well-being. Deforestation and Farming. The Disposal of Waste

Continuous generation and accumulation of waste, including hazardous waste (Persistent Organic Pollutants (POPs), medical waste, etc.). Almost one third of industrial waste in the country is accumulated in Karaganda region - more than 8.5 billion tons by the end of 2012.

The main volume of emissions in Karaganda region accounts for metallurgical sphere and equals to 70%. This is the share of two major metallurgic companies - Arcelor Mittal Temirtau JSC and Kazakhmys Corporation. In the period from 2008 to 2012 Arcelor Mittal Temirtau has paid the penalty of more than one billion tenge for noncompliance of emission standards, Kazakhmys Corporation - more than 300 million tenge. According to the results of 2012 the expected decline in total emissions in Karaganda equals to 31% compared to 2008. This is mainly the merit of environmental protection program of a single enterprise - "BalkhashTsvetMet", a subsidiary of Kazakhmys Corporation. In 2008 "BalkhashTsvetMet" has commissioned production on the basis of sulfuric acid. This action has reduced the emissions of sulfur dioxide from 578,000 tons in 2008 to 254,000 tons in 2012. In Kazakhstan, as elsewhere in the world, liquid and gaseous types of industrial waste are a major concern for environmental organizations. Industrial discharges and gaseous emissions are priorities for state control and regulation of all enterprises in Kazakhstan. Methods of inspection, as well as ecological and economic standards have been developed in the country for all categories of liquid and gaseous waste products. At the same time, solid waste produced by the mining, concentrating, refining and power industries has accumulated all over the country without controls. This is largely caused by the following factors:

- Lack of research on environmental impact assessment (EIA) of solid waste and health risks;

- Absence of legal and economic leverage to control the entities storing waste in breach of sanitary regulations;

- Lack of a regulatory framework and technical equipment for environmental monitoring at the local level;

- Lack of access to information on waste composition and class of hazard.

The issue of solid waste accumulation is of special concern in Kazakhstan given its economy's reliance on extraction and processing industries, which produce a huge amount of waste. About 21 billion tons of solid waste of all types has accumulated in the country. Its incremental growth amounts to 1 billion tons a year (Annex). Most solid waste is stored in Karaganda (29.4%), Eastern Kazakhstan (25.7%), Kostanai (17.0%) and Pavlodar (14.6%) oblasts.

### 1.1 Environmental Legislation

The Constitution of the Republic of Kazakhstan, which was adopted in 1995, forms the basis of national environmental legislation. Article 31 of the Constitution states that “the state shall set an objective to protect the environment favorable for the life and health of the citizen". Consequently, though the right to an environment favorable for life and health is not stipulated as a constitutional right, the fundamental law affirms the responsibility of the state to provide favorable environmental conditions for its citizens. The Constitution also states that officials shall be held accountable for the concealment of facts and circumstances endangering the life and health of the people. The access of the citizens of Kazakhstan to information on the environmental situation is viewed as a guarantee of the implementation of state policy on environmental protection. In addition to the above-mentioned law, Kazakhstan has adopted a number of specific laws in the area of environmental protection:

- The Law on Protection, Reproduction and Use of Wildlife of October 21, 1993

- The Law on Environmental Expertise of March 18, 1997;

- The Law on Specially Protected Natural Territories of July 15, 1997;

- The Law on Protection of Atmospheric Air of March 11, 2002;

The Criminal Code in force from July 16, 1997 includes a special Article (#11) on environmental crime. This Article covers the following types of environmental offences:

- Violation of environmental statutory regulations of economic and other types of activity;

- Violation of environmental requirements during the process of production and use of potentially hazardous chemical, radioactive and biological substances;

- Violation of safety regulations during the process of handling microbiological and other biological agents or toxins;

- Violation of veterinary rules and regulations of pest and plant disease control;

- Water pollution, clogging and depletion of water resources;

- Air pollution;

- Pollution of the marine environment;

- Violation of legislation on the continental shelf of Kazakhstan and special economic zone of the country;

- Damage of land;

- Violation of regulations on protection and use of subsoil;

- Illegal capture of aquatic species and plants;

- Illegal hunting;

- Violation of regulations of wildlife protection;

- Illegal use and exploitation of rare and endangered species and plants;

- Illegal cutting down of trees and bushes;

- Destruction or damage of forests;

- Violation of the regime of specially protected natural territories;

- Failure to take measures to ameliorate the consequences of environmental pollution.

Waste processing and utilisation

Kazakhstan has practically no waste recycling enterprises. The few currently operating recyclers face a number of financial and organizational constraints. For instance, in Pavlodar there are two enterprises employing unique technologies (up to 96% of waste is used in the production process). They are: Pavlodar Ash-Sludge Waste Processing Plant JSC and EMEKO JSC. The base raw materials used by these enterprises are bauxite slime of Aluminum of Kazakhstan JSC and ash waste of Pavlodar TES-1, resulting from combustion of Ekibastuz coal with high ash content. The plant produces 14 kinds of construction materials, such as brick (including fire-resistant), cement, etc. Provided the plant operates on a two-shift basis, it can annually process 32,000 tons of slime and 38 tons of ash. However, it cannot operate at full capacity due to lack of working capital. Enterprises of the ore dressing and metallurgical complexes, petrochemical, heat-and-power generating facilities, coal pits, etc. use industrial waste for the mining-technical reclamation of tailings, ash dumps, overburden dumps and hosting rocks. Thus, in waste-intensive areas (East Kazakhstan, Karaganda, Kostanai and Pavlodar oblasts) the percentage of industrial waste usage varies, from 1.5-2% in Pavlodar oblast to 25% in Karaganda oblast. A high level of waste utilization has been achieved in this oblast in the last two years mainly due to the use of overburden and hosting rocks for technical reclamation of up to 85% of the used and disturbed land. The metallurgical slime stored on test sites is usually buried. This is largely due to lack of efficient and cost-effective recycling technologies. A small part of the waste is used in production and construction technologies by Ispat-Karmet JSC (Karaganda oblast), where practically all metallurgical slag is processed and reused. Zhairem Tsvet Мет JSC and the Balkhash smelter of Kazakhmys Corporation process all the metallurgical slag accumulated throughout the year, together with ore, in the production of copper concentrate.

The  following technologies of waste processing and neutralization are applied at Sokolovsko-Sarbaisky smelter (Kostanai Oblast): crushing of rocks for construction; use of mill tailings in the extraction of collective sulfide concentrate by a floatation unit and its further processing at the mining-chemical integrated works, although this technology has yet to be finalized. In 2000 Maikainzoloto OJSC (Pavlodar Oblast) started to use off-balance ores from overburdened soil dumps, including dumps located in the Maikain settlement. Maikainzoloto JSC, jointly with Technopark- Stepnogorsk JSC, plans to apply environmentally friendly technology in the production of the copper-zinc concentrate without cyanides.

Ferrochrome JSC (Aktyube Oblast) is an example of successful industrial waste utilization and processing. This JSC has started to produce crushed stone from the scoria of high and low-carbon ferrochrome (more than 150.0 tons per year) and ferrodust (more than 4,000 tons per year) which is further used in the manufacturing of lime-and-sand brick (more than 12,000 pieces per year). In addition, the company plans to use a bin for the reception and sorting out of the waste for further recycling. A dedicated area will be used to store metal scrap.

Private sector participation in solid waste disposal helps to raise significantly the efficiency of recycling. For instance, in Pavlodar the situation improved after solid waste management was transferred to Polygon MDS JSC (accumulator of solid waste of III-IV class of hazard) and Spetsmashiny JSC (the city solid waste land fill). Another private company, which is part of Pavlodar Chemical Plant JSC, is engaged in collection, employing the technology of Tolyatti IFC Chelnok JSC (Russia). The enterprise has so far recycled more than 90,000 lamps. Experimental work has been conducted on ‘demercurization’ in the use of fluorescent lamps of Petropavlovsk by ZIKSTO JSC. In Almaty oblast the mercury-containing waste and fluorescent lamps are delivered for demercurization to the specialized enterprise Synap JSC (Almaty). A facility for the centralized collection, storage and recycling of utilized luminescent, mercury lamps and devices has been opened in Uralsk, western Kazakhstan.

Recycling efforts are being made in West Kazakhstan also. For instance, Stroitekh JSC has begun to recycle broken glass and produce consumer goods from them. A similar project is being developed by Zheksengaly JSC. Uralvtorma Research and Production Association has designed a project on recycling wood-shavings, scrap paper, wool to produce composite building slabs used in the construction industry. In order to resolve the problem of waste management, it is necessary to develop a National Waste Management Program, spelling out waste management policies, a legal framework and methodology for waste disposal, an economic mechanism for waste management and monitoring. Implementation of this program requires: an inventory of existing tailings and dumps and assessment of their technical condition; specification and analysis of flows, volumes and components of accumulated and buried waste; assessment of risks for decision-making.

A waste management system is the strategy an organization uses to dispose, reduce, reuse, and prevent waste. Possible waste disposal methods are recycling, composting, incineration, landfills, bioremediation, waste to energy, and waste minimization.

As for waste management, it is the measures utilized to manage waste in its entire life cycle, from waste generation to disposal or recovery.

Over the past few years, waste management has become a global concern. Commonly, waste is generated in households, factories, construction sites, refineries and nuclear power plants. As the population is increasing, consumption trends are changing. Along with the improvement in lifestyle, it has posed alarming threats to the environment. If waste is not managed properly it can cause serious problems to human health and the environment. So, it is necessary for people and businesses to know the importance of waste management.

Waste management is important as it saves the environment from the toxic effects of inorganic and biodegradable element present in waste. Mismanagement of waste can cause water contamination, soil erosion and air contamination.
    Waste can be recycled if collected and managed efficiently. Waste materials such as plastic, glass and paper can be segregated into different columns in order to process them to make new products which will save the natural resources. Moreover, when this waste is not recycled, it usually ends up in landfills or oceans posing threat to human health and marine life. In most areas of the world, sewage treatment is not done properly, leading to eutrophication and beach closures. According to World Wildlife Fund (WWF) an estimate of 8 million ton plastic ends up in oceans each year.

# 1.2 The Earth is getting Warmer and detrimental effect on human well-being

If  it is getting warmer all over the world, you need information from all over the world. For example, one recent report on this question, by the American Meteorological Society or AMS, used the information of more than 450 scientist from 62 different countries.When scientists ask a question, they look for evidence,

evidence means the facts or signs that help prove something.

In other words, to show that it is true, at least as true as we can know now based on the science that we have now. Some examples of the evidence the scientists study to answer, is the Earth getting warmer, are temperatures, how hot or cold it is in the air, the oceans, and on the land.

They also look at how many clouds are in the sky, how high or low the water in the ocean is. And how much of the earth is covered with snow.

We mentioned the AMS, the AMS is a group of scientists called meteorologist. Meteorologist study the weather and the earth's atmosphere,

atmosphere means the gases or air around a planet. What does the AMS say?

Is the Earth getting warmer? Well, isn't it true that the Earth has always had times where it became hotter, and then different periods of time, when it became colder?

Well, the Earth has switched, between periods of time where it was, either hotter, or colder. This chart shows temperature changes around the world since the late 1800s.

Temperature anomaly 1880-2016

This line 0, here, shows the normal temperature for the time of the year, anything below 0 is cooler for that time of year. Everything above 0 is warmer for that time of year.

As we can see since the 1970s, temperatures around the world have continued to go higher and higher. In fact, there is evidence that shows the earth is warming ten times faster than it has in the past.

If things continue as they are now, scientist believe that the earth will continue warming 20 times faster than it has in the past. So looking at all the evidence, we know the answer to the question, is the Earth getting warmer? 97% of all of the scientists who study climate change believe they have evidence to prove the Earth is getting warmer. Examples like measuring temperatures, clouds, snow, and ocean levels. They also believe there is evidence to prove that people are causing this warming. Mostly by pollution from things like cars, factories, and electricity.

The United Nations weather agency says the past 10 years were likely the hottest decade since scientists began keeping records. In a new report, the World Meteorological Organization, or WMO, blames the rising temperatures mainly on greenhouse gases produced by human activities.

“Average temperatures for the five-year, 2015-to-2019 and 10-year, 2010-to-2019 periods are almost certain to be the highest on record,” the WMO said.

The report predicted that 2019 would become the second- or third-warmest year on record. Final temperature measurements will not be available for several months.

Petteri Taalas is the Secretary-General of the WMO. He says severe heatwaves and floods that used to happen about once every hundred years have become “regular” events.

“Countries ranging from the Bahamas to Japan to Mozambique suffered the effect of devastating tropical cyclones. Wildfires swept through the Arctic and Australia,” he noted.

Taalas said that one of the main effects of climate change is the lack of predictable rainfall. This presents a threat to successful harvests and will likely create food security problems in some countries in the future, he said.

The report said climate change was a main driver of a recent increase in world hunger after 10 years of decrease. It estimated that about 820 million people suffered from hunger in 2018.

The WMO said that weather disasters have displaced millions of people this year and affected rainfall from India to northern Russia, to the central United States and other areas.

Oceans, which take in an estimated 90 percent of the extra heat produced by greenhouse gases, are now at their highest recorded temperatures. Sea water is also 25 percent more acidic than it was 150 years ago, the report said. This is threatening ocean environments that provide food and jobs for billions of people.

Levels of carbon dioxide in the atmosphere reached a record 407.8 parts per million in 2018 and continued to rise in 2019. Experts say carbon dioxide can be damaging because it can remain in the atmosphere for hundreds of years and in the ocean for even longer.

On Monday, at the opening of a climate meeting in Spain, United Nations Secretary-General Antonio Guterres warned that 400 parts per million had once been considered “unthinkable.” The UN Climate Change Conference brings together representatives from around the world to seek solutions to reduce greenhouse gas emissions. Last year, the UN’s Intergovernmental Panel on Climate Change set a goal for countries to limit temperature increases to 1.5 Celsius or below. The group said this target could be reached by reducing greenhouse gases and restructuring the world economy to expand renewable energy sources.

The UN reported last week that the world needed to cut carbon emissions by 7.6 percent each year, every year, until 2030, to reach the 1.5 Celsius temperature goal.

1.3 The Impact on Our Health

Including an increase in the high heat and poor air quality, an increase in disease, and an increase in mental health conditions.

Scientist believe that by the end of the century, climate change will increase the number and strength of heatwaves which are periods of unusually hot weather. These changes could increase the number of heat related deaths by tens of thousands in the United States alone.

Some people are more in danger of heat related deaths than others. Some examples are people who work or play a lot outside. People who do not have air conditioning. Or people for whom it is too harsh on their bodies. Some examples of people who are more affected by heat are very young children, pregnant women, or the elderly.

Climate change is also likely to impact the air that we breathe. One concern is the increased amount of ground level ozone. The Ozone layer is a layer of ozone gases high up in the atmosphere that block a lot of the sun's ultraviolet rays.

The problem is, the warmer temperatures from climate change could increase the number of days we have ground level ozone. Which is an unhealthy part of the air pollution we call smog.

Smog is when fog or haze becomes darker by smoke or other air pollutants.

People who breathe in high levels of smog or ground-level ozone, are at a higher risk of dying or being put in the hospital for breathing problems.

There are also numerous types of diseases that are expected to increase

due to climate change. Diseases that people will get from insect bites, water, or food supplies. One example is the West Nile virus that is transmitted or given to humans by mosquitoes. West Nile virus can be a very serious illness.

Climate change could increase the number of people who get it because the warmer weather will allow the mosquitoes to be active in larger and larger areas for longer and longer times of year.

Climate change could threaten the safety of our water supply. Bacteria are single-celled organisms that live on, in, and around most living and nonliving things.

They are so tiny, millions of them would fit on the head of a pin.

Although many bacterial species are beneficial, some bacteria are harmful.A rise in hurricanes or floods from climate change could cause a city's sewage system where waste material such as human urine and feces is carried away in pipes to mix with clean drinking water or bathing water.

People living in areas of the world where cities and foods are less clean are at a greater risk. All of these challenges and the stress that climate change can cause can also be a threat to the mental health of the people in a community. It is especially stressful if someone loses a loved one in the event.

This stress is dangerous for us all. But it is even more dangerous for people who have mental illness. A mental illness is a condition that affects a person's thinking, feeling, or mood.

People with mental illness are three times the risk of death during heatwaves.

1.4 The Threat to Plants and Animals

For animals these impacts can include changes to habitats, and migration patterns. While for plants the impacts can be the loss of insulation from cold weather, and an increase in wildfires. First let's look at how climate change impacts animals' habitats and migration patterns.

A habitat is the place where a plant or animal lives or grows. The four main elements of an animal's habitat are the right temperatures, fresh water, food sources, and places to raise their young.

Animals depend on these elements to survive. If any of these elements change it can cause problems for the animals. They either have to find a way to adapt, which is to change, so that it is easier to live in a particular place or situation, or

if they cannot adapt, they have to find a new place to live.

One example of climate change impacting the habitat of animals would be the struggles polar bears now face because of Arctic Sea ice melting. Polar bears depend on the ice for hunting, because they are not as good at swimming as many of the animals they hunt especially the main source of food, seals.

So with the ice melting, they have much shorter hunting seasons and unable to catch as much food as they need. In the long run, this has had an impact on the polar bear's ability to raise their young.

Without enough food, polar bear reproduction which means having babies is down 20%. Ducks and geese are also impacted by climate change. You see, ducks and geese have migration patterns. A migration pattern is when animals move together from one place to another because of the seasons.

For example, in winter they move to places they know that will have warmer temperatures and hopefully, better food supplies. Because global warming is changing the areas that they live, they are being forced to change their migration patterns.

This not only causes problems for the duck and geese but also for others as well. For them, if they change their migration pattern, they may find that the plants that were in bloom when they used to go there before are not growing at this time. They may also find that there are different animals around at this new time than before. Some of those animals may compete with them for the same food or maybe eat them. For others, animals or people that were used to their old migration patterns, they might be disappointed to see that they are no longer there, especially if they used to hunt them for food.

Climate change also impacts trees and plants. For example, climate change is reducing the amount of snow in large parts of the world.

The snow is important for the plants and trees because it insulates the soil below it.Insulate means to add a material to something to stop heat, electricity, or sound from going in or out. So in other words, the thick layer of snow keeps

the freezing air out from the warm soil underground. If there is not enough snow to cover the ground well, the soil below the trees and plants freezes more.

This damages the roots and makes the soil lose some of it's nutrients the plants need to live. Climate change has also created extremely dangerous conditions for an increase in wildfires. A wildfire is a large destructive fire that spreads quickly in the wilderness. In areas where the temperature is rising and there is a drought, the trees and plants dry out due to the lack of water. Wildfires can make an area unlivable because they destroy all the plants and all the homes of the animals and humans who live in and around them.

Climate change has forced changes to animals' habitats and migration patterns. It has also impacted plants by decreasing insulation from the cold while increasing the risk of wildfires.

As the rising temperatures limit the amount of fresh water and food sources and places to raise their young. All living things must find ways to adapt or move to new, healthier places in order to live.

The Stress on Our Marine Ecosystem

An ecosystem is a community of living things and its specific environment. So, what is the marine ecosystem? The marine ecosystem is part of the Earth's largest water system covering over 70% of the Earth's surface. It includes the oceans, salt marshes, coral reefs and more.

There are approximately 4,000 known plants that live in the marine ecosystem, including phytoplankton, seaweed also known as algae, sea grasses and mangroves.

In addition to plant life, there are also many animals that call the marine ecosystem home. These animals include zooplankton, coral, sea urchins, parrot fish, squid, sardines, sharks, seals and other mammals. So, how does climate change affect this very large ecosystem? As you've learned, temperatures are rising due to human activity. This increase in temperatures also affects our oceans.

The worlds oceans have been getting warmer over the past 50 years and

are continuing to heat up. This directly affects the growth and production of many things. One of them being phytoplankton, which is decreasing in numbers. It may seem like decreasing phytoplankton wouldn't be a problem since it's such a small plant, but don't be fooled by its size.

Phytoplankton plays a huge role in the marine food chain. A food chain is the feeding order of species in an ecosystem.

Phytoplankton is the basis or starting point of the marine food chain as you can see in this picture.

Food chain

If phytoplankton begins to disappear, there will be less food for

zooplankton, creating less food for small fish, etc., etc.This chain reaction could damage the entire marine food chain and marine life as we know it.

In addition to its importance in the marine food chain, phytoplankton also helps to remove carbon dioxide from the atmosphere. In fact, through the process of photosynthesis, phytoplankton removes half of all carbon dioxide from the atmosphere. As we learned earlier, photosynthesis is extremely important. Because during that process, plants release oxygen and store carbon.

Phytoplankton does just this. It releases oxygen in the air for us to breath and

drops carbon into the ocean that collects on the ocean's bottom. With less phytoplankton, there will be even carbon dioxide in our atmosphere.

Lastly, warmer temperatures have had a huge impact on coral reefs. Coral reefs are underwater ecosystems built by corals that are homes to thousands of tiny marine species. One-third of ocean life depends on coral reefs to live. Unfortunately, corals need specific temperatures to survive and the warmer temperatures caused by climate change have been causing what scientists call coral bleaching. Coral bleaching means that corals are turning white, starving and dying, because of higher temperatures. This in turn affects all the other animals that depends on coral for their ecosystem.

1.5 Deforestation and Farming

Deforestation is the destruction or clearing of forests to make land available for other uses.

Deforestation happens for many reasons. Sometimes deforestation happens so people can use the wood from trees to create fuel, paper, and other commercial goods.

Most often though, deforestation happens so people can use the land for things like farming and housing. There are things in our environment called the carbon sinks.

Carbon sinks absorb greenhouse gasses from the atmosphere and store them.

Forests are a primary carbon sink in our environment. Trees take carbon dioxide from the air and turn it into oxygen during photosynthesis.

Photosynthesis is a chemical process by which plants make food using sunlight to turn carbon dioxide, water, and certain salts into sugar or carbohydrates.

So basically, trees absorb the carbon dioxide in the air to make food, and release oxygen for us to breathe. By destroying the forests, we are destroying trees and these very important carbon sinks.

Furthermore, when we cut down trees, they release the carbon that is stored inside them. Each year, deforestation is the cause of about one billion tons of carbon dioxide being released into the atmosphere.

Overall, deforestation creates about 15% of our greenhouse gases in our atmosphere. In addition to this, deforestation limits the amount of water vapor in the atmosphere. A fully grown tree releases about 1,000 liters of water vapor a day.

When we cut down forests, we are losing this very important source of water vapor that creates clouds and rain, leading to dry weather and other severe climate effects.

As mentioned earlier in the video, the land clear from deforestation, is often used for farming or agriculture. After all, we need food to survive.  Most farming today uses chemicals and fossil fuels that add even more carbon dioxide, methane, and nitrous oxide to our atmosphere.

In fact, according to the inter-governmental panel in climate change, agriculture land use creates 12% of greenhouse gases globally. So not only does the process of deforestation cause greenhouse emissions to be released. But the use of the land for farming afterwards, also adds greenhouse gases to our atmosphere.

Across Africa, men carrying tools for cutting down trees spend weeks deep inside forests. They cut down the trees to burn them in order to make fuel for fires known as charcoal. Because they work at night, mainly on public land, they operate without fear of the law while destroying forests in many countries.

Fires in Brazil Amazon rainforest   have brought attention to the difficulties of protecting the Earth’s forest lands. The Congo Basin tropical rainforest is about the size of Western Europe. After the Amazon, it is the world’s second largest rainforest. Some people call it Earth’s second lung. Africa is home to over 1.2 billion people and poverty on the continent remains a problem. Many nations have struggled to protect forests as growing populations demand plant-based energy resources that are considered low cost. Charcoal is widely used.

The European Space Agency says that 25 to 35 percent of climate-changing greenhouse gases come from biomass burning. Biomass burning is natural or manmade burning of organic material, including seasonal fires people set to clear land for farming. Most of this kind of burning happens in tropical areas of Africa.

A 2018 report from the United Nations Food and Agriculture Organization found dependence on charcoal or firewood to be highest in Africa and Asia. Some African cities are almost completely dependent on charcoal for cooking. In Kinshasa, the capital of The Democratic Republic of the Congo, 90 percent of residents depend on it, the report said. The country is already suffering a wave of extremist violence.

The U.N. report said that the estimated value of the charcoal export trade from East Africa to the Middle East and other places is over $360 million a year. This is the case although the trade is banned. About 8.2 million trees were cut down for charcoal between 2011 and 2017, the U.N. said.

Ugandan government officials have warned about the charcoal trade for a long time. The trade continues although the electrical power system now extends deep into the country. Hydroelectric power is still too costly for many people even in the capital, Kampala.

Edwin Muhumuza is an environmental activist who runs the group Youth Go Green in Kampala. He told the Associated Press that charcoal has become a valuable product, like gold or coffee.

Now the National Environment Management Authority, a government agency, is urging officials to lift taxes on liquid petroleum gas. It is another cooking fuel that could be used instead of charcoal.

Africa’s forests are shrinking, researchers warnResearch shows a difficult situation. Uganda’s forest cover as a percentage of land area was nine percent in 2015. Government data shows that is down from 24 percent in 1990.

But officials in northern parts of Uganda such as Gulu, which provides much of the charcoal entering Kampala, are taking action. Their campaign has seized many charcoal trucks since 2015. Gulu chairman Martin Mapenduzi organizes raids in hopes of arresting charcoal burners.

“Illegal logging has gone down but the destruction of forests for charcoal burning is still high,” Mapenduzi said. “It’s something that is giving us a lot of headache, but we are fighting.”

The price of enough charcoal to support a small family for several weeks has been rising in Kampala. In August, it reached about $28. That is largely because of reduced supply from places such as Gulu.

The cost is still far too much for families, said Rose Twine. She is a business woman who sells a cooking device called the eco-stove. Twine says one eco-stove comes with volcanic rocks that, she said, can last for up to two years. It costs $110. Yearly charcoal costs for an average family, can grow to more than $300.

“Charcoal is actually not cheaper,” Twine said. “People are just stuck in their old ways.”

Widespread destruction of forests has led to campaigns to take action in some African countries. This week, Gabon became the first African country   to receive payments for provable efforts to reduce greenhouse gases resulting from the destruction of forests. The U.N. Development Program said payments would reach $150 million over 10 years. It called the payments “historic in many ways.”In July, The U.N. Environment Program said that Ethiopia’s prime minister led an effort in which over 350 million trees were planted across the country in one day.

But some activists say tree-planting may not be enough to save Africa’s forests. They urge governments to spend more on alternative energy sources for the poorest people.Mapenduzi, the Ugandan official campaigning against charcoal burning, called for laws against it and urged officials to make electricity cheaper.

Words in This Story

tropical – adj. of, relating to, occurring in, or used in the part of the world that is near the equator where the weather is very warm

continent – n. one of the great divisions of land, such as North America, South America, Europe, Asia, Africa, Australia, or Antarctica, of the Earth

greenhouse gas(es) – n. pollution that causes the Earth’s atmosphere to warm

desertification – n. the process by which an area becomes a desert

hydroelectric – adj. of or relating to the production of electricity by using machines that are powered by moving water

replace – v. to put someone or something new in the place or position of someone or something

log(ging) – v. to cut down trees in an area for wood

headache – n. a difficult or annoying situation or problem

cheaper – comparative adj. costing less money than something else

alternative – adj. offering or expressing a choice

1.6 The Disposal of Waste

Disposal of Waste means removing, throwing away something that we do not want. Examples of this might be the box and the other packaging that your mobile phone came in, the can that is left after you drink your soda, or the peel of a banana after you eat it. Think about what you might do with the waste. We're going to look at four ways to dispose of waste: landfill, incineration, composting, and recycling. Let's look at a landfill first.

The definition of a landfill is a place to dispose of waste material by burying it and covering the waste with soil. This method does have the benefit of creating more usable land which can be used for other purposes. However, there are many disadvantages to landfills.  Dangerous chemicals can contaminate both the soil and the groundwater in areas around a landfill site. Also, landfills produce large amounts of methane gas, CH4 which contributes to global warming.

A second method of waste disposal is incineration. This is a process that involves the burning of waste materials. People all over the world have been burning waste for centuries. Its disadvantages are that it releases harmful pollutants into the air, including greenhouse gases which are breathed in by people or can cause problems for water, plants, wildlife, and the climate.  Recently, some countries have created very large incineration plants. These places can keep in the harmful pollutants caused by burning waste. Also, the heat created by burning the waste is turned into steam which then produces power.  Another method of waste disposal that has been going on for a long time is composting. Composting is the process of decomposing, breaking down into separate parts, organic matter like plants to provide more nutrients for the soil.

In other words, over a period of time, the waste is broken down into separate parts.

These parts then help the soil become richer which means that plants can grow more easily. More composting also means less waste is sent to landfill, which means that less methane is in the atmosphere. The last method of waste disposal, we're going to look at today, is recycling which as you probably know means to change waste into something useful. The best example might be the soda can, we talked about at the beginning of the video. When you recycle this, it goes into a plant where it is cleaned and melted. It is then rolled out and made into another aluminum product. While workers in the recycling industry run a small risk of exposure to harmful materials, the benefits of recycling strongly outweigh the disadvantages. It reduces the amount of waste sent to landfills, reduces greenhouse gas emissions, and stops the pollution created by finding and processing new raw materials.

Questions to test knowledge

1.Nowadays, human activities are changing the natural greenhouse effect.

a general scientific fact

something new happening now

2.Water vapor increases as the Earth’s temperature warms.

a general scientific fact

something new happening now

3.Sources in the “Carbon Cycle” release carbon dioxide into the atmosphere.

a general scientific fact

something new happening now

4.Nature plays a role in the levels of greenhouse gases in the atmosphere.

a general scientific fact

something new happening now

5.As climate change continues, ice wedges are melting at a faster rate in the Arctic.

a general scientific fact

something new happening now

6.Now, mayors in large cities are trying to control air pollution.

a general scientific fact

something new happening now

7.Deforestation increases the threat of climate change.

a general scientific fact

something new happening now

8.As temperatures rise, climate change is forcing some animals to change their migration.

a general scientific fact

something new happening now

Chapter 2

The main properties of wastes and waste classification

2.1 Definition and classification of wastes

Wastes are substances (or mixtures of substances) which are unsuitable for further use within the existing technologies.

From the point of view of natural sciences, any substance can be theoretically used in one way or another. The natural limitation for use is economic inexpediency. The main sources of wastes are industry, agriculture and households. The volume of industrial wastes is 20 times more than that of household wastes. Figure 1 shows the Classification of industrial wastes.

Industrial waste is a waste produced as a result of chemical and thermal transformations of materials of natural origin.

Industrial waste

Liquid

Gaseous

Solid

Anhydrous solutions and suspensions

Aqueous solutions and suspensions

Burning residues

Smoke

Gas

Dusts

Figure 1. Classification of the industrial waste

Wastes of some products are the unused residues of raw materials and/or substances and energy produced in some technological processes that are not subject to utilization. Part of wastes that can be used in the same production is called returnable wastes. It includes the residues of raw materials and other material resources formed in the process of production of goods. Due to the partial loss of some consumer properties, returnable wastes can be used with reduced requirements for the product, or with increased consumption; sometimes they are not used for their initial purpose, but only in a suitable production (for example, automotive waste oils — for lubrication of non-essential units of equipment).

At the same time, the residues of raw materials and other material values that are transferred to other units as full-fledged raw materials, in accordance with the technological process, associated products obtained as a result of the technological process do not belong to returnable wastes.

Wastes, which in the framework of this production cannot be used, but can be used in other productions, are referred to as secondary raw materials.

Previously, industrial wastes were simply taken to landfills, sent to dumps and tailings. Currently, wastes are mainly stored on specially designed and equipped disposal sites.

Part of the wastes is temporarily accumulated at enterprises, in accordance with the established limits on formation and accumulation of wastes.

Waste Neutralization

Some wastes require neutralization before disposal in landfills, sites or dumps. For example, wastes of titanium production containing volatile and toxic anhydrous aluminum chloride are pre-treated with lime.

In the 20-th century, the amount of waste production increased so quickly that the formation of waste has become an important problem of large cities and large industries.

The danger of wastes is  determined by their physical and chemical properties, as well as the conditions of their storage, or location in the environment.

For wastes, it is necessary to make a waste passport, determine the hazard class and limits on the placement of waste in the environment, limits on the accumulation at the enterprise and other documents.

The term "Hazardous waste" is used in the following cases:

* Waste contains harmful substances, including infectious, toxic, explosive and fire hazardous agents, with high reactivity, for example, corrosive, radioactive;

* Waste poses a risk to human health and/or to the normal state of the natural environment.

Residual waste including wood waste, waste paper, textile residues, rubber residues, defective oils, solvents can be burned. These wastes are placed in a ceramic tube to make them harmless and release heat in the production cycle or for disposal of incombustible waste. As a result of the combustion process, waste is disposed for burial.

Waste depends on its qualitative and quantitative composition. The chemical composition of the waste is uneven, because it contains a mixture of complex polycarbonate substances with different physical and chemical properties. Waste containing compounds such as pesticides, radioactive waste, mercury, arsenic and theircompounds, etc., is classified as hazardous waste.

The main properties of wastes that are environmentally hazardous

Dust formation

volatility

instability

solubility

Figure 2. Main properties of environmentally hazardous wastes

Radioactive wastes are produced by various sources of radiation. Radioactive wastes are grouped according to their activities and aggregate status (Figure 3).

Radioactive waste

gas

liquid

solid

Low active

Medium active

Highly active

Figure 3. Classification of radioactive waste

Emissions of toxic substances very harmful to human health, as well as to the environment are called toxic waste. It also describes the toxicity of a substance that destroys the life style during introduction or use.

Industrial wastes are divided into 4 classes depending on their toxicity:

Class I toxicity is extremely dangerous. It includes radioactive substances, benz(a) pyrene, dimethylthiophosphate, lead metal, mercury and their inorganic compounds;

Class II toxicity has a high level of hazard. It  includes: methylmercaptan, nitrogen oxides, nickel, manganese, hydrogen sulfide, formaldehyde, hydrogen fluoride.

Class III toxicity is a moderate hazard. It includes carbohydrates, sulfur carbon, methyl alcohol, and tobacco;

Class IV toxicity is a minor hazard. Ammonia, ammonia-carbamide fertilizers, bauxites, iron oxides and limestone;

Some of these unfavorable properties complicating the use of solid waste are shown in Figure 4.

Corrosive effect

mechanicalconnection

Compaction,

solidification

Diverse

types

Waste properties complicating technological processes.

Not spilled

Conglutination, agglutination

Figure 4. Adverse properties of solid waste for treatment

There are 4 methods of waste decontamination and utilizations: burning, chemical or biological neutralization, and burial.

Designs of special plants for waste incineration with different thermal properties have been developed. It is advisable to burn waste at 1000-1200 ℃, as this will reduce the amount of pollutants emitted into the atmosphere. The thermal use of waste requires compliance with certain technological standards.

Together with smoke and ash, harmful substances can enter the environment. Simple combustion of waste causes temperature changes in the atmosphere: from the combustion temperature of the lower layer to the environment temperature. In the process of high-temperature combustion, drying, pyrolysis of organic substances (thermal decomposition) and combustion of the released substances occur, when the combustion proceeds simultaneously at all levels of the layer, part of the flue gases passes into another layer without falling into the combustion layer. Some part of the formed gases does not penetrate from high-temperature layers, which ensures complete disinfection.

An alternative method of simple heat treatment of waste is the development of technologies that first decompose organic waste compounds in anaerobic conditions (pyrolysis), then direct the resulting vapor-gas compounds (VGC) into the combustion chamber, converting toxic substances into safe substances.

This technology is used in many waste disposal facilities, which are currently developed, including compact Eucoco units.

They utilize waste in places of their formation in eco-friendly way. This equipment processes medical (class A,B,C) and veterinary, household and food enterprises, industrial and technical wastes. Processing waste in this way saves the money needed for transportation to landfills. In addition, the installation of ECU-TO-150.03 can provide heat supply to production facilities when removing 50 kg of waste up to 30,000 Kcal/hour.

Disposal of non-combustible solid and pasty wastes of 2nd and 3rd toxicity class made by filling in clay or concrete pits with a high density up to 1m. This process leads to formation of special landfills. Landfills are special environmental facilities aimed at neutralization and disposal of industrial waste that cannot be utilizated.

Lands used as disposal sites are provided for 20-25 years for insulated containers which are well ventilated, not formed under water during heavy rain, snowmelt and floods. As a rule, non-toxic incombustible solid waste is stored in open place or simply buried. 1-3 class hazardous wastes are pre-dehydrated and converted into a paste form. Very toxic wastes, such as arsenic and cadmium were mixed with cement and buried in waterproof landfills.

One of the ways of disposal of solid and highly hazardous liquid waste is biotechnological method.

Different types of microbiological organisms can not only absorb some organic substances, but also turn them into harmless or useful products. The application of biological method is significantly limited due to its long-term need. However, in biotechnology there are few advances in obtaining new types of microorganisms. For example, in 1990 American company JCJ received a thermoplastic "biofield", which for the first time in the world biologically decomposed waste by bacterial fermentation. In addition, many microorganisms are able to recycle wastes.

2.2. Еcotoxicants and chemical waste

The development of production is associated with a large demand and use of chemicals. The use of pesticides, fertilizers, chemicals and other substances determined byagriculture development. The danger of chemicals to the environment depends on human actions.

Ten years of agroproduction dumped chemical waste into the environment: it means that the incoming gases had to evaporate to the atmosphere, and the liquid substances should gradually dissolve in water and disappear, pesticides and fertilizers self-destruct in the ground. Despite the fact that solid urban waste is collected, the safety of the generated waste is not considered safe. The use of pesticides and fertilizers is economically advantageous, since its costs were small compared to the release of toxic substances into nature.

Rachel Carlson’s book “Silent Spring” (1962) describes how birds and fish suffer from the influence pesticides. According to the results of Carlson, the influence of pollutants on wildlife threatens their lives. This book has attracted the attention of the whole society. After that, the environmental protection organization issued laws restricting the release of xenobiotics, a new direction of science – ecotoxicology appeared.

In 1969, Rene Trout singled out ecotoxicology as the first science that combines two subjects: ecology (according to Krebs - it is the science of interconnection that determines the life activity and distribution of living things) and toxicology.

In 1978 Butler supposed that ecotoxicology is the science that studies the toxic effects of chemical agents on a living organism, especially on living organisms at the population level. In 1989 Levin and others determined that the area of the science implies the influence of chemicals on the ecosystem. In 1994, R. Forbes issued the following definitions of ecotoxicology: field of knowledge about environmental and toxic effects of chemical pollutants: the population affects on the ecosystem, and control of pollutants in the environment (transformation and elimination).

Thus, ecotoxicology studies the effects of pollutants on different types of living organisms (from microorganisms to humans), as well as the transformation and decomposition of chemicals in the biogeocenosis system.

Later, one of the directions of ecotoxicology was identified -toxicology of the environment. According to Walker and others (1996), ecotoxicology studies the effects of chemicals on the ecosystem.

2.3. Xenobiotic types of environment

Most of the biologically light compounds are removed from the body with the participation in the process of material and energy exchange in the environment. Others get into the body of animals and plants, and do not use them as a source of energy or plastic material, but if they affect in certain quantities and concentrations, they are able to change the physiological processes. Such compounds are referred to xenobiotic.

The main elements of xenobiotics are foreign chemicals insoluble in water fixed in solids and not evaporating in the air (for example, glass, plastics and so on), they can be considered as a source of formation of xenobiotic types.

Rapidly developing society has led to the accumulation of huge amounts of waste, especially in the countries with highly developed industry. Further development of technologies and industry leads to creation of new xenobiotics, not previously encountered in nature. The accumulation of chemicals in large quantities leads to changes in the xenobiotic form, so they can be considered as an ecopollutants (Table.1)

Table1. List of the main ecopollutants

| Air pollutants | Water and soil pollutants |
| --- | --- |
| Gas: sulfur oxide nitric oxide ozone chlorine hydrocarbons freons dust of asbestos carbon silicon metals | Metals(lead, cadmium, arsenic) organochlorine pesticides(aldrin, dieldrin, chlorine) nitrates, phosphates, oil and petroleum sources of organochlorine compounds (toluene, benzene, tetracloroethylene) low molecular weight halogenated hydrocarbons (chloroform, bromodichloromethane, bromoform, carbon tetrachloride,dichloroethane) polycyclic aromatic aromatic carbohydrates (PAC) polychlorinated biphenyls dioxins, dibenzofurans |

The accumulation of foreign substances in the environment (water, soil, air and living organisms) in the aggregate state, allows them to enter into chemical, physical and chemical bonds with biological objects of ecosystems, which creates xenobiotic form of biocenosis. Xenobiotic types can change due to the influence of certain environmental factors (temperature, humidity, light, trophic conditions, etc.).

In small quantities, they do not harm wildlife and society. An ecopolutant accumulated in the environment in a certain volume, if this volume is sufficient to start a toxic process in the biocenosis, can be considered an ecotoxicant.

2.4 Ecotoxicity (Ecotoxikinetics)

Ecotoxicity is a section of ecotoxicology, exploring the process of transformation of xenobiotics in biological system, i.e. the issues which are considered by ecotoxicity:

* sources of xenobiotics,

* distribution of xenobiotics in abiotic and biotic components of the environment;

* conversion of xenobiotics in the environment;

* elimination (removal) of xenobiotics from the environment.

The main pollution sources:

* Natural sources are wind-borne dust particles, sea salt aerosol, volcanic activity, forest fires, biogenic particles, biogenic volatile substances.

* Anthropogenic sources are chemical, petrochemical, metallurgical, pulp and paper and other types of production; automobile, aviation, sea and river transport; production and use of radioactive substances for various purposes, radioactive waste; the use of plant protection chemicals in agriculture and forestry (fungicides, insecticides, herbicides); the use of chemistry in everyday life and as medicines.

All biological systems are aimed at elimination (removal) of xenobiotics through abiotic (occurring without participation of living organisms) and biotic (occurring with participation of living organisms) processes.

Substances that are not subjected to the processes of destruction have long-lasting persistence in the environment and, as a rule, are potentially dangerous. These substances include:

1.	Heavy metals (especially, arsenic and mercury compounds),

2.	Some pesticides (Dildrin – 12 years, DDT – 10 years, Atrazine – 25 months).

Among the chemicals rapidly decomposing in nature are organophosphorus compounds (duration of decay in the soil is not more than 3 months).

Xenobiotics undergo transformation in the environment. This can be an abiotic transformation, where a toxicant interacts with non-biological environmental factors, such as light (photolysis), water (hydrolysis), oxygen (oxidation) and faster biotic transformation, when the destruction of xenobiotics occurs due to participation of living organisms.

The biotic transformation is based on hydrolysis, dehalogenation, splitting of cyclic structures of molecules, removal of alkyl radicals (dealkylation).

The main problem is the fact that degradation of toxicants rarely goes to the final stage (mineralization) when such substances as water, carbon oxide (II), etc. are formed. In most cases more or less toxic intermediate products are formed. Often, the removal of the toxicant from the biological system is helped by the processes not associated with transformation. It is evaporation, wind movement, sorption and redistribution.

Dangerous phenomenon is the process of bioaccumulation, in which living organisms accumulate xenobiotics, extracting them from the abiotic phase (water, soil, air) and from food (trophic transfer). The accumulation of toxicant will lead to a critical point of its concentration in the organs and tissues of a living organism.

This phenomenon is most characteristic of the aquatic environment. The concentration of toxicants in the tissue of some hydrobionts is higher than its concentration in water. For this process, the indicator was calculated as a bioaccumulation factor, i.e. as the ratio of the concentration of the pollutant in the tissues of hydrobionts to its concentration in water.

Bioaccumulation is affected by the following factors:

- Resistance of xenobiotics in the environment. If a xenobiotic is long-lasting in the environment, the degree of its accumulation by living organisms is high. Rapidly excreted pollutants are accumulated poorly except the areas where the introduction of pollutants is a constant process.

- Chemical properties of xenobiotics. Fat-soluble (lipophilic) substances have the best ability of accumulation. Adipose tissue is the main place of long-term deposition of xenobiotics.

The phenomenon of bioaccumulation underlies not only chronic but also sudden acute toxic effects.

Accumulated by one or another organism xenobiotics can move along the food chain from the victim to the predator. This phenomenon is called biomagnification.

Natural environment can be considered as a catalytic system containing a complex of biotic (enzymes, viruses, bacteria, fungi, algae, fauna) and abiotic components (H2 and OH ions, radical particles and peroxides, solar radiation, clay minerals, oxides, hydroxides, non-crystalline components, organic substances). Several of these components can be simultaneously involved in the same reactions, so that the transformation of compounds is a complex process.

Questions for self-control

- What is bioremediation? Define the methods of waste management.

- Describe in-situ and ex-situ methods.

- Which kind of materials can raise biodegradation of soil.

- What is a polluting material? Describe Xenobiotics.

- Compare bioaugmentation and biostimulation

- Give definitions to Еcotoxicants, Ecotoxicity (Ecotoxikinetics).

- Characterize the Biosorption process.

- Describe pollutants which can be toxic for living organisms and people in small concentrations.

- Name and describe the most harmful metals.

- Estimate and describe biosorbents, biosorptions and types of sorptions.

Chapter 3

Industrial waste. Classification and treatment. Toxic waste. nuclear and radioactive waste.

3.1 Industrial waste. Causes of Industrial Pollution

Industrial waste is the waste produced by industrial activity which includes any material that is rendered useless during a manufacturing process such as that of factories, industries, mills, and mining operations.

Types of industrial waste include dirt and gravel, masonry and concrete, scrap metal, oil, solvents, chemicals, scrap lumber, even vegetable matter from restaurants. Industrial waste may be solid, liquid or gaseous. It may be hazardous or non-hazardous waste.

Hazardous waste may be toxic, ignitable, corrosive, reactive, or radioactive. Industrial waste may pollute the air, the soil, or nearby water sources, eventually ending up in the sea.

Industrial waste is often mixed into municipal waste, making accurate assessments difficult. Most countries have enacted legislation to deal with the problem of industrial waste, but strictness and compliance regimes vary. Enforcement is always an issue.

With the coming of the Industrial Revolution, humans were able to advance further into the 21st century. Technology developed rapidly, science became advanced and the manufacturing age came into view. With all of these came one more effect, industrial pollution. Earlier, industries were small factories that produced smoke as the main pollutant.

However, since the number of factories were limited and worked only a certain number of hours a day, the levels of pollution did not grow significantly. But when these factories became full-scale industries and manufacturing units, the issue of industrial pollution started to take on more importance

Toxic waste/Hazardous waste, chemical waste, industrial solid waste and municipal solid waste are designations of industrial wastes.

Sewage treatment plants can treat some industrial wastes, i.e. those consisting of conventional pollutants such as biochemical oxygen demand (BOD).

Industrial wastes containing toxic pollutants or high concentrations of other pollutants (such as ammonia) require specialized treatment systems.

Industrial wastes can be classified on the basis of their characteristics:

- Waste in solid form, but some pollutants within are in liquid or fluid form, e.g. crockery industry or washing of minerals or coal e liquid form, e.g. dairy industry

Causes of Industrial Pollution

1. Lack of Policies to Control Pollution:

2. Unplanned Industrial Growth

3. Use of Outdated Technologies

4. Presence of a Large Number of Small Scale Industries

5. Inefficient Waste Disposal

6. Leaching of Resources From Our Natural World

Factories and power plants are typically located near bodies of water due to the need for large amounts of water as an input to the manufacturing process, or for equipment cooling.

Many areas that are becoming industrialized do not yet have the resources or technology to dispose off the waste with lesser effects on the environment. Both untreated and partially treated wastewater are commonly fed back into a near lying body of water.

Metals, chemicals and sewage released into bodies of water directly affect marine ecosystems and the health of those who depend on the waters as food or drinking water sources.

Toxins from the wastewater can kill off marine life or cause varying degrees of illness to those who consume these marine animals, depending on the contaminant. Metals and chemicals released into bodies of water affect the marine ecosystems. Effective manners in properly removing waste .

Wastewater containing nitrates and phosphates often causes eutrophication which can kill off existing life in the water. A Thailand study focusing on water pollution origins found that the highest concentrations

Air Pollution

Another obvious effect of industrial waste is air pollution resulting from fossil fuel burning. This affects the lives of many people because this spreads illnesses.

Over time, this issue that has been widespread. Several environmental issues have a devastating effect on third world countries because they don't have sufficient resources to solve this particular issue. This also effects the quality of soil because farmers have to try and deal with this massive issue.

Water pollution

One of the most devastating effects of industrial waste is water pollution. For most industrial processes, heavy amount of water is used which comes in contact with harmful chemicals.

These chemicals are usually metals or radioactive material. This heavily effects the environment because most of waste ends up in oceans, lakes, or rivers. As a result, water becomes polluted posing as health hazard to everyone.

Farmers rely on this water but if the water is polluted, then crops that are produced can become polluted. These effect the health of society because if industrial companies can't clean up their waste, this begins to affect the life of humans but also animals.

Soil pollution

Soil pollution is creating problems in agriculture and destroying local vegetation.

It also causes chronic health issues to the people which come in contact with such soil.

Global Warming

With the rise in industrial pollution, global warming has been increasing at a steady pace. Smoke and greenhouse gases are being released by industries into the air which causes an increase in global warming. Melting of glaciers, extinction of polar bears, floods, tsunamis, hurricanes are few of the effects of global warming.

The issue of industrial pollution concerns every nation on the planet. As a result, many steps have been taken to seek permanent solutions to the problem. Better technology is being developed for the disposal of waste and recycling as much polluted water in the industries as possible.

3.2 Toxic waste

Toxic waste is any unwanted material in all forms that can cause harm (e.g. by being inhaled, swallowed, or absorbed through the skin). Mostly generated by industry, consumer products like televisions, computers and phones contain toxic chemicals that can pollute the air and contaminate soil and water. Disposing of such waste is a major public health issue.

The above definition of toxic waste is the one most people would use. There are many wastes that are harmful to human health or the environment, but they are not toxic.  The Environmental Protection Agency (EPA) classifies hazardous wastes as having one or more of four characteristics:

Figure 5. Four characteristics of hazardous

- Corrosive - A corrosive material can wear away (corrode) or destroy a substance. For example, most acids are corrosives that can eat through metal, burn skin on contact, and give off vapors that burn the eyes.

- Ignitable - An ignitable material can burst into flames easily. It poses a fire hazard; can irritate the skin, eyes, and lungs; and may give off harmful vapors. Gasoline, paint, and furniture polish are ignitable.

- Reactive - A reactive material can explode or create poisonous gas when combined with other chemicals. For example, chlorine bleach and ammonia are reactive and create a poisonous gas when they come into contact with each other.

- Toxic - Toxic materials or substances can poison people and other life. Toxic substances can cause illness and even death if swallowed or absorbed through the skin. Pesticides, weed killers, and many household cleaners are toxic.

To simplify the identification of hazardous (toxic) waste the EPA defines three types of hazardous waste:

Universal Waste (batteries, pesticides, mercury containing items)

Characteristic Waste (has the properties characteristic of hazardous waste)

Listed Waste (waste streams classified as hazardous based on their source)

The by-products of many of the products we use every day can result in the production of toxic waste. For example, common materials such as paints, batteries, pesticides, and solvents create toxic wastes during their manufacture or when these products are thrown out. We could eliminate toxic wastes by not using products such as paint, compact florescent light bulbs, and batteries, but that is not practical. That means we must properly manage the toxic waste resulting from the manufacture and disposal of these products.

Figure 6. Hazardous wastes

The United Nations Environment Programme (UNEP) has identified 11 key substances that pose a risk to human health:

- Arsenic: used in making electrical circuits, as an ingredient in pesticides, and as a wood preservative. It is classified as a carcinogen.

- Asbestos: is a material that was once used for the insulation of buildings, and some businesses are still using this material to manufacture roofing materials and brakes. Inhalation of asbestos fibers can lead to lung cancer and asbestosis.

- Cadmium: is found in batteries and plastics. It can be inhaled through cigarette smoke, or digested when included as a pigment in food. Exposure leads to lung damage, irritation of the digestive tract, and kidney disease.

- Chromium: is used as brick lining for high-temperature industrial furnaces, as a solid metal used for making steel, and in chrome plating, manufacturing dyes and pigments, wood preserving, and leather tanning. It is known to cause cancer, and prolonged exposure can cause chronic bronchitis and damage lung tissue.

- Clinical wastes: such as syringes and medication bottles can spread pathogens and harmful microorganisms, leading to a variety of illnesses.

- Cyanide: a poison found in some pesticides and rodenticides. In large doses it can lead to paralysis, convulsions, and respiratory distress.

- Lead: is found in batteries, paints, and ammunition. When ingested or inhaled can cause harm to the nervous and reproductive systems, and kidneys.

- Mercury: used for dental fillings and batteries. It is also used in the production of chlorine gas. Exposure can lead to birth defects and kidney and brain damage

- PCBs, or polychlorinated biphenyls, are used in many manufacturing processes, by the utility industry, and in paints and sealants. Damage can occur through exposure, affecting the nervous, reproductive, and immune systems, as well as the liver.

- POPs, persistent organic pollutants. They are found in chemicals and pesticides, and may lead to nervous and reproductive system defects. They can bio-accumulate in the food chain or persist in the environment and be moved great distances through the atmosphere.

- Strong acids and alkalis used in manufacturing and industrial production. They can destroy tissue and cause internal damage to the body.

3.3 Utilization and disposal of nuclear and radioactive waste

Classification and rules of circulation

Like all industries, the generation of electricity produces waste. Whatever fuel is used, the waste produced in generating electricity must be managed in ways that safeguard human health and minimize the impact on the environment.For radioactive waste, this means isolating or diluting it such that the rate or concentration of any radionuclides returned to the biosphere is harmless. To achieve this, practically all radioactive waste is contained and managed, with some clearly needing deep and permanent burial. From nuclear power generation, unlike all other forms of thermal electricity generation, all waste is regulated – none is allowed to cause pollution.

Nuclear Waste

  - Some radioactive

Types of radioactive waste

  - Low-level waste

  - High-level waste

Disposal and storage

  - Low-level waste disposal

  - Spent fuel storage

  - Waste isolation

Reprocessing

Decommissioning

Figure 7. Types of waste from nuclear power plants

- Low-level waste is usually packed in boxes or drums and shipped to disposal sites where it is

- Buried in trenches

- Covered with soil and a cap that sheds water and

- Monitored to detect radiation.

Figure 8.Options and challenges

- Store waste in only approved bags and container.

- Bags – Yellow with Radiation label

- Boxes – Line with yellow bag

- Bottles – Attach Radiation label

- DO NOT use anything that can be mistaken for a ordinary trash container

- When full label with user’s name and dept., isotope, activity, fill date and seal with a liquid tight tie-off

Figure 9.Waste conteiner

Decay in Storage

  - Store waste in the laboratory if the half-life is no more than 120 days.

  - “DIS” Isotopes must be held for decay for at least 10 half-lives.

  - Survey monitoring of material must read close to background.

  - All radioactive labeling must be defaced.

Chapter 4

Biological and  non-biological processes of transformation of pollutants

4.1 Types of transformation

In the absence of living organisms, chemical pollutants may undergo abiotic transformation.

The most important non-biological processes of organic xenobiotics transformation are hydrolysis, oxidative reactions, catalytic decomposition, photolysis and polymerization. These processes except polymerization, contribute to the degradation of organic xenobiotics. As a result of polymerization, substances with bigger molecular weight than the original compound are formed.

Hydrolysis of chemical contaminants actively occurs in aqueous media and often leads to loss of their toxic properties.

Hydrolysis makes a significant contribution to the destruction of chlorinated aliphatic acids, esters and amides of carboxylic acids, phosphorus compounds and carbamates and plays a role in the destruction of polymers related to polyamides, polyurethanes, aliphatic and aromatic polyesters.

During decomposition of herbicides related to chloroacetamide in the soil, a hydrolysis of the bond of the chlorine atom with the carbon atom and the amide bond occurs. The half-decay period is 16-18 days for different soil and climatic zones. Hydrolysis of organophosphorus compounds is accompanied by removal of carboxyl or methyl group attached to phosphorus.

Under natural conditions, chemical hydrolysis of most pollutants is slow. The speed depends on the nature of the side groups, the characteristics of the aggregate state of the pollutant, its solubility in water, temperature, pH of the medium, the composition of the solution. In water organophosphorus pesticide typos collapses by 50% for 120 days, and in slightly alkaline lime mortar for 8h. The other pesticide groups that Malathion are hydrolyzed relatively fast at pH = 5.0-7.0.

The hydrolysis process can catalyze various ions of metal cations, phosphate ions, anions of monobasic and polybasic acids present in the medium.

In the soil, the rate of hydrolysis of adsorbed xenobiotic molecules on the surface of the solid phase in the presence of cations Мn, А1, Са, Ма, Сu can increase. Thus, Si ions accelerate the hydrolysis of TAE, flowing on the mineral surface of soil in contact with Сu. Minerals of clays and metal oxides can also be directly involved in hydrolysis reactions. The surface of amorphous silica usually does not show catalytic properties.

In general, hydrolytic reactions contribute little to the transformation of organic pollutants in natural environments.

Photolysis - transformation of molecules of substance under the action of absorbed light (e.g., oxidation). Photolysis is one of the mechanisms of natural detoxification of pollutants. The basis of the hotolysis is photochemical reactions. Photo periodism is alternation of light and dark periods within 24 hours of the day.

Flash photolysis, the second type of active particle generator, is relatively clean, since the choice of the wavelength of the source (usually pulsed UV lamp) allows you to choose the type of particles formed and the degree of their excitation. One of the main limitations of this method is a relatively low concentration of products but the progress of laser technology allows the use of more power in the pulse and thus increases the concentration. The characteristic time scale of the studied phenomenon is 1 µs, three orders of magnitude less than in the discharge jet method.

In addition to photolysis, aquatic plants improve water quality by absorbing a number of dissolved and dispersed substances. Thus, they are an important component of the biological wastewater treatment process.

Direct photolysis of cellulose leads to the rupture of chemical bonds absorbed by light in the cellulose macromolecule. Therefore, the energy of the absorbed quantum should be sufficient to cause such a gap. Breaking C—C or C—O bonds in the cellulose macromolecule requires energy of 80-90 kcal/mol. There are 100 kcal/mol is needed to break the C-H bond.

Oxidative UV-photolysis of water samples has the lowest reactive background due to the minimum use of reagents, good reproducibility of results, and does not depend on the high chloride content, which allows using this method of sample preparation for both fresh and sea water. The efficiency of photochemical destruction of dissolved organic forms largely depends on the presence of oxidants and the pH of irradiated sample. It was found that the photochemical destruction of dissolved organic substances without the addition of oxidants proceeds very slowly and incompletely. However, UV photolysis of sea waters provides effective destruction of organic complexes of heavy metals by adding only hydrochloric acid (pH 1.5).

Anthropogenic impact on the environment was destructive. The law of historical development of Biosystems does not work fully or does not work at all because the role of biotic influence on the environment has decreased. Transformative human activity prevails. After the direct destruction of one species, one should expect the destruction of the other according to the relationship of all living organisms .This process actually goes in the form of mass reproduction of individual organisms destroying the developed ecosystems. All this depends on the pace of change. Due to the increased anthropogenic impact, there is an intensive transformation not only of the abiotic components of the biosphere — hydrosphere, atmosphere, upper part of the lithosphere, but also of biotic communities (flora and fauna). The stability of the biosphere is impossible without providing favorable living conditions for all biotic communities in all their diversity. The destruction of forests, other vegetation and wildlife is the destruction of the natural habitat of man with unpredictable consequences. The loss of biodiversity also threatens its very existence.

The largest amounts of synthetic organic compounds and their transformation products are emitted into the atmosphere or discharged into water bodies in densely populated areas, which leads to high pollution of the human environment at the local and regional levels. Some of these components have undesirable resistance (persistence) to biotic and abiotic factors and can therefore be included in migration processes and cause pollution of the environment at the large-regional or even global levels.

Self-purification of natural waters is a variant of biotic transformation of the environment, the process of purification of water from pollutants by their decomposition and deposition. Self-purification of natural waters occurs in both anaerobic and aerobic environments.

Self-purification of natural water is more active if the concentration of oxygen in the water is high. Not only bacteria but also algae and animals take part in self-purification of natural water. In running water self-purification occurs more actively than in standing. When a large amount of wastewater enters the water bodies (this is the case in large cities), the ability to self-purification of natural water bodies is insufficient.

Special treatment facilities and reduction of discharges through the use of low-waste technologies are needed.

4.2 Biotic transformation of organic pollutants

It is known that plants for hundreds of millions of years synthesized organic compounds from carbon dioxide but no significant accumulation of organic substances during this time has occurred. Only a small part of them in conditions without access to air remained in the form of strongly reduced carbon compounds - oil, natural gas and coal. In nature, there is always a microorganism that can completely or partially transform very complex substances, and the products of this transformation will be used by other microorganisms.Transformation of organic residues (plant, animal and microbial origin) is an important process that determines the existence of the biological cycle of elements in nature. The bulk of organic substances undergoing transformation in the soil is polymeric compounds of cellulose, pectin, chitin, lignin, humus, etc. Moreover, easily decomposing materials are subjected to rapid and sufficiently complete oxidation; the polymer compounds are hardly cleaved by microorganisms and therefore remain in the soil for a long time as its organic components. Consequently, the organic matter of the soil is partly composed of not completely disintegrated residues of plants and partly of humus. Humus is an amorphous, usually dark-colored material of biological, mainly microbial origin. The composition of humus includes compounds that are difficult to decompose by microorganisms – primarily lignin, as well as fats, waxes, carbohydrates and protein components. They turn into polymer compounds that are not amenable to precise chemical characteristics. Polymer compounds are subjected to microbial transformations, which involve a variety of groups of saprotrophic organisms, including representatives of chemo-organoheterotrophic bacteria.

The rate of transformation of polymeric organic substances in the soil, the nature of microorganisms that carry it out, has a great influence on the processes of soil formation, the creation of its structure and the level of fertility (the distribution of humus on the soil profile, etc.). Due to the intensification of agricultural production (widespread use of mineral fertilizers, intensive soil treatment systems, reclamation and highly specialized crop rotations), the processes of microbial decomposition of soil organic matter are significantly accelerated. This leads to the degradation of humus, accompanied by obstructionism, decrease in absorption capacity, buffering capacity, water retention capacity and other negative processes that, ultimately, determine decrease in soil fertility. Hence, there is a need to slow down the process of transformation of organic matter in the soil. However, to solve this problem it is necessary to know some basic principles of microbial transformation of organic substances in the soil (humus, lignin and other polymeric substances).

In the resistance to transformation, the chemical structure of organic matter is of great importance, and secondly, the nature of the microbial cenosis of the soil and the conditions of its functioning. However, at present, not only the composition of many polymeric compounds included in the organic matter of the soil, but also the specifics of its transformation by microbial cenosis and its individual representatives remains insufficiently studied. In addition, it is known that the current level of anthropogenic impact on the soil is accompanied by a significant change in its ecology. It leads to a violation of the natural balance in the microbial cenosis of the soil, restructuring in its functional and taxonomic structure. It significantly changes the intensity and direction of the processes of transformation of organic substances in the soil. Therefore, in order to develop reliable methods for regulating the rate of transformation of organic substances in the soil, it is necessary to study in detail the kinetics of its decomposition under various conditions and microorganisms that dominate the processes of destruction of an organic substance.

The transformation of organic substances in the soil is carried out by both aerobic and anaerobic microorganisms. Among the anaerobic bacteria particularly stands out grouping of microorganisms, which belongs to the genus Clostridium. It is one of the most widespread groups in all (or almost all) soil, taking part in many soil processes associated with the transformation of simple and complex organic compounds (carbohydrates, polysaccharides, proteins, nucleic acids, purine and pyrimidine bases, lipids, humus substances, oil and other substances (Emtsev, 1987).

It is also known that Clostridium participates in the transformation of xenobiotics insecticides and herbicides in soil (Emtsev, 1982). Since the genus Clostridium includes a number of physiological subgroups of organisms with different enzyme systems, it is necessary to study the participation of each such group of anaerobic bacteria in the destruction of various organic substances, as well as the conditions of these processes.

4.3 Destruction of oil pollutants in the soil

4.3.1 Ecological and geochemical characteristics of oil composition

Oil pollutants closely interact with three groups of environmental factors:

- Unique multi-component composition of oil, which is in the process of constant change;

- Heterogeneity of the composition and structure of any ecosystem that is in the process of continuous development;

- Diversity and variability of external factors that influence the ecosystem: temperature, pressure, humidity, state of the atmosphere, hydrosphere, etc.

It is quite obvious that it is necessary to evaluate the consequences of pollution of an ecosystem with oil and to outline ways of elimination of these effects, taking into account the specific combination of these three groups of factors.

Oil is a complex mixture of components that are difficult to dispose of in nature. 70-90% of all petroleum substances are hydrocarbons of three classes: paraffinic, naphthenic, and aromatic. The smaller the molecular weight of oil components, the higher the rate of its spread in the environment. Toxic gasoline-kerosene fractions and aromatic compounds, dissolving well in water, are able to migrate over long distances in the horizontal plane and deeply spread over the soil profile. Mononuclear hydrocarbons – benzene and its homologues – have a faster toxic effect on organisms than polyaromatic hydrocarbons, as large molecules of polyaromatic hydrocarbons penetrate cell membranes more slowly. More than 208 oil fields have been discovered in Kazakhstan. Oil and oil products are one of the most dangerous and large-scale environmental pollutants. Therefore, the ecological problem of rebuilding a biosystem, contaminated with oil and oil products, is an extremely urgent task. Elimination of pollution by only mechanical and physicochemical methods does not always achieve the desired effect, since often there are problems of recycling of waste generated after treatment.

The most promising are biological methods of cleaning oil-contaminated objects, as they do not cause additional damage to the environment and they are the cheapest. A great deal of preference is given to bacterial preparations, since they are more viable and competitive in the environment.

The process of destruction of petroleum products with the help of biopreparations proceeds from a few days or weeks to several months, depending on the degree of contamination of the facilities, chemical composition of the contaminant, climatic and physicochemical parameters of the environment. In general, the degradation of hydrocarbons with the use of biological oil-oxidizing action occurs 100 times faster than the process of natural decomposition.

The main natural destructors of oil and oil products are oil-oxidizing bacteria, fungi and yeast. To date, some strains of these microorganisms are already used to clean contaminated areas of hydrocarbons, especially in large quantities; they are found in places heavily polluted with oil and oil products.

For biodegradation of petroleum and petroleum products specialists of biotechnology select specialized types of microorganisms – hydrocarbon oil destructors in contaminated sites. Such microorganisms are well adapted to local conditions and at high specific petro-biodestructive activity are very effective. A large number of works has been devoted to the problems of microbial purification of water and soil from oil and petroleum products. They consider microorganisms that are used in cleaning: Pseudomonas, Arthrobacter, Rhodococcus, Acinetobacter, Flavobacterium, Corynebacterium, Xanthomonas, Alcaligenes, Nocardia, Brevibacterum, Mycobacterium, Beijerinckia, Bacillus, Enterobacteriaceae, Klebsiella, Micrococcus, Spaerotilus, Serratia, microscopic fungi and yeast.

At the current level of the development of the oil industry, it is not possible to exclude its impact on the environment. Pollution of the natural environment with oil and petroleum products leads to significant economic and environmental damage. Large-scale development and production of hydrocarbon raw materials are carried out on land in Atyrau, Mangystau, Shymkent, West-Kazakhstan and Aktobe regions. Due to the fact that these territories mainly belong to steppe, semi-desert and desert zones with sharply continental foothill climate conditions, ecosystems of these areas are characterized by high vulnerability and low self-healing potential. From the chemical point of view, the process of natural complete destruction of oil proceeds for at least 25 years, but the toxic properties of oil disappear after 10-12 years, the products of its decomposition are partially included in humus, partially dissolved and removed from the soil profile.

The ability to use oil as an energy source is not unique to some forms, including many fungi, yeasts and bacteria. The use of microbial biologics for elimination of environmental pollution is a well-known technique described by many researchers.

Oil is a complex mixture of difficult-to-utilize components in nature. 70-90% of all oil substances are hydrocarbons of three classes: paraffinic, naphthenic, and aromatic. The smaller the molecular weight of the components of oil, the higher the rate of its spread in the environment. Toxic gasoline-kerosene fractions and aromatic compounds, readily soluble in water, are able to migrate over considerable distances in the horizontal plane and deep to spread along the soil profile. Mononuclear hydrocarbons - benzene and its homologues have a faster toxic effect on organisms than polyaromatic hydrocarbons, since large molecules of polyaromatic hydrocarbons penetrate more slowly through cell membranes. The main role in the biodestruction of oil pollution belongs to microorganisms.

The development of new purification biological methods on the basis oil-destructive strains can significantly intensify the processes of natural environment restoration. Currently, more than 100 species of microorganisms belonging to 40 genera of eubacteria, yeasts, and fungi capable of utilizing hydrocarbons are known.

One of the main and most important industries of Kazakhstan is oil production and development of petroleum products. According to confirmed oil reserves, Kazakhstan is among the top 15 countries in the world, with 3% of the world's oil reserves. The area of prospected and used oil and gas bearing areas is 17,000 thousand km2, which is more than 62% of the country's total territory and includes 208 oil deposits. More than 90% of the oil reserves are concentrated in 15 largest fields - Tengiz, Kashagan, Karachaganak, Uzen, Zhetybai, Zhanazhol, Kalamkas, Kenkiyak, Karazhanbas, Kumkol, Kenbai, Severnye Buzachi, Alibekmola, Central and Eastern Prorva, Kenbai.

The increase in the scale of hydrocarbon production is due to the need to meet the needs of industry and transport. Under these conditions, the measures taken at the oil production and processing sites do not remove the problem of pollution of ecosystems by oil and oil products due to a large number of the sources of pollution.

The chemical composition of oil depends on the production area and is determined on the average by the following data: carbon (83-87%), hydrogen (12-14%), nitrogen, sulfur, oxygen (1-2%, rarely 3-6% due to sulfur). Tenth and one-hundredths of a percent are numerous trace elements, the set of which in any oil is approximately the same.

It is quite obvious that it is necessary to evaluate the consequences of pollution of the ecosystem with oil and outline ways to eliminate these effects, taking into account a specific combination of these three groups of factors.

As for ecological and geochemical characteristics of the basic composition of oil, it contains the light fraction (boiling point of 200 °C), methane hydrocarbons (including solid paraffins), cyclic hydrocarbons, resins and asphaltenes, and sulfur compounds.

Many researchers note a strong toxic effect of the light fraction on microbial communities and soil animals. The light fraction, migrating along the soil profile and aquifers, expands, sometimes significantly, the halo of the original contamination.

On the surface, this fraction is firstly subjected to physico-chemical decomposition processes, the hydrocarbons entering into its composition are most rapidly processed by microorganisms.

4.3.2 Methods for eliminating oil contamination in soil

A review of McGill (1977) provides the data of researchers from different countries on the establishment of safe limits for the content of oil and oil products in soils.

Based on the analysis of world experience and experimental data, McGill compiled a table of indicative standards for the content of oil and oil products in soils that are subject to reclamation (Table 2).

Table 2. Relative degree of disturbance of soils containing different amounts of oil

| Degree of degradation | The content of oil in the soil, mg/kg of dry soil |
| --- | --- |
| From mild to moderate: in the absence of any special measures, some temporary weakening of vegetation growth | 5000-20000 |
| From moderate to high: only certain types of plants are able to develop normally; soil restoration is possible within three years; Without reclamation restoration will require 2-3 times more time | 20000-50000 |
| From high to very high: oil fronts the soil to a depth of 10 cm; only a few plants survive; With a rational reclamation, soil restoration will take 20 years or more | More than 50000 |

At present, a number of methods for eliminating oil contamination of the soil has been developed, including mechanical, physico-chemical, and biological methods (Table 3).

Natural self-purification of natural objects from oil pollution is a long process, especially in conditions where the temperature is high for a long time. In this regard, the development of methods for cleaning the soil from oil pollution with hydrocarbons is one of the most important tasks in addressing the problem of reducing the anthropogenic impact on the environment. From the chemical point of view, the process of natural oil destruction completely ends in at least 25 years, however, the toxic properties of oil disappear after 10-12 years, the products of its decomposition are partially dissolved and removed from the soil profile.

Table3. Methods for Eliminating Oil Contamination in Soil

| Methods | Ways of liquidation | Application features |
| --- | --- | --- |
| Mechanical | Polishing pollution, pumping oil in a container | Primary measures for large spills in the presence of appropriate equipment and reservoirs (the problem of cleaning the soil during the infiltration of oil into the ground is not solved) |
|  | Soil replacement | Removal of soil for landfill for natural decomposition |
| Physico-chemical | Burning | Emergency measure in case of threat of breakthrough of oil into water sources. Depending on the type of oil and petroleum product, from 50 to 70% of the spill is destroyed, the rest is leaked into the soil. Due to the insufficiently high temperature, the products of sublimation and partial oxidation of oil enter the atmosphere; land after burning must be taken to the landfill. |
|  | Prevention of fire | When spills of flammable products occur in shops, residential areas, on motorways, where combustion is more dangerous than soil contamination; isolate the spill from above with fire-fighting foams or cover with sorbents. Soil washing is carried out in washing drums with the use of surfactants, washing water is settled in waterproof ponds or containers. |
|  | Soil drainage | Variety of washing the soil in situ with the help of drainage systems; can be combined with the use of oil-forming bacteria. |
|  | Extraction with solvents | It is usually carried out in flushing drums with volatile solvents followed by distilling off their residues with steam. |
|  | Sorption | Spills on a relatively hard surface (asphalt, concrete, compacted soil) are covered with sorbents to absorb oil products and reduce fire risk during spills of flammable products. |
|  | Thermal desorption | It is used rarely with the availability of appropriate equipment, it allows us to obtain useful products up to the black oil fraction. |
| Biological | Bioremediation | Apply oil-forming microorganisms. It is necessary to cultivate them in the soil. Periodic fertilizing with fertilizer solutions, limitation in depth of treatment, soil temperature (above 15 ° C), the process takes 2-3 seasons. |
|  | Phytoremediation | Elimination of oil residues by sowing oil resistant grasses (creeping clover, sorrel, sedge, etc.) that activate the soil microflora in the final stage of reclamation of contaminated soils. |

In the initial period of development and operation of oil fields on a production scale, only the most primitive methods of liquidating oil spills were used: pumping out, burning out oil, piling oil contaminated sites with sand.

Until recently, the most common and cheap method of eliminating oil pollution was simple combustion. This method is not effective and harmful for two reasons:

1) incineration is possible, if oil lies on the surface with a thick layer or collected in accumulators; soaked soil or soil will not burn;

2) in the place of burned oil products, the productivity of soils is not usually restored, and among the combustion products remaining in place or dispersed in the environment, many toxic, in particular, carcinogenic substances appear.

The qualitative removal of oil pollutants at high levels of pollution is often not without the use of various kinds of sorbents. Among the possible raw materials for the production of sorbents, the most attractive are natural organic raw materials and waste products of plant origin. Such raw materials include peat, sapropels, waste from processing of agricultural crops, etc. Such sorbents as Sorbest, RS, Leszor, etc. have been developed on the basis of such raw materials

There is a technology for purifying soils and groundwater by washing them with surfactants. This way one can remove up to 86% of oil and petroleum products. It is unlikely to use it on a large scale, since surface-active substances themselves pollute the environment and the problem of their collection and utilization will arise. The existing mechanical, thermal and physico-chemical methods for cleaning soils from oil contamination are costly and effective only at a certain level of pollution (as a rule, at least 1% of oil in the soil). They are often associated with additional pollution and do not ensure the completeness of treatment.

Thus, a literary review showed that Kazakhstan has a powerful oil reserve. However, the problem of cleaning of oil-polluted ecosystems, as well as their recovery in Kazakhstan is acute. To this end, various technologies are being developed and put into practice, including biological methods. Microbial products, the basis of which is active microorganisms-destructors of petroleum hydrocarbons, are widely used. Microorganisms are capable of oxidizing various components of petroleum, such as aliphatic, mono- and polyaromatic hydrocarbons, heterocyclic, halogenated and methylated organic compounds. The study of the group composition of oil must be considered when developing a technology to clean up natural ecosystems from oil pollution. The study of the ability of microorganisms to oxidize specific classes of hydrocarbons in the composition of oil, allows us to create targeted biological products.

Natural self-purification of natural objects from oil pollution is a long process, especially in conditions where a low temperature regime remains for a long time. In this regard, the development of methods for cleaning soil from hydrocarbon pollution of oil is one of the most important tasks in solving the problem of reducing the anthropogenic impact on the environment.

4.3.3 Biological methods based on different groups of microorganisms

Oil pollution differs from many other anthropogenic impacts in that it gives not a gradual, but, as a rule, sudden load on the environment, causing a quick response. When assessing the effects of such pollution, it is not always possible to say whether the ecosystem will return to a sustainable state or will irreversibly degrade. The essence of the restoration of polluted ecosystems is the maximum mobilization of the ecosystems’ internal resources to restore their original functions. Self-healing and remediation are an inseparable biogeochemical process. Currently, the most promising for the treatment of oil contaminated soils, both economically and environmentally, are biological methods that do not cause additional damage to the environment and are cheaper: if the cost of cleaning 1m3 of contaminated soil by burning is about 3 thousand US dollars, then with the use of bio-cleaning it is from 100 to 300 dollars.

Biological methods based on different groups of microorganisms are based on high ability of such microorganisms to biodestruct the components of oils and petroleum products. Biological methods include agrotechnical methods, bioremediation, composting, and bioventilation. Bioremediation (biological recovery) is a well-known biological method of soil purification based on activating the activity of native microflora by introducing various stimulating additives or active microorganisms-destructors into the contaminated ecosystem.

The methods used in bioremediation of soils can be divided into In-situ and Ex-situ. In turn, Ex-situ methods are subdivided into On-site and Off-site methods.

In-situ method is based on carrying out measures for cleaning oil-polluting soils directly on the contamination site, without soil removal. Isolated bioaugmentation and biostimulation of WIP.

Bioaugmentation consists in processing of WIP with cultures of oil-oxidizing microorganisms in combination with the introduction of a complex of mineral fertilizers.

Biostimulation is a complex of agrotechnical measures (plowing of the soil surface to improve aeration, introduction of structurators, mineral substances), which are carried out to stimulate native soil microflora.

The Ex-situ methods provide for the removal of WIP from the contamination site and for subsequent measures to intensify the process of biological soil purification. Introduction of UVOM crops and a complex of agrotechnical measures allow us to intensify the process of WIP purification. This group of methods is divided into On-site and Off-site.

The On-site method includes withdrawal of WIP and treatment of contaminated soil at the site of formation and can be implemented by using mobile bioreactors.

The Off-site method includes withdrawal of WIP and treatment of contaminated soil outside its site of formation. This method is implemented through stationary and industrial bioreactors, as well as by creating specialized technological platforms for bioremediation.

Further research is aimed at improving In-situ and Ex-situ methods by creating On-site and Off-site bioreactors.

The advantage of bioremediation is 10-70-fold reduction in the content of oil and oil products in water and soil per season. The use of microbes-destructors isolated from natural microbiocenosis excludes unpredictable ecological consequences possible with the use of extraneous types of microorganisms, and a complex of isolated strains of microbes-destructors ensures complete mineralization of oil and oil products to carbon dioxide and water. The technology allows us in each case to develop a set of agrotechnical measures aimed at elimination of the activity of microbial destructors isolated from native microflora. All components are introduced into the soil at the same time. Concentrated biological preparation is of low cost. Bioremediation is safe for human life. This method has its drawbacks. The total duration of the reclamation process depends on the soil-climatic conditions and the nature of the contamination. Stimulation of the soil microflora should begin only when the concentration of the total oil product is reduced to 23-25% in organogenic or 15-18% in mineral soil horizons on average over the site.

Intensive growth and vital activity on soils contaminated with oil hydrocarbons depend on:

- the genus and species of microorganisms;

- ambient temperature;

- pH of the medium;

- preservation of organic and mineral components necessary for vital activity of microorganisms in the polluted ecosystems;

- the degree of aeration of contaminated soil;

- the type and composition of oil pollution;

- the depth of penetration into the soil;

- concentrations of different classes of petroleum hydrocarbons in the soil;

- their resistance to biodegradation, etc.

In addition, weighed or gypsum is used to reduce the acidity or alkalinity of the soil, since the most optimal for most microorganisms are pH values close to neutral. In arid conditions irrigation is necessary.

The search for and isolation of microorganisms possessing hydrocarbon activity allowed scientists to create preparations for cleaning soils and water from oil contamination. At present, microbial preparations are used to purify soils from crude oil, oxidizing mainly paraffinic hydrocarbons of oil. At the same time, the problems of cleaning soils and water from hardly decomposable petroleum products, for example, fuel oil, lubricating oils, etc., are not fully developed.

Microorganisms using hydrocarbons that are always present in the soil are widely distributed. The ability of biodegradation of various petroleum hydrocarbons has more than 20 genera of bacteria (Arthrobacter, Pseudomonas, Bacillus, Agrobacterium, Alcaligenes, Flavobacterium), 24 genera of microscopic fungi (Trichoderma, Pénicillium, Aspergillus, Mortirella), 19 genera (Aureobasidium, Candida and Rhodotorula), and various actinomycetes. On their basis, biopreparations are produced, which are used for bioremediation of oil-contaminated soils.

Bacterial preparations recommended and used for bioremediation may include monocultures or a special combination of microorganisms. In the world practice, methods of microbial cleaning of oil-contaminated surfaces are successfully used. These are such well-known firms as Occidental Chemical (USA), Beistritent (England), Biodetox (Germany). Only in the last ten years in the CIS countries more than two dozens of biopreparations have been created on the basis of oil-oxidizing microorganisms. The main contribution to the development in this area belongs to Russian scientists.

In Russia, "Putidol", "Devoroyl", "Valentis", "Neftoks", and others are widely used, in Byelorussia - "Rodobel-T". A series of preparations "Pseudomin", "Naftok", purifying the soil from aromatic and paraffin compounds is known. Biological preparations "Ecoil" and "Lestan" are preparations of a wide spectrum of action. There are other biopreparations of various activity spectra, such as "Putidol", "Biodestructor", "Roder", "Avalon", "Bioset", etc. , designed to clean oil-contaminated areas. However, when introducing new types of microorganisms, it is necessary to take into account the natural conditions of soil formation and the types of soils that have a certain effect on the life activity of the microbial flora.

In Ukraine, the most famous preparations are: Desna, and Devoroil, which is produced under a Russian license and is recently recommended by Simbinal (the development of the Institute of Botany of the National Academy of Sciences).

In the process of vital activity, microorganisms actively synthesize their own enzymes and biological surface-active substances, which reproach the decomposition of oil. Biotransit "Petro Trit" is used for cleaning soil and water from oil and oil products, fuel oil, diesel fuel, gasoline, motor oils. "Soilthrit" cleans soil and water from polychlorinated biphenyls (PCBs) - transformer oil, sovtol, autol. "Roder" preparation is effective in various soils, fresh and sea water. "Econadin" is a new generation of bacterial drug based on avirulent oil-oxidizing bacteria, exhibiting sorption and destructive activity in relation to petroleum hydrocarbons.

Biopreparations of the series "Biodestructor" are intended for liquidation of pollution by oil, oil products, condensate and other organic compounds of various environmental objects. Biological preparations Allegro, Tornado, Gera decompose crude oil, gasoline, diesel fuel, phenols, alcohols, vegetable oils, animal fats, heptyl, pesticides, and dioxins. Biological preparations "Valentis", "Leader", "MAG" decompose crude oil, mazut, engine and motor oil, gasoline, heptyl, dioxins, pesticides, vegetable oils, animal fats and alcohols.

In Kazakhstan, a composition based on natural strains of Pseudomonas putida GNPO PE-R-6, Pseudomonas fluorecens GNPO PE-R-5, Bacillus subtilis GNPO PE-R-7 and a complex of mineral fertilizers-ammonium dihydrogen phosphate and potassium hydrogen fluoride was developed. This composition is intended for biochemical treatment of oil-contaminated soils. The investigations, carried out under the conditions of the experimental production of the "Kazmechanobr" State Unitary Enterprise, showed a complete and rapid 100% destruction of oil and oil products hydrocarbons.

In the South Kazakhstan M. Auezov State University. microbiologists have developed a biopreparation "Peroyl", consisting of cultures of Micrococcusluteus B1Ag 8 G and Rhodococcuserythropolis LN 304 B-7, decomposing monocycloaromatic, bicycloaromatic compounds, toluene resins and asphalts. The effectiveness of the biopreparation for cleaning the soil of arid regions from oil and oil products is 96.5%. Scientists of the RSE "Institute of Microbiology and Virology" developed biochemicals of the Bakoil KZ series based on oil-oxidizing microorganisms. These preparations are competitive, applicable for soil and climatic conditions in Kazakhstan and do not refer to expensive preparations. They were tested in Western Kazakhstan on 43 hectares. The efficiency of soil cleaning from oil was 98%. A wide range of approaches to bioremediation of WIP allows us to choose methods based on natural and climatic conditions of the contaminated area, the volumes of WIP produced, and the concentration of petroleum products in the soil. Bioremediation has a great potential for preventing environmental pollution and for dealing with already contaminated areas. Compared with other methods of cleaning the environment from pollution, bioremediation In-situ is much cheaper. In contrast to industrial biotechnology, where it is possible to control all the parameters of the technological process, bioremediation, as a rule, is carried out, literally, in an open system, i.e. in the environment. Thus, in Europe and in particular in Germany, mostly In-situ methods are used, which is caused by small amounts of WIPs formed, a local nature of contamination with oil products or mineral oils, as a rule, these are filling stations, or conserved areas of chemical enterprises.

Due to natural and climatic peculiarities of Europe, namely, a long vegetation period, this method has fewer drawbacks than when it is applied in Russia. In some regions of the Russian Federation and Kazakhstan, the implementation of this method is limited by natural and climatic conditions, but in some southern regions it can be widely used.

The process of biodegradation of hydrocarbon oil in natural conditions is most effective at the following parameters:

- the temperature of the medium is not lower than + 10 ° C and not higher than + 50 ° C;

- the soil surface moisture is about 70% of the total moisture capacity;

- introduction of basic biogenic elements (ammonium nitrogen, water-soluble salts of phosphorus, potassium, magnesium, etc.);

- absence of toxic compounds for microorganisms.

For example, the lack of use of Ex-situ methods in Germany is explained by the fact that technogenic impact of oil pollution of natural soils is not so important in the country. That is why, there are no enterprises specializing in bioremediation of WIP.

Russia, unlike Germany,widely used the methods of Ex-situ bioremediation, namely, bioremediation technology on technological platforms. Priority of the application of this method is due to the large volumes of generated WIP, due to the fact that oil production, oil processing and a network of main oil pipelines are intensively developed on its territory. At high concentrations of oil hydrocarbons in the soil or the impossibility of carrying out the process using technological platforms (due to natural and climatic conditions) the application of bioreactor technologies is actual.

The use of bioreactor technology is expedient with the aim of eliminating a high level of soil contamination (more than 30 g/kg). With the help of intensive bioreactor technologies it is possible to reduce the high level of oil pollution to the permissible residual content of oil products in the soil or to some specified values with further post-treatment at technological areas. The disadvantage of intensive bioreactor technologies is the limited volume of loading in comparison with the technological sites on which large volumes of WIP are processed.

Due to the fact that bioremediation on open technological areas can be limited by natural and climatic conditions, scientists develop technologies that allow them to maintain optimal parameters of WIP bioremediation process. The most promising direction is the development and application of bioreactor technologies, which allow us to carry out the process with given parameters. The advantages of these methods are minimization of risks of environmental pollution by preventing migration of oil pollution to adjacent environments, as well as the possibility of intensifying the purification process. Especially relevant is the use of bioreactor technologies in the northern regions, where the implementation of bioremediation in open areas is difficult due to low rates of self-cleaning of the soil, due to complex natural and climatic conditions.

Thus, for the restoration of oil contaminated lands the bioremediation method with various technological solutions is widely used; this approach is most suitable for Kazakhstan, where the oil industry is actively developing.

The advantage of bioremediation is 10-70-fold reduction in the content of oil and oil products in water and soil per season. The use of destructive microbes, isolated from the natural microbiocenosis, eliminates unpredictable environmental consequences possible with the use of extraneous species of microorganisms, as well as a complex of isolated destructive microbial strains provides complete mineralization of oil and petroleum products to carbon dioxide and water. The technology allows us in each case to develop a complex of agrotechnical measures aimed at optimization of activities of the culture of microbes-destructors isolated from the indigenous microflora. All components are introduced into the soil at the same time. Concentrated biological product has a low cost. Bioremediation is safe for human life. This method has its drawbacks. The total duration of the reclamation process depends on the soil and climatic conditions and the nature of the pollution. Stimulation of soil microflora should begin only with a decrease in the concentration of total petroleum product to 23–25% in organogenic or to 15–18% in mineral soil horizons on average over the site.

4.3.4 Investigation and isolation of hydrocarbon-oxidizing microorganisms

The search for and isolation of microorganisms with hydrocarbon activity made it possible to create preparations for cleaning the soil and water from oil pollution. At present, microbial preparations are used to clean the soil from crude oil, which oxidize mainly paraffinic hydrocarbons. At the same time, the problems of cleaning up soils and water from difficult-to-decompose oil products, such as fuel oil, lubricating oils, etc., are badly studied. Investigation and isolation of hydrocarbon-oxidizing microorganisms allowed scientists to create preparations for purifying soils and water from oil contamination. Hydrocarbon-oxidizing microorganisms are a constant component of various ecosystems. The ability of biodegradation of various petroleum hydrocarbons was discovered in more than 20 genera of bacteria, 19 yeasts of genera and 24 genera of microscopic fungi. Biological preparations are made on their basis, which are used for bioremediation of oil-contaminated soils and water. In recent years, microbiological technologies for purification of natural environments from oil pollution, based on the use of associations of hydrocarbon-oxidizing microorganisms in combination with various substances that stimulate their activity, have been successfully developed. An important condition for the microbiological purification of contaminated soils is the ability of various groups of microorganisms (bacteria, actinomycetes, yeasts and micromycetes) to possess high initial vital activity, to jointly destroy hydrocarbons of oil.

At present, a large number of active associations of hydrocarbon-oxidizing microorganisms have been created to purify soils and waters from pollution by oil and oil products, but each of them works effectively only if there is a certain set of external conditions (soil-climatic factors, oil concentration). There are some biopreparations created on the basis of bacterial monocultures or consisting of several strains. And the latter can consist both of model combinations of selected strains and of consortia of microorganisms entirely isolated from natural sources. According to the researchers, the use of individual cultures and mixed associations of microbial hydrocarbon-degradation cultures, which are created by combining strains with known destructive capacities, has its drawbacks. Supporters of the use of associations believe that any bacterial strain destroys a limited number of oil components, and the use of a consortium with many strains is necessary to achieve complete and effective biodegradation of oil. Nevertheless, the activity of biopreparations based on monocultures, according to the described results, is not inferior to the activity of microbial consortia. For example, the preparation Rhodotrin (Rhodococcuserythropolis VKM AS-1339D) performs the destruction of oil sludge by 91% in the upper (10 cm) and 87% in the lower (20 cm) layer over 12 months; "Pseudomin" (Pseudomonas putida 91-96) contributes to the decomposition of oil with its concentration in peat of 19.7% to 75.6% for 3 months; "Bioprin" or "Oleovorin" (Acinetobacteroleovorans (strains VSB-712 or VSB-568)) is able to destroy oil up to 20 g / kg for 1 month. The biopreparation "Roder" (R-dissociates of two strains (Rhodococcusrhuber VKM As-1513D and RhodococcuserythropolisVKM As-1514D) provides destruction of oil in the soil in laboratory conditions by 81%, in natural conditions by 65% in one season. Bioset "(three types of aerobic microorganisms Micrococcus varians and two - the genus Arthrobacter) with a concentration of petroleum products in soil 0.6% leads to their destruction by 77% for 4 months.  Biopreparation "Devoroyl" (Rhodococcus sp., Rhodococcusmaris,  Rhodococcuserytrhopolis, Pseudomonas stutzeri, Candidasp.) At an oil concentration of 60, 120 and 180 m3/ha, it contributes to its destruction by 81.9, 35.1 and 58.1% for 12 months and by 82.4, 50.0 and 72.6% for 17 months, respectively, the microbiological preparation "COBE-10" based on 6 bacterial strains identified as Bacillussp., Rhodococcussp., Providenciasp., and Citrobactersp. , contributes to the removal of hydrocarbons of diesel fuel in the soil by 91% in 8 weeks compared to a 44% decrease in the control area of soil.

Biological product "Naftox", as shown by the researchers, is characterized by the ability to destroy oil at a high (up to 584 g/kg) pollution level, which is associated with the physiological and biochemical characteristics of its microorganisms. The scientists evaluated the addition of a bacterial consortium to stimulate bioremediation of oil-contaminated soil, in which the number of aboriginal UOM was low (103-104 cells/g soil). The processing of sites by the consortium in combination with nutritional supplements led to the removal of hydrocarbons of oil by 89.7-92.0% for 1 year.

A consortium of oil-oxidizing bacteria Bacillus brevis and Arthrobacter species, which are the basis of the biologic oil destructor "Lenoil", was obtained at the Institute of Biology of Ufa Scientific Center of the Russian Academy of Sciences. The biopreparation has a high carbon-oxidizing activity, is able to adapt to high doses of carbons and actively utilizes the substrate in various types of soils.

The authors showed that a consortium of microorganisms consisting of cultures of Rhodococcussp., Pseudomonas sp. and Arthrobactersp., when applied to oil-contaminated soil intensively activates the vital activity of native oil-oxidizing microflora and the processes of oxidation of petroleum products in soils of different climatic zones of the Khanty-Mansiysk Autonomous Okrug. There are other microorganisms, for example, the authors reported on the study of aboriginal psychoactive UOMs of the genus Cytophaga, isolated from oil contaminated soils of the Komi Republic. Among psychrotolerant oil-destructive strains isolated from various regions of Siberia, the authors describe microorganisms of the genus Arthrobacter, Bacillus, yeast of the genus Yarrowia.

UOM strains that are part of the preparations, when applied to oil contaminated sites, do not always compete with a natural microflora, which can quickly suppress artificially introduced strains, and the intensity of biodegradation is lower than expected. Potential reasons for the rapid decline in the population level of introducents include both biotic (predation by protozoa, competition with other soil microorganisms), and abiotic factors (presence of minerals, organic carbon, moisture, pH, temperature, toxic substances). Veen et al. also suggested that the content of substrates is so reduced over time that microorganisms are only able to maintain their metabolism, but are unable to grow and multiply.

Scientists of al-Farabi University have created several highly active consortiums of oil-oxidizing microorganisms.  They used microorganisms isolated from oil-contaminated soils and collection strains. For further field remediation works:  2 associations consisting of 2 cultures - Pseudomonas ssp. BCS-1: Pseudomonas aeruginosa H14; and consisting of 3 cultures of microorganisms - Pseudomonas. ssp.ZG-2; Pseudomonas. ssp. BShS-1: Pseudomonas aeruginosa H14 will be used.

The Institute of Microbiology and Virusology used the bacterial association Arthrobacter sp. P1 + K3 and the association of bacteria and yeast Arthrobacter sp. as oil-recovery microorganisms. The application of P1 and K3 + Candida sp. FS-4AT led to an increase in the activity of soil respiration by 1.6 - 1.8 times compared with the original soil and 1.2 - 1.3 times compared with the oil contaminated soil. The literature data indicate that the associations of microorganisms are capable of more complete and faster decomposition of hydrocarbon substrates in comparison with individual strains. Immobilization of biological preparations on carriers makes it possible to further increase efficiency and shorten the cleaning time. The destruction of oil in the environment is a complex multi-component process, influenced by the composition, concentration and duration of action of the pollutant, the type of soil, the diversity and variability of external factors acting on the ecosystem.

Thus, scientists have shown that the use of microorganism associations for bioremediation significantly influences the microbiological indices of the soil, activating not only the enzymatic activity of oil contaminated soils, significantly increasing the dehydrogenase, catalase and lipase activity of the soil, but also increasing the activity of respiration and, thereby, positively affecting its fertility.

It is shown that over the last 10-15 years, many biochemical preparations for cleaning ecosystems have been developed in Russian and foreign practice. However, the most effective are biological products based on natural hydrocarbon oxidizing microorganisms isolated in a specific climatic zone, since a microflora that is not inherent in one or another ecosystem introduced as biological products can be suppressed by aboriginal microbial populations. Therefore, Kazakhstan microbiologists search for effective aboriginal hydrocarbon-oxidizing microorganisms in order to create a full-fledged specialized consortium of microorganisms and to introduce them into the initially clean environment, as well as to study the ability of microorganisms to oxidize specific classes of hydrocarbons in the composition of oil, which will allow microbiologists to create biopreparations for a specific purpose.

Kazakhstani authors compiled a consortium of hydrocarbon-oxidizing microorganisms: Dietzia maris B21, Acinetobacter sp. B1, Rhodococcus sp. B 22, Microbacterium sp. B5, Bacillus sp. B10, which have the efficiency of 79%. This consortium can be recommended for use in cleaning oil-contaminated territories of West Kazakhstan.

Thus, to eliminate oil pollution, biological preparations are widely used. It is a specially selected, balanced association of hydrocarbon-oxidizing microorganisms using petroleum hydrocarbons as a source of energy and transforming them into organic matter of their own biomass. They use the mass of viable cells of biodegradable microorganisms and differ in the strains used for their production, which are characterized by various physiological and biochemical properties, such as thermotolerance, osmophilia, optimal for pH growth, the ability to include different classes of hydrocarbons and spectra of n -alkanes in the metabolic processes. These physiological and biochemical properties of biodegradable strains determine the effectiveness of application of biopreparations in different climatic zones in removing contamination specific for the chemical composition. The main advantages of this technology are the ecological purity, high profitability, the absence of secondary pollution, and minimal costs.

4.4 Bioremediation, Bioaugmentation, Biostimulation

Bioremediation is a waste management technique that involves the use of organisms to remove or neutralize pollutants from a contaminated site. Bioremediation is a “treatment that uses naturally occurring organisms to break down hazardous substances into less toxic or non-toxic substances”. Technologies can be generally classified as in situ or ex situ. In situ bioremediation involves treating the contaminated material at the site, while ex situ involves the removal of the contaminated material to be treated elsewhere. Some examples of bioremediation-related technologies are  phytoremediation, bioleaching, landfarming, bioreactor, composting, bioaugmentation, rhizofiltration, and biostimulation.

Bioremediation may occur on its own (natural attenuation or intrinsic bioremediation) or may only effectively occur through the addition of fertilizers, oxygen, etc., that help encourage the growth of the pollution-eating microbes within the medium (biostimulation). For example, US Army Corps of Engineers demonstrated that windrowing and aeration of petroleum-contaminated soils enhanced bioremediation using the technique of landfarming. Depleted soil nitrogen status may encourage biodegradation of some nitrogenous organic chemicals, and soil materials with a high capacity to adsorb pollutants may slow down biodegradation owing to limited bioavailability of the chemicals to microbes.

Recent advancements have also proven successful via the addition of matched microbe strains to the medium to enhance the ability of resident microbe population to break down contaminants. Microorganisms used to perform the function of bioremediation are known as bioremediators.

However, not all contaminants are easily treated by bioremediation using microorganisms. For example, heavy metals such as cadmium and lead are not readily absorbed or captured by microorganisms. A recent experiment, however, suggests that fish bones have some success in absorbing lead from contaminated soil. Bone char has been shown to bioremediate small amounts of cadmium, copper, and zinc. The assimilation of metals such as mercury into the food chain may worsen the situation. Phytoremediation is useful in these circumstances because natural plants or transgenic plants are able to bioaccumulate these toxins in their above-ground parts, which are then harvested for removal.

Heavy metals in the collected biomass may be further concentrated by incineration or even recycled for industrial use. Some damaged artifacts in museums contain microbes which could be specified as bioremediating agents. In contrast to this situation, other contaminants, such as aromatic hydrocarbons, common in petroleum, are relatively simple targets for microbial degradation, and some soils may even have some capacity to autoremediate owing to the presence of autochthonous microbial communities capable of degrading these compounds.

The elimination of a wide range of pollutants and wastes from the environment requires better understanding of the relative importance of different pathways and regulatory networks for carbon flows in different environments and for different compounds, and it will certainly accelerate the development of bioremediation technologies and biotransformation processes.

Bioremediation largely relies on the enzymatic activities of living organisms, usually microbes, to catalyze the destruction of pollutants or their transformation into less harmful forms. It is a complex process depending on many factors including: ambient environmental conditions, composition of the microbial community,nature and amount of pollution.

Types of pollutants. Organic pollutants → catabolized,  naturally occurring, Xenobiotics-substances foreign to an entire biological system, i.e. artificial substances, which did not exist in nature before their synthesis by humans, metals from ore extraction and manufacturing (Table 4).

Fundamentals of cleanup reactions. Aerobic metabolism: Microbes use O2 in their metabolism to degrade contaminants.

Anaerobic metabolism: Microbes substitute another chemical for O2 to degrade contaminants: nitrate, iron, sulfate, carbon dioxide, uranium, technicium, and perchlorate.

Table 4. Types of pollutants

| Rather easily degradable | Degradable to some extent | Badly degradable | Generally recalcitran |
| --- | --- | --- | --- |
| fuel oils, gasoline ketones and alcohols, monocyclic aromatics, bicyclic aromatics (naphthalene) | creosote, coal tars, pentachloro-phenol (PCP) | chlorinated solvents, some pesticides and herbicides | dioxins polychlorinated biphenyls |

Cometabolism- Bacterium uses some other carbon and energy source to partially destruct contaminants (organic aromatic ring compounds).

Advantages: permanence, contaminant is destructed, potentially low cost, 60-90% less than other technologies. Cheaper in-situ vs. ex-situ remediation of contaminated soils: Cost of treating contaminated soil in situ is $80-$100 per ton. Cost of excavating and trucking the contaminated soil off for incineration is $400 per ton. Over 90% of the chemical substances classified as hazardous today can be biodegraded.

Bioaugmentation vs. biostimulation: Biostimulation involves modification of the environment by stimulation of existing microorganisms capable of bioremediation. Indigenous populations may not be capable of destructing the xenobiotics or a wide range of potential substrates present in complex pollutant mixtures. Bioaugmentation is the introduction of a group of natural microbial strains or a genetically engineered variant to treat contaminated soil or water. Many factors control biodegradability of a contaminant in the environment. Before attempting to employ bioremediation technology, one needs to conduct a thorough investigation of the environment where the contaminant exists, including microbiology, geochemistry, mineralogy, geophysics, and hydrology of the system.

Bioremediation refers to the use of microorganisms to destruct contaminants that pose environmental and especially human risks. Due to its safety and convenience, it has become an accepted remedy for cleaning polluted soil and water. Bioremediation processes typically involve many different microbes acting in parallel or in sequence to complete the degradation process. The ability of microbes to destruct a large number of pollutants makes bioremediation a technology that can be applied in different soil conditions.

A widely used approach to bioremediation involves stimulating a group of organisms in order to shift the microbial ecology toward the desired process. This approach is termed "Biostimulation." Biostimulation can be achieved through changes in pH, moisture, aeration, or nutrient additions. The other widely used approach is termed "Bioaugmentation" where organisms selected for high degradation abilities are used to inoculate the contaminated site. These two approaches are not mutually exclusive – they can be used simultaneously.

Bioremediation is accomplished either in situ or ex situ. In situ remediation efforts focus on treating the contaminant at the polluted site. Ex situ remediation refers to the treatment of contaminated soil at an off-site location. In such cases, soils from the contaminated site are transported to a place like a bioreactor, where conditions favorable for biological degradation can be controlled and enhanced.

Questions for self-control

- Can you analyze waste generation and classify it?

- Give a definition of Еcotoxicants and Xenobiotic types of the environment.

- Determine the main properties of waste that are environmentally hazardous.

- List the adverse properties of solid waste for treatment.

- Describe the types of transformation – abiotic transformation, species, their effect on transformation.

- Define the process of biotic transformation of organic pollutants in cleaning up the environment.

- Describe the process of transformation of naphthenic, naphthenic-aromatic and aromatic hydrocarbons.

- Give information about studying and isolation of microorganisms with hydrocarbon-oxidizing activity.

- Make an analysis of biological methods based on the use of different groups of microorganisms. Identify their advantages and disadvantages.

- Assess the value of Bioremediation, Bioaugmentation and Biostimulation in environmental biotechnology.

Chapter 5

Heavy metal pollution. The main absorber of heavy metals

5.1 Bioabsorption, active and passive biosorption

The basis of biosorption or extraction of metals from solutions with biosorbents is the processes of interaction with surface structures of cells, their metabolites and exo-polymers. Biosorbents are different biologically active cells, both living and dead - bacteria, algae, plants, fungi, etc.

Biosorbents can selectively accumulate on the surface of metal ions. They behave similar to physicochemical surfactants, such as ion exchange resins, natural zeolites and other hydro-apatites.

The sorption is divided into active and passive.

Active sorption occurs due to metabolic processes of inclusion of metals in the cell structure. At the same time ion-exchange rate is rather low.

Passive adsorption occurs on the surface of cells and is explained by physical-chemical interactions of metal ions with ion-exchange groups of cell membranes. The process is relatively fast – within a few hours. Ability of the sorbent to accumulate metals characterized by a quantity called the sorption capacity of the biomass.

The sorption capacity of biomass is the amount of adsorbed metal in mg or mmol per 1 g of biomass. Typically, this average value is used because the actual value of the capacitance can vary within wide limits. Some methods of biosorption are designed for use in industry.

Bacterial Zooglocaramigera exopolysaccharides are used for recovery of copper, cadmium and uranium from a solution containing salts of these metals. The sorption of each metal separately occurs at different pH - 3.5; 6.5 to 5.5 for U, Cd and C, respectively. The sorption time with stirring at 800 rev/min was 15 min. Sewage from the uranium used emulsan obtained through the culture Acinetobacter calcoaceticus RAG-1 (ATCC 31012).

Emulsan is an environmentally safe drug which does not contain phosphorus, is not toxic, is easily degradable and can be used in industrial wastewater purification schemes. Wastewater from the uranium and nitrates can also be treated using biomass denitrifying bacteria immobilized on the particles of coal. Over 8 minutes of contact time give up to 98% of adsorbed uranium and nitrates, by using denitrizing bacteria with formation of gaseous nitrogen.

Penicilliumchrysogenum micelle extracted from waste of antibiotics production treated by urea polycondensate, gives a solid product called metal biosorbent. This drug is very effective for wastewater treatment from radioactive elements.

The choice of natural biosorbents for rehabilitation of industrial solutions with complex multi-component composition is determined by the following requirements:

•  the ability of active sorption of heavy metals and radionuclides;

•  low cost and high consumption of the technology;

• the availability of accessible natural biosorbents near the industrial zone;

• possibility of making technological schemes with biosorbents directly on the production or cleaned areas;

• a minimum amount of waste of  biotechnological processes.

Multicomponent biological solutions were used as test solutions for interaction with biosorbents and bacterial leaching of ores and rocks via microbial community of peat slurry.

Studies have shown that the considered types of sorbents (seeds tea, green tea leaves, tea production waste, moss and microorganisms Aspergillus niger and Penicilliumnotatum) are able to extract heavy and radioactive metals from multicomponent solutions in varying degrees.

At the same time the sorption capacity of each of the sorbents in different conditions is different and depends on a number of factors, the main ones are:

- chemical composition of the multicomponent solution;

- concentration of metal in the solution;

- the content of the same metal in the sorbent;

- the degree of activity of microorganisms and their metabolites are present in the solution;

- duration of contact between solution and sorbent.

5.2 The sources of soil contamination by heavy metals

Heavy metal pollution has different sources:

1. Waste of the metalworking industry;

2. Industrial emissions;

3. Products of combustion of fuel;

4. Automobile exhausts of gases;

5. Chemicalization of agriculture.

Heavy metals are a fuzzy group of inorganic chemical hazards including lead (Pb), chromium (Cr), arsenic (As), zinc (Zn), cadmium (Cd), copper (Cu), mercury ( Hg) and nickel (Ni). As defined by the World Health Organization (WHO), lead, mercury and cadmium are the most dangerous heavy metals, representing the "terrible trinity" in the natural environment. In recent years, a high toxicity of beryllium, arsenic, selenium, antimony, thallium, nickel, tin, vanadium, which are biologically active, has been registered. In accordance with GOST toxic chemical elements are divided into classes of hygiene hazard. According to the soils, they are:

Class I: arsenic (As), beryllium (Be), mercury (Hg), selenium (Sn), cadmium (Cd), lead (Pb), zinc (Zn), fluorine (F);

Class II: chromium (Cr), cobalt (Co), boron (B), molybdenum (Mn), nickel (Ni), copper (Cu), antimony (Sb);

Class III: barium (Ba), vanadium (V), tungsten (W), manganese (Mn), strontium (Sr).

Lead is a metal belonging to group IV and period 6 of the periodic table with atomic number 82, atomic weight 207.2, density 11.4 g cm-3, melting point 327.4 ° C and boiling point 1725 ° C. This is a natural, bluish gray metal, usually found as a mineral in combination with other elements such as sulfur (i.e., PbS, PbSO4) or oxygen (PbCO3), and ranges from 10 to 30 mg/kg-1 in the crust. The typical average Pb concentration in surface soils worldwide is on average 32 mg/kg-1 and ranges from 10 to 67 mg kg-1. In the industrial production of metals it occupies the fifth place behind Fe, Cu, Al and Zn. Approximately half of the Pb is used in the US for production of Pb batteries.

In Kazakhstan, to date, there is no reliable data on the extent of the spread of lead pollution and its impact on public health. The main sources of pollution:

- the main sources of lead in the environment and its impact on the health of the population are emissions of industrial enterprises, the use of leaded gasoline, the use of lead-containing solders in the canning industry, lead-based paints, the use of lead materials in water supply systems;

- the main source of lead in the human body is soil (dust and food chains) and atmospheric air;

- a direct indicator of lead intoxication is the level of lead in blood;

- exposure to lead is particularly damaging to a child's body, which is much more sensitive than an adult to the toxic effects of xenobiotics, including lead. It is shown that even low levels of lead in children’s organism lead to a significant reduction in mental development.

With exhaust gases, more than 250,000 tons of lead per year are deposited on the soil surface; this is the main pollutant of soils with lead.

Heavy metals fall into the soil together with pesticides and fertilizers, which they enter as an admixture and with biocides.

The source of pollution generally determines the quality and quantity of the product emitted. In this case, the degree of its dispersion depends on the height of ejection. The zone of maximum contamination extends over a distance equal to 10-40 times the pipe height with high and hot emission, 5-20 times the pipe height with low industrial emission. The duration of presence of the emitted particles in the atmosphere depends on their mass and physical and chemical properties. The heavier the particles, the faster they settle.

5.3 Main absorber of heavy metals

Pollution by heavy metals is associated with their widespread use in industrial production. Because of imperfect systems of cleaning, heavy metals fall into the environment, including soil, contaminating and poisoning it. Heavy metals are classified as special pollutants, observations of which are mandatory in all environments.

The soil is exposed to pollutants coming from the atmosphere, with surface runoff, from subsoil and groundwater. This is especially true of the humus horizon of soils.

Soils are the main absorber of heavy metals released into the environment by the above-mentioned anthropogenic activity, and unlike organic contaminants that are oxidized to carbon monoxide (IV) by microbial action, most metals are not subjected to microbiological or chemical degradation, and their total concentration in soils persists for a long time after their introduction.

However, changes in their chemical forms (speciation) and bioavailability are possible. The presence of toxic metals in the soil can seriously inhibit biodegradation of organic contaminants. Soil contamination with heavy metals can be dangerous for humans and ecosystems by direct intake or contact with contaminated soil, food chain (soil-plant-man or soil-plant-animal-man), use of contaminated groundwater, reduced food quality (safety and competitiveness) due to phytotoxicity, reduction of land use for agricultural production causing food shortages and land use problems.

Increasing attention to environmental protection has caused particular interest in the impact of heavy metals on the soil.

From a historical point of view of interest to this problem came with the study of soil fertility, since elements such as iron, manganese, copper, zinc, molybdenum and, optionally, cobalt, are very important for the life of plants and, hence, to animals and humans.

They are also known by the name of trace elements, because plants need them in small quantities. To the group of trace elements also refer metals in the soil, the concentration of which is rather high, for example, iron, which is included in most soils and is ranked fourth in the composition of the crust (5%) after oxygen (46.6%), silica (27.7 %) and aluminum (8.1%).

All trace elements can have a negative impact on plants if the concentration of their available forms exceeds certain limits. Some heavy metals, such as mercury, lead and cadmium, which are apparently not very important for plants and animals, are dangerous for human health, even at low concentrations.

5.4 Accumulation of heavy metals by microorganisms

Heavy metals accumulated by water-living and soil-living microorganisms, and by plants get into food of the domestic animals and by the natural food chain into the human body. The concentration of heavy metals in the dissolved state in the wastewater depends on the temperature of water, the total salt content, the presence of inorganic and organic compounds and pH. Heavy metal ions in the wastewater often form complexes with organic substances, especially, humic compounds in alkaline and neutral media.

In the fermenters supporting anaerobic conditions, already formed biocoenosis helps to restore chromium Cr6 + into chromium Cr3 +, which in the secondary settling tank precipitates out as Cr(OH)3 together with the active sludge.

To create sulphate-reducing bacteria in the bioreactor it is necessary to introduce the organic substrate. Restoration of chromium Cr6 + to chromium Cr3 + is mostly carried out by microorganisms Pseudomonas. If the wastewater contains sulphate ions, biocenosis is formed that includes species Desulfovibrio and Desulfotomaculum.

These types of bacteria in anaerobic conditions restored sulphates to hydrogen sulphide, which is chemically reacting with heavy metal ions and forming insoluble sulphides, which are precipitated in the secondary settling tanks. Some microorganisms and algae can accumulate metals, which they need for functioning of their enzymes. Algae have special transport systems, which are responsible for penetration of metals, both in cationic and anionic forms, into the cell.

In natural ponds and bioponds purification of water from heavy metals carried out by tallness amphibians’ larvae. Plankton’s microorganisms can accumulate copper compounds in their biomass, in concentrations 9000 times higher than the concentration in water, as well, they can accumulate compounds of lead in their biomasses in concentrations 12,000 times higher than in water, and cobalt – 16,000 times higher than in water.

High accumulative ability of microalgae in relation to heavy metals makes prospects for their use in wastewater treatment. The current experience in biotechnology demonstrates that the efficiency of accumulation reaches 95%.

Biological cleaning methods can be viewed as an alternative way, instead of using chemical reagents. Sulphate-reducing bacteria (SRB) attracted the attention of researchers as potential agents for cleaning wastewater that contains both components: heavy metals and sulphates.

Sulfate-reducing bacteria are bacteria that can obtain energy by oxidizing organic compounds or molecular hydrogen (H2) while reducing sulfate (SO2−4) to hydrogen sulfide (H2S). In a sense, these organisms "breathe" sulfate rather than oxygen, in a form of anaerobic respiration.

Sulfate-reducing bacteria can be traced back to 3.5 billion years and are considered to be among the oldest forms of microorganisms, having contributed to the sulfur cycle soon after life emerged on Earth.

Many bacteria reduce small amounts of sulfates in order to synthesize sulfur-containing cell components; this is known as assimilatory sulfate reduction. By contrast, the sulfate-reducing bacteria considered here reduce sulfate in large amounts to obtain energy and expel the resulting sulfide as waste; this is known as dissimilatory sulfate reduction. They use sulfate as the terminal electron acceptor of their electron transport chain. Most of them are anaerobes.

Most sulfate-reducing bacteria can also reduce other oxidized inorganic sulfur compounds, such as sulfite, thiosulfate, or elemental sulfur (which is reduced to sulfide as hydrogen sulfide). In addition, there are sulfate-reducing bacteria that can reduce fumarate, nitrate and nitrite, iron (Fe (III)) and some other metals, dimethyl sulfoxide and even oxygen.

Sulfates occur widely in seawater, sediment, or water rich in decaying organic material. Sulfate-reducing bacteria are common in anaerobic environments where they aid in the degradation of organic materials. In these anaerobic environments, fermenting bacteria extract energy from large organic molecules; the resulting smaller compounds such as organic acids and alcohols are further oxidized by acetogens and methanogens and the competing sulfate-reducing bacteria.

In hydrofracturing fluids used to frack shale formations to recover methane (shale gas), biocide compounds are often added to water to inhibit the microbial activity of sulfate-reducing bacteria in order to avoid anaerobic methane oxidation and to minimize potential production loss.

Taking into account that in the wastewater sulphates alkaline and alkaline earth metals are always present, this technology is perspective for textile and wood-working industries.

Questions for self-control

- What is bioabsorption, active and passive biosorption?

- Explain the use of sulfate-reducing bacteria in deposition of metals. Describe conditions of their functioning.

- What is the accumulative ability of microorganisms to heavy metals?

- What is sorption capacity of biomass?

- Name the requirements to natural biosorbents.

- What are the main sources of soil contamination by heavy metals?

- Describe the main absorbers of heavy metals.

- Describe characteristics of heavy metals, their physical and chemical features. The negative effect of heavy metals mainly depends on their mobility, i.e. solubility.

- Assess the value of Remediation of Heavy Metal-Contaminated Soils. Describe the process of Vitrification, Phytoremediation, Phytoextraction (Phytoaccumulation).

- Estimate the accumulative ability of microorganisms to heavy metals.

Chapter 6

WATER RESOURCES. The basic characteristics of sewage, types of water flows, their structure and criteria of quality assessment

6.1 Water as the most important natural resource

Water is the most valuable natural resource. It plays a crucial role in the metabolic processes that form the basis of life. Water has great importance in industrial and agricultural production. It is well known that it is needed for domestic human needs as well as for all plants and animals. For many living creatures it serves habitat. The growth of cities, rapid development of industry, agricultural intensification, a significant expansion of irrigated land, improved cultural conditions and other factors complicate the problems of water supply.

Water demands are enormous and annually increasing. Annual consumption of water on the globe for all types of water is 3300-3500 km3. Thus, 70% of the total water consumption is used in agriculture. A lot of water is used by chemical and pulp paper industry, ferrous and nonferrous metallurgy. Energy development also leads to a sharp increase in the demand for water. Significant quantities of water are spent on the needs of the livestock industry, as well as for domestic needs of the population. Most of the water after using it for domestic needs is returned to rivers as wastewater.

Deficiency of fresh water has already become a world problem. Ever-growing requirements of industry and agriculture mean that it is necessary to solve this problem.

At the present time, the problem of rational use of water resources is solved by better utilization and expanded reproduction of freshwater resources; development of new technological processes preventing water pollution and minimizing consumption of fresh water.

Growth of population, expansion of old and new cities has significantly increased the flow of domestic sewage into inland waters. These drains have become a source of pollution of rivers and lakes by pathogenic bacteria and worms. These drains contain even more polluting waters than the washing synthetic agents that are widely used in everyday life. They are widely used in industry and agriculture. They contain chemical substances, discharged with the wastewater into rivers and lakes; have a significant impact on the biological and physical treatment of ponds. This reduces the ability of water to oxygen saturation, paralyze the bacteria and mineralize organic substances.

6.2 Sewage treatment

Sewage treatment generally involves three stages, called primary, secondary and tertiary treatment (Figure 10).

Primary treatment consists of temporarily holding the sewage in a quiescent basin where heavy solids can settle to the bottom while oil, grease and lighter solids float to the surface. The settled and floating materials are removed and the remaining liquid may be discharged or subjected to secondary treatment.

Figure 10.Sewage treatment process

Secondary treatment removes dissolved and suspended biological matter. Secondary treatment is typically performed by indigenous, water-borne micro-organisms in a managed habitat. Secondary treatment may require a separation process to remove micro-organisms from the treated water prior to discharge or tertiary treatment.

Tertiary treatment is sometimes defined as anything more than primary and secondary treatment in order to allow rejection into a highly sensitive or fragile ecosystem (estuaries, low-flow rivers, coral reefs). Treated water is sometimes disinfected chemically or physically (for example, by lagoons and microfiltration) prior to discharge into a stream, river, bay, lagoon or wetland, or it can be used for the irrigation of a golf course, green way or park. If it is sufficiently clean, it can also be used for groundwater recharge or agricultural purposes.

Biological systems include biological filters and rotating biological contactors. These systems are effective unit processes in treating wastewater. However, trickling filters and RBCs are temperature sensitive, remove less BOD, and trickling filters cost more to build than activated sludge systems. Although they are more expensive to build, the activated sludge systems are much more expensive to operate because of the need for energy to run pumps and blowers.

Activated sludge refers to biological treatment processes that use suspended growth of organisms to remove BOD and suspended solids. As shown below, the process requires an aeration tank and a settling tank (Figure 11).

Figure 11.Activated Sludge Processing.

In addition, support equipment, including return pumps, waste pumps, flow measurement devices for return and waste, as well as equipment to provide aeration (mixers and/or blowers) is also required (Figure 12).

Figure 12.Treatment of sewages in an aeration tank of the Shymkent system of wastewater treatment

The excess solids and organisms are periodically removed from the system (waste activated sludge). Failure to remove waste solids will result in poor performance and loss of solids out of the system over the settling tank effluent weir (Figure 13).

Figure 13.Activated sludge in wastewater treatment

The activated sludge in the Shymkent system of waste water treatment is shown in Figure 14.

Figure 14. Activated sludge in the Shymkent system of wastewater treatment

Living organisms together with a solid carrier form zoogloeas – symbiosis of microorganism populations covered by a common mucosa. Microorganisms isolated from activated sludge belong to different genera: Actynomyces, Azotobacter, Bacillus, Bacterium, Corynebacterium, Desulfomonas, Pseudomonas, Sarcina. The most abundant bacteria are of the genus Pseudomonas.

A significant role in the formation and operation of activated sludge belongs to the simplest microorganisms (Figure 15). They have a simple but diverse function; they are not directly involved in the consumption of organic matter but regulate the age and composition of species of microorganisms in the activated sludge keeping them at a certain level. Absorbing a large number of bacteria protozoa contribute to the output of bacterial exoenzymes concentrated in the mucosa and thereby participate in the degradation of contaminants. In active muds there are representatives of four classes of simple microorganisms: sarkodovyh (Sarcodina), flagellate ciliates (Mastigophora), ciliated infusoria species (Ciliata), sucking ciliates (Suctoria).

Figure 15. The simplest types of microorganisms of activated sludge

The quality indicator of the activated sludge is a protozoal coefficient, which reflects the ratio of cell protozoa to the number of bacterial cells. In the high-quality sludge 1 million bacterial cells should fall per 10-15 cells of protozoa. If you change the composition of the wastewater, the number of one of the species of microorganisms can increase, but other cultures still remain in the ecological community.

The formation of the activated sludge cenoses depends on the seasonal fluctuations in temperature, oxygen supply, the presence of mineral components. All this makes the structure complicated and irreproducible. The effectiveness of the treatment facilities depends on the concentration of microorganisms in the wastewater and the age of the activated sludge.

Sludge moves for natural drying on the sludge platform after squeezing out (Figure 16).

Figure 16. Drying of activated sludge in the Shymkent system of wastewater treatment

6.3 Biofiltration, biofilters and problems of their work.

1. Biofilter is a structure in which the waste water is filtered through the feed material coated with a biofilm formed by colonies of microorganisms.

- Mechanical  filter

- Aerator

- Biofilter element

- Section for plants

Figure 17.Scheme of a biofilter

The biofilter consists of the following main parts:

a) filter media (filter body) from the slag, gravel, expanded clay, gravel, plastic, asbestos, usually placed in a tank of water with permeable or impermeable walls;

b) water distribution device that provides a uniform short intervals for surface load of irrigation wastewater biofilter;

c) drainage device for removing the filtered water;

d) air distribution device, through which air required for air oxidation process flows.

Oxidation processes proceed in the biofilter, similar processes proceed in other biological treatment plants, and especially in the fields of irrigation and filtration fields. However, in the biofilter these processes proceed much more intensively.

.

Figure 18. General system of the biofilter

Passing through the biofilter the contaminated water leaves therein undissolved impurities not settled in the primary settling tanks, as well as colloidal and dissolved organic substances sorbed by biological membrane. Densely populating the biofilm microorganisms oxidize organic matter and hence derive the energy needed for their lives. Some organic substances such as microorganisms use plastic material to increase their mass. Thus, waste water is removed from the organic matter and at the same time increases the mass of active biofilm within the body of the biofilter. The waste and dead membrane is washed off with water and sewage flowing is removed from the body of the biofilter. Oxygen required for biochemical process enters the body from the air through the natural load and ventilation filter.

Biofilters are classified according to various criteria:

1. Purity - biofilters operating at full and part-biological treatment;

2. The way of air supply - biofilters with natural and artificial air supply;

3. Operating mode - biofilters operating with recirculation.

A biofilm is an aggregate of microorganisms in which cells adhere to each other on the surface. These adherent cells are frequently embedded within a self-produced matrix of an extracellular polymeric substance (EPS).

Microbes form a biofilm in response to many factors, which may include cellular recognition of specific or non-specific attachment sites on a surface, nutritional cues, or in some cases, by exposure of planktonic cells to sub-inhibitory concentrations of antibiotics. When a cell switches to the biofilm mode of growth, it undergoes a phenotypic shift in its behavior in which large suites of genes are differentially regulated (Stages of biofilm development).

Figure 19. Five stages of biofilm development: (1) Initial attachment, (2) Irreversible attachment, (3) Maturation I, (4) Maturation II, and (5) Dispersion. Each stage of development in the diagram is paired with a photomicrograph of a developing P. aeruginosa biofilm. All photomicrographs are shown at the same scale.

Biofilms are ubiquitous. Nearly every species of microorganisms, not only bacteria and archaea, have mechanisms by which they can adhere to surfaces and to each other. Biofilms will form on virtually every non-shedding surface in a non-sterile aqueous (or very humid) environment.

Biofilms can be found on rocks and pebbles at the bottom of most streams or rivers and often form on the surface of stagnant pools of water. In fact, biofilms are important components of food chains in rivers and streams and are grazed by the aquatic invertebrates upon which a lot of fish feed.

6.4 Purification of industrial wastewater by Pseudomonas putida

In the activated sludge produced during purification of industrial wastewater, the species composition of microorganisms does not change so much, despite extremely large variety of oxidizable contaminants. In most parts of the sludge the dominant microbiota is microbiota of genus Pseudomonas.

Pseudomonas putida is a gram-negative rod-shaped saprotrophic soil bacterium. Based on 16S rRNA analysis, P. putida has been placed in the P. putida group, to which it lends its name.

It is the first patented organism in the world. Because it is a living organism the patent was disputed and brought before the United States Supreme Court in the historic court case won. It demonstrates a very diverse metabolism, including the ability to degrade organic solvents such as toluene. This ability has been put to use in bioremediation, or the use of microorganisms to biodegrade oil. Use of P. putida is preferable to some other Pseudomonas species capable of such degradation as it is a safe species of bacteria, unlike P. aeruginosa, for example, which is an opportunistic human pathogen.

The diverse metabolism of P. putida may be exploited for bioremediation; for example, it is used as a soil inoculant to remedy naphthalene contaminated soils.

P. putida is capable of converting styrene oil into the biodegradable plastic. This may be of use in the effective recycling of Polystyrene foam, otherwise thought to be non-biodegradable.

The Pseudomonas putida KT2440 chromosome is characterized by strand symmetry and intra-strand parity of complementary oligonucleotides. Each tetranucleotide occurs with similar frequency on the two strands. Tetranucleotide usage is biased by G+C content and physicochemical constraints such as base stacking energy, dinucleotide propeller twist angle or trinucleotide bendability. The 105 regions with atypical oligonucleotide composition can be differentiated by their patterns of oligonucleotide usage into categories of horizontally acquired gene islands, multidomain genes or ancient regions such as genes for ribosomal proteins and RNAs. A species-specific extragenic palindromic sequence is the most common repeat in the genome that can be exploited for the typing of P. putida strains. In the coding sequence of P. putida LLL is the most abundant tripeptide.

As a result of long targeted selection of microorganisms grown up on a single substance serving as their sole carbon source, such cultures can be prepared that will absorb this substance even at high concentrations. These cultures can be successfully used in the purification of waste waters polluted by a single agent, such as phenol; in most cases advisable to use the biocoenosis of microorganisms (activated sludge).

In the biocenosis systems, types of microorganisms are selected in the process of long work of a certain biooxidant on the wastewater under a certain composition. Changing the quality of the cleaning water and its concentration necessitates the adaptation of microorganisms. Their ability to adapt is very important for biological treatment of industrial wastewater.

The purification process is more stable and complete in those cases where there is a mixture of purified industrial and household wastewater. This is explained by the fact that household water contains the necessary biogenic elements and also serves for dilution. For a quick inoculation of microorganisms, treatment facilities often add household water to industrial waters, especially in the start-up period.

After "maturation" of clearing constructions when microorganisms adapt to the utilization of specific contaminants of industrial water, the household water’s inflow can be reduced or stopped altogether.

6.5 Sum parameters in waste water analysis

About 40 million organic compounds known in the environment which cannot be defined individually without considerable analytical effort and in short time. Therefore, the so-called sum parameters are used. These parameters reflect the effect and material characteristics of one or more substances. The most popular sum parameters in waste water analysis are the BOD (biochemical oxygen demand), COD (chemical oxygen demand), TOD (total oxygen demand) and TOC (total organic carbon). The TOC reflects the organic pollution on the basis of a direct carbon determination. The other parameters are based on oxygen, which is required to reduce or to oxidise the samples of substances.

Formation of sum parameters

The BOD (biological or biochemical oxygen demand) can be considered as the 'mother' of the sum parameters. It had been already found and defined in the 19th century, apparently as a consequence of construction of the sewerage system in the big cities. The sewerage system had provided definite advantages for hygiene. Waste water and refuse had started to be taken underground by channels. However, some waste water had a high oxygen demand and led to an oxygen deficiency in water bodies, into which the waste water was discharged. Hence, the oxygen content of these waters decreased to zero. A higher fish mortality was the consequence. Since then the engagement with biological and biochemical oxygen demand has intensified. In the following years, further parameters have been defined.

BOD, biochemical oxygen demand

The BOD indicates the content of oxygen needed to decompose organic compounds in waste water by bacteria. In most cases the special factor BOD5 is perceived as the BOD, which requires a detailed definition (5 represents the 5-day analysis time). For the determination of BOD5 there are nitrification inhibitors added to the samples, which suppress the degradation of nitrogen compounds. Consequently, it results in the determination of the decomposition of carbon compounds only (carbonaceous BOD, cBOD). Due to this limitation, an essential process of sewage treatment is not considered: The nitrification. Obviously, a WWTP can only be controlled and monitored reliably by total BOD measurements, instead of the determination of the insufficient BOD5. In the real sense, BOD measurements are respiration measurements. Due to their rapidity, respiration measurements are preferred for online analysis. Provided that the conditions are known, respiration measurements [mg/(l*min)] can be converted into BOD measurements [mg/l].

Due to the 5-day analysis time and the measurement of the carbonaceous BOD instead of the total BOD, the BOD5 is not suitable to assess the current capacity of the waste water degradation by bacteria. The BOD5 is only time-delayed information about the pollution of waste water and cannot be used for optimization or control of a WWTP. Alternatively, BOD analyzers or respiration analyzers can be used, which enable measurements within 5 to 60 minutes. Thus, estimations can be made promptly for the biodegradability of the waste water and its behavior in the plants.

COD, chemical oxygen demand

The COD value has been developed analogically to the BOD measurement. Since there are many organics which are rather hard or not possible to decompose biologically, a parameter has been defined indicating the amount of oxygen which would be needed when all organic ingredients would be oxidised completely. As, according to the name, the oxidation takes place chemically, the chemical oxygen demand can only be defined indirectly. A chemical oxidant is added to the sample in question, the consumption of which is then determined. The internationally dominant method today is the so-called 'Dichromate' method which is characterized by the acidification of the sample with sulphuric acid and the addition of silver sulphate. To avoid false measurements in chloride-containing samples, the chloride must be masked by mercuric sulphate first. Due to the application of hazardous chemicals and having an analysis time of 2 hours the method is not suitable for online use.

TOD, total oxygen demand

According to its name, this parameter defines the total oxygen demand of the water sample. It is based on the same principle as the COD. Correspondingly, this parameter has existed already for over 40 years. However, it fell into oblivion in Germany. In contrast, the parameter TOD has been standardized as a reference parameter for the assessment of organic pollution in water in the US.

To define the TOD the sample is thermally oxidised in an oven and the emerging oxygen is measured directly in the carrier gas by a NDIR detector. The TOD value can be affected by inorganic compounds. However, these influences are generally very small and thus enables a mathematically compensation. The parameter is particularly suited for the correlation with COD and BOD, since inorganic carbons do not affect the correlation and the non-carbonaceous BOD (nitrogen compounds) are also considered. Due to the similarities of the determination of the TOC, the TOD is well suited for online monitoring.

TOC, total organic carbon

The content of TOC in water also reflects the organic contamination. As the name of the parameter already suggests, it is supposed to and has to detect the total organic carbon of the sample. Therefore, the inorganic carbon, literally carbon dioxide dissolved in water and its dissolved ions, have to be excluded from the sample.

Generally, the determination of TOC is done by thermal or wet chemical oxidation, so that CO2 is formed, which is subsequently measured by a NDIR detector. TOC measurements are well suited for online measurements since its provides fast and meaningful results depending on the function of the process analyser. The TOC takes a special position in well-known regulatories.

Biochemical oxygen demand (BOD) and chemical oxygen demand (COD) are two different ways used to measure how much oxygen the water will consume when it enters the recipient. In both cases the oxygen-consuming substances are mainly of organic origin. These substances should be reduced to a minimum in the wastewater treatment plant. Industries normally focus more on COD and municipalities on BOD removal.

With chemical treatment, removal of COD is improved at basically all kinds of wastewater plants. The more particle-bound COD, the more efficient the removal rate. Our products can also separate some dissolved substances, but when the COD consists of small organic molecules a biological treatment process is normally preferred.

If our products are used before the secondary treatment, the biological process will work more efficiently and consume less energy. Or if needed, the plant capacity can be increased without any major investments. A wastewater treatment plant using chemicals to support BOD/COD removal will always be the most compact plant and leave the smallest possible environmental footprint.

Questions for self-control

1. What is the role of water for living organisms?

2. What natural phenomena and factors of human activity can affect the quantity and quality of water?

3. What is the volume of annual water consumption in the world?

4. What kind of human activity consumes water?

5. What is the waste water?

6. What are the classes of sewage?

7. How could waste water be recycled?

8. What is the difference of industrial wastewater from household?

9. What is the treatment of wastewater?

10. What is activated sludge and biological treatment processes?

11. How many stages does cleaning of waste water include?

12. What is the role of Pseudomonas putida in purification of industrial wastewater?

13. What total parameters are used in the analysis of sewages?

14. How is biochemical oxygen demand (BOD) defined?

15. How is chemical oxygen demand (COD) defined?

Chapter 7

Anaerobic digestion of wastes

7.1 Microorganisms involved in the process of anaerobic digestion

The key process of anaerobic digestion

Anaerobic digestion facilities have been recognized as one of the most useful decentralized sources of energy supply, as they are less capital-intensive than large power plants. With increased focus on climate change mitigation, the re-use of waste as a resource and new technological approaches which have lowered capital costs, in recent years anaerobic digestion has received increased attention among governments of a number of countries, among these the United Kingdom (2011), Germany and Denmark (2013).

Anaerobic digestion is a series of processes in which microorganisms break down biodegradable material in the absence of oxygen. It is used for industrial or domestic purposes to manage waste and/or to release energy. Much of the fermentation used industrially to produce food and drink products, as well as home fermentation, uses anaerobic digestion. Silage is also produced by anaerobic digestion.

The digestion process begins with bacterial hydrolysis of the input materials to break down insoluble organic polymers, such as carbohydrates, and make them available for other bacteria. Acidogenic bacteria then convert the sugars and amino acids into carbon dioxide, hydrogen, ammonia, and organic acids. Acetogenic bacteria then convert these resulting organic acids into acetic acid, along with additional ammonia, hydrogen, and carbon dioxide. Finally, methanogens convert these products to methane and carbon dioxide. The methanogenicarchaea populations play an indispensable role in anaerobic wastewater treatments. It is used as part of the process to treat biodegradable waste and sewage sludge. As part of an integrated waste management system, anaerobic digestion reduces the emission of landfill gas into the atmosphere. Anaerobic digesters can also be fed with purpose-grown energy crops, such as maize.

Anaerobic digestion is widely used as a source of renewable energy. The process produces a biogas, consisting of methane, carbon dioxide and traces of other ‘contaminant’ gases. This biogas can be used directly as cooking fuel, in combined heat and power gas engines or upgraded to natural gas-quality biomethane. The use of biogas as a fuel helps to replace fossil fuels. The nutrient-rich digestate produced can also be used as fertilizer.

Scientific interest in the manufacturing of gas produced by the natural decomposition of organic matter was first reported in the 17-th century by Robert Boyle and Stephen Hale, who noted that flammable gas was released by disturbing the sediment of streams and lakes. In 1808, Sir Humphry Davy determined that methane was present in the gases produced by cattle manure. The first anaerobic digester was built by a leper colony in Bombay, India, in 1859. In 1895, the technology was developed in Exeter, England, where a septic tank was used to generate gas for the sewer gas destructor lamp, a type of gas lighting. Also in England, in 1904, the first dual-purpose tank for both sedimentation and sludge treatment was installed in Hampton. In 1907, in Germany, a patent was issued for the Imhoff tank, an early form of digester.

Through scientific research, anaerobic digestion gained academic recognition in the 1930s. This research led to the discovery of anaerobic bacteria, the microorganisms that facilitate the process. Further research was carried out to investigate the conditions under which methanogenic bacteria were able to grow and reproduce. This work was developed during World War II, during which in both Germany and France, there was an increase in the application of anaerobic digestion for the treatment of manure.

Many microorganisms are involved in the process of anaerobic digestion, including acetic acid-forming bacteria (acetogens) and methane-forming archaea (methanogens). These organisms feed upon the initial feedstock, which undergoes a number of different processes, converting it to intermediate molecules, including sugars, hydrogen, and acetic acid, before finally being converted to biogas.

Different species of bacteria are able to survive in different temperature ranges. The ones living optimally at temperatures between 35 and 40 °C are called mesophiles or mesophilic bacteria. Some of the bacteria can survive at the hotter and more hostile conditions of 55 to 60 °C; these are called thermophiles or thermophilic bacteria. Methanogens come from the domain of archaea. This family includes species that can grow in the hostile conditions of hydrothermal vents, so are more resistant to heat, and can, therefore, exist at high temperatures, a unique property for thermophiles.

As for aerobic systems, the bacteria, growing and reproducing microorganisms within anaerobic systems, require a source of elemental oxygen to survive, but in anaerobic systems, there is no gaseous oxygen. Gaseous oxygen is prevented from entering the system through physical containment in sealed tanks. Anaerobes obtain oxygen from sources other than the surrounding air, which can be an organic material itself or may be supplied by inorganic oxides within the input material. When the oxygen source in an anaerobic system is derived from the organic material itself, the 'intermediate' end products are primarily alcohols, aldehydes, and organic acids, plus carbon dioxide. In the presence of specialised methanogens, the intermediates are converted to the 'final' end products of methane, carbon dioxide, and trace levels of hydrogen sulfide. In an anaerobic system, the majority of the chemical energy contained within the starting material is released by methanogenic bacteria as methane.

Populations of anaerobic microorganisms typically need a large amount of time to make themselves fully effective. Therefore, common practice is to introduce anaerobic microorganisms from materials with existing populations, a process known as "seeding" the digesters, typically accomplished with the addition of sewage sludge or cattle slurry.

The key process stages of anaerobic digestion are shown in Figure 20.

Figure 20 . The key process stages of anaerobic digestion.

7.2  Septic. Potential problems

A septic tank is a key component of the septic system, a small-scale sewage treatment system common in areas with no connection to main sewage pipes provided by local governments or private corporations (Other components, typically mandated and/or restricted by local governments, optionally include pumps, alarms, sand filters, and clarified liquid effluent disposal means such as a septic drain field, ponds, natural stone fiber filter plants or peat moss beds.). Septic systems are a type of On-Site Sewage Facility (OSSF). In North America, approximately 25% of the population relies on septic tanks; this can include suburbs and small towns as well as rural areas (Indianapolis is an example of a large city where many of the city's neighborhoods are still on separate septic systems). In Europe, they are  generally limited to rural areas only. Since a septic system requires a drainfield that uses a lot of land area, therefore, they are not suitable for densely built cities.

The term "septic" refers to the anaerobic bacterial environment that develops in the tank which decomposes or mineralizes the waste discharged into the tank. Septic tanks can be coupled with other on-site wastewater treatment units such as biofilters or aerobic systems involving artificial forced aeration.

A septic tank generally consists of a tank (or sometimes more than one tank) of between 4000 and 7500 liters (1,000 and 2,000 gallons) in size connected to an inlet wastewater pipe at one end and a septic drain field at the other. In general, these pipe connections are made via a T pipe, which allows liquid entry and exit without disturbing any crust on the surface. Today, the design of the tank usually incorporates two chambers (each of which is equipped with a manhole cover), which are separated by means of a dividing wall that has openings located about midway between the floor and roof of the tank.

Waste water enters the first chamber of the tank, allowing solids to settle and scum to float. The settled solids are anaerobically digested, reducing the volume of solids. The liquid component flows through the dividing wall into the second chamber, where further settlement takes place, with the excess liquid then draining in a relatively clear condition from the outlet into the leach field, also referred to as a drain field or seepage field, depending upon locality. A Percolation Test is required to establish the porosity of the local soil conditions for the drain field design.

The remaining impurities are trapped and eliminated in the soil, with the excess water eliminated through percolation into the soil (eventually returning to the groundwater), through evaporation, and by uptake through the root system of plants and eventual transpiration. A piping network, often laid in a stone-filled trench (see weeping tile), distributes the wastewater throughout the field with multiple drainage holes in the network. The size of the leach field is proportional to the volume of wastewater and inversely proportional to the porosity of the drainage field. The entire septic system can operate by gravity alone or, where topographic considerations require, with inclusion of a lift pump.

Tank is a two-stage septic system where the sludge is digested in a separate tank. This avoids mixing of digested sludge with incoming sewage. Also, some septic tank designs have a second stage where the effluent from the anaerobic first stage is aerated before it drains into the seepage field.

Waste that is not decomposed by the anaerobic digestion eventually has to be removed from the septic tank, or else the septic tank fills up and undercomposed wastewater discharges directly to the drainage field. This is not only  bad for the environment but, if the sludge overflows the septic tank into the leach field, it may clog the leach field piping or decrease the soil porosity itself, requiring expensive repairs.

How often the septic tank has to be emptied depends on the volume of the tank relative to the input of solids, the amount of indigestible solids, and the ambient temperature (as anaerobic digestion occurs more efficiently at higher temperatures). Anaerobic decomposition is rapidly re-started when the tank re-fills.

A properly designed and normally operating septic system is odor-free and, besides periodic inspection and pumping of the septic tank, should last for decades with no maintenance (Picture 8).

A well-designed and maintained concrete, fiberglass or plastic tank should last for about 50 years.

Periodic preventive maintenance is required to remove the irreducible solids that settle and gradually fill the tank, reducing its efficiency. In most jurisdictions this maintenance is required by law, yet often not enforced. Those who ignore the requirement will eventually be faced with extremely costly repairs when solids escape the tank and destroy the clarified liquid effluent disposal means. A properly maintained system, on the other hand, can last for decades or possibly even a lifetime.

Figure 21.  Septic tank scheme

The sewer line from the house to the septic tank may be plastic sewer pipe with glued joints or cast iron with stainless steel clamps or leaded joints. The joints must be glued so they are watertight and resist root penetration. (picture 22).

Figure 22.  Deep septic tank installation. Suitable for use where a high water table is not present.

Potential problems of the septic tank

Excessive dumping of cooking oils and grease can cause the inlet drains to block. Oils and grease are often difficult to degrade and can cause odor problems and difficulties with the periodic emptying.

Flushing non-biodegradable items such as cigarette butts and hygiene products such as sanitary napkins, tampons, and cotton buds/swabs will rapidly fill or clog a septic tank; these materials should not be disposed off in this way.

The use of garbage disposals for disposal of waste food can cause a rapid overload of the system and early failure.

Certain chemicals may damage the components of a septic tank, especially pesticides, herbicides, materials with high concentrations of bleach or caustic soda (lye) or any other inorganic materials such as paints or solvents.

Other chemicals can destroy septic bacteria itself, most notably silver nitrate even in very small quantity will kill an entire culture.

Roots from trees and shrubbery growing above the tank or the drainfield may clog and/or rupture them.

Playgrounds and storage buildings may cause damage to a tank and the drainage field. In addition, covering the drainage field with an impermeable surface, such as a driveway or parking area, will seriously affect its efficiency and possibly damage the tank and absorption system.

Excessive water entering the system will overload it and cause it to fail. Checking for plumbing leaks and practicing water conservation will help the system's operation.

Very high rainfall, rapid snow-melt, and flooding from rivers or the sea can all prevent a drain field from operating and can cause flow to back up and stop the normal operation of the tank.

Over time, biofilms develop on the pipes of the drainage field, which can lead to blockage. Such a failure can be referred to as "biomat failure".

Septic tanks by themselves are ineffective at removing nitrogen compounds that have a potential to cause algae blooms in receiving waters; this can be remedied by using a nitrogen-reducing technology, or by simply ensuring that the leach field is properly sited to prevent direct entry of effluent into bodies of water.

7.3 Methane fermentation system of sewage sludge and raw garbage, and carbonization-activation for utilization

The methane fermentation technology system of sewage sludge and raw garbage, and carbonization-activation for utilization achieves two goals:
	- Making beneficial use of methane gas recovered via methane fermentation from a mixture of sewage sludge and organic waste, such as kitchen refuse, received from local communities, viewing a wastewater treatment plant as a facility for using local biomass; and

- Promoting carbonization-activation and recycling of fermentation residue, or dewatered sludge.

This technology system integrates two processes, which are methane fermentation of sewage sludge-biomass mixture followed by power generation using the resultant biogas and carbonization-activation of fermentation residue.
	The use of biomass is under assessment in Japan, but progress is slow regarding the use of wet biomass due to its high water content, which accounts for a high weight percentage. Methane fermentation is attracting great interest as a low-cost means of recovering energy from wet biomass. However, one drawback of the process is the treatment cost for filtrate from the dewatering machine after fermentation. In this regard, a wastewater treatment plant has large wastewater treatment equipment and consumes electricity generated within the plant, and so is expected to be a useful facility for making use of wet biomass. Moreover, methane gas can be used as supplementary fuel for carbonization or carbonization-activation of residual dewatered sludge to promote recycling without consuming fossil fuel. Furthermore, in the case of receiving kitchen refuse included in general domestic waste, the refuse incineration plant can be downsized when rehabilitated.

The four major components of this technology are: (1) Pretreatment equipment, (2) Methane fermentation tank, (3) Biogas power generating equipment and (4) Carbonization-activation equipment (Figure 22).

These components have the following functions.

(1) Pretreatment equipment
Receives kitchen refuse and other biomass, removes foreign matter and solubilizes the biomass mixed with sewage sludge.

(2) Methane fermentation tank
Performs methane fermentation of the solubilized liquid and another portion of sewage sludge to produce biogas.

(3) Biogas power generator

Refines biogas, generates power by gas engine and supplies electricity to plant facilities.

(4) Carbonization-activation equipment
Dries dewatered digested sludge (dewatered sludge) and produces activated carbon.

Real conditions may vary depending on the local situation. For example, the type of biomass that the plant can receive may vary or the plant may receive dewatered sludge from other plants for treatment. This technology allows for optimization, such as considering whether or not to incorporate every component or changing the capacity to reduce the final cost.

This technology is designed to receive wet biomass including kitchen refuse, recover a larger quantity of methane gas and earn revenue from the treatment. By producing valuable activated carbon from dewatered sludge without disposing it as industrial waste and earning revenue from its sales, this technology substantially improves the economy of the overall system.

(1)Wastewater treatment plant receives kitchen refuse and other wet biomass.
→ Earns waste disposal fees.

(2)Performs methane fermentation of a mixture of wet biomass and sewage sludge, and uses the resultant biogas for high-efficiency power generation.
→ Increases the biogas recovery rate and reduces the amount of electric power purchased.

(3)Produces activated carbonized products from dewatered sludge.
→ Reduces disposal costs for dewatered sludge, and earns revenue from selling valuable products.

⇒Substantially improves the economy of the system.

The features of this technology are detailed below.

(1) Pretreatment equipment

Figure 22.  Integration of both technologies

The pretreatment equipment is comprised of a crusher-separator and solubilization tank. The pretreatment equipment crushes kitchen refuse,  separates and removes foreign matter. The kitchen refuse is then mixed with some sewage sludge and made soluble by the activity of microorganisms within a retention period of approximately 24 hours at 40 to 50°C. The viscosity of the mixture decreases through this process, meeting the required fluidity in the methane fermentation tank. In this process, part of the organic matter becomes organic acids via acid fermentation. Moreover, high-melting-point animal fats are also solubilized without becoming lumpy. A dryer, if installed in the plant, eliminates the need for a solubilization tank heater since dryer exhaust can warm the sewage sludge to be fed into the solubilization tank.

(2) Horizontal methane fermentation tank

Synergistic effects can be attained by raising the operating temperature, thus allowing a compact fermentation tank to be used with a volume for a retention period of approximately seven days. Compactness also reduces the amount of heat radiation and hence heating energy in comparison with the conventional complete-mixing ones used for high-temperature fermentation. At the section where the raw material is fed in, some 40% of sludge is returned to prevent rancidification and maintain a high microorganism concentration. The retention periods required for different raw materials are easy to forecast by laboratory batch testing.

(3) Sludge heating tower for digestion tank

The scrubber-equipped gas-liquid contact tower allows dryer exhaust to come in direct contact with sewage sludge to condense the steam from the exhaust and also heat the sewage sludge. This system has been installed and is in operation at seven locations in Japan. Usually 30 to 50% of the produced digestion gas is fired in a dedicated boiler to produce hot water or steam for heating. This technology system eliminates this need, thus achieving substantial digestion gas savings. Sludge in the digestion tank can be recycled to the gas-liquid contact tower depending on the tank temperature.

(4) Carbonization-activation equipment

The apparatus comprises a dryer and an activated carbonization furnace. The dryer is a flash dryer using steam as medium. The activated carbonization furnace is an externally heated screw furnace. The dryer obtains heat from the exhaust of the activated carbonization furnace via a heat exchanger. The steam-rich exhaust is dehumidified in the sludge heating tower, leaving an extremely small quantity of odorous gas. The odorous gas is used for combustion in the activated carbonization furnace, eliminating the need for a hot air generating furnace and deodorization furnace. In the activated carbonization furnace, dried sludge and steam contained therein are sent to the lowest part of the screw and a high operating temperature is used to facilitate the activation reaction to produce adsorptive activated carbonized products. Activated carbonized products can be sold at high prices as a dioxin adsorbent. Moreover, carbonized products produced at a low operating temperature are used as soil improvers or fuel. Schematic diagram of the technology is shown Figure 23.

Figure 23. Schematic diagram of the technology

Questions for self-control

- What is anaerobic processing?

- Why was anaerobic processing of waste recognized by the Program of Development of the United Nations as one of the most useful decentralized power supply sources?

- Anaerobic processing is widely used as a source of renewable energy. What is a final or an intermediate result?

- When was the first anaerobic systematization constructed by the leper colony? What is the septic tank? What is the volume of the septic tank?

- What stages of anaerobic processing do you know?

- Are substances that do not decay in a septic tank during anaerobic digestion dangerous or not?

- Describe one disvantage of methane fermentation process.

- What are the four main components of this technology  of biogas production?

- What is the horizontal tank of methane fermentation?

- What technological system relieves burning of the gas?

- Where can carbonized products be used?

- What does the equipment for carbonization activation include?

- Why dryer is used in methane tanks?

- What dryer is used as nutrient media?

- How is methane formed?

- Why do we need tank in methane fermentation?

- What are the disvantages of the methane fermentation process ?

- Where do we use the carbonized products made at a low working temperature?

Chapter 8

Utilization of agrIcultUral and industrial waste

8.1 Bioconversion of organic waste

The technology of microbiological bioconversion of waste is designed for processing raw materials not used in traditional feed production into high-quality carbohydrate-protein feed additives and mixed fodder. The technology of microbiological bioconversion is intended for processing of agricultural waste, food and grain processing industry into feed additives and mixed fodder.

The essence of bioconversion technology is as follows: raw materials (waste) containing complex polysaccharides – pectin, cellulose, hemicellulose are exposed by complex enzyme preparations containing pectinase, hemicellulase and cellulase.

Enzymes are purified extracellular protein capable of deep destruction of the cell walls and individual structural polysaccharides, i.e. splitting of complex polysaccharides into simple ones, followed by construction on their basis of easily digestible feed product.

The following wastes can be used as raw material components:

- plant components of agricultural crops: stalks of grain and industrial crops, baskets and stalks of sunflower, flax waste, corn cob rods, potato pulp, grass legumes, waste haylage and silage, waste vines, waste tea plantations, tobacco stems;

- waste of the grain processing industry: bran, waste produced during cleaning and sorting of grain mass (grain waste), grain weed admixture, injured grains, puny and sprouted grains, seeds of wild plants, substandard grain;

- canning wastes, wine industry and fruit waste: peels, seeds and defective fruit pomace, grape waste, waste vegetables, cut ends of fruit, cake, defective, vegetables, waste green peas (greens, scattered grain, broken grain, pieces of leaves, leaves), waste cabbage, beets, carrots, potatoes;

- wastes of sugar industry: beet pulp, molasses, refinery molasses, sludge cake, beet waste, tails of beets;

- waste of brewing and the alcohol industry: alloy of barley (shrunken grain barley, chaff, straw and other impurities), polishing waste, particles of crushed shells, endosperm, broken grain, malt dust, brewer's grain, molasses, starchy foods (potatoes and various types of grain), distillery grains, mash;

- waste of tea industry: tea dust, stalks;

- wastes of essential oil industry: waste of herbaceous and floral raw materials;

- wastes of oil and fat industry: sunflower husks, cotton husks;

- wastes of confectionery and dairy industry.

Thus, all vegetable raw material and its derivatives, such as a lignocellulosic source, are available for microbiological bioconversion in carbohydrate-protein feed and feed additives.

In the process of bioconversion substandard components are destroyed by pathogenic microflora, eggs of helminths, causative agents of severe diseases (brucellosis, tuberculosis, cholera, typhoid, etc.), various  protozoa parasites (roundworms, tapeworms, etc.). At the same time, the feed value of substandard raw materials after appropriate processing is 1.4-1.8 times higher than the feed value of conditioned analogues.

After completion of the bioconversion process, the final product is a feed additive - carbohydrate and protein concentrate (CPC), which acquires feed properties 1.8-2.4 times superior to feed grain of good quality, and also has a number of important properties that traditional grain raw materials do not have.

A key element of the technological chain is a bioreactor, in which the process of microbiological bioconversion of waste into feed is carried out. Reactors are universal and allow us to work with any raw material and to produce various feed additives. The technology provides year-round operation of the enterprise, low requirements for the skills of most workers, and low energy costs. The technology is environmentally safe and has no waste water and emissions.

A wet (55%) mixture of various wastes is loaded into the bioreactor. Since the loading of raw materials, the process of microbiological bioconversion takes place in the bioreactor for 4-6 days (depending on the desired zootechnical parameters of the final product). The result is a wet feed additive-carbohydrate and protein concentrate (CPC). Then it is dried to a moisture content of 8-10 % and crushed. After grinding, the concentrate can be used for the production of feed, where CPC is used as the main component (65 – 25% depending on the recipe and the purpose of feed).

Technological scheme of the production complex for microbiological processing of plant waste into feed:

1-reception of bulk and wet raw materials; 2-reception of liquid raw materials; 3-dosing bins; 4- a mixer; 5-a bio-reactor; 6- a compressor; 7- a steam generator; 8 – a dryer; 9 – a shredder; 10 – shipment in bags.

8.2 Characteristics of wastes from food industry

Industrial waste includes all waste that is generated as a result of the processing of raw materials into products, as well as any material that becomes useless in the production process, for example, in factories and mining or during excavations. Some examples of industrial waste are paints, sand paper, paper products, industrial by-products, metals, and radioactive wastes.

Toxic waste, chemical waste, industrial solid waste and municipal solid waste are designations of industrial waste. Sewage treatment plants can treat some industrial wastes, i.e. those consisting of conventional pollutants such as biochemical oxygen demand (BOD). Industrial wastes containing toxic pollutants require specialized treatment systems.

Industrial wastewater treatment includes mechanisms and processes used to treat waters that have been contaminated in some way by anthropogenic industrial or commercial activities prior to its release into the environment or its re-use.

Most industries produce some wet waste although recent trends in the developed world have been to minimise such production or recycle such waste within the production process. However, many industries remain dependent on the processes that produce wastewaters.

Wastewater generated from agricultural and food operations has distinctive characteristics that set it apart from common municipal wastewater managed by public or private sewage treatment plants throughout the world, i.e. it is biodegradable and nontoxic, but has high concentrations of biochemical oxygen demand (BOD) and suspended solids (SS). The constituents of food and agriculture wastewater are often complex to predict due to the differences in BOD and pH in effluents from vegetable, fruit, and meat products and due to the seasonal nature of food processing and post harvesting.

Processing of food from raw materials requires large volumes of high grade water. Vegetable washing generates waters with high loads of particulate matter and some dissolved organic matter. It may also contain surfactants.

Animal slaughter and processing produces very strong organic waste from body fluids, such as blood, and gut contents. This wastewater is frequently contaminated by significant levels of antibiotics and growth hormones from the animals and a variety of pesticides used to control external parasites.

Processing food for sale produces wastes generated from cooking which are often rich in plant organic material and may also contain salt, flavourings, colouring material and acids or alkali. Very significant quantities of oil or fats may also be present.

### Pulp and paper industry. Effluent from the pulp and paper industry is generally high in suspended solids and BOD. Stand alone paper mills using imported pulp may only require simple primary treatment, such as sedimentation or dissolved air flotation. Increased BOD or chemical oxygen demand (COD) loadings, as well as organic pollutants, may require biological treatment such as activated sludge or upflow anaerobic sludge blanket reactors. For mills with high inorganic loadings such as salt, tertiary treatments may be required, either general membrane treatments such as ultrafiltration or reverse osmosis or treatments to remove specific contaminants, such as nutrients.

Wastes of food industry are a good medium for reproduction of different types of bacteria, fungus and other types of dangerous microorganism. Wastes of food industry are not dangerous and do not present any threats. But under the influence of high temperature such waste turns out into a good medium for  reproduction of flies, rats and cockroach, carriers of infectious and other dangerous diseases. Therefore well-timed utilization of waste of food industry is an important problem for the administration of the enterprise.

Collection  , transportation, conversion and salvaging of wastes of food industry must be carried out  in strict schedule to avoid arising threat of  infectious diseases and epidemies. For prevention decomposition and fermentations, waste must be removed in a short time in special tanks and containers with hermetic closing.

For recycling waste of food industry such methods as instillation and composting are used.Waste of food industry disposed in landfills has a large amount of liquid. The liquid contains organic acids that can react with heavy metals in the soil. As a result of the formation of compounds, it has a negative impact on the environment.

The method of composting is formation of special drives for waste. Inside these drives, natural decomposition of waste occurs under the influence of certain temperatures. Waste treatment methods are constantly improving. One of these improvements was a special device with which you can grind such waste directly from the packaging.The use of such units is to obtain a substance which can be used, for example, in the production of concrete.

8.2.1 Waste waters collected from dairy industry

Milk occupies an important place in human life. The dairy industry involves processing of raw milk into such products as consumer milk, butter, cheese, etc. The quantity of water required in a milk processing plant depends upon the size of the plant. It is generally expressed in terms of the maximum weight of milk handled in a day and the involved processes. The daily volume of required water may vary widely mainly depending on the availability of water and the control of all water used in operations at the plant. The operations where the process involves continuous water flow, the amount of water needed for rinsing and washing is not necessarily proportional to the amount of product processed. Most of the waste water discharged into water bodies, disturbs the ecological balance and deteriorates the water quality. The casein precipitation from waste decomposes further into highly odorous black sludge. Effluents from milk processing unit contain soluble organics, suspended solids and trace organics, which release gases, cause taste and odor, impart colour and turbidity, and promote eutrophication. How it affects and disturbs the environment can be evaluated by studying the physicochemical characteristics of waste water generated from dairy industry with suitable treatment.

As management of dairy wastes becomes an ever-increasing concern, treatment strategies will need to be based on state and local regulations. Because the dairy industry is a major water user and wastewater generator, it is a potential candidate for wastewater reuse. Purified wastewater can be utilized in boilers and cooling systems as well as for washingplants, and so on. Even if the purified wastewater is initially not reused, the dairy industry will still benefit directly from in-house wastewater treatment, since levies charged for wastewater reception will be significantly reduced.

All these facts underline the need for efficient dairy wastewater management. Before selecting any treatment method, a complete process evaluation should be undertaken along with economic analysis. This should include the wastewater composition, concentrations, volumes generated, treatment susceptibility, as well as the environmental impact of the solution to be adopted. All options are expensive, but an economic analysis may indicate that slightly higher maintenance costs may be less than increased operating costs. What is appropriate for one site may not be suitable for another.

The most useful processes are those that can be operated with a minimum of supervision and are inexpensive to construct or even mobile enough to be moved from site to site. The changing quantity and quality of dairy wastewater must also be included in the design and operational procedures. From the literature it can be found that biological methods are the most cost-effective for the removal of organics, with aerobic methods being easier to control, but anaerobic methods have lower energy requirements and lower sludge production rates. Since no single process for treatment of dairy wastewater is by itself capable of complying with the minimum effluent discharge requirements, it is necessary to choose a combined process especially designed to treat a specific dairy wastewater.

All wastewatert reatment systems are unique. Before a treatment strategy is chosen, careful consideration should be given to proper wastewater sampling and composition analysis as well as a process survey. This would help prevent an expensive and unnecessary or overdesigned treatment system. A variety of different local and international environmental engineering firms are able to assist in conducting surveys. These firms can also be employed to install effective patented industrial-scale installations for dairy processing wastewater treatment.

The dairy industry wastewaters are primarily generated from cleaning and washing operations in the milk processing plants. Dairy wastewaters are characterized by high biological-oxygen demand (BOD) and chemical oxygen demand (COD) concentrations, and generally contain fats, nutrients, lactose, as well as detergents and sanitizing agents. Dairy effluents decompose rapidly and deplete the dissolved oxygen level of the receiving streams immediately resulting in anaerobic conditions and release of strong foul odours due to nuisance conditions. The casein precipitation from waste, which decomposes further into a highly odorous black sludge at certain dilutions, also causes toxicity to fish.

Milk whey is an important by-product of the dairy industry, nutrient-rich and potentially used as a growth medium for the production of commercial products. Milk whey is now a high BOD waste but, since it has the proper composition for PHA production, its use will serve a double purpose: to lower food production costs and to reduce the amount of waste produced by the cheese industry.

During the last years, the amounts of whey increased to such an extent that they cannot be simply used as animal feed as the most common application. To overcome these problems, a sustainable alternative is to upgrade whey and its derivatives to a resource for many value-added industrial products, making whey not only a waste but also a valuable resource.

8.2.2 Utilization of winery wastes

Wine making process is one of the most historical applications of biotechnology. The earliest known wine production may have been in the region of Iran as long ago as 6000 BC. Many centuries later, this process without any knowledge was combined with technological development and became a large industrial area. Therefore the requirements to raw material also increased.

In terms of organic chemistry, wine is a complex mixture of a large number of compounds including carbohydrates, alcohols, aldehydes, esters, acids, proteins and vitamins. It is also home to a number of polyhydroxy aromatic compounds, such as tannins, anthocyanins and flavonols, which contribute hugely to colour and taste. The basic raw material for a wine fermentation is a fermentable sugar, such as fructose or sucrose, rather than the less soluble, non-fermentable starch, which is the raw material for most beers.

Wine is usually produced from grapes, honey, grains, rice and sugarcane. Depending on the cultivation conditions of the region, one of these ingredients can be fermented up to ethanol, which is the most desirable chemical compound in alcoholic beverages. Conversion of sugar to ethanol finishes with a liquid phase that contains ethanol. Starting with solid phase to obtain alcoholic phase generally needs separation, discharging and sedimentation steps. In winemaking process, it is possible to have stalks, pulp, skin and lees. Most of them can be called as a waste for wineries but reducing sugar, cellulose, hemicellulose and pectin content shows that these can be called as a substrate for different biotechnological pathways.

Discharging of winery waste to soil is a different concern for the environment. According to recent studies, germination properties of soil are inhibited by discharging of winery wastes because of the biological oxygen demand (BOD), carbon and phenolic compounds. Grape pomace is the major waste generated in the winemaking process, and utilization of its components, such as skins, pulp, stalks and seeds, plays an important role in waste reduction and, thus, reduction of the environmental impact and production of value-added products. Grape pomace is usually used as fertilizer, animal feed or extraction raw material of seed oil and polyphenols. But utilization of winery waste is promising in the light of new biotechnological applications. Reducing sugar content can be extracted from red or white grape pomace. Also, complex carbohydrates (cellulose, hemicellulose and pectin) participating in grape pomace can be hydrolyzed up to reducing sugar by different methods such as extraction, acid hydrolyses and enzymatic hydrolyses.

Two different grape types of Syrah (red) and Muscat (white) were collected in the middle of the harvest season and were hydrolysed by enzymatic and acid hydrolysis. After screening possible fermentable sugars of grape pomace, lactic acid fermentation was performed from grape pomace suspension and liquid extract phase.

Lactic acid fermentation by Lactobacillus casei showed that grape pomace can be used as a substrate for lactic acid production. Different solid loadings and yeast extract concentrations effect the lactic acid production yield from grape pomace.

Maximum 84 % of fermentable sugar in dry grape pomace was converted to lactic acid by L. casei. The studies of yeast extract showed that commercial yeast (bakers’ yeast) can be used as nitrogen source instead of yeast extract, and 10 g/l of yeast extract was the most suitable concentration for lactic acid production from grape pomace by L.casei. This study showed the potential of the grape pomace for fermentative processes.

8.2.3 Efficient recycling of brewer’s spent grain

In the brewing industry during processing of raw materials, brewer’s spent grain (BSG), hop grains, protein sludge, residual brewer's yeast, carbon dioxide are produced from 100 kg of grain, 125-130 kg of brewer’s spent grain are produced with 20-25% dry matter content. If it dries, it will give dry pellets weighing 27 kg and humidity of 12%. Usually in breweries grains are not dried, they are used for animal feed.

Hop pellet is not used for livestock due to high bitterness, so it has no use in national economy. From 1 kg of dry hops about 4 kg of wet hop pellets are obtained.

Protein sludge is obtained by cooling and clarification of wort. From 100 kg of processed grain products 2-3 kg of protein sludge with humidity of 80% are obtained. It has intense bitterness and it is not used for animal feed either.

Residual brewer's yeast is a valuable food product consisting of proteins, carbohydrates, fats, which are rich in vitamins. Due to the high content of vitamins, brewer's yeast has a high biological value and is used for medicinal purposes, as well as additives in some foods. 100 liters of beer produced is approximately 1 kg of yeast dough containing 0.14 kg of dry matter. Yield of purified yeast from 1L of liquid yeast is 80-85%.

Carbon dioxide is produced mainly in the main fermentation. It can be turned into marketable products, liquefied carbon dioxide and solid (in the form of briquettes) carbon dioxide. In the process of fermentation in closed fermenter,100 liters of beer produced 1.25-1.5 kg of commercial liquefied carbon dioxide.

Recent advances in biotechnology ensure that brewer’s spent grain (BSG) is no longer regarded as a waste but rather a feedstock for producing several products. Based on this, it is an undeniable fact that BSG has its own potential for sustainable reuse through biotechnological processes. Thus, efficient recycling of BSG requires extensive work towards exploring newer applications and maximizing use of existing technologies for a sustainable and environmentally sound management. Finally, more insight is required for large scale utilization, which involves both laboratory and field experiments with proper control processes.

Many microorganisms have been utilized for the isolation of amylases and starch hydrolysis, mainly bacteria such as Bacillus amyloliquefaciensand B. licheniformis. Filamentous fungi, especially Aspergillus strains such as A. oryzae, A. nigerand A. awamori, are also used for industrial enzyme production due to their superior ability to secrete enzymes. The aim was to upgrade BSG through hydrolysis of their residual starch using strains of A. oryzae, and to evaluate the possibility of use of the treated material as protein enriched animal feed.

The fungus with the highest frequency of occurrence was identified as Aspergillusoryzae. The proximate analyses revealed a significant increase in the percentage of protein from 18.22% in the unfermented to 28.33% in the solid state fermented spent grain after 35 days of fermentation. The ash content of the spent grain also increased from 3.66 % to 7.4% in the fermented sample and a significant decrease in the carbohydrate from 22.05% to19.05% was observed, ether extract from 5.53% decreased  to 3.23%, and the fiber content also decreased from 18.11% to 13.99% in the brewery spent grain with time, which means that BSG after fermentation will be suitable for farm animal feeding.

BSG slurries were treated either directly with Aspergillus spores or with crude enzyme solutions, to evaluate the possibility of using the treated substrates as protein enriched animal feeds. Nevertheless, because BSG contains low amounts of residual starch, cheap and abundant food by-products (molasses) were employed as carbon sources and their mixtures with BSG hydrolysates were used as substrates for yeast production without the use of other nutrients. Specifically, in the first set of experiments, BSG slurries of different pH (4, 6 and 8) were inoculated with various amounts of spore suspensions (9-105, 106, 2-106 spores/ml). Hydrolysis was carried out at 30 °C with simultaneous fungal growth. No significant effect of pH and initial spore concentration was observed. The systems were monitored for residual starch conversion for 40-45 h, but the process times required for growth, enzyme secretion and starch hydrolysis were 15-25 h for Aoryzae. The best starch conversion with A. oryzae,was achieved at pH 8. On the other hand, at pH 4 and 6, the hydrolysis of starch  was not efficient and did not proceed further after a period of 20 h.

To avoid transformation of sugars to biomass, crude enzyme solutions were prepared by growth of the fungi in various substrates made from cheap by-products of the food industry, like potato peel and molasses. The crude enzyme solutions were used to hydrolyze BSG residual starch at 45 °C. It was found that the medium containing blended potato peel was far the most efficient, leading to fast growth and higher amylase activity within 25-30 h for both strains. The activities of the crude enzyme solutions were higher when CaCI2 was added. When the produced crude enzyme solutions were used for hydrolysis of BSG starch at 45 °C, it was found that Aoryzaeamylase activity (1-2 h) was high. The sugars produced by hydrolysis of BSG starch using A. oryzae were in very low levels to support the use of hydrolysates as raw materials for yeast production. Therefore, it was thought that apart from using BSG treated directly with fungi as animal feed, their hydrolysates (BSGH) could also be utilized as sources of minerals or N-compounds if mixed with other materials, rich in carbohydrates, for biomass production. Therefore, experiments were carried out using media containing mixtures of molasses, with and without addition of BSGH. It is obvious that biomass yields (g of dry yeast per g of sugar utilized) were improved when BSGH was added to the growth medium, for both fungi and for all the tested media.

The above results demonstrate that A. oryzae was efficient as far as the process times and enzyme stability were concerned. Hydrolysis was faster with crude enzyme solutions than directly with spores, avoiding transformation of sugars to biomass. Therefore, the hydrolyzed BSG could be utilized for e.g. yeast growth within the brewery. Since BSG do not contain high amounts of residual starch, the addition of molasses can enhance the capacity of the hydrolyzed BSG slurries. The remaining solids may also be used as animal feeds. In all cases, the BSG can be utilized to create added value and avoid environmental problems caused by their disposal.

8.3 Wastes of cellulose and paper industry

This process is aimed at obtaining pulp, paper, cardboard and other related products of final or intermediate processing.

Paper first was mentioned in Chinese chronicles in the 12-th century BC. Raw materials for its production were stalks of bamboo and cork mulberry tree. In the year 105 CaiLun generalized and improved the existing methods for preparation of paper.

In Europe, the paper appeared in the XI-XII centuries. It replaced the papyrus and parchment (which was too expensive). For the manufacture of paper crushed hemp and linen rags were first used.

In 1719 Reaumur made the assumption that e timber can serve as raw material for paper production. However, the need to use wood appeared only at the beginning of the XIX century when paper machine was invented, which increased productivity,  resulting in shortage of raw materials for paper plants.

In 1853 Mello (France) patented a method of producing pulp from straw cooking with 3% solution of sodium hydroxide in a hermetically sealed furnace at a temperature of about 150 °C (soda pulping). Almost simultaneously Watt (England) and Burges (USA) got patents for production of pulp from wood in a similar way. The first plant for the production of soda pulp was built in 1860 in the United States.

In 1866, B. Tilgman (USA) invented a method of producing sulphite pulp.

In 1879 K.F. Dahl (Sweden) modified soda pulping and invented sulfate (kraft) pulp production method, which to this day is the main method for its production.

As for the production of timber, a lot of water is required, thus, pulp and paper mills are usually placed on the banks of large rivers, where it is possible to use the river for rafting timber serving as the main raw material for production.

For paper and paperboard, the following fibrous semi-finished materials are produced (data 2000):

waste paper - 43%

Kraft pulp - 36%

pulp - 12%

sulphite pulp - 3%

hemicellulose - 3%

cellulose from plant raw material is not wood - 3%

For manufacturing of high-grade paper to print money and important documents, shreds textiles are used.

Paper wastes include a high percentage of various types of packaging material, which has adverse effects on the environment.

Waste paper is the main raw material for paper industry of the XXI century. The amount of waste paper in every country directly depends on the level of paper consumption per capita. In Russia this figure now stands at around 35 kg, which is 8-10 times lower than in Germany and the United States. Despite the fact that only some regions of Russia are facing the problem of recycling waste paper, not far off the time it can grow into a nation-wide. Already, according to experts in urban waste disposed at landfills, about 40% comes from recycling. Thus, it is necessary to develop technologies for recycling and reuse of waste paper.

Furthermore, to impart special properties to the paper, sizing agent mineral fillers and special dyes are added,.

One way  is the use of recycled paper as the base material for the production of liquid wallpaper.

The composition includes a liquid wall from 60 to 80% of the pulp. This is an excellent method for the processing of paper waste.

The processing of waste paper in the raw material for making decorative plaster consists of several stages:

Crushing - Waste paper to produce a uniform dry pulp;

Mixing - Dry pulp with water for better mixing with further components;

Addition - In the wet pulp cellulose glue (as binder), quartz mica, silk fiber, various decorators;

Drying - Resulting pasty mass in the roller dryer;

Crushing - Dried material for convenience in use.

As a result of the above-mentionedsteps the waste paper are converted into valuable finishing material.

8.4 Polyethylene as an ecological problem

About 40 countries of the world refused or took the toughening measures, on use of plastic bags,the countries with advanced economy, and developing countries. In the nature throw out more than 4 trillion packages year, plastic packages are to 10% of all mass of household waste, plastic packages are to 90% of the garbage floating in the Pacific Ocean. All these emissions bring to the state and the nature big damages, pollution, death of animals, the hammered sewerages, etc. It is also possible to carry to shortcomings of plastic bags that they are made from the major non-restorable natural resources - gas, coal, andoil. Besides, they are very durable and hardly decay that seriously threatens our ecology. After all for decomposition of one plastic bag are required more than 1000, and thus polyethylene doesn't decay in the earth, and in the course of combustion it emits carbon dioxide.

Many experts, from the different countries, suggest to replace plastic bags with the paper. But paper packages have the highest impact on global warming in comparison with other types of bags. By production of paper packages more carbon dioxide is emitted for 70%, than by production of plastic bags, and by 50 times bigger quantity of water becomes soiled. Production of paper packages also promotes global warming and on the other hand – there is a large-scale cutting down of trees. However, a certain British company Symphony Environmental Ltd developed the d2w technology based on simple the principle of development, in which plastic not simply falls to pieces - it collapses to fragments with such low molecular weight that it allows microorganisms to get access to carbon and hydrogen.

In our country there isn't similar fight being conducted against plastic bags. After all, the cost of plastic bags is many times lower than the cost of any other packing. Shops aren't ready to refuse so cheap and effective advertizing, as plastic bags with a logo.

However not everything depends on the government. Inhabitants can refuse whenever possible use of plastic bags, try to use them most long.

Quoting an English proverb: "Don't raise children all the same they will be similar to you. Bring up yourself", and after all and the truth it is worth reflecting, and to draw for itself a conclusion, that we have to remain in any situation a person, be able to understand, create, and, above all to be an example for the children and to make this world at least slightly better and purer.

8.5 Bacterial leaching

Bacterial leaching extraction chemical elements from ores, concentrates and rocks using a bacteria or their metabolites.Most of the bacterial leaching is combined with leaching with weak solutions of sulphuric acid, which has bacterial or chemical origin, as well as with solutions containing organic acids, proteins, peptides, polysaccharides, etc.

Bacterial leaching for refractory sulphide concentrates is carried out by parallel flow way, in series consistently connected of tanks with a mixing and aeration by aero-elevator at 30 ° C degree, at pH is 2,0 -2,5 and a cell concentration of Th. ferrooxidans 1010-1011 in 1 ml of slurry. Schemes of recycling sulphide concentrates are always closed. Circulating solutions after partial or complete regeneration is used as a medium for the bacteria and leach solution.

The most active bacterial cultures are adapted to a complex of factors (pH, heavy metals, type of concentrate, etc.).) under conditions of active bacterial leaching process. Examples of bacterial leaching in a container, so from a complex metal mixture is extracted for 72-96 hours to 90-92% Zn and Cd, with the extraction of Cu and Fe, respectively, about 25% and 5%; from the concentrates can be completely removed Cu, Zn and Cd. In solution, the metal concentrations are reached: Cu to 50 g / l, Zn to 100 g / l, etc.

In tin- and gold-arsenic concentrates, arsenopyrite was almost completely destroyed in 120 hours which allows in some cases to clean concentrates from harmful admixtures arsenic, in others - in the subsequent cyanidation to extract up to 90% gold.

Introduction of bacterial leaching, as well as other ways of hydrometallurgical extraction of metals has great economic importance. Expanding source of raw materials is due to the use of poor and lost in the depths of ores, etc. Bacterial leaching  provides a comprehensive and fuller use of mineral raw materials and increases culture of production, it is does not require the creation of complex mining complexes, and good for the environment.

In nature, sulfidic ores are decayed by weathering under the influence of oxygen and water. Microbiological investigations reveal that certain bacteria are the main agent in this process. Several bacteria, especially Thiobacilli, are able to solubilize heavy metal minerals by oxidizing ferrous to ferric iron as well as elemental sulfur, sulfide and other sulfur compounds to sulfate. So they enhance leaching of heavy metals from sulfidic ores under aerobic conditions about 104 fold or more compared with weathering without bacteria.

The principal bacterium in ore leaching is Thiobacillusferrooxidans, which is capable of oxidizing ferrous iron as well as sulfur and sulfur compounds. But there are some other bacteria which may also be involved. For example the thermophilicSulfolobus plays a role in leaching at elevated temperatures. Thiobacillusthiooxidans, which oxidizes merely sulfur and sulfur compounds but not iron, and Leptospirillumferrooxidans, which contrarily oxidizes only ferrous iron, may play a role if they work together or with other bacteria.

Bacterial ore leaching can be applied to extract heavy metals from low grade ores, industrial wastes and other materials on an industrial scale by different procedures: dump leaching, in situ leaching, tank leaching, leaching in suspension. Sulfidic copper and uranium ores are the principle ores leached in several countries. So 20% to 25% of the copper production in the U.S.A. and about 5% of the world copper production is obtained by bacterial leaching. This process is a very slow one and needs a long time (years) for good recovery, but its main advantages are low investment costs and low operating costs.

Current investigations deal with the leaching of ores other than those mentioned, leaching industrial wastes to recover metals, desulfurizing of coal, developing methods for in situ leaching and using other microorganisms than those used until now. Basic microbiological research focuses on the biochemistry, physiology and genetics of the involved microorganisms and on the complex interrelationships in the microbial community of leaching biotopes.

8.6 Utilization of livestock waste, manure for the production of biogas and fertilizers

The anaerobic digestion method is most suitable from the point of view of hygiene and environmental protection for the processing of animal waste, because it gives the highest disinfection of the residue and elimination of pathogenic microorganisms.

In anaerobic digestion, organic matter decomposes in the absence of oxygen. This process involves two steps. At the first stage, complex organic polymers (cellulose, proteins, fats, etc.) under the action of the natural community of various types of anaerobic bacteria, decompose to simpler compounds: volatile fatty acids, lower alcohols, hydrogen and carbon monoxide, acetic and formic acids, methyl alcohol . At the second stage, methane-forming bacteria convert organic acids into methane, carbon dioxide and water.

Primary anaerobes are represented by various physiological groups of bacteria: cell-destructive, carbon-fermented (such as butyric acid bacteria), ammonifying (decomposing proteins, peptides, amino acids), bacteria, decomposing fats, etc. Due to this composition, primary anaerobes can use a variety of organic compounds of plant and animal origin, which is one of the most important features of the methane community. The close connection between these groups of bacteria ensures sufficient stability of the process.

Methane fermentation occurs at medium (mesophilic) and high (thermophilic) temperatures. The highest productivity is reached at thermophilic methane fermentation. The feature of a consortium of methane allows us to make the fermentation process uninterrupted. For the normal course of the anaerobic digestion process, optimal conditions in the reactor are necessary: ​​temperature, anaerobic conditions, sufficient nutrient concentration, acceptable range of pH values, and absence or low concentration of toxic substances.

The temperature strongly influences the anaerobic digestion of organic materials. The best fermentation takes place at a temperature of 30-40 ° C (development of mesophilic bacterial flora), as well as at a temperature of 50-60 ° C (development of thermophilic bacterial flora). The choice of mesophilic or thermophilic mode of operation is based on the analysis of climatic conditions. If significant energy costs are required to provide thermophilic temperatures, the operation of reactors at mesophilic temperatures will be more efficient.

Along with the temperature conditions of the methane fermentation process, the amount of biogas produced affects the waste treatment time.

When operating reactors, it is necessary to monitor the pH value, the optimal value of which is in the range of 6.7-7.6. This indicator is regulated by adding lime.Under normal operation of the reactor, the biogas obtained contains 60-70% of methane, 30-40% of carbon dioxide, a small amount of hydrogen sulfide, as well as admixtures of hydrogen, ammonia and nitrogen oxides. The most effective reactors operate in thermophilic mode at 43-52oС. When the duration of manure treatment is 3 days, the biogas yield at such plants is 4.5 liters per liter of the useful volume of the reactor. Organic catalysts are added to the initial mass to intensify the process of anaerobic digestion of manure and release of biogas, which change the ratio of carbon and nitrogen in the fermented mass (optimal ratio C/N=20/1-30/1). Glucose and cellulose are used as such catalysts.

In fermentation chambers, vigorous stirring is necessary to prevent the formation of a pop-up substance in the upper part of the layer. This significantly accelerates the fermentation process and biogas yield. Without mixing, the volume of reactors must be significantly increased to obtain the same capacity. Hence, the consequence is high costs and higher cost of installation.

Mixing is carried out by:

- mechanical agitators of various shapes or submersible pumps driven by an electric motor,

- hydraulic nozzles due to the jet energy pumped by the manure fermentation pump or recirculation,

- overpressure of biogas passed through a bubbler or tube located at the bottom of the reducer.

The liquid phase of manure after anaerobic digestion usually meets the requirements of environmental authorities for wastewater quality. The spent liquid organic mass enters through the discharge chamber into the tank of the fermented mass, and from there it is pumped into tanks, with the help of which the usual manure mass is introduced into the fields.

The residue formed in the process of biogas production contains a significant amount of nutrients and can be used as a fertilizer. The composition of the residue obtained during anaerobic digestion of livestock waste depends on the chemical composition of the feedstock loaded into the reactor. In conditions favorable for anaerobic digestion, about 70% of organic substances are usually decomposed, and 30% are contained in the residue.

The main advantage of anaerobic digestion is the preservation in organic or ammonium form of almost all nitrogen contained in the feedstock.

Questions for self-control

- What is the process of Bioremediation? What is the method of waste management?

- Describe categorization of technologies as in situ or ex situ.

- What material can increase biodegradation of soil?

- Name the types of polluting materials. Describe the definition of Xenobiotics.

- Make the analysis of Bioaugmentation and Biostimulation

- Describe the culture Pseudomonas putida. What is needed to use this culture for peeling of oil polluted soil?

- Describe the process of Biosorption.

- Name and describe the most pressing problem arising during remediation of pollutants which can be toxic for wild life and people in small concentration.

- Name and describe the most harmful metals.

- Give the estimation and describe biosorbents, biosorptions, and types  of sorptions.

- Explain the usage of sulfate-reducing bacteria in deposition of metals. Describe the conditions of their functioning.

- What is the accumulative ability of microorganisms to heavy metals?

- What is bacterial leaching?

- What does dairy industry include?

- What is treatment of dairy processing wastewaters?

- How do we produce milk whey?

- What is a complex mixture of a large number of organic compounds found in wine?

- What is wine made from? What affects the taste, color and smell?

- What are the main winewastes?

- What  secondary products can be obtained by recycling wine production?

- What  winery wastewater treatment do you know?

- Describe the full process of beer production. At what stage is brewer’s spent grain formed?

- What is the composition of the brewer’s spent grain?

- Where is it possible to use spent grains as a raw material?

- What methods are the basis of enrichment of brewers grain by proteins and vitamins?

- What is the main type of fungi used in upgrading spent grains?

- Describe the scheme of pulp and paper industry.

- What is pulp and paper industry?

- What can be done with the waste of paper production?

- Where and how was the word "paper" first used?

Chapter 9

Bioenergy and biofuels

Bioenergy is the energy produced from biofuels. Its purpose is to produce electricity or heat, as well as their joint production.

The name of this industry comes from the English word “bioenergy”, which has long been used as an energy term. Bioenergy is the production of energy from both solid biofuels, biogas, and liquid biofuels of various origins.

The main place in bioenergy belongs to biofuels, which can be of several types:  solid biofuels, liquid biofuels, gaseous biofuels.

Biofuels can be an effective replacement for traditional fuels only when their production costs are lower or energetically equivalent to the drilling and oil refining costs.

9.1 Bioethanol

Bioethanol uses alcohol made by fermentation, mostly from carbohydrates produced from sugar or starch crops such as corn or sugarcane. Cellulosic biomass, derived from non-food sources such as trees and grasses, is also being developed as a feedstock for ethanol production.

Technologies for the first (sugar or starch feedstock) and second generation (lignocellulosic feedstock) of bioethanol basically involve two stages: (i) the conversion of sunlight into chemical energy (such as carbohydrates and lipids); and (ii) the conversion of chemical energy into biofuel.

Figure 24. Bioethanol production by fermentation

The top five ethanol producers till 2010 were:

Brazil – 16500 billion liters;

The United States – 16270 billion liters;

China – 2000 billion liters;

The European Union – 950 billion liters; and

India – 300 billion liters

Figure 25. Raw materials for ethanol production in Europe (2008)

Bioethanol properties: colorless and clear liquid; used to substitute petrol fuel for road transport vehicles: one of the widely used alternative automotive fuels in the world (Brazil and USA are the largest ethanol producers); much more environmentally friendly; and with a lower toxicity level. The principle fuel used as a petrol substitute, bioethanol, is mainly produced by the sugar or cellulose fermentation process. Bioethanol is an alternative to gasoline for flexi fuel vehicles. Ethanol is a high-octane fuel and has replaced lead as an octane enhancer in petrol.

Advantages:

Exhaust gases of ethanol are cleaner due to complete combustion.

Ethanol-blended fuels such as E85 (85% ethanol and 15% gasoline) reduce up to 37.1% of GHGs (Greenhouse gases).

Output of energy during the production is higher than the input

Any plant containing sugar/starch can be used for production of bioethanol

Carbon neutral i.e.CO2 released in bioethanol production process= the amount the crops previously absorbed during photosynthesis.

Decreases ozone formation

Renewable energy resource

Energy security

Reduces the amount of high-octane additives

Fuel spills are more easily biodegraded or diluted to non-toxic concentrations

Disadvantages and Concerns:

Not as efficient as petroleum, energy content of the petrol is much higher than that of bioethanol. Its energy content is 70% of that of petrol.

Engines made for working on Bioethanol can burn working with much higher compression ratio.

Usage of phosphorous and nitrogen in the production produces a negative effect on the environment.

Cold start difficulties, pure ethanol is difficult to vaporize.

9.2 Microalgae cultivation and carbohydrate accumulation

Biochemical composition of microalgae grown under normal conditions, that is, without nutrient limitation, primarily encompasses proteins (30–50%), carbohydrates (20–40%), and lipids (8–15%)

Microphytes or microalgae are microscopic algae, typically found in freshwater and marine systems living in both water columns and sediment. They are unicellular species which exist individually, in chains or groups.

Cyanobacteria, also known as Cyanophyta, are a phylum of bacteria obtaining their energy through photosynthesis and the only photosynthetic prokaryotes able to produce oxygen.

Ethanol can be produced from microalgae and cyanobacteria.

Hydrolysis of biomass and fermentation have been extensively studied so far, however, feasibility of these processes needs further investigation.  Photo-fermentation is promising for the production of bioethanol. Only little information about photo-fermentation is available from industrial applications

There are three routes for producing bioethanol from such microorganisms:

1.The traditional one involving hydrolysis and fermentation of biomass with bacteria or yeast.

2.The dark fermentation route, and the use of engineered cyanobacteria or “photo-fermentation.”

3. In recent years, the use of engineered cyanobacteria for direct production of ethanol has gained enormous attention, mainly after the successful use of these microorganisms in industrial plants.

Figure 26. Production of different types of biofuels from microalgae

The increasing global demand for energy and advances in new biofuel production routes has intensified investigations of the potential of microalgae and cyanobacteria as a third generation of biofuels. Most investigations have been focused on using this type of biomass for producing biodiesel and biogas; however, more recent developments have indicated the potential of microalgae and cyanobacteria for the production of bioethanol.

However, only little information is available on the efficiency and the real advantages and disadvantages of these processes, and particularly, a comparison between traditional processes and engineered cyanobacteria. This study compiles the main publications on the production of bioethanol from microalgae and cyanobacteria, and summarizes the main features, advantages, and key aspects for each type of the process. The industrial implementation of these technologies depends on the capability of reducing production costs through more scientific research and technical development, and possibility to make them more competitive despite the lower cost of fossil fuels, also thanks to public subsidies.

The fact that the future of transportation is associated with the exploitation of liquid biofuels and the development of their production process is widely recognized.

However, biofuels can be an effective replacement for traditional fuels only when their production costs are lower or energetically equivalent to the drilling and oil-refining costs. At present, the biofuel sustainability is limited by the cost of production. In this study, the development of new cellular processes or enhancement of preexisting metabolic pathways in host microorganisms, and the production of rapid prototypes for testing are considered as crucial steps to converting biofuels into more feasible alternatives to the traditional fuels.

The use of algae, microalgae, and cyanobacteria for production of the third  generation of biofuels has many advantages over higher plants in view of producing first and second-generation biofuels.

This is due to their faster growth; capability of growing under various conditions, including wastewater; reduced need for water and other resource inputs; and the possibility of not occupying arable lands for their cultivation. The biochemical composition of microalgae grown under normal conditions, that is, without nutrient limitation, primarily encompasses proteins (30–50%), carbohydrates (20–40%), and lipids (8–15%).

Effort to increase yields of biofuels produced by microalgae is underway, including the optimization of light technologies to modify the carbon uptake pathways, aimed at a higher accumulation of biomass or specific compounds such as carbohydrates and lipids or, more recently, the use of genetic engineering for producing bioethanol, biohydrogen, and other special fermentation products.

Photosynthetic organisms are favorable for the production of biofuels, mainly because of their low cost of cultivation, but biofuel yields obtained under normal conditions are not satisfactory.  In addition to the production of biodiesel, microalgae and cyanobacteria serve as attractive feedstock for the production of bioethanol, although scientific and technological knowledge on this context is still scarce. On the contrary, numerous studies have documented that the contents of oil and carbohydrates in microalgae cells can be increased under stress conditions, resulting, for instance, in a decrease of the protein content under nitrogen depletion. This approach could be applied to cultivate microalgae biomass richer in carbohydrates, thereby leveraging their use for the production of bioethanol, which is currently the most widely used biofuel in the world.

Technologies for the first (sugar or starch feedstock) and second generation (lignocellulosic feedstock) of bioethanol basically involve two stages: (i) the conversion of sunlight into chemical energy (such as carbohydrates and lipids) and (ii) the conversion of chemical energy into biofuel. These two stages are related to each other and result in increased production costs. As an improvement of this process, the use of a single-stage system that is capable of capturing sunlight directly and converting it into biofuel (bioethanol) would avoid one step, thereby reducing the cost of production and increasing the sustainability of the bioethanol production process.

Figure 27. Biochemical processes of biofuel production

The first one is the traditional process in which the biomass undergoes pretreatment steps, enzymatic hydrolysis, and yeast fermentation. The second route is the use of metabolic pathways in dark conditions, redirecting photosynthesis to produce hydrogen, acids, and alcohols (such as ethanol). The third way is via “photofermentation,” which is impracticable in nature. The last route requires the use of genetic engineering to redirect the pre-existing biochemical pathways of microalgae for a more subjective and efficient production of bioethanol.

Photosynthesis is a vital process that drives the synthesis of all biofuels, by converting light energy into biomass, carbon storage products (e.g., carbohydrates and lipids), and a small amount of H2. In green algae, the light-harvesting complex (LHC) (chlorophylls and carotenoids) absorbs photons from sunlight as chemical energy. This energy is used by the photosystem II (PS II) for the catalytic oxidation of water to form protons, electrons, and molecular oxygen.

The production pathways and operating conditions vary for each biofuel. Several studies have already demonstrated the viability of industrial processes for the production of biodiesel, some of them suggesting the anaerobic digestion after extraction of lipid from algal biomass. However, studies aimed at consolidating suitable process for the production of bioethanol are still ongoing.

The production of bioethanol from microalgae and cyanobacteria is a feasible technological development, as they showed higher productivity than certain crops such as sugarcane and corn (already consolidated as feedstock for bioethanol production). Microalgae and cyanobacteria can reach 50% of their dry weight (DW) in carbohydrates, which can then be hydrolyzed and fermented with high yields. This fact will be better discussed later. They can also be easily concentrated, but the costs of cultivation are still high. As described earlier, three routes exist for the production of bioethanol from microalgae: (i) hydrolysis and fermentation of biomass; (ii) dark fermentation; and (iii) “photofermentation.”

This route is based on the production of microalgae biomass in photobioreactors succeeded by pretreament steps (breakdown of the cell structure and hydrolysis of the biomass), and frequently by the addition of enzymes. The treated biomass is then fermented with yeasts or bacteria to obtain ethanol. The main drawbacks of this route are the multistep processes required, which demands more energy, and the use of enzymes and yeasts, which accounts for a considerable proportion of the costs. On the contrary, the hydrolysis/fermentation process converts biomass at the highest rate, because of the well-known high efficiency of enzymes and yeasts in converting biomass into products.

Microorganisms with potential for bioethanol production in this way are selected primarily in accordance with their ability to accumulate carbohydrates, which depends on environmental and nutritional conditions. The main environmental factors are light intensity, pH, salinity, and temperature, while the nutritional factors include availability of the source for nitrogen, carbon, phosphorus, sulfur, and iron. The genera Scenedesmus, Chlorella, Chlorococcum, and Tetraselmis of the Clorophyta division and Synechococcus among other cyanobacteria have been extensively studied as feedstock for this type of bioethanol production.

The most common carbohydrates present in the microalgae and cyanobacteria that are used for the production of bioethanol are starch, glycogen, and cellulose. Starch is one of the largest microalgal sources of carbon, and it is an important feedstock for the production of bioethanol. The cellulose present inside the microalgae cell wall is also suitable as a feedstock for bioethanol production. The most common microorganisms used for ethanolic fermentation are yeasts of the genus Saccharomyces or bacteria of the genus Zymomonas.

Table. Carbohydrate content in some microalgae and cyanobacteria

Glycogen is a glucose polymer synthesized as an energy storage compound by cyanobacteria, and named cyanophycean starch. It has several interesting characteristics similar to starch, such as higher solubility in water and shorter polymer chains, apart from the fact that cyanobacteria can be easily hydrolyzed for producing bioethanol.

Regarding the process operating conditions, the microalgae biomass appears to require mild conditions for hydrolysis as well as for fermentation. Moreover, the acid and enzyme hydrolysis of microalgal biomass requires low amount of reactants, particularly in the case of enzymatic hydrolysis, to achieve high yields of conversion Biomass.

The most common carbohydrates present in the microalgae andcyanobacteria that are used for the production of bioethanol are starch, glycogen, and cellulose.

9.3 Fermentation of microalgae biomass

Regarding the conditions of operating process, the microalgae biomass appears to require mild conditions for hydrolysis as well as for fermentation.

Table. Hydrolysis and fermentation of microalgae biomass

Dark fermentation has been referred to as the conversion of organic substrates into biohydrogen. Fermentative and hydrolytic microorganisms hydrolyze complex organic polymers into monomers, which are subsequently converted into a mixture of organic acids of low molecular weight and alcohols, mainly acetic acid and ethanol. Various microalgae and cyanobacteria that are capable of expelling ethanol through the cell wall by means of intracellular process in the absence of light (46) include C. reinhardtii, Chlamydomonas moewusii, C. vulgaris, Oscillatoria limnetica, Oscillatoria limosa, Gleocapsa alpνcola, Cyanothece sp., Chlorococcum littorale, and Spirulina sp. e Synechococcus sp.

However, dark fermentation is disadvantageous in terms of hydrogen productivity, because approximately 80–90% of the initial chemical oxygen demand (COD) remains in the form of acids and alcohols after the process. Even under optimal operating conditions, typical yields vary only between 1 and 2 mol H2 per mol of glucose. The production of ethanol is favored by the accumulation of carbohydrates in the microalgae cells through photosynthesis, and then the microalgae are forced to synthesize ethanol through fermentative metabolism directly from their carbohydrate and lipid reserves when switching the growth to dark conditions. However, it can be concluded that dark fermentation of microalgae is not an efficient process for the production of bioethanol.

Photofermentation is a process of growing interest principally after the announcement of the installation of industrial plants where modified cyanobacteria are used to produce bioethanol directly. The “photofermentative” route (simply, Photanol) is a natural mechanism of converting sunlight into products of fermentation through a highly efficient metabolic pathway. Photanol is not only limited to ethanol production, but it is also used for a large number of naturally occurring products resulting from glycolysis-based fermentation.

Thus, several cyanobacteria species can be genetically modified by introducing specific fermentation cassettes through molecular engineering procedures, and then tested as a fermentative organism. Many studies have been performed on two models of cyanobacteria, Synechocystis sp. PCC 6803 and Synechococcus elongatus sp. PCC 7992. In addition to these Synechococcus sp., PCC 7002 and Anabaena sp. PCC 7120 have received much attention. Synechococcus sp. is a unicellular cyanobacterium that lives in freshwater that has been relatively well characterized.

It is capable of tolerating insertion of a foreign DNA to be transformed and replicated using shuttle vectors between Escherichia coli and cyanobacteria, or insertion of a foreign DNA into the chromosome through homologous recombination at selected active sites. Synechocystis sp. PCC 6803 was the first photosynthetic organism that had its genome sequenced, and one of the best characterized cyanobacteria. Thermosynechococcus is also naturally transformable.

9.4 The largest sources of carbon dioxide -- fossil fuels

The Rise of Renewable Energy. renewable energy sources, sun, wind and water

The carbon cycle and how sources release carbon dioxide into the atmosphere and sinks absorb or take in carbon dioxide from the atmosphere and store it. We will learn how fossil fuels were formed and how they're used to provide heat, energy, and transportation. And finally, we will learn about the impact of burning fossil fuels on the greenhouse effect.

Fossil fuels are natural materials, such as oil, coal, or gas, that are burned to create energy. Fossils are what remains of plants or animals that lived long, long ago. One example is the impression of a leaf or a small animal from millions of years ago that we see in a rock today.

Fossil fuels are also what remains from living things from millions and millions of years ago. Fossil fuels are formed from the decomposition of living organisms. The most common examples are oil, natural gas, and coal. Oil and natural gas were created when plants and animals that died sank to the bottom of the oceans and rivers and were buried by sediment. Those sediments would have been things like stones in sand that washed into the water and then sink to the bottom.

After time, there were many layers of sediment, and a great deal of pressure and heat were created. In this intense environment, the remains were decomposed. Compounds are composed of two or more separate elements. So to make a compound simpler, you take them apart, so that each compound has fewer elements. Millions of years later, these compounds became oil or natural gas.

Coal is created in a similar way. For coal, it was trees or other plants that were buried in wet swamplands. When the plants died, they fell into the bottom of the swamps. As time went by, more plants died, and they formed a thick layer at the bottom of the water. These layers were then buried by dirt and water. The heat and pressure under, at the bottom of the piles, cause chemical reactions. In the end, after the oxygen was pushed out and mostly carbon remained,

the materials that were left became coal. Oil, natural gases, and coal have all played an important part in helping the world develop since the Industrial Revolution, when modern machines were invented to make our lives better.

In modern society, many places in the world get their electricity so easily by simply plugging in a cord into a wall, but that electricity must be created somewhere. The largest share of greenhouse gas emissions come from burning fossil fuels, mostly coal and natural gases, to make electricity. In modern society, it is also very convenient and easy to get from one place toanother. You can visit another city or another country in one day, but most forms of transportation also depend on fossil fuels. Planes, trains, cars, trucks, ships -- they burn oil products, such as gasoline, petrol, or diesel, to make them run.

Factories that make everything that we fill our stores with, such as clothes, toys, electronics, and cars, all use the burning of fossil fuels to make their machines work and to heat their buildings. As we can see, most of us depend on burning fossil fuels for making almost everything we use and taking us almost everywhere we go. But burning so many fossil fuels does have an impact on the environment.

It pushes all the carbon dioxide that was stored in the Earth up into the atmosphere in a way that would not happen naturally in the carbon cycle. And as we learned before, carbon dioxide is a greenhouse gas. So when there is more of it floating around the atmosphere, that means more heat is trapped from escaping our greenhouse -- the Earth's atmosphere. In this video, we learned more about fossil fuels: how they were formed and how they are used to provide heat, energy, and transportation. And finally, we looked at the impact of burning fossil fuels on the greenhouse effect.

Fossil fuels release greenhouse gases into the atmosphere. And that has caused the earth to get warmer at a very fast and dangerous pace.

Scientists, are therefore, trying to discover new types of energy that won't have such a harmful effect on the environment. We'll start by looking at the different types of renewable energy. Then we'll look at some of their advantages and challenges of each.

Finally, we'll introduce some advances in technology that may help increase the use of renewable energy in the future. Let's start by explaining what renewable energy is.

Renewable energy is energy from a source that is not depleted when used.

Depleted means used up or emptied. Some of the most common examples are wind, water or solar power.

For example, if you get your energy from a windmill, you don't have to worry that someday you'll use up all the wind the same way that fossil fuels will someday be used up or depleted.

The main types of renewable energy sources are biomass energy, hydropower, geothermal energy, wind energy and solar energy.

Biomass energy is energy made from non-fossilized plant materials. In other words, plants that have not been turned into fossil fuels.

Wood and wood waste are common forms of biomass energy. In fact, a couple hundred years ago, before the use of coal and oil became so important, wood supplied 90% of the energy needed in the United States.

Another example of biomass is ethanol, which is fuel made of corn. For the most part, sources for biomass energy similar to fossil fuels have to be burned to release the energy. But there are other forms of renewable energy that are not burnt and do not release any greenhouse gases into the atmosphere. Let's look at some types of clean renewable energy.

Hydropower is energy created by moving water.

Geothermal energy is energy taken from the hot interior of the Earth.

geothermal

Wind energy is power from the wind, and solar energy is power created by capturing the radiation of the Sun.

These types of energies are different, because they are clean sources of renewable energy. This means, they do not pollute our atmosphere with greenhouse gases.

One challenge is these natural sources are not always available,.

There may be cloudy days, or days with low winds that can decrease the power from solar or wind energy.

Another challenge, in the past creating these types of energy has been more expensive than fossil fuels, so some countries that are developing quickly have chosen to use cheaper and easier methods to get fossil fuels to power their growth.

But many scientists are working hard to create new ways to help increase the use of these clean renewable sources of energy.

For example, they are discovering ways to store the extra energy collected on extra sunny days, so it can be used later on cloudy days.

All of this research is paying off. In the United States, the amount of electricity produced by renewable energy increased from 13% in 2012 to 22% in 2013.

It's predicted by 2020, 26% of the electricity in the US will be produced by renewable sources, and this increase is happening all over the world.

One great example is Denmark. In 2015, Denmark produced over 40% of its power by wind energy.

We're  going to take a closer look at some of the most common renewable energy sources, sun, wind and water. We'll look at the technologies, how they produce energy, and look at the advantages and disadvantages of each one. Let's begin by looking at how we can get power from the sun.

We call this solar power. The sun gives off energy in the form of heat. We can use something called a solar panel to turn that heat into energy. A solar panel is made up of lots of silicon cells joined together.

Silicon, which has the chemical symbol Si, is a semiconductor. This means that it sometimes conducts electricity. The technology is quite complicated.

But in simple terms, a solar panel is like a sandwich of two layers of silicon.

The bottom layer does not have enough electrons, and the top layer has more than it needs. When photons, small particles of light from the sun, hit the silicon sandwich, they knock electrons from the bottom layer to the top.

This creates electricity. Solar panels have many advantages compared to fossil fuels.

They produce clean energy with very little waste that harms the environment. They can be installed on the tops of houses, so each home can be responsible for creating its own energy.

After the solar panels have been installed, they save people money.

Most importantly, any country in the world can used the power of the sun to create energy. So countries do not have to depend on other countries for their energy.

There are two strong disadvantages, the cost of buying the solar panels and the fact that the sun does not shine all the time.Consequently, the panels either need an efficient way to store energy,or they need to be used with another power source.

Second, let's talk about the power of wind.

Modern windmills are called wind turbines. And many of them are placed together to create a wind farm.A turbine is a machine for producing power in which a wheel goes around.When the wind turns the blades of the wind turbine,

it is connected to a shaft, which spins a generator and creates electricity

Similar to solar power, wind power is a clean, naturally occurring energy that can be created wherever the wind blows.After the wind farm has been set up, it is easy and cheap to fix, and allows countries to create their own electricity without help from other countries.In contrast to solar power, it can be generated 24 hours a day because winds blow at night, too. However, there are a few disadvantages to wind power. A wind farm can be expensive to create, and some people believe that wind farms are unattractive to look at.

The best wind sites are not usually close to cities, which use the electricity, so the power often has to be transported a long way

Third, let's look at water power.When we refer to energy created by water, we use the word hydro from Greek, so we call it hydroelectric power. Hydroelectric power is created by converting falling water into electricity. The process is similar to wind power. The water falls onto a turbine that spins a generator, creating electricity.Similar to solar and wind power, hydroelectric power is a clean energy with no waste product.

The water is either used again or just continues flowing. The water also flows constantly, so like wind power it can produce electricity 24 hours a day.

Also, since the flow of water can be controlled, more energy can be created at peak times, the times of day when people use more energy.

There are a couple of disadvantages. The water has to fall a long way to create the force to turn the turbines. Therefore, you need to build a dam on a river

to stop most of the water from flowing. This can be expensive to create, and it can cause problems for the land around the river, such as flooding.

Also, people often need to leave their towns or farms when the dam is built.

Solar power from the sun, wind power, and hydroelectric power from water.

We looked at the way that they work, and talked about some of the advantages and disadvantages of each.

Chapter 10

Waste management in biotechnological industry

Today, biotechnology is developing at a high pace. As a science, it studies the production processes, which are based on the practical use of microorganisms and all kinds of biological systems. These are not only plant or animal tissues, but also protoplasts, recombinant DNA, and fully genetically modified organisms. Advances in biological science over the past 30 years have led to the development of new methods of work with microorganisms. There is a real opportunity to create living beings with unusual properties useful for humans. The prospects in many areas of biotechnology based on modern methods promise fantastic economic and environmental benefits in the future.

There are such problems in the world as:

* lack of fresh or purified water (in some countries);

* environmental pollution by various chemicals;

* energy resource deficit;

* the need to improve and produce completely new environmentally friendly materials and products; and

* to improve the level of medicine.

Scientists believe that it is possible to solve these and many other problems with the help of biotechnology.

The study of biotechnology is not only related to the biological sciences. Ion-selective field effect transistors (HpaI) have been developed in microelectronics. Biotechnology is needed to improve oil recovery. The most developed area is the use of biotechnology in the environment for treatment of industrial and domestic wastewater. Many other disciplines have contributed to the development of biotechnology, therefore biotechnology should be referred to integrated sciences.

Biotechnological processes cause lower pollution of the environment with waste and by-products than other methods, in addition, they do not significantly depend on climatic and weather conditions, do not require large land areas, do not need the use of pesticides, herbicides and other agents alien to the environment. Therefore, biotechnology as a whole and its individual sections are among the most priority areas of scientific and technological progress and are a powerful example of "new technologies", which are associated with the prospects for the development of many industries. All highly developed countries of the world consider biotechnology as one of the most important modern industries, considering it as a key method of industrial reconstruction in accordance with the needs of the time, and take measures to stimulate its development.

A special place and role in the complex of biotechnological sciences and processes belongs to environmental biotechnology in connection with the increase in environmental problems. Environmental biotechnology is a special application of biological systems and processes to solution of the problems of environmental protection and environmental management. These processes include the disposal of agricultural, household and industrial waste, treatment of effluents and air and gas emissions, degradation of xenobiotic, production of effective and non-toxic drugs to combat diseases and pests of cultivated plants and domestic animals, as well as creation of alternative and environmentally friendly ways for producing energy and mining.

The conceptual basis of environmental education should be considered the concept of transition of the Republic of Kazakhstan to sustainable development for 2007-2024. Sustainable development is necessary to achieve the goals of Kazakhstan development strategy until 2030. For Kazakhstan, the transition to sustainable development is an urgent need. Environmental education intend to develop and consolidate more advanced stereotypes of human behavior aimed at:

- saving natural resources;

- prevention of environmental pollution;

- universal preservation of natural ecosystems;

- promotion of joint environmental action and implementation of a common environmental policy in the state.

Biotechnology  is one of the main disciplines in the training of future specialists - biotechnologists, ecologists, biologists, as it has a high potential for the formation and development of environmental concepts and knowledge; for practical solutions to the environmental problems and the rational use of natural resources. Specific application of biotechnological methods to solution to the environmental problems, such as waste treatment, water, soil and air purification, pollution removal, is one of the subjects of environmental biotechnology. Biotechnology  is a modern approach to the protection and preservation of the environment by combining the achievements of biochemistry, microbiology, genetic engineering, and chemical technologies. Biotechnology companies have developed a number of methods for the identification of chemical and biological pollutants in soil using monoclonal antibodies and polymerase chain reaction. Researchers in laboratories based on monoclonal antibodies have developed biosensors to detect the presence of explosives in the soil of the old war affected areas. These techniques are much cheaper and faster than the laboratory methods the implementation of which requires complex and expensive equipment, and they are portable and can be used in field conditions.

In the field of bioremediation the most actual studies are purification of soil from oil products, pesticides, heavy metals, chlororganic compounds by the methods in situ. In the field of recycling solid waste the methods of composting, vermicomposting and vermicultivation are widely used. Biological preparations, biofertilizers and other biological materials for recultivation, restoration, greening of urban landscapes, restoration of soil fertility, protection of soils, coastlines, engineering structures and others have been created in the last 5-10 years.

Extensive experience and research in the field of studying the composition of biocenoses in the process of pollution and self-purification of water from pollutants (in particular, from oil pollution, heavy metals); the use of specialized populations of genetically engineered strains of microorganisms, sensitive and selective methods of analysis of toxicants in the environment based on biosensors, biotests and bioindicator systems have been developed.

One of the big obstacles to the establishment of bio-processing plants is the cost of fermented sugars (carbohydrates). Today, in the process of typical fermentation of ethanol, starch of food crops is mainly used. Biotechnology can help in the conversion of plant cellulose in the form of fermentable sugars. New advanced enzymes are the key to this process.

In the microbiological techniques bacteria are used for extraction of metals, the so-called "leaching" in the processing of poor ores or dump materials, which are accumulated during the development of the open production of ore. This technique called biogeotechnology is used to produce gold, silver and nonferrous metals.

Environmental biotechnology allows us to produce various bioproducts to protect and restore nature: biosorbents, biocatalysts, preparations for technologies of bioremediation of contaminated environments, biotechnological methods for phytoremediation of waste recycling and by-products in the industry and agriculture.

Nowadays the bank of ecologically safe beneficial types of microorganisms is created. Preparations based on them are characterized by a complex action. They have a stimulating effect on plant growth, suppress a number of diseases, improve the mineral nutrition of plants, enhance soil fertility, and significantly reduce the pesticide load. Application of a new generation of biological products not only increases the productivity of the plants but also allows us to get an earlier production improving its safety.

-  All microbial drugs have a wide range of action, they can reduce the norms of application of mineral fertilizers and toxic chemicals, which will has a positive effect on the content of nitrate and nitrite in products and reduce the pesticide load on ecosystems.

-  Biotechnology of composting organic waste occupies a special place. In the natural process of composting many types of microorganisms take part e.g., more than 2,000 species of bacteria and about 50 species of fungi.

-  Microorganisms are used in aerobic biological treatment of wastewater in the aeration tank or biofilters. The activated sludge contains 70% of living organisms, mainly, bacteria (about 30 species). In the activated sludge there is also simple species (protozoa), which regulate bacterial population.

-  Microbial products find their application in recycling of waste and also in oil industry for improvement of oil recovery, oil prospecting, search for the place of oil origin, wastewater treatment in petroleum industry. In the mining industry they are used for leaching of metals from ores, fight with aggressive waters in the mines for the desulfurization of coal mining and purification of mining waste water.

- In the future environmental biotechnology should create completely harmless and rational processes of conversions of agricultural products and chemical raw materials into biologically harmless forms.

10.1 Medical Waste. Major Types of Medical Waste. Effective Medical Waste Management

Medical waste is a kind of hazardous waste generated by hospitals, clinics, health centers and laboratories, etc. Medical waste must be properly handled and disposed of so as not to cause environmental and sanitary pollution and endanger public health.

Gient, founded in 1995, has dedicated in medical waste treatment industry for 20 years. As the leading supplier of medical waste treatment systems in China, we are committed to providing comprehensive, reliable and environmentally friendly medical waste disposal solutions for centralized medical waste treatment plants as well as hospitals.

Nowadays, Gient medical waste treatment systems are running successfully nearly 40 countries and across 5 continents. Our clients include UNITED NATIONS organizations, centralied medical waste treatment plants and hospitals, etc.

Two Major Types of Medical Waste

  - General solid waste

    - Similar to domestic waste

    - Includes paper, plastic, packaging, food prep

    - No patient contact

    - 75 – 90% of healthcare facility waste is hazardous

  - Hazardous waste

Hazardous Waste includes:

    - Infectious waste (except sharps and waste from patients with highly infectious diseases)

    - Small quantities of chemicals and pharmaceuticals

    - Non-recyclable pressurized containers

  - Highly Hazardous waste is a separate category

  - Highly Hazardous includes:

    - Sharps

    - Highly infectious non-sharp waste

    - Stools from cholera patients

    - Bodily fluids of patients with highly infectious diseases

    - Large quantities of expired or unwanted pharmaceuticals and hazardous chemicals and radioactive wastes

    - Genotoxic wastes (affecting genetic composition and multiple generations)

    - Teratogenic wastes (affecting development of the exposed individual)

What are the Risks

- Physical injury

    - Cuts, punctures (e.g., from sharps)

- Disease transmission

    - Greatest and most immediate threat

    - HIV/AIDS, hepatitis B & C

- Water supply contamination

    - Infectious stools or bodily fluids

- Chemical and toxic

    - Pharmaceuticals

    - Heavy metals (mercury, cadmium)

Mandatory Environmental Review

Reg. 216 applies!

Establishment or rehabilitation of health centers or clinics

Training or technical assistance to health care providers or institutions—”health system strengthening”

Environmental analysis (EA or IEE) prepared to determine nature and extent of risks

EA/IEE conditions specify mitigation and monitoring criteria for medical waste management

Segregation and disposal

Use of personal protective equipment (PPE); training

Effective Medical Waste Management

Segregation & Disposal: Non-hazardous solid waste

Treat same as “domestic waste”

Reduces waste quantity, cost, risk from and to scavengers and workers

Manage as close to point of generation as possible

Disposal options include landfilling, incineration

Do not incinerate plastic, PVC or packaging—incineration produces dioxins, furans etc.

Wastes and Appropriate Receptacles

10.2 Effective management of agricultural wastes. Classification and causes/sources of agricultural wastes

Agricultural waste refers to waste produced from agricultural operations, including waste from farms, poultry houses, and slaughterhouses.

In addition to being potentially damaging to the environment, agricultural waste may expose workers to harmful biological material (biohazards).

However, these wastes may constitute a serious threat to human health through environmental pollution and handling them may result in huge economic loss. Unfortunately, in many developing countries where large quantities of these wastes are generated, they are not properly managed because little is known about their potential risks and benefits if properly managed.

Classification and causes/sources of agricultural solid wastes

- Pesticides and herbicides are substances used to help control pests like insects, arachnids, rodents, and weeds. While these products are undeniably helpful, they also pose certain dangers.

- Accidental exposure to pesticides can occur through ingestion, inhalation, and/or skin absorption. Once exposed, pesticides can harm organisms including pets, livestock, wildlife, and people.

- In addition to poisoning, “The potential [negative] environmental impacts from pesticide disposal are air, soil, and water contamination from releases and accidental exposure of humans and animals.”

- The environmental implications concerning improper disposal are the same as for the application process, except that the concentration of the pesticide may be stronger because of the quantity and mass of the disposed pesticide. The disposal of pesticides is a critical process; if not properly conducted it can have immediate, detrimental effects on the environment.

Effective management of agricultural wastes

There are options on how agricultural solid wastes could be handled. Traditionally, shafts from palm oil processing could be used as fuel in fuel wood for cooking and heating. In the recent time, some of these wastes are put into better uses. Some of these agricultural solid wastes could be used as additives in cement mixes, water glass manufacturing, paper making, ethanol production, animal feed, electricity and biogas generation, heavy metal removal, mulching, organic fertilizers, and compost. An effective means of managing agricultural solid wastes is to recycle them to produce useful products. This can be achieved through:

Food wastage is an important source of agricultural solid wastes. Hence, the prevention of food wastage at all levels before they are created will salvage some of these wastes and prevent unnecessary ill-health and environmental disadvantages as well as huge economic losses. This can be achieved through proper education and awareness of those involved with agricultural activities at all levels as well as being a little more generous by feeding hungry people with fresh food instead of keeping them till they are spoilt. There are hungry people everywhere in the world. Feeding animals saves food scraps and bioconversion of agricultural by-products, which may turn to agricultural solid wastes if their values are not enhanced and will go a long way in preventing such wastes as well. Composting and conversion of agricultural solid wastes to a renewable energy source is another effective way of managing agricultural solid wastes. It is high time attention is focused on turning these huge potential agricultural solid wastes to wealth, particularly in developing countries. To make our world safer for us to live, all hands must be on deck. Research activities should be geared toward commercial scaling of some productive findings made toward the efficient recycling of agricultural wastes.

10.3 Food waste and its disposal. The problem of population growth, hunger, malnutrition, emissions and efficient processing and management of food waste

Food loss and waste is food that is not eaten. The causes of food waste or loss are numerous and occur throughout the food system, during production, processing, distribution, retail and food service sales, and consumption. Overall, about one-third of the world's food is thrown away.

A 2021 metaanalysis that did not include food lost during production, by the United Nations Environment Programme found that food waste was a challenge in all countries at all levels of economic development.

The analysis estimated that global food waste was 931 million tonnes of food waste (about 121 kg per capita) across three sectors: 61 per cent from households, 26 per cent from food service and 13 per cent from retail.

Landfills and greenhouse gases Dumping food waste in a landfill causes odour as it decomposes, attracts flies and vermin, and has the potential to add biological oxygen demand (BOD) to the leachate. The European Union Landfill Directive and Waste Regulations, like regulations in other countries, enjoin diverting organic wastes away from landfill disposal for these reasons.

Fodder and insect feed                                    Large quantities of fish, meat, dairy and grain are discarded at a global scale annually, when they can be used for things other than human consumption. The feeding of food scraps or slop to domesticated animals such as pigs or chickens is, historically, the most common way of dealing with household food waste. The animals turn roughly two thirds of their ingested food into gas or fecal waste, while the last third is digested and repurposed as meat or dairy products. There are also different ways of growing produce and feeding livestock that could ultimately reduce waste.

- As alternatives to landfill, food waste can be composted to produce soil and fertilizer, fed to animals or insects, or used to produce energy or fuel. Some wasted fruit parts, can also be biorefined to extract useful substances for the industry (i.e. succinic acid from orange peels, lycopene from tomato peels).

- Composting                                                                                                                                  Food waste can be biodegraded by composting, and reused to fertilize soil. Composting is the aerobic process completed by microorganisms in which the bacteria break down the food waste into simpler organic materials that can then be used in soil. By redistributing nutrients and high microbial populations, compost reduces water runoff and soil erosion by enhancing rainfall penetration, which has been shown to reduce the loss of sediment, nutrients, and pesticide losses to streams by 75–95%.

The problem of population growth, hunger, malnutrition

- Reducing food losses and food waste has a triple advantage – for the climate, for food security and for the sustainability of our food systems. In our time, when food shortages, malnutrition and hunger are growing all over the world, this task should become a priority.

Today, for many of us on the planet, food has become taken for granted. But for millions of people who are hungry

We live in turbulent times. Two years of the COVID-19 pandemic, climate change and the war in Ukraine disrupted the global food system, putting millions of people in conditions of food insecurity. According to the report "The State of Food security and Nutrition in the world" released in 2022, in 2021 the number of people suffering from hunger worldwide increased to 828 million, which is 46 million more compared to 2020 and 150 million more than in 2019. In total, approximately 3.1 billion people worldwide do not have access to a healthy diet.

To feed 9 billion man – and this will soon be the population of the Earth – it is necessary to increase agricultural productivity by at least 70%. It is impossible to do this using traditional methods, so there is only one way out — a biotechnological revolution in agriculture. For example, to increase food production, new varieties of wheat, barley, sunflower and rapeseed were created in Australia, the USA, Canada and some other countries.

Such varieties are resistant to frost, drought, pests, respectively, compared with previous years, the yield has increased markedly.

# 10.4 Biofilter made from peanut shell degrades air pollutants

(Nanowerk News) In order to clean the air of pollutants such as methanol and solvents used in the industry, biotechnology expert Raul Pineda Olmedo, of the National University of Mexico (UNAM), designed a biofilter that uses microorganisms living in the shell of the peanut.

The research from the department of Environmental Technology noted that microorganisms grow naturally on peanut shell, which can be used to clean the air. Furthermore, in Mexico this material is generated in large amounts and is considered a worthless agricultural residue.

The idea is a prototype filter with peanut shells, which cultivates the microorganisms to degrade toxic pollutants into carbon dioxide and water, thereby achieving clean air.

"The peanut shell is special for these applications because it is naturally hollow and has an area of contact with air, which favors the development of microorganisms," said Pineda Olmedo.

He also said it has been observed that this organic material can be applied to biotechnology as biological filters similar to those used by cars, but instead of stopping dust it can degrade the contaminants.

Olmedo Pineda development focuses on solving the problem of air pollution in companies dedicated to handling inks or solvents, which have a contaminated workplace.

The experiment was developed in collaboration with doctors Frédéric Thalasso Sire and Fermin Perez Guevara from the Research Center of Advanced Studies (CINVESTAV) in Mexico.

The prototype is similar to a bell or kitchen extractor, but it not only absorbs and stores polluting vapors, it degrades and purifies the air.

The design consists of a filter made with peanut shells containing microorganisms, which purify the air. For optimum development it should be in a temperature controlled environment.

Olmedo Pineda explained that the filter takes on average 28 days to synthesize microorganisms such as Fusarium and Brevibacterium. Bacteria and fungi take the carbon from pollution to reproduce and breathe.

In Mexico this technology has not been exploited extensively. The researcher currently seeks to commercialize the innovation, which is a solution applicable to everyday life. They will create a demonstration prototype for schools, making it accessible to students, who can apply and replicate it.

# 10.5 Major Advance in Artificial Photosynthesis Poses Win/Win for the Environment

A potentially game-changing breakthrough in artificial photosynthesis has been achieved with the development of a system that can capture carbon dioxide emissions before they are vented into the atmosphere and then, powered by solar energy, convert that carbon dioxide into valuable chemical products, including biodegradable plastics, pharmaceutical drugs and even liquid fuels.

Scientists with the U.S. Department of Energy (DOE)’s Lawrence Berkeley National Laboratory (Berkeley Lab) and the University of California (UC) Berkeley have created a hybrid system of semiconducting nanowires and bacteria that mimics the natural photosynthetic process by which plants use the energy in sunlight to synthesize carbohydrates from carbon dioxide and water. However, this new artificial photosynthetic system synthesizes the combination of carbon dioxide and water into acetate, the most common building block today for biosynthesis.

“We believe our system is a revolutionary leap forward in the field of artificial photosynthesis,” says Peidong Yang, a chemist with Berkeley Lab’s Materials Sciences Division and one of the leaders of this study. “Our system has the potential to fundamentally change the chemical and oil industry in that we can produce chemicals and fuels in a totally renewable way, rather than extracting them from deep below the ground.”

Yang, who also holds appointments with UC Berkeley and the Kavli Energy NanoSciences Institute (Kavli-ENSI) at Berkeley, is one of three corresponding authors of a paper describing this research in the journal Nano Letters. The other corresponding authors and leaders of this research are chemists Christopher Chang and Michelle Chang. Both also hold joint appointments with Berkeley Lab and UC Berkeley. In addition, Chris Chang is a Howard Hughes Medical Institute (HHMI) investigator. (See below for a full list of the paper’s authors.)

The more carbon dioxide that is released into the atmosphere the warmer the atmosphere becomes. Atmospheric carbon dioxide is now at its highest level in at least three million years, primarily as a result of the burning of fossil fuels. Yet fossil fuels, especially coal, will remain a significant source of energy to meet human needs for the foreseeable future. Technologies for sequestering carbon before it escapes into the atmosphere are being pursued but all require the captured carbon to be stored, a requirement that comes with its own environmental challenges.

The artificial photosynthetic technique developed by the Berkeley researchers solves the storage problem by putting the captured carbon dioxide to good use.

“In natural photosynthesis, leaves harvest solar energy and carbon dioxide is reduced and combined with water for the synthesis of molecular products that form biomass,” says Chris Chang, an expert in catalysts for carbon-neutral energy conversions. “In our system, nanowires harvest solar energy and deliver electrons to bacteria, where carbon dioxide is reduced and combined with water for the synthesis of a variety of targeted, value-added chemical products.”

By combining biocompatible light-capturing nanowire arrays with select bacterial populations, the new artificial photosynthesis system offers a win/win situation for the environment: solar-powered green chemistry using sequestered carbon dioxide.

“Our system represents an emerging alliance between the fields of materials sciences and biology, where opportunities to make new functional devices can mix and match components of each discipline,” says Michelle Chang, an expert in biosynthesis. “For example, the morphology of the nanowire array protects the bacteria like Easter eggs buried in tall grass so that these usually-oxygen sensitive organisms can survive in environmental carbon-dioxide sources such as flue gases.”

The system starts with an “artificial forest” of nanowire heterostructures, consisting of silicon and titanium oxide nanowires, developed earlier by Yang and his research group.

“Our artificial forest is similar to the chloroplasts in green plants,” Yang says. “When sunlight is absorbed, photo-excited electron−hole pairs are generated in the silicon and titanium oxide nanowires, which absorb different regions of the solar spectrum. The photo-generated electrons in the silicon will be passed onto bacteria for the CO2 reduction while the photo-generated holes in the titanium oxide split water molecules to make oxygen.”

Once the forest of nanowire arrays is established, it is populated with microbial populations that produce enzymes known to selectively catalyze the reduction of carbon dioxide. For this study, the Berkeley team used Sporomusa ovata, an anaerobic bacterium that readily accepts electrons directly from the surrounding environment and uses them to reduce carbon dioxide.

“S. ovata is a great carbon dioxide catalyst as it makes acetate, a versatile chemical intermediate that can be used to manufacture a diverse array of useful chemicals,” says Michelle Chang. “We were able to uniformly populate our nanowire array with S. ovata using buffered brackish water with trace vitamins as the only organic component.”

Once the carbon dioxide has been reduced by S. ovata to acetate (or some other biosynthetic intermediate), genetically engineered E.coli are used to synthesize targeted chemical products. To improve the yields of targeted chemical products, the S. ovata and E.coli were kept separate for this study. In the future, these two activities – catalyzing and synthesizing – could be combined into a single step process.

A key to the success of their artificial photosynthesis system is the separation of the demanding requirements for light-capture efficiency and catalytic activity that is made possible by the nanowire/bacteria hybrid technology. With this approach, the Berkeley team achieved a solar energy conversion efficiency of up to 0.38-percent for about 200 hours under simulated sunlight, which is about the same as that of a leaf.

The yields of target chemical molecules produced from the acetate were also encouraging – as high as 26-percent for butanol, a fuel comparable to gasoline, 25-percent for amorphadiene, a precursor to the antimaleria drug artemisinin, and 52-percent for the renewable and biodegradable plastic PHB. Improved performances are anticipated with further refinements of the technology.

“We are currently working on our second generation system which has a solar-to-chemical conversion efficiency of three-percent,” Yang says. “Once we can reach a conversion efficiency of 10-percent in a cost effective manner, the technology should be commercially viable.”

10.6 Management of household waste. The problem of solid waste disposal. Bioplastics as alternative

Household waste is waste that accompanies human activity throughout life. Or, so to speak, "household garbage": food waste, waste paper, glass and plastic containers, etc.

The problem of solid waste disposal, especially in large cities, is particularly acute. Household garbage is partially exported to suburban landfills intended for their burial, partially "settles" in unauthorized landfills. However, these household wastes have negative effects on the environment, health, and cause negative impacts to any living organisms in the surrounding.

Since these organic wastes will nevertheless, decay and decompose, the decomposing mass becomes good breeding grounds for most bacteria and fungi, which poses a serious health threat to human life. Environmentally, rotting food products changes the ecosystem by affecting the eating habits of the surrounding animals.

Large amounts of household wastes pose major problems especially when they have accumulated to high levels and the systems of waste management are poor. The type of waste disposal adapted for the household waste disposal determines whether the problems occurring from the accumulation of the waste continues or not.

Poorly designed or poorly managed landfills create a number of adverse environmental impacts including windblown litter and attraction of vermin. This problem is mainly experienced in the urban settings especially in developing countries where plastic shopping bags and wrapping papers are highly used. Winds blowing normally carry the light papers from the dump pits and litter the surrounding. On the other hand, the attraction of vermin such as mice and rats becomes evident in the dumpsites from where they get food and shelter consequently becoming their breeding grounds.

With organic wastes especially from the kitchen, decaying is inevitable and the decay results from anaerobic breakdown of the organic waste releasing methane and carbon dioxide.

These gases cause a major problem, air pollution, health problems and can destroy surface vegetation. Health wise, these gases cause respiratory problems, which are expensive to treat.

Large amounts of garbage also cause damage to the infrastructure especially roads in areas where heavy trucks are used to collect and transport the waste to the dumpsites. The fatal accidents here involve scavengers buried in the waste piles. Damage to these infrastructures together with the landfill operations poses environmental noise pollution to both animals and human beings living in the surroundings. Moreover, due to the decay and decomposition of the organic waste in these landfills there is a problem of contamination of the underground water and aquifer through leeching.

For people living close to these dumpsites, they frequently suffer from diseases such as cholera due to contamination of both water and food by dirt from the rotting garbage.

Burning of plastics in the dump sites also contribute to health problems in that, smoke causes respiratory problems especially to small children.

Large household waste being a major problem in most developing countries on both the environment and health of the people, proper waste management policies and mitigation procedures have been set in place to curb these problems.

The growth of the comfort of human existence is accompanied by problems that are caused by the introduction of new technologies into modern life. For example, the cheapness of disposable plastic packaging expensively affects the health of people and the environment.

Pollution of the environment by household waste affects a person through air, water, food of plant origin, grown on soil poisoned by garbage. Chemical compounds entering the soil accumulate and lead to a gradual change in its chemical and physical properties, reduce the number of living organisms, worsen fertility. Together with pollutants, pathogenic bacteria, helminth eggs and other harmful organisms often enter the soil.

Mankind has invented compounds that do not decompose. These include various packaging materials, containers for storing liquids, rubber, dacron, synthetic polymers, detergents, dyes. All of them emit substances harmful to the environment and people.

Currently, recycling of garbage is becoming a traditional phenomenon only in a few countries, but the need for its more active application is important. Waste disposal methods such as landfill disposal and incineration are not harmless. Landfills emit methane gas, which creates a greenhouse effect threatening our planet, keeping heat in the earth's atmosphere.

Waste incineration also leads to the release of dangerous gases containing toxic heavy metals: cadmium, mercury, lead. Entering the body, they can affect the function of hematopoiesis, cause changes in blood composition, contribute to the development of carcinogenic, genetic and other long-term biological effects. Increased release of methane, oxygen, carbon dioxide, can cause suffocation of a person.

The role of synthetic materials, plastic objects in accumulated places

Plastics are made up of synthetic organic polymers which are widely used in different applications ranging from water bottles, clothing, food packaging, medical supplies, electronic goods, construction materials, etc. In the last six decades, plastics became an indispensable and versatile product with a wide range of properties, chemical composition and applications. Although, plastic was initially assumed to be harmless and inert, however, many years of plastic disposal into the environment has led to diverse associated problems. Environmental pollution by plastic wastes is now recognized widely to be a major environmental burden, especially in the aquatic environment where there is prolong biophysical breakdown of plastics, detrimental negative effects on wildlife, and limited plastic removal options.

In many instances, sheeting and packaging plastics are disposed of after usage, however, because of their durability, such plastics are located everywhere and persistent in the environment. Research on the monitoring and impacts of plastic wastes is still at the infancy stage, but thus far, the reports are worrisome. In human occupational and residential environment, plastics made of petrol-based polymer are present in high quantity. At the end-of-life of these plastics, they are usually land-filled together with municipal solid waste. Plastics have several toxic constituents among which are phthalates, poly-fluorinated chemicals, bisphenol A (BPA), brominated flame retardants and antimony trioxide which can leach out to have adverse effects on environmental and public health. Plastics in electronic waste (e-waste) have become a serious global environmental and public health concern due to its large production volume and the presence of inadequate management policies in several countries.

Plastic is a polymeric material—that is, a material whose molecules are very large, often resembling long chains made up of a seemingly endless series of interconnected links. Natural polymers such as rubber and silk exist in abundance, but nature’s “plastics” have not been implicated in environmental pollution, because they do not persist in the environment. Today, however, the average consumer comes into daily contact with all kinds of plastic materials that have been developed specifically to defeat natural decay processes—materials derived mainly from petroleum that can be molded, cast, spun, or applied as a coating. Since synthetic plastics are largely nonbiodegradable, they tend to persist in natural environments. Moreover, many lightweight single-use plastic products and packaging materials, which account for approximately 50 percent of all plastics produced, are not deposited in containers for subsequent removal to landfills, recycling centres, or incinerators. Instead, they are improperly disposed of at or near the location where they end their usefulness to the consumer. Dropped on the ground, thrown out of a car window, heaped onto an already full trash bin, or inadvertently carried off by a gust of wind, they immediately begin to pollute the environment. Indeed, landscapes littered by plastic packaging have become common in many parts of the world. (Illegal dumping of plastic and overflowing of containment structures also play a role.) Studies from around the world have not shown any particular country or demographic group to be most responsible, though population centres generate the most litter. The causes and effects of plastic pollution are truly worldwide.

According to the trade association PlasticsEurope, worldwide plastic production grew from some 1.5 million metric tons (about 1.7 million short tons) per year in 1950 to an estimated 275 million metric tons (about 303 million short tons) by 2010 and 359 million metric tons (nearly 396 million short tons) by 2018; between 4.8 million and 12.7 million metric tons (5.3 million and 14 million short tons) are discarded into the oceans annually by countries with ocean coastlines.

Compared with materials in common use in the first half of the 20th century, such as glass, paper, iron, and aluminum, plastics have a low recovery rate. That is, they are relatively inefficient to reuse as recycled scrap in the manufacturing process, because of significant processing difficulties such as a low melting point, which prevents contaminants from being driven off during heating and reprocessing. Most recycled plastics are subsidized below the cost of raw materials by various deposit schemes, or their recycling is simply mandated by government regulations. Recycling rates vary dramatically from country to country, and only northern European countries obtain rates greater than 50 percent. In any case, recycling does not really address plastic pollution, since recycled plastic is “properly” disposed of, whereas plastic pollution comes from improper disposal.

Recommendations on Reduction and Control of Plastic Wastes

Many countries are laboring on controlling environmental pollution by plastic wastes by reducing plastics and plastic product's production, prohibition of excessive packaging, litter capture and recycling. In the struggle against plastic pollution, the following recommendations might be helpful:

Policy making

To combat and curb persistent environmental pollution by plastics, there is need for realistic policies which must be properly followed and enforced. This should include the need for global convention on environmental pollution by plastics to mandate plastic producers to declare all ingredients in their plastic products and put a warning on the products for consumers about the potentially health effects of such constituents. Policies to classify some of the harmful ingredients in plastic products should be enacted.

Plastic waste management and recycling

In reducing toxic effects of plastic wastes on the environment and public health, waste management plays a major role. For global reduction of plastic litters and ocean pollution, there is need for improvement in proper plastic waste collection, treatment and disposal. Inadequate management of landfills will make way for harmful chemicals in plastic wastes to leach into the environment, polluting the soil, air and underground water.

Education and public awareness

Efforts must be made to educate the general populace on the potential environmental and public health effect of pollution by plastic wastes. This will go a long way to reduce the pollution rate and preserve the quality of the environment. There is need for people to be aware of the chemical constituents of plastic products and their health effects. Educational curriculums at different levels must include ways of plastic pollution reduction and waste management systems as information resources.

Bioplastics as alternative

Bioplastics is a plastic produced from cellulose that is made of wood pulp by a British chemist in the 1850s. Now, bioplastics can be produced from different biodegradable and non-biodegradable materials including weeds, hemp, plant oil, potato starch, cellulose, corn starch, etc. Sugar-based bioplastics can biodegrade under normal conditions for composting. Bioplastics are environmentally friendly since they require less fossil fuels during production in comparison to other types of plastic.

Although bioplastics have been used commercially in just few applications, they are widely used in consumer goods for items that are disposable like cutlery, bowls, pots, crockery, straws and packaging. In principle, bioplastics can replace petroleum-derived plastics in many applications, however, the problem lies with the cost and performance of bioplastics. If there are no specific regulations globally to limit the use of conventional plastics, there may be no favourable usage of bioplastics. For example, Italy has since 2011 enacted law that made it compulsory for biodegradable plastic bags to be used for shopping. In the production of bioplastics, substitute for fossil fuel resources like wood, cellulose, sugar and starch are used. This has made bioplastic production more sustainable and environmentally friendly in comparison to conventional plastic production. The production of bioplastics decreases consumption of non-renewable energy and reduces the emission of greenhouse gases.

Conclusion

In conclusion, large household wastes involve wastes from the kitchen in most cases package in plastic bags. Accumulation of these waste pose a series of problems to the environment, health, and life of many living organisms in the affected areas.

To the environment, the wastes cause pollution of the air due to emission of gases such as methane, causes pollution of the soil and contamination of the underground water coupled with noise pollution especially the operations in the dumpsites.The effect of plastic wastes on marine organisms, humans and the environment at large is of public concern, and calls for the need to salvage the ecosystems and lives therein. Despite the fact that plastics are very useful in everyday life, the toxic chemicals used in the production need to be thoroughly monitored so as to ensure environmental and health safety. Reducing community's exposure to toxicants from plastic wastes will increase the chances of having a clean environment and healthy society. There is a urgent need for government agencies and health authorities to enact and enforce environmental laws that will monitor production, usage and disposal of plastics. In addition, some harmful chemical constituents used in the production of plastics (e.g. phthalates, BPA, etc) should be banned in consumer goods and in plastic products that are in direct contact with food, beverages and children.

Test questions:

1. Environmental conditions in the Republic of Kazakhstan characterized by significant violations of the environment are caused by the following factors: reasons

a) Technogenic impact.

b) Anthropogenic impact

c) Ecological disbalance

d) Reduction in Natural resources

e) Reduction in Flora and Fauna

2. Refund of the chemical elements (nitrogen, carbon, phosphorus) in the cycle is primarily provided by

a) producers

b) decomposers

c) industrial enterprises

d) consumers

e) human

3. The sources of high environmental hazard:

a) Machine building plants

b) Food plants

c) Industrial plants

d) Textile plants

e) Educational Institutions

4. Regions that are declared zones of ecological disaster:

a) Almaty and  Taldykurgan

b) Karaganda and Karaganda regions

c) Shymkent and Taraz

d) The Aral Sea and Semipalatinsk regions

e) Pavlodar and Kustanay

5. Industrial emissions include ...... ..

a) Uranium

b) Emissions of all enterprises

c) Emissions of non-ferrous metallurgy

d) Emissions of all enterprises

e) Emissions of chemical Industry

6. Spaceport where one of the facts of launching into outer space and emergency was registered... ..

a) Zhaylau

b) Alatau

c) Zhusan

d) Aktobe

e) Baikonur

7. Consumption of solar energy in the life cycle occurs in

a) rhizobia

b) mildew

c) animals

d) plants

e) fungi

8. The cycling of matter and energy transformation constantly going on in the biosphere, in which the main role is played by

a) climate change

b) factors of inanimate nature

c) living organisms

d) seasonal changes in nature

e) the microorganisms

9. Within one year from the spaceport "Baikonur" how many space rockets have been launched:

a) 55

b) 15

c) 8

d) 34

e) 2

10. The greenhouse effect is caused by ...... ..

a) global man-made (technogenic) air pollution

b) malfunction of the atmosphere

c) industrial enterprises

d) Plants

e) emissions from the chemical industry

11. pathogens that get including in the cycling of matter in the biosphere,

a) decompose organic matter to inorganic

b) involved in the formation of ozone screen

c) participate in the formation of limestone

d) involved in neutralization of radioactive substances in the soil

e) participate in the formation of minerals

12. some large enterprises placed  in Karaganda region are:

a) metallurgy, coal mines

b) Metallurgy

c) Thermal Power

d) non-ferrous metallurgy

e) coal mines

13. Nodule bacteria play an important role in the biosphere by participating in the cycle

a) oxygen

b) Nitrogen

c) Phosphorus

d) carbon

e) hydrogen

14. In the Aktobe region a negative impact on the environment is caused by.........

a) pharmaceutical plants

b) ferroalloy plant and chromium compounds

c) Ferroalloy Plant

d) non-ferrous metallurgy

e) emissions from the chemical industry

15. The main consumer of carbon dioxide in the biosphere are

a) decomposers

b) Consumers

c) producers

d) detritophages

e) ksenobioty

16. In South Kazakhstan region the main air pollutants are ... ..

a) Ferroalloy Plant

b) Heavy metals, pharmaceutical plants

c) Lead-phosphate, cement and refineries

d) power system, metallurgy, coal mines.

e) Metallurgy

17. In the nitrogen cycle are involved

a) nitrogen fixing bacteria only

b) people

c) all animals

d) all organisms

e) only producers

18. In the nitrogen cycle in the biosphere decisive role belongs to

a) the animal

b) man

c) plants

d) bacteria

e) algae

19. Carbon in the water cycle is activated through the

a) human activities

b) marine organic sediments

c) volcanic activity

d) Mineral Reserves

e) photosynthesis

20. Biological purify of waters:

а) Is not applied at the industrial enterprises

b) Now  it is not carried out at plant

c) Economically itself does not justify

d) Is not applied at the food enterprises

e) Kind of clearing of household and industrial waste water

21. The biofilter is:

а) Device for biological purify of waste water

b) Filter - press

c) Vacuum filter

d) The filter which detains only bacteria

e) Device for filtering biological liquids

22.Anabioz - status of a convertible oppression of ability to live organizm:

а) Under influence of the factors of external environment

b) After which the microorganisms perish

c) After which  about it is impossible to subject cultivation

d) Under influence of normal temperature

e) under influence only chemical reagent

23. Anaerobe is the microorganism capable normal to live:

а) In absence of free oxygen

b) At the presence of free oxygen

c) In absence of sources of carbon

d) In absence of mineral salts

e) At the presence of other microorganisms

24. The set of consecutive biochemical reactions proceeding in a cell:

a) Metobolism

b) Compilation

c) Transduscya

d) Katobalizm

e) Repression

25. Through the complex processing of wood it is possible to receive:

а) Cellulose lactose and fructose

b) Alcohol, fodder yeast, a paper

c) Waste water

d) Alcohol, СО2 clay

e) Fodder yeast, wastewater

26. The aerofilter is:

а) Filter established on air stations

b) Biofilter with compulsory aeration

c) Biofilter for clearing wastewater

d) Biofilter for cultivation of microorganizm

e) Filter with compulsory circulation of a liquid

27. Bacterial leaching is:

а) Dissolution of connections in water environment

b) Selective extraction of microorganisms, chemical elements from multicomponent connections

c) With application of simultaneous influence of alkali and temperature

d) Dissolution in water various biological objects under action of alkali

e) Extraction of metals by chemical substance

28.  The devices intended for division of non-uniform systems by a method of filtering through a partition (a fabric, metal grid, etc.), name:

a) Compressors

b) Filters

c) Membranes

d) Separators

e) Crane

29. Continuous methane fermentation goes at temperature:

а) 5 – 10 °С

b) 100 – 150 °С

c) 53 – 55 °С

d) 1 – 2 °С

e) 20 – 25 °С

30. According to the temperature analysis of wastewater brewing of manufactures is close:

а) 90 °С, рН  14

b) 2 °С, рН  2

c) 20 °С, рН  7

d) 50 °С, рН  5,5

e) 10 °С, рН  9

31. Gases obtained during alcohol fermentation on 99 % consist from:

а) Carbonic acid

b) Ammonia

c) Nitrogen

d) Charcoal gas

e) sulphurous of gas

32. Afteryeast bard possible to use in quality:

а) Medium for reception fodder В12

b) Medium for reception of baking yeast

c) Medium for reception ferments

d) Medium for reception of antibiotics

e) Medium for reception of vaccines

33. Ways of clearing of wastewater:

а) Mechanical, chemical, physico -chemical, biological

b) Physico -mechanical, biological

c) Biological, ecological

d) Chemical, physical

e) Microbiological, physical

34. In sand trap separate:

A) Microorganisms;

B) Large disperse  admixture;

C) Heavy mineral weighted material;

D) Chemical contamination;

E) Remainder products

35. Purpose of clearing wastewater:

а) Reduction price on product;

b) Increase of the weighed and dissolved substances

c) Removal from them of the weighed and dissolved substances Reduction price on product;

d) Increase of the price on production

e) Removal chemical poison

36. What amount of organic substances does domestic sewage contain?

- 50–60%

- 70-80%

- 10-15%

- 45-50%

- 90-95%

37. What amount of minerals contains domestic sewage?

- 10-20%

- 40–50%

- 30-35%

- 70-80%

- 20-30%

38. Select the wrong answer. Classes of wastewater include:

- Sanitary

- Industrial

- Commercial

- Agricultural

- State

39.  What is the annual water consumption in the world in all types of water?

- 3300-3500 km3

- 1000-1200 km3

- 1500-1600 km3

- 4000-4500 km3

- 900-980 km3

40. Surface runoff - is:

- Industrial

- Commercial

- Agricultural

- Sanitary

- Storm (rain), rinsing or surface runoff

41. What is the amount of wastewater without treatment?

- 80% of the waste water

- 99% o fthe waste water

- 90% ofthewastewater

- 70 % ofthewastewater

- 50% ofthewastewater

42. Select the wrong answer. Factors more complicated by problems of water supply:

- The growth of cities

- The rapid development of industry

- Intensification of agriculture

- Significant expansion of irrigated land

- Deterioration of cultural and living conditions

43. The chemicals contained in the wastewater affect:

- Increasing water demand

- Biological and Physical mode of reservoirs

- The problem of water supply

- The annual water consumption

- Household needs of the population

44. Sewage includes:

- Domestic waste

- Production waste

- Liquid wastes

- gaseous waste

- Construction waste

45. During a growth phase observed the largest number of cells formed in the shortest possible time:

- Latent phase

- Stationary phase

- Exponential growth phase

- Death phase

- Phase breeding

46. Why use untreated wastewater in agriculture:

- For the growth of microorganisms in the soil

- To separate the waste water from the suspended solids

- For the mineralization of soil

- For the formation of the activated sludge

- For irrigation

47. Select the wrong answer. Classes of wastewater include:

- Sanitary wastewater

- Industrial Wastewater

- Agricultural runoff

- Runoff

- Domestic wastewater

48. Sewage 99% consists of:

- Water

- Activated sludge

- Microorganisms

- Mineral substances

- Bacteria

49. Select the wrong answer. Treatment processes used for stormwater include:

- Pools

- Vortex separator

- Wetlands

- Biological ponds

- Vaults with different types of media filters

50. When a sewage treatment plant microorganisms are used:

- Primary

- Secondary

- Tertiary

- Quaternary

- Fifth

51. Filter’s action divided on:

a) Periodic and continuous

b) Poorly filtering and strongly filtering

c) Itself cleared and not cleared

d) Oil and water

e) Constant and variable

52. Pressure of the filters can classified to:

a) Gravitational, vacuum - filters, filter - press

b) Vacuum - filters

c) Gravitational, vacuum - filters, screw

d) Gravitational, screw

e) Vacuum - filters, atmospheric

53. Aerob - microorganism requiring the ability to live:

а) Free molecular oxygen

b) Absence of oxygen

c) Additives of mineral substances

d) Hydrogen or carbonic gas

e) Various to simbiotic

54. Methane fermentation tank include:

a) Performs methane fermentation of the solubilized liquid and another portion of sewage sludge to produce biogas.

b) Receives kitchen refuse and other biomass, removes foreign matter and solubilizes the biomass mixed with sewage sludge.

c) Refines biogas, generates power by gas engine and supplies electricity to plant facilities.

d) Dries dewatered digested sludge (dewatered sludge) and produces activated carbon.

e) All answers are correct

55.  What is anaerobic digestion?

a) It is a series of processes in which microorganisms break down biodegradable material in the absence of oxygen

b) Receives kitchen refuse and other biomass, removes foreign matter and solubilizes the biomass mixed with sewage sludge

c) Refines biogas, generates power by gas engine and supplies electricity to plant facilities

d) Dries dewatered digested sludge (dewatered sludge) and produces activated carbon

e) All answers are correct

56.  Silage produced by:

a) Analytic digestion

b) Aerobic digestion

c) Chemical digestion

d) Physical digestion

e) Anaerobic digestion

57.  Anaerobic digestion is widely used as:

a) All answers are correct

b) Source of renewable fermentation

c) Source of renewable activation.

d) Source of renewable carbonization.

e) Source of renewable energy

58.  Some bacteria that can survive in hot and more hostile conditions are called:

a) Bacilli

b) Psyhophiles

c) Mesophiles

d) Cocci

e) Thermophiles

59.  Optimal temperatures for thermophiles is:

a) 25 - 30 °C

b) 5 - 6 °C

c) 55 - 600 °C

d) 55 - 100 °C

e) 55 - 60 °C

60.  Optimal temperatures for mesophiles is:

a) 5 - 40 °C

b) 30 - 70 °C

c) 35 - 145 °C

d) 0 - 10 °C

e) 35 - 40 °C

61. Pollution started from the prehistoric times…

- when man created an instrument of labor

- when man made the first fires

- when man created an instrument of labor

- when man  created gunpowder

- when man learnt to hunt

- when man began to breed cattle

62. In the 2010 issues, the ten top nominees in pollution are located in:

- United States, Peru, Russia, Ukraine and Zambia

- Azerbaijan, China, India, Peru, Russia, Ukraine and Zambia

- China, India, Peru, Russia, Ukraine and Kazakhstan

- Azerbaijan, China, India, Peru, Russia and Malaysia

- Singapore, Kazakhstan, Russia, Ukraine and Zambiya

63. Key turning point in creation of significant air pollution levels outside homes is …

- Forging of metals

- Search for gold

- trip around the world

- when man began to breed cattle

- when people began to develop agriculture

64. Intensive pollution began with the period …
a) the Industrial Revolution,
b)October Revolution
c)during the Second World War
d)prehistory
e)From the the period of the emergence of nuclear weapons

65. The Great Smog of 1952 occurred in…
a) London
b) Paris
c) Moscow
d) Almaty
e) Beijing

66. The Great Smog of 1952 in London killed at least…
a) 4000 people

b) 100 people
c) 1million people
d) 10 people
e) 20,000 people

67. The most polluted lake on earth, because it was used as a testing ground for the Soviet Union during the 1950s and 1960s

a) Balkhash
b) Kapchagai
c) Issyk Kul
d) Karachai 
e) Aral

68. Oil spills from sinking ships contains:
a) approximately 100%
b) about 20%
c) about 50%
d) about 10%

e) Do not harm

69. Offshore oil spills occur in refueling ships:
a) 45%
b) 15%
c) about 50%
d) approximately 100%
e) do not harm

70. Alien substances for living organisms resulting from human activities that can cause a violation of biotic …

a)токсикант Toxicant 
загрязнитель b) contaminant 
поллютант c) Pollutant

антибиотик d) Antibiotic

ксенобиотик e) Xenobiotic 

Существенное изменение (чаще упрощение) структуры вещества под действием организмов 71. A significant change (more simplification) structure of matter under the influence of organisms:  
биоремедиация a) Bioremediation 
конъюгация b) Conjugation 
трансформация c) Transformation 
минерализация d) Mineralization

деградация e) Degradation 

72. Трансформация нетоксичного или малотоксичного ксенобиотика в токсичное соединение Non-toxic or low-toxic transformation in xenobiotic toxic compound:  
a)детоксикация Detoxication

изомеризация b) Isomerization 
токсификация c) Toxification 
обезвреживание d) Rendering harmless

уничтожение e) Destruction

73.В анаэробных условиях конечными продуктами деградации многих ксенобитиков являются Ready products of degradation under anaerobic conditions of many xenobiotic are:
метан и углекислый газ a) Methane and carbon dioxide 
алканы и углекислый газ b) Alkanes and carbon dioxide 
водород и углекислый газ c) Hydrogen and carbon dioxide 
кислород и этан d) Oxygen and ethane

этиловый спирт и водород e) Ethanol and hydrogen 

Базовый элемент биосферы, способный адсорбировать, нейтрализовать и минерализовать загрязнения, выполняя важную роль в самоочищении экосистем от органических отходов и остатков 74. Basic element of the biosphere capable of adsorbing to neutralize and mineralize dirt, performing an important role in self-cleaning ecosystem from organic waste and residues is…
a)почвавода Water

b) Soil 
c) атмосфераAtmosphere 
микробоценозd) Microbiocenosis

e)сточные воды Effluent 

75. В экосистемах редуцентами являются Decomposers ecosystems are:
a)растения и животные Plants and animals 
бактерии и грибы b) Bacteria and fungi 
вирусы c) Viruses 
детрит d) Detritus

альдегид e) Aldehyde 

Из водорослей нашли применение в качестве очистителей сточных вод в биопрудах 76. Which type of Algae used as a cleaning wastewater in bioponds?
Gelidium, Phyllophora a) Gelidium, Phyllophora

Laminariab) Laminaria 
Pleurococcus c) Pleurococcus 
Airococcus d) Airococcus 
Chlorella, Scendesmus e) Chlorella, Scendesmus

77. Plants using Для очистки сточных вод с помощью растений используютfor treatment of wastewater: 
a)поля фильтрации, биоплато field of filtration, constructed wetlands 
иловые карты, иловые площадки b) silt card sludge beds 
аэротенки, метантенки c) aeration tanks, digesters 
окситенки, биотенки d) oxytanks, biotanks

зоогели e) zoogels

78. Специально подготовленные и спланированные земельные участки, предназначенные для очистки сточных вод с одновременным использованием для выращивания технических культур растений ### Specially trained and planned land intended for wastewater treatment with simultaneous use for the cultivation of commercial crops of plants 
поля орошенияполя фильтрации a) filtration fields 
иловые площадки b) sludge beds 
биопруды c) bioponds

зоогели d) zoogels

e) sewage farm

Фиторемедиационная технология, основанная на способности растений поглощать корневой системой токсиканты, находящиеся в почве и воде, и транспортировать их в надземные органы 79. Phytoremediation technology based on the ability of plants to absorb toxicants by their root system in soil and water and transport them to the aerial organs 
фитоэкстрация a) phytoextraction

фитодеградация b) phytodegradation  
фитовыпаривание c) phytoevaporation 
фитоселекция d) phytoselection

фотосинтез e) photosynthesis 

80.Перифитонные слизеобразующие организмы, способные образовывать бактериальные скопления с общей слизистой капсулой, играющие важную роль в системах биологической очистки сточных вод Periphyton slime organisms capable of forming clusters with common bacterial mucous capsule, which play an important role in biological wastewater treatment:

a)Pseudomonas putida, Pseudomonas aeruginosa Pseudomonas putida, Pseudomonas aeruginosa 
Zoogloea ramigera, Sphaerotilus natansb) Zoogloea ramigera, Sphaerotilus natans 
Rhodococcus erythropolis, Arthrobacter luteusc) Rhodococcus erythropolis, Arthrobacter luteus 
Bacillus subtilis, Bacillus thuringiensisd) Bacillus subtilis, Bacillus thuringiensis

Pseudomonas, Arthrobacter, Rhodococcuse) Pseudomonas, Arthrobacter, Rhodococcus

Наиболее целесообразным видом биоремедиации участков со старыми нефтяными загрязнениями является 81. The most appropriate method of bioremediation of sites with old oil pollution is: 
a)внесение новых штаммов-деструкторов introduction of new strains-destructors

сжиганиеb) burning

стимулирование аборигенной микробиоты с применением удобренийc) encouraging indigenous microbiota using fertilizers 
засыпка пескомd) filling with sand 
внесение фитофаговых грибовe) Adding fitofagovyh mushrooms

Благоприятными условиями для биодеградации нефтепродуктов в окружающей среде являются 82. Favorable conditions for biodegradation of petroleum products in the environment are:

анаэробные условия, температура 0-3ºС a) anaerobic conditions, the temperature 0-3ºС
аэробные условия, температура 20-35ºС b) aerobic conditions, the temperature 20-35ºС
анаэробные условия, температура 20-35ºС c) anaerobic conditions, the temperature 20-35ºС
анаэробные условия, температура 5-15ºС d) anaerobic conditions, the temperature 5-15ºС
аэробные условия, температура 5-15ºС e) aerobic conditions, the temperature 5-15ºС

83. Наиболее трудно утилизируемыми фракциями нефти для микроорганизмов являются The most difficult recyclable oil fractions for microorganisms are:

бензин a) gasoline 
смолы и асфальтены b) resins and asphaltenes 
предельные углеводороды c) saturates 
непредельные углеводороды d) unsaturated hydrocarbons 
циклические углеводороды e) cyclic hydrocarbons 

84. S: Полную минерализацию ксенобиотиков способны осуществитьComplete mineralization of xenobiotics can be carry out by …
a)бактерии bacteria

растенияb) plants 
водорослиc) algae 
микрофаунаd) microfauna

животныеe) animals

В процессе окисления загрязнений сточных вод основная роль принадлежит 85. In the oxidation of polluted wastewater the main role belongs to 
a)бактериям bacteria

водорослямb) algae 
грибамc) fungi 
простейшимd) protozoa

амебамe) amoebas 

86. Facility for S: Сооружение для биологической очистки сточных вод, представляющее собой открытую систему проточных резервуаров с активной аэрацией ###biological wastewater treatment is an open system flow tanks with active aeration …
 a)аэротенк aerotank

метантенк b) digester 
септитенк c) septic tank 
биопруд d) bioponds

активный ил e) activated sludge

87. Герметичный ферментер объемом в несколько кубических метров с перемешиванием, который обязательно оборудуется газоотделителями с противопламенными ловушками87.87Hermetic fermenter of a volume of several cubic meters, with stirring, equipped with a gas separator 
метантенк a) methane tank
аэротенк b) aerotank 
окситенк c) oxytank
фильтротенк d) filtertank

Активный ил e) activated sludge 

Система анаэробной очистки стоков 88. Anaerobic wastewater treatment system proceeds in…
 a)аэротенк, окситенк aerotank, oxytank 
метантенк, септитенк b) methane tank, septic tank 
экструдер, ферментер c) extruder fermenter 
биопруд, иловая карта d) bioponds, silt map

Активный ил e) activated sludge 

89. Горизонтальный отстойник закрытого типа, в котором образовавшийся на дне осадок твердых частиц перегнивает и разлагается анаэробными микроорганизмами без дополнительного перемешивания и нагревания A horizontal closed type clarifier, where in the bottom sediment resulting solids rot and decomposed by anaerobic microorganisms without additional mixing and heating: 
аэротенкa) aerotank 
b) метантенкdigester 
c) септитенкseptic tank
d) биопрудbioponds

активный илe) activated sludge 
Сооружение для анаэробного сбраживания осадка сточных вод, а также высококонцентрированных сточных вод при повышенных температурах

90. Facility for anaerobic digestion of sewage sludge, as well as highly concentrated wastewater at elevated temperatures… 
аэротенк a) aerotank 
метант енк b) methane tank
септитенк c) septic tank 
биопруд d) bioponds

активный ил e) activated sludge 

91. Формирование биоценоза обрастаний начинается с адсорбции или осаждения твердых частиц и колонизации клеток Formation of biocenosis fouling starts with adsorption or deposition of particulate matter and cell colonization:  
бактерий, способных образовывать слизистую капсулу a) freely moving bacteria
свободно передвигающихся бактерий b) bacteria capable of forming a mucous capsule 
инфузорий c) ciliates 
водорослей d) algae

дрожжей e) yeasts 

Если вода богата кислородом и загрязнена органическими веществами, то в биообрастаниях доминируют 92. If the water is rich in oxygen and contaminated with organic substances, the biofouling dominate 
бактерии Zoogloea ramigera, Sphaerotilus natans a) filamentous iron bacteria
нитчатые железобактерии b) Zoogloea ramigera, Sphaerotilus natans 
грибы c) fungi 
актиномицеты d) actinomycetes

Rhizobium e) Rhizobium 

93.К системе механической очистки сточных вод относят ### To the system of mechanical wastewater treatment, include…
a)решетки и пескоуловители lattice and sand catchers 
аэротенкиb) aerotanks 
метантенкиc) digesters 
циркуляционные окислительные каналыd) circulating oxidative channels

активный илe) activated sludge 
Эндоглюканаза, экзоглюканаза, экзоглюкозидаза, целлобиаза представляют собой комплекс
В результате деятельности сульфатвосстанавливающих бактерий из сточных вод осаждаются 95. Because of sulfate-reducing bacteria from sewage deposited:  
a)сульфиды тяжелых металлов sulphides of heavy metals 
b)сульфаты тяжелых металлов heavy metal sulfates 
c)сульфиты тяжелых металлов heavy metal sulfites

серосодержащие пептидыd)sulfur-containing peptides

e)сера sulfur

96. Инокуляция бобовых растений препаратами, содержащими Rhizobium Inoculation of legume preparations containing Rhizobium 
a)способствует азотфиксации inhibits the growth of weeds 
производится для обеспечения фунгицидного действия b) is made ​​to ensure the fungicidal activity 
угнетает рост сорняков c) promotes nitrogen fixation

защищает корни от нематодd) protect the roots from nematodes

образует антибиотики e) create antibiotics 

Конечный продукт связывания азота азотфиксирующими бактериями, свидетельствующий об окончании связывания азота и включении его в метаболизм 97. The final product of nitrogen fixation by nitrogen-fixing bacteria, indicating the end of nitrogen fixation and its inclusion in the metabolism: 
 a)аммиак diimide
гидразин b) hydrazine 
диимид c) ammonia 
белок d) protein

аргинин e) arginine 

98.Преобразование нитритов и нитратов в бескислородной среде очистных сооружений с выделением газообразного азота Transformation of nitrite and nitrate in the absence of oxygen treatment facilities with the release of nitrogen gas: 
a)денитрификация ammonification
нитрификация b) nitrification 
аммонификация c) denitrification 
азотфиксация d) nitrogen fixation

азотнитрацияПоложительное влияние бактерий Pseudomonas , стимулирующих рост растений, заключается вФиксацию атмосферного азота могут 99. Fixation of atmospheric nitrogen can 
клубеньковые бактерии в симбиозе с бобовыми растениями a) Bacteria Pseudomonas,
клубеньковые бактерии без бобовых растений b)nodule bacteria without leguminous plants бобовые растения без клубеньковых бактерий

c)legumes without nodule bacteria

бактерий Pseudomonas , d) nodule bacteria in symbiosis with leguminous plants

e)Chlorella, Scendesmus

| Keys to correct answers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # question | Correct answer | # question | Correct answer | # question | Correct answer | # question | Correct answer | # question | Correct answer |
| 1 | a | 11 | a | 21 | a | 31 | a | 41 | c |
| 2 | b | 12 | a | 22 | a | 32 | a | 42 | e |
| 3 | c | 13 | b | 23 | a | 33 | a | 43 | b |
| 4 | d | 14 | b | 24 | a | 34 | c | 44 | a |
| 5 | e | 15 | c | 25 | b | 35 | c | 45 | c |
| 6 | e | 16 | c | 26 | b | 36 | a | 46 | e |
| 7 | d | 17 | d | 27 | b | 37 | b | 47 | e |
| 8 | c | 18 | d | 28 | b | 38 | e | 48 | a |
| 9 | b | 19 | e | 29 | c | 39 | a | 49 | d |
| 10 | a | 20 | e | 30 | c | 40 | e | 50 | b |

| # question | Correct answer | # question | Correct answer | # question | Correct answer | # question | Correct answer | # question | Correct answer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | a | 61 | b | 71 | c | 81 | c | 91 | b |
| 52 | a | 62 | b | 72 | c | 82 | b | 92 | b |
| 53 | a | 63 | a | 73 | a | 83 | b | 93 | b |
| 54 | a | 64 | a | 74 | b | 84 | a | 94 | a |
| 55 | a | 65 | a | 75 | b | 85 | a | 95 | a |
| 56 | e | 66 | a | 76 | e | 86 | a | 96 | a |
| 57 | e | 67 | d | 77 | a | 87 | a | 97 | c |
| 58 | e | 68 | d | 78 | e | 88 | b | 98 | c |
| 59 | e | 69 | b | 79 | a | 89 | c |  |  |
| 60 | e | 70 | e | 80 | b | 90 | b |  |  |

Exam questions

Level 1

1 Describe  the environmental situation in the Republic of Kazakhstan

2 The main sources of air pollution in the Republic of Kazakhstan

3 The most important problems in Kazakhstan regions

4  How we can use of biotechnology for solving ecological problems?

5 List basic methods used successfully in practice of eco biotechnology

6 What is Environmental biotechnology?

7When pollution became a popular issue?

8 What is especially adversely manifested in Kazakhstan?

9	What is the greenhouse effect?

10	What is acid rain?

11	Describe the role of biotechnology in solution of ecological problems.

12	List the main sources of high environmental hazards?

13	Which regions of Kazakhstan have ecological problems?

14	List the main sources of air pollution in the Republic of Kazakhstan.

15	Which consequences of the test the nucleus weapons on Semipalatinsk firing range?

16	What is the main reason of high levels of air pollution?

17      What is bioabsorption, active and passivebiosorption?

18     List the requirement for naturalbiosorbents.

19The main sources of soilcontamination by heavymetals.

20     What is accumulativeability of microorganisms to heavymetals?

21      Describe the main absorber of heavymetals.

22What is the wastewater?

23    How much is the annualwaterconsumption in the world?

24What are the classes of sewage?

25How manystagesincludecleaning of wastewater?

26What is the role of water for living organisms?

27  Give the definition for anaerobic processing

28Describe one disvantage of methanefermentationprocess

- How do the three categories of municipal solid waste (compost, recycling and landfill) differ?

- Why are landfills environmentally harmful?

- What are the current executive orders on waste management?

- What are common barriers to implementing waste diversion programs?

- Explain the benefits of maximizing waste diversion

- What waste management services does your locality provide?

- What is waste management?

- Give some examples of the rules and regulations that guide waste management in Kazakhstan?

- Describe the common methods of waste disposal

- Describe the aerobic composting

- What is anaerobic composting?

- Give definition of the incineration

- What is a sanitary dump?

- Simple and effective ways of waste management for individuals

LEVEL 2

1.	What is the biogeochemical cycle?

2.	Classification of biogeochemical cycles. Give the definitions to following terms: exchange fund and reserve fund cycle.

3.	Describe three main types of biogeochemical cycles.

4.	What is a biodegradation?

5.	What are the main groups of microorganisms involved inoxygen cycle?

6.	What is a nitrification?Characterize Nitrifying bacteria.

7.	What is a nitrogen fixation?

8.	List common anaerobic respiratory inhabitants of soil?

9.	Prokaryotes ofsulfur cycle.Define their role.

10.	Give a brief description of phosphorus cycling. Role of phosphorus in exchange process of the cell

11.	Evaluate the Microbiocenosis as part of the ecosystem

12.	Determine the role of Microbiocenosis in biogeochemical cycles.

13.     What is sorptioncapacity of biomass?

14. Whatnaturalphenomena and factors of humanactivity can affect the quantity and quality of water?

15.    Whatkind of humanactivityconsumeswater?

16.   What is the differenceindustrialwastewater from household?

17.   Whatcould be the recycling of wastewater?

18. Anaerobicprocessing’s is being widelyusedas a source of renewableenergy. What’shappened in a final or intermediateresult?

19. Whatstages of anaerobicprocessing do you know?

20. What is the septictank? What is spaciousness of septictank?

21. Wherecarbonized products can be used for?

22. Why the dryer is used in methanetanks?

23. How methane is formed?

24. Why weneed tank in methanefermentation?

25. What are the four main components of this technologyreceivingbiogas?

26. Characterizeprocess Biosorption?

27. What is the process Bioremediation? What is necessarymethod of wastemanagement?

28. Enumerate and describe the mostpressingproblemwhileremedingpollutantswhich can be toxic for alivenature and people in smallconcentration.

- What are the examples of eco-innovations that reduce environmental pollution and the process of global warming?

- Will replacing all plastic packaging with its biodegradable substitutes solve the problem of garbage?

- How to improve the waste separation system?

- Should the scale of environmental education in schools be expanded?

- Will the development of robotics and intelligent technologies solve the problem of sharply increasing environmental pollution by natural ecosystems?

- What bacteria can help eliminate the unpleasant smell of garbage and avoid the spread of diseases?

- How can people improve the environment at the expense of the waste in which we live?

- Describe the effects that pollution can have on human health?

- What are the most important environmental problems related to waste in the modern world? Should governments be solely responsible for protecting the environment from waste?

- Do you think that waste recycling is important?

- What strategies help improve the environment we live in?

- What are the rules that guide waste management in Kazakhstan?

- Describe the common methods of waste disposal

- What is the aerobic composting

LEVEL 3

1.	What isbioremediation? Define methods of waste management?

2.	Describe in-situ and ex-situ methods.

3.	Which kind of material can raise biodegradation of soil?

4.	Define polluting material. Describe the Xenobiotics.

5.	Compare bioaugmentation and biostimulation

6.	Give definitions to Еcotoxicants,Ecotoxicity(Ecotoxikinetic).

7.	Characterize Biosorptionprocess.

8.	Describe pollutants which can be toxic for alive nature and people in small concentration.

9.	Enumerate and describe the most harmful metals.

10.	Estimate and describebiosorbents, biosorptions and types of sorptions.

11.	Determine the main properties of waste that are environmentally hazardous

12.	Recount the adverse properties of solid waste for treatment

13.	Describe the types of transformation- abiotic transformation, species, effect on transformation.

14.	Definethe process biotic transformation oforganicpollutantsin cleaning up the environment.

15. Describe the processtransformationofnaphthenic, naphthenic-aromatic and aromatichydrocarbons.

16. Giveinformationabout research and isolation of microorganisms with hydrocarbon-oxidizingactivity.

17. Make an analysis about biologicalmethods are based on the use of different groups of microorganisms. Identify the advantages and disadvantages.

18. Assess the value of Bioremediation, Bioaugmentation, biostimulation in environmentalbiotechnology.

19. Characteristics of heavymetals, physical and chemicalfeatures. The negativeeffect of heavymetalsdepends, in essence, on theirmobility, i.e. solubility.

20. Explain the use of sulfate-reducingbacteria in deposition of metals. Describe conditions of theirfunctioning.

21. Estimateaccumulativeability of microorganisms to heavymetals.

22. How biochemical oxygen demand (BOD) is defined?

23. How chemical oxygen demand (COD) is defined?

24. In whatyear the first anaerobicsystematizationwasconstructed by lepercolony in?

25. Where do we use the carbonized products made at a lowworkingtemperature?

26. Anydisvantage of methanefermentationprocess ?

27. Whattechnologicalsystemrelieves of burning of the madegas?

28. Deviationswhichdon'tdecay in a septictankduring an anaerobic digestion are dangerous are not?

- What ways can you reduce environmental pollution in this country?

- What are the possible consequences associated with deforestation?

- What are the consequences of land pollution?

- Describe how deforestation can contribute to global warming.

- What environmental problems can deforestation lead to?

- What is deforestation and in which parts of the world do the greatest forest losses occur?

- What are the ways to reduce indoor air pollution?

- List some of the main harmful environmental consequences of deforestation.

- What technologies do you know that could help stop environmental problems?

- What are the different types of waste?

- What are the ways to store waste at home?

- Why is waste considered one of the main environmental problems?

- What is an ecosystem?

- Give some examples of things that can be recycled?

References

- The concept of ecological security of the Republic of Kazakhstan for 2004-2015 approved by the Decree of the President of the Republic of Kazakhstan dated December 3, 2003.(UNEP-CHW-CFS-Kazakhstan-1.English.doc - Basel Convention).

- Environmental Code of the Republic of Kazakhstan dated January 9, Astana, 2007.(http://adilet.zan.kz/eng/docs/K070000212_)

- Law of the Republic of Kazakhstan "On health and safety" of 28 February 2004.

- "Semipalatinsk: 60 years later (collection of articles)". Bulletin of the Atomic Scientists. September 2009. Retrieved 2009-10-01.

- Biogeochemical Cycles (Web site).  http://www.bsi.vt.edu/chagedor/biol_4684/Cycles/cycles.html

- "Reports". WorstPolluted.org. Archived from the original on 11 August 2010. Retrieved - 2010-08-29.- 14 p.

- Состояние окружающей среды Восточно- Казахстанской области. 2011 год. Экология Восточного Казахстана: проблемы и решения. Усть-Каменогорск. Изд-во ВКГУ. 2011. с. 14-48.

- Калиев. С. Воздействие ГМК на окружающую среду. Промышленность Казахстана. – 2012.- №12 – С. 12-14.

- Заядан Б.К.Экологическая биотехнология. Учебник. – Алматы:Казахский университет, 2014.- 316 с.

- Кузнецова А.Е., Градова Н.Б., Научные основы экобиотехнологии.- Москва: Мир, 2006-504 с.

- Новиков Ю.В. Экология, окружающая среда и человек. М.: ФАИР-ПРЕСС,2003.

- Современные глобальные изменения природной среды /Под научной редакцией Н.С. Касимова. - М.: Научный Мир, 2006. - Т. 1. - 696 с.; Т. 2. - 775 с.

- Горшков С.П. Учение о биосфере. Введение. - М.: Географический ф-т МГУ, 2007.-118с.

- Горелов А.А. Концепции современного естествознания: учеб. пособие для студ. вузов / А. А. Горелов.5-е изд., перераб. и доп..М.: Академия, 2010., 512 с.

- Shepard N, Krech N,  Encyclopedia of World Environmental History: A-E. Routledge. pp. 70–71. 2011.

- Hancock, Paul L., Brian J. Skinner. The Oxford Companion to the Earth. New York: Oxford University Press, 2012, Life and Biogeochemical Cycles (Web site).  http://essp.csumb.edu/esse/climate/climatebiogeo.html

- "Infant Inhalation Of Ultra-fine Air Pollution Linked To Adult Lung Disease". Sciencedaily.com. 2009-07-23. - Retrieved 2010-08-29.

- Vasilyeva S.V. Primenenie chemical mutagenesis to biodegradable industrial chemical waste // purification and bio-indication contaminated vod.M.2008.- 65 p.

- Dmitriev A.A. The influence of the concentration of activated sludge on the rate of biochemical oxidation / Industrial wastewater treatment: Sat. Nauchn.tr. . M., 2005.

- Zhukov D.D. Process Wastewater Treatment to aeration tanks,Moscow, - 2012. - 44 p.

- Sevastianova G.S. Improved method of biological treatment of wastewater.,Moscow, - 2012. (Review. Inform. / VNIIIS.Ser.4Inzh.obespecheniya objects page Islands, Vol. 2).

- G. Oniszhenko, S. Avaliani, S. Novikov, U. Rakhmanin and K. Bushtueva, “Basis for human Health Risk Assessment Resulting from Chemical Pollutants,” NII ECH & GOS, Moscow City, 2008. doi:10.1016/S0305-750X(99)00086-8

- ShvetsovA.F., The use of technical oxygen for biological wastewater treatment. // Water and sanitary tehnika.- 2013, - №4.- P12.

- Astakhov V.A. Intensification of process wastewater bioremediation microimpurities ozone water purification // Microbiology Abstracts .: Vsesoyuz.konf .., M, 2012.

- Henze et al. Wastewater treatment. M., Mir, 2014.

- StepanovA.S., Intensification of biological treatment at wastewater treatment plants. Samara .// Water is an itarnayatehnika.M., -  2006, - №9.

- Кузнецов А.Е., Градова Н.Б. Научные основы экологической биотехнологии. - М. Мир, 2008. -447c.

- Slavinskaya A.S, The intensification of post-treatment of biologically treated waste water // Water Supply and Sanitary Equipment, 2011, №2.

- Данилович Д. Л., Монгайт А. И. Анаэробная очистка концентрированных сточных вод. – М.: Стройиздат, 2010. -420с.

- VasudevanP.UtilisationofAgro-residuesasanimalfeedthroughBioconversion.Bioresour.Technol.59:5-8. - 2013.

- EssienJP,UdotongIR(2008).AminoAcidProfileofBiodegradedBrewersSpentGrains(BSG).J.Appl.Sci.Environ.Manage.12(1): 109-111

- Begon, M.; Townsend, C. R., Harper, J. L. (2006). Ecology: From individuals to ecosystems. (4th ed.). — Blackwell. ISBN 1-4051-1117-8.

- Ахатов А. Г. Экология и международное право.Ecology & International Law.-М.: АСТ-ПРЕСС, 2006. — 512 с. (тираж 1000), ISBN 5-214-00225-4

- Пианка Эрик. Эволюционная экология. — М.: Мир,2001. — 399 с.

| CONTENT |  |
| --- | --- |
| Introduction | 3 |
| # Chapter 1 Industrial waste. Environmental Legislation. The Earth is getting Warmer and detrimental effect on human well-being. Deforestation and Farming. The Disposal of Waste | 7 |
| ### 1.1 Environmental Legislation | 8 |
| # 1.2 The Earth is getting Warmer and detrimental effect on human well-being | 11 |
| 1.3 The Impact on Our Health | 13 |
| 1.4 The Threat to Plants and Animals | 14 |
| 1.5 Deforestation and Farming | 16 |
| 1.6 The Disposal of Waste | 19 |
| Chapter 2 The main properties of wastes and waste classification | 21 |
| 2.1 Definition and classification of wastes | 21 |
| 2.2. Еcotoxicants and chemical waste | 25 |
| 2.3. Xenobiotic types of environment | 26 |
| 2.4 Ecotoxicity (Ecotoxikinetic) | 27 |
| Chapter 3 Industrial waste. Classification and treatment. Toxic waste. nuclear and radioactive waste | 30 |
| 3.1 Industrial waste. Causes of Industrial Pollution | 30 |
| 3.2 Toxic waste | 32 |
| 3.3 Utilization and disposal of nuclear and radioactive waste | 34 |
| Chapter 4 Biological and non-biological processes of transformation of pollutants |  |
| 4.1 Types of transformation | 37 |
| 4.2 Biotic transformation of organic pollutants | 39 |
| 4.3 Destruction of oil pollution in the soil | 41 |
| 4.3.1 The ecological and geochemical characteristics of the basic composition of oil | 43 |
| 4.3.2 Methods for Eliminating Oil Contamination in Soil | 39 |
| 4.3.3 Biological methods are based on the use of different groups of microorganisms | 46 |
| 4.3.4 The research and isolation of microorganisms with hydrocarbon-oxidizing activity | 51 |
| 4.4 Bioremediation, Bioaugmentation, biostimulation | 55 |
| Chapter 5 Heavy metal pollution. The main absorber of heavy metals | 58 |
| 5.1 Bioabsorption, active and passive biosorption | 58 |
| 5.2 The sources of soil contamination by heavy metals | 59 |
| 5.3 The main absorber of heavy metals | 61 |
| 5.4 Accumulative ability of microorganisms to heavy metals | 61 |
| Chapter 6 Water resources. The basic characteristics of sewage, kind of water flows, their structure and criteria of quality assessment | 64 |
| 6.1 Water as the most important natural resource | 64 |
| 6.2 Sewage treatment | 64 |
| 6.3 Biofiltration, biofilters and problems of their work. | 69 |
| 6.4 Purification of industrial wastewater by Pseudomonas putida | 71 |
| 6.5 Sum parameter in waste water analysis | 73 |
| Chapter 7 Anaerobic digestion of wastes | 76 |
| 7.1 Microorganisms involved in process of the anaerobic digestion The key process of anaerobic digestion | 76 |
| 7.2 Septic. Potential problems | 78 |
| 7.3 Methane Fermentation System of Sewage Sludge and Raw Garbage, and Carbonization-activation for Utilization | 81 |
| Chapter 8 the Utilization of agrocultaral and industrial waste | 86 |
| 8.1 Bioconversion of organic waste | 86 |
| 8.2 Characteristics of wastes from food industry | 87 |
| 8.2.1 Waste waters are collected from dairy industry | 89 |
| 8.2.2 Utilization of winery wastes | 90 |
| 8.2.3 Efficient recycling of brewer’s spent grain | 92 |
| 8.3 Wastes of cellulose and paper industry | 94 |
| 8.4 Polyethylene as ecological problem | 95 |
| 8.5 Bacterial leaching | 96 |
| 8.6 Utilization of livestock waste, manure for the production of biogas and fertilizers | 97 |
| Chapter 9 BIOENERGY and biofuels | 101 |
| 9.1 Bioethanol | 101 |
| 9.2 Microalgae cultivation and carbohydrate accumulation | 102 |
| 9.3 Fermentation of microalgae biomass | 108 |
| 9.4 The largest sources of carbon dioxide -- fossil fuelsю The Rise of Renewable Energy. renewable energy sources, sun, wind and water | 109 |
| Chapter 10 Waste management in biotechnological industry | 115 |
| 10.1 Medical Waste. Major Types of Medical Waste. Effective Medical Waste Management | 118 |
| 10.2 Effective management of agricultural wastes. Classification and causes/sources of agricultural wastes | 120 |
| 10.3 Food waste and its disposal. The problem of population growth, hunger, malnutrition, emissions and efficient processing and management of food waste | 122 |
| # 10.4 Biofilter made from peanut shell degrades air pollutants | 124 |
| # 10.5 Major Advance in Artificial Photosynthesis Poses Win/Win for the Environment | 126 |
| 10.4 Management of household waste. The problem of solid waste disposal. Bioplastics as alternative | 128 |
| Tests questions | 134 |
| Exam questions | 152 |
| References | 156 |