Ф.7.03-09

Ministry of Education and Science of the Republic of

Kazakhstan

M.O. Auezov SOUTH KAZAKHSTAN STATE UNIVERSITY

Chair "Automation, telecommunications and control"

Methodical instruction for laboratory works
on the discipline of

Electronics and circuitry analog devices 2

for specialty: 5B071900 - “Radio engineering, electronics and telecommunication”

SHYMKENT-2013

УДК 658.012.011

ББК 6П7.1

The composer senior lecturer, cand.tech.sci. Sarybaev A.A., Aslanov M.T Methodical instruction for laboratory works on the discipline of «Electronics and circuitry analog devices 2» for students of a speciality 5B071900 - “Radio engineering, electronics and telecommunication”, Shymkent: SK.SU, 2013, 62p.

The methodical instruction for laboratory works is made according to requirements ГОСО and the discipline program « Electronics and circuitry analog devices 2» and includes all necessary data on performance of course laboratory researches by that.

Laboratory practical work the « Electronics and circuitry analog devices 2» contains instructions to performance of laboratory on discipline. Necessary recommendations are resulted in each laboratory work and similar examples are considered. At the end of each section the list of control questions are resulted.

Reviewer: Tasanbaev S.E - cand.tech.sci., the associate professor of chair «Automation, telecommunications and control» SKSU of M.Auezov

It is considered and is recommended for printing by faculty meeting the methodical commission of faculty «Information technology» (the report № __ from «__ » ___ 2013)

It is recommended to the edition Education-methodical council SKSU of M.Auezov, the report № __ from «___» ____ 2013

©  the South Kazakhstan State University (SKSU), 2013

Contents

Introduction	5

- - ADDERS	1

- - AMPLIFIER	13

- - ANALOG I/O - NI ELVIS Function	14

- - CHANNEL MODULE	15

- - DIGITAL I/O - NI ELVIS Function	16

- - DUAL ANALOG SWITCH	17

- - FUNCTION GENERATOR - NI ELVIS Function	18

- - HEADPHONE AMPLIFIER	19

- - LINE CODE ENCODER	20

- - MASTER SIGNALS	22

- - MULTIPLIER	24

- - NOISE GENERATOR	25

- - PCM ENCODER	26

- - PCM DECODER	27

- - PHASE SHIFTER	28

- - SEQUENCE GENERATOR	29

- - SERIAL TO PARALLEL	34

- - SPEECH	35

- - TUNEABLE LPF	36

- - TWIN PULSE GENERATOR	37

- - UTILITIES	38

- - VARIABLE DC V - NI ELVIS Function	39

DATEx™ SOFTWARE INSTALLATION & SFP OPERATION

- - INSTALLING THE EMONA DATEx SOFTWARE	40

- - USING THE DATEx-MAIN & DATEx-TOOLBAR SFPs	41

- - CONFIRMING PC-to-DATEx™ COMMUNICATIONS	41

USING DATEx™ VIs within LabVIEW PROGRAMS FOR PC-CONTROL

- - PC-CONTROL MODE	42

### The DATEx™ Telecommunications Add-in Module for NI ELVIS

DATEx, or Digital Ananlog Telecommunications Experimenter, as its name implies, is used to help students learn about communications and telecommunications principles. The Emona DATEx add-in module is fully integrated with the NI ELVIS platform and NI LabVIEW environment. All DATEx™ knobs and switches can be varied either manually or under the control NI LabVIEW VIs.

Equipment Required

Experiments make use of the Emona DATEx telecommunications trainer kit together with the NI EL platform and NI LabVIEW running on a PC. The functionality and range of the virtual instrumentation available depends on the NI DAQ that is coupled with NI ELVIS platform.

IMPOR TANTNO TE CONCERNING DAQ SELCTION Ideally the NI-DAQ should have an oscilloscope sample rate of 1MS/s or more, such as the USB-6251 or equivalent. If the NI-DAQ being used has a slower oscilloscope sample rate, then only baseband signals generated by the Emona DATEx add-in module will be v

### DATEx™ bringing BLOCK DIAGRAMS to life

Using the DATEx module students bring to life the block diagrams that fill communications textbooks. A "block diagram" is a simplified representation of a more complex circuit. An example is shown in Figure 1 below.

Block diagrams are used to explain the principle of operation of electronic systems (like a radio transmitter for example) without worrying about how the circuit works. Each block represents a part of the circuit that performs a separate task and is named according to what it does. Examples of common blocks in communications equipment include the adder, multiplier, oscillator, and so on.

### Didactic philosophy behind the ETT-202 DATEx™ System - Emona TIMS™ and the "Block Diagram" approach

The Emona DATEx telecommunications trainer draws on a well established experimental methodology that brings to life the "universal language" of telecommunications, the BLOCK DIAGRAM.

Originally developed in the 1970's by Tim Hooper, a senior lecturer in telecommunications at The University of New South Wales, Australia, and further developed by Emona Instruments, Emona TIMS™, or "Telecommunications Instructional Modeling System", is used by thousands of students around the world, to implement practically any form of modulation or coding.

# Introduction

DATEx™ Integartion with NI ELVIS

DATEx uses the NI ELVIS for all electronic measurements and test equipment.

OPERATION MODE SWITCH

MANUAL: allows for finger adjustment of knobs and switches.

PC-CONTROL: knobs

and switches are controlled on-screen using DATEx SFPs.

""""" DATEx functions - controlled in MANUAL mode or under PC-CONTROL mode using NI LabVIEW

The Emona DATEx add-in module, illustrated below, has a collection of functional blocks (called modules) that the students can patch together to implement dozens of communications and telecommunications block diagrams.

The Emona DA TEx Add-in Module has a collection ofblocks (called modules) that are patched together to implement dozens of telecommunicationsexperiments.

One-to-One Relationship The figure on the right illustrates the one- to-one relationship between each block of the BLOCK DIAGRAM and the independent functional circuit blocks of the DATEx trainer board.

The functional blocks of the DATEx board are used and re-used in experiments, just as blocks of the block diagram reappear in many different implementations.

Examples of DATEx ™ functional blocks

NI ELVIS inputs and outputs

available at the DATEx front panel

DATEx™ System Conventions

Making lab experiments interesting and informative is important when introducing new technologies and concepts for the first time to students.

In order to help students use the DATEx, so they spend their time learning about the experiment rather than learning how to use the lab equipment, the front panel of the DATEx has been laid out following a series of front panel conventions.

All DATEx modules conform to the following mechanical and electrical conventions.

A - FRONT PANEL SOCKETS

Signal interconnections are made via front panel, 2mm sockets. Sockets on the LEFT HAND SIDE are for signal INPUTS.

All inputs are high impedance, either 10k ohms or 56k ohms depending on the module, in order to reduce effects when connections are made and broken.

Sockets on the RIGHT HAND SIDE are for signal OUTPUTS.

All analog outputs are low impedance, typically 330 ohms. Again, this is to reduce effects when connections are made and broken. Digital outputs are typically 47 ohms.

ROUND sockets, " ® are only for ANALOG signals.

ANALOG signals are typically held near the ETT-202 standard reference level of 4V pk-pk.

SQUARE sockets, " M are only for DIGITAL signals. DIGITAL level signals are TTL level, 0 to 5 V.

ROUND sockets labeled GND, " ® are common, or system GROUND.

Note that input and output impedances are intentionally mismatched, so that signal connections may be made or broken without changing signal amplitudes at module outputs.

Input and output sockets are protected from damage due to mis-wiring.

B - LABELING

All modules are identified as to the function they perform.

Inputs, outputs, controls and switches are labeled so that a student who has had only a brief introduction to DATEx can use the modules without needlessly referring back to this USER MANUAL.

Front panel input and output conventions are illustrated below:

It should be noted that variable controls do NOT have calibration marks. This is intentional, as the philosophy behind DATEx is that students setup and adjust systems by observing and measuring signals. This assists the student in gaining a much greater understanding, feel and insight into the operation of a communications implementation.

# C - PATCHING ERRORS and MAKING EXPERIMENTS

Input and output sockets are protected from damage due to mis-wiring.

An important part of the Emona DATEx teaching philosophy is that students are free to make mistakes while patching together experiments. By observing the result of their mistake - when the signals do not look as expected - they can experiment and self-correct to obtain the desired result.

Students are also free to experiment with different ideas by testing their own understanding of how the mathematics or theories actually function in real life implementations. The DATEx is ideally suited to helping students experience abstract concepts and making mathematics actually come to life.

# How to handle, install and power up DATEx

TM

Handling DA TEx

When holding DATEx, always hold the circuit board by the edges, as illustrated.

Ensure NI EL VIS Prototype BoardPower OFF

Before installing the DATEx add-in module in the NI ELVIS PROTOTYPE PCI SLOT, always check the PROTOTYPING BOARD POWER switch is in the OFF position.

Ensure Prototype Board Power is OFF

Installing DA TEx on NIELVIS

When installing the DATEx add-in module in the NI ELVIS PROTOTYPE PCI SLOT, always carefully check the alignment is correct before pushing DATEx into position.

Power up DA TEx

After DATEx is correctly positioned, turn the NI ELVIS Prototyping Board Power switch ON.

Before removing DATEx™ from the NI ELVIS

Ensure NI EL VIS Prototype BoardPower is OFF

Before removing the DATEx add-in module from the NI ELVIS PROTOTYPE PCI SLOT, always turn the PROTOTYPING BOARD POWER switch OFF.

Ensure Prototype Board Power is OFF before removing DATEx

# DATEx ™ QUICK START - INSTALLATION AND OPERATION

## The following guide outlines the steps required to install and use the NI ELVIS platform and the

## Emona DATEx add-in module.

- Installation of NI components

- Install LabVIEW on your PC - follow NI LabVIEW Installation Guide

- Install NI ELVIS software - follow the NI ELVIS installation Guide

  - Installation of Emona DATEx components

- Install Emona DATEx software using the DATEx CD-ROM supplied. Refer to page 29 of this User Manual for detailed installation instructions. The installation will include 2 SFPs (DATEX- Main and DATEx-Toolbar), DATEx module VIs and DATEx Examples.

- Setting-up the NI hardware components

- Make all power and interface connections between the PC, NI DAQ and NI ELVIS

- Power-up NI DAQ and NI ELVIS; Launch the NI ELVIS software.

- Set-up of the Emona DATEx add-in module

- Confirm the NI ELVIS' "PROTOTYPING BOARD POWER" switch is OFF.

- Holding the DATEx module by its outer edges, position it on the top of the NI ELVIS platform. Carefully align the DATEx circuit board with the NI ELVIS PCI socket and slide into position.

- Operation

- Power-up the DATEX add-in module by switching the NI ELVIS' "PROTOTYPING BOARD POWER" switch to ON. Confirm the the 3 red power LEDs at the left hand side are lit.

- The DATEx add-in module has 2 operating MODES: MANUAL and PC-CONTROL.

For MANUAL mode, slide the switch to the MANUAL position. All front panel knobs and switches are varied manually.

For remote control under NI LabVIEW, slide the switch to PC-CONTROL position. All front panel knobs and switches can be controlled by the DATEx SFP (Soft Front Panels) or DATEx VIs (Virtual Instruments). These must be installed in step 2 above.

- Patch together experiments following the instructions of the DATEx Lab Manual. The DATEx kit includes all equipment required: patching wires, oscillscope leads, DMM leads and headphones.

- The NI ELVIS platform powers the DATEx add-in module, provides the test instruments and interface between NI LabVIEW and the DATEx add-in module.

- DATEx front panel conventions

- The DATEx add-in module follows strict labelling conventions to assist the student in building and exploring experiments. For details see page "iii" of this User Manual. In breif:

Terminals with a CIRCLE label only take analog signals. (typically + 2 V) Terminals with a SQUARE label only take digital type signals. ( +5V TTL-level) INPUT terminals are always on the LEFT of each functional block. OUTPUT terminals are always on the RIGHT of each functional block.

- Power down and unplug DATEx

- Always switch the NI ELVIS' "PROTOTYPING BOARD POWER" switch to OFF before removing the DATEx add-in module.

- Please handle the DATEx add-in module by the outer edges of the circuit board and store in the anti-static bag provided.

# ADDER

The ADDER module is used to sum two signals in real-time.

Two analog input signals A(t) and B(t) may be added together in adjustable proportions G and g. The resulting sum is presented at the output.

## USE

The ETT-202 provides two independent ADDER modules, as pictured above.

The first ADDER includes adjustable GAINS. The second ADDER module has fixed GAINS of unity (x1).

Care must be taken when adjusting the gains to avoid overloading the following modules. Overloading will not cause any damage but it means non-linear operation, which is to be avoided in analog systems. The ADDER is capable of delivering a signal well in excess of the standard reference level, 4 V pk-pk, given a standard level input.

The ADDER can also be used as a normal amplifier by using only one input and turning the gain of the other input to minimum. It is not necessary to ground the unused input.

Note that gains G and g are negative. All inputs and outputs are DC coupled.

## BASIC SPECIFICATIONS

Gain Range 0 < G < 2 (inverting);

0 < g < 2 (inverting); Bandwidth approx 600kHz

AMPLIFIER

Another name for the BUFFER is AMPLIFIER. The BUFFER is a variable gain amplifier.

## USE

The BUFFER may be used to amplify (increase) small signals or attenuate (reduce) large signals.

The BUFFER has a gain, (or amplitude), control on the front panel labeled GAIN.

Care should be taken to ensure that later modules are not overloaded due to excessive gain. Overload will not cause any damage but it means non-linear operation (distortion), which is to be avoided in analog systems. If overload occurs, turn the gain control counter clockwise.

## BASIC SPECIFICATIONS

Bandwidth DC to approx 700kHz Gain 0.2 to 10

### ANALOG I/O - NI ELVIS Function

Two NI ELVIS analog inputs and two NI ELVIS analog outputs are provided at the front panel of the DATEx add-in module.

###### USE

NI ELVIS analog inputs ACH0 and ADCH1, and analog outputs DAC0 and DAC1 are provided from the NI ELVIS PCI connector.

These signals are provided directly, without buffering.

These four signals can be accessed through LabVIEW programs such as the NI ELVIS Bode Analyzer and NI ELVIS Arbitrary Function Generator.

Refer to the NI ELVIS User Manual for programming and user information.

###### BASIC SPECIFICATIONS

ACH0 Stimulus signal connection ACH0+;

ACH0- is connected to GROUND. ACH1 Stimulus signal connection ACH1+;

ACH1- is connected to GROUND. DAC0 Function Output signal DAC0; DAC1 Function Output signal DAC1;

# CHANNEL MODULE

A channel is the space (or medium) between the transmitter and the receiver. Two types of channels are provided in the CHANNEL MODULE.

CHANNEL BPF is used to model a wireless channel, such as a satellite link, radio or TV channel.

BASEBAND LPF is used to model a wire or cable channel, such as a telephone or modem link, or hardwired link between computer networks.

## USE

Both channel modules take signals which are at the ETT-202's analog signal levels of 4Vpk-pk.

CHANNEL BPF is a bandpass filter used for signals which are modulated at the ETT-202 carrier frequency of 100kHz. The filter is centered at 100kHz with a bandwidth of 20kHz.

BASEBAND LPF is a lowpass filter used for data signals which are not modulated. The cut-off frequency of the filter is 2kHz.

### BASIC SPECIFICATIONS CHANNEL BPF Fcenter = 100kHz;

Passband = 18kHz; (approximately at 91kH & 109kHz) Stopband = 140kHz, -40dB (approximately at 30kHz & 170kHz); Gain = 1;

Type: 6th order Chebychev with 0.1dB ripple

BASEBAND LPF Fcut-off = 1.65kHz; Gain = 0.9;

Type: 4th order Butterworth

# DIGITAL I/O - NI ELVIS Function

Four NI ELVIS digital inputs and four NI ELVIS digital outputs are provided at the front panel of the DATEx add-in module.

## USE

NI ELVIS digital inputs DI0 to DI3, and digital outputs D00 to Do3 are provided from the NI ELVIS PCI connector.

These signals are provided directly, without buffering.

These eight signals can be accessed through LabVIEW programs written for the NI ELVIS platform.

Refer to the NI ELVIS User Manual for programming and user information.

## BASIC SPECIFICATIONS

DI0 to DI3 Digital Reader inputs from NI ELVIS connection DI0 to DI3; Do0 to DO3 Digital Writer outputs from NI ELVIS connection D00 to D03;

# DUAL ANALOG SWITCH

The ANALOG SWITCH is just like a normal switch, but it is controlled by a digital signal instead of being controlled physically.

Two identical analog switches are controlled by digital, 0 to 5 V level signals. The outputs of the two switches are added internally and presented at the output of the module.

This module also provides an additional and independent function called "S&H", or Sample-and- Hold". The Sample -and-Hold is the first step in converting "analog" signals to "digital" signals.

## USE

ANALOG SWITCHES

Each ANALOG SWITCH may be closed independently by a digital high, or 5V, signal at the respective control input. The switch outputs are combined internally and are presented at the common output socket, as is illustrated on the module's front panel.

When the ANALOG SWITCH is closed, the signal at the input terminal, IN1 or IN2, is passed to the OUT terminal until the switch is opened.

SAMPLE-AND-HOLD

The Sample-and-Hold has a completely different function. When a digital signal at the CONTROL 1 terminal transitions from a logical low, 0V, to logical high, 5V, (i.e. the positive edge of the digital signal), then the voltage at that instant in time at the IN 1 terminal is sampled and held at the S&H OUT terminal until the next positive edge at the CONTROL 1 terminal.

## BASIC SPECIFICATIONS

### Analog Input Bandwidth 50kHz

### Maximum CONTROL clock 100kHz

CONTROL Input Levels digital-level only, 0V and 5V

### Maximum Analog Input Level 4Vpk-pk

FUNCTION GENERATOR - NI ELVIS Function

Three signals associated with the NI ELVIS Function Generator are provided at the front panel of the DATEx add-in module.

# USE

The NI ELVIS Function Generator is a general purpose signal source which is used extensively in experiments with the DATEx add-in module. The NI ELVIS Function Generator can be controlled in MANUAL mode from the front panel of the NI ELVIS platform or on-screen via the NI ELVIS Virtual Instrument front panel.

The output frequency of the Function Generator can also be controlled by applying a voltage to the VCO input.

Two output signals are provided: the Signal Output (DATEx label FUNC OUT) and the TTL- Compatible Signal Output (DATEX label SYNC). As well, the Frequency Modulation Input is also provided (DATEX label VCO).

These signals are provided directly, without buffering.

Refer to the NI ELVIS User Manual for programming and user information.

# BASIC SPECIFICATIONS

Frequency Range 5Hz to 250kHz; sinewave, triangle and squarewave; VCO Input Voltage -5V < VCO < 5V

FUNC OUT NI ELVIS Function Generator signal output FUNC_OUT;

SYNC NI ELVIS Function Generator TTL-Compatible Clock Signal output SYNC_OUT;

VCO NI ELVIS Function Generator FM input signal FM_IN;

##### HEADPHONE AMPLIFIER

The ETT-202 allows students to listen to sounds, their voice and demodulated messages.

The BUFFER provides a headphone socket output. The volume, or amplitude, of the signal to the headphone is controlled by the GAIN control knob.

USE

This module serves as an electro-acoustic interface between the audio signals within the system and the student.

The BUFFER may be used to amplify (increase) small signals or attenuate (reduce) large signals. The BUFFER has a gain, (or amplitude), control on the front panel labeled GAIN. WARNING: always ensure the GAIN is set to minimum before plugging-in the headphones.

BASIC SPECIFICATIONS

Ouput power 125mW, stereo socket

Headphone Type and Connector 3.5mm stereo, > 8ohm impedance

# LINE CODE ENCODER

Line codes are digital data which has been converted to bipolar voltage levels in a specific manner, following the rules of the line code selected.

Line codes are utilized to assist in minimizing distortion and errors due to external effects. For example, before digital data is transmitted along a computer network cable, the digital, 0 V to 5 V, data is line code encoded to bipolar, say -2V to +2V, data which is optimized for that environment.

The ETT-202 offers 4 Line Codes as part of the SEQUENCE GENERATOR module.

The LINE CODE ENCODER converts the "X" data stream of the SEQUENCE GENERATOR module into one of the selected line codes. The line codes are selected via front panel switch.

## USE

Four line codes are available:

NRZ-L : Non Return to Zero - Level; Bi-^ : Bi-phase Line, otherwise know as Manchester; RZ-AMI : Return to Zero - Alternate Mark Invert NZR-M : Non Return to Zero - Mark

These different line codes will all carry the same original digital information but the signals will have different characteristics and applications - such as wireless transmission, computer network cabling and so on.

The line code is selected as illustrated in the following table:

| LINE CODE | SWITCH |  |
| --- | --- | --- |
|  | LEFT | RIGHT |
| NRZ-L | UP | UP |
| Bi-^ | UP | DOWN |
| RZ-AMI | DOWN | UP |
| NRZ-M | DOWN | DOWN |

The ETT-202 line code waveforms are either bipolar, +2V and -2V, or tri-level, +2V, 0 V and - 2V.

The encoded waveforms are described in the following manner: Line-Code's name, description and <output level>

input data state : resulting output Line-Code waveform; input data state : resulting output Line-Code waveform.

NRZ-L Non-Return to Zero - Level; <bipolar> 1 : high level;

- : low level.

Bi-^ Biphase - level ( Manchester ); <bipolar>

- : transition from high to low in the middle of the bit interval;

  - : transition from low to high in the middle of the bit interval.

RZ-AMI Return to Zero - Alternate Mark Invert; <3-level>

  - : pulse in the first half of the bit width, alternating polarity pulse to pulse;

    - : no pulse.

NRZ-M Non-Return to Zero - Mark; <bipolar>

    - : transition at beginning of interval; 0 : no transition.

BASIC SPECIFICATIONS

Input data from SEQUENCE GENERATOR "X" data sequence

CLK same digital-level clock as SEQUENCE GENERATOR CLK signal, fmax > 100kHz

Output LINE-CODE signal +/-2Vp-p

# MASTER SIGNALS

MASTER SIGNALS provides a set of useful analog and digital signals.

Three of the signals have a frequency of 100kHz, which in the context of the ETT-202 is equivalent to radio or wireless frequencies used in telecommunications.

The other signals are lower in frequency and referred to in telecommunications as "baseband". In the context of the ETT-202 this means that they can be heard with the human ear. These baseband signals can be used in place of a human voice or music in modulation and coding experiments.

## USE

100kHz - CARRIER or WIRELESS signals

The ETT-202 models telecommunications systems at frequencies that are lower than real- world telecommunications systems, so that students can easily and safely view the advanced modulation and coding schemes used in modern telecommunications.

The "carrier" frequency we have chosen in 100kHz. A 100kHz sinusoidal and a 100kHz cosine signal are provided. A digital-level, 0 V to 5 V, squarewave signal is also provided to allow more complex experiments to be implemented.

8kHz - SAMPLE CLOCK signal

In modern telecommunications, analog (voice, music, etc) signals need to be converted to digital signals using a process called "sampling".

The ETT-202 allows students to investigate the principles of sampling, hence an 8kHz digital, squarewave signal is provided for this purpose.

2kHz - ANALOG MESSAGE signal

An analog signal at a frequency of 2kHz is within the range of the human voice and can be heard by the human ear. Therefore it is useful to provide an analog signal which can be used in modulation and coding experiments. This signal is easily viewable and provides very repeatable displays for student investigation.

A digital, squarewave version of the 2kHz signal is also provided. This helps students in contrasting analog and digital signals, and also assists in broadening the range of possible fundamental telecommunications experiments.

SYNCHRONIZATION of MASTER SIGNALS -

It is important to note that a special feature of the MASTER SIGNALS module's signals is that they are synchronized so that students will always obtain clear and stable textbook-like displays.

BASIC SPECIFICATIONS

Output Frequencies carrier: 100kHz in quadrature and a third digital signal sample clock 8.333kHz (sub-multiple of the carrier) message: 2.083kHz sinusoidal and digital, (sub-multiple of the carrier) Output Levels 4V pk-pk, analog

Digital level, 0V to 5V

# MULTIPLIER

The MULTIPLIER module is used to multiply two signals in real-time.

Two analog input signals X(t) and Y(t) may be multiplied together. The resulting product is scaled by a factor of approximately 1/2 so that, with standard level inputs, later modules are not overloaded.

## USE

The ETT-202 provides three independent MULTIPLIER modules, as pictured above.

The first MULTIPLIER includes both AC and DC coupled inputs. The other two MULTIPLIER modules have DC coupled inputs, as per their front panel labels.

The purpose of the AC coupled inputs is to remove any DC components in the input signal, so that only the AC components of each input signal are multiplied.

Note that the output of each MULTIPLIER is input X times input Y times a scale factor "k", that is, k . X . Y

The "k" factor (a scaling parameter associated with "four quadrant" multipliers) is approximately one half. It is defined with respect to the OUTPUT from the module and may be measured experimentally.

## BASIC SPECIFICATIONS

Bandwidth approx 600kHz Characteristic k.X(t).Y(t) k approx 1

# NOISE GENERATOR

The NOISE GENERATOR module provides a wide source of electrical noise. Three different amplitudes of noise level are provided.

Electrical noise is usually added to signals before they pass through the CHANNEL MODULE to model the effects on a signal due to real-world disturbances. Examples of disturbances are other nearby signals, static, nearby electrical machines, electrical transformers and so on.

## USE

The module requires no input or control signals.

The output noise level can be varied in three discrete steps:

0dB, known as the reference level; -6dB, half the reference level; and -20dB, one tenth of the reference level.

If required, the characteristics of the output noise signal can be altered by: filtering, using any of the ETT-202 filter modules; or attenuated or amplified using the ETT-202 BUFFER or ADDER modules.

## BASIC SPECIFICATIONS

Bandwidth 10Hz to < 240kHz, "white" noise Maximum level approx 4.8Vrms

Attenuator steps 0dB (approx 4.8Vrms), -6dB (approx 2.4Vrms) and -20dB (approx 0.48Vrms)

# PCM ENCODER

The PCM ("Pulse Code Modulation") ENCODER is the fundamental building block in telecommunications which converts sound, music and speech into digital data. The heart of a PCM ENCODER is known as an analog-to-digital converter.

The ETT-202 PCM ENCODER is an audio frequency analog-to-digital converter which outputs the digitized data in serial, digital-level PCM format.

Since the digital data comes out in a serial format - one bit after another from a single output - another signal, known as the FS or FRAME SYNCHRONIZATION pulse is required. The FS pulse marks the start of each byte of digital data.

The PCM ENCODER has two modes which can be selected at the PCM/TDM front panel switch:

PCM mode digitizes only one signal at a time, presented at the INPUT 1 socket. TDM mode (Time Division Multiplex) digitizes two signals simultaneously, presented at the INPUT 1 and INPUT 2 sockets.

The PCM ENCODER always requires a digital clock signal, either the 100kHz DIGITAL or 8kHz DIGITAL signals, or the VCO module's DIGITAL output socket.

A minimum of two input signals are required for correct operation: the analog signal to be digitized, INPUT 1, and the sampling "bit" clock, CLK.

INPUT 1 (and INPUT 2) will accept any audio frequency analog-level, bipolar signal ranging from DC up to several kilohertz. Note that the INPUTs are not band limited, so that aliasing may be observed if desired.

The bit clock, CLK, must be a digital-level signal, such as the MASTER SIGNALS 100kHz DIGITAL signal.

## USE

INPUT SIGNALS

Note that careful consideration must be given regarding the sampling theorem, when selecting the relative frequencies of both INPUT and CLK.

PCM DATA

The digital-level digitized data is output serially, one byte at a time. Each data byte represents the analog voltage value at the time of sampling and conversion by the analog-to-digital converter. The data bytes are referred to as code words.

ETT-202 PCM code words are in standard offset binary format.

FRAME SYNCHRONISATION

In digital electronics and telecommunications a FRAME SYNCHRONIZATION pulse, FS, is used to indicate the start of the PCM code word.

The FS signal is normally low and only goes high for one bit period, at the time of the least significant bit of the PCM code word, bit 0.

TDM MODE (TIME DIVISION MUILTIPLEX)

When TDM mode is selected two signals are alternately converted into PCM code words and output at the PCM DATA terminal.

The code words are output in multiplexed format, that is, INPUT 1 code word is output followed by the INPUT 2 code word.

FS in TDM MODE

When observing the PCM DATA signal in TDM MODE, the user can identify which is code word 1 (for INPUT 1) and which is code word 2 (for INPUT 2), by observing the FS signal.

The FS signal will be high in the least significant bit of the PCM code word, bit 0, for INPUT 1, but the FS signal will be low in the least significant bit of the PCM code word, bit 0, for INPUT 2.

# TECHNICAL DETAILS

1. TIMING DIAGRAMS

The following timing diagram describes PCM ENCODER operation in PCM mode.

The following timing diagram describes PCM ENCODER operation in TDM mode.

TIMING DIAGRAM DESCRIPTIONS:

INPUT 1 is the input voltage applied at INPUT 1. The waveform is shown as presented to the analog-to-digital converter by the PCM ENCODER module's internal sample-and-hold circuit.

INPUT 2 is the input voltage applied at INPUT 2. The waveform is shown as presented to the analog-to-digital converter by the PCM ENCODER module's internal sample-and-hold circuit.

CLK is the applied bit clock at the CLK socket.

PCM DATA is the serial data signal at the PCM DATA output.

FS is the frame synchronization signal as provided at the FS output.

2. DATEx PCM CODE WORD RANGES

PCM DATA format is an 8-bit LINEAR Frame where 00000000 = -2.0V (approximately) to 11111111 = +2.0V (approximately). The mid point 10000000 = 0V (approximately). The precise voltage reading at 00000000, 10000000 and 11111111 should be measured, if this information is an important part of the experiment.

# BASIC SPECIFICATIONS

Input Vin +/-2Vpk, DC coupled

Bit Clock Input >128kHz, digital-level

Output Signal serial, digital-level data stream in offset binary format Output Format 8 bits data

Frame Synchronization FS synchronization signal coincident with frame's LSB TDM Mode two input Time Division Multiplex system

# PCM DECODER

The PCM ("Pulse Code Modulation") DECODER performs the reverse function of the PCM ENCODER. The PCM DECODER module converts digital data back to sound, music or speech. The heart of a PCM DECODER is known as a digital-to-analog converter.

The ETT-202 PCM DECODER is an audio frequency digital-to-analog converter which accepts digitized data in serial, digital-level PCM format and outputs an analog signal.

The PCM DECODER module needs the FS or FRAME SYNCHRONIZATION pulse so that it can determine which bit is the start of each byte of digital data.

The PCM DECODER has two modes of operation PCM and TDM. When in TDM mode, the TDM LED will be illuminated.

The PCM DECODER always requires the same digital clock signal as is being used at the PCM ENCODER.

## USE

INPUT SIGNALS

Three digital-level signals are required for correct operation:

PCM DATA, the serial digital data to be converted to an analog signal;

CLK, a synchronized and in-phase bit clock, usually taken directly from the PCM

ENCODER

module's CLK socket; FS, the frame synchronization signal from the PCM ENCODER module.

All these signals must be "clean", squared digital signals, 0 V to 5 V.

PCM DATA

The format of the serial data expected at the PCM DATA input is exactly as generated by the PCM ENCODER module: PCM code words in standard offset binary, with the 8 bits allocated for data.

ANALOG OUTPUT

The OUTPUT terminal provides a bipolar, standard analog-level signal, decoded from the input digital data at PCM DATA. Note that the signal at the OUTPUT terminal is taken directly from the digital-to-analog converter without reconstruction filtering so that individual steps in the conversion process may be observed if desired.

TDM MODE

When TDM mode is selected at the PCM ENCODER module, the PCM DECODER module's "TDM" LED will be lit, confirming that the PCM DECODER is expecting TDM formatted data. Therefore the PCM DECODER will output two analog signals at OUTPUT 1 and OUTPUT 2.

# BASIC SPECIFICATIONS

Input PCM DATA serial, digital-level data stream in offset binary format Input Format 8 bits

Bit Clock Input >128kHz, digital-level; positive edges of CLK & PCM DATA coincident Output Signal approximately +/-2Vpk, DC coupled TDM Mode two channel TDM system,

# PHASE SHIFTER

In telecommunications systems, signals often need to be lined-up or delayed due to the effects of nature or signal processing blocks or circuits. The PHASE SHIFTER is used to help re-align signals as may be required by theory or due the implementation of the task at hand.

The PHASE SHIFTER module introduces a phase shift (delay in time) between its input and output. The amount of phase shift is controlled by the user, via the front panel PHASE knob and 0/180 degree switch.

The PHASE SHIFTER is a fundamental analog function building block and only performs a phase shift on analog signals.

The frequency range of operation, high frequency range or low frequency range, is automatically selected and displayed by LED.

## USE

This variable PHASE SHIFTER is capable of varying the magnitude of the phase shift through 340 degrees in two steps. The 180 degree switch selects the step or region of interest.

If the input is COS(m(t)), then the output is COS(m(t)- F), where F lies between 0 and 180 degrees.

Although the PHASE SHIFTER will operate from a few hertz up to 200kHz it has been optimized to operate in the neighborhood of two frequencies: around 100kHz in the HI range and around 2kHz in the LO range. Frequency range selection is automatically detected and displayed.

The gain through the PHASE SHIFTER is essentially unity for all phases, but note that the amount of phase shift is a function of frequency. This is NOT a wideband phase changer: thus all the frequency components of a complex signal's spectra are not shifted by the same phase.

# SEQUENCE GENERATOR

In the real world there are essentially two types of message signals which need to be transmitted from one location to another: "analog" and "digital". Voice, speech and sound are referred to as "analog" messages. Computer data and digitized analog messages are referred to as "digital" messages. (For clarification regarding digitized analog signals, please refer to the PCM ENCODER module).

In ETT-202 experiments, the SEQUENCE GENERATOR module outputs what could be considered as typical "digital" messages. The advantage of using a SEQUENCE GENERATOR as a digital message source is that it outputs a stable and repeating set of digital data, which means that it is easier to observe the data in experiments.

The SEQUENCE GENERATOR module outputs two different digital sequences - digital sets of data - labeled "X" and "Y". The SYNC terminal outputs a pulse at the start of the X data sequence only.

The data needs to be clocked by a signal at the CLK input. Typically this would be either the 8kHz or 2kHz DIGITAL signals from MASTER SIGNALS module, the DIGITAL output of the VCO module.

## USE

CLK

An external clock signal, CLK, must be provided to operate the SEQUENCE GENERATOR. This must be a digital-level signal. CLK only accepts digital-level signals.

SYNC

The SYNC terminal outputs a digital pulse at the beginning of each X sequence. SYNC is a digital-level signal.

X (short) and Y (long)

The length of sequence X is 31 bits. The length of sequence Y is 255 bits. X and Y are digital level-signals.

## BASIC SPECIFICATIONS

Input Clock Range TTL 1Hz to 200kHz Number of Sequences 2, X and Y Sequence Lengths X = 31 bits, Y = 255 bits Sync indicates start of sequence X

# SERIAL TO PARALLEL

SERIAL TO PARALLEL performs a basic digital logic function, breaking a single serial digital data stream into two parallel streams of data. This module is a fundamental building block used primarily in digital communications experiments.

The two inputs, SERIAL data and digital CLK, are digital level signals.

The two outputs X1 and X2 are square wave signals at bipolar voltages of -2V and +2V.

## USE

SERIAL input accepts any serial digital data stream such as the X or Y output of the SEQUENCE GENERATOR module

A digital-level CLK signal must always be provided. If the SEQUENCE GENERATOR module's X or Y data signals are being used, then the SEQUENCE GENERATOR module's CLK must also be patched to the SERIAL TO PARALLEL module's CLK input.

Outputs X1 and X2 are digital signals at bipolar voltage levels of -2V and +2V.

OPERATION

Alternate bits from the SERIAL input are presented sequentially to outputs X1 and X2. This is also known as a "multiplexing" action.

## BASIC SPECIFICATIONS

Inputs SERIAL digital-level data;

CLK is the digital-level clock signal; Maximum CLK Rate approx 100kHz Outputs bipolar parallel data output

SPEECH

The SPEECH module allows speech and audio signals to be converted into an electrical signal.

# USE

LIVE CHANNEL

The SPEECH module includes a sensitive microphone which will easily pick-up normal speech and background noise. You do not need to lean towards or speak into the microphone.

The microphone will continuously output an electrical of about 0.6Vrms (that is, an effective AC voltage equivalent to 0.6V DC).

# BASIC SPECIFICATIONS

Microphone electret-type with frequency response of 500Hz to 3kHz Output typically 0.6Vrms

# TUNEABLE LPF

Filters are important building block in electronics and telecommunications. A filter is used to pass some signals and block other signals. A low pass filter passes low frequencies and blocks high frequencies.

The TUNEABLE LPF module allows the user to vary which frequencies are passed by adjusting the front panel fc control knob. fc is known as the cutoff frequency of the lowpass filter.

## USE

The TUNEABLE LPF module accepts analog-level signals and outputs analog-level signals.

The cutoff frequency is adjustable by the user. The cutoff range is from 300Hz to 16kHz.

The amplitude of the output signal can be control via front panel GAIN control.

A digital-level squarewave signal is output which has a frequency 100 times higher than the selected cutoff frequency. Measuring the frequency of the digital signal and dividing the frequency by 100 will give the user an instantaneous reading of the TUNEABLE LPF cutoff frequency.

## BASIC SPECIFICATIONS

Filter Range 200 Hz to 12 kHz Filter Order 8th order, Elliptic

Stopband Attenuation > -50dB at 1.4 fc and Passband Ripple < 0.5dB Gain Control 0 to x1.6

# TWIN PULSE GENERATOR

A pulse in electronics or telecommunications is a single digital-level signal which may or may not repeat over a particular time frame. Pulses have many uses, such as markers in time, and control signals.

The TWIN PULSE GENERATOR module accepts a digital-level clock signal and outputs two consecutive digital pulses. The width of the pulses and the time delay between the two pulses can be controlled by the user via the front panel WIDTH and DELAY control knobs. The pulses repeat at the occurrence of each CLK input signal.

## USE

A digital-level signal is applied to the CLK input. The TWIN PULSE GENERATOR then sequentially outputs two pulses.

Two equal width positive pulses occur as a result of each CLK signal positive edge. Pulse Q1 always occurs before pulse Q2. The width of both pulses is controlled by the front panel WIDTH control. The DELAY control varies the spacing between the two pulses.

The pulse WIDTHs can be varied from 5us to 40us. The delay between Q1 and Q2 can be varied from 40us to 350us.

## BASIC SPECIFICATIONS

Clock Frequency Range < 8kHz

Pulse WIDTH 5us < tw < 40us

Pulse DELAY Q2-Q1 40us < td < 300us

# UTILITIES

The Utilities Module houses four independent functional blocks:

- A signal COMPARATOR with digital-level output. The COMPARATOR is used for squaring analog waveforms. The COMPARATOR's threshold level may be set as required by applying a DC voltage to the REF input terminal.

- Precision halfwave RECTIFIER.

- Simple diode and single pole, audio range, Resistor-Capacitor (RC) Lowpass Filter.

- Single pole, audio range, RC Lowpass Filter.

## USE

COMPARATOR

The COMPARATOR will square any analog signal and provide a standard digital-level output. The switching threshold level is determined by the voltage level applied to the REF input.

NOTE: For correct COMPARATOR operation, the REF input must never be left unconnected. The REF input may be connected to GROUND, VARIABLE DC or any other signal source.

## BASIC SPECIFICATIONS

COMPARATOR

Operating Range > 100kHz

TTL Output Risetime 500nsec (typically)

RECTIFIER

Bandwidth DC to 100kHz (approx) DIODE & LPF LPF -3dB 2.6kHz (approx) RC LPF

LPF -3dB 2.6kHz (approx)

# VARIABLE DC - NI ELVIS Function

Two signals associated with the NI ELVIS Variable Power Supply are provided at the front panel of the DATEx add-in module.

## USE

The NI ELVIS Variable Power Supply is a general purpose, variable, bipolar output DC source. The NI ELVIS Variable Power Supply can be controlled in MANUAL mode from the front panel of the NI ELVIS platform or on-screen via the NI ELVIS Virtual Instrument front panel.

Two output signals are provided: positive going voltage and negative going voltages.

These signals are provided directly, without buffering.

Refer to the NI ELVIS User Manual for programming and user information.

## BASIC SPECIFICATIONS

+ NI ELVIS Variable Power Supply positive signal output SUPPLY+; - NI ELVIS Variable Power Supply negative signal output SUPPLY-;

# DATEx™ SOFTWARE INSTALLATION & SFP OPERATION

## Installing NI DAQ and NI ELVIS software

Before installing the EMONA DATEx software, ensure that the NI DAQ and NI ELVIS software has been correctly installed as per the accompanying NI DAQ and NI ELVIS user instructions.

Using NI MAX (Measurement and Automation Explorer) ensure that the NI DAQ used with NI ELVIS (6251 device) is set to either DEVICE 1 or DEVICE 2.

Confirm the correct operation of NI ELVIS before proceeding.

## Installing the EMONA DATEx software

The CD-ROM supplied with the DATEx Kit includes the following items:

- DATEx-Main SFP (Soft Front Panel).

A large SFP which allows all DATEx switches and knobs to be controlled on-screen.

- DATEx-Toolbar SFP.

A compact sized SFP which allows all DATEx switches and knobs to be controlled onscreen.

- DATEx polymorphic VIs for control of the individual DATEx modules.

These VIs can be used by any LabVIEW program to control any of DATEx's variable parameters.

- Example LabVIEW programs which demonstrate LabVIEW control of the DATEx.

A selection of simple example programs for using DATEx block functions.

The DATEx CD-ROM also includes a soft copy of the Lab Manual,

- Emona DATEx Lab Manual Volume One in PDF format Installation procedure

The DATEx software is self installing. Insert the CD-ROM in your PC drive, open the CD-ROM directory and run DATEx-setup.exe

# Using the DATEx-Main and DATEx-Toolbar SFPs

To launch the DATEx SFP go to START > click on DATEx and select either DATEx-Main-SFP or DATEx-Toolbar-SFP.

Ensure that the Device selection radio button is set to match the NI DAQ unit device number, either DEV 1 or DEV 2.

Use the mouse left button to click and drag the knob or switch to the desired position.

TAB can also be used to select knobs and switches. Keyboard arrow keys (left & right) can then be used to change knob positions.

# Confirming PC-to-DATEx™ Communications

To confirm that the on-screen mouse control signals are being passed from the PC through the NI-DAQ to NI ELVIS and the DATEx add-in module, do the following:

Toggle any DATEx on-screen switch and look at the 4 LEDs located within the DATEx add-in module. Refer to the diagram to the right.

As the switch (or potentiometer) is varied, the communications LEDs should flicker on-and-off.

If the LEDs (A, B, D) flicker, then communication signals are being passed from the PC through to the DATEx.

If the communications LEDs do not flicker, then the check the following:

- That the correct DEV number is selected for the NI-DAQ in the DATEx SFP.

- That the DATEx SFP is "running".

- That the PROTOTYPING BOARD POWER switch is in the ON position.

- .That the PC, NI-DAQ and NI ELVIS are connected correctly.

- That the NI ELVIS virtual instruments are functioning correctly.

- That the DATEx add-in module is correctly positioned and plugged-into the NI ELVIS PCI slot.

# USING DATEx™ VIs within LabVIEW PROGRAMS

# FOR PC-CONTROL

PC-Control Mode

When operated in PC-CONTROL mode, the EMONA DATEx add-in module has all user controls adjustable under LabVIEW program control. The PC-CONTROL slide switch on DATEx board MUST be in PC-CONTROL position.

The circuit blocks : ADDER, AMPLIFIER, PCM ENCODER, PHASE SHIFTER, SEQUENCE GENERATOR, TUNEABLE LPF & TWIN PULSE GENERATOR all have adjustable controls, either potentiometers or switches.

Each one of these controls can be independently controlled from within LabVIEW program code to achieve PC-controlled experimental setups and systems.

A unique LabVIEW function block exists for each of these DATEx circuit blocks, and can be inserted into a VI as follows:

- Browse for, or Select VI, and open the DATEx function.LLB from the directory selected during installation.

- Note that DATEx software can also be run off the supplied CD without installing if preferred.

- From within DATEx functions.LLB, select the circuit block VI you wish to use and insert into your program VI.

- Follow the basic program flow of INITIALISE DAQ: DATEx function block: STOP DAQ

- Connect controls to the inputs in accordance with the guidelines as described in the DATEx Functions controls Valid Input Range. Guidelines are displayed in the contextual help for each block, when placed on the block diagram. See the example

below:

Example programs are supplied which demonstrate both very simple test setups as well as more considered sample programs.

The simple test programs are titled:

adder_vary.vi:

pcm_tdm_test.vi:

tpg_amp.vi:

sg_test.vi:

width_tpg.vi:

tlpf_ramp_freq.vi: (screen image of simple vi below)

The practical example files supplied are titled: eye-graph.vi

auto-null-using-ps-elvis.vi gain-control-with-amp-elvis.vi

nulling-using-ps-adder.vi (screen images displayed below)

List of recommended literature

Basic

- Pryanishnikov V.А. Electronics: Course of lectures. - SPb.: Teacher and student: KORONA print, 2006.

- PasynkovV.V., Chorkin L.К. Semiconductor devices. Textbook for HEI. - SPb.: “Lan”, 2003.

- Opadchyi U.F., Gludkin О.P., Gurov А.I. Analog and digital electronics (general course): Textbook for HEI. Under edition of О.P. Gludkin. - M.: Hot Line – Telecom, 2005.

Additional

- Gusev V.G., Gusev U.M. Electronics: Textbook for HEI. - М.: High school, 2005.

- Paton B. Bases of analog and digital electronics. - М.: 2002.

- Vuchkov I., Boyadzhieva L., Solakov Е. Applied linear regress analysis. - М.: “Finance and Statistics”, 1987.

- Bystrov U.А. and others. Electric devices and equipment on its base: Reference book. - М.: IP RadioSoft, 2002.

- Electrical techniques and electronics in experiments and exercises: Laboratory practical work on Electronics Workbench: In 2 b. /Under edition of D.I. Panphilov - Т.2: Electronics. - М.: DODEKA, 2000

- Trevis G. LabVIEW for everyone. - М.:DMK Press; PriborKomplect, 2004.

- Page L.I., Tocholin D.А., Pollak B.P. LabVIEW for beginners and specialists. - М.: HotLine – Telecom, 2004.

Sarybaev A.A., Aslanov M.T

METHODICAL INSTRUCTION FOR LABORATORY WORKS
ON THE DISCIPLINE OF

on the discipline “Electronics and circuitry analog devices 2”

It is sent for printing _______2013

Format of paper Х*Y 1/16

Paper is typographical. Press is offset. Volume 2 items of l.

Circulation – 50 copies. Order № ___

©  Edition of М. Auezov South Kazakhstan State University

Publishing center of M. Auezov SKSU, Shymkent city,

Tauke-khan avenue, 5