Ф.7.03-09

Ministry of Education and Science of the Republic of

Kazakhstan

M.O. Auezov SOUTH KAZAKHSTAN STATE UNIVERSITY

Chair "Automation, telecommunications and control"

Methodical instruction for laboratory works
on the discipline of

Systems and devices of sound and television broadcasting

for students of a specialty: 5B071900 - “Radio engineering, electronics and telecommunication”

SHYMKENT-2013

УДК 658.012.011

ББК 6П7.1

The composer senior lecturer, cand.tech.sci. Sarybaev A.A., Aslanov M.T Methodical instruction for laboratory works on the discipline of «Theory of electric communication » for students of a speciality 5B071900 - “Radio engineering, electronics and telecommunication”, Shymkent: SK.SU, 2013, 32p.

The methodical instruction for laboratory works is made according to requirements ГОСО and the discipline program «Theory of electric communication » and includes all necessary data on performance of course laboratory researches by that.

The methodical instruction for laboratory works the «Theory of electric communication » contains instructions to performance of laboratory on discipline. Necessary recommendations are resulted in each laboratory work and similar examples are considered. At the end of each section the list of control questions are resulted.

Reviewer: Tasanbaev S.E - cand.tech.sci., the associate professor of chair «Automation, telecommunications and control» SKSU of M.Auezov

The methodical instruction for laboratory works is discussed and recommended for publication at the meeting of the chair “Automation, Telecommunication and Control” (Minutes № 6 “05” January 2013)

The methodical instruction for laboratory works is recommended for printing by faculty meeting the methodical commission of faculty «Information technology» (the report №4 from «18 » January 2013)

The methodical instruction for laboratory works is recommended for publication by Methodical Council SKSU of M.Auezov, the report № __ from «___» ____ 2013

©  the South Kazakhstan State University (SKSU), 2013

Contents

Introduction	i - iv

How to install and power up DATEx™ for NI ELVIS II/+	v

How to install and power up DATEx™ for NI ELVIS I	vii

ESD Warning, Unpacking, Handling, Set-up and

ESD Preventative Measures	ix

- - An introduction to the NI ELVIS II test equipment	1

- - An introduction to the DATEx experimental add-in module	1

- - An introduction to soft front panel control	1

Introduction

The ETT-202 DATEx ™ Lab Manual Overview

The ETT-202 Lab Manual Volume One covers a broad range of introductory digital and analog telecommunications topics through a series of 20 carefully paced, hands-on laboratory experiments. Each experiment is written to support the theoretical concepts introduced in the class work of a first course in modern telecommunications.

Each DATEx experiment presents an interesting, hands-on learning experience for the student. In each experiment the student is challenged to build, measure and consider: there are no "instant" or "cookbook-style" experiments. DATEx is actually a true engineering modeling system where students see that the block diagrams so common in their textbooks represent real functioning systems.

The Emona DA TEx Add-in Module has a collection of blocks (called modules) that are patched together to implement dozens of telecommunications experiments.

Equipment Required

Experiments make use of the Emona DATEx telecommunications trainer kit together with the NI ELVIS I or II platform and NI LabVIEW running on a PC. The functionality and range of the virtual instrumentation available depends on the NI DAQ that is coupled with NI ELVIS I platform. ELVIS II has all necessary interfacing circuitry built into it.

Refer to the ETT-202 DATEx USER MANUAL for further details, as well as information on the installation and use of the DATEx/NI ELVIS experiment system.

Student Academic Level

Experiments in this volume have been prepared for students with only a basic knowledge of mathematics and a limited background in physics and electricity.

Students with a higher level of competence in mathematics will also gain a deeper understanding of telecommunications theory by using the DATEx system. Due to the engineering "modeling" nature of the DATEx system, they will be able to investigate more complex issues, carry out additional measurements and then contrast their findings to their theoretical understanding and mathematical analysis.

Didactic philosophy behind the ETT-202 DATEx™ System - Emona TIMS™ and the "Block Diagram" approach

The Emona DATEx telecommunications trainer draws on a well established experimental methodology that brings to life the "universal language" of telecommunications, the BLOCK DIAGRAM. Originally developed in the 1970's by Tim Hooper, a senior lecturer in telecommunications at The University of New South Wales, Australia, and further developed by Emona Instruments, Emona TIMS™, or "Telecommunications Instructional Modeling System", is used by thousands of students around the world, to implement practically any form of modulation or coding.

Block Diagrams

Block diagrams are used to explain the principle of operation of electronic systems (like a radio transmitter for example) without worrying about how the circuit works. Each block represents a part of the circuit that performs a separate task and is named according to what it does. Examples of common blocks in communications equipment include the adder, multiplier, oscillator, and so

The TIMS™ and hence DATEx™ approach to implementing telecommunications experiments through realizing BLOCK DAIAGRAMS has the following benefits in the educational environment:

- Students gain practical experience with true mathematical modeling hardware, designed specifically for implementing telecommunications theory.

- Students actually build each experiment stage-by-stage, in an engineering manner, by following the BLOCK DIAGRAM.

- Students are free to try "what-if" scenarios to validate their understanding of the theory being investigated, by viewing real, real-time electrical signals.

- DATEx is designed to allow students to make mistakes, hence students will learn from their hands-on experiences as they investigate their findings.

One-to-One Relationship The figure on the right illustrates the one- to-one relationship between each block of the BLOCK DIAGRAM and the independent functional circuit blocks of the DATEx trainer board.

The functional blocks of the DATEx board are used and re-used in experiments, just as blocks of the block diagram reappear in many different implementations.

NI Lab VIEW™ and DATEx ™

The Emona DATEx add-in module is fully integrated with the NI ELVIS platform and NI LabVIEW environment. All DATEx™ knobs and switches can be varied either manually or under the control NI LabVIEW VIs.

DATEx™ VIs are provided in the DATEx kit so that the student has the ability further enhance the experiment capabilities of the DATEx hardware, by utilizing the resources of NI LabVIEW and even integration with NI's wide range of RF products.

Guidelines for Using the Lab Manual

The experiments in this volume have been prepared for students with only a basic knowledge of mathematics. However, due to the engineering "modeling" nature of the DATEx add-in module, students with a higher level of competence in mathematics will equally gain a deeper understanding of telecommunications theory by carrying out these experiments.

The 20 chapters cover a broad range of telecommunications concepts, from fundamental topics familiar to all students, such as AM and FM broadcasting, through to the underlying technologies used in the latest mobile telephones and wireless systems. In each experiment, the core technology is revealed to the student, at its most fundamental level. The first chapters also provide a solid introduction to the NI ELVIS platform and the use of NI LabVIEW virtual instrumentation.

Chapters can be covered in any order, however, it is imperative that all students complete the first four chapters before proceeding to the subsequent chapters.

- Chapter 1 introduces the NI ELVIS test equipment.

- Chapter 2 introduces the Emona DATEx experimental add-in module.

- Chapter 3 introduces the DATEx Soft Front Panel control, and

- Chapter 4 introduces the concept of mathematical modeling using electronic functional blocks.

In order to make the student's learning experience more memorable, the student is usually able to both view signals on the NI ELVIS oscilloscope and then listen to their own voice undergoing the modulation or coding being investigated.

Making Mistakes and Mis-wiring

An important factor which makes the learning experience more valuable for the student is that the student is allowed to make wiring mistakes. DATEx inputs and outputs can be connected in any combination, without causing damage. As the student builds the experiment, they need to make constant observations, adjustments and corrections. If signals are not as expected then the student needs to make a decision as to whether the correction required is an adjustment or an incorrectly placed patching wire.

Structure of the Experiments and Topics

Each experiment in the DATEx Lab Manual provides a basic introduction to the topic under investigation, followed by a series of carefully graded hands-on activities. At the conclusion of each sub section the student is asked to answer questions to confirm their understanding of the work before proceeding.

It should be noted that the DATEx add-in module can implement many more experiments than are documented in this Volume One Lab Manual and further experiments will be released in later manuals.

Finally, since the ETT-202 Trainer is a true modeling system, the instructor has the freedom to modify existing experiments or even create completely new experiments to convey new and course specific concepts to students.

How to install and power up DATEx™ with NI ELVIS II/+

Handling DA TEx

When holding DATEx, always hold the circuit board by the edges, as illustrated.

Ensure NI EL VIS II/+ Prototype Board Power OFF

Before installing the DATEx add-in module in the NI ELVIS II/+ PROTOTYPE PCI SLOT, always check the PROTOTYPING BOARD POWER switch is in the OFF position.

Installing DA TEx on NI ELVIS II A When installing the DATEx add-in module in the NI ELVIS II/+ PROTOTYPE PCI SLOT, always carefully check the alignment is correct before pushing DATEx into position.

Power up DA TEx

After DATEx is correctly positioned, turn the NI ELVIS II/+ Prototyping Board Power switch ON.

Before removing DATEx™ from the NI ELVIS II/+

Ensure NI EL VIS II/+ Prototype Board Power is OFF

Before removing the DATEx add-in module from the NI ELVIS ИЛ PROTOTYPE PCI SLOT, always turn the PROTOTYPING BOARD POWER switch OFF.

How to install and power up DATEx™ for NI ELVIS I

Handling DA TEx

When holding DATEx, always hold the circuit board by the edges, as illustrated.

Ensure NI EL VIS I Pro to type Board Power OFF

Before installing the DATEx add-in module in the NI ELVIS I PROTOTYPE PCI SLOT, always check the PROTOTYPING BOARD POWER switch is in the OFF position.

Installing DA TEx on NI ELVIS I

When installing the DATEx add-in module in the NI ELVIS I PROTOTYPE PCI SLOT, always carefully check the alignment is correct before pushing DATEx into position.

Power up DA TEx

After DATEx is correctly positioned, turn the NI ELVIS I Prototyping Board Power switch ON.

Before removing DATEx™ from the NI ELVIS I

Ensure NI EL VIS I Prototype Board Power is OFF

Before removing the DATEx add-in module from the NI ELVIS I PROTOTYPE PCI SLOT, always turn the PROTOTYPING BOARD POWER switch OFF.

ESD Warning - ESD Sensitive Product

Caution Although this product has been designed to be as robust as possible, ESD (Electrostatic Discharge) can damage or upset this product. This product must be protected at all times from ESD. Static charges may easily produce potentials of several kilovolts on the human body or equipment, which can discharge without detection. Industry-standard ESD precautions must be employed at all times.

The Emona ETT-202 board is designed and intended for use as an experimental platform for hardware or software in an educational/professional laboratory environment. To facilitate usage, the board is manufactured with its components and connecting traces openly exposed to the operator and the environment. As a result, ESD sensitive (ESDS) components on the board, such as the semiconductor integrated circuits, can be damaged when exposed to an ESD event. To indicate the ESD sensitivity of the Emona ETT-202 board, it carries the symbol shown at left.

Unpacking, Transporting, and Storage

When unpacking the Emona ETT-202 board from its shipping carton, do not remove the board from the antistatic packaging material until you are ready to complete the installation. Before unwrapping the antistatic packaging, discharge yourself by touching a grounded bare metal surface, touching an approved anti-static mat, or wearing an ESD strap. When transporting or storing the Emona ETT-202 board, first place it in an antistatic container or packaging.

Handling and Setup

Handling the Emona ETT-202 board can damage the board components if ESD prevention measures are not applied. Before handling or setup, equalize your potential with the board by touching one of the integrated ESD discharge pads. During all handling and setup, ESD prevention measures must be applied. In addition, the Emona ETT-202 board should be handled by the edges. Touching exposed circuits, components or connectors could result in an ESD event. When setting up Emona ETT-202 board, observe the following guidelines to minimize the potential impact of ESD:

- Patch the desired experiment blocks using 2mm patch cords.

- Set switches and other controls to initial settings.

- Ensure the NI ELVIS is powered correctly by plugging the NI ELVIS AC/DC power supply brick into an appropriate AC outlet.

- Move the board power switch ON.

Operation

When operating the Emona ETT-202 board, ESD can cause upset as well as damage to the board components. Therefore, apply ESD prevention measures whenever operating the Emona ETT-202 board. In addition, observe the following guidelines:

- Do not touch exposed traces or components on the board while the board is powered on.

- Exercise caution when manipulating switches, buttons, knobs, and other controls while the board is powered on.

ESD Prevention Measures

ESD prevention measures focus on reducing or eliminating the build-up of static charge that may result in an ESD event that could damage or upset sensitive electronics. To minimize the potential for an ESD event, implement the following measures:

- Perform all work at an approved work station.

- Use an approved antistatic mat to cover your work surface.

- Wear a conductive wrist strap attached to the antistatic mat and a good earth ground.

- Before handling or beginning work, equalize your potential with the board by touching one of the "GND" pads.

1 - An introduction to the N1 ELVIS II test equipment

Experiment 1 - An introduction to the N1 ELVIS II test equipment

Preliminary discussion

The digital multimeter and oscilloscope are probably the two most used pieces of test equipment in the electronics industry. The bulk of measurements needed to test and/or repair electronics systems can be performed with just these two devices.

At the same time, there would be very few electronics laboratories or workshops that don't also have a DC Power Supply and Function Generator. As well as generating DC test voltages, the power supply can be used to power the equipment under test. The function generator is used to provide a variety of AC test signals.

Importantly, N1 ELVIS II has these four essential pieces of laboratory equipment in one unit (and others). However, instead of each having its own digital readout or display (like the equipment pictured), NI ELVIS II sends the information via USB to a personal computer where the measurements are displayed on one screen.

On the computer, the NI ELVIS II devices are called "virtual instruments". However, don't let the term mislead you. The digital multimeter and scope are real measuring devices, not software simulations. Similarly, the DC power supply and function generator output real voltages.

The experiments in this manual make use of all four NI ELVIS II devices and others so it's important that you're familiar with their operation.

The experiment

This experiment introduces you to the NI ELVIS II digital multimeter, variable DC power supplies (there are two of them), oscilloscope and function generator. Importantly, the oscilloscope can be a tricky device to use if you don't do so often. So, this experiment also gives you a procedure that'll set it up ready to display a stable 2kHz 4Vp-p signal every time. Importantly, it's recommended that you use this procedure as a starting point for the other experiments in this manual.

It should take you about 50 minutes to complete this experiment.

Equipment

Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Some things you need to know for the experiment

This box contains definitions for some electrical terms used in this experiment. Although you've probably seen them before, it's worth taking a minute to read them to check your understanding.

The amplitude of a signal is its physical size and is measured in volts(V). It is usually measured either from the middle of the waveform to the top (called the peak voltage) or from the bottom to the top (called the peak-to-peak voltage).

The period of a signal is the time taken to complete one cycle and is measured in seconds(s). When the period is small, it is expressed in milli seconds (ms) and even micro seconds (^s).

The frequency of a signal is the number of cycles every second and is measured in hertz (Hz). When there are many cycles per second, the frequency is expressed in kilo hertz (kHz) and even mega hertz (MHz).

A sinewave is a repetitive signal with the shape shown in Figure 1.

Figure 1

A squarewave is a repetitive signal with the shape shown in Figure 2.

Figure 2

Procedure

Part A - Setting started

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to Manual.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software per the instructor's directions.

Note: If the NI ELVISmx software has launched successfully, the window called "ELVIS - Instrument Launcher" will be visible (see Figure 3).

Part В - The NI ELVIS II Digital Multimeter

The NI ELVIS II Digital Multimeter (DMM) is an instrument that can measure the following electrical properties: DC & AC voltages, DC & AC currents, resistance, capacitance and inductance. Its operation is briefly introduced next.

  - Use the mouse to click on the "DMM" button on the NI ELVISmx Instrument Launcher.

Note: If the digital multimeter virtual instrument has launched successfully, the instrument's window will be visible (see Figure 4).

The digital multimeter's measurement options are selected using the Measurement and Settings controls on the virtual instrument (near the mouse-pointer in Figure 4).

  - Move the mouse-pointer over these controls but don't click on any of them yet.

Note: As you do this, you'll notice that a pop-up appears to tell you by name what measurement mode the controls activate.

Ask the instructor to check your work before continuing.

  - Click back and forth between one of the Voltage controls (marked V) and one of the Current controls (marked A).

Note 1: As you do, notice that the buttons on the virtual instrument are animated. The selected control fades as though it has been physically pressed in.

Note 2: Notice also that the Banana Jack Connections window updates to tell you which of the DMM's banana jacks to use on the left side of the NI ELIVS II for that particular measurement.

Importantly, simply launching the DMM virtual instrument doesn't activate the instrument's hardware. This must be done every time the DMM virtual instrument is launched using its Run control (the button with the green arrow).

  - Click on the DMM's Run control.

  - Click on each of the Measurement and Settings controls in turn while watching the DMM's readout.

Note: As you do, notice that the readout updates to tell you the unit of measurement (eg Vfor volts, A for amps, etc). The readout also indicates the relative size of the measurement (for example, m for milli, M for mega, etc). See the instructor for more information if you're not familiar with the metric system of multiples and sub-multiples.

Question 1

Given you've not been asked to connect the digital multimeter's inputs to anything yet, why does the DMM read very small values of voltage and current instead of zero?

If you examine the DMM virtual instrument closely you'll notice that there are other settings on the DMM virtual instrument that can be adjusted including the Mode, Null Offset and Acquisition Mode. These controls default to appropriate settings for regular use so we'll not discuss them further here. Where adjustment of these controls is necessary, they'll be explained at the appropriate place in the experiments.

Part С - The NI ELVIS II Variable Power Supplies

The NI ELVIS II Variable Power Supplies (VPS) is an instrument that can simultaneously output two DC voltages (one positive and one negative) to terminals on the Emona DATEx. Its operation is briefly discussed next.

  - Use the mouse to click on the "VPS" button on the NI ELVISmx Instrument Launcher.

Note 1: Don't close the NI ELVISmx DMM virtual instrument because you'll be using it to verify the operation of the Variable Power Supplies.

Note 2: If the Variable Power Supplies virtual instrument has launched successfully, the instrument's window will be visible (see Figure 5).

  - Put the NI ELVIS II Variable Power Supplies into Manual mode by checking the boxes next to the word Manual on both the positive and negative sides of the virtual instrument.

Tip: One of the boxes is near the mouse-pointer in Figure 5.

Note: Once you've performed this step, you'll notice that the virtual controls fade. This tells you that the virtual power supplies' outputs are controlled manually using the controls on the top right of the NI ELVIS II (directly below the USB Ready & Active indicators).

  - ClickontheDMM's control to put the unit into DC voltage measuring mode.

  - Set the two Variable Power Supplies Voltage controls to about half of their travel.

  - Connect the set-up shown in Figure 6 below.

Tip: Use the 4mm banana plug to 2mm banana plug patch lead.

Note: As you perform this step, you should see some activity on the DMM virtual instrument and the measurement on its readout change to about 6V.

  - Use the Voltage control to determine the Variable Power Supplies' minimum and maximum positive output voltages. Record these in Table 1 below.

  - Connect the DMM to the Variable Power Supplies' negative output and repeat Step 18.

| Table 1 | Minimum output voltage | Minimum output voltage |
| --- | --- | --- |
| Positive (+) output |  |  |
| Negative (-) output |  |  |

Ask the instructor to check your work before continuing.

While the DMM can be used for measuring the Variable Power Supplies' outputs, the instrument can monitor its own outputs freeing the digital multimeter for other uses. The next steps demonstrate this.

  - Check the box at the top of the NI ELVIS II Variable Power Supplies' virtual instrument shown in Figure 7 below.

  - Vary the Variable Power Supplies' negative Voltage control and compare the values on the displays of the two virtual instruments - they should be the same.

Ask the instructor to check your work before continuing.

Part D - The NI ELVIS II Oscilloscope

The NI ELVIS II Oscilloscope (or just "scope") is a fully functional dual channel oscilloscope that allows engineers and technicians to measure AC waveforms and view their shape. Its operation is briefly discussed next.

  - Close the virtual instruments for the digital multimeter and Variable Power Supplies.

  - Use the mouse to click on the "Scope" button on the NI ELVISmx Instrument Launcher.

Note: If the scope virtual instrument has launched successfully, the instrument's window will be visible (see Figure 8).

The NI ELVIS II Oscilloscope is operated using the controls on its virtual instrument. Although operating the NI ELVIS II Oscilloscope is much easier than operating other types of scopes, it can still be a little tricky to use when you're new to this piece of test equipment. The procedure on the next page is one that you can use to set it up ready to reliably view waveforms and take measurements when undertaking DATEx experiments.

Procedure for setting up the NI ELVIS II Oscilloscope

  - Follow the procedure below. Call the instructor for assistance if you can't find a particular control.

Note: Much of this procedure simply involves checking that control settings are in the default positions used by the NI ELVIS II Oscilloscope at the time of writing this manual.

General

i) Check that the Cursors On box is doesn't have a tick in it. Vertical

    - Check that the Channel 0 Source control is set to SCOPE CH 0 and the Channel 1 Source control is set to SCOPE CH 1.

    - Check that the Probe control for both channels is set to 1*.

iii Set the Coupling control for both channels to AC. )

      - Check that the Scale Volts/Divcontrol for both channels is set to 1V/div.

      - Check that the Vertical Position control for both channels is in the middle of their travel.

Timebase

i) Set the Time/Divcontrol to the 500jus/div position. Trigger

        - Set the Type control to Edge.

        - Set the Source control to CH 0 Source.

        - Check that the Level control is set to 0.

iii) Check that the Slope control is set to the _f position.

          - Activate the scope's hardware by clicking on its Run control.

Ask the instructor to check your work before continuing.

The next part of this experiment lets you familiarize yourself with NI ELVIS II Oscilloscope by observing and measuring a DATEx signal.

26. Connect the set-up shown in Figure 9 below.

Note: Notice that the connection to the Master Signals' 2kHz SINE output must be made with the red banana plug. The black banana plug should be connected to any one of the ground (GND)) sockets on the Emona DATEx.

When measuring the amplitude of an AC waveform using a scope, it's common to measure its peak-to-peak voltage. That is, the difference between its lowest point and its highest point. This is shown in Figure 10. Importantly, knowing the waveform's peak-to-peak voltage allows us to calculate its RMS voltage where required.

The other dimension of an AC waveform that's important to measure is its period. The period is the time it takes

to complete one cycle and this is also shown in Figure 10. While knowing the waveform's period may be useful in its own right, it also allows us to calculate the signal's frequency using the equation:

Measuring the amplitude of signals and determining their frequency using conventional scopes is a little more involved that using a digital multimeter. As such, it can be easy for the novice to make mistakes. Helpfully, the NI ELVIS II Oscilloscope includes meters that measure voltage and frequency for you and readout the information on the display. The location of this information on the virtual instrument is below the graticule as shown in Figure 11 below.

- Record the scope's measured values for voltage (RMS and peak-to-peak) and frequency in Table 2 below.

- Use the signal's frequency to work backwards to calculate and record its period.

Tip: You'll have to transpose the equation on the previous page to make period (P) the subject.

| Table 2 |  |  |
| --- | --- | --- |
| RMS voltage |  |  |
| Frequency |  |  |
| Pk-Pk voltage |  |  |
| Period |  |  |

Ask the instructor to check your work before continuing.

Part E - The NI ELVIS II Function Generator

The NI ELVIS II Function Generator (FGEN) is an instrument that can output AC signals of various shapes and at various frequencies to terminals on the Emona DATEx. Its operation is briefly discussed next.

- Use the mouse to click on the "FGEN" button on the NI ELVISmx Instrument Launcher.

Note 1: Don't close the NI ELVISmx Scope virtual instrument because you'll be using it to verify the operation of the function generator.

Note 2: If the function generator virtual instrument has launched successfully, the instrument's window will be visible (see Figure 12).

- Check the box that puts the function generator into Manual Mode.

Note: Once you've performed this step, you'll notice that the virtual controls fade. This tells you that the function generator's output is now a sinewave whose amplitude and frequency are controlled manually by the Frequency and Amplitude controls on the right side of the NI ELVIS II.

- Set the function generator's Amplitude control on the right side of the NI ELVIS II to about half its travel.

- To observe the function generator's output, connect the set-up shown in Figure 13 below.

Note: Again, the connection to the function generator's output must be made using the lead's red banana plug.

- Adjust the scope's Timebase control for 1ms/div.

Note: Once this step has been performed, you should see one complete cycle of a sinewave.

- Vary the function generator's Amplitude control left and right and observe the effect on the function generator's output.

- Determine the function generator's manually adjustable minimum and maximum output voltages and record your measurements in Table 3 below.

Table 3

| Minimum output V |  |
| --- | --- |
| Maximum output V |  |

36. Vary the function generator's Frequency control on the right side of the NI ELVIS II and observe the effect on the function generator's output.

Note 1: If this control doesn't seem to have an effect, keeping turning it.

Note 2: This control can vary the frequency of the function generator's output between 0.2Hz and 5MHz. However, it would take a lot of turns of the manual Frequency control to sweep between them. Experiment 3 introduces you to the controls on the function generator's virtual instrument which are more convenient to use.

Ask the instructor to check your work before finishing.

List of recommended literature

Basic

- Звуковое и телевизионное вещание" / Кохно М. Т., Мн.: Экоперспектива, 2001

- Теория электрической связи. Под редакцией профессора Д.Д.Кловского - М.: «Радио и связь», 1998.

- Г. П. Катунин, Г. В. Мамчев, В. П. Попантонопуло, В. П. Шувалов “Телекоммуникационные системы и сети” радиосвязь, радиовешание, телевидение Том 2

Additional

- "Звуковое и телевизионное вещание" / Выходец Коваленко, Кохно М. Т., М.: Радио и связь, 1987

- "Основы радиосвязи, радиовещания и телевидения", составитель Семененко М. И., 2004

- Bystrov U.А. and others. Electric devices and equipment on its base: Reference book. - М.: IP RadioSoft, 2002.

- Electrical techniques and electronics in experiments and exercises: Laboratory practical work on Electronics Workbench: In 2 b. /Under edition of D.I. Panphilov - Т.2: Electronics. - М.: DODEKA, 2000

- Trevis G. LabVIEW for everyone. - М.:DMK Press; PriborKomplect, 2004.

- Page L.I., Tocholin D.А., Pollak B.P. LabVIEW for beginners and specialists. - М.: HotLine – Telecom, 2004.

Sarybaev A.A., Aslanov M.T

METHODICAL INSTRUCTION FOR LABORATORY WORKS
ON THE DISCIPLINE OF

on the discipline “Systems and devices of sound and television broadcasting”

It is sent for printing _______2013

Format of paper Х*Y 1/16

Paper is typographical. Press is offset. Volume 2 items of l.

Circulation – 50 copies. Order № ___

©  Edition of М. Auezov South Kazakhstan State University

Publishing center of M. Auezov SKSU, Shymkent city,

Tauke-khan avenue, 5