Ф.7.03-09

Ministry of Education and Science of the Republic of

Kazakhstan

M.O. Auezov SOUTH KAZAKHSTAN STATE UNIVERSITY

Chair "Automation, telecommunications and control"

Laboratory practical work
on the discipline of

Systems and devices of sound and television broadcasting

for students of a specialty: 5B071900 - “Radio engineering, electronics and telecommunication”

SHYMKENT-2013

УДК 658.012.011

ББК 6П7.1

The composer senior lecturer, cand.tech.sci. Sarybaev A.A., Aslanov M.T Laboratory practical work contains methodical instructions to laboratory works on the discipline of «Systems and devices of sound and television broadcasting» for students of a specialty 5B071900 - “Radio engineering, electronics and telecommunication”, Shymkent: SKSU, 2013, 80p.

The laboratory practical work is made according to requirements ГОСО and the discipline program «Systems and devices of sound and television broadcasting » and includes all necessary data on performance of course laboratory researches by that.

Laboratory practical work of the «Systems and devices of sound and television broadcasting » contains instructions to performance of laboratory on discipline. Necessary recommendations are resulted in each laboratory work and similar examples are considered. At the end of each section the list of control questions are resulted.

Reviewer: Tasanbaev S.E - cand.tech.sci., the associate professor of chair «Automation, telecommunications and control» SKSU of M.Auezov

Laboratory practical work is discussed and recommended for publication at the meeting of the chair “Automation, Telecommunication and Control” (Minutes № 6 “05” January 2013)

Laboratory practical work is recommended for printing by faculty meeting the methodical commission of faculty «Information technology» (the report №4 from «18 » January 2013)

Laboratory practical work is recommended for publication by Methodical Council SKSU of M.Auezov, the report № __ from «___» ____ 2013

©  the South Kazakhstan State University (SKSU), 2013

Contents

1- Double Sideband DSBSC demodulation	Expt 1 - 4

2- Single Sideband (SSB) modulation & demodulation	Expt 2 - 14

3- Frequency Modulation (FM)	Expt 3 - 22

4- FM demodulation	Expt 4 – 29

5- Sampling and reconstruction	Expt 5 – 39

6- PCM encoding	Expt 6 – 51

7- PCM decoding	Expt 7 – 59

1- DSBSC demodulation

Experiment 1 - DSBSC demodulation

Preliminary discussion

Experiment 8 shows how the envelope detector can be used to recover the original message from an AM signal (that is, demodulate it). Unfortunately, the envelope detector cannot be used to demodulate a DSBSC signal.

To understand why, recall that the envelope detector outputs a signal that is a copy of its input's envelope. This works well for demodulating AM because the signal's envelopes are the same shape as the message that produced it in the first place (that is, as long as it's not over- modulated). However, recall that a DSBSC signal's envelopes are not the same shape as the message.

Instead, DSBSC signals are demodulated using a circuit called a product detector (though product demodulator is a more appropriate name) and its basic block diagram is shown in Figure 1 below. Other names for this type of demodulation include a synchronous detector and switching detector

As its name implies, the product detector uses multiplication and so mathematics are necessary to explain its operation. The incoming DSBSC signal is multiplied by a pure sinewave that must be the same frequency as the DSBSC signal's suppressed carrier. This sinewave is generated by the receiver and is known as the local carrier

To see why this process recovers the message, let's describe product detection mathematically:

DSBSC demodulator's output = the DSBSC signal * the local carrier

Importantly, recall that DSBSC generation involves the multiplication of the message with the carrier which produces sum and difference frequencies (the preliminary discussion in Experiment 6 summarises DSBSC generation). That being the case, this information can be substituted for the DSBSC signal and the equation rewritten as:

DSBSC demodulator's output = [(carrier + message) + (carrier - message)] * carrier

When the equation is solved, we get four sinewaves with the following frequencies:

- Carrier + (carrier + message)

- Carrier + (carrier - message)

- Carrier - (carrier + message) which simplifies to just the message

- Carrier - (carrier - message) which also simplifies to just the message

(If you're not sure why these sinewaves are produced, it's important to remember that whenever two pure sinewaves are multiplied together, two completely new sinewaves are generated. One has a frequency equal to the sum of the original sinewaves' frequencies and the other has a frequency equal to their difference.)

Importantly, notice that two of the products are sinewaves at the message frequency. In other words, the message has been recovered. As the two message signals are in phase, they simply add together to make one larger message.

Notice also that two of the products are non-message sinewaves. These sinewaves are unwanted and so a low-pass filter is used to reject them while keeping the message.

The experiment

For this experiment you'll use the Emona DATEx to generate a DSBSC signal by implementing its mathematical model. Then you'll set-up a product detector by implementing its mathematical model also.

Once done, you'll connect the DSBSC signal to the product detector's input and compare the demodulated output to the original message and the DSBSC signal's envelopes. You'll also observe the effect that a distorted DSBSC signal due to overloading has on the product detector's output.

Finally, if time permits, you'll investigate the effect on the product detector's performance of an unsynchronised local carrier.

It should take you about 1 hour to complete the whole experiment.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

- One set of headphones (stereo)

Procedure

Part A - Setting up the DSBSC modulator

To experiment with DSBSC demodulation you need a DSBSC signal. The first part of the experiment gets you to set one up.

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Oscilloscope VI.

  - Set up the scope per the procedure in Experiment 1 ensuring that the Trigger Source control is set to CH 0.

  - Connect the set-up shown in Figure 2 below.

This set-up can be represented by the block diagram in Figure 3 below. It generates a 100kHz carrier that is DSBSC modulated by a 2kHz sinewave message.

    - Adjust the scope's Timebase control to view two or so cycles of the Master Signals module's 2kHz SINE output.

    - Activate the scope's Channel 1 input to view the DSBSC signal out of the Multiplier module as well as the message signal.

Note: If the Multiplier module's output is not a DSBSC signal, check your wiring.

    - Set the scope's Channel 0 Scale control to the 1V/divposition and the Channel 1 Scale control to the 2V/divposition.

    - Draw the two waveforms to scale in the space provided on the next page leaving room to draw a third waveform.

Tip: Draw the message signal in the upper third of the graph and the DSBSC signal in the middle third.

Ask the instructor to check your work before continuing.

Part В - Recovering the message using a product detector

    - Launch the DATEx soft front-panel (SFP) and check that you have soft control over the DATEx board.

    - Locate the Tuneable Low-pass Filter module on the DATEx SFP and set its soft Gain control to about the middle of its travel.

    - Turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control fully clockwise.

    - Modify the set-up as shown in Figure 4 below.

The additions to the set-up can be represented by the block diagram in Figure 5 on the next page. The Multiplier and Tuneable Low-pass Filter modules are used to implement a product detector which demodulates the original message from the DSBSC signal.

The entire set-up is represented by the block diagram in Figure 6 below. It highlights the fact that the modulator's carrier is "stolen" to provide the product detector's local carrier. This means that the two carriers are synchronised which is a necessary condition for DSBSC communications. 7877

19. Draw the demodulated DSBSC signal to scale in the space that you left on the graph paper.

Question 1

Why must a product detector be used to recover the message instead of an envelope detector?

Tip: If you're not sure, refer to the preliminary discussion.

Ask the instructor to check your work before continuing.

Part C - Investigating the message's amplitude on the recovered message

- Locate the Amplifier module on the DATEx SFP and turn its soft Gain control to about a quarter of its travel.

- Disconnect the plugs to the Master Signals module's 2kHz SINE output.

- Use the Amplifier module to modify the set-up as shown in Figure 7 below.

The addition to the set-up can be represented by the block diagram in Figure 8 below. The amplifier's variable gain allows the message's amplitude to be adjustable.

- Vary the message signal's amplitude up and down a little (by turning the Amplifier module's soft Gain control left and right a little) while watching the demodulated signal.

Remember: You can use the keyboard's TAB and arrow keys for fine adjustments of DATEx controls.

Question 2

What is the relationship between the amplitude of the two message signals?

- Slowly increase the message signal's amplitude to maximum until the demodulated signal begins to distort.

Question 3

What do you think causes the distortion of the demodulated signal?

Tip: If you're not sure, connect the scope's Channel 0 input to the DSBSC modulator's output and set its Trigger Source control to the CH1 position.

Ask the instructor to check your work before continuing.

Part D - Transmitting and recovering speech using DSBSC

This experiment has set up a DSBSC communication system to "transmit" a 2kHz sine wave. The next part of the experiment lets you use it to modulate, transmit, demodulate and listen to speech.

- If you moved the scope's Channel 0 input and adjusted its Trigger Source control to help answer Question 3, return them to their previous positions.

- Disconnect the leads to the Amplifier module and modify the set-up as shown in Figure 9 below.

- Set the scope's Timebase control to the 2ms/divposition.

- Turn the Amplifier module's soft Gain control fully anti-clockwise.

- Without wearing the headphones, plug them into the Amplifier module's headphone socket.

- Put the headphones on.

- As you perform the next step, set the Amplifier module's soft Gain control to a comfortable sound level.

- Hum and talk into the microphone while watching the scope's display and listening on the headphones.

Ask the instructor to check your work before continuing.

Part E - Carrier synchronisation

Crucial to the correct operation of a DSBSC communications system is the synchronisation between the modulator's carrier signal and the product detector's local carrier. Any phase or frequency difference between the two signals adversely affects the system's performance.

The effect of phase errors

Recall that the product detector generates two copies of the message. Recall also that they're in phase with each other and so they simply add together to form one bigger message. However, if there's a phase error between the carriers, the product detector's two messages have a phase error also. One of them has the sum of the phase errors and the other the difference. In other words, the two messages are out of phase with each other.

If the carriers' phase error is small (say about 10°) the two messages still add together to form one bigger signal but not as big as when the carriers are in phase. As the carriers' phase error increases, the recovered message gets smaller. Once the phase error exceeds 45° the two messages begin to subtract from each other. When the carriers' phase error is 90° the two messages end up 180° out of phase and completely cancel each other out.

The next part of the experiment lets you observe the effects of carrier phase error.

- Turn the Amplifier module's soft Gain control fully anti-clockwise again.

- Return the scope's Timebase control to about the 100ps/divposition.

- Locate the Phase Shifter module on the DATEx SFP and set its soft Phase Change control to the 180°position.

- Set the Phase Shifter module's soft Phase Adjust control to about the middle of its travel.

- Disconnect the leads to the Speech output and modify the set-up as shown in Figure 10 below.

The set-up in Figure 10 on the previous page can be represented by the block diagram in Figure 11 below. The Phase Shifter module allows a phase error between the DSBSC modulator's carrier and the product detector's local carrier to be introduced.

- Slowly increase the Amplifier's module's gain until you can comfortably hear the demodulated 2kHz tone.

- Vary the Phase Shifter module's soft Phase Adjust control left and right while watching and listening to the effect on the recovered message.

- Use the keyboard's TAB and left arrow keys to turn the Phase Shifter module's soft Phase Adjust control anti-clockwise until the recovered message is smallest.

Question 4

Given the size of the recovered message's amplitude, what is the likely phase error between the two carriers?

Tip: If you're not sure about the answer to this question (and the next one), reread the notes on page 9-13.

41. Verify your answer to Question 4 by connecting the scope's Channel 0 input to the

Master Signals module's 100kHz SINE output, its Channel 1 input to the Phase Shifter module's output and setting its Timebase control to the 5ps/divsetting.

  - Use the keyboard's TAB and left arrow keys to adjust the Phase Shifter module's soft Phase Adjust control until the two signals are in phase.

Question 5

Given the two carriers are in phase, what should the amplitude of the recovered message be?

  - Verify your answer to Question 5 by reconnecting the scope's Channel 0 input to the Master Signals module's 2kHz SINE output, reconnecting its Channel 1 input to the Tuneable Low-pass Filter module's output and setting its Timebase control back to the 100jus/div setting.

Ask the instructor to check your work before continuing.

The effect of frequency errors

When there's a frequency error between the DSBSC signal's carrier and the product detector's local carrier, there is a corresponding frequency error in the two products that usually coincide. One is at the message frequency minus the error and the other is at the error frequency plus the error.

If the error is small (say 0.1Hz) the two signals will alternately reinforce and cancel each other which can render the message periodically inaudible but otherwise intelligible. If the frequency error is larger (say 5Hz) the message is reasonably intelligible but fidelity is poor. When frequency errors are large, intelligibility is seriously affected.

The next part of the experiment lets you observe the effects of carrier frequency error.

  - Launch and run the NI ELVIS II Function Generator VI.

  - Adjust the function generator's soft controls for an output with the following specifications:

- Waveshape: Sine

- Frequency: 100kHz exactly

- Amplitude: 4Vpp

- DC Offset: 0V

  - Disconnect the leads to the Phase Shifter module and modify the set-up as shown in Figure 12 below.

The entire set-up can be represented by the block diagram in Figure 13 below. The function generator allows the local oscillator to be completely frequency (and phase) independent of the DSBSC modulator.

  - If you're not doing so already, listen to the recovered message using the headphones.

  - Compare the scope's frequency measurements for the original message and the recovered message.

Note 1: You should find that they're very close in frequency.

Note 2: You'll notice that the volume of the recovered messages varies. This is due to the phase error between the two carriers and should be ignored for the following steps.

  - Reduce the function generator's output frequency to 99.8kHz.

  - Give the function generator a moment to achieve the correct frequency and note the change in the tone of recovered message.

Tip: If you can't remember what 2kHz sounds like, set function generator's output to 100kHz for a moment then return it to 99.8kHz.

  - Experiment with other local carrier frequencies around 100kHz and listen to the effect on the recovered message.

  - Return the function generator's output to 100kHz.

  - Disconnect the plugs to the Master Signals module's 2kHz SINE output and connect them to the Speech module's output.

  - Hum and talk into the microphone to check that the whole set-up is still working correctly.

  - Vary the function generator's frequency again and listen to the effect of an unsynchronised local carrier on speech.

Ask the instructor to check your work before finishing.

2 - SSBSC modulation and demodulation

Experiment 2 - SSBSC modulation and demodulation

Preliminary discussion

Comparing the two communications systems considered earlier in this manual, DSBSC offers considerable power savings over AM (at least 66%) because a carrier is not transmitted. However, both systems generate and transmit sum and difference frequencies (the upper and lower sidebands) and so they have the same bandwidth for the same message signal.

As its name implies, the Single Sideband Suppressed Carrier (SSBSC or just SSB) system transmits only one sideband. In other words, SSB transmits either the sum or the difference frequencies but not both. Importantly, it doesn't matter which sideband is used because they both contain all of the information in the original message.

In transmitting only one sideband, SSB requires only half the bandwidth of DSBSC and AM which is a significant advantage.

Figure 1 below shows a simple message signal and an unmodulated carrier. It also shows the result of modulating the carrier with the message using SSBSC. If you look closely, you'll notice that the modulated carrier is not the same frequency as either the message or the carrier.

A common method of generating SSB simply involves generating a DSBSC signal then using a filter to pick out and transmit only one of the sidebands. This is known as the filter method. However, the two sidebands in a DSBSC signal are close together in frequency and so specialised filters must be used. This means that the filters for non-mainstream communications systems can be expensive.

Another way of generating SSB that is becoming increasingly popular is called the phasing method. This uses a technique called phase discrimination to cancel out one of the sidebands at the generation stage (instead of filtering it out afterwards).

In telecommunications theory, the mathematical model that defines this process is:

SSB = (message * carrier) + (message with 90° of phase shift * carrier with 90° of phase shift)

If you look closely at the equation you'll notice that it's the sum of two multiplications. When the message is a simple sinewave the solution of the two multiplications tells us that four sinewaves are generated. Depending on whether the message's phase shift is +90° or -90° their frequencies and phase differences are:

Regardless of whether the message's phase shift is +90° or -90°, when the four sinewaves are added together, two of them are in phase and add together to produce one sinewave (either carrier + message or carrier - message) and two of the sinewaves are phase inverted and completely cancel. In other words, the process produces only a sum or difference signal (that is, just one sideband).

The block diagram that implements the phasing type of SSB modulator is shown in Figure 2 below.

As SSB signals don't contain a carrier, they must be demodulated using product detection in the same way as DSBSC signals (the product detector's operation is summarised in the preliminary discussion of Experiment 9).

The experiment

For this experiment you'll use the Emona DATEx to generate an SSB signal by implementing the mathematical model for the phasing method. You'll then use a product detector (with a stolen carrier) to reproduce the message.

Importantly, you'll only do so for a sinewave message (that is, you'll not SSB modulate then demodulate speech). There's a practical reason for this. The phase shift introduced by the DATEx Phase Shifter module is frequency dependent (that is, for any given setting, the phase shift is different at different frequencies). A wideband phase shifting circuit is necessary to provide 90° of phase shift for all of the sinewaves in a complex message like speech.

It should take you about 40 minutes to complete this experiment.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Procedure

Part A - Generating an SSB signal using a simple message

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Function Generator VI.

  - Adjust the function generator using its soft controls for an output with the following specifications:

- Waveshape: Sine

- Frequency: 10kHz exactly

- Amplitude: 4Vpp

- DC Offset: 0V

- Minimise the function generator's VI.

- Connect the set-up shown in Figure 3 below.

This set-up can be represented by the block diagram in Figure 4 below. It is used to set up two message signals that are out of phase with each other.

  - Launch the DATEx soft front-panel (SFP) and check that you have soft control over the DATEx board.

  - Locate the Phase Shifter module on the DATEx SFP and set its soft Phase Change control to the 180°position.

  - Set the Phase Shifter module's soft Phase Adjust control to about the middle of its travel.

  - Launch and run the NI ELVIS II Oscilloscope VI.

  - Set up the scope per the procedure in Experiment 1 ensuring that the Trigger Source control is set to CH 0.

  - Adjust the scope's Timebase control to view two or so cycles of the function generator's output.

  - Activate the scope's Channel 1.

  - Check that the two message signals are out of phase with each other. Note: At this stage, it doesn't matter what the phase difference is.

  - Modify the set-up as shown in Figure 5 below.

This set-up can be represented by the block diagram in Figure 6 on the next page. It is used to multiply the two message signals with two 100kHz sinewaves (the carriers) that are exactly 90° out of phase with each other.

  - Use the scope to check that the lower Multiplier module's output is a DSBSC signal.

Tip: Temporarily set the scope's Channel 1 Scale control to the 2V/divposition to do this.

  - Disconnect the scope's Channel 1 input from the lower Multiplier module's output and connect it to the upper Multiplier module's output.

  - Check that the upper Multiplier module's output is a DSBSC signal as well.

  - Locate the Adder module on the DATEx SFP and set its soft & and g controls to about the middle of their travel.

  - Modify the set-up as shown in Figure 7 below.

This set-up can be represented by the block diagram in Figure 8 below. The Adder module is used to add the two DSBSC signals together. The phase relationships between the sinewaves in the DSBSC signals means that two of them (one in each sideband) reinforce each other and the other two cancel each other out.

Question 1

The signal out of the Adder module is highly unlikely to be an SSB signal at this stage. What are two reasons for this? Tip: If you're not sure, one of them can be worked out by reading the preliminary discussion.

Ask the instructor to check your work before continuing.

The next part of the experiment gets you to make the fine adjustments necessary to turn the set-up into a true SSB modulator.

    - Deactivate the scope's Channel 0 input.

    - Disconnect the patch lead to the Adder module's B input.

Note: This removes the signal on the Adder module's B input from the set-up's output.

    - Adjust the Adder module's soft G control to obtain a 4Vp-p output.

Tip: Remember that you can use the keyboard's TAB and arrow keys for fine adjustment of the DATEx SFP's controls.

    - Reconnect the Adder module's B input and disconnect the patch lead to its A input. Note: This removes the signal on the Adder module's A input from the set-up's output.

    - Adjust the Adder module's soft g control to obtain a 4Vp-p output.

    - Reconnect the patch lead to the Adder module's A input.

The gains of the Adder module's two inputs are now nearly the same. Next, the correct phase difference between the messages must be achieved.

    - Slowly vary the Phase Shifter module's soft Phase Adjust control left and right and observe the effect on the envelopes of the set-up's output.

Note: For most of the soft Phase Adjust control's travel, you'll get an output that looks like a DSBSC signal. However, if you adjust the control carefully, you'll find that you're able to flatten-out the output signal's envelope.

    - Set the scope's Channel 1 Scale control to the 500m V/div position.

    - Adjust the Phase Shifter module's soft Phase Adjust control to make the envelopes as "flat" as possible.

The phase difference between the two messages is now nearly 90°.

    - Tweak the Adder module's soft & control to see if you can make the output's envelopes flatter.

    - Tweak the Phase Shifter module's soft Phase Adjust control to see if you can make the output's envelopes flatter still.

Once the envelopes are as flat as you can get, the gains of the Adder module's two inputs are very close to each other and the phase difference between the two messages are very close to 90°. That being the case, the signal out of the Adder module is now SSBSC.

Question 2

How many sinewaves does this SSB signal consist of?

Tip: If you're not sure, see the preliminary discussion.

Question 3

For the given inputs to the SSB modulator, what two frequencies can this signal be?

Part B - Spectrum analysis of an SSB signal

The next part of this experiment let's you analyse the frequency domain representation of the SSB signal to see if its spectral composition matches your answers to Questions 2 and 3.

    - Suspend the scope's operation by clicking on its Stop control once.

Note: The scope's display should freeze and its hardware has been deactivated. This is a necessary step as the scope and signal analyzer share hardware resources and so they cannot be operated simultaneously.

    - Launch and run the NI ELVIS II Dynamic Signal Analyzer VI.

    - Set up the signal analyzer per the procedure in Experiment 7 (page 7-10).

    - Activate the signal analyzer's cursors by checking the Cursors On box.

    - Align C1 with the most significant sinewave in the signal's spectrum and determine its frequency.

Question 4

Based on your measurement for the step above, which sideband does your SSB modulator generate?

    - Align C1 with some of the other significant sinewaves close to this sideband and note their frequencies.

Note: You should find that there's a sinewave at the carrier frequency and another at the frequency for the other sideband. Importantly, despite appearances, these signals are very small relative to the significant sideband (the scale used for the Y-axis is decibels which is not a linear unit of measurement).

Question 5

Give two reasons for the presence of a small amount of the other sideband?

    - Tweak the Phase Shifter module's soft Phase Adjust control and note the effect on the size of the carrier and other sideband.

Note: Give the signal analyzer's display time to update after each adjustment. Question 6

Why doesn't varying the Phase Shift module's Phase Adjust control affect the size of the carrier in the SSBSC signal?

    - Adjust the two controls to obtain the smallest size for the insignificant sideband.

    - Close the Signal Analyzer's VI.

    - Restart the scope's VI by clicking on its Run control once.

    - Note whether there is any improvement in the SSB signal's envelope (that is, note whether the envelope is any flatter).

Ask the instructor to check your work before continuing.

Part С - Using the product detector to recover the message

    - Reactivate the scope's Channel 0 input and return the Channel 1 Scale control to the 1V/div position.

    - Locate the Tuneable Low-pass Filter module on the DATEx SFP and set its soft Gain control to about the middle of its travel.

    - Turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control fully clockwise.

    - Modify the set-up as shown in Figure 9 below.

The additions to the set-up shown in Figure 9 on the previous page can be represented by the block diagram in Figure 10 below. The Multiplier and Tuneable Low-pass Filter modules are used to implement a product detector which demodulates the original message from the SSB signal.

52. Use the scope to compare the original message with the recovered message. Question 7

What is the relationship between the original message and the recovered message?

Ask the instructor to check your work before finishing.

3 - Frequency modulation

Experiment 3 - Frequency modulation

Preliminary discussion

A disadvantage of the AM, DSBSC and SSB communication systems is that they are susceptible to picking up electrical noise in the transmission medium (the channel). This is because noise changes the amplitude of the transmitted signal and the demodulators of these systems are designed to respond to amplitude variations.

As its name implies, frequency modulation (FM) uses a message's amplitude to vary the frequency of a carrier instead of its amplitude. This means that the FM demodulator is designed to look for changes in frequency instead. As such, it is less affected by amplitude variations and so FM is less susceptible to noise. This makes FM a better communications system in this regard.

There are several methods of generating FM signals but they all basically involve an oscillator with an electrically adjustable frequency. The oscillator uses an input voltage to affect the frequency of its output. Typically, when the input is OV, the oscillator outputs a signal at its rest frequency (also commonly called the free-running or centre frequency). If the applied voltage varies above or below OV, the oscillator's output frequency deviates above and below the rest frequency. Moreover, the amount of deviation is affected by the amplitude of the input voltage. That is, the bigger the input voltage, the greater the deviation.

Figure 1 below shows a bipolar squarewave message signal and an unmodulated carrier. It also shows the result of frequency modulating the carrier with the message.

There are a few things to notice about the FM signal. First, its envelopes are flat - recall that FM doesn't vary the carrier's amplitude. Second, its period (and hence its frequency) changes when the amplitude of the message changes. Third, as the message alternates above and below OV, the signal's frequency goes above and below the carrier's frequency. (Note: It's equally possible to design an FM modulator to cause the frequency to change in the opposite direction to the change in the message's polarity.)

Before discussing FM any further, an important point must be made here. A squarewave message has been used in this discussion to help you visualise how an FM carrier responds to its message. In so doing, Figure 1 suggests that the resulting FM signal consists of only two sinewaves (one at a frequency above the carrier and one below). However, this isn't the case. For reasons best left to your instructor to explain, the spectral composition of the FM signal in Figure 1 is much more complex than implied.

This highlights one of the important differences between FM and the modulation schemes discussed earlier. The mathematical model of an FM signal predicts that even for a simple sinusoidal message, the result is a signal that potentially contains many sinewaves. In contrast, for the same sinusoidal message, an AM signal would consist of three sinewaves, a DSBSC signal would consist of two and an SSBSC signal would consist of only one. This doesn't automatically mean that the bandwidth of FM signals is wider than AM, DSBSC and SSBSC signals (for the same message signal). However, in the practical implementation of FM communications, it usually is.

There's another important difference between FM and the modulation schemes discussed earlier. The power in AM, DSBSC and SSBSC signals varies depending on their modulation index. This occurs because the carrier's RMS voltage is fixed but the RMS sideband voltages are proportional to the signals' modulation index. This is not true of FM. The RMS voltage of the carrier and sidebands varies up and down as the modulation index changes such that the square of their voltages always equal the square of the unmodulated carrier's RMS voltage. That being the case, the power in FM signals is constant.

Finally, when reading about the operation of an FM modulator you may have recognised that there is a module on the Emona DATEx that operates in the same way - the VCO output of the function generator. In fact a voltage-controlled oscillator is sometimes used for FM modulation (though there are other methods with advantages over the VCO).

The experiment

For this experiment you'll generate a real FM signal using the VCO module on the Emona DATEx. First you'll set up the VCO module to output an unmodulated carrier at a known frequency. Then you'll observe the effect of frequency modulating its output with a squarewave then speech. You'll then use the NI ELVIS II Dynamic Signal Analyzer to observe the spectral composition of an FM signal in the frequency domain and examine the distribution of power between its carrier and sidebands for different levels of modulation.

It should take you about 40 minutes to complete this experiment.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Procedure

Part A - Frequency modulating a squarewave

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Function Generator VI.

  - Adjust the function generator using its soft controls for an output with the following specifications:

- Waveshape: Sine

- Frequency: 20kHz

- Amplitude: 4Vpp

- DC Offset: 0V

- Modulation type: FM

  - Connect the set-up shown in Figure 2 below.

This set-up can be represented by the block diagram in Figure 3 below. The Master Signals module is used to provide a 2kHz squarewave message signal and the VCO is the FM modulator with a 20kHz carrier.

- Launch and run the NI ELVIS II Oscilloscope VI.

- Set up the scope per the procedure in Experiment 1 with the following changes:

- Timebase control to the 100ps/divposition instead of 500ps/div

- Trigger Level to the 2.5Vposition instead of 0V

- Activate the scope's Channel 1 input to view the FM signal on the VCO's output as well as the message signal.

Question 1

Why does the frequency of the carrier change?

Ask the instructor to check your work before continuing.

Part В - Generating an FM signal using speech

So far, this experiment has generated an FM signal using a squarewave for the message. However, the message in commercial communications systems is much more likely to be speech and music. The next part of the experiment lets you see what an FM signal looks like when modulated by speech.

- Return the scope's Trigger Level control to OV.

- Disconnect the plugs to the Master Signals module's 2kHz SINE output.

- Connect them to the Speech module's output as shown in Figure 4 below.

- Set the scope's Timebase control to the 2OOps/divposition.

- Hum and talk into the microphone while watching the scope's display.

Ask the instructor to check your work before continuing.

Part С - Power in an FM signal

As mentioned earlier, the power in an FM signal is constant regardless of its level of modulation. This part of the experiment lets you see this for yourself.

- Launch the DATEx soft front-panel (SFP) and check that you have soft control over the DATEx board.

- Locate the Amplifier module on the DATEx SFP and turn soft Gain control fully anti- clockwise.

- Completely dismantle the previous set-up.

- Connect the set-up shown in Figure 5 below.

This set-up can be represented by the block diagram in Figure 6 on the next page. With the VCO's input connected to ground, its output is a single sinewave at 20kHz.

- Use the scope to check that the VCO's output is a 20kHz sinewave.

- Close the scope's VI.

- Launch and run the NI ELVIS II Dynamic Signal Analyzer VI.

- Adjust the signal analyzer's controls as follows: Input Settings

- Source Channel to SCOPE CH 0 ■ Voltage Range to+10V

FFT Settings	Averaging

- Frequency Span to 60,000	■ Mode to RMS

- Resolution to 400	■ Weighting to Exponential

- Window to 7 Term B-Harris	■ # of Averages to 3

Trigger Settings

- Type to Edge

Frequency Display	Cursor settings

- Units to Linear	■ Cursors On box unchecked (for now)

- Mode to RMS

- Scale to Auto

  - Once done, one significant sinewave should be displayed.

Note: It's important at this point to double-check that the signal analyzer's Frequency Display Units option is set to Linear.

  - Activate the signal analyzer's cursors by checking the Cursors On box.

  - Use the scope's C1 cursor to measure the frequency of the sinewave and verify that it's the VCO's rest frequency (that is, 20kHz).

Note: Recall that cursor measurements of frequency and voltage are difference values between C1 and C2. However, when cursor C2 is moved to the extreme left side of the display, its frequency and voltage is zero so the cursor measurements become absolute values for C1.

  - To the left of the cursor's frequency measurement readout is the measurement of the signal's RMS voltage. Record this in Table 1 below.

  - Square and record this voltage.

Why square the signal's RMS voltage? To answer this question, remember that we're investigating the power in an FM signal but signal analyzers (and most other test equipment) V2 measure power. However, one of the power equations (P = -RML) tells us that power and the square of a signal's RMS voltage (that is, VRMS) are proportional values. That being the case, we can investigate power in an FM signal indirectly by investigating the square of the signal's RMS voltage because whatever is true of one must also be true of the other (regardless of R).

  - Disconnect the plug to the GND socket and modify the set-up as shown in Figure 7 below.

This set-up can be represented by the block diagram in Figure 8 below. Importantly, as the Amplifier module's minimum gain isn't zero, the carrier will now be frequency modulated by a low level message signal. This means that the signal analyzer's display will show about four sidebands. As these sidebands are small relative to the carrier, they can be better observed by temporarily setting the analyzer's Units option to dB instead of Linear.

  - If you haven't already done so, return the analyzer's Units option to Linear.

  - Use the Amplifier module's soft Gain control to adjust the modulation of the FM signal slightly until only five sinewaves are clearly visible in the signal's spectrum.

  - Use the cursor to measure the RMS voltage of these sinewaves and record them in Table 2 on the next page.

  - Square and record the voltages.

  - Add and record the squared voltages.

  - Use the Amplifier module's soft Gain control to increase the modulation of the FM signal until the carrier drops to zero for the first time.

  - Repeat Steps 35 and 37 for the six most significant sinewaves in the signal recording your measurements in Table 3 below.

Question 2

How do the totals in Tables 2 and 3 compare with each other and the value in Table 1?

Question 3

What do these measurements help to prove? Explain your answer.

Ask the instructor to check your work before continuing.

Part D - Bandwidth of an FM signal

The spectral composition of an FM signal can be complex and consist of many sidebands. Usually, many of them are relatively small in size and so an engineering decision must be made about how many of them to include as part of the signal's bandwidth. There are several standards in this regard and a common one involves including all sidebands that are equal to or greater than 1% of the unmodulated carrier's power (or VRMS). This part of the experiment lets you use this criterion to measure FM signal bandwidth.

  - Use the signal analyzer's C1 cursor to identify the lowest frequency sinewave in the FM signal with a VRMS equal to or greater than 1% of the value in Table 1.

Note: You have to do this by measuring the RMS voltage of the smallest sinewaves and square the value until you find the first one with a VRMS equal to or greater than 1% of the value in Table 1.

  - Use the signal analyzer's C2 cursor to identify the highest frequency sinewave in the FM signal with a voltage equal to or greater than 1% of the value in Table 1.

  - The signal analyzer's df (Hz) reading is a measurement of the difference in frequency between its cursors. Following Steps 40 and 41, this reading is the FM signal's bandwidth. Record this value in Table 4 below.

Question 4

Calculate the bandwidth of a 20kHz carrier amplitude modulated by a 2kHz sinewave.

Question 5

How does the FM signal's bandwidth compare to an AM signal's bandwidth for the same inputs?

Ask the instructor to check your work before continuing.

  - Increase the Amplifier module's gain until the cursor on its soft Gain control points to the 9 o'clock position.

  - Repeat steps 40 to 42 recording your measurement in Table 5 below.

  - Question 6

What is the relationship between the message signal's amplitude and the FM signal's bandwidth?

Ask the instructor to check your work before finishing.

4 - FM demodulation

Experiment 4 - FM demodulation

Preliminary discussion

There are as many methods of demodulating an FM signal as there are of generating one. Examples include: the slope detector, the Foster-Seeley discriminator, the ratio detector, the phase-locked loop (PLL), the quadrature FM demodulator and the zero-crossing detector It's possible to implement several of these methods using the Emona DATEx but, for an introduction to the principles of FM demodulation, the zero-crossing detector is used here.

The zero-crossing detector

The zero-crossing detector is a simple yet effective means of recovering the message from FM signals. Its block diagram is shown in Figure 1 below.

The received FM signal is first passed through a comparator to clip it heavily, effectively converting it to a squarewave. This allows the signal to be used as a trigger signal for the zero- crossing detector circuit (ZCD).

The ZCD generates a pulse of fixed duration every time the squared-up FM signal crosses zero volts (either on the positive or the negative transition but not both). Given the squared-up FM signal is continuously crossing zero, the ZCD effectively converts the squarewave to a rectangular wave with a fixed mark time.

When the FM signal's frequency changes (in response to the message), so does the rectangular wave's frequency. Importantly though, as the rectangular wave's mark is fixed, changing its frequency is achieved by changing the duration of the space and hence the signal's mark/space ratio (or duty cycle). This is shown in Figure 2 on the next page using an FM signal that only switches between two frequencies (because it has been generated by a squarewave for the message).

Recall from the theory of complex waveforms, pulse trains are actually made up of sinewaves and, in the case of Figure 2 above, a DC voltage. The size of the DC voltage is affected by the pulse train's duty cycle. The greater its duty cycle, the greater the DC voltage.

That being the case, when the FM signal in Figure 2 above switches between the two frequencies, the DC voltage that makes up the rectangular wave out of the ZCD changes between two values. In others words, the DC component of the rectangular wave is a copy of the squarewave that produced the FM signal in the first place. Recovering this copy is a relatively simple matter of picking out the changing DC voltage using a low-pass filter.

Importantly, this demodulation technique works equally well when the message is a sinewave or speech.

The experiment

For this experiment you'll use the Emona DATEx to generate an FM signal using a VCO. Then you'll set-up a zero-crossing detector and verify its operation for variations in the message's amplitude.

It should take you about 50 minutes to complete this experiment.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

- One set of headphones (stereo)

Procedure

Part A - Setting up the FM modulator

To experiment with FM demodulation you need an FM signal. The first part of the experiment gets you to set one up. To make viewing the signals around the demodulator possible, we'll start with a DC voltage for the message.

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Function Generator VI.

  - Adjust the function generator for an output with the following specifications:

- Waveshape: Sine

- Frequency: 15kHz

- Amplitude: 4Vpp

- DC Offset: OV

- Modulation type: FM

- Check that the function generator's Signal Route option (near the bottom right-hand corner of the VI is set to Prototyping Board.

Important note: There is the potential for a hardware conflict when using the scope's external triggering input (TRIG) at the same time as the function generator. This is because the connector for the TRIG input doubles as an output for the function generator when its Signal Route option is set to FGEN BNC. To avoid hardware conflicts always set the Signal Route to Prototyping Board.

- Launch and run the NI ELVIS II Variable Power Supplies VI.

- Turn the Variable Power Supplies negative output soft Voltage control fully anti- clockwise to output 0V.

- Connect the set-up shown in Figure 3 below.

This set-up can be represented by the block diagram in Figure 4 on the next page. The negative output of the Variable DC Power Supplies is being used to provide a simple DC message and the VCO implements the FM modulator with a carrier frequency of 15kHz.

- Launch and run the NI ELVIS II Oscilloscope VI.

- Set up the scope per the procedure in Experiment 1 with the following changes:

- Scale control for Channel 0 to 2V/div instead of 1V/div

- Trigger Type control to Digital instead of Edge

- Coupling controls for both channels to DCinstead of AC

- Activate the scope's Channel 1 input to view the FM signal on the VCO's output as well as the DC message signal.

- Set the scope's Timebase control to view two or so cycles of the VCO output.

- Vary the Variable Power Supplies negative output soft Voltage control and check that the VCO's output frequency changes accordingly.

Ask the instructor to check your work before continuing.

Part В - Setting up the zero-crossing detector

- Launch the DATEx soft front-panel (SFP) and check that you have soft control over the DATEx board.

- Locate the Twin Pulse Generator module on the DATEx SFP and turn its soft Width control fully anti-clockwise.

- Set the Twin Pulse Generator module's soft Delay control fully anti-clockwise.

- Locate the Tuneable Low-pass Filter module on the DATEx SFP and turn its soft Gain control fully clockwise.

- Turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control fully clockwise.

- Modify the set-up as shown in Figure 5 below.

The additions to the set-up can be represented by the block diagram in Figure 6 on the next page. The comparator on the Utilities module is used to clip the FM signal, effectively turning it into a squarewave. The positive edge-triggered Twin Pulse Generator module is used to implement the zero-crossing detector. To complete the FM demodulator, the RC Low-pass Filter module and Tuneable Low-pass Filter module combination is used to pick-out the changing DC component of the Twin Pulse Generator module's output.

Note: The RC LPF and Tuneable LPF modules are used in combination because the Tuneable LPF is a clocked "switched capacitor" filter which uses internal sampling technology. These types of filters are subject to aliasing (a concept that is covered in later experiments) which can cause problems in sampled systems. To avoid such problems, we use the RC LPF as an anti-aliasing pre-filter for the Tuneable LPF. If time permits, explore this part of the experiment without the RC LPF.

The entire set-up can be represented by the block diagram in Figure 7 below.

You will now tune the Tuneable Low-pass Filter to isolate the DC component of the ZCD for the voltage range of the input signal we plan to use.

- Set the Variable Power Supplies negative output to -2V.

- Set the scope's Channel 1 Scale control to the 100mv/divposition.

Note: You should now see a 430mVp-p sinewave with a DC offset of approximately 230mV. This is the filtered pulse train out of the ZCD.

- Slowly turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control anti-clockwise until the sine wave becomes a DC voltage. Stop the moment this happens.

Note 1: You have now eliminated all non-DC components from the signal. That said, the filter will still allow the message signal to pass.

Note 2: Don't change the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control setting for the rest of the experiment unless instructed to.

- Vary the Variable Power Supplies negative output between 0V and a maximum of -2V.

Note 1: As you do, you should notice that the DC voltage out of the Tuneable Low-pass Filter module varies as well.

Note 2: If this doesn't happen, check that the scope's Channel 1 Coupling control is set to the DC position.

Ask the instructor to check your work before continuing.

Part С - Investigating the operation of the zero-crossing detector

The next part of the experiment lets you verify the operation of the zero-crossing detector.

29. Rearrange the scope's connections to the set-up as shown in Figure 8 below.

The new scope connections can be shown using the block diagram in Figure 9 below.

  - Vary the Variable Power Supplies negative output in small steps using the up and down arrow buttons on the VI.

Note: This will cause small but noticeable changes in the FM signal's frequency.

  - As you vary the FM signal's frequency, pay close attention to the mark-space ratio (that is, the duty cycle) of the Comparator's output.

Tip: You may find it helpful to adjust the scope's Vertical Position controls to separate the signals on the display.

Question 1

Does the mark-space ratio of the signal on the Comparator's output change? Question 2

What does this tell us about the DC component of the comparator's output?

Ask the instructor to check your work before continuing.

  - Rearrange the scope's connections to the set-up as shown in Figure 10 below. The new scope connections can be shown using the block diagram in Figure 11 below.

    - Vary the Variable Power Supplies negative output in small steps again to model an FM signal's changing frequency.

    - As you perform the step above, note how the frequency of the two signals changes.

    - Turn on the scope's cursors.

    - Use the scope's cursors to measure the width of the ZCD output's mark and space for different DC input voltages.

Note: The time difference between the two cursors is displayed directly above the Channel O & 1 measurements and is denoted as dT.

Tip: You may find it helpful to turn the scope's Channel O off as you do this and set its Timebase control to 1Ops/div when measuring the mark's width.

Question 3

As the FM signal changes frequency so does the ZCD's output. What aspect of the ZCD's output signal changes to achieve this?

- Neither the signal's mark nor space

- Only the signal's mark

- Only the signal's space

- Both the signal's mark and space

Question 4

What does this tell us about the DC component of the comparator's output?

Ask the instructor to check your work before continuing.

The next part of the experiment lets you verify your answer to the previous question.

  - If you deactivated the scope's Channel 0 then reactivate it and return its Timebase control to 50ps/div.

  - Rearrange the scope's connections to the set-up as shown in Figure 12 below.

  - The new scope connections can be shown using the block diagram in Figure 13 below.

  - If you've adjusted the scope's Channel 1 Vertical Position control, re-zero it.

  - Vary the Variable Power Supplies negative output in small steps again to model an FM signal's changing frequency.

  - As you perform the step above, compare the outputs from the Twin Pulse Generator module (the ZCD) and the Tuneable Low-pass Filter module.

Question 5

Why does the Tuneable Low-pass Filter module's DC output go up as the mark-space ratio of the ZCD's output goes up?

Question 6

If the original message is a sinewave instead of a variable DC voltage, what would you expect to see out of the Tuneable Low-pass Filter module?

Ask the instructor to check your work before continuing.

Part D - Transmitting and recovering a sinewave using FM

This experiment has set up an FM communication system to "transmit" a message that is a DC voltage. The next part of the experiment lets you use the set-up to modulate, transmit and demodulate a test signal (a sinewave).

  - If it's not already, turn the Tuneable Low-pass Filter module's soft Gain control fully clockwise.

  - Modify the set-up as shown in Figure 14 below.

This modification to the FM modulator can be shown using the block diagram in Figure 15 on the next page. Notice that the message is now provided by the Master Signals module's 2kHz SINE output.

  - Make the following adjustments to the scope's controls:

- Scale control for Channel 0 to 2V/divand to 100m V/divfor Channel 1

- Input Coupling control for both channels to AC

- Trigger Type to Edge

- Trigger Source to CH 0

- Timebase control to 200ps/div

  - Use the TAB and arrow keys to turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control slightly anti-clockwise to fine tune the filter's cut-off frequency.

Note: You will already be viewing the demodulated 2kHz message sinewave with an amplitude of approximately 250mVp-p.

Question 7

What does the FM modulator's output signal tell you about the ZCD signal's duty cycle?

Part E - Transmitting and recovering speech using FM

The next part of the experiment lets you use the set-up to modulate, transmit and demodulate speech.

  - Disconnect the plugs to the Master Signals module's 2kHz SINE output.

  - Modify the set-up as shown in Figure 16 below.

  - Set the scope's Timebase control to the 2ms/divposition.

  - Locate the Amplifier module on the DATEx SFP and turn its soft Gain control fully anti- clockwise.

  - Without wearing the headphones, plug them into the Amplifier module's headphone socket.

  - Put the headphones on.

  - As you perform the next step, set the Amplifier module's soft Gain control to a comfortable sound level.

  - Hum and talk into the microphone while watching the scope's display and listening on the headphones.

60. Once you have completed viewing the signal with the scope, stop the scope and start the NI ELVIS II Dynamic Signal Analyzer to view the spectrum of the frequency modulated speech. Set the DSA frequency setting to 40kHz. Try whistling into the microphone. This will help you to see the difference between single tones and speech during modulation.

Ask the instructor to check your work before finishing.

5 - Sampling and reconstruction

Experiment 5 - Sampling and reconstruction

Preliminary discussion

So far, the experiments in this manual have concentrated on communications systems that transmit analog signals. However, digital transmission is fast replacing analog in commercial communications applications. There are several reasons for this including the ability of digital signals and systems to resist interference caused by electrical noise.

Many digital transmission systems have been devised and several are considered in later experiments. Whichever one is used, where the information to be transmitted (called the message) is an analog signal (like speech and music), it must be converted to digital first. This involves sampling which requires that the analog signal's voltage be measured at regular intervals.

Figure la below shows a pure sinewave for the message. Beneath the message is the digital sampling signal used to tell the sampling circuit when to measure the message. Beneath that is the result of "naturally" sampling the message at the rate set by the sampling signal. This type of sampling is "natural" because, during the time that the analog signal is measured, any change in its voltage is measured too. For some digital systems, a changing sample is unacceptable. Figure lb shows an alternative system where the sample's size is fixed at the instant that the signal measured. This is known as a sample-and-hold scheme (and is also referred to as pulse amplitude modulation).

Regardless of the sampling method used, by definition it captures only pieces of the message. So, how can the sampled signal be used to recover the whole message? This question can be answered by considering the mathematical model that defines the sampled signal:

Sampled message = the sampling signal * the message

As you can see, sampling is actually the multiplication of the message with the sampling signal. And, as the sampling signal is a digital signal which is actually made up of a DC voltage and many sinewaves (the fundamental and its harmonics) the equation can be rewritten as:

Sampled message = (DC + fundamental + harmonics) * message

When the message is a simple sinewave (like in Figure 1) the equation's solution (which necessarily involves some trigonometry that is not shown here) tells us that the sampled signal consists of:

- A sinewave at the same frequency as the message

- A pair of sinewaves that are the sum and difference of the fundamental and message frequencies

- Many other pairs of sinewaves that are the sum and difference of the sampling signals' harmonics and the message

This ends up being a lot of sinewaves but one of them has the same frequency as the message. So, to recover the message, all that need be done is to pass the sampled signal through a low- pass filter. As its name implies, this type of filter lets lower frequency signals through but rejects higher frequency signals.

That said, for this to work correctly, there's a small catch which is discussed in Part E of the experiment.

The experiment

For this experiment you'll use the Emona DATEx to sample a message using natural sampling then a sample-and-hold scheme. You'll then examine the sampled message in the frequency domain using the N1 ELVIS II Dynamic Signal Analyzer. Finally, you'll reconstruct the message from the sampled signal and examine the effect of a problem called aliasing.

It should take you about 50 minutes to complete this experiment.

Equipment

- Personal computer with appropriate software installed

- N1 ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Part A - Sampling a simple message

The Emona DATEx has a Dual Analog Switch module that has been designed for sampling. This part of the experiment lets you use the module to sample a simple message using two techniques.

Procedure

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable.

Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

8. Connect the set-up shown in Figure 2 below.

Note: Insert the black plugs of the oscilloscope leads into a ground (GND) socket.

This set-up can be represented by the block diagram in Figure 3 below. It uses an electronically controlled switch to connect the message signal (the 2kHz SINE output from the Master Signals module) to the output. The switch is opened and closed by the 8kHz DIGITAL output of the Master Signals module.

    - Launch and run the NI ELVIS II Oscilloscope VI.

    - Set up the scope per the procedure in Experiment l (page l-l2) with the following change:

■ Timebase control to the 100ps/divposition instead of 500ps/div

    - Adjust the scope's Timebase control to view two or so cycles of the Master Signals module's 2kHz SINE output.

    - Activate the scope's Channel 1 input by (by checking the Channel 1 Enabled box) to observe the sampled message out of the Dual Analog Switch module as well as the message.

Tip: To see the two waveforms clearly, you may need to adjust the scope so that the two signals are not overlayed.

    - Draw the two waveforms to scale in the space provided on the next page leaving room to draw a third waveform.

Tip: Draw the message signal in the upper third of the graph and the sampled signal in the middle third.

Question 1

What type of sampling is this an example of?

- Natural

- Sample-and-hold

Question 2

What two features of the sampled signal confirm this?

Ask the instructor to check your work before continuing.

    - Modify the set-up as shown in Figure 4 below.

Before you do...

The set-up in Figure 4 below builds on the set-up that you've already wired so don't pull it apart. To highlight the changes that we want you to make, we've shown your existing wiring as dotted lines.

This set-up can be represented by the block diagram in Figure 5 on the next page. The electronically controlled switch in the original set-up has been substituted for a sample-and- hold circuit. However, the message and sampling signals remain the same (that is, a 2kHz sinewave and an 8kHz pulse train).

    - Draw the new sampled message to scale in the space that you left on the graph paper.

Question 3

What two features of the sampled signal confirm that the set-up models the sample- and-hold scheme?

Ask the instructor to check your work before continuing.

Part B - Sampling speech

This experiment has sampled a 2kHz sinewave. However, the message in commercial digital communications systems is much more likely to be speech and music. The next part of the experiment lets you see what a sampled speech signal looks like.

- Disconnect the plugs to the Master Signals module's 2kHz SINE output.

- Connect them to the Speech module's output as shown in Figure 6 below.

Remember: Dotted lines show leads already in place.

- Set the scope's Timebase control to the 500jus/divposition.

- Hum and talk into the microphone while watching the scope's display.

Ask the instructor to check your work before continuing.

Part С - Observations and measurements of the sampled message in the frequency domain

Recall that the sampled message is made up of many sinewaves. Importantly, for every sinewave in the original message, there's a sinewave in the sampled message at the same frequency. This can be proven using the NI ELVIS II Dynamic Signal Analyzer. This device performs a mathematical analysis called Fast Fourier Transform (FFT) that allows the individual sinewaves that make up a complex waveform to be shown separately on a frequency- domain graph. The next part of the experiment lets you observe the sampled message in the frequency domain.

- Return the scope's Timebase control to the 100ps/divposition.

- Disconnect the plugs to the Speech module's output and reconnect them to the Master Signals module's 2kHz SINE output.

Note: The scope should now display the waveform that you drew for Step 15.

- Suspend the scope VI's operation by clicking on its Stop control once.

Note: The scope's display should freeze and its hardware has been deactivated. This is a necessary step as the scope and signal analyzer share hardware resources and so they cannot be operated simultaneously.

- Launch the NI ELVIS II Dynamic Signal Analyzer VI.

Note: If the signal analyzer's VI has launched successfully, the instrument's window will be visible (see Figure 7).

- Adjust the Signal Analyzer's controls as follows: Input Settings

- Source Channel to SCOPE CH1 ■ Voltage Range to+10V

FFT Settings	Averaging

- Frequency Span to 40,000	■ Mode to RMS

- Resolution to 400	■ Weighting to Exponential

- Window to 7 Term B-Harris	■ # of Averages to 3

Trigger Settings

- Type to Edge

Frequency Display

- Units to dB (for now)	■ Cursors On box unchecked (for now)

- Mode to RMS

- Scale to Auto

  - Click on the signal analyzer's Run control.

Note: If the Signal Analyzer VI has been set up correctly, its display should look like Figure 8 below.

If you've not attempted Experiment 7, the signal analyzer's display may need a little explaining here. There are actually two displays, a large one on top and a much smaller one underneath. The smaller one is a time domain representation of the input (in other words, the display is a scope).

The larger of the two displays is the frequency domain representation of the complex waveform on its input (the sampled message). The humps represent the sinewaves and, as you can see, the sampled message consists of many of them. As an aside, these humps should just be simple straight lines, however, the practical implementation of FFT is not as precise as the theoretical expectation.

If you have done Experiment 7, go directly to Step 33 on the next page.

  - Activate the signal analyzer's cursors by checking (that is, ticking) Cursors On box.

Note: When you do, green horizontal and vertical lines should appear on the signal analyzer's frequency domain display.

The NI ELVIS II Dynamic Signal Analyzer has two cursors C1 and C2 that default to the left most side of the display when the signal analyzer's VI is launched. They're repositioned by "grabbing" their vertical lines with the mouse and moving the mouse left or right.

  - Use the mouse to grab and move the vertical line of cursor C1

Note: As you do, notice that cursor C1 moves along the signal analyzer's trace and that the vertical and horizontal lines move so that they always intersect at C1.

  - Repeat Step 27 for cursor C2.

Note: Fine control over the cursors' position is obtained by using the cursor's Position control in the Cursor Settings area (below the display).

The NI ELVIS II Dynamic Signal Analyzer includes a tool that measures the difference in magnitude and frequency between the two cursors. This information is displayed in green between the upper and lower parts of the display.

  - Move the cursors while watching the measurement readout to observe the effect.

  - Position the cursors so that they're on top of each other and note the measurement.

Note: When you do, the measurement of difference in magnitude and frequency should both be zero.

Usefully, when one of the cursors is moved to the extreme left of the display, its position on the X-axis is zero. This means that the cursor is sitting on 0Hz. It also means that the measurement readout gives an absolute value of frequency for the other cursor. This makes sense when you think about it because the readout gives the difference in frequency between the two cursors but one of them is zero.

  - Move C2 to the extreme left of the display.

  - Align C1 with the highest point of any one of the humps.

Note: The readout will now be showing you the frequency of the sinewave that the hump represents.

Recall that the message signal being sampled is a 2kHz sinewave. This means that there should also be a 2kHz sinewave in the sampled message.

  - Use the signal analyzer's C1 cursor to locate sinewave in the sampled message that has the same the frequency as the original message.

Ask the instructor to check your work before continuing.

As discussed earlier, the frequency of all of the sinewaves in the sampled message can be mathematically predicted. Recall that digital signals like the sampling circuit's clock signal are made up out of a DC voltage and many sinewaves (the fundamental and harmonics). As this is a sample-and-hold sampling scheme, the digital signal functions as a series of pulses rather than a squarewave. This means that the sampled signal's spectral composition consists of a DC voltage, a fundamental and both even and odd whole number multiples of the fundamental. For example, the 8kHz sampling rate of your set-up consists of a DC voltage, an 8kHz sinewave (fs), a 16kHz sinewave (2fs), a 24kHz sinewave (3fs) and so on.

The multiplication of the sampling signal's DC component with the sinewave message gives a sinewave at the same frequency as the message and you have just located this in the sampled signal's spectrum.

The multiplication of the sampling signal's fundamental with the sinewave message gives a pair of sinewaves equal to the fundamental frequency plus and minus the message frequency. That is, it gives a 6kHz sinewave (8kHz - 2kHz) and a 10kHz sinewave (8kHz + 2kHz).

In addition to this, the multiplication of the sampling signal's harmonics with the sinewave message gives pairs of sinewaves equal to the harmonics' frequency plus and minus the message frequency. That is, the signal also consists of sinewaves at the following frequencies: 14kHz (16kHz - 2kHz), 18kHz (16kHz + 2kHz), 22kHz (24kHz - 2kHz), 26kHz (24kHz + 2kHz) and so

All of these sum and difference sinewaves in the sampled signal are appropriately known as aliases.

34. Use the signal analyzer's C1 cursor to locate and measure the exact frequency of the sampled signal's first six aliases. Record your measurements in Table 1 below.

Tip: Their frequencies will be close to those listed above.

Ask the instructor to check your work before continuing.

Why aren't the alias frequencies exactly as predicted?

You will have notice that the measured frequencies of your aliases don't exactly match the theoretically predicted values. This is not a flaw in the theory. To explain, the Emona DATEx has been designed so that the signals out of the Master Signals module are synchronised. This is a necessary condition for the implementation of many of the modulation schemes in this manual. To achieve this synchronisation, the 8kHz and 2kHz signals are derived from a 100kHz master crystal oscillator. As a consequence, their frequencies are actually 8.3kHz and 2.08kHz.

Part D - Reconstructing a sampled message

Now that you have proven that the sampled message consists of a sinewave at the original message frequency, it's easy to understand how a low-pass filter can be used to "reconstruct" the original message. The LPF can pick-out the sinewave at the original message frequency and reject the other higher frequency sinewaves. The next part of the experiment lets you do this.

    - Suspend the Signal Analyzer VI's operation by clicking on its Stop control once. Note: The analyzer's display should freeze.

    - Restart the scope's VI by clicking its Run control once.

    - Launch the DATEx soft front-panel (SFP).

    - Check you now have soft control over the DATEx by activating the PCM Encoder module's soft PDM/TDM control on the DATEx SFP.

Note: If you're set-up is working correctly, the PCM Decoder module's LED on the DATEx board should turn on and off.

    - Locate the Tuneable Low-pass Filter module on the DATEx SFP and set its soft Gain control to about the middle of its travel.

    - Turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control fully anti-clockwise.

    - Modify the set-up as shown in Figure 9 below.

The set-up in Figure 9 can be represented by the block diagram in Figure 10 below. The Tuneable Low-pass Filter module is used to recover the message. The filter is said to be "tuneable" because the point at which frequencies are rejected (called the cut-off frequency) is adjustable.

At this point there should be nothing out of the Tuneable Low-pass Filter module. This is because it has been set to reject almost all frequencies, even the message. However, the cut- off frequency can be increased by turning the module's Cut-off Frequency Adjust control clockwise.

42. Slowly turn the Tuneable Low-pass Filter module's soft Cut-off Frequency control clockwise and stop when the message signal has been reconstructed and is roughly in phase with the original message.

Part E - Aliasing

At present, the filter is only letting the message signal through to the output. It is comfortably rejecting all of the other sinewaves that make up the sampled message (the aliases). This is only possible because the frequency of these other sinewaves is high enough. Recall from your earlier measurements that the lowest frequency alias is 6kHz.

Recall also that the frequency of the aliases is set by the sampling signal's frequency (for a given message). So, suppose the frequency of the sampling signal is lowered. A copy of the message would still be produced because that's a function of the sampling signal's DC component. However, the frequency of the aliases would all go down. Importantly, if the sampling signal's frequency is low enough, one or more of the aliases pass through the filter along with the message. Obviously, this would distort the reconstructed message which is a problem known as aliasing.

To avoid aliasing, the sampling signal's theoretical minimum frequency is twice the message frequency (or twice the highest frequency in the message if it contains more than one sinewave and is a baseband signal). This figure is known as the Nyquist Sample Rate and helps to ensure that the frequency of the non-message sinewaves in the sampled signal is higher than the message's frequency. That said, filters aren't perfect. Their rejection of frequencies beyond the cut-off is gradual rather than instantaneous. So in practice the sampling signal's frequency needs to be a little higher than the Nyquist Sample Rate.

The next part of the experiment lets you vary the sampling signal's frequency to observe aliasing.

      - Launch and run the NI ELVIS II Function Generator VI.

      - Adjust the function generator for an 8kHz output.

Note: It's not necessary to adjust any other controls as the function generator's SYNC output will be used and this is a digital signal.

      - Modify the set-up as shown in Figure 11 below.

This set-up can be represented by the block diagram in Figure 12 below. Notice that the sampling signal is now provided by the function generator which has an adjustable frequency.

At this point, the sampling of the message and its reconstruction should be working as before.

        - Set the scope's Timebase control to the 500ps/divposition.

        - Reduce the frequency of the frequency generator's output by 1000Hz and observe the effect this has (if any) on the reconstructed message signal.

Note: Give the function generator time to output the new frequency before you change it again.

        - Disconnect the scope's Channel 1 input from the Tuneable Low-pass Filter module's output and connect it to the Dual Analog Switch module's S&Houtput.

        - Suspend the scope VI's operation.

        - Restart the signal analyzer's VI.

Question 4

What has happened to the sampled signal's aliases?

        - Suspend the signal analyzer VI's operation.

        - Restart the scope's VI.

        - Return the scope's Channel 1 input to the Tuneable Low-pass Filter module's output.

        - Repeat Steps 47 to 53 until the function generator's output frequency is 3000Hz.

Question 5

What's the name of the distortion that appears when the sampling frequency is low enough?

Question 6

What happens to the sampled signal's lowest frequency alias when the sampling rate is 4kHz?

Ask the instructor to check your work before continuing.

        - If you've not done so already, repeat Steps 51 to 53.

        - Increase the frequency of the frequency generator's output in 200Hz steps and stop the when the recovered message is a stable, clean copy of the original.

        - Record this frequency in Table 2 below.

Question 7

Given the message is a 2kHz sinewave, what's the theoretical minimum frequency for the sampling signal? Tip: If you're not sure, see the notes on page 13-18.

Question 8

Why is the actual minimum sampling frequency to obtain a reconstructed message without aliasing distortion higher than the theoretical minimum that you calculated for Question 5?

6- PCM encoding

Experiment 6 - PCM encoding

Preliminary discussion

As you know, digital transmission systems are steadily replacing analog systems in commercial communications applications. This is especially true in telecommunications. That being the case, an understanding of digital transmission systems is crucial for technical people in the communications and telecommunications industries. The remaining experiments in this book use the Emona DATEx to introduce you to several of these systems starting with pulse code modulation (PCM).

PCM is a system for converting analog message signals to a serial stream of Os and Is. The conversion process is called encoding. At its simplest, encoding involves:

- Sampling the analog signal's voltage at regular intervals using a sample-and-hold scheme (demonstrated in Experiment 13).

- Comparing each sample to a set of reference voltages called quantisation levels.

- Deciding which quantisation level the sampled voltage is closest to.

- Generating the binary number for that quantisation level.

- Outputting the binary number one bit at a time (that is, in serial form).

- Taking the next sample and repeating the process.

An issue that is crucial to the performance of the PCM system is the encoder's clock frequency. The clock tells the PCM encoder when to sample and, as the previous experiment shows, this must be at least twice the message frequency to avoid aliasing (or, if the message contains more than one sinewave, at least twice its highest frequency).

Another important PCM performance issue relates to the difference between the sample voltage and the quantisation levels that it is compared to. To explain, most sampled voltages will not be the same as any of the quantisation levels. As mentioned above, the PCM Encoder assigns to the sample the quantisation level that is closest to it. However, in the process, the original sample's value is lost and the difference is known as quantisation error. Importantly, the error is reproduced when the PCM data is decoded by the receiver because there is no way for the receiver to know what the original sample voltage was. The size of the error is affected by the number of quantisation levels. The more quantisation levels there are (for a given range of sample voltages) the closer they are together. This means that the difference between the quantisation levels and the samples is smaller and so the error is lower.

A little information about the PCM Encoder module on the Emona DATEx

The PCM Encoder module uses a PCM encoding and decoding chip (called a codec) to convert analog voltages between -2V and +2V to an 8-bit binary number. With eight bits, it's possible to produce 256 different numbers between 00000000 and 11111111 inclusive. This in turn means that there are 256 quantisation levels (one for each number).

Each binary number is transmitted in serial form in frames. The number's most significant bit (called bit-7) is sent first, bit-6 is sent next and so on to the least significant bit (bit-0). The PCM Encoder module also outputs a separate Frame Synchronisation signal (FS) that goes high at the same time that bit-0 is outputted. The FS signal has been included to help with PCM decoding (discussed in the preliminary discussion of Experiment 15) but it can also be used to help "trigger" a scope when looking at the signals that the PCM Encoder module generates.

Figure 1 below shows an example of three frames of a PCM Encoder module's output data (each bit is shown as both a 0 and a 1 because it could be either) together with its clock input and its FS output.

The experiment

For this experiment you'll use the PCM Encoder module on the Emona DATEx to convert the following to PCM: a fixed DC voltage, a variable DC voltage and a continuously changing signal. In the process, you'll verify the operation of PCM encoding and investigate quantisation error a little.

It should take you about 1 hour to complete this experiment.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Procedure

Part A - An introduction to PCM encoding using a static DC voltage

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable.

Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Function Generator VI.

  - Adjust the function generator for a 1OkHz output.

Note: It's not necessary to adjust any other controls as the function generator's SYNC output will be used and this is a digital signal.

10. Connect the set-up shown in Figure 2 below.

Note: Insert the black plugs of the oscilloscope leads into a ground (6ND) socket.

This set-up can be represented by the block diagram in Figure 3 below. The PCM Encoder module is clocked by the function generator output. Its analog input is connected to 0V DC.

    - Launch the DATEx soft front-panel (SFP).

    - Check you now have soft control over the DATEx by activating the PCM Encoder module's soft POM/TDM control on the DATEx SFP.

Note: If you're set-up is working correctly, the PCM Decoder module's LED on the DATEx board should turn on and off.

    - Locate the PCM Encoder module on the Emona DATEx SFP and set its soft Mode switch to the PCM position.

    - Launch and run the NI ELVIS II Oscilloscope VI.

    - Set up the scope per the procedure in Experiment 1 (page 1-12) with the following changes:

- Scale control for both channels to 2V/div instead of 1V/div

- Coupling control for both channels to DCinstead of AC

- Trigger Level control to 2Vinstead of OV

- Timebase control to 2OOjss/div instead of 5OOjss/div

  - Set the scope's Slope control to theposition.

Setting the Slope control to the "-" position makes the scope start its sweep across the screen when the FSsignal goes from high to low instead of low to high. You can really notice the difference between the two settings if you flip the scope's Slope control back and forth. If you do this, make sure that the Slope control finishes on the "-" position.

  - Set the scope's Timebase control to the 1OOjus/div position.

Note 1: The FSsignal's pulse should be one division wide as shown in Figure 4. If it's not, adjust the function generator's output frequency until it is.

Note 2: Setting the function generator this way makes each bit in the serial data stream one division wide on the graticule's horizontal axis.

  - Activate the scope's Channel 1 input (by checking the Channel 1 Enabled box) to observe the PCM Encoder module's CLK input as well as its FS output.

Tip: To see the two waveforms clearly, you may need to adjust the scope so that the two signals are not overlayed.

  - Draw the two waveforms to scale in the space provided below leaving enough room for a third digital signal.

Tip: Draw the clock signal in the upper third of the graph paper and the FS signal in the middle third.

Ask the instructor to check your work before continuing.

20. Connect the scope's Channel 1 input to the PCM Encoder module's output as shown in Figure 5 below.

Remember: Dotted lines show leads already in place.

This set-up can be represented by the block diagram in Figure 6 below. Channel 1 should now display 10 bits of the PCM Encoder module's data output. Reading from the left of the display, the first 8 bits belong to one frame and the last two bits belong to the next frame.

  - Draw this waveform to scale in the space that you left on the graph paper.

Question 1

Indicate on your drawing the start and end of the frame. Tip: If you're not sure where these points are, see the preliminary discussion.

Question 2

Indicate on your drawing the start and end of each bit.

Question 3

Indicate on your drawing which bit is bit-0 and which is bit-7.

Question 4

What is the binary number that the PCM Encoder module is outputting?

Question 5

Why does the PCM Encoder module output this code for 0V DC and not 0000000?

Ask the instructor to check your work before continuing.

Part В - PCM encoding of a variable DC voltage

So far, you have used the PCM Encoder module to convert a fixed DC voltage (OV) to PCM. The next part of the experiment lets you see what happens when you vary the DC voltage.

- Launch and run the NI ELVIS II Variable Power Supplies VI.

- Set the Variable Power Supplies two outputs to OV.

- Unplug the patch lead connected to the ground socket.

- Modify the set-up as shown in Figure 7 below.

This set-up can be represented by the block diagram in Figure 8 on the next page. The NI ELVIS II Variable Power Supplies is used to let you vary the DC voltage on the PCM Encoder module's input. The scope's external trigger input is used to obtain a stable display.

26. Determine the code on the PCM Encoder module's output.

Tip: Remember, the first eight horizontal divisions of the scope's graticule correspond with one frame of the PCM Encoder module's output.

Note: You should find that the PCM Encoder module's output is a binary number that is reasonably close to the code you determined earlier when the module's input was connected directly to ground.

Ask the instructor to check your work before continuing.

  - Increase the Variable Power Supplies' negative output voltage in -O.1V increments and note what happens to the binary number on the PCM Encoder module's output.

Tip: This is easiest to do by simply typing the required voltage in the field under the negative output's Voltage control. When you do, don't forget to put a minus sign in front of the voltage you enter.

Question 6

What happens to the binary number as the input voltage increases in the negative direction?

  - Determine the first instance of the changing negative voltage that produces the number OOOOOOOO on the PCM Encoder module's output.

  - Record this voltage in Table 1 below.

Ask the instructor to check your work before continuing.

  - Modify the set-up as shown in Figure 9 below.

This set-up can be represented by the block diagram in Figure 10 below.

    - Increase the Variable Power Supplies' positive output voltage in +O.1V increments and note what happens to the binary number on the PCM Encoder module's output.

Question 7

What happens to the binary number as the input voltage increases in the positive direction?

    - Determine the lowest positive voltage that produces the number 11111111 on the PCM Encoder module's output.

    - Record this voltage in Table 2 below.

Question 8

Based on the information in Tables 1 & 2, what is the maximum allowable peak-to-peak voltage for an AC signal on the PCM Encoder module's INPUT?

Question 9

Calculate the difference between the PCM Encoder module's quantisation levels by subtracting the values in Tables 1 & 2 and dividing the number by 256 (the number of codes).

Part C - PCM encoding of continuously changing voltages

Now let's see what happens when the PCM encoder is used to convert continuously changing signals like a sinewave.

    - Close the Variable Power Supplies VI.

    - Disconnect the plugs to the Variable Power Supplies positive output.

    - Modify the set-up as shown in Figure 11 below.

    - Set the function generator's output frequency to 50kHz.

    - Watch the PCM Encoder module's output on the scope's display.

Question 10

Why does the code on PCM Encoder module's output change continuously?

7 - PCM decoding

Experiment 7 - PCM decoding

Preliminary discussion

The previous experiment introduced you to the basics of pulse code modulation (PCM) which you'll recall is a system for converting message signals to a continuous serial stream of binary numbers (encoding). Recovering the message from the serial stream of binary numbers is called decoding.

At its simplest, decoding involves:

- Identifying each new frame in the data stream.

- Extracting the binary numbers from each frame.

- Generating a voltage that is proportional to the binary number.

- Holding the voltage on the output until the next frame has been decoded (forming a pulse amplitude modulation (PAM) version of the original message signal).

- Reconstructing the message by passing the PAM signal through a low-pass filter.

The PCM decoder's clock frequency is crucial to the correct operation of simple decoding systems. If it's not the same frequency as the encoder's clock, some of the transmitted bits are read twice while others are completely missed. This results in some of the transmitted numbers being incorrectly interpreted, which in turn causes the PCM decoder to output an incorrect voltage. The error is audible if it occurs often enough. Some decoders manage this issue by being able to "self-clock".

There is another issue crucial to PCM decoding. The decoder must be able to detect the beginning of each frame. If this isn't done correctly, every number is incorrectly interpreted. The synchronising of the frames can be managed in one of two ways. The PCM encoder can generate a special frame synchronisation signal that can be used by the decoder though this has the disadvantage of needing an additional signal to be sent. Alternatively, a frame synchronisation code can be embedded in the serial data stream that is used by the decoder to work out when the frame starts.

A little information about the DATEx PCM Decoder module

Like the PCM Encoder module on the Emona DATEx, the PCM Decoder module works with 8-bit binary numbers. For 00000000 the PCM Decoder module outputs -2V and for 11111111 it outputs +2V. For numbers in between, the output is a proportional voltage between +2V. For example, the number 10000000 is half way between 00000000 and 11111111 and so for this input the module outputs 0V (which is half way between +2V and -2V).

The PCM Decoder module is not self-clocking and so it needs a digital signal on the CLK input to operate. Importantly, for the PCM Decoder module to correctly decode the PCM data generated by the PCM Encoder module, it must have the same clock signal. In other words, the decoder's clock must be "stolen" from the encoder.

Similarly, the PCM Decoder module cannot self-detect the beginning of each new frame and so it must have a frame synchronisation signal on its FS input to do this.

The experiment

For this experiment you'll use the Emona DATEx to convert a sinewave and speech to a PCM data stream then convert it to a PAM signal using the PCM Decoder module. For this to work correctly, the decoder's clock and frame synchronisation signal are simply "stolen" the PCM Encoder module. You'll then recover the message using the Tuneable Low-pass filter module.

It should take you about 45 minutes to complete this experiment. Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Two BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

- One set of headphones (stereo)

Procedure

Part A - Setting up the PCM encoder

To experiment with PCM decoding you need PCM data. The first part of the experiment gets you to set up a PCM encoder.

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Variable Power Supplies VI.

  - Set the Variable Power Supplies' positive output to OV.

  - Launch the DATEx soft front-panel (SFP) and check that you have soft control over the DATEx board.

  - Set the PCM Encoder module's soft Mode switch to the PCM position.

12. Connect the set-up shown in Figure 1 below.

Note: Insert the black plugs of the oscilloscope leads into a ground (GND) socket.

This set-up can be represented by the block diagram in Figure 2 below. The PCM Encoder module is clocked by the Master Signals module's 100kHz DIGITAL output. Its analog input is the Variable Power Supplies' positive output.

    - Launch and run the NI ELVIS II Oscilloscope VI.

    - Set up the scope per the procedure in Experiment 1 (page 1-12) with the following changes:

- Scale control for both channels to 2V/div instead of 1V/div

- Coupling control for both channels to DCinstead of AC

- Trigger Level control to 2Vinstead of OV

- Timebase control to 10ps/div instead of 500ps/div

  - Set the scope's Slope control to the "-" position.

  - Activate the scope's Channel 1 input (by checking the Channel 1 Enabled box) to observe the PCM Encoder module's PCM DATA output as well as its FS output.

  - Vary the Variable Power Supplies positive output Voltage control left and right (but don't exceed 2.5V).

Note: If your set-up is working correctly, this last step should cause the number on PCM Encoder module's PCM DATA output to go down and up. If it does, carry on to the next step. If not, check your wiring or ask the instructor for help.

  - Close the Variable Power Supplies VI.

  - Launch and run the NI ELVIS II Function Generator VI.

  - Adjust the function generator using its soft controls for an output with the following specifications:

- Waveshape: Sine

- Frequency: 500Hz

- Amplitude: 4Vpp

- DC Offset: 0V

- Disconnect the plug to the Variable Power Supplies' positive output.

22. Modify the set-up as shown in Figure 3 below.

Remember: Dotted lines show leads already in place.

This set-up can be represented by the block diagram in Figure 4 below. Notice that the PCM Encoder module's input is now the function generator's output.

As the PCM Encoder module's input is a sinewave, the module's input voltage is continuously changing. This means that you should notice the PCM DATA output changing continuously also.

Ask the instructor to check your work before continuing.

Part В - Decoding the PCM data

  - Deactivate the scope's Channel 1 input.

  - Modify the set-up as shown in Figure 5 below.

The entire set-up can be represented by the block diagram in Figure 6 on the next page. Notice that the decoder's clock and frame synchronisation information are "stolen" from the encoder.

  - Adjust the scope as follows:

- Scale control for both channels to 1V/div

- Coupling control for both channels to AC

- Trigger Level control to 0V

- Slope control back to the "+" position

- Timebase control to 500ps/div

  - Activate the scope's Channel 1 input to observe the PCM Decoder module's output as well as the message signal.

Question 1

What does the PCM Decoder's "stepped" output tell you about the type of signal that it is?

Tip: If you're not sure, see the preliminary discussion for this experiment or for Experiment 13.

The PCM Decoder module's output signal looks very similar to the message. However, they're not the same. Remember that a "sampled" message contains many sinewaves in addition to the message. The next part of this experiment lets you verify this using the NI ELVIS II Dynamic Signal Analyzer.

  - Suspend the scope's VI.

  - Launch and run the NI ELVIS II Dynamic Signal Analyzer VI.

  - Adjust the signal analyzer's controls as follows:

Input Settings

- Source Channel to SCOPE CH1 ■ Voltage Range to+10V

FFT Settings	Averaging

- Frequency Span to 40,000	■ Mode to RMS

- Resolution to 400	■ Weighting to Exponential

- Window to 7 Term B-Harris	■ # of Averages to 3

Trigger Settings

- Type to Edge

Frequency Display

- Units to dB	■ Cursors On box unchecked (for now)

- Mode to RMS

- Scale to Auto

- Activate the signal analyzer's cursors (by checking Cursors On box).

- Use the signal analyzer's C1 cursor to examine the frequency of the significant sinewaves that make up the sampled message.

- Use the C1 cursor to locate the sinewave in the sampled message that has the same the frequency as the original message.

You have probably just noticed that some of the extra sinewaves in the sampled message are at audible frequencies (that is, between about 20Hz and 20kHz, and in particular those about 12.5kHz). This means that, although the message and sampled messages are similar in shape, you may be able to hear a difference between them. Keep in mind that many of the components below the -40dB line are not significant. Signals that are visually below this level (but bigger numerically) are less than 1% of the level of any signals at 0dB.

Where do these "extra" components come from?

As the data clock rate is 100kHz, the "frame rate" is 1/8th of that because there are 8 bits per frame. Therefore, the PCM system is sampling the message at 12.5kHz or every 80^s.

Importantly, this means that the PCM module's output changes voltage in steps every 80^s. The spectral composition of these steps creates extra "images" of the message at 12.5kHz, 2 x 12.5kHz (25kHz), 3 x 12.5kHz (37.5kHz) and so on. It is these "images" that you can see in the spectrum.

- Add the Amplifier module to the set-up as shown in Figure 7 below leaving the scope's connections as they are.

- Locate the Amplifier module on the DATEx SFP and turn its soft Gain control fully anti- clockwise.

- Without wearing the headphones, plug them into the Amplifier module's headphone socket.

- Put the headphones on.

- Turn the Amplifier module's soft Gain control clockwise until you can comfortably hear the PCM Decoder module's output.

- Listen to how the sampled message sounds and commit it to memory.

- Disconnect the Amplifier module's lead where it plugs to the PCM Decoder module's output.

- Modify the set-up as shown in Figure 8 below, again leaving the scope's connections as they are.

- Compare the sound of the two signals. You should notice that they're similar but clearly different.

Question 2

What must be done to the PCM Decoder module's output to reconstruct the message properly?

Part C - Encoding and decoding speech

So far, this experiment has encoded and decoded a sinewave for the message. The next part of the experiment lets you do the same with speech.

- Close the signal analyzer's VI and restart the scope's VI.

- Adjust the scope so that you can observe two or so cycles of the original and sampled messages again.

Tip: Don't forget to check that the scope's Trigger Source control is set to the CH 0 position.

- Completely remove the Amplifier module from the set-up while leaving the rest of the leads in place.

- Disconnect the plugs to the function generator's output.

- Modify the set-up as shown in Figure 9 below.

- Hum and talk into the microphone while watching the scope's display.

Part D - Recovering the message

As mentioned earlier, the message can be reconstructed from the PCM Decoder module's output signal using a low-pass filter. This part of the experiment lets you do this.

- Locate the Tuneable Low-pass Filter module on the DATEx SFP and set its soft Gain control to about the middle of its travel.

- Turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control fully anti-clockwise.

- Disconnect the plugs to the Speech module's output.

- Modify the set-up as shown in Figure 10 below.

The entire set-up can be represented by the block diagram in Figure 11 on the next page. The Tuneable Low-pass Filter module is used to reconstruct the original message from the PCM Decoder module's PAM output.

- Slowly turn the Tuneable Low-pass Filter module's soft Cut-off Frequency control clockwise and stop the moment the message signal has been reconstructed (ignoring phase shift).

The two signals are clearly the same so let's see what your hearing tells you.

- Add the Amplifier module to the set-up as shown in Figure 12 below leaving the scope's connections as they are.

- Turn the Amplifier module's soft Gain control fully anti-clockwise.

- Put the headphones on.

- Turn the Amplifier module's soft Gain control clockwise until you can comfortably hear the Tuneable Low-pass Filter module's output.

- Commit the recovered message's sound to memory.

- Disconnect the Amplifier module's lead where it plugs to the PCM Decoder module's output and connect it to the function generator's output (in the same way that you did when wiring the set-up in Figure 8).

- Compare the sound of the two signals. You should find that they're very similar.

Question 3

Even though the two signals look and sound the same, why isn't the reconstructed message a perfect copy of the original message? Tip: If you're not sure, see the preliminary discussion for Experiment 14.

Ask the instructor to check your work before finishing.

8 - Bandwidth limiting and restoring digital signals

Experiment 8 - Bandwidth limiting and restoring digital signals

Preliminary discussion

In the classical communications model, intelligence (the message) moves from a transmitter to a receiver over a channel. A number of transmission media can be used for the channel including: metal conductors (such as twisted-pair or coaxial cable), optical fibre and free-space (what people generally call the "airwaves").

Regardless of the medium used, all channels have a bandwidth. That is, the medium lets a range of signal frequencies pass relatively unaffected while frequencies outside the range are made smaller (or attenuated). In this way, the channel acts like a filter.

This issue has important implications. Recall that the modulated signal in analog modulation schemes (such as AM) consists of many sinewaves. If the medium's bandwidth isn't wide enough, some of the sinewaves are attenuated and others can be removed altogether. In both cases, this causes the demodulated signal (the recovered message) to no-longer be a faithful reproduction of the original.

Similarly, recall that digital signals are also made up of many sinewaves (called the fundamental and harmonics). Again, if the medium's bandwidth isn't wide enough, some of them are attenuated and/or removed and this can change the signal's shape.

To illustrate this last point, Figure 1 below shows what happens when all but the first two of a squarewave's sinewaves are removed. As you can see, the signal is distorted.

Making matters worse, the channel is like a filter in that it shifts the phase of sinewaves by different amounts. Again, to illustrate, Figure 2 below shows the signal in Figure 1 but with one of its two sinewaves phase shifted by 40°.

Imagine the difficulty a digital receiver circuit such as a PCM decoder would have trying to interpret the logic level of a signal like Figure 2. Some, and possibly many, of the codes would be misinterpreted and incorrect voltages generated. The makes the recovered message "noisy" which is obviously a problem.

The experiment

For this experiment you'll use the Emona DATEx to set up a PCM communications system. Then you'll model bandwidth limiting of the channel by introducing a low-pass filter. You'll observe the effect of bandwidth limiting on the PCM data using a scope. Finally, you'll use a comparator to restore a digital signal and observe its limitations.

It should take you about 50 minutes to complete this experiment and an additional 20 minutes to complete the Eye-6raph addendum.

Equipment

- Personal computer with appropriate software installed

- NI ELVIS II plus USB cable and power pack

- Emona DATEx experimental add-in module

- Three BNC to 2mm banana-plug leads

- Assorted 2mm banana-plug patch leads

Procedure

Part A - The effects of bandwidth limiting on PCM decoding

As mentioned in the preliminary discussion, bandwidth limiting in a channel can distort digital signals and upset the operation of the receiver. This part of the experiment demonstrates this using a PCM transmission system.

  - Ensure that the NI ELVIS II power switch at the back of the unit is off.

  - Carefully plug the Emona DATEx experimental add-in module into the NI ELVIS II.

  - Set the Control Mode switch on the DATEx module (top right corner) to PC Control.

  - Connect the NI ELVIS II to the PC using the USB cable. Note: This may already have been done for you.

  - Turn on the NI ELVIS II power switch at the rear of the unit then turn on its Prototyping Board Power switch at the top right corner near the power indicator.

  - Turn on the PC and let it boot-up.

  - Launch the NI ELVISmx software.

  - Launch and run the NI ELVIS II Function Generator VI.

  - Adjust the function generator using its soft controls for an output with the following specifications:

- Waveshape: Sine

- Frequency: 20Hz

- Amplitude: 4Vpp

- DC Offset: 0V

- Minimise the function generator's VI.

- Launch the DATEx soft front-panel (SFP) and check that you have soft control over the DATEx board.

- Set the PCM Encoder module's soft Mode switch to the PCM position.

- Connect the set-up shown in Figure 3 below.

This set-up can be represented by the block diagram in Figure 4 below. The PCM Encoder module converts the function generator's output to a digital signal which the PCM Decoder returns to a sampled version of the original signal. Importantly, the patch lead that connects the PCM Encoder module's PCM DATA output to the PCM Decoder module's PCM DATA input is the communication system's "channel".

  - Launch and run the NI ELVIS II Oscilloscope VI.

  - Set up the scope per the procedure in Experiment 1 with the following change: ■ Timebase control to 10ms/div instead of 500ps/div

  - Activate the scope's Channel 1 input to observe the PCM Decoder module's output as well as the PCM Encoder module's input.

Note: If the set-up is working, you should see a 20Hz sinewave for the message and its sampled equivalent out of the PCM Encoder module.

Ask the instructor to check your work before continuing.

  - Locate the Tuneable Low-pass Filter module on the DATEx SFP and set its soft Gain control to about the middle of its travel.

  - Set the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control to about the middle of its travel.

  - Modify the set-up as shown in Figure 5 below.

The set-up in Figure 5 can be represented by the block diagram in Figure 6 below. The Tuneable Low-pass Filter module models bandwidth limiting of the channel.

  - Slowly turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control anti-clockwise.

Tip: Use the keyboard's TAB and arrow keys to make fine adjustment of this control.

  - Stop the moment the PCM Decoder module's output contains the occasional error.

Question 1

What's causing the errors on the PCM Decoder module's output?

Tip: If you're not sure, see the preliminary discussion.

Question 2

If this were a communications system transmitting speech, what would these errors sound like when the message is reconstructed?

22. Reduce the channel's bandwidth further to observe the effect of severe bandwidth limiting of the channel on the PCM Decoder module's output.

Ask the instructor to check your work before continuing.

You have just seen what bandwidth limiting has done to the sampled signal in the time domain so now let's look at what happens in the frequency domain.

    - Increase the channel's bandwidth just until the PCM Decoder's output no-longer contains errors.

    - Suspend the scope VI's operation by clicking on its Stop control once.

    - Launch and run the NI ELVIS II Dynamic Signal Analyzer VI.

    - Adjust the signal analyzer's controls as follows:

Input Settings

- Source Channel to SCOPE CH1 ■ Voltage Range to+10V

FFT Settings	Averaging

- Frequency Span to 1,000	■ Mode to RMS

- Resolution to 400	■ Weighting to Exponential

- Window to 7 Term B-Harris	■ # of Averages to 3

Trigger settings

- Type to Edge

Frequency Display

- Units to dB	■ Cursors On box unchecked (for now)

- Mode to RMS

- Scale to Auto

  - Activate the signal analyzer's cursors (by checking Cursors On box).

  - Use the signal analyzer's C1 cursor to examine the frequency of the sinewaves that make up the sampled message.

  - Use the C1 cursor to locate the sinewave in the sampled message that has the same the frequency as the original message.

  - Reduce the channel's bandwidth so that the PCM Decoder module's output contains occasional errors and observe the effect on the signal's spectral composition.

Tip: Use the signal analyzer's lower display (which is basically a scope) to help you set the level of errors.

  - Reduce the channel's bandwidth so that the PCM Decoder module's output is severely bandwidth limited and observe the effect on the signal's spectral composition.

Question 3

The signal analyzer's trace should now be much smoother than it was before (that is, fewer peaks and troughs). What is this telling you about the spectral composition of the PCM Decoder module's output?

Question 4

These extra sinewaves are heard as noise. Why doesn't the Tuneable Low-pass Filter module remove them?

Ask the instructor to check your work before continuing.

Part B - The effects of bandwidth limiting on a digital signal's shape

You've seen how a channel's bandwidth can upset a receiver's operation. Now let's have a look at how it affects the shape of the digital signal at the receiver's input.

Importantly, digital signals that are generated by a message such as a sinewave, speech or music cannot be used for this part of the experiment. This is because the data stream is too irregular for the scope to be able to lock onto the signal and show a stable sequence of 1s and 0s. To get around this problem the Sequence Generator module's 32-bit sequence is used to model a digital data signal.

  - Close the signal analyzer's VI.

  - Completely dismantle the previous set-up.

  - Set the Tuneable Low-pass Filter module's soft Gain control to about the middle of its travel.

  - Turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control fully clockwise.

  - Locate the Sequence Generator module on the DATEx SFP and set its soft dip-switches to 00.

  - Connect the set-up shown in Figure 7 below.

This set-up can be represented by the block diagram in Figure 8 on the next page. The Sequence Generator module is used to model a digital signal and its SYNC output is used to trigger the scope to provide a stable display.

  - Restart the scope's VI by clicking its Run control once.

  - Adjust the following scope controls:

- Trigger Type control to Digital

- Timebase control to 1ms/div

- Note the effects of making the channel's bandwidth narrower by turning the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control anti-clockwise.

Question 5

What two things are happening to cause the digital signal to change shape? Tip: If you're not sure, see the preliminary discussion.

Ask the instructor to check your work before continuing.

An obvious solution to the problem of bandwidth limiting of the channel is to use a transmission medium that has a sufficiently wide bandwidth for the digital data. In principle, this is a good idea that is used - certain cable designs have better bandwidths than others. However, as digital technology spreads, there are demands to push more and more data down existing channels. To do so without slowing things down requires that the transmission bit rate be increased. This ends up having the same effect as reducing the channel's bandwidth. The next part of the experiment demonstrates this.

- Turn the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control fully clockwise to make the channel's bandwidth as wide as possible (about 13kHz).

- Launch and run the NI ELVIS II Function Generator VI.

- Adjust the function generator for a 2kHz output.

Note: It's not necessary to adjust any other controls as the function generator's SYNC output will be used and this is a digital signal.

- Check that the function generator's Signal Route option (near the bottom right-hand corner of the VI) is set to Prototyping Board.

Note: This must be done to prevent the function generator's output from being sent to the scope's TRIG input and causing a hardware conflict.

- Modify the set-up as shown in Figure 9 below.

Note: As you have set up the function generator's output for a signal that's the same as the Master Signals module's 2kHz DIGITAL output, the signals on the scope shouldn't change.

The set-up in Figure 9 can be represented by the block diagram in Figure 10 below. Notice that the Sequence Generator module's clock is now provided by the function generator's output and so it is variable.

46. To model increasing the transmission bit-rate, increase the function generator's output frequency in 5,000Hz intervals until the clock is about 50kHz.

Tip: As you do this, you'll need to adjust the scope's Timebase control as well so that you can properly see the digital signals.

Question 6

What other change to your communication system distorts the digital signal in the same way as increasing its bit-rate?

Part C - Restoring digital signals

As you have seen, bandwidth limiting distorts digital signals. As you have also seen, digital receivers such as PCM decoders have problems trying to interpret bandwidth limited digital signals. The trouble is, bandwidth limiting is almost inevitable and its effects get worse as the digital signal's bit-rate increases.

To manage this problem, the received digital signal must be cleaned-up or "restored" before it is decoded. A device that is useful for this purpose is the comparator. Recall that the comparator amplifies the difference between the voltages on its two inputs by an extremely large amount. This always produces a heavily clipped or "squared-up" version of any AC signal connected to one input if it swings above and below a DC voltage on the other input.

As you know, ordinarily we avoid clipping but in this case it's very useful. The bandwidth limited digital signal is connected to one of the comparator's inputs and a variable DC voltage is connected to the other. The bandwidth limited digital signal swings above and below the DC voltage to produce a digital signal on the comparator's output. Then, the variable DC voltage is adjusted until this happens at the right points in the bandwidth limited digital signal for the comparator's output to be a copy of the original digital signal.

Unfortunately, this simple yet clever idea has its limitations. First, bandwidth limiting can distort the digital signal too much for the comparator to restore accurately (that is, without errors). Second, the channel can cause the received digital signal (and the hence the restored digital signal) to become phase shifted. For reasons not explained here this can cause other problems for receivers.

This part of the experiment lets you restore a bandwidth limited digital signal using a comparator and observe these limitations.

  - Launch and run the NI ELVIS II Variable Power Supplies VI.

  - Set the Variable Power Supplies' positive output to 0V.

  - Set the scope's Timebase control to the 1ms/divposition.

  - Disconnect the patch lead to the function generator's output then modify the set-up as shown in Figure 11 below.

The entire set-up can be represented by the block diagram in Figure 12 below. The comparator on the Utilities module is used to restore the bandwidth limited digital signal.

  - Compare the signals.

Question 7

Although the restored digital signal is almost identical to the original digital signal, there is a difference. Can you see what it is?

Tip: If you can't, set the scope's Timebase control to the 100{ss/divposition.

Question 8

Can this difference be ignored? Why?

Ask the instructor to check your work before continuing.

    - Return the scope's Timebase control to the 1ms/divposition.

    - Increase the Variable Power Supplies' positive output in 0.2V intervals and observe the effect.

Question 9

Why do some DC voltages cause the comparator to output the wrong information?

Tip: If you're not sure, see the notes on page 16-17.

    - Return the Variable Power Supplies positive output to 0V.

    - Slowly make the channel's bandwidth narrower by turning the Tuneable Low-pass Filter module's soft Cut-off Frequency Adjust control anti-clockwise.

Note: As you do this, the phase difference between the two digital signals will increase but ignore this.

Question 10

Why does the comparator begin to output the wrong information when this control is turned far enough?

    - Make the channel's bandwidth wider and stop when the comparator's output is the same as the original digital signal (ignoring the phase shift).

    - Compare the restored digital signal with the bandwidth limited digital signal by modifying the set-up as shown in Figure 13 below.

Question 11

How can the comparator restore the bandwidth limited digital signal when it is so distorted?

Ask the instructor to check your work before finishing.

Eye diagrams

Regardless of whether the digital data is received from a satellite or the optical head of a CD drive, it's important to be able to inspect and test its distortion (that is, the channel bandwidth & phase characteristics) and degradation (that is, the channel noise). One method of doing so involves using the received digital signal to develop an Eye Diagram.

Eye diagrams can be readily set-up using a stand-alone scope or an Eye Diagram Virtual Instrument if the NI ELVIS II test equipment is being used. For both, multiple sweeps of the scope are overlayed one upon another producing a display much like Figure 1 below.

Figure 1

As you can see, the spaces between the logic-1s and logic-0s produce "eyes" in the centre of the display. Importantly, the greater the effect of bandwidth limiting and phase distortion, the less ideal the logic levels become and so the eyes begin to "close". In addition, channel noise appears as erratic traces across the centre of the eye though a scope with a very long persistence is needed to capture them if the Eye Diagram VI is not being used.

If time permits, this activity gets you to develop an Eye Diagram and observe the effect of noise and bandwidth limiting on its eyes.

- Completely dismantle the existing set-up.

Note: If you're attempting this part of the experiment without having just completed the previous part, perform Steps 1 to 7 on page 16-4 and launch the DATEx SFP.

- Check that the Sequence Generator module's soft dip-switches are set to 00.

- Connect the set-up shown in Figure 2 below.

This set-up can be represented by the block diagram in Figure 3 below.

The Sequence Generator module is used to model a digital signal and its bit-clock is provided by the function generator so the data rate can be varied. An Adder is used to add noise to the digital signal that can be varied from -20dB (lowest) to 0dB (highest). The signal is finally bandwidth limited by the Baseband LPF.

- Launch and run the NI ELVIS II Oscilloscope VI.

- Set up the scope per the procedure in Experiment 1 with the following changes:

- Trigger Type control to Digital

- Timebase control to 1ms/div instead of 500ps/div

  - Activate the scope's Channel 1 input to observe the Sequence Generator module's bit- clock as well as the digital data on the Adder module's output.

  - Set the function generator's output frequency to 2kHz using its soft controls. Note: Once done, you should observe a digital signal with an obvious noise component.

  - Increase the digital signal's noise component to -6dB and observe the effect.

  - Increase the digital signal's noise component to 0dB and observe the effect.

  - Return the digital signal's noise component to -20dB.

  - Modify the set-up as shown in Figure 4 below.

This set-up can be represented by the block diagram in Figure 5 below.

  - Repeat Steps 9 and 10 and observe the effect on the digital signal.

Question 1

Why has the noise disappeared?

Note: Although much of the noise has been removed, this doesn't mean that the digital signal is now unaffected. The remaining noise can still distort the digital signal enough to cause errors at the receiver. You can see the errors for yourself if you compare the signals with - 20dB and 0dB of noise.

    - Set the digital signal's noise component to -6dB.

    - Close the scope's VI.

    - Launch the DATEx Eye-Graph virtual instrument per the instructor's directions.

Note 1: The DATEx Eye-Graph VI will launch running.

Note 2: Within a couple of seconds of the Eye-Graph VI starting, multiple traces of the scope's sweep for Channel 0 (the noisy bandwidth limited digital signal) are written on the VI's screen. This will produce an eye graph similar to the one shown in Figure 1.

Note 3: If this doesn't happen, you may need to select a different device number to the default. See the instructor for help on this.

Ask the instructor to check your work before continuing.

    - Stop the DATEx Eye-Graph VI by clicking its STOP button.

    - Increase digital signal's noise component to 0dB.

    - Run the Eye-Graph VI again and watch it for a couple of minutes to observe the effect.

Question 2

What's the relationship between the size of the eye and the level of noise that the channel introduces to digital signal?

    - Stop the DATEx Eye-Graph VI.

    - Increase the digital signal's data rate by increasing the Sequence Generator module's bit-clock to 4kHz.

Note: By increasing the digital signal's data rate, you'll increase the effect of bandwidth limiting.

    - Run the Eye-Graph VI again and watch it for a couple of minutes to observe the effect.

Question 3

What's the relationship between the size of the eye and the distortion level of the received digital signal?

Ask the instructor to check your work before finishing.

List of recommended literature

Basic

- Звуковое и телевизионное вещание" / Кохно М. Т., Мн.: Экоперспектива, 2001

- Теория электрической связи. Под редакцией профессора Д.Д.Кловского - М.: «Радио и связь», 1998.

- Г. П. Катунин, Г. В. Мамчев, В. П. Попантонопуло, В. П. Шувалов “Телекоммуникационные системы и сети” радиосвязь, радиовешание, телевидение Том 2

Additional

- "Звуковое и телевизионное вещание" / Выходец Коваленко, Кохно М. Т., М.: Радио и связь, 1987

- "Основы радиосвязи, радиовещания и телевидения", составитель Семененко М. И., 2004

- Bystrov U.А. and others. Electric devices and equipment on its base: Reference book. - М.: IP RadioSoft, 2002.

- Electrical techniques and electronics in experiments and exercises: Laboratory practical work on Electronics Workbench: In 2 b. /Under edition of D.I. Panphilov - Т.2: Electronics. - М.: DODEKA, 2000

- Trevis G. LabVIEW for everyone. - М.:DMK Press; PriborKomplect, 2004.

- Page L.I., Tocholin D.А., Pollak B.P. LabVIEW for beginners and specialists. - М.: HotLine – Telecom, 2004.

Sarybaev A.A., Aslanov M.T

LABORATORY PRACTICAL WORK

on the discipline “Systems and devices of sound and television broadcasting”

It is sent for printing _______2013

Format of paper Х*Y 1/16

Paper is typographical. Press is offset. Volume 2 items of l.

Circulation – 50 copies. Order № ___

©  Edition of М. Auezov South Kazakhstan State University

Publishing center of M. Auezov SKSU, Shymkent city,

Tauke-khan avenue, 5